3GPP TSG RAN WG1 #119	R1-2409481

Orlando, US, November 18th – 22nd, 2024

Title: 	Discussion on specification support for AI CSI prediction

Source: 	ZTE Corporation, Sanechips

Agenda item:	9.1.3

Document for:	Discussion/Decision

Introduction

In RAN#105 meeting, a revised WID on AI/ML for air interface was approved [1]. After the study phase in Rel-19, the AI/ML based CSI prediction has been converted into normative phase. The following objectives related to CSI prediction is included in the WID.

- CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025 -------------- Other parts are omitted --------------

Note that the normative work in RAN1 for CSI prediction will start from Q1 of 2025. In the upcoming two meetings in Q4 of 2024, RAN1 will focus on the consistency issue.

Discussion on the consistency issue

Background

In this section, we first wrap up the background agreements/conclusions agreed in Rel-19 AI others agenda and AI beam agenda, which can be the starting point for the study of consistency issue for Rel-19 AI CSI prediction.

In RAN1#116bis meeting, RAN1 agreed the following agreements for consistency issue in others agenda, where step A/B/C is for consistency issue only and step A/B/C/D is for model identification.

Agreement From RAN1 perspective, for UE-sided model(s) developed (e.g., trained, updated) at UE side, following procedure is an example (noted as AI-Example1) of MI-Option1 for further study (including the feasibility/necessity) - A: For data collection, NW signals the data collection related configuration(s) and it/their associated ID(s) - Associated IDs for each sub use case in relation with NW-sided additional conditions - B: UE(s) collects the data corresponding to the associated ID(s) - C: AI/ML models are developed (e.g., trained, updated) at UE side based on the collected data corresponding to the associated ID(s). - D: UE reports information of its AI/ML models corresponding to associated IDs to the NW. Model ID is determined/assigned for each AI/ML model - relationship between model ID(s) and the associated ID(s) - How model ID(s) is determined/assigned, e.g., - Alt.1: NW assigns Model ID - Alt.2: UE assigns/reports Model ID - Alt.3: Associated ID(s) is assumed as model ID(s) - “Model ID is determined/assigned for each AI/ML model” in D is not needed - Alt.4: Model ID is determined by pre-defined rule(s) in the specification - FFS: how to report - Note: D is to facilitate AI/ML model inference - Note: Step A/B/C and additional interaction of associated IDs between UE and NW can be considered as a different solution for resolving the consistency without model identification.

RAN1 agreed the following working assumption for consistency issue in others agenda in RAN1#117 meeting and confirmed it in RAN1#118 meeting. The baseline assumption of associated ID is cell-specific.

Agreement Confirm the following Working assumption. Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study)

In RAN1#118 meeting, RAN1 agreed the following agreements for consistency issue in AI beam agenda. The associated ID can be configured within CSI framework and UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID.

Agreement [UE-sided model, consistency] For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

Simulation results

In order to investigate the necessity of specifying solutions for consistency of training/inference, the first step is to identify the potential additional conditions that impact the performance of UE-side model for AI CSI prediction. According to the study in Rel-19, the following aspects may impact the performance of UE-side model for AI CSI prediction.

- UE speed

- Carrier frequency

- Scenarios

- Observation window

- Prediction window

Regarding the UE speed, it’s UE side additional condition instead of network side additional condition. We are open to discuss this if majority companies prefer to address this issue.

Regarding the carrier frequency, UE can obtain the carrier frequency information via the existing band and BWP configuration. There is no need to introduce associated ID for carrier frequency.

Regarding the scenarios, e.g., UMA or UMI, it has limited performance impact for the UE-side model for CSI prediction based on the simulation results, e.g., majority companies observed less than 2% performance degradation. Furthermore, UE may also identify the scenarios via implementation, e.g., positioning or history information. Thus, there is no need to associated ID for scenarios.

Regarding the observation/prediction window, they can be indicated by the RRC configuration directly instead of introducing associated ID.

In RAN1#118bis meeting, the following two agreements were agreed. Companies are encouraged to provide simulation results at least for 1) Various tilt angle (e.g., [102] degrees, [110] degrees)  and 2) Various TXRU mapping.

Agreement For consistency between training and inference, study to identify which potential NW-side additional conditions, if any, may impact on UE assumption for CSI prediction using UE-sided model, resulting non-negligible degradation on model generalization performance. Note: Companies are encouraged to provide generalization performance evaluation on the impact of NW-side additional conditions, if considered. Agreement For generalization evaluation to identify potential NW-side additional conditions to ensure consistency between training and inference, consider one or more of the following aspects, if evaluated: - [Optional evaluation] Various tilt angle (e.g., [102] degrees, [110] degrees) - [Optional evaluation] Various TXRU mapping - With TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (8x8x2) and (12 x 8 x 2) antenna elements) - Without TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (2x8x2) and (4 x 4 x 2) antenna elements, respectively) - Other configurations are not precluded Note: for other evaluation assumptions, reuse baseline of TR 38.843 Note: report how to map antenna port to antenna elements Note: report the backbone/structure of AI/ML model used

Generalization across different down tilt angles

The simulation results are shown in the following Table 1. Dataset A and dataset B are generated with down tilt angle 102 degrees and 120 degrees, respectively. According to the simulation results, down tilt angle has almost no impact on the SGCS performance, where using a dataset generated from a different angle only incurs 0.28% SGCS impact. Thus, there is no need to consider down tilt angle as additional condition for AI CSI prediction.

Table 1. Simulation results of generalization across different down tilt angles

| Train | Test | SGCS |
| --- | --- | --- |
| Dataset B | Dataset B | 0.9602 |
| Dataset A | Dataset B | 0.9575 |
| Dataset A+B | Dataset B | 0.9726 |
| Note: 32 ports: (8,8,2,1,1,2,8) Dataset A = down tile 102 degrees, Dataset B = down tile 120 degrees |  |  |

Observation 1: AI CSI prediction can generalize well across different down tile angles, where using a dataset generated from a different angle only incurs 0.28% SGCS impact.

Generalization across different TXRU mappings

The simulation results for generalization across different TXRU mappings with TXRU virtualization and without TXRU virtualization are shown in Table 2 and Table 3, respectively. The simulation results for generalization across different TXRU virtualizations are shown in Table 4. Based on the simulation results in Table 2 and 3, it is observed that different TXRU mapping may result with different SGCS performance. Model trained in one TXRU mapping configuration may incur performance loss in another scenario with different TXRU mapping. However, mixed dataset can well address the potential performance loss incurred by different TXRU mappings. Based on the simulation results in Table 4, the model trained with dataset with TXRU mapping can be generalized to the case without TXRU virtualization.

Table 2. Simulation results of generalization across different TXRU mappings with TXRU virtualization

| Train | Test | SGCS |
| --- | --- | --- |
| Dataset B1 | Dataset B1 | 0.9362 |
| Dataset A1 | Dataset B1 | 0.8785 |
| Dataset A1+B1 | Dataset B1 | 0.9531 |
| Note: down tile 102 degrees Dataset A1 = 32 ports: (8,8,2,1,1,2,8) Dataset B1 = 32 ports: (12,8,2,1,1,4,4) |  |  |

Table 3. Simulation results of generalization across different TXRU mappings without TXRU virtualization

| Train | Test | SGCS |
| --- | --- | --- |
| Dataset B2 | Dataset B2 | 0.949 |
| Dataset A2 | Dataset B2 | 0.8825 |
| Dataset A2+B2 | Dataset B2 | 0.9655 |
| Note: down tile 102 degrees Dataset A2 = 32 ports: (2,8,2,1,1,2,8) Dataset B2 = 32 ports: (4,4,2,1,1,4,4) |  |  |

Table 4. Simulation results of generalization across different TXRU virtualizations

| Train | Test | SGCS |
| --- | --- | --- |
| Dataset A1 | Dataset A1 | 0.9541 |
| Dataset A2 | Dataset A1 | 0.9484 |
| Note: Dataset A1 = 32 ports: (8,8,2,1,1,2,8 without TXRU virtualization Dataset A2 = 32 ports: (2,8,2,1,1,2,8) with TXRU virtualization |  |  |

Observation 2: Model trained in one TXRU mapping configuration may incur performance loss in a scenario with different TXRU mapping. However, mixed dataset can well address the potential performance loss incurred by different TXRU mappings.

To summarize, there is no need to consider down tile angle or TXRU mapping as additional condition based on the above simulation results.

Proposal 1: Conclude that neither down tile angle nor TXRU mapping is considered as additional condition for AI CSI prediction because the performance can be guaranteed by model generalization.

Conclusion

Overall, we have the following proposals for AI CSI prediction.

Observation 1: AI CSI prediction can generalize well across different down tile angles, where using a dataset generated from a different angle only incurs 0.28% SGCS impact.

Observation 2: Model trained in one TXRU mapping configuration may incur performance loss in a scenario with different TXRU mapping. However, mixed dataset can well address the potential performance loss incurred by different TXRU mappings.

Proposal 1: Conclude that neither down tile angle nor TXRU mapping is considered as additional condition for AI CSI prediction because the performance can be guaranteed by model generalization.

Reference

- RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN#105.

- 3GPP TR 38.843 V1.3.0, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18).

Appendix A

Table 5-1.  Assumptions for AI/ML based CSI prediction

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD, OFDM |
| Multiple access | OFDMA |
| Scenario | Dense Urban (Macro only) |
| Frequency Range | FR1 only, 2GHz |
| Inter-BS distance | 200m |
| Channel model | According to TR 38.901 |
| Antenna setup and port layouts at gNB | With TXRU virtualization Case1: 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ Case2: 32 ports: (12,8,2,1,1,4,4), (dH,dV) = (0.5, 0.8)λ Without TXRU virtualization Case3: 32 ports: (2,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ Case4: 32 ports: (4,4,2,1,1,4,4), (dH,dV) = (0.5, 0.8)λ Down tilt angle Case 1: 102 degrees Case 2: 120 degrees |
| Antenna setup and port layouts at UE | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) |
| BS Tx power | 41 dBm for 10MHz |
| BS antenna height | 25m |
| UE antenna height & gain | Follow TR36.873 |
| UE receiver noise figure | 9dB |
| UE distribution | 100% outdoor (30km/h) |
| UE receiver | MMSE-IRC |
| Feedback assumption | Realistic |
| Channel estimation | Realistic |

Table 5-2  Training parameters of AI/ML model for CSI prediction

| Parameter | Value |
| --- | --- |
| Backbone | LSTM |
| Parameter type | Real value |
| Input CSI type | Historical channel matrices measured by UE |
| Output CSI type | Predicted channel matrix by AI/ML model in UE |
| Model input size | () |
| Model output size | () |
| Training dataset | 80K |
| Validation dataset | 5K |
| Testing dataset | 5K |
| Batch size | 200 |
| Optimizer | Adam |
| Loss function | MSE |