3GPP TSG RAN WG1 #119			R1-2409505

Orlando, US, November 18th – 22nd, 2024

Source: 	CMCC

Title:	Discussion on CSI enhancements

Agenda item:	9.2.2

Document for:	Discussion & Decision

# Introduction

Rel-19 NR MIMO evolution WI was revised in [1], we will discuss the following objectives in this contribution.

- Specify CSI support for up to 128 CSI-RS ports, targeting FR1

  - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks

  - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s)

  - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design

  - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook

    - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format

    - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI

- Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD

  - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH

# CSI report for up to 128 CSI-RS ports

In Rel-19 MIMO WID, there are three aspects of enhancement on the CSI report for up to 128 CSI-RS ports:

- Type-I/II codebook refinement for up to 128 CSI-RS ports

- CRI based CSI report for hybrid beamforming

- SRS port grouping

In this section, we will discuss the above enhancements respectively.

## Type-I/II codebook refinement

### Soft scaling for RI=2

In RAN1#117 and RAN1#118bis meeting [4] [6], we have the following agreements on the soft scaling feature:

Agreement @RAN1#117 For the Rel-19 Type-I codebook refinement for 48, 64, and 128 CSI-RS ports, for RI=v=1, support the following: - for each group of X1X2 SD basis vectors, a 3-bit scaling factor can be NW-configured via higher-layer (RRC) signalling, where the scaling factors are defined as scalings on the power control offset configured for the associated CSI-RS resources - The values of X1 and X2 for this feature are separately configured from those for CBSR - Separate configuration (RRC signalling) from CBSR - The candidate values of X1 and X2 are the same as those agreed for CBSR - The codepoints of each of the group-specific 3-bit scaling factors are mapped to values of 1, 1/2, 1/3 ,1/4,1/6,1/8,1/12,1/16 - Note: This feature is a separate UE capability FFS: Whether this can be extended to RI=v>1 as well as Type-II codebook refinement Agreement @RAN1#118bis For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding the support for the 3-bit scaling factor(s) for RI=v >1, support only for RI=v=2 without per-SD-basis-vector/layer power adjustment/boosting - FFS: Details on per-layer scaling factor applied to each of the selected SD basis vectors, extending the agreed scaling factor for RI= v =1 si∈1, 12, 13 ,14,16,18,112,116 (in RAN1#117) This feature is a separate UE capability from soft scaling for RI=v=1. Introduce new RRC parameter to enable the feature. Note: This doesn’t preclude the use of power boosting at the NW side by implementation Agreement @RAN1#118bis For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding per-layer scaling factor applied to each of the selected SD basis vectors associated with RI=v ∈{2} for the 3-bit scaling factor(s), decide, by RAN1#119, from the following alternatives: - Alt1: min1υ,si2ri - Alt2: min1υ,si2k∈Girk - Alt3: si2 - Alt4: υsi2ri - Alt5: minriυ,si2 - Alt6: si2υ - Alt7: - if two different vectors for υ=2, min1υ,si2 for the vector with smaller scaling factor si≤sj, and min⁡(1-min⁡(1υ,si2),sj2) for the other vector configured with larger scaling factor sj; - otherwise (υ=1 or one same vector for υ=2), min1,si2 for the vector Where ri denotes the number of layers associated with the i-th SD basis vector. The same scheme applies to both Mode-A and Mode-B. Note: si∈1, 12, 13 ,14,16,18,112,116 as agreed in RAN1#117. Normalization of precoder should be taken into account in the final specification design.

For RI=1, the scaling factors are defined as scalings on the power control offset configured for the associated CSI-RS resources, and the scaling formula is simply as si.

While for RI=2, there are 7 alternatives for the scaling formula, and it can be observed that some alternatives of formula have considered the precoder normalization, while some other haven’t.

However, based on the above agreement, the normalization of precoder will be taken into account in the final specification design, and it will make sure the same transmit power assumption across different RI values, but the transmit power among different RI values can be different. So, in this sense, some alternative involving the factor of RI value can be merged into other similar alternatives:

- Alt 1, Alt 4, Alt 5 merged to Alt 4

- Alt 3, Alt 6 merged to Alt3

However, for Alt7, it uses power borrowing/boosting, which is not aligned with the previous agreement.

So, there will be three further directions for down-selection:

- Direction 1: si

- Direction 2: siri

- Direction 3: sik∈Girk

Considering Direction 1 is just same as RI=1, and some companies’ evaluation results do show Direction 1 have some performance benefits. So, we prefer Direction 1.

Besides, in the pre-RAN1#119 meeting discussion, FL raised an issue on the configuration of the si value, whether the si value is RI-specific or RI-common. The 3-bit indicator per SD basis vector group is raised to restrict the transmit power for some specific beams, not related with the rank value, so it is not natural to say the si value is RI-specific from the perspective of physical meaning. However, we are open to RI-specific si value configure, so that the total power associated with one specific beam for Rank=2 can be same as the power for Rank=1 based on gNB implementation.

Proposal 1: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding per-layer scaling factor applied to each of the selected SD basis vectors associated with RI=v=2 for the 3-bit scaling factor(s):

- Reuse the scaling formula for RI=v=1, i.e. si

  - Reuse legacy precoder normalization

Proposal 2: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding per-layer scaling factor applied to each of the selected SD basis vectors associated with RI=v=2 for the 3-bit scaling factor(s):

- Regarding the configuration of the si value (3-bit indicator per SD basis vector group), prefer RI=1 and RI=2 are separately configured (RI-specific)

### Extension of Type-I SP to <=32 ports

In RAN1#118bis meeting [6], we have the following FL proposal on the extension of Type-I SP to <=32 ports:

Proposal 1.E: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, extend the agreed Scheme-A and Scheme-B to the legacy number of CSI-RS ports (i.e. 4, 8, 12, 16, 24, and 32 ports) for all applicable RI values (1, …, min(PCSI-RS,8)) with K=1 only, and without any further modification/enhancement of the sub-features pertinent to the Rel-19 Type-I SP design (including, e.g. the Rel-19 Type-I SP CBSR, soft scaling). - [The following of Rel-19 Type-I SP are four separate UE features: (1) Scheme-A >32port; (2) Scheme-B >32port; (3) Scheme-A ≤32port; (4) Scheme-B ≤32 port.]

We think extension to R19 Type-I codebook with <=32 ports will not bring extra spec impacts and no other UE implementation efforts with separate UE capability. It can be supported to further improve the performance of Type-I SP codebook with <=32 ports.

Proposal 3: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, extend the agreed Scheme-A and Scheme-B to the legacy number of CSI-RS ports (i.e. 16, 24, and 32 ports) for all applicable RI values (1, …, min(PCSI-RS,8)) with K=1 only, and without any further modification/enhancement of the sub-features pertinent to the Rel-19 Type-I SP design (including, e.g. the Rel-19 Type-I SP CBSR, soft scaling).

- For the Rel-19 Type-I SP codebook, the support for ≤32ports is a separate UE capability from the support for the previously agreed number of ports (48, 64, 128 ports)

# UE reporting enhancement for CJT calibration

Rel-18 MIMO WI specified enhanced Type II codebook to support coherent joint transmission (CJT) assuming ideal time/frequency synchronization and backhaul among TRPs, i.e., it is assumed that the DL timings and carrier frequencies of multiple TRPs are perfectly aligned. However, in practical deployments of CJT, e.g., inter-site CJT, there will be time and frequency misalignments among TRPs due to non-ideal synchronization and backhaul.

Rel-19 MIMO WI will specify the CSI reporting enhancements for CJT deployments under non-ideal synchronization and backhaul, including inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH [1].

The previous RAN1 meetings have almost resolved all major issues on UE reporting enhancement for CJT calibration. In this section, we will discuss some remaining issues regarding the delay/frequency/phase offset reporting for CJT calibration.

## Delay offset reporting

A major issue discussed in the previous two meetings is the linkage of Rel-18 eType-II CJT CSI report and Rel-19 CJTC delay offset (DO) report. The motivation of such linkage is to achieve the PMI calculation for the Rel-18 eType-II CJT CSI report assuming pre-compensation using the UE-reported delay offset. The RAN1#118 meeting discussed two cases where the two reports are separately or jointly triggered and made the following working assumptions [5]. In the last RAN1#118bis meeting, these working assumptions were confirmed with some refinements [6].

Agreement@118 For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, to facilitate UE-specific delay offset pre-compensation on PDSCH by the NW, support configuring a UE (via RRC signaling) to perform PMI calculation for the Rel-18 eType-II CJT CSI report assuming pre-compensation using the UE-reported delay offset (when ReportQuantity is ‘cjtc-Dd’). - The two separately configured reports (i.e. Rel-18 eType-II CJT CSI report and the CJTC delay offset report) can be separately or jointly triggered [and carried on a same PUSCH (hence on a same slot)] following legacy joint triggering mechanism - (Working Assumption) When separately triggered, the delay offset value to be compensated is the latest reported delay offset (DO) whose reporting instance’s last symbol is before the first symbol of DCI triggering of the CJT CSI reporting - FFS: whether some expiration time interval is needed - (Working Assumption) When jointly triggered, the delay offset value to be compensated is the reported delay offset (DO) in the same reporting instance FFS: NW indicates the delay offset value to be compensated for the Rel-18 eType-II CJT CSI report FFS: Whether only AP-CSI-RS, or any type of CSI-RS (P, SP, or AP) can be configured as the CMR for the Rel-18 eType-II CJT reporting FFS: Support for SP-CSI for CJTC report FFS: Whether this is also applicable for Rel-19 Type-I MP codebook The above only applies when the CMRs do not share common QCL source for average delay indication The above is UE optional feature. Agreement@118bis For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured, confirm the following working assumptions as agreement with the following refinement: - When separately triggered, the delay offset value to be compensated is the latest reported delay offset (DO) whose reporting instance’s last symbol is before the first symbol of DCI triggering of the CJT CSI reporting - When jointly triggered, the delay offset value to be compensated is the reported delay offset (DO) in the same reporting instance FFS: Whether an additional UE procedure is needed when the reported DO value is ‘out of range’ FFS: Whether the Dd report codepoints need to be reinterpreted from intervals/ranges to values when the linkage mechanism is configured, or in general, a rule is needed to determine the value of DO for the compensation for each CMR for CJT CSI report (including if multiple Dos are reported)

Regarding the first FFS, the last meeting discussed the following proposal but made no consensus. From our perspective, if the reported DO value corresponding to a TRP is ‘out of range’, this TRP is not expected to be selected in CJT transmission. In Rel-18 eType-II CJT CSI report, dynamic TRP selection can be configured for a UE to select TRPs used for CJT transmission based on measured RSRP values of TRPs. Therefore, an alternative way to go is to enable the dynamic TRP selection at UE side when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured. In this case, when the reported DO value corresponding to a TRP is ‘out of range’, UE will remove the corresponding TRP in Rel-18 eType-II CJT CSI report.

In Moderator Summary#3 on Rel-19 CSI enhancements: Round 3 [7] Proposal 3.C.9: For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with two separate triggers, when at least one of the NTRP reported delay offset (DO) values in a linked CJTC Dd report is ‘out of range’, the UE does not perform DO compensation on the triggered Rel-18 eType-II CJT CSI associated with TRP(s) that are ‘out of range’.

Regarding the second FFS, we fail to see the necessity since it can be addressed by network implementation.

Proposal 4: For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured, support that dynamic TRP selection is enabled for eType-II CJT CSI reporting.

When linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with a joint trigger, it was agreed in the last meeting that the CPU occupation and active resource counting for the Rel-18 eType-II CJT can be reused. One remaining issue is whether to reuse or further relax the timeline for the Rel-18 eType-II CJT CSI report. From our perspective, the baseline is to reuse the timeline for the Rel-18 eType-II CJT CSI since it denotes the worst of the two reports. In this case, no specification impact is involved.

Agreement For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with a joint trigger: - Reuse the CPU occupation and active resource counting for the Rel-18 eType-II CJT - FFS (RAN1#119): Re timeline, decide whether to reuse or further relax the timeline for the Rel-18 eType-II CJT

Proposal 5: For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with a joint trigger, reuse the timeline for the Rel-18 eType-II CJT report.

When linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with a joint trigger, another issue that should be discussed is the UCI mapping order of the two reports in the same PUSCH. When the PUSCH carrying these two CSI reports overlaps with the PUSCH/PUCCH carrying other CSI reports, the legacy omission rules of CSI reports can be followed.

Proposal 6: For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with a joint trigger, the UCI mapping order of CJTC Dd and Rel-18 eType-II CJT CSI reports in the same PUSCH should be discussed.

- Reuse the legacy omission rules of CSI reports when the PUSCH carrying these two CSI reports overlaps with the PUSCH/PUCCH carrying other CSI reports.

In addition, the last meeting agreed to introduce a 1-bit indicator per CSI trigger state to indicate whether or not the UE should perform delay offset (DO) compensation based on the latest linked CJTC Dd report when calculating the Rel-18 Type-II CJT CSI. The corresponding agreement is as follows.

Agreement For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with two separate triggers, to indicate whether or not the UE should perform delay offset (DO) compensation based on the latest linked CJTC Dd report when calculating the Rel-18 Type-II CJT CSI, introduce a 1-bit indicator per CSI trigger state: - The 1-bit indicator is an RRC parameter - This is done without increasing the bit-width of the CSI request field of the DCI triggering a Rel-18 CJT eType-II CJT CSI report - FFS (RAN1#119): Whether the 1-bit indicator applies to all the NTRP CSI-RS resources, or to 1-bit indicator per CSI-RS resource - FFS (RAN1#119): How this applies to a single CSI trigger state associated with >1 CSI reports

Regarding the first FFS, we don’t see the necessity to support a 1-bit indicator per CSI-RS resource. Besides, it is also not clear to us how to realize this based on the 1-bit indicator configured per CSI trigger state. For the second FFS, if a CSI trigger state is associated with multiple CSI reports, the 1-bit indicator should apply to all the CSI reports that are configured to link with a CJTC Dd report.

Proposal 7: For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with two separate triggers, regarding the 1-bit indicator introduced per CSI trigger state, support the following:

- The 1-bit indicator applies to all the NTRP CSI-RS resources.

- The 1-bit indicator should apply to all the CSI reports that are configured to link with a CJTC Dd report.

RAN1#118 meeting also discussed the linkage of CMRs associated with CJTC Dd and Rel-18 eType-II CJT CSI reports and achieved the following agreement. The remaining issue is how to realize the linkage of CMRs when the number of CSI-RS resources configured for CJT CSI is less than the number of TRS resource sets configured for CJTC Dd, in case of separate triggers. Assume that the number of CSI-RS resources configured for CJT CSI is denoted by N1 and the number of TRS resource sets configured for CJTC Dd is denoted by N2, when N1<N2, it should be guaranteed by NW implementation that the first N1 TRS resource sets configured for CJTC Dd are linked to the N1 CSI-RS resources configured for CJT CSI. Regarding the remaining N2-N1 TRS resource sets configured for CJTC Dd, the corresponding delay offsets can be calculated but they do not correspond to the calculation of Rel-18 Type-II CJT CSI.

Agreement For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured, support linking the CMRs in the two CSI Report Settings so that UE knows which CMRs in the two report settings correspond to the same TRP. - Based on a fixed correspondence between the set of TRS resource set IDs in sequential order and the set of CSI-RS resource IDs in sequential order of configuration in their respective Resource Setting FFS: linking, when the number of resources configured for CJT CSI is < number of resource sets configured for CJT Dd, in case of separate triggers

Proposal 8: For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with two separate triggers, if the number of resources configured for CJT CSI report (e.g., N1) is less than the number of resource sets configured for CJTC Dd report (e.g., N2), support the following:

- It should be guaranteed by NW implementation that the first N1 TRS resource sets configured for CJTC Dd report are linked to the N1 CSI-RS resources configured for CJT CSI report.

- The remaining N2-N1 TRS resource sets configured for CJTC Dd report can be used for calculating delay offset only.

In addition, the last RAN1 meeting also discussed the resource type of CMR for the Rel-18 eType-II CJT reporting when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured. The following agreement and conclusion were made.

Agreement For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured, support at least AP-CSI-RS as the CMR for the Rel-18 eType-II CJT reporting - FFS: The support for P/SP CSI-RS Conclusion For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with a joint trigger, there is no consensus in supporting P/SP CSI-RS as the CMR for the Rel-18 eType-II CJT reporting (in addition to the agreed AP CSI-RS).

## Frequency offset reporting

For frequency offset reporting, a remaining issue is whether to support configuring a UE (via RRC signaling) to perform PMI calculation for the Rel-18 eType-II CJT CSI report assuming pre-compensation using the UE-reported frequency offset (when ReportQuantity is ‘cjtc-F’), using the same mechanism as that for UE-reported delay offset (when ReportQuantity is ‘cjtc-Dd’).

Unlike the delay offset, the phase rotation due to frequency offset will accumulate with time after UE reports the frequency offset, thus gNB needs to re-compensate the frequency offset when scheduling the PDSCH. From this perspective, it is unnecessary for UE to perform PMI calculation for the CJT CSI report assuming pre-compensation using the UE-reported frequency offset.

Proposal 9: Do not support configuring a UE (via RRC signaling) to perform PMI calculation for the Rel-18 eType-II CJT CSI report assuming pre-compensation using the UE-reported frequency offset (when ReportQuantity is ‘cjtc-F’).

## DL/UL phase offset reporting

In realistic TDD systems, there would be phase offsets between UL and DL channels due to the non-ideal reciprocity caused by different Tx/Rx RF characteristics. In TDD CJT deployment, there are distinct UL-DL phase offsets over the UL-DL channels for each pair of UE and TRP. With such inter-TRP reciprocity error, the performance of TDD CJT would significantly degrade. Hence, the DL/UL phase offset reporting is supported to assist the NW in compensating for the inter-TRP reciprocity error.

The RAN1#118bis meeting discussed the details for DL/UL phase offset, including the selection of PSRS=1 SRS port, resource type of configured associated SRS resource, UCI mapping order and maximum value of configured NSB-P for subband reporting, active resource counting and CPU occupation, etc. The following agreements were made.

Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), the selection of PSRS=1 SRS port (corresponding to the ‘reference UE antenna port’) out of the y available SRS ports (from an xTyR SRS resource for antenna switching) can be configured per CSI reporting setting. Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the configured associated SRS resource, - Confirm the following working assumption as agreement: “the configured associated SRS resource can be either periodic, semi-persistent, or aperiodic” - When periodic or semi-persistent associated SRS resource is configured, the SRS transmission occasion for determining the reference UE antenna port corresponds to the latest SRS transmission occasion before the occasions of the NTRP CSI-RS resources used for measuring ‘cjtc-P’ report - FFS: Whether ‘the earliest SRS transmission occasion after the NTRP CSI-RS occasions’ is also supported as an option - FFS: Whether determination of SRS transmission occasion is needed for aperiodic associated SRS resource, and if so, how Agreement For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when ReportQuantity is ‘cjtc-P’ (PO) and >1: - The UCI parameters are captured in the tables below: Parameter; Details/description. nref; Reference CSI-RS resource index, based on the ordering from RRC configuration: logNTRP bits. {(n,, n,, n,NSB-P), n=0, 1, …, NTRP – 1, n≠nref}; DL/UL phase offsets for CSI-RS resource n, (n=0, 1, …, NTRP – 1, n≠nref): NTRP-1NSB-P logMΦbits. - The UCI mapping order is as follows: - nref, - {(n,, n,, n,NSB-P), n=0, 1, …, NTRP – 1, n≠nref} ordered from the lowest to highest CSI-RS resource ID Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, regarding active resource counting and OCPU, when ReportQuantity is ‘cjtc-P’ (phase offset), for both =1 and >1, fully reuse those from Rel-18 TDCP reporting OCPU =X.NTRP where X≥1 is defined based on UE capabilities and determined by the UE Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), - after the CSI report (re)configuration, serving cell activation, BWP change, the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for each of the CSI-RS resources in the corresponding CSI-RS Resource Set for channel measurement no later than the CSI reference resource within the same DRX active time, when DRX is configured, and drop the report otherwise. Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the support of sub-band reporting (>1), the maximum value of configured NSB-P is 16 per CSI reporting setting

Regarding the configured associated SRS resource, it was agreed that when periodic or semi-persistent associated SRS resource is configured, the SRS transmission occasion for determining the reference UE antenna port corresponds to the latest SRS transmission occasion before the occasions of the NTRP CSI-RS resources used for measuring ‘cjtc-P’ report. In the last meeting, it was proposed that ‘the earliest SRS transmission occasion after the NTRP CSI-RS occasions’ also can be supported. However, in the case of precoded CSI-RS transmission scheme, gNB needs to calculate the precoder of CSI-RS based on the estimation of SRS, so the SRS transmission occasion for determining the reference UE antenna port should be prior to the NTRP CSI-RS occasions.

Another remaining issue that should be addressed in this meeting is whether determination of SRS transmission occasion is needed for aperiodic associated SRS resource. Considering that the aperiodic associated SRS resource can be triggered multiple times, determination of SRS transmission occasion is also needed. To be specific, the SRS transmission occasion for determining the reference UE antenna port corresponds to the most recent transmission of the aperiodic associated SRS resource before the occasions of the NTRP CSI-RS resources used for measuring ‘cjtc-P’ report.

Proposal 10: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the configured associated SRS resource, support the following:

- When aperiodic associated SRS resource is configured, the SRS transmission occasion for determining the reference UE antenna port corresponds to the most recent transmission of the aperiodic associated SRS resource before the occasions of the NTRP CSI-RS resources used for measuring ‘cjtc-P’ report.

# Conclusions

In this contribution, we discuss CSI enhancements in MIMO, including support for up to 128 CSI-RS ports on FR1 and UE reporting enhancement for CJT calibration, and the following proposals are made.

Proposal 1: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding per-layer scaling factor applied to each of the selected SD basis vectors associated with RI=v=2 for the 3-bit scaling factor(s):

- Reuse the scaling formula for RI=v=1, i.e. si

  - Reuse legacy precoder normalization

Proposal 2: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding per-layer scaling factor applied to each of the selected SD basis vectors associated with RI=v=2 for the 3-bit scaling factor(s):

- Regarding the configuration of the si value (3-bit indicator per SD basis vector group), prefer RI=1 and RI=2 are separately configured (RI-specific)

Proposal 3: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, extend the agreed Scheme-A and Scheme-B to the legacy number of CSI-RS ports (i.e. 16, 24, and 32 ports) for all applicable RI values (1, …, min(PCSI-RS,8)) with K=1 only, and without any further modification/enhancement of the sub-features pertinent to the Rel-19 Type-I SP design (including, e.g. the Rel-19 Type-I SP CBSR, soft scaling).

- For the Rel-19 Type-I SP codebook, the support for ≤32ports is a separate UE capability from the support for the previously agreed number of ports (48, 64, 128 ports)

Proposal 4: For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured, support that dynamic TRP selection is enabled for eType-II CJT CSI reporting.

Proposal 5: For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with a joint trigger, reuse the timeline for the Rel-18 eType-II CJT report.

Proposal 6: For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with a joint trigger, the UCI mapping order of CJTC Dd and Rel-18 eType-II CJT CSI reports in the same PUSCH should be discussed.

- Reuse the legacy omission rules of CSI reports when the PUSCH carrying these two CSI reports overlaps with the PUSCH/PUCCH carrying other CSI reports.

Proposal 7: For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with two separate triggers, regarding the 1-bit indicator introduced per CSI trigger state, support the following:

- The 1-bit indicator applies to all the NTRP CSI-RS resources.

- The 1-bit indicator should apply to all the CSI reports that are configured to link with a CJTC Dd report.

Proposal 8: For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with two separate triggers, if the number of resources configured for CJT CSI report (e.g., N1) is less than the number of resource sets configured for CJTC Dd report (e.g., N2), support the following:

- It should be guaranteed by NW implementation that the first N1 TRS resource sets configured for CJTC Dd report are linked to the N1 CSI-RS resources configured for CJT CSI report.

- The remaining N2-N1 TRS resource sets configured for CJTC Dd report can be used for calculating delay offset only.

Proposal 9: Do not support configuring a UE (via RRC signaling) to perform PMI calculation for the Rel-18 eType-II CJT CSI report assuming pre-compensation using the UE-reported frequency offset (when ReportQuantity is ‘cjtc-F’).

Proposal 10: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the configured associated SRS resource, support the following:

- When aperiodic associated SRS resource is configured, the SRS transmission occasion for determining the reference UE antenna port corresponds to the most recent transmission of the aperiodic associated SRS resource before the occasions of the NTRP CSI-RS resources used for measuring ‘cjtc-P’ report.

# References

- RP-242394, WID revision: NR MIMO Phase 5, RAN#105, Melbourne, Australia, September 9-12, 2024.

- Chairman’s Notes RAN1#116, Athens, Greece, February 26th - March 1st, 2024.

- Chairman’s Notes RAN1#116-bis, Changsha, Hunan Province, China, April 15th – 19th, 2024.

- Chairman's Notes RAN1#117, Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024.

- Chairman's Notes RAN1#118, Maastricht, NL, August 19th – 23rd, 2024.

- Chairman's Notes RAN1#118bis, Hefei, China, October 14th – 18th, 2024.

- R1-2409059, Moderator Summary#3 on Rel-19 CSI enhancements: Round 3, RAN1#118bis, Hefei, China, October 14th – 18th, 2024.