3GPP TSG RAN WG1 #119 	                                           R1-2409527

Orlando, US, November 18th – 22nd, 2024

Source: 	CMCC

Title:	Discussion on NR-NTN DL coverage enhancement

Agenda item:	9.11.1

Document for:	Discussion & Decision

# Introduction

During the RAN#102 meeting, a new work item was proposed for further enhancements for NR NTN Phase 3 [1]. One of the objectives is to focus on the downlink coverage enhancements for NR NTN, and the objective was updated in the RAN#104 meeting for further clarification on the scope of link-level and system-level enhancements, which is shown as below [2].

Objective The work item aims at specifying further enhancements for NG-RAN based NTN (Non-Terrestrial Networks) with the following assumptions: - GSO (Geo Synchronous Orbit) and NGSO (Non-Geo Synchronous Orbit). NGSO includes Low Earth Orbit (LEO) and Medium Earth Orbit (MEO). - Earth fixed tracking area. Earth fixed & Earth moving cells for NGSO - FDD mode - UEs with GNSS (Global Navigation Satellite Systems) capabilities - In frequency band above 10 GHz, both Terminal Type 1 (Electronic steering antenna) and Type 2 (Mechanical steering antenna) to be considered for GSO and NGSO - Implicit compatibility to support HAPS (High Altitude Platform Station) and ATG (Air To Ground) scenarios, where relevant Note 1: In Rel-19 WID, “VSAT” device with external antenna on moving platform is equivalent to a device that operate on platforms in motion, and this is referred to as ESIM (Earth Station In Motion). The objectives of the work item are the following: - Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

During last RAN1 meeting, detailed coverage enhancements for both system-level study and link-level study have been discussed, and some progress on supporting extended SSB periodicity and PDCCH/PDSCH link level enhancements are achieved [3]. In this contribution, we will further share our views on the impacts of SSB periodicity extension and potential DL coverage enhancements, including system-level enhancements and detailed link-level enhancements of bottleneck DL channels/signals based on the LLS	results.

# Discussion

## 2.1 System-level study

For system level study of NTN DL coverage, several key issues, including the reference satellite parameters assumptions, antenna model, evaluation metrics and traffic models have been discussed, some progress on baseline configuration, assumptions and DL coverage ratio evaluation were achieved. For LEO-600 in FR1 band, three sets of reference satellite parameters are assumed for different NTN deployment scenarios, referred to Set1-1 FR1, Set1-2 FR1 and Set1-3 FR1 listed below [4].

RAN1#116 Agreement For DL coverage study, consider the following additional reference satellite parameters scenarios for LEO600km Set1 in FR1 (i.e., S-band), referred to as Set1-1 FR1, Set1-2 FR1 and Set1-3 FR1: LEO600km Set1-1 FR1 (i.e., S-band). Maximum Bandwidth per beam; 5 MHz. SCS; 15 kHz. Beam size(Note 1); 50km. Satellite EIRP density /beam (dBW/MHz); 34. Payload Total DL power level (dBW); 31.24. Aggregated EIRP (Total) (dBW); 61.24*. Satellite Tx max Gain; 30 dBi. Maximum EIRP per Satellite beam (dBW); 41. Total number of beam footprints***; 1058. Total number of simultaneously active beams **; 106. % simultaneously active beams**; 10.02 %. *Note: EIRP limit is 61.24 dBm for the reference configuration. **Assuming 100 % Resource Block utilization within the same beam at max power. Absolute number of simultaneously active beams is up to 212 (due to limitation of RF) *** For a constellation design at 600km with low elevation angle with 30° and selected (i.e Set 1 parameters) beam size Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies. LEO600km Set1-2 FR1 (i.e., S-band). Maximum Bandwidth per beam; 5 MHz. SCS; 15 kHz. Beam size (note 1); 50km. Satellite EIRP density /beam (dBW/MHz); 34. Payload Total DL power level (dBW); 23. Aggregated EIRP (Total) (dBW); 53*. Satellite Tx max Gain; 30 dBi. Maximum EIRP per Satellite beam (dBW); 41. Total number of beam footprints; 1058. Total number of simultaneously active beams**; 16. % simultaneously active beams**; 1.5 %. *Note: EIRP limit is 53 dBm for the reference configuration. **Absolute number of simultaneously active beams is up to 16 (due to limitation of RF) Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies. LEO600km Set 1-3 FR1 (i.e., S-band). Maximum Bandwidth per beam; 5 MHz. SCS; 15 kHz. Beam size (note 1); 50km. Satellite EIRP density /beam (dBW/MHz); 26. Payload Total DL power level (dBW); 23.24. Aggregated EIRP (Total) (dBW); 53.24*. Satellite Tx max Gain; 30 dBi. Maximum EIRP per Satellite beam (dBW); 33. Total number of beam footprints; 1058. Total number of simultaneously active beams**; 106. % simultaneously active beams**; 10.02 %. *Note: EIRP limit is 53.24 dBm for the reference configuration. **Absolute number of simultaneously active beams is up to 212 (due to limitation of RF) Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies. Note: RAN1 will aim to identify necessary enhancements for these scenarios in the study phase. At the end of the study phase, RAN1 will further discuss whether the potential enhancements will be specified within Rel-19 framework.

### 2.1.1 SSB periodicity extension

Three sets of reference satellite parameters present different satellite capabilities and using scenarios. LEO-600km Set 1-1 FR1 represents the case which has a relatively large number of simultaneously active beams, i.e. 106 beams, to cover 1058 planned beam footprints on the earth. Set 1-2 FR1 has the same DL transmit power as Set 1-1, but with a smaller number of simultaneously active beams, i.e. 16 beams, due to the limitation of satellite payload. The intended coverage area of Set 1-2 is the same as that of Set 1-1, which is also 1058 beam footprints on the earth. Set 1-3 FR1 has the same number of the simultaneously active beams as Set 1-1, but with a lower transmit power at the satellite side, which may cause further link level coverage issue for the DL coverage.

For the three satellite parameter sets, the ratio of active beam is limited by 10% or worse, by 1.5% due to limitation of the hardware and the payload of satellites. It is observed that even if all the active beams are used for common channels, it cannot satisfy 100% coverage ratio with the default SSB periodicity. Therefore, increasing SSB periodicity to improve DL coverage ratio and reduce control channel overhead has been studied, and the following observations are achieved during previous RAN1 meeting [5], [6].

RAN1#117 Observation Based on the results of DL coverage ratio evaluation at system level collected from 7 sources for all the three LEO600km satellite parameter sets where the beam footprint diameter is 50 km: - For Set 1-1/1-3, the coverage ratio can be improved from 10% to 100% if the SSB periodicity is increased from 20ms to 80ms and beam hopping is applied - For Set 1-2, the coverage ratio can be improved from 1.5% to 96.8% if the SSB periodicity is increased from 20ms to 320ms and beam hopping is applied. - Note: coverage ratio is N2+N3/ total beam footprints - Note: the baseline assumes no beam hopping. TDM between SIB1 and SIB19 is assumed in those results, following current specs. Based on the results of DL coverage ratio evaluation at system level collected from 3 sources for a deployment scenario implementing wide beam footprint: - 1 source reports that with a deployment of wide beam covering 4 narrow (of 50km size) beams, which means Set 1-2 FR1 with additional EIRP reduction of 6dB, using SSB periodicity of 80 ms can provide coverage ratio of 96.8%, and Set 1-1/1-3 FR1 with additional EIRP reduction of 6dB, SSB periodicity of 80 ms can provide coverage of 100%. - 1 source observed that for Set 1-1, 1-2 and 1-3, the coverage ratio can be improved from 1.5% to 100% using the legacy default SSB periodicity of 20ms during initial access, by choosing a wide beam footprint with beam footprint sizes of 84 km and 56 km respectively. - Note: the PDCCH and the PDSCH for SIB19 is assumed to be transmitted within 2 OFDM symbols and 5 MHz bandwidth. the PDSCH for SIB1 is assumed to be transmitted within 3 OFDM symbols and 5 MHz bandwidth. This assumes no SIB1 and SIB19 transmission in N2 beam footprints. This assumes non-aligned SFN timing across different beams. - 1 source observed, for Set 1-1 with increased beam size, that the legacy SSB periodicity of 20ms during initial access is usable with NTN beam hopping, by choosing a deployment scenario implementing wide beam footprint with beam footprint sizes of 70.7 km and 86.6 km, leading to a total of 529 and 353 beam footprints within the satellite coverage area, respectively, and the coverage ratio is 80% and 90%, respectively, and a ratio of simultaneously active beam footprints to the total number of beam foot prints equal to 20% and 30%. - Note: Beam footprint size is increased by increasing only the adjacent beam spacing without increasing the 3dB beamwidth. Note: RAN1 will further investigate the impact of SSB periodicity extension Note: Any needed clarification “SSB channel enhancement is not considered” in the WID is up to RAN plenary Note: RAN1 will further investigate the impact of wider beam of SSB and/or other channels on performance (e.g. link budget, capacity...) RAN1#118 Observation Based on the results of DL coverage evaluation at system level collected from different sources, it is observed that extending the default value of SSB periodicity (different from 20ms) in NTN with LEO600km satellite parameter sets where the beam footprint diameter is 50 km, is beneficial in terms of reduction of common control channel overhead, when targeting a full coverage of 1058 beam footprints: - With Set 1-1 FR1 and Set 1-3 FR1, the common messages (SSB, SIB1) overhead is around 40% assuming 5 MHz BW when SSB/SIB1 periodicity of 20ms is in use, this overhead ratio could be reduced to less than 14% when 160ms SSB/SIB1 periodicity is used. - With Set 1-2 FR1, the common message (SSB, SIB1) overhead is greater than 100% assuming 5 MHz BW when SSB/SIB1 periodicity of 20ms is in use, this overhead could be reduced to around 25.8% when 640ms SSB/SIB1 periodicity is used. - Note: the overhead of SIB19 was included in some of the results - Note: an observation when SSB/SIB1 periodicity is 320 ms will be discussed and added to the observation

Based on the analytical evaluation results, it can be observed that coverage ratio is improved with longer duration of SSB periodicity from UE side. Extension of the periodicity of SSBs is a prerequisite for reaching almost 100% coverage ratio for all the scenarios using the parameter Set 1-1/1-2/1-3 FR1. Besides, based on the evaluation results of common control channel overhead with different default SSB/SIB1 periodicity summarized in the last RAN1 meeting, it is observed that extending SSB periodicity is also beneficial for reducing common channel overhead, which can be reflected when 160ms or larger SSB periodicity is used.

Furthermore, the determination of default value(s) of SSB periodicity if SSB periodicity extension is supported for NTN scenarios has been discussed. According to the agreement achieved in the RAN1#118 meeting, several candidate values of extended periodicity are provided. And in the last RAN1 meeting, it is suggested to take 160ms as the default value of extended SSB periodicity, which can be used to improve coverage ratio with less specification works. Whether 320ms of default value can be supported needs to be further discussed, corresponding agreements are listed as below.

RAN1#118 Agreement As part of the NTN DL coverage enhancements at both system level and link level, RAN1 to consider: - Extending the periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - Default value[s] with extended periodicity assumed by NTN UE for initial access can be: - One [or more] values from the list {40ms, 80 ms, 160 ms, 320ms, 640ms} - Potential enhancements for transmitting the DL common channels using a wider beam footprint, while DL/UL dedicated channels (incl. PRACH) may be transmitted using a narrower beam footprint - Link-level enhancements for the following channels: - PDCCH - PDSCH with Msg 4 - PDSCH with SIB1/SIB19. - Note: link-level enhancements for PDSCH with SIB1/SIB19 may be applicable to other SIBs, without additional specification impact. - Note: the above does not imply that all the channels above will be enhanced, but all of them should be considered based on this agreement RAN1#118bis Agreement For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is at least 160 ms. - FFS: whether 320ms can be supported as the maximum of the additional default value instead of or in addition to 160ms

To identify the default value of extended SSB periodicity, the coverage ratio with different values of SSB is summarized as in Table 1 based on the current assumptions on total number of beam footprints and number of active beams.

Table 1. System level evaluation for LEO-600 Set1-1/1-2/1-3

| Cases | Case 1 | Case 2 | Case 3 | Case 4 | Case 5 | Case 6 | Case 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Satellite parameter set | Set1-1/1-3 | Set1-1/1-3 | Set1-1/1-3 | Set 1-2 | Set 1-2 | Set 1-2 | Set 1-2 |
| Total number of beam footprints | 1058 | 1058 | 1058 | 1058 | 1058 | 1058 | 1058 |
| Total number of simultaneously active beams | 106 | 106 | 106 | 16 | 16 | 16 | 16 |
| SSB Periodicity | 20ms | 40ms | 80ms | 20ms | 80ms | 160ms | 320ms |
| Number of beam footprints being served | 424 | 828 | 1058 | 64 | 256 | 512 | 1024 |
| Coverage ratio (number of beam footprints being served/total number of beam footprints) | 40.08% | 80.16% | 100% | 6.05% | 24.2% | 48.4% | 96.8% |

To ensure UE access the network successfully, more beam footprints are expected to be served with SSBs and necessary system information. Assuming that 4 SSBs per SSB burst is transmitted within SSB periodicity and one-shot SSB detection is performed without any enhancement of power reduction or wide beam. If the default SSB periodicity is extended to 80ms or longer duration, 100% coverage ratio can be satisfied for Set1-1/1-3 FR1. While only 24.2% of beam footprints can be served by 16 active beams for Set 1-2 FR1 with 80ms SSB periodicity. And when the SSB periodicity of Set 1-2 is extended to 320ms, 96.8% coverage ratio can be achieved. To achieve 100% coverage for all beam footprints, 640ms of default SSB periodicity from UE side may be needed.

From our perspective, for NTN UEs, if the UE needs to connect to the satellites, the default periodicity should be at least 320ms for all the 3 implementation scenarios. When default SSB periodicity as 320ms is adopted in the scenario of Set 1-1/1-3, and the actual SSBs are transmitted by the gNB in every 80ms, the impact of the power consumption for the detection of SSB is marginal, since the UE will finally find the SSB in an 80ms period. The power consumption is almost the same as if the default period of detection is configured as 80ms and the UE is working under the set 1-1/1-3 scenarios. Then it is proposed to extend the default detection periodicity for SSB in NTN scenarios to 320ms.

Observation 1: Without enhancement of power reduction or wider beam, to reach an almost 100% coverage ratio, the default detection period for SSBs in NTN scenarios needs to be extended to 320ms for all cases of Set1-1/1-2/1-3.

Considering whether to support more than one default value of extended SSB periodicity, from our understanding, the default SSB periodicity is a predefined transmission pattern that allows the UE to efficiently search for and identify the SSBs within the serving cell. For initial cell selection, a legacy UE may assume that half frames with SS/PBCH blocks occur with a periodicity of 2 frames. While for Rel-19 NTN coverage enhancements, it is efficient to extend the default SSB periodicity and the UEs scan the frequency resources based on the extended SSB periodicity. If multiple default SSB periodicities are specified, since the network configuration of periodicity for reception of the SSB is not provided prior to initial cell search, a criterion needs to be defined to enable UE to identify a specific SSB periodicity, which will increase the complexity of UE implementation. Thus, the necessity of defining more than one default value of extended SSB periodicity needs to be clarified.

Proposal 1: To support SSB periodicity extension for improving DL coverage ratio and reducing common control channel overhead, one default value of {160ms, 320ms} can be considered, and 320ms is preferred to achieved almost 100% coverage ratio.

Proposal 2: It needs to be clarified on UE’s behavior when multiple default values are supported, e.g. how to identify which default value can be used for initial access under one beam footprint or frequency bands.

In the current specification, legacy UEs assume the 20ms default detection periodicity for SSBs. Considering the impact to backward compatibility when SSB periodicity is extended in NTN scenarios, different SSB periodicities can be supported for covering different beam footprints by dynamic beam scheduling. That is, the legacy UEs will be served by some beam footprints being illustrated with 20ms period, and longer default SSB detection period can be used for other beam footprints serving NTN UEs.

Proposal 3: Different SSB periodicities can be used to solve the backward compatibility issue, wherein one set of legacy 20ms SSB periodicity can be reserved for legacy NTN UE in some beam footprints, and other sets of longer SSB periodicities can be used to serve Rel-19 NTN UE in other beam footprints.

### 2.1.2 Potential system-level enhancements

As mentioned in the objectives of DL coverage enhancement in NR NTN, system level coverage enhancement via efficient power sharing is proposed for further study. Currently it assumes that a large amount of DL beams from satellites will be configured, but only a limited portion of beams can be activated simultaneously. In order to provide services to all UEs covered by the satellite footprint, TDM beam scheduling/beam hopping can be performed to improve the network throughputs.

Considering system level enhancement with restricted number of simultaneously active beams and power sharing among active beams, the following potential schemes can be further discussed to improve the coverage performance.

- Time domain enhancements

  - Long revisit period and dwelling window for each beam should be defined to improve the coverage ratio

  - Extending the default periodicity for common channels/message (e.g., SSB, SIB1, SIB19)

- Spatial domain enhancements

  - Broader/wide beam size to serve larger area based on the link level evaluation

As analyzed in the Section 2.1.1, the SSB periodicity for Set 1-1/1-2/1-3 needs to be extended, and nearly 100% coverage ratio can be achieved for Set 1-2 when the SSB periodicity is 320ms. Furthermore, if larger default SSB periodicity than 20ms is supported, the periodicity of common control channel, necessary system information and UE measurement needs to be extended correspondingly, to ensure that the initial access process can be completed during the duration when the beam footprint is illuminated.

Proposal 4: Considering the extended SSB periodicity, the periodicity of common message (e.g. CORESET0/SIB1, SIB19) and UE measurement needs to be extended correspondingly.

Combined with long detection period of SSBs and important common control channel for initial access procedure, a revisit period and a dwelling window should be introduced to facilitate the operation under NTN scenarios. It is proposed to support the system level enhancements to improve the coverage ratio in NR-NTN phase 3, at least considering to introduce the revisit periodicity and dwelling window.

From our understanding, the revisit time of the beam to serve each beam footprint is determined by beam hopping pattern and residence time on the beam footprint. Assuming that each footprint is uniformly illuminated based on TDM beam scheduling for common channels, according to the coverage ratio analysis in Table 1, if default 20ms SSB periodicity is assumed, since maximum 40.08% coverage of common control channels (e.g. SSB, CORESET0/SIB1, SIB19) can be satisfied for LEO-600 Set1-1/1-3, longer revisit time than 20ms is needed in order to serve all the 1058 beam footprints.

Observation 2: For LEO-600 Set1-1/1-2/1-3, longer revisit period can be considered to improve the coverage ratio.

Proposal 5: Support the system level enhancements to improve the coverage ratio in NR-NTN phase 3, with the consideration of introducing the revisit periodicity and dwelling window.

Furthermore, if extension of SSB/SIB1 periodicity in NTN scenarios is supported, the allocation of SSB and necessary system information for each satellite beam or beam footprint need to be further studied. Taking the satellite parameters Set 1-2 as an example, the operation and necessary signals/channels are illustrated in the Figure 1. 4 SSBs are supported for the S-band. Each SSB can provide DL synchronization over one beam footprint. With one of the 16 simultaneously active beams from Set 1-2, 4 SSBs can be transited at different time instance into 4 different beam footprints. And within one beam footprint, the SIB1 and SIB19 can be transmitted following the same SSB providing the system information and the necessary information for initial access. The SSBs, the related SIB1 and SIB19 can be transmitted in 4 beam footprints in a TDM manner. The transmission of those information over 4 beam footprints will occupy at least 10ms of the 20ms within dwelling window. Then the residual time-domain resources not used for SSB, SIB1 and SIB 19 (e.g., the last 10ms of the dwelling window) can be used for the dynamic scheduling depending on the traffic, such as scheduling for the same beam footprints as SSB or other beam footprints. For the initial access for the uplink, after UE successfully decoded the information of SIB19, the UE can transmit the PRACH at corresponding RACH occasion associated with the specific SSB index within the dwelling window. Excluding the RACH occasions whose beam direction should be determined according to the associated DL SSB, the other uplink slots can be used dynamically, including the last 10ms of the 20ms dwelling window. Considering that one of the 16 simultaneously active beams can cover 4 beam footprints over 20ms dwelling window, it needs 16 time-domain occasions for each of 16 simultaneous active beam to cover all candidate 1058 beams to reach a 96.8% coverage ratio. Based on the assumption above, the active beam will periodically visit one beam footprint in a 320ms periodicity. The UE can be scheduled every 320ms for at least 10ms for the traffic. If the UEs are not uniformly located in all the beam footprints or triggering any traffic, the gNB can allocate the last 10ms of the 20ms dwelling window to the other beam footprint, or extend the dwelling window for one beam footprint and reduce the dwelling time of other beam footprints.

Figure 1. An example of 20ms dwelling window and the 320ms revisit period for Set 1-2

Observation 3: Within the dwelling window, the SSB and necessary system information can be broadcasted for the target beam footprints, which can be utilized by UEs to access to the satellite/gNB within the dwelling window. For the residual slots of the dwelling window, it can be used for the dynamic scheduling for the same beam footprints as SSB or other beam footprints.

Proposal 6: In order to provide services to all UEs covered by the satellite footprint, TDM beam scheduling/beam hopping can be performed to illuminate different beam footprint for SSB and necessary system information transmission.

Proposal 7: The time span of the dynamic scheduled dwelling window for a given beam footprint can be allocated based on UE distribution and traffic distribution.

Another kind of solution is that allocating the maximum number of simultaneously active beams separately for specific usages. For example, with Set1-2 FR1, when 16 active beams are simultaneously serving for communication, 8 beams will be used specifically for the DL broadcasting and the other 8 beams can be used for UE specific traffic. The 8 beams for the broadcast and initial access can be periodically hopping over the total 1058 beam footprints. The other 8 beams for UE specific traffic can be used to illuminate parts of the beam footprints continuously until the traffic ends. This kind of different functionality allocation for the simultaneously active beams can support both periodically broadcast to maintain the basic coverage for the UE initial access and the continuous service for specific areas.

Proposal 8: For the system level enhancements to improve the coverage ratio, both the following functionality allocation of active beams can be considered.

- Using one shared beam for both broadcast and UE specific traffic.

- Using separate beams for different functions respectively.

DL coverage ratio at system level with wider beam footprint is also evaluated and some results are collected in the last RAN1 meeting. As we observed in link level analysis, for LEO-600 set1-1/1-2, there are 5dB coverage margin for single SSB detection and 8.89dB/10.5dB coverage margin for 4 SSB combination within 80ms if no additional loss is considered. Considering the limited number of active beams can be supported for system level coverage enhancement, wider beams to illuminate larger coverage area can be supported for SSB transmission or other common signals with coverage margins. Also considering the implementation issues that more SSBs can facilitate the detection and synchronization of UEs, such as AGC tuning and some estimation for the Doppler shift, multiple SSB transmissions and receptions will facilitate the DL synchronization of UEs.

However, as 4 SSB combinations is considered to ensure the DL coverage performance for the case with 20ms SSB periodicity, if the default SSB periodicity is extended to 80ms by UE behavior, the detection of 4 SSBs combination within a larger duration (320ms) can be difficult. On the other hand, the multiple SSB transmission within dwelling window may also prolong the revisit period to reach the full coverage of all the beam footprints. Thus, the impacts of increasing SSB periodicity needs to be further studied.

Figure 2. Examples of multiple (e.g., 4) SSBs combination within one dwelling window

Observation 4: Some channels, e.g. PBCH with 4 SSB combination detection, can provide additional margins to support wider beams.

Observation 5: Additional SSB detection, e.g. SSB combinations, within the dwelling window would facilitate the UE implementation for the DL synchronization.

Proposal 9: Additional SSB detection, e.g. SSB combinations, within the dwelling window can be considered to facilitate the UE implementation for DL synchronization.

In addition to extending the default SSB periodicity to improve coverage ratio, whether wider beam can be supported has been discussed. Based on RAN1’s evaluation, for Set 1-1/1-3 FR1 with higher number of simultaneous active beam, the SSB periodicity needs to be extended from 20ms to 80ms to reach the 100% coverage ratio. For set 1-2 with less simultaneous active beam, larger than 320ms of SSB periodicity is needed to achieve nearly 100% coverage ratio. Then for set 1-2, three cases are considered and illustrated in Figure 3. The Case A in the Figure 3 is the baseline case defined in Set 1-2, in which the SSB periodicity needs to be extended from 20ms to 320ms to achieve nearly 100% coverage. Based on the RAN1’s evaluation for link-level study, under the assumption of Set 1-2, DL coverage margins for several channels/signals are observed. To reduce the SSB periodicity, multiple beams with reduced per beam power (Case B) and wider beam (Case C) can be considered. In Case B, the transmit power of one beam in Set 1-2 can be shared or split with 4 beams to extend the coverage and shorten the SSB periodicity. The transmit power of one beam in Case B would be reduced with 6dB, which may induce issues in the link level coverage. In Case C, a wider beam is used to extend the coverage on the earth and shorten the SSB periodicity. It may have similar issue as Case B that the link budget may not fulfill the requirements. For both Case B and C, the link level enhancement is needed to compensate the reduced transmit power or the reduced antenna gain due to the wider beam. However, the performance may be different between Case B and C. In the Case C, all the UEs under the original 4 beam areas will share the same set of frequency domain resources. But in the Case B, with the different beams or the spatial isolation between 4 beam areas, the UEs in the different beam area can use the same set of frequency domain resources. The UE performance or date rate in the Case B would be better than that of Case C, while the implementation of Case B requires more complex on-board processing capability. With the extension of the coverage of one beam from Set 1-2, the SSB periodicity can be reduced to 80ms. While due to the reduced power and antenna gain, further link level enhancements may be needed.

Figure 3. Examples to extend the coverage of one beam and reducing the SSB periodicity

Observation 6: To extend the coverage ratio and shorten the SSB periodicity, more beams with reduced power per beam, and wider beams can be considered.

If wider beam size for DL common channel is supported, considering the issue on whether potential enhancements for determining narrower beam footprint to transmit DL/UL dedicated channels (incl. PRACH) is necessary, from our perspective, the narrow beam selection can be implemented based on current specification, such as the report of CSI-RS RSRP. When the DL common channels are transmitted by using a wider beam footprint, while DL/UL dedicated channels are transmitted using a narrower beam footprint, UE can receive and measure CSI-RS configured for several narrower beams, one of the narrow beams for dedicated channels will be identified based on CSI-RSRP reported by UE. Therefore, the motivation on introducing additional mechanism to identify narrow beams for dedicated channels needs to be clarified first. Otherwise, the study of potential enhancements on using wide beams for DL common channels transmission and narrow beams for dedicated channels transmission can be de-prioritized.

Observation 7: For the case when DL common channels are transmitted by using wider beams, identifying narrower beams for dedicated channels can be performed based on current specification, e.g., based on CSI-RS measurement and reporting mechanism.

Proposal 10: It should further clarify the necessity and the motivation to support additional enhancements to identify the narrow beams for dedicated channels.

## 2.2 Link-level study

According to the agreements achieved in the RAN1#116 meeting, NGSO at LEO-600 operating in FR1 is considered in priority for the link level evaluation of NTN-specific downlink coverage. Regarding the target services and target data rate, the study in Rel-19 will continue to focus on the handset terminals (including smartphones with -5.5 dBi antenna gain). Also, the target services (i.e., VoIP and low-data rate services for commercial handset terminals) and the target data rate for each service determined in Rel-18 can be re-used to evaluate DL coverage performance in NR NTN.

For link level DL coverage evaluation, the evaluation methodology and performance metrics are determined. Also, the reference parameters and simulation assumptions have been agreed in the RAN1#116bis meeting.

### 2.2.1 Observations on the evaluation results

Considering the link budget calculation of each NTN study case, the two different satellite EIRP density, i.e., 34dBm/MHz for Set1-1/Set1-2 FR1, 26dBm/MHz for Set1-3 FR1, agreed in satellite parameter tables are considered in the link budget calculation. By using the formula specified in Section 6.1.3.1 of TR 38.821, the CNR for each study case in NR NTN are calculated, and it is concluded that the CNRs for the satellite payload parameters Set 1-1, Set 1-2 and Set 1-3 are equal to -1.9 dB, -1.9 dB and -9.9 dB respectively based on the observation by companies.

The coverage performance of following DL channels/signals are evaluated to identify the bottleneck channel(s). And observations on DL coverage performance of each channel/signal have been summarized based on evaluation results provided in the RAN1#117 meeting.

- PDSCH for VoIP

- PDSCH for low data rate service-3kbps

- PDSCH for low data rate service-1Mbps

- PDSCH Msg.2

- PDSCH Msg.4

- PDSCH carry SIB1

- PDSCH carry SIB 19

- PDCCH

According to the evaluation results observed in the previous RAN1 meetings, for LEO-600 Set 1-1/1-2 scenario, since EIRP density is not decreased due to dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint or other additional loss, it is observed that there is no coverage issue for all the evaluated physical channels/signals.

While for LEO-600 Set 1-3 FR1 satellite parameters, EIRP density is 8dB less than that of Set 1-1/1-2 FR1, it can be observed that there are coverage gaps for some evaluated physical channels, including PDCCH, PDSCH carrying SIB1 or SIB19, PDSCH Msg4 and PDSCH for 1Mbps low data rate service, and the coverage gap of each DL channel are summarized in the Table 2. Furthermore, the CNR target for LEO-600 Set 1-3 FR1 was discussed in the last meeting, it is proposed to adjust the CNR target to -8dB based on the coverage performance of PSS detection, as agreed in the last meeting, at least for PDCCH and PDSCH carrying Msg4, it will focus on coverage enhancement with a target CNR of -8 dB for link-level study. Therefore, the coverage gaps of each DL channel/signal compared with CNR target of -8dB are also provide as in Table 2. The link level analysis and potential coverage enhancements will be discussed by taking the observations on average evaluation results collected in the meeting and our simulation into consideration.

Table 2. Coverage gap for LEO-600 Set1-3 FR1

| DL Channel/signal | Coverage gap compared with -9.9dB CNR |  | Coverage gap compared with -8dB CNR |  |
| --- | --- | --- | --- | --- |
|  | Average results | CMCC results | Average results | CMCC results |
| PDCCH | 3.9 | 3.52 | 2 | 1.62 |
| PDSCH for low data rate-1Mbps | 5.5 | 7.73 | 3.6 | 5.83 |
| PDSCH Msg4 | 4.7 | 4.49 | 2.8 | 2.59 |
| PDSCH SIB1 (800bits) | 4.1 | 3.76 | 2.2 | 1.86 |
| PDSCH SIB1 (1280bits) | 6.5 | 5.77 | 4.6 | 3.87 |
| PDSCH SIB19(616 bits) | 3.5 | 3.05 | 1.6 | 1.15 |

### 2.2.2 Potential link-level enhancements

As agreed in the RAN1#118 meeting, RAN1 to consider link-level enhancements for the following channels:

- PDCCH

- PDSCH with Msg 4

- PDSCH with SIB1/SIB19.

  - Note: link-level enhancements for PDSCH with SIB1/SIB19 may be applicable to other SIBs, without additional specification impact.

- Note: the above does not imply that all the channels above will be enhanced, but all of them should be considered based on this agreement

Considering limited TU assigned for NTN DL coverage enhancements, it would be challenging to study all the potential DL coverage enhancements, including beam hopping, wide/narrow beams and link level enhancements for bottleneck channels. Therefore, the workload of link-level enhancements needs to be lessened and the following options can be taken into consideration:

- Option 1: Unified solution can be adopted for DL channels coverage enhancement.

- Option 2: Prioritize discussing coverage enhancement schemes for one or two DL channels/signals

Proposal 11: The work load of link-level enhancements needs be reduced, and the following options can be considered,

- Option 1: A unified solution can be adopted for all the link-level enhancement

- Option 2: Only one or two channels can be enhanced

For PDCCH in LEO-600 scenarios with Set1-3 FR1 parameters, it is observed that 1.62dB coverage gap needs to be enhanced based on our simulation results, and the average coverage gap is about 2dB. In order to ensure the PDSCH/PUSCH can be scheduled successfully for both common channels and UE specific channels, coverage enhancements for PDCCH in Set1-3 FR1 needs to be considered. Some potential enhancements have been proposed for further study in the last meeting, the agreement is listed as the following.

RAN1#118bis Agreement Support PDCCH CSS Link level enhancement in Rel-19 for all CSS types except type 3. - The following techniques are for further study: - PDCCH repetition, including: - Option 1: Intra-slot PDCCH repetition - Option 2: Inter-slot PDCCH repetition - CORESET length (i.e. number of OFDM symbols) extension - DCI format optimization (e.g. size reduction, etc) - Note: the same technique is intended to apply to all search space types targeted for link level enhancements - For the above techniques, at least the following aspects should be discussed for the relevant candidate techniques: - Configuration - Backward compatibility and UE behaviour of legacy UE - Linked Search Space - Blocking probability - DCI format size budget - Resource overhead - Impact on CORESET0 - Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level. - FFS: whether to apply the selected solution to PDCCH CSS type3 and PDCCH USS

Considering to improve the coverage performance of PDCCH in NTN scenarios, PDCCH repetition can be supported to provide coding gain. Currently, the link-level evaluations of DL coverage in NTN scenarios focus on S-band and the beam bandwidth is assumed as 5MHz, resulting in limited downlink frequency domain resources. In this case, with maximum 3 OFDM symbols for current CORESET configuration, the maximum supported aggregation level of PDCCH is 8. It is difficult to improve the coverage performance of PDCCH by increasing the aggregation level of PDCCH. The time-domain PDCCH repetition can be applied as an effective coverage enhancement scheme. To eliminate the coverage gap of no more than 2dB, it is sufficient to support 2 PDCCH repetitions, which theoretically can provide nearly 3dB of performance gain by the additional PDCCH repetition transmission.

The PDCCH repetition scheme has been supported in multi-TRP scenarios to improve reliability, two linked Search Spaces within the same slot are configured with the same periodicity and offset (monitoringSlotPeriodicityAndOffset), and the same duration. However, as clarified in current specification, the following SS sets cannot be linked with another SS set for PDCCH repetition: SS set 0, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, searchSpaceMCCH, searchSpaceMTCH, peiSearchSpace, and sdt-SearchSpace. PDCCH repetition by search space linking for scheduling system information has not been supported. While for PDCCH monitoring in connected states, the PDCCH repetitions can be monitored in linked Search Space with additional search space configuration in NTN scenarios.

From our perspective, PDCCH coverage enhancements for scheduling in initial access are necessary for NTN scenarios. To support PDCCH repetition schemes for CSS except CCS type 3, one key issue is how to configure the time/frequency domain resources for multiple PDCCH repetitions transmission, and indicate UEs to perform PDCCH monitoring in the associated COERSET and search space successfully. Specifically, for PDCCH scheduling SIB1, the resource allocation for CORESET0 is indicated through controlResourceSetZero and searchSpaceZero in MIB instead of RRC configuration, then the time-domain repetition for PDCCH scheduling SIB1 can be further enabled/configured in the MIB.

Furthermore, inter-slot PDCCH repetition can be supported for Type-0 PDCCH by pre-defined/network configured PDCCH monitoring windows for additional PDCCH repetition, which can be efficiently extended to support more than 2 PDCCH repetitions. And for intra-slot PDCCH repetition, the two PDCCH repetition can be configured with the same numbers of symbols and RBs in a slot, while the symbols occupied by each PDCCH candidate needs to be pre-configured/indicated in MIB.

Considering the scheme on CORESET length extension, i.e., increasing number of OFDM symbols, the maximum number of symbols occupied by CORESET is 3 in current specification, which is determined based on considerations of coverage performance, scheduling flexibility, complexity control and resource utilization efficiency. Increasing the number of symbols occupied by CORESET may affect the data transmission, as more available resource blocks are used to carry control information, which may reduce the transmission efficiency of user data. And it leads to less scheduling flexibility compared with repetition mechanism.

And the mechanism of DCI format optimization (e.g. size reduction, etc) needs to be further clarified. From our perspective, DCI size reduction may be achieved by introducing new DCI format or designing 2-stage DCI for Rel-19 NTN link-level enhancement, the performance gain of these DCI size reduction methods is not clear, and may increase the UE implementation complexity of performing DCI size alignment considering the constraints of the DCI size budget.

Proposal 12: PDCCH repetition can be supported for DL coverage enhancements in NTN scenarios. And for Type-0 PDCCH coverage enhancements, the PDCCH repetition can be enabled/configured in the MIB.

- Inter-slot PDCCH repetition and intra-slot PDCCH repetition can be considered

For PDSCH in NTN scenarios, when Set1-3 FR1 parameters are assumed, it is agreed to focus on coverage enhancement with the CNR target of -8dB, which is listed as below. It is observed that there are 3.5~6.5dB coverage gap for PDSCH carrying SIB1, SIB19 and Msg4 with large payload size. Thus, coverage enhancement schemes need to be considered. With similar bandwidth limitation issue as PDCCH, it is reasonable to support time-domain PDSCH repetition method to improve coverage performance.

RAN1#118bis Agreement For PDSCH with Msg4 Link level enhancement: - Continue studying PDSCH repetition - Further discuss the specification impact for at least the following: - Procedure and signaling - Repetition factor - Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level.

To alleviate the workload for PDSCH coverage enhancements, unified mechanism to support PDSCH repetition for transmitting SIB1/SIB19 and Msg4 can be further discussed. One straightforward way is to carry the PDSCH repetition indication in system information. While the repetition factors applied for each PDSCH carrying different information can be further discussed. Specifically, based on the coverage evaluation of each PDSCH channel/signal, for PDSCH Msg4, PDSCH SIB1 (800bits) and PDSCH SIB19, the coverage gaps are less than 3dB, which can be eliminated through performance gain provided by 2 PDSCH repetitions, while for PDSCH SIB1 with larger payload size with 4.6dB coverage gap, repetition number of 4 can be considered. If multiple repetition factors are considered for different channel conditions, dynamic indication of repetition number is necessary and the corresponding DCI scheduling the PDSCH transmission can be used to indicate the repetition number respectively.

Proposal 13: Considering DL coverage enhancements for PDSCH carrying SIB1, SIB19 and Msg4 in NTN scenarios, PDSCH repetition methods can be further discussed.

- PDSCH repetition for SIB1/SIB19/Msg4 can be enabled/configured via SIB.

- Multiple PDSCH repetition number can be considered, and dynamic indication (e.g., DCI format) needs to be further studied.

## 2.3 RAN2 LS on DL coverage enhancements

RAN2 has sent a LS to RAN1 regarding the DL coverage enhancements in NTN scenarios [8]. In the LS, RAN2 provides questions on DL coverage enhancements related to SSB, common control signaling, UL beams and beam status, and RAN1 has sent reply to answer Question 1 and Question 4. For the remaining questions, whether to send response has been discussed in the last RAN1 meeting while no consensus was achieved. The content of the LS is listed as below.

RAN2 has started to study on RAN2 aspects of DL coverage enhancement. To progress the study, RAN2 has identified questions to RAN1 for aspects where the input is required. Related to this, RAN2 would like to request RAN1 to provide feedback on the following questions. Question 1: Can RAN1 provide the information on their progress on whether the existing SSB pattern for an NR cell (e.g. SSB position in burst, SSB index number, etc.) is changed in Rel-19 NR NTN, and whether the SSB periodicity is extended compared with existing TN values? Question 2: Can RAN1 provide the information on whether/how the solution RAN1 is investigating is expected to impact common control signalling for UEs in RRC idle / RRC inactive? Question 3: Can RAN1 provide the feedback on whether UL beam hopping is also being studied in RAN1 (and whether this is separate from DL beam hopping)? Question 4: RAN2 would like to remind RAN1 that satellite beams are currently not visible to UEs and any decision about different beam status (i.e. "off", "common messages only" and "active traffic") will likely have to relate to beams visible to the UE (e.g. SSB beams). RAN2 would also like to know whether RAN1 intends to define beam status for beams not visible to the UE or to define new beam status for beams visible to the UE. Question 5: Can RAN1 provide the feedback on whether the beam status in different beams (visible to the UE) of one cell are the same or can be different in any given time, i.e. the beam status is cell specific or beam specific? # Actions: # To RAN1: # RAN2 kindly request RAN1 to provide feedback on above questions.

Considering Question 2, as the extension of SSB periodicity is supported, the enhancements to the corresponding common control channel/signals may be needed. While currently RAN1 does not have any discussion on the enhancements to the common control signaling for UEs in RRC idle/RRC inactive mode. For even there are more progresses on the extended periodicity of common control channel, RAN2 can directly refer to the agreements from RAN1. Further replies may not be needed.

Considering Question 3, RAN1 has not discussed the DL or UL beam hopping separately. For even the solution of beam hopping, it was still under the discussion. And for Question 5, currently RAN1 does not have any discussion on beam status or beam visibility. Based on the previous discussion, it seems there will not be any “beam status” introduced for the NTN DL enhancement. Then there will not be any conclusion on that.

Based on the current progress of RAN1, the above issues have not been resolved or consensus cannot be reached by RAN1. Since no valid information can be provided currently, it is recommended to postpone the responses to RAN2.

Proposal 14: Considering RAN2 LS on DL coverage enhancements, there is not any progress on the other three questions and there is no need to reply the LS at this stage. It is proposed to postpone the responses until RAN1 made new progress.

# Conclusions

In this contribution, we share our views on DL coverage enhancement for both system-level study and link-level study in NTN scenarios. The observations and proposals are summarized as follows.

Observation 1: Without enhancement of power reduction or wider beam, to reach an almost 100% coverage ratio, the default detection period for SSBs in NTN scenarios needs to be extended to 320ms for all cases of Set1-1/1-2/1-3.

Observation 2: For LEO-600 Set1-1/1-2/1-3, longer revisit period can be considered to improve the coverage ratio.

Observation 3: Within the dwelling window, the SSB and necessary system information can be broadcasted for the target beam footprints, which can be utilized by UEs to access to the satellite/gNB within the dwelling window. For the residual slots of the dwelling window, it can be used for the dynamic scheduling for the same beam footprints as SSB or other beam footprints.

Observation 4: Some channels, e.g. PBCH with 4 SSB combination detection, can provide additional margins to support wider beams.

Observation 5: Additional SSB detection, e.g. SSB combinations, within the dwelling window would facilitate the UE implementation for the DL synchronization.

Observation 6: To extend the coverage ratio and shorten the SSB periodicity, more beams with reduced power per beam, and wider beams can be considered.

Observation 7: For the case when DL common channels are transmitted by using wider beams, identifying narrower beams for dedicated channels can be performed based on current specification, e.g., based on CSI-RS measurement and reporting mechanism.

Proposal 1: To support SSB periodicity extension for improving DL coverage ratio and reducing common control channel overhead, one default value of {160ms, 320ms} can be considered, and 320ms is preferred to achieved almost 100% coverage ratio.

Proposal 2: It needs to be clarified on UE’s behavior when multiple default values are supported, e.g. how to identify which default value can be used for initial access under one beam footprint or frequency bands.

Proposal 3: Different SSB periodicities can be used to solve the backward compatibility issue, wherein one set of legacy 20ms SSB periodicity can be reserved for legacy NTN UE in some beam footprints, and other sets of longer SSB periodicities can be used to serve Rel-19 NTN UE in other beam footprints.

Proposal 4: Considering the extended SSB periodicity, the periodicity of common message (e.g. CORESET0/SIB1, SIB19) and UE measurement needs to be extended correspondingly.

Proposal 5: Support the system level enhancements to improve the coverage ratio in NR-NTN phase 3, with the consideration of introducing the revisit periodicity and dwelling window.

Proposal 6: In order to provide services to all UEs covered by the satellite footprint, TDM beam scheduling/beam hopping can be performed to illuminate different beam footprint for SSB and necessary system information transmission.

Proposal 7: The time span of the dynamic scheduled dwelling window for a given beam footprint can be allocated based on UE distribution and traffic distribution.

Proposal 8: For the system level enhancements to improve the coverage ratio, both the following functionality allocation of active beams can be considered.

- Using one shared beam for both broadcast and UE specific traffic.

- Using separate beams for different functions respectively.

Proposal 9: Additional SSB detection, e.g. SSB combinations, within the dwelling window can be considered to facilitate the UE implementation for DL synchronization.

Proposal 10: It should further clarify the necessity and the motivation to support additional enhancements to identify the narrow beams for dedicated channels.

Proposal 11: The work load of link-level enhancements needs be reduced, and the following options can be considered,

- Option 1: A unified solution can be adopted for all the link-level enhancement

- Option 2: Only one or two channels can be enhanced

Proposal 12: PDCCH repetition can be supported for DL coverage enhancements in NTN scenarios. And for Type-0 PDCCH coverage enhancements, the PDCCH repetition can be enabled/configured in the MIB.

- Inter-slot PDCCH repetition and intra-slot PDCCH repetition can be considered

Proposal 13: Considering DL coverage enhancements for PDSCH carrying SIB1, SIB19 and Msg4 in NTN scenarios, PDSCH repetition methods can be further discussed.

- PDSCH repetition for SIB1/SIB19/Msg4 can be enabled/configured via SIB.

- Multiple PDSCH repetition number can be considered, and dynamic indication (e.g., DCI format) needs to be further studied.

Proposal 14: Considering RAN2 LS on DL coverage enhancements, there is not any progress on the other three questions and there is no need to reply the LS at this stage. It is proposed to postpone the responses until RAN1 made new progress.

# References

- RP-234078, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, December 11-15, 2023.

- RP-241667, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, June 17-20, 2024.

- Chairman's Notes RAN1#118bis, October 14th – 18th, 2024.

- Chairman's Notes RAN1#116, February 26th - March 1st, 2024.

- Chairman's Notes RAN1#117, May 20th - 24th, 2024.

- Chairman's Notes RAN1#118, August 19th - 23rd, 2024.

- 3GPP TR38.821, Solutions for NR to support non-terrestrial networks (NTN).

- R1-2405785, LS on DL coverage enhancements, 3GPP TSG-RAN WG1 Meeting #118