3GPP TSG-RAN WG1 Meeting #119	Tdoc R1-2409563

Orlando, US, November 18th – 22nd, 2024

Agenda Item:	9.2.1

Source:	Ericsson

Title:	Remaining issues for UE-initiated beam management

Document for:	Discussion

# 1	Introduction

The scope of NR MIMO phase 5 was endorsed in RAN#102 meeting including the following objective:

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management

  - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching

  - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

In previous RAN1 meetings, some agreements were made on UE-initiated/event-driven beam management. In this contribution, we discuss next level of details on UE-initiated or event-driven reporting.

# 2	Discussion

## 2.1	Event

In RAN1#117, the following agreement was made:

Agreement

Regarding the triggering event determination for Event 2:

- If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs.

  - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam

Above feature is subject to UE capability.

- Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs

FFS: Whether the above is captured in RAN1 or RAN2 specification.

The agreement is illustrated in Figure 1. The UE counts the number of event-2 instances in a window, and as soon as M event-2 instances occur within the window, the UE initiates the beam reporting, using the first available instance of the first UL channel.

Figure 1: Illustration of the agreement of event evaluation. The UE evaluates how many event-2 instances occur within a (sliding) window. Every event-2 instance is represented by “Ind” in the figure.

Up till now, events have been captured in RAN1 and RAN2 specifications. This includes beam failure detection, where L1 provides indications to MAC, which are then counted. Based on the counting, beam failure is then declared in MAC, and the associated actions are initiated in MAC. On the other hand, RAN1 specifications do not contain any counters: RAN1 specifications are stateless. We prefer to keep this separation, and propose:

The event evaluation for UE-initiated beam reporting is captured in a RAN2 specification.

The same proposal was made in the RAN2 contributions [2][3][4][5][6], which indicates that it is feasible to capture the event evaluation in the RAN2 specifications. Furthermore, RAN2 has made the following agreements for LTM:

MAC layer handles the event evaluation and measurement report triggering. (RAN2#127)

The entire event evaluation procedure is handled by MAC based on the latest L1 measured results reported by L1. (RAN2#127bis)

Hence, RAN2 will describe event evaluation for LTM in the MAC specification, and it would make a lot of sense to reuse that description also for UE-initiated beam management.

Note that the implementation in the UE is independent on where the triggering is captured, and that there is no performance impact on this decision. For example, the latency does not depend on the document where the functionality is captured.

### 2.1.1	Further details on Event-2

In RAN1#118bis, the following agreement was reached:

Agreement

Regarding RS measurement for the current beam for Event 2, for enabling one of either Scheme-1 or Scheme-2 by NW in Option-2a, the following implicit manner is supported:

- If the RS(s) for new beam are CSI-RS configured in a CSI-RS resource set configured with repetition, Scheme-1 is enabled; otherwise, Scheme-2 is enabled.

In legacy, we rely on the CSI-ResourceConfig to define measurement resources:

We propose to reuse the CSI-ResourceConfig also for UE-initiated beam management:

Reuse the RRC IE CSI-ResourceConfig to define the new beams for UE-initiated beam management.

This is also summarized in Section 2.4, where the RRC parameter discussion is summarized.

In RAN1#118bis, the following was agreed:

Agreement

Regarding the triggering event determination for Event 2, down-select among the following alternatives for the evaluation periodicity for determining Event-2 instance [at least when DRX is not configured]

- Alt-1: The periodicity of the current beam RS should be the same as that of the new beam RS(s).

  - The evaluation periodicity is the same as the periodicity of the current and new beam RS(s)

- Alt-2: The periodicity of the current beam RS can be different from that of the new beam RS(s)

  - Alt-2_1: The evaluation periodicity is the same as the periodicity of the current beam RS;

  - Alt-2_2: The evaluation periodicity is the same as periodicity of the new beam RS;

  - Alt-2_3: The evaluation periodicity is the same as shortest periodicity of the current beam RS and new beam RS(s):

  - Alt-2_4: The evaluation periodicity is the maximum of {X ms, shortest periodicity of the current beam RS and new beam RS(s)}:

  - Alt-2_5: The evaluation periodicity is the same as largest periodicity of the current beam RS and new beam RS(s):

Note: There is the same periodicity for the new beam RS(s).

The periodicity of the event determination for event-2 is closely related to the measurement period of the corresponding RS – the UE cannot evaluate an event more often than it performs the corresponding measurements. Currently, RAN4 has not defined any measurement period for UE-initiated beam reporting, but the corresponding requirements for L1-RSRP may be relevant to study. The legacy measurement period requirements are provided in Table 9.5.4.1-1 and Table 9.5.4.1-2 in [7] for FR1 and FR2, respectively. For ease of reference, we replicate Table 9.5.4.1-2 below:

Table 9.5.4.1-2: Measurement period TL1-RSRP_Measurement_Period_SSB for FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

Table 1: Table 9.5.4.1-2 from [7].

In Table 1, M is related to if measurement restriction is configured, N is related to the Rx beam sweeping, and P is related to if the L1-RSRP measurement occasions can be shared with L3 measurements. We realize that the measurement period is always larger than the periodicity of the RS. We understand that RAN4 is going to define measurement period for event triggered UE initiated measurement report.

It would seem appropriate that the indication periodicity is the same as the measurement period, which would be defined by RAN4. Therefore, we propose

The periodicity of the event determination for event-2 is identical to the measurement periodicity defined by RAN4.

RAN1 could however decide if the periodicity is given by the current beam RS or new beam RS. Rather than going into the discussion on how the various beams are configured, we propose to use a solution that is agnostic to the configuration:

The periodicity of the event determination for event-2 is identical to the measurement periodicity of the current beam RS.

One additional advantage of Proposal 4 is that it is also applicable to event-1.

In RAN1#118bis, the following was agreed

Agreement

Regarding the triggering event determination for Event 2, the event instance(s) counting is per new beam. Further study candidate condition(s) of resetting the counting including whether resetting is needed.

It is likely that there will be a need to reset the counter. However, this discussion can be postponed: there is no functionality that relies on the decision on the resetting mechanism – it can even be made during maintenance:

Postpone the discussion on the resetting mechanism of the counter.

In RAN1#118, the following was agreed:

Agreement

Regarding explicit RS configuration for new beam measurement for Event 2, at least Option-1 is supported

  - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration

  - If legacy UE capability signaling cannot be reused, introduce a UE capability signaling of indicating the maximum number of the configured RS(s) in the RS resource set.

  - FFS: The RS in the RS resource set can be updated by MAC-CE

There is an FFS in the agreement, on if RS in the RS resource set can be updated by MAC-CE how the RS(s) for the new beam(s) are configured. The claimed advantage of a MAC CE update is that the NW could activate a smaller set of beams, e.g., the neighbor beams of the indicated TCI state, which would reduce the UE measurement complexity. However, even without any MAC CE update, the measurement procedure would be the same as in legacy, which indicates it is not an issue. Therefore, we propose

Do not support MAC CE update of the RSs in the RS resource set.

### 2.1.2	Additional events

In RAN1#118bis, the following agreement was made

Working Assumption

The following working assumption in RAN1#118 is revised in red.

On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported.

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th Q-th best quality.

  - M Q is RRC configured with subjective to UE capability signalling

    - UE may only indicate a single candidate value or not support Event-7.

- The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise

- The additionally supported events will be lower priority compared to event 2.

With this working assumption, RAN1 has closed a discussion that has been ongoing since the start of the work item. The working assumption paves the way to develop the remaining details for these two new events. To streamline the future discussion, we propose to confirm the working assumption:

Confirm the working assumption to support event-1 and event-7.

In the working assumption, there is a statement that event-1 and event-7 will reuse the same design as event-2 as far as possible. In our view, this is straightforward for most properties of event-2. In the following, we highlight how to do this. Since we aim to reuse as much as possible, we focus on how to make it work, rather than discussing if certain properties are useful or not.

For event-1 and event-7 we summarize our proposals in Table 2.

|  | Event-1 | Event-7 |
| --- | --- | --- |
| Quality measure | L1-RSRP | L1-RSRP |
| Current beam | Scheme-1 and scheme-2 based on indicated TCI state | Scheme-1 and scheme-2 based on activated TCI state with Qth best quality, where Q=1,2,…,L, where L is the number of activated TCI states. |
| Report | N new beams Current beam (if configured) | N2 new beams Current beam+identifier (if configured) Additional contents FFS |
| Configuration of new beams | Explicitly configured in one RS resource set | Explicitly configured in one RS resource set. FFS: The reporting of RSs corresponding to activated TCI states |
| Report container | Mode A and mode B | Mode A and mode B |
| Window/counter | M instances within a window | M instances within a window |

Table 2: Details for event-1 and event-7.

From Table 2, we realize that for event-1, the properties from Event-2 can be reused as is.

For event-7, some additional explanations are provided below.

In event-7, the UE compares RSs in the activated TCI state with the Q-th best quality with a set of new beams. For the definition of the current beam, it thus makes sense to define the current beam based on the activated TCI state with the Qth best quality, (Q=1,2,…,L, where L is the number of activated TCI states).

For the report after event-7, it is relevant for the UE to report N2 new beams, with the corresponding configuration. We use N2, rather than N, for event-7, to highlight that up to L RSs should be reported, where L can be up to 8:

For event-7, it should be possible to configure the UE to report up to 8 RSs.

The idea with event-7 is to update the set of activated TCI states, the report should include information on what TCI states to activate, and what TCI states to deactivate:

The report for event-7 should include information on what TCI states to activate, and what TCI states to deactivate.

Note that including an identifier is necessary, since the NW does not know which TCI state corresponds to the activated TCI state with the Qth best quality.

## 2.2	Requesting UL resources for transmission of the report

One central issue when designing a UE-initiated beam report is how the resources for the transmission of the UL report are provided to the UE. In RAN1#117, the following was agreed

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-A, the DCI format in Step-2 comprises UL-grant DCI format, and the second channel in Step-3 is at least PUSCH.

- The UL-grant DCI format at least comprises DCI format 0_1/0_2.

  - FFS: DCI format 0_3

- FFS: How to trigger the UEI beam report by the UL-grant DCI format

- FFS: the DCI format in Step-2 comprises DL-grant DCI format, and the second channel in Step-3 is PUCCH.

  - 1-bit field in the DL-grant DCI format is introduced to indicate the transmission of the UEI beam report

    - The PUCCH resource for HARQ-ACK transmission can be reused to carry both the HARQ-ACK and UEI beam report.

  - The DL-grant DCI format at least comprises DCI format 1_1/1_2.

    - FFS: DCI format 1_3

The agreement contains a number of FFSs. The first FFS discusses if UL-grant after step-1 can be DCI format 0_3. In our view, DCI format 0_3 should be supported:

The UL-grant DCI format comprises DCI format 0_1/0_2/0_3.

Note that if the UE is scheduled using DCI format 0_3, the beam report will be mapped to the cell with the smallest serving cell index among the scheduled cells indicated by scheduled cells indicator field or frequency domain resource assignment field, just as in legacy.

When a UE is scheduled using DCI format 0_3, the UCI is transmitted on one of the scheduled cells.

In RAN1#118, the following was agreed

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the dedicated RRC signaling for first PUCCH channel configuration for Mode-A, down-select one of the following

- Alt-1 (dedicated SR for mode-A): Introduce RRC parameter, e.g., reportResourceRequest-UEIBR, corresponding to the one-bit indication in the first PUCCH channel

  - The RRC parameter is associated with the dedicated SchedulingRequestId.

    - Note: The detailed signaling is up to RAN2.

- Alt 2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig-ModeA-UEIBR, for the periodic PUCCH resource configuration.

  - Note: The RRC parameter is NOT associated with SchedulingRequestId.

  - FFS: how to encode 1-bit to PUCCH resource, e.g., reuse encoding mechanism of positive/negative SR.

  - The dedicated RRC parameter at least comprises the following:

    - periodicityAndOffset

    - PUCCH-ResourceID

- Above applies at least for the single CC case.

For mode A, by transmitting the first channel, the UE informs the NW that an event has occurred and requests resources to transmit the corresponding report. When the NW receives the first channel, it sends an UL-grant to the UE. If the UE transmits the first channel, but does not get any response, the desired behaviour is that the UE retransmits the first channel, until it receives a response from the NW:

The UE retransmits the first UL channel until it receives a DCI format that indicates a resource for a second UL channel: the received DCI serves as an acknowledgment that the NW has received the first UL channel.

It is preferable that the retransmissions are rather frequent, to reduce the latency. A typical case is that the UE would transmit the first UL channel in every possible occasion until it receives a response from the NW.

When looking at Observation 3, we realize that for mode A the properties of the first channel is very similar to a scheduling request (SR). Therefore, we propose

For mode A, support Alt-1: Introduce an RRC parameter, e.g., reportResourceRequest-UEIBR, associated with a dedicated SchedulingRequestId.

For the SR procedure, there are three central concepts:

- Trigger: This is what starts the SR procedure. Originally, an SR was triggered according to the BSR procedure, to provide the NW with information about UL data volume in the MAC entity. In later versions of the specifications, SR can also be triggered by procedures not related to BSR, e.g., beam failure recovery or consistent LBT failure. For UE initiated beam reporting, a new triggering criterion must be introduced.

- Pending: An SR procedure is considered pending until it is cancelled.

- Cancellation: This is what stops the SR procedure. All pending SR(s) for BSR triggered according to the BSR procedure prior to the MAC PDU assembly shall be cancelled when the MAC PDU is transmitted and this PDU includes a Long, Refined Long or Short BSR MAC CE. For SRs not triggered according to the BSR procedure, e.g., beam failure recovery or consistent LBT failure, other conditions apply. For UE initiated beam reporting, another condition must be added.

Adding support for an SR procedure triggered by UE initiated reporting seems possible, and in line with earlier extensions.

Furthermore, SR has the attractive property that if the SR is pending, i.e., that the UE is waiting for the response, the UE continuously (i.e., not only during the DRX on duration) monitors PDCCH. If a new type of UCI is introduced, the corresponding DRX behaviour would also have to be modified.

We note that in 38.321, it is stated that a scheduling request is used to request resources for UL-SCH transmissions. By sending the first channel after the event has occurred, the UE is not requesting resources to transmit UL-SCH. However, the same is true for the use cases when the UE transmits SR to inform the NW that beam failure, or LBT failure, has occurred. Clearly, if Proposal 10 is agreed, RAN2 would have to capture the behaviour in the MAC specification.

Overall, we note that we have the following procedures:

Event evaluation, as discussed in section 2.1.

Transmission of the first channel, as discussed above.

Prohibit timers to avoid frequent triggering. This was discussed during RAN1#118bis, but without any agreement.

For all these procedures, we need to consider the relation to DRX. The specification work would be greatly simplified if all these procedures were described in 38.321.

Regarding mode B, RAN1 made the following agreement in RAN1#118:

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the dedicated RRC signaling for first PUCCH channel configuration for Mode-B, down-select one of the following

- Alt 1 (dedicated SR for mode-B): Introduce RRC parameter, e.g., reportNotification-UEIBR, corresponding to the one-bit indication in the first PUCCH channel

  - The RRC parameter is associated with the dedicated SchedulingRequestId.

    - Note: The detailed signaling is up to RAN2.

- Alt 2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig-ModeB-UEIBR, for the periodic PUCCH resource configuration.

  - Note: The RRC parameter is NOT associated with SchedulingRequestId.

  - FFS: how to encode 1-bit to PUCCH resource, e.g., reuse encoding mechanism of positive/negative SR.

  - The dedicated RRC parameter at least comprises the following:

    - periodicityAndOffset

    - PUCCH-ResourceID

- Above is at least applied to a single CC case.

Compared to mode A, the situation for mode B is a bit different. Since the NW does not provide any DL response when it receives the first UL channel, the UE cannot repeat the transmission of the first UL channel until it receives a response from the NW – there is simply no such DL signal defined. During the RAN1 discussions, the possibility to have the same solution for Mode A and Mode B has been mentioned. Having the same solution may seem attractive, since only one solution must be specified. However, this is only true if the requirements for the two solutions are the same.  Otherwise, postulating the same solution beforehand solution is clearly inappropriate, and may lead to the situation depicted in Figure 2.

Figure 2: Trying to enforce the same solution (fitting one shape) to two problems (holes with different shapes) may lead to a bad solution.

Based on this, we propose

Do not impose a constraint beforehand that the first channel should be the same for mode A and mode B.

Note that the solutions can still be the same, but it would be an outcome of the design process, not an a priori condition.

In RAN1#118bis, the following was agreed:

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the triggering procedure in Step-2 of Mode-A

- Reuse CSI request field in DCI format 0_1/0_2 to trigger the transmission of the UEI beam report

  - If a CSI trigger state associated with UEI beam report configuration(s) is indicated by the CSI request field in DCI format 0_1/0_2, the UE transmits the corresponding UEI beam report(s) in the second PUSCH scheduled by the DCI format 0_1/0_2

  - FFS: DCI format 0_3

- FFS: Whether a CSI trigger state should be dedicated to UE-initiated/event-driven beam reporting, i.e., not associated with legacy AP-CSI report configuration.

The report triggered in step-2 of mode A is very similar to a normal aperiodic beam report: in particular, the configuration will be similar: the CSI report configuration will be similar, as will the configuration of the triggering offsets. As we see it, we could also reuse the CSI processing criteria, and report priorities: there seems to be no reason to deviate from what we have:

The report triggered in step-2 of mode A reuses properties from a normal aperiodic beam report, when it comes to configuration, CSI processing criteria and report priorities.

Note that false SR detection happens from time to time: as a rule of thumb, SR is dimensioned to have 0.1% false detection probability, but it is difficult to achieve that level of accuracy in practice. This means that the UE will receive the grant to transmit the step-2 beam report also when no event has occurred. To avoid complicated error cases, the UE should be prepared to send the step-2 beam report in response to a request from the NW, even when no event has been triggered:

The UE sends the step-2 beam report of mode A in response to the CSI request from the NW even when no event has occurred.

## 2.3	Reporting

In RAN1#117, RAN1 agreed

Agreement

Regarding the triggering event determination for Event 2:

- If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs.

  - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam

Above feature is subject to UE capability.

- Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs

FFS: Whether the above is captured in RAN1 or RAN2 specification.

With the time window/counter functionality agreed above, the beam that triggered the event is not necessarily the beam with the highest RSRP at the beam reporting time. This is illustrated in Figure 3 below.

Figure 3: An illustration of varying beam quality. In this event-2 example, candidate beam 2 is consistently better than the current beam.

In the example in Figure 3, the NW configures event-2 with a 2dB threshold. The NW also configures a time window of 80ms, and M=4, i.e., 4 instances must fulfill the condition within 80ms for the event to be triggered. In this example, candidate beam 1 triggers at time 400ms, but candidate beam 2 has the largest RSRP. With the window/counter, this is generally true:

The beam that triggered the event is not necessarily the beam with the highest RSRP.

The window/counter was introduced to avoid ping pong beam switches, and there was a wide understanding in RAN1 that this was an important feature. However, to avoid ping pong, the window/counter in the UE is not sufficient: the NW must also know which beam triggered the event. Otherwise, the NW has no choice but to switch to the beam with the highest RSRP. Clearly, there is no reduction in ping pong in this case:

There is no reduction in ping pong beam switches if the NW is unaware of which beam triggered the event, i.e., which beam consistently fulfilled the condition.

Based on this argument, the following proposal follows:

For every reported new beam, the UE indicates if it fulfilled the event condition or not.

The current agreements facilitate the configuration of multiple events to the UE: different event types, different CCs, different new beam configurations. When the UE is configured with multiple events, it must be possible for the NW to determine which event has occurred. One option is to define multiple instances of the first UL channel, and associate different events with different instances. However, this solution is very inefficient. Therefore, it is much more efficient to introduce information about the event in the second UL channel:

Introduce an event identifier in the report sent over the second channel.

## 2.4	RRC parameters

In RAN1#118, the first RRC parameters were collected. It is assumed that all these parameters are configured inside a CSI-ReportConfig. Here we will comment on these parameters and suggest a few new.

Parameters currently in the RRC parameter excel:

reportTransmissionMode: This parameter is used to differentiate between mode A and mode B. This parameter should be replaced by a configuration for the second UL channel: when a configuration for the second UL channel is present, the UE should apply mode B. If a configuration for the second UL channel is absent, the UE should apply mode A.

eventThreshold: This parameter is used to control the threshold in the event evaluation. This parameter should be applicable for event-1 and event-7 as well.

eventInstanceCount, eventDetectionTimeWindowLength: These parameters control the event evaluation. The event triggers when a condition is fulfilled eventInstanceCount within the time eventDetectionTimeWindowLength.  These parameters should be applicable for event-1 and event-7 as well.

enabledCurrentBeamReport: This parameter is used to instruct the UE to include a measurement of the current beam in the report. This parameter should be applicable for event-1 and event-7 as well.

nrofReportedRS-UEIBR: This parameter is used to control how many new beams are included in total in the report. The UE always includes the beam that triggered the event.

newBeamResourceSetEvent2: This parameter is used to configure the new beams. We note that the CSI-ReportConfig already includes the mandatory IE resourcesForChannelMeasurement, and we propose to reuse that rather than defining a new IE.

New proposed parameters:

uplinkChannelConfiguration: This parameter provides the configuration of the second UL channel, which is a Type I CG-PUSCH. This  indicates a ConfiguredGrantConfigIndex. When this parameter is present, the UE reports according to mode B, and if the parameter is absent, the UE reports according to mode A.

eventIndicatorChannel: This parameter provides the configuration for the first UL channel and is associated with a dedicated SchedulingRequestId at least for Mode A.

eventId: The UE includes this parameter when it sends the report. The NW would use this to determine which event has occurred. In principle, we could use the reportConfigId for this purpose, but that identifier would be unnecessarily long, since not all CSI report configurations are associated with UE-initiated beam reporting.

Parameters in the CSI report configuration that are reused and relevant:

resourcesForChannelMeasurement: This parameter is used to provide the new beams to the UE. This can be either a list of CSI-RS resource sets or a list of CSI SSB resource sets. For UE initiated beam reporting, we do not support a combination of CSI-RS resource sets and CSI SSB resource sets.

carrier: This field describes in which serving cell the resourcesForChannelMeasurement are configured.

reportConfigType: this field is only valid for mode A and must in this case be set to aperiodic. The reportSlotOffsetList describes when the UE should send the report when triggered with this report. In practice, this parameter must be the same for all mode A events: otherwise, the NW will not be able to receive the report. Alternatively, we could introduce a new report in parallel (UEinitiated), which would in that case require the inclusion of slot offset as for an aperiodic report.

- In summary, we propose the following RRC parameter updates:

| Parameter name in the spec | Description |
| --- | --- |
| reportTransmissionMode-r19 | This parameter is used to inform one report transmission mode on UCI based beam report procedure for UE-initated/event-driven beam reporting. - ModeA: UCI in a dynamically scheduled PUSCH - ModeB: UCI in a pre-configured UL resource(s) Note: This par may be removed later if it can be inferred from other configurations |
| eventThreshold-r19 | This parameter is used to inform a threshold value for trigger event detection regarding Event-1, Event-2 and Event-7. |
| eventDetectionTimeWindowLength-r19 | This parameter is used to inform the time window for triggering event determination regarding Event-1, Event-2 and Event-7 |
| eventInstanceCount-r19 | This parameter is used to inform the number of event instances for at least one same new beam within a configured time window that the UE can initiate UEIBM report. This parameter applies to Event-1, Event-2 and Event-7. |
| enabledCurrentBeamReport-r19 | This parameter is to enable or disable whether current beam is always reported. - When enabled by RRC, the current beam + N beams from the measurement RSs for new beam(s) are reported l - Note: The reported current beam is NOT counted in the N reported beams. - When disabled by RRC, N beams are reported. |
| nrofReportedRS-UEIBR-r19 | This parameter is used to inform the number of reported RS(s) in a report format for UE-initiated/event-driven beam reporting. |
| newBeamResourceSetEvent2-r19 | This parameter is used to configure the new beam resource set for Event 2 (Opt1) |
| uplinkChannelConfiguration | The configuration of the UL channel used in mode B. |
| eventIndicatorChannel | This parameter is used to inform the UE how to transmit the first UL channel |
| reportId | The UE inserts this parameter in the report to inform the NW which event occurred. |

# Conclusion

In the previous sections we made the following observations:

Observation 1	The report for event-7 should include information on what TCI states to activate, and what TCI states to deactivate.

Observation 2	When a UE is scheduled using DCI format 0_3, the UCI is transmitted on one of the scheduled cells.

Observation 3	The UE retransmits the first UL channel until it receives a DCI format that indicates a resource for a second UL channel: the received DCI serves as an acknowledgment that the NW has received the first UL channel.

Observation 4	The beam that triggered the event is not necessarily the beam with the highest RSRP.

Observation 5	There is no reduction in ping pong beam switches if the NW is unaware of which beam triggered the event, i.e., which beam consistently fulfilled the condition.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The event evaluation for UE-initiated beam reporting is captured in a RAN2 specification.

Proposal 2	Reuse the RRC IE CSI-ResourceConfig to define the new beams for UE-initiated beam management.

Proposal 3	The periodicity of the event determination for event-2 is identical to the measurement periodicity defined by RAN4.

Proposal 4	The periodicity of the event determination for event-2 is identical to the measurement periodicity of the current beam RS.

Proposal 5	Postpone the discussion on the resetting mechanism of the counter.

Proposal 6	Do not support MAC CE update of the RSs in the RS resource set.

Proposal 7	Confirm the working assumption to support event-1 and event-7.

Proposal 8	For event-7, it should be possible to configure the UE to report up to 8 RSs.

Proposal 9	The UL-grant DCI format comprises DCI format 0_1/0_2/0_3.

Proposal 10	For mode A, support Alt-1: Introduce an RRC parameter, e.g., reportResourceRequest-UEIBR, associated with a dedicated SchedulingRequestId.

Proposal 11	Do not impose a constraint beforehand that the first channel should be the same for mode A and mode B.

Proposal 12	The report triggered in step-2 of mode A reuses properties from a normal aperiodic beam report, when it comes to configuration, CSI processing criteria and report priorities.

Proposal 13	The UE sends the step-2 beam report of mode A in response to the CSI request from the NW even when no event has occurred.

Proposal 14	For every reported new beam, the UE indicates if it fulfilled the event condition or not.

Proposal 15	Introduce an event identifier in the report sent over the second channel.

# References

RP-234007, New WID: NR MIMO Phase 5, Samsung (Moderator), RAN#102, Edinburgh, Scotland, December 2023

R2-2409200, Initial analysis on RAN2 impact for Rel-19 NR MIMO Ph5, Huawei/HiSilicon, RAN2#127bis, Hefei, China, October 2024

R2-2408092, RAN2 Impacts of Rel-19 NR MIMO, CMCC, RAN2#127bis, Hefei, China, October 2024

R2-2408125, Discussion on UE-initiated/event-driven beam management, vivo, RAN2#127bis, Hefei, China, October 2024

R2-2408667, User plane and Control plane impacts from MIMO, Ericsson, RAN2#127bis, Hefei, China, October 2024

R2-2408795, Discussion on the design considerations for MIMO Phase 5, Qualcomm Incorporated, RAN2#127bis, Hefei, China, October 2024

3GPP TS 38.133, NR; Requirements for support of radio resource management, RAN#101, Bangalore, India, v18.3.0, September 2023

3GPP TS 38.214 V18.2.0, NR; Physical layer procedures for data, RAN1#103, Maastricht, The Netherlands, March 2024