3GPP TSG RAN WG1 Meeting #119	R1-2409593

Orlando, US, November 18th – 22nd, 2024

Agenda Item:	9.3.1

Source:	Samsung

Title:	SBFD operation and procedures

Document for:	Discussion

1. Introduction

RAN approved the updated WID [1] including the following objectives.

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of time location of SBFD subbands in SIB is not precluded - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of frequency domain location of SBFD subbands in SIB is not precluded - […] - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2] - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon - UL transmissions within UL subband only - DL receptions within DL subband(s) only, except for CLI measurement by the UE outside of the DL subbands Note: When flexible symbols are used, it is not expected that any legacy Uplink symbol is converted to Downlink/SBFD symbols - Enhancement on resource allocation in frequency domain in SBFD symbols, including - resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols - handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols, including - resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots - CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol - Followings are assumed based on TR 38.858 - SBFD at the gNB side - Half duplex operation at the UE side - FR1 and FR2-1 - SBFD operation Option 4, i.e., both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs - Coexistence between non-SBFD aware UEs (including legacy UEs) and SBFD aware UEs in the cell operating SBFD at gNB side - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies - One UL subband for SBFD operation in an SBFD symbol (excluding legacy UL symbol/slot) within a TDD carrier - Mechanisms for SBFD operation shall also consider the adjacent channel coexistence between two operators - […]

In this contribution, we discuss remaining issues on semi-static indication of subbands and UE’s transmission, reception and measurement behaviours in SBFD symbol and non-SBFD symbols.

# 2. Details on SBFD Designs

  - 2.1. Remaining Issus on Semi-static indication of SBFD subbands

    - 2.1.1. Frequency domain indication of SBFD subbands

UE specific configuration on frequency location of SBFD subbands

The remaining issue is whether to support UE-specific frequency domain location of subbands. As shown in the agreement below, RAN1 agreed to support cell-specific configuration on time domain location, but the cell-specific frequency domain indication is still working assumption.

Agreement For RRC connected mode UEs, at least cell-specific configuration on time and frequency(working assumption) location of SBFD subbands is supported within a TDD carrier. - FFS: Additional support of UE-specific configuration on time and/or frequency locations of SBFD subbands

From RAN4’s study and Section 9 of TR38.858, the SBFD operation requires additional components like RF/IF subband filtering or analog filter to avoid RX blocking. These components should be tuned offline for a specific UL subband configuration. Therefore, the frequency configuration of subbands should be cell-specific or gNB-specific, not UE-specific.

One argument of benefit of the UE-specific frequency configuration of subband is to reduce inter-UE CLI. It is because the UE-specific frequency configuration of subbands can provide larger guardband compared to the guardband of cell-specific frequency configuration of subbands. However, the guardband between two UEs (TX UE and RX UE) can be implicitly created by gNB’s scheduling. Since RAN1 agreed to support L1-based CLI measurement and reporting, if the inter-UE CLI between two UE is severe, gNB will not schedule the two UEs in the same symbols. Therefore, without the UE-specific frequency configuration of subbands, gNB can avoid strong inter-UE CLI.

Proposal 1. Confirm the cell-specific configuration on frequency location of SBFD subbands.

- UE-specific frequency domain location of SBFD subbands is not necessary

    - 2.1.2. Time domain indication of SBFD subbands

Explicit Guard Symbols

To take into account transient period from SBFD symbols to non-SBFD symbols (or from non-SBFD symbols to SBFD symbol), explicit configuration for guard symbols may be necessary. In the previous RAN4#111 meeting, RAN4 discussed the necessity of transient period and their location.

- Agreement: - FFS requirement and cases for transient period between SBFD and non-SBFD: - Option 1: The existing TDD BS transmitter transient period, i.e., 10us for FR1 and 3us for FR2-1 - Other options are not precluded. - FFS location of transient period between SBFD and non-SBFD: - Option 1: The transmitter transient period shall be located within the SBFD slot - Other options are not precluded.

Since RAN4 still do not conclude the necessity and location of transient period, RAN1 can wait the discussion of explicit configuration for guard symbols. Regarding the necessity of explicit configuration for guard symbols, we see the necessity of explicit guard symbols to avoid collision of DL symbols right before SBFD symbols and UL subband by TA offset. (More detailed analysis for transient period is shown in our contribution [2]) For example, the guard symbol is applied to first SBFD symbols and the UL usable PRBs in the explicit guard symbols are not used for UL transmission. Note that the DL reception is not interrupted by the explicit guard symbol.

Figure 1. Necessity of explicit guard symbols [2]

Proposal 2. RAN1 waits the discussion on explicit configuration for guard symbols until RAN4 concludes the necessity and location of transient period

Interaction with legacy TDD slot configuration indications

The following three alternatives were identified in the previous RAN1 meeting for interaction with legacy TDD slot configuration indication via TDD-UL-DL-ConfigDedicated.

- Alt. 1: An SBFD aware UE does not expect to be provided with TDD-UL-DL-ConfigDedicated.

- Alt. 2: TDD-UL-DL-ConfigDedicated is not applicable to SBFD symbols. An SBFD aware UE interprets TDD-UL-DL-ConfigDedicated as legacy for non-SBFD symbols.

- Alt. 3: TDD-UL-DL-ConfigDedicated is applicable to both non-SBFD and SBFD symbols. An SBFD aware UE interprets TDD-UL-DL-ConfigDedicated as legacy for non-SBFD symbols, and interprets TDD-UL-DL-ConfigDedicated as link direction for SBFD symbols, i.e., whether to transmit within UL usable PRBs or to receive within DL usable PRBs.

In our views, TDD-UL-DL-ConfigDedicated is not essential to a cell supporting SBFD. The signaling TDD-UL-DL-ConfigDedicated is mainly used for flexible/dynamic TDD operation. The joint operation with SBFD and flexible/dynamic TDD was not considered and evaluated during Rel-18 Duplex study. Therefore, its benefit and use case are unclear. Nevertheless, this signaling TDD-UL-DL-ConfigDedicated can be applied to flexible symbol where SBFD subbands are not configured, which does not make any impacts to SBFD operation.

Proposal 3. An SBFD aware UE does not expect to be provided with TDD-UL-DL-ConfigDedicated for SBFD symbols

- Follow the legacy UE behavior for non-SBFD symbol, i.e., direction of a flexible symbol without SBFD subbands can be indicated by TDD-UL-DL-ConfigDedicated

The following three alternatives were considered in the previous RAN1 meeting for interaction with dynamic SFI in DCI format 2_0.

- Alt. 1: An SBFD aware UE does not expect to be configured to monitor DCI format 2_0 for the serving cell.

- Alt. 2: SFI in DCI format 2_0 is not applicable to SBFD symbols. An SBFD aware UE interprets SFI in DCI format 2_0 as legacy for non-SBFD symbols.

- Alt. 3: SFI in DCI format 2_0 is applicable to both non-SBFD and SBFD symbols. An SBFD aware UE interprets SFI in DCI format 2_0 as legacy for non-SBFD symbols, and interprets SFI in DCI format 2_0 as link direction for SBFD symbols, i.e. whether to transmit within UL usable PRBs or to receive within DL usable PRBs.

Similarly, the joint operation with SBFD and dynamic TDD makes UE behaviors complicated. And, the gNB-gNB CLI and the UE-UE CLI are not evaluated. If the dynamic DL/UL resource change is deemed necessary in a cell supporting SBFD operation, dynamic SBFD operation can be further studied and specified in later release. In this release, we propose any dynamic DL/UL resource change (including dynamic TDD operation and dynamic SBFD operation) is not considered

Proposal 4. An SBFD aware UE does not expect to be configured to monitor DCI format 2_0 for the serving cell configured with SBFD subbands

    - 2.1.3. Container for SBFD subband configuration

To indicate SBFD frequency/time indication, SIB or dedicated RRC signaling can be used. We compare two containers for SBFD subband configuration.

Table 1. Comparison of SIB and Dedicated RRC signaling

|  | SIB | Dedicated RRC signaling |
| --- | --- | --- |
| Pros/Motivations | - If RA operation in SBFD symbol for a UE during RRC idle/inactive UE is supported, SIB-based solution should be mandatorily supported. - At least, SBFD frequency indication is cell-common and SBFD time indication can be also cell-common. - It can be helpful to reduce signaling overhead if the number of SBFD-aware UEs is large. | - Dedicated RRC signaling is required to configure information in SIB to a RRC connected UE for Scell addition case. For example, ServingCellConfigCommon is used to configure information in ServingCellConfigCommonSIB to a UE. |
| Cons | - Since SIB1 is periodically transmitted, the SIB1 overhead can be concerned. | - If many SBFD-aware UEs exist in a cell, multiple dedicated RRC signaling should be transmitted - It cannot enable RA operation in SBFD for a UE during RRC idle/inactive UE |

Based on Table 1, SIB and dedicated RRC signaling have different motivations. In this end, we prefer to support both for SBFD subband configuration. Note that if SIB is used, the SBFD subband configuration can be added in SIB1 to avoid additional latency to receive other SIBx.

Observation 1. The motivations of supporting SIB1 and dedicated RRC signaling for SBFD configuration are

- The motivation of SIB1 is to indicate SBFD configuration to RRC connected/[idle/inactive] UE, like ServingCellConfigCommonSIB

- The motivation of dedicated RRC signaling is to configure cell-specific parameters for RRC connected UE for Scell Addition case, like ServingCellConfigCommon

Proposal 5. Cell-common SBFD frequency indication and SBFD time indication can be carried by both SIB1 and dedicated RRC signaling

  - 2.2. UE Behaviour and Procedures in SBFD symbol and non-SBFD symbol

    - 2.2.1. Enhancement on resource allocation in frequency domain

      - 2.2.1.1 Remaining Issues on FDRA type-0 in a single slot

Wideband PRG of PDSCH

In the last RAN1#118 meeting, RAN1 concluded that wideband precoder is only supported for contiguous PRBs as follow:

Conclusion If PRG is determined as wideband, UE does not expect to be scheduled with non-contiguous PRBs in SBFD symbols.

In case of prb-BundlingType=staticBundling, n2, n4 or wideband can be configured. If ‘wideband’ is configured, as per the conclusion, only one DL subband can be utilized for PDSCH reception. In other words, considering D-U-D SBFD configuration with 20MHz UL subband and 40MHz DL subband, the peak DL throughput is severely degraded when ‘wideband’ is configured (less than half peak DL throughput). To avoid such a degradation, ‘n2’ may be used when the scheduled PRB is non-contiguous as a fallback PRG granularity. Note that the PRG size of a PDSCH scheduled by DCI format 1_0 is always 2.

In case of prb-BundlingType=dynamicBundling, 1-bit PRB bundling size indicator in a DCI format may indicate PRG size. For example, if ‘0’ is indicated, the PRG size is determined from ‘bundleSizeSet2’. Otherwise, the PRG size is determined from ‘bundleSizeSet1’ If ‘bundleSizeSet1’ includes ‘n2-wideband’ or ‘n4-wideband’, the PRG size is further determined by checking the number of scheduled PRBs exceeds half of DL BWP size. That is, wideband PRG is determined only if the number of scheduled PRBs exceeds half of DL BWP. Consider the case where DL BWP contains NBWP=273 PRBs and each of two DL subbands contains NDLSB=106 PRBs (51 PRBs for UL subband and 5 RBs for each of two guardbands). Then, even if all of PRBs in one DL subband are scheduled in a PDSCH, the PRG granularity cannot be wideband since the # of scheduled contiguous PRBs < NBWP/2. To resolve this issue, RAN1 may consider the DL subband size, instead of DL BWP size, when determining PRG size.

Proposal 6. In case of prb-BundlingType=staticBundling and bundleSize=wideband, UE may assume the PRG size is equal to 2 when the scheduled PRBs overlap with two DL subbands in SBFD symbol.

Proposal 7. In case of prb-BundlingType=dynamicBundling and bundleSizeSet1= n2-wideband or n4-wideband, if PRB bundling size indicator field indicates ‘1’, the DL subband size can be used for PRG size determination, instead of DL BWP size.

Figure 2. Wideband PRG size across two symbols types

Another remaining issue is how to apply wideband PRG size when SPS PDSCHs, PDSCH repetitions and multi-PDSCHs across non-SBFD symbols and SBFD symbols (Configuration 2). As shown in Figure 2, suppose that ‘wideband’ is configured and PDSCH repetition#0 is mapped to non-SBFD symbols while the PDSCH repetition#1 is mapped to SBFD symbols. PDSCH repetition#0 in non-SBFD symbols may be consecutive so that the UE is able to transmit the repetition or PDSCH with ‘wideband’ PRB size. However, PDSCH repetition#1 in non-SBFD symbols may be non-consecutive since the PRBs within DL usable PRBs are only valid. In this case, the UE behavior is unclear. The potential options are 1) UE does not expect such a scheduling (i.e., if ‘wideband’ is configured, gNB should ensure the valid PRBs in all repetitions/PDSCHs are contiguous in SBFD symbol and non-SBFD symbol) or 2) UE does not drop the repetition/PDSCH if the valid PRBs in the repetition/PDSCH are non-contiguous in SBFD symbol or 3) UE receive the repetition/PDSCH by assuming PRG size = 2 if the valid PRBs in the repetition/PDSCH are non-contiguous in SBFD symbol.

Proposal 8. When SPS PDSCHs, PDSCH repetitions and multi-PDSCHs across non-SBFD symbols and SBFD symbols (Configuration 2), RAN1 further discusses how to apply wideband PRG size when the scheduled PRBs in a repetition/SPS PDSCH are contiguous in non-SBFD symbols, but the valid PRBs in a repetition/SPS PDSCH are non-contiguous in SBFD symbols.

    - 2.2.2. Tx/Rx across SBFD symbols and non-SBFD symbols

In the previous RAN1#118bis meeting, RAN1 identified three options for granularity of configuration 1 and 2.

Agreement: For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with Configuration 1 or Configurat 2 via RRC configuration. Select from the following options - Option 1: SBFD-aware UE is configured with Configuration 1 or Configuration 2 on a per UE per serving cell basis - Option 2: SBFD-aware UE is configured with Configuration 1 or Configuration 2 on a per UE per BWP per channel/signal basis - Separate configurations for configured and dynamic transmission/receptions for a given channel/signal are not precluded. - Option 3: Configuration 1 or Configuration 2 is on a per cell basis. - No separate UE capabilities for Configuration 1 and 2.

To support UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots, a lot of new UE behaviors are required. For example, in order to transmit PUSCH repetitions, the frequency offset to shift the frequency allocation of PUSCH repetition on SBFD symbols is needed. Also, for PDSCH repetitions, a different TB size determination is required. Therefore, we would prefer to introduce separate UE capability for configuration 1 and 2. That is, Option 3 is not a practical solution.

Regarding Option 1 and Option 2, we think option 1 is too restrictive. For example, if configuration 2 is configured to a UE, gNB should configure all necessary information for all DL/UL channels and signals to support configuration 2. Also, if gNB does not configure information for a certain channel/signal, UE behaviors on the channel/signal might be unclear.

Proposal 9. The granularity of the configuration is per UE per BWP per channel/signal basis (Option 2). If the configuration is not provided for a certain channel/signal, the default configuration is configuration 1.

How to determine which symbol type is used for transmission/reception in case of configuration 1

In the last RAN1#118 meeting, RAN1 reached the following agreement, where two options were captured to determine the valid symbol type.

Agreement For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only, the valid symbol type is determined as following. - For semi-statically configured transmissions/receptions without activation DCI, the valid symbol type is explicitly configured by RRC. - It is not applicable to PDCCH - For dynamically scheduled transmissions/receptions, the valid symbol type is determined based on the symbol type of the first transmission/reception. - For SP-CSI on PUCCH or PUSCH, type 2 CG PUSCH, SPS PDSCH and semi-persistent SRS, down-select from the following options: - Option 1: - The valid symbol type for SP-CSI on PUCCH or PUSCH is explicitly configured in CSI-ReportConfig. - The valid symbol type for type 2 CG PUSCH is explicitly configured in ConfiguredGrantConfig. - The valid symbol type for SPS PDSCH is explicitly configured in SPS-Config. - The valid symbol type for semi-persistent SRS is explicitly configured in [SRS-Config /SRS-ResourceSet/SRS-Resource]. - Option 2: - The valid symbol type for SP-CSI on PUCCH or PUSCH is determined based on the symbol type of the first PUSCH/ PUCCH after activation. - The valid symbol type for type 2 CG PUSCH is determined based on the symbol type of the first CG PUSCH associated with activation DCI. - The valid symbol type for SPS PDSCH is determined based on the symbol type of the first SPS PDSCH associated with activation DCI. - The valid symbol type for semi-persistent SRS is determined based on the symbol type of the first SRS after activation.

The valid symbol type is explicitly configured in the corresponding higher-layer parameters in option 1, while it is determined by the first transmission/reception occasion after scheduled DCI or activation DCI in option 2. In terms of scheduling flexibility, option 2 is more preferred since gNB can determine the valid symbol types by the scheduling/activation DCI. But, option 1 may provide more clean way to determine the valid symbol types without any interactions of the scheduling/activation DCI. The similar discussion has been done in Rel-16 URLLC, where it was proposed a physical layer priority of a channel/signal is indicated by DCI or explicitly configured in higher-layer parameters. At that time, RAN1 decided to support explicit physical layer configuration in higher-layer parameters. Also, there were very long discussions on ambiguities on the first SPS PDSCH and the first type-2 CG PUSCH after activation. Even though the activation DCI includes a DCI field (e.g., DAI in the DCI activating CG PUISCH), RAN1 agreed to ignore the DCI field for the first SPS PDSCH and the first type-2 CG PUSCH after activation. It means the first SPS PDSCH and the first type-2 CG PUSCH after activation are considered as CG, not DG. Only for timeline determination and corresponding collision handling determination, the first SPS PDSCH and the first type-2 CG PUSCH after activation are considered as DG. So, in order to keep the same design policy, we prefer option 1.

Proposal 10. Support Option 1 to determine the valid symbol type for Configuration 1.

Another issue is whether to apply the same rule to PUCCH transmission for HARQ-ACK for SPS PDSCH. Once activating SPS PDSCH reception, the PUCCH transmission for HARQ-ACK for SPS PDSCH is periodically transmitted. The PUCCH transmissions is allowed to be mapped to non-SBFD symbols or SBFD symbols in Configuration 2, but it is unclear this is allowed or not in Configuration 1. Our understanding is that one of merits of Configuration 1 is totally separate two symbol types in terms of scheduling. That, gNB’s single scheduling only affects single symbol types and gNB does not require to consider interaction between two symbol types. So, we prefer to keep the same rule to PUCCH transmission for HARQ-ACK for SPS PDSCH, i.e., the PUCCH transmission is only limited to one symbol type.

Proposal 11. PUCCH transmissions for HARQ-ACK for SPS PDSCH are only allowed to transmit one symbol types in Configuration 1.

      - 2.2.2.1 SPS, PDSCH repetitions and multi-PDSCH scheduling

In the previous RAN1#118 meeting and RAN1#118bis meeting, RAN1 made the following working assumption and agreement on determining the valid PRBs for PDSCH repetitions, SPS PDSCHs, multi-PDSCH scheduled by a single DCI.

Working Assumption For an SPS PDSCH configuration without repetitions, if the reception occasions are across SBFD symbols and non-SBFD symbols where each reception occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), PDSCH repetitions across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PDSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PDSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), - Option 5: Only the assigned PRBs within DL usable PRBs in SBFD symbols are considered to be valid. - For SPS PDSCH, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only - For PDSCH repetitions, FFS whether the number of PRBs for TBS determination is based on assigned PRBs, or based on assigned PRBs within DL usable PRBs only, or based on assigned PRBs within DL usable PRBs of the first repetition. - For multi-PDSCH scheduled by a single DCI, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only. Agreement - For multi-PDSCH scheduled by a single DCI with Configuration 2, the number of PRBs for TBS determination for a PDSCH in SBFD symbols is based on assigned PRBs within DL usable PRBs only. The number of PRBs for TBS determination for a PDSCH in non-SBFD symbols is based on assigned PRBs.

The basic idea of the working assumption is to use the PRBs within DL usable PRBs in SBFD symbol for PDSCH reception. The remaining issue is how to determine the number of PRBs for TBS determination. Note that, for single PDSCH reception, RAN1 already agreed to use the assigned PRBs within DL usable PRBs for TBS determination.

Table 2 shows the characteristic of three scheduling methods, PDSCH repetition, multi-PDSCH scheduled by a single DCI and SPS PDSCHs across non-SBFD symbols and SBFD symbols. Note that RAN1 agreed in the last RAN1 meeting to support separate TB size determination for multi-PDSCH scheduled by a single DCI

For PDSCH repetition, the same TB is repeatedly transmitted over multiple slots. The main motivation of PDSCH repetition is to provide a good coverage. Hence, it is reasonable to use the smaller number of PRBs for TBS determination. That is, the assigned PRBs within DL usable PRBs only. In the last RAN1 meeting, some companies raised a concern on additional complexity for checking the PDSCH repetition overlaps with SBFD symbols. Our understanding is it can be simply done since the SBFD time configuration is semi-static and the slots for PDSCH repetition are the consecutive slots from the first PDSCH reception slot.

For SPS PDSCHs, different TBs are transmitted on different SPS PDSCH reception occasion while their TBS is the same. If the smaller number of PRBs is used for TBS determination, unnecessarily low code-rate is applied. Otherwise, if the larger number of PRBs is used for TBS determination, high code-rate may result in PDSCH decoding failure. So, it would be beneficial to use the number of valid PRBs in each SPS occasion for TBS determination.

Table 2. Different PDSCH scheduling methods

|  | PDSCH repetition | Multi-PDSCH scheduling | SPS PDSCHs |
| --- | --- | --- | --- |
| Same TB or different TBs for each PDSCH occasion | Same TB | Different TBs | Different TBs |
| Same TB size or different TB size for each PDSCH occasion | Same TB size | Different TB size (agreed) | Same TB size |

Proposal 12. For SPS PDSCH without repetitions, TBS is determined by the number of valid PRBs in each occasion

Proposal 13. For PDSCH repetitions scheduled by a DCI, TBS is determined by the number of the assigned PRBs within DL usable PRBs for SBFD symbols if at least one PDSCH repetition overlaps with SBFD symbols. Otherwise, TBS is determined by the number of the scheduled PRBs in non-SBFD symbol.

      - 2.2.2.2 CG, PUSCH repetitions and multi-PUSCH scheduling

In the last RAN1#118bis meeting, RAN1 made the following agreement and listed options to determine frequency shift equations.

Agreement #118b At least in case when PUSCH frequency hopping is not enabled, for a CG PUSCH configuration without repetitions, if the transmission occasions are across SBFD symbols and non-SBFD symbols where each transmission occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), for PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PUSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for TBoMS across SBFD symbols and non-SBFD symbols in different slots, - The number of PRBs for PUSCH transmissions in SBFD and non-SBFD symbols is determined as legacy. - The PRBs for PUSCH transmissions in non-SBFD symbols are determined as legacy. - Consider the following options for determining starting PRB for PUSCH transmissions in SBFD symbols: - Option 1: One or multiple RBoffsetSBFD are configured and the starting PRB for PUSCH in SBFD symbol is determined according to one of the following equation: - Equation 1-A: RBstartSBFD=RBstartnon-SBFD+RBoffsetSBFD - Negative values of RBoffsetSBFD are not precluded. - Equation 1-B: RBstartSBFD=RBstartnon-SBFD+RBoffsetSBFDmodNBWPsize - Equation 1-C: RBstartSBFD=RBstartUL SB+RBstartnon-SBFD+RBoffsetSBFDmodNUL SBsize - Equation 1-D: RBstartSBFD=RBstartUL SB+RBstartnon-SBFD-RBstartUL SB+RBoffsetSBFDmodNUL SBsize - Option 2: The starting PRB for PUSCH in SBFD symbol is determined according to one of the following equation: - Equation 2-A: RBstartSBFD = RBstartUL SB+mod(RBstartnon-SBFD,NUL SBsize -NPUSCHsize) - Equation 2-B: RBstartSBFD = RBstartUL SB+mod(RBstartnon-SBFD,NUL SBsize ) - Equation 2-C: RBstartSBFD = round(RBstartnon-SBFD* NUL SBsizeNBWPsize) + RBstartUL SB - Equation 2-D: RBstartSBFD = RBstartUL SB+RBstartnon-SBFD - The variables are defined as follows: - RBstartSBFD is the starting PRB index of PUSCH in SBFD symbol with reference to the start of UL active BWP - RBstartnon-SBFD is the starting PRB index of PUSCH in non-SBFD symbol with reference to the start of UL active BWP - RBstartUL SB is the starting PRB index of UL usable PRBs with reference to the start of UL active BWP - NBWPsize is the number of PRBs of UL BWP - NUL SBsize is the number of PRBs of UL usable PRBs - NPUSCHsize is the number of PRBs for PUSCH transmissions - No additional configuration/indication to enable/disable whether to apply RBSBFDoffset in SBFD symbols. - FFS: Condition to enable/disable - UE does not expect that the PRBs for PUSCH transmissions in SBFD symbols after applying RBSBFDoffset to be overlapped with PRBs outside UL usable PRBs. - It applies at least to RA type 1. FFS for RA type 0. - Note: Other equations are not precluded.

Regarding Option 1 and option 2, we think both options are necessary. For example, if gNB does not configure the offset value for frequency shift, a UE can use option 2 to determine the starting PRB index on SBFD symbol. It can be also applied to msg3 PUSCH repetition if Configuration 2 is supported for msg3 PUSCH. Option 1 can be used when gNB further controls the frequency shift to avoid collision of PUSCHs among multiple UEs. It is worth noting that if UE’s active UL BWP configuration is different to UEs, Option 2 could not provide a good PUSCH multiplexing property. It is because the modulo operation is defined by the number of PRBs of UL usable PRBs.

Regarding the equations, Example 1-A and Example 2-D are the simplest. But, in Example 2-D, PUSCH allocated in PRB above UL usable PRBs cannot be shifted into UL usable PRBs. So, it is not flexible in terms of resource utilization. Equation 1-C and Equation 2-B can be considered.

Proposal 14. Support both option 1 and option 2. Option 2 is applied when no frequency shift offset is configured.

- For Option 1, use RBstartSBFD=RBstartUL SB+RBstartnon-SBFD+RBoffsetSBFDmodNUL SBsize

- For Option 2, use RBstartSBFD=RBstartUL SB+RBstartnon-SBFDmodNUL SBsize

If multiple frequency shift offsets (RBoffsetSBFD) are configured, one frequency shift offset can be indicated by a DCI format scheduling PUSCH. The simple way is to reuse FH offset indication by FDRA. For example, if FH is disabled, X MSB bits are used to indicate frequency shift offset, where X = 1 or 2 bits according to the number of the configured frequency shift offsets. If FH is enabled, NUL_hop  bits are used to indicate both frequency hopping offset and frequency shift offset.

Proposal 15. If multiple frequency shift offsets (RBoffsetSBFD) are configured, one frequency shift offset is indicated by a DCI format scheduling PUSCH.

Regarding the frequency offset is applicable for FDRA type 0, some companies argued the shifted PRBs on SBFD symbols are not aligned with RBG boundary. Our understanding is that FDRA type 0 is not a practical configuration for PUSCH transmission in UL subband. For example, there is the following scheduling restriction for PUSCH with FDRA type-0.

TS38.214 6.1.2.2.1 Uplink resource allocation type 0 In frequency range 1, only 'almost contiguous allocation' defined in [8, TS 38.101-1] is allowed as non-contiguous allocation per component carrier for UL RB allocation for CP-OFDM. In frequency range 2, non-contiguous allocation per component carrier for UL RB allocation for CP-OFDM is not supported. TS38.101-1 If CP-OFDM allocation satisfies following conditions, it is considered as almost contiguous allocation NRB_gap / (NRB_alloc + NRB_gap ) ≤ 0.25 and NRB_alloc + NRB_gap is larger than 106, 51 or 24 RBs for 15 kHz, 30 kHz or 60 kHz respectively where NRB_gap is the total number of unallocated RBs between allocated RBs and NRB_alloc is the total number of allocated RBs. The size and location of allocated and unallocated RBs are restricted by RBG parameters specified in clause 6.1.2.2 of TS 38.214 [10]. For UE that indicates support for almostContiguousCP-OFDM-UL, the almost contiguous signals in power class 1.5, 2 and 3, the allowed maximum power reduction defined in Table 6.2.2-2, Table 6.2.2-1 and Tables 6.2D.2-1 to 6.2D.2-5 are increased by CEIL{ 10 log10(1 + NRB_gap / NRB_alloc), 0.5 } dB, where CEIL{x,0.5} means x rounding upwards to closest 0.5dB. The parameter of LCRB which is used to specify valid RB allocation ranges for Outer and Inner RB allocations is replaced by (NRB_alloc + NRB_gap) for almost contiguous allocation cases

In short, the bandwidth of scheduled PRBs (including allocated PRBs and unallocated PRB between the allocated PRBs) should be larger than 19.08MHz, 18.36MHz, or 17.28MHz for 15kHz, 30kHz, or 60kHz SCS, respectively. The bandwidth is almost same as the UL subband. Also, the non-contiguous resource allocation by FDRA type-0 results in the allowed maximum power reduction, which is contradictive of SBFD operation for UL coverage extension. So, we think no optimization for FDRA type-0 is needed and the same frequency shift can be applied to FDRA type-0 as well.

On the other hands, FDRA type-1 with RBG granularity is supported in DCI format 0_2 or 0_3. The similar issue (RBG boundary mismatch) can occur for FDRA type-1 with RBG granularity. Note that the same issue occurs when frequency hopping is enabled, but it can be avoided by a proper FH offset configuration/indication. The same way can be reused for frequency shift offset values.

Proposal 16. The frequency shift is applicable to FDRA type-0 without any optimization.

If frequency hopping is enabled (also frequency hopping is disabled as well), we think that the frequency shift is firstly applied and then frequency hopping is applied. More detailed steps are listed below.

- Determine RBstart and number of scheduled RBs by FDRA (as legacy)

- If frequency shift is applied, the intermediate starting RB index (before applying frequency hopping) is

- RBstart,FSnon-SBFD=RBstart  if the PUSCH is scheduled in non-SBFD symbols

- RBstart,FSSBFD=fRBstart  if the PUSCH is scheduled in SBFD symbols

  - f(x) is the equation to shift a PUSCH into UL usable PRBs in frequency domain from the agreement

    - For example, fRBstart=RBstartUL SB+RBstart+RBoffsetshiftmodNUL SBsize, where RBoffsetshift is the frequency shift offset.

- If frequency shift is not applied, RBstart,FSnon-SBFD=RBstart, RBstart,FSSBFD=RBstart.

- If frequency hopping is applied, the starting RB index is

- RBstartnon-SBFD=RBstart,FSnon-SBFD                       for the 1st hopgnon-SBFDRBstart,FSnon-SBFD for the 2nd hop if the PUSCH is scheduled in non-SBFD symbols

  - gnon-SBFDx is the frequency hopping equation for non-SBFD symbols, i.e., gnon-SBFDx=x+RBoffsetnon-SBFDmod NBWPsize, where RBoffsetnon-SBFD is the frequency hopping offset for non-SBFD symbols

- RBstart SBFD=RBstart,FSSBFD               for the 1st hopgSBFDRBstart,FSSBFD for the 2nd hop if the PUSCH is scheduled in SBFD symbols

  - gSBFDx is the frequency hopping equation for SBFD symbols, i.e., gSBFDx=RBUL SB start+(x[-RBUL SB start]+RBoffsetSBFD) modNUL SBsize, where RBoffsetSBFD is the frequency hopping offset for SBFD symbols

- If frequency hopping is not applied, the starting RB index is

  - RBstartnon-SBFD=RBstart,FSnon-SBFD, if the PUSCH is scheduled in non-SBFD symbols

  - RBstart SBFD=RBstart,FSSBFD, if the PUSCH is scheduled in SBFD symbols

Regarding condition to enable/disable whether to apply frequency shift for SBFD symbols or not, we think no additional condition is needed. That is, if a UE is configured with Configuration 2 for PUSCH transmission and an occasion of PUSCH repetition type-A/CG PUCSH/multi-PUSCH overlaps with SBFD symbols, the frequency shift is applied.

Proposal 17. If both frequency shift and frequency hopping are applied, the frequency shift is firstly applied and then frequency hopping is applied. In other words, the starting PRB of the first hop of PUSCH in SBFD symbols is determined by the frequency shift.

    - 2.2.3 Enhancements on Frequency hopping for PUSCH and PUCCH

Frequency hopping equation update

In the last RAN1#118bis meeting, RAN1 made the following agreement with two alternatives.

Agreement: #118b For PUSCH intra-slot frequency hopping in SBFD symbols, the starting RB in each hop is given by: RBstart=RBstart, i=0RBUL SB start+(RBstart[-RBUL SB start]+RBoffset)modNUL SBsize, i=1 For PUSCH inter-slot frequency hopping in SBFD symbols and when pusch-DMRS-Bundling is not enabled, or for inter-slot frequency hopping for a PUSCH in SBFD symbols scheduled by RAR UL grant or DCI format 0_0 with CRC scrambled by TC-RNTI, the starting RB during slot nsμ is given by: RBstart(nsμ)=RBstart, nsμmod2=0RBUL SB start+(RBstart[-RBUL SB start]+RBoffset)modNUL SBsize, nsμmod2=1 Where - RBUL SB start is the starting PRB index of UL usable PRBs with reference to the start of UL active BWP. - NUL SBsize is the number of UL usable PRBs. - RBstart is the starting PRB index of the first PUSCH hop with reference to the start of UL active BWP. - RB offset is the frequency hopping offset for PUSCH in SBFD symbols - Note: Definition of nsμ is unchanged from existing specifications Decide in RAN1#119, one of the following options: - Alt1: Remove -RBUL SB start in the above equations - Alt2: Remove square brackets from [-RBUL SB start] in the above equations

Two alternatives were captured in the agreement. For Alt2, there was a concern for negative value by [-RBUL SB start]. We can see RBstart-RBUL SB start is always non-negative for both configurations.

- Configuration 1. If PUSCH is scheduled in SBFD symbol, the starting RB index of the first hop is derived from FDRA field as legacy. It means, the starting RB index is no less than RBUL SB start. Therefore, RBstart-RBUL SB start is always non-negative.

- Configuration 2. the starting RB index of the first hop is derived from FDRA field as legacy. It can be less than RBUL SB start since PUSCH can be also scheduled on non-SBFD symbol. However, as we agreed, the frequency shift offset is applied and then the shifted PRB index is again no less than RBUL SB start. Therefore, if RBstart is the shifted PRB index, RBstart-RBUL SB start is always non-negative.

Observation 2. For both configurations, RBstart-RBUL SB start is non-negative.

The frequency hopping equation is user for msg3 PUSCH as well. In FL summary [x], we described the drawback of Alt1 when considering the frequency hopping offsets defined in Table 8.3-1 in TS38.213. The table below is the first PRB index of the 2nd hop of msg3 PUSCH transmission. In the table, the SBFD configuration is (106-5-51-5-106), i.e., RBUL SB start=111,  NUL SBsize=51. If Alt1 is adopted, some of RB offsets does not provide a proper frequency gap. For example, when the starting PRB index of the first hop is 114 (RBstart=114) and RBoffset is -⌊NUL SBsize/4⌋=-12, the staring PRB index of the second hop is 111+114-12mod 51=111. Hence, only 3 RB difference exists. If Alt2 is adopted, the staring PRB index of the second hop is 111+114-111-12mod 51=153. See the highlighted values, which only provides 3dB gap.

|  | 00: RB_offset =N_size/2 |  | 01: RB_offset =N_size/4 |  | 10: RB_offset = -N_size/4 |  |
| --- | --- | --- | --- | --- | --- | --- |
| RB_start | Alt2 | Alt1 | Alt2 | Alt1 | Alt2 | Alt1 |
| 111 | 136 | 145 | 123 | 132 | 150 | 159 |
| 112 | 137 | 146 | 124 | 133 | 151 | 160 |
| 113 | 138 | 147 | 125 | 134 | 152 | 161 |
| 114 | 139 | 148 | 126 | 135 | 153 | 111 |
| 115 | 140 | 149 | 127 | 136 | 154 | 112 |
| 116 | 141 | 150 | 128 | 137 | 155 | 113 |
| 117 | 142 | 151 | 129 | 138 | 156 | 114 |
| 118 | 143 | 152 | 130 | 139 | 157 | 115 |
| 119 | 144 | 153 | 131 | 140 | 158 | 116 |
| 120 | 145 | 154 | 132 | 141 | 159 | 117 |
| 121 | 146 | 155 | 133 | 142 | 160 | 118 |
| 122 | 147 | 156 | 134 | 143 | 161 | 119 |
| 123 | 148 | 157 | 135 | 144 | 111 | 120 |
| 124 | 149 | 158 | 136 | 145 | 112 | 121 |
| 125 | 150 | 159 | 137 | 146 | 113 | 122 |
| 126 | 151 | 160 | 138 | 147 | 114 | 123 |
| 127 | 152 | 161 | 139 | 148 | 115 | 124 |
| 128 | 153 | 111 | 140 | 149 | 116 | 125 |
| 129 | 154 | 112 | 141 | 150 | 117 | 126 |
| 130 | 155 | 113 | 142 | 151 | 118 | 127 |
| 131 | 156 | 114 | 143 | 152 | 119 | 128 |
| 132 | 157 | 115 | 144 | 153 | 120 | 129 |
| 133 | 158 | 116 | 145 | 154 | 121 | 130 |
| 134 | 159 | 117 | 146 | 155 | 122 | 131 |
| 135 | 160 | 118 | 147 | 156 | 123 | 132 |
| 136 | 161 | 119 | 148 | 157 | 124 | 133 |
| 137 | 111 | 120 | 149 | 158 | 125 | 134 |
| 138 | 112 | 121 | 150 | 159 | 126 | 135 |
| 139 | 113 | 122 | 151 | 160 | 127 | 136 |
| 140 | 114 | 123 | 152 | 161 | 128 | 137 |
| 141 | 115 | 124 | 153 | 111 | 129 | 138 |
| 142 | 116 | 125 | 154 | 112 | 130 | 139 |
| 143 | 117 | 126 | 155 | 113 | 131 | 140 |
| 144 | 118 | 127 | 156 | 114 | 132 | 141 |
| 145 | 119 | 128 | 157 | 115 | 133 | 142 |
| 146 | 120 | 129 | 158 | 116 | 134 | 143 |
| 147 | 121 | 130 | 159 | 117 | 135 | 144 |
| 148 | 122 | 131 | 160 | 118 | 136 | 145 |
| 149 | 123 | 132 | 161 | 119 | 137 | 146 |
| 150 | 124 | 133 | 111 | 120 | 138 | 147 |
| 151 | 125 | 134 | 112 | 121 | 139 | 148 |
| 152 | 126 | 135 | 113 | 122 | 140 | 149 |
| 153 | 127 | 136 | 114 | 123 | 141 | 150 |
| 154 | 128 | 137 | 115 | 124 | 142 | 151 |
| 155 | 129 | 138 | 116 | 125 | 143 | 152 |
| 156 | 130 | 139 | 117 | 126 | 144 | 153 |
| 157 | 131 | 140 | 118 | 127 | 145 | 154 |
| 158 | 132 | 141 | 119 | 128 | 146 | 155 |
| 159 | 133 | 142 | 120 | 129 | 147 | 156 |
| 160 | 134 | 143 | 121 | 130 | 148 | 157 |

Proposal 18. For determining PUSCH frequency hopping in UL subband, Support Alt2: Remove square brackets from [-RBUL SB start] in the above equations

PUCCH Frequency Hopping on SBFD symbol

In the last RAN1#118 meeting, RAN1 agreed to support separate frequency configures for PUCCH transmissions in SBFD symbols and non-SBFD symbols, respectively.

Agreement Support separate frequency configurations for SBFD symbols and non-SBFD symbols in the same PUCCH-Resource. - pucch-ResourceId is not separately configured for SBFD and non-SBFD symbols - Support separate configurations of startingPRB and secondHopPRB for SBFD symbols and non-SBFD symbols - Introduce new RRC parameters in PUCCH-Resource to configure starting PRB and second hop PRB for SBFD symbols - FFS whether to support separate configurations of intraSlotFrequencyHopping for Configuration 1 or for both Configuration 1 and 2 - No change on the maximum number of PUCCH resources supported by a UE - Above PUCCH resources with the same pucch-ResourceId is counted as 1 resource FFS: UE behaviour when no separate configuration is provided for SBFD symbols, e.g. PUCCH transmissions in SBFD symbols for this pucch-ResourceId is not expected, or configurations for non-SBFD symbols are applied for SBFD symbols (in which case it is not expected that the configurations would lead to unexpected transmissions) etc.

The remaining issue is about UE behaviours when no separate frequency configuration for SBFD symbols. In this case, the same frequency configuration can be applied to both non-SBFD symbol and SBFD symbol unless the frequency configuration is not outsize of UL subband.

Proposal 19. When no separate configuration is provided for SBFD symbols, the frequency configuration for non-SBFD symbols is applied unless the frequency configuration for non-SBFD symbols is not outside of UL subband.

    - 2.2.4. Enhancement on CSI-RS Configurations and Reports

CSI-RS resource across two DL subbands

For CSI-RS resource across two DL subband, the following was agreed in RAN1#118bis.

Agreement For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands, - Option 3: It is up to UE capability to indicate one of the following: - CSI processing timeline and CPU counting are the same as legacy. Scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy.

The associated timeline for CSI-RS across two DL subbands was agreed as above. Apart from CSI processing timeline, the non-contiguous CSI-RS in frequency may also have impact to the counting of active resources and active ports. Non-contiguous CSI-RS have two chunks of consecutive PRBs for channel estimation which is doubled in comparison with legacy. Hence, it is good for RAN1 to further discuss whether to change the counting rule for non-contiguous CSI-RS in frequency (e.g., double the number of resource counts and port counts for the CSI-RS across two DL subbands).

Proposal 20. In case of CSI-RS across the two DL subbands, consider to double the counting of active resources and active ports for the CSI-RS.

Another issue for CSI-RS across the two DL subbands is the associated minimum bandwidth. In the legacy, the bandwidth of CSI-RS within DL BWP is equal or greater than min⁡(24,NBWPsize). In case of a CSI-RS across the two DL subbands, similar restriction should be defined with extension. In case of a CSI-RS across the two DL subbands, the frequency domain resources of the CSI-RS consists of 2 parts in different DL subbands. the number of PRBs for each of the 2 parts is ≥ min⁡(24,NDL-SBsize), where NDL-SBsize is the number of PRBs of DL subband which includes the associated part of the CSI-RS.

Proposal 21. In case of a CSI-RS across the two DL subbands, the frequency domain resources of the CSI-RS consists of 2 parts in different DL subbands.

- the number of PRBs for each of the 2 parts is ≥ min⁡(24,NDL-SBsize), where NDL-SBsize is the number of PRBs of DL subband which includes the associated part of the CSI-RS

CSI report enhancements

As agreed in RAN1#117, two options are identified for CSI report associated with P/SP CSI-RS.

Agreement For CSI report associated with periodic/semi-persistent CSI-RS, discuss and decide whether to support the following options. - Option A: For separate CSI reports on SBFD and non-SBFD, one CSI-ReportConfig is associated with CSI-RS(s) restricted to SBFD symbols only and the second CSI-ReportConfig is associated with CSI-RS(s) restricted to non-SBFD symbols only. - gNB configuration may not ensure that the CSI-RS associated with each CSI-ReportConfig is confined to either SBFD symbols or non-SBFD symbols only. - For the CSI-ReportConfig associated with CSI-RS(s) restricted to SBFD symbols only, only CSI-RS transmission occasions within SBFD symbols are used for CSI derivation. For the CSI-ReportConfig associated with CSI-RS(s) restricted to non-SBFD symbols only, only CSI-RS transmission occasions within non-SBFD symbols are used for CSI derivation. - Option B: Enhance Rel-18 NES CSI reporting framework to support one CSI-ReportConfig with one sub-configuration associated with SBFD symbols and the other sub-configuration associated with non-SBFD symbols

We are supportive of Option B. It is notable that the maximum number of configured CSI report for a particular UE is subject to UE capability. To avoid the increase of the number of CSI report configuration for SBFD operation (i.e., for SBFD symbols and for non-SBFD symbols), it is beneficial to support a CSI-ReportConfig with CSI for SBFD symbols and CSI for non-SBFD symbols in one reporting instance. Similar as the discussion above, for the CSI for SBFD symbols, only CSI-RS transmission occasions within SBFD symbols are used for CSI derivation; for the CSI for non-SBFD symbols, only CSI-RS transmission occasions within non-SBFD symbols are used for CSI derivation. To support this type of CSI-ReportConfig, the design of Rel-18 CSI reporting introduced for NES can be largely reused. A CSI-ReportConfig for NES is configured with multiple sub-configurations, where each configuration can be associated with spatial/power parameters. For SBFD operation, a sub-configuration can either associated with SBFD symbols or non-SBFD symbols, additionally with the spatial/parameter associated with the SBFD symbols or the non-SBFD symbols (e.g., different symbol types may correspond to different transmission antenna ports or different DL transmission power). Based on NW triggering, UE can report the CSIs for the sub-configuration associated with SBFD symbols and CSIs for the sub-configuration associated with non-SBFD symbols. For option B, an issue for further study is how to identify a sub-configuration is for SBFD symbols or non-SBFD symbols.

Proposal 22. For CSI report associated with periodic/semi-persistent CSI-RS, support option B.

- For option B, further study how to identify a sub-configuration is for SBFD symbols or for non-SBFD symbols

Agreement For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation.

In RAN#118bis, the valid symbol type for CSI derivation was agreed. A follow-up issue is how to determine the CSI reference resource for the associated CSI report. In TS38.214, the definition of a CSI reference resource for a given CSI report is as follows.

#### TS38.214 #### 5.2.2.5 CSI reference resource definition The CSI reference resource for a serving cell is defined as follows: - In the frequency domain, the CSI reference resource is defined by the group of downlink physical resource blocks corresponding to the band to which the derived CSI relates. - In the time domain, the CSI reference resource for a CSI reporting in uplink slot n' is defined by a single downlink slot n-nCSI_ref-Koffset⋅2μDL2μKoffset, where Koffset is a parameter configured by higher layer as specified in clause 4.2 of [6 TS 38.213], and where μKoffsetis the subcarrier spacing configuration for Koffset with a value of 0 for frequency range 1 and for FR2-NTN, - where +Nslot,offset,ULCA2μoffset,UL-Nslot,offset,DLCA2μoffset,DL∙2μDL and and are the subcarrier spacing configurations for DL and UL, respectively, and Nslot, offsetCA and are determined by higher-layer configured ca-SlotOffset for the cells transmitting the uplink and downlink, as defined in clause 4.5 of [4, TS 38.211] - where for periodic and semi-persistent CSI reporting - if a single CSI-RS/SSB resource is configured for channel measurement nCSI_ref is the smallest value greater than or equal to 4⋅2µDL, such that it corresponds to a valid downlink slot, or - if multiple CSI-RS/SSB resources are configured for channel measurement nCSI_ref is the smallest value greater than or equal to , such that it corresponds to a valid downlink slot. - where for aperiodic CSI reporting, if the UE is indicated by the DCI to report CSI in the same slot as the CSI request, nCSI_ref is such that the reference resource is in the same valid downlink slot as the corresponding CSI request, otherwise nCSI_ref is the smallest value greater than or equal to , such that slot n- nCSI_ref corresponds to a valid downlink slot, where Z' corresponds to the delay requirement as defined in Clause 5.4. - when periodic or semi-persistent CSI-RS/CSI-IM or SSB is used for channel/interference measurements, the UE is not expected to measure channel/interference on the CSI-RS/CSI-IM/SSB whose last OFDM symbol is received up to Z' symbols before transmission time of the first OFDM symbol of the aperiodic CSI reporting. A slot in a serving cell shall be considered to be a valid downlink slot if: - it comprises at least one higher layer configured downlink or flexible symbol, and - it does not fall within a configured measurement gap for that UE If there is no valid downlink slot for the CSI reference resource corresponding to a CSI Report Setting in a serving cell, CSI reporting is omitted for the serving cell in uplink slot n'.

Note that the valid downlink slot is used to identify the slot where the associated CSI derivation is based on. However, the existing valid slot has no differentiation between SBFD symbol and non-SBFD symbol which is incorrect for the CSI derivation for a particular the valid symbol type (i.e., SBFD symbol or non-SBFD symbol). Hence, we have the following proposal to re-define the valid downlink slot associated with CSI reference resource in this case.

Proposal 23. If the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is configured, the valid downlink slot associated with CSI reference resource for the CSI report is defined as follows:

- A slot comprises at least one higher layer configured downlink or flexible symbol, where the symbol is with the same symbol type as the configured valid symbol type; and

- The slot does not fall within a configured measurement gap for that UE

Currently, if timeRestrictionForChannelMeasurements or timeRestrictionForInterferenceMeasurements in CSI-ReportConfig is set to "Configured", the latest measurement occasion of the resource for channel measurement and/or interference measurement is used for CSI computation. However, if the valid symbol type for CSI derivation is configured, the latest occasion of the associated periodic/semi-persistent should be within the symbols with the same symbol type as the configured valid symbol type for CSI derivation.

Proposal 24. If the timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to "Configured", and if the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is configured,

- The most recent occasion of the periodic/semi-persistent CSI-RS resource for channel measurement for CSI derivation is within the symbols with the same symbol type as the configured valid symbol type

Proposal 25. If the timeRestrictionForInterferenceMeasurements in CSI-ReportConfig is set to "Configured", and if the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is configured,

- The most recent occasion of periodic/semi-persistent CSI-IM and/or NZP CSI-RS for interference measurement for CSI derivation is within the symbols with the same symbol type as the configured valid symbol type

  - 2.3 Configurations on SBFD symbols and non-SBFD symbols

In this section, we discuss separate configurations for UL transmission on SBFD symbols and non-SBFD symbols. Note that PUSCH/PUCCH frequency hopping is discussed in 2.2.3.1.

Separate UL power control parameters

In the last RAN1#118 meeting, it was agreed to down-select from the following three options.

Agreement For a single TRP scenario, for separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols based on unified TCI state framework, down-select from the following options to support separate configurations of UL power control parameters. - Option 1: One TCI codepoint is mapped to separate TCI states for SBFD symbols and non-SBFD symbols respectively - FFS: No RRC impact - FFS: New MAC CE or reuse existing MAC CE - Option 2: Same unified TCI state is associated with separate UL power control parameters for SBFD symbols and non-SBFD symbols - FFS whether a new Uplink-powerControlId is introduced for SBFD symbols, or new P0AlphaSets are introduced in Uplink-powerControl for SBFD symbols for PUSCH, PUCCH and SRS respectively - FFS: Impact on MAC CE - Option 3: Same unified TCI state is associated with same UL power control parameters for SBFD symbols and non-SBFD symbols - A power offset is introduced for transmission in SBFD symbols for PUSCH, PUCCH, and SRS - FFS: Same offset or different offsets for all channels - FFS: RRC or MAC CE introduced to convey power offset - FFS whether the maximum number of closed power control loops is increased to be more than 2. - FFS: Whether/How to handle the case where UL power control is not associated within TCI state.

It is worth noting that there was a parallel discussion on whether to support separate beam/spatial configuration for non-SBFD symbol and SBFD symbols. If the separate beam/spatial configuration is supported, option 1 gives a unified framework to separately configure UL power control parameters and beam/spatial parameters. Also, NR specification already supports joint TCI indication for M-TRP deployment, where each TCI is mapped to each TRP. Also, we expect no RRC impact and the existing MAC CE (e.g., Enhanced Unified TCI States Activation/Deactivation MAC CE for Joint TCI States) can be reused.

Option 2 and option 3 aim to configure UL power control parameters separately while keeping the same TCI state. Regarding two options, option 2 would be preferred since it can reuse the existing RRC configuration, Uplink-PowerControl. Option 3 can be considered but it may impact RRC design and MAC CE design.

Proposal 26. For separate UL power control, support Option 1: One TCI codepoint is mapped to separate TCI states for SBFD symbols and non-SBFD symbols respectively.

- No RRC impact and reuse the existing MAC CE, Enhanced Unified TCI States Activation/Deactivation MAC CE for Joint TCI States

Beam/spatial relation

Figure 3. Best beam/spatial relation for Non-SBFD symbol and SBFD symbol

From RAN4’s study of TR38.858, the beam isolation value is important to avoid RX blocking and to obtain 1dB desense for self-interference. It is evident that amount of self-interference is determined by TX-RX beam pair. For example, as in Figure 3, if there is a clutter in TX/RX beam boresight direction, UL performance is suffered from strong self-interference. In other words, some Tx-Rx beam pair can be restricted in SBFD symbol, but all Tx-Rx beam pair is allowed in non-SBFD symbol.

Also, RAN-P endorsed the following objective to handle gNB-gNB CLI. If an aggressor gNB receives the strongest DL beam information and CLI-mitigation request, the aggressor gNB may avoid using the beam corresponding to the strong DL beam information in SBFD symbol. But the aggressor gNB still keep using the beam in non-SBFD symbol. In short, to properly handle the gNB-gNB CLI, gNB may use different beams for non-SBFD symbols and SBFD symbols.

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request

To indicate separate beams, the existing unified TCI frameworks for M-TRP can be reused. This is the same as in Option 1 under separately UL power control.

Proposal 27. RAN1 to support separate UL beam (QCL/TCI states) for SBFD symbols.

- One TCI codepoint is mapped to separate TCI states for SBFD symbols and non-SBFD symbols respectively

- No RRC impact and reuse the existing MAC CE, Enhanced Unified TCI States Activation/Deactivation MAC CE for Joint TCI States

Separate SRS configurations

In the last RAN1#118bis meeting, two options for separate SRS configurations for SBFD and non-SBFD symbols were discussed.

Agreement #118b Support separate SRS configurations for SBFD and non-SBFD symbols. Select one from the following options. - Option 1: Support separate SRS-ResourceSets configurations for SBFD and non-SBFD symbols for a given usage. - SRS-ResourceSet configured for SBFD symbol is only applicable to SRS transmission occasions in SBFD symbols. - For periodic and semi-persistent SRS, the SRS transmissions in non-SBFD symbols are dropped. - For aperiodic SRS with available slot counting, only SBFD symbols are available for SRS transmission. - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the SBFD symbols. - SRS-ResourceSet configured for non-SBFD symbol is only applicable to SRS transmission occasions in non-SBFD symbols. - For periodic and semi-persistent SRS, the SRS transmissions in SBFD symbols are dropped. - For aperiodic SRS with available slot counting, only non-SBFD symbols are available for SRS transmission. - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the non-SBFD symbols. - FFS applicable usages - Option 2: Support separate configurations for SBFD and non-SBFD symbols in the same SRS-Resource. - At least freqDomainPosition, freqDomainShift and freqHopping are separately configured. - FFS separate configurations of other parameters - FFS applicable usages

Regarding option 1 and option 2, it is clear that option 1 is more flexible. For example, SRS-ResourceSet can contain UL power control parameters (these are applicable when unified TCI is not enabled) so that SRS-ResourceSet for SBFD symbol can include proper UL power control parameters. Also, we think different time domain configurations, resourceMapping, are required since # of UL symbols and # of SBFD symbols in SBFD slots are different. Furthermore, different periodicity and offset is needed for semi-persistent/periodic SRS transmission. Having said that, we see the limitation of Option 2.

Proposal 28. RAN1 to support option 1 for separate SRS configurations for SBFD and non-SBFD symbols

  - 2.4 Collision Handling

The following agreements were made in RAN1#116 meeting regarding the collision handling of overlapping channels.

Agreement For SBFD-aware UE transmission and reception in an SBFD symbol, consider the following options to determine link direction, i.e. whether to transmit or to receive in the SBFD symbol. - Option 1: UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any). - Option 2: link direction is indicated by gNB explicitly. Other options are not precluded. Agreement For SBFD-aware UEs, collisions between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol may be addressed or alleviated with proper scheduling. The following cases of potential collisions, [if link direction indication is not supported or provided], can be further studied to see if any change to the current specs is necessary: - Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH - Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH - Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission - Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission - Case 5: SSB vs. dynamically scheduled or configured UL transmission - e.g., PUSCH, PUCCH, PRACH, SRS - Case 6: Dynamic or semi-static DL vs. valid RO Note: In addition to collision between UL transmission and DL reception in the same SBFD symbol(s), collision between UL transmission and DL reception in different symbol(s) due to lack of sufficient transition time between Tx/Rx at UE side is also included.

The determination of link direction is not explicitly within the scope of the WID and may not need to be specified. Instead of determining the link direction, the discussion should focus on the UE behaviour on receiving/transmitting a channel/signal. The issue is whether a UE can be scheduled/configured with a DL reception in DL sub-band and a UL transmission in UL sub-band on SBFD symbols, if it is allowed, how to deal with the overlapping for a DL reception and a UL transmission. Actually, a similar issue exists in legacy flexible symbols as well as HD-UE and link direction was not introduced in legacy. The collision issues have no significant difference from legacy cases and the agreed UE behaviours for SBFD are the same as legacy flexible symbols or HD-UEs if link direction is not supported nor provided.

Observation 3. The link direction does not need to be specified in an SBFD symbol.

In previous meetings, the collision issues focused on two overlapping channels, a group of more than two overlapping channels should also be specified. An example is given by Figure 4. Different orders of resolving the collision would result in different UE behaviour. If the UE first resolves the overlapping of DG PDSCH and CG PUSCH, the PUCCH can be transmitted, however, if the UE multiplexes the UCI in the CG PUSCH first and then resolves the overlapping of DG PDSCH and CG PUSCH, the UCI would be dropped together with the CG PUSCH.

Figure 4. An example of a group of three overlapping channels

Proposal 29. RAN1 should specify the order of resolving the collision of overlapping channels for a group of more than two overlapping channels.

  - 2.5 Other Considerations

    - 2.5.1 UL transmit timing

For Rel-19 SBFD, the UL transmit timing of the UL channels/signals transmitted by the UE in the SBFD UL subband needs to be decided.

For reception by the gNB in the UL slot, the Rx timing misalignment between the UL signals received from different UEs is controlled such that ideally it falls within the CP.

In Rel-15 NR, the UL transmit timing procedure uses MAC-CE signaled timing advance as a negative transmission time offset at the UE between the start of a DL slot as observed by the UE and the start of a slot in the UL. The UL frame number i for transmission from the UE is defined as TTA=NTA+NTA,offsetTc before the start of the corresponding DL frame at the UE in 38.211. For NR single carrier, the UE maintains a single TA loop. The sequence of gNB signaled TA commands in the MAC-CEs is used by the UE together with UE-autonomous TA adjustment when tracking the DL reference timing to set the UL transmit timing.

A default value depending on the FR and FDD/TDD is assumed by the UE for NTA,offset, or RRC signaling configures the UE with a value through parameter n-TimingAdvanceOffset for the serving cell. Note that if the UE is configured with two UL carriers for a serving cell, the same timing advance offset value NTA,offset applies to both carriers.

In Rel-15 NR, up to 2 different/separate timing advance groups (TAGs) can be configured to better support CA and DC deployments. When the different UL component carriers from the UE are all received at the same geographical location, i.e., gNB reception is co-located, the same TA value can usually be employed for all the UL component carriers. When different UL component carriers are received at different geographical locations, i.e., non-co-located, the different UL carriers can require different TA values to best align the UL Rx timings at these distinct reception sites. An example is the case of RRH-based gNB CA deployments or NR-NR dual connectivity where different UL component carriers are usually terminated at different BS sites. RRC then configures the UE with UL CCs in 2 different/separate timing advance groups (TAGs). Separate TA commands apply to these 2 different TAGs. All CCs in the same TAG are subject to the same TA command. The UE maintains 2 separate TA loops, one for each TAG.

Rel-18 NR then introduced the possibility to configure the UE with two TAs on the same serving cell in support of multi-DCI based operation. This is motivated by UL performance improvements to enhance the support of UL multi-TRP deployments.

For Rel-19 SBFD, the open design question is if the UL transmit timing from the UE perspective should be the same/shared as in the UL slot, or if support of different/separate UL transmit timings in the UL subband and UL slot can be beneficial for improved UL reception performance in the gNB.

The underlying issue is the gNB-side SIC implementation. Analog and digital SIC components can work better if self-interference is cancelled on pair-wise aligned (from the gNB perspective) DL and UL symbols. Equalization of the received UL symbols across DL symbol boundaries is best avoided. Note that aligned UL Rx timing (usually to within the CP) for UL receptions from multiple UEs by the gNB does not imply DL-UL alignment with respect to the gNB DL Tx timing. Another consideration is SIC complexity. When symbol-level equalization is desired, SIC processing of the received UL symbol across 2 partial DL symbols results in doubled coefficients.

The question of UL transmit timing affects both UL demodulation performance in the gNB and gNB complexity required for SIC.

In the Rel-18 Duplex enhancements, UL transmit timing was somewhat discussed but more detailed considerations then left for the work item phase. One key aspect is the assumed gNB side SIC implementation. In presence of RF degradations, the net effect of DL-UL mis-aligned symbol trains due to SIC processing can be reduced. For BB-centric implementations, the net effect and benefit to allow for DL-UL aligned symbol trains can be larger. 38.858 summarized these findings.

38.858 Section 6.1.3 BS self-interference due to time misalignment

Time misalignment at gNB between UL receptions and DL transmissions due to configuration of non-zero NTA,offset at UE can lead to increased interference assuming no gNB transmit chain impairments and no filtering of DL subband(s) in the gNB Rx chain.

Simulation results from one source [41] show that the increase of self-interference on the UL subband due to misaligned timing between UL reception and DL transmission at the gNB can be quite small (~1dB) when impairments in the gNB transmit chains and filtering of DL subbands in the gNB Rx chains are considered. Filtering that suppresses self-interference from DL subbands in the gNB Rx chains could incur some switching time/delay to bypass the filter in UL symbols and could introduce some insertion loss.

For Rel-19 SBFD, we see 2 possible design options for the UL transmit timing of an UL channel/signal in the SBFD UL subband from the UE to down-select from.

Option 1: The UL transmit timing in the SBFD UL subband and in the UL slot is the same.

Note: The SBFD-aware UE then follows existing Rel-15 behavior to determine its UL transmit timing in the SBFD UL subband.

Option 2: The UL transmit timings in the SBFD UL subband and in the UL slot can be different.

- Option 2a: using a single TA on the serving cell but 2 separately configured NTA,Offset values for the UL subband the UL slot, respectively.

- Option 2b: using 2 separate TAs on the serving cell, one for the UL subband and the other for the UL slot.

Note: The SBFD-aware UE then follows the existing Rel-18 2-TA feature on the serving cell introduced for mTRP.

Option 1 (same/shared) can simplify the UE-side operation. The SBFD-aware UE follows Rel-15 UL transmit timing for any UL transmission in the SBFD UL subband and the UL slot. gNB-side complexity can be increased and UL demodulation performance by the gNB in the SBFD UL subband can be negatively impacted due to DL-UL symbol mis-alignment from the gNB perspective.

Options 2a and 2b (different/separate) allow to separately adjust the UL Rx timings from the same UE at the gNB for UL transmissions received in the SBFD UL subband and the UL slot, respectively.

Options 1 (same/shared) and 2b (different/separate: using 2-TA from mTRP) are supported by existing Rel-18 specifications in the sense that the UL transmit timing procedure in 38.213 and corresponding MAC-CE signalling in 38.321 exists. Option 2a (different/separate: using 2 configured NTA,Offset values) would require some changes in RRC.

For Rel-19 SBFD, both Options 2a and 2b (different/separate) may require additional specification support to address and resolve potential collision cases arising when the UL transmit timing of the UE at the end of the UL subband must be changed prior to UL transmission in the UL slot from the UE. For example, dropping, skipping priority-based rule may be applied to the last symbol of the UL transmission in the SBFD UL subband when overlap with the first UL symbol of the UL slot would result. Note that Rel-18 mTRP already provides some rules to resolve such cases.

In summary, we propose to capture these possible design options for down-selection.

Proposal 30. For UL transmissions of an UL channel/signal by the SBFD-aware UE in the SBFD UL subband and the UL slot, the following design options are identified:

- Option 1: The UL transmit timing in the SBFD UL subband and in the UL slot is the same.

Note: The SBFD-aware UE then follows existing Rel-15 behavior to determine its UL transmit timing in the SBFD UL subband.

- Option 2: The UL transmit timings in the SBFD UL subband and in the UL slot can be different.

- Option 2a: using a single TA on the serving cell but 2 separately configured NTA,Offset values for the UL subband the UL slot, respectively.

- Option 2b: using 2 separate TAs on the serving cell, one for the UL subband and the other for the UL slot.

Note: The SBFD-aware UE then follows the existing Rel-18 2-TA feature on the serving cell introduced for mTRP.

# 3. Conclusion

This contribution discussed the SBFD operation design and following observations and proposals are given:

Observation 1. The motivations of supporting SIB1 and dedicated RRC signaling for SBFD configuration are

- The motivation of SIB1 is to indicate SBFD configuration to RRC connected/[idle/inactive] UE, like ServingCellConfigCommonSIB

- The motivation of dedicated RRC signaling is to configure cell-specific parameters for RRC connected UE for Scell Addition case, like ServingCellConfigCommon

Observation 2. For both configurations, RBstart-RBUL SB start is non-negative.

Observation 3. The link direction does not need to be specified in an SBFD symbol.

Proposal 1. Confirm the cell-specific configuration on frequency location of SBFD subbands.

- UE-specific frequency domain location of SBFD subbands is not necessary

Proposal 2. RAN1 waits the discussion on explicit configuration for guard symbols until RAN4 concludes the necessity and location of transient period

Proposal 3. An SBFD aware UE does not expect to be provided with TDD-UL-DL-ConfigDedicated for SBFD symbols

- Follow the legacy UE behavior for non-SBFD symbol, i.e., direction of a flexible symbol without SBFD subbands can be indicated by TDD-UL-DL-ConfigDedicated

Proposal 4. An SBFD aware UE does not expect to be configured to monitor DCI format 2_0 for the serving cell configured with SBFD subbands

Proposal 5. Cell-common SBFD frequency indication and SBFD time indication can be carried by both SIB1 and dedicated RRC signaling

Proposal 6. In case of prb-BundlingType=staticBundling and bundleSize=wideband, UE may assume the PRG size is equal to 2 when the scheduled PRBs overlap with two DL subbands in SBFD symbol.

Proposal 7. In case of prb-BundlingType=dynamicBundling and bundleSizeSet1= n2-wideband or n4-wideband, if PRB bundling size indicator field indicates ‘1’, the DL subband size can be used for PRG size determination, instead of DL BWP size.

Proposal 8. When SPS PDSCHs, PDSCH repetitions and multi-PDSCHs across non-SBFD symbols and SBFD symbols (Configuration 2), RAN1 further discusses how to apply wideband PRG size when the scheduled PRBs in a repetition/SPS PDSCH are contiguous in non-SBFD symbols, but the valid PRBs in a repetition/SPS PDSCH are non-contiguous in SBFD symbols.

Proposal 9. The granularity of the configuration is per UE per BWP per channel/signal basis (Option 2). If the configuration is not provided for a certain channel/signal, the default configuration is configuration 1.

Proposal 10. Support Option 1 to determine the valid symbol type for Configuration 1.

Proposal 11. PUCCH transmissions for HARQ-ACK for SPS PDSCH are only allowed to transmit one symbol types in Configuration 1.

Proposal 12. For SPS PDSCH without repetitions, TBS is determined by the number of valid PRBs in each occasion

Proposal 13. For PDSCH repetitions scheduled by a DCI, TBS is determined by the number of the assigned PRBs within DL usable PRBs for SBFD symbols if at least one PDSCH repetition overlaps with SBFD symbols. Otherwise, TBS is determined by the number of the scheduled PRBs in non-SBFD symbol.

Proposal 14. Support both option 1 and option 2. Option 2 is applied when no frequency shift offset is configured.

- For Option 1, use RBstartSBFD=RBstartUL SB+RBstartnon-SBFD+RBoffsetSBFDmodNUL SBsize

- For Option 2, use RBstartSBFD=RBstartUL SB+RBstartnon-SBFDmodNUL SBsize

Proposal 15. If multiple frequency shift offsets (RBoffsetSBFD) are configured, one frequency shift offset is indicated by a DCI format scheduling PUSCH.

Proposal 16. The frequency shift is applicable to FDRA type-0 without any optimization.

Proposal 17. If both frequency shift and frequency hopping are applied, the frequency shift is firstly applied and then frequency hopping is applied. In other words, the starting PRB of the first hop of PUSCH in SBFD symbols is determined by the frequency shift.

Proposal 18. For determining PUSCH frequency hopping in UL subband, Support Alt2: Remove square brackets from [-RBUL SB start] in the above equations

Proposal 19. When no separate configuration is provided for SBFD symbols, the frequency configuration for non-SBFD symbols is applied unless the frequency configuration for non-SBFD symbols is not outside of UL subband.

Proposal 20. In case of CSI-RS across the two DL subbands, consider to double the counting of active resources and active ports for the CSI-RS.

Proposal 21. In case of a CSI-RS across the two DL subbands, the frequency domain resources of the CSI-RS consists of 2 parts in different DL subbands.

- the number of PRBs for each of the 2 parts is ≥ min⁡(24,NDL-SBsize), where NDL-SBsize is the number of PRBs of DL subband which includes the associated part of the CSI-RS

Proposal 22. For CSI report associated with periodic/semi-persistent CSI-RS, support option B.

- For option B, further study how to identify a sub-configuration is for SBFD symbols or for non-SBFD symbols

Proposal 23. If the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is configured, the valid downlink slot associated with CSI reference resource for the CSI report is defined as follows:

- A slot comprises at least one higher layer configured downlink or flexible symbol, where the symbol is with the same symbol type as the configured valid symbol type; and

- The slot does not fall within a configured measurement gap for that UE

Proposal 24. If the timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to "Configured", and if the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is configured,

- The most recent occasion of the periodic/semi-persistent CSI-RS resource for channel measurement for CSI derivation is within the symbols with the same symbol type as the configured valid symbol type

Proposal 25. If the timeRestrictionForInterferenceMeasurements in CSI-ReportConfig is set to "Configured", and if the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is configured,

- The most recent occasion of periodic/semi-persistent CSI-IM and/or NZP CSI-RS for interference measurement for CSI derivation is within the symbols with the same symbol type as the configured valid symbol type

Proposal 26. For separate UL power control, support Option 1: One TCI codepoint is mapped to separate TCI states for SBFD symbols and non-SBFD symbols respectively.

- No RRC impact and reuse the existing MAC CE, Enhanced Unified TCI States Activation/Deactivation MAC CE for Joint TCI States

Proposal 27. RAN1 to support separate UL beam (QCL/TCI states) for SBFD symbols.

- One TCI codepoint is mapped to separate TCI states for SBFD symbols and non-SBFD symbols respectively

- No RRC impact and reuse the existing MAC CE, Enhanced Unified TCI States Activation/Deactivation MAC CE for Joint TCI States

Proposal 28. RAN1 to support option 1 for separate SRS configurations for SBFD and non-SBFD symbols

Proposal 29. RAN1 should specify the order of resolving the collision of overlapping channels for a group of more than two overlapping channels.

Proposal 30. For UL transmissions of an UL channel/signal by the SBFD-aware UE in the SBFD UL subband and the UL slot, the following design options are identified:

- Option 1: The UL transmit timing in the SBFD UL subband and in the UL slot is the same.

Note: The SBFD-aware UE then follows existing Rel-15 behavior to determine its UL transmit timing in the SBFD UL subband.

- Option 2: The UL transmit timings in the SBFD UL subband and in the UL slot can be different.

- Option 2a: using a single TA on the serving cell but 2 separately configured NTA,Offset values for the UL subband the UL slot, respectively.

- Option 2b: using 2 separate TAs on the serving cell, one for the UL subband and the other for the UL slot.

Note: The SBFD-aware UE then follows the existing Rel-18 2-TA feature on the serving cell introduced for mTRP.

# Reference

[1] RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD),” Huawei, 3GPP TSG RAN Meeting #104

[2] R4-2418098, “On the potentially new requirements for SBFD operation for FR1 and FR2-1,” Samsung, 3GPP TSG RAN WG1 Meeting #113