3GPP TSG RAN WG1 #119	R1-2409603

Orlando, US, November 18th – November 22th, 2024

Agenda item:	9.5.2

Source:	Samsung

Title:	On-demand SIB1 for idle/inactive mode UEs

Document for:	Discussion and decision

- Introduction

To reduce energy consumption of common signal and channel for UEs in idle/inactive mode, in the WID of Rel-19 network energy saving [1], the following objective has been specified:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3]

  - Specify procedures and signaling method(s) for Case 2 [RAN1/2]

    - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell

    - Triggering method by UL WUS using PRACH

  - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3]

  - Note 1: No modification of SSB will be discussed under this objective

  - Note 2:

    - UL WUS: Uplink wake-up signal

    - Cell A: A cell that is periodically transmitting at least its own SIB1

    - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE

  - Note 3:

    - RAN1 strives to minimize impact to legacy UE

    - RAN1 specification impact to support this feature should be minimized

In this contribution, we discuss details on supporting on-demand SIB1 operation for UEs in idle/inactive mode.

- How to use pdcch-configSIB1 provided by NES cell

In the previous meeting [3], the following proposal has been discussed to define how UE identifies a NES cell with on-demand SIB1.

Agreement For further work of on-demand SIB1 in idle/inactive mode, it is assumed that: - SSB transmitted in the NES cell supporting OD-SIB1 is with K_SSB > 23 for FR1 and K_SSB >11 for FR2 if it is transmitted on sync raster - UE determines a cell supporting OD-SIB1 based at least on UL WUS configuration from Cell A. Agreement For type 0 PDCCH monitoring occasions of on demand SIB1, searchSpaceZero and controlResourceSetZero for on-demand SIB1 are provided from UL WUS configuration if SSB on NES cell is on sync raster and K_SSB is not equal to 30 in FR1 or 14 in FR2.

There is still a remaining issue on UE behavior after first detecting SSB transmitted by NES cell. Even if a UE can determine whether or not the SSB is NCD-SSB from NES cell, the UE can’t know which Cell A provides information of the target NES cell for on-demand SIB1 operation. It would be a burden for the UE to search the Cell A to acquire information for on-demand SIB1, and it would also bring additional latency. To solve the burden and latency to search the Cell A, pdcch-ConfigSIB1 can be used to indicate information of Cell A, like frequency location of SSB transmitted by Cell A.

When K_SSB is set to 24≤K_SSB≤29 for FR1 or with 12≤K_SSB≤13 for FR2, same as legacy UE behavior supporting NCD-SSB, pdcch-ConfigSIB1 can be used to indicate frequency location of the nearest CD-SSB on sync raster within the same band. For example, as illustrated in Figure 1, when UE first detects SSB from NES cell (at GSCN: 8051) and K_SSB is set to 28 with pdcch-ConfigSIB1: ‘01010011’, UE can determine GSCN of CD-SSB for Cell A (i.e. Cell#1) is 7711 using NGSCNReference+NGSCNSize×NGSCNOffset. It would be beneficial in UE complexity and latency to search Cell A. Furthermore, for type 0 PDCCH monitoring occasions of on-demand SIB1, searchSpaceZero and controlResourceSetZero for on-demand SIB1 are provided from UL WUS configuration as agreed in the above agreement.

Proposal 1: For the usage of pdcch-ConfigSIB1, reuse legacy UE behavior in [13, TS38.213] to provide information for CD-SSB if UE detects a first SS/PBCH block with 24≤K_SSB≤29 for FR1 or with 12≤K_SSB≤13 for FR2.

Also, when K_SSB is set to 31 for FR1 or 15 for FR2, legacy UE behavior explained in [13, TS 38.213] will be reused to indicate a GSCN range without CD-SSB of Cell A, i.e. If UE detects a first NCD-SSB with K_SSB = 31, UE determines that there is no CD-SSB within a GSCN range [NGSCNReference-NGSCNStart,NGSCNReference+NGSCNEnd]. Same as above, searchSpaceZero and controlResourceSetZero for on-demand SIB1 are provided from UL WUS configuration.

Proposal 2: For the usage of pdcch-ConfigSIB1, reuse legacy UE behavior in [13, TS38.213] to provide information for CD-SSB if UE detects a first SS/PBCH block with 31 for FR1 or with 15 for FR2.

Last but not the least, when K_SSB is set to 30 for FR1 or 14 for FR2, there isn’t legacy UE behavior using pdcch-ConfigSIB1. For the usage of pdcch-ConfigSIB1, the following options can be taken into account for Rel-19 NES capable UE.

- Option 1: pdcch-ConsigSIB1 is reserved as legacy UE behavior

- Option 2: Use to configure searchSpaceZero and controlResourceSetZero similar as legacy UE behavior when K_SSB is set to K_SSB≤23 for FR1 or with K_SSB≤11 for FR2.

- Option 3: Use to indicate SSB location of Cell A similar as legacy UE behavior when K_SSB is set to 24≤K_SSB≤29 for FR1 or with 12≤K_SSB≤13 for FR2.

There are pros. and cons. for each option. With Option 2, to reduce the payload size of UL WUS configuration on Cell A, pdcch-ConfigSIB1 can be used to indicate type 0 PDCCH monitoring occasion. But, UE burden to search Cell A remains. In terms of Option 3, to solve issue on UE complexity and latency to search Cell A, pdcch-ConfigSIB1 can be used to indicate frequency location of CD-SSB for Cell A with overhead to indicate type 0 PDCCH monitoring occasion in UL WUS configuration.

However, the above legacy UE behavior supports very limited scenario since it is designed for single band operation. Therefore, it has a limitation on configurable frequency range to indicate frequency location of Cell A’s SSB. As illustrated in Figure 1, when UE first detects SSB of NES cell (i.e. Cell#0 at band n78), gNB can’t indicate Cell A (i.e. Cell#2 at band n28) which is out of a band of NES cell. As a results, in a scenario that Cell A has large coverage with low carrier frequency and NES cell provides good data rate with high carrier frequency, legacy UE behavior may not be applicable. Therefore, to support a scenario that Cell A is out of band of NES cell, it is proposed that pdcch-ConfigSIB1 with reserved bits can be used to indicate SSB location of Cell A when K_SSB is set to 30 for FR1 or 14 for FR2 (as Option 3).

Proposal 3: Support to use pdcch-ConfigSIB1 to indicate SSB location of Cell A with extended frequency offset, when K_SSB is set to 30 for FR1 or 14 for FR2.

Figure 1 Illustration of the usage of pdcch-ConfigSIB1 for searching Cell A

- Type0-PDCCH monitoring occasion for on-demand SIB1

In the previous meetings [2], [4], the following agreement has been made to define reference time point for the window starting time.

Agreement At least for Case-2: For further work on type 0 PDCCH monitoring occasions for on demand SIB1, on reference time point to determine the window starting time, RAN1 to down select from the following two options: - Option 1: The reference time point is defined based on the RAR reception time of the UL-WUS transmission - FFS: Definition of RAR reception time - Option 2: The reference time point is defined based on the UL-WUS transmission time - Option 3: The reference time point is defined based on the RAR window of the UL-WUS transmission ### FL Proposal 5-3 The reference time point to determine the window starting time for on-demand SIB1 is the slot UE receives the PDSCH of RAR response for UL WUS. ### FL Proposal 5-3-2 The reference time point to determine the window starting time for on-demand SIB1 is the ending slot of RAR window for UL WUS.

To minimize the UE power consumption in on-demand SIB1 operation, a reference time point in the above agreement has been introduced to determine the starting time of a window for monitoring Type0-PDCCH scheduling on-demand SIB1. In previous meeting, Option 1 and Option 3 has been down selected to further discuss as shown in FL proposal 5-3 and 5-3-2, respectively. Both Option 1 and Option 3 will work. However, with Option 1, it will be more efficient operation compared to Option 3. For example, as illustrated in Figure 2 for Option 3, when each UE#1 (red) and UE#2 (green) transmit WUS#1 at 9th slot of SF#1 and UL WUS#2 at 19th slot of SF#1 respectively, each RAR window would be started at the earliest RAR monitoring occasion, and each different reference time point would be determined based on the ending slot of RAR window when both UEs receive RAR at 3rd slot of SF#2. Even if both UE receive RAR before Type0-PDCCH monitoring occasions (blue), only UE#1 can monitor it but UE#2 can’t monitor it. As a results, when Type0-PDCCH monitoring occasions are located within the RAR window, it can’t be monitored by UE. Otherwise, Type0-PDCCH monitoring occasions should be delayed beyond the RAR window which cause some latency to receive on-demand SIB1.

Figure 2 Example of timeline to determine reference time point and starting time for on-demand SIB1 based on the RAR window of the UL WUS (Option 3).

On the other hand, as illustrated in Figure 3 for Option 1, when each UE#1 (red) and UE#2 (green) transmit WUS#1 at 9th slot of SF#1 and UL WUS#2 at 19th slot of SF#1 respectively, each RAR window would be started at the earliest RAR monitoring occasion. When both UEs receive RAR at 3rd slot of SF#2, reference time point for WUS#1 and WUS#2 would be determined based on the slot for RAR reception of UL WUS, then both UEs can monitor Type0-PDCCH monitoring occasions after time offset from the determined reference time point.

Figure 3 Example of timeline to determine reference time point and starting time for on-demand SIB1 based on the RAR reception time of the UL WUS (Option 1).

As described in Figure 2 and Figure 3, Option 3 can provide a benefit when Type0-PDCCH monitoring occasion is located within RAR window. Hence, if both options can be applicable, more efficient option should be selected and then Option 3 would be desirable to be down-selected for reference time point.

Proposal 4: Support the RAR reception time of the UL WUS transmission as the reference time point for determining Type0-PDCCH monitoring window (Option 1 in RAN1#118 agreement).

In addition, the following agreements were made to determine how to indicate starting time, which is time offset for RAR processing, and duration of the time window for on-demand SIB1.

Agreement At least for Case-2: For further work on type 0 PDCCH monitoring occasions for on demand SIB1, on the starting time and duration of the time window of type 0 PDCCH monitoring occasions, RAN1 to down select from the following two options: - Option 1: starting time and duration are indicated in RAR of the UL-WUS transmission - Option 2: starting time and duration are indicated in the UL WUS configuration Agreement RAN1 to discuss the contents of RAR in response to UL WUS using the legacy RAR for OSI as a starting point FL Proposal 5-1-4 The starting time (relative to the reference time point) and duration of the time window for on-demand SIB1 is indicated in the UL WUS configuration. - Unit of the starting time and duration is in slot FFS: The details of the first SIB1 PDCCH monitoring occasion, e.g., when reference time point plus indicated starting time falls into the middle of SIB1 monitoring occasion for SSB #n and that for SSB #n+1.

To configure starting time and duration, both RAR of UL WUS and UL WUS configuration are discussed in previous meeting. Both options are workable since starting time for RAR processing time and duration of the time window can be the preconfigured value. Additionally, as stated in the above agreement for details of RAR contents, legacy RAR for OSI would be considered for study on contents of RAR for on-demand SIB1 as starting point. Current design of RAR in response to OSI only has a MAC header for acknowledgment of OSI, and has no field for content. In that sense, to reuse legacy RAR design for OSI, the main bullet of FL Proposal 5-1-4 (e.g. Option 2) can be selected, but if Option 1 is supported, then we need to further check with RAN2. Regarding the FFS of FL Proposal 5-1-4, we think a UE will only monitor PDCCH monitoring occasion associated with a SSB beam. Therefore, we don’t think FFS is necessary.

Proposal 5: Support UL WUS configuration to indicate starting time (relative to the reference time point) and duration of the time window for on-demand SIB1 (FL Proposal 5-1-4 and Option 2 in RAN1#118 agreement).

- SSB adaptation based on SIB1 transmission

RAN2 made the following agreement to allow for UE to overhear SIB1 triggered by other UE.

RAN2 agreement Proposal 3 (modified): If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell. => Agreed.

According to the RAN2 agreement, after UE acquires UL WUS configuration for on-demand SIB1 from Cell A, UE will check whether SIB1 is broadcasted or on-demand SIB1 is provided before transmitting UL WUS for triggering on-demand SIB1. In this scenario, the signal/channel that can provide information on whether SIB1 is broadcasted or on-demand SIB1 is provided may only be the SSB on the NES cell. Furthermore, RAN1 made an agreement that SSB transmitted in the NES cell supporting OD-SIB1 is with K_SSB > 23 for FR1 and K_SSB >11 for FR2 if it is transmitted on sync raster. Therefore, when the gNB starts to transmit on-demand SIB1, the gNB can transmit SSB with K_SSB≤23 for FR1 or with K_SSB≤11 for FR2 which is associated with the on-demand SIB1 to allow other UE(s) not transmitting UL WUS to receive on-demand SIB1. Then, the UE can easily check whether SIB1 is currently being broadcasted or not based on value of K_SSB.

Proposal 6: RAN1 can further discuss whether the K_SSB value can be adapted based on the on-demand SIB1 transmission.

- Contents of UL WUS configuration

In RAN1#118bis, the following agreements have been made for the required parameters/contents for UL WUS configuration [3].

Agreement For the purpose of “to which cell does the configuration applies”, at least the following parameters are included in the UL-WUS configuration. Purpose; Parameters. To which cell does the configuration applies; NES-CellId; PhysCellId. ARFCN-ValueNR. Note: ARFCN-ValueNR is used to indicate the absolute radio frequency channel number (ARFCN) for SSB of NES cell. Agreement For the purpose of “WUS transmission”, at least the following parameters are included in the UL-WUS configuration: - rsrp-ThresholdSSB - prach-RootSequenceIndex - msg1-SubcarrierSpacing - restrictedSetConfig Note: In legacy spec, the parameters above are under the IE RACH-ConfigCommon Agreement For the purpose of “WUS transmission”, at least the following parameters to indicate “frequency information for UL-WUS transmission” are included in the UL-WUS configuration. Purpose; Parameters. WUS transmission; frequencyBandList. absoluteFrequencyPointA. offsetToCarrier. p-Max. frequencyShift7p5khz (working assumption)ULSubCarrierSpacing. Agreement For the purpose of “WUS transmission”, at least the following parameters are included in the UL-WUS configuration. Purpose; Parameters. WUS transmission; SIB1-RequestConfig; ss-PBCH-BlockPower. SSB-positionInBurst. tdd-UL-DL-ConfigurationCommon. rach-OccasionsSIB1; Prach-ConfigurationIndex. msg1-FDM. msg1-FrequencyStart. zeroCorrelationZoneConfig. preambleReceivedTargetPower. preambleTransMax. powerRampingStep. ra-ResponseWindow. ssb-perRACH-Occasion. sib1-RequestPeriod. sib1-RequestResources; ra-PreambleStartIndex. ra-AssociationPeriodIndex. ra-ssb-OccasionMaskIndex. Note: - In legacy spec, ss-PBCH-BlockPower/SSB-positionInBurst/tdd-UL-DL-ConfigurationCommon are under the IE ServingCellConfigCommon/ServingCellConfigCommonSIB Msg1-FrequencyStart is with respect to the first RB of the UE carrier determined by offsetToCarrier and absoluteFrequencyPointA

To reduce unnecessary UE behavior to determine accessibility on NES cell, UL WUS configuration can include accessibility information of NES cell, like cell selection information and accessible UE type. By informing the accessibility to UE, it can prevent not only for UE to request and receive on-demand SIB1 on NES cell, which doesn’t support the UE, but also for gNB to save energy consumption for transmission triggered by non-accessible UE.

Furthermore, the following remaining RRC parameters will be needed for supporting on-demand SIB1 operation:

- First, to define CORESET and PDSCH resource for RAR reception, the following RRC parameters will be needed:

  - FrequencyInfoDL

  - genericParameters in initialDownlinkBWP

  - pdsch-ConfigCommon

  - commonControlResourceSet

  - K_SSB

- For initial UL BWP for the configuration of RO for UL WUS, the following additional RRC parameter is still needed with rach-ConfigCommon agreed in previous meeting:

  - genericParameters in initialUplinkBWP

- For remaining RRC parameter related to SSB, the following additional RRC parameter is needed (e.g., for RO validation, the information on the time domain location of SSB is needed):

  - ssb-PeriodicityServingCell

- To transmit UL WUS, the following RRC parameter is needed to acquire timing advance offset:

  - n-TimingAdvanceOffset

Proposal 7: Support the following additional RRC parameters in UL WUS configuration:

- Remaining configurations related to CORESET and PDSCH resource for RAR:

  - FrequencyInfoDL

  - genericParameters in initialDownlinkBWP

  - pdsch-ConfigCommon

  - commonControlResourceSet

  - K_SSB

- Remaining configuration related to initial UL BWP for the configuration of RO:

  - genericParameters in initialUplinkBWP

- Remaining configuration related to SSB :

  - ssb-PeriodicityServingCell

- Remaining configuration related to UL WUS :

  - n-TimingAdvanceOffset

- Configurations related to accessibility, e.g. cell selection information and accessible UE type

Note: details are up to RAN2

- Conclusion

In this contribution, we discussed details on supporting on-demand SIB1 operation for UEs in idle/inactive mode. The following proposals were made:

Proposal 1: For the usage of pdcch-ConfigSIB1, reuse legacy UE behavior in [13, TS38.213] to provide information for CD-SSB if UE detects a first SS/PBCH block with 24≤K_SSB≤29 for FR1 or with 12≤K_SSB≤13 for FR2.

Proposal 2: For the usage of pdcch-ConfigSIB1, reuse legacy UE behavior in [13, TS38.213] to provide information for CD-SSB if UE detects a first SS/PBCH block with 31 for FR1 or with 15 for FR2.

Proposal 3: Support to use pdcch-ConfigSIB1 to indicate SSB location of Cell A with extended frequency offset, when K_SSB is set to 30 for FR1 or 14 for FR2.

Proposal 4: Support the RAR reception time of the UL WUS transmission as the reference time point for determining Type0-PDCCH monitoring window (Option 1 in RAN1#118 agreement).

Proposal 5: Support UL WUS configuration to indicate starting time (relative to the reference time point) and duration of the time window for on-demand SIB1 (FL Proposal 5-1-4 and Option 2 in RAN1#118 agreement).

Proposal 6: RAN1 can further discuss whether the K_SSB value can be adapted based on the on-demand SIB1 transmission.

Proposal 7: Support the following additional RRC parameters in UL WUS configuration:

- Remaining configurations related to CORESET and PDSCH resource for RAR:

  - FrequencyInfoDL

  - genericParameters in initialDownlinkBWP

  - pdsch-ConfigCommon

  - commonControlResourceSet

  - K_SSB

- Remaining configuration related to initial UL BWP for the configuration of RO:

  - genericParameters in initialUplinkBWP

- Remaining configuration related to SSB :

  - ssb-PeriodicityServingCell

- Remaining configuration related to UL WUS :

  - n-TimingAdvanceOffset

- Configurations related to accessibility, e.g. cell selection information and accessible UE type

Note: details are up to RAN2

- References:

- RP-242354, “Revised WID: Enhancements of network energy savings for NR”

- Chairman’s notes, RAN1#118, August 2024.

- Chairman’s notes, RAN1#118bis, October 2024.

- R1-2409016, “FL summary 5 for on-demand SIB1 in idle/inactive mode”, Moderator (MediaTek), RAN1#118bis, October 2024.