3GPP TSG RAN WG1 #119	R1-2409674

Orlando, US, November 18th – 22nd, 2024

Source:	vivo

Title:	Remaining issues on Rel-19 CSI enhancements

Agenda Item:	9.2.2

Document for:	Discussion and Decision

Introduction

The following two revised objectives related to CSI enhancement are listed in MIMO Evolution WID [1].

- Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH

In this contribution, we discuss CSI enhancement for up to 128 CSI-RS ports and UE reporting enhancement for CJT calibration.

Views on CSI enhancement for up to 128 CSI-RS port

In this chapter, we will focus on the remaining topics related to the support of up to 128 CSI-RS ports.

3-bit scaling factor for v=1

Regarding a per-layer scaling factor calculation based on the 3-bit scaling factor si associated with the ith SD basis vector, the following agreement was reached in RAN1 #118bis for selecting one or more alternatives.

Agreement For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding per-layer scaling factor applied to each of the selected SD basis vectors associated with RI=v ∈{2} for the 3-bit scaling factor(s), decide, by RAN1#119, from the following alternatives: - Alt1: min1υ,si2ri - Alt2: min1υ,si2k∈Girk - Alt3: si2 - Alt4: υsi2ri - Alt5: minriυ,si2 - Alt6: si2υ - Alt7: - if two different vectors for υ=2, min1υ,si2 for the vector with smaller scaling factor si≤sj, and min⁡(1-min⁡(1υ,si2),sj2) for the other vector configured with larger scaling factor sj; - otherwise (υ=1 or one same vector for υ=2), min1,si2 for the vector Where ri denotes the number of layers associated with the i-th SD basis vector. The same scheme applies to both Mode-A and Mode-B. Note: si∈1, 12, 13 ,14,16,18,112,116 as agreed in RAN1#117. Normalization of precoder should be taken into account in the final specification design.

Given the soft scaling for RI=2 is a separate UE capability from that for RI=1, we agree with the FL observation in pre-RAN1#119 discussion paper, i.e., it is natural to assume that the value of si2 is independently configured between RI=1 and RI=2, and RI-specific setting of si2 value offers more flexibility to NW and also facilitates a simpler scaling formula for RI=2.

Support rank-specific 3-bit scaling factor configuration.

As for down-selection of Alt1, Alt2, Alt3, Alt4, Alt5 and Alt6, we also believe that they can be merged into AltA, AltB, and AltC, which are listed by FL in pre-RAN1#119 discussion paper and shown below.

- AltA (baseline, same as RI=1): si - AltB: siri - AltC: sik∈Girk

For Alt7, there is a power boosting issue, which violates the agreement reached at the last meeting and can therefore be excluded. For example, when si2=14 and sj2=1, min1-min1υ,si2,sj2=34 that is greater than 1v=12.

Among the 3 merged alternatives, AltA is simpler and easier for PMI acquisition. UE can directly use si2 of each layer for each W1 in the codebook without having to differentiate between multiple W1 matrices, so that the energy per layer is  si2υ. Besides, AltA also produces the same UE behavior as the use of si2 for rank=1.

AltB denotes that the per-layer power scaling factor depends on the value of ri. Therefore, for PMI acquisition, the UE needs to distinguish between W1 matrices with the same SD basis vectors and the W1 matrices with different SD basis vectors in the codebook.

With respect to AltC, when UE obtains the per-layer power scaling factor, UE also needs to distinguish between W1 matrices with same SD basis vector group and W1 matrices with different SD basis vector groups in the codebook, in addition to W1 matrices with same SD basis vectors and the W1 matrices with different SD basis vectors in the codebook. Due to the introduction of more conditions, AltC implies that the complexity of UE is much higher than that of AltA and AltB.

Therefore, from the UE implementation perspective, we support AltA.

Support AltA, i.e., the scaling factor for rank 2 is si assuming si is configured per rank value.

Extension of Rel-19 Type-I SP to <= 32ports

The following proposal was listed in FL summary in the last RAN1 meeting regarding the extension of Rel-19 Type-I SP to <= 32ports.

Proposal 1.E: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, extend the agreed Scheme-A and Scheme-B to the legacy number of CSI-RS ports (i.e. 4, 8, 12, 16, 24, and 32 ports) for all applicable RI values (1, …, min(PCSI-RS,8)) with K=1 only, and without any further modification/enhancement of the sub-features pertinent to the Rel-19 Type-I SP design (including, e.g. the Rel-19 Type-I SP CBSR, soft scaling). - [The following of Rel-19 Type-I SP are four separate UE features: (1) Scheme-A >32port; (2) Scheme-B >32port; (3) Scheme-A ≤32port; (4) Scheme-B ≤32 port.]

In order to understand the performance gain of doing this, we performed some evaluations via SLS. The following figure shows the results of our simulation. In our simulation, rank adaptation with rank=1 or rank=2 is enabled and the baseline is Rel-15 Type-I CB for each antenna port layout. Other simulation parameters are shown in the Appendix.

Cell mean SE comparison for different CB schemes

From the result, at least for rank=1/2, there is virtually no gain for 8T4R/16T4R/32T4R.

At least for rank=1/2, there is no gain for extension of Rel-19 Type-I SP to 8T4R/16T4R/32T4R.

In addition, extension of Rel-19 Type-I SP to <= 32 CSI-RS ports would require the introduction of another CB for <= 32 CSI-RS ports, in addition to the mandatory Rel-15 Type-I CB. We believe that there is no need to introduce another CB on top of Rel-15 Type-I CB without any benefit, and additional codebooks would increase the complexity of UE and also fragment the market.

As a result, we do not support extending the agreed Scheme-A and Scheme-B to the legacy number of CSI-RS ports (i.e. 4, 8, 12, 16, 24, and 32 ports) for all applicable RI values.

Do not support extending the agreed Scheme-A and Scheme-B to the number of CSI-RS ports (i.e. 4, 8, 12, 16, 24, and 32 ports) for all applicable RI values.

CSI reference resource for Rel-19 CSI reporting

In the previous meeting, we agreed on scaling the legacy timeline Z/Z’ by ceil(P/32), which is the Capability 2 timeline for the Rel-19 Type-I SP and Type-II codebook refinements for 48, 64, and 128 CSI-RS ports. The principle of capability 2 is to extend the timeline to handle the processing of the increased number of ports while maintaining the number of occupied CPUs unchanged. Following this principle, the processing time for periodic or semi-persistent CSI reporting also requires extension for capability 2, which is determined by CSI reference resource.

With respect to the CSI reference resource, the following description in TS 38.214[2] implies that the time-domain location of CSI reference resource is determined by nCSI_ref , and the minimum nCSI_ref for periodic CSI reporting, semi-persistent CSI reporting, and aperiodic CSI reporting are 4 milliseconds, 5 milliseconds and Z'/Nsymbslot, respectively.

<omitted text> - In the time domain, the CSI reference resource for a CSI reporting in uplink slot n' is defined by a single downlink slot n-nCSI_ref-Koffset⋅2μDL2μKoffset, where Koffset is a parameter configured by higher layer as specified in clause 4.2 of [6 TS 38.213], and where μKoffsetis the subcarrier spacing configuration for Koffset with a value of 0 for frequency range 1, - where +Nslot,offset,ULCA2μoffset,UL-Nslot,offset,DLCA2μoffset,DL∙2μDL and and are the subcarrier spacing configurations for DL and UL, respectively, and Nslot, offsetCA and are determined by higher-layer configured ca-SlotOffset for the cells transmitting the uplink and downlink, as defined in clause 4.5 of [4, TS 38.211] - where for periodic and semi-persistent CSI reporting - if a single CSI-RS/SSB resource is configured for channel measurement nCSI_ref is the smallest value greater than or equal to 4⋅2µDL, such that it corresponds to a valid downlink slot, or - if multiple CSI-RS/SSB resources are configured for channel measurement nCSI_ref is the smallest value greater than or equal to , such that it corresponds to a valid downlink slot. - where for aperiodic CSI reporting, if the UE is indicated by the DCI to report CSI in the same slot as the CSI request, nCSI_ref is such that the reference resource is in the same valid downlink slot as the corresponding CSI request, otherwise nCSI_ref is the smallest value greater than or equal toZ'/Nsymbslot, such that slot n- nCSI_ref corresponds to a valid downlink slot, where Z' corresponds to the delay requirement as defined in Clause 5.4. - when periodic or semi-persistent CSI-RS/CSI-IM or SSB is used for channel/interference measurements, the UE is not expected to measure channel/interference on the CSI-RS/CSI-IM/SSB whose last OFDM symbol is received up to Z' symbols before transmission time of the first OFDM symbol of the aperiodic CSI reporting. <omitted text>

For nCSI_ref equals to 5ms because of multiple CSI-RS/SSB resources are configured for channel measurement, the following table shows a comparison of  nCSI_ref and Z'2 for a 32-port PMI versus a 128-port PMI. Obviously, when 128-port PMI is calculated, Z'2 may exceed 10ms, which is more than twice the minimum nCSI_ref, due to scaling the legacy timeline Z/Z’ by ceil(P/32).

CSI computation delay requirement

| μ | 32port[symbols] |  | 128 port[symbols] |  |
| --- | --- | --- | --- | --- |
|  | Smallest nCSI_ref | Z'2 | Smallest nCSI_ref | Z'2 |
| 0 | 70 | 37 | 70 | 148 |
| 1 | 140 | 69 | 140 | 276 |
| 2 | 280 | 140 | 280 | 560 |

When 128-port PMI is calculated, Z'2 may exceed 10ms, which is more than twice of the minimum nCSI_ref, due to scaling the legacy timeline Z/Z’ by ceil(P/32).

Therefore, for periodic or semi-persistent CSI reporting, the definition of nCSI_ref also needs to be extended to match the Capability 2 of Z/Z’. One straightforward way to extend nCSI_ref is that if K CSI-RS resources are aggregated for channel measurement, nCSI_ref is the smallest value greater than or equal to ceilP32*4*2μDL, such that it corresponds to a valid downlink slot.

To match the timeline capability 2, support scaling of the nCSI_ref by ceil(P/32) to determine the CSI reference resource.

Determination of the number of PRBs for a PUCCH resource or a number of Part 2 CSI reports.

In Subclause 9.2.5 of TS38.213[3], rank=1 is specified to determine the number of PRB for a PUCCH resource or the number of Part 2 CSI reports. When the payload size of Part 2 CSI reports is beyond the size determined based on the max code rate, a set of Part 2 reports would be omitted until the max code rate requirement is satisfied.

We believe that the above assumption is reasonable for the Rel-15 Type-I codebook that PMI payload size based on rank1 is close to PMI payload size based on other rank values. However, for Rel-19 Type-I CB Scheme-B RI=1-4, the PMI payload size approximately doubles as the number of layers doubles. Therefore, the determination of the number of PRB for a PUCCH resource or a number of Part 2 CSI reports based on rank=1 is not suitable. If the current rank assumption is reused, instead of determining the appropriate number of Part 2 CSI reports, the UE may determine an excessive number of Part 2 CSI reports, resulting in frequent CSI omission according to TS38.214[2] when the actual reported rank is larger than 1.

Agreement For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding UCI parameters for Scheme-B RI=v=1-4: - SD basis vector selection indicator for each layer is in Part 2 (wideband) and log2N1N2 bits per layer l=1, …, v - Inter-pol co-phase selection indicator for each layer is in Part 2 (wideband or subband) and 2 bits (representing {+1, +j, -1, -j}) per layer l=1,…,v

A straightforward approach to address this issue is to change rank = 1 to the maximum allowed rank value, i.e., the maximum value of all allowed rank candidates is specified to determine the number of PRB for a PUCCH resource or a number of Part 2 CSI reports.

The maximum value of all allowed rank candidates is supported to determine the number of PRBs for a PUCCH resource or the number of Part 2 CSI reports.

Antenna port assumption for Rel-19 Type-II Doppler codebooks

- For the Rel-19 Type-II codebook refinement for up to 128 CSI-RS ports based on the Rel-18 Type-II Doppler regular codebook, the following agreement was achieved for aperiodic CMR configuration.

Agreement For the Rel-19 Type-II codebook refinement for 48, 64, and 128 CSI-RS ports based on the Rel-18 Type-II Doppler codebook, support the following aperiodic CMR configuration: - A UE can be configured with KDOPP = {4, 8, 12} CSI-RS resource groups for the purpose of aperiodic CMR as needed by Type-II Doppler CSI - The time separation between the first resources from two consecutive groups (=m) can be configured from {1, 2} - FFS: The need for additional restriction in time domain - Each CSI-RS resource group comprises K NZP CSI-RS resources (K defined in previous agreements) for aggregation associated with a same CSI-RS resource set assuming the agreed resource set rules for Rel-19 Type-I/II codebooks - All the KDOPP CSI-RS resource groups are associated with a same CSI-RS resource set configuration FFS: the determination of CSI-RS resource group that a CSI-RS resource is associated with

In a legacy aperiodic CMR configuration for Rel-18 Type-II Doppler codebook, the UE shall assume that the antenna port with the same port index of the K aperiodic CSI-RS resources is the same. However, KDOPP = {4, 8, 12} CSI-RS resource groups, each comprising multiple CSI-RS resources, are introduced for Type-II Doppler CSI. This means that within a CMR group, there may be multiple CSI-RS ports with the same CSI-RS port index mapped to different antenna ports, e.g., the port 3000s for the different CSI-RS resources in the following figure are associated with the different PMI port indices. Therefore, the antenna port assumption needs to be further clarified when the UE is configured with KDOPP = {4, 8, 12} CSI-RS resource groups.

The corresponding relationship between CSI-RS ports and PMI ports for Mapping method 1

KDOPP = {4, 8, 12} CSI-RS resource groups, each comprising multiple CSI-RS resources, are introduced for Type-II Doppler CSI. This means that within a CMR group, there may be multiple CSI-RS ports with the same CSI-RS port index mapped to different antenna ports.

Considering we have two port mapping methods, a common method for associating CSI-RS ports with the same PMI row index of the K aperiodic CSI-RS resources with the same antenna port can be considered, where each PMI row index corresponds to KDOPP CSI-RS ports.

For antenna port assumption, UE shall assume that the CSI-RS ports associated with the same PMI row index of the K aperiodic CSI-RS resources are associated with the same antenna port, where each PMI row index corresponds to KDOPP CSI-RS ports.

Views on UE reporting enhancement for CJT calibration

This section provides our views on the remaining issues on enhancement for CJT calibration (CJTC).

Joint reporting of PO and DO

For PO reporting, the following two approaches may be popular.

| Approach1(precoded CSI-RS) | Approach2(non-precoded CSI-RS) |
| --- | --- |
| Step1: NW calculates the UL DO values between TRPs, and the channel phase at each TRP (including Tx phase of UE, Rx phase of TRP, raw channel phase) on SRS . Step2: NW sends a precoded single port CSI-RS precoded by the conjugation of the channel phase from each TRP. Step3: UE receives the CSI-RS from all TRPs. UE calculates the DL DO between TRPs, and the phase of channel of each TRP (including Tx-Rx phase difference of TRP, Tx phase of UE, and Rx phase of UE). Note that raw channel phase has been cancelled out. Step4: UE computes the PO of channels between TRPs. Note that Tx and Rx phase of UE are cancelled out by conjugate multiplication. Step5: UE reports the DL DO between TRPs, and the PO between TRPs. Step6: gNB calculates the TX-RX timing misalignment between TRPs by the UL DO between TRPs and the DL DO between TRPs, and compensate the UL channel by the UE reported PO. | Step1: Same as Approach1 Step2: NW sends a non-precoded single port CSI-RS at each TRP. Step3: UE receives the CSI-RS from all TRPs. UE calculates the DL DO between TRPs, and the phase of channel of each TRP (including Tx phase of TRP, phase of raw channel, Rx phase of UE). Step4: UE computes the PO of channels between TRPs. Note that Rx phase of UE are cancelled out by conjugate multiplication. Step5: Same as Approach1 Step6： gNB calculates the TX-RX timing misalignment between TRPs by the UL DO between TRPs and the DL DO between TRPs, gNB calculates the PO of uplink channels between TRPs (Note that Tx phase of UE are cancelled out by conjugate multiplication.), and compensate the UL channel with the difference between the PO of uplink channels and the PO reported by UE (Note that phase of raw channel is cancelled out.). |

From the above description, it can be seen that when there is a timing error on each TRP, the UE needs to report the DL DO between the TRPs and the PO of channels between the TRPs.

For DL DO reporting, it is yet to be decided on how to indicate the DO between TRPs to gNB. The following methods have been considered to address this issue.

Method 1: Indication of DL DO values to the gNB via already supported DO reporting design, i.e. feedback DO values directly.

Method 2: Indication of DL DO values to the gNB via subband PO values, i.e., converting DO values to PO values.

Since Method 2 has been agreed in the previous meeting (copied below), there is no need to introduce multiple designs to support the same functionality. Therefore, there is no need to support joint reporting of PO and DO.

Do not support joint reporting of PO and DO.

Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the support of sub-band reporting (>1): - Denoting the number of reported sub-band phase-offset values as NSB-P, and the sub-bands are indexed as {0, 1, …, NSB-P –1}, support, as a separate UE capability from wideband (=1) phase offset reporting, reporting, in one CSI reporting instance, {(n,, n,, n,NSB-P), n=0, 1, …, NTRP – 1, n≠nref} - The alphabet for n, follows the previously agreed alphabet for =1, including the ‘invalid’ state - FFS: - Supported sub-band size(s), e.g. following the legacy CSI sub-band definition, vs {1, 2, 4, 8, 16 PRBs} - Whether the UE performs measurement over the entire configured CSI reporting band WCSI - If needed, mechanism to limit CSI reporting overhead (e.g. maximum NSB-P) - If needed, configuration of whether the CSI-RSs are MRT-beamformed in frequency-domain or not Note: UE is not required to perform DO/FO compensation for sub-band phase offset calculation Note: There is a source R1-2407184 showing for frequency-domain-MRT-precoded CSI-RSs, inter-TRP phase has a linear rotation in frequency-domain, and is associated with a single delay component. Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the support of sub-band reporting (>1) {(n,, n,, n,NSB-P), n=0, 1, …, NTRP – 1, n≠nref}: - Supported sub-band size(s) {1, 2, 4, 8, 16} PRB - The NW configures, via higher-layer (RRC) signalling, which NSB-P sub-band(s) the UE reports Agreement For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when ReportQuantity is ‘cjtc-P’ (PO) and >1: - The UCI parameters are captured in the tables below: Parameter; Details/description. Nref; Reference CSI-RS resource index, based on the ordering from RRC configuration: logNTRP bits. {(n,, n,, n,NSB-P), n=0, 1, …, NTRP – 1, n≠nref}; DL/UL phase offsets for CSI-RS resource n, (n=0, 1, …, NTRP – 1, n≠nref): NTRP-1NSB-P logMΦbits. - The UCI mapping order is as follows: - nref, - {(n,, n,, n,NSB-P), n=0, 1, …, NTRP – 1, n≠nref} ordered from the lowest to highest CSI-RS resource ID Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, regarding active resource counting and OCPU, when ReportQuantity is ‘cjtc-P’ (phase offset), for both =1 and >1, fully reuse those from Rel-18 TDCP reporting - OCPU =X.NTRP where X≥1 is defined based on UE capabilities and determined by the UE Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the support of sub-band reporting (>1), the maximum value of configured NSB-P is 16 per CSI reporting setting

Identification of SRS/CSI-RS occasions for PO reporting

For precoded CSI-RS for PO reporting, the identified SRS transmission occasion must take place before the CSI-RS transmission occasion since gNB needs to acquire the uplink channel information for downlink precoding. Therefore, the identified SRS transmission occasion should be the most recent SRS occasion before the CSI-RS occasion for periodic or semi-persistent SRS resources.

For non-precoded CSI-RS for PO reporting, there is no restriction on the order of CSI-RS transmission occasion and SRS transmission occasion. For example, as shown in the following figures, SRS transmission can take place either after or before the CSI-RS transmission. Therefore, the identified SRS transmission occasion can be the most recent SRS occasion before or after the CSI-RS occasion for periodic or semi-persistent SRS resource.

For precoded CSI-RS for PO reporting, the identified SRS transmission occasion must take place before the CSI-RS transmission occasion since gNB needs to obtain the uplink channel for downlink precoding.

For non-precoded CSI-RS for PO reporting, there is no restriction on the order of CSI-RS transmission occasion and SRS transmission occasion.

Procedure 1 of obtaining reciprocity misalignment for gNB

Procedure 2 of obtaining reciprocity misalignment for gNB

According to the following agreement achieved at the last meeting, it is agreed that the identified SRS transmission occasion should be the most recent SRS occasion before the CSI-RS occasion for periodic or semi-persistent SRS resource.

Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the configured associated SRS resource, - Confirm the following working assumption as agreement: “the configured associated SRS resource can be either periodic, semi-persistent, or aperiodic” - When periodic or semi-persistent associated SRS resource is configured, the SRS transmission occasion for determining the reference UE antenna port corresponds to the latest SRS transmission occasion before the occasions of the NTRP CSI-RS resources used for measuring ‘cjtc-P’ report - FFS: Whether ‘the earliest SRS transmission occasion after the NTRP CSI-RS occasions’ is also supported as an option - FFS: Whether determination of SRS transmission occasion is needed for aperiodic associated SRS resource, and if so, how

One remaining issue is whether the earliest SRS transmission occasion after the NTRP CSI-RS occasions is also supported. Considering a unified design of PO reporting with precoded CSI-RS or non-precoded CSI-RS, we do not support the identified SRS transmission occasion as the earliest SRS transmission occasion after the NTRP CSI-RS occasions for periodic or semi-persistent SRS resource.

Do not support the identified SRS transmission occasion as the earliest SRS transmission occasion after the NTRP CSI-RS occasions for periodic or semi-persistent SRS resource.

Another remaining issue is whether determination of SRS transmission occasion is needed for aperiodic associated SRS resource. In our view, for aperiodic SRS resource, the SRS transmission occasion can be triggered by NW either after or before the CSI-RS transmission occasion, depending on whether CSI-RS resources are precoded or not. Therefore, there is no ambiguity on AP SRS transmission occasion identification.

There is no ambiguity on AP SRS transmission occasion identification. The SRS transmission occasion can be triggered either after or before the CSI-RS transmission occasion, depending on whether CSI-RS resources are precoded or not.

Determination of SRS transmission occasion is not needed for aperiodic associated SRS resource

Furthermore, since determination of the reference UE antenna port used to obtain the PO values corresponds to the latest SRS transmission occasion before the occasions of the NTRP CSI-RS resources, and the time domain behavior of CSI-RS resources can be either periodic or semi-persistent in addition to aperiodic CSI-RS. And, PO measurement also needs alignment of CSI-RS transmission occasions between UE and NW. Otherwise, it may be possible that there is no SRS transmission occasion before the NTRP CSI-RS occasions, or the reference UE antenna port used for PO calculation is wrong.

For example, in Figure 5, when P CSI-RS or SP CSI-RS is configured, if UE determines the PO by using of CSI-RS occasion 1 instead of CSI-RS occasion 2, there is no associated SRS occasion hence precoding of the CSI-RS at occasion 1 is ambiguous. Therefore, the PO reporting may be dropped even if there is one SRS occasion before the CSI-RS occasion 2. Whereas in Figure 6, if NW determines the uplink PO by using of SRS occasion 2, NW expects UE to obtain downlink PO by using of CSI-RS occasion 2 and the reference UE antenna port associated with SRS occasion 2. However, UE may use CSI-RS occasion 1 and the reference UE antenna port associated with SRS occasion 1 to determine the PO, which may result into some incorrect PO values.

Illustration of misaligned CSI-RS transmission occasions between UE and NW

Illustration of misaligned CSI-RS transmission occasions between UE and NW

To address this ambiguity, when P CSI-RS or SP CSI-RS is configured, UE shall derive the PO only based on the most recent occasion of CSI-RS resources, no later than the CSI reference resource associated with the PO reporting, which is similar to the CSI reporting that higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is restricted to be set as "Configured".

When P CSI-RS or SP CSI-RS is configured, UE shall derive the PO only based on the most recent occasion of CSI-RS resources, no later than the CSI reference resource associated with the PO reporting.

To summarize, the table below shows the identification of SRS transmission occasion and CSI-RS transmission occasion for a total of 9 combinations of SRS and CSI-RS.

The identification of SRS transmission occasion and CSI-RS transmission occasion

|  | AP CSI-RS | SP CSI-RS | P CSI-RS |
| --- | --- | --- | --- |
| AP SRS | No SRS transmission occasion identification required, and no CSI-RS transmission occasion identification required | No SRS transmission occasion identification required, and, the latest CSI-RS occasion before the CSI reference resource | No SRS transmission occasion identification required, and, the latest CSI-RS occasion before the CSI reference resource |
| SP SRS | The latest SRS occasion before the CSI-RS occasion, and no CSI-RS transmission occasion identification required | The latest SRS occasion before the CSI-RS occasion, and, the latest CSI-RS occasion before the CSI reference resource | The latest SRS occasion before the CSI-RS occasion, and, the latest CSI-RS occasion before the CSI reference resource |
| P SRS | The latest SRS occasion before the CSI-RS occasion, and no CSI-RS transmission occasion identification required | The latest SRS occasion before the CSI-RS occasion, and, the latest CSI-RS occasion before the CSI reference resource | The latest SRS occasion before the CSI-RS occasion, and, the latest CSI-RS occasion before the CSI reference resource |

Finally, since the channel is time-varying, it is necessary to control the transmission interval between the SRS and the CSI-RS in order to counteract the phase of the UL channel and the phase of the DL channel for each TRP as much as possible. This is also to avoid UE searching a too-long window to identify the SRS occasion, where the window may exceed UE coherence time or channel coherence time. This will reduce the performance of this PO reporting and compensation.

Restrict the time gap between the received CSI-RS and the transmitted associated SRS within a certain time range.

Linkage between DO reporting and Rel-18 CJT CSI reporting

In previous meetings, we have agreed that gNB will perform pre-compensation on PDSCH to align timing among TRPs. The pre-compensation of DO needs to work together with PDSCH precoding based on the Rel-18 CJT CSI report to ensure the PDSCH reception performance. Thus, the CJT CSI report somehow needs to be linked with the CJTC DO report. Regarding linkage between DO reporting and Rel-18 CJT CSI reporting, we have a consensus on associating the CJT CSI report with the CJTC DO report via RRC signaling, and separately or jointly triggering the two linked reports.

When linking between CJTC DO report and Rel-18 eType-II CJT CSI report is configured with two separate triggers, the following agreement was made to reduce the CSI Reporting Setting budget by introducing a 1-bit indicator per CSI trigger state to indicate whether or not the UE should perform DO compensation. If the 1-bit indicator is not configured, UE performs CJT CSI acquisition without DO compensation. Otherwise, as shown in Figure 7, UE compensates the UE-specific DO values using the previously UE-reported delay offset, and performs PMI calculation for the Rel-18 eType-II CJT CSI report after pre-compensation.

Agreement For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with two separate triggers, to indicate whether or not the UE should perform delay offset (DO) compensation based on the latest linked CJTC Dd report when calculating the Rel-18 Type-II CJT CSI, introduce a 1-bit indicator per CSI trigger state: - The 1-bit indicator is an RRC parameter - This is done without increasing the bit-width of the CSI request field of the DCI triggering a Rel-18 CJT eType-II CJT CSI report - FFS (RAN1#119): Whether the 1-bit indicator applies to all the NTRP CSI-RS resources, or to 1-bit indicator per CSI-RS resource - FFS (RAN1#119): How this applies to a single CSI trigger state associated with >1 CSI reports

CJT CSI report associated with CSI-RS resources without timing compensation

One remaining issue for this approach is whether 1-bit indicator is resource-specific or resource-common. In our view, the use case for UE to compensate DOs for a subset of TRPs is unclear, and resource-specific 1-bit indicators introduce a large number of configuration combinations that also increase the trigger state budget. Therefore, we support resource-common 1-bit indicator and do not support resource-specific 1-bit indicators.

Support resource-common 1-bit indicator only.

Another remaining issue is how to apply the 1-bit indicator when a single CSI trigger state is associated with >1 CSI reports.  In our view, it is better to restrict the number of CSI reports to 1 when the 1-bit indicator is configured for the trigger state. If the asymmetric number of TRPs for CJT CSI report and DO report is supported, a straightforward way is to apply the 1-bit indicator to all linked CJT CSI reports.

Regarding how the 1-bit indicator applies to a single CSI trigger state associated with NR>1 CSI reports,

1st preference is the Alt 1 from the FL observation, i.e., when the 1-bit indicator is used, restrict NR to 1.

2nd preference is to apply the 1-bit indicator to all linked CJT CSI reports, if the asymmetric number of TRPs for CJT CSI report and DO report is supported.

When linking between CJTC DO report and Rel-18 eType-II CJT CSI report is configured with a joint trigger, according to the following agreement, one remaining issue is whether to relax the timeline for the Rel-18 CJT report. Given that the serial calculation may be needed for a joint trigger, hence we support relaxing the timeline.

Agreement For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with a joint trigger: - Reuse the CPU occupation and active resource counting for the Rel-18 eType-II CJT - FFS (RAN1#119): Re timeline, decide whether to reuse or further relax the timeline for the Rel-18 eType-II CJT

Given that a normal timeline Z2,Z2' and a relaxed timeline (Z2+Z2',2Z2') are supported for Rel-18 CJT eTypeII CB, and the timeline of Rel-18 TDCP reporting which is reused for CJT calibration reporting is Z2,Z2', we support reusing the relaxed timeline of legacy Rel-18 CJT eTypeII CB for the linked Rel-18 eType-II CJT, i.e., (Z2+Z2',2Z2').

Support timeline relaxation for joint triggering and reusing the relaxed timeline of legacy Rel-18 CJT eTypeII CB for linked Rel-18 CJT eTypeII CB, i.e., (Z2+Z2',2Z2').

The slot offsets of NTRP CSI-RS resources for PO reporting

The following proposal was listed in FL summary of the last RAN1 meeting, regarding the slot offsets of NTRP CSI-RS resources for PO reporting.

Proposal 3.F: For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), the slot offsets of NTRP P/SP CSI-RS resources are configured within X={1,[2]} slots, without DL/UL switching in between the NTRP resources, where X=1 implies that the NTRP resources are configured within a same slot, and X=2 implies that the NTRP resources are configured within two adjacent slots.

Since both  Approach1 (precoded CSI-RS) and Approach2 (non-precoded CSI-RS) described above should eliminate the Rx phase of UE, the slot offsets of NTRP CSI-RS resources should be as small as possible from the perspective of phase coherence. Considering 1-port CSI-RS resources only, we support restricting the P/SP CSI-RS resources in a same CSI-RS set are configured within same slot. In addition, for AP CSI-RS current specification already restricts that the resources should be all in a slot. Thus, no further enhancement is needed for AP CSI-RS.

Support restricting the P/SP CSI-RS resources in a same CSI-RS set are configured within same slot.

Clarifications on CP and ppm

In RAN1#116bis and RAN1#117, the following agreements were reached on the values of AD and AFO.

Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, the dynamic range and resolution parameters for delay offset reporting Dn,offset, i.e. (AD, MD), are NW-configured via higher-layer (RRC) signalling from the following candidate values: - AD ={0.5CP, 0.75CP, CP, 1.5CP, 2CP, 14∆f, 112∆f, 124∆f} where CP and ∆f denote the length of the cyclic prefix according to the current specifications (for normal CP) within a slot and the SCS, respectively - FFS: Further down-selection of the above candidate values for AD, including the use of a same unit for all supported values - MD ={32, 64} - FFS: If TDD TX/RX timing misalignment report is supported, whether different set of candidate MD values is needed In addition, the inside/outside range for the 1-bit indicator dn is equal to [0, CP]. FFS: Further implicit/explicit restriction(s) on candidate value(s) depending on the CSI-RS configuration Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the dynamic range for delay offset reporting Dn,offset, i.e. AD, at least support the following values: {0.5CP, CP} - Decide, by RAN1#117, whether any of the following candidate values are supported: {0.75CP, 1.5CP, 14∆f, 112∆f,124∆f} Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, the dynamic range and resolution parameters for frequency offset reporting FOn, i.e. (AFO, MFO), are NW-configured via higher-layer (RRC) signalling from the following candidate values: - AFO = {0.01ppm, 0.1ppm, 0.2ppm, f, f/2, f/4,f/8, 1/(4t), 1/(8t), 1/(16t), 1/(32t), 1/(512t)} where f and t denote the SCS and duration of one OFDM symbol, respectively - FFS: Further down-selection of the above candidate values for AFO, including the use of a same unit for all supported values - MFO = {16,32} FFS: Whether additional restriction(s) based on CSI-RS configuration is supported, including implicit configuration of quantization range Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the dynamic range for frequency offset reporting FOn, i.e. AFO, at least support the following values: {0.1ppm, 0.2ppm} - Decide, by RAN1#117, whether any of the following candidate values are supported: {0.025ppm, 0.05ppm, 1/(8t), 1/(16t), 1/(32t)}

Regarding AD, one clarification that may be needed is that the subcarrier spacing configuration of CSI-RS is used to determine the length of CP.

The subcarrier spacing configuration of CSI-RS is used to determine the length of CP for AD.

Regarding AFO, the reference frequency in relation to ppm should be clarified. A straightforward method is to define the reference frequency in relation to ppm is the frequency of Point A.

Support defining the reference frequency in relation to ppm is the frequency of Point A.

Linkage between FO reporting and CJT CSI reporting

Similar to timing offset reporting, different UE may report different frequency offsets between TRPs due to UE-specific Doppler shifts for each TRP. Therefore, the gNB is unable to adjust the oscillator for each UE. It can only compensate for the phase offsets between TRPs caused by the FO at different times.

Figure 8 shows the effect of the frequency offset of each TRP on the channel between two TRPs as an example.

The effect of frequency offset of each TRP on channel

Based on the above channel variations, the FO compensation procedure can take the following steps:

Step1: UE reports the FO f1-f2 based on a configured FO reporting setting.

Step2: UE estimates the channel to obtain h1nand h2n based on CSI-RS1 and CSI-RS2.

Step3: UE adjusts the phases of h1n and h2n based on FO of each TRP to obtain h3n=h1nej2πf1t1, h4n=h2nej2πf2t3, where f1 and f2 are frequency offset for CSI-RS1 and CSI-RS2 respectively.

Step4: UE reports CJT PMI based on FO-compensated channel [h3n, h4n].

Step5: TRP2 pre-compensates for the phase offset with ej2π(f1-f2)t2 and uses the PMI reported by UE.

Step6: UE estimates the channel h5n based on DMRS which corresponds to t2 starting from CSI reference resource, and

h5n=h1n2ej2πf1t2+h2n2ej2πf2t2ej2π(f1-f2)t2=(h1n2+h2n2)ej2πf1t2

Step7: UE receives the PDSCH based on h5n and f1. Note that the subsequent phase offset caused by (f1-f2) is compensated by TRP2.

Since the FO values are used for both UE and gNB, the FO values between TRPs need to be aligned. The FO report and the CJT CSI report can be linked to each other using a similar linking method to the DO report, i.e., configuring the CSI report ID associated with the FO report in the CSI report setting of the CJT CSI.

Linkage between the FO report and the CJT CSI report is supported so that the FO values for both UE and gNB compensation are aligned.

In legacy PMI acquisition, the PMI is determined based on the reference resource, which may mean that for UE FO compensation, the UE can adjust the phase caused by the FO of each CSI-RS, based on the time duration from the CSI-RS occasion to the reference resource. Since the reference resource is a slot, it depends on the UE implementation whether the FO compensated in PMI is calculated based on the start symbol of the reference resource, or the end symbol of the reference resource.

Therefore, it may happen that the UE compensates the phase offset of each TRP up to the first symbol of reference resource, while the gNB compensates the phase offsets between the TRPs starting from the last symbol of reference resource, resulting in a phase compensation mismatch of up to one-slot phase rotation between TRPs.

Assuming the gNB is unable to adjust the oscillator and +/-0.1 ppm frequency error per TRP, the following table shows the maximum frequency errors for different carriers.

Maximum frequency errors for different carrier frequencies

| Carrier(GHz) | Max frequency error(Hz) |
| --- | --- |
| 1 | 200 |
| 2 | 400 |
| 4 | 800 |
| 6 | 1200 |
| 7 | 1400 |

Based on the maximum frequency errors in the above table, the following table gives the phase rotation values for a slot length with different maximum frequency errors.

The phase rotation values for a slot length with different maximum frequency errors.

| Max FE(Hz) slot length(ms) | 200 | 400 | 800 | 1200 | 1400 |
| --- | --- | --- | --- | --- | --- |
| 1 | 0.4π | 0.8π | 1.6π | 2.4π | 2.8π |
| 0.5 | 0.2π | 0.4π | 0.8π | 1.2π | 1.4π |
| 0.25 | 0.1π | 0.2π | 0.4π | 0.6π | 0.7π |

From our simulation results in [4], it can be seen that at a carrier frequency of 2.2GHz, a maximum frequency error of 0.01ppm, and CSI period of 5ms, i.e., phase error ∆φ=2*π*2.2*109*0.01ppm*5*10-3=0.22π, will not cause significant performance loss. Therefore, if the gNB is unable to adjust the oscillator, the FO compensated in CJT PMI should not be based on the reference resource slot but on a given symbol of the reference resource slot.

For FO compensation, define a reference symbol in CSI reference resource as the reference time to align UE compensation of FO during CJT CSI calculation and gNB pre-compensation of FO for PDSCH, i.e., the UE compensates the phase offset of each TRP up to the given symbol of reference resource, and the gNB compensates the phase offsets between the TRPs starting from the reference symbol in CSI reference resource.

Conclusions

To summarize, we have the following proposals and observations.

Views on CSI enhancement for up to 128 CSI-RS port

Proposal 1:	Support rank-specific 3-bit scaling factor configuration.

Proposal 2: Support AltA, i.e., the scaling factor for rank 2 is si assuming si is configured per rank value.

Proposal 3: Do not support extending the agreed Scheme-A and Scheme-B to the number of CSI-RS ports (i.e. 4, 8, 12, 16, 24, and 32 ports) for all applicable RI values.

Proposal 4: To match the timeline capability 2, support scaling of the nCSI_ref by ceil(P/32) to determine the CSI reference resource.

Proposal 5: The maximum value of all allowed rank candidates is supported to determine the number of PRBs for a PUCCH resource or the number of Part 2 CSI reports.

Proposal 6: For antenna port assumption, UE shall assume that the CSI-RS ports associated with the same PMI row index of the K aperiodic CSI-RS resources are associated with the same antenna port, where each PMI row index corresponds to KDOPP CSI-RS ports.

Observation 1: At least for rank=1/2, there is no gain for extension of Rel-19 Type-I SP to 8T4R/16T4R/32T4R.

Observation 2: When 128-port PMI is calculated, Z'2 may exceed 10ms, which is more than twice of the minimum nCSI_ref, due to scaling the legacy timeline Z/Z’ by ceil(P/32).

Observation 3: KDOPP = {4, 8, 12} CSI-RS resource groups, each comprising multiple CSI-RS resources, are introduced for Type-II Doppler CSI. This means that within a CMR group, there may be multiple CSI-RS ports with the same CSI-RS port index mapped to different antenna ports.

Views on UE reporting enhancement for CJT calibration

Proposal 7: Do not support joint reporting of PO and DO.

Proposal 8: Do not support the identified SRS transmission occasion as the earliest SRS transmission occasion after the NTRP CSI-RS occasions for periodic or semi-persistent SRS resource.

Proposal 9: Determination of SRS transmission occasion is not needed for aperiodic associated SRS resource

Proposal 10: When P CSI-RS or SP CSI-RS is configured, UE shall derive the PO only based on the most recent occasion of CSI-RS resources, no later than the CSI reference resource associated with the PO reporting.

Proposal 11: Restrict the time gap between the received CSI-RS and the transmitted associated SRS within a certain time range.

Proposal 12: Support resource-common 1-bit indicator only.

Proposal 13:	Regarding how the 1-bit indicator applies to a single CSI trigger state associated with NR>1 CSI reports,

1st preference is the Alt 1 from the FL observation, i.e., when the 1-bit indicator is used, restrict NR to 1.

2nd preference is to apply the 1-bit indicator to all linked CJT CSI reports, if the asymmetric number of TRPs for CJT CSI report and DO report is supported.

Proposal 14: Support timeline relaxation for joint triggering and reusing the relaxed timeline of legacy Rel-18 CJT eTypeII CB for linked Rel-18 CJT eTypeII CB, i.e., (Z2+Z2',2Z2').

Proposal 15: Support restricting the P/SP CSI-RS resources in a same CSI-RS set are configured within same slot.

Proposal 16: The subcarrier spacing configuration of CSI-RS is used to determine the length of CP for AD.

Proposal 17: Support defining the reference frequency in relation to ppm is the frequency of Point A.

Proposal 18: Linkage between the FO report and the CJT CSI report is supported so that the FO values for both UE and gNB compensation are aligned.

Proposal 19: For FO compensation, define a reference symbol in CSI reference resource as the reference time to align UE compensation of FO during CJT CSI calculation and gNB pre-compensation of FO for PDSCH, i.e., the UE compensates the phase offset of each TRP up to the given symbol of reference resource, and the gNB compensates the phase offsets between the TRPs starting from the reference symbol in CSI reference resource.

Observation 4: For precoded CSI-RS for PO reporting, the identified SRS transmission occasion must take place before the CSI-RS transmission occasion since gNB needs to obtain the uplink channel for downlink precoding.

Observation 5: For non-precoded CSI-RS for PO reporting, there is no restriction on the order of CSI-RS transmission occasion and SRS transmission occasion.

Observation 6: There is no ambiguity on AP SRS transmission occasion identification. The SRS transmission occasion can be triggered either after or before the CSI-RS transmission occasion, depending on whether CSI-RS resources are precoded or not.

Appendix

Evaluation parameters for chapter 2.2

The following table shows the possible evaluation parameters of SLS.

Evaluation parameters for SLS

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (macro only) with 200m ISD |
| Frequency Range |  | 2GHz |
| Channel generation model |  | TR38901. No spatial consistency |
| Antenna setup and port layouts at gNB |  | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ 8 ports: (4,4,2,1,1,1,4), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4Rx: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| BS Tx power |  | 41dBm |
| BS antenna height |  | Depending on scenarios (cf. table A.2.1-1 of TS 38.802) |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs |  | 52 |
| Simulation bandwidth |  | 10 MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU-MIMO with rank adaptation is a baseline Rank candidates: {1,2} |
| CSI feedback |  | CSI periodicity: 5 ms CSI delay: 4ms |
| Traffic model |  | Full buffer, 10 UEs per cell |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |

References

RP-242394, New WID: NR MIMO Phase 5, RAN#105.

3GPP TS 38.214 v17.4.0: “Physical layer procedures for data(Release 17)”.

3GPP TS 38.213 v17.4.0: “Physical layer procedures for control(Release 17)”.

R1-2404171, Discussion on Rel-19 CSI enhancement, RAN1#117.