3GPP TSG RAN WG1 #119                                  			               R1-2409720

Orlando, US, November 18th – 22nd, 2024

Agenda Item:	9.11.2

Source:	InterDigital, Inc.

Title:		         Discussion on half-duplex RedCap issues for NTN FR1 operation

Document for:	Discussion

# Introduction

In RAN #102 meeting, the WI for R19 NR NTN for Phase 3 [1] is approved and revised in RAN # 104 meeting as [2]. To support RedCap devices for NTN operation, this WID has an objective to support RedCap devices with NR NTN. The justification for this objective is as in the following:

- Address RedCap UE within FR1 NTN The support of RedCap devices (e.g. handheld and IoT) operating in FR1 band NR-NTN networks can offer enhanced service capabilities (wideband/broadband) compared to IoT-NTN while ensuring low-complexity devices. Global coverage would clearly benefit RedCap devices. RF and RRM requirements were defined for RedCap devices only for terrestrial networks in releases 17 and 18.

This justification is linked to the following objective:

- Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following HD collisions cases (cases 3 and 4): - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded. - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution. - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

Following agreements were made in RAN1 #116 [3] meeting:

Agreement Study at least the following scenarios for (e)RedCap HD-FDD UEs for NTN: - Whether existing handling rules for the following cases should be reused or updated when taking into account TA mismatch between actual TA used by UE and assumed TA at the gNB based on available TA report: - Case 1: Dynamically scheduled DL reception collides with semi-statically configured UL transmission - Case 2: Semi-statically configured DL reception collides with dynamically scheduled UL transmission - Case 3: Semi-statically configured DL reception collides with semi-statically configured UL transmission - Case 4: Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Case 5: Configured SSB collides with dynamically scheduled or configured UL transmission - Case 6: Dynamic or semi-static DL collides with valid RO - Case 7: Collision due to direction switching - At least the following potential issues can be further considered for (e)RedCap HD-FDD UEs - Error cases in case 3 and case 4 - SIB19 reception collides with UL transmission - Slot counting for UL repetition transmission colliding with SSB reception - Invalid symbol determination for PUSCH repetition type B - Actual TDW determination due to the collision between DL reception and UL transmission with DMRS bundling - CPU occupation due to omitted DL reception or UL transmission Note: Both GSO and Non-GSO should be considered.

Following conclusion was reached in RAN1 #118Bis [5] meeting:

Conclusion The different use cases (e.g. the collision happening in different channel types or different scheduling cases) from RAN1#118 agreement for collision case 3 (Semi-statically configured DL reception collides with semi-statically configured UL transmission) consist of pairs of one semi-statically configured DL reception and one semi-statically configured UL transmission, among: - Semi-statically configured DL reception includes Type-0/0A/0B/1/2-PDCCH CSS and dedicated semi-statically configured PDCCH USS/PDSCH SPS/CSI-RS/PRS/[Type 3 PDCCH CSS] - Semi-statically configured UL transmission includes semi-statically configured PUSCH/PUCCH/SRS

This document discusses some aspects of HD-FDD RedCap collisions in the context of NTN operation.

# HD-FDD RedCap Operation in NTN

RedCap operation support is fundamental to Rel-19 NR NTN work. This will add the support of additional IoT use cases through NTN networks beyond what is supported by IoT NTN, where NB-IoT and eMTC based devices operate in IoT NTN networks.

Among RedCap devices, HD-FDD devices are of high importance being the low cost version of RedCap devices. These devices are not equipped with a duplexer, and thus they cannot transmit and receive simultaneously. This leads to HD operation for these devices even when they operate in paired spectrum, so called FDD operation. Thus, despite being operating in FDD mode/system, these devices operate in a time division transmission/reception mode, which results in some collisions/overlaps of different UL and DL transmissions from system point of view.

RAN #104 has updated the objective on HD-FDD collision handling to address at least the collision cases for case 3 and case 4.

## Case 3 Collisions

Due to UE determined TA and UE based TA compensation, the network does not know the precise timing at the UE. Even with UE reporting its TA, there can still be considerable uncertainty with the TA value know at the gNB. A direct consequence of network not able to determine UL and DL timing at the UE is the network inability to provide semi-static DL and UL resources not colliding in time, unless very large TA variation margin is assumed at the network which will result in significant scheduling restriction.

Observation 1: The network cannot always determine non-colliding semi-static UL and semi-static DL configurations for a HD-FDD UE due to TA drift.

RAN1 #118 made an agreement for case 3 where different options are formulated to specify the priority rules for case 3.

Agreement

The collision case 3 (Semi-statically configured DL reception collides with semi-statically configured UL transmission) can’t be assumed as error case. When the collision happens at UE side for collision case 3, one of the following options on priority rules is supported.

- Option A: the priority of keeping one direction and overriding another direction is based on different use cases (e.g. the collision happening in different channel types or different scheduling cases) without network signalling

  - It is not precluded that some or all use cases are left to UE implementation regarding whether DL reception or UL transmission is dropped

  - Note: it is not precluded that the same direction is prioritized for all different use cases

- Option B: network indicates the priority configuration to the UE

  - It is not precluded that some use cases are left to UE implementation regarding whether DL reception or UL transmission is dropped

  - It is not precluded to define default rules, if necessary

RAN1 needs to consider different use cases for case 3 collisions where a variety of semi-static DL transmissions may potentially collide with a variety of semi-static UL transmissions.

One important use case can be the semi-static PUSCH for voice which may have potential collisions with semi-static DL, e.g., PDCCH reception scheduling SIBs/SIB19 etc. PDCCH scheduling SIB19 is through Type 0A PDCCH, so this is the collision between Type 0A PDCCH and semi-static PUSCH. If the collision priority is specified to prioritize DL, the UE may not transmit CG PUSCH colliding with all configured occasions for Type 0A PDCCH  (for SIB19 and other SIBs if configured) which may be an issue for UEs in a voice call - which may require 8/16 repetitions every 20 ms. If the collision priority is specified to prioritize UL, the UE may transmit CG PUSCH voice data successfully but will deprioritize reception of Type 0A PDCCH. This may eventually lead to SIBs expiry. This may particularly be an issue for SIB19 as expiry of ephemeris information results in loss of UL synchronization.

Proposal 1: Support to specify the priorities for case 3 based upon the type of channels undergoing collisions.

One simple option to define the collision priority could be to specify a static or fixed priority, e.g., to prioritize UL or DL transmission when such collisions occur at the UE. Nevertheless, this option can only work if exception cases are clearly defined.

Another option can be to define a configurable priority where the network can indicate whether to prioritize DL or UL as part of the transmission/reception configuration. For case 3 (semi-static DL vs semi-static UL), the network can provide the priority indication as part of the semi-static resource configuration.

In some cases, the priority rules may need to be defined based upon the network decisions for system operation. For example, in some cases, the network may provide/reserve certain types of resources (e.g., semi-static RO or dynamic scheduled resources) which may collide, and the network expects a specific UE behavior for such collisions. For these reasons and to keep a simple behavior, the default priority rules may be specified for different use case (traffic types), and the network may (re-)configure the priorities if need be.

Proposal 2: Support network configurable priorities for case 3 collisions.

## Case 4 Collisions

Similar to the discussion in the previous section for case 3 collisions, the network does not know the precise timing at an NTN UE due to UE based TA compensation. Further, due to TA drift despite the TA reporting, the network may have outdated TA information. Thus, the network cannot ensure non-colliding dynamic UL transmission and dynamic DL transmission for a HD-FDD RedCap UE without imposing significant scheduling restrictions.

Observation 2: The network cannot ensure scheduling non-colliding dynamic UL vs dynamic DL transmissions for a HD-FDD UE due to TA drift unless large gap is introduced by the network between such transmissions.

The large gap and the scheduling restrictions to avoid such collisions will result in less resource available for a scheduled HD-FDD UE to avoid error cases 3 and 4, as has been captured in the observations made during RAN1 #116Bis.

RAN1 #116Bis Observation

To avoid the occurrence of error cases 3 and 4 through network scheduling, there are less resources available for a scheduled HD-FDD RedCap/eRedCap UE in NTN compared to TN when there is TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB.

RAN1 #118 made an agreement for case 3 where different options are formulated to specify the priority rules for case 3. To avoid the restriction for the scheduler and to avoid performance degradation at the UEs, the UEs need to know the priority rules for case 4 as well.

Proposal 3: Support to define the priority rules for case 4.

The UE behavior for case 4 collisions can be specified similar to case 3 collision scenarios. As part of the investigation, RAN1 needs to carefully examine if a single priority rule can be workable for all different types of transmissions or different rules may be needed. Based upon the type of the underlying UL or DL transmissions, in some cases UL may have higher priority while in others DL may have higher priority.

Proposal 4: Support to specify the priorities for case 4 based upon the type of signals/channels undergoing collisions.

A simple solution can be to define the collision priority as static or fixed priority, e.g., to prioritize UL or DL transmission when such collisions occur at the UE. Nevertheless, at least the collisions involving SIB19 transmissions and PDCCH ordered PRACH transmission need to be discussed and potentially treated under exceptions for a prioritized direction.

## TA Reporting Enhancements

Defining the priorities for the collisions makes the UE behavior clear but it does not help avoid performance degradation or future collisions from happening. In one category of techniques to combat performance degradation and avoid future collisions, the UEs provide information to the network in a more dynamic manner and with more refined granularities so that based upon this information, the network can avoid the resource collisions at HD-FDD devices operating in NTN. Based upon the enhanced reporting of TA, TA drift etc., or UE indication upon detecting HD-FDD collisions, the network may (re-)configure semi-static scheduling in a manner to avoid HD-FDD collisions. Similarly, based upon the enhanced reporting from the UE(s), the network may be able to avoid dynamic collisions occurring at the UE(s).

RAN1 #118 Agreement

To mitigate the collisions of case3 and case4, the following TA reporting enhancements for Rel-19 NTN HD-FDD (e)Redcap UE can be further studied:

- Finer TA report granularity

- Smaller TA offset threshold

- Trigger a TA report when the collision is detected at the UE

- TA reporting with a new triggering mechanism from gNB

- TA drifting rate reporting

Note: The benefit, complexity, power consumption and signaling overhead of each scheme is to be further investigated.

Note: other solutions can also be studied

One solution to improve TA reporting is to introduce smaller TA offset threshold which will result in more frequent TA reporting. The frequent TA reporting can be easily implemented by providing a smaller TA offset threshold. The drawback would be that the UEs configured with smaller TA offset threshold would be reporting TA very frequently resulting in large UL overhead, which may lead to poor system efficiency.

Observation 3: Frequent TA reporting, for example by using a smaller TA offset threshold, from several UEs will result in large UL overhead and may lead to poor system efficiency.

Another solution to improve TA reporting is to introduce finer TA granularity. Currently TA reporting is based upon 1 ms granularity. A sub-ms or symbol level granularity may be advantageous in some cases but does not provide any help against TA drift and will still result in collisions at the UE despite the increased UL overhead.

Observation 4: Finer TA granularity provides little help against TA drift despite the increased reporting overhead.

For this reason, a better option could be to trigger TA reporting based upon collisions. The UE may be configured to report TA or collision indication to the network based upon detecting a configured number of collisions, collisions of a specific nature (e.g., signal, channel or transmission), or collisions related to a specific (group) of HARQ process.

Observation 5: The TA reporting based upon UE collision detection can enable the network to avoid future collisions through updated scheduling, e.g., by introducing suitable gap intervals between the UL and the DL transmissions.

The TA reporting triggered upon UE collision detection can be the same as legacy or the network may configure the UE with a different TA reporting. The UE may be configured to choose a suitable TA format and/or granularity based upon configuration or collision scenario or number of detected collisions.

Proposal 5: Support UE configuration to trigger TA reporting upon detecting HD-FDD collision.

In one strategy, a UE may report a collision indication (e.g., one bit) to the network that there has been a collision, or the network may estimate missing UL or missing ACK for the DL transmissions. Upon receiving collision indications or detecting the potential collisions at the UE, the network may choose to trigger a full reporting. In one example, the full reporting may comprise of finer TA reporting from the UE. In another example, the full reporting may include the additional details for the past collisions, e.g., collision identification, number of detected collisions for different collision types, traffic types, or collisions for different priorities etc.

Proposal 6: Support the enhanced TA reporting from the UE upon network request where the network may request the reporting granularity or additional details on the past collisions.

## Collision Avoidance by Scheduling and Resource Configurations

In one broad category, we may consider the collision avoidance by network scheduling. The network scheduler may provide semi-static configuration and dynamic scheduling such that at least Case 3 and Case 4 collisions won’t occur. The network scheduler may keep some scheduling margin among the resource configurations to avoid such collisions. As the network may not always know TA or TA drift, the scheduling based avoidance comes at the expense of introducing additional gaps or delays, which may lead to resource inefficiency, QoS degradation or both.

A more effective technique to avoid HD-FDD collisions can be to update the resource configurations periodically based upon TA and/or TA drift etc. One design can be to network updating the resource configuration and signaling it to the UE.

In an alternative strategy, the UE can be provided with multiple resource configurations, and after each period, the UE may select and indicate one suitable resource configuration to the network. This may allow the UE to select a suitable configuration based upon its TA, TA drift, etc. which will avoid HD-FDD collisions at the UE. This technique can be implemented in a window based approach, where UE selects one configuration in a first window based upon its estimated TA. Then UE selection of a configuration in the first timing window may activate a second resource configuration in the second timing window.

Proposal 7: Support UE reporting of a preferred resource configuration to avoid HD-FDD collisions.

This scheme may allow the resource configuration adaptation based upon TA and TA drift. In this manner, without loss of scheduling efficiency or scheduling many resource configurations to a UE, the network may avoid HD-FDD semi-static collisions.

## SIB19 Collisions with UL Transmissions

RAN1 made an agreement in RAN1#116 meeting to study the potential collision cases of SIB19 with UL transmissions for which further discussions didn’t happen due to lack of time.

The reception of SIB19 is fundamental to NTN operation. Typically, SIB19 is transmitted multiple times within a validity period. If a HD-FDD UE is provided with UL resources colliding with SIB19 reception, the following may happen:

- If a HD-FDD UE always prioritizes the SIB19 reception, the UE will drop colliding UL transmissions, resulting in performance degradation for UL data. This may particularly be an issue when the UE does not actually need to receive SIB19, e.g., when it has valid ephemeris data.

- If a HD-FDD UE always prioritizes the UL transmissions over SIB19 reception, the ephemeris data available to the UE may expire and the UE may have issues with NTN operation. Thus, from this perspective, always prioritizing the UL transmissions over SIB19 reception may not be a good idea.

With the above discussion, it is clear that in case of SIB19 collisions, having a fixed rule will either lead to severe UL throughput degradation (with SIB19 prioritized) or potential loss of NTN connection (with SIB19 de-prioritized).

Observation 6: A UE prioritizing the UL transmissions over SIB19 reception may have ephemeris data expired, while a UE prioritizing the SIB19 reception may have serious UL performance degradation.

Given that SIB19 is transmitted through dynamic scheduling, both PDCCH and PDSCH transmissions are involved in SIB19 transmission. So, a UE needs to receive both PDCCH (semi-static DL) and PDSCH (dynamic DL) to be able to successfully decode SIB19. A UE will not receive PDCCH (scheduling SIB19) whenever it collides with a semi-statically configured UL transmission, if it has UL priority indication for UL vs DL for case 3 collisions, resulting in the expiry of SIB19 and ephemeris data.

Observation 7: If case 3 defines a fixed rule to prioritize UL, a UE may not receive PDCCH (scheduling SIB19) whenever it collides with a semi-statically configured UL transmission, resulting in the expiry of SIB19 and ephemeris data.

On the other hand, if case 3 specifies a fixed priority for DL, the UEs will always receive PDCCH (scheduling SIBs, e.g., SIB19) upon detecting a case 3 collision, even if they have valid ephemeris data and don’t need to receive SIB19. This frequent discarding of UL traffic will lead to UL throughput degradation for the UEs and will contribute to poor spectral efficiency for the system.

Observation 8: If case 3 defines a fixed rule to prioritize DL, a UE will receive PDCCH (scheduling SIB19) whenever it collides with a semi-statically configured UL transmission, even if the UE has valid SIB19 leading to loss of UL throughput and poor system efficiency.

This means that the SIB19 reception needs to be investigated for the following collision scenarios:

- semi-static DL (PDCCH for SIB19) colliding with UL transmissions

- Dynamic DL (PDSCH for SIB19) colliding with UL transmissions.

Proposal 8: Investigate the following collision scenarios in relation to SIB19 reception:

- PDCCH for SIB19 (Semi-static DL) colliding with UL transmissions

- PDSCH for SIB19 (Dynamic DL) colliding with UL transmissions.

SIB19 transmissions are periodic DL transmissions with multiple SIB19 transmissions in each validity interval and a UE may select a suitable window/interval to acquire SIB19. The issue is that the network does not know the validity of SIB19 at the UE and when UE will acquire SIB19. A simple solution to avoid SIB19 expiry is to always prioritize DL to secure SIB19 data, although that may be very sub-optimal for UE UL data QoS. Another option could be the network configuration based upon UE assistance, e.g., the UE informing the network about the validity of its ephemeris data and then network providing some configurable prioritization to the UE in relation to the SIB19 transmissions. This may be technically feasible but may incur large signaling overhead to keep the network informed for the UE ephemeris data. Another option could be the UE autonomous determination of prioritization where, e.g., the UE may determine to prioritize SIB19 reception based upon a condition on the validity duration of its ephemeris data. This can be beneficial as the UE wouldn’t need to send assistance information to the network to receive SIB19 prioritization which may overload the network resources when there are a lot of UEs sending the assistance data.

Proposal 9: For the SIB19 collisions with UL transmissions, at least, the following broad schemes may be considered as baseline for prioritization:

  - Network configuration

  - UE autonomous determination

## Collision scenarios with more than two Overlapping Transmissions

For HD-FDD Redcap device operating in NTN, due to TA and TA drift, more than two transmissions may collide at the UE. A multi-transmission collision involves multiple (i.e., more than two) transmissions a UE is scheduled/configured to transmit/receive having at least one overlapping symbol in time. As an example, types of colliding transmissions which may be considered include dynamic (e.g., DCI scheduled) DL signaling (e.g., PDSCH, CSI-RS), dynamic (e.g., DCI scheduled) UL signaling (e.g., PUSCH, PUCCH, PRACH, SRS), semi-static DL signaling (e.g., PDCCH, PDSCH, CSI-RS, DL-PRS) and semi-static UL signaling (e.g., PUSCH, PUCCH, SRS)

An exemplary scenario is considered in Figure 2-1 below involving the collision of a series of three transmissions. Case 1 (left) involves the collision of two downlink transmissions with one uplink transmission. Alternatively, Case 2 (right) involves the collision of two uplink transmissions with one downlink transmission.

Figure 2-1: Collision scenarios for 3 overlapping transmissions

Figure 2-1 further depicts two time periods: tov1_2 which may be defined as the duration of overlap (e.g., in time) between the first colliding transmission and the second colliding transmission; and tov2_3, which may be defined as the duration of overlap (e.g., in time) between the second colliding transmission and the third colliding transmission. Due to TA and TA drift at NTN UE, these scenarios may exist.

It’s not clear to us whether the existing collision rules specified in the specification lead to a clear UE behavior upon detecting of such multi-transmission collisions.

The UE behavior may be ambiguous for multi-transmission collision from which transmissions to prioritize or drop, for example:

- UE may drop the two UL transmissions when 1st UL transmission is lower priority

- UE may drop DL transmission if any one of UL transmissions is higher priority

- UE may drop first UL transmission and receive a portion of DL and switch to UL for higher priority UL transmission, etc.

Such undefined UE behavior may cause issues at the gNB scheduler as there would be different UEs having different behaviors. RAN1 can discuss such multi-transmission collision scenarios and clarify if the UE behavior is properly defined in the specification.

Proposal 10: Clarify the UE behavior upon detecting the multi-transmission (>2) collision scenarios for HD-FDD RedCap NTN operation.

# Conclusions

In this contribution, the following observations are made:

Observation 1: The network cannot always determine non-colliding semi-static UL and semi-static DL configurations for a HD-FDD UE due to TA drift.

Observation 2: The network cannot ensure scheduling non-colliding dynamic UL vs dynamic DL transmissions for a HD-FDD UE due to TA drift unless large gap is introduced by the network between such transmissions.

Observation 3: Frequent TA reporting, for example by using a smaller TA offset threshold, from several UEs will result in large UL overhead and may lead to poor system efficiency.

Observation 4: Finer TA granularity provides little help against TA drift despite the increased reporting overhead.

Observation 5: The TA reporting based upon UE collision detection can enable the network to avoid future collisions through updated scheduling, e.g., by introducing suitable gap intervals between the UL and the DL transmissions.

Observation 6: A UE prioritizing the UL transmissions over SIB19 reception may have ephemeris data expired, while a UE prioritizing the SIB19 reception may have serious UL performance degradation.

Observation 7: If case 3 defines a fixed rule to prioritize UL, a UE may not receive PDCCH (scheduling SIB19) whenever it collides with a semi-statically configured UL transmission, resulting in the expiry of SIB19 and ephemeris data.

Observation 8: If case 3 defines a fixed rule to prioritize DL, a UE will receive PDCCH (scheduling SIB19) whenever it collides with a semi-statically configured UL transmission, even if the UE has valid SIB19 leading to loss of UL throughput and poor system efficiency.

The observations have led to the following proposals:

Proposal 1: Support to specify the priorities for case 3 based upon the type of channels undergoing collisions.

Proposal 2: Support network configurable priorities for case 3 collisions.

Proposal 3: Support to define the priority rules for case 4.

Proposal 4: Support to specify the priorities for case 4 based upon the type of signals/channels undergoing collisions.

Proposal 5: Support UE configuration to trigger TA reporting upon detecting HD-FDD collision.

Proposal 6: Support the enhanced TA reporting from the UE upon network request where the network may request the reporting granularity or additional details on the past collisions.

Proposal 7: Support UE reporting of a preferred resource configuration to avoid HD-FDD collisions.

Proposal 8: Investigate the following collision scenarios in relation to SIB19 reception:

- PDCCH for SIB19 (Semi-static DL) colliding with UL transmissions

- PDSCH for SIB19 (Dynamic DL) colliding with UL transmissions.

Proposal 9: For the SIB19 collisions with UL transmissions, at least, the following broad schemes may be considered as baseline for prioritization:

  - Network configuration

  - UE autonomous determination

Proposal 10: Clarify the UE behavior upon detecting the multi-transmission (>2) collision scenarios for HD-FDD RedCap NTN operation.

# Reference

- RP-234078, “New WID: Non-Terrestrial Networks (NTN) for NR Phase 3”, RAN#102, December 2023.

- RP-241667, “New WID: Non-Terrestrial Networks (NTN) for NR Phase 3”, RAN#104, June 2024

- RAN1 Chairman’s note, RAN1 #116, February 2024.

- RAN1 Chairman’s note, RAN1 #118, August 2024.

- RAN1 Chairman’s note, RAN1 #118Bis, October 2024.

- NR; Physical layer procedures for control, TS38.213 v 18.1.0.