3GPP TSG RAN WG1 #119                                                                     R1- 2409796

Orlando, USA, November 18th – 22nd, 2024

Agenda Item:	9.3.1

Source:	Apple Inc.

Title:	Discussion on SBFD TX/RX/measurement procedures

Document for:	Discussion/Decision

# Introduction

In RAN#102, a new work item was endorsed in RP- 234035, later revised in RP-241614, to specify Evolution of NR duplex operation: Sub-band full duplex (SBFD) in Rel-19, where one of the objectives is related SBFD TX/RX/measurement procedures as follows (with some arrangements):

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of time location of SBFD subbands in SIB is not precluded - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of frequency domain location of SBFD subbands in SIB is not precluded - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2] - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2] - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon - UL transmissions within UL subband only - DL receptions within DL subband(s) only, except for CLI measurement by the UE outside of the DL subbands Note: When flexible symbols are used, it is not expected that any legacy Uplink symbol is converted to Downlink/SBFD symbols - Enhancement on resource allocation in frequency domain in SBFD symbols, including - resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols - handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols, including - resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots - CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol

In this contribution, we share our views on objectives related to SBFD TX/RX/measurement procedures.

# Enhancements for SBFD operation at gNB

## Indication of time/frequency location of SBFD subbands

For SBFD sub-band indication in frequency, some related agreements were achieved so far, which define SBFD sub-bands in frequency for SBFD operation from gNB perspective. Indeed, none of the previous agreements mandate UE in SBFD slot to support being scheduled across both DL sub-bands. In fact, some UE implementations may prefer to be scheduled in only one of the DL sub-bands in SBFD slots/symbols. Some other UEs may not be able to cancel CLI in SBFD slot, and thus indicate to be scheduled in SBFD slots only in UL. In general, different SBFD-aware UEs can be defined as follows:

Proposal 1: a SBFD-aware UE can exist subject to different UE capabilities

- Cap1: Support being scheduled in SBFD symbol in UL sub-band only

- Cap2: Support being scheduled in SBFD symbol in UL sub-band and/or in only one of the DL sub-bands

- Cap3: Support being scheduled in SBFD symbol in UL sub-band and/or across both DL sub-bands

In the above formulation, Cap1 is the least capable UE in terms of UE-to-UE CLI handling, as such a UE avoids being scheduled in downlink in SBFD symbols in any of the DL sub-bands. Cap3 is the most capable UE as it can be scheduled in downlink across the two DL sub-bands. Cap2 UE is in the middle. For a SBFD-aware UE of Cap2, SBFD switching delay is defined as the time offset between switching from non-SBFD symbols to SBFD symbols (and vice versa), or from one sub-band to another sub-band within SBFD symbols.

Proposal 2: For a SBFD-aware UE which can receive over only one of DL sub-bands in SBFD symbols, SBFD switching delay is defined as the time offset between switching from non-SBFD symbols to SBFD symbols (and vice versa), or from one sub-band to another sub-band within SBFD symbols.

SBFD switching delay can reuse the numbers already specified in 38.133 Table 8.6.2-1, for BWP switch delay without BWP switch indication, or subject to UE capability new slot or sub-slot offsets can be specified. Here it must be noted that there will be no active BWP switching even for low capable UEs, s no new design/specification is needed in terms of BWP switch indication, number of configured BWP etc.

Figure 1: SBFD switching delay for a UE incapable of being scheduled across both DL sub-bands

In the example shown in Figure 1, a guard band is assumed between UL and DL sub-bands. In practice, UE should be able to indicate as part of UE capability, minimum required guard band between UL and DL sub-bands.

Proposal 3: UE indicates as part of UE capability signaling, the minimum required guard band between UL and DL sub-bands, in the unit of frequency, or number of PRBs for each tone spacing.

## More on resource configurations for SBFD operation

For uplink transmission (or downlink reception) with Configuration 2, where different occasions may span different symbol types, the following agreement (and working assumption) was achieved in RAN1#118

Agreement (118)

For a CG PUSCH configuration without repetitions, if the transmission occasions are across SBFD symbols and non-SBFD symbols where each transmission occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), for PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PUSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for TBoMS across SBFD symbols and non-SBFD symbols in different slots, where each transmission within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2),

- Option 2: Single resource configuration/indication for non-SBFD symbols and RB offset(s) configuration/indication/determination to determine frequency resource for SBFD symbols

  - The numbers of PRBs are the same for PUSCH transmissions in SBFD symbols and PUSCH transmissions in non-SBFD symbols

Working Assumption

For an SPS PDSCH configuration without repetitions, if the reception occasions are across SBFD symbols and non-SBFD symbols where each reception occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), PDSCH repetitions across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PDSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PDSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2),

- Option 5: Only the assigned PRBs within DL usable PRBs in SBFD symbols are considered to be valid.

  - For SPS PDSCH, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only

  - For PDSCH repetitions, FFS whether the number of PRBs for TBS determination is based on assigned PRBs, or based on assigned PRBs within DL usable PRBs only, or based on assigned PRBs within DL usable PRBs of the first repetition.

  - For multi-PDSCH scheduled by a single DCI, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only.

The concern of the above agreements is mainly on frequency domain resource allocation. In general for Configuration 2, for an activated CG-PUSCH, where different repetitions span different slot types (non-SBFD vs SBFD), different CG-PUSCH parameters may be desired. This is due to the facts that UE may follow different power control parameters, and/or gNB may experience self-interference, wider beam indication etc for SBFD slots vs non-SBFD slots. In Fig. 2, an example is shown where CG PUSCH is configured with RRC parameter periodicity as 2 slots (sym2x14). In this example, MCS corresponding to those occasions within legacy non-SBFD slots (CG_PUSCH 1 and CG_PUSCH_3) is different from the MCS for occasions that happen in a SBFD slot (CG_PUSCH_2).

Figure 2: CG-PUSCH configuration with different MCS configurations

Proposal 4: For a type-1 CG-PUSCH, ConfiguredGrantConfig may include a pair of parameters for some of the fields like as follows, where the second value indicates the parameter for CG-PUSCH transmission in SBFD slot:

- (p0-PUSCH-Alpha, p0-PUSCH-Alpha2)

- (srs-ResourceIndicator, srs-ResourceIndicator2)

- (pathlossReferenceIndex, pathlossReferenceIndex2)

- (mcsAndTBS, mcsAndTBS2)

In current specification a single DCI can schedule multiple PDSCHs (or PUSCHs), where most of the parameters are the same across different grants, e.g., they all follow the same MCS and FDRA, although TDRA (SLIV) could be different for different grants. Here again, similar o CG-PUSCH (or SPS-PDSCH), it is desired to apply different parameters to different grants, depending on the slot type, i.e., SBFD vs non-SBFD.

Proposal 5: DCI for multi-grant scheduling can have two different MCS fields for PDSCHs (or PUSCHs) in SBFD and non-SBFD slots, the first MCS bit-field is applied to grants in SBFD slots and the second bit-field is applied to grants in non-SBFD slots (or vice versa)

For SRS transmissions across both SBFD and non-SBFD symbol types, the following agreement is made in RAN1#118b

Agreement

Support separate SRS configurations for SBFD and non-SBFD symbols. Select one from the following options.

- Option 1: Support separate SRS-ResourceSets configurations for SBFD and non-SBFD symbols for a given usage.

  - SRS-ResourceSet configured for SBFD symbol is only applicable to SRS transmission occasions in SBFD symbols.

    - For periodic and semi-persistent SRS, the SRS transmissions in non-SBFD symbols are dropped.

    - For aperiodic SRS with available slot counting, only SBFD symbols are available for SRS transmission.

    - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the SBFD symbols.

  - SRS-ResourceSet configured for non-SBFD symbol is only applicable to SRS transmission occasions in non-SBFD symbols.

    - For periodic and semi-persistent SRS, the SRS transmissions in SBFD symbols are dropped.

    - For aperiodic SRS with available slot counting, only non-SBFD symbols are available for SRS transmission.

    - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the non-SBFD symbols.

  - FFS applicable usages

- Option 2: Support separate configurations for SBFD and non-SBFD symbols in the same SRS-Resource.

  - At least freqDomainPosition, freqDomainShift and freqHopping are separately configured.

    - FFS separate configurations of other parameters

  - FFS applicable usages

In our view, to configure separate SRS resource sets, has the following drawbacks:

- For most of scenarios, SRS is unnecessarily dropped, even if the configured SRS resource bandwidth is within UL BWP (for SRS-ResourceSet configured for non-SBFD symbol), or within UL usable PRBs (for SRS-ResourceSet configured for SBFD symbol).

- Additional specification is needed to make sure a clear distinguish between this use-case, and mTRP PUSCH repetition, which is specified for TDM operation in Rel-17 and SDM/SFN operation in Rel-18. This is not a trivial effort, especially considering that justification behind such separate configuration per SRS-ResourceSet is not really met.

We should note that for other uplink channels where the resource in frequency domain is RRC configured, like PUCCH, we already agreed to have separate RRC configurations for transmissions in SBFD and non-SBFD. In our view, we must follow the same principle. There is no justification to break that procedure for SRS. Based on the above discussion, we support Option 2, and the following is proposed:

Proposal 6: Support separate configurations for SBFD and non-SBFD symbols in the same SRS-Resource.

- At least freqDomainPosition, freqDomainShift and freqHopping are separately configured.

On how to indicate to UE configuration 1 or Configuration 2, the following agreement is made in RAN1#118b.

Agreement

For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with Configuration 1 or Configurat 2 via RRC configuration. Select from the following options

- Option 1: SBFD-aware UE is configured with Configuration 1 or Configuration 2 on a per UE per serving cell basis

- Option 2: SBFD-aware UE is configured with Configuration 1 or Configuration 2 on a per UE per BWP per channel/signal basis

  - Separate configurations for configured and dynamic transmission/receptions for a given channel/signal are not precluded.

- Option 3: Configuration 1 or Configuration 2 is on a per cell basis.

  - No separate UE capabilities for Configuration 1 and 2.

In our view, the configuration should not be per BWP. Now whether it is UE specific per serving cell or cell-specific per serving cell, both procedures will work. The difference is UE specific signaling is limited to UE in RRC connected while cell specific works for UE in RRC idle as well. Given that the focus of this agenda is UEs in RRC connected we support Option 1.

Proposal 7: The SBFD-aware UE is provided with Configuration 1 or Configuration 2 per serving cell through RRC signaling.

## UL Power control

Uplink power control is a technique to manage UE-to-UE and/or gNB-to-gNB CLI. In RAN1#118, it was agreed that

Agreement

For a single TRP scenario, for separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols based on unified TCI state framework, down-select from the following options to support separate configurations of UL power control parameters.

- Option 1: One TCI codepoint is mapped to separate TCI states for SBFD symbols and non-SBFD symbols respectively

  - FFS: No RRC impact

  - FFS: New MAC CE or reuse existing MAC CE

- Option 2: Same unified TCI state is associated with separate UL power control parameters for SBFD symbols and non-SBFD symbols

  - FFS whether a new Uplink-powerControlId is introduced for SBFD symbols, or new P0AlphaSets are introduced in Uplink-powerControl for SBFD symbols for PUSCH, PUCCH and SRS respectively

  - FFS: Impact on MAC CE

- Option 3: Same unified TCI state is associated with same UL power control parameters for SBFD symbols and non-SBFD symbols

  - A power offset is introduced for transmission in SBFD symbols for PUSCH, PUCCH, and SRS

    - FFS: Same offset or different offsets for all channels

  - FFS: RRC or MAC CE introduced to convey power offset

- FFS whether the maximum number of closed power control loops is increased to be more than 2.

- FFS: Whether/How to handle the case where UL power control is not associated within TCI state

In our view, although gNB may utilize different antenna elements when operating in SBFD and non-SBFD slots, for a given UL (DL) channel, same TCI can be applied for transmission (reception) in SBFD and non-SBFD slots. Under unified TCI framework, P0 (not the nominal part), alpha, PL-RS and loop index may be indicated to UE as part of TCI state configuration.

Given that uplink transmissions in SBFD and non-SBFD symbols may have to follow different requirements to address CLI management in SBFD symbols, it is desired to define separate power control parameters. In Rel-17, power control parameter setting through unified TCI framework was defined where a set of power control parameters per channel (PUSCH/PUCCH/SRS) associated with a joint or UL TCI State is RRC configured. In this procedure, PL-RS should be always configured while other power control parameters (p0, alpha, loop index) are optionally configured, as shown in Fig. 3

Proposal 8: Under unified TCI framework, two Uplink-powerControlIds are configured for the same TCI state

Figure 3: Two Uplink-powerControlIds per TCI state under unified TCI framework

# Conclusion

In this contribution, we shared our views on different aspects related to UE TX/RX/measurement procedures for SBFD operation at the gNB. Based on what we discussed, the following proposals are made:

Proposal 1: a SBFD-aware UE can exist subject to different UE capabilities

- Cap1: Support being scheduled in SBFD symbol in UL sub-band only

- Cap2: Support being scheduled in SBFD symbol in UL sub-band and/or in only one of the DL sub-bands

- Cap3: Support being scheduled in SBFD symbol in UL sub-band and/or across both DL sub-bands

Proposal 2: For a SBFD-aware UE which can receive over only one of DL sub-bands in SBFD symbols, SBFD switching delay is defined as the time offset between switching from non-SBFD symbols to SBFD symbols (and vice versa), or from one sub-band to another sub-band within SBFD symbols.

Proposal 3: UE indicates as part of UE capability signaling, the minimum required guard band between UL and DL sub-bands, in the unit of frequency, or number of PRBs for each tone spacing.

Proposal 4: For a type-1 CG-PUSCH, ConfiguredGrantConfig may include a pair of parameters for some of the fields like as follows, where the second value indicates the parameter for CG-PUSCH transmission in SBFD slot:

- (p0-PUSCH-Alpha, p0-PUSCH-Alpha2)

- (srs-ResourceIndicator, srs-ResourceIndicator2)

- (pathlossReferenceIndex, pathlossReferenceIndex2)

- (mcsAndTBS, mcsAndTBS2)

Proposal 5: DCI for multi-grant scheduling can have two different MCS fields for PDSCHs (or PUSCHs) in SBFD and non-SBFD slots, the first MCS bit-field is applied to grants in SBFD slots and the second bit-field is applied to grants in non-SBFD slots (or vice versa)

Proposal 6: Support separate configurations for SBFD and non-SBFD symbols in the same SRS-Resource.

- At least freqDomainPosition, freqDomainShift and freqHopping are separately configured.

Proposal 7: The SBFD-aware UE is provided with Configuration 1 or Configuration 2 per serving cell through RRC signaling.

Proposal 8: Under unified TCI framework, two Uplink-powerControlIds are configured for the same TCI state.