3GPP TSG RAN WG1 #119        	          	      		            	R1- 2409830

Orlando, US, November 18th – 22nd, 2024

Agenda Item:   	5

Source:   	LG Electronics

Title: 	Discussion on LS on OCC for CB-msg3 NPUSCH

Document for:   	Discussion and decision

- Introduction

In this contribution, we will provide our view on the applicability of NPUSCH’s OCC solution to CB-msg3 NPUSCH, which is a question in RAN2 LS [1].

- Discussion

The following question regarding the support of OCC for CB-msg3 NPUSCH was asked for RAN1 to provide an answer in LS [1]:

- Question: Whether the OCC solution for NPUSCH on which RAN1 is working can be applied to CB-msg3 NPUSCH or not.

First of all, for the OCC design of NPUSCH format 1 and NPRACH, the following agreements were reached at the last RAN1 meeting, but we believe that RAN1 still needs further work on its remaining details. Also, as shown in the yellow marked FFS points below, RAN1 has not yet made a decision on whether to support OCC for CB-msg3 NPUSCH. Therefore, at this point, it seems difficult to clearly answer whether the OCC solution for NPUSCH that RAN1 has worked on so far can be applied to CB-msg3 NPUSCH.

Agreement At least the following schemes are supported for single-tone: - For 3.75kHz SCS OCC for NPUSCH format 1: - OCC length 2, Symbol-level - FFS: DMRS pattern(s) - For 15kHz SCS OCC for NPUSCH format 1: - OCC length 2, Slot-level, CDM DMRS with legacy pattern - FFS: CDM details, e.g. with or without spreading Agreement For NPRACH transmission, inter-symbol group OCC is not further studied. Agreement For support of single-tone OCC for NPUSCH format 1, RAN1 studies: - The parameters that need to be signalled, considering the following: - OCC codeword - Enabling of OCC feature - FFS: other parameters - For dynamic grant in RRC CONNECTED, study which and whether any parameters are signalled via DCI and which and whether any parameters are signalled by RRC - FFS: whether/how to support cases other than dynamic grant in RRC CONNECTED, e.g. Msg3, PUR, CB Msg3 EDT

Observation 1: Considering the completion level of OCC design for NPUSCH that RAN1 has worked on up to the last RAN1 meeting, it seems difficult for RAN1 to clearly answer at this point whether the OCC solution for NPUSCH can be applied to CB-msg3 NPUSCH.

We think that there are several additional factors (as described in Section 2.1 of [2]) that should be considered when supporting OCC in CB-msg3 NPUSCH. For example, since CB-msg3 EDT is performed in IDLE state, it is impossible for eNB to dynamically indicate/assign different OCC codewords between UEs that are multiplexed, unlike RRC_CONNECTED state. As one possible way to resolve this issue, the eNB provides a set of OCC codeword candidates via SIB and let the UE randomly selects one of them. Additionally, since Msg1/2 does not exist in CB-msg3 EDT, it needs to be clarified how to define the RNTI value used to determine the scrambling sequence of CB-msg3 NPUSCH as described in TS 36.211 Section 10.1.3.1 below. In other words, for CB-msg3 EDT, the TC-RNTI value cannot be used for this purpose since there is no TC-RNTI value that is typically provided via Msg2 during the initial access procedure. To address this problem, we can consider having the UE select a virtual NPRACH resource and use the RA-RNTI value derived from it, or having the UE use the contention resolution ID conveyed in Msg3. Note that these RNTI values and/or virtual NPRACH resource related parameters can also be used by the UE to select the OCC codeword to apply to its CB-msg3 NPUSCH transmission. Lastly, for OCC multiplexing between different UEs to work properly, the start time of CB-msg3 NPUSCH transmission of the UE needs to be restricted from being performed at any point in time. This is because OCC multiplexing may fail when CB-msg3 NPUSCH transmissions with different OCC codewords partially overlap in time domain. To be specific, it can be defined that CB-msg3 NPUSCH transmission is allowed to start only in the first subframe of the granularity to which the OCC codeword applies.

10.1.3.1 Scrambling Scrambling shall be done according to clause 5.3.1. The scrambling sequence generator shall be initialised with where is the first slot of the transmission of the codeword. In case of NPUSCH repetitions, the scrambling sequence shall be reinitialised according to the above formula after every transmissions of the codeword with and set to the first slot and the frame, respectively, used for the transmission of the repetition. The quantity is given by clause 10.1.3.6.

Observation 2: At least the following aspects should be additionally considered to support OCC in CB-msg3 NPUSCH:

- How to make multiplexed UEs select as different OCC codewords as possible, considering that the eNB cannot dynamically indicate/assign the UEs to select OCC codewords

- How to define the RNTI value used to determine the scrambling sequence of CB-msg3 NPUSCH, since there is no TC-RNTI value in CB-msg3 EDT that is typically provided via Msg2 during the initial access procedure

- How to limit the start time of CB-msg3 NPUSCH transmission of the UE, considering the possibility that OCC multiplexing fails when CB-msg3 NPUSCH transmissions with different OCC codewords partially overlap in time domain

- Conclusion

In this contribution, we discussed how RAN1 provides feedback on RAN2’s question in LS [1]. The following observations are given.

Observation 1: Considering the completion level of OCC design for NPUSCH that RAN1 has worked on up to the last RAN1 meeting, it seems difficult for RAN1 to clearly answer at this point whether the OCC solution for NPUSCH can be applied to CB-msg3 NPUSCH.

Observation 2: At least the following aspects should be additionally considered to support OCC in CB-msg3 NPUSCH:

- How to make multiplexed UEs select as different OCC codewords as possible, considering that the eNB cannot dynamically indicate/assign the UEs to select OCC codewords

- How to define the RNTI value used to determine the scrambling sequence of CB-msg3 NPUSCH, since there is no TC-RNTI value in CB-msg3 EDT that is typically provided via Msg2 during the initial access procedure

- How to limit the start time of CB-msg3 NPUSCH transmission of the UE, considering the possibility that OCC multiplexing fails when CB-msg3 NPUSCH transmissions with different OCC codewords partially overlap in time domain

- Reference

[1] R1-2409343, “LS on OCC for CB-msg3 NPUSCH,” RAN2, vivo.

[2] R1-2409834, “Discussion on IoT-NTN uplink capacity/throughput enhancement,” LG Electronics.