3GPP TSG RAN WG1 #119		R1- 2409862

Orlando, US, November 18th – 22nd, 2024

Agenda item:	9.6.3

Source:	NEC

Title:              	Discussion on LP-WUS operation in RRC CONNECTED mode

Document for:	Discussion and Decision

Introduction

In the RAN1#118b, the following agreements on LP-WUS operation in RRC CONNECTED mode were achieved:

Agreement For Option 1-1 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. - LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer - UE is configured with legacy C-DRX configurations as Rel-18 - UE behaviors related to CDRX active time triggered by all legacy DRX timers are not affected unless included in this proposal - No impact on RRM/RLM/BFD measurement requirements is assumed - For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP) - Periodic CSI/L1-RSRP is not reported during the time given by the configured drx-onDurationTimer if UE is not indicated to wake-up - Periodic CSI/L1-RSRP is periodically reported during the time given by the configured drx-onDurationTimer regardless if UE is indicated to wake-up or not - Note: Periodic CSI/L1-RSRP is reported during CDRX active time as in legacy specification Agreement Confirm following working assumption Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Note: Above can be revisited considering RAN2 decisions. Agreement For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. - FFS: X - FFS: definition of reported candidate value - UE can indicate its preference for configured time offset between LP-WUS and PDCCH reception using UAI mechanism. FFS details. UAI is optional. - FFS: A single time offset is configured for a UE at a given time Agreement LP-WUS is supported when UE is configured with NR-DC in RRC CONNECTED mode - Above is supported for the case PDCCH monitoring is triggered by LP-WUS in the same cell group - FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable Agreement - For LP-WUS CONNECTED mode operation, Option 1-2 can be configured separately from Option 1-1 - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Agreement - Regarding the potential restriction for LP-WUS configuration in relation with C-DRX configuration for option 1-2 of LP-WUS CONNECTED mode operation, from RAN1 perspective, - AltOpt2: At least LP-WUS periodicity is no larger than long C-DRX cycle, FFS other restrictions - Introducing monitoring window is not precluded

In this contribution, we provide our views on LP-WUS operation in RRC CONNECTED mode.

Discussion

First of all, in RRC_IDLE/INACTIVE, it is expected that a LP-WUS configuration applied to a group of RRC_IDLE/INACTIVE UEs will be introduced, e.g., the LP-WUS configuration is provided in system information for all UEs, and each LP-WUS capable UE may, based on its specific parameter such as UE-ID to determine which LP-WUS resource it will apply. It is noted that in this way, multiple UEs will be associated with the same LP-WUS resource, which causes false alarm issues (i.e., unnecessary wake-ups by indications intended for the other UEs). However, for LP-WUS operation in RRC_CONNECTED, the UE can receive dedicated RRC signaling and has UE-specific monitoring resources, so besides common LP-WUS configuration, the UE-specific LP-WUS configuration is possible for RRC_CONNECTED UE.

For UE-specific LP-WUS configuration, the network could use dedicated RRC signaling such as RRCSetup, RRCResume during RRC connection establishment, or RRCReconfiguration during RRC_CONNECTED to provide the UE-specific LP-WUS configuration to the UE. Since the LP-WUS monitoring resource is dedicated to each individual UE, no false alarm issue will be introduced then.

On the other hand, considering a LP-WUS may occupy relatively large time-frequency resources, e.g., ~ 5MHz bandwidth and several OFDM symbols, supporting UE-specific LP-WUS in RRC connected mode may introduce considerable overhead. Therefore, group based LP-WUS can also be considered for RRC_CONNCTED to improve network resource efficiency. The group based LP-WUS can be jointly designed with the legacy DCP mechanism (i.e., wake up signal for C-DRX) to improve the false alarm of wake-up. For example, the UE could firstly monitor a group-common LP-WUS, after a wake-up indication is received, UE may monitor a DCP to further check whether a wake-up indication for it is comprised, and UE may fallback to LP-WUS monitoring if no wake-up indication is received. Through this method, a better tradeoff between power saving and false wake-up rate can be achieved. .

In RAN1#116b, several options for case 1 for the LP-WUS procedures to trigger PDCCH monitoring were agreed for further study. In our view, case 1, i.e., LP-WUS combined with C-DRX, can be supported to further improve the power saving of C-DRX. The following figure gives an example for option 1-1 of case 1, where an LP-WUS monitoring occasion is configured for each drx-onDurationTimer based on e.g., offset in every C-DRX cycle, once LP-WUS is detected, the UE starts drx-onDurationTimer to enter Active Time for PDCCH monitoring. Otherwise the UE keeps in inactive mode until the next C-DRX cycle. Since the power consumption of LP-WUS monitoring is less than DCP monitoring by MR, further power saving gain can be achieved by replacing DCP with LP-WUS.

Proposal 1: support LP-WUS in combination with DCP to further reduce power consumption without introducing too much overhead.

The difference between option 1-2 and option 1-1 is whether UE can start PDCCH monitoring irrespective of the on-duration timer. In our view, option 1-2 may be beneficial for the traffic with unpredictable jitter of arrival time, e.g., XR services, therefore it can also be further considered. Regarding option 1-2-1 (i.e., support Option 1-1 and Option 1-2 simultaneously) and option 1-2-2, similar power saving gain can be achieved if option 1-2-1 can be jointly used with option 1-1. However, the standard impact of option 1-2-1 is less than option 1-2-2 because the UE behavior in C-DRX active time will not be changed for option 1-2-1.

In addition, it is preferred that an LP-WUS monitoring occasion can be located at any time without C-DRX config. In this way, the configuration of LP-WUS could be more flexible so that UE can detect data/signal arrival by LP-WUS in a timely manner. When LP-WUS is detected, a new active time can be started, e.g., drx-InactivityTimer (with C-DRX config) or new activeTimerLPWUS (without C-DRX config) to let UE monitor PDCCH within the timer.

Proposal 2: support Option 1-1 and Option 1-2 simultaneously configured for the same UE.

Conclusion

In this contribution, we discuss the LP-WUS operation in RRC CONNECTED mode, and the following proposals are made:

Proposal 1: support LP-WUS in combination with DCP to further reduce power consumption without introducing too much overhead.

Proposal 2: support Option 1-1 and Option 1-2 simultaneously configured for the same UE.

References

- RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), CMCC, RAN meeting #102, Dec. 11-15, 2023