3GPP TSG RAN WG1 #119			R1-2409896

Orlando, US, November 18th – 22nd, 2024

Source: 	     Xiaomi

Title:	Discussion on ambient IoT device architectures

Agenda item: 	9.4.1.2

Document for:  Discussion

Introduction

The revised Rel-19 study item on ambient internet of things (A-IoT) for NR has been approved in RAN#103 plenary meeting [1], which can be found in Appendix. Furthermore, we discussed initial issues for A-IoT device architectures during previous meetings, and some agreements were achieved [2-6], which can be found in Appendix.

In this contribution, we present our views on the potential basic blocks/components of A-IoT device architectures.

Evaluation on complexity and power consumption

The industrialization of A-IoT devices hinges on their complexity and power consumption, necessitating a design approach that prioritizes low complexity and power consumption at the outset of discussions. From this perspective, it becomes imperative to evaluate the complexity and power consumption associated with the current ambient IoT design, including aspects such as waveform. However, at this stage, there is a lack of available conditions for assessing the implementation complexity and power consumption of the A-IoT device. Although RAN1 offers the lowest complexity and power consumption for device design, it does not guarantee meeting coverage requirements. Therefore, after ensuring basic KPIs are met, an evaluation of complexity and power consumption should be conducted to provide a basis for selecting appropriate technical solutions.

Observation 1: At this stage, there is a lack of available conditions for assessing the implementation complexity and power consumption of the A-IoT device.

Proposal 1: The evaluation on complexity and power consumption should be addressed subsequent to further clarification of the technical proposal.

In the design of A-IoT devices, high implementation complexity often leads to increased product costs, thus emphasizing the need for lower implementation complexity. Implementation complexity generally encompasses two aspects: hardware and algorithm complexities, which are interdependent. For instance, intricate hardware designs often necessitate complex algorithm implementations. Conversely, reducing hardware design complexity can help mitigate the requirement for complex algorithms to some extent. Therefore, it is preferable to primarily focus on hardware design complexity while interspersing discussions on algorithm implementation within relevant sections.

Observation 2: High implementation complexity often leads to increased product costs.

Observation 3: Reducing hardware design complexity can help mitigate the requirement for complex algorithms to some extent.

Proposal 2: RAN1 should focus on the discussion of hardware complexity for A-IoT device architectures, if need.

Before evaluating complexity, it is necessary to determine a universally applicable method. For example, by dividing a modular circuit into a certain number of basic electronic components, the resulting hardware design with fewer basic electronic components is less complex.

Proposal 3: A universally applicable method should be studied for evaluating complexity, if need.

Compared to legacy UE, A-IoT devices exhibit ultra-lower peak power consumption. To meet power requirements, it is necessary to address the most energy-intensive components of the A-IoT device in further discussion.

Proposal 4: To evaluate power consumption, the most power hungry components the A-IoT device needs to be addressed in the discussion based on the blocks in architectures, and a universally applicable method should be studied, if need.

Architectures for A-IoT device

The discussion on A-IoT device architecture should not impose any limitations on the implementation of the final product. This implies that if the ultimate architecture of the A-IoT device satisfies peak power limits and other corresponding KPIs, the inclusion of components depends on the implementation. A-IoT devices with diverse architectures may exhibit varying capabilities.

Proposal 5: The discussion on A-IoT device architecture should not impose any limitations on the implementation of the final product.

Clock/LO

In RAN1#118bis meeting, the following agreement was achieved for purpose of clock in device:

Agreement - Adopt following updated table format. - One or more rows under a single purpose may exist to capture different options, if any.

As it is discussed previously, the clock in the device potentially serves five distinct purposes, which necessitate diverse performance requirements. In our last meeting, we examined the fundamental characteristics of clocks pertaining to purposes #3, #4, and #5 based on the updated table format provided above. Regarding purpose #1, the clock can be utilized for signal sampling. This function is supported by device 1, 2a, and 2b. In some reference papers, the preferred power consumption of an Ultra-Low-Power clock for sampling is at a few MHz clock speed and can be controlled within tens to hundreds of nW [7,8]. Therefore, the power consumption of the clock can meet the limited peak power requirement of device 1. For device 2a and 2b, there is no significant improvement in clock speed, so the power consumption remains similar to that of device 1. Due to the peak power limitation of device1, achieving clock synchronization by receiving reference signals becomes challenging. Furthermore, calibration on the device side necessitates corresponding hardware support, thereby impeding control over hardware cost and power consumption. Consequently, even with accurate clock correction implementation, further enhancement of clock accuracy remains difficult. Conversely, for device2a and 2b with higher peak power capabilities, superior hardware can be employed on the device side while enabling clock synchronization through received reference signals.

Observation 4: The power consumption is directly proportional to the clock speed, and when the clock speeds of different type devices are similar, their power consumption tends to be comparable.

Observation 5:The performance of device1 and device2a,2b in correcting clock accuracy differs due to the influence of peak power.

Proposal 6: For clock purpose #1, adopt following update.

Regarding purpose #2, When the clock is utilized for a small frequency shift, device 1 and device 2a are capable to support this functionality. This is because the small frequency shift range does not exceed 10 MHz and is directly depending on the clock speed, which in this case operates at a few MHz. Consequently, the power consumption of the clock for small frequency shifts will remain below 1 µW [7,8].  Similar to the purpose#1 clock, device1 faces challenges in achieving accurate timekeeping compared to device2a and 2b due to variations in peak power.

Observation 6: The power consumption of the clock used for small frequency shift shows no significant difference between device1 and device2a, as the range of small frequency shift is not substantially varied.

Proposal 7: For clock purpose #2, adopt following update.

Frequency shift

In previous RAN1 meetings [4][5][6], the following observations and agreements on large frequency shift were achieved.

Observation For large frequency shift: - Large frequency shift can be used in shifting reflected signal in tens of MHz, e.g., from FDD DL to FDD UL frequency or vice versa. - Large frequency shift consumes 10s of uW to 100s of uW. - Large frequency shift is not feasible for device 1. - Large frequency shift requires a clock for IF generation which is accurate enough to avoid large guard band and interference to adjacent channels/bands. - Large frequency shift requires image suppression and may require harmonics suppression - Note: details of image suppression and harmonics suppression are not discussed in RAN1 - FFS: whether large frequency shift is necessary and feasible for device 2a Agreement Add following additional observations for large frequency shifter for device 2a. - Large frequency shift may allow the reader to avoid implementing in-band full duplex capability for scenario e.g., D1T1-A2 and D2T2-A2 - Large frequency shift may result in [e.g., 5kHz~50kHz] of frequency uncertainty in target frequency for IF clock accuracy of [e.g., 0.01%~0.1%] assuming the large frequency shift range is 50MHz Agreement TR captures following descriptions for large frequency shift for device 2a. - Large frequency shift may be technically feasible in terms of power consumption, and device complexity. - Remaining technical feasibility of large frequency shift in terms of image suppression, harmonics suppression may be relevant to device emission requirement, spectrum usage. Additional complexity and power consumption may be needed to achieve image suppression and harmonics suppression. - The necessity or effectiveness of large frequency shift may depend on spectrum for R2D/CW2D/D2R, full duplex capability of reader, whether device 1 and 2a are supported by a single reader, spectrum regulation/usage, frequency error, coverage, etc.

- Small frequency shift

In order to implement the frequency division multiplexing of D2R in the device, a large frequency shift (FS) is introduced into the discussion. Based on observations from previous meetings, it has been noted that a large FS can enable D2R and D2R to transmit and receive on different bands, such as from FDD DL to FDD UL, with a frequency shift in the tens of MHz range. In scenarios where only a slight stagger in transmission frequency between D2R and R2D is desired through frequency shift, a small frequency shift (FS) can be employed. The use of small FS with low-frequency shifts tends to result in lower hardware power consumption, which is beneficial for supporting this feature in peak power-limited device 1. For the purpose of discussion, a frequency shift range of less than 10 MHz, limited to movement within either the uplink or downlink spectrum, can be referred to as small FS.

Proposal 8: Support for defining Small FS capable of shifting reflected signals less than 10MHz, intended for research purposes.

When a device supports frequency shift, it is imperative to ensure precise offset accuracy to avoid potential interference between the D2R of the device and other NR transmissions or the D2R of other devices. The small FS has lower power consumption requirements, which makes it possible for device1 to support this feature. However, due to limitations in peak power of device 1, there are challenges in supporting frequency shifts that guarantee high accuracy. Furthermore, it is imperative to provide a more comprehensive elucidation of the advantages associated with supporting device 1, particularly in relation to its capability of accommodating small FS. The clock speed of device 2a can support tens of MHz, enabling it to handle small LS due to its peak power reaching hundreds of µW. Besides, for device 2a, employing small FS compensates for the coverage limitations of large FS in terms of frequency shifts and enhances flexibility in frequency domain scheduling.

Observation 7: The small FS could be enabled to compensate for the frequency shift blind spot of large FS for device 2a.

Proposal 9: Further clarification is needed regarding the ability of device 1 to support small FS within peak power limits and the associated benefits of such support.

Proposal 10: Small FS could be support in device 2a for better scheduling flexibility of D2R.

- Large frequency shift

According to the observations made during the previous meeting, large FS can effectively prevent the reader from achieving in-band full duplex capability, thereby reducing the complexity associated with reader implementation. Furthermore, if a device supports frequency shifter, especially large FS, the reader can mitigate collisions in D2R transmission by implementing scheduling techniques. Moreover, the reader could have the capability to simultaneously receive D2R from different devices backscatter at varying frequencies, thereby enhancing overall inventory efficiency. Additionally, from a device perspective, it enables reduction of interference between R2D signals and backscatter D2R signals. As for the range of frequency shift, this can be hundreds of kH or tens of MHz, depending on the structure of the device implementation.

Observation 8: Large FS can reduce the complexity associated with reader implementation by preventing its in-band full duplex capability.

Observation 9: The support for frequency shifter in a device will significantly enhance the flexibility of reader scheduling, thereby improving spectrum utilization and inventory efficiency.

Proposal 11: RAN1 should support Large FS in device 2a for better spectrum utilization and inventory efficiency.

Reflection amplifier for Device 2a

In RAN1#117 meeting, the following observation on reflection amplifier was achieved:

Observation Reflection amplifier with following characteristics could be considered for device 2a. - Direction of amplification - Uni-directional reflection amplifier (baseline) can amplify backscattered signal in D2R which can improve D2R link budget. - Bi-directional amplifier can amplify both signal in R2D and backscatter signal in D2R at least when R2D and D2R are in the same spectrum. - Bi-directional amplifier has higher complexity, higher noise figure, and reduced isolation between tx and rx path. - Amplification gain ranges from 10 to 20dB. - Power consumption of reflection amplifier is in the range of a tens of uW to 100s of uW. - Reflection amplifier can operate in FDD frequency bands. - Reflection amplifier bandwidth can support 10s of MHz. - Note: reflection amplifier can get unstable when the input power exceeds a certain value, which may be frequency-dependent.

It is observed that bi-directional amplifier has higher complexity, higher noise figure and reduced isolation between Tx and Rx path, and it is also mentioned by some companies that the gain of bi-directional amplifier is lower than that of uni-directional amplifier. Furthermore, it may be suffered from performance loss when transitioning between R2D signal amplification and D2R signal amplification within a bi-directional amplifier. This issue can be mitigated by allowing plenty of time for switching process, however the hardware limitation of A-IoT device can have impact on the scheduling procedure, e.g., switching process may introduce additional latency. By considering the obvious drawbacks, bi-directional amplifier is not considered to be implemented by A-IoT devices. Whether to apply uni-directional amplifier on device 2a relies on the specific implementation by the manufacturer, as long as it ensures that the output peak power can satisfy the requirement.

Proposal 12: Considering the lack of significant benefits and potential disruptions to scheduling mechanisms, bi-directional amplifier is not supported by device 2a. Whether to support uni-directional amplifier by device 2a is up to implementation.

Wake up mechanism for devices

In the RAN1#118 meeting, the following proposal for device wake up mechanism was discussed.

FL Proposal 12: Device can wake up based following mechanisms Energy level-based: voltage level across capacitor can trigger device wake up Preamble-based: device BB logics can wake up when R2D preamble is detected. Timer-based: device can wake up when timer expires. Incident Rx power-based: device wakes up when incident rx power triggers devices wake up.

For preamble-based wake-up mechanism, additional wake-up indications need to be introduced by the preamble.  However, due to the proliferation of devices, even grouping instructions can result in significant overhead, thereby compromising the reliability of the preamble design itself.  In the case of timer-based wake-up mechanism, the accuracy of the clock plays a crucial role; a low-precision clock may cause failure in waking up at the designated time while opting for a high-precision clock often incurs substantial costs. Insufficient clock accuracy will cause the device and reader to have inconsistent understanding of the wake-up time, and thus miss the detection. Energy level-based and incident Rx power-based wake-up mechanisms can be preferred, which does not introduce additional protocol complexity. Furthermore, in terms of the device 1 and device 2a's functionality, it possesses the capability to harvest energy. This implies that when its own power level is depleted, the device becomes non-operational and can only function in an energy collection state. Consequently, it is justifiable to trigger devices wake up based on its power status or incident rx power.

Observation 10: The introduction of a wakeup mechanism based on the preamble would introduce additional complexity to the protocol and could potentially compromise the reliability of the preamble.

Observation 11: Insufficient clock accuracy will cause the device and reader to have inconsistent understanding of the wake-up time, and thus miss the detection.

Proposal 13:  Energy level-based and incident Rx power-based mechanisms should be prioritized for discussion, if needed.

Energy harvester and storage

- Power source

SA1 is exploring the potential of harvesting energy from various sources such as radio waves, light, motion, heat, or any other suitable power source. Each source requires a specific energy harvesting and storage module. For instance, if an A-IoT device utilizes light for energy source, an integrated photovoltaic circuit becomes essential. Conversely, for radio wave harvesting modules, components like matching networks, voltage multipliers and energy storage is required. While any power resource can be harvested for A-IoT devices, natural resources like light are susceptible to weather conditions and other factors leading to unstable supply. In contrast to light and other sources, radio waves offer relatively stable power supply which can be provided to A-IoT devices by gNBs or UEs.

Observation 12: Different energy harvesting and storages could be designed for different types of energy.

Observation 13: Compared with light and other power sources, radio wave is a relatively stable power source which can be provided to A-IoT passive device by gNB or UE or other nodes.

Proposal 14: Radio wave should be regarded as baseline power source for A-IoT passive device, while not precluding the exploration of alternative resources as potential power sources in the product.

- Charging time

According to existing studies, the efficiency of RF-based energy harvester varies with different input power levels. In [13~15], at input powers of -19.5 dBm, -11.1 dBm, and -1 dBm respectively, the 900MHz RF-based energy harvester module achieved efficiencies of 13%, 27%, and 49%. The input power of the energy harvester module in the actual process is influenced by factors such as device and charging device location, as well as the transmitting power of the charging device, making it challenging to quantitatively analyse the actual charging efficiency. To enable a quantitative evaluation of 900MHz device performance, assuming a starting point for discussion with a charging efficiency range of 10~50% would be appropriate.

Observation 14: The RF-based energy harvester module operates at the same frequency, and its operational efficiency is contingent upon the input power. Specifically, a higher input power corresponds to an increased efficacy in energy harvesting.

Proposal 15: Power conversion efficiency for RF-based energy harvester module at 900MHz could be assumed a starting point for discussion with a charging efficiency range of 10~50%.

The charging time is significantly influenced by factors such as power conversion efficiency (PCE), capacitance size, input RF power, and other variables. Previous observations have indicated that the PCE value ranges from 10% to 50% at a frequency of 900MHz. Table 1 presents the charging time values under different assumptions.

Table 1: Evaluation of charging time

| Capacitance (uF) | Max. Energy (uJ) | Input RF power (dBm) | PCE | Charging time (s) |
| --- | --- | --- | --- | --- |
| 1 | 0.5(Vmax=1V) | -30 | 10% | 5 |
| 1 | 0.5(Vmax=1V) | -30 | 30% | 1.67 |
| 1 | 0.5(Vmax=1V) | -30 | 50% | 1 |
| 1 | 0.5(Vmax=1V) | -23 | 10% | 1 |
| 1 | 0.5(Vmax=1V) | -23 | 30% | 0.33 |
| 1 | 0.5(Vmax=1V) | -23 | 50% | 0.2 |
| 1 | 1(Vmax=2V) | -30 | 10% | 10 |
| 1 | 1(Vmax=2V) | -30 | 30% | 3.33 |
| 1 | 1(Vmax=2V) | -30 | 50% | 2 |
| 10 | 5(Vmax=1V) | -30 | 10% | 50 |
| 10 | 5(Vmax=1V) | -30 | 30% | 16.67 |
| 10 | 5(Vmax=1V) | -30 | 50% | 10 |

Observation 15: Under varying assumptions, there exists a significant disparity in the charging time gap.

- RF EH sensitivity

In the RAN1#118 meeting, the following proposal for device wake up mechanism was discussed.

FL Proposal 15: - Following RF EH sensitivity values were reported as typical values. - [-35, -30]dBm, [-35, -30]dBm, -30dBm, [-20, -10]dBm

In RAN1#118, multiple candidate values for RF energy harvesting sensitivity are proposed. According to reference materials provided by various companies, the majority of energy collection sensitivities range from -30 to -35 dBm; however, some companies have observed a narrower range of -10 to -20 dBm [5]. Unlike communication, where the signal strength can be controlled, during actual operation the device will rely on any available wireless signal for charging purposes. However, in scenarios where no other RF signals are present in the working environment and the device needs to be charged solely through signals sent by the reader, it is crucial for the reader to adjust its RF signal based on both communication sensitivity and RF energy harvesting sensitivity differences. If the gap between communication sensitivity and RF energy harvesting sensitivity is large, it will bring difficulties to the transmission of the reader's RF signal.

Observation 16: If there exists a significant disparity between the communication sensitivity and the RF energy acquisition sensitivity, it may pose certain challenges in achieving successful transmission of the RF signal by the reader.

Proposal 16: The RF EH sensitivity discussion could be initiated with the reference points of -35 and -30 dBm.

Sustainable operation time

In this section, we present a concise analysis of device operation time. The primary factors influencing device operation time include capacitance capacity, PCE, charging time, input RF power, sleep power consumption, monitoring power consumption, communication power consumption, sleep duration ratio, monitoring duration ratio, and communication duration ratio. Sleep duration ratio, monitoring duration ratio, and communication duration ratio are , , and , respectively. And . Sustainable operation time T is equal to the sum of sleep duration , monitoring duration , and communication duration . As the device utilizes the same antenna for both charging and communication, it is imperative that the charging time does not overlap with the communication time. However, when the device is in a sleep state, there is no need for antenna usage in communication, thereby allowing its utilization for charging purposes. Consequently, it can be inferred that the duration required for charging equals that of sleep. Then we can get the formula related to T: . Input RF power is assumed as -30dbm. Sleep power consumption is assumed as 0.1uw, and monitoring power consumption is assumed as 1uw. Other assumptions can be found in Table 2.

Table 2: Evaluation of sustainable operation time

| Capacitance (uF) | Max. Energy (uJ) | PCE | Tx/Rx(uW) | α(%） | β(%） | γ (%） | Sustainable operation time T(ms) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.5(Vmax=1V) | 10% | 10 | 50% | 40% | 10% | 357 |
| 1 | 0.5(Vmax=1V) | 10% |  | 85% | 10% | 5 | 833 |
| 1 | 0.5(Vmax=1V) | 30% |  | 50% | 40% | 10% | 385 |
| 1 | 0.5(Vmax=1V) | 30% |  | 85% | 10% | 5 | 1163 |
| 1 | 0.5(Vmax=1V) | 50% |  | 50% | 40% | 10% | 417 |
| 1 | 0.5(Vmax=1V) | 50% |  | 85% | 10% | 5 | 1923 |
| 10 | 5(Vmax=1V) | 10% |  | 50% | 40% | 10% | 3571 |
| 10 | 5(Vmax=1V) | 10% |  | 85% | 10% | 5 | 8333 |
| 10 | 5(Vmax=1V) | 30% |  | 50% | 40% | 10% | 3846 |
| 10 | 5(Vmax=1V) | 30% |  | 85% | 10% | 5 | 11628 |
| 10 | 5(Vmax=1V) | 50% |  | 50% | 40% | 10% | 4167 |
| 10 | 5(Vmax=1V) | 50% |  | 85% | 10% | 5 | 19231 |
| 1 | 0.5(Vmax=1V) | 10% | 100 | 50% | 40% | 10% | 48 |
| 1 | 0.5(Vmax=1V) | 10% |  | 85% | 10% | 5 | 98 |
| 1 | 0.5(Vmax=1V) | 30% |  | 50% | 40% | 10% | 49 |
| 1 | 0.5(Vmax=1V) | 30% |  | 85% | 10% | 5 | 101 |
| 1 | 0.5(Vmax=1V) | 50% |  | 50% | 40% | 10% | 49 |
| 1 | 0.5(Vmax=1V) | 50% |  | 85% | 10% | 5 | 105 |
| 10 | 5(Vmax=1V) | 10% |  | 50% | 40% | 10% | 481 |
| 10 | 5(Vmax=1V) | 10% |  | 85% | 10% | 5% | 980 |
| 10 | 5(Vmax=1V) | 30% |  | 50% | 40% | 10% | 485 |
| 10 | 5(Vmax=1V) | 30% |  | 85% | 10% | 5% | 1014 |
| 10 | 5(Vmax=1V) | 50% |  | 50% | 40% | 10% | 490 |
| 10 | 5(Vmax=1V) | 50% |  | 85% | 10% | 5 | 1050 |

Under varying assumptions, the sustainable operation time can span from tens of milliseconds to tens of thousands of milliseconds. The key determinants influencing the duration of sustainable operation are the capacitor size and the proportion of sleep time within the overall sustainable operation time.

Observation 17: The proportion of sleep time within the overall sustainable operating period and capacitance capacity are two crucial determinants of sustainable operating duration.

Conclusion

In this contribution, initial views on ambient IoT device architectures were provided. Based on the discussion, the following observations and proposals are proposed:

Observation 1: At this stage, there is a lack of available conditions for assessing the implementation complexity and power consumption of the A-IoT device.

Observation 2: High implementation complexity often leads to increased product costs.

Observation 3: Reducing hardware design complexity can help mitigate the requirement for complex algorithms to some extent.

Observation 4: The power consumption is directly proportional to the clock speed, and when the clock speeds of different type devices are similar, their power consumption tends to be comparable.

Observation 5: The performance of device1 and device2a,2b in correcting clock accuracy differs due to the influence of peak power.

Observation 6: The power consumption of the clock used for small frequency shift shows no significant difference between device1 and device2a, as the range of small frequency shift is not substantially varied.

Observation 7: The small FS could be enabled to compensate for the frequency shift blind spot of large FS for device 2a.

Observation 8: Large FS can reduce the complexity associated with reader implementation by preventing its in-band full duplex capability.

Observation 9: The support for frequency shifter in a device will significantly enhance the flexibility of reader scheduling, thereby improving spectrum utilization and inventory efficiency.

Observation 10: The introduction of a wakeup mechanism based on the preamble would introduce additional complexity to the protocol and could potentially compromise the reliability of the preamble.

Observation 11: Insufficient clock accuracy will cause the device and reader to have inconsistent understanding of the wake-up time, and thus miss the detection.

Observation 12: Different energy harvesting and storages could be designed for different types of energy.

Observation 13: Compared with light and other power sources, radio wave is a relatively stable power source which can be provided to A-IoT passive device by gNB or UE or other nodes.

Observation 14: The RF-based energy harvester module operates at the same frequency, and its operational efficiency is contingent upon the input power. Specifically, a higher input power corresponds to an increased efficacy in energy harvesting.

Observation 15: Under varying assumptions, there exists a significant disparity in the charging time gap.

Observation 16: If there exists a significant disparity between the communication sensitivity and the RF energy acquisition sensitivity, it may pose certain challenges in achieving successful transmission of the RF signal by the reader.

Observation 17: The proportion of sleep time within the overall sustainable operating period and capacitance capacity are two crucial determinants of sustainable operating duration.

Proposal 1: The evaluation on complexity and power consumption should be addressed subsequent to further clarification of the technical proposal.

Proposal 2: RAN1 should focus on the discussion of hardware complexity for A-IoT device architectures, if need.

Proposal 3: A universally applicable method should be studied for evaluating complexity, if need.

Proposal 4: To evaluate power consumption, the most power hungry components the A-IoT device needs to be addressed in the discussion based on the blocks in architectures, and a universally applicable method should be studied, if need.

Proposal 5: The discussion on A-IoT device architecture should not impose any limitations on the implementation of the final product.

Proposal 6: For clock purpose #1, adopt following update.

Proposal 7: For clock purpose #2, adopt following update.

Proposal 8: Support for defining Small FS capable of shifting reflected signals less than 10MHz, intended for research purposes.

Proposal 9: Further clarification is needed regarding the ability of device 1 to support small FS within peak power limits and the associated benefits of such support.

Proposal 10: Small FS could be support in device 2a for better scheduling flexibility of D2R.

Proposal 11: RAN1 should support Large FS in device 2a for better spectrum utilization and inventory efficiency.

Proposal 12: Considering the lack of significant benefits and potential disruptions to scheduling mechanisms, bi-directional amplifier is not supported by device 2a. Whether to support uni-directional amplifier by device 2a is up to implementation.

Proposal 13:  Energy level-based and incident Rx power-based mechanisms should be prioritized for discussion, if needed.

Proposal 14: Radio wave should be regarded as baseline power source for A-IoT passive device, while not precluding the exploration of alternative resources as potential power sources in the product.

Proposal 15: Power conversion efficiency for RF-based energy harvester module at 900MHz could be assumed a starting point for discussion with a charging efficiency range of 10~50%

Proposal 16: The RF EH sensitivity discussion could be initiated with the reference points of -35 and -30 dBm.

Reference

- RP-240826, Revised WID: Study on solutions for Ambient IoT (Internet of Things) in NR, RAN#103, March 18-21, 2024.

- 3GPP TSG RAN WG1 #116 RAN1 Chair’s Notes, February 26th – March 1st, 2024.

- 3GPP TSG RAN WG1 #116-bis RAN1 Chair’s Notes, 15th-19th April, 2024.

- 3GPP TSG RAN WG1 #117 RAN1 Chair’s Notes, May 20th – 24th, 2024.

- 3GPP TSG RAN WG1 #118 RAN1 Chair’s Notes, August 19th – 23rd, 2024.

- 3GPP TSG RAN WG1 #118bis RAN1 Chair’s Notes, October 14th – 18th, 2024.

- Kamalinejad P, Keikhosravy K, Molavi R, et al. An ultra-low-power CMOS voltage-controlled ring oscillator for passive RFID tags[J]. IEEE, 2014.

- Cilek F, Seemann K, Brenk D, et al. Ultra-low power oscillator for UHF RFID transponder[J]. IEEE, 2008.

- Rosenthal, James, and M. S. Reynolds. "All-Digital Single Sideband (SSB) Bluetooth Low Energy (BLE) Backscatter with an Inductor-free, Digitally-Tuned Capacitance Modulator." 2020 IEEE/MTT-S International Microwave Symposium (IMS) IEEE, 2020.

- Appendix

Revised Rel-19 study item on A-IoT for NR

The following objectives are set, within the General Scope: - Evaluation assumptions - Conclude at least the following aspects of design targets left to WGs in Clause 5 (RAN design targets) of TR 38.848 [RAN1]. - Clause 5.3: Applicable maximum distance target values(s) - Clause 5.6: Refine the definition of latency suitable for use in RAN WGs - Clause 5.8: 2D distribution of devices - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations [RAN1, RAN4] - Identify basic blocks/components of possible Ambient IoT device architectures, taking into account state of the art implementations of low-power low-complexity devices which meet the RAN design target for power consumption and complexity. [RAN1] - Define link budget calculation for coverage, including whether/how to model carrier wave from node(s) inside or outside the connectivity topology. NOTE: Assessment performance of the design targets is within the study of feasibility and necessity of proposals in the following objectives, e.g. by inspection of reference implementations in the field, simulations, analytically. NOTE: strive to minimize evaluation cases in RAN1. - Study necessary and feasible solutions for Ambient IoT as prescribed in the General Scope, including decisions on which functions, procedures, etc. are needed and not needed, and ensuring at least the required functionalities in Section 6.2 of TR 38.848. Study of positioning in Rel-19 is RAN3-led, limited to functionalities which would have no, or minimal, specification impact (note: this does not imply any decision relating to WI creation). Study the feasibility and required functionalities for proximity determination, which is the determination of whether BS or intermediate UE and ambient IoT device are near each other or not (coordination with SA3 is required for privacy aspects). - RAN1-led: For the Ambient IoT DL and UL: - Frame structure, synchronization and timing, random access - Numerologies, bandwidths, and multiple access - Waveforms and modulations - Channel coding - Downlink channel/signal aspects - Uplink channel/signal aspects - Scheduling and timing relationships - Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. For Topology 2, no difference in physical layer design from Topology 1. - RAN2-led: - Study and decide which functions are needed for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission, and study those functions. For example: - Paging - Random access - Data transmission, including necessary radio resource control aspects, respecting the limitation in the General Scope - Interactions with upper layers For functionalities not listed above, they are studied only if found essential. - RAN3-led: - Identify necessary impacts on signaling and procedures for CN-RAN interface, to enable: - Paging - Device context management - Data transport - Identify RAN architecture aspects, including whether support for split architecture is necessary. - Identify potential solutions for locating an Ambient IoT device with no specification impact, e.g. reusing existing user location report, or minimal specification impact to convey location information to core network. - RAN4-led: - Coexistence study of Ambient IoT and NR/LTE. - RF requirements study for Ambient IoT: - Ambient IoT BS transmission and reception - Ambient IoT Device, as per the General Scope, transmission and reception - Intermediate node (UE), as per the General Scope, transmission and reception RAN2 and RAN3 are expected to identify RAN-CN functional split in coordination with SA2. Note: This study shall target for an IoT segment well below the existing 3GPP IoT technologies, e.g. NB-IoT, eMTC, RedCap, etc. The study shall not aim to replace existing 3GPP LPWA technologies.

Agreements in the RAN1#116 meeting

Agreement For the purpose of the study, RAN1 uses the following terminologies: - Device 1: ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - Device 2a: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - Device 2b: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission is generated internally by the device. Agreement Study at least the following blocks for device 1 architecture. - Antenna could be either shared or separate for RF energy harvester and receiver/transmitter. - Matching network is to match impedance between antenna and other components (including RF energy harvester and receiver related blocks). - RF energy harvester can include rectifier performing RF signal (AC) to DC conversion. - Energy storage (e.g., capacitor) stores harvested energy from RF energy harvester. - Power management unit (PMU) manages storing energy to energy storage from energy harvester and suppling power to active component blocks which needs power supply. - Digital BB logic includes functional blocks like encoder, decoder, controller, etc. - Memory can include two types of memory: 1) Non-Volatile Memory (NVM) such as EEPROM for permanently storing device ID, etc, and 2) registers for temporarily keeping any information required for its operation only while energy is available in energy storage. - Clock generator provides required clock signal(s). - Reception related blocks - RF BPF for improving selectivity. - Depending on implementation, it may not exist. RAN4 RF requirement (if any, e.g., ACS) and peak power consumption target also need to be considered. - RF Envelope Detector converts RF signal to baseband. - BB LPF can filter out harmonics and high frequency components to improve input signal quality to comparator. - Depending on implementation, it may not exist. Presence of BB LPF is assumed for the study. - Comparator determines high/low of input signal. - Transmission related blocks - Backscatter modulator switches impedance to modulate backscattered signal with tx signal from BB logics. Waveform/Modulation type is FFS. Agreement Study at least following blocks for device 2a architecture w/ RF-ED receiver. - Antenna could be either shared or separate for RF energy harvester (if present) and receiver/transmitter. - Matching network is to match impedance between antenna and other components (including RF energy harvester (if present) and receiver related blocks). - Energy harvester. - Energy storage (e.g., capacitor) stores harvested energy from energy harvester. - Power management unit (PMU) manages storing energy to energy storage from energy harvester and suppling power to active component blocks which needs power supply. - Digital BB logic includes functional blocks like encoder, decoder, controller, etc. - Memory can include two types of memory: 1) Non-Volatile Memory (NVM) such as EEPROM for permanently storing device ID, etc, and 2) registers for temporarily keeping any information required for its operation only while energy is available in energy storage. - Clock generator provides required clock signal(s). - Reflection amplifier can amplify reflected backscattered signal. - FFS study applicability of amplification of rx signal, power consumption. - At least one of R2D/CW2D and D2R could be amplified by either reflection amplifier or LNA. - Reception related blocks - RF BPF filter for improving selectivity. - Depending on implementation, it may not exist. RAN4 RF requirement (if any, e.g., ACS) and peak power consumption target also need to be considered. - FFS: LNA for improving signal strength and sensitivity of receiver. - At least one of R2D/CW2D and D2R could be amplified by either reflection amplifier or LNA. - RF envelope detector (RF-ED) detects envelope from RF signal. - BB amplifier amplifies BB signal to improve signal strength. - BB LPF can filter out harmonics and high frequency components to improve input signal quality to comparator/ADC. - Depending on implementation, it may not exist. - Comparator or N-bit ADC - Transmission related blocks - Backscatter modulator switches impedance to modulate backscattered signal with tx signal from BB logics. - FFS: Large Frequency shifter (e.g., tens of MHz) for shifting backscattered signal from one frequency (e.g., FDD-DL frequency) to another frequency (e.g., FDD-UL frequency).

Agreements in the RAN1#116-bis meeting

Agreement Study device 2b architecture w/ RF-ED receiver with following blocks. - Antenna could be either shared or separate for RF energy harvester (if present) and receiver/transmitter. - Matching network is to match impedance between antenna and other components (including RF energy harvester (if present) and receiver related blocks). - Energy harvester for harvesting energy from e.g., RF signal, solar, vibration/movement, temperature difference, etc - Energy storage (e.g., capacitor) stores harvested energy from energy harvester. - Power management unit (PMU) manages storing energy to energy storage from energy harvester and suppling power to active component blocks which needs power supply. - Digital BB logic includes functional blocks like encoder, decoder, controller, etc. - Memory can include two types of memory: 1) Non-Volatile Memory (NVM) such as EEPROM for permanently storing device ID, etc, and 2) registers for temporarily keeping any information required for its operation only while energy is available in energy storage. - Clock generator provides required clock signal(s). - Reception related blocks - RF BPF filter for improving selectivity. - Depending on implementation, it may not exist. RAN4 RF requirement (if any, e.g., ACS) and peak power consumption target also need to be considered. - FFS: LNA for improving signal strength and sensitivity of receiver, if present - RF envelope detector (RF-ED) detects envelope from RF signal. - BB amplifier amplifies BB signal to improve signal strength. - BB LPF can filter out harmonics and high frequency components to improve input signal quality to comparator/ADC. - Depending on implementation, it may not exist. - Comparator or N-bit ADC - Transmission related blocks - Tx Modulator: baseband bits are modulated according to modulation scheme. This block could be the part of BB logic. - Digital to Analog Converter (DAC) converts digital signal to analog signal. - Low pass filter for filtering out undesired signal - Mixer performs up converting baseband signal to RF range. - Local oscillator (LO) for carrier frequency generation - FFS: PLL/FLL - FFS: Power amplifier (PA) amplifies tx signal, if present - Details on transmitter related blocks depends on tx waveform/modulation. Agreement Further study reflection amplifier for Device 2a, considering following aspects: - Types of reflection amplifier - Uni-directional/one-way (for D2R) - Bi-directional/two-way (for both R2D and D2R) - FFS: switching loss (if applicable) - One-way Amplification Gain - E.g. [10, 15, 25] dB - Considering stability, operating frequency, and power consumption characteristics - Bandwidth Agreement Further study the feasibility of large frequency shift (large FS, i.e. between DL/UL spectrum of an FDD band) for device 2a considering at least following aspects. - Power consumption characteristics - Frequency shift range and granularity - Image suppression or SSB backscatter for large FS - IF carrier frequency accuracy - Harmonics suppression Note: the necessity (including applicable potential scenarios) of large FS can still be discussed in other agendas of the SI Agreement Study device 2b architecture with ZIF receiver with following blocks. - Antenna could be either shared or separate for RF energy harvester (if present) and receiver/transmitter. - Matching network is to match impedance between antenna and other components (including RF energy harvester (if present) and receiver related blocks). - Energy harvester for harvesting energy from e.g., RF signal, solar, vibration/movement, temperature difference, etc - Energy storage (e.g., capacitor) stores harvested energy from energy harvester. - Power management unit (PMU) manages storing energy to energy storage from energy harvester and suppling power to active component blocks which needs power supply. - Digital BB logic includes functional blocks like encoder, detector, decoder, controller, etc. - Memory can include two types of memory: 1) Non-Volatile Memory (NVM) such as EEPROM for permanently storing device ID, etc, and 2) registers for temporarily keeping any information required for its operation only while energy is available in energy storage. - Clock generator provides required clock signal(s). - Local oscillator (LO) for generating carrier frequency for Tx and Rx - FFS: PLL/FLL - FFS: one LO or separate LOs for Tx and Rx - Reception related blocks - RF BPF filter for improving selectivity. - Depending on implementation, it may not exist. RAN4 RF requirement (if any, e.g., ACS) and peak power consumption target also need to be considered. - FFS: LNA for improving signal strength and sensitivity of receiver. - Mixer down converts RF signal to BB stage. - Depending on implementation, there could be one or two mixers for Rx and Tx - BB amplifier amplifies BB signal - BB LPF can filter out undesired frequency components to improve input signal quality to comparator/ADC. - Depending on implementation, it may not exist. - Comparator or N-bit ADC - Transmission related blocks - Tx Modulator: baseband bits are modulated according to modulation scheme. This block could be the part of BB logic. - Digital to Analog Converter (DAC) converts digital signal to analog signal. - Low pass filter for filtering out undesired signal - Mixer performs up converting baseband signal to RF range. - FFS: Power amplifier (PA) amplifies tx signal, if present - Details on transmitter related blocks depends on e.g., waveform/modulation, etc Agreement Study device 2b architecture with IF-ED receiver with following blocks. - Antenna could be either shared or separate for RF energy harvester (if present) and receiver/transmitter. - Matching network is to match impedance between antenna and other components (including RF energy harvester (if present) and receiver related blocks). - Energy harvester for harvesting energy from e.g., RF signal, solar, vibration/movement, temperature difference, etc - Energy storage (e.g., capacitor) stores harvested energy from energy harvester. - Power management unit (PMU) manages storing energy to energy storage from energy harvester and suppling power to active component blocks which needs power supply. - Digital BB logic includes functional blocks like encoder, decoder, controller, etc. - Memory can include two types of memory: 1) Non-Volatile Memory (NVM) such as EEPROM for permanently storing device ID, etc, and 2) registers for temporarily keeping any information required for its operation only while energy is available in energy storage. - Clock generator provides required clock signal(s). - Local oscillator (LO) for generating carrier frequency for Tx, or for generating carrier frequency offset by the IF for Rx - FFS: PLL/FLL - FFS: one LO or separate LOs for Tx and Rx - Reception related blocks - RF BPF filter for improving selectivity. - Depending on implementation, it may not exist. RAN4 RF requirement (if any, e.g., ACS) and peak power consumption target also need to be considered. - FFS: LNA for improving signal strength and sensitivity of receiver, if present - Mixer down converts RF signal to IF stage. - Depending on implementation, there could be one or two mixers for Rx and Tx - IF amplifier amplifies IF signal - IF filter for filtering out unwanted RF and LO signals - IF envelope detector (IF-ED) detects envelope from IF signal. - BB amplifier - Depending on implementation, one or both of IF amplifier and BB amplifier may exist. - BB LPF can filter out harmonics and high frequency components to improve input signal quality to comparator/ADC. - Depending on implementation, it may not exist. - Comparator or N-bit ADC - Note: image rejection is required - Transmission related blocks - Tx Modulator: baseband bits are modulated according to modulation scheme. This block could be the part of BB logic. - Digital to Analog Converter (DAC) converts digital signal to analog signal. - Low pass filter for filtering out undesired signal - Mixer performs up converting baseband signal to RF range. - FFS: Power amplifier (PA) amplifies tx signal, if present - Details on transmitter related blocks depends on e.g., waveform/modulation, etc

Agreements and Observations in the RAN1#117 meeting

Observation

Reflection amplifier with following characteristics could be considered for device 2a.

- Direction of amplification

  - Uni-directional reflection amplifier (baseline) can amplify backscattered signal in D2R which can improve D2R link budget.

  - Bi-directional amplifier can amplify both signal in R2D and backscatter signal in D2R at least when R2D and D2R are in the same spectrum.

  - Bi-directional amplifier has higher complexity, higher noise figure, and reduced isolation between tx and rx path.

- Amplification gain ranges from 10 to 20dB.

- Power consumption of reflection amplifier is in the range of a tens of uW to 100s of uW.

- Reflection amplifier can operate in FDD frequency bands.

- Reflection amplifier bandwidth can support 10s of MHz.

- Note: reflection amplifier can get unstable when the input power exceeds a certain value, which may be frequency-dependent.

Observation

For large frequency shift:

- Large frequency shift can be used in shifting reflected signal in tens of MHz, e.g., from FDD DL to FDD UL frequency or vice versa.

- Large frequency shift consumes 10s of uW to 100s of uW.

- Large frequency shift is not feasible for device 1.

- Large frequency shift requires a clock for IF generation which is accurate enough to avoid large guard band and interference to adjacent channels/bands.

- Large frequency shift requires image suppression and may require harmonics suppression

  - Note: details of image suppression and harmonics suppression are not discussed in RAN1

- FFS: whether large frequency shift is necessary and feasible for device 2a

Agreement

For study purpose, assume that A-IoT device has a single antenna for both communication (tx/rx) and RF energy harvesting purposes.

Agreement

The following template is used for capturing descriptions on clock/LO.

|  | Purpose | Applicable device types | Clock speed | Power consumption | [Initial clock Accuracy] | [Accuracy after clock sync] | [Clock drift] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Purpose #1 of the clock |  |  |  |  |  |  |  |
| Purpose #N of the clock |  |  |  |  |  |  |  |
| Etc |  |  |  |  |  |  |  |

Agreements and Observations in the RAN1#118 meeting

Agreement

For device 2a with RF-ED, make following update.

- FFS: LNA for improving signal strength and sensitivity of receiver, if present.

  - At least one of R2D/CW2D and D2R could be amplified by either reflection amplifier or LNA.

For device 2b with RF-ED, make following update.

- FFS: LNA for improving signal strength and sensitivity of receiver, if present.

For device 2b with ZIF, make following update.

- FFS: LNA for improving signal strength and sensitivity of receiver, if present.

For device 2b with IF, make following update.

- FFS: LNA for improving signal strength and sensitivity of receiver, if present.

Agreement

For Device 2b with RF-ED/ZIF/IF, make following update.

- FFS: Power amplifier (PA) amplifies tx signal, if present

Agreement

For the architecture of Device 2b with RF-ED receiver, make following updates.

- Local oscillator (LO) for carrier frequency generation

  - FFS: PLL/FLL

  - FLL(/PLL) can be used for the frequency synthesis and calibration

    - FLL(/PLL) may not exist depending on architecture

For the architecture of Device 2b with ZIF receiver, make following update:

- Local oscillator (LO) for generating carrier frequency for Tx and Rx

  - FFS: PLL/FLL

  - FLL(/PLL) can be used for the frequency synthesis and calibration

  - FFS: o One LO or separate LOs for Tx and Rx.

For the architecture of Device 2b with ZIF receiver, make following update:

- Local oscillator (LO) for generating carrier frequency for Tx, or for generating carrier frequency offset by the IF for Rx generating carrier frequency for Tx and Rx

  - FFS: PLL/FLL

  - FLL(/PLL) can be used for the frequency synthesis and calibration

  - FFS: o One LO or separate LOs for Tx and Rx

Agreement

Adopt the following table for clock

|  | Description | Applicable device types | Clock speed | Power consumption | Initial clock accuracy | Accuracy after clock sync / calibration | Clock drift |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Purpose #1 of the clock | Sampling | Device 1, 2a, 2b | a few MHz | < [1]uW for device 1 FFS for device 2a/2b | [10^4 ~ 10^5] ppm for device 1 [10^3~ 10^4] ppm for device 2a/2b | FFS (if applicable for device 1) | FFS |
| Purpose #2 of the clock | Small frequency shift | At least for device 1 and device 2a |  |  |  |  |  |
| [Purpose #3 of the clock] | [Time counting (if supported)] | [Device 1, 2a, 2b] | FFS | FFS the same or different for different devices | FFS | FFS (if applicable) | FFS |
| Purpose #4 of the clock | Large frequency shift (if supported for device 2a) | Device 2a | 10s MHz | [10s] uW | FFS | FFS | FFS |
| Purpose #5 of the clock | LO for carrier frequency (for up/down conversion) | Device 2b | e.g., [900] MHz | [10s ~ 100s] uW | FFS | FFS | FFS |
| Note: It does not necessarily imply that different purposes of LOs/clocks correspond to separate discrete LOs/clocks, which is up to implementation. |  |  |  |  |  |  |  |

Agreement

Make the following update for device 2a diagram.

- FFS: Large Frequency shifter (e.g., tens of MHz) for shifting backscattered signal from one frequency (e.g., FDD-DL frequency) to another frequency (e.g., FDD-UL frequency).

- Note: this does not mean that RAN1 has concluded on the feasibility of Large Frequency shifter for device 2a

Agreement

Add following additional observations for large frequency shifter for device 2a.

- Large frequency shift may allow the reader to avoid implementing in-band full duplex capability for scenario e.g., D1T1-A2 and D2T2-A2

- Large frequency shift may result in [e.g., 5kHz~50kHz] of frequency uncertainty in target frequency for IF clock accuracy of [e.g., 0.01%~0.1%] assuming the large frequency shift range is 50MHz

Conclusion

RAN1 will not further discuss the following:

Adopt following diagram as example tx modulators. Provided diagrams are not intended to constrain actual implementation.

Following blocks could be used as an alternative tx architecture in device architecture diagrams.

- Device 1/2a with OOK/BPSK

  - Tx modulator is based on impedance switching between two states.

  - OOK/PSK can be realized by the choice of two impedance values.

- Device 1/2a with BFSK

  - Tx modulator is based on impedance switching between two states.

  - BFSK can be realized by the choice of IF frequency f1 and f2 based on baseband information bits.

- Device 2b with OOK

  - Baseband information bits control the switch between LO and output.

  - [Matching network may or may not exist.]

- Device 2b with BPSK

  - Baseband information bits select a phase of differential carrier frequency signal.

- Device 2b with BFSK

  - Baseband input signal directly controls the choice of carrier frequency f1 and f2 generated from LO.

Agreement

Companies are encouraged to provide receiver sensitivity numbers used in their evaluations with justification of feasibility to agenda 9.4.1.1, along with their evaluation assumptions.

Agreements and Observations in the RAN1#118bis meeting

Agreement

RAN1 deprioritize IF receiver for device 2a.

Agreement

RAN1 deprioritize ZIF receiver for device 2a.

Agreement

- Adopt following updated table format.

- One or more rows under a single purpose may exist to capture different options, if any.

Agreement

TR captures following descriptions for large frequency shift for device 2a.

- Large frequency shift may be technically feasible in terms of power consumption, and device complexity.

- Remaining technical feasibility of large frequency shift in terms of image suppression, harmonics suppression may be relevant to device emission requirement, spectrum usage. Additional complexity and power consumption may be needed to achieve image suppression and harmonics suppression.

- The necessity or effectiveness of large frequency shift may depend on spectrum for R2D/CW2D/D2R, full duplex capability of reader, whether device 1 and 2a are supported by a single reader, spectrum regulation/usage, frequency error, coverage, etc.

Agreement

- For clock purpose #3, adopt following update.

Agreement

- For clock purpose #5, adopt following update.

- Further study above options and strive to down select considering its implication on performance, system design, necessity, etc.

- Note: study of whether additional signal is necessary or not for calibration is discussed under agenda 9.4.2.2.

- Agreement

- For clock purpose #4, adopt following update.