3GPP TSG-RAN WG1 Meeting #119	R1-2409920

# Orlando, US, November 18th – 22nd, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | xxxx | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on TPMI determination for UL transmissions in 38.214 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-Core |  |  |  |  |  | Date: |  |  | 2024-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In current version of 38.214, the clarification on in which cases maxRankSDM, maxRankSDMmaxRankSDM-DCI-0-2, maxRankSFN or maxRankSFNmaxRankSFN-DCI-0-2 is applied is missed. - For codebook based PUSCH transmission scheduled by DCI format 0_2, the maximum transmission rank is configured by the higher layer parameter maxRankDCI-0-2. Therefore, when multipanelSchemeSDM or multipanelSchemeSFN is configured and two SRS resource sets are configured in srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook', for PUSCH transmissions scheduled by DCI format 0_2,if codepoint "00" or "01" of SRS Resource Set indicator is indicated, the first TPMI is used to indicate the precoder to be applied over layers {0…v-1}, where v ≤ maxRankDCI-0-2. - When multipanelSchemeSDM is configured and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook', and codepoint "00" or "01" of SRS Resource Set indicator is indicated, maxRank is defining the maximum number of layers applied over the first SRS resource set or the second SRS resource set, not “over the first SRS resource set or the second SRS resource”. - A “is” is missing in sentence “where v ≤ maxRankSFN or maxRankSFNmaxRankSFN-DCI-0-2 and maxRankSFN or maxRankSFNmaxRankSFN-DCI-0-2 defining the maximum number of layers applied over the first SRS resource set and over the second SRS resource set separately.” - In 38.331, maxRankSDM, maxRankSDM-DCI-0-2, maxRankSFN and maxRankSFN-DCI-0-2 are used to indicate the maximum transmission rank for each SRS resource set when SFN or SDM is configured. There is a misalignment for the parameters in 38.331 and 38. 214. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that to configuring the maximum transmission rank of each panel in SDM scheme, maxRankSDM is used for PUSCH scheduled with DCI format 0_1 or 0_3 and maxRankSDMmaxRankSDM-DCI-0-2 is used for PUSCH scheduled with DCI format 0_2, and to configuring the maximum transmission rank of each panel in SFN scheme, maxRankSFN is used for PUSCH scheduled with DCI format 0_1 or 0_3 and maxRankSFNmaxRankSFN-DCI-0-2 is used for PUSCH scheduled with DCI format 0_2. - Clarify that when multipanelSchemeSDM or multipanelSchemeSFN is configured and two SRS resource sets are configured in srs-ResourceSetToAddModListDCI-0-2, for PUSCH transmissions with codepoint "00" or "01" of SRS Resource Set indicator indicated, the first TPMI is used to indicate the precoder to be applied over layers {0…v-1}, where v ≤ maxRankDCI-0-2. - Add “set” in sentence “maxRank is defining the maximum number of layers applied over the first SRS resource set or the second SRS resource.” - Add “are” behind “maxRankSFNmaxRankSFN-DCI-0-2” in sentence “where v ≤ maxRankSFN or maxRankSFNmaxRankSFN-DCI-0-2 and maxRankSFN or maxRankSFNmaxRankSFN-DCI-0-2 defining the maximum number of layers applied over the first SRS resource set and over the second SRS resource set separately.” - Change maxRankSDM, maxRankSDMmaxRankSDM-DCI-0-2, maxRankSFN and maxRankSFNmaxRankSFN-DCI-0-2 to maxRankSDM, maxRankSDM-DCI-0-2, maxRankSFN and maxRankSFN-DCI-0-2 respectively. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - In which cases maxRankSDM, maxRankSDMmaxRankSDM-DCI-0-2, maxRankSFN or maxRankSFNmaxRankSFN-DCI-0-2 is applied is not clear. -The determination of TPMI is not correct for PUSCH transmissions scheduled by DCI format 0_2 and with codepoint "00" or "01" of SRS Resource Set indicator indicated when multipanelSchemeSDM or multipanelSchemeSFN is configured and two SRS resource sets are configured in srs-ResourceSetToAddModListDCI-0-2. And the description of the configuration of the maximum number of layer for both the first SRS resource set and the second SRS resource set for SDM and SFN schemes is not clear and aligned for SDM and SFN schemes. - UE’s behaviour is not correct when multipanelScheme set to 'SFNscheme' and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook', and codepoint "00" or "01" of SRS Resource Set indicator is indicated. - There is a misalignment for the parameters for indicating the maximum transmission rank for each SRS resource set when SFN or SDM is configured in 38.331 and 38. 214. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.1.1.1	Codebook based UL transmission

For codebook based transmission, PUSCH can be scheduled by DCI format 0_0, DCI format 0_1, DCI format 0_2, DCI format 0_3 or semi-statically configured to operate according to Clause 6.1.2.3. If this PUSCH is scheduled by DCI format 0_1, DCI format 0_2, or semi-statically configured to operate according to Clause 6.1.2.3, the UE determines its PUSCH transmission precoder(s) based on SRI(s), TPMI(s) and the transmission rank, where the SRI(s), TPMI(s) and the transmission rank are given by DCI fields of one or two SRS resource indicators and one or two Precoding information and number of layers in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2  or given by srs-ResourceIndicator and precodingAndNumberOfLayers according to clause 6.1.2.3 or given by srs-ResourceIndicator, srs-ResourceIndicator2, precodingAndNumberOfLayers, and precodingAndNumberOfLayers2 according to clause 6.1.2.3. If this PUSCH is scheduled by DCI format 0_3, the UE determines its PUSCH transmission precoder based on SRI, TPMI and the transmission rank, where the SRI, TPMI and the transmission rank are given by DCI fields of one SRS resource indicator and one Precoding information and number of layers in clause 7.3.1.1.4 of [5, TS 38.212] for DCI format 0_3. The SRS-ResourceSet(s) applicable for PUSCH scheduled by DCI format 0_1 and DCI format 0_2 are defined by the entries of the higher layer parameter srs-ResourceSetToAddModList and srs-ResourceSetToAddModListDCI-0-2 in SRS-config, respectively. Only one or two SRS resource sets can be configured in srs-ResourceSetToAddModList with higher layer parameter usage in SRS-ResourceSet set to 'codebook', and only one or two SRS resource sets can be configured in srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook'.

When only one SRS resource set is configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook', SRI and TPMI are given by the DCI fields of one SRS resource indicator and one Precoding information and number of layers in clauses 7.3.1.1.2, 7.3.1.1.3 and 7.3.1.1.4 of [5, TS 38.212] for DCI format 0_1, 0_2 and 0_3 or given by srs-ResourceIndicator and precodingAndNumberOfLayers according to clause 6.1.2.3. A UE does not expect two SRS resource sets are configured in srs-ResourceSetToAddModList with higher layer parameter usage in SRS-ResourceSet set to 'codebook' for a serving cell, when the serving cell is included in schedulingCellListDCI-0-3-r18 for a set of serving cells provided by mc-DCI-SetOfCellsToAddModList-r18. The TPMI is used to indicate the precoder to be applied over the layers {0…ν-1} and that corresponds to the SRS resource selected by the SRI when multiple SRS resources are configured, or if a single SRS resource is configured TPMI is used to indicate the precoder to be applied over the layers {0…ν-1} and that corresponds to the SRS resource. The transmission precoder is selected from the uplink codebook that has a number of antenna ports equal to higher layer parameter nrofSRS-Ports or nrofSRS-Ports-n8 in SRS-Config, as defined in Clause 6.3.1.5 of [4, TS 38.211]. When the UE is configured with the higher layer parameter txConfig set to 'codebook', the UE is configured with at least one SRS resource. The indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI.

When neither multipanelSchemeSDM nor multipanelSchemeSFN is configured and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook', one or two SRI(s), and one or two TPMI(s) are given by the DCI fields of two SRS resource indicator and two Precoding information and number of layers in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2.  The UE applies the indicated SRI(s) and TPMI(s) to one or more PUSCH repetitions according to the associated SRS resource set of a PUSCH repetition according to clause 6.1.2.1. Each TPMI, based on indicated codepoint of SRS Resource Set indicator, is used to indicate the precoder to be applied over the layers {0…ν-1} and that corresponds to the SRS resource selected by the corresponding SRI when multiple SRS resources are configured for the applicable SRS resource set, or if a single SRS resource is configured for the applicable SRS resource set TPMI is used to indicate the precoder to be applied over the layers {0…ν-1} and that corresponds to the SRS resource. For one or two TPMI(s), the transmission precoder is selected from the uplink codebook that has a number of antenna ports equal to the higher layer parameter nrofSRS-Ports in SRS-Config for the indicated SRI(s), as defined in Clause 6.3.1.5 of [4, TS 38.211]. When two SRIs are indicated, the UE shall expect the nrofSRS-Ports for the two indicated SRS resources to be the same. When the UE is configured with the higher layer parameter txConfig set to 'codebook', the UE is configured with at least one SRS resource. Each of the indicated one or two SRI(s) in slot n is associated with the most recent transmission of SRS resource of associated SRS resource set identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI. When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook', the UE is not expected to be configured with different number of SRS resources in the two SRS resource sets.

When the higher layer parameter multipanelScheme is set to 'SDMScheme' and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook', two SRI(s), and two TPMI(s) are given by the DCI fields of two SRS resource indicator and two Precoding information and number of layers in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2 or given by srs-ResourceIndicator, srs-ResourceIndicator2, precodingAndNumberOfLayers, and precodingAndNumberOfLayers2 in configuredGrantConfig:

-	When codepoint "10"  of SRS Resource Set indicator is indicated or when srs-ResourceIndicator2 and precodingAndNumberOfLayers2 are provided, the first TPMI is used to indicate the precoder to be applied over layers {0…v1-1}, where v1 is the number of layers indicated by the first TPMI, that corresponds to the SRS resource selected by the corresponding SRI when multiple SRS resources are configured for the applicable SRS resource set or if single SRS resource is configured for the applicable SRS resource set, and the second TPMI is used to indicate the precoder to be applied over layers {v1…. v2+v1-1}, where v2 is the number of layers indicated by the second TPMI, that corresponds to the SRS resource selected by the corresponding SRI when multiple SRS resources are configured for the applicable SRS resource set or if single SRS resource is configured for the applicable SRS resource set, v1 ≤ maxRankSDM or maxRankSDM-DCI-0-2 and v2 ≤ maxRankSDM or maxRankSDM-DCI-0-2 and maxRankSDM or maxRankSDM-DCI-0-2 is defining the maximum number of layers applied over the first and the second SRS resource sets, separately.

-	When codepoint "00" or "01" of SRS Resource Set indicator is indicated, the second SRI and second TPMI are reserved, the first TPMI is used to indicate the precoder to be applied over layers {0…v-1}, where v ≤ maxRank or maxRankDCI-0-2, where maxRank or maxRankDCI-0-2 is defining the maximum number of layers.

-	Codepoint "11" of SRS Resource Set indicator is reserved.

-	For one or two TPMI(s), the transmission precoder is selected from the uplink codebook that has a number of antenna ports equal to the higher layer parameter nrofSRS-Ports in SRS-Config for the indicated SRI(s), as defined in Clause 6.3.1.5 of [4, TS 38.211]. When two TPMIs are indicated, the UE shall expect that the precoder indicated by the first TPMI and the precoder indicated by the second TPMI are mapped to different PUSCH antenna ports.

-	When two SRIs are indicated, the UE shall expect that the number of SRS antenna ports associated with two indicated SRIs would be the same. When the UE is configured with the higher layer parameter txConfig set to 'codebook', the UE is configured with at least one SRS resource. Each of the indicated one or two SRI(s) in slot n is associated with the most recent transmission of SRS resource of associated SRS resource set identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI. When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook', the UE is not expected to be configured with different number of SRS resources in the two SRS resource sets.

When higher layer parameter multipanelScheme set to 'SFNscheme' and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook', two SRI(s), and two TPMI(s) are given by the DCI fields of two SRS resource indicator and two Precoding information and number of layers in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2 or given by srs-ResourceIndicator, srs-ResourceIndicator2, precodingAndNumberOfLayers, and precodingAndNumberOfLayers2 in configuredGrantConfig:

-	When  codepoint "10" of SRS Resource Set indicator is indicated or when srs-ResourceIndicator2 and precodingAndNumberOfLayers2 are provided, the first TPMI is used to indicate precoder to be applied over layers {0…v-1} and the second TPMI is used to indicate the precoder to be applied over layers {0…v-1}, where  v ≤ maxRankSFN or maxRankSFN-DCI-0-2 and maxRankSFN or maxRankSFN-DCI-0-2 is defining the maximum number of layers applied over the first SRS resource set and over the second SRS resource set separately.

-	When codepoint "00" or "01" of SRS Resource Set indicator is indicated, the second SRI and second TPMI are reserved, the first TPMI is used to indicate precoder to be applied over layers {0…v-1}, where v ≤ maxRank or maxRankDCI-0-2, where maxRank or maxRankDCI-0-2 is defining the maximum number of layers applied over the first SRS resource set or the second SRS resource set.

-	Codepoint "11" of SRS Resource Set indicator is reserved.

-	For one or two TPMI(s), the transmission precoder is selected from the uplink codebook that has a number of antenna ports equal to nrofSRS-Ports in SRS-Config for the indicated SRI(s), as defined in Clause 6.3.1.5 of [4, TS 38.211]. When two TPMIs are indicated, the UE shall expect that the precoder indicated by the first TPMI and the precoder indicated by the second TPMI are mapped to different PUSCH antenna ports.

-	When two TPMIs are indicated, the UE shall expect that the number of SRS antenna ports associated with two indicated SRIs to be the same. When the UE is configured with the higher layer parameter txConfig set to 'codebook', the UE is configured with at least one SRS resource. Each of the indicated one or two SRI(s) in slot n is associated with the most recent transmission of SRS resource of associated SRS resource set identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI. When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook', the UE is not expected to be configured with different number of SRS resources in the two SRS resource sets.

When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the purpose of determining the most recent transmission of SRS resource identified by the SRI, the PDCCH candidate that starts earlier in time is used.

For codebook based transmission with two or four antenna ports, the UE determines its codebook subsets based on TPMI(s) and upon the reception of higher layer parameter codebookSubset in pusch-Config for PUSCH associated with DCI format 0_1 or 0_3 and codebookSubsetDCI-0-2 in pusch-Config for PUSCH associated with DCI format 0_2 which may be configured with 'fullyAndPartialAndNonCoherent', or 'partialAndNonCoherent', or 'nonCoherent' depending on the UE capability for two or four antenna ports.

For codebook based transmission with eight antenna ports, the UE determines its codebook based upon the reception of higher layer parameter[s] CodebookTypeUL in pusch-Config for PUSCH associated with DCI format 0_1 and 0_2, depending on the UE capability. According to the configured CodebookTypeUL, coherent UL MIMO operation applies within antenna port groups as defined in Table 6.3.1.5-8 of [4, TS 38.211]. According to the configured CodebookType, requirements for coherent UL MIMO in clause 6.4D.4 of [8, TS 38.101-1] and [21, TS 38.101-2] apply within an antenna port group.

When higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2' and the higher layer parameter codebookSubset or the higher layer parameter codebookSubsetDCI-0-2 is set to 'partialAndNonCoherent', and when the SRS-resourceSet with usage set to "codebook" includes at least one SRS resource with 4 ports and one SRS resource with 2 ports, the codebookSubset associated with the 2-port SRS resource is 'nonCoherent'.

When higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2' and the higher layer parameter CodebookTypeUL is set to 'codebook2' or 'codebook3', and the SRS-resourceSet with usage set to 'codebook' includes one SRS resource with 8 ports, and at least one SRS resource with 2 ports or 4 ports, subject to UE capability,

-	when CodebookTypeUL is set to 'codebook2', the codebookSubset associated with the 2-port SRS resource is 'nonCoherent'.

-	when CodebookTypeUL is set to 'codebook2', the codebookSubset associated with the 4-port SRS resource can be configured as 'partialAndNonCoherent' or 'nonCoherent', subject to UE capability.

-	when CodebookTypeUL is set to 'codebook3', the codebookSubset associated with the 4-port SRS resources is 'nonCoherent'.

The maximum transmission rank may be configured by the higher layer parameter maxRank in pusch-Config for PUSCH scheduled with DCI format 0_1 or 0_3 and maxRankDCI-0-2 for PUSCH scheduled with DCI format 0_2.

When the higher layer parameter multipanelSchemeSDM is configured in pusch-Config, the maximum transmission rank for each SRS resource set in srs-ResourceSetToAddModList with higher layer parameter usage in SRS-ResourceSet set to 'codebook' may be configured by the higher layer parameter maxRankSDM for PUSCH scheduled with DCI format 0_1 or 0_3 and the maximum transmission rank for each SRS resource set in srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook' may be configured by the higher layer parameter maxRankSDM-DCI-0-2 for PUSCH scheduled with DCI format 0_2.

When the higher layer parameter multipanelSchemeSFN is configured in pusch-Config, the maximum transmission rank for each SRS resource set in srs-ResourceSetToAddModList with higher layer parameter usage in SRS-ResourceSet set to 'codebook' may be configured by the higher layer parameter maxRankSFN in pusch-Config for PUSCH scheduled with DCI format 0_1 or 0_3 and the maximum transmission rank for each SRS resource set in srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook' may be configured by the higher layer parameter maxRankSFN-DCI-0-2 for PUSCH scheduled with DCI format 0_2.

A UE reporting its UE capability of 'partialAndNonCoherent' transmission shall not expect to be configured by either codebookSubset or codebookSubsetDCI-0-2 with 'fullyAndPartialAndNonCoherent' for two or four antenna ports.

A UE reporting its UE capability of 'nonCoherent' transmission shall not expect to be configured by either codebookSubset or codebookSubsetDCI-0-2 with 'fullyAndPartialAndNonCoherent' or with 'partialAndNonCoherent' for two or four antenna ports.

A UE does not expect to be configured with a value of CodebookTypeUL that does not correspond to one of the values reported in its capability for codebook based transmission with eight antenna ports.

A UE shall not expect to be configured with the higher layer parameter codebookSubset or the higher layer parameter codebookSubsetDCI-0-2 set to 'partialAndNonCoherent' when higher layer parameter nrofSRS-Ports in an SRS-ResourceSet with usage set to 'codebook' indicates that the maximum number of the configured SRS antenna ports in the SRS-ResourceSet is two.

For codebook based transmission, only one SRS resource can be indicated based on the SRI from within the SRS resource set. Except when higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2', the maximum number of configured SRS resources for codebook based transmission is 2. If aperiodic SRS is configured for a UE, the SRS request field in DCI triggers the transmission of aperiodic SRS resources.

A UE shall not expect to be configured with higher layer parameter ul-FullPowerTransmission set to 'fullpowerMode1' and codebookSubset or codebookSubsetDCI-0-2 set to 'fullAndPartialAndNonCoherent' simultaneously.

A UE shall not expect to be configured with higher layer parameter ul-FullPowerTransmission set to 'fullpowerMode1' and CodebookTypeUL set to 'codebook1' simultaneously.

The UE shall transmit PUSCH using the same antenna port(s) as the SRS port(s) in the SRS resource(s) indicated by the DCI format 0_1, 0_2 or 0_3 or by configuredGrantConfig according to clause 6.1.2.3.

The DM-RS antenna ports  in Clause 6.4.1.1.3 of [4, TS38.211] are determined according to the ordering of DM-RS port(s) given by Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 in Clause 7.3.1.1.2 of [5, TS 38.212].

Except when higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2', when multiple SRS resources are configured by SRS-ResourceSet with usage set to 'codebook', the UE shall expect that a same higher layer parameter nrofSRS-Ports or nrofSRS-Ports-n8 is configured in SRS-Resource in SRS-ResourceSet, and the higher layer parameter nrofSRS-Ports or nrofSRS-Ports-n8 shall be configured with the same value for all these SRS resources.

When higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2',

-	the UE can be configured with one SRS resource or multiple SRS resources with same or different number of SRS ports within an SRS resource set with usage set to 'codebook'.

-	up to 2 different spatial relations can be configured for all SRS resources in the SRS resource set with usage set to 'codebook' when multiple SRS resources are configured in the SRS resource set.

-	subject to UE capability, a maximum of 2 or 4 SRS resources are supported in an SRS resource set with usage set to 'codebook'.