3GPP TSG RAN WG1 #119		                                                      R1-2409934

Orlando, US, November 18th – 22nd, 2024

Source:	CATT

Title:	Views on NR MIMO CSI enhancements Phase 5

Agenda Item:	9.2.2

Document for:	Discussion and Decision

- Introduction

In RAN #105 meeting, in addition to already endorsed objectives on specifying UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul as well as specifying CSI support for up to 128 CSI-RS ports, a new item of specifying SRS port grouping was approved in the WID for Rel-19 NR MIMO[1]. The current work item objectives are:

Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI

In RAN1#118b meeting, regarding UE reporting enhancement for CJT, several remaining details on delay offset calibration and subband phase offset calibration have been discussed. For CSI-RS for up to 128 ports, SRS port grouping and remaining details on Type I 3-bit soft scaling and multi-CRI-based CSI reporting UCI design have been agreed.

In this contribution, for UE reporting enhancement for CJT, remaining details on phase offset reporting and the linkage between CJTC reporting and CJT reporting are discussed. For CSI support for up to 128 CSI-RS ports, our views on remaining details on Type I codebook soft scaling as well as possible extension of agreed Rel-19 type I SP codebook schemes to legacy (inferior to 32) ports are provided.

- UE reporting enhancement for CJT

  - Phase offset measurement and reporting

Phase offset reporting

In RAN1#118bis meeting, the following agreement on associated SRS for reciprocity calibration was achieved, and some remaining issues need further study.

Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the configured associated SRS resource, - Confirm the following working assumption as agreement: “the configured associated SRS resource can be either periodic, semi-persistent, or aperiodic” - When periodic or semi-persistent associated SRS resource is configured, the SRS transmission occasion for determining the reference UE antenna port corresponds to the latest SRS transmission occasion before the occasions of the NTRP CSI-RS resources used for measuring ‘cjtc-P’ report - FFS: Whether ‘the earliest SRS transmission occasion after the NTRP CSI-RS occasions’ is also supported as an option - FFS: Whether determination of SRS transmission occasion is needed for aperiodic associated SRS resource, and if so, how

In RAN1#118bis meeting, it was agreed that the latest SRS transmission occasion before the occasions of the NTRP CSI-RS resources is used for determining the reference UE antenna port. One issue is whether ‘the earliest SRS transmission occasion after the NTRP CSI-RS occasions’ is also supported as an option. Actually, the SRS transmission occasion which is after NTRP non-precoded CSI-RS resources can also work for reciprocity calibration. If only the SRS transmission occasion before the CSI-RS resources can be used, the scheduling flexibility of the network will be restricted. Considering that non-precoded CSI-RS is an important use case in reciprocity calibration, it is preferred that the earliest SRS transmission occasion after the NTRP CSI-RS occasions is also supported. Whether the SRS transmission occasion before or after NTRP CSI-RS resources is used for determining the reference UE antenna port depends on network configuration.

Proposal 1: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the configured associated periodic/semi-persistent SRS resource, the earliest SRS transmission occasion after the NTRP CSI-RS occasions is also supported as an option.

- Whether the SRS transmission occasion before or after NTRP CSI-RS resources is used for determining the reference UE antenna port depends on network configuration.

Aperiodic associated SRS resource can be triggered many times, and there might be multiple transmission occasions. In this case, the transmission occasion utilized to determine the reference UE antenna port is unclear. Therefore, the determination of SRS transmission occasion is also needed for aperiodic associated SRS resource. Similar to periodic or semi-persistent associated SRS resource, the transmission occasion is the latest SRS transmission occasion before the NTRP CSI-RS occasions or the earliest SRS transmission occasion after the NTRP CSI-RS occasions.

Proposal 2: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), for associated aperiodic SRS, the transmission occasion is the latest SRS transmission occasion before the NTRP CSI-RS occasions or the earliest SRS transmission occasion after the NTRP CSI-RS occasions.

It can be seen that SRS transmission occasion is determined depending on NTRP CSI-RS resources. If there are multiple CSI-RS transmission occasions for P/SP-CSI, which NTRP CSI-RS resources are used to determine SRS transmission occasion is unclear. A predefined rule is needed to ensure that the network and the UE have the same understanding. Similar to time restricted measurement, the most recent occasion of CSI-RS resources, no later than the CSI reference resource associated with the ‘cjtc-P’ reporting can be selected to determine SRS transmission occasion.

Proposal 3: For the Rel-19 aperiodic standalone CJT calibration reporting, when P/SP-CSI-RSs are used as CMR and ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), the most recent occasion of NTRP P/SP CSI-RS resources, no later than the CSI reference resource associated with the ‘cjtc-P’ reporting can be used to determine SRS transmission occasion.

  - Delay offset/frequency offset measurement and reporting

PMI calculation for delay offset measurement and reporting

In RAN1#118bis meeting, the following agreements and conclusions were made on time offset measurement and reporting:

Agreement For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured, confirm the following working assumptions as agreement with the following refinement: - When separately triggered, the delay offset value to be compensated is the latest reported delay offset (DO) whose reporting instance’s last symbol is before the first symbol of DCI triggering of the CJT CSI reporting - When jointly triggered, the delay offset value to be compensated is the reported delay offset (DO) in the same reporting instance FFS: Whether an additional UE procedure is needed when the reported DO value is ‘out of range’ FFS: Whether the Dd report codepoints need to be reinterpreted from intervals/ranges to values when the linkage mechanism is configured, or in general, a rule is needed to determine the value of DO for the compensation for each CMR for CJT CSI report (including if multiple Dos are reported) Agreement For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with two separate triggers, to indicate whether or not the UE should perform delay offset (DO) compensation based on the latest linked CJTC Dd report when calculating the Rel-18 Type-II CJT CSI, introduce a 1-bit indicator per CSI trigger state: - The 1-bit indicator is an RRC parameter - This is done without increasing the bit-width of the CSI request field of the DCI triggering a Rel-18 CJT eType-II CJT CSI report - FFS (RAN1#119): Whether the 1-bit indicator applies to all the NTRP CSI-RS resources, or to 1-bit indicator per CSI-RS resource - FFS (RAN1#119): How this applies to a single CSI trigger state associated with >1 CSI reports Agreement For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured, support linking the CMRs in the two CSI Report Settings so that UE knows which CMRs in the two report settings correspond to the same TRP. - Based on a fixed correspondence between the set of TRS resource set IDs in sequential order and the set of CSI-RS resource IDs in sequential order of configuration in their respective Resource Setting FFS: linking, when the number of resources configured for CJT CSI is < number of resource sets configured for CJT Dd, in case of separate triggers RRC. Agreement For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with a joint trigger: - Reuse the CPU occupation and active resource counting for the Rel-18 eType-II CJT - FFS (RAN1#119): Re timeline, decide whether to reuse or further relax the timeline for the Rel-18 eType-II CJT

Remaining issues are details of Dd linkage with separate triggering and Dd linkage with joint triggering.

When the reported DO value is ‘out of range’, it might be caused by either the measured delay offset is too large or the received power of the corresponding TRS is low. An additional UE procedure for a fixed value compensation for CJT CSI report cannot solve the misalignment. In this case, it can be solved by network implement, such as triggering another CJTC Dd and Rel-18 eType-II CJT CSI reports for the same TRPs or other TRPs. Therefore, an additional UE procedure is unnecessary when the reported DO value is ‘out of range’.

Proposal 4: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-Dd’, an additional UE procedure for a fixed value compensation for CJT CSI report is unnecessary when the reported DO value is ‘out of range’.

When the linkage mechanism is configured, discussion on whether the Dd report codepoints should be reinterpreted from intervals/ranges to values is needed. Without such reinterpretation, the maximum misalignment of the UE and the network, according to the adopted configuration parameters, is dozens of nanoseconds. There is no evidence that the remaining time misalignment will lead to significant performance loss for FDD transmission. Therefore, it is unnecessary to reinterpret the Dd report codepoints from intervals/ranges to values.

Proposal 5: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-Dd’, it is unnecessary to reinterpret the Dd report codepoints from intervals/ranges to values.

For Dd linkage with separate triggering, a 1-bit indicator per CSI trigger state has been introduced to indicate whether or not the UE should perform DO compensation based on the latest linked CJTC Dd report when calculating the Rel-18 Type-II CJT CSI. One of the remaining issues is whether the 1-bit indicator is resource-common or per CSI-RS resource. 1-bit indicator per CSI-RS resource can facilitate DO compensation for only a subset of TRPs. However, since CJTC Dd report is used for delay offsets compensation among TRPs to assist CJT transmission, the motivation for compensation only at a subset of TRPs is not clear. Besides, facilitating DO compensation for only a subset of TRPs may cause that the agreed QCL assumptions for PDSCH for a UE indicated with two TCI states in Rel-19 are not satisfied. As a consequence, the performance of PDSCH may be affected. Thus, the 1-bit indicator should be applies to all the NTRP CSI-RS resources.

Proposal 6: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-Dd’, the 1-bit indicator is applied to all the NTRP CSI-RS resources.

Another issue is how the 1-bit indicator applies to a single CSI trigger state associated with >1 CSI reports. We already have the agreement that  the linked CJTC Dd and Rel-18 eType-II CJT CSI reports are configured based on a fixed correspondence between the set of TRS resource set IDs and the set of CSI-RS resource IDs in sequential order in their respective resource setting. If the 1-bit indicator applies to a single CSI trigger state associated with >1 CSI reports, these CJT CSI reports will be compensated with the same DO, the motivation of NR>1 reports associated with the same trigger state is to save some DCI triggers. Thus, when the 1-bit indicator is used, the 1-bit indicator per CSI trigger state applies to all the NR reports.

Proposal 7: For UE delay offset measurement and reporting enhancement for CJT, when a single CSI trigger state associated with NR >1 CSI reports, the 1-bit indicator per CSI trigger state is applied to all the NR reports.

In joint triggering, the number of resources configured for CJT CSI should be the same as the number of resource sets configured for CJT Dd reporting, the agreement in RAN#118b on fixed correspondence between CJTC CMR and CJT CSI CMR can guarantee which CMRs in the two report settings correspond to the same TRP. In separate triggering, it is argued that if the reported DO value is ‘out of range’, CSI-RS resource for corresponding CJT CSI report should not be configured because generally the corresponding TRP will not be selected for CJT transmission. In such case, the number of resources configured for CJT CSI report is less than the number of resource sets configured for CJT Dd report and the agreed fixed correspondence between CJTC CMR and CJT CSI CMR is not operational. However, RRC reconfiguration is needed to cancel the configured CSI-RS resource(s) for TRP(s) with ‘out of range’ DO values and a new mapping between the two CMRs should be introduced when the cardinality of the two CMRs are unequal. To simplify the design, we prefer that the number of resources configured for CJT CSI is restricted to be the same as the number of resource sets configured for CJT Dd in case of separate triggering.

Proposal 8: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-Dd’, the number of resources configured for CJT CSI report is restricted to be the same as number of resource sets configured for CJT Dd report in case of separate triggering.

PMI calculation for frequency offset measurement and reporting

Remaining issue is whether support configuring a UE (via RRC signaling) to perform PMI calculation for the Rel-18 eType-II CJT CSI report assuming pre-compensation using the UE-reported frequency offset.

Different from the frequency-selective specific phase rotations caused by time misalignment, PMI can be calculated by the channels of multiple TRPs in one time instant. Even if TRSs of multiple TRPs are transmitted in different slots, the effect of the time-varied channels can be compensated by the network in the PDSCH/DMRS transmission, and special handling of the UE is not needed in PMI calculation.

Since it is unnecessary to align the frequency offset values for the transceiver compensation, there is no need to establish the linkage between the frequency offset reporting and the CJT CSI reporting.

Proposal 9: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-F’, there is no need to establish the linkage between the frequency offset reporting and the CJT CSI reporting.

Timeline for joint triggered CJTC and CJT reporting

For Dd linkage with joint triggering, our first preference is to reuse the timeline for the Rel-18 eType-II CJT as a baseline. However, if the need is justified, joint trigger timeline can also be relaxed. Considering the fact that UEs may process the CJT Dd calibrationand CJT eType II CSI in sequential order when CJTC and CJT CSI reporting are jointly triggered, the timeline can be determined as Z/Z’+ Drelax . Z/Z’ is associated with the Rel-18 eType-II CJT and the value of Drelax is a UE capability, taken from {0, drelax}. The drelax value can be FFS.

Proposal 10: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-Dd’, the timeline (Z/Z’) is determined as Z/Z’ associated with the Rel-18 eType-II CJT, plus Drelax. The value of Drelax is a UE capability, taken from {0, drelax}. drelax can be FFS.

- Discussion on CSI support for up to 128 CSI-RS ports

  - Port mapping for non-codebook based transmission

In RAN1 #117 meeting, the following agreement on the association between SRS resource set(s) and a CSI-RS resource set was achieved.

Agreement On the NZP CSI-RS resource aggregation of K=2, 3 or 4 legacy NZP CSI-RS resources to attain a total of 48, 64, and 128 ports (for Rel-19 Type-I/II codebook refinement), support to configure a CSI-RS resource set with the K CSI-RS resources as the associated NZP CSI-RS for each of the SRS resource set(s) with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook', - The previously agreed restrictions on the K resources for Rel-19 Type-I/II codebook refinement apply - Reuse the legacy approach for triggering of the NZP-CSI-RS resources and the legacy timeline for the NZP-CSI-RS resources and SRS

In Rel-18 non-codebook based PUSCH transmission, UE can calculate precoder based on the CSI-RS resource which is associated with the SRS resource set(s). When a CSI-RS resource set with K CSI-RS resources is used to calculate precoder, the port mapping of these CSI-RS resources and the mapping from CSI-RS resource index/port index per resource and port index to precoder calculation shall be defined. Otherwise, the precoder or the DL channel information derived by UE based on an arbitrary port indexing assumption and an arbitrary mapping order may not be matched with the realistic UL channel information. Correspondingly, the uplink transmission performance cannot be guaranteed.

Regarding port mapping of CSI-RS resources, port mapping method defined for Rel-19 Type-I and Type-II codebook refinement can be reused directly.

In order to determine the mapping from CSI-RS resource to precoder calculation, the following options can be considered:

- Option 1: Mapping method 1 defined for Rel-19 Type-I and Type-II codebook refinement.

- Option 2: Mapping method 2 defined for Rel-19 Type-I and Type-II codebook refinement.

- Option 3: Other simple mapping methods.

Option 1 and Option 2 have already been supported by Rel-19 Type-I and Type-II codebook refinement. However, mapping method related higher layer parameter portMappingMethod-r19 is configured in CSI-ReportConfig, which cannot be used for the associated CSI-RS resource set directly.

In option 3, a simple mapping method can be defined. For example, antenna ports of CSI-RS resources are ordered sequentially. Taking four CSI-RS resources as an example, in precoder calculation, the mapping method is (1st resource, all K ports), (2nd resource, all K ports), (3rd resource, all K ports), (4th resource, all K ports).

Actually, all these mapping methods can work. In order to minimize standardization impact, mapping method 1 or mapping method 2 defined for Rel-19 Type-I and Type-II codebook refinement is preferred. Mapping method 1 or mapping method 2 can either be configured for the UE or be predefined for the UE.

Proposal 11: For CSI enhancement for 48, 64, and 128 CSI-RS ports, port mapping (indexing) method defined for Rel-19 Type-I and Type-II codebook refinement can be used for CSI-RS resource set which is associated with SRS resource set(s) in non-codebook based PUSCH transmission.

Proposal 12: For CSI enhancement for 48, 64, and 128 CSI-RS ports, mapping method 1 or mapping method 2 defined for Rel-19 Type-I and Type-II codebook refinement can be used for CSI-RS resource set which is associated with SRS resource set(s) in non-codebook based PUSCH transmission.

- Either mapping method 1 or mapping method 2 can be configured or predefined for the UE.

  - Potential extension of Rel-19 Type-I SP codebook schemes to legacy port numbers

In RAN1#118b meeting, the extension of agreed scheme A and B in Rel-19 Type I SP codebook to the legacy number of CSI-RS ports for all applicable RI values has been discussed. The agreed scheme A and scheme B are low complexity and high performance codebooks for 32-128 ports, respectively. Our view is that potential extension of scheme A and scheme B to the legacy number should be discussed separately.

Regarding the extension of scheme A, Rel-15 Type I SP codebook has already specified the codebook for legacy port number (i.e., ≤32 ports). It is a low complexity codebook with very similar design principle as scheme A. Many simulation results have demonstrated that there is only marginal gain to extend scheme A to legacy port number [2]. Apart from the negligible performance gain, introducing scheme A which is too similar to the Rel-15 codebook may cause unnecessary burden for UE in terms of implementation and IODT. In addition, too many codebook types are not good for commercialization and can also cause confusion for the market.

Regarding the extension of scheme B, simulation results have revealed that it can bring promising performance gain compared with Rel-15 Type I SP codebook. Introducing scheme B for legacy port number can be seen as a complementary high performance codebook design in addition to the already specified low complexity Rel-15 Type I SP codebook.  Such extension renders a uniform codebook design for 1-128 CSI-RS ports, with both low complexity and high performance codebook to be supported. Therefore, extension of scheme B to legacy port number in Type I SP codebook can be considered.

Proposal 13: For potential extensions of Rel-19 Type I SP codebook schemes to legacy port numbers (i.e., ≤32 ports), do not support extension of scheme A.

  - Rel-19 Type-I SP codebook soft scaling for RI>1

In RAN1#118bis meeting, the following agreements on 3-bit scaling factors for RI=v>1 were achieved. How to determine scaling factor for RI=v=2 shall be studied.

Agreement For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding the support for the 3-bit scaling factor(s) for RI=v >1, support only for RI=v=2 without per-SD-basis-vector/layer power adjustment/boosting - FFS: Details on per-layer scaling factor applied to each of the selected SD basis vectors, extending the agreed scaling factor for RI= v =1 si∈1, 12, 13 ,14,16,18,112,116 (in RAN1#117) This feature is a separate UE capability from soft scaling for RI=v=1. Introduce new RRC parameter to enable the feature. Note: This doesn’t preclude the use of power boosting at the NW side by implementation Agreement For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding per-layer scaling factor applied to each of the selected SD basis vectors associated with RI=v ∈{2} for the 3-bit scaling factor(s), decide, by RAN1#119, from the following alternatives: - Alt1: min1υ,si2ri - Alt2: min1υ,si2k∈Girk - Alt3: si2 - Alt4: υsi2ri - Alt5: minriυ,si2 - Alt6: si2υ - Alt7: - if two different vectors for υ=2, min1υ,si2 for the vector with smaller scaling factor si≤sj, and min⁡(1-min⁡(1υ,si2),sj2) for the other vector configured with larger scaling factor sj; - otherwise (υ=1 or one same vector for υ=2), min1,si2 for the vector Where ri denotes the number of layers associated with the i-th SD basis vector. The same scheme applies to both Mode-A and Mode-B. Note: si∈1, 12, 13 ,14,16,18,112,116 as agreed in RAN1#117. Normalization of precoder should be taken into account in the final specification design.

At first, whether RI-common or RI-specific scaling factor is needed shall be determined. According to previous agreement, scaling factor is configured for each SD-basis group, and it is independent of rank. Therefore, RI-common scaling factor configuration shall be supported.

According to offline discussion, the above 7 alternatives are combined or down-selected to the following 3 alternatives:

    - Alt A (baseline, same as RI=1): si

    - Alt B: siri

    - Alt C: sik∈Girk

Simulation results in the offline discussion [2] show that Alt A outperforms Alt B with RI-common scaling factor configuration. Besides, no simulation result shows the performance gain of Alt C. Therefore, the motivation of supporting Alt B and Alt C is unclear, and Alt A (i.e., Alt3) is preferred.

Proposal 14: For CSI enhancement for 48, 64, and 128 CSI-RS ports, regarding per-layer scaling factor applied to each of the selected SD basis vectors associated with RI=v ∈{2} for the 3-bit scaling factor(s), the following alternative is supported.

- Alt3: si2.

- Conclusions

In this contribution, we provided our views remaining details on phase offset reporting and the linkage between CJTC reporting and CJT reporting. We also provided our views on remaining details on Type I codebook soft scaling as well as possible extension of agreed Rel-19 type I SP codebook schemes to legacy (inferior to 32) ports. Finally, we made the following proposals:

Proposal 1: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the configured associated periodic/semi-persistent SRS resource, the earliest SRS transmission occasion after the NTRP CSI-RS occasions is also supported as an option.

- Whether the SRS transmission occasion before or after NTRP CSI-RS resources is used for determining the reference UE antenna port depends on network configuration.

Proposal 2: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), for associated aperiodic SRS, the transmission occasion is the latest SRS transmission occasion before the NTRP CSI-RS occasions or the earliest SRS transmission occasion after the NTRP CSI-RS occasions.

Proposal 3: For the Rel-19 aperiodic standalone CJT calibration reporting, when P/SP-CSI-RSs are used as CMR and ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), the most recent occasion of NTRP P/SP CSI-RS resources, no later than the CSI reference resource associated with the ‘cjtc-P’ reporting can be used to determine SRS transmission occasion.

Proposal 4: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-Dd’, an additional UE procedure for a fixed value compensation for CJT CSI report is unnecessary when the reported DO value is ‘out of range’.

Proposal 5: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-Dd’, it is unnecessary to reinterpret the Dd report codepoints from intervals/ranges to values.

Proposal 6: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-Dd’, the 1-bit indicator is applied to all the NTRP CSI-RS resources.

Proposal 7: For UE delay offset measurement and reporting enhancement for CJT, when a single CSI trigger state associated with NR >1 CSI reports, the 1-bit indicator per CSI trigger state is applied to all the NR reports.

Proposal 8: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-Dd’, the number of resources configured for CJT CSI report is restricted to be the same as number of resource sets configured for CJT Dd report in case of separate triggering.

Proposal 9: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-F’, there is no need to establish the linkage between the frequency offset reporting and the CJT CSI reporting.

Proposal 10: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-Dd’, the timeline (Z/Z’) is determined as Z/Z’ associated with the Rel-18 eType-II CJT, plus Drelax. The value of Drelax is a UE capability, taken from {0, drelax}. drelax can be FFS.

Proposal 11: For CSI enhancement for 48, 64, and 128 CSI-RS ports, port mapping (indexing) method defined for Rel-19 Type-I and Type-II codebook refinement can be used for CSI-RS resource set which is associated with SRS resource set(s) in non-codebook based PUSCH transmission.

Proposal 12: For CSI enhancement for 48, 64, and 128 CSI-RS ports, mapping method 1 or mapping method 2 defined for Rel-19 Type-I and Type-II codebook refinement can be used for CSI-RS resource set which is associated with SRS resource set(s) in non-codebook based PUSCH transmission.

- Either mapping method 1 or mapping method 2 can be configured or predefined for the UE.

Proposal 13: For potential extensions of Rel-19 Type I SP codebook schemes to legacy port numbers (i.e., ≤32 ports), do not support extension of scheme A.

Proposal 14: For CSI enhancement for 48, 64, and 128 CSI-RS ports, regarding per-layer scaling factor applied to each of the selected SD basis vectors associated with RI=v ∈{2} for the 3-bit scaling factor(s), the following alternative is supported.

- Alt3: si2.

- References

- RP-242394, WID revision: NR MIMO Phase 5, Samsung (Moderator), RAN#105, September 9th – 12nd, 2024.

- R1-2409587, Moderator summary for OFFLINE discussion on Rel-19 CSI enhancements, Samsung, RAN1#119, August 18th – 22nd, 2024.