3GPP TSG RAN WG1 #119	R1-2410020

Orlando, USA, Nov. 18th – 22nd, 2024

Source:	Lenovo

Title:	On AI/ML for CSI prediction

Agenda Item:	9.1.3

Document for:	Discussion and Decision

# Introduction

In RAN#105 [1], the following has been agreed as a revision of the Rel-19 AI/ML for air interface WID:

Specify CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025

Furthermore, the following has been agreed in RAN1#118bis [2]:

Agreement For consistency between training and inference, study to identify which potential NW-side additional conditions, if any, may impact on UE assumption for CSI prediction using UE-sided model, resulting non-negligible degradation on model generalization performance. Note: Companies are encouraged to provide generalization performance evaluation on the impact of NW-side additional conditions, if considered. Agreement For generalization evaluation to identify potential NW-side additional conditions to ensure consistency between training and inference, consider one or more of the following aspects, if evaluated: - [Optional evaluation] Various tilt angle (e.g., [102] degrees, [110] degrees) - [Optional evaluation] Various TXRU mapping - With TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (8x8x2) and (12 x 8 x 2) antenna elements) - Without TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (2x8x2) and (4 x 4 x 2) antenna elements, respectively) - Other configurations are not precluded Note: for other evaluation assumptions, reuse baseline of TR 38.843. Note: report how to map antenna port to antenna elements. Note: report the backbone/structure of AI/ML model used.

In this document we discuss AI/ML consistency over training and inference procedures for AI/ML-based CSI prediction operation via UE-side models.

# Study on training/inference consistency for CSI prediction

In TR 38.843 [3], the following was captured from the study on consistency between training and inference for UE-sided models under NR air interface:

For an AI/ML-enabled feature/FG, additional conditions refer to any aspects that are assumed for the training of the model but are not a part of UE capability for the AI/ML-enabled feature/FG. It does not imply that additional conditions are necessarily specified. Additional conditions can be divided into two categories: NW-side additional conditions and UE-side additional conditions. Note: whether specification impact is needed is a separate discussion. For inference for UE-side models, to ensure consistency between training and inference regarding NW-side additional conditions (if identified), the following options can be taken as potential approaches (when feasible and necessary): - Model identification to achieve alignment on the NW-side additional condition between NW-side and UE-side - Model training at NW and transfer to UE, where the model has been trained under the additional condition - Information and/or indication on NW-side additional conditions is provided to UE - Consistency assisted by monitoring (by UE and/or NW, the performance of UE-side candidate models/functionalities to select a model/functionality) - Other approaches are not precluded - Note: the possibility that different approaches can achieve the same function is not denied

For UE-sided CSI prediction, the following NW-side additional conditions are considered:

- TXRU virtualization assumptions, e.g., trivial virtualization, i.e., each antenna element is mapped to a Tx antenna port in a one-to-one fashion, vs. non-trivial virtualization, i.e., multiple antenna elements are mapped to a same antenna port.

- Variations in gNB antenna downtilt and/or panning angles.

In addition to NW-side additional conditions, the data collection for training and input for inference may not be consistent with respect to the underlying environment conditions, e.g., scenario, UE speed range, etc. For instance, the input for inference may correspond to a UE speed that is higher than the range of UE speeds corresponding to the training dataset points. The impact of the training/inference consistency for CSI prediction due to variations of these parameters needs to be evaluated.

Evaluation is needed of training/inference consistency under UE-sided CSI prediction due to variations in NW-side additional conditions, e.g., TXRU virtualization and gNB antenna tilt/panning, in addition to environment conditions, e.g., scenario and UE speed.

Regarding the generalization over different TXRU mappings for two-sided AI/ML-based CSI compression, the following was captured in TR 38.843:

For generalization Case 3, generalized performance of the AI/ML model can be achieved (0%~-4.4% loss or positive gain) for TxRU mapping#B subject to any of [2,8,2], [4,4,2], and [8,2,2] from the perspective of the layouts of antenna ports, or subject to any of 8x8x2 and 2x8x2 from the perspective of the layouts of antenna element mapping, if the training dataset is constructed with data samples subject to TxRU mappings including TxRU mapping#B, as observed by 4 sources. - Minor loss (0%~-2%) are observed by 4 sources. - Moderate loss (-2.5%~-4.4%) are observed by 1 source. - Positive gains are observed by 1 source.

From the perspective of PMI calculation, the performance degradation due to TXRU mapping inconsistency across training and inference appears to be marginal. However, the same needs to be verified for UE-sided AI/ML-based CSI prediction. Since electrical tilt and panning can be incorporated within the TXRU virtualization, they can be considered jointly in training/inference consistency evaluation.

Verify whether UE-sided AI/ML-based CSI prediction performance degradation resembles that of two-sided AI/ML-based CSI compression with respect to training/inference consistency in TXRU virtualization and electrical tilt/panning variation.

- For variations in UE speed over different training/inference consistency, the following has observed and captured in TR 38.843 for Rel-18 study of AI/ML-based CSI prediction:

Generalization over UE speeds For the generalization verification of AI/ML based CSI prediction over various UE speeds compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain UE speed#B and applied for inference with a same UE speed#B, - For generalization Case 2, generalized performance may be achieved for certain combinations of UE speed#A and UE speed#B but not for others: - If UE speed#B is 10 km/h & UE speed#A is 30 km/h, 2 sources observe a generalized performance of less than -1.4% degradation. - Note: 1 company still observes significant degradation (-11.3%~-13.4% loss). - If UE speed#B is either 30 km/h or 60 km/h or 120 km/h, or if UE speed#B is 10km/h and UE speed#A is either 60km/h or 120km/h, 11 sources observe that moderate/significant performance degradations are suffered: - For UE speed#B is 10 km/h & UE speed#A is either 60 km/h or 120 km/h, 1 source observes moderate degradation (-2.3% loss), 3 sources observe significant degradation (-5.5%~-61% loss). - For UE speed#B is 30 km/h & UE speed#A is either 10 km/h, 60 km/h or 120 km/h, 2 sources observe moderate degradation (-2.01%~-4.62% loss), 9 sources observe significant degradation (-5%~-72.37% loss). - For UE speed#B is 60 km/h & UE speed#A is either 10 km/h, 30 km/h or 120 km/h, 1 source observes moderate degradation (-3% loss), 10 sources observe significant degradation (-7.8%~-76.85% loss). - For UE speed#B is 120 km/h & UE speed#A is either 30 km/h or 60 km/h, 1 source observes moderate degradation (-3.4% loss), 5 sources observe significant degradation (-7.55%~-56.3% loss). - For generalization Case 3, generalized performance of the AI/ML model can be achieved in general (0%~-4.45% loss) for UE speed#B subject to any of 10 km/h, 30 km/h, 60 km/h and 120 km/h, if the training dataset is constructed with data samples subject to multiple UE speeds including UE speed#B, as observed by 11 sources. - For UE speed#B is 10 km/h, minor loss (-0.2%~-1.7%) are observed by 4 sources. - For UE speed#B is 30 km/h, minor loss (-0.2%~-1.34%) or positive gain are observed by 5 sources, moderate loss (-4.07%~-4.2%) are observed by 2 sources. - For UE speed#B is 60 km/h, minor loss (-0.05%~-2%) are observed by 4 sources, moderate loss (-3.76%~-4.65%) are observed by 2 sources. - For UE speed#B is 120 km/h, moderate loss (-2%~-4.45%) are observed by 4 sources. - - Note: For generalization Case 3, 6 sources observe significant performance degradations (-5%~-43.6% loss) for UE speed#B subject to 10 km/h, 30 km/h, 60 km/h, but compared with generalization Case 2, in general the performance is still improved.

- As illustrated above in the study, the performance degradation due to training/inference inconsistency based on UE speed variations is negligible whenever the training data comprises sufficient points with UE speeds equal to or higher than the UE speed applied for inference.

For UE-sided AI/ML-based CSI prediction, training the model with various UE speed values that are equal to or exceed the UE speed values associated with inference process leads to negligible performance degradation compared with a scenario in which the UE speed for inference matches that of the training dataset points.

# Methods to maintain training/inference consistency

Based on the Rel-18 study on the issue of consistency between training and inference for UE-sided models, four approaches have been considered, as follows:

- Model training at NW and transfer to UE.

- Information and/or indication on NW-side additional conditions is provided to UE.

- Model identification between NW-side and UE-side.

- Model monitoring-based consistency assistance.

In the remainder of this contribution, we discuss the previous approaches for mitigating consistency issues, if any, across training and inference for UE-sided CSI prediction.

## Model training at the NW side with model transfer to the UE

This approach is based on building a from-ground-up consistent training and inference processes via training the UE-sided model at the NW side, followed by sharing the model with the UE. However, this approach requires support of a model transfer mechanism, either via proprietary methods or specification-based methods. In our opinion, model training at the network side and transferring the model to the UE side is not practical due to the significant overhead and substantial coordination required for model sharing, whether via proprietary or specification-based methods. Moreover, for CSI prediction in FDD networks, the training data is naturally available at the UE side based on CSI-RS received at the UE that enables channel estimation and CSI acquisition.

Maintaining training/inference consistency via model training at the network side and transferring the model to the UE side incurs significant challenges in model transfer.

Maintaining consistency of training/inference via model training at the NW side followed by model transfer to UE side is not considered for the study of training/inference consistency under UE-sided CSI prediction.

## Indication of NW-side additional conditions to the UE side

On the other hand, indication of the NW-side additional conditions provided to the UE can also enable achieving consistency between training and inference for UE-sided CSI prediction. This requires the support of a signaling framework from the NW side to the UE side to allow signaling a “quantified” set of parameters corresponding to the NW-side additional conditions. However, the following aspects need to be considered for explicit indication of additional conditions, as follows:

- The number of NW-side additional condition parameters and the corresponding codepoints. In case the number of additional condition parameters is large, and/or if corresponding parameter values and combinations (to be mapped to IDs) is massive, the additional condition indication becomes challenging.

- A generalized model may be trained based on dataset points corresponding to multiple configurations, making it difficult to map the training phase to a fixed set of additional conditions.

- The interpretation of additional conditions at both the UE and the NW side may not be matched.

Maintaining training/inference consistency via signaling information corresponding to NW-side additional conditions may be challenging in case of a large number of additional condition parameters/values and combinations, and in case of generalized models that are trained with dataset points corresponding to a mixture of different additional conditions.

Based on the previous observation, the efficiency of maintaining consistency of training/inference via indication of NW-side additional conditions depends on the number of NW-side additional conditions/parameters, as well as for localized/cell-specific/site-specific models, or non-generalized models. The study can follow the progress made for training/inference consistency study for BM-Case1 and BM-Case2 under Rel-19 normative work.

Further study maintaining consistency of training/inference via indication of NW-side additional conditions, with focus on scenarios with a few number/values of NW-side additional conditions, as well as scenarios with localized/cell-specific/site-specific models.

## Model identification between NW side and UE side

One other approach that enables achieving training/inference consistency is via model identification between the network side and UE side. Model identification is based on corresponding training dataset identification, i.e., in the presence of a finite number of training datasets, model(s) trained by a distinct training dataset are allocated a distinct ID. This approach requires matching the conditions under which training and inference processes are pursued. For instance, values of parameters including UE speed, antenna layout/virtualization at the gNB/UE, channel model, LoS conditions, etc., may be common per one model. Note that the model ID would not carry explicit information on the additional conditions (except conditions that can be inferred from metadata, if any, corresponding to training dataset). In order to map a model with corresponding conditions, a matching phase may be needed so that both the network side and the UE side can share side information related to the model/training dataset identification process, either via specification-based methods or proprietary methods, to ensure consistency. This approach seems reasonable for scenarios in which a few generalized models are available with limited information on how the training dataset was aggregated to train a given model.

Model identification is simple to implement but requires implicit/implementation-based approaches to match the training and inference processes.

In our opinion, training/inference consistency via model identification is realizable only in scenarios with a few generalized models with lack of information on the additional conditions. Moreover, model identification can be considered a special case of NW-side additional condition identification in the presence of a single condition, wherein the single condition corresponds to an aggregation of parameter values that are mapped to one codepoint. Therefore, our preference is to deprioritize model identification for training/inference consistency.

Deprioritize model identification-based techniques for training/inference consistency for UE-sided CSI prediction.

## Model monitoring for maintaining training/inference consistency

Finally, the last approach to be discussed that may improve training/inference consistency is UE-based performance monitoring, i.e., performance monitoring feedback is utilized to minimize the training/inference consistency. Using performance monitoring, either Type-1, Type-2 and/or Type-3 may help reduce the gap between training and inference, especially if the monitoring process reports parameters that are correlated with the NW-side additional conditions. Further details on the performance monitoring types for UE-sided CSI prediction as part of the Rel-18 study can be found in TR38.843, as follows:

Performance monitoring: For CSI prediction using UE side model use case, at least the following aspects have been proposed by companies on performance monitoring for functionality-based LCM: - Type 1: - UE calculates the performance metric(s) - UE reports performance monitoring output that facilitates functionality fallback decision at network - Performance monitoring output details can be further defined - NW may configure threshold criterion to facilitate UE side performance monitoring (if needed). - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Type 2: - UE reports predicted CSI and/or the corresponding ground-truth - NW calculates the performance metrics. - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Type 3: - UE calculates the performance metric(s) - UE reports performance metric(s) to the NW - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Functionality selection/activation/deactivation/switching as defined for other UE side use cases can be reused, if applicable. - Configuration and procedure for performance monitoring - CSI-RS configuration for performance monitoring - Performance metric including at least intermediate KPI (e.g., NMSE or SGCS) - UE report, including periodic/semi-persistent/aperiodic reporting, and event driven report - Note: down selection is not precluded. - Note: UE may make decision within the same functionality on model selection, activation, deactivation, switching operation transparent to the NW.

- We provide more emphasis on the different monitoring types below, as follows:

- Type 1 monitoring for CSI prediction:

- UE-based monitoring approach in which the UE reports an explicit recommendation of a monitoring decision. In the context of monitoring-based reporting of training/inference consistency, the UE is expected to report an indication of a decision based on the evaluated consistency. Examples of monitoring output may include: (i) No model change, (ii) CSI parameter(s) update, (iii) Model switching, and (iv) Fallback to non-AI/ML scheme.

For CSI prediction Type 1 monitoring, output may include: (i) No model change, (ii) CSI parameter(s) update, (iii) Model switching, and (iv) Fallback to non-AI/ML scheme.

- Type 2 monitoring for CSI prediction:

- NW-based monitoring approach in which the UE reports the ground truth CSI to the network for further performance metric evaluation and monitoring output. In the context of monitoring-based reporting for consistency, the UE is expected to report the ground truth CSI, wherein the network evaluates a correlation measure of the ground truth CSI feedback and compares it with the training dataset characteristics, if available at the NW side and/or the reported predicted CSI. In general, this approach may require large feedback corresponding to the monitoring payload corresponding to the ground truth CSI coefficients.

For CSI prediction Type 2 monitoring, the NW side can evaluate the training/inference consistency based on comparison of the ground truth CSI with the training data characteristics and/or the reported predicted CSI.

- Type 3 monitoring for CSI prediction:

- UE-assisted monitoring approach in which the UE reports performance metrics that improve the ability of the network to take a suitable monitoring decision. In the context of monitoring-based reporting of training/inference consistency, the UE reports performance metric(s) corresponding to a measure of the quality or resolution of the precoding matrix computed via AI/ML scheme(s), or any other measure of a correlation between the training data labels and the inference output. The network can configure the UE with reporting a subset of pre-defined performance metrics based on RRC, MAC-CE or DCI signaling.

For CSI prediction Type 3 monitoring, output is in a form of performance metric(s) that correspond to a measure of a correlation of the training and inference information.

- As mentioned above, Type 2 monitoring requires significant overhead and can be deprioritized. Type 1 monitoring adopts a UE-assisted monitoring decision approach, whereas Type 3 monitoring adopts a NW-based monitoring decision based on performance metric feedback from the UE. In our opinion, both Type 1 and Type 3 performance monitoring for the purpose of training/inference consistency should be considered for further study. One advantage of using model monitoring for training/inference consistency is that knowledge of the additional conditions is not needed, and instead the performance of different models is monitored to identify the most consistent model, including model switching and/or fallback to non-AI/ML schemes.

Monitoring-based approach for training/inference consistency is motivated for scenarios with lack of knowledge on the additional conditions.

Further study Type 1 and Type 3 performance monitoring techniques for CSI prediction for the purpose of training/inference consistency.

- To summarize, our preference is to down select the focus for training/inference consistency issues for AI/ML-based CSI prediction, if supported, to additional condition associated ID techniques, as well as monitoring-assisted consistency techniques based on Type 1 and Type 3 monitoring for UE-sided CSI prediction.

For training/inference consistency issues for UE-sided CSI prediction, if specified, scope is limited to additional condition associated ID techniques, as well as monitoring-based techniques.

# Conclusion

This contribution studied some aspects on training and inference consistency for UE-sided AI/ML-based CSI prediction for Rel-19 WI. We have the following observations:

- Evaluation is needed of training/inference consistency under UE-sided CSI prediction due to variations in NW-side additional conditions, e.g., TXRU virtualization and gNB antenna tilt/panning, in addition to environment conditions, e.g., scenario and UE speed.

- Maintaining training/inference consistency via model training at the network side and transferring the model to the UE side incurs significant challenges in model transfer.

- Maintaining training/inference consistency via signaling information corresponding to NW-side additional conditions may be challenging in case of a large number of additional condition parameters/values and combinations, and in case of generalized models that are trained with dataset points corresponding to a mixture of different additional conditions.

- Model identification is simple to implement but requires implicit/implementation-based approaches to match the training and inference processes.

- For CSI prediction Type 1 monitoring, output may include: (i) No model change, (ii) CSI parameter(s) update, (iii) Model switching, and (iv) Fallback to non-AI/ML scheme.

- For CSI prediction Type 2 monitoring, the NW side can evaluate the training/inference consistency based on comparison of the ground truth CSI with the training data characteristics and/or the reported predicted CSI.

- For CSI prediction Type 3 monitoring, output is in a form of performance metric(s) that correspond to a measure of a correlation of the training and inference information.

- Monitoring-based approach for training/inference consistency is motivated for scenarios with lack of knowledge on the additional conditions.

- Based on the observations above, we have the following proposals:

- Verify whether UE-sided AI/ML-based CSI prediction performance degradation resembles that of two-sided AI/ML-based CSI compression with respect to training/inference consistency in TXRU virtualization and electrical tilt/panning variation.

- For UE-sided AI/ML-based CSI prediction, training the model with various UE speed values that are equal to or exceed the UE speed values associated with inference process leads to negligible performance degradation compared with a scenario in which the UE speed for inference matches that of the training dataset points.

- Maintaining consistency of training/inference via model training at the NW side followed by model transfer to UE side is not considered for the study of training/inference consistency under UE-sided CSI prediction.

- Further study maintaining consistency of training/inference via indication of NW-side additional conditions, with focus on scenarios with a few number/values of NW-side additional conditions, as well as scenarios with localized/cell-specific/site-specific models.

- Deprioritize model identification-based techniques for training/inference consistency for UE-sided CSI prediction.

- Further study Type 1 and Type 3 performance monitoring techniques for CSI prediction for the purpose of training/inference consistency.

- For training/inference consistency issues for UE-sided CSI prediction, if specified, scope is limited to additional condition associated ID techniques, as well as monitoring-based techniques.

# References

| [1] | Qualcomm, "Revised WID on AI/ML for NR Air Interface," Melbourne, Australia, Sep. 9-12, 2024. |
| --- | --- |
| [2] | 3GPP TSG RAN WG1#118bis, "Draft Meeting Report," Hefei, Anhui, China, Oct. 14-18, 2024. |
| [3] | 3GPP TR 38.843, "Study on AI/ML for NR air interface," Oct. 2023. |