3GPP TSG RAN WG1 #119			R1-2410147

Orlando, US, November 18th – 22nd, 2024

Title:	Draft Reply LS on SSB relation in On-demand SSB and SSB adaptation on SCell

Response to:	R1-2409350 (R4-2416913)

Release:	Release 19

Work Item:	Netw_Energy_NR_enh

Source:	Google, [RAN1]

To:	RAN4

Cc:	RAN2

Contact Person:

Name:	 Yushu Zhang

E-mail Address: yushuzhang@google.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: -

1. Overall Description:

RAN1 would like to thank RAN4 for the questions on impacts on SSB relation between on-demand SSB and SSB adaptation, and would like to provide the following answers to the questions.

Obj. 1 related to on-demand SSB SCell operation_

Based on the discussion, RAN4 identified that the relation between always-on SSB and on-demand SSB (i.e. Case#2 as agreed in RAN1) has impacts on UE’s behavior and requirements in SCell. RAN4 would like RAN1 to clarify at least the following aspects:

- Q1: What is the relation in terms of periodicity between always-on SSB and OD-SSB?

- A1: The always-on SSB and OD-SSB can be configured based on the same or different periodicities.

- Q2: What is the relation in terms of time location between always-on SSB and OD-SSB?

- A2: The always-on SSB and OD-SSB can be configured based on the same or different time locations.

- Q3: What is the relation in terms of frequency location between the always-on SSB and OD-SSB?

- A3: The always-on SSB and OD-SSB can be configured based on the same or different frequency location.

- Q4: What is the spatial relation between the always-on SSB and OD-SSB?

- A4: The always-on SSB and OD-SSB should be based on the same SSB indexes. The OD-SSB and always-on SSB based on the same SSB index are QCLed with the average delay, delay spread, Doppler spread, Doppler shift, spatial reception parameter and average gain.

Obj.3 related to SSB adaptation in time domain

For the SSB adaption in the objective of adaptation of common signal/channel transmits, RAN4 would also like RAN1 to clarify the relation for SSB adaptation at least for following aspects.

- Q1: What is the relation in terms of time location before and after SSB adaptation?

- A1: The time location may be the same or different before and after SSB adaptation.

- Q2: What is the relation in terms of frequency location before and after SSB adaptation?

- A2: The frequency location is not changed before and after SSB adaptation.

- Q3: What is the spatial relation before and after SSB adaptation?

- A3: The spatial relation is not changed before and after SSB adaptation.

2. Actions:

To RAN4:

RAN1 respectfully asks RAN4 to take into the response above for further work.

3. Date of Next TSG-RAN1 Meetings:

TSG-RAN1 #120			17 Feb. – 21 Feb. 2025	Athens, Greece