3GPP TSG RAN WG1 #118bis			R1-2410160

Orlando, US, November 18th – 22nd, 2024

Agenda Item:	9.11.1

Source:	Sharp

Title:	NR-NTN downlink coverage enhancement techniques

Document for:	Discussion and Decision

- Introduction

NTN downlink coverage enhancements consider the NTN deployment constraints such as payload power limitation, large satellite footprint and limited feeder link bandwidth. In the last several meetings, extensive link level simulations were presented, and the options to fill the link budget gap for PDCCH and PDSCH were discussed. In this contribution, we present our views on the remaining issues, and the principles to use beam groups and DRX patterns for efficient operations.

- Discussions

2.1. 	Additional SSB/PBCH default periodicity

In RAN1 #118bis, it was agreed to support extened SSB/PBCH periodicity to at least 160ms. A beam group is formed by the set of simulatenous transmitted beams. For the NTN satellite and gNB, each SSB index can be used to transmit multiple beams, e.g. beams for a beam group and/or beam pattern. The gNB can scheduled different sets of SSBs for different beam groups although the beam group is transparent to UE as the NTN UE will only detect SSBs from one beam. Therefore, 160ms is good enough to ensure that the SSBs are transmitted on all beams of the coverage footprint. Therefore

Proposal 1: For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access, with the maximum additional default value as 160ms.

2.2. 	Coverage enhancement techniques for different channels

The techniques are studied for different channel tyes, including PDCCH and Msg4 PDSCH, etc. The maim method for coverage enhancement is repetition. Different number of repetitions can be configured based on the gap values, e.g. a 2 times repetition provides 3 dB, and 3 time repetition provides 4.7dB, and so on.

For PDCCH, intra-slot repetition will require monitoring DCI beyond the defaulted CORSET region. Similarly, extend the CORSET region will not be compatible with legacy UEs, and reduced DCI size will require new DCI formats. To minimize the specification impact, it is better to support inter-slot PDCCH repetition only. The PDCCH repetition methods can be applied to all types of PDCCHs in CSS and USS as a unified solution.

Proposal 2: Support inter-slot PDCCH repetition for PDCCH in both CSS and USS.

The repetition levels can be separately configured for CSS and USS. For PDCCH for Msg2 and Msg4 during initial access, the repetition level should be signaled to UE before RRC connection. Thus, the number of PDCCH repetitions in CSS should be indicated in MIB or SIB. For PDCCH in USS, the repetition number may be configured by RRC. If not configured, the USSD PDCCH repetition number can be the same as CSS PDCCH as default.

Proposal 3: The number of repetitions for CSS PDCCH is indicated in MIB/SIB, and number of repetitions for USS PDCCH can be configured by RRC.

Similarly, repetition can be the default method for PDSCH coverage enhancement. The repetition factor for different types of PDSCHs can be different depending on the link budget deficit, e.g. PDSCH with Msg 4 and PDSCH with SIB1/SIB19, etc. Sicne RACH procedure is performed first for the NTN access, the Msg. 2/Msg.4 performance should be guaranteed to establish the connectivity. Thus, for Msg2 and Msg4, the number of repetitions, if applicable, should be known before RRC connection. Thus, the number of PDSCH repetitions for Msg2/Msg4 should be indicated in SIB.

Proposal 4: The numbers of repetitions for Msg2 and Msg4 PDCCH are indicated in SIB.

If dynamic power sharing is employed, the gNB may limit the simultaneous transmitted beams and allocate power to channels with coverage gaps, and more power can be allocated to the channels with larger coverage gaps. The actual number of active beams can then be dynamically scheduled by gNB based on total available power. Techinques like repetition can be jointly considered for the link budget enhancement. With dynamic power sharing, the number of repetitions required for a channel may be smaller if power boosting is already applied. However, how to indicate the modified number of repetitions should be studied further.

Proposal 5: Dynamic power sharing can be applied to reduce the number of repetitions required to fill the coverage gap of a channel.

2.3	Beam group and beam hopping

With a large number of beams, to effectively scheduling for data transmissions in the whole coverage area, beam patterns or beam groups are needed. A beam group with a beam pattern contains the beams that may be simultaneous generated. A pre-defined or well-defined beam pattern can be reused for different NTN satellites to provide easy scheduling at the gNB. With a well-defined beam pattern, the beam patterns may be generated by a formula or algorithm without complicated allocations among beams, the beam group index may be determined automatically based on the formula.

In design of beam group/pattern, the distances among the beams are important parameters, esp. the distances between the active beams for simultaneous transmissions. Intuitively, more distributed patterns with large sparations will have better performance since the cross interference among the active beams can be reduced.

Proposal 6: Beam group/patterns can be defined for efficient gNB scheduling. A distributed beam pattern can reduce the interference among simultaneous transmitted beams.

The gNB can cyclically transmit among the beam groups/patterns so that all coverage area is served. To reduce system complexity, the active beam transmissions can be limited to the beams within a beam group or a beam pattern. In this case, the UEs in a given beam group do not need to monitor the DL transmissions targeted for other beam groups.

Therefore, the gNB can indicate a DL monitoring timeslot pattern, e.g. a beam group specific DTX/DRX pattern or timeslot bitmap, to minimize the unnecessary DL reception at the NTN UEs. From the gNB point of view, multiple DTX/DRX patterns can be configured for a cell. A DTX/DRX pattern can be defined for each beam group for on all beams in the beam group. And the UE only needs to know the DTX/DRX pattern for the given beam group.

The increased SSB periodicity can also provide more flexibility on the DTX/DRX pattern since the period of DTX/DRX can be the same as the SSB periodicity.

Proposal 7: Beam group based DTX/DRX pattern can be introduced to minimize the UE monitoring periods.

- Conclusions

In this contribution, we discussed the remaining issues of NTN DL coverage enhancements with the following proposals.

Proposal 1: For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access, with the maximum additional default value as 160ms.

Proposal 2: Support inter-slot PDCCH repetition for PDCCH in both CSS and USS.

Proposal 3: The number of repetitions for CSS PDCCH is indicated in MIB/SIB, and number of repetitions for USS PDCCH can be configured by RRC.

Proposal 4: The numbers of repetitions for Msg2 and Msg4 PDCCH are indicated in SIB.

Proposal 5: Dynamic power sharing can be applied to reduce the number of repetitions required to fill the coverage gap of a channel.

Proposal 6: Beam group/patterns can be defined for efficient gNB scheduling. A distributed beam pattern can reduce the interference among simultaneous transmitted beams.

Proposal 7: Beam group based DTX/DRX pattern can be introduced to minimize the UE monitoring periods.

- References

- Draft Report of 3GPP TSG RAN WG1 #118bis, v010