3GPP TSG RAN WG1 #119	R1-2410255

Orlando, USA, November 18th – 22th, 2024

Agenda Item:	9.1.1

Source:	Fraunhofer HHI, Fraunhofer IIS

Title:	Specification support for beam management

Document for:	Discussion & Decision

# Introduction

A new work item on AI/ML for NR air interface was approved in the RAN#102 plenary meeting [1] the support of AI for beam management by the specification was agreed. Furthermore, in RAN1#118b discussion, the following agreements were made:

Agreement Answer to Q2 in R1-2407604 as below: RAN 1 did not have agreement on the content of NW-side additional condition. RAN1 agreed to support associated ID and it can be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID, while FFS whether/how to define similar properties of a DL Tx beam or beam set/list.. Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), - At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation - FFS: on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric - FFS other alternatives Agreement For BM-Case 2 of UE-side model, for the reference time of the earliest time instance for the predicted results, consider at least the following alternatives for potential down-selection: - Option 1: Based on the uplink slot for the report - Option 2: Based on the CSI reference resource corresponding to the report - Option 3: Based on the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report, wherein the transmission occasion is no later than the CSI reference resource Agreement For UE-side model, existing CPU mechanism is used as a starting point for AI/ML-based CSI processing. - FFS whether the overall CPU should be shared or separately counted between legacy CSI reporting and AI/ML-based CSI reporting, and among AI/ML features/functionalities. - FFS whether it is fully applicable for BM-Case 1 and/or BM-Case 2 Agreement For UE-side AI/ML model, for BM-Case1, at least for inference, at least for Set B, support the following CSI-RS resource types for CMR: - Periodic (P) CSI-RS - Semi-persistent (SP) CSI-RS - Aperiodic (AP) CSI-RS For UE-side AI/ML model, for BM-Case 2, at least for inference, at least for Set B, support the following CSI-RS resource types for CMR: - Periodic (P) CSI-RS - Semi-persistent (SP) CSI-RS - FFS: Aperiodic (AP) CSI-RS Note: above CSI-RS resource refers to that used for beam management. Agreement At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), consider the following options with potential down selection for the configuration for monitoring: - Option 1: The resource set(s) for monitoring and report configuration for monitoring are configured (when applicable) within CSI report configuration used for inference - FFS: the resource set(s) for monitoring - UE measures the resource set(s) for monitoring. - FFS how/when to report the monitoring results. - Option 2: Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The dedicated report configuration used for monitoring links to an inference report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - UE measures the resource set(s) for monitoring. - FFS when to report the monitoring results. Agreement RAN 1 further study the following options for applicability for inference for UE-side model: Option 1: - In Step 3, following configurations are provided from NW to UE: - 1) UE is allowed to do UAI reporting via OtherConfig, - 2)+3) NW configures one or more CSI-ReportConfig for inference configuration, where the associated ID may be configured in CSI framework as working assumption applied. - FFS on whether some IEs in the CSI report configuration can be removed or modified - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - In Step 4, UE reports applicability(ies) of the above CSI-ReportConfig - FFS on one or more of the above CSI-ReportConfig to be reported - FFS on activation (including when/how) of inference report after obtaining the applicability from UE Step 4 - FFS: whether Step 5 is needed, Option 2: - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - NW configures one set or multiple sets of inference related parameters - Note: the set of inference related parameters is not configured by CSI-ReportConfig - FFS on the set of inference related parameters, at least including: - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - The associated ID(s) may be configured - wherein the associated ID(s) may be - FFS: a) part of one set of the inference related parameters, or - FFS: b) independently from the one set of the inference related parameters. - In Step 4, UE reports applicability of the above one or multiple sets of inference related parameters, where the associated ID information may be associated. - In Step 5, NW configures configuration(s) for CSI report for inference Option 3: - In Step 3, following configurations are provided from NW to UE: - 1) UE is allowed to do UAI reporting via OtherConfig, - 2) The associated ID(s) may be provided to UE, e.g., a new RRC parameter. - In Step 4, UE reports by UAI - the applicable one or multiple sets of inference related parameters may be included. - FFS on the set of inference related parameters, at least including: - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - Note: not applicable may also be replied by UE - Note: if the inference related parameters are not supported for reporting, only the applicability(ies) or not is reported in Step 4. - the associated ID(s) may be included - FFS: a) as part of the inference related parameters, or - FFS: b) independently from the set of the inference related parameters. - In Step 5, NW configures configuration(s) for CSI report for inference. Note: There is no impact of configuring CSI report configuration for non-AI beam management in RRCReconfiguration. Agreement For UE-side model, for beam management, for inference report, support periodic CSI report, aperiodic CSI report, and semi-persistent CSI report. Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability. Agreement Incorporating below text into the general part of the LS In RAN1’s discussion of RAN 2 terminologies on beam management, - The concept/terminology “functionality“ of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-specific FGs - The concept/terminology “ functionality“ of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters or information/parameters indicated by UE - The Activated functionalities may be enabled based on CSI framework. Therefore, the meaning and the granularity of “functionality“for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same, depends on certain option in RAN1, and the discussion is still ongoing Agreement Answer to Q1 in R1-2407604 as below, In Step 2, RAN1 expects that UE reports its UE-capability information/parameters, i.e., Rel-19 AI/ML-specific FGs (including components and corresponding value ranges). These AI/ML-specific UE capability information/parameters will depend on how FGs are defined including the granularity, that will be discussed in RAN1 later in the WI..

In the AI/ML SI [2], two use cases for beam management have been studied, BM-Case 1 and BM-Case 2. Furthermore, NW-sided models and UE-sided models have been considered for these use cases. As per the WID, it was agreed to specify both flavors having different requirements to the specification. Thus, we structure our contribution according to the WID and will highlight aspects for NW-sided and UE-sided Assistance information for BM-Case 1

# Performance Monitoring of UE-sided models

Model monitoring is an extremely crucial aspect for employing AI/ML in the field. While AI models perform very well under theoretical or well-defined testing conditions, their practical performance can be significantly worse than that. Hence, monitoring and validating the model performance is of uttermost importance.

Figure 1 Monitoring phases with less measurements, and validation phase with increased number of measurements, e.g., CSI measurements.

In Figure 1, we show a 2-phase monitoring approach in which the UE adapts its monitoring depending on the phase. In a monitoring phase, the monitoring requirements can be more relaxed. Thus, the overhead of reference signals is reduced compared to the validation phase. In the validation phase, the UE reports more frequently and possibly more measurements or KPIs to the network. For example, the UE measures all beams of Set A in the validation phase. Thus, the UE can reliably assess the performance of its AI model and furthermore, gather measurements for training of another model.

Proposal 1: For monitoring UE-sided models, support 2-phase monitoring with varying frequencies and reporting detail.

In particular, during the monitoring phase only a subset of Set A may be measured while during the validation phase all beams of Set A may be monitored. This would lower the monitoring overhead especially during longer monitoring phases.

Proposal 2: For monitoring, support monitoring of only a subset of Set A beams.

An important matter is how to determine the switch between the monitoring and validation phases. In particular, an indication-based or event-based strategy may be used, both with their advantages and disadvantages. For the indication-based strategy, the NW may initiate the UE to switch into a validation phase, in which the UE may measure more beams, e.g., whole Set A beams. However, it comes at the drawback that the UE potentially knows at an earlier stage that more extensive monitoring may be required. Hence, an event-based indication offers faster adaptation to changing environments and reduces the reporting overhead, since during the monitoring phases the UE may not need to report the performance to the NW. Thus, the UE would only calculate a performance metric in the monitoring phase and only report, if the performance metric drops below a certain threshold. This may also trigger a switch into the validation phase, in which more extensive monitoring, e.g., more beams, reporting, is performed.

Proposal 3: Consider indication-based and event-based switching into a validation phase. Events may be defined based on agreed performance metrics.

Events have to characterize the prediction performance. For that reason, we think that the difference between the predicted values and the actual values and the prediction accuracy are good indicators. Furthermore, also a certain KPI, e.g. BLER, indicates the overall performance of the beam prediction.

Proposal 4: For the activation of monitoring, support Event-1, Event-4 and Event-5.

# Configuration of Set A and Set B

In the WID, it is stated that both UE-sided and NW-sided models for beam management shall be supported. In general, UE-sided models offer multiple merits over NW-sided models. In particular, the measurement signaling overhead is significantly reduced. This is due to the UE rather processing the measurements directly instead of transmitting them to the NW. Essentially, this also allows a shorter measurement-to-inference delay as the step of transmitting the measurements to the NW is omitted. Finally, the UE can use more measurements as input to the beam prediction as there is no cost associated to transferring measurements over the air.

For BM-Case2, the UE reports inference results of N future time instances. The inference result is obtained based on Set B measurements. Hence, the predicted time instances of the AI/ML model will be based on the timing of the transmission of the Set B resources. For this reason, we believe that the reported time instances N should be determined based on the transmission of Set B CSI-RS/SSB resources. However, the CSI reference resource may also be a suitable reference time for determining the future time instances.

Proposal 5: For BM-Case2, for inference, the reference time of the predicted time instances shall be determined based on the CSI reference resource or the transmission occasion of Set B resources.

In BM-Case2, the UE predicts the CSI for future time instances based on past time instances. However, in certain circumstances, it may be required that some of the past time instances do not provide helpful information in terms of the prediction task. Even to the contrary, a measurement based on wrong assumptions may harm the prediction performance and hence, should be excluded. For example, the gNB may have adapted its Tx codebook or certain beams or TRPs may have been not utilized or deactivated temporarily. In these cases, the UE should not utilize measurements based on these beams.

Proposal 6: For BM-Case2, for UE-sided models, introduce a configuration that allows the exclusion of certain past measurements for the inference.

## Configuration of Set A of beams

In some cases, the Set A of beams can be the resources for radio link monitoring or resources for radio link quality measurements. Based on the RS resources configured for the two procedures, the RS resources configured for radio link monitoring can be used for model monitoring procedures associated with CSI beam reporting. The linking between the two can be determined based on the beams configured for the two purposes, the TCI-states of the beams and beam prediction methods used. If such linking for model monitoring between the two frameworks is possible, a separate set of configured resources for model monitoring is not necessary.

Proposal 7: Explore the possibility of using the resources configured for radio link monitoring for model monitoring purposes with respect to CSI beam reporting.

Furthermore, such a model monitoring configuration can also be used to collect data, which allows the UE to train a new AI/ML model. In addition, the UE also determines the performance of other inactive models and can use these results to decide whether switching to another beam management model is beneficial or not.

Proposal 8: Support a model monitoring configuration that allows for collecting data for model training and monitoring of inactive models.

## Radio link monitoring and recovery

Like CSI beam reporting, radio link monitoring and link recovery procedures also involve the reporting of L1-RSRP values or status messages associated with L1-RSRP measurements to higher layers. Considering the possibility of temporal and spatial AI/ML beam prediction models deployed at the UE side, RS configuration and reporting mechanisms for radio link monitoring or link recovery exploiting such prediction models need to be studied by RAN1.

Proposal 9: Study the support for radio link monitoring and link recovery procedures considering spatial and temporal beam prediction at the UE-side.

## Overhead reduction in configuration

As stated above, the CSI framework can be reused for the configuration of Set A/Set B of beams and for reporting information on the predicted beams and/or L1-RSRP values. The agreement in RAN1#118 to configure both sets separately for UE-side BM-Case1, which naturally leads to considerably increased overhead compared to legacy beam reporting. With the typical or most useful use-case in spatial beam prediction being the measurement of a small set of beams to predict a larger set of beams, the configuration of set B being a subset of set A should be the priority. This relationship between set A and set B should be exploited for overhead reduction of the CSI resource and report configuration to avoid duplication of configuration(s) across set A and set B.

Proposal 10: For the configuration of set A and set B of beams for UE-side BM-Case1, the relationship in use-cases where set B of beams is a subset of set A of beams shall be exploited for overhead reduction of CSI resource and/or report configuration.

# Measurement report for NW-sided models

In the TR [2], an important role of data collection for NW-sided models in enabling different use cases has been identified, and the following considerations were made:

A set of general data collection principles is expected to be considered for network-side model training. These include: -UE to support data logging, -UE to report the collected data periodically, event-based, and on-demand, -The UE memory, processing power, energy consumption, signalling overhead should be considered. Note: The above principles can be revised depending on RAN1 requirements. Furthermore, and regarding the use cases in this study, the following is considered. For CSI and beam management use cases, the training of network-side models can consider both gNB and OAM-centric data collection mechanisms. The gNB-centric data collection implies that the gNB can configure the UE to initiate/terminate the data collection procedure. The potential impact of L3 signalling for the reporting of collected data should be assessed.

Moreover, multiple existing data collection methods have been analyzed in terms of the maximum payload size per reporting, end-to-end report latency, report type and security information. According to Table 7.3.1.2-1 [2], L1 measurements (CSI reporting) have indeed the smallest air interface signaling latency of 1 TTI over PUCCH but come along with the restricted payload size of 1706 bits in PUCCH and 3840 bits in PUSCH and no AS security. In comparison, L3 measurements support approximately 9kbyte of payload capacity at the cost of increased latency of approx. 20 ms when transmitted via RRC signalling.

In general, data can be collected for multiple purposes, including model inference, monitoring, training and re-training, which can be listed regarding where the AI/ML model or functionality is located:

| NW-sided model | UE-sided model |
| --- | --- |
| - Reporting for model training or re-training - Reporting for model inference - Reporting for model monitoring - Reporting for AI/ML model/functionality management | - Reporting based on the results of model inference - Reporting for model monitoring |

Each of the above-mentioned purposes might have its own set of requirements, including the amount of data to be reported, acceptable latency, security and reliability. Therefore, it is important to identify and analyze them separately to enable a proper selection of the container for reporting purposes.

Proposal 11: Prior to the selection of a container for data collection, study the AI/ML purposes separately in terms of the amount of data to be reported, acceptable latency, security and reliability.

At the previous 3GPP meeting, a beam report sent from the UE to the network for inference purpose has been agreed to be signaled in Layer 1 for NW-sided models, which is primarily driven by efficiency and timeliness of the Layer 1. However, training or re-training purposes for the NW-side model are characterized by a relaxed latency requirement and come along with a larger payload size per UE report for beam management use case, since both measurements for Set A and Set B are necessary. Therefore, Layer 3 container might be used for this case to report raw L1-RSRP measurements given its increased payload size, relaxed latency requirement and higher reliability based on the usage of error-correction mechanisms.

Proposal 12: Support L3 measurements as a container for L1-RSRPs reporting for training/re-training purposes given its increased payload size, relaxed latency requirement and higher reliability based on the usage of error-correction mechanisms.

## Beam Information

When M is smaller than the measurement resource set, the beam information for the reported beams has to be reported. Different methods have been discussed in the past. The current CSI framework employs the CRI/SSBRI. However, depending on the number of reported beams the overhead of different reporting methods varies significantly in size. The following table summarizes the bit sizes associated with different beam information.

Table 1: Bit sizes of different beam information indications.

| Indexing | N – number of measured/predicted beams M – number of reported beams |
| --- | --- |
| CRI/SSBRI | M∙log2N |
| Bitmap | M+log2N |
| Combinatorial | log2N-1M-1+log2N |

The following figures show the report size overhead due to beam information for the different indications.

Figure 2: Overhead in bits due to beam information for M=8, M=32 and M=64, respectively.

Clearly, it can be noted that the bitmap-based indication has the lowest offset in almost all ranges of N. However, on the other side it comes with the drawback that information on the ordering of beams is lost. Nevertheless, in combination with the RSRP information the ordering can be reconstructed without any issues at the network side.

Observation 1: Bitmap indexing results in the lowest overhead compared to CRI/SSBRI and Combinatorial Indexing.

Proposal 13: For NW-sided models, adopt bitmap-based indexing for reporting measurements.

## Quantization enhancements

To enhance the quantization, mainly three different dimensions of potential improvement can be classified:

- Multi-resolution quantization,

- Increased step sizes,

- Adaptive reference beam for differential L1-RSRPs.

Each of these adaptions can work on their own but also in combination with each other. We have summarized the Pros and Cons of the different approaches in the table below.

|  | Pros | Cons |
| --- | --- | --- |
| Multi-resolution quantization: Higher resolution for stronger beams and lower resolution for weaker beams, i.e. adaptive step size depending on the strength of beams | - Balanced loss of information | - Running more complex AI/ML models can lead to increased energy and battery consumption. |
| Increased step sizes: Increase the step size, in particular for the differential RSRPs | - Lowest specification effort | - Highest loss of information |
| Adaptive reference beam for differential L1-RSRPs: Instead of defining the differential RSRPs with reference to the strongest beam, the differential RSRP can be defined with reference to the n-th strongest or next-strongest beam | - Lowest loss of information | - Potential signaling overhead. - Running more complex AI/ML models can lead to increased energy and battery consumption. |

Proposal 14: Consider the following approaches to enhance the quantization of differential RSRPs:

- Multi-resolution quantization,

- Increased step sizes,

- Adaptive reference beam for differential RSRPs.

# Beam indication for UE-sided and NW-sided models

In RAN1#116, the use of a predicted beam that is not measured/received by the UE for beam indication was discussed. As discussed earlier, the beam reporting shall include predicted beams with predicted L1-RSRP values. The use of predicted beams when they are among top-K beams should be enabled not just for seamless beam management, but also for robustness of beam switching or refinement. RAN1 shall study the QCL references to be used by the UE when beams that are not measured/received are used in beam indication.

Proposal 15: The use of a predicted beam that is not measured/received by the UE for beam indication is supported.

Another aspect of beam indication discussed in RAN1#116 was the indication of beams for future time instances. The following issues with such a beam indication need to be addressed:

- With a unified TCI indication possible via the DCI itself, there is very little advantage in terms of latency that such a beam indication offers.

- It is possible that such a beam indication involves including multiple beams in the same TCI state or indicating multiple beams via multiple TCI states. The former method involves specifying a new TCI-state configuration comprising multiple beams, the latter involves interpretation of a DCI field with multiple beams applied at different times. Note, that this method should also be differentiated from previous schemes involving TDM-based MTRP schemes. Both methods involve specifying/indicating the UE behaviour for application of the beams at different times.

- In terms of overhead, indication of multiple beams incurs higher DCI overhead in one instance, while reducing the overhead in some future instances. The overall DCI overhead across these multiple instances could be very similar compared to legacy beam indication.

Beam indication for multiple future time instances offers very little advantage in terms of latency and overhead but incurs high specification workload and considerable modification in UE behaviour. Therefore, it is preferred to not have such a beam indication.

Proposal 16: Beam indication for one or more future time instances is not supported.

# Others

An additional issue arises due to different processing capabilities of UEs and the complexity of the used models. Depending on these parameters, the time required at the UE to obtain the prediction can vary significantly. Nevertheless, one can apply a worst-case inference time which fits all models. However, this unnecessarily sacrifices the overall performance due to delayed reporting of the inference outcome. Hence, the UE needs be able to report is inference time to the gNB, so that the inference reporting procedure can be adapted to the UE’s performance.

Proposal 17: For UE-sided models, for inference, study the UE reporting its inference time to the gNB.

# Conclusion

In this contribution, we have the following proposals:

Proposal 1: For monitoring UE-sided models, support 2-phase monitoring with varying frequencies and reporting detail.

Proposal 2: For monitoring, support monitoring of only a subset of Set A beams.

Proposal 3: Consider indication-based and event-based switching into a validation phase. Events may be defined based on agreed performance metrics.

Proposal 4: For the activation of monitoring, support Event-1, Event-4 and Event-5.

Proposal 5: For BM-Case2, for inference, the reference time of the predicted time instances shall be determined based on the CSI reference resource or the transmission occasion of Set B resources.

Proposal 6: For BM-Case2, for UE-sided models, introduce a configuration that allows the exclusion of certain past measurements for the inference.

Proposal 7: Explore the possibility of using the resources configured for radio link monitoring for model monitoring purposes with respect to CSI beam reporting.

Proposal 8: Support a model monitoring configuration that allows for collecting data for model training and monitoring of inactive models.

Proposal 9: Study the support for radio link monitoring and link recovery procedures considering spatial and temporal beam prediction at the UE-side.

Proposal 10: Support Alt 2, one CSI-ResourceConfigId is configured for both Set A and Set B, and the Set B of beams is provided as a subset of the CSI resource configuration.

Proposal 11: Prior to the selection of a container for data collection, study the AI/ML purposes separately in terms of the amount of data to be reported, acceptable latency, security and reliability.

Proposal 12: Support L3 measurements as a container for L1-RSRPs reporting for training/re-training purposes given its increased payload size, relaxed latency requirement and higher reliability based on the usage of error-correction mechanisms.

Proposal 13: For NW-sided models, adopt bitmap-based indexing for reporting measurements.

Proposal 14: Consider the following approaches to enhance the quantization of differential RSRPs:

- Multi-resolution quantization,

- Increased step sizes,

- Adaptive reference beam for differential RSRPs.

Proposal 15: The use of a predicted beam that is not measured/received by the UE for beam indication is supported.

Proposal 16: Beam indication for one or more future time instances is not supported.

Proposal 17: For UE-sided models, for inference, study the UE reporting its inference time to the gNB.

# References

- 3GPP RP-234039, “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, 3GPP TSG-RAN #102, Edinburg, Scotland, Dec 11th – Dec 15th, 2023.

- 3GPP TR38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”, version 18.0.0.