3GPP TSG RAN WG1 #119                                R1- 2410285

Orlando, US, November 18th – 22nd, 2024

Agenda Item:	9.4.1.1

Source: 	LG Electronics

Title: 	Discussion on Ambient IoT evaluation

Document for:	Discussion and decision

- Introduction

The Re1-19 SI on solutions for Ambient IoT was approved in RAN#102 meeting. The SI objective relevant to this agenda item is as follows [1].

## 4 Objective ### 4.1 Objective of SI or Core part WI or Testing part WI … - Evaluation assumptions - Conclude at least the following aspects of design targets left to WGs in Clause 5 (RAN design targets) of TR 38.848 [RAN1]. - Clause 5.3: Applicable maximum distance target values(s) - Clause 5.6: Refine the definition of latency suitable for use in RAN WGs - Clause 5.8: 2D distribution of devices - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations [RAN1, RAN4] - Identify basic blocks/components of possible Ambient IoT device architectures, taking into account state of the art implementations of low-power low-complexity devices which meet the RAN design target for power consumption and complexity. [RAN1] - Define link budget calculation for coverage, including whether/how to model carrier wave from node(s) inside or outside the connectivity topology. NOTE: Assessment performance of the design targets is within the study of feasibility and necessity of proposals in the following objectives, e.g. by inspection of reference implementations in the field, simulations, analytically. NOTE: strive to minimize evaluation cases in RAN1. …

- Discussion

In this contribution, we present our views on Ambient IoT evaluation.

  - Remaining design targets

2.1.1 Applicable maximum distance target value

In RAN1#118bis, following was agreed about applicable maximum distance target.

Agreement For Rel-19 Ambient IoT, the target value(s) for applicable maximum distance are refined as follows, - For device 1 in D1T1: - a value within [10 - 20]m - For device 2a in D1T1: - a value within [20 - 30]m - For device 2b in D1T1: - a value within [30 - 50]m - For device 1 in D2T2: - 10 m - For device 2a in D2T2: - a value within [10 - 20]m - For device 2b in D2T2: - a value within [30 - 50]m The distance target is compared to the results which assumes mandatory assumptions (as agreed).

Based on the link budget template agreed in RAN1#117 and link budget table collected in RAN1#118, maximum distance that can be achieved may vary according to deployment scenarios, topologies, or spectrum. The reason is that various parameters (e.g. total Tx power, CW Tx power, antenna gain, etc.) can vary depending on deployment scenarios, topologies, or spectrum. For example, using UL spectrum has more stringent regulation of total Tx power compared to DL spectrum leading to smaller coverage. In this case, coverage enhancement methods may need to be applied to meet the expectation of maximum distance target. However, using UL spectrum may be a typical scenario (e.g. D2T2 scenario). As discussed in RAN1#118bis, mandatory assumptions (e.g. spectrum, path loss model, etc.) are assumed to compare the results for applicable maximum distance target.

According to the link budget result collected in RAN1#118bis, it is observed that R2D link is the bottleneck of D1T1/D2T2 scenarios due to high receiver sensitivity of AmIoT device. Based on our link budget analysis shown in the below table, it is observed that to achieve the applicable maximum distance target in A1/A2 scenarios as agreed above, receiver sensitivity below 30 dBm is required for AmIoT devices.

| Scenario | CW case | Device | Tx power | Ant gain | Rx BW | Receiver Sensitivity | MPL | Distance |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| D1T1-A1/A2 | Case 1-1 | Device 1 | 33 | 2 | 20MHz | -30dBm | 54.1 | 6.89 |
| D2T2-A1/A2 | Cases 2-2 | Device 1 | 23 | 0 | 20MHz | -30dBm | 40.9 | 4.47 |
| D1T1-A1/A2 | Case 1-1 | Device 1 | 33 | 2 | 20MHz | -40dBm | 64.1 | 26.32 |
| D2T2-A1/A2 | Cases 2-2 | Device 1 | 23 | 0 | 20MHz | -40dBm | 50.9 | 13.25 |

Table 1. (R2D) Link budget calculation in D1T1-A1/A2 and D2T2-A1/A2 scenario

Observation#1: Receiver sensitivity below 30 dBm is required for AmIoT device (device type 1) to meet the applicable maximum distance target (20m for D1T1, 10m for D2T2)

  - Latency

In RAN1#118, follow table was agreed to calculate latency for single-device.

Agreement Companies to report the calculation of single-device latency according to the follow tables Table XX: calculation of single-device latency for “inventory-only” Steps; Procedures; Time (us). Step A: A-IoT paging; A-IoT paging message. Gap A (if any). Step B: D2R data transmission.; A-IoT Random access procedure. Gap B1. D2R data transmission. Total time for one attempt, T1= Tstep_A+TGap_A+ Tstep_B. Average time considering all the re-attempts, e.g., T2 = T1 /(1-X). Note: the following is assumed - Data rate: 1kbps or 7kbps - The rate for re-attempt probability X = [10] % - Message size for each procedure: FFS. Table YY: calculation of single-device latency for “inventory and command” Steps; Procedures; Time (us). Step A: A-IoT paging; A-IoT paging message. Gap A (if any). Step B: D2R data transmission.; A-IoT Random access procedure. Gap B1. D2R data transmission. Gap B (if any). Step C: Data transmission; R2D data transmission. Gap C (if any). Step D (optional, pending RAN2 decision); D2R data transmission. Total time for one attempt, T1= Tstep_A+TGap_A+ Tstep_B+TGap_B+ Tstep_C+ +TGap_C +Tstep_D. Average time considering all the re-attempts, e.g., T2 = T1 / (1-X). Note: the following is assumed - Data rate: 1kbps or 7kbps - The rate for re-attempt probability X = [10] % - Message size for each procedure: FFS.

In this section, we calculate the single-device latency for “inventory” and “inventory and command” cases based on our views on message payload size and data rate for R2D and D2R. The table below shows the calculation of latency based on our understanding.

| Steps | Procedures |  | Time (us) |
| --- | --- | --- | --- |
| Step A: A-IoT paging | A-IoT paging message: 26 bit payload (including Query Rep.) |  | 3715 |
| Gap A (if any) |  |  | 200 |
| Step B: D2R data transmission | A-IoT Random access procedure | Msg 1: 16 bit payload | 2286 |
|  |  | Gap between Msg.1 and Msg.2 | 200 |
|  |  | Msg. 2:16 bit payload | 2286 |
|  | Gap B1 |  | 200 |
|  | D2R data transmission (Msg. 3): 96 bit payload |  | 13715 |
| Total time for one attempt, T1= Tstep_A+TGap_A+ Tstep_B |  |  | 22,602 |
| Average time considering all the re-attempts, e.g., T2 = T1 /(1-X) |  |  | 25,114 |
| Note: the following is assumed - Data rate for R2D and D2R = 7kbps - The rate for re-attempt probability X = 10% |  |  |  |

Table 2. Calculation of single-device latency for “inventory-only”

| Steps | Procedures |  | Time (us) |
| --- | --- | --- | --- |
| Step A: A-IoT paging | A-IoT paging message: 26 bit payload (including Query Rep.) |  | 3715 |
| Gap A (if any) |  |  | 200 |
| Step B: D2R data transmission | A-IoT Random access procedure | Msg 1: 16 bit payload | 2286 |
|  |  | Gap between Msg.1 and Msg.2 | 200 |
|  |  | Msg. 2:16 bit payload | 2286 |
|  | Gap B1 |  | 200 |
|  | D2R data transmission (Msg. 3): 96 bit payload |  | 13715 |
| Gap B (if any) |  |  | 400 |
| Step C: Data transmission | R2D data transmission: 96 bit |  | 13715 |
| Gap C (if any) |  |  | 200 |
| Step D (optional, pending RAN2 decision) | D2R data transmission: 16 bit |  | 2286 |
| Total time for one attempt, T1= Tstep_A+TGap_A+ Tstep_B+TGap_B+ Tstep_C+ +TGap_C +Tstep_D |  |  | 39,203 |
| Average time considering all the re-attempts, e.g., T2 = T1 /(1-X) |  |  | 43,559 |
| Note: the following is assumed - Data rate for R2D and D2R = 7kbps - The rate for re-attempt probability X = 10% |  |  |  |

Table 3. Calculation of single-device latency for “inventory and command”

Note that the message transmission duration is calculated based on the 7kbps data rate and only considered message payload size excluding X-amble, repetition, CRC, etc.

Observation#2: For the “inventory-only” case considering 10% re-attempt probability, assuming Msg.3 size of 96 bits, single-device latency is 25.114ms with 7kbps data rate in both R2D and D2R transmission

Observation#3: For the “inventory and command” case considering 10% re-attempt probability, assuming Msg.3 size of 96 bits and Step D D2R message size of 16 bits, single-device latency is 43.559ms with 7kbps data rate in both R2D and D2R transmission

  - Link budget

As discussed in RAN1#118, there is still a concern about coverage evaluation for RF-EH link is needed or not. Before discussing about necessity about coverage evaluation for RF-EH link, predefined threshold which is defined for Budget Alt-1 should be first clarified. Below table is the definition of Budget Alt-1.

Budget-Alt1: Receiver sensitivity is derived by a predefined threshold and no link-level simulation is needed for link budget calculation - The results rely on the received sensitivity and maximum transmit power, and directly calculate the maximum distance / pathloss based on these values and other related parameters. The link-level simulation performances, such as required SINR, can be satisfied for such case and no link-level simulation is needed for link budget calculation.

Predefined threshold may be defined in two manners. First way is to define two independent thresholds of turning on the EH circuit and threshold of turning on the Tx/Rx processing. Second way is to define a predefined threshold implying both threshold of turning on the EH circuit and threshold of turning on the Tx/Rx processing. When the predefined threshold is defined in the first manner, when threshold of turning on the EH circuit is higher than the threshold of turning on the Tx/Rx processing, coverage evaluation for RF-EH link may be needed. However, when the predefined threshold is defined in the second manner, since the threshold already implies both of the two thresholds, no separate coverage evaluation for the RF-EH link may be needed.

Proposal#1: Related to the discussion on whether coverage evaluation for RF-EH link is needed or not, clarification on the definition of the predefined threshold in Budget-Alt 1 is needed.

- Conclusion

In this contribution, we shared our views on Ambient IoT evaluation.

Observation#1: Receiver sensitivity below 30 dBm is required for AmIoT device (device type 1) to meet the applicable maximum distance target (20m for D1T1, 10m for D2T2)

Observation#2: For the “inventory-only” case considering 10% re-attempt probability, assuming Msg.3 size of 96 bits, single-device latency is 25.114ms with 7kbps data rate in both R2D and D2R transmission

Observation#3: For the “inventory and command” case considering 10% re-attempt probability, assuming Msg.3 size of 96 bits and Step D D2R message size of 16 bits, single-device latency is 43.559ms with 7kbps data rate in both R2D and D2R transmission

Proposal#1: Related to the discussion on whether coverage evaluation for RF-EH link is needed or not, clarification on the definition of the predefined threshold in Budget-Alt 1 is needed.

- References

RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR

- Appendix A

# 4 Objective ### 4.1 Objective of SI or Core part WI or Testing part WI This study targets a further assessment at RAN WG-level of Ambient IoT, a new 3GPP IoT technology, suitable for deployment in a 3GPP system, which relies on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications. The study shall provide clear differentiation, i.e. addressing use cases and scenarios that cannot otherwise be fulfilled based on existing 3GPP LPWA IoT technology e.g. NB-IoT including with reduced peak Tx power. General Scope The definitions provided in TR 38.848 are taken into this SI, and the following are the exclusive general scope: - The overall objective shall be to study a harmonized air interface design with minimized differences (where necessary) for Ambient IoT to enable the following devices: - ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - ≤ a few hundred µW peak power consumption1, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally. - X is to be decided in WGs. - Coverage design target: Maximum distance of 10-50 m with device indoors as per TR 38.848: “…a range that WGs can sub-select within”. - For Topologies 1 & 2 (UE as intermediate node under NW control) per TR 38.848, with no RRC states, no mobility (i.e. at least no cell selection/re-selection -like function), no HARQ, no ARQ. NOTE 1: It is to be understood that “≤ a few hundred µW” means WGs are not tasked with setting a particular value, and that it will be for WG discussions to determine if a presented design with corresponding power consumption satisfies the “≤ a few hundred µW” requirement. - Deployment Scenarios with the following characteristics, referenced to the tables in Clause 4.2.2 of TR 38.848: - Deployment scenario 1 with Topology 1 - Basestation and coexistence characteristics: Micro-cell, co-site - Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control - Basestation and coexistence characteristics: Macro-cell, co-site - The location of intermediate node is indoor - FR1 licensed spectrum in FDD. - Spectrum deployment in-band to NR, in guard-band to LTE/NR, in standalone band(s). - Traffic types DO-DTT, DT, with focus on rUC1 (indoor inventory) and rUC4 (indoor command). - From RAN#104, the study will assess whether the harmonized air interface design (per bullet ‘A’ above) can address the DO-A (Device-originated autonomous) use case, only to identify which part(s) of the harmonized air interface design (per bullet ‘A’ above) is/are not sufficient for the DO-A use case. Transmission from Ambient IoT device (including backscattering when used) can occur at least in UL spectrum.