3GPP TSG RAN WG1 #119                                                       R1-2410325

Orlando, US, November 18th – 22nd, 2024

Source: Google

Title:	On the gNB-to-gNB CLI and the UE-to-UE CLI handling

Document for: Discussion and Decision

# Introduction

The Rel-19 Work item for the evolution of NR duplex operation has been approved in RAN#102 [1]. One of the main constraints in effective SBFD operation is the gNB-to-gNB CLI and the UE-to-UE CLI.

Conventional TDD operation mitigate interference between neighboring UEs and gNBs by segregating UL and DL transmissions into distinct time slots/symbols, hence splitting the time domain between UL and DL. In contrast, SBFD allows simultaneous UL and DL transmissions within the same time slots/symbols at the gNB. This introduces the potential for:

- Inter-UE CLI: the UL transmission of one aggressor UE can degrade the DL signal reception quality at a neighboring victim UE.

- Inter-gNB CLI: the DL transmission of one aggressor gNB can interfere with the UL signal reception at a neighboring victim gNB.

Effective CLI mitigation techniques are needed to be specified in Rel-19.  The following WI objectives for the CLI mitigation have been captured as below:

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3]:

  - Support gNB-to-gNB co-channel CLI handling scheme(s) (the detailed schemes are to be down-selected from those in TR38.858 by RAN1#117)

  - Support UE-to-UE co-channel CLI handling scheme(s) (the detailed schemes are to be down-selected from those in TR38.858 by RAN1#117)

  - Note: Without dedicated optimization for dynamic/flexible TDD.

In this Tdoc, we share our views and propose techniques to specify the above enhancements.

# Remaining CLI handling issues

The SRS transmission from the aggressor UE may not be always guaranteed, e.g., due to collision with other uplink signals, and the aggressor UE may drop the SRS transmission. Then the victim UE may not be able to measure the SRS.

- Study the behaviour of the victim UE when the SRS transmission is dropped by the aggressor UE, e.g. due to collision with other UL signals.

Also, the L1-SRS-RSRP report from the victim UE may also overlap with other uplink signals, e.g., other CSI report, SRS, etc ... Then how to transmit the L1-SRS-RSRP report could be another issue to be discussed and addressed.

The UE can measure the L1-SRS-RSRP for each SRS resource based on the entire bandwidth of the SRS resource or on the overlapping between the SRS bandwidth and the UL subband of the victim UE.

It should also be discussed whether the UE is supposed to report the highest L1-SRS-RSRPs measured for all the SRS resources configured as CMR or if it should also report highest and lowest L1-SRS-RSRPs.

- The UE to report a specified/configurable number N of the highest L1-SRS-RSRPs from all the measured SRS resources of different potential agressor UEs.

The network can configure an SRS resource subset restriction, which indicates a subset of SRS resources from the SRS resources configured as CMR and the UE should report the L1-SRS-RSRP  based on the SRS resources in the SRS resource subset.

- The network to configure an SRS resource subset restriction and the UE to report the L1-SRS-RSRP  based on the SRS resources in the SRS resource subset.

For an SBFD UE with multiple antenna ports, it can measure and report the L1-SRS-RSRP per antenna port. If receiver diversity is in use by the SBFD UE, the reported L1-SRS-RSRP value should not be lower than the corresponding L1-SRS-RSRP of any of the individual receiver branches. If a multiple ports SRS resource is configured for L1-SRS-RSRP measurement, the UE can measure/report the L1-SRS-RSRP for each SRS port.

- If a multiple ports SRS resource is configured for L1-SRS-RSRP measurement, the UE can measure/report the L1-SRS-RSRP for each SRS port.

Regarding the configuration of ‘typeD’ QCL assumptions for the CLI measurement resource, we support the configuration of typeD’ QCL assumptions for L1 CLI measurement resource.

- Support the configuration of typeD’ QCL assumptions for L1 CLI measurement resource.

UE can be semi-statically  or dynamically scheduled to receive a DL signal and is also expecting an SRS transmission from a another UE for UE-to-UE CLI measurements based on different QCL -TypeD, the victim UE cannot receive both the SRS and the DL signal simultaneously.

Some solutions to explore include:

- Dropping SRS: when the SRS and the DL signal are configured with different QCL- Type D, the first UE is configured to prioritize the DL reception and drop the SRS reception (e.g., the L1-SRS-RSRP/SRS-RSSI) if the two transmissions are overlapping in time.

- Drop the DL signal: when the SRS  and the DL signal are configured with different QCL- Type D, the first UE can be configured to prioritize the SRS reception and drop the DL reception if the two transmissions are overlapping in time

- Scheduling restriction: scheduling restrictions could be specified to avoid such collision scenarios. For example, the UE is not expected to receive a DCI scheduling DL transmission overlapping with the reception of an SRS, where the DL transmission and the SRS may be configured with different QCL-TypeD properties. The gNB should ensure the SRS and the DL signal are configured with the same QCL-TypeD property.

- Priority rules: priority rules could be established to determine which signal – a DL data transmission or an SRS transmission should be prioritized for reception by the UE in the event of a scheduling conflict.

- If a vicitm UE is to receive a DL signal and is also expecting an SRS transmission from  agressor UE for UE-to-UE CLI measurements based on different QCL -TypeD, it cannot receive both the SRS and the DL signal simultaneously. Some solutions could be specified:

- Dropping SRS

- Dropping DL signal

- Follow priority rules

If priority rules are used, then a priority adjustment based on the CLI level can be spcified. The priority of an SRS configuration can be dynamically adjusted based on the level of CLI generated by the other UE, which is transmitting the SRS. If the interfering UE is causing a high level of interference (as indicated by a high CLI), the priority of its SRS measurements may be increased to facilitate more accurate interference assessment and mitigation.

While measuring inter-UE CLI based on SRS from adjacent UEs offers great benefits, UE complexity must be carefully considered. This approach can be computationally intensive and power-consuming for the UE.  To reduce the complexity, the victim UE could initiate CLI measurement upon detecting multiple DL transmission failures, suggesting potential CLI. The UE could then assess overall CLI using group-based simultaneous SRS transmissions from suspected aggressor UEs. Based on this assessment, the UE could dynamically adjust the frequency of CLI measurement and reporting, and reducing these activities when CLI levels fall below specified/configured thresholds.

- Inter-UE CLI measurement and reporting mechanisms should prioritize UE complexity reduction. This includes minimizing CLI measurement and reporting frequency and optimizing signaling overhead.

- For L1 based UE-to-UE CLI, support CLI level threshold  for semi-persistent/ aperiodic CLI reporting.

# Conclusion

In this contribution, we have presented several proposals to specify the support of the gNB-to-gNB CLI and the UE-to-UE CLI handling. We made the following proposals:

- Study the behaviour of the victim UE when the SRS transmission is dropped by the aggressor UE, e.g. due to collision with other UL signals.

- The UE to report a specified/configurable number N of the highest L1-SRS-RSRPs from all the measured SRS resources of different potential agressor UEs.

- The network to configure an SRS resource subset restriction and the UE to report the L1-SRS-RSRP  based on the SRS resources in the SRS resource subset.

- If a multiple ports SRS resource is configured for L1-SRS-RSRP measurement, the UE can measure/report the L1-SRS-RSRP for each SRS port.

- Support the configuration of typeD’ QCL assumptions for L1 CLI measurement resource.

- If a vicitm UE is to receive a DL signal and is also expecting an SRS transmission from  agressor UE for UE-to-UE CLI measurements based on different QCL -TypeD, it cannot receive both the SRS and the DL signal simultaneously. Some solutions could be specified:

- Dropping SRS

- Dropping DL signal

- Follow priority rules

- Inter-UE CLI measurement and reporting mechanisms should prioritize UE complexity reduction. This includes minimizing CLI measurement and reporting frequency and optimizing signaling overhead.

- For L1 based UE-to-UE CLI, support CLI level threshold  for semi-persistent/ aperiodic CLI reporting.

# References

- RP-234035 “New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, CMCC (Moderator, RAN1 VC), RAN#102, Edinburgh, Scotland, December 11-15, 2023.

- 3GPP TR 38.858 V18.0.0 “Study on Evolution of NR Duplex Operation”

- R1-2308336, Discussion on evaluation and methodologies on evolution of NR duplex operation, Huawei, HiSilicon

- R1-2307922, On Deployment scenarios and evaluation Methodology for NR duplex evolution, Qualcomm Incorporated

- R1-2306874, On the evaluation methodology for NR duplexing enhancements, Nokia, Nokia Shanghai Bell

- R1-2307922, On Deployment scenarios and evaluation Methodology for NR duplex evolution, Qualcomm Incorporated