3GPP TSG-RAN WG1 Meeting #119	  R1-2410326

Orlando, US, November 18 – 22, 2024

Agenda Item:	5

Source:	Huawei, HiSilicon

Title:	Discussion on the LS reply to RAN2 on functionality in AI/ML

Document for:	Discussion and Decision

# Introduction

After the RAN2#127 meeting, RAN2 has sent a LS [1] to RAN1 to ask questions on functionality reporting for beam management.

RAN2 agreed the following understandings on terminologies: Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling) Applicable functionalities refer to functionalities that the UE is ready to apply for inference Activated functionalities refer to functionalities already enabled for performing inference. RAN2 further has made following agreements and signalling procedure (see the attached figure) on applicable functionality reporting for beam management UE-sided model: - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - (Between “Step 3” and “Step 4”) UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration). - Step 5: 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not. RAN2 also agreed the applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5. FFS the initial activation state. FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation. FFS if multiple applicable functionalities can be activated at the same time. FFS what is the granularity of functionality. The above agreements were made based on the following assumptions: NW-side additional condition is assumed as associated ID in RAN2 (which is assumed by majority of companies). To further progress life cycle management for beam management UE-sided model, RAN2 has following questions for which RAN2 would like to check RAN1’s understanding: On General - Q1: In Step 2, what is the granularity of functionality? For example, whether it is a use case (e.g. beam management), whether it is a sub-use case (e.g. beam management Case 1), or others? On NW-side additional condition and configuration - Q2: What is the content of NW-side additional condition, i.e. is it correct the RAN2 assumption of a NW-side additional condition assumed as associated ID? - Q3: Is NW-side additional condition functionality specific? - Q4: RAN2 wonders what information is needed in Step 3 for UE to decide whether a functionality is applicable before Step 4. More specifically, RAN2 would like to ask the following questions (Q4-1 to Q4-5): - Q4-1: In RAN2, it is FFS whether NW-side additional condition is mandatory or optional. In order to discuss further, RAN2 would like to understand whether it is feasible for UE to decide the applicable functionalities without NW-side additional condition? - Q4-2: In RAN2, it is FFS whether configuration (e.g. inference configuration) other than NW-side additional condition can be included in Step 3. RAN2 would like to understand whether it is feasible and required for gNB to provide configuration (e.g. inference configuration) other than NW-side additional condition in Step 3 for UE to determine applicable functionalities? - Q4-3: For UE evaluating applicable functionality reporting, if the answer to Q4-2 is Yes, what is the relationship between NW-side additional condition and configuration (e.g. inference configuration)? For example, is NW-side additional condition part of inference configuration, or is inference configuration part of NW-side additional condition, or is NW-side additional condition separate from inference configuration, etc? - Q4-4: If the answer to Q4-2 is Yes, what is the content of configuration (e.g. inference configuration) for UE to determine applicable functionalities? - Q5: What is the content of applicable functionality reporting in Step 4? - Q6: What is the content of inference configuration in Step 5? On Functionality Activation - Q7: If inference configuration is provided in Step 3, does it activate the functionality immediately upon receiving Step 3? - Q8: If inference configuration is not provided in Step 3, does configuration in Step 5 activate the functionality immediately upon receiving Step 5? - Q9: If more than one functionality are configured in Step 3 or Step 5, whether multiple/all applicable functionalities can be activated? - Q10: Is L1/L2 signaling for functionality activation/deactivation needed?

In the RAN1#118bis meeting, the following agreements have been achieved related to Q1, Q2, and Q4-2/4-4.

Agreement Answer to Q2 in R1-2407604 as below: RAN 1 did not have agreement on the content of NW-side additional condition. RAN1 agreed to support associated ID and it can be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID, while FFS whether/how to define similar properties of a DL Tx beam or beam set/list.. Agreement RAN 1 further study the following options for applicability for inference for UE-side model: Option 1: - In Step 3, following configurations are provided from NW to UE: - 1) UE is allowed to do UAI reporting via OtherConfig, - 2)+3) NW configures one or more CSI-ReportConfig for inference configuration, where the associated ID may be configured in CSI framework as working assumption applied. - FFS on whether some IEs in the CSI report configuration can be removed or modified - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - In Step 4, UE reports applicability(ies) of the above CSI-ReportConfig - FFS on one or more of the above CSI-ReportConfig to be reported - FFS on activation (including when/how) of inference report after obtaining the applicability from UE Step 4 - FFS: whether Step 5 is needed, Option 2: - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - NW configures one set or multiple sets of inference related parameters - Note: the set of inference related parameters is not configured by CSI-ReportConfig - FFS on the set of inference related parameters, at least including: - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - The associated ID(s) may be configured - wherein the associated ID(s) may be - FFS: a) part of one set of the inference related parameters, or - FFS: b) independently from the one set of the inference related parameters. - In Step 4, UE reports applicability of the above one or multiple sets of inference related parameters, where the associated ID information may be associated. - In Step 5, NW configures configuration(s) for CSI report for inference Option 3: - In Step 3, following configurations are provided from NW to UE: - 1) UE is allowed to do UAI reporting via OtherConfig, - 2) The associated ID(s) may be provided to UE, e.g., a new RRC parameter. - In Step 4, UE reports by UAI - the applicable one or multiple sets of inference related parameters may be included. - FFS on the set of inference related parameters, at least including: - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - Note: not applicable may also be replied by UE - Note: if the inference related parameters are not supported for reporting, only the applicability(ies) or not is reported in Step 4. - the associated ID(s) may be included - FFS: a) as part of the inference related parameters, or - FFS: b) independently from the set of the inference related parameters. - In Step 5, NW configures configuration(s) for CSI report for inference. Note: There is no impact of configuring CSI report configuration for non-AI beam management in RRCReconfiguration. Agreement Incorporating below text into the general part of the LS In RAN1’s discussion of RAN 2 terminologies on beam management, - The concept/terminology “functionality“ of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-specific FGs - The concept/terminology “ functionality“ of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters or information/parameters indicated by UE - The Activated functionalities may be enabled based on CSI framework. Therefore, the meaning and the granularity of “functionality“ for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same, depends on certain option in RAN1, and the discussion is still ongoing. Agreement Answer to Q1 in R1-2407604 as below, In Step 2, RAN1 expects that UE reports its UE-capability information/parameters, i.e., Rel-19 AI/ML-specific FGs (including components and corresponding value ranges). These AI/ML-specific UE capability information/parameters will depend on how FGs are defined including the granularity, that will be discussed in RAN1 later in the WI..

This contribution discusses the remaining issues of the LS reply from the RAN1 perspective.

# Discussion on the framework of functionality-based LCM

Since RAN1 has not discussed on the functionality-based LCM yet, RAN1 needs to first achieve common understanding on the concept before answering the questions from RAN2. This section discusses a couple of fundamental issues related with functionality.

## Granularity/number of functionalities (related with Q1, Q9)

During the Rel-18 discussions, the functionality is defined as AI/ML-based feature/feature group enabled by configurations (seen as TR 38.843 in below). However, RAN1 has not clearly decided the granularity of the functionality, e.g., whether one functionality corresponds to a feature/feature group (i.e., use case of sub-use case), or finer granularity (such as a group of RRC parameters).

4.2.1 LCM Flavours …… In functionality-based LCM, network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signalling (e.g., RRC, MAC-CE, DCI). Models may not be identified at the Network, and UE may perform model-level LCM. Whether and how much awareness/interaction NW should have about model-level LCM requires further study. For functionality identification, there may be either one or more than one Functionalities defined within an AI/ML-enabled feature, whereby AI/ML-enabled Feature refers to a Feature where AI/ML may be used. Note: UE may have one AI/ML model for the functionality, or UE may have multiple AI/ML models for the functionality. For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models, functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG.

### Granularity of applicable functionalities (Q1)

In the RAN1#118bis meeting, it has been clarified by RAN1 that the meaning and the granularity of “functionality” for applicable functionalities, activated functionalities and supported functionalities may or may not be the same. E.g., supported functionality may imply the supported parameters/value ranges by UE capability, while applicable functionality could imply the input/output combination of one or more applicable models.

In the following, we will focus on the analysis of applicable functionality, with two options of its granularity.

Option 1: An applicable functionality corresponds to a use case (e.g., beam management corresponds to functionality#1, CSI prediction corresponds to functionality#2) or sub-use case (e.g., BM-Case 1 corresponds to functionality#1, BM-Case 2 corresponds to functionality#2). Under Option 1, there is only one applicable functionality for per [sub] use case, and any inference configuration (e.g., Set B/Set A/Observation window/Prediction window) or configured NW side additional condition for this sub use case should be applicable to the functionality.

Option 2: An applicable functionality corresponds to a group/combination of conditions/configurations. As one example, for BM-Case 1, Set B/Set A = 16/64 beams corresponds to functionality#1, and Set B/Set A = 32/128 beams corresponds to functionality#2. As another example, for BM-Case 2, N = 4 prediction instances corresponds to functionality#1, and N = 8 prediction instances corresponds to functionality#2. Under Option 2, UE may report more than one applicable functionality subject to one sub use case, each of which corresponds to specific inference configurations/conditions.

Due to limited scalability of the models, it may be challenging for UE side to develop and train a single model being scalable to all candidate inference configurations. As an alternative, it is more realistic to train multiple models each corresponding to a limited set of specific configuration(s). On the other hand, even for the same set of inference configurations, it is possible for UE side to train multiple models corresponding to different complexity and performance, e.g., one with high performance and high complexity, while another with low performance and low complexity.

If Option 1 is adopted, UE side has to pack all models subject to the same sub use case into a single functionality, and performs internal switch among the models based on the inference configurations from NW and preferred performance/complexity from UE. From NW perspective, as the model switching is transparent, the change of the performance is hardly controlled by NW. From UE perspective, when the functionality is activated, it has to always deploy the full package of models subject to the whole sub use case to the AI/ML processing memory, which consumes substantial storage memory. Otherwise, additional model switching delay (which is transparent to NW) may cause ambiguity on the inference timeline.

On the other hand, if Option 2 is adopted, the functionality has finer granularity with potentially smaller pack of models, e.g., UE can pack models with similar performance/complexity into one functionality. Thus, from NW perspective, it has finer awareness/controllability to the change of performance along with the change of the functionality, and from UE perspective, the needed AI/ML processing memory can be potentially reduced compared with Option 1, as only the model(s) subject to the activated functionality are deployed to the AI/ML processing memory, which could be a subset of the models trained for the whole sub use case.

Observation 1: Finer granularity of an applicable functionality may enable a potentially smaller number of models to be packed into this applicable functionality.

- It may benefit NW since it may have finer awareness/controllability to the change of performance along with the change of the applicable functionality.

- It may benefit UE to efficiently use the AI/ML processing memory since potentially smaller number of models are deployed to the AI/ML processing memory when the applicable functionality is activated.

### Number of simultaneously activated functionalities (Q9)

In RAN1#118bis, it has been agreed that multiple CSI reports can be configured/activated/triggered. This refers the principle that multiple CSI processing can run simultaneously in legacy CSI framework.

Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability.

In our view, AI/ML-based beam management should also support such flexibility to outperform the legacy CSI report. E.g., functionalities subject to different Set A/Set B can be activated with separate CSI reports for different carriers based on traffic load, or functionalities subject to different output types can be activated with separate CSI reports for different time domain behaviors (e.g., P-CSI, SP-CSI, A-CSI).

That is to say, multiple applicable functionalities can be activated simultaneously.

Figure 1 Multiple applicable functionalities can be activated with separate CSI reports

Proposal 1: Regarding the granularity/number of functionalities,

- The granularity of the applicable functionality should be a group/combination of conditions/configurations.

- Multiple applicable functionalities can be activated so that the corresponding CSI processing can run simultaneously.

## Representation of an applicable functionality (related with Q2, Q3, Q4, Q5, Q6)

Regarding the representation of an applicable functionality, it should include the information that have to be perceived by gNB, so that a functionality could be selected by gNB based on these perceivable factors. We will analyze the following candidate aspects.

### Inference configurations (Q4-4, Q5, Q6)

As mentioned previously, due to limited scalability of UE side model(s), an applicable functionality should be associated with a group/combination of inference configurations as demonstrated in the table in below. The inference configurations include input related parameters and output related parameters.

For input related parameters, it may at least include the following:

- Set B related configurations. E.g., number of beams in Set B, type of SSB or CSI-RS, distance/periodicity of SSB/CSI-RS, mapping from Set B beam(s) to Set A beams, etc.

- Observation window related configurations. E.g., number of observation instances, distance/periodicity of observation instances. Considering the observation window may be subject to UE implementation at least for P/SP-CSI-RS resources, whether this item needs to be explicitly reported can be FFS.

  - Note: When the number of observation instance(s) is 1, the functionality is subject to BM-Case 1 (e.g., Functionality#1/#2 in the following table); otherwise the functionality is subject to BM-Case 2 (e.g., Functionality#3 in the following table).

For output related parameters, it may at least include the following:

- Set A related configurations. E.g., number of beams in Set A, distance/periodicity of SSB/CSI-RS.

- Type of prediction output. E.g., predicted beam ID or predicted RSRP.

- Max number of predicted beams for per instance, i.e., value of maximum Top-K.

- Prediction window related configurations. E.g., number of predicted instances (i.e., value of maximum N), reference time of predicted instances, size/duration of prediction time unit.

  - Note: When the number of prediction instance(s) is 1, the functionality is subject to BM-Case 1 (e.g., Functionality#1/#2 in the following table); otherwise the functionality is subject to BM-Case 2 (e.g., Functionality#3 in the following table).

Table 1 Example of the reported applicable functionalities and their corresponding inference configurations

|  | Set B | Set A | Observation window | Value of Max Top-K | Value of Max N | Duration of prediction time unit | …… |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Functionality#1 | 8 beams | 32 beams | 1 | 4 | 1 | N/A | …… |
|  | 16 beams | 64 beams | 1 | 8 | 1 | N/A | …… |
| Functionality#2 | 16 beams | 128 beams | 1 | 8 | 1 | N/A | …… |
| Functionality#3 | 16 beams | 64 beams | 4 | 8 | 4 | 20ms | …… |
| …… | …… | …… | …… | …… | …… | …… | …… |

The above inference configurations should be reported by UE along with the applicable functionality in Step 4 of RAN2 LS. In addition, one applicable functionality reported by UE may include more than one combination of inference configurations. E.g., as shown in the table above, Functionality#1 includes two combinations of inference configurations with different numbers of input beams and output beams. Therefore, the specific inference configuration (including input related parameters and output related parameters) for the functionality still needs to be explicitly configured by gNB in Step 5 of RAN2 LS.

### Performance/requirements (Q5)

Different from the inference configuration, this part of the representation should be provided to the gNB (e.g., reported in Step 4) but do not need to be configured by gNB in a later step (e.g., Step 5). This part is used to describe the expected performance and complexity requirement of the functionality, including the following items [2]:

- Target performance/robustness. Providing this information benefits NW to quickly understand the expected performance under the current environment when receiving the functionality from UAI report, so that NW can select the appropriate functionality based on the trade-off between performance and UE requirements, e.g., between two functionalities one with better prediction accuracy but longer required timeline while the other with lower prediction accuracy but shorter timeline. Note the target performance includes the prediction accuracy, while the target robustness includes the confidence of performing inference with the functionality. If the target performance/robustness changes with environment, UE can update this information of the functionality via UAI report.

- Required inference timeline. Different models may have different inference complexities, including required processing timeline, required CSI processing unit (CPU), and required memory storage. Hence, these complexity related requirements should be reported for per functionality, e.g., in Step 4.

- Required CPU. Similar to inference timeline, the required CPU should be reported in together with the associated functionality.

- Required memory storage. Similar to inference timeline, the required AI/ML processing memory should be reported in together with the associated functionality.

### NW side additional condition (Q2, Q3, Q4-1, Q5)

Regarding NW side additional condition, RAN1 has the following agreements:

Agreement (RAN1#116bis) Further study, for the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, where the NW-side additional condition may at least impact UE assumption on beams of Set A/Set B: - Opt1: Based on associated ID (Referring to AI 9.1.3.3) - FFS on what can be assumed by UE with the same associated ID across training and inference - FFS on how associated ID is introduced, e.g., within CSI framework, or outside of CSI framework - Opt 2: Performance monitoring based - FFS details - Other options are not precluded. Agreement (RAN1#118) For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

That is to say, it is correct understanding of RAN2 that the NW side additional condition is assumed in forms of associated ID.

Assuming the granularity of functionality is a group/combination of conditions/configurations (i.e., Option 2 in Section 2.1), there may be mapping relationship between one or more associated ID and one or more functionality – it could be one to one, one to multiple, or multiple to one. As one example, one functionality may be associated with multiple associated IDs if the functionalities are trained to be generalized to the data subject to these associated IDs. As another example, multiple functionalities may be associated with one associated ID where the functionalities correspond to different inference configurations. Thus, from the UE report perspective (i.e., Step 4), UE can report the set of associated ID(s) along with the reported applicable functionality, i.e., with functionality specific manner.

Figure 2 Mapping between functionality and associated ID

On the other hand, it should be noted that the introduction of NW side additional condition for BM case is to alleviate the UE side burden on categorizing the data and to facilitate UE side to train models overfitted to the data subject to the associated ID, but it does not mean the UE side model can NOT work without it. Without the associated ID, the UE side can either categorize the data by its own data categorization implementation for training overfitting models, or train a generalized model by mixing the dataset from various NW side additional conditions.

Similar to performance/requirement, this part of the representation should be provided to the gNB (e.g., reported in Step 4) but do not need to be configured by gNB in a later step (e.g., Step 5).

Observation 2: Associated ID is not a mandatory part of the BM feature, i.e., UE side does not necessarily need to rely on the indication of associated ID for enabling the consistency of UE side models of beam management between training and inference.

- E.g., the UE side can either categorize the data by its own data categorization implementation for training overfitting models, or train a generalized model by mixing the dataset from various NW side additional conditions.

### UE side additional condition (Q5)

From the RAN2 agreement, the selection of UE side model between “Step 3” and “Step 4” is based on NW side additional condition and UE side additional condition. If UE side trains different models to adapt to different UE side additional conditions (e.g., power consumption, UE speed, etc.), these models should be packed into a single functionality rather than being separated to different functionalities, since these UE side additional conditions are not perceived by gNB. UE can select the model based on UE side additional condition with gNB transparent manner.

### Summary

Based on the above discussions from Section 2.2.1 to Section 2.2.4, the following proposal is provided regarding the content/representation of a functionality.

Proposal 2: Regarding the representation of an applicable functionality, it may include at least the following contents:

- Part 1: Inference configurations of the functionality, including input related parameters and output related parameters of at least the following:

  - Input related parameters: Set B related configurations, FFS observation window related configurations

  - Output related parameters: Set A related configurations, type of prediction output, max number of predicted beams for per instance, prediction window related configurations

- Part 2: Performance/requirements of the functionality, including at least target performance/robustness, required inference timeline, required CPU, required memory storage.

- Part 3: Applicable associated ID(s) of the functionality (if any).

- Note: Part 1 needs to be explicitly configured by NW in Step 5.

## Procedure and signaling for functionality alignment (related with Q4, Q7, Q8, Q10)

### Inference configuration procedure (Q4-2, Q4-3, Q4-4)

In the RAN2 LS, there is FFS on whether the inference configuration is provided from NW to UE in Step 3. This FFS is actually the question whether the configuration of the functionality is provided before Step 4 (UE reporting applicable functionalities), or after Step 4.

For the 3 options from RAN1#118bis agreement, analysis is provided in the following.

- Option 1: NW configures one or more CSI-ReportConfig for inference configuration at Step 3.

  - Pros: The inference configurations in Step 3 can serve to filter the supported models at UE side OTT server, so that UE could download a limited number of models satisfying the indicated inference configurations and associated ID (if any) before Step 4.

  - Cons: In Step 3, the corresponding CSI report is not actually enabled but “virtually” enabled (e.g., P-CSI report is on hold instead of immediately running after Step 3). NW needs to always reconfigure the actual CSI report after UE reports the applicable functionality at Step 4, as the applicable functionalities are a subset of the “virtually” configured functionalities in Step 3. This corresponds to a new rule different from the legacy CSI framework. In addition, the configuration of “virtual” CSI report may cause waste on the number of CSI reports as some of them configured in Step 3 will never be reconfigured in Step 5 due to inapplicability reported in Step 4. This incurs burden to NW with respect to unnecessary CSI report configuration and release.

- Option 2: NW configures one set or multiple sets of inference related parameters other than using CSI-ReportConfig at Step 3.

  - Pros: Similar to Option 1, inference configurations in Step 3 can filter the supported UE side models, so that the overhead for model downloading and memory for model storage can be saved. In addition, the CSI reports are only configured at Step 5 (i.e., CSI reports are not “virtually” in an earlier step), so that the procedure is more consistent with legacy CSI framework.

  - Cons: New RRC signalling is introduced to indicate the set of inference related parameters beyond the current CSI framework.

- Option 3: UE reports the applicable one or multiple sets of inference related parameters at Step 4, while NW configures configuration(s) for CSI report for inference at Step 5.

  - Pros: Similar to Option 2, the CSI reports are only configured at Step 5, which is more consistent with legacy CSI framework. Moreover, the handshake between gNB and UE is also consistent with legacy, where UE first reports the applicable feature, after which gNB enables the feature.

  - Cons: As UE does not have potential inference configuration information at Step 3, it may have less information than in Option 1/2 to guide the model downloading from OTT server before Step 4. However, it should be noted that the potential inference configuration information is relatively static, and can be obtained by UE side with implementation approaches, e.g., from historic UEs which have performed inference in the same cell. Such historic information can also serve to filter the supported models at UE side.

Observation 3: Regarding Option 1 for applicability for inference for UE-side model related to Q4-2, the CSI reports are “virtually” configured in Step 3, which may cause waste on the number of CSI reports as some of them will not be actually configured/activated in Step 5 due to inapplicability as reported in Step 4.

For Option 2 and Option 3, regarding the relationship between associated ID and inference configuration (Q4-3), as analyzed in our companion contribution [2] (Section 2.3.3), from the definition of additional condition, it is not a part of UE capability, therefore, it does not cover the inference configurations that could be reported/represented by UE capability, e.g., configuration of the size of Set A/Set B, Top-K, observation window size, prediction window size, etc. Therefore, associated ID is separate from inference configuration.

For Option 2 and Option 3, regarding the content of the set of inference related parameters (Q4-4), it can refer to our analysis in Section 2.2.1 and Section 2.2.5, with the difference that such input related parameters and output related parameters are indicated by NW at Step 3 for Option 2 while being reported by UE at Step 4 for Option 3.

Proposal 3: Regarding the procedure of inference configuration, deprioritize Option 1, and consider Option 2 and Option 3 with potential down selection, i.e., the CSI report for inference is only configured by NW in Step 5, which are more consistent with the CSI configuration/activation/triggering procedure of the legacy CSI framework.

- NW-side additional condition represented by associated ID is separate from inference configuration.

### Indication of a functionality (Q7, Q8)

In Step 4, UE may report one or more applicable functionality each with associated inference configurations, performance/requirements, and applicable associated ID(s) as analyzed in Section 2.2. Assuming the granularity of functionality is a group/combination of configurations (i.e., Option 2 in Section 2.1), there may be multiple functionalities corresponding to the same combination of configurations. Thus, NW may need to indicate the specific functionality ID in Step 5, e.g., by indicating the functionality ID inside the CSI-ReportConfig.

Proposal 4: Regarding the indication of a functionality, NW indicates the functionality along with the inference configuration for the corresponding CSI report in Step 5.

### Signaling to activate the functionality (Q7, Q8, Q10)

As analyzed in our companion contribution [2] (Section 5.1.2), there is no need to introduce dedicated MAC CE or RRC signaling to activate or deactivate the model, since the activation of a model subject to beam management cases could be as fast as TTI level. A simpler way is to activate the functionality by configuring/activating/triggering a corresponding CSI report for P-CSI/SP-SCI/A-CSI (which could reuse the legacy CSI report signaling as much as possible), and incorporate the model activation delay into the inference timeline.

E.g., as shown in the figure in below, for P-CSI report, it is configured by P-CSI configuration; for SP-CSI, it is activated by MAC CE or activation DCI; for A-CSI, it is triggered by DCI. It should be noted that for P-CSI report and SP-CSI report, the functionality is activated before the reported CSI on PUCCH/PUSCH (model deployed to AI/ML processing memory) and deactivated after the CSI report (model flushed from AI/ML processing memory), so that the AI/ML processing memory is efficiently used.

Figure 3 Signaling to configure/activate/trigger the P-CSI/SP-CSI/A-CSI report, respectively

Following this principle, the functionality does not need to be activated immediately upon receiving Step 5. For P-CSI and SP-CSI, the functionality is activated based on the time instance of the CSI report, e.g., earlier than the PUCCH/PUSCH carrying the CSI, rather than immediately activated upon receiving the RRC signaling configuring the CSI report configurations. For A-CSI, the functionality is activated after receiving the DCI triggering the CSI report, so it is not immediately activated upon receiving the RRC signaling configuring configurations either.

Proposal 5: Regarding the signaling to activate a functionality, it can be achieved by reusing the legacy signaling of CSI framework to configure/activate/trigger a corresponding P-CSI/SP-CSI/A-CSI report.

- The functionality is not immediately activated upon receiving the RRC signaling configuring configurations for P-CSI/SP-CSI/A-CSI report.

# Replies to RAN2 questions

RAN1 has achieved agreement on the replies to Q1 and Q2 in RAN1#118bis meeting, so we discuss the reply to Q3~Q10 in the following.

## Reply to Q3

The question by itself is not crystal clear: NW side additional condition is provided by NW, while the functionality is reported by UE, depending on the UE side model development and training. If the assumption of RAN2 is that the granularity of an applicable functionality is a use case or sub use case, it may be correct to say the associated ID is use case specific or sub use case specific.

However, if the assumption is that an applicable functionality has finer granularity, how NW indicates the associated ID information at the training phase is NW implementation, and NW has no information of the applicable functionality which is to be reported by UE at inference phase.

For the latter assumption (i.e., with finer granularity, which is preferred by us), based on the discussions at Section 2.2.3 “NW side additional condition (Q2, Q3, Q4-1, Q5)”, we can clarify that UE can report the set of associated ID(s) along with the reported applicable functionality in Step 4 in functionality specific manner (subject to Option 2 and Option 3 of inference configuration procedure).

Proposal 6: For Q3 of the LS, reply as follows:

- Assuming the granularity of the applicable functionality is a group/combination of conditions/configurations, i.e., applicable functionality has finer granularity than a sub use case, RAN1 would like to clarify that how NW side indicates the NW side additional condition subject to a use case or sub use case is NW implementation irrespective of the applicable functionality.

  - On the other hand, UE can report the set of associated ID(s) corresponding to the reported applicable functionality in Step 4 with functionality specific manner.

## Reply to Q4

Reply to Q4-1

From NW perspective, whether to provide associated ID is always optional. If the intention is to say, whether it is possible for UE to support the AI/ML-based beam management feature by relying on (namely mandatory) or without relying on (namely optional) the indication of associated ID, then based on the discussions at Section 2.2.3 “NW side additional condition (Q2, Q3, Q4-1, Q5)”, UE side does not necessarily need to rely on the indication of associated ID for enabling the consistency of UE side models of beam management between training and inference.

Proposal 7: For Q4-1 of the LS, reply as follows:

- Provision of NW side additional condition is optional.

- It is feasible for UE to decide the applicable functionalities without NW-side additional condition.

  - E.g., the UE side can either categorize the data by its own data categorization implementation, or train a generalized model by mixing the dataset from various NW side additional conditions.

Reply to Q4-2

Based on the discussions at Section 2.3.1 “Inference configuration procedure (Q4-2, Q4-3, Q4-4)”, Option 2 and Option 3 are preferred with higher priority, i.e., NW configures the CSI report configurations to UE in Step 5 after applicable functionality reporting in Step 4.

Proposal 8: For Q4-2 of the LS, reply as follows:

- RAN1 is discussing whether one set or multiple sets of inference related parameters are configured in Step 3 (RAN1 Option 2) or reported in Step 4 (RAN1 Option 3).

- NW configures configuration(s) for CSI report for inference to UE in Step 5 after applicable functionality reporting in Step 4.

Reply to Q4-3

Based on the discussions at Section 2.3.1 “Inference configuration procedure (Q4-2, Q4-3, Q4-4)”, regardless one set or multiple sets of inference related parameters are configured in Step 3 (Option 2) or reported in Step 4 (Option 3), NW-side additional condition represented by associated ID is separate from inference configuration.

Proposal 9: For Q4-3 of the LS, reply as follows:

- Regardless the answer to Q4-2 is “Yes” or “No”, it can be understood that inference configuration is separate from NW side additional condition.

Reply to Q4-4

Based on the discussions at Section 2.3.1 “Inference configuration procedure (Q4-2, Q4-3, Q4-4)”, if the one set or multiple sets of inference related parameters are configured in Step 3 (RAN1 Option 2), i.e., answer to Q4-2 is “Yes”, the content of the inference related parameters can refer to the FFS part of the RAN1 agreement on Option 2.

If the one set or multiple sets of inference related parameters are reported in Step 4 (RAN1 Option 3), i.e., answer to Q4-2 is “No”, we do not need to reply to Q4-4. UE does not need to determine the applicable functionalities based on NW indicated inference configuration. Actually, UE can report all supported inference configurations in together with the report of applicable functionalities at Step 4.

Proposal 10: For Q4-4 of the LS, reply as follows:

- If the one set or multiple sets of inference related parameters are configured in Step 3 (RAN1 Option 2), i.e., answer to Q4-2 is “Yes”, the content of inference related parameters include: Set A related information, Set B related information, report content related information, time instances related information for measurements, and time instances related information for prediction.

- If the one set or multiple sets of inference related parameters are reported in Step 4 (RAN1 Option 3), i.e., answer to Q4-2 is “No”, then Q4-4 is not applicable, i.e., UE does not determine the applicable functionalities based on inference configuration indicated by NW.

## Reply to Q5

Based on the discussions at Section 2.2.1 “Inference configurations (Q4-4, Q5, Q6)”, Section 2.2.2 Performance/requirements (Q5), and Section 2.2.3 “NW side additional condition (Q2, Q3, Q4-1, Q5)”, we can provide the following reply on the content of a functionality in Step 4.

Proposal 11: For Q5 of the LS, reply as follows:

- Regarding the content of a functionality, UE can report the following information in Step 4 corresponding to an associated applicable functionality:

  - Part 1: Inference configurations of the functionality, including input related parameters and output related parameters of at least the following:

    - Input related parameters: Set B related configurations, FFS observation window related configurations.

    - Output related parameters: Set A related configurations, type of prediction output, max number of predicted beams for per instance, prediction window related configurations.

  - Part 2: Performance/requirements of the functionality, including at least target performance/robustness, required inference timeline, required CPU, required memory storage.

  - Part 3: Applicable associated ID(s) of the functionality (if any).

## Reply to Q6

Based on the discussions at Section 2.2.1 “Inference configurations (Q4-4, Q5, Q6)”, the content of the inference configuration includes the contents listed in Section 2.2.1, but does not include the contents listed in Section 2.2.2 and Section 2.2.3.

Proposal 12: For Q6 of the LS, reply as follows:

- Regarding the content of inference configuration in Step 5, it may include at least the following input related parameters and output related parameters:

  - Input related parameters: Set B related configurations, FFS observation window related configurations.

  - Output related parameters: Set A related configurations, type of prediction output, max number of predicted beams for per instance, prediction window related configurations.

## Reply to Q7

Based on the discussions at Section 2.3.3 “Signaling to activate the functionality (Q7, Q8, Q10)”, the functionality can be activated by reusing the legacy signaling of CSI framework to configure/activate/trigger a corresponding CSI report, i.e., the functionality is not activated immediately upon receiving Step 3.

Proposal 13: For Q7 of the LS, reply as follows:

- Regardless the inference related parameters are configured in Step 3 (RAN1 Option 2) or reported in Step 4 (RAN1 Option 3), the functionality is not activated immediately upon receiving Step 3.

## Reply to Q8

Based on the discussions at Section 2.3.3 “Signaling to activate the functionality (Q7, Q8, Q10)”, the functionality is activated based on the time instance of the CSI report, e.g., starting at a time slot earlier than the PUCCH/PUSCH carrying the CSI, rather than immediately activated upon receiving the RRC signaling configuring the CSI report configurations. That is to say, the functionality does not need to be activated immediately upon receiving Step 5.

Proposal 14: For Q8 of the LS, reply as follows:

- The functionality does not need to be activated immediately upon receiving Step 5.

## Reply to Q9

Based on the discussions in Section 2.1.2 “Number of simultaneously activated functionalities (Q9)”, we prefer the multiple applicable functionalities can be activated simultaneously.

Proposal 15: For Q9 of the LS, reply as follows:

- If more than one functionality is configured in Step 5, multiple applicable functionalities can be activated as separate CSI reports so that the corresponding CSI processing can run simultaneously.

## Reply to Q10

Based on the discussions at Section 2.3.3 “Signaling to activate the functionality (Q7, Q8, Q10)”, the functionality can be activated by reusing the legacy signaling of CSI framework to configure/activate/trigger a corresponding CSI report.

Proposal 16: For Q10 of the LS, reply as follows:

- The signalling to activate of a functionality can reuse the legacy signaling of CSI framework to configure/activate/trigger a corresponding CSI report. In particular,

  - If the functionality is activated with P-CSI report, the activation signalling is RRC signalling.

  - If the functionality is activated with SP-CSI report, the activation signalling is MAC CE (L2 signalling) or activation DCI (L1 signaling).

  - If the functionality is activated with A-CSI report, the activation signalling is DCI (L1 signaling).

# Conclusions

In this contribution, we analyzed the principles related with functionality, and draft replies to the questions raised by the RAN2 LS. The observations and proposals are provided in below.

Observation 1: Finer granularity of an applicable functionality may enable a potentially smaller number of models to be packed into this applicable functionality.

- It may benefit NW since it may have finer awareness/controllability to the change of performance along with the change of the applicable functionality.

- It may benefit UE to efficiently use the AI/ML processing memory since potentially smaller number of models are deployed to the AI/ML processing memory when the applicable functionality is activated.

Observation 2: Associated ID is not a mandatory part of the BM feature, i.e., UE side does not necessarily need to rely on the indication of associated ID for enabling the consistency of UE side models of beam management between training and inference.

- E.g., the UE side can either categorize the data by its own data categorization implementation for training overfitting models, or train a generalized model by mixing the dataset from various NW side additional conditions.

Observation 3: Regarding Option 1 for applicability for inference for UE-side model related to Q4-2, the CSI reports are “virtually” configured in Step 3, which may cause waste on the number of CSI reports as some of them will not be actually configured/activated in Step 5 due to inapplicability as reported in Step 4.

Proposal 1: Regarding the granularity/number of functionalities,

- The granularity of the applicable functionality should be a group/combination of conditions/configurations.

- Multiple applicable functionalities can be activated so that the corresponding CSI processing can run simultaneously.

Proposal 2: Regarding the representation of an applicable functionality, it may include at least the following contents:

- Part 1: Inference configurations of the functionality, including input related parameters and output related parameters of at least the following:

  - Input related parameters: Set B related configurations, FFS observation window related configurations

  - Output related parameters: Set A related configurations, type of prediction output, max number of predicted beams for per instance, prediction window related configurations

- Part 2: Performance/requirements of the functionality, including at least target performance/robustness, required inference timeline, required CPU, required memory storage.

- Part 3: Applicable associated ID(s) of the functionality (if any).

- Note: Part 1 needs to be explicitly configured by NW in Step 5.

Proposal 3: Regarding the procedure of inference configuration, deprioritize Option 1, and consider Option 2 and Option 3 with potential down selection, i.e., the CSI report for inference is only configured by NW in Step 5, which are more consistent with the CSI configuration/activation/triggering procedure of the legacy CSI framework.

- NW-side additional condition represented by associated ID is separate from inference configuration.

Proposal 4: Regarding the indication of a functionality, NW indicates the functionality along with the inference configuration for the corresponding CSI report in Step 5.

Proposal 5: Regarding the signaling to activate a functionality, it can be achieved by reusing the legacy signaling of CSI framework to configure/activate/trigger a corresponding P-CSI/SP-CSI/A-CSI report.

- The functionality is not immediately activated upon receiving the RRC signaling configuring configurations for P-CSI/SP-CSI/A-CSI report.

Proposal 6: For Q3 of the LS, reply as follows:

- Assuming the granularity of the applicable functionality is a group/combination of conditions/configurations, i.e., applicable functionality has finer granularity than a sub use case, RAN1 would like to clarify that how NW side indicates the NW side additional condition subject to a use case or sub use case is NW implementation irrespective of the applicable functionality.

  - On the other hand, UE can report the set of associated ID(s) corresponding to the reported applicable functionality in Step 4 with functionality specific manner.

Proposal 7: For Q4-1 of the LS, reply as follows:

- Provision of NW side additional condition is optional.

- It is feasible for UE to decide the applicable functionalities without NW-side additional condition.

  - E.g., the UE side can either categorize the data by its own data categorization implementation, or train a generalized model by mixing the dataset from various NW side additional conditions.

Proposal 8: For Q4-2 of the LS, reply as follows:

- RAN1 is discussing whether one set or multiple sets of inference related parameters are configured in Step 3 (RAN1 Option 2) or reported in Step 4 (RAN1 Option 3).

- NW configures configuration(s) for CSI report for inference to UE in Step 5 after applicable functionality reporting in Step 4.

Proposal 9: For Q4-3 of the LS, reply as follows:

- Regardless the answer to Q4-2 is “Yes” or “No”, it can be understood that inference configuration is separate from NW side additional condition.

Proposal 10: For Q4-4 of the LS, reply as follows:

- If the one set or multiple sets of inference related parameters are configured in Step 3 (RAN1 Option 2), i.e., answer to Q4-2 is “Yes”, the content of inference related parameters include: Set A related information, Set B related information, report content related information, time instances related information for measurements, and time instances related information for prediction.

- If the one set or multiple sets of inference related parameters are reported in Step 4 (RAN1 Option 3), i.e., answer to Q4-2 is “No”, then Q4-4 is not applicable, i.e., UE does not determine the applicable functionalities based on inference configuration indicated by NW.

Proposal 11: For Q5 of the LS, reply as follows:

- Regarding the content of a functionality, UE can report the following information in Step 4 corresponding to an associated applicable functionality:

  - Part 1: Inference configurations of the functionality, including input related parameters and output related parameters of at least the following:

    - Input related parameters: Set B related configurations, FFS observation window related configurations.

    - Output related parameters: Set A related configurations, type of prediction output, max number of predicted beams for per instance, prediction window related configurations.

  - Part 2: Performance/requirements of the functionality, including at least target performance/robustness, required inference timeline, required CPU, required memory storage.

  - Part 3: Applicable associated ID(s) of the functionality (if any).

Proposal 12: For Q6 of the LS, reply as follows:

- Regarding the content of inference configuration in Step 5, it may include at least the following input related parameters and output related parameters:

  - Input related parameters: Set B related configurations, FFS observation window related configurations.

  - Output related parameters: Set A related configurations, type of prediction output, max number of predicted beams for per instance, prediction window related configurations.

Proposal 13: For Q7 of the LS, reply as follows:

- Regardless the inference related parameters are configured in Step 3 (RAN1 Option 2) or reported in Step 4 (RAN1 Option 3), the functionality is not activated immediately upon receiving Step 3.

Proposal 14: For Q8 of the LS, reply as follows:

- The functionality does not need to be activated immediately upon receiving Step 5.

Proposal 15: For Q9 of the LS, reply as follows:

- If more than one functionality is configured in Step 5, multiple applicable functionalities can be activated as separate CSI reports so that the corresponding CSI processing can run simultaneously.

Proposal 16: For Q10 of the LS, reply as follows:

- The signalling to activate of a functionality can reuse the legacy signaling of CSI framework to configure/activate/trigger a corresponding CSI report. In particular,

  - If the functionality is activated with P-CSI report, the activation signalling is RRC signalling.

  - If the functionality is activated with SP-CSI report, the activation signalling is MAC CE (L2 signalling) or activation DCI (L1 signaling).

  - If the functionality is activated with A-CSI report, the activation signalling is DCI (L1 signaling).

# References

R2-2407848, LS on applicable functionality reporting for beam management UE-sided model, 3GPP TSG RAN WG2 Meeting #127, August 19 - 23, Maastricht, Netherlands.

R1-2409395, “Discussion on AI/ML for beam management”, RAN1#119, Orlando, US, November 18 - 22, 2024.