3GPP TSG-RAN WG1 Meeting #119	R1-2410354

Orlando, US, November 18th – November 22nd, 2024

Agenda Item:	9.1.1

Source:	Ericsson

Title:	AI/ML for beam management

Document for:	Discussion

# Introduction

In RAN#102 meeting, a new WID is approved for Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface [2]. In this contribution, we focus on the normative work for the use case of beam management enhancements.

# General data collection enhancements

It is agreed that two resource sets can be configured for Set A and Set B separately in the CSI report configuration for the report. Majority of the evaluations results from the Rel-18 Study Item assumed 32 or 64 beams in Set A and 4, 8, 16, or even 32 in Set B. From the current specification in TS 38.331, it is noted that the maximum number of NZP CSI-RS resources per resource set (i.e., maxNrofNZP-CSI-RS-ResourcesPerSet) is 64 and the maximum number of aperiodic (AP) CSI-RS resources per resource set (i.e., maxNrofAP-CSI-RS-ResourcesPerSet) is 16. The aPeriodic reporting configuration is needed to ensure efficient data collection, for example activated only when NW have sufficient resources available. However, given the current limitation, set A or set B can have a maximum of 16 beams if aPeriodic reporting should be supported.

Two resource sets can be configured for Set A and Set B separately in the CSI report configuration for the report, but only 16 beams (i.e., aperiodic CSI-RS resources) within a resourceSet are supported for AP CSI report.

It is unclear whether Set A needs to be larger than 64 or not. At least the number of CSI-RS resources for AP and P should be the same, and hence we should have 64 beams as a starting point for the resourceSet in the aPeriodicReportingConfiguration.

For UE-sided or NW-sided data collection, at least support UE measurement of a resourceSet with 64 NZP CSI-RS resources in the aperiodic reporting configuration

  - FFS on whether to support a resourceSet larger than 64 resources for aPeriodic or periodic measurements

# UE-sided model

## Configuration of Set A and Set B for UE-sided model

Agreement For UE-sided model at least for BM Case-1, CSI-ReportConfig is used for the configuration of inference results reporting - FFS on the details in the CSI-ReportConfig, at least considering: - Alt 1: one CSI-ResourceConfigId is configured for Set B - FFS: how UE can determine the information about set A - Alt 2: one CSI-ResourceConfigId is configured for both Set A and Set B - FFS: How to configure resource set(s) for Set A and Set B in CSI-ResourceConfig - Alt 3: two CSI-ResourceConfigId s are configured for Set A and Set B separately - Alt 4: one CSI-ResourceConfigId is configured for Set B, Set A is configured using separate resource set(s) other than that represented by CSI-ResourceConfigId - FFS: how to configure/indicate separate resource set(s) for Set A - Note: separate CSI-ReportConfig for Set A and Set B are not precluded. - Note: Not perform measurement for Set A and only perform measurement for Set B subject to the CSI-ReportConfig - FFS on the association between Set A and Set B with or without additional IE - Other necessary configuration are not precluded. Agreement For UE-sided model at least for BM Case-1, for inference results report - Two resource sets can be configured for Set A and Set B separately in the CSI report configuration for the report - FFS whether support only resource set for Set B is configured - UE performs measurement on the resource set for Set B for inference, and UE is not expected to measure resource set for Set A for inference, - The beam information in the inference report refers to the resource set for Set A

### Configuration of Set A and Set B for Inference Reporting

Alt 1 and 4: one CSI-ResourceConfigId is configured for Set B

Regarding alternative 1, our understanding is that resourceSet A is configured by directly relating to the ResourceSet  and not another CSI-ResourceConfig. Since the working assumption is that the CSI framework should be used, our understanding is that such solution comprises that the NW configures the ResourceSetA in the CSI report configuration, for example as the reporting quantity. This has the benefit of separating the measured resources from the predicted resources, and the NW does not need to indicate that the setA resourceSet is “virtual”. However, having resourceSets indicated as part of a report quantity may lead to unnecessary complex specification impact. Hence, our view is that alternative 2 or 3 are simpler solutions.

Alt 2: one CSI-ResourceConfigId is configured for both Set A and Set B/ Alt 3: two CSI-ResourceConfigIds are configured for Set A and Set B separately

Regarding alternative 2, this requires that the NW to configure multiple resourceSets in one ResourceConfiguration, and indicate if each resourceSet belongs to set A or set B. The UE should also be able to indicate for which resourceSet that it reports.

In Alt 3), the UE receives two resourceConfigurations and an indication of whether the resourceConfiguration is for set A or set B. This alternative also enables the NW to configure multiple set A within one resource configuration similar to Alt 2. The differences between alternative 2 and 3 are minor, it is mainly on what level set A/B resourceSets are indicated.  Alt 3 is slightly preferrable since using separate ResourceConfigs for Set A and Set B allow different resource types to be configured for Set A and Set B (note that resource type is indicated in ResourceConfig).

For UE-sided model, regarding configuration of set A/B, support alternative 3.

In some deployment scenarios, to reduce potential inter-cell interference, it may be desirable to avoid DL transmissions at certain spatial directions (e.g., direction pointing towards neighbouring cells). In NR, this can be achieved via codebook subset restriction (CBSR) that is specified for CSI feedback. For the case where the UE predicts a beam using a UE sided model, it is likely that the predicted beam may potentially cause interference to adjacent cells. From the perspective of reducing/controlling inter-cell interference, it is beneficial to specific beam subset restriction using similar principle as CBSR that currently exists for CSI feedback. Hence, we make the following proposal:

For UE-sided model inference, enable NW to specify set A beam subset restriction similar to codebook subset restriction (CBSR) that is specified for CSI feedback

### Configuration for temporal beam prediction

Agreement For BM-Case 2 of UE-side model, for the reference time of the earliest time instance for the predicted results, consider at least the following alternatives for potential down-selection: - Option 1: Based on the uplink slot for the report - Option 2: Based on the CSI reference resource corresponding to the report - Option 3: Based on the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report, wherein the transmission occasion is no later than the CSI reference resource

The configuration of the N future time instances and the two FFSs above can be resolved using a similar mechanism supported in Rel-18 for the ’Enhanced Type II codebook for predicted PMI’ feature as defined in 38.214 below:

“Subject to UE capability, a UE configured with a CSI-ReportConfig with the higher layer parameter N4 and reportQuantity set to 'cri-RI-PMI-CQI' is assumed to support UE-side CSI prediction. The reported PMI indicates predicted precoder matrices associated with N4 consecutive slot intervals, each with duration of d slots, where the value of N4∈{1,2,4,8} is configured by higher layer parameter N4. If the UE is configured with an aperiodic CSI-RS resource set for channel measurement, the value, in number of slots, of the time unit d∈{1,m} is configured by higher layer parameter d, where m is defined in Clause 5.2.1.4.1. If the UE is configured with a periodic or semi-persistent CSI-RS resource set for channel measurement, the value of d is equal to the periodicity of the CSI-RS resource. The earliest of the N4 slot intervals starts at slot l=n+δ, where n is the uplink slot in which the CSI is reported and the slot offset δ∈{-nCSI_ref,0,1,2} is configured by higher layer parameter delta, where nCSI_ref defined in Clause 5.2.2.5 and the value δ=-nCSI_ref can be configured subject to UE capability.”

Similar to the configuration of parameter N4 parameter the ‘N future time instance(s) for inference’ can be configured to the UE as part of CSI reporting configuration. The reference time for the time instance(s) can be slot n+δ where n is the slot in which the AI/ML beam prediction is reported and δ is a slot offset configured to the UE as part of CSI reporting configuration.  The duration d of each of the N future time instance(s) can be configured as part of the CSI reporting configuration.

For UE-sided model for BM-Case 2, for inference results report, support “Option 1: Based on the uplink slot for the report”, with the following clarifications:

  - The N future time instances are configured as part of CSI reporting configuration;

  - The reference time for the time instance(s) can be slot n+δ where n is the slot in which the AI/ML beam prediction is reported and δ is a slot offset configured to the UE as part of CSI reporting configuration;

  - The duration d of each of the N future time instance(s) can be configured as part of the CSI reporting configuration.

## Configuration of the associated ID

The support for associated ID should be introduced either on an individual TX beam level, and/or a TX beam set level. This corresponds to in the CSI framework either an NZP-CSI-RS-Resource or an NZP-CSI-RS-ResourceSet. The signalling overhead might be large in case the associated ID is defined on a resource level, in particular considering the ongoing discussion on indicating the associated ID as part of the functionality framework discussed in RAN2. Hence our view is the following:

For UE-sided model, support that each ResourceSet should have an associated ID, and the associated ID is configured separately for Set A and Set B

## Beam Indication Enhancements for Measuring a Set A Subset

Figure 3: Overview of the basic inference steps of AI/ML spatial beam prediction.

There is a common understanding that K>1 is needed for accurately finding the strongest beam, this would imply that a Top-K measurement would be needed according to the figure below. This Top-K beam measurement would correspond to the P2 procedure in legacy framework.

In existing P2 procedure, for example when using hierarchical beamforming in left figure below, the NW has a limited number of narrow beam measurement combinations during P2. For example, the NW can configure a limited set of aPeriodicTriggerStates to enable UE to measure the narrow beams within each wide beam, where the main difference of the aPeriodicTriggerStates is the qcl-info for each state.

Figure 4: The large number of possible Top-K configurations with AI/ML is complex for NW to configure using legacy CSI framework

With the introduction of AI/ML beam predictions, one possibility is that the Top-K narrow beams are part of separate wide beams as highlighted in the right part of Figure 4. It is not clear how the NW could dynamically indicate the subset of set A beams that are part of the top-K beam measurements within the CSI framework. This information would be needed for the UE to understand which TCI state to apply for its Top-K measurements. Similar issue occurs if transmitting a subset of set A for the purpose of monitoring as discussed in subsequent sections.

One solution is that the NW transmits Top-K beams according to the UE predictions, however, such solution is not feasible since it does not include NW-specific information (e.g. which beams that are not feasible to transmit due to interference, or “new” beams outside of set A).

Another solution is to use the existing CSI framework; however this would require complex CSI-resource configuration by the NW. In current specifications, the TCI state IDs providing qcl-info for the resources triggered are semi-statically configured per CSI-AssociatedReportConfigInfo. Hence, depending on the which beams the UE predicts as Top-K beams, it may not be possible to indicate the TCI state IDs providing qcl-info for the Top-K beams via the existing mechanism as not all combinations of Top-K beams can be taken into account when configuring CSI-AssociatedReportConfigInfo. The semi-static configuration of QCL relations in the CSI-AssociatedReportConfigInfo can make it challenging for the NW to configure any combination of Top-K beams according to Figure 4. Moreover, a dynamic number of K due to different uncertainty in UE/NW predictions can be challenging to configure without substantially increasing the signaling overhead.

For UE/NW-sided models, further study how Top-K beam measurements (P2 sweep) can be introduced in the existing CSI framework, with the following aspects as a starting point

  - How NW can configure/indicate a dynamic number K RS resources for measurements

  - How NW can dynamically indicate the QCL relationship for the RS resources of the Top-K beams

    - FFS: In addition to the QCL relations, whether the UE further needs to know which  beams in set A that are part of the Top-K beams (e.g. for monitoring purposes)

## Inference result report for UE-sided model

### Inference result report for UE-sided model for BM-Case 1

Agreement For UE-sided model, at least for BM-Case1, for content in the report of inference results, support - Opt 1: Beam information on predicted Top K beam(s) among a set of beams - Opt 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams - At least K=1 and more, FFS on max value - FFS on beam information - FFS on the definition of predicted Top K beam(s) - FFS on definition of reported RSRP when applicable - FFS on other information in the report with potential down selection among the following options - Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams - FFS on the quantization method of probability information - Probability information is the probability of the beam to be the Top 1 or Top K beam - Opt 4: Beam information on predicted Top K beam(s) among a set of beams, RSRP of predicted Top K beam(s) among a set of beams, and confidence information of the RSRP - FFS on definition of reported RSRP - FFS on the definition and quantization method of confidence information - Other options are not precluded. where the set of beams is Set A, i.e., the beams for UE prediction.

Beam information

The method for the UE to report information on the predicted beam are related to the configuration of set A. For efficient introduction of the beam prediction feature, it should be done via minimal extensions to the existing CSI framework. Hence it is proposed to at least reuse CRI and SSBRI to indicate the Top-K beams.

For UE-sided model inference, regarding the FFS on beam information, agree that such information at least comprises the CSI-RS resource indicator (CRI) and SSB resource indicator (SSBRI)

FFS on option 3 and option 4

Uncertainty is a key notion in AI/ML and uncertainty quantification is a key element for trustworthy and explainable AI/ML. A UE report of an uncertainty of a predicted beam can be used by the NW when triggering subsequent Top-K measurements. In case the UE reports a high uncertainty in the predicted strongest beam, the NW can choose to trigger the UE with a larger number of K measurements. Moreover, it can also enable the NW to fallback to a legacy beam management method in case the uncertainty is high. For a properly trained model, the uncertainty is a method for a UE to indicate a possible data distribution change, where a new sample not seen during training should result in a high uncertainty. Note that option 3 and option 4 are special cases of option 1 and option 2, respectively. For example, Option 3 is Option 1 + probability information of predicted Top K beam(s) among a set of beams and Option 4 is Option 2 + confidence information of the RSRP. Confidence of the RSRP prediction can comprise a prediction interval where the UE prediction with a certain probability resides within, for example with 90% confidence, the RSRP prediction is within interval [RSRP1, RSRP2].

Uncertainty can capture a possible data drift in a UE-sided model, and if reported, it can enable NW to configure different number of Top-K beams, or fallback to legacy method.

For UE-sided model inference, regarding the FFS on potential down selection of option 3 and 4,

  - support option 3: probability related information of predicted Top K beam(s)

  - support option 4, the confidence is for example a prediction interval where the UE prediction with a certain probability resides within (e.g. with 90% confidence, the RSRP prediction is within interval [RSRP1, RSRP2]

Adaptive vs fixed value of Top-K

Based on the UE model output, the UE might be flexible in the required number of Top-K beams to guarantee with a certain probability that the strongest beam is selected. Hence, to save signalling overhead in the UE report of the predicted Top-K beams, K should be adaptive and based on the UE-side model output.

For UE-sided model inference, support that value of K could be adaptive and based on the UE-sided model output (e.g. support that UE report K beams so the probability of one of them being strongest is above a certain threshold)

### Inference result report for UE-sided model for BM-Case 2

The inference result reporting for BM-Case2 can be more efficient by enabling the UE to report N>1 future time instances in one report. After the UE has reported such N future time instances in t0, the NW will for example perform Top-K beam measurements (P2 sweep) in t1, t2,…,tN. During the P2 sweep in t1 for example, the UE will measure parts of the set A beams that it also predicted during t0 for t2,…,tN. Note that the observations from P2 sweep in t1 are not used in the predictions for the future time instances. The UE could detect that the predictions are way-off based on the P2 procedure during t1, and hence would benefit from being able to update its predictions.

For UE-side AI/ML model inference, for BM-Case2, the reported inference results of N future time instances in t0 can be invalid after the reporting, for example due to:

  - P2 procedure in t1, t2,…,tN-1 when UE measures a subset of set A beams

  - Unexpected change of UE speed or unexpected blockage

For UE-side AI/ML model inference, for BM-Case2, support that UE can update reported inference results of N future time instances after such report.

## Performance monitoring of UE-sided model

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring

### UE-assisted performance monitoring (Type I option 2)

Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including: - Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring - Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring - Note: resources for Set B for monitoring are not precluded and can be study. - Note: this is only applicable when the model can predict RSRP - Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam - Note: this is only applicable when the model can generate probability information - FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference. - For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window) Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), - At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation - FFS: on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric - FFS other alternatives Agreement At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), consider the following options with potential down selection for the configuration for monitoring: - Option 1: The resource set(s) for monitoring and report configuration for monitoring are configured (when applicable) within CSI report configuration used for inference - FFS: the resource set(s) for monitoring - UE measures the resource set(s) for monitoring. - FFS how/when to report the monitoring results. - Option 2: Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The dedicated report configuration used for monitoring links to an inference report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - UE measures the resource set(s) for monitoring. - FFS when to report the monitoring results.

For a UE-sided model, it is agreed to support reporting a predicted RSRP value. Due to this support, it is necessary to support a relevant metric to monitor the UE predictions. The RSRP difference could be reported per sample, or via statistical information to reduce signalling overhead. Statistical information could comprise of the 5th,10th, …, 90th percentile of the L1-RSRP error.

For UE-sided model performance metric reporting, support alternative 3 (The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring)

  - Both statistical metric (e.g., 10th, 50th, 90th percentile) and per sample metric

How to configure performance monitoring

The performance monitoring metrics should be indicated within the CSI framework to enable an efficient standard support. If the monitoring metric would be introduced in a dedicated report, it motivates a need to introduce a linkage of the monitoring and inference resource sets which can be challenging. Instead, introducing the performance metric report as part of the model inference report can enable a straightforward solution for UE to indicate its model performance. Regarding the FFS on “FFS how/when to report the monitoring results.” The NW can for example configure an aPeriodic measurement report wherein the related performance metrics are included along with inference related information, for example as a new report quantity.

For UE-sided model performance metric reporting Type 1 Option 2, support Option 1 (The resource set(s) for monitoring and report configuration for monitoring are configured (when applicable) within CSI report configuration used for inference).

  - UE can report the performance metrics via introducing a new report quantity

Event-based

To achieve sufficient statistics to determine at the NW whether to activate/deactivate a functionality or not, there is a need to have a lot of samples. One issue occurs for the low-accuracy (erroneous) models, the NW might spend unnecessary number of resources for the purpose on performance metric calculation on models that are performing significantly bad, e.g. an accuracy below e.g. 50%. There could be a benefit to define an event, where the UE can indicate early if a functionality is not working properly.

For UE-assisted performance metric calculation, at least support an event where the UE can early indicate if a functionality is not working properly, e.g. prediction accuracy is below 50%

### NW-side performance metric calculation of UE-sided model (Type I option 1)

Regarding the NW-side performance metric calculation, our understanding is that the NW receives the inference results from the UE and then configures measurements to validate the UE predictions. Based on the predictions and measurements, the NW can calculate any metric that it needs to evaluate the UE-sided models. The above procedure is possible without any standardization impact.

For NW-side performance metric calculation of UE-sided model (type1, option1), conclude that there is no specification impact specific to enable the performance metric calculation at NW-side

# NW-sided model

## Configuration of RS for Set A and Set B for NW sided model

Regarding enhancements for NW-sided model training, options such as UE can report information on two resource sets are already supported by existing standard. Hence, there is no specification impact related to training except the support for UE to report more than 4 beams. Enhancements such as UE only report a CRI instead of CRI+L1-RSRP is only providing minor signaling overhead reduction and can be discussed as part of the signaling overhead reduction.

For NW-sided model, regarding configuration of Set A and Set B, RAN1 to conclude there is no critical specification impact for configuring UE with set A or set B beams

  - Note: Enhancements such as UE only report a CRI instead of CRI+L1-RSRP for a resourceSet is only providing minor signalling overhead reduction

## UE measurement report for NW sided model

Agreement For NW-sided model, for inference, in a beam report initiated by network, based on one measurement resource set, support the report of more than 4 beam related information in L1 signaling - Note: Purpose, such as above “For NW-sided model, for inference”, will not be specified in RAN 1 specifications - FFS on the report content for beam related information - FFS on max number of reported beam related information in one report

During the Rel-18 evaluations, set B typically comprised a ratio that is 1/4 or 1/8 of Set A for spatial beam prediction, and setB equal to setA for BM-case2 in some evaluations. Hence, the possible number of beams of set B could be equal to the number of beams in set A (for BM-case2), therefore, one straightforward solution is to consider the number of beams of 32, or 64 or 256 beams as the number of beams for UE to report.

From the Table 6.3.1.1.2-8 in 38.212, up to 4 beams can be supported to report on UCI over PUSCH/PUCCH. From the Table 6.3.1.1.2-6 in 38.212, the number of bits needed for CRI and SSBRI depends on the total needed reported beams. For example, if 128 CSI-RS resources in total are configured by NW in RRC, which means 7 bits needed for reporting the CRI. So, for each beam, it needs 7 bits for CRI with 7 or 4 bits for absolute or differential RSRP. Based on the limit of Polar code (e.g., 1706 bits), the max potential number of beams reporting on UCI for the considered example above could be supported up to 1706/11=155. One straightforward way to support this with less spec impact is simply to extend the value range of the parameter nrofReportedRS in 38.331.

Furthermore, in case all beams are reported, the CRI field is not needed, hence one could also support the scenario of 256 beams (1706/4=426) evaluated in the study item. Note that with this extension, one would also need to extend the maxNrofNZP-CSI-RS-Resources to number larger than 192. Further discussion needs to comprise on how to support the NW configuration and UE report of such extended number of beams.

For NW-sided model, regarding max number of reported beam related information in one report, use 256 beams as a starting point.

## UE measurement reporting overhead reduction

### Mechanisms to reduce inference data collection overhead for NW sided model

Agreement For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, support - L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB - If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set - FFS: L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB, and whether/how to report number of reported beams - FFS on the maximum value of M (where M can be larger than 4) based on UE capability (M may or may not be different for different reporting contents) - FFS on beam information - Note: Purpose, such as above “For NW-sided model, for inference report, at least for BM-Case 1”, will not be specified in RAN 1 specifications

Spatial-domain beam prediction

In the Rel-18 evaluations, it was shown that good prediction accuracy could be obtained even with reporting of RSRP only for a subset of set B beams, thereby significantly reducing the reporting overhead. For example, it was found [1][3] that a 70% reporting overhead could be achieved while maintaining close to optimal prediction accuracy by having the UE report RSRP only for the strongest beam and any other beams within 15 dB of the strongest beam. In some cases, it may also be beneficial for the network to impose a bound on the maximum number of beams that the UE can report. For example, it was found in [1][3] that for one evaluation scenario, almost 70% reporting overhead reduction could be obtained with minimal prediction accuracy loss if 5 strongest beams were reported.

For NW-sided model inference, support NW configuration for UEs to pre-process set B beams to reduce reporting overhead, via:

  - Support configuring reporting of only beams within X dB of the strongest beam,

  - Support configuring reporting of at most N strongest set B beams.

Temporal-domain beam prediction

When reporting information about measurements of multiple past time instances (in one reporting instance for BM-Case2), it is important to explore the possibility of reducing the reporting overhead. For example, the UE can report the past measurements as a temporal variance or a polynomial approximation.

For NW-sided model inference, support methods for UEs to compress the set B temporal domain measurement results to reduce the reporting overhead.

### Mechanisms to reduce training data collection overhead for NW sided model

Regarding data collection for NW-side AI/ML model of BM-Case1 and BM-Case2, the following approaches have been identified for overhead reduction: - the omission/selection of collected data - the compression of collected data - Note1: For the different purposes of data collection, the overhead reduction mechanisms and corresponding specification impacts may be different. - Note2: Support of any mechanism(s) (if necessary) for each LCM purpose and the potential spec impact (if any) are separate discussions - Note 3: UE complexity and power consumption should be considered

Reducing radio resource overhead for data collection can be achieved by means of minimizing the radio measurements and assistance information that needs to be collected for model training/inference/monitoring. For example, by only collecting the relevant samples, and configuring such data collection in certain favorable conditions. One example for collecting relevant data samples is to avoid reporting duplicated/similar samples from the UE, a static UE might experience similar channel conditions over a certain time period and should in such scenario avoid reporting duplicated samples when collecting data for training an AI/ML model. Also, there can be other issues related to the data collection, for example there is a sudden change in UE channel conditions during a large beam sweep. In this case such sample could be disregarded (removed) for overhead reduction. RAN1 could provide recommendations to RAN2 on the possibility to reduce the data collection overhead, but it is up to RAN2 on how/whether data collection on MDT/RRC should be supported, hence our proposals are,

For NW-sided data collection, RAN1 studies possible “omission/selection of collected data” by the following aspects as a starting point,

  - Possibility for UE to avoid signalling “duplicated” samples,

  - Possibility for UE to avoid signalling data based on certain events, one event can comprise that the UE experienced large channel variation during set A measurements.

  - Note: RAN2 can use such study when designing data collection procedures

For NW-sided data collection, conclude that it is up to RAN2 on whether RRC/MDT procedures should be supported

# Conclusions

In the previous sections we made the following observations:

Observation 1	Two resource sets can be configured for Set A and Set B separately in the CSI report configuration for the report, but only 16 beams (i.e., aperiodic CSI-RS resources) within a resourceSet are supported for AP CSI report.

Observation 2	Uncertainty can capture a possible data drift in a UE-sided model, and if reported, it can enable NW to configure different number of Top-K beams, or fallback to legacy method.

Observation 3	For UE-side AI/ML model inference, for BM-Case2, the reported inference results of N future time instances in t0 can be invalid after the reporting, for example due to:

	P2 procedure in t1, t2,…,tN-1 when UE measures a subset of set A beams

	Unexpected change of UE speed or unexpected blockage

Based on the discussion in the previous sections we propose the following:

Proposal 1	For UE-sided or NW-sided data collection, at least support UE measurement of a resourceSet with 64 NZP CSI-RS resources in the aperiodic reporting configuration

	FFS on whether to support a resourceSet larger than 64 resources for aPeriodic or periodic measurements

Proposal 2	For UE-sided model, regarding configuration of set A/B, support alternative 3.

Proposal 3	For UE-sided model inference, enable NW to specify set A beam subset restriction similar to codebook subset restriction (CBSR) that is specified for CSI feedback

Proposal 4	For UE-sided model for BM-Case 2, for inference results report, support “Option 1: Based on the uplink slot for the report”, with the following clarifications:

	The N future time instances are configured as part of CSI reporting configuration;

	The reference time for the time instance(s) can be slot n+δ where n is the slot in which the AI/ML beam prediction is reported and δ is a slot offset configured to the UE as part of CSI reporting configuration;

	The duration d of each of the N future time instance(s) can be configured as part of the CSI reporting configuration.

Proposal 5	For UE-sided model, support that each ResourceSet should have an associated ID, and the associated ID is configured separately for Set A and Set B

Proposal 6	For UE/NW-sided models, further study how Top-K beam measurements (P2 sweep) can be introduced in the existing CSI framework, with the following aspects as a starting point

	How NW can configure/indicate a dynamic number K RS resources for measurements

	How NW can dynamically indicate the QCL relationship for the RS resources of the Top-K beams

i.	FFS: In addition to the QCL relations, whether the UE further needs to know which  beams in set A that are part of the Top-K beams (e.g. for monitoring purposes)

Proposal 7	For UE-sided model inference, regarding the FFS on beam information, agree that such information at least comprises the CSI-RS resource indicator (CRI) and SSB resource indicator (SSBRI)

Proposal 8	For UE-sided model inference, regarding the FFS on potential down selection of option 3 and 4,

	support option 3: probability related information of predicted Top K beam(s)

	support option 4, the confidence is for example a prediction interval where the UE prediction with a certain probability resides within (e.g. with 90% confidence, the RSRP prediction is within interval [RSRP1, RSRP2]

Proposal 9	For UE-sided model inference, support that value of K could be adaptive and based on the UE-sided model output (e.g. support that UE report K beams so the probability of one of them being strongest is above a certain threshold)

Proposal 10	For UE-side AI/ML model inference, for BM-Case2, support that UE can update reported inference results of N future time instances after such report.

Proposal 11	For UE-sided model performance metric reporting, support alternative 3 (The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring)

	Both statistical metric (e.g., 10th, 50th, 90th percentile) and per sample metric

Proposal 12	For UE-sided model performance metric reporting Type 1 Option 2, support Option 1 (The resource set(s) for monitoring and report configuration for monitoring are configured (when applicable) within CSI report configuration used for inference).

	UE can report the performance metrics via introducing a new report quantity

Proposal 13	For UE-assisted performance metric calculation, at least support an event where the UE can early indicate if a functionality is not working properly, e.g. prediction accuracy is below 50%

Proposal 14	For NW-side performance metric calculation of UE-sided model (type1, option1), conclude that there is no specification impact specific to enable the performance metric calculation at NW-side

Proposal 15	For NW-sided model, regarding configuration of Set A and Set B, RAN1 to conclude there is no critical specification impact for configuring UE with set A or set B beams

	Note: Enhancements such as UE only report a CRI instead of CRI+L1-RSRP for a resourceSet is only providing minor signalling overhead reduction

Proposal 16	For NW-sided model, regarding max number of reported beam related information in one report, use 256 beams as a starting point.

Proposal 17	For NW-sided model inference, support NW configuration for UEs to pre-process set B beams to reduce reporting overhead, via:

	Support configuring reporting of only beams within X dB of the strongest beam,

	Support configuring reporting of at most N strongest set B beams.

Proposal 18	For NW-sided model inference, support methods for UEs to compress the set B temporal domain measurement results to reduce the reporting overhead.

Proposal 19	For NW-sided data collection, RAN1 studies possible “omission/selection of collected data” by the following aspects as a starting point,

	Possibility for UE to avoid signalling “duplicated” samples,

	Possibility for UE to avoid signalling data based on certain events, one event can comprise that the UE experienced large channel variation during set A measurements.

	Note: RAN2 can use such study when designing data collection procedures

Proposal 20	For NW-sided data collection, conclude that it is up to RAN2 on whether RRC/MDT procedures should be supported

# References

3GPP TR 38.843 V18.0.0 (2024-01), Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18).

RP-234039, "New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface," Qualcomm (Moderator), 3GPP TSG RAN Meeting #102, Edinburgh, Scotland, December 11-15, 2023.

R1-2304749, Evaluations of AIML for beam management, Ericsson, 3GPP TSG-RAN WG1 Meeting 113, May 22-26, 2023.