| 3GPP TSG RAN WG1 #119 | R1-2410508 |
| --- | --- |
| Orlando, United States, November 18th – 22nd, 2024 |  |

Agenda Item: 9.1.4.1

Source: MediaTek Inc.

Title:	Aditional study on AI/ML - CSI Compression

Document for: Discussion & Decision

# Introduction

Release 19 additional study on CSI  compression has been agreed for extension due to significant remaining aspects to be addressed before normative work. In this regard, our efforts in the rest of Rel-19 are suggested to be on the following directions:

Training feasibility and required logistics: There has been a discussion on pros and cons of different training options with no consensus on several aspects; however, there are some  common understandings that none of the existing options is universal, and their level of complexity, performance, and logistic requirement are totally different. In the remaining time, it should be clarified what is the procedure of training.

Reference AI/ML model: Reference AI/ML models seems promising for easing the training collaboration. The most important issues remained to be addressed are as follows:

- Boundary of AI/ML models: The exact steps where AI/ML model begins or start  as a part of a bigger system. It is not clear yet whether pre-/post-processing and/or (de)quantization are parts of AI/ML models or not.

- Level and place of standardization: It should be concluded what level of standardization is needed, e.g., parameters and structure, structure only, dataset, etc. Also, it should be clarified which parts of a two-sided AI/ML models undergo standardization to have a reference AI/ML model.

- Procedure of selecting reference AI/ML model:  It needs to be clarified how the candidates for reference AI/ML models or the structure of AI/ML models will be selected. Whether we need the benchmark data for training and/or testing, what kind of metrics are important, etc.

Localized AI/ML models: The AI/ML models are a promis9ing solution to reduce complexity and increase the performance of AI/ML models by intentional overfitting to specific features of underlying data associated to a specific area. After its introduction in our discussion, this concept is overlooked. We believe it should be discussed and clarified which entities and how will collaborate in developing and maintaining AI/ML localized AI/ML models.

Conformance test: Given the large number of possible encoders and decoders from UE and NW vendors, it seems infeasible to test inter-operability of all in a reasonable time. This large number is impacted by number of vendors at both side, training types, number of scenarios/configs to be covered by AI/ML, etc. Thereby, this test space should be confined to a more reasonable size, thereby, ensuring  feasibility of the tests for RAN4.

Clarify the boundary of AI/ML model, i.e., in which step it starts or ends.

# Temporal-domain CSI Compression

In RAN1#116, we have agreed on including temporal domain in CSI compression evaluations. How to use temporal information for compression and decompression varies on different possible designs and can be classified by agreed table below.

Agreement ‎[2] For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following categorization for study: Case; Target CSI slot(s); Whether the UE uses past CSI information; Whether the network uses past CSI information. 0; Present slot; No; No. 1; Present slot; Yes; No. 2; Present slot; Yes; Yes. 3; Future slot(s); Yes; No. 4; Future slot(s); Yes; Yes. 5; Present slot; No; Yes. Note 1: For the UE, the past CSI information may include past model inputs and/or any information derived from them. For the network, the past CSI information may include past CSI feedback instances and/or any information derived from them. Note 2: For case 3 and case 4, the UE may perform prediction as a separate step or jointly with compression. Similarly, the network may perform prediction as a separate step or jointly with reconstruction. Companies to report which option is selected, the number of future slots, and whether the prediction is AI/ML-based or not. Note 3: “Target CSI slot(s)” refers to the slot(s) to which the CSI feedback in the report corresponds. “Present slot” refers to the slot of the most recent CSI-RS measurement used to generate the CSI report. “Future slot(s)” includes at least one slot after the present slot and may include the present slot as well. Note 4: Down-selection is not precluded.

Also, in RAN1#118, we had the following agreement:

Agreement For temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, among Cases 1, 2, 3, 4, and 5, prioritize further discussion on Case 2 and Case 3

We have evaluated case 0 as the baseline which is SF compression (see Figure ‎2-1(a)) and two possible scenarios for case 3: “case 3-AR” which uses auto-regression (AR) for CSI prediction and cascades it with AI/ML based CSI compression as shown in Figure ‎2-1(b); and “case 3-Ideal” which uses ideal CSI prediction and cascades it with AI/ML based CSI compression as shown in Figure ‎2-1(c). In both implementations we use 5 consecutive historical CSI samples starting from present CSI and use those 5 samples for prediction and CSI compression for the next 3 CSI samples in future.  We have also evaluated case 1 where 3 consecutive historical CSI samples are used, and target time slot is present slots.

- Case 0

(b) Case 3-AR: AR-based CSI prediction and CSI compression

(c) Case 3-Ideal: Ideal CSI prediction and CSI compression

Figure ‎2-1: Evaluated scenarios for TSF CSI compression.

For test and train datasets, we followed assumptions agreed in RAN1#116 ‎[2]. The training dataset includes 400k CSI samples and test dataset includes 30k samples generated with 5ms periodicity, distribution option 1 (80% indoor, 20% outdoor) with 3km/h for indoor samples and 30km/h for outdoor samples. CSI reporting periodicity is also 5ms. The observation window length is 25ms which means five subsequent historical CSI samples will be used as input of AI/ML model in case 3-AR and case 3-Ideal.

Agreement For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following as baseline options for UE distribution: - Option 1: 80% indoor, 20% outdoor - Option 2: 100% outdoor Note: Indoor speed is 3 km/h, outdoor speed is chosen from the following options: 10 km/h, 20 km/h, 30 km/h, 60 km/h, 120 km/h. Assumption on O2I car penetration loss and spatial consistency follow the R18 AI based CSI prediction. Agreement For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following evaluation assumptions: - CSI-RS configuration - Periodic: 5 ms periodicity (baseline), 20 ms periodicity(encouraged) - Aperiodic (for cases with prediction): Optional, CSI-RS burst with K resources and time interval m milliseconds (based on R18 MIMO eType-II) - CSI reporting periodicity: {5, 10, 20} ms; other values are not precluded - For cases with the use of past CSI information, to report observation window, including number/time distance of historic CSI/channel measurements. - For cases with prediction, to report prediction window, including number/time distance of predicted CSI/channel.

We evaluate case 0 and case 3-AR and case 3-ideal in terms of SGCS performance. Also, represented complexity of corresponding  AI/ML models below.

Table ‎2-1: Performance and complexity of AI/ML models used in TSF CSI compression evaluations

| Case | Input slots | Output slots | SGCS-256bits | #Param [M] |  | #FLOPs [M] |  | Benchmark |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Enc. | Dec. | Enc. | Dec. | Type | Baseline result |
| Case0 | 1 (present) | 1 (present) | 0.864 (+2.2%) | 3.30 | 3.99 | 83.8 | 85.2 | R16 CB | PC5 - 0.838 PC6 - 0.853 |
| Case1 | 3 (past) | 1 (present) | 0.870 (+3.0%/+0.7%) (R16 CB/Case0) | 3.33 | 5.59 | 84 | 88 | R16 CB and case 0 |  |
| Case3-AR | 5 (past) | 3 (future) | 0.813 (+2.3%) | 3.36 | 9.81 | 85.5 | 98.4 | AR prediction + R18 CB | PC4 - 0.787/0.777/0.770 Avg. 0.778 PC5 - 0.822/0.811/0.804 Avg. 0.812 |
| Case3-Ideal | 3 (past) | 3 (future) | 0.847 (+2.4%) | 3.33 | 9.81 | 84.6 | 98.4 | Ideal prediction + R18 CB | PC4 - 0.808/0.810/0.808 Avg. 0.809 PC5 - 0.844/0.846/0.844 Avg. 0.845 |

We could observe case 0, case 1, case 3-AR and case 3-Ideal respectively show 2.2%, 1.8%, 2.3%, and 2.4% compared to its baselines on average over time slots and ParCombs. Also, we have examined feasibility of using AR for CSI compression and observed a good perf compared to ideal prediction before CSI compression. It only introduces 3.9% degradation.

We have also concern, handling CSI prediction and compression through one generic AI/ML model will increase the size of AI/ML models which have been the issue so far in CSI prediction. Amy issue with CSI prediction will be two sided naturally after unifying two AI/ML models in one. Further, we have discussed promising non-AI methods in 9.1.3.1, with very low computation and storage complexity. This unification deprives us from leveraging non-AI methods for CSI prediction part. Finally, we have not observed any solid justification or gain from combination of these two tasks in one AI/ML model.

# Error tolerance of AI/ML models

In this section we test whether the errors in CSI feedback completely ruins AI/ML models’ performance or causes slight degradation. For this evaluation, we have assumed a certain number of bits in feedback will be flipped (0 to 1 or 1 to 0). The simulation assumptions are presented in Table ‎6-1. The performance of AI/ML models under various number of erroneous bits for the following three cases are evaluated.

Table ‎3-1: Evaluation cases for studying tolerance of AI/ML model against CSI feedback errors

|  | Training | Inference |
| --- | --- | --- |
| Case 0 | No error in training | No error in inference |
| Case 1 | No error in training | Unintentional error in inference |
| Case 2 | Intentional error in training | Unintentional error in inference |

Case 0 [Benchmark]: This is the benchmark where the AI/ML model is not exposed to any error in the CSI feedback at the training and inference stages.

Case 1 [Non-robust model]:  In this case, the AI/ML model is trained in conventional manner whit no CSI feedback error.  In the inference stage, it will be exposed to CSI feedback error.

Case 2 [Robust model]:  In this case, the AI/ML model is trained with intentional CSI feedback errors to learn how to leverage inter-relation of all bits in CSI feedback for accurate CSI reconstruction. In the inference, AI/ML model will be exposed to CSI feedback error too.

We have evaluated the cases in terms of error tolerance with following definition:

Definition: Error tolerance is defined as the average SGCS degradation caused by introducing a certain  number of errors in CSI feedback during inference stage.

Table ‎3-2: Error tolerance of AI/ML models for case 0 (benchmark), case 1(non-robust model) and case 2 (robust model)

| Cases |  | 52 bits |  | 128bits |  | 256bits |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SGCS | Perf. loss | SGCS | Perf. loss | SGCS | Perf. loss |
| Case 0 |  | 0.726 | - | 0.806 | - | 0.869 | - |
| Case 1 | Err. bits = 1 | 0.687 | 5.4% | 0.792 | 1.7% | 0.865 | 0.5% |
|  | Err. bits ≤ 2 | 0.644 | 12.7% | 0.778 | 3.5% | 0.860 | 1.0% |
|  | Err. bits ≤ 5 | 0.526 | 27.5% | 0.731 | 9.3% | 0.846 | 2.6% |
| Case 2 | Err. bits = 1 | 0.693 | 4.5% | 0.798 | 1.0% | 0.871 | 0.0% |
|  | Err. bits ≤ 2 | 0.682 | 6.1% | 0.794 | 1.5% | 0.865 | 0.5% |
|  | Err. bits ≤ 5 | 0.647 | 10.8% | 0.758 | 6.0% | 0.852 | 2.0% |

Our results show that the model trained from case 1 has intrinsically error tolerance to some extent. Errors on few CSI feedback bits do not completely ruin the performance of AI/ML models. If error only occurs in inference (case 1), on average for CSI feedback of 52 bits, 128bits, and 256 bits, the inference error causes 15.2%, 4.8%, and 1% SGCS degradation respectively.

Comparing the feedback of 52 bits with 1 error bit and feedback of 256 bits with 5 error bits, we observe for the same error rate, AI/ML models’ tolerance against feedback error enhances as the feedback overhead increases. We have already observed this trend for other values of error rates as well.

The following figure also depicts the impact of CSI feedback error on throughput of case 1 compared case 0 and eType II baseline with no training or inference error. Traffic is full buffer and max rank is 2. We can see even with few bits of error the AI/ML models work better than eType II  feedback. The gap between throughput gap from Case 1 and Case 0 is gradually closing  for higher overhead bits.

Figure ‎3-1: Impact of AI/ML model’s error on mean throughput

Finally, we enhanced the error tolerance of AI/ML model by intentionally exposing it to errors in training stage and learning how to compensate the errors (flipped bits) in CSI feedback at the CSI reconstruction parts. The results are shown in the previous table where case 2 significantly outperform case 1 for all overheads lengths.  If error occurs in both training and inference (case 2), on average for CSI feedback of 52 bits, 128 bits, and 256 bits, the inference error causes 7.1%, 2.8%, and 0.8% SGCS degradation respectively.  It indicates introducing intentional error in CSI feedback at the training stage, makes AI/ML model more robust against CSI feedback errors.

Evaluate the feedback error tolerance of eType II and compare it with that of AI/ML model.

# Alleviation of Inter-vendor Collaboration

In RAN1#118 the following direction has been imagined for the rest of Rel 19:

Agreement [4] Continue the study of following directions - on-device operation and UE side offline engineering - Direction A: Sharing parameters/reference model/dataset that enables UE-side offline engineering (Inter vendor collaboration option 3a/5a/4) - Potential down-selection into one or more among sub-options 3a/5a-1, 3a/5a-2, 3a/5a-3, 4-1, 4-2, and 4-3 considering their feasibility and performance, including at least the following issues - [Issue 1] What additional information should be shared from NW-side to UE-side to enable UE-side encoder training, validation, and testing? - [Issue 2] Is there concern for NW’s proprietary information disclosure, and if so, how to address it? - [Issue 3] Is there an overhead concern, and if so, how to address it? - [Issue 4] Is there performance impact due to mismatch between NW side data distribution and UE side data distribution, and if so, how to address it? - Direction B: Sharing NW side encoder parameter to UE side for UE side inference directly with on-device operation (Inter vendor collaboration option 3b), including at least the following issues - [Issue 3] Is there an overhead concern, and if so, how to address it? - [Issue 5] Whether it is feasible to use a common encoder across UEs, and whether it is feasible for NW-side to train multiple encoders for different UEs? - [Issue 6] Is there performance impact due to mismatch between NW side data distribution and UE side inference data distribution, and if so, how to address it? - [Issue 7] Is there concern for NW’s and UE’s proprietary information disclosure, and if so, how to address it? - Direction C: Fully standardized reference model(s) and parameters with specified CSI generation part and/or CSI reconstruction part (Inter vendor collaboration option 1), including at least the following issues - [Issue 8] Whether to consider 3GPP’s statistical channel model or field data for reference model(s) training. In case of the latter, how does RAN1 collect field data and agree on them? - [Issue 9] Is there performance impact due to mismatch between the distribution of the dataset used for reference model(s) training, UE-side data distribution, and NW-side data distribution, and if so, how to address it? - [Issue 10] What additional information should be specified to enable UE-side encoder training, validation, and testing, and NW-side decoder training, validation, and testing? - Note: - 1-1: Only reference encoder is specified, and NW-side and/or UE-side may train their actual CSI generation part and actual CSI-reconstruction part separately compatible to the reference encoder. - 1-2: Only reference decoder is specified, and NW-side and/or UE-side may train their actual CSI generation part and actual CSI-reconstruction part separately compatible to the reference decoder. - 1-3: Both reference encoder and reference decoder are specified, and NW-side and/or UE-side may train their actual CSI generation part and actual CSI-reconstruction part separately that are compatible to the reference decoder/encoder. - Note: UE-side data and NW-side data in “UE-side data distribution” and “NW-side data distribution” are field data. - Note: Some issues identified in one direction may/may not be applicable for other Direction. - Note: potential down selection among the 3 directions is not precluded - Study of data distribution mismatch to consider the use of synthetic data and/or field data.

## Direction A

This direction focuses on transferring reference model or data from NW to UE, making it able to come up with a proper decoder. The following progress have been made in the last meeting on direction A.

Agreement[5] For Directions of addressing inter-vendor collaboration complexity for two-sided CSI compression, - For Direction A: - Further study the following options with potential down-selection - 4-1: dataset is {target CSI, CSI feedback}, - 3a-1: encoder parameter exchange, with {target CSI} - Note: Additional information beyond what’s listed above may need to be exchanged if necessary.

In our view, sending AI/ML model  in direction A is preferable than dataset exchange due to following reasons:

- Transmitting data to UE is not feasible due to the scale of problem especially for localized AI/ML model. The UE cannot handle training of millions of AI/ML models using numerous datasets.

- Offline engineering efforts to reduce number of AI/ML models by generalizable design of AI/ML models are more manageable compared to sending & mixing numerous gigantic datasets.

- AI/ML models will be available from more capable entities to those who are less capable and unable to do engineering in reasonable amount of time.

- In our domain problem, typical overhead of transferring certain knowledge through AI/ML model exchange is much lower than dataset transfer. Furthermore, the typical asymmetric complexity of AI/ML models for CSI compression further reduces the encoder part if Direction A focuses on encoder only.

- The architecture mismatch in encoder and decoder side cause a non-negligible performance degradation. For example, a CNN-based encoder may not be able to mimic sophisticated mapping of a transformer even if the relevant data is shared. So, in the engineering stage, UE-side should have some hint. So, AI/ML model transfer solves this problem automatically.

- If data is transferred for training encoder, the dataset should be diverse enough to avoid any overfitting. This means the size and property of datasets needs further regulation to form a proper one. While Sending AI/ML model is free from unimportant hassles

For direction A, prioritize transferring AI/ML model rather than transferring dataset

For direction A, prioritize transferring AI/ML encoder through 3a-1, either with or without {Target CSI}

## Direction B

- Direction B: This direction is about sending NW’s encoder  to be used by UE. This scheme shall remove any performance discrepancy. It should be noted that NW’s computation resources are much higher than that in UE side, thereby enabling NW to design and train relatively large and complex but more capable models. It is possible NW’s encoder would be computationally unbearable by NW.

- For Direction B, discuss how to consider UE’s computational constraints when encoder is provided by NW.

For Direction B, discuss whether re-engineering on reference AI/ML models are allowed right before their deployment, and if yes, what are the acceptable re-engineering efforts and constraints to do that.

Agreement [5] For issues listed for inter-vendor collaboration direction B, further study the overhead in Direction B based on the size of the encoder (i.e., number of parameters and quantization level), the number of encoders, and how often the parameters need to be transferred.

In direction B, over-the-air signaling of AI/ML models may be a big drawback considering the size and frequency of update. Occurrence of updates should be contingent on monitoring event detection or decision in the following cases:

- UE-side detection-based

- UE-side’s additional condition changes

- Abnormal performance detected using UE-side nominal encoder.

- Significant changes in statistics and/or semantics of measured CSI

-  NW-side decision-based

- NW-side decides to change encoder/decoder based on abnormal performance, additional condition changes, Output CSI features, etc.

For direction B, limit the updates occurrence to the following occasions:

- UE-side detection

  - UE-side’s additional condition changes

  - Abnormal performance detected using UE-side nominal encoder.

  - Significant changes in statistics and/or semantics of measured CSI

- NW-side decision

  - NW-side decides to change encoder/decoder based on abnormal performance, additional condition changes, Output CSI features, etc.

## Direction C

Standardization of AI/ML models: To further relax inter-operability test, one concrete step is setting reference AI/ML models in part or completely. During ‎[3], different options have been discussed and it is concluded that option 1 (Fully standardized reference model) fixes inter-vendor collaboration issue. This model serves as the reference AI/ML model, and it is not necessarily the one finally deployed in UE and NW. UE and NW uses the model to mimic its encoder/decoder mapping through a desired AI/ML model, or they perform re-engineering and optimization on top of it to fit available budget. Nevertheless, any deviation from the reference model may impact the performance and compatibility of encoder and decoder. To still ensure compatibility of two parts, there should be limit on the manipulation of reference model which will be represented by performance. While this a common understanding, there is no formal progress in this regard, making it ambiguous and  prone to misinterpretation. We believe it should be discussed among companies whether re-engineering on AI/ML models are allowed right before their deployment, and if yes, what are the acceptable re-engineering and constraints to do that.

Agreement [5] - For Direction C: - It is clarified that; the purpose of a fully specified CSI generation (reconstruction) part is to enable development/training of compatible CSI reconstruction (generation) parts at the NW (UE) side. - Study whether the fully specified reference models or their compatible models may provide reasonable performance for deployment at NW and UE - Including performance evaluation of training/finetuning the models by modeling data distribution mismatch between synthetic and field data, and/or by using actual field data - Including performance evaluation of training/finetuning the models by modelling data distribution mismatch between NW-side field data and UE-side field data, and/or by using actual field data from both NW-side and UE-side - Note: Training/finetuning is done at either or both sides without inter-vendor collaboration. - FFS whether the reference model is the encoder (1-1) or decoder (1-2) or both (1-3) - FFS whether the reference model is the same as RAN4 specified model - FFS whether a single or multiple reference (and how many) models need to be specified

# AI/ML Model Monitoring

In our discussion during study item phase, the general procedure of AI/ML monitoring, different types of it, and possible monitoring actions are discussed, and a common understanding. has been reached.  However, the details of monitoring method as a core of AI/ML model monitoring have not been discussed sufficiently. Here we explain details of some mainstream solutions for monitoring.

## I/O-based Monitoring

I/O based monitoring look into the statistical or semantic properties of input or output CSI samples to detect a monitoring event. The main idea is any drastic change in environment and config/scenario will flow into CSI samples and change their property. So have a temporal inspection on how these properties are changing enable us to find monitoring event and take subsequent decision.

UE-side monitoring

We have used a new metric called Power Spectral Entropy (PSE) as an important property of a sample. Tracking temporal changes of PSE is an effective way of  detecting monitoring event. Let us explain how it is calculated and later introduce a simple monitoring metric like PSE[t]-PSE[t-1] as a simple metric to detect changes in underlying RF environment/config/scenario.

Let us assume that a block of CSI samples is available with dimension of  𝑀×𝑁×𝑊 in time-spatial-frequency domain, i.e., 𝑥[𝑚,𝑛,𝑤]. Naturally, M=1 in SF CSI compression and M>1 in TSF CSI compression. We explain the general case with arbitrary value of M. For such CSI sample(s), PSE can be calculated through the following steps.

- Conversion to spectral domain: Xl,i,j=1MNWm=0M-1n=0N-1w=0W-1xm,n,we-j2π(lMm+iNn+jWw)

- Normalized power profile: Pl, i, j= Xl,i,j2lijXl,i,j2

- Entropy of power profile: PSE=-1log2MNWlijP[l,i,j].log2⁡P[l,i,j]

PSE is a normalized value between zero and one and it has a direct indication on compressibility of CSI samples. The two extreme cases of PSE are a fixed-valued input (DC signal on all dimensions) which results PSE=0 and white noise which results PSE=1 as shown in Figure ‎2-27 for 2D input CSI (M=1).

Figure 5-1: PSE’s extreme cases: DC input and noise input

We know DC signal is the most compressible signal as it can be represented by only one of its elements. On the other hand, white noise has no correlation among its elements, and it is not possible to provide any abstract representation of it; in fact, all elements should be represented to realize the white noise. As such, PSE has a reverse relation with compressibility of CSI samples. The physical notion behind PSE in SF compression is that it quantifies the  smoothness/correlation of CSI samples in spatial-frequency domain or equivalently  quantifies the sparsity of CSI samples in delay-beam domain, both of which favour AI/ML models in compressing CSI samples. This motivates us to track the changes in RF environment based on PSE of input CSI at UE side or recovered CSI at the NW side.

As discussed, PSE quantifies sparsity of CSI samples in beam-delay domain and their smoothness in SF domain, which both finally will be translated to intermediate KPI achieved by an AI/ML model. We observed the average PSE of the samples in the training dataset finally determines the SGCS achieved by the AI/ML model. An evaluation with three datasets is shown in Table 5-1.

Table ‎5-1: Illustration of inverse relation between PSE and CSI compressibility

|  | Dataset 1 | Dataset 2 | Dataset 3 |
| --- | --- | --- | --- |
| Average PSE of dataset | 0.555 | 0.531 | 0.486 |
| SGCS | 0.890 | 0.921 | 0.937 |

We can also use the historical changes of PSE as a metric to find monitoring events. The simplest metric you can think of is Dt=PSEt-PSE[t-1] where PSE[t] refers to PSE of current CSI sample and PSEt-1 refers to AI/ML We have evaluated its effectiveness of such a metric on few cases scenarios. Let’s assume a UE moves from environment one (a.k.a. Env 1) to environment 2 (a.k.a. Env 2) on 300th sample and after spending some time in Env 2, it will return to Env 1 on the 600th sample. You can now see how our simple metric has detected the changes.

| (a) UMA-LOS 300 UMi-NLOS 600 UMa-LOS | (b) UMi-LOS 300 UMa-NLOS 600 UMi-LOS |
| --- | --- |
| (c) UMa-LOS300 UMa-NLOS -> UMa-LOS. | (d) UMa-LOS300 UMi-LOS 600 UMa-LOS |

Figure ‎5-2: Evaluation of PSE-based model monitoring when UE changes its RF environment at 300th and 600th CSI sample.

As you can see in the above figures, putting a simple threshold like |PSE[t]-PSE[t-1]≥thr we can capture monitoring events. We have also observed if AI/ML model is generalizable over two environments, PSE will not be sensitive to transitions between two environments, compared to two non-generalizable environments.

We can see the input-based monitoring is an effective tool which does not need to use ground truth CSI transmission, nor violates proprietariness of NW’s and UE’s AI/ML model.

NW-side monitoring

We can similarly apply PSE in uplink at NW side to explore changes. We can do it through  calculating D(t) over output CSI or calculating Dt over uplink CSI samples collected from SRS. The rationale behind the second option is, AI/ML models are generalizable in frequency domain; moreover, the RF environmental changes will impact uplink as well, so such changes can be equivalently captured in downlink by inspecting CSI collected by CSI-RS or uplink by inspecting CSI samples collected from SRS.

Discuss NW-side AI/ML model I/O-based monitoring using uplink CSI samples collected from SRS.

## NW-side intermediate KPI-based monitoring

Intermediate KPI-based monitoring is another candidate to detect monitoring events through changes in intermediate KPI, such as SGCS, GCS, NMSE. However, the problem is any KPI calculation needs a reference/ground truth CSI. At UE and NW one of these two CSIs forms are missing. At UE side, recovered/output CSI is missing which should be sent by NW. At NW side,  the ground truth CSI is missing which needs to be sent by UE. Either of these two solutions come with significant overhead. Any improvement on resolution of ground truth CSI makes the issue even worse.

Sending ground truth CSI from UE to NW is discussed more in our previous meeting. The idea is calculation of SGCS between output CSI and ground truth CSI. Since it is not practical to send CSI in float 32 format, we have no choice other than doing some sort of quantization of it. One quantization option is using a eType II-like scheme which only reports the beam and delay elements with higher power. To calibrate our monitoring, we first must decide how much quantization is good for subsequent metric. To do so, the difference between KPI calculation using quantized ground truth (KPIactual) and using float 32 ground truth CSI (KPIgenie) is measured. We  can show this measure as KPIgenie-KPIactual≤thr where thr refers to a threshold on allowable difference. We also have some candidate values for threshold, i.e., 0.02, 0.05, 0.1. We have following agreements in this regard.

Agreement​ For the intermediate KPI monitoring of CSI compression, for the FFS issue on the value of threshold of KPIth_1 in Option 1, the candidate threshold values are set as 0.02, 0.05 and 0.1​ Agreement​ For the intermediate KPI monitoring of CSI compression, between the two options to calculate KPIdiff achieved in the RAN1#112bis-e meeting, as baseline for calibration purpose, consider Option 1 (Gap between KPIActual and KPIGenie). ​ - Option 2 (Binary state of KPIActual and KPIGenie relationship) as optional and up to companies to report.​ - Results subject to Option 2, may be captured as a note in observation

To this end, we have evaluated this proposed metric and saw how increasing resolution affect the accuracy of monitoring event detection through accuracy, miss detection rate, and false alarm rate trough the following 6 different settings, e.g., Env1 to Env 6. We should note that the AI/ML model is trained over 100k samples obtained from ENV 1.

Table ‎5-2: Different environments/settings used for monitoring evaluation.

| Environment | Config/Scenario difference w/ agreed EVM |
| --- | --- |
| Env1 | Baseline EVM |
| Env2 | 60kmph(outdoor) |
| Env3 | TXRU1 |
| Env4 | LoS |
| Env5 | TXRU2 |
| Env6 | NLoS |

Let us assume UE remains in each environment for 50 CSI samples and then do transition to another environment. UE does this 100 times of each of Env 1 to Env 6.  You can see how KPIactual using PC8 of eType II is tightly matched with KPIgenie. The RF environmental/setting changes is obvious by significant jump or dip once per every 50 samples. From this figure, we can see, The KPI actual using PC8 (KPIPC8 ) is so close to KPgenie so that it tightly follows KPI genie and additional resolution of ground truth CSI only offers marginal improvement.

Figure ‎5-3: KPIactual using eType II’s PC8 and KPIgenie using float 32 format over six different RF environments/configs/scenarios.

Furthermore, we are observing the transitions to Env 3 and Env 6 result higher drops compared to others. It seems the generalizability of AI/ML model on those environments is poor, and for monitoring accuracy investigation it is fair to consider transitions to Env3 and Env 6 as a monitoring event to be detected. Hereby, we limit our focus only to these two environments.

First let us use the KPIgenie-KPIactual≤thr to calibrate the ground truth CSI’s resolution. We have examined PC8 as an existing resolution and PC9 which is higher resolution with (L=10, p=0.9, beta=0.31). From the existing the above metric it is evident that as the resolution increases the difference between KPIs reduces.

Table ‎5-3: Closeness of KPI_genie and KPI_actual with PC6, PC8, and PC9

| Target CSI format | thr = 0.02 | thr = 0.05 | thr = 0.1 | Overhead[bits] |
| --- | --- | --- | --- | --- |
| eTypeII ParaComb6 {L=4, p=0.5, beta=0.5} | 39.1% | 69.7% | 88.1% | 279 |
| eTypeII ParaComb8{L=6, p=0.25, beta=0.75} | 41.1% | 70.6% | 90.3% | 328 |
| Rel-16 Type II CB with new parameters: PC9 - {L=10, p=0.9, beta=0.31} | 79.6% | 94.3% | 98.0% | 1014 |

Based on the metric we have used for calibration it is inevitable to reach higher similarity by increasing resolution, but the question is how to detect a monitoring event after decision about the resolution of ground truth CSI, a metric that works based on KPIactual and shows the AI/ML model malfunctions as an indication of a monitoring event’s occurrence, such as temporal difference of KPI (KPIactualt-KPIactualt-1≥thr1) or absolute value of  KPI (KPIactual≥thr2). Let us continue our study with the latter monitoring metric. Now the question is how much monitoring accuracy increases after increasing the resolution of CSI samples, What is the monitoring performance difference if we choose PC9 or PC8? Let us show how much value of the two KPIs are close as shown in the below figure.

Figure ‎5-4: Difference of KPIactual  using PC9 (high resolution) and PC8 (existing resolution)

We have evaluated the mentioned resolution and their resultant false alarm rate (where monitoring event is detected while AI/ML models is in ENV1 ) and miss detection rate (Monitoring event is not detected while AI/ML model is Env3 or Env 6) . Below tables shows what is the effect of different resolutions on the false alarm and miss detection rates.  Here are our important observations during this evaluation:

- Increasing resolution of CSI samples improves false alarm rate by 0.3%

- Increasing resolution of CSI samples improves miss detection rate by 0.7%

- False alarm rate degradation  of choosing PC8 and PC9 compared to KPI_genie (float 32) is 7% and 0.4%, respectively.

- Miss detection rate’s degradation  of choosing PC8 and PC9 compared to KPI_genie (float 32) is 0.7% and 0.3%, respectively.

- Enhancement of CSI resolution’s is not necessary as it only offers an insignificant improvement.

Table ‎5-4: Impact of CSI sample’s resolution on accuracy of NW-side monitoring

| thr2 |  | PC8 | PC9 | Float32 |
| --- | --- | --- | --- | --- |
| 0.3 | False alarm rate | 0.3% | 0.2% | 0.03% |
|  | Miss detection rate | 41.8% | 41.4% | 41.4% |
| 0.4 | False alarm rate | 1.2% | 0.9% | 0.5% |
|  | Miss detection rate | 37.8% | 36.8% | 36.6% |
| 0.5 | False alarm rate | 3.1% | 2.8% | 2.2% |
|  | Miss detection rate | 32.8% | 32.1% | 31.9% |
| 0.6 | False alarm rate | 7.4% | 6.7% | 6.2% |
|  | Miss detection rate | 27.9% | 27.6% | 27.4% |

## Associate ID

Associate ID indicate consistency of the situation an AI/ML model is working in that. Associate ID helps the UE vendors to select proper AI/ML models for functioning. From monitoring perspective, as long as associate ID remains same, AI/ML model should be able to handle the task it has been designed for, and whenever it changes, there is a need to Fallback, Switch, or deactivate AI/ML model in use, or at least check its performance for subsequent monitoring decisions. Thereby, granularity of associate ID impacts the number of AI/ML models should be developed by UE vendors and determine how general they should be. If one associate ID is assigned to multiple cells, UE naturally cannot develop low-complex cell-specific AI/ML models. Instead, it should use leverage more general models to work on more cells. This will be a set back for CSI compression use case, where it’s complexity-performance tradeoff has been questionable. The general AI/ML models we have observed so far are very complex in terms of computational and storage complexity. In other words, such a coarse assignment of single associate ID to multiple cells inevitably prohibits the usage of cell-specific AI/ML models which have been regarded as a key solution to improve the complexity-performance tradeoff of AI/ML models for CSI compression.

Deprioritize coarse granularity of associate ID, i.e., assigning one associate ID to multiple cells.

# Data Collection for Model Training

## Using SRS/Uplink CSI Collection  for Training NW-side Models

AI/ML model will be used in the inference stage to compress and decompress downlink (DL) CSI samples measured by UE using CSI-RS sent in the downlink. Does this necessarily mean only DL CSI samples collected from CSI-RS are useful for training? Downlink and uplink (UL) CSI samples share common semantic features. We will show an AI/ML model can be successfully trained using a dataset merely consists of UL CSI  and works well in the inference on DL CSI samples. We put one step further and show that the UL CSI can be collected in a frequency other than what is targeted for inference stage. This is because the AI/ML models for CSI compressions have natural ability of generalization over carrier frequency. To show that using UL CSI is possible for training, we have evaluated different approaches of NW-side data collection shown in the below figure.

Figure ‎6-1: Possible approaches for NW-side data collection for training

Approach 1 (DL CSI only): In the most popular approach, it is assumed that NW sends CSI-RS toward UE, and then UE measures and reports DL CSI. The reported DL CSI samples will be stored as a part of dataset for training purposes at NW.

Approach 2 (UL CSI only): We are proposing approach 2 where UE sends an SRS to NW, and NW itself measures and stores CSI samples; Albeit it will be UL CSI and it may be collected at a frequency different from target frequency in the inference stage.

Approach 3 (DL and UL CSI): This approach is essentially the mixture of previous approaches, where NW mainly relies on UL CSI in data collection, but it may receive DL CSI samples for finetuning or forming a mixed dataset.

We have evaluated the above approaches where SRS is sent with 200 MHz shift (2.2 GHz) from the frequency used for sending CSI-RS (2GHz) in the downlink. The following table shows our evaluation results which indicate feasibility of using UL CSI at NW side for training AI/ML models.

Table ‎6-1: Impact of different NW-side data collection methods on training AI/ML models

| NW Data collection | Description of training and dataset at NW | Test dataset | FB OH 52 bits | FB OH 128 bits | FB OH 256 bits |
| --- | --- | --- | --- | --- | --- |
| Approach 1 | 300k DL CSI samples | 30k DL CSI | 0.689 | 0.764 | 0.825 |
| Approach 2 | 300k UL CSI samples | 30k DL CSI | 0.694(-0%) | 0.755(-1.2%) | 0.813(-1.5%) |
| Approach 3 | 285k UL CSI, finetune with 15k DL CSI | 30k DL CSI | 0. 687(-0.0%) | 0. 756(-1.0%) | 0.815(-1.2%) |
|  | 150k+150k mixed UL/DL CSI | 30k DL CSI | 0.690(-0%) | 0.759(-0.6%) | 0.821(-0.5%) |

For NW-side AI/ML model training, NW can rely on UL CSI samples collected from SRS sent by UEs. Use of DL CSI can be limited to finetuning purposes.

# Conclusion

We have the following proposals in this contribution:

- Clarify the boundary of AI/ML model, i.e., in which step it starts or ends.

- Evaluate the feedback error tolerance of eType II and compare it with that of AI/ML model.

- For direction A, prioritize transferring AI/ML model rather than transferring dataset

- For direction A, prioritize transferring AI/ML encoder through 3a-1, either with or without {Target CSI}

- For direction B, discuss how to consider UE’s computational constraints when encoder is provided by NW.

- For direction B, discuss whether re-engineering on reference AI/ML models are allowed right before their deployment, and if yes, what are the acceptable re-engineering efforts and constraints to do that.

- For direction B, limit the updates occurrence to the following occasions:

- UE-side detection

  - UE-side’s additional condition changes

  - Abnormal performance detected using UE-side nominal encoder.

  - Significant changes in statistics and/or semantics of measured CSI

- NW-side decision

  - NW-side decides to change encoder/decoder based on abnormal performance, additional condition changes, Output CSI features, etc.

- Discuss NW-side AI/ML model I/O-based monitoring using uplink CSI samples collected from SRS.

- Deprioritize coarse granularity of associate ID, i.e., assigning one associate ID to multiple cells.

- For NW-side AI/ML model training, NW can rely on UL CSI samples collected from SRS sent by UEs. Use of DL CSI can be limited to finetuning purposes.

# References

- 3GPP TR 38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”, Nov 2023.

- R1-2401563,  “Final summary for additional study on AI/ML for NR air interface: CSI compression”, Moderator (Qualcomm), RAN WG1 #116, Athens, Greece, Feb 26th – Mar 1st, 2024.

- R1-2403504, “Final summary for additional study on AI/ML for NR air interface: CSI compression”, Moderator (Qualcomm), RAN WG1 #116bis, Changsha, China, April 15th – 19th, 2024.

- R1-2407347, “Final summary of additional study on AI/ML for NR air interface: CSI compression”, Moderator (Qualcomm), RAN WG1 #118, Maastricht, NL, August 19th – 23rd, 2024.

- R1-2409161, “Final summary for additional study on AI/ML for NR air interface: CSI compression”, Moderator (Qualcomm), RAN WG1 #118-bis, Hefei, China, October 14th – 18th, 2024.