3GPP TSG-RAN WG1 Meeting #119	R1-2410602

# Orlando, USA, November 18-22, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev | - | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on skipping uplink transmission in case of BWP switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, TEI17 |  |  |  |  |  | Date: |  |  | 2024-11-8 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TS 38.213, when UE detects a DCI format for an UL BWP change for a cell, UE shall switch to a new BWP as the DCI indicated, and perform UL transmission on that new BWP. gNB would acknowledge that the BWP switching is successful according to the UL transmission on the new BWP. In TS 38.321, if UE is configured with skipUplinkTxDynamic with value true, and if no data is available for any LCG, UE may not generate a MAC PDU for uplink transmission. In this case, if an uplink grant schedules an uplink transmission and simultaneously indicates an active UL BWP change, a UE should skip UL transmission for this UL grant if no data is available. For UE side, it correctly decodes the UL grant, and it will operate on new BWP. However, for gNB side, it is not sure whether the BWP switch is successful or not since no UL transmission is received for confirmation. Therefore, misunderstanding may happen beween UE and gNB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To clarify that UE transmits a PUSCH indicated by a DCI format triggering a UL BWP switch with the assumption of skipUplinkTxDynamic value as false. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | gNB has no method to know whethter UE has switched to a new BWP or not, when skipUplinkTxDynamic is configured with value true, and thus gNB does not know in which BWP to schedule the UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

12  Bandwidth part operation

< Unchanged parts are omitted >

A UE does not expect to detect a DCI format with a BWP indicator field that indicates an active DL BWP or an active UL BWP change with the corresponding time domain resource assignment field providing a slot offset value for a PDSCH reception or PUSCH transmission that is smaller than a delay required by the UE for an active DL BWP change or UL BWP change, respectively [10, TS 38.133].

If a UE detects a DCI format indicating an active UL BWP change for a cell, and if a UE is configured with skipUplinkTxDynamic with value true, the UE transmits a PUSCH indicated by that DCI with the assumption of skipUplinkTxDynamic value as false.

If a UE detects a DCI format with a BWP indicator field that indicates an active DL BWP change for a cell, the UE is not required to receive or transmit in the cell during a time duration from the end of the third symbol of a slot where the UE receives the PDCCH that includes the DCI format in a scheduling cell until the beginning of a slot indicated by the slot offset value of the time domain resource assignment field in the DCI format or determined by the slot offset value corresponding to the first PDSCH of the more than one PDSCH scheduled by the DCI format for the cell.

< Unchanged parts are omitted >