3GPP TSG-RAN WG1 Meeting #118		R1-2407208

Maastricht, The Netherlands, August 19th –23rd, 2024

Agenda Item:	9.11.3

Source:	Moderator (MediaTek)

Title:	Feature lead summary #3 of AI 9.11.3 on NR-NTN uplink capacity and throughput

Document for:	Discussion

# 0 Introduction

## 0.1 Background

In RAN#104 [1], a revised WID on NR NTN enhancements was endorsed for Release 19, as copied below [1].

Uplink Capacity/Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4]

- Study then specify, if beneficial, DFT-s-OFDM PUSCH enhancements via Orthogonal Cover Codes (OCC)

  - Determine the achievable capacity improvement to be targeted taking into account realistic impairments (e.g. Doppler, time variation, phase distortion, etc)

  - Specify necessary signalling, if needed

  - Update RF requirements accordingly, if needed

  - Note: The study can consider orthogonal cover codes across OFDM symbols, across slots, and/or within an OFDM symbol.

  - Note: the study phase is targeted to be completed by RAN#104

- Notes for this objective:

  - The enhancement is not targeting improvements/impacts of MU-MIMO capability

  - The enhancement is not targeted to PUSCH DMRS

  - No enhancement for initial access

  - Enhancements to PRACH are not in scope.

  - This feature may be applicable for UEs operating in terrestrial networks based on a common design

This Feature Lead summary aims to discuss issues on Uplink Capacity/Throughput Enhancement for FR1-NTN based on companies contributions and discussions in RAN1#118.

The following agreement and conclusion were made in RAN1#117

Conclusion

OCC with PUSCH can support at least multiplexing of 2 or 4 UEs and achieve up to 2 or 4 times capacity gains respectively, when repetitions are used.

Note: the actual gain may be less due to e.g. intra/inter cell interference.

Agreement

For the normative phase, at least one of the OCC techniques will be specified:

- Inter-slot time-domain OCC with PUSCH repetition Type A with OCC length 2 or 4

- Inter-symbol(s) time domain OCC with OCC length 2 or 4

- Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2 or 4

- FFS Combination of OCC techniques including multiplexing of 8 UEs

- FFS Use of OCC techniques with TBoMS

- FFS Backward compatibility with non-Rel-19 UEs

## 0.2 Contact Information

Please help to fill in the contact information for the FL summary.

| Company | Name | E-mail |
| --- | --- | --- |
| CMCC | Yi Zheng Ting Ke Yongchang Liu | Zhengyi@chinamobile.com keting@chinamobile.com liuyongchang@chinamobile.com |
| China Telecom |  |  |
| DoCoMo | Shohei Yoshiuoka Luhua You | syouhei.yoshioka.py@NTTDOCOMO.COM youlh@docomolabs-beijing.com.cn |
| OPPO | Hao Lin Zuomin Wu Nande Zhao | v-linhao1@oppo.com wuzuomin@oppo.com zhaonande@oppo.com |
| Huawei, HiSilicon | Xiaolei TIE | tiexiaolei@huawei.com |
| Vivo | Siqi liu | liusiqi@vivo.com |
| ZTE | Nan Zhang Fangyu Cui Ziyang Li | zhang.nan152@zte.com.cn cui.fangyu@zte.com.cn li.ziyang1@zte.com.cn |
| Nokia, NSB | Frank Frederiksen Jingyuan Sun | Frank.frederiksen@nokia.com jingyuan.sun@NOKIA-SBELL.COM |
| Ericsson | Stefan Eriksson G Talha Khan | stefan.g.eriksson@ERICSSON.COM talha.khan@ericsson.com |
| CATT | Deshan Miao | miaodeshan@catt.cn |
| Spreadtrum | Zhenzhu Lei | reven.lei@unisoc.com |
| Qualcomm | Alberto Rico Alvarino Xiaofeng Wang | albertor@QTI.QUALCOMM.COM wangxiao@qti.qualcomm.com |
| Lenovo | Sher Ali Cheema Zhi Yan | scheema@lenovo.com yanzhi1@LENOVO.COM |
| Xiaomi | Yajun Zhu Xuemei Qiao | zhuyajun@xiaomi.com qiaoxuemei@xiaomi.com |
| Samsung | Sungjin Park | sj100.park@SAMSUNG.COM |
| Apple | Chunxuan Ye Chunhai Yao | Chunxuan_ye@apple.com Chunhai_yao@apple.com |
| SONY | Martin Beale | martin.beale@sony.com |
| LG | Hanjun Park Daesung Hwang Seungmin Lee | hanjun0128.park@lge.com daesung.hwang@lge.com edison.lee@lge.com |
| NEC |  |  |
| Panasonic | Akihiko Nishio | nishio.akihiko@jp.panasonic.com |
| Sharp | Tomoki Yoshimura | yoshimurat@sharplabs.com |
| Google |  |  |
| ETRI | Junghoon Lee | jh.lee@etri.re.kr |
| NICT | Masafumi Moriyama Michiharu Nakamura | m.moriyama@nict.go.jp michi.nakamura@nict.go.jp |
| Fujitsu | Shogo Uchida | uchida-shogo@fujitsu.com |
| TCL | Yu Ding Yiwei Deng | yu10.ding@tcl.com yiwei1.deng@tcl.com |
| InterDigital | Umer Salim | Umer.salim@interdigital.com |
| THALES | Mohamed El Jaafari | mohamed.el-jaafari@THALESALENIASPACE.COM |
| HONOR | Liangang Chi | chiliangang@honor.com |
| MediaTek (Moderator) | Gilles Charbit | Gilles.charbit@mediatek.com |

# 1 [CLOSED] Performance of OCC Techniques

## 1.1Company contributing views

Inter-slot time-domain OCC with PUSCH repetition Type A

- 2 DM RS per 14-symbol slot with DM RS mapping Type A with OCC 2 or 4.

- Can utilize all OFDM symbols in slot

- One non Rel-19 UE can be multiplexed with Rel-19 UEs with OCC

Inter-symbol time-domain OCC based on PUSCH repetition Type B

- 1 DM RS per 6-symbol slot or 3-symbol slot with DM RS mapping Type B with OCC 2 or 4.

- Cannot utilize all OFDM symbol in slot for OCC 4

- One non Rel-19 UE can be multiplexed with Rel-19 UEs with OCC

Intra-symbol pre-DFT-s OCC

- Rel-19 UE cannot be multiplexed with Rel-19 UEs with OCC

- Requires updates of the TBS/rate matching design (i.e. TBS division by spreading factor)

- Requires higher MCS, split the TB into smaller TBs or enable TBoMS with low MCS to increase NRE (to keep the same throughput)

| Contribution | Observation/Proposals |
| --- | --- |
| Ericsson [3] | Link level evaluation: Observation 11: For VoIP, Inter-slot OCC performs well in the presence of RF impairments with two UEs but not with four UEs. Observation 12: For VoIP with grouping of UEs with similar frequency offsets onto the same time-frequency resources, inter-slot OCC with four UEs performs well if the CFO spread on a resource is kept within 100 or 200 Hz. Observation 13: For inter-slot OCC with four UEs, low data rate service works well. There are significant throughput gains compared with no OCC. Observation 14: For VoIP, inter-symbols OCC performs well in the presence of RF impairments with both two UEs and four UEs. Observation 15: For VoIP, intra-symbol OCC with TB splitting performs well in the presence of RF impairments with both two UEs and four UEs. System-level evaluation: Observation 7: OCC will increase the number of active UEs per UL radio resource, which will increase the levels of inter-cell interference. At some load level, an unacceptable fraction of users with bad quality of service is reached. Observation 8: With FRF=1, system-level simulations show that the system is interference limited. The SINR is reduced by almost 3 dB for each time the number UEs is doubled. Observation 9: With FRF=3, system-level simulations show that the system is partly interference limited. OCC levels of 2 and 4 give only limited reductions in SINR (0.5 and 1.5 dB compared to no OCC, respectively), while an OCC level of 8 gives a SINR reduction of 3 dB compared to no OCC. Observation 10: With FRF=3, OCC multiplexing levels 2 and 4 give a VoIP capacity gain of 2 and 3.9 times, respectively, while the additional capacity gain of OCC level 8 is small and results in a dramatic increase in number of UEs with unacceptable voice quality. Proposal 1: Down-prioritize OCC multiplexing level 8. Comparison with FDM: Observation 16: Intra-symbol OCC, when applied prior to the DFT precoder, is equivalent to a sub-PRB allocation where every nth subcarrier is used by a given UE. Observation 17: When more than one PRB is available, multiplexing can also be achieved with FDM using separate PRBs per UE. Observation 18: Intra-symbol pre-DFT OCC for 2 UEs across 2 PRBs has the same capacity and performance as 2 UEs allocated to one PRB each. Grouping of UEs with CFO: Proposal 3: Further study grouping of UEs with similar frequency offset. 8 UE multiplexing: Proposal 4: Do not specify support for OCC with more than 4 UEs. |
| ZTE [4] | Observation 5: Inter-symbol time domain OCC has higher tolerance to frequency offset. Observation 8: For inter-slot time domain OCC without TBoMS, at least 2 UEs in VoIP scenario and at least 4 UEs in Low data rate scenario can be supported without BLER performance degradation. Observation 9: For inter-slot time domain OCC without TBoMS, at least 4 UEs can be supported in VoIP scenario and Low data rate scenario with significant throughput improvement compared to single UE performance. |
| Nokia [6] | Observation 1: Motivation for introducing an intra-symbol or inter-symbol OCC scheme is unclear. Observation 2: Intra-symbol and inter-symbol OCC will impact the signal generation mechanisms yielding a loss in baseline PUSCH performance for the single UE. Observation 3: Intra-symbol OCC suffers from inter subcarrier emissions lowering its performance in actual deployments. Observation 7: It is crucial that the codes used for OCC are aligned to maintain orthogonality between the UEs using the same physical resources. Proposal 1: RAN1 to support only OCC with PUSCH repetition Type A in Rel-19 NR NTN. Proposal 2: RAN1 to discuss mechanisms for UE frequency offset control by the gNB. Proposal 3: The design for OCC should not impose any limitations in the PRB domain. Proposal 8: For UEs in RRC connected mode, short-term frequency offset requirements should be tighter than 0.1 ppm for operation with OCC. Proposal 9: RAN1 to discuss mechanisms for UE frequency offset control by the gNB. Observation 9: It is crucial that the codes used for OCC are aligned to maintain orthogonality between the UEs using the same physical resources. Proposal 12: RAN1 to introduce OCC timing to align the time (i.e. repetitions) of OCC application among UEs scheduled on the same physical resources. Observation 10: Applying GNSS update during OCC time window may break OCC orthogonality. Proposal 13: RAN1 to explore methods to address potential issues with GNSS update when OCC is applied. The methods should specify rules and requirement at UE side and coordination with NW to avoid timing misalignment. |
| Qualcomm [9] | Observation 1: Pre-DFTS comb, overloading and Cross-symbol OCC with SF2 and pre-DFT-s OCC (comb structure) with SF4 are robust to all impairments. Cross-slot OCC is impacted most by impairments – mainly CFO. Observation 2: - Pre-DFTS comb-based OCC schemes (including comb + symbol-level OCC) can offer up to 8x multiplexing gains at lower SNRs while 2x gains at higher SNRs for different payload sizes and different target BLERs, within 1 dB degradation loss. For a fixed payload size, these schemes undergo performance loss as available resources (repetitions) are reduced due to increased coding rate. - Overloading schemes can get 2x multiplexing gains at lower SNRs and higher target BLERs, within 1 dB degradation loss. The performance of these schemes saturates as number of multiplexed users is increased and/or as operating SNR is increased (systems become interference limited). - Cross-slot OCC can get 2x multiplexing gains at lower SNRs and higher target BLERs, within 1 dB degradation loss. The performance of cross-slot OCC is similar to overloading because of CFO impacting interference cancellation capability (orthogonality) of OCC. Observation 3: Slot-level OCC suffers from BLER flooring problem because CFO severely impacts the orthogonality OCC, increasing inter-UE interference. Pre DFTS comb-based OCC does not suffer from the BLER flooring problem. The BLER flooring problem becomes worse as more UEs are multiplexed for slot-level OCC, while Pre-DFTS comb-based OCC is robust in this setting. Slot-level OCC cannot support 4 UE multiplexing while Pre-DFTS comb-based OCC can support 4 UEs at lower SNRs with minimal performance degradation. Other companies have observed the same behavior in RAN1#117 (see Appendix III for more details) Proposal 1: Pre-DFTS comb-based is specified to achieve higher uplink capacity gains using OCC while keeping good per-UE performance Proposal 2: Support the following schemes for PUSCH OCC: - Inter-symbol time domain OCC with spreading factor of 2 - Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with spreading factor of up to 4. - Both techniques can be simultaneously applied to achieve a spreading factor of 8. |
| OPPO [7] | Observation 1: Almost N times throughput can be achieved for PUSCH with OCC when compared to PUSCH without OCC for all the evaluated OCC schemes except for the cases that OCC length is 4 and FO is considered in inter-slot OCC with PUSCH repetition Type A. Observation 2: For the cases that OCC length is 4 and FO is considered in inter-slot OCC with PUSCH repetition Type A, throughput gain is obvious when compared to PUSCH without OCC. Observation 3: For the cases that OCC length is 4 and FO is considered in inter-slot OCC with PUSCH repetition Type A, the impact of FO would be reduced and the performance would be enhanced if the FO is estimated and compensated at gNB side. |
| MediaTek [11] | Observation 1: Timing drift or CFO may not have significant impact on Inter-slot OCC or inter-symbol(s) OCC with small values of L for OCC length 2 or 4. |
| Panasonic [16] | Observation 3: Intra-symbol OCC shows the best BLER performance among the three OCC options. Degradation due to inter-OCC interference caused by the impairments is seen especially for inter-slot OCC. Observation 4: Performance in a low BLER region like 1.0E-2 is important in NTN because the number of HARQ retransmissions would be limited due to the long propagation delay. Observation 5: For “no OCC” (UE multiplexing without OCC), large degradation is observed especially for 4 UE case with low BLER region. The exception is the case of 2UEs with 16 repetitions (i.e. small number of UEs and large repetition factor) in high BLER region. OCC (either inter-slot, inter-symbol or intra-symbol) is essential. Proposal 3: From the link level simulation results, intra-symbol OCC should be considered as the first priority. |
| CMCC [18] | Observation 7: The legacy redundancy version cycling as defined in current specification for DG PUSCH repetition may not be workable as the repetition number increases. The enhancements to the RV cycling for PUSCH repetition when slot level OCC multiplexing is adopted. Observation 8: For the traffic of small data rate, the performance comparison between PUSCH OCC and single UE without multiplexing are listed below, - With PUSCH repetition =8 - The performance loss of OCC =2 compared with SU equals to 0.46. - The performance loss of OCC =4 compared with SU equals to 1.45. - With PUSCH repetition =16 - The performance loss of OCC =2 compared with SU equals to 0.29. - The performance loss of OCC =4 compared with SU equals to 0.85. - With PUSCH repetition =32 - The performance loss of OCC =2 compared with SU equals to 0.11. - The performance loss of OCC =4 compared with SU equals to 0.4. Observation 9: For the traffic of VoIP, the performance comparison between PUSCH OCC and single UE without multiplexing are listed below, - With PUSCH repetition =16 - The performance loss of OCC =2 compared with SU equals to 0.47. - The performance loss of OCC =4 compared with SU equals to 1.26. - With PUSCH repetition =32 - The performance loss of OCC =2 compared with SU equals to 0.27. - The performance loss of OCC =4 compared with SU equals to 1.32. Observation 10: The performance difference between different solutions, including inter-slot OCC, inter-symbol OCC and intra-symbol OCC is marginal when a relative high repetition number is applied. |
| Vivo [28] | Observation 2. When TO/FO is considered for TBS=96 bits with OCC length=2, inter-symbol/intra-symbol OCC schemes show around 1.06 dB performance loss, inter-slot OCC schemes about 1.57 dB performance loss compared with baseline. Observation 3. When TO/FO is considered for TBS=96 bits with OCC length=2, inter-slot OCC is slightly worse than intra-symbol OCC/inter-symbol OCC. Observation 4. When different FO error distinctions are considered between multiplexed UEs, the performance of OCC schemes becomes worse. Observation 5. When TO/FO is considered for TBS=96 bits with OCC length=4, inter-symbol/intra-symbol OCC schemes show around 2.6 dB performance loss, inter-slot OCC schemes about 3.86 dB performance loss, compared with baseline. Observation 6. When TO/FO is considered for TBS=96 bits with OCC length=4, inter-slot OCC suffers an error floor. Observation 7. When TO/FO is considered for TBS=96 bits with OCC length=2 or 4, inter-symbol OCC and intra-symbol OCC have a similar performance. Proposal 8. Down select one of the following - Inter-slot time-domain OCC with PUSCH repetition Type A with OCC length 2 and 4 - Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2 and 4 |
| Xiaomi [13] | Observation 1: Pre-DFT based OCC spreading is not sensitive to UE impairment, doppler shift and delay spread. Observation 4: In theory, inter-symbol(s) time domain OCC multiplexing is more sensitive to the UE impairment and doppler shift than pre-DFT based OCC multiplexing. Observation 5: For inter-slot OCC multiplexing, the performance degradation caused by UE impairment and doppler shift could be compensated by a lower code rate, as opposed to solutions with symbol spreading. Observation 6: For pre-DFT based OCC spreading with 8 repetitions, the performance loss is ~0.5 dB and ~4.1 dB for 2 and 4 multiplexed UEs respectively compared to single UE transmission for VOIP service with 2% BLER. Observation 7: For pre-DFT based OCC spreading with 4-slot TBoMS and 2 repetitions, the performance loss is 0 dB and 0.4 dB for 2 multiplexed UEs and 4 multiplexed UEs respectively compared to single UE transmission for VOIP service with 2% BLER. Observation 8: For pre-DFT based OCC spreading with 8 repetitions, the performance loss is 0.7 dB and 1.9 dB for 2 and 4 multiplexed UEs respectively for low data rate service with 10% BLER. Observation 9: For pre-DFT based OCC spreading with 4-slot TBoMS and 2 repetitions, the performance loss is 0 dB and 0.6 dB for 2 and 4 multiplexed UEs respectively for low data rate service with 10% BLER. Observation 10: In general, pre-DFT based OCC spreading combining with TBoMS and repetitions has better link performance than pre-DFT based OCC spreading only with repetitions. Observation 11: In general, pre-DFT based OCC spreading can effectively enhance system throughput, even in the case of degraded link performance for a single UE. Observation 12: Pre-DFT based OCC spreading with TBoMS provides a better increase in system throughput compared to pre-DFT based OCC spreading with repetitions at low SNR. Observation 13: For repetition-based OCC multiplexing, the performance loss is 0.5 dB and 0.6 dB for 2 multiplexed UEs for VOIP with 2% BLER and for low data rate with 10% BLER respectively. Observation 14: For repetition-based OCC multiplexing, the performance loss is more than 10 dB and 3.9 dB for 4 multiplexed UEs for VOIP with 2% BLER and for low data rate service with 10% BLER respectively. Observation 15: In general, inter-slot OCC multiplexing can effectively enhance system throughput, even at low SNR and with single UE’s performance degradation. Observation 16: pre-DFT OCC spreading with TBoMS shows superior transmission performance compared to repetition-based OCC multiplexing. Proposal 1: Don’t support 8 UE multiplexing for NR-NTN uplink capacity enhancement. |
| Samsung [10] | Observation 5: OCC schemes shows similar aggregated throughput performances in case of OCC length 2/4. Proposal 3: RAN1 does not discuss further whether to limit the number of PRBs applicable to OCC PUSCH. Proposal 4: RAN1 does not pursue code length 8 for OCC PUSCH. |
| NEC [21] | Proposal 2: Study the complexity impact on the receiver of the network to despread the PUSCH with OCC and cancel inter-user interferences for NR-NTN uplink capacity/throughput enhancement. |
| DoCoMo [17] | Observation 1: For inter-slot OCC length 2 with TBoMS option1, in case of 2-slot TBoMS, the BLER performance will be over 3 dB worse than OCC without TBoMS at 2% BLER; in case of 4-slot TBoMS, the BLER performance is around 3 dB worse than OCC without TBoMS at 2% BLER. Observation 2: For inter-slot OCC length 2 with TBoMS option 2, the BLER performance is equal to or even slightly better compared to the inter-slot OCC without TBoMS case. |
| CMCC [18] | Observation 7: The legacy redundancy version cycling as defined in current specification for DG PUSCH repetition may not be workable as the repetition number increases. The enhancements to the RV cycling for PUSCH repetition when slot level OCC multiplexing is adopted. Observation 8: For the traffic of small data rate, the performance comparison between PUSCH OCC and single UE without multiplexing are listed below, - With PUSCH repetition =8 - The performance loss of OCC =2 compared with SU equals to 0.46. - The performance loss of OCC =4 compared with SU equals to 1.45. - With PUSCH repetition =16 - The performance loss of OCC =2 compared with SU equals to 0.29. - The performance loss of OCC =4 compared with SU equals to 0.85. - With PUSCH repetition =32 - The performance loss of OCC =2 compared with SU equals to 0.11. - The performance loss of OCC =4 compared with SU equals to 0.4. Observation 9: For the traffic of VoIP, the performance comparison between PUSCH OCC and single UE without multiplexing are listed below, - With PUSCH repetition =16 - The performance loss of OCC =2 compared with SU equals to 0.47. - The performance loss of OCC =4 compared with SU equals to 1.26. - With PUSCH repetition =32 - The performance loss of OCC =2 compared with SU equals to 0.27. - The performance loss of OCC =4 compared with SU equals to 1.32. Observation 10: The performance difference between different solutions, including inter-slot OCC, inter-symbol OCC and intra-symbol OCC is marginal when a relative high repetition number is applied. Proposal 1: The enhancement to the redundancy version cycling of the PUSCH repetition should be considered, when the slot-level PUSCH OCC multiplexing is adopted. Observation 11: Based on current specifications, the UCI multiplexing with PUSCH repetition and the dropping of PUSCH repetition happens, which will break the orthogonality of the OCC enhanced PUSCH repetitions. Proposal 3: Support reusing DMRS port indication in DCI field to additional indicate OCC index to reduce signalling overhead. Proposal 4: Potential enhancements for the cases of PUSCH repetition dropping and the UCI multiplexing can be considered when the time domain-based OCC enhancements are applied. Proposal 5: It should be further studied whether to support the frequency hopping in the NTN scenario and with the OCC multiplexing in the time domain. |
| China Telecom [19] | Proposal 1: The impact on coherent combining issue in inter-slot time domain OCC caused by dropping rules or other reasons needs to be further considered. |

Phase rotation due to timing drift and CFO:

MediaTek [11] observed phase rotation over time duration X ms is calculated as

- Δφ = 2π (rad) × timing drift (ppm) × X (ms) × channel bandwidth (Hz) /2,

- Δφ = 2π (rad) × CFO × X (ms) × Fc/2

Where max time drift = 80 ppm and max CFO = ±0.1 ppm.

CFO seems a relatively more significant factor than timing drift for OCC as shown in tables below. Typical UEs may have some margin for CFO, hence a uniform distribution for CFO was assumed in evaluation resulting in much smaller phase rotations due to CFO. With smaller values of L, the impact of CFO can be marginal for OCC length 2 or 4.

- The maximum phase rotation due to timing drift of OCC applied to slots of PUSCH with L=14  is 5.18 degree for OCC length 2 and 10.36 degrees for OCC length 4; and with L=1 is 0.37 degree for OCC length 2 and 0.73 degrees for OCC length 4.

- The maximum phase rotation due to CFO ±0.1 ppm of OCC applied to slots of PUSCH with L=14 is 72 deg for OCC length 2 and 144 degrees for OCC length 4; and L=1 is 5.1 deg for OCC length 2 and 10.3 degrees for OCC length 4.

MediaTek [11] Table 1: Max delay drift with OCC schemes

| OCC scheme | Max delay drift |  |
| --- | --- | --- |
|  | OCC length = 2 | OCC length = 4 |
| Inter-slot PUSCH repetition Type B with PUSCH mapping Type B Inter-slot PUSCH repetition Type A with PUSCH mapping Type A | (L=1) 0.014 us (L=4) 0.057 us (L=14) 0.2 us | (L=1) 0.028 us (L=4) 0.114 (L=14) 0.4 us |
| Inter-symbols | (Msymbgroup=1) 0.014 us | (Msymbgroup=1) 0.028 us |
| Intra-symbol | 0.007 us | 0.007 us |

MediaTek [11] Table 2: Max CFO with OCC schemes

| Timing drift 80 ppm | Max ΔΘ [deg] | Max CFO ±0.1 ppm @ 2 GHz | Max ΔΘ [deg] |
| --- | --- | --- | --- |
| OCC 2 @ 1 RB | 5.18 (L=14) 1.48 (L=4) 0.37 (L=1) | OCC 2 | 72 (L=14) 20 (L=4) 5.1 (L=1) |
| OCC 4 @ 1 RB | 10.36 (L=14) 2.96 (L=4) 0.73 (L=1) | OCC 4 | 144 (L=14) 41.1 (L=4) 10.3 (L=1) |

Grouping of UEs with same CFO:

Nokia [6] showed that if the multiplexed UEs have similar FO values, no large impairment from a larger OCC size is to be expected. Propose short-term frequency offset requirements for connected UEs should be tighter than 0.1 ppm for operation with OCC and discuss and, if needed, define mechanisms for controlling the FO difference of UEs using OCC across PUSCH repetitions. It’s not clear why RAN1 should limit the design of the OCC features to a certain number of RBs, especially for the case of OCC applied to PUSCH repetitions Type A. UE multiplexing in the same resources makes the most sense in the cases where UEs need to transmit large payloads and hence occupy a large number of resources for their transmission, leaving no space for frequency or time domain multiplexing.

Ericsson [3] proposed to further study grouping of UEs with similar frequency offset

Nokia [6] Figure 1 The BLER of four UEs employing OCC across repetitions, with a 30-degree phase shift in four slots.

Alignment of OCC codes of multiplexed UEs

Nokia [6] observed it is crucial that the codes used for OCC are aligned to maintain orthogonality between the UEs using the same physical resources and proposed RAN1 introduce OCC timing to align the time (i.e. repetitions) of OCC application among UEs scheduled on the same physical resources.

UE multiplexing with no OCC and comparison to single UE :

We copy below the evaluation results from Panasonic [16]. For “no OCC” (UE multiplexing without OCC), large degradation is observed especially for 4 UE case with low BLER region. The exception is the case of 2UEs with 16 repetitions (i.e. small number of UEs and large repetition factor) in high BLER region. The performance of the 3 OCC schemes (inter-slot, inter-symbol, intra-symbol) is relatively closer at high number of repetitions at lower SNR.

Panasonic [16] BLER and Throughput performance

Qualcomm observed that compare to single UE and pre DFTS comb with 2 UEs, the gap between no OCC and OCC increases as number of repetitions are decreased, which is expected since the increase in coding rate starts to control the performance of the system.

Qualcomm [9] BLER performance of different schemes for VoIP with OCC 4

Qualcomm [9] discuss a semi-analytical approach relies on calculating the post-processing SINR that a given symbol would experience after de-OCC, where the 2% point of the CDF is used when comparing perfect OCC, slot-level OCC, random scrambling (for UE multiplexing with no OCC), and symbol-level OCC (equivalent to the BLER target for VOIP).

- At -10dB SNR, the cross-user interference is negligible, so the performance of all schemes is within 1dB of the perfect OCC. Symbol-level OCC offers virtually the same performance of perfect OCC, while random scrambling (“overloading”) and slot-level SNR have a 0.6dB and 0.8dB loss.

- At -5dB SNR, the differences between schemes become more apparent. While symbol-level OCC still offers the same performance as perfect OCC, random scrambling has a ~2.1dB loss with respect to perfect OCC, and slot-level OCC has a ~1.7dB loss with respect to perfect OCC.

Further, the loss in postprocessing SNR at the 10% point of the CDF (BLER for LDR) for different SNR values and OCC lengths of 2 and 4 is observed:

- While the degradation of slot-level OCC is small at low SNRs, the performance improvement with respect to “random scrambling” is minimal, especially for OCC-4.

- At higher SNRs, the degradation of slot-level is not acceptable.

- Symbol level OCC offers good performance across all SNRs, with a slight degradation (~0.4dB) observed at 0dB SNR for OCC4.

Qualcomm [9] Figure 4 CDF of postprocessing SINR for OCC length of 2, different schemes, -10dB SNR (left) and -5dB SNR (right)

Qualcomm [9] Figure 4: SNR loss at 10% of the CDF for OCC-2 (left) and OCC-4 (right)

8 UE multiplexing:

Ericsson showed with FR1, interference results in large SINR loss. With FRF=3, OCC multiplexing levels 2 and 4 give a VoIP capacity gain of 2 and 3.9 times, respectively, while the additional capacity gain of OCC level 8 is small and results in a dramatic increase in number of UEs with unacceptable voice quality.

Ericsson [2]: Table 3: Relative capacity gain counting voice users with acceptable quality.

| OCC level | SNR @ 2% BLER | Fraction of UEs with acceptable voice quality4) |  |
| --- | --- | --- | --- |
|  |  | FRF=1 | FRF=3 |
| 1 | -3.7 dB1) | 71% | 100% |
| 2 | -3.5 dB2) | 17% | 100% |
| 4 | -3.3 dB2) | 0% | 98% |
| 8 | -2.7 dB3) | 0% | 53% |
| - Baseline performance without OCC from section 2.4. - Inter-symbols OCC from section 2.4.2. - Comb 4x2 OCC gives a loss of ~1 dB compared to baseline according to simulations by Qualcomm in [6]. - Based on SINR CDFs in Figure 6 and Figure 7. |  |  |  |

Ericsson [2] Table 3: Relative capacity gain counting voice users with acceptable quality.

| OCC level | Relative capacity (FRF=3) |
| --- | --- |
| 1 | 1 |
| 2 | 2 |
| 4 | 3.9 |
| 8 | (4.3) |

Huawei [5] discussed the number of DMRS symbols per slot might be doubled when more than 4 UE are multiplexed with OCC. According to Table 7.3.1.1.2-6 and Table 7.3.1.1.2-6A in TS38.212, there are only 2 bits of Antenna port(s) field in DCI format 0-1 when transform precoder is enabled, dmrs-Type=1, maxLength=1 (single-symbol DMRS without time domain OCC), which can indicate one of at most 4 DMRS ports. In order to multiplex more than 4 UEs, the number of orthogonal DMRS ports for scheduling should be increased to 8 for accurate channel estimation. Thus, the DMRS with maxLength=2 (double-symbol DMRS with time domain OCC) should be used, which obviously will impact per UE throughput.

Huawei [5], Samsung [10] proposed to only support multiplexing 2 or 4 UEs without combination of OCC schemes in Rel-19 NR-NTN.

DoCoMo [17] discussed 2 options for inter-slot OCC length 2 with TBoMS. For inter-slot OCC length 2 with TBoMS option 2, the BLER performance is equal to or even slightly better compared to the inter-slot OCC without TBoMS case (Option 1 has 3 dB performance loss).

Timing Synchronization with GNSS:

Release-17 and 18 design assumes that the NTN UE is equipped with Global Navigation Satellite System (GNSS) capability which can be used by the NTN UE to know its position. This information, combined with satellite ephemeris information (either broadcasted by the network or provisioned via other means), allows the UE to carry out frequency pre-compensation and timing adjustment. UE applies orbit propagation to determine the delay and Doppler frequency of the service link at any time. Some NTN systems architecture requires the UE to estimate the feeder link delay and delay variation. For this purpose, the network provides the UE with another information called TA_common, which is in the form of coefficients of a quadratic polynomial and indicates the feeder link variation over time. Combined with its GNSS location knowledge, the UE can apply a timing advance described by the following equation:

TTA = (NTA  +NTA, UE-specific + NTA,common + NTA, offset) × Tc

Nokia [6] observed applying GNSS update during OCC time window may break OCC orthogonality and proposed RAN1 to explore methods to address potential issues with GNSS update when OCC is applied. The methods should specify rules and requirement at UE side and coordination with NW to avoid timing misalignment.

Nokia [6] Figure 8 GNSS fix might break the OCC orthogonality.

Moderator summary and views on performance of OCC schemes:

Ericsson [3], ZTE [4], CMCC [18], vivo [27], Xiaomi [13], Samsung [10], DoCoMo [17] contributed on link-level performance of OCC techniques. It was noticed that the simulation results were mainly re-submission or updates of link-level evaluation with summary captured in Table 1 and Table 2 in moderator summary in RAN1#117 [30]. We copy the Table 1 and Table 2. Contributing companies are encouraged to review and update directly Table 1 and Table 2 if needed.

The moderator’s view based on these tables below and further inputs from companies contributions and evaluation (note Table 1 and Table 2 were not captured as observations in RAN1#117 as consensus to capture these tables as observation could not be achieved during the meeting).

- Intra-symbol OCC with TBoMS is robust to CFO (which has significantly more impact than timing drift) and can meet target of multiplexing with 4 UEs. OCC within an OFDM symbol, when applied prior to the DFT precoder, is equivalent to a sub-PRB allocation.

- Inter-symbol(s) OCC (symbol-level repetition) with TBoMS or Inter-symbol OCC with PUSH repetition B is robust to CFO and can meet target of multiplexing with 4 UEs.

- Inter-slot OCC Inter-slot OCC  can only be supported with multiplexing of 2 UEs due to impact of CFO, which is the dominant factor (i.e. more significant than timing drift).

- Comparison intra-slot OCC with FDM: When more than one PRB is available, multiplexing of UEs can similarly be supported with FDM using separate PRBs per UE.

- OCC with 1 PRB only: There is no enough consensus on limiting the number of PRBs.

- Multiplexing with 8 UEs: There is not enough consensus on multiplexing with 8 UEs.

FL Summary Table 1: PUSCH of VoIP

|  |  | Inter-slot | Inter-symbol | Intra-symbol |
| --- | --- | --- | --- | --- |
| baseline | SNR@2%BLER | HW/ -5.6dB, ZTE/ -5 dB, E/// -3.7 dB, CATT -2.6 dB, MTK -2.4 dB, Xiaomi -4.2 dB, CMCC -5.45, vivo -0.73dB for 4 slots, CMCC -5.45(with 20 reps), -4.81(with 16 reps) |  |  |
|  | Aggr thpt @required SNR | HW/ 10.2kbps, ZTE/ 9 kbps, MTK 10 kbps, vivo 45kbps for 4 slots |  |  |
| OCC2 w/ TBoMS | SNR@2%BLER | NA | HW/ -5.4dB ZTE/ -5 dB Xiaomi -4.2 dB vivo -0.44dB for 4 slots CMCC -4.78Db w/ 16 reps -5.27dB w/20 reps | HW/ -5.6dB E/// -2.7 dB Xiaomi -4.2 dB vivo -0.44dB for 4 slots CMCC -4.79Db w/ 16 reps -5.39dB w/20 reps |
|  | Aggr thpt @required SNR | NA | HW/ 20.2kbps ZTE/ 18 kbps vivo 89.4kbps for 4 slots | HW/ 20.3kbps vivo 89.4kbps for 4 slots |
| OCC4 w/ TBoMS | SNR@2%BLER | NA | HW/ -5.2dB ZTE/ -5 dB Xiaomi -3.8 dB CMCC -4.21Db w/ 16 reps -4.74dB w/20 reps | HW/ -5.6dB E/// -2.3 dB Xiaomi -3.8 dB CMCC -4.52 Db w/ 16 reps -5.2dB w/20 reps |
|  | Aggr thpt @required SNR | NA | HW/ 40.4kbps ZTE/ 36 kbps | HW/ 40.7kbps |
| OCC2 w/o TBoMS | SNR@2%BLER | HW/ -5.2dB ZTE/ -5 dB E/// -3.5 dB CATT -2.1 dB MTK -2 dB Xiaomi -3.7 dB CMCC -5.18dB vivo 0.54dB for 4 slots CMCC -4.34Db w/ 16 reps -5.18dB w/20 reps | HW/ 2.1dB E/// -3.6 dB CATT -2.6 dB vivo 1.27dB for 4 slots | HW/ 1.7dB ZTE/ Not reach CATT -2.4 dB Xiaomi -3.7 dB E/// -3.9 dB vivo 1.18dB for 4 slots |
|  | Aggr thpt @required SNR | HW/ 20.2kbps ZTE/ 18 kbps MTK 20.1 kbps vivo 87.3kbps for 4 slots | HW/ 6.5kbps vivo 83kbps for 4 slots | HW/ 7.0kbps ZTE/ 16 kbps vivo 83kbps for 4 slots |
| OCC4 w/o TBoMS | SNR@2%BLER | HW/, ZTE, E///, Xiaomi Not reach CMCC -4.13 E/// -0.6 dB CMCC -3.55Db w/ 16 reps -4.13dB w/20 reps | HW/ not reach E/// -3.2 dB | HW/, ZTE/ not reach Xiaomi -0.1 dB E/// -3.3 dB |
|  | Aggr thpt @required SNR | HW/ 37.4 kbps ZTE/ 35 kbps | HW/ 1.5kbps | HW/ 3.0kbps ZTE/ not reach |

FL summary Table 2: PUSCH of low data rate.

|  |  | Inter-slot | Inter-symbol | Intra-symbol |
| --- | --- | --- | --- | --- |
| baseline | SNR@10%BLER | HW/ -6.5dB, ZTE/ -11.5 dB, E/// -7.7 dB, Apple -11 dB, OPPO -3.5 dB, Xiaomi -6.1 dB, CMCC -11.12, Pana {-4.6 dB (8 slots), 1.6 dB (4 slots)}, Sharp -5.9 dB, vivo -5.42dB for 4 slots, CMCC -7.3Db w/ 8 reps, -9.18dB w/16 reps, -11.12dB w/32 reps |  |  |
|  | Aggr thpt @required SNR | HW/ 6.5kbps, ZTE/ 2.7 kbps, E/// 5.9 kbps, OPPO 12.6 kbps, Pana {11 kbps (8 slots), 21.8kbps (4 slots)}, Sharp 9 kbps, vivo 21.5kbps for 4 slots |  |  |
| OCC2 w/ TBoMS | SNR@10%BLER | HW/ NA OPPO -2.3 dB | HW/ -6.2dB ZTE/ -11.4 dB Xiaomi -6.1 dB OPPO -3 dB Pana {-4.5 dB (8 slots), -1.1 dB (4 slots)} vivo -4.47dB for 4 slots CMCC -6.97Db w/ 8 reps, -9.12dB w/16 reps, -11.05dB w/32 reps | HW/ -6.4dB OPPO -3 dB Pana {-4.5 dB (8 slots), -1.2 dB (4 slots)} Sharp -5.9 dB Xiaomi -6.1 dB vivo -4.50dB for 4 slots CMCC -6.99dB w/ 8 reps, -9.14dB w/16 reps, -11.09dB w/32 reps |
|  | Aggr thpt @required SNR | HW/ NA OPPO 24 kbps | HW/ 12.9kbps ZTE/ 5.3 kbps OPPO 24.6 kbps Pana {21.6 kbps (8 slots), 43.2 kbps (4slots)} vivo 37kbps for 4 slots | HW/ 12.9kbps OPPO 24.6 kbps Pana {21.6 kbps (8 slots), 43.2 kbps (4slots)} Sharp 18 kbps vivo 37kbps for 4 slots |
| OCC4 w/ TBoMS | SNR@10%BLER | HW/ NA OPPO not reach | HW/ -6.0dB ZTE/ -11.4 dB Xiaomi -5.5 dB OPPO -2.3 dB Pana {-3.9 dB (8 slots), 0.4 dB (4 slots)} vivo -2.78dB for 4 slots CMCC -6.58Db w/ 8 reps, -8.61dB w/16 reps, -10.7dB w/32 reps | HW/ -6.3dB OPPO -2.4 dB Pana {-4.0 dB (8 slots), 0.4 dB (4 slots)} Sharp -5.6 dB Xiaomi -5.5 dB vivo -2.78dB for 4 slots CMCC -6.85Db w/ 8 reps, -8.89dB w/16 reps, -10.95dB w/32 reps |
|  | Aggr thpt @required SNR | HW/ NA OPPO 42 kbps | HW/ 25.8kbps ZTE/ 10.7 kbps OPPO 50 kbps Pana {43 kbps (8 slots), 86 kbps (4 slots)} | HW/ 25.8kps OPPO 50 kbps Pana {43 kbps (8 slots), 86 kbps (4 slots)} Sharp 36 kbps |
| OCC2 w/o TBoMS | SNR@10%BLER | HW/ -6.0dB ZTE/ -11.5 dB E/// -7.9 dB Apple -10.8 dB Xiaomi -5.5 dB CMCC -11.01 Pana {-4.4 dB (8slots), -1.0 dB (4slots)} Sharp -5.4 dB vivo -4.35dB for 4 slots CMCC -6.84Db w/ 8 reps, -8.89dB w/16 reps, -11.01dB w/32 reps | HW/ -0.2dB Sharp -4.3 dB vivo -4.47dB for 4 slots | HW/ -0.4dB ZTE/ -10.0 dB Xiaomi -5.4 dB Sharp -5.8 dB vivo -4.47dB for 4 slots |
|  | Aggr thpt @required SNR | HW/ 12.9kbps ZTE/ 5.3 kbps E/// 11.7 kbps Pana {21.6 kbps (8slots), 43.2kbps (4slots)} Sharp 18 kbps vivo 38.7kbps for 4 slots | HW/ 8.4kbps Sharp 18 kbps vivo 39.1kbps for 4 slots | HW/ 8.5kbps ZTE/ 4.8 kbps Sharp 18 kbps vivo 39kbps for 4 slots |
| OCC4 w/o TBoMS | SNR@10%BLER | HW/ -3.3dB ZTE/ -11.4 dB E/// -7.2 dB Apple -10.7 dB Xiaomi -2.2 dB CMCC -10.72 Pana {-3.6 dB (8slots), 1.2dB (4slots)} Sharp -2.0 dB vivo -1.15dB for 4 slots CMCC -5.85Db w/ 8 reps, -8.33dB w/16 reps, -10.72dB w/32 reps | HW/ not reach | HW/ not reach ZTE/ -9.0 dB Xiaomi -4.2 dB Sharp -4.8 dB |
|  | Aggr thpt @required SNR | HW/ 23.7kbps ZTE/ 10.7 kbps E/// 23.4 kbps Pana {43kbps (8slots), 86kbps (4slots)} Sharp 36 kbps | HW/ 1.4kbps | HW/ 2kbps ZTE/ 8.6 kbps Sharp 36 kbps |

## 1.2 First Round Discussion

FL Recommendation 1-1:

Contributing Companies are encouraged to review and update directly Table 1 and Table 2 at end of Section 1.1 for summary of evaluation simulations if needed, and indicate in the comment table below that they revised the Table 1 or Table 2 for their results.

Initial proposal 1-2:

Observation

Phase rotation over time duration X ms is calculated as

- Δφ = 2π (rad) × timing drift (ppm) × X (ms) × channel bandwidth (Hz) /2,

- Δφ = 2π (rad) × CFO × X (ms) × Fc/2

Where max time drift = 80 ppm, max CFO = ±0.1 ppm with uniform distribution between [-0.1 ppm, +0.1 ppm], and Fc is carrier frequency.

Initial proposal 1-3:

Observation

Inter-slot OCC can only be supported with multiplexing of 2 UEs.

Initial proposal 1-4:

Multiplexing of 8 UEs is not supported with OCC with PUSCH

Companies are encouraged to provide comments within the following table:

| Companies | Comments |
| --- | --- |
| Panasonic | FL Recommendation 1-1: Panasonic results are added in above Table (same as the last meeting). Initial proposal 1-2: The necessity of this observation is not clear to us. Initial proposal 1-3: According to our simulation results in R1-2406591, inter-slot OCC can be supported with 2 UEs or 4 UEs multiplexing when the number of repetitions is large (e.g. >8) and target BLER is high (e.g. 10%). On the other hand, degradation is seen in lower BLER region due to inter-OCC interference. Because lower BLER needs to be achieved in NTN due to limited applicability of HARQ retransmissions for the long propagation delay, inter-slot OCC would be less useful for NTN. Initial proposal 1-4: Agree. |
| LGE | [Initial proposal 1-2] We do not think that this observation is necessary. [Initial proposal 1-3] We think that this is proposal rather than observation. In general, whether inter-slot OCC can be applicable or not may depend on the scenario. Moreover, there is no discussion/evaluation for inter-slot OCC with TBoMS. If TBoMS is not applied, all evaluated OCC schemes may not applicable with OCC length 4. [Initial proposal 1-4] Support. |
| Sharp | Table 2 has been updated with Sharp’s evaluation results. |
| Qualcomm | For the table, we tried to agree in the previous meeting but companies were including the results that supported their preferred technique, so we suggest to not repeat that futile exercise. Proposal 1-2: Unclear what is the target of this proposal. Proposal 1-3: We would be OK with this observations as high level, but I don’t think it would be agreeable to companies. If the SNR is very low, inter-slot OCC would work for any number of repetitions (also doing nothing would work). Proposal 1-4: We are not OK to agreeing to this at this stage. |
| Viasat | Initial proposal 1-2 This seems to be a description of a methodology, and not a necessary observation Initial proposal 1-3 This is a fair observation considering real-world scenarios. However, we have a broader concern on the performance of inter-slot OCC in the presence of CFO and their applicability within realistic scheduling restrictions. We think we should focus on other techniques. Initial proposal 1-4 Does this refer to the number of OCC supported, or to the network signalling? We should not prevent (and probably cannot prevent it anyways) the network from attempting to schedule up to 8 (or more) UE if it’s capable of resolving the conflicts by implementation. |
| Ericsson | FL Recommendation 1-1: We have updated and added results based on our simulations. Initial proposal 1-2: We don’t see a need to agree this as an observation. Initial proposal 1-3: No. Good performance can be achieved for low date rate service also with 4 UE. Good performance can be achieved for VoIP with 4 UE through grouping of UEs with similar CFO and/or improved RX algorithm. Initial proposal 1-4: Agree. Multiplexing of 8 UEs will generate high interference levels and may exceed available PDCCH capacity for dynamic grant. |
| Lenovo | Proposal 1-3: We support the proposal to limit the number of UEs to 2 for inter-slot multiplexing. Proposal 1-4: No. If we support multiple OCC techniques, then we don’t see any significant impact for combining these methods to multiplex more UEs. |
| Nokia | Proposal 1-1: OK with encouragement, but unclear where the actual proposal would lead us. Proposal 1-2: It should be clarified that the phase rotation given by these equations will be reflecting the worst case phase rotation. Additionally, this will again only show the phase rotation for a single UE. If two UE have same frequency error (for instance both at 0.1 ppm), the relative phase rotation between these two UE will be “0 degrees”, which would be manageable by the gNB. Proposal 1-3: Tentatively OK – but our preference is to support OCC with up to 4 UEs. Proposal 1-4: OK |
| NTT DOCOMO | FL Recommendation 1-1: As the applied repetitions may impact the performance of OCC techniques especially inter-slot OCC, it can be reported by companies together with the simulation results. Initial proposal 1-2: We couldn’t find the necessity of this observation. Initial proposal 1-3: Inter-slot OCC with length 4 may be support in some cases, which can be separately considered. Initial proposal 1-4: Support. |
| Samsung | FL Recommendation 1-1: Not clear this recommendation is necessary to be captured in Chair Note. Initial proposal 1-2: Not clear what motivation/purpose the proposal would like to achieve. Initial proposal 1-3: It’s more like the proposal, not observation. Actually, it doesn’t preclude the number of OCC for inter-slot at this time. If it is supported, spec should be transparent without any restriction. Initial proposal 1-4: Okay |
| Apple | Initial proposal 1-2: The intention of this observation is unclear. Initial proposal 1-3: Inter-slot OCC can support multiplexing 4 UEs in some cases. Initial proposal 1-4: Fine |
| Xiaomi | FL Recommendation 1-1: We have updated results based on our simulations. Initial proposal 1-2: We can’t see the necessity for this observation. Initial proposal 1-3: Share the same view as Ericsson. Besides, even with low SNR, inter-slot OCC could also achieve the aim of cell throughput enhancement. Initial proposal 1-4: Agree. |
| Vivo1 | FL Recommendation 1-1: we also included the results we submitted in the last meeting [Initial proposal 1-2] This observation seems unnecessary. [Initial proposal 1-3] We share similar view as LG this is more like a proposal [Initial proposal 1-4] Support. |
| CMCC | FL Recommendation 1-1: We have added our results in the tables. Initial proposal 1-2: the motivation of this proposal should be clarified. Initial proposal 1-3: from our understanding and simulations, there is no need to limited inter-slot OCC only supporting 4 UEs. Initial proposal 1-4: Agree. |
| ZTE | Initial proposal 1-2: Fine with the observation Initial proposal 1-3: We don’t agree. According to companies’ simulation results, it’s clear that inter-slot OCC could also achieve good performance with multiplexing of 4 UEs, e.g. in LDR scenario. Initial proposal 1-4: We are fine with the proposal. |
| Huawei, HiSilicon | Proposal 1-2: this observation may not be necessary. It just clarifies how the phase rotation caused by CFO is calculated. Not sure how it can help to reach consensus on the other issue. Proposal 1-3: if it is observation, we do not agree. As mentioned by previous comment, OCC4 is still possible for small data. The gap to VoIP is with the acceptable range assuming the receiver used in simulation. It can be optimized. If it is a proposal, we propose to change the wording as following: Inter-slot OCC with PUSCH repetition type A can be supported at least with multiplexing of 2 UEs. Proposal 1-4: support. |
| CATT | Initial proposal 1-2: We don’t see a need to agree this as an observation. Initial proposal 1-3: No. based on companies’ simulation results, inter-slot OCC with 4 users can have acceptable performance. Initial proposal 1-4: Agree. |
| IDC | 1-2: Not necessary. 1-3: We prefer not to restrict this to 2 UE. There may be scenarios where it may be applicable for 4 UEs. |

## 1.3 Summary of First Round Discussion

Based on first round discussions on performance of OCC techniques, there was not enough consensus achieved on Proposals 1-2, 1-3, and 1-4 for discussion in online sessions.

## 1.4 Conclusion

The following agreement was made in second online session.

Conclusion

Multiplexing of 8 UEs with PUSCH OCC is not discussed in RAN1 until the work for multiplexing of less than 8 UEs has been completed.

RAN1#118 made agreement on SNR range for R19 IoT NTN UL capacity and throughput enhancements as below:

- For 3.75kHz SCS, NPUSCH format 1 simulations are performed using an appropriate MCS with SNR at least in the range of -8dB to 0dB.

The moderator view is that a similar SNR range could be used for R19 NR NTN UL capacity and throughput enhancements

FL Recommendation 1-4:

Companies are encouraged to further contribute on performance evaluation on inter-slot OCC with PUSCH repetition Type A and pre-DFT-s OCC (comb-like structure as in PUCCH format 4) based on RAN1#118 agreement (copied in Section 5 Conclusion). Performance simulations performed using an appropriate MCS with SNR at least in the range of -8dB to 0dB would be very helpful to determine the CFO impact and OCC gain compared to single UE without OCC. To the moderator understanding, a good way to proceed will be to consider the assumptions of Rel-18 UL coverage enhancements for low data rates and for VoIP. With an appropriate MCS to fit a TB within a PUSCH repetition Type B. 2 and 4 repetitions should be considered. Larger number of repetitions may also be considered is needed for the lower SNR values.

# [CLOSED] Mapping of OCC schemes

## 2.1 Company contributing views

| Contribution | Observation/Proposals |
| --- | --- |
| Ericsson [3] | Inter-slot time domain OCC Observation 1: For inter-slot OCC based on PUSCH repetition Type A, one of the OCC multiplexed UEs can potentially be a UE without Rel-19 OCC capability. This facilitates a smooth introduction of the feature, since OCC multiplexing is possible even when the penetration of OCC capable UEs is low. Inter-symbol(s) time domain OCC Observation 2: For inter-symbols OCC based on PUSCH repetition Type B, one of the OCC multiplexed UEs can potentially be a UE without Rel-19 OCC capability. Observation 3: An OCC scheme based on current specification of four PUSCH repetitions Type B within one slot will have a high DMRS overhead and cannot utilize all OFDM symbols in the slot. Intra-symbol(s) time domain OCC Observation 4: For intra-symbol OCC, multiplexing with a UE without OCC capability on the same time-frequency resource is not possible. Observation 5: Intra-symbol OCC requires updates of the TBS/rate matching design while inter-slot OCC (based on PUSCH repetitions Type A) or inter-symbols OCC (based on PUSCH repetitions Type B) do not. Observation 16: Intra-symbol OCC, when applied prior to the DFT precoder, is equivalent to a sub-PRB allocation where every nth subcarrier is used by a given UE. Observation 17: When more than one PRB is available, multiplexing can also be achieved with FDM using separate PRBs per UE. Observation 18: Intra-symbol pre-DFT OCC for 2 UEs across 2 PRBs has the same capacity and performance as 2 UEs allocated to one PRB each. TBoMS: Observation 6: TBoMS can help intra-symbol OCC to keep a low MCS. Comparison /preference of OCC Proposal 2: Specify support for inter-slot OCC based on PUSCH repetition Type A for up to 4 UEs. |
| ZTE [4] | Inter-slot time domain OCC Observation 1: Inter-slot time domain OCC can provide the promising gain with limited spec effort and implementation complexity considering the tolerance to frequency offset. Observation 3: Inter-slot time domain OCC is the only scheme that supports backward compatibility with non-Rel-19 UEs. Proposal 1: In NR NTN with inter-slot time domain OCC, the redundancy versions for different repetitions should be kept the same. Inter-symbol(s) time domain OCC Observation 4: Inter-symbol time domain OCC requires significant change on the legacy resource mapping. Observation 5: Inter-symbol time domain OCC has higher tolerance to frequency offset. Intra-symbol pre-DFT-s OCC Observation 7: Intra-symbol Pre-DFT OCC may have impact on the required frequency domain resources, coding rate, PAPR and transmission power. TBoMS: Observation 2: TBoMS is not required to be configured along with inter-slot time domain OCC. Observation 6: Inter-symbol time domain OCC is feasible only if the enhanced TBoMS is jointly supported to maintain reasonable coding rate. Observation 8: Intra-symbol Pre-DFT OCC is feasible only if the TBoMS is jointly supported to maintain reasonable coding rate in some scenarios. Comparison /preference of OCC Proposal 2: Considering the trade-off between performance and specification impact, inter-slot time domain OCC should be supported. |
| Huawei, HiSilicon [5] | Inter-slot time domain OCC Observation 1: The existing resource mapping can be reused for inter-slot time-domain OCC with PUSCH repetition type A. Inter-symbol time domain OCC Observation 2: The existing resource mapping can be reused for inter-symbol OCC with PUSCH repetition type B under the condition that the nominal repetition is not across slot boundary. Intra-symbol pre-DFT-s OCC Observation 3: Intra-symbol pre-DFT-s OCC can be applied to PUSCH with repetition type A/B and TBoMS. The legacy time domain resource mapping can be reused. TBoMS: Observation 11: TBoMS is an optional feature and bonding with it will greatly limit the flexibility of employing OCC techniques. Proposal 6: TBoMS can be used when OCC is applied, while there is no need to bond it with OCC. Comparison /preference of OCC Proposal 7: For FR1-NTN, support to specify inter-slot time-domain OCC with PUSCH repetition type A. - For UE support PUSCH repetition type B, inter-symbol time-domain OCC with repetition type B can also be considered when the nominal repetition is not across slot boundary. |
| Nokia [6] | Proposal 1: RAN1 to support only OCC with PUSCH repetition Type A in Rel-19 NR NTN. |
| Qualcomm [9] | Bandwidth limitation: Proposal 3: OCC schemes can only be applied when the number of allocated PRBs is 1. TBoMS Proposal 4: OCC is supported for TBoMS at least for the case of repetitions |
| OPPO [7] | TBoMS Proposal 4: Support TBoMS for PUSCH transmission with OCC. Comparison /preference of OCC Proposal 1: Support at least one of the following OCC schemes for PUSCH transmission: inter-slot OCC, inter-symbol OCC, intra-symbol pre-DFT-s OCC. |
| Apple [8] | Comparison /preference of OCC Proposal 4: Consider inter-slot time domain OCC with PUSCH repetition type A, with minimal change to existing UCI multiplexing rule, and/or intra-symbol pre-DFT-s OCC. |
| MediaTek [11] | Inter-slot time domain OCC Observation 3: Inter-slot OCC with PUSCH repetition Type A and Type B can re-use legacy resource allocation in time domain for block-wise spreading of DFT-s-OFDM symbols y(n) with OCC sequence wi(m) of length m=NSFPUSCH based on equation. zmMscPUSCHNsymbslot+n=wimyn n=0,..,MscPUSCHNsymbslot-1 m=0..NSFPUSCH-1 NSFPUSCH≤K Where Nsymbslot is the number of DFT-s-OFDM symbols per slot and K is the number of repetitions. The repetitions are done slot wise for PUSCH repetition Type A with PUSCH mapping Type A and PUSCH repetition Type B with PUSCH mapping Type B. Observation 7: Multiplexing Rel-19 UEs that support OCC for PUSCH with non-Rel-19 UEs may be feasible with inter-slot OCC schemes. Inter-symbol time domain OCC Observation 4: Inter-symbol(s) OCC for block-wise spreading of DFT-s-OFDM symbols y(n) using OCC sequence wi(m) of length m=NSFPUSCH based on equation. zmMscPUSCH+n=wimyn n=0,..,Nsymbgroup MscPUSCH-1 m=0..NSFPUSCH-1 NSFPUSCH≤K Where Nsymbgroup is the number of symbols per group and K is the number of repetitions. The repetitions are done symbol(s)-wise, which can be implemented with PUSCH repetition Type B with PUSCH mapping Type B to minimize change to resource allocation in TS 38.214 Clause 6.1.2.1. Observation 6: PUSCH repetition Type B with PUSCH mapping Type B is a straightforward implementation of inter-symbol(s) OCC. Intra-symbol pre-DFT-s OCC Observation 2: Intra-symbol (comb-like structure as in PUCCH format 4) requires doubling of transmission bandwidth. Observation 5: Intra-symbol (comb-like structure as in PUCCH format 4) can re-use legacy resource allocation in frequency domain and legacy block-wide spreading in TS-38.211 Clause 6.3.2.6.3 for PUCCH 4 with MscPUCCH,s replaced by MscPUSCH in following equation, where MscPUSCH=MRBPUSCHMscRB xlMscPUSCH+k=wnkNSFPUSCHMscPUSCHdlMscPUSCHNSFPUSCH+k mod MscPUSCHNSFPUSCH k=0..NscPUSCH-1 l=0,1,…,NSFPUSCHMsymbMscPUSCH-1 Comparison /preference of OCC Proposal 1: RAN1 to support Inter-slot OCC with PUSCH repetition Type A or Type B for OCC length 2 and 4. Proposal 2: Postpone discussion on combination of OCC techniques until RAN1 makes agreement on down-scoping of OCC techniques. |
| CATT [12] | Proposal 1: Based on standardization impact and complexity analysis, it is proposed to adopt inter-slot OCC scheme in UL capacity enhancement of NR NTN. |
| ETRI [26] | Proposal 2. RAN1 to support OCC for TBoMS transmission, at least for the following cases: - Case #1: numberOfSlotsTBoMS = 2, numberOfRepetitions = 2 0; 1; 2; 3; Slot number. 0; 0; 0; 0; Used RV index per slot. 1; 1; 1; 1; OCC #1. 1; 1; -1; -1; OCC #2. - Case #2: numberOfSlotsTBoMS = 2, numberOfRepetitions = 4 0; 1; 2; 3; 4; 5; 6; 7; Slot number. 0; 0; 0; 0; 2; 2; 2; 2; Used RV index per slot. 1; 1; 1; 1; 1; 1; 1; 1; OCC #1. 1; 1; -1; -1; 1; 1; -1; -1; OCC #2. - Case #3: numberOfSlotsTBoMS = 2, numberOfRepetitions = 8 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; 10; 11; 12; 13; 14; 15; Slot number. 0; 0; 0; 0; 2; 2; 2; 2; 3; 3; 3; 3; 1; 1; 1; 1; Used RV index per slot. 1; 1; 1; 1; 1; 1; 1; 1; 1; 1; 1; 1; 1; 1; 1; 1; OCC #1. 1; 1; -1; -1; 1; 1; -1; -1; 1; 1; -1; -1; 1; 1; -1; -1; OCC #2. |
| Spreadtrum [14] | Proposal 1: Inter-symbol time-domain OCC with PUSCH repetition Type B is deprioritized, due to potential segmentation and scheduling restrictions. Proposal 2: Time-domain OCC with TBoMS is deprioritized. Since TBoMS option 1 suffer from larger timing drift, and fundamental change to resource mapping of TBoMS in option 2. Proposal 3: Only inter-slot time-domain OCC with PUSCH repetition Type A with OCC length 2 and 4 is specified for R19 NTN. |
| Vivo [27] | TBoMS Observation 1: If same code rate as well as same TBS is to be guaranteed between inter-slot OCC and inter-symbol OCC/intra-symbol OCC, across slot mapping (i.e., TBoMS) should be enabled for inter-symbol OCC/intra-symbol OCC. Comparison /preference of OCC Proposal 2. Support a same OCC scheme for OCC length=2 and OCC length=4. |
| Xiaomi [13] | Observation 2: In the case of pre-DFT based OCC spreading for UE multiplexing, the actual number of coded bits carried in a slot will be reduced compared to the transmission of a single UE with the same number of resources and modulation order. Observation 3: In the case of inter-symbol(s) OCC spreading for UE multiplexing, the actual number of coded bits carried in a slot will be reduced compared to the transmission of a single UE with the same number of resources and modulation order. |
| Samsung [10] | Observation 1: Inter-slot time-domain OCC with PUSCH repetition Type A may have the least specification impact(s). Observation 2: Inter-symbol(s) time domain OCC may have the most specification impact(s). Observation 3: Inter-slot time-domain OCC with PUSCH repetition Type A doesn’t have any issue for backward compatibility with non-Rel-19 UEs. Observation 4: Inter-symbol time-domain OCC and Intra-symbol pre-DFT-s OCC have an issue for backward compatibility with non-Rel-19 UEs. TBoMS: Proposal 5: RAN1 defers the decision whether to support TBoMS for PUSCH with OCC in Rel-19 NTN. Comparison /preference of OCC Proposal 2: RAN1 supports only one OCC technique in Rel-19 NTN. |
| NEC [21] | Proposal 1: Study the PAPR impact on the transmitter of a terminal to spread the PUSCH with OCC for NR-NTN uplink capacity/throughput enhancement. |
| DoCoMo [17] | Proposal 4: For PUSCH inter-slot time-domain OCC, the inter-slot OCC is assigned to repetitions as follows: - If OCC length = repetition factor, each repetition is assigned with one bit of OCC sequence - If OCC length < repetition factor, study the mapping of the OCC bits and repetitions - E.g., for OCC length = 4 and repetition=8, the mapping of the OCC bits can be [w0 w1 w2 w3 w0 w1 w2 w3], or [w0 w0 w1 w1 w2 w2 w3 w3] - The slots which are associated with one OCC sequence (i.e., [w0 w1 w2 w3] ) are assigned with the same RV. Proposal 7: For PUSCH with intra-symbol pre-DFT OCC, a code of an OCC sequence (e.g., +1 or -1) is assigned within a PRB and assigned for each set of subcarriers which consists of 6 or 3 subcarriers. TBoMS: Proposal 5: For inter-slot time-domain OCC with TBoMS, discuss how to assign OCC sequence. Comparison /preference of OCC Proposal 1: Support at least one of following OCC techniques with code length 2 or 4: - Inter-slot time-domain OCC with PUSCH repetition Type A and/or TBoMS - Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) Proposal 2: Combinations of OCC techniques is not supported. Proposal 3: Support OCC for PUSCH with TBoMS. |
| Panasonic [16] | Observation 2: Inter-symbol time domain OCC would have a problem on the setting of OCC length because possible OCC length depends on the number of PUSCH symbols in a slot, which is affected by DMRS configuration and/or SRS symbol(s). |
| Google [20] | Proposal 1: Inter-slot (PUSCH repetition Type A) OCC can be the base line for UL capacity enhancement. Observation 1: With OCC length 2 and 4, the performance difference between OCC with PUSCH repetition Type B and OCC with interlaced repetitions might be marginal. TBoMS: Proposal 4: If inter-symbol OCC with PUSCH repetition Type B is supported, TBoMS should be applied. Comparison /preference of OCC Proposal 2: With regard to the performance, PAPR, and spec impact, inter-symbol OCC with PUSCH repetition Type B is preferred. Proposal 3: Intra-symbol OCC is not considered for R19 NTN. |
| Lenovo [15] | Comparison /preference of OCC Proposal 2: RAN1 to support the inter-slot time domain OCC (OCC across slots) with fixed redundancy version and OCC length 2. Proposal 3: Support intra-slot time domain OCC with OCC length 2 or 4. Proposal 4: RAN1 to consider the symbol mapping types for intra-slot time domain OCC. Proposal 5: For intra-slot OCC, consider the simultaneous use of OCC length 2 and 4 in a slot to multiplex more UEs and for providing full flexibility to the network to schedule the resources. Proposal 6: RAN1 to deprioritize the intra-symbol OCC scheme. Proposal 7: Support the use of combination of inter-slot technique with OCC length 2 and intra-slot technique with OCC length 2 or 4. |
| CMCC [18] | Observation 1: PUSCH repetition type A is required for all data traffic transmission in uplink for NR-NTN, including both small data rate and VoIP traffic. Observation 2: The inter-slot time-domain OCC has the least specification impacts for PUSCH RE mapping and is easiest one for realization, compared with the other two OCC solutions. Observation 3: Inter-slot time-domain OCC schemes need to pay more attention on realistic impairments. Observation 4: The inter-symbol time domain OCC would have a large specification impact for either the realization of consecutive symbol repetitions within one slot or PUSCH repetition Type B-like operations. Observation 5: The performance of PUSCH transmission adopting two PRBs with intra-symbol pre-DFT-s OCC with OCC length 2 has almost the similar or even worse performance than PUSCH transmission with single UE and single PRB. The latter scheme will not suffer from the inter-comb interference and can be realized without any specification efforts. TBoMS: Observation 6: TBoMS should be introduced to maintain the same data rate and lower the code rate for PUSCH transmission if the inter-symbol OCC or the intra-symbol pre-DFT-s OCC is introduced for multiple UE multiplexing. Comparison /preference of OCC Proposal 2: Considering both the performance and the specification impact, the inter-slot OCC scheme is preferred for the normative phase. Proposal 6: Only one solution should be down-selected from the current candidate solutions. Proposal 7: If a combined solution with multiple candidate solutions is supported, only one UE capability is defined or supported without differentiation of any sub-capability or sub-solutions. |
| China Telecom [19] | Comparison /preference of OCC Proposal 2: Considering the capacity gains and specification impact, at least one of the OCC techniques should be specified: - Inter-slot time-domain OCC with PUSCH repetition Type A with OCC length 2. - Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2 or 4 |
| Fujitsu [30] | Observation 1: For inter-symbol OCC requires more discussion on data structure, i.e. how to repeat the OCCed OFDM symbols including the avoidance of orphan symbols while that for inter-slot and intra-symbol OCC is straightforward. Observation 3: For inter-symbol OCC, RAN1 needs to specify the multiplication of OCC sequence and the mapping procedure to each symbol/slot. Observation 4: For inter-slot OCC, RAN1 needs to specify the multiplication of OCC sequence, but the spec impact on the mapping procedure over slot is not expected. TBoMS: Proposal 6: TBoMS with OCC should be supported. Comparison /preference of OCC Proposal 1: For multiplexing 2 or 4 UEs, inter-slot OCC is slightly preferred to select in Rel-19 due to less specification impact. Proposal 2: OCC combination is not preferable due to specification impact. |
| InterDigital [22] | Observation 4: Sub-slot OCC based PUSCH transmissions may result in less parity bits being repeated in each PUSCH repetition. TBoMS: Proposal 7: Support OCC based PUSCH transmissions with TBoMS where OCC length is equal to the TBoMS length in number of slots. Comparison /preference of OCC Observation 1: Intra-symbol and inter-symbol OCC based PUSCH transmissions involve higher specification impact and effort compared to inter-slot OCC based PUSCH transmissions. Proposal 3: Inter-slot OCC is supported for slot based PUSCH repetitions. Proposal 4: Support Inter-slot and one of the symbol level OCC schemes. |
| LG [23] | Comparison /preference of OCC Observation #1: Inter-symbol(s) time domain OCC and inter-symbol(s) pre-DFT-s OCC have shown similar performance. Observation #2: When TB over multiple PUSCHs or slots is not applied, BLER performance of inter-symbol(s) OCC may be degraded since the coding rate increases as OCC length increases. Proposal #1: Support inter-slot time-domain OCC with PUSCH repetition Type A in in Rel-19 NR NTN. Proposal #2: If inter-symbol(s) OCC is supported in Rel-19 NR NTN, one of the following two OCC technique(s) is down-selected: - Inter-symbol(s) time domain OCC - Inter-symbol pre-DFT-s-OCC TBoMS: Proposal #9: Study TBoMS enhancement to support orthogonality of OCC for inter-slot time domain OCC. |
| NICT [25] | Comparison /preference of OCC Observation 1: The OCC across OFDM symbols and OCC within an OFDM symbol (pre-DFT OCC) involve significant changes to the specifications, while the OCC scheme across slots requires fewer modifications. |
| TCL [26] | Proposal 3: For OCC across slots, at least the total number of repetitions after OCC spreading should be studied. Proposal 5: For the scheme of OCC across symbols, the number of symbols after OCC spreading should be clarified. Proposal 6: For the scheme of OCC across symbols, it is necessary to study whether to support Type B PUSCH with OCC. TBoMS: Proposal 7: TBoMS for PUSCH with OCC can be studied for the enhancement of uplink performance and the slot number should be scaled by the OCC length. |
| Sharp [29] | TBoMS: Proposal 1: At least intra-symbol OCC with TBoMS should be supported. |

Backward compatibility:

Ericsson [2], Huawei [5], MediaTek [11] observed that for inter-slot OCC and inter-symbol(s) OCC with PUSCH repetition type B, one of the OCC multiplexed UEs can potentially be a UE without Rel-19 OCC capability. Configured grant can be configured to use a fixed RV, while PUSCH repetition Type A scheduled by dynamic grant always performs RV cycling per repetition. His would allow a smooth introduction of OCC features when there are not many OCC-capable UEs and be beneficial for system capacity. Backward compatibility is not possible for intra-symbol OCC.

ZTE[4] observed inter-slot time domain OCC is the only scheme that supports backward compatibility with non-Rel-19 UEs.

Resource mapping with OCC:

Huawei [5], Spreadtrum [5] observed the existing resource mapping can be reused for inter-slot time-domain OCC with PUSCH repetition type A; it can also be used for inter-symbol OCC with PUSCH repetition type B under the condition that the nominal repetition is not across slot boundary.

Spreadtrum [14] Figure 1: 6 symbols in PUSCH nominal repetition

Huawei [5] observed as intra-symbol pre-DFT-s OCC can be applied to PUSCH with repetition type A/B and TBoMS, the legacy time domain resource mapping can be reused.

Inter-symbol(s) OCC with PUSCH repetition Type B:

Ericsson [2], Huawei [5], Apple [8], MediaTek [11] mentioned PUSCH repetition typeB can be considered as a method of performing OCC across the symbols. To our understanding, the group mapping way is inter-symbol(s) OCC with symbol cluster and can be seen as PUSCH repetition type B with SLIV indicating the symbol position S and the number of symbols L as specified in TS 38.214 Clause 6.1.2.1 (S=0 and L=6 in the example below from MediaTek [11]). There is no change in specifications as TBS calculation and resource mapping are unchanged from legacy behavior.

Huawei [5] proposed for UE support PUSCH repetition type B, inter-symbol time-domain OCC with repetition type B can also be considered when the nominal repetition is not across slot boundary.

CMCC [18] discussed inter-symbol OCC with PUSCH repetition Type B with 7 symbols can be other way, but for other value of symbol numbers, e.g. 6 or 8 symbols, the operation for across slot repetitions or the actual repetitions should be studied further.

MediaTek [11] observed inter-slot OCC with PUSCH repetition Type A and Type B can re-use legacy resource allocation in time domain for block-wise spreading of DFT-s-OFDM symbols y(n) with OCC sequence wi(m) of length m=NSFPUSCH  based on equation.

zmMscPUSCHNsymbslot+n=wimyn

n=0,..,MscPUSCHNsymbslot-1

m=0..NSFPUSCH-1

NSFPUSCH≤K

Where Nsymbslot is the number of DFT-s-OFDM symbols per slot and K is the number of repetitions. The repetitions are done slot wise for PUSCH repetition Type A with PUSCH mapping Type A and PUSCH repetition Type B with PUSCH mapping Type B.

Ericsson [3] showed 2 DM RS per 14-symbol slot are used with DM RS mapping Type A with OCC 2 or 4. All OFDM symbols in the slot are used.

Ericsson [3] Figure 1: Schematic view of inter-slot OCC.

Comparison /preference of OCC

Proposal 1: RAN1 to support Inter-slot OCC with PUSCH repetition Type A or T/ype B for OCC length 2 and 4.

Inter-symbol(s) OCC with symbol-level repetition:

Panasonic [16] discussed because the number of PUSCH OFDM symbols in a slot depend on the number of DMRS symbols and SRS symbols, possible OCC length is restrictive due to DMRS symbol and SRS symbol. OCC spread symbols does not fit into the available PUSCH symbols in a slot when available PUSCH symbols are not multiple of OCC length.

MediaTek [11] discussed inter-symbol time domain OCC with PUSCH repetition Type B can have higher tolerance to timing drift and CFO with smaller values of L (e.g. with L=1, it should be as robust to these impairments as intra-symbol OCC).

TS 38.214 Clause  6.1.2.1 “Resource allocation in time domain”

The UE shall consider the S and L combinations defined in table 6.1.2.1-1 as valid PUSCH allocations

TS 38.214 Table 6.1.2.1-1: Valid S and L combinations

| PUSCH mapping type | Normal cyclic prefix |  |  | Extended cyclic prefix |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | S | L | S+L | S | L | S+L |
| Type A (repetition Type A only) | 0 | {4,…,14} | {4,…,14} | 0 | {4,…,12} | {4,…,12} |
| Type B | {0,…,13} | {1,…,14} | {1,…,14} for repetition Type A, {1,…,27} for repetition Type B | {0,…, 11} | {1,…,12} | {1,…,12} for repetition Type A, {1,…,23} for repetition Type B |

MediaTek [11] Figure 1: Example PUSCH repetition Type B with PUSCH Mapping Type B

ZTE [4] observed inter-symbol(s) time-domain OCC with symbol-level repetition requires significant change on the legacy resource mapping. Though inter-symbol time domain OCC has higher tolerance to frequency offset, it is feasible only if the enhanced TBoMS is jointly supported to maintain reasonable coding rate.

Several companies mentioned the TBS should be scaled by OCC length. To the moderator understanding this is to maintain the same code rate. The other way is to increase the code rate by a factor equal to OCC length if the TBS is not scaled by OCC length.

ZTE [4] Figure 1Illustration of inter-slot time domain OCC and inter-symbol time domain OCC

MediaTek [11] observed PUSCH repetition Type B with PUSCH mapping Type B is a straightforward implementation of inter-symbol(s) OCC for block-wise spreading of DFT-s-OFDM symbols y(n) using OCC sequence wi(m) of length m=NSFPUSCH   based on equation.

zmMscPUSCH+n=wimyn

n=0,..,Nsymbgroup  MscPUSCH-1

m=0..NSFPUSCH-1

NSFPUSCH≤K

Where Nsymbgroup  is the number of symbols per group and K is the number of repetitions. The repetitions are done symbol(s)-wise, which can be implemented with PUSCH repetition Type B with PUSCH mapping Type B to minimize change to resource allocation in TS 38.214 Clause 6.1.2.1.

Huawei [5], Spreadtrum [5], Ericsson [3] discussed the existing resource mapping can be reused for inter-symbol OCC with PUSCH repetition type B. It can be used under the condition that the nominal repetition is not across slot boundary. The specification mandates the legacy DMRS mapping Type B when PUSCH repetition Type B is used. At least one DMRS symbol per repetition (located in the first symbol of each repetition) is needed. By comparison, inter-slot OCC with PUSCH repetition Type A has 2 DM RS per slot for OCC 2 or 4. It can be noted that the DMRS overhead is large and not all OFDM symbols in the slot can be utilized. To the moderator understanding, only one configuration 6 OFDM symbols and 1 DM RS symbol will allow inter-symbol OCC with PUSCH repetition B with use of OFDM symbols in the slot and OCC length 2. All other configurations will have at least 2 unused OFDM symbols or more per slot to ensure nominal repetition is not across slot boundary. The un-used symbols may be used for legacy scheduling of UL transmission with PUSCH repetition B with resource mapping B. Examples are shown in Figure below:

- Configuration with 6 OFDM symbols and 1 DM RS symbol with OCC length 2 uses all OFDM symbol per slot and 1/6 DM RS overhead

- Configuration with 2 OFDM symbols and 1 DM RS symbol with OCC length 4 has 2 un-used OFDM symbol per slot and 1/3 DM RS overhead

We list below the possible configurations with un-used OFDM symbols and DM RS overhead for 2 UE multiplexing and 4 UE multiplexing.

- 1 OFDM symbol and 1 DM RS symbol (2 or 6 OFDM symbols unused with OCC length 2 or 4, 1/2 DM RS overhead)

- 2 OFDM symbols and 1 DM RS symbol (2 OFDM symbols unused with OCC length 2 or 4, 1/3 DM RS overhead)

- 3 OFDM symbols and 1 DM RS symbol (6 OFDM symbols unused with OCC length 2, and OCC length N/A, 1/4 DM RS overhead)

- 4 OFDM symbols and 1 DM RS symbol (4 OFDM symbols unused with OCC length 2 and OCC length 4 N/A, 1/5 DM RS overhead)

- 5 OFDM symbols and 1 DM RS symbol (2 OFDM symbols unused with OCC length 2 and OCC length 4 N/A, 1/6 DM RS overhead)

- 6 OFDM symbols and 1 DM RS symbol (all OFDM symbols used with OCC length 2 and OCC length 4 N/A, 1/7 DM RS overhead)

Spreadtrum [14] Figure 1: 6 symbols in PUSCH nominal repetition

Ericsson [3] Schematic view of inter-symbols OCC with 4 UEs using PUSCH repetitions Type B.

Intra-symbol OCC

MediaTek [11] observed a comb-like structure where two UEs can be frequency division multiplexed with each UE using every other sub-carrier requires a twofold increase in UL transmission bandwidth with a 3 dB link budget loss. For example, assuming 2 RBs are needed for a voice packet @ 4.75 kbps, then 4 RBs will be used for the transmission with such scheme and result in a 3 dB link loss.

Ericsson [3] discussed OCC within an OFDM symbol, when applied prior to the DFT precoder, is equivalent to a sub-PRB allocation where every nth subcarrier is used by a given UE. When the number of allocated PRBs is larger than 1, there is no gain observed. When more than one PRB is available, multiplexing can also be achieved with FDM using separate PRBs per UE. Intra-symbol pre-DFT OCC for 2 UEs across 2 PRBs has the same capacity and performance as 2 UEs allocated to one PRB each.

Ericsson [2] Link performance with OCC versus FDM

/MediaTek [11] observed intra-symbol (comb-like structure as in PUCCH format 4) requires doubling of transmission bandwidth. Intra-symbol (comb-like structure as in PUCCH format 4) can re-use legacy resource allocation in frequency domain and legacy block-wide spreading in TS-38.211 Clause 6.3.2.6.3 for PUCCH 4 with MscPUCCH,s replaced by MscPUSCH in following equation,  where MscPUSCH=MRBPUSCHMscRB

xlMscPUSCH+k=wnkNSFPUSCHMscPUSCHdlMscPUSCHNSFPUSCH+k mod MscPUSCHNSFPUSCH

k=0..NscPUSCH-1

l=0,1,…,NSFPUSCHMsymbMscPUSCH-1

TBoMS:

Several companies mentioned TBS in intra-symbol pre-DFT-s OCC is scaled down by a factor of OCC length from the legacy TBS determination. We copy Ericsson’s discussion on TBoMS for intra-symbol OCC. The legacy rules for TBS calculation are based on following equation: Ninfo= NRE∙R∙Qm∙v. In the rate matching, NRE∙Qm∙v coded bits will be taken out from the circular buffer. In intra-symbol OCC, the modulation symbols need to be spread several times. This will result in that not all the modulation symbols can be mapped to the resources, which implies that parts of coded bits will be dropped. The characteristics of the LDPC code will be destroyed, causing decoding failure at the receiver. In order to avoid this, when calculating TBS and rate matching, the TBS and number of coded bits from circular buffer need to be divided by the spreading factor, where TBS calculation is modified as Ninfo= NRE∙R∙Qm∙v/NSF and NRE∙Qm∙v/NSF coded bits taken out from the buffer. After this, to keep the same throughput, OCC within an OFDM symbol needs to use a higher MCS, split the TB into smaller TBs or enable TBoMS to increase NRE. Thus TBoMS can help intra-symbol OCC keep low MCS to get better coverage performance.

Several companies discussed loss of systematic bits with high MCS (or TBS scaled down if code rate is not increased) in intra-symbol OCC and inter-symbol OCC (symbol-level repetitions) and observed TBoMS is a pre-requisite for these OCC techniques.

Ericsson [2] Figure 5: Rate-matching options for OCC within an OFDM symbol.

Vivo [27] Observed if same code rate as well as same TBS is to be guaranteed between inter-slot OCC and inter-symbol OCC/intra-symbol OCC, across slot mapping (i.e., TBoMS) should be enabled for inter-symbol OCC/intra-symbol OCC.

Vivo [27] Figure 1. Example of OCC across slots/ across symbols with OCC length=2

Vivo [27] Figure 2. Example of OCC within symbols with OCC length=2

Huawei [5] observed TBoMS is an optional feature. TBoMS can be used when OCC is applied, but there is no need to bond it with OCC as it will greatly limit the flexibility of employing OCC techniques.

Huawei mentioned inter-symbol OCC with PUSCH repetition Type B with 7 OFDM symbols per repetition over 2 slots for OCC factor 4 has similar CFO experienced as inter-slot OCC with PUSCH repetition Type A over 2 slots with OCC factor 2..

ZTE [4] observed intra-symbol Pre-DFT OCC is feasible only if the TBoMS is jointly supported to maintain reasonable coding rate in some scenarios. TBoMS is not required to be configured along with inter-slot time domain OCC.

OPPO [7] mentioned in current specification, TBoMS is supported for PUSCH repetition Type A to reduce TB code rate and enhance coverage level, which should also be supported with OCC if inter-slot OCC is supported. For inter-symbol OCC (to the moderator understanding, this is for inter-symbol OCC not based on PUSCH repetition B) and intra-symbol pre-DFT-s OCC, the available RE number for TBS determination should be scaled by the OCC length. If the allocated frequency resource for PUSCH transmission is small to obtain high transmit power density for coverage enhancement, then the determined TBS would be very small after taking OCC scaling into account. In this case, TBoMS is beneficial and should be considered.

Spreadtrum [14] observed impact of timing drift is higher if one transmission resource for TBoMS is several slots and OCC applies on top of one transmission occasion of TBoMS. The performance loss would larger than inter-slot OCC with PUSCH repetition type A (without TBoMS to the moderator understanding).

Qualcomm [9] proposed OCC is supported for TBoMS at least for the case of repetitions.

Number of PRBs with OCC:

Qualcomm [9] discussed the addition of OCC to the encoding chain of PUSCH is a non-trivial modification (from implementation point of view). Having to perform OCC “at the envelope” (i.e., without limiting the number of PRBs, modulation order, etc.) would require to overdimension the UE hardware and processing to accommodate spreading + OCC with very large allocations, or to have to run the hardware at higher clock rates (with the corresponding increasing power consumption) to meet the timeline requirements. OCC schemes can only be applied when the number of allocated PRBs is 1.

Combination of OCC Techniques:

Several companies proposed only support multiplexing 2 or 4 UEs without combination of OCC schemes in Rel-19 NR-NTN, or postpone discussions until decision is made on which OCC techniques should be supported.

Moderator summary and views on mapping of OCC schemes:

- Greater than 1 PRB with OCC with PUSH: There is not enough consensus to limit OCC with PUSCH to 1 PRB. It is up to gNB implementation.

- Inter-slot time-domain OCC: can reuse existing resource mapping for PUSCH repetition type A.

- Intra-symbol pre-DFT OCC can be applied to PUSCH with repetition type A/B and TBoMS, and re-use the existing time-domain resource resource mapping. It can be configured with 4 symbols to 14 symbols ove4/14 if r 12 sub-carriers with OCC spreading factor 2 or 4. The DM RS with 14 symbols is 2/14 or 4/14 with additional DM RS symbols.

- Inter-symbol(s) OCC with PUSCH repetition B The resource mapping for PUSCH repetition type B can be reused when the number of symbols per symbol group is same as the length of PUSCH and the nominal repetition is not across slot boundary. This results in limitation of configuration of symbols per repetition, high DM RS overhead, and un-used symbols. One possible configuration is to use 7 OFDM symbols per repetition over 2 slots for OCC factor 4, which has similar CFO experienced as inter-slot OCC over 2 slots with OCC factor 2. To the moderator understanding there is no difference of meaning between inter-slot OCC with PUSCH repetition B or inter-symbol OCC with PUSCH repetition B as the repetition are done over a group of symbols (i.e. not done symbol-level repetition).

- TBoMS is a necessary pre-requisite for Intra-symbol OCC and Inter-symbol(s) OCC (symbol-level repetition) assuming available RE per slot (NRE) for TB size determination should be scaled down by occ-length for inter-symbol(s) time domain OCC and intra-symbol pre-DFT OCC.

- Backward compatibility: Inter-slot time-domain OCC with PUSCH repetition A and Inter-symbol OCC with PUSCH repetition can be used with 1 non-Release 19 UE assigned the code [+1, +1] or [+1, +1, +1, +1] for OCC length 2 or 4 respectively.

- Inter-slot time-domain OCC with PUSCH repetition Type A, Inter-symbol OCC with PUSCH repetition Type B, and intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2 or 4 should be potential OCC techniques prioritized to minimize overall impact on the specifications.

## 2.2 First Round Discussion

FL Recommendation: Companies are encouraged to input / update their 1st preference, 2nd preference and 3rd preference, for OCC techniques in table below (as copied from FL summary in RAN1#117 [2]), and whether they would want RAN1 to consider combinations of OCC techniques. Separate comments on the companies preferences can be made in the comment table further below.

Companies preference on OCC schemes:

| Table to indicate companies preferences on OCC technique options A, Option B, and Option C: - Option A: Inter-slot time-domain OCC with PUSCH repetition Type A - Option B: Inter-symbol(s) time domain OCC (w/ or w/o PUSCH repetition Type B) - Option C: Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) - Option D: Others |  |  |  |  |
| --- | --- | --- | --- | --- |
| Company | 1st preference | 2nd preference | 3rd preference | Combination |
| Qualcomm | C | B |  | C+B |
| LGE | A | C | B |  |
| NTT DOCOMO | A | C | B |  |
| Nokia | A |  |  | No |
| Panasonic | C | A | B |  |
| OPPO | A | C |  | A+C |
| Apple | A |  |  |  |
| Huawei, HiSilicon | A | B (with repetition type B) |  | No |
| ZTE | A | B (with repetition type B) |  | No |
| MediaTek | A | B (with repetition type B) | C |  |
| CATT | A |  |  |  |
| Fujitsu | A | B | C | No |
| Ericsson | A | B (with repetition type B) | C |  |
| Spreadtrum | A | B (with repetition type B) | C | No |
| NICT | A |  |  |  |
| Sharp | C | A | B |  |
| Viasat | C |  |  |  |
| TCL | A | B | C |  |
| Lenovo | A | B | C | A+B |
| Xiaomi | A | C | B | No |
| CMCC | A |  |  | No combinaiton |
| vivo | A | C |  | No |
| HONOR | A | B |  |  |
| IDC | A | B | C |  |
| Moderator summary |  |  |  |  |

Initial proposal 2-1:

Scheduling PUSCH with OCC with 1 PRB or greater than 1 PRB is up to gNB implementation.

Initial proposal 2-2:

Inter-symbol(s) with PUSCH repetition B is only considered under the condition that the nominal repetition is not across slot boundary.

Initial proposal 2-3:

Observation

Intra-symbol OCC or Inter-symbol(s) OCC (symbol-level repetition) should be used with TBoMS assuming available RE per slot (NRE) for TB size determination are scaled down by occ-length to allow gNB scheduling with low MCS.

Initial proposal 2-4:

Intra-symbol OCC and inter-symbol OCC (symbol-level repetition) are only considered with TBoMS.

Initial proposal 2-5:

Intra-symbol pre-DFT OCC can be applied to PUSCH with repetition type A/B and TBoMS, and re-use the existing time-domain resource resource mapping.

Initial proposal 2-6:

Observation

Inter-slot time-domain OCC with PUSCH repetition A and Inter-symbol OCC with PUSCH repetition can be used with 1 non-Release 19 UE assigned the code [+1, +1] or [+1, +1, +1, +1] for OCC length 2 or 4 respectively.

Initial proposal 2-7:

Observation:

Inter-symbol OCC with PUSCH repetition Type B with 7 OFDM symbols per repetition over 2 slots with OCC factor 4 has similar CFO impact as inter-slot OCC with PUSCH repetition Type A over 2 slots with OCC factor 2.

Note: Inter-symbol OCC with PUSCH repetition Type B and Inter-slot OCC with PUSCH repetition Type B have same meaning.

Initial proposal 2-8:

The following OCC techniques are prioritised:

- Inter-slot time-domain OCC with

  - PUSCH repetition Type A with OCC length 2

  - PUSCH repetition Type B with 7 symbols per repetition over 2 slots with OCC length 4

- Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 4

Companies are encouraged to provide further comments in the comment table below as needed:

| Companies | Comments |
| --- | --- |
| Panasonic | Initial proposal 2-1: Agree. Initial proposal 2-2: Agree. Initial proposal 2-3, Initial proposal 2-4: Use of TBoMS for intra-symbol OCC and inter-symbol OCC is preferable to keep enough large TBS and/or use lower MCS, but it’s up to gNB implementation. We propose to modify as follows. Use of Intra-symbol OCC or Inter-symbol(s) OCC (symbol-level repetition) should be used together with TBoMS should be supported assuming available RE per slot (NRE) for TB size determination are scaled down by occ-length to allow gNB scheduling with low MCS. Initial proposal 2-5: Agree. Initial proposal 2-6: Inter-slot OCC with PUSCH repetition type A and inter-symbol OCC with PUSCH repetition type B needs the same RV for each repetition unit. This observation is only applicable to the case with non-Release 19 UE with configure grant PUSCH because dynamic grant PUSCH always applies RV cycling. Initial proposal 2-7: We don’t see the necessity of this observation. CFO impact for OCC length 4 with multiplexing 4 UEs and OCC length 2 with multiplexing 2 UEs would be different. Initial proposal 2-8: We don’t see the necessity to limit the OCC length to 2 for inter-slot OCC with PUSCH repetition type A and 4 for inter-slot OCC with PUSCH repetition type B and intra-symbol pre-DFT OCC. Any OCC scheme should support OCC length 2 and 4. |
| Spreadtrum | Initial proposal 2-1: support Initial proposal 2-2: support. In additional PUSCH repetition type B should be scheduled per DCI, such as DCI 0_1 or 0_2, one DCI cannot scheduled both of PUSCH repetition type A and B. In addition, if no DCI format configured as repetition type B, none of CG-PUSCH cannot use repetition type B, either. So besides the restriction of the proposal, there are still some restrictions, too. So PUSCH repetition type A is prioritized over type B. Initial proposal 2-3 and Initial proposal 2-4: not in favor. The resource of TBoMS in each slot is based on PUSCH repetition type A. if symbol-level repetition applied, it is not TBoMS at all. And PUSCH repetition type A does not consider Inter-symbol OCC and intra-symbol OCC, why to extend two OCC schemes to TBoMS. Initial proposal 2-6: Besides OCC code, there are other aspects which also impact back compatibility of legacy UE, such as RV, UCI multiplexing, FH, etc. DG-PUSCH cannot use a fixed RV when PUSCH repetition type A. And if FH is enabled, phase continuity would also be a problem. Initial proposal 2-8: PUSCH repetition type A can be prioritized. |
| LGE | In general, we think that it would be desirable to firstly down-select OCC scheme(s) to be specified and discuss further details on each OCC scheme, which can avoid spending an unnecessary time for the scheme(s) that is(are) not finally supported. [Initial proposal 2-1] Support. [Initial proposal 2-2] If inter-symbol OCC with PUSCH repetition Type B is supported, we support this proposal. [Initial proposal 2-3] We think that this is proposal rather than observation. In our observation, if TBoMS is not applied for intra-symbol or inter-symbol OCC schemes, the performance can be degraded. However, we think that whether to use these OCC schemes with or without TBoMS is still up to network implementation. [Initial proposal 2-4] Refer to the comment on initial proposal 2-3. [Initial proposal 2-5] We can discuss details of OCC schemes after the down-selection of OCC scheme(s). [Initial proposal 2-6] Support. [Initial proposal 2-7] Support. [Initial proposal 2-8] We are not convinced of the benefit of supporting both inter-symbol OCC and intra-symbol pre-DFT-s OCC. Therefore, we propose the following update: The following OCC techniques are at least supported prioritised: - Inter-slot time-domain OCC with - PUSCH repetition Type A with OCC length 2 - PUSCH repetition Type B with 7 symbols per repetition over 2 slots with OCC length 4 - Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 4 |
| Sharp | Initial proposal 2-3: Initial proposal 2-4: TBoMS is useful tool at least for intra-symbol OCC to aid performance gain. However, we don’t need to mandate usage of intra-symbol OCC only with TBoMS. Initial proposal 2-6: UCI piggyback on PUSCH when happened in legacy UEs will break the OCC property. So, we don’t agree with the observation. Initial proposal 2-8: We don’t understand why intra-symbol OCC with length 2 is not prioritized. |
| Qualcomm | Initial proposal 2-1: Not OK for the reasons stated in our paper. Initial proposal 2-2: OK Initial proposal 2-3: Although in general this is true, in some cases (e.g. very low MCS) we may not need TBoMS Initial proposal 2-4: Same comment as 2-3 Initial proposal 2-5: OK Initial proposal 2-6: It is unclear whether this is the case or not, since in several cases legacy UEs will perform RV cycling. The intention of this proposal is unclear. Initial proposal 2-7: It is not obvious to us why this is true. The separation between two symbols that are OCC’d for 2-slot is 1ms, but for 4 mini-slots is actually 1.5ms. Initial proposal 2-8: Could we agree to specify one of the time domain + pre-DFT-s |
| TCL | Initial proposal 2-1: Support Intitial proposal 2-2: Supoort Intitial proposal 2-3 and 2-4: We support the use of TBoMS for OCC but it should not be bound to the shcemes such as intra-symbol OCC. Intitial proposal 2-6：We should discuss this after some issues such as UCI multiplex and RV cycling have been determined. Initial proposal 2-8: Intra-symbol OCC with the length of OCC lenght 2 should also be supported. |
| Ericsson | Initial proposal 2-1: We are open to discuss a limit on the number of PRBs. Initial proposal 2-2: Agree. Initial proposal 2-3: No. Use of TBoMS or not depends on UE capability, NW scheduling, TBS etc. There are cases that do not need TBoMS, eg., for small TBS. Initial proposal 2-4: No. Same reason as for Initial proposal 2-3. Initial proposal 2-5: Ok as an observation. Initial proposal 2-6: Agree. Initial proposal 2-7: Ok if it is clarified that “similar CFO impact” means in terms of phase shift (the performance impact may be different). Initial proposal 2-8: We support to prioritize inter-slot Type A with OCC length 2 and 4, while the other options can be left ffs. |
| Lenovo | Proposal 2-1: Support Proposal 2-2: Agree. Proposal 2-3: We don’t need to mandate TBoMS with Intra-symbol or inter-symbol OCC techniques. It should be left to gNB implementation. Support the proposed amendment by Panasonic. Proposal 2-4: Depend on number of RBs agreed for intra-symbol. We should first select the OCC technique and later discuss this. Proposal 2-5: Fine but first we need to down select the scheme. Proposal 2-6: Agree. Proposal 2-7: Agree. Proposal 2-8: We are not sure why Inter-symbol(s) time domain OCC with OCC length 2 or 4 is not listed as it has the similar performance as of intra-symbol. Based on the table that lists the different companies’ priority, inter-symbol OCC has more support than the intra-symbol. We prefer to have inter-symbol with length 2 or 4 and inter-slot OCC with length 2. |
| Nokia | Proposal 2-1: OK for the gNB to have scheduling freedom. However, since this feature would only make sense in the case of coverage shortage, it would not make sense for the gNB to apply the UL capacity enhancements for more than 2 PRBs (at maximum). For cases of more than 2 PRBs, the gNB could reach orthogonalization between the UEs by simply scheduling UEs on different PRBs (with more power and with “pure” orthogonality). Proposal 2-2: From our viewpoint PUSCH repetition type B does not seem to reflect a valid use case. This type of scheduling is intended for shorter duration (and larger PRB allocations). Additionally, if PUSCH repetition type B is intended for UL capacity enhancements, it would be natural that this is mandatorily supported as well? We prefer that PUSCH repetition type B is not considered for this WID at all. Proposal 2-3: We do not agree with this observation, even that it at first glance does look trivial. If the transport block size is scaled down for TBoMS, the UE throughput is also correspondingly scaled down, which will mean that the network throughput (or capacity) is scaled down as well. Hence, there would be no capacity gains from implementing OCC here. Proposal 2-4: Not support – see comment above for proposal 2-3. Proposal 2-5: Not support. Proposal 2-6: Agree with addition. This statement is only true if a number of conditions for the non-Rel-19 UE are met. These are for instance: No PUCCH on PUSCH for any of the slots, No changes of UE tx power, no changes of RV by the UE. Proposal 2-7: Agree – but two such schemes would experience same performance (where one is relying on additional UE features). Proposal 2-8: Agree in principle, even that the inter-slot methods/alternatives are effectively the same (apart for relying on additional UE feature support). For the intra-symbol pre-DFT-s OCC approach we have concerns related to supported TBS, and the actual gain mechanism is questionable – allocating more PRBs for having sufficiently large TBS will be possible to implement in a similar manner by simply scheduling fewer PRBs per UE. |
| NTT DOCOMO | Initial proposal 2-1: Agree. Initial proposal 2-2: Agree only if inter-symbol OCC with PUSCH repetition Type B is supported. Burt it’s not preferable case for us. Initial proposal 2-3/2-4: Support the use of TBoMS with OCC, but no need to make it mandatory. Initial proposal 2-5: Agree. Initial proposal 2-6: Other aspects may also impact the backward compatibility, such as UCI multiplexing, RV cycling. Initial proposal 2-7: More clarification is needed for necessity of this proposal. Initial proposal 2-8: We support to prioritize inter-slot OCC with PUSCH repetition Type A and intra-symbol pre-DFT OCC. |
| Samsung | Initial proposal 2-1: Agree. It seems conclusion. It’s okay not to capture in Chair Note since this is no spec impact. Initial proposal 2-2: Okay if it is supported. Initial proposal 2-3: It seems not necessary. Down-selection should be done first. Initial proposal 2-8: We prefer to support only one option. Supporting multiple options should be avoided since there is no motivation to do that. |
| Apple | We can consider intra-symbol OCC as the second preference. Initial proposal 2-1: Fine as a conclusion Initial proposal 2-2: We need to add a condition in the proposal that “if inter-symbol OCC is supported” Initial proposal 2-3/2-4: We do not see the necessary linkage between intra-symbol OCC/inter-symbol OCC with TBoMS. Initial proposal 2-5: Not support. We think the resource mapping is changed for intra-symbol OCC. Initial proposal 2-6: Fine Initial proposal 2-7: We do not see the wide usage of PUSCH repetition type B in NTN. Furthermore, for PUSCH repetition type B, the determination of actual repetition is more than the slot boundary separation. Hence, we need to consider more on PUSCH repetition type B with OCC. Initial proposal 2-8: the sub-bullet on PUSCH repetition type B should be further discussed. |
| Xiaomi | Initial proposal 2-1: Agree. Initial proposal 2-2: Down-selection between the three or four OCC schemes should be performed at first before starting the detailed design of inter-symbol occ. Initial proposal 2-3: Agree. Initial proposal 2-4: Agree. Initial proposal 2-5: Agree. Initial proposal 2-6: Agree, it is up to gNB scheduling. Initial proposal 2-7: Further simulation is necessary if RAN 1 want to reach on such a conclusion or observation. Initial proposal 2-8: It seems that nothing is prioritized compared with the agreement achieved in the last RAN1 meeting. Only the combination scheme is precluded and inter-symbol OCC based on repetition type B is further designed in this proposal. One more question is, does it mean that all the solutions will be specified within this proposal? Anyway, we support to specify only one OCC scheme due to the limited TU and the heavy workload. Furthermore, inter-slot OCC is preferred by us. |
| Vivo1 | [Initial proposal 2-1] Support. [Initial proposal 2-2] Support the proposal only if inter-symbol OCC scheme is supported. [Initial proposal 2-3] OK with the observation. [Initial proposal 2-4] If same code rate as well as same TBS should be guaranteed for inter-slot OCC and inter-symbol OCC/intra-symbol OCC, this proposal should be supported [Initial proposal 2-5] We think it is an observation rather than a proposal. [Initial proposal 2-6] Support. [Initial proposal 2-7] Motivation of having this observation is unclear to us. It seems this observation can be true under some certain assumptions. The note is also ambiguous, does ‘have same meaning’ mean that Inter-symbol OCC with PUSCH repetition Type B and Inter-slot OCC with PUSCH repetition Type B have same resource mapping/pattern after applying OCC? [Initial proposal 2-8 It's unclear whether the prioritization is intended for study purposes or for actual support. |
| CMCC | Initial proposal 2-1: no need to limit the scheduled PRB numbers. But for some candidate solutions, there is no need to introduce multiple PRBs’ transmission. If the 2 UE intra-symbol OCC is introduced with 4 PRB scheduling, the gNB can schedule 2 PRBs for each UE. The performance is the same. We do not need to solve this more than 1 PRB issue at this stage. Initial proposal 2-3: We are fine with the observation. Initial proposal 2-4: There is no need to combine two different feature together. TBOMS is supported in the Rel-17 CE. It could be a separated capability. If the intra-symbol or inter-symbol OCC is supported, it still can depends on UE’s capability to decide whether TBOMS is supported or not. If the TBOMS is not supported, the UE has to be scheduled with less data rate. Initial proposal 2-5: Partially agree. The existing time-domain resource mapping of intra-symbol pre-DFT OCC can only reuse the existing RE mapping of PUCCH, but not that of PUSCH. Initial proposal 2-6: Support the observation. |
| ZTE | Initial proposal 2-1: Fine with the proposal. Initial proposal 2-2:Fine with the proposal. Initial proposal 2-3 and 2-4: The observation is not clear. TBoMS is an optional feature up to gNB configuration, maybe we can say “The performance of Intra-symbol and inter-symbol OCC significantly degrades without TBoMS.” Initial proposal 2-5: Can be OK as an observation. Initial proposal 2-6: OK Initial proposal 2-7: OK with the observation. Regarding the terminology, we prefer to separately define inter-symbol OCC with repetition type B instead of calling it as inter slot OCC, because this is actually a compromised solution, our 1st preference is inter-slot OCC with repetition type A only. Initial proposal 2-8: We think inter-slot OCC with repetition type A should be prioritized. |
| Huawei, HiSilicon | proposal 2-1: agree proposal 2-2: agree proposal 2-3: To our understanding, this proposal applies to intra symbol and inter symbol OCC (without repetition type B). The inter-symbol OCC with PUSCH repetition type B do not need to change the TBS determination. For PUSCH repetition type B, TBoMS is not necessary. UE can split the big TB into two TB and fit into individual repetition. proposal 2-4: Similar as P2-3, it should be clarified the inter symbol OCC is without PUSCH repetition type B. in addition, we observed that without TBoMS, there are significant performance degradation for intra symbol OCC and inter symbol OCC w/o PUSCH repetition type B. Intra-symbol OCC and inter-symbol OCC (symbol-level repetition) should be used together with TBoMS, if Intra-symbol OCC or inter-symbol OCC (symbol-level repetition) is supported.. proposal 2-5: Not support. the resource mapping should be changed because of pre DFT spreading. proposal 2-6: Agree. A typo “Inter-symbol OCC with PUSCH repetition B” proposal 2-7: we suggest to capture the observation as following Inter-symbol OCC with PUSCH repetition Type B with 7 OFDM symbols per repetition over 2 slots with OCC factor 4 has similar BLER performance as intra symbol OCC with OCC factor 4 assuming same spectrum efficiency. proposal 2-8: we do not think it is necessary to restrict the applicable OCC length for different schemes. As we commented 2-4, it is still possible to mux 4 UE with inter slot OCC with rep type A. In addition, all candidates are still on the table, there seems no prioritization. We prefer to prefer inter slot OCC with repetition type A |
| CATT | Initial proposal 2-1: OK Initial proposal 2-2: not support inter-symbol solution. Initial proposal 2-3: it is not relevant with TBoMS. Initial proposal 2-4: not support TBoMS. Initial proposal 2-5: Not support intra-symbol OCC since it will change the TBS mapping for different PRB allocations. Secondly it will be linked to TBoMS, so it will make OCC scheme complicated. Initial proposal 2-6: Ok. Initial proposal 2-7: the observation is not aligned with simulation result. No need to limit inter-slot OCC length equal to 2. Initial proposal 2-8: inter-slot OCC is prioritized. |
| IDC | Initial proposal 2-1: Yes – we don’t need to specify OCC based upon PRB number. Initial proposal 2-3: Support. Initial proposal 2-4: Support. Initial proposal 2-5: Not support. Initial proposal 2-8: Our first preference is Inter-slot. Then among the symbol level, we have slight preference for inter-symbol. |

## 2.3 Summary of First Round Discussion

Based on first round discussions on mapping of OCC schemes, proposals were made for first offline discussions in Section 4.1. Revised proposals were discussed in the first online session in Section 4.2. The inter-symbol OCC (symbol-level repetition without PUSCH repetition Type B) and inter-slot OCC with PUSCH repetition B will not be pursued based on online session agreement copied below.

Agreement

At least one of the OCC techniques when PUSCH repetitions are used will be specified:

- Inter-slot time-domain OCC with OCC length 2

- Inter-slot time-domain OCC with OCC length 2 and 4

- Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2

- Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2 and 4

- Note: combination of techniques is not precluded

- PUSCH repetition Type B is not considered

## 2.4 Second Round Discussion

Based on first online session agreement, we revised first round proposals 2.3 and 2.6 as 2.3a and 2.6a.

Initial proposal 2-6a:

Observation

Inter-slot time-domain OCC with PUSCH repetition A can be used with CG-PUSCH and no PUCCH on PUSCH for any of the slots assigned the code [+1, +1] or [+1, +1, +1, +1] for OCC length 2 or 4 respectively for the following cases

- 1 non-Release 19 UE

- 1 Release-19 UE without OCC with PUSCH capability

Initial proposal 2-3a:

Observation

If supported, Intra-symbol OCC should be used with TBoMS assuming available RE per slot (NRE) for TB size determination are scaled down by occ-length to allow gNB scheduling with very low MCS.

Note:

- Since inter-slot OCC with PUSCH repetition Type B is not considered, further discussions on proposal 2-2a and 2.5 in first online session in Section 4.2 will not be necessary.

Companies are encouraged to provide further comments in the comment table below as needed:

| Companies | Comments |
| --- | --- |
| Ericsson | Initial proposal 2-6a: Support with the following comments: - The term “no PUCCH on PUSCH” is not clear to us. Should it be “no UCI on PUSCH”? - The sub-bullets are not needed, can be simplified to just say “UE without PUSCH OCC capability” (the UE release does not matter). Initial proposal 2-3a: The use of TBoMS depends on TB size, UE capability and other factors. We do not think this observation is needed. The use of TBoMS can be discussed in the next meeting. |

## 2.5 Conclusion

The following agreement was made in first online session

Agreement

At least one of the OCC techniques when PUSCH repetitions are used will be specified:

- Inter-slot time-domain OCC with OCC length 2

- Inter-slot time-domain OCC with OCC length 2 and 4

- Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2

- Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2 and 4

- Note: combination of techniques is not precluded

- PUSCH repetition Type B is not considered

This agreement means that inter-symbol OCC (symbol-level repetition) and OCC schemes with PUSCH repetition B will not be pursued in Release 18.

In second offline, backward compatibility for inter-slot OCC was discussed. It was understood that at least a UE without PUSCH OCC with repetitions Type A can be multiplexed with Rel-19 UE with inter-slot PUSCH OCC if at least the following conditions are met:

•	CG-PUSCH with same RV

•	No UCI on PUSCH for any of the slots

•	No frequency hopping

There was not enough consensus to propose to capture these conditions as an observation. Several companies discussed other conditions which would require more discussions to align understanding of contributing companies before these other conditions can be captured as observation.

FL recommendation 2-1:

Companies are encouraged to contribute in next meeting in RAN1#118bis on conditions that must be met to multiplex a UE without inter-slot OCC with PUSCH repetitions Type A with a Rel-19 UE with inter-slot PUSCH OCC with PUSCH repetitions Type A. At least the following conditions should be met based on second offline discussion:

- CG-PUSCH with same RV

- No UCI on PUSCH for any of the slots

- No frequency hopping

# [CLOSED] Signalling aspects of PUSCH with OCC

## 3.1 Company contributing views

| Contribution | Observation/Proposals |
| --- | --- |
| Ericsson [2] | TBS calculation / Rate matching Observation 5: Intra-symbol OCC requires updates of the TBS/rate matching design while inter-slot OCC (based on PUSCH repetitions Type A) or inter-symbols OCC (based on PUSCH repetitions Type B) do not. PDCCH Capacity for OCC: Observation 19: The capacity of PDCCH is a bottleneck limiting the possibility to grant PUSCH resources for OCC-based transmissions with high OCC multiplexing levels. Observation 20: If OCC multiplexing level 8 were intended to be supported, it would become essential studying DCI enhancements to alleviate the downlink overhead. Even OCC multiplexing level 4 results in a non negligible downlink overhead impacting the resource availability for other physical channels e.g., PDSCH. Proposal 5: RAN1 should study DCI enhancements to alleviate the downlink overhead when scheduling an OCC-based PUSCH transmission, especially if OCC multiplexing level 8 were intended to be supported. |
| Huawei [5] | TBS calculation / Rate matching Observation 5: TBS determination procedure in clause 6.1.4.2 of TS38.214 can be reused for inter-slot time domain OCC with repetition type A and inter-symbol time domain OCC with repetition type B. The number of available RE per slot (NRE) for TB size determination should be scaled down by occ-length for intra-symbol pre-DFT-s OCC. UCI multiplexing Observation 6: - For inter-slot time domain OCC with PUSCH repetition type A and inter-symbol time domain OCC with repetition type B, UCI should be multiplexed on every slot/repetition of the time span of an OCC sequence. - For intra-symbol pre-DFT-s OCC, existing RE mapping for UCI multiplexing can be reused. Observation 7: When multiplexing UCI on PUSCH, the calculation of the number of symbols for HARQ-ACK transmission, CSI part 1 transmission and CSI part 2 transmission can be reused for inter-slot time domain OCC with repetition type A and inter-symbol time-domain OCC with PUSCH repetition type B. Whereas the number of REs that can be used for transmission of UCI should be scaled by occ-length for intra-symbol pre-DFT-s OCC. RV cycling across repetitions Observation 4: RV cycling period should be aligned with the span of an OCC sequence for inter-slot time domain OCC with PUSCH repetition type A and inter-symbol time-domain OCC with PUSCH repetition type B. Legacy RV cycling can be reused for intra-symbol OCC. Frequency hopping, e.g. intra /inter slot Observation 8: - For inter-slot time-domain OCC with PUSCH repetition type A and inter-symbol time-domain OCC with PUSCH repetition type B, the time interval of inter-slot frequency hopping should be extended to a number of slots aligned with the time span of an OCC sequence. Intra-slot frequency hopping and inter-repetition frequency hopping should be avoided for inter-slot time-domain OCC with PUSCH repetition type A and inter-symbol time-domain OCC with PUSCH repetition type B, respectively. - For intra symbol pre-DFT-s OCC, both intra-slot and inter-slot frequency hopping can be used without change. Power control Observation 9: For inter-slot time-domain OCC with PUSCH repetition type A and inter-symbol time-domain OCC with PUSCH repetition type B, the existing BPRE of the PUSCH power control can be reused. For intra-symbol OCC, the number of resource elements calculating BPRE for PUSCH power control should be the total resource elements after OCC spreading. OCC indication/configuration Proposal 1: For PUSCH transmission corresponding to the configured grant type 1, OCC-related parameters, such as occ-Index and occ-Length, can be configured within configuredGrantConfig. Proposal 2: For PUSCH transmission corresponding to the configured grant type 2 or dynamic granted based PUSCH, OCC-related parameters, such as occ-Index can be included in the activation/scheduling DCI, and occ-Length can be configured within configuredGrantConfig/pusch-config. OCC sequences: Proposal 4: For OCC length 2, support to define the OCC sequences from the rows of matrix W2=111-1. Proposal 5: The choice of OCC sequence of length 4 should take the following perspectives into account: - Flexibility to multiplex UE with different OCC lengths; - Orthogonality under residual CFO. UE Multiplexing: Proposal 3: Only support multiplexing 2 or 4 UEs without combination of OCC schemes in Rel-19 NR-NTN. Observation 10: It is beneficial from system capacity perspective to support multiplexing of Rel-19 PUSCH repetitions with OCC on the time/frequency resource allocated to non-Rel-19 PUSCH repetitions (i.e. backward compatibility with non-Rel-19 UE). For inter-slot OCC with PUSCH repetition type A and inter-symbol OCC with PUSCH repetition type B, it is possible to multiplex Rel-19 PUSCH with OCC on non-Rel-19 Configured Grant PUSCH with repetitions if the PUSCH repetitions are aligned and RV sequence is configured as all 0. |
| ZTE [4] | UCI multiplexing Proposal 3: If UCI is multiplexed on one of repetitions, OCC is not applied on these repetitions. OCC indication/configuration Proposal 4: For the UL capacity enhancement, the OCC sequences defined in Table 6.3.2.5A-1 and Table 6.3.2.5A-2 can be reused. Proposal 5: The dynamic and semi-static indication of the OCC sequence index can be supported, e.g., different per type of PUSCH. Proposal 6: The indication of the OCC sequence index can be per UE or per UE group. |
| Nokia [6] | TBS calculation / Rate matching Observation 4: No adaptation of the TBS determination or rate matching procedure is necessary in the case of OCC applied to PUSCH repetitions Type A. Proposal 2: In case intra-symbol OCC is supported, RAN1 to consider scaling the parameters related to rate matching by the applied OCC size. UCI multiplexing Observation 5: based on current specification, too many resources will be occupied by a UCI if the UCI is multiplexed on the PUSCH repetitions with OCC enabled. Meanwhile, scaling down the resource occupied by the UCI by OCC length will be too aggressive and impact the reliability of UCI transmission. Proposal 3: RAN1 to study the possible solution to resolve the problem that too many resources are occupied by a UCI when it is multiplexed on PUSCH repetitions with OCC enabled. Proposal 4: RAN1 to study and potentially modify the procedures related to UCI multiplexing on PUSCH, with focus on how to integrate the UCI multiplexing into the inter-slot OCC scheme. Observation 6: based on current specification, there will be more than one CSI reference resource for a CSI report if the CSI report is multiplexed on PUSCH repetitions with OCC enabled, which is not as expected. Proposal 5: RAN1 to study possible solutions to avoid multiple CSI reference resources issue when a CSI report is multiplexed on PUSCH repetitions with OCC enabled. RV cycling across repetitions Proposal 6: RAN1 to introduce RV grouping to allow application of OCC across PUSCH repetitions. Frequency hopping, e.g. intra /inter slot Observation 7: In the case of inter-symbol and inter-slot (i.e. PUSCH repetitions Type A) OCC, frequency hopping might impair applicability of OCC. Proposal 7: Discuss extension of the inter-slot frequency hopping with inter-slot bundling for PUSCH feature in case of PUSCH repetitions Type A with OCC. OCC indication/configuration Observation 8: Signaling and indication of both UE’s capability and OCC configuration and indication need to be further discussed in RAN1. Proposal 10: RAN1 to define signalling mechanisms for indication of OCC parameters based on either introducing an additional field in the DCI scheduling the PUSCH or based on repurposing some of the existing bits of the DCI scheduling the PUSCH. Proposal 11: Dynamic DCI signalling might feature an index that specifies to the UE the position of OCC application in relation to the first PUSCH transmission. Power control Proposal 14: In case intra-slot OCC is supported, RAN1 to adapt the power control determination when OCC is enabled to consider the size of the applied OCC. Alignment of OCC codes Observation 9: It is crucial that the codes used for OCC are aligned to maintain orthogonality between the UEs using the same physical resources. Proposal 12: RAN1 to introduce OCC timing to align the time (i.e. repetitions) of OCC application among UEs scheduled on the same physical resources. |
| Qualcomm [9] | OCC indication/configuration Proposal 5: For DG-PUSCH: - The OCC feature is enabled/disabled by RRC. - FFS: Details (e.g. parameters to be configured) - The OCC codeword is indicated dynamically in DCI. - FFS: Details (e.g. whether explicit or implicit) Proposal 6: The indication of DMRS port, OCC factor and OCC CW index are jointly encoded in DCI. Proposal 7: Network may enable/disable OCC implicitly based on values of certain fields that meet certain conditions, e.g. OCC is only applied if the following conditions are met: - FDRA = 1 PRB. - Modulation order Q_m≤2 - UL-SCH flag = 1 Proposal 8: Other aspects (rate matching / TB calculation / FH / etc.) are postponed until RAN1 concludes on the specific OCC scheme(s) to be specified. |
| Apple [8] | OCC Sequences: Proposal 1: RAN1 is to use Walsh codes to generate OCC sequence. TBS calculation / Rate matching Observation 1: The inter-slot time-domain OCC with PUSCH repetition Type A does not have specification impact on TBS calculation and resource mapping but has specification impact on UCI multiplexing and RV cycling across repetitions. Observation 2: The inter-symbol(s) time-domain OCC has specification impact on TBS calculation, resource mapping, but does not have specification impact on UCI multiplexing and RV cycling across repetitions. Observation 3: The intra-symbol pre-DFT-s OCC has specification impact on TBS calculation, resource mapping, but does not have specification impact on UCI multiplexing and RV cycling across repetitions. RV cycling across repetitions Proposal 2: If inter-slot time-domain OCC with PUSCH repetition Type A is used, consider applying RV cycling over every X consecutive PUSCH repetitions, where X is OCC length. Proposal 3: If inter-slot time-domain OCC with PUSCH repetition Type A is used, consider the following options for UCI multiplexing on PUSCH: - Option 1: UCI multiplexing on PUSCH is disabled (FFS: prioritization rule) - Option 2: Same UCI is multiplexed on all PUSCH repetitions in an OCC period (FFS: timeline consideration, dropping rule, whether/how to multiplex UCIs over different slots with PUSCH) OCC indication/configuration Proposal 5: For PUSCH enhancement via OCC spreading, RAN1 to consider dynamic grant PUSCH, type 1 configured grant PUSCH and type 2 configured grant PUSCH. Proposal 6: The contents of OCC related information include OCC length and OCC sequence index. RAN1 to study the signaling to support PUSCH with OCC spreading. |
| CATT [12] | TBS calculation / Rate matching Observation 1: The change of TBS calculation is needed when intra- symbol OCC is used. UCI multiplexing Observation 2: For inter-slot OCC, the multiplexing needs to be applied to all repetition slots using the same OCC sequences when UE takes the multiplexing of PUCCH and PUSCH in a slot. Observation 3: For inter-symbol OCC and intra-symbol OCC, the enhancement for UCI multiplexing is needed. RV cycling across repetitions Observation 4: For inter-slot OCC and inter-symbol OCC, further simulation is needed to determine whether RV cycling across repetitions is used for dynamic transmission. Frequency hopping, e.g. intra /inter slot Observation 5: It is not recommended to use frequency hopping in NTN. OCC indication/configuration Observation 6: For OCC indication/configuration, at least RRC configuration is supported. Observation 7: For intra-symbol OCC, it needs to be clarified whether the resources indicated by the Resource Allocation field of the DCI are referred to before or after the OCC. |
| MediaTek [11] | TBS calculation / Rate matching Observation 8: Inter-slot OCC does not require enhancements for TBS determination, power control, and multiplexing with non-R19 UEs. UCI multiplexing RV cycling across repetitions Frequency hopping, e.g. intra /inter slot OCC indication/configuration |
| OPPO [7] | Proposal 5: For selected OCC schemes, the corresponding spec impacts in Table 1 should be considered. UCI multiplexing Observation 4: For inter-slot OCC with PUSCH repetition Type A, multiplexing UCI on PUSCH may extend the delay of UCI reception due to the UCI multiplexing timeline and multiplexing slots. Observation 5: For inter-symbol OCC with PUSCH mapping Type A, the RE number for TBS determination for UCI and data would be much less, and it is hard to multiplexing UCI on PUSCH with OCC using legacy mapping rule. Observation 6: For intra-symbol pre-DFT-s OCC, the RE number for TBS determination for UCI and data would be much less. Observation 7: For UCI on PUSCH with OCC in TBoMS case, the issue of extension of the delay of UCI reception would be much more serious. Observation 8: Option 2 can achieve a trade-off between UCI performance and uplink capacity/throughput. OCC indication/configuration Proposal 2: The following information should be provided to UE for PUSCH with OCC: - Enable/disable of PUSCH with OCC - OCC scheme - OCC table and OCC length - OCC index and/or the association between OCC index and DMRS port OCC sequences: Proposal 3: Reuse legacy table for OCC sequence with length 2 or 4. |
| Spreadtrum [14] | TBS calculation / Rate matching Observation 1: For inter-slot time-domain OCC with PUSCH repetition type A, there is no change on TBS calculation/Rate matching. RV cycling across repetitions Proposal 9: For inter-slot time-domain OCC with PUSCH repetition Type A, repetitions in a PUSCH OCC group use same RV. - RV cycling is across different PUSCH OCC groups - RV in TBoMS can be used as a baseline, such as ((n-(n mod M))/M) mod 4 determines the RV ID, where M is OCC length UCI multiplexing Proposal 7: Only when PUCCH overlaps with PUSCH in the first repetition of a PUSCH OCC group, the UCI of PUCCH can be multiplexed on PUSCH, otherwise the UCI on PUCCH is dropped. - UCI of PUCCH multiplex on all repetitions of the PUSCH OCC group Proposal 8: When a DCI triggers A-CSI report on PUSCH, the A-CSI multiplex on all repetitions of the first PUSCH OCC group. . Frequency hopping, e.g. intra /inter slot Proposal 10: For inter-slot time-domain OCC with PUSCH repetition Type A, frequency only hops across different PUSCH OCC group, and there is no hop among repetitions in one PUSCH OCC group. - The RB start index in inter-slot frequency hopping with DMRS bundling can be used as baseline. OCC indication/configuration Proposal 6: For discussion purpose, PUSCH OCC group can be defined, which is PUSCH repetitions covered by one OCC sequence. Proposal 11: For PUSCH transmission with DCI dynamic scheduling, the following two OCC determination methods can be considered. - Option 1: The OCC used for PUSCH transmission is implicitly determined by legacy parameter. - Option 2: The OCC used for PUSCH transmission is explicitly indicated by the scheduling DCI. Proposal 12: For configured grant type 1, the OCC used for PUSCH transmission can be configured through RRC. Proposal 13: For configured grant type 2, the following two OCC determination methods can be considered. - Option 1: The OCC used for PUSCH transmission is implicitly determined by legacy parameter. - Option 2: The OCC used for PUSCH transmission is explicitly indicated by the scheduling DCI. Proposal 14: Dynamic UL grant for UE group can be considered for uplink capacity and throughput enhancement. Observation 2: UE grouping for OCC based PUSCH transmission in RRC connected state can be implemented based on the network. OCC Sequences: Proposal 4: For OCC length 2, OCC sequences are +1+1 and +1-1. Proposal 5: For OCC length 4, RAN1 needs to do down selection between Walsh sequence and DFT sequence. |
| Vivo [27] | RV Cycling Proposal 1. RV for time resources applying OCC should be the same. OCC indication/configuration Proposal 3. OCC scheme can be enabled for dynamic grant scheduled PUSCH and configured uplink grant. Proposal 4. OCC scheme is configured by RRC for at least configured uplink grant type1. Proposal 5. OCC scheme is can be indicated by DCI for dynamic grant scheduled PUSCH. Proposal 6. OCC scheme can be enabled for all PUSCH repetitions, or partial PUSCH repetitions in a PUSCH transmission, which is up to NW configuration/indication. Proposal 7. Clarify the actual number of repetition resources of a PUSCH for the value indicated by numberOfRepetitions or numberOfRepetitionsExt when OCC is enabled together. |
| Samsung [10] | Proposal 1: Conclude that DFT-s-OFDM PUSCH enhancements via OCC is not targeted to Msg3 PUSCH. |
| NEC [21] | Observation 1: Some potential issues related to multiplex UCI with PUSCH need to be considered under the uplink capacity/cell throughput enhancement study : - When is the UCI ready and suitable to be multiplexed with PUSCH? - How to multiplex UCI with PUSCH under inter-slot OCC, inter-symbol(s) OCC, and intra-symbol OCC? - Which repetition(s) of the PUSCH should be multiplexed with UCI? - How to handle the priority differences between PUSCH, HARQ-ACK, CSI Part 1, and CSI Part 2? 5. How to balance the performance of UL-SCH and UCIs in limited uplink resources? |
| Panasonic [16] | Proposal 4: Because OCC is used together with repetition and OCC length is closely related with repetition factor, similar way as signaling of repetition should be considered for signaling of OCC length. Proposal 7: If inter-slot OCC or inter-symbol OCC is supported, UE behavior for non-contiguous PUSCH transmission with OCC due to e.g. collision with PRACH resource and SRS transmission should be discussed. Proposal 8: Combination of DMRS bundling and OCC should be supported. Relation between OCC and TDW should be discussed. UCI multiplexing Observation 1: For inter-slot time domain OCC, issues on UCI multiplexing and non-contiguous PUSCH transmission due to collision e.g. with PRACH and SRS, need to be discussed. Proposal 6: If inter-slot OCC is supported, UE behavior of UCI and PUSCH collision case to avoid loss of OCC orthogonality should be discussed, e.g. drop all UCI in an OCC duration or the same UCI multiplexing in all slots in an OCC duration. OCC indication/configuration Proposal 5: dynamic indication via DCI for OCC sequence and DMRS port should be supported. |
| DoCoMo [17] | UCI multiplexing Proposal 6: For UCI multiplexing of PUSCH with inter-slot time-domain OCC, UCI is multiplexed on the PUSCH in an overlapped slot and on other PUSCHs that are associated with the same set of OCC code as the overlapped slot. Proposal 8: For PUSCH with intra-symbol pre-DFT OCC, in TBS determination and UCI multiplexing, NSCPUSCH is replaced by NSCPUSCH/M, where M is OCC length. - Data and (if any) UCI are mapped to the first set of (NSCPUSCH/M) subcarriers, and then block-wise spreading is performed for the remaining set(s). OCC Sequences: Proposal 9: Legacy sequences (i.e., Walsh sequences and DFT sequence as defined in TS38.211) are used as baseline for generating OCC for PUSCH. OCC indication/configuration Proposal 10: Study whether the indication of OCC related parameters, e.g., OCC length, OCC index, is performed in a semi-static manner or in a dynamic manner. |
| CMCC [18] | RV cycling Proposal 1: The enhancement to the redundancy version cycling of the PUSCH repetition should be considered, when the slot-level PUSCH OCC multiplexing is adopted. |
| China Telecom [19] | UCI multiplexing: Proposal 4: RAN1 to discuss and modify the potential enhancements for the UCI multiplexing when using the inter-slot time domain OCC. At least the following two solutions can be considered: - Repeat UCI messages in all slots of an OCC period. - Indicate the UCI transmission through higher layer signalling and pre-reserving UCI resources in other slots. RV cycling: Proposal 5: Support new RV cycling scheme for inter-slot time domain OCC, and fixed RV for repetitions can be prioritised. OCC indication/configuration Proposal 3: OCC related parameters can be configured/indicated by RRC or DCI. |
| Lenovo [15] | OCC indication/configuration Proposal 1: OCC schemes should be studied by considering legacy dynamic grant and configured grant-based resource allocation procedures as baseline. |
| Sharp [29] | Power Control: Proposal 4: For power control, the total number of resource elements for a PUSCH in a slot NRE should be scaled by the OCC length for intra-symbol pre-DFT OCC when TBS scaling is applied. Proposal 6: Study whether Rel-17/18 DMRS bundling solves the cross-slot power control issue for inter-slot OCC. TBS calculation / Rate matching Observation 2: TBS scaling is not necessary for inter-symbol OCC. RV Cycling Observation 1: Incremental redundancy gain will be reduced by intra-symbol OCC with the legacy RV cycling compared to TBoMS mapping. Proposal 2: Study the following options for intra-symbol pre-DFT OCC. - Scale down the TBS based on the OCC length - Change the starting position in the virtual circular buffer for each PUSCH repetition (like TBoMS) Proposal 7: Support new RV cycling schemes on a cross-slot basis for inter-slot OCC. UCI multiplexing Observation 3: A new UCI multiplexing timeline needs to be introduced for inter-slot OCC. Proposal 3: For UCI multiplexing, the total number of resource elements for a PUSCH in a slot l=0Nsymb,allPUSCH-1MscUCIl should be scaled by the OCC length for intra-symbol pre-DFT OCC when TBS scaling is applied. Proposal 5: Further study the impact of UCI multiplexing which leads to a risk of not transmitting systematic bits for intra-symbol pre-DFT OCC when TBoMS-like rate matching is performed. |
| InterDigital [22] | UCI multiplexing Observation 2: UCI multiplexing to OCC based PUSCH transmissions will require additional timing restrictions on the UCI availability. Observation 3: Re-defining the UCI multiplexing rules for OCC based PUSCH transmissions may involve significant complexity and specification effort. Proposal 6: UCI is transmitted via higher layer signalling when UCI overlaps with an OCC based PUSCH transmission. OCC indication/configuration Proposal 1: Uplink capacity enhancements are supported for both dynamic grant and configured grant types of uplink transmissions. Proposal 2: The signaling for OCC can be dynamic and/or semi-static. The use of dynamic vs semi-static can be based upon the type of uplink PUSCH transmission. Others Proposal 5: Support UE based de-activation of HARQ feedback for OCC based PUSCH transmissions. |
| Fujitsu [30] | TBS calculation / Rate matching Observation 2: For intra-symbol OCC, RAN1 need to specify comb-like structure for PUSCH transmission procedure, which may impact the TBS calculation. OCC indication/configuration Proposal 3: The largest value of supported OCC length is configured by cell specific signalling (e.g. SIB19) or UE specific signalling (e.g. RRC), and other OCC length smaller than that is implicitly indicated. Proposal 4: OCC sequence index should be associate with DMRS antenna port one by one to reduce signalling overhead. Proposal 5: For OCC indication with inter-slot OCC, OCC application can control without UE specific signal when UE interpret that OCC length is less than the actual number of PUSCH repetitions. Frequency hopping: Proposal 7: For inter-slot OCC, RAN1 needs to discuss whether PUSCH transmission with frequency hopping is restricted in subcarriers close to DC subcarrier to avoid the performance degradation by timing drift and doppler shift. |
| ETRI [26] | Observation 1. On the (maximum) number of RBs to apply OCC for PUSCH in NTN, the network has full responsibility to choose a proper frequency resource allocation and OCC indication/configuration, and some of possible OCC indication methods (e.g., DCI-based OCC indication) may be able to implicitly cover such tight correlation between OCC indication/configuration and frequency resource allocation. Proposal 1. RAN1 to study following details on OCC sequence configuration/indication: - OCC sequence type (e.g., DFT or Walsh): RAN1 to support at least Walsh code. - OCC sequence granularity (time resource size for 1 OCC element): A sequence value spreads over OFDM symbols for a PUSCH transmission. - RAN1 to support time-domain OCC with PUSCH repetition type B, in addition to the agreed PUSCH repetition type A. - RAN1 to support time-domain OCC with TBoMS. - OCC configuration: It is preferred to configure OCC sequence type (if multiple sequence types are supported), OCC sequence length, and a default value of OCC sequence index via SIB or RRC. - OCC indication: It is preferred to indicate/update OCC sequence index via MAC CE or DCI. - OCC indication may include implicit/explicit OCC ON-OFF indication. |
| NICT [24] | UCI multiplexing Observation 2: Since PUSCH with UCI disrupts OCC orthogonality, it is important to discuss how to transmit PUSCH repetitions using OCC when UCI occurs. Proposal 1: The following methods can be taken to maintain OCC orthogonality when UCI occurs: - Transmit PUSCH with UCI in all slots - Skip UCI transmission when UCI occurs in the middle of an OCC transmission - Allow simultaneous transmission of PUSCH and PUCCH |
| TCL [26] | UCI multiplexing: Observation 1: - For inter-slot OCC, the UCI block-wise spread with OCC after multiplexing or the repetition of the UCI or PUCCH should be supported and the UCI multiplexing can be postponed to the next OCC period when the overlap does not occur in the first slot of the OCC period. - For inter-symbol(s) OCC, the bits for UCI occupied after multiplexing should be limited to improve the transmission efficiency of PUSCH. OCC indication/configuration Proposal 9: A group-common DCI to schedule the transmission of multiple UEs with OCC on the same resources can be studied to improve the overhead of signaling. Proposal 1: The OCC for configured grant Type 1 PUSCH can be accomplished through RRC. Proposal 2: Special fields of the CS-RNTI scrambled DCI can be utilized for explicit indication of OCC for configured grant Type 2 PUSCH. Proposal 4: For the PUSCH with OCC, the RV cycling period can be modified according to the length of the OCC sequence. OCC Sequences: Proposal 8: Both Walsh sequences and DFT sequences can be considered for PUSCH with OCC. |
| LG [23] | UCI multiplexing Proposal #5: Support new UCI multiplexing rules for PUSCH with OCC including the following design principles: - UCI RE mapping before OCC - UCI repetition/expansion under OCC - Timeline requirements based on units to which OCC applies RV cycling: Proposal #6: For inter-slot time domain OCC, support RV cycling including the follows: - Fixed RV - RV cycling per resource unit to which OCC applies OCC indication/configuration Proposal #3: If inter-symbol(s) OCC is supported in Rel-19 NR NTN, support TBS determination rule that scale down the initial TBS by the OCC length. Proposal #4: If inter-symbol(s) OCC is supported in Rel-19 NR NTN, study the following issues rate-matching. - Whether/how to support multiple DM-RS configuration(s) - Whether/how to support PT-RS configuration Proposal #7: Support OCC indication/configuration for PUSCH with OCC as follows: - OCC parameters include at least OCC index and OCC length. - For Type 1 CG PUSCH, OCC parameters are configured by RRC signaling. - For Type 2 CG PUSCH and DG PUSCH, OCC parameters are indicated/configured by DCI and RRC signaling. Proposal #8: Study OCC indication/configuration to mitigate UL interference among cell(s) and/or satellite beam(s). Proposal #10: Study UE grouping for OCC based PUSCH transmission to prevent near-far problem. |

In RAN1#116bis, the following agreement was made

RAN1 to at least further study the potential specification aspects on OCC techniques:

- TBS calculation / Rate matching

- UCI multiplexing

- RV cycling across repetitions

- Frequency hopping, e.g. intra /inter slot

- OCC indication/configuration

- Power control

- FFS others aspects

Spreadtrum [14] provided a useful diagram for PUSCH scheduling in NR

Spreadtrum [14] Figure: PUSCH scheduling in NR

TBS calculation / Rate matching:

…….

The UE shall first determine the number of REs (NRE) within the slot:

-	A UE first determines the number of REs allocated for PUSCH within a PRB  by

-	, where is the number of subcarriers in the frequency domain in a physical resource block,  NsymbslotNsymbslotis the number of symbols L of the PUSCH allocation according to Clause 6.1.2.1 for scheduled PUSCH or Clause 6.1.2.3 for configured PUSCH,  is the number of REs for DM-RS per PRB in the allocated duration including the overhead of the DM-RS CDM groups without data, as described for PUSCH with a configured grant in Clause 6.1.2.3 or as indicated by DCI format 0_1, 0_2 or 0_3 or as described for DCI format 0_0 in Clause 6.2.2, and  is the overhead configured by higher layer parameter xOverhead in PUSCH-ServingCellConfig. If the  is not configured (a value from 6, 12, or 18), the  is assumed to be 0. For Msg3 or MsgA PUSCH transmission the  is always set to 0. In case of PUSCH repetition Type B,  is determined assuming a nominal repetition with the duration of L symbols without segmentation.

-	A UE determines the total number of REs allocated for PUSCH  as follows

-	For TB processing over multiple slots, NRE=N*min156,NRE'⋅nPRB where  is the total number of allocated PRBs for the UE and N is the number of slots used for TBS determination indicated by numberOfSlotsTBoMS.

-	Otherwise, NRE=min156,NRE'⋅nPRB.

-	Next, proceed with steps 2-4 as defined in Clause 5.1.3.2

-	For a PUSCH scheduled by fallbackRAR UL grant, UE assumes the TB size determined by the UL grant in the fallbackRAR shall be the same as the TB size used in the corresponding MsgA PUSCH transmission.

……

The determination of TBS is given in clause 6.1.4.2 of TS38.214 and is shown as the following:

Ericsson [2] provided some analysis on the legacy rules for TBS calculation which use equation Ninfo= NRE∙R∙Qm∙v, with NRE∙Qm∙v coded bits taken out from a circular buffer for rate matching. In OCC within an OFDM symbol, the modulation symbols need to be spread several times, which result in that not all the modulation symbols can be mapped to the resources and parts of coded bits will be dropped. The characteristics of LDPC encoding will be destroyed, causing decoding failure at the receiver. In order to avoid this, when calculating TBS and rate matching, the TBS and number of coded bits from circular buffer need to be divided by the spreading factor, where TBS calculation is modified as Ninfo= NRE∙R∙Qm∙v/NSF and NRE∙Qm∙v/NSF coded bits taken out from the buffer. After this, to keep the same throughput, OCC within an OFDM symbol needs to use a higher MCS, split the TB into smaller TBs or enable TBoMS to increase NRE.

Apple [8] mentioned the inter-symbol(s) time-domain OCC has specification impact on resource mapping, since the same OFDM symbol will be repeated by OCC length in a PUSCH.

Several companies observed TBS determination procedure in clause 6.1.4.2 of TS38.214 can be reused for:

- Inter-slot time domain OCC with repetition type A

- Inter-symbol time domain OCC with repetition type B

Several companies observed the number of available RE per slot (NRE) for TB size determination should be scaled down by occ-length for:

- Inter-symbol time domain OCC (with symbol-level repetitions)

- Intra-symbol pre-DFT-s OCC

UCI Multiplexing:

Huawei [5] provided analysis of the specifications for UCI multiplexing, which we copy here.

- Rate matching for UCI multiplexed on PUSCH is given in clause 6.3.2.4 of TS38.212. This specified how to calculate the number of modulation symbols for HARQ-ACK transmission Q'ACK, the number of symbols for CSI part 1 transmission Q'CSI-1, and the number of symbols for CSI part 2 transmission Q'CSI-2.

- The procedures to multiplex UCI on PUSCH so as to form a codeword is given in clause 6.2.7 of TS38.212.In clause 9 of TS38.213, if a UE transmits a PUSCH over one or more slots or multiple PUSCHs over one or more slots that are scheduled by a DCI format, and the UE would transmit a PUCCH with HARQ-ACK and/or CSI information over a single slot that overlaps with the PUSCH transmission in the one or more slots, and the PUSCH transmission in the one or more slots fulfills the conditions in clause 9.2.5 for multiplexing the HARQ-ACK and/or CSI information, the UE multiplexes the HARQ-ACK and/or CSI information in the PUSCH transmission in the one or more slots.

- If a UE transmits a PUSCH with repetition Type B and the UE would transmit a PUCCH with HARQ-ACK and/or CSI information over a single slot that overlaps with the PUSCH transmission in one or more slots, the UE expects all actual repetitions of the PUSCH transmission [6, TS 38.214] that would overlap with the PUCCH transmission to fulfill the conditions in clause 9.2.5 for multiplexing the HARQ-ACK and/or CSI information, and the UE multiplexes the HARQ-ACK and/or CSI information in the earliest actual PUSCH repetition of the PUSCH transmission that would overlap with the PUCCH transmission and includes more than one symbol.

Several companies observed UCI should be multiplexed on every slot/repetition of the time span of an OCC sequence for

- Inter-slot time domain OCC with PUSCH repetition type A.

- Inter-symbol time domain OCC with repetition type B.

Several companies observed UCI multiplexing can be reused for:

- Inter-symbol OCC (with symbol-level repetitions).

- Intra-symbol pre-DFT-s OCC (but CATT [12] observed a different UCI mapping after multiuser multiplexing compared to the UE without OCC is needed because of RE reduction).

Huawei [5] When multiplexing UCI on PUSCH, the calculation of the number of symbols for HARQ-ACK transmission, CSI part 1 transmission and CSI part 2 transmission can be reused for inter-slot time domain OCC with repetition type A and inter-symbol time-domain OCC with PUSCH repetition type B.

OPPO [7] observed multiplexing UCI on PUSCH may extend the delay of UCI reception due to the UCI multiplexing timeline and multiplexing slots for inter-slot OCC with PUSCH repetition Type A (TBoMS further add to the delay of UCI reception). Three options on how to deal with collisions between PUSCH and PUCCH by UE for case PUSCH and PUCCH are overlapped in time domain are discussed:

- Option 1: UCI is multiplexed on PUSCH, and UCI uses the same OCC scheme and OCC sequence as PUSCH.

- Option 2: PUCCH is transmitted, and PUSCH in one OCC group is dropped (trade-off between UCI performance and uplink capacity/throughput).

- Option 3: Both PUCCH and PUSCH are transmitted.

OPPO [7] Figure 1. Example of UCI on PUSCH with OCC (Option 1)

Apple [8] discussed two approaches to ensure the same contents used in all PUSCH repetitions within an OCC period in case multiple PUCCH resources over different slots have time overlap with PUSCH repetitions.

- Multiplex one UCI with PUSCH repetitions, where a dropping rule needs to be designed on which UCI is to be multiplexed with PUSCH repetitions

- Multiplex UCIs over multiple slots over PUSCH repetitions with jointly process these UCIs with a new control signaling (e.g., for type 2 HARQ-ACK codebook construction) or separately process these UCIs (significant impact on specification and complexity)

Since UCI multiplexing on PUSCH is performed before PUSCH resource mapping with OCC spreading, inter-symbol(s) time-domain OCC does not have specification impact for UCI multiplexing on PUSCH.

Apple [8]Figure 2: UCI multiplexing for inter-slot OCC with PUSCH repetition Type A

Huawei [5] observed the number of REs that can be used for transmission of UCI should be scaled by occ-length for intra-symbol pre-DFT-s OCC.

ZTE [4] proposed if UCI is multiplexed on one of repetitions, OCC is not applied on these repetitions.

Nokia [6] observed based on current specification, too many resources will be occupied by a UCI if the UCI is multiplexed on the PUSCH repetitions with OCC enabled. Meanwhile, scaling down the resource occupied by the UCI by OCC length will be too aggressive and impact the reliability of UCI transmission. Nokia proposed RAN1 study and potentially modify the procedures related to UCI multiplexing on PUSCH with inter-slot OCC scheme and study possible solutions to avoid multiple CSI reference resources issue when a CSI report is multiplexed on PUSCH repetitions with OCC enabled.

RV cycling:

For the transmission of PUSCH with dynamic grant or uplink grant received in a MAC RAR, the sequence of redundancy versions (RVs) is determined in clause 6.1.2.1 of TS38.214. For the transmission of PUSCH with configured grant, the sequence of RVs is determined in clause 6.1.2.3 of TS38.214. Legacy RV cycling can be reused for inter-symbol(s) OCC and intra-symbol OCC.

Several companies observed RV cycling period should be aligned with the span of an OCC sequence for:

- Inter-slot time domain OCC with PUSCH repetition type A

- Inter-symbol time-domain OCC with PUSCH repetition type B.

Several companies observed legacy RV cycling can be reused for:

- Intra-symbol OCC - i.e. within a symbol.

- Inter-symbol OCC (symbol-level repetitions - i.e. within a slot repetition.

ZTE [4] observed in NR NTN with inter-slot time domain OCC, the redundancy versions for different repetitions should be kept the same.

Frequency hopping:

The phase continuity may not be guaranteed after frequency hopping depending on OCC technique.

Huawei [5] observed the time interval of inter slot frequency hopping should be extended to every OCC-length slots for

- Inter-slot time-domain OCC with PUSCH repetition type A.

- Inter-symbol time-domain OCC with PUSCH repetition type B.

Huawei [5] observed Intra-slot frequency hopping and inter-repetition frequency hopping should be avoided for

- Inter-slot time-domain OCC with PUSCH repetition type A.

- Inter-symbol time-domain OCC with PUSCH repetition type B.

Huawei [5] observed both intra-slot and inter-slot frequency hopping can be used without change for intra symbol pre-DFT-s OCC.

Nokia [6] observed frequency hopping might impair applicability of inter-symbol OCC and inter-slot OCC with PUSCH repetitions Type A and proposed to discuss extension of the inter-slot frequency hopping with inter-slot bundling for PUSCH feature in case of inter-slot OCC with PUSCH repetitions Type A.

CATT [12] observed it is not recommended to use frequency hopping in NTN. For DMRS-Bundling to enhance the performance in NTN, phase continuity needs to be ensured. Frequency hopping has weak gain with low SNR and complexity of implementation

Power Control:

Huawei [5] provided analysis of the specifications for power control mechanism for PUSCH in TS 38.213, which we copy below. The potential impact on the PUSCH power control by OCC is the calculation of BPRE. For a given TB to be transmitted, the BPRE value is impacted by the TB size (r=0C-1Kr) and number of resource elements (NRE) available for TB mapping. For inter-slot time domain OCC with PUSCH repetition type A and inter-symbol time-domain OCC with PUSCH repetition type B, existing definition of NRE can be reused. For intra-symbol OCC, NRE depends on how the TDRA in DCI is interpreted.  If the TDRA is interpreted as time domain resource after  OCC, existing definition of NRE can be reused. If the TDRA is interpreted as time domain resource before OCC, then the value of NRE should be multiplied by NOCC since the actual PUSCH resource is NOCC times of the indicated one.

If a UE transmits a PUSCH on active UL BWP b of carrier f of serving cell c using parameter set configuration with index j and PUSCH power control adjustment state with index l, the UE determines the PUSCH transmission power PPUSCH,b,f,c(i,j,qd,l) in PUSCH transmission occasion i as

[dBm]

where,

……

- ∆TF,b,f,ci=10log102BPRE⋅Ks-1⋅βoffsetPUSCH for Ks=1.25 and ∆TF,b,f,ci=0 for Ks=0 where Ks is provided by deltaMCS for each UL BWP b of each carrier f and serving cell c. If the PUSCH transmission is over more than one layer [6, TS 38.214], ∆TF,b,f,ci=0. BPRE and βoffsetPUSCH, for active UL BWP b of each carrier f and each serving cell c, are computed as below

- BPRE=r=0C-1KrNRE for PUSCH with UL-SCH data and BPRE=Qm⋅RβoffsetPUSCH for CSI transmission in a PUSCH without UL-SCH data, where

- C is a number of transmitted code blocks, Kr is a size for code block r, and NRE is a number of resource elements determined as NRE=N⋅MRB,b,f,cPUSCH(i)⋅j=0Nsymb,b,f,cPUSCHi-1Nsc,dataRB(i,j), where N≥1 is provided by numberOfSlotsTBoMS as described in [6, TS 38.214] and N=1 if numberOfSlotsTBoMS is not provided, Nsymb,b,f,cPUSCHi is a number of symbols for PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c, Nsc,dataRB(i,j) is a number of subcarriers excluding DM-RS subcarriers and phase-tracking RS samples [4, TS 38.211] in PUSCH symbol j and assuming no segmentation for a nominal repetition in case the PUSCH transmission is with repetition Type B, 0≤j<Nsymb,b,f,cPUSCHi, and c, Kr are defined in [5, TS 38.212]

Huawei [5] observed the existing BPRE of the PUSCH power control can be reused for

- Inter-slot time-domain OCC with PUSCH repetition type A

- Inter-symbol time-domain OCC with PUSCH repetition type B

Huawei [5] observed the number of resource elements calculating BPRE for PUSCH power control should be the total resource elements after OCC spreading for intra-symbol OCC.

MediaTek [11] observed inter-slot OCC does not require enhancements for power control.

OCC indication/configuration:

Huawei [5] provided an analysis of the specifications for the scheduling of PUSCH transmission In clause 6.1 of TS38.214, as copied below. Under the configured grant Type 1, the PUSCH transmission is usually performed after reception of configuredGrantConfig.

PUSCH transmission(s) can be dynamically scheduled by an UL grant in a DCI, or the transmission can correspond to a configured grant Type 1 or Type 2. The configured grant Type 1 PUSCH transmission is semi-statically configured to operate upon the reception of higher layer parameter of configuredGrantConfig including rrc-ConfiguredUplinkGrant without the detection of an UL grant in a DCI. The configured grant Type 2 PUSCH transmission is semi-persistently scheduled by an UL grant in a valid activation DCI according to clause 10.2 of [6, TS 38.213] after the reception of higher layer parameter configuredGrantConfig not including rrc-ConfiguredUplinkGrant. If configuredGrantConfigToAddModList is configured, more than one configured grant configuration of configured grant Type 1 and/or configured grant Type 2 may be active at the same time on an active BWP of a serving cell.

Several companies proposed OCC spreading on PUSCH should be applicable for all of dynamic grant PUSCH, type 1 and type 2 configured grant PUSCH. This may OCC related parameters such as OCC configuration, OCC sequence index, and OCC length.

Several companies proposed DG-PUSCH for the OCC feature, where the indication of OCC parameters (explicitly or implicitly) is based on either introducing an additional field in the DCI scheduling the PUSCH or based on repurposing some of the existing bits of the DCI scheduling the PUSCH.

CATT [12] observed it needs to be clarified whether the resources indicated by the Resource Allocation field of the DCI are referred to before or after the OCC for intra-symbol OCC.

Spreadtrum [14] proposed PUSCH OCC group can be defined, which is PUSCH repetitions covered by one OCC sequence. Further proposals related to PUSCH OCC groups are made for UCI multiplexing, A-CSI triggers, RV cycling, frequency hopping van be found in their contribution.

PDCCH Capacity for OCC

Ericsson [2] observed the capacity of PDCCH is a bottleneck limiting the possibility to grant PUSCH resources for OCC-based transmissions with high OCC multiplexing levels. If OCC multiplexing level 8 were intended to be supported, it would become essential studying DCI enhancements to alleviate the downlink overhead. Using a single DCI granting a group of UEs to perform simultaneously an OCC-based transmission on the same time-frequency resources could be studied to alleviate the downlink overhead when scheduling an OCC-based PUSCH transmission, especially if OCC multiplexing level 8 were intended to be supported.

Ericsson [3] Table 6: Fraction of DL resources used for DCI with PDCCH aggregation level 8 and 20 PUSCH slot repetitions.

| OCC level | Number of UEs per 20 ms | Fraction of DL resources used for PDCCH |  |
| --- | --- | --- | --- |
|  |  | PDCCH AL=4 | PDCCH AL=8 |
| 1 | 24 | 9% | 17% |
| 2 | 48 | 17% | 34% |
| 4 | 96 | 34% | 69% |
| 8 | 192 | 69% | >100% |

OCC sequence:

Huawei [5] discussed that to multiplex 2 UEs with inter slot OCC with PUSCH repetition type A or inter symbol OCC with PUSCH repetition type B, the minimum length of an OCC sequence is 2. In order to multiplex with non-Rel-19 PUSCH with repetition, there should be a row with all “1” elements. Then, two OCC sequences can be defined from the rows of matrix W2, which is equivalent to Walsh or DFT matrix of size 2.

Huawei [5] prosed for OCC length 2, support to define the O CC sequences from the rows of matrix W2=111-1.

Huawei [5] propose the choice of OCC sequence of length 4 should take the following perspectives into account:

- Flexibility to multiplex UE with different OCC lengths;

- Orthogonality under residual CFO.

ZTE [4] proposed the OCC sequences defined in Table 6.3.2.5A-1 and Table 6.3.2.5A-2 can be reused.

Apple[8]  proposed RAN1 is to use Walsh codes to generate OCC sequence.

Signalling aspect specification:

Several companies commented signalling aspects aspects (rate matching / TB calculation / FH / UCI multiplexing, RV cycling etc.) are postponed until RAN1 concludes on the specific OCC scheme(s) to be specified.

Moderator summary and views on signalling aspects of OCC schemes:

- Detailed specification aspects specific to the type of OCC scheme such as rate matching, TBS calculation, Frequency Hopping, UCI multiplexing, RV cycling should be de-prioritized in discussions in RAN1 meeting until support of the OCC scheme is confirmed by RAN1 agreement.

- OCC sequences defined in TS 38.211 Table 6.3.2.5A-1 and Table 6.3.2.5A-2 can be reused for inter-slot time-domain OCC and Inter-symbol OCC with PUSCH repetition Type B..

- OCC sequences defined in TS 38.211 Table 6.3.2.6.3-1 and Table 6.3.2.6.3-2 can be reused for Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2 or 4.

- OCC spreading on PUSCH should be applicable for all of dynamic grant PUSCH, type 1 and type 2 configured grant PUSCH with indication of OCC configuration, OCC sequence index, and OCC factor.

- DG-PUSCH for the OCC feature, where the indication of OCC parameters (explicitly or implicitly) is based on either introducing an additional field in the DCI scheduling the PUSCH or based on repurposing some of the existing bits of the DCI scheduling the PUSCH.

## 3.2 First Round Discussion

Initial proposal 3-1:

Observation

OCC sequences defined in TS 38.211 Table 6.3.2.5A-1 and Table 6.3.2.5A-2 can be reused for inter-slot time-domain OCC and Inter-symbol OCC with PUSCH repetition Type B.

Initial proposal 3-2:

Observation

OCC sequences defined in TS 38.211 Table 6.3.2.6.3-1 and Table 6.3.2.6.3-2 can be reused for Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2 or 4.

Initial proposal 3-3:

The following should be configured for OCC spreading on PUSCH:

- DM RS port

- OCC sequence index

- OCC factor

FFS indication for DG-PUSCH (explicit or implicit), CG-PUSCH Type 1 and Type 2

Companies are encouraged to provide comments within the following table:

| Companies | Comments |
| --- | --- |
| Panasonic | Initial proposal 3-1, Initial proposal 3-2: Agree. Initial proposal 3-3: Because “configured” gives an impression of RRC signaling, we prefer to use “indicated” rather than “configured” because whether to use RRC signaling or DCI/MAC CE is FFS. The following should be indicated configured for OCC spreading on PUSCH: |
| LGE | [Initial proposal 3-1] Support. [Initial proposal 3-2] Support. [Initial proposal 3-3] DM-RS port should be removed from the list since it is not a property of OCC spreading. We propose the following update: The following should be configured/indicated for OCC spreading on PUSCH: - DM RS port - OCC sequence index - OCC factor length FFS indication for DG-PUSCH (explicit or implicit), CG-PUSCH Type 1 and Type 2 |
| Ericsson | Initial proposal 3-1: Time domain OCC schemes could also use DFT sequence. Suggest to use “OCC sequences defined in TS 38.211 Table 6.3.2.5A-1 and Table 6.3.2.5A-2 or Table 6.3.2.6.3-1 and Table 6.3.2.6.3-2 can be reused for inter-slot time domain OCC …”. Initial proposal 3-2: Ok Initial proposal 3-3: Ok. Suggest to use “At least the following…” since more parameters are not excluded. |
| Lenovo | Proposal 3-1: Support in general. We prefer to remove PUSCH repetition type B as inter-symbol OCC may be used for PUSCH repetition type A. Proposal 3-2: Agree Proposal 3-3: We need to add OCC technique type if more than one OCC technique is supported. Better to postpone the proposal until down selection is agreed. The following should be configured for OCC spreading on PUSCH: - DM RS port - OCC technique - OCC sequence index - OCC factor FFS indication for DG-PUSCH (explicit or implicit), CG-PUSCH Type 1 and Type 2 |
| Nokia | Proposal 3-1: OK. Proposal 3-2: OK. Proposal 3-3: The list might not be complete, since the “starting time” or “phase” may also be changed. Consider multiplexing of 4 UEs (on the same resources) with 20 repetitions, but the scheduling has not been initiated at the exact time (some slot offset may happen due to PDCCH being provided at different time instances). Here it would be good to also be able to control the exact “start time” of the OCC sequence. |
| NTT DOCOMO | Initial proposal 3-1/3-2: Agree. Initial proposal 3-3: Whether DMRS port is needed for OCC indication can be further discussed. We agree with LGE’s revision. |
| Apple | Initial proposal 3-1: OK. Initial proposal 3-2: OK Initial proposal 3-3: suggest to change “configured” to “indicated”. We are also not sure whether DMRS port is needed for OCC indication. |
| Xiaomi | Initial proposal 3-1: OK. Initial proposal 3-2: OK. Initial proposal 3-3: We suggest to discuss it until the specific solution is down-selected. Besides, for DG-PUSCH maybe the dynamic indication for OCC sequence index and the current DMRS port indication scheme are enough. |
| CMCC | Initial proposal 3-1: Support Initial proposal 3-3 In general fine with the proposal. It can be updated as Initial proposal 3-3: The following should be configured/ indicated for OCC spreading on PUSCH: - DM RS port - OCC sequence index - OCC factor FFS indication for DG-PUSCH (explicit or implicit), CG-PUSCH Type 1 and Type 2 |
| ZTE | Initial proposal 3-1 and 3-2: As an observation, it should be clarified that both Walsh code and DFT code can be applied for inter-slot OCC, but only DFT code as in 3-2 can be used for intra-symbol OCC. Initial proposal 3-3: DMRS port is the legacy indication, and should be separated from the OCC indication. As for other 2 parameters, we think at least sequence index should be included, if OCC factor refers to OCC length, it will be directly determined by the configured OCC sequence. |
| Huawei, HiSilicon | Proposal 3-1/3-2: the sequence depends on OCC scheme. It should be determined after we make the selection. Proposal 3-3: DMRS port is already indicated/configured. Only OCC length and OCC index are sufficient. |
| CATT | For Initial proposal 3-3, it needs more discussion. It is too early to agree it. For example, DMRS port, sequence and OCC index/length can be separately indicated or jointly. Explicit or implicit indication can be considered further. |
| HONOR | Initial proposal 3-1: OK. Initial proposal 3-2: OK For Initial proposal 3-3, it needs more discussion. |

## 3.3 Summary of First Round Discussion

Based on first round discussions on signalling aspects, proposals were made for offline discussions in Section 4.1 and the proposals 3-1a and 3-2 on sequence aspects were discussed in the first online session. One company mentioned some new sequence design may be considered and asked for discussions on sequences to be delayed for inter-slot OCC. ,

Proposal 3-2 is not needed, since intra-symbol OCC (comb-like structure as in PUCCH format 4 with spreading factor 2 and 4 is specified, the sequence design is already specified.

Proposal 3-3 can be postponed to next meeting.

## 3.4 Conclusion

There was no specific agreements made on signaling aspects. Companies are encouraged to further contribute on these aspects taking into account the agreement and conclusion made at RAN1#118.

# 4 Proposals

## 4.1 Proposals for First Offline Sessions

Initial proposal 1-3a:

Observation

Inter-slot OCC can be supported at least with multiplexing of 2 UEs.

Mapping of OCC techniques:

Initial proposal 2-1a:

Whether to schedule PUSCH with OCC with 1 PRB or 2 PRBs is up to gNB implementation.

Initial proposal 2-2:

If supported, Inter-symbol OCC with PUSCH repetition Type B is only considered under the condition that the nominal repetition is not across slot boundary.

Initial proposal 2-3:

Observation

If supported, Intra-symbol OCC or Inter-symbol(s) OCC (symbol-level repetition with no PUSCH repetition B) should be used with TBoMS assuming available RE per slot (NRE) for TB size determination are scaled down by occ-length to allow gNB scheduling with very low MCS.

Note: Whether to use TBoMS with intra-symbol OCC is up to network and UE capability

Initial proposal 2-5:

Observation:

Intra-symbol pre-DFT OCC can be applied to PUSCH with repetition type A/B and TBoMS, and re-use the existing time-domain PUCCH resource mapping.

Initial proposal 2-6:

Observation

Inter-slot time-/domain OCC wih PUSCH repetition A and Inter-symbol OCC with PUSCH repetition B can be used for case with 1 non-Release 19 UE with CG-PUSCH and no PUCCH on PUSCH for any of the slots assigned the code [+1, +1] or [+1, +1, +1, +1] for OCC length 2 or 4 respectively.

Initial proposal 2-8a:

At least one of the OCC techniques will be specified are prioritised:

- Inter-slot time-domain OCC with PUSCH repetition Type A with OCC length 2 [and 4]

  - FFS at least FPUSCH repetition Type B with 7 symbols per repetition over 2 slots with OCC length 2 and 4, other combinations not precluded

- Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2 and 4

Signalling aspects:

Initial proposal 3-1:

Observation

OCC sequences defined in TS 38.211 Table 6.3.2.5A-1, and Table 6.3.2.5A-2, Table 6.3.2.6.3-1 and Table 6.3.2.6.3-2  can be reused for inter-slot time-domain OCC and Inter-symbol OCC with PUSCH repetition Type B.

Initial proposal 3-2:

Observation

OCC sequences defined in TS 38.211 Table 6.3.2.6.3-1 and Table 6.3.2.6.3-2 can be reused for Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2 or 4.

## 4.2 Proposal for First Online session

Mapping of OCC techniques:

Initial proposal 2-8b:

At least one of the OCC techniques will be specified:

- Inter-slot time-domain OCC with PUSCH repetition Type A with OCC length 2 [and 4]

- FFS Inter-slot time-domain OCC with PUSCH repetition Type B with OCC length 2 and 4

- Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2 and 4

Initial proposal 2-2a:

If supported, Inter-slot OCC with PUSCH repetition B is only considered under the condition that the repetition is not across slot boundary.

Initial proposal 2-5:

Observation

Intra-symbol pre-DFT OCC can be applied to PUSCH with repetition type A/B and TBoMS, and re-use the existing time-domain PUCCH resource mapping.

Initial proposal 2-6:

Observation

Inter-slot time-/domain OCC wih PUSCH repetition A and Inter-symbol OCC with PUSCH repetition B can be used for case with 1 non-Release 19 UE with CG-PUSCH and no PUCCH on PUSCH for any of the slots assigned the code [+1, +1] or [+1, +1, +1, +1] for OCC length 2 or 4 respectively.

Signalling aspects:

Initial proposal 3-1a:

Observation

OCC sequences defined in TS 38.211 Table 6.3.2.5A-1, Table 6.3.2.5A-2, Table 6.3.2.6.3-1 and Table 6.3.2.6.3-2  can be reused for inter-slot time-domain OCC and Inter-symbol OCC with PUSCH repetition Type B.

Initial proposal 3-2:

Observation

OCC sequences defined in TS 38.211 Table 6.3.2.6.3-1 and Table 6.3.2.6.3-2 can be reused for Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2 or 4.

## 4.3 Proposals for Second Offline session

Initial proposal 1-4:

Multiplexing of 8 UEs with PUSCH OCC is not discussed in RAN1 until the work for multiplexing of 2 UEs and/or 4 UEs has been completed

Initial proposal 2-6b:

Observation

A UE without PUSCH OCC with repetitions Type A can be multiplexed with Rel-19 UE with inter-slot PUSCH OCC if at least the following conditions are met:

- CG-PUSCH with same RV

- No UCI on PUSCH for any of the slots

- No frequency hopping

## 4.4 Proposals for Second Online session

Initial proposal 1-4:

Multiplexing of 8 UEs with PUSCH OCC is not discussed in RAN1 until the work for multiplexing of 2 UEs and/or 4 UEs has been completed.

# 5 Conclusions

The following agreement and conclusion were made at RAN1#118

Performance of OCC Techniques:

Conclusion

Multiplexing of 8 UEs with PUSCH OCC is not discussed in RAN1 until the work for multiplexing of less than 8 UEs has been completed.

Mapping of OCC schemes:

Agreement

At least one of the OCC techniques when PUSCH repetitions are used will be specified:

- Inter-slot time-domain OCC with OCC length 2

- Inter-slot time-domain OCC with OCC length 2 and 4

- Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2

- Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2 and 4

- Note: combination of techniques is not precluded

- PUSCH repetition Type B is not considered

# 6 Appendix

The following agreements were made in RAN1#117

Conclusion

OCC with PUSCH can support at least multiplexing of 2 or 4 UEs and achieve up to 2 or 4 times capacity gains respectively, when repetitions are used.

Note: the actual gain may be less due to e.g. intra/inter cell interference.

Agreement

For the normative phase, at least one of the OCC techniques will be specified:

- Inter-slot time-domain OCC with PUSCH repetition Type A with OCC length 2 or 4

- Inter-symbol(s) time domain OCC with OCC length 2 or 4

- Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2 or 4

- FFS Combination of OCC techniques including multiplexing of 8 UEs

- FFS Use of OCC techniques with TBoMS

- FFS Backward compatibility with non-Rel-19 UEs

The following agreements were made in RAN1#116bis

Agreement

Support OCC for PUSCH in Rel-19 NR NTN:

- At least PUSCH with Type A repetition

  - FFS PUSCH without Type A repetition for intra-symbol and/or inter-symbol cases

- At least code length 2 or 4, FFS code length 8

- FFS: number of RBs

- Potential OCC techniques listed below are for further down-selection:

  - Inter-slot time-domain OCC with PUSCH repetition Type A

  - Inter-symbol(s) time domain OCC

  - Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4)

  - Combinations of OCC techniques

- TBoMS for OCC techniques is FFS

Agreement

RAN1 to at least further study the potential specification aspects on OCC techniques:

- TBS calculation / Rate matching

- UCI multiplexing

- RV cycling across repetitions

- Frequency hopping, e.g. intra /inter slot

- OCC indication/configuration

- Power control

- FFS others aspects

The following agreements were made in RAN1#116

Agreement

Adopt the table below for assumptions for Evaluation parameters for link level evaluation in NR NTN UL capacity and throughput enhancements

| Parameter | Value |
| --- | --- |
| Channel model | - NTN-TDL-C Rural, 30° elevation angle |
| Carrier frequency | - 2 GHz |
| Subcarrier spacing | - 15 kHz |
| UE speed | - 3 km/h |
| Frequency hopping | - No frequency hopping |
| PUSCH mapping type A with | - 14 OS- for OCC across slots including DMRS |
| HARQ configuration | - No HARQ |
| Channel coding | - LDPC |
| TBS | Reported by companies, e.g. - ≈184 bits payload @AMR 4.75kbps96 bits @Low data rate |
| DMRS configuration / port / bundling | 1 port per UE Reported by companies - DMRS positions for single-symbol DMRS and optional double-symbol DMRS for PUSCH mapping type A defined in Table 6.4.1.1.3-3 and Table 6.4.1.1.3-4 respectively with ld=14, l0=2 and pos1 in [38.211]. - up to 8 DMRS Ports Optional DMRS Bundling |
| PRBs/MCS | Reported by companies, e.g. - 1 PRB, 2 PRBs - MCS in Table 6.1.4.1-2 in [TS 38.214] |
| Max repetition number | - Reported by companies – up to 20 for VoIP, up to 32 for low data rates |
| OCC length | Reported by companies, e.g. - Up to 8 |
| OCC sequence | Reported by companies, e.g. - Walsh sequences in Table 6.3.2.6.3-1 in TS38.211 - DFT sequence in Table 6.3.2.6.3-2 in TS38.211 |
| Antenna configuration at Satellite | - 1Rx |
| Antenna configuration at UE | - 1Tx |

Agreement

-

| Parameter | Value |
| --- | --- |
| TO | Reported by companies - With TO: Uniform selection from [-0.94us, 0.94us], where 0.94us=29Ts - Optional without TO |
| FO | Reported by companies - Uniform selection from [-0.1 ppm, +0.1 ppm], Variation of frequency error is negligible. - Optional: with lower maximum residual FO, to be reported by companies |
| Timing drift | Optional |
| Receiver algorithm | To be reported by companies, e.g. - MMSE |
| Channel estimation | - Real channel estimation |

Agreement

Adopt the table below for assumptions for KPIs for link level evaluation in NR NTN UL capacity and throughput enhancements

| Parameter | Value |
| --- | --- |
| Number of code-division multiplexed users | Reported by companies (up to 8) |
| KPI – SNR for a target BLER per UE | As in Rel-18 (otherwise reported by companies) - VoIP: SNR @2% BLER - For other cases: SNR @10% BLER |
| KPI - Aggregated throughput | Reported by companies Total throughput according to number of code-division multiplexed users (up to 8) Note: companies should also report the throughput for the case without OCC |

# 7 References

RP-241667, Moderator (Huawei), New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, RAN#104, Shanghai, June, 2024

R1-2405685, Moderator (MediaTek), Feature lead summary #5 of AI 9.11.3 on NR-NTN uplink capacity and throughput , Fukuoka, Japan, May 20th – 24th, 2024

R1-2406123, Ericsson, On uplink capacity/throughput enhancement for NR NTN, RAN1#118, Maastricht, The Netherlands, August 19th–23rd, 2024

R1-2406132, ZTE, Discussion on UL capacity enhancement for NR NTN, RAN1#118, Maastricht, The Netherlands, August 19th–23rd, 2024

R1-2405840, Huawei, HiSilicon, Discussion on uplink capacity/throughput enhancement for FR1-NTN, RAN1#118, Maastricht, The Netherlands, August 19th–23rd, 2024

R1-2406756, Nokia, Nokia Shanghai Bell, NR-NTN uplink capacity/throughput enhancements, RAN1#118, Maastricht, The Netherlands, August 19th–23rd, 2024

R1-2406231, OPPO, Discussion on NR-NTN uplink capacity/throughput enhancement, RAN1#118, Maastricht, The Netherlands, August 19th–23rd, 2024

R1-2406865, Apple, On NR-NTN Uplink Capacity Enhancement, RAN1#118, Maastricht, The Netherlands, August 19th–23rd, 2024

R1-2407051, Qualcomm, NR-NTN uplink capacity/throughput enhancement, RAN1#118, Maastricht, The Netherlands, August 19th–23rd, 2024

R1-2406672, Samsung, Discussion on uplink capacity/throughput enhancement for NR-NTN, RAN1#118, Maastricht, The Netherlands, August 19th–23rd, 2024

R1-2406779, MediaTek, NR-NTN uplink capacity and throughput, RAN1#118, Maastricht, The Netherlands, August 19th–23rd, 2024

R1-2406361, CATT, Discussion on UL capacity enhancement for NR NTN, RAN1#118, Maastricht, The Netherlands, August 19th–23rd, 2024

R1-2406277, Xiaomi, Discussion on NR-NTN PUSCH capacity enhancement, RAN1#118, Maastricht, The Netherlands, August 19th–23rd, 2024

R1-2405927, Spreadtrum, Discussion on NR-NTN uplink capacity and throughput enhancement, RAN1#118, Maastricht, The Netherlands, August 19th–23rd, 2024

R1-2406757, Lenovo, Discussion on NR-NTN Uplink Capacity/Throughput Enhancement, RAN1#118, Maastricht, The Netherlands, August 19th–23rd, 2024

R1-2406591, Panasonic, Uplink capacity/throughput enhancement for NR-NTN, RAN1#118, Maastricht, The Netherlands, August 19th–23rd, 2024

R1-240722, DoCoMo, Discussion on NR-NTN uplink capacity/throughput enhancement, RAN1#118, Maastricht, The Netherlands, August 19th–23rd, 2024

R1-2406005, CMCC, Discussion on the NR-NTN uplink capacity/throughput enhancements, RAN1#118, Maastricht, The Netherlands, August 19th–23rd, 2024

R1-2406101, China Telecom, Discussion on NR-NTN uplink enhancement, RAN1#118, Maastricht, The Netherlands, August 19th–23rd, 2024

R1-2406967, Google, Uplink Capacity Enhancement for NR NTN, RAN1#118, Maastricht, The Netherlands, August 19th–23rd, 2024

R1-2406555, NEC, NR-NTN uplink capacity/throughput enhancement, RAN1#118, Maastricht, The Netherlands, August 19th–23rd, 2024

R1-2406110, InterDigital, Inc., NR-NTN uplink capacity/throughput enhancement, RAN1#118, Maastricht, The Netherlands, August 19th–23rd, 2024

R1-2406448, LG, Discussion on NR-NTN uplink capacity/throughput enhancement, RAN1#118, Maastricht, The Netherlands, August 19th–23rd, 2024

R1-2406053, NICT, “NR-NTN uplink capacity/throughput enhancement”, RAN1#118, Maastricht, The Netherlands, August 19th–23rd, 2024

R1-2406076, TCL, Discussion on NR-NTN uplink capacity/throughput enhancement, RAN1#118, Maastricht, The Netherlands, August 19th–23rd, 2024

R1-2406740, ETRI, Discussion on NR-NTN uplink capacity/throughput enhancement, RAN1#118, Maastricht, The Netherlands, August 19th–23rd, 2024

R1-2406204, Vivo, Discussions on NR-NTN uplink capacity enhancement, RAN1#118, Maastricht, The Netherlands, August 19th–23rd, 2024

R1-2406967, Inmarsat, Viasat, Views on NR-NTN PUSCH enhancement, RAN1#118, Maastricht, The Netherlands, August 19th–23rd, 2024

R1-2406802, Sharp, NTN NR uplink capacity enhancement, RAN1#118, Maastricht, The Netherlands, August 19th–23rd, 2024

R1-2406513, Fujitsu, Discussion on uplink capacity/throughput enhancement for FR1-NTN, RAN1#118, Maastricht, The Netherlands, August 19th–23rd, 2024