3GPP TSG-RAN WG1 Meeting #118	Tdoc R1-2407267

Maastricht, Netherlands, August 19th – August 24th, 2024

Agenda Item:	9.1.2

Source:	Moderator (Ericsson)

Title:	Summary #1 of specification support for positioning accuracy enhancement

Document for:	Discussion, Decision

# Introduction

This document summarizes the discussions during RAN1#118 for the agenda item 9.1.2, Specification support for positioning accuracy enhancement.

This discussion corresponds to the objectives related to the positioning use case described in RP-234039 (WID) below.

RP-234039 (WID), Objective: Provide specification support for the following aspects: - AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]: - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback - Identification related signalling is part of the above objective - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models - Signalling mechanism of applicable functionalities/models - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2 - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above two use cases. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring. Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - CSI feedback enhancement [RAN1]: - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843. - For CSI prediction (one-sided model), further study performance gain over Rel-18 non-AI/ML based approach and associated complexity, while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843 (e.g., cell/site specific model could be considered to improve performance gain). - Necessity and details of model Identification concept and procedure in the context of LCM [RAN2/RAN1] - CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]: - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods - Model transfer/delivery [RAN2/RAN1]: - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study - Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis NOTE: offline training is assumed for the purpose of this project. NOTE: the outcome of the study objectives should be captured in TR 38.843 for future reference. NOTE: Coordination with SA/SA WGs of the ongoing study/work as it may relate to their required work.

In the following, the delegates from participating companies are listed. Please check and add the contact information. We can use this list for future offline discussion, if such a need arises.

| Company/organization name | Contact person | Email of contact person |
| --- | --- | --- |
| Ericsson | Yufei Blankenship | yufei.blankenship@ericsson.com |
| Ericsson | Florent Munier | florent.munier@ericsson.com |
| Fujitsu | Xin WANG Yujia SHAN | wangxin@fujitsu.com shanyujia@fujitsu.com |
| MTK | harrison | harrison.chuang@mediatek.com |
| ZTE | Cong WANG | wang.cong8@zte.com.cn |
| CMCC | Yi Zheng Yongchang Liu | zhengyi@chinamobile.com liuyongchang@chinamobile.com |
| InterDigital | Fumihiro Hasegawa | fumihiro.hasegawa@interdigital.com |
| vivo | Huaming Wu | huaming.wu@vivo.com |
| vivo | Yuanyuan Wang | yuanyuan.wang.txyj@vivo.com |
| CATT | Yongqiang Fei | feiyongqiang@catt.cn |
| NEC | Chen Wei Miao Zhaobang | chen_wei@nec.cn miao_zhaobang@nec.cn |
| Xiaomi | Qin MU | muqin@xiaomi.com |
| OPPO | Zhihua Shi | szh@oppo.com |
| Pengcheng laboratory | Pingye XIANG | xiangpy@pcl.ac.cn |
| NVIDIA | Xingqin Lin | xingqinl@nvidia.com |
| TCL | Tianqi Wu | tianqi1.wu@tcl.com |
| LG Electronics | Jaehoon Chung | jhoon.chung@lge.com |
| Qualcomm | Mohammed Hirzallah (Ali) | mhirzall@qti.qualcomm.com |
| ETRI | Seungjae Bahng | sjbahng@etri.re.kr |
| Baicells | Xiang YUN Xiaonan WANG | yunxiang@baicells.com wangxiaonan@baicells.com |
| Apple | Kome Oteri | ooteri@apple.com |
| Fraunhofer | Mohammad Alawieh Georgios Kontes | mohammad.alawieh@iis.fraunhofer.de georgios.kontes@iis.fraunhofer.de |
| Sony | Basuki Priyanto Yujie Zhang | Basuki.priyanto@sony.com Yujie.zhang@sony.com |
| Samsung | Qi XIONG | Q1005.xiong@samsung.com |
| NTT DOCOMO | Luhua You Kosuke Shima | youlh@docomolabs-beijing.com.cn kousuke.shima.nr@nttdocomo.com |
| Intel | Chatterjee, Debdeep | debdeep.chatterjee@INTEL.COM; |
| Nokia | Dick Carrillo Melgarejo | dick.carrillo_melgarejo@nokia.com; |
| CEWiT | Shiv Shankar Dhivagar Baskaran | shivshankar@cewit.org.in dhivagar.b@cewit.org.in |
| Lenovo | Robin Thomas | rthomas7@lenovo.com |
| HW/HiSi | Thorsten Schier | Thorsten.schier@huawei.com |
| Honor | Guozeng Zheng | zhengguozeng@honor.com |
| IIT Madras | K Sai Prasanth | saiprasanth@5gtbiitm.in |

# Model input

## Sample-based vs path-based measurements for model input

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Qualcomm (R1-2407020) Proposal 3: For AI/ML positioning, in Alt-A sample-based measurements definition, study ranges for timing granularity factor k and number of samples Nt’ based on values in existing specifications (up to Rel-18) (e.g., k integer in [-6,5]) and Nt’ can be up to 9 samples). FFS: potential combination of k and Nt’ values. Proposal 4: For AI/ML positioning, in Alt-A sample-based measurements definition, LMF can signal to device the recommended timing granularity factor k and number of samples Nt’, and device selects the final values to be considered for measurement and reporting. Proposal 5: For AI/ML positioning, in Alt-A sample-based measurements definition, consider criteria for selecting Nt’ samples based on the first detected path, device can report up to Nt’ samples anchored with respect to the first detected path. FFS: Anchoring options. Observation 10: For AI/ML positioning channel measurement generation for model input Case2b/3b, the following are uncertainties that can be realized due to different implementations and can result in timing errors/shifts or changes in shape of time-domain channel response: - Uncertainties common to Alt-A and Alt-B: - Uncertainty due to NW synchronization errors (e.g., relative time difference between TRPs) - Uncertainty due to gNB TX/RX timing errors (e.g., RF group delays) - Uncertainty due to UE TX/RX timing errors (e.g., RF group delays) - Uncertainty due to TRP/gNB clock timing errors and drifts - Uncertainty due to UE clock timing errors and drifts - Uncertainty due to determining OFDM symbol boundaries (e.g., uncertainty in FFT window alignment in time domain (if FFT is used)) - Uncertainty due to generation of time-domain channel response (e.g., algorithm/methodology used to generate power delay profile starting from a frequency-domain channel response (if such conversion is used)) - Uncertainties specific to Alt-A: - Uncertainty associated with determining how the sampling grid to be aligned to channel time-domain response (including timing location of grid’s first sample measurement) - Uncertainty associated with selection of Nt’ samples - Uncertainties specific to Alt-B: - Uncertainty associated with path finding methodology Proposal 7: For time domain channel measurement in AI/ML positioning Case2b/3b, support Alt-B path-based measurements as the default and baseline measurement reporting for LMF-side model (Case2b/3b). - FFS: necessity of Alt-A.

- Huawei, HiSilicon (R1-2406978) Proposal 1: Regarding the type of channel measurements for Case 2b/3b, support the use of legacy mechanism subject to path-based measurements. • Sample-based method does not need to be introduced since they require a substantial specification effort and may not resolve potential ambiguity issues. Proposal 2: For Case 2b/3b, support enhancing the legacy reporting by increasing the number of reported delay and power values from the UE/gNB to LMF.

- InterDigital, Inc. (R1-2406206) Proposal 20: For case 3b and case 1, support Alternative (b) (path based measurements) for representation of time domain channel measurements and reuse path-based reporting with the granularity up to k=-6 as specified in Release 18. Proposal 21: For case 3a, support Alternative (b) (path based measurements) since an AIML model may be able to estimate ToA that is not aligned with sampling periods. Proposal 22: For direct AI/ML based positioning, for case 3b, no enhancements are needed for UL measurements (UL-RTOA, SRS-RSRP and SRS-RSRPP) reported from the gNB to LMF

- CATT, CICTCI (R1-2406354) Proposal 10: For case 1 and case 2a, at least when UE-side model is trained by UE-side, UE generates the measurement for AI/ML model inference based on the PRS transmission. The input type or format is up to UE implementation. Proposal 11: For case 3a, at least when gNB-side model is trained by gNB/NW-side, gNB/TRP generates the measurement for AI/ML model inference based on the SRS-pos transmission. The input type or format is up to gNB/TRP implementation. Proposal 12: At least for case 3b and 2b, support sample-based channel measurements as the AI/ML model input. Proposal 14: For sample-based measurement, the following alternative rules can be considered for selecting Nt’ samples. • Rule Alt 1: The selected Nt’ samples are the Nt’ strongest power samples. • Rule Alt 2: Selecting samples whose power is higher than a threshold. Nt’ can be varying. • Rule Alt 3: Selecting Nt’ samples whose power is higher than a threshold. Nt’ should satisfy the value range configured by LMF. Proposal 15: For sample-based measurement, at least for case 2b and case 3b, a measurement window (Nt samples) is introduced from which to select Nt’ samples. Proposal 17: For case 3b, if both sample-based reporting and path-based reporting are supported, the choice of sample-based reporting and path-based reporting is based on gNB/TRP implementation or LMF indication. Proposal 19: For case 2b, if both sample-based reporting and path-based reporting are supported, the choice of sample-based reporting and path based reporting is based on UE implementation or LMF indication.

- Ruijie Networks Co. Ltd (R1-2406594) Proposal 2: The time domain window size Nt should be the maximum value of Nt'. Proposal 3: It is proposed to reuse Nt' = 9 as a starting point for the minimum value of Nt'. Proposal 4: The rule for selecting Nt' samples is suggested to be Nt' samples with the strongest power selected from the estimated channel response in time domain.

- TCL (R1-2406394) Proposal 4: The sampling rule for the sample-based measurement should be clarified, e.g., the timing granularity T can be derived based on the bandwidth of PRS or SRS. Proposal 5: The value range of integer k is 2 <= k <= 8 for PRS and 2 <= k <= 11 for SRS. Proposal 6: The number of selected samples or selection range can be configured for selecting Nt' samples. Proposal 8: The symbol level measurement window could be configured for AI/ML based positioning.

- CMCC (R1-2405976) Proposal 1: Sample-based measurements for AI/ML positioning is slightly preferred. Proposal 5: For AI/ML based positioning, additional configurations or limitations can be supported to improve the efficiency of the measurement data reporting for positioning. Proposal 12: For selecting Nt' samples, assuming that these Nt' samples are the strongest power samples in the sampling window or introducing a threshold could be considered. Proposal 13: For selecting Nt' samples, entity derive AI model could have opportunity to report its recommended number of samples.

- Samsung (R1-2406638) Proposal 8. Path-based measurement could be considered with potential overhead reduction.

- Sony (R1-2406464) Proposal 6: Support sample-based time domain channel measurements as the AI/ML model input.

- CEWiT (R1-2407076) Proposal 1: In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, time domain sample-based measurement reporting is preferred. Proposal 2: Support the unified sample based approach where model input is truncated channel measurement rather than selecting paths based on strongest power.

- Sharp (R1-2406970) Proposal 1: For model input of the AI/ML based positioning, sample-based measurement should be supported. Proposal 2: For AI/ML based positioning, sample-based measurement with selecting samples of strongest power should be supported.

- OPPO (R1-2406255) Proposal 2: In Rel-19 AI/ML based positioning, regarding the time domain channel measurements for UE-assisted/LMF based positioning with LMF-side model (Case 2b) and NG-RAN node assisted positioning with LMF-side model (Case 3b) - The existing reporting (from UE to LMF or gNB to LMF), which are path-based measurement, can be used as the input of LMF-side model - It can be done by LMF’s implementation. Thus, there is no RAN1 specification impact - If the detailed scheme for sample-based measurement and the performance gain is justified, it can be supported as an optional UE feature for Case 2b / Case 3b.

- Intel Corporation (R1-2406015) Proposal 11: • Support path-based measurement for representation of time domain channel measurements in AI/ML based positioning with potential enhancement of the number of reported paths.

- vivo (R1-2406173) Proposal 5: It is necessary to further specify the rules of selecting samples from channel measurement if sample-based measurement is specified. Proposal 6: As for the rules of selecting samples from channel measurement, at least selecting earliest Nt’ samples and selecting strongest Nt’ samples are feasible, and can be considered as a start point. Proposal 7: Specify sample-based channel measurement reporting for Case 2b and 3b of AI/ML based positioning.

- Xiaomi (R1-2406270) Proposal 1: - For Case 2b and Case 3b, support sample-based input. - For Case 1, Case 2a and Case 3a, it is up to implementation to determine the model input Proposal 2: If there is need for the model input specification, down select the following options for the time domain samples selection by comparing the positioning accuracy performance and the signaling overhead - Option 1: Select Nt consecutive time domain samples as model input - Option 2: Select N't,. time domain samples with the strongest power as model input Proposal 3: If there is need for the model input specification, specify multiple values of Nt or N't, for configuration. Proposal 22: For inference in Case 3b and Case 2b - Input data format for inference need to be specified and common input data format can be used for both data collection and inference - If multiple input data formats are defined, LMF configure the input data format for inference - The signaling exchange between gNB/UE and LMF is left to RAN3/RAN2

- NEC (R1-2406536) Proposal 1: For the definition of sample-based measurement, the rule for selecting the Nt' samples should be, selecting the strongest power from the estimated channel response in time domain. Proposal 2: For the definition of sample-based measurement, the value range of Nt' is advised to: − Less than or equal to 9, or − Large than or equal to 9 but less than or equal to 256. Proposal 3: For the definition of sample-based measurement, the value range of granularity k is advised to: − Large than or equal to 0 but less than 5. Proposal 4: For case 2b/3b, LMF determines a consistent k value, a consistent Nt' value and a consistent rule (if more than one rules are agreed) to select the Nt' samples and informs them to TRP/gNB/UE/PRU for generating sample-based measurement. Proposal 5: Prioritized sample-based measurements over path-based measurement for model input, if down-selection between sample-based input and path-based input is needed. Proposal 6: Support applying the same alternative, i.e., sample-based measurement, for all the cases, unless new evaluation demonstrates additional benefits from applying the different alternatives for each cases.

- Fujitsu (R1-2406306) Proposal 1 For case 3b/2b model inference, LMF may configure at least the following rules to gNB/UE to select the number of samples for reporting:  Option 1 Nt’-1 consecutive samples after first sample/path.  Option 2 Nt’ strongest non-consecutive samples which the power is larger than a threshold.  Option 3 Nt’ samples with the largest delay spread which the power is larger than a threshold. Proposal 2 For sample-based measurement configuration, the minimum configurable k value is dependent on the sample period, and it is suggested that minimum k value is equal to or less than 3 for model performance purpose.

- Google (R1-2405951) Proposal 3: Support the NW to configure whether the UE should report the sample-based or path-based measurement results. Proposal 2: Support the L1-SINR as the quality indicator of channel measurement.

- NVIDIA (R1-2406493) Proposal 2: Support both of the following alternatives for time domain channel measurements for AI/ML based positioning: • Alternative (a). Sample-based measurements, where the timing information is an integer multiple of sampling periods. • Alternative (b). Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods.

- Apple (R1-2406827) Proposal 1: Rel-19 AI/ML based positioning, when comparing sample-based measurement input and path-based measurement input: - Overhead: Path-based measurements have a lower overhead (6.4124 samples on average for 100MHz) than sample-based measurements (16.5393 samples on average for 100 MHz). - Performance: - Path based measurements showed worse performance than sample-based measurements in low complexity and/or smaller bandwidth scenarios. - Path based measurements showed approximately the same performance as sample-based measurements in large bandwidth scenarios with high complexity models. Proposal 2: On the applicability to the different sub-use cases in the LCM stages of AI/ML positioning: Path/Sample Input for different Sub-use cases Training Measurement Input; Inference; Monitoring. Case 1; Specify both; N/A; Depends on location and monitoring input required. Case 2a; Specify both; Specify both. Case 2b; Specify both; N/A. Case 3a; Specify both; Specify both. Case 3b; Specify both; N/A. Proposal 3: Support both path-based measurement input and sample-based measurement input. - Sample based model input can be viewed as a special case of path-based input with equi-spaced timing samples Proposal 4: Support modification of path-based feedback to support sample type feedback for case 2b and 3b (for inference) and all types (for data collection in model training). The existing path-based feedback specification can be modified to support sample-based model input by: - Timing is eqi-spaced on a grid with signaling that indicates the equal spacing size - The relative time difference of each reported path (nr-RelativeTimeDifference) is an integer multiple of a configured timing and represents a sample timing - The reference path may be the 1st path or the largest path by magnitude - Increasing the number of additional paths supported to [16, 32, 64, 128]. - Option 1: The samples reported may be Nt consecutive samples - Option 2: The samples reported are the the Nt strongest samples - Support both absolute and differential RSRPP mapping for RSRPP values - The magnitude depends on the input type - Delay profile: no magnitude - Power Delay Profile: RSPP - CIR: RSPP and phase

- ZTE Corporation, Pengcheng Laboratory (R1-2406055) Proposal 10: The procedure for determining the number of samples and timing granularity k for sample-based measurement report follows the current handshake framework, while the selection of timing granularity should follow LMF’s configuration. Proposal 11: Support defining sample selection rule for sample-based measurement report, and the following alternatives can be considered: - Alternative (a): UE reports N’t samples with the strongest power. - Alternative (b): UE reports N’t samples with extremum strongest power. - Detailed selection criterion can be further discussed, e.g., interval between selected samples. Proposal 1: Support LMF configuring Nt for sample-based measurement. Proposal 2: For path-based measurement, enhancement on the number of reported paths is not needed. Observation 4: Compared with path-based measurement, the implementation details of sample-based measurement is more clear, which provides unified UE/TRP behavior for channel measurement. Observation 5: Ambiguity exists in both sample-based and path-based measurements, and the ambiguity in sample-based measurement can be removed, but the ambiguity in path-based measurement cannot be removed. Proposal 14: In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, support sample-based measurements.

- Lenovo (R1-2406441) Proposal 1: The value range of Nt’ samples needs to take into account total number of continuous sample Nt, i.e. Nt’ < Nt for sample-based measurements. Proposal 2: Legacy values for the reporting range factor (k) may be used as a starting point, while the value range (v) of [-v×Tc, v× Tc], e.g., v = [-985024: 985024] or other min/max values, may be further discussed. Proposal 3: Further consideration may be given to extend the maximum number of reported additional paths for path-based measurements. FFS the value of the maximum number of reported paths. Proposal 4: On the time domain channel information representation, consider support for a hybrid approach in relation to the cases: - For Cases 1, 2b, and 3b: Support sample-based time domain representation - For Cases 2a and 3a: Support legacy-based path timing time domain representation - Note: For each of the above use cases, use of one type time domain representation (e.g. sample-based approach) in training and another type of time domain representation (e.g. path-based approach) during inference is not supported. Proposal 5: It is up to implementation to resolve any hardware ambiguities/inconsistencies of either sample-based or path-based time domain channel measurements during training and inference. Proposal 6: RAN1 to at least support the following selection rules for sample-based measurement: - Sample selection based on configured path power threshold or between min and max path power - Sample selection based on a configured measurement window with a start time and window length FFS whether selection rules also apply to path-based measurements for selecting up to N additional paths. Proposal 7: Consider the following additional conditions to reduce sample-based measurement overhead: 1) reduction in the number of TPs, 2) reduction in ground truth locations to be measured or 3) based on measurement quality metrics. These additional conditions can also assist in ensuring consistency between training and inference.

- NTT DOCOMO, INC. (R1-2406921) Proposal 4: Support sample-based measurement for AI/ML based positioning in Rel-19. Proposal 5: For the report granularity k of sample-based measurement， - K is indicated by LMF to UE via LPP for case 1, 2a, 2b, and to gNB via NRPPa for case 3a, 3b. - Legacy parameter timingReportingGranularityFactor can be considered as baseline, the value range can be reused, i.e., (0…5). Proposal 6: For the number of samples Nt’ which composes the measurement for sample-based measurement, - Nt’ is indicated by LMF to UE via LPP for case 1, 2a, 2b, and to gNB via NRPPa for case 3a, 3b - Legacy parameter nr-AdditionalPathList can be considered as baseline, and larger value can be introduced for sample-based measurement, e.g., 32, 64, 128, etc. - Predefine the selection rule for Nt’ samples e.g., UE reports the Nt’ samples with strongest power.

- Fraunhofer IIS, Fraunhofer HHI (R1-2406711) Proposal 1: To minimize discrepancies between training and inference, the desired selection criteria can be signaled: - First paths - Strongest paths - First paths and strongest paths Proposal 2: To minimize discrepancies between training and inference, the sample/path can be differentiated as: - The sample-based estimates samples from the channel response without processing - The path-based estimates from the channel response include internal processing to detect the path Proposal 3: Support reporting measurement times which comprises a measurement window of Nt consecutive samples or Nt' samples from a measurement window of Nt consecutive samples.

- MediaTek Korea Inc. (R1-2407105) Proposal 2a-1: For sample-based method, the reporting granularity is in terms of the RS transmission bandwidth Proposal 2a-2: For sample-based method, the reporting granularity corresponds to the adoption of the IDFT size 2x <= RS BW (PRB number x 12) < 2x+1, where both 2x and 2x+1 are suitable IDFT size. It means, the reporting granularity could be 1/(SCS* 2^x ) or 1/(SCS* 2^(x+1) ) Proposal 2a-3: For sample-based method, if a single reporting granularity is needed, consider to choose the reporting granularity with the corresponding IDFT size closest to RS BW (in subcarriers) Proposal 2a-4: The measurement entity reports to LMF which granularity is used. This is because, if the reporting granularity is related to RS transmission BW and SCS, the two parameters have been signalled to the UE. UE could choose the granularity from two candidates

- Indian Institute of Tech (M), IIT Kanpur (R1-2407063) Proposal 1: Given the performance gain shown by sample-based model input as well as the ambiguity in ob-taining path-based measurements, we suggest the use of sample-based model inputs.

- Nokia (R1-2406587) Proposal 2: RAN1 to consider both: path-based and sample-based representation for AI/ML positioning cases. Proposal 3: RAN1 to include in the definition of sample-based measurement the parameter Nt, which can be also signalled along with Nt’ and k parameters. Proposal 4: When sample-based representation is used, LMF configures the UE to determine N’t and choose sample selection strategy/rule-based for a fair trade-off between positioning requirements and signaling overhead.

- Ericsson Inc. (R1-2405945) Proposal 3 For Rel-19 AI/ML based positioning Case 3b, LMF includes an indicator in the measurement request to gNB, where the indicator is on the LMF preferred measurement report: either (a) sample measurement, or (b) legacy path-based measurement. Proposal 4 Define the starting time of the timing window as the timing of the first detected path. Proposal 6 For the definition of sample-based measurement, adopt the sample selection rule used in Rel-18 SI: select the Nt' samples with the highest Nt' powers among the Nt samples in the timing window, Nt'<=Nt. Proposal 7 For Case 3b, parameters {Nt, Nt', k} are chosen by LMF and signaled to gNB. Proposal 8 For Case 2b, sample-based reporting and (if supported) enhanced path-based reporting are new capabilities. For sample-based reporting, maximum values of Nt' and Nt are UE capabilities. Proposal 9 For Case 3b, a small number of k values, e.g., k={4}, k={3,4}, or k={2,3,4,5}, is supported for timing reporting granularity factor in Rel-19.

### 1st round discussion

Regarding the issue of sample-based vs path-based measurements for model input, the investigation follows from the agreement made in RAN1#116 (Feb 2024) and RAN1#117 (May 2024).

Agreement (RAN1#117) Sample-based measurement is defined as: - The measurement is composed of Nt' samples of the estimated channel response in time domain. The timing information for the Nt' samples are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The corresponding measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' samples. - Nt' and k can be signalled - FFS: the value range of Nt'; the value range of integer k for the timing granularity T. - The timing information is defined relative to a reference time Further discussion is expected on the determination of Nt' and k (including signaling) , and a rule to be introduced for selecting Nt' samples. Note: It doesn’t imply the definition of Sample-based measurement will be captured into the spec. Agreement (RAN1#117) Path-based measurement refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed). Agreement (RAN1#116) In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, RAN1 investigate the following alternatives: - Alternative (a). Sample-based measurements, where the timing information is an integer multiple of sampling periods. - Alternative (b). Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods. The issues to be studied include, but not limited to, the following: - Tradeoff of positioning accuracy and signaling overhead - Impact and necessary details of gNB/UE implementation to obtain the channel measurement values. - Whether the same Alternative(s) applies to all cases or not - Applicability and necessity of specifying the Alternative(s) to different cases - Note: different sub-cases may have different issues. Note: In addition to timing information, the components for the channel measurement for model input may also include power and potentially phase. To provide the type of the channel measurement in their investigation.

For this topic, agreements were made in RAN1#117 to define sample-based and/or path-based measurements. However, no resolution could be reached on standardizing sample-based and/or path-based measurements. Coming to RAN1#118, companies continue to provide their analysis and evaluations about this issue. Based on the submitted contributions to RAN1#118, companies’ preference between Alternative A and Alternative B is as follow (please update/correct if your view is not captured correctly):

| Method | Supporters |
| --- | --- |
| Alternative A (sample-based measurement), 22 sources | vivo, CEWiT, Fujitsu, CATT/CICTCI, NEC, CMCC, Sony, ETRI, Mediatek, ZTE/Pengcheng laboratory, Ericsson, Sharp, Xiaomi, TCL, IITM/IIT Kanpur, Ruijie Networks, DOCOMO, OPPO (optional), Lenovo (Case 1,2b,3b) |
| Alternative B (path-based measurement), 8 sources | Huawei/HiSilicon, Qualcomm, Samsung, InterDigital, Intel, OPPO, Lenovo (Case 2a,3a) |
| Specify both, 5 sources | Lenovo, Apple, Nokia, NVIDIA, Google (configure) |
| Unclear 1 source | Fraunhofer |

As a reference, the RAN1#117 status is copied below (see R1-2405389):

| Method | Supporters |
| --- | --- |
| Alternative A (sample-based measurement), 20 sources | vivo, CEWiT, Fujitsu, CATT/CICTCI, NEC, CMCC, Sony, ETRI, Google, Fraunhofer, Mediatek, ZTE/Pengcheng laboratory, Ericsson, Sharp, Xiaomi, TCL, IITM/IIT Kanpur |
| Alternative B (path-based measurement), 6 sources | Huawei/HiSilicon, Qualcomm, Samsung, InterDigital, Intel |
| Specify both, 5 sources | Lenovo, Apple, OPPO, Nokia, NVIDIA |
| Unified/hybrid approach of sample-based and path-based, 3 sources | Ericsson, Intel, Apple |
| Undecided (Support one of Alternative A, B), 1 source | DOCOMO |

Based on the analysis and proposals submitted by companies, Alternative A (sample-based measurement) still has majority support in RAN1#118. However, there are 8 supporters of path-based measurement. The situation is very similar to previous meetings. Thus, it is very difficult for RAN1 to converge to a single design choice, given that extensive evaluations and analysis have been done by many companies since the start of the work item.

Therefore, it is suggested that RAN1 support both sample-based and path-based measurement at least for Case 3b/2b, where the channel measurement has to be reported over the interface for model inference. Further details of sample-based and path-based measurement can be discussed further. Regarding path-based measurement, companies have different views on whether the maximum number of paths should be increased from the existing 1 (first path) + 8 (additional paths), and this point can be further investigated, as stated in the RAN1#117 agreement.

Proposal 2.1.2-1

For Rel-19 AI/ML based positioning Case 3b and 2b, for determining model input, support both Alternative (a) sample-based measurement and Alternative (b) path-based measurement report.

For a given case (3b or 2b), LMF configures whether Alternative (a) or Alternative (b) is used.

Signaling is provided to ensure that the same type of measurement is used during training and inference.

|  | Company |
| --- | --- |
| Support |  |
| Not support | InterDigital, HwHiSi |

| Company | Comments |
| --- | --- |
| ZTE | Current proposal is a comprised solution for the majority. The last bullet is not necessary, the detailed signaling to ensure the ignalingy between training and inference is not clear. Furthermore, case 2b and 3b is LMF side mode, it’s up to LMF’s implementation to ensure the same type of measurement if we support both alt.a and alt.b. |
| InterDigital | Supporting both alternatives is not the solution. We should at least support what can be found in the specification first, i.e., Alternative b. We support Alternative b and make progress on other discussion assuming Alternative b is supported. |
| Fraunhofer | We are fine with the direction of the proposal. It might be helpful, though, to finalize the definition of “sample-based” so that we all have a common understanding of each method’s scope or differences. |
| Nokia | Agree with the direction, however, adding additional details may delay the discussion, based on this we suggest the following rewording: Proposal 2.1.2-1 (updated) At least for Rel-19 AI/ML based positioning Case 3b and 2b, for determining model inference input, support both Alternative (a) sample-based measurement and Alternative (b) path-based measurement report. For a given case (3b or 2b), LMF configures whether Alternative (a) or Alternative (b) is used. Signaling is provided to ensure that the same type of measurement is used during training and inference. |
| Baicells | Support this direction with concerns: Case 3b and 2b may need to be discussed seperately given that in Case 2b, measurements are transferred via air interface while case 3b is not. Suggest the following update for Case 3b: For Rel-19 AI/ML based positioning Case 3b, for determining model input, support both Alternative (a) sample-based measurement and Alternative (b) path-based measurement report. FFS: How Signaling is provided to ensure the same type of measurement is used during training and inference. |
| OPPO | The principle is fine. |
| Sharp | We are fine to support both types of measurement and support Nokia’s updated Proposal. |
| CATT, CICTCI | Fine with the direction. But the last bullet seems not needed. If sample-based or path-based is up to LMF to configure (of course also subject to UE/TRP capability), it is LMF’s implementation to guarantee the same type of measurement during training and inference. Or at least it can be discussed latter. Signaling is provided to ensure that the same type of measurement is used during training and inference. |
| Mtk | 1, We support both measurements, as UE capability 2, The restriction on UE implementation is not preferred. Do we restrict that only native English speaker can speak English when AI model is used for acoustic purpose? 3, the definition of “sampling period“ may need to be discussed further |
| Apple | We are fine with supporting both in the specification (subject to UE capability) |
| Ericsson | Support. Regarding the last bullet, we are ok to further discuss what supporting both implies. However it should be clear that if say alt(b) measurement is requested, the response cannot be a alt(a) type measurement. This can be discussed further in e.g. capability ignaling. |
| HW/HiSi | Do not support. For path reporting it is clear what it means, since the mechanism already is supported in legacy. Before discussing this proposal, we need to understand the details of the sample-based measurement mentioned in the proposal, e.g. how is the sampling rate determined, what is the start and length of the sampling window, how to select the Nt’ samples. For example, based on the contributions to this meeting, several methods have been proposed for the selection of the Nt’ samples. Depending on the selection, the implementation complexity is very different and also risks do disclose proprietary aspects about the implementation. We do agree to discuss this proposal further until we know further details on the sample based method, especially what values to take for Nt and Nt’. We suggest to study Alternative Proposal 2.1.2-1 For Rel-19 AI/ML based positioning Case 3b and 2b, for the study of sample based reporting, RAN1 studies, - Detarmination of samplng rate - Start of sampling wondow - Length of samplng window - Number of Nt samples - Selection method of Nt‘ samples and their maximum values. |
| DOCOMO | We prefer not to support both and prefer to support sample-based measurement. However, based on the current situation, we are fine with this direction if a mojority thinks so. |
| CMCC | Generally fine. For alternative (b) path-based measurement report, the agreement made in the last meeting said that path-based measurement refers to the measurement in the existing specifications. So if no specific enhancement for AI based method is needed, it may not necessary to have a agreement for path-based measurement report cause no spec impact is needed. |
| IIT Madras | Fine with the direction it is going. We are fine with using both the measurements if most of the companies agree. We need to decide on whether down selection for measurements is available or not. With our simulation results we have observed that Alternative A is better. |
| Qualcomm | Let’s agree on Alternative (b) path-based measurements for now. For Alternative (a) sample-based measurement, the definition is still pending many aspects that we can further discuss. |
| Lenovo | We are generally supportive of the compromised the proposal, in principle. Would other cases 1, 2a, and 3a, be further discussed separately ? The 2nd bullet is also a bit ambiguous regarding the aspect “Signalling is provided …” – Would RAN1 or RAN2 discuss this signalling? |
| Fujitsu | Fine with the direction. Keep the main bullet and leave the details on the configuation/indication of sample-based/path-based method may be a better choice for progress. |
| CEWiT | Fine with the proposal, however we still support Alternative A as model input. Also,as LMF has the flexibility to configure the input to its model, its should be upto LMF to ensure the necessary data preprocessing and consistency of measurement during training and inference. |

For the definition of sample-based measurements, companies provided input on sample selection criteria, and how to determine the parameter values for Nt, Nt’, k. Based on companies’ input, the following sample selection criteria has more support:

For sample selection criteria, select the Nt’ samples with strongest power within the time window defined by Nt. That is, the same rule used in Rel-18 study item phase.

For the starting time of the timing window, several companies proposed to use the timing of the first detected path.

Proposal 2.1.2-2

For sample-based measurement, the rule for selecting Nt’ samples is: select Nt’ samples with the highest power in the channel measurement window.

- The starting point of the channel measurement window is the timing of the first detected path.

- The duration of the channel measurement window is Nt * T, where T=2kxTc is the timing granularity.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| ZTE | With this proposal, how to signal Nt’ and k is still not clear, i.e., whether follow the current framework or define new framework for AI/ML. Furthermore, the configuration of Nt is not included. One example of this proposal can be: For sample-based measurement. - Nt: LMF configures Nt, where Nt is the number of consecutive time domain samples. - Nt’: LMF configures candidate values of Nt’, where Nt’ is the samples with the strongest power selected from Nt consecutive time domain samples. - t: LMF configures candidate values of t, where T=2kxTc is the timing granularity. - Note: UE/TRP reports the measurement based on the configured Nt’ and k. |
| Fraunhofer | Share views with ZTE on Nt configuration. Regarding the selection criteria, support first paths, strongest paths, and optionally consistency of reported paths over multiple time instants. If we choose an Nt’ value that’s lower than the number of detected paths, the strongest path might actually be suboptimal. Strong paths from random clusters (like a moving object) aren’t always relevant for representing a CIR. |
| Nokia | Before to deep on details on sample based representation, we suggest to close ASAP Proposal 2.1.2-1 to do further progress on details. |
| CATT, CICTCI | Support the main text. But for the first bullet, we think it cannot address the ambiguty issue if the starting point of the channel measurement is the first detected path – unless all vendors have the same behaviour to detect the so-call first path. This should be FFS and better to be strictly aligned. So suggestion: For sample-based measurement, the rule for selecting Nt‘ samples is: select Nt‘ samples with the highest power in the channel measurement window. - The starting point of the channel measurement window is the timing of the first detected path. - The duration of the channel measurement window is Nt * T, where T=2kxTc is the timing granularity. - FFS the starting point of the channel measurement window |
| mtk | 1, we dont think that using a rule for selection is the way to distinguish path and sample 2, the describe the channel, the good way should be the increase of Nt‘, 3, if it is argued that path-based measurement has no rule, we would say, the legacy path-based measurement is to determine the first path candidates. The intention is quite clear 4, each vendor’s implementation on derving PDP is also different. Then to restrict sample selection by power is not a critical step 5, the starting point of measurement window should be discussed later, not now |
| Apple | We are fine with the proposal. |
| Ericsson | support |
| HW/HiSi | This proposal should be discussed before the draft proposal 2.1.2-1. As mentioned above, how the measurement report is used at the LMF would be transparent to the gNB and UE. A corresponding note should be added. In addition, the use of the term channel measurement window is not needed and may be misleading. We suggest the following updates to the proposal: For sample-based measurement, if supported, the rule for selecting Nt' samples is: select Nt' samples with the highest power in a time window. - The starting point of the channel measurement time window is the timing of the first detected path. - The duration of the time window is Nt * T, where T=2kxTc is the timing granularity. Note: The purpose of the measurement report is transparent to the gNB (Case 3b) and UE (Case 2b) and should not be specified. |
| CMCC | Fine. For the starting point of the window, a reference point like T0+T_SRS may can also be considered. Also, how to determine/ configure the duration of the window should be discussed. |
| Qualcomm | It is good to have alignment on the further aspects of the definition. It seems the channel measurement window Nt was not explicilty mentioned in Alt (a) definition. Let’s first discuss amending the definition. |
| Fujitsu | Support |
| CEWiT | Similar view as HW/HiSi |
| TCL | If the starting point of the channel measurement window is the timing of the first detected path, we should clarify how to distinguish the true path from the noises, especially in the NLOS scenario. We think it is better to set a starting point before the so-called first detected path in order to preserve the channel information as much as possible. The starting point can be determined based on the time difference between the starting point and the adopted reference time. |

For value range of Nt, Nt', k, companies have different views, thus it is difficult to resolve the FFS on value range at this meeting. Regarding the signaling, RAN1#117 has agreed that: “Nt' and k can be signaled”. The proposal below further clarifies that Nt is also signaled. Furthermore, the protocol for Case 3b and 2b follows NRPPa and LPP as in legacy, as proposed by companies. It can be further discussed whether gNB/UE can select its preferred value and signal the parameters to LMF in the measurement report.

Proposal 2.1.2-3

For sample-based measurement, for training data collection of Part A and model inference,

For Case 3b, LMF signals parameters Nt, Nt', k to gNB via NRPPa.

For Case 2b, LMF signals parameters Nt, Nt', k to the UE via LPP.

|  | Company |
| --- | --- |
| Support | OK |
| Not support |  |

| Company | Comments |
| --- | --- |
| ZTE | The detailed signalling can be discussed when Proposal 2.1.2-2 is agreed. |
| Nokia | Before to deep on details on sample based representation, we suggest to close ASAP Proposal 2.1.2-1 to do further progress on details. |
| CATT, CICTCI | Yes. We are also open to discusss whether UE or gNB/TRP can make the final dicision and then signal the actual Nt, Nt‘ and k to LMF. |
| mtk | Discuss this later |
| Ericsson | Support, the LMF should be controlling these parameters. For case 2b, we can add “according to the UE capabilities” |
| HW/HiSi | This proposal is unclear, as currently only Nt' and k can be signalled to the gNB and UE via NRPPa and LPP, respectively, as the parameter Nt is not considered in NRPPa and LPP. Before handling this proposal, we should discuss the value of Nt,. In addition, the parameter Nt', with Nt' up to 9 paths, can be indicated in legacy by the LMF. Thus, a note should be added that the number of paths Nt' that can be reported can be increased. |
| Qualcomm | It looks too early to discuss this proposal. Let’s first finish the supported alternative. Not urgent at this level, we can come back if needed. |
| Fujitsu | Can be discussed later depending on the discussions of Proposal 2.1.2-1 and Proposal 2.1.2-2. |
| CEWiT | Fine with the proposal |
| TCL | For the parameter k, we need to decide whether it is determined by LMF or by rule. The rule can be, for example, the parameter k is calculated based on the subcarrier spacing and the smallest power of two that is not less than the number of REs in the PRBs carrying the reference signal. In this way, the parameter k is self-determined and it is not necessary to signal the parameter. |

## Timing measurement for model input

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Qualcomm (R1-2407020) Proposal 6: For AI/ML positioning, in Alt-A sample-based measurements definition, consider reference timing based on the reference time of existing legacy DL timing measurements (e.g., reference timing considered in DL RSTD as defined in TS 38.215 Clauses 5.1.29). Proposal 8: For time domain channel measurement in AI/ML positioning Case2b/3b, study the necessity of following potential solutions for enhancing uncertainties in Alt-B measurement (Case2b/3b): - Mixed dataset training - Recommended enhancements to RAN4 accuracy tests to reduce uncertainty bounds for additional path measurements Proposal 9: For time domain channel measurement in AI/ML positioning Case2b, as starting point, discuss reference timing considered in existing legacy DL timing measurements (e.g., reference timing considered in DL RSTD as defined in TS 38.215 Clauses 5.1.29).

- Huawei, HiSilicon (R1-2406978) Proposal 3: For Case 2b, for UE channel measurements reported to LMF, the timing information is represented relative to a reference time, where the reference time is selected from the following with potential further down-selection. • Option 1: The reference time is TSubframeRxi, as defined in TS 38.215, clause 5.1.29. • Option 2: The reference time is TUE-TX, as defined in TS 38.215, clause 5.1.30. Proposal 4: For Case 3b/2b, reuse the legacy reporting of timing information or timing and power information from gNB/UE to LMF. • The use of phase information for the measurement reporting would need further justification.

- InterDigital, Inc. (R1-2406206) Proposal 33: Support multi-RTT positioning method under Case 3b without any specification enhancement Proposal 34: Support multi-RTT positioning method under Case 3a with a request from the LMF to the gNB to use an AIML model for generating gNB Rx-Tx time

- CATT, CICTCI (R1-2406354) Proposal 13: For case 2b, the TSubframeRxi defined in TS 38.215 is reused to determine the timing information of sample-based measurement reported from UE to LMF. Proposal 16: For case 3b, the following one or two options of timing information reporting of sample-based measurements are considered: • Option 1: Sample-based reporting -  Time offset: the time offset is the difference between the timing of first sample and UL RTOA reference time; -  Bitmap: the bitmap is used to represent the timing information of Nt’ samples and the first bit is corresponding to the first sample of Nt’ samples; • Option 2: Path-based reporting  Resolution step of the timing information of path should be an integer multiple of sampling periods; -  Path-based reporting may be enhanced to support reporting more samples. Proposal 18: For case 2b, the following one or two options of timing information reporting of sample-based channel measurements are considered: • Option 1: Sample-based reporting  Reference time: the TSubframeRxi defined in TS 38.215;  Time offset: for the measurement of each TRP, the time offset is the difference between the timing of first sample corresponding to a TRP and the reference time. The time offset may consist of, e.g., propagation time difference, symbol offset and the slot offset. The symbol offset and slot offset is relative to the start of slot and subframe which contain the reference signal respectively.  Bitmap: for the measurement of each TRP, the bitmap is used to represent the timing information of Nt’ samples and the first bit is corresponding to the first sample of Nt’ samples; • Option 2: Path-based reporting  Resolution step of the timing information of path should be an integer multiple of sampling periods  Path-based reporting may be enhanced to support reporting more samples.

- ITL (R1-2407166) Proposal 1: For AI/ML based positioning Case 3b, the existing UL RTOA reference time T0+tSRS is applicable for only UL part, and TRP transmit timing of DL subframe should be used to support multi-RTT. Proposal 2: For AI/ML based positioning Case 2b, option 1 should be used to support DL-TDOA, and option 2 should be used to support multi-RTT. - Option 1: The reference time is TSubframeRxi, as defined in TS 38.215, clause 5.1.29. - Option 2: The reference time is TUE-TX, as defined in TS 38.215, clause 5.1.30.

- TCL (R1-2406394) Proposal 7: The DL-RTOA reference time can be introduced for downlink AI/ML based positioning.

- Sharp (R1-2406970) Proposal 3: The bitmap format should be introduced for channel measurement for sample-based measurement of AI/ML based positioning. Proposal 4: How to properly use legacy-like format and bitmap format for each AI/ML positioning case can be further discussed. Proposal 6: DL RTOA reference time is not supported for AI/ML based positioning Case 2b. Proposal 7: For AI/ML based positioning Case 2b, for UE channel measurements reported to LMF, the timing information is represented relative to a reference time, where the reference time is selected from the following. • Option 1: The reference time is TSubframeRxi, as defined in TS 38.215, clause 5.1.29. • Option 2: The reference time is TUE-TX, as defined in TS 38.215, clause 5.1.30.

- Intel Corporation (R1-2406015) Proposal 12: - For representation of the timing information for CIR/PDP/DP to serve as model input at least for AI/ML based positioning Case 2b, consider one of the following two options: - Option DLRefTime-1: One of the TRPs, namely TRP j, is used as a reference TRP. - Following the existing definition of RSTD in TS38.215, the reference time t0n is defined as the time when the UE receives the start of one DL subframe from the reference TRP j that is closest in time to the DL subframe received from TRP n. - Option DLRefTime-2: The reference time t0n is defined as the start of the DL subframe containing the measurement RS received from TRP n. - The start of the DL subframe is determined from the UE Rx perspective. - Solutions to convey the information on propagation delay are pursued, e.g., where the propagation delay information is conveyed separately in addition to the timing information related to CIR/PDP/DP. Proposal 13: - Support relative differential time instance representation for representation of timing information of the channel response for model input in AI/ML based positioning using CIR/PDP/DP. Proposal 14: - If path-based measurements is supported, the existing representation/quantization framework for timing-based metric reporting (e.g., RSTD, RTOA), with the smallest value of ReportingGranularityfactor (‘k’) of -6 as introduced in Rel-18, can be reused for representation of timing information of the channel response for model input in AI/ML based positioning.

- Xiaomi (R1-2406270) Proposal 9: - Study the reference timing for the measurement report in Case 2b - Consider the following options as starting point o Option 1: Reuse the definition of reference timing for UL-RTOA o Option 2: Utilize the receiving sub frame boundary as the reference timing.

- Fujitsu (R1-2406306) . Proposal 3 For AI/ML based positioning case 2b, the reference time of the input related timing information may re-use legacy definitions. TSubframeRxi, as defined in TS 38.215, clause 5.1.29 (option 1) can be baseline.

- Apple (R1-2406827) Proposal 8: On the reference time for UE channel measurements reported to the LMF, RAN1 to re-use the legacy reference time for UE side channel measurements as follows: - Option 1: The reference time is TSubframeRxi, as defined in TS 38.215, clause 5.1.29. - Option 2: The reference time is TUE-TX, as defined in TS 38.215, clause 5.1.30. - We prefer at least option 1

- ZTE Corporation, Pengcheng Laboratory (R1-2406055) Proposal 17: For AI/ML based positioning case 2b, for UE channel measurement reported to LMF, reuse the reference time of current DL RSTD and/or UE Rx-Tx time difference, the following options can be considered: - Option 1 (1st priority): The reference time is TSubframeRxi, as defined in TS 38.215, clause 5.1.29. - Option 2 (2nd priority): The reference time is TUE-TX, as defined in TS 38.215, clause 5.1.30.

- MediaTek Korea Inc. (R1-2407105) Proposal 2c-1: For LMF side model (case 2b) to support DL positioning method (similar to DL-TDOA), the model input for training and inference could also re-use the legacy reference time, which is the first path delay of signal from the reference TRP Proposal 2c-2: The channel responses (PDP) observed from signals of the TRPs, including the reference TRP, start from the reference time

- Nokia (R1-2406587) Proposal 12: For AI/ML based positioning Case 2b (using DL measurement) the reference time is based on the legacy reference time.

- Ericsson Inc. (R1-2405945) Proposal 17 Update the agreement for Case 3b channel measurement reference time by deleting the FFS. Proposal 18 For AI/ML based positioning Case 3a, the reference time for uplink channel measurements is the same as that of Case 3b, i.e., the timing information of DP and PDP is represented relative to the UL RTOA reference time T0+tSRS as defined in TS 38.215. Proposal 20 Option 1: For model input with embedded timing information (e.g., DP, PDP), for training data collection and model inference, the timing information of DL measurement is represented relative to DL RTOA reference time. Proposal 23 As Option 2 for channel measurement reference time for Case 2b, T*SubframeRxi is used. T*SubframeRxi is the time when the UE receives the start of downlink subframe n from the reference TP i, defined by the first detected path in time. The timing information in DL channel measurement (e.g., PDP, DP) of TRP j is obtained using the downlink subframe of TRP j that is closest in time to the subframe n of reference TRP i. Observation 39 Option 3 (TSubframeRxi) is not robust against network synchronization error. Observation 40 Option 4 (TUE-TX) is not appropriate as a reference time for DL channel measurement.

## Power measurement for model input

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- CMCC (R1-2405976) Proposal 2: For path-based measurement reporting, current DL PRS-RSRPP and UL-SRS-RSRPP can be used as a starting point.

- Samsung (R1-2406638) Proposal 6. For paired timing information and power information, following two types are considered: (b-1): separate timing and power values with jointly reporting (b-2): joint time and power value and reporting

- Intel Corporation (R1-2406015) Proposal 15: - For path-based measurements, re-use the existing definitions of DL PRS-RSRPP and UL SRS-RSRPP to represent power information of CIR/PDP/DP paths as AI/ML model input. - For sample-based measurements, re-use the existing definitions of DL PRS-RSRPP and UL SRS-RSRPP to represent power information for sample-based CIR/PDP/DP taps/samples as AI/ML model input. - FFS: Further adaptations to the existing definitions.

- Fujitsu (R1-2406306) Proposal 4 For AI/ML based positioning, the existing DL/UL measurements (e.g., DL PRS-RSRPP/UL SRS RSRPP) defined in TS38.215 can be re-used as the power information of model input.

- ZTE Corporation, Pengcheng Laboratory (R1-2406055) Proposal 9: For AI/ML positioning, regarding the power information for determining the model input: • For DL positioning, DL PRS-RSRPP defined in TS 38.215 can be a starting point for power measurement information. • For UL positioning, UL SRS-RSRPP defined in TS 38.215 can be a starting point for power measurement information. • The mapping of the measurement quality defined in TS 38.133 can be reused for power measurement information.

- MediaTek Korea Inc. (R1-2407105) Proposal 2d-1: The measured channel response, PDP, from each of a set of RS resources could be combined among the set of RS resources to produce a composite PDP, under a same receiver branch Proposal 2d-2: The RSRP with respect to the composite PDP for reporting are not lower than the RSRP of the PDP obtained in any of the individual receiver branches Proposal 2d-3: The receiver branch(es) used for combination are under the same RX TEG

- Ericsson Inc. (R1-2405945) Proposal 10 For the model input types for Case 3b (1st priority) and Case 2b (2nd priority), consider input based on DP or PDP samples containing sample powers summed over all receive antenna ports, i.e., total-power PDP.

### 1st round discussion

In TS 38.215, DL PRS-RSRPP and UL SRS-RSRPP are defined as following.

5.1.35		DL PRS reference signal received path power (DL PRS-RSRPP)

| Definition | DL PRS reference signal received path power (DL PRS-RSRPP), is defined as the power of the linear average of the channel response at the i-th path delay of the resource elements that carry DL PRS signal configured for the measurement, where DL PRS-RSRPP for the 1st path delay is the power contribution corresponding to the first detected path in time. For frequency range 1, the reference point for the DL PRS-RSRPP shall be the antenna connector of the UE. For frequency range 2, DL PRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE for DL PRS-RSRPP measurements, the reported DL PRS-RSRPP value included in the higher layer parameter NR-DL-AoD-MeasElement for the first and additional measurements shall be provided for the same receiver branch(es) as applied for DL PRS-RSRP measurements. |
| --- | --- |
| Applicable for | RRC_CONNECTED, RRC_INACTIVE, RRC_IDLE |

5.2.6	UL SRS reference signal received path power (UL SRS-RSRPP)

| Definition | UL SRS reference signal received path power (UL SRS-RSRPP) is defined as the power of the linear average of the channel response at the i-th path delay of the resource elements that carry the received UL SRS signal configured for the measurement, where UL SRS-RSRPP for 1st path delay is the power contribution corresponding to the first detected path in time The reference point for UL SRS-RSRPP shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: based on the combined signal from antenna elements corresponding to a given receiver branch - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. For frequency range 1 and 2, if receiver diversity is in use by the gNB for UL SRS-RSRPP measurements: - The reported UL SRS-RSRPP value for the first and additional paths shall be provided for the same receiver branch(es) as applied for UL SRS-RSRP measurements, or - The reported UL SRS-RSRPP value for the first path shall not be lower than the corresponding UL SRS-RSRPP for the first path of any of the individual receiver branches and the reported UL SRS-RSRPP for the additional paths shall be provided for the same receiver branch(es) as applied UL SRS-RSRPP for the first path. |
| --- | --- |

The signaling in 37.355 and 38.455 is copied below.

37.355 (LPP)

NR-AdditionalPath-r16 ::= SEQUENCE {

…

...,

[[

nr-DL-PRS-RSRPP-r17				INTEGER (0..126)						OPTIONAL

]]

}

38.455 (NRPPa)

9.2.72		UL SRS-RSRPP

This information element contains the UL SRS Reference Signal Received Path Power measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| First Path RSRP Power | M |  | INTEGER (0..126) |  |

Appendix B shows the report mapping tables for DL PRS-RSRPP and UL SRS RSRPP in 38.133. They show the measured power values corresponding to INTEGER (0..126).

As proposed in by several companies (e.g., ZTE, CMCC, Intel, Fujitsu), the existing definitions of DL PRS-RSRPP and UL SRS-RSRPP can be used as a starting point. If sample-based measurement is used, “path” in the existing definition should be changed to “sample”. Further Enhancement (e.g., composite PDP, total-power PDP) to the power measurement, which are proposed by some companies (e.g., MediaTek, Ericsson), can be discussed further.

Proposal 2.3.2-1

For AI/ML positioning, regarding the power information for determining the model input:

- For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point.

  - For measurement report of DL PRS-RSRPP, reuse the existing measurement report mapping table for PRS-RSRPP in 38.133.

- For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point.

  - For measurement report of UL PRS-RSRPP, reuse the existing measurement report mapping table for SRS-RSRPP in 38.133.

Note: when sample-based measurement is used, “path” is replaced by “sample” in the existing definitions.

|  | Company |
| --- | --- |
| Support | Baicells, CATT/CICTCI, TCL, |
| Not support |  |

| Company | Comments |
| --- | --- |
| ZTE | The principle is generally fine. |
| OPPO | The proposal in fine. |
| Sharp | The Note is not needed. We think it's not enough to simply replace the wording of "path" to "sample". For example, the existing spec says "detected path", but "detected sample" is little strange. It should be "selected sample". Detail discription of the definition can be further discussed. |
| Nokia | There are a lot of aspects to consider on this proposal based on the discussion and potential agreement on the sample-based and path-based representation. The Note is not enough, we agree with Sharp. Our suggestion is to delay the discussion until we have a solid agreement on Proposal 2.1.2-1, we believe that there are dependencies on sample-based, if agreed. |
| Apple | Fine with the proposal. May want to wait on the note. |
| Ericsson | Ok with the proposal. Maybe we can insert the following: For AI/ML positioning, for both alt(a) and alt(b) measurement reporting, regarding the power information for determining the model input: |
| HW/HiSi | OK, but for sample-based measurement we should add, “if supported” in the note Note: when sample-based measurement is used, if supported, “path” is replaced by “sample” in the existing definitions. |
| DOCOMO | Generally fine. Regarding the note, details should be further discussed as sharp said. |
| CMCC | Fine. |
| Qualcomm | Please include the relevant cases (i.e., Case2b/3b?). We delete the last Note until we settle down on alternatives. |
| Lenovo | Also fine with common understanding of the power measurement. Further discussion is needed on “pairing” of this power measurement with the timing information as part of the report. |
| Fujitsu | It is fine in general. |
| CEWiT | Fine with the proposal but not agreeing with the Note. |

## Phase measurement for model input

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Qualcomm (R1-2407020) Proposal 10: For Case2b/3b, no support for reporting phase information (e.g., CIR) for model input running at LMF side.

- Huawei, HiSilicon (R1-2406978) Proposal 4: For Case 3b/2b, reuse the legacy reporting of timing information or timing and power information from gNB/UE to LMF. • The use of phase information for the measurement reporting would need further justification.

- InterDigital, Inc. (R1-2406206) Proposal 26: Do not introduce CIR where all paths or samples (if supported) have phase information Proposal 27: Support first-path phase measurement, namely RSCP and RSCPD, for AIML based positioning

- CATT, CICTCI (R1-2406354) Proposal 21: Phase measurement and reporting is supported for case 2b and case 3b. LMF makes the decision on whether to report the phase measurement and send the corresponding indication to UE/gNB/TRP.

- CMCC (R1-2405976) Proposal 4: For AI/ML based positioning, more discussion is needed for the comparison between CIR and PDP as model inputs.

- Sony (R1-2406464) Proposal 1: Support radio channel characteristics reporting in a form of channel impulse response (CIR) for AI/ML positioning. Proposal 2: Support configurable CIR measurement report (e.g., report size, measurement window size) in an effort to reduce the signalling overhead.

- Sharp (R1-2406970) Proposal 5: For AI/ML based positioning, continuous phase information is supported for model input.

- OPPO (R1-2406255) Proposal 3: For R19 AI-based positioning, NOT support the reporting based on phase information (in additional to timing information and power information).

- Intel Corporation (R1-2406015) Proposal 16: - If phase information as model input is supported, consider extending the existing definitions of DL RSCP and UL RSCP to represent phase information for CIR as AI/ML model input to include phase information for the additional detected paths for path-based measurement or additional samples for sample-based measurement of the CIR as model input. - FFS: Further adaptations to the existing definitions.

- vivo (R1-2406173) Proposal 9: In addition to delay and power, at least RSCP and RSCPD should be supported for AI/ML based positioning. Proposal 10: Raw CIR reporting can be supported, and LMF can eliminate the impact of random initial phases of transceiver by implementation.

- Xiaomi (R1-2406270) Proposal 4: The support of CIR should be deprioritized

- NEC (R1-2406536) Proposal 16: For case 1 and case 2a, whether support using phase as model input is up to UE implementation. Proposal 17: For case 3a, whether support using phase information as model input is up to gNB implementation. Proposal 18: For case 2b and case 3b, support reporting phase information as a component of the triplet {timing information, power information, phase information} for determining model input. The format of phase information reuses the measurement introduced in Rel-18 CPP, e.g., DL RSCPD, DL RSCP, UL RSCP.

- Fujitsu (R1-2406306) Proposal 5 There is no need to have phase information as model input to support AI/ML positioning for the following reasons: a) CIR: Adding phase information can bring only negligible performance gains of AI/ML positioning accuracy with significant overhead increasing. b) Rel-18 measurements DL RSCPD, DL RSCP, UL RSCP: The phase information for the first path only may not be helpful to increase AI/ML positioning accuracy in NLOS scenarios.

- NVIDIA (R1-2406493) Proposal 1: Support reporting phase information in time domain channel measurements for AI/ML based positioning.

- Apple (R1-2406827) Proposal 24: Enhance current measurement input reports to LMF for cases 2b and 3b to support feedback of the CIR, PDP, and DP. - Examples include adding support to signal more elements and support for phase information.

- ZTE Corporation, Pengcheng Laboratory (R1-2406055) Proposal 8: In AI/ML based positioning, support using phase information for determining model input.

- Lenovo (R1-2406441) Proposal 8: RAN1 to support the following additional model input types for DL-based Direct AI/ML positioning measurements for case 2b based on DL-PRS: - Support channel observation measurements in the form of DL CIR measurements Proposal 9: RAN1 to support the following additional model input types for UL-based Direct AI/ML positioning measurements for case 3b based on SRS for positioning: - Support channel observation measurements in the form of UL CIR measurements - Support the inclusion of additional channel profiles such as UL-based angle-delay domain (paired angle/phase and timing information). Proposal 8: RAN1 to support the following additional model input types for DL-based Direct AI/ML positioning measurements for case 2b based on DL-PRS: - Support channel observation measurements in the form of DL CIR measurements Proposal 9: RAN1 to support the following additional model input types for UL-based Direct AI/ML positioning measurements for case 3b based on SRS for positioning: - Support channel observation measurements in the form of UL CIR measurements - Support the inclusion of additional channel profiles such as UL-based angle-delay domain (paired angle/phase and timing information).

- NTT DOCOMO, INC. (R1-2406921) Proposal 7: For direct AI/ML positioning, in addition to timing information and power information, phase information report is considered for determining model input. - Rel-18 measurements (e.g., DL RSCPD, DL RSCP, UL RSCP) are considered as a baseline for phase information report.

- Fraunhofer IIS, Fraunhofer HHI (R1-2406711) Proposal 4: Support a complex valued sample-based reporting offering: - Lossless reporting of the channel impulse response - Supporting future enhancements of the AI/ML model

- Nokia (R1-2406587) Proposal 6: CIR is not supported for inference input when the model running at the LMF-side, gNB-side or UE-side cases. Proposal 5: If RAN1 considers introducing legacy RSCP/RSCPD as inference input for AI/ML positioning, the LoS determination and integer ambiguity needs to be studied.

- Ericsson Inc. (R1-2405945) Proposal 12 Do not support phase information for determining model input, including CIR and single phase value for first path/sample.

### 1st round discussion

Regarding the issue of using phase information (in addition to time and power) for determining model input, the investigation follows from the agreement made in RAN1#116 (Feb 2024).

Agreement (RAN1#116) For AI/ML based positioning for all use cases, RAN1 investigate the necessity and feasibility of using phase information (in addition to timing information and power information) for determining model input. The issues to study include: - Tradeoff of positioning accuracy and signaling overhead - The impact of transmitter and receiver implementation - Specification impact - Other aspects are not precluded Note: the phase information may be used in different ways, e.g., one phase value for the first path or first sample only; triplet of {timing information, power information, phase information} for CIR, etc.

For this topic, no decision could be made in RAN1#117. In contributions to RAN1#118, companies provided further analysis and expressed their views as follows (please update/correct if your view is not captured correctly).

| Support reporting CIR (i.e., phase info in addition to timing info and power info)? |  |
| --- | --- |
| Yes (14) | vivo, Sharp, DOCOMO, Apple, Sony, Fraunhofer IIS/Fraunhofer HHI, ZTE/Pengcheng laboratory, NVIDIA, Lenovo, NEC, CATT/CICTCI |
| No (10) | QC, Nokia, Fujitsu, MediaTek(?), OPPO, Huawei/HiSilicon, Xiaomi, InterDigital, Ericsson |
| Undecided (1) | CMCC |

| Support reporting only first path phase or first sample phase? |  |
| --- | --- |
| Yes (3) | vivo, InterDigital, ZTE |
| No (7) | QC, Fujitsu, Huawei/HiSilicon, OPPO, Ericsson, Intel |

For reference, RAN1#117 status (see R1-2405389) was:

| Support reporting CIR (i.e., phase info in addition to timing info and power info)? |  |
| --- | --- |
| Yes (13) | Sharp, DOCOMO, Apple, Sony, Fraunhofer, ZTE/Pengcheng laboratory, vivo, NVIDIA, Lenovo, NEC, CATT/CICTCI |
| No (10) | QC, Nokia, Fujitsu, MediaTek, OPPO, Huawei/HiSilicon, Xiaomi, InterDigital, Ericsson |

| Support reporting only first path phase or first sample phase? |  |
| --- | --- |
| Yes (3) | vivo, InterDigital, ZTE |
| No (6) | QC, Fujitsu, Huawei/HiSilicon, OPPO, Ericsson |

As shown above, companies’ view is about the same from RAN1#117 to RAN1#118. No consensus is expected to be achieved at this meeting. Based on the discussion status since the start of the work item, it is unlikely that RAN1 can achieve convergence even if more time is given.

Therefore, it is suggested to conclude this discussion as below.

Conclusion 2.4.2-1

For Rel-19 AI/ML based positioning for all use cases, there is no consensus in RAN1 to support CIR for determining model input.

|  | Company |
| --- | --- |
| Support |  |
| Not support | New H3C, ZTE |

| Company | Comments |
| --- | --- |
| New H3C | From our perspective, CIR is very important information to AI/ML based positioning, further investigation should be required. It is a little bit too early to draw conclusion because we have about 10 months to work on this. |
| ZTE | For the collection table, we are OK to support only the phase of first path/sample. Share similar view with New H3C. |
| Baicells | We think at least CIR can be optional. |
| OPPO | The proposal is ok. Since many aspects were suggested to be considered when study CIR-based reporting, e.g., tradeoff of performance and overhead, impact of implementation, spec impacts and so on, it needs much efforts to support CIR or phase information. |
| CATT, CICTCI | We already see some gain by using phase info. We think this should be supported by spec, and whether to report should be subject to UE/TRP capability and up to LMF configuration (2b, 3b). The spec impact is small since RSCP and RSCPD are already specified. |
| Apple | CIR optional as a compromise |
| Ericsson | Ok with the conclusion. For first path reporting, we think it can be supported via rel18 procedures, but we don’t see a reason for adding it to the rel19 AI specific cases. |
| HW/HiSi | Ok |
| DOCOMO | Whether/how CIR is supported can be further discussed, as the performance gain of applying CIR as input is clear. |
| Lenovo | As a compromise, phase information of the 1st path should be considered. |
| Fujitsu | OK |

Conclusion 2.4.2-2

For Rel-19 AI/ML based positioning for all use cases, in terms of using phase information (in addition to timing information and power information) for determining model input, there is no consensus in RAN1 to support using one phase value (e.g., Rel-18 measurements DL RSCPD, DL RSCP, UL RSCP) for the first path or first sample only.

|  | Company |
| --- | --- |
| Support |  |
| Not support | New H3C, InterDigital |

| Company | Comments |
| --- | --- |
| New H3C | From our perspective, phase information is very important to AI/ML based positioning, further investigation should be required. It is a little bit too early to draw conclusion because we have about 10 months to work on this. |
| ZTE | The current specification already support phase measurement and report. Whether taking these measurement as model input is up to LMF’s implementation. We can not limit LMF’s behavior. |
| InterDigital | As NRCP and NRCPD are already supported in specification, these measurements should be supported for AIML positioning by default. |
| OPPO | The proposal is fine. Some issues are mentioned in companies’ tdocs when applying Rel-18 measurements in Rel-19 AI/ML based positioning. These issues need to be resolved first, if we want to support phase information. |
| CATT, CICTCI | Not support, due to the same reason in 2.4.2-1. |
| Ericsson | Ok with the conclusion. |
| HW/HiSi | Ok |
| DOCOMO | Support of phase information report can be considered jointly with support of CIR. |
| CMCC | Although the benifit for reporting phase information for AI may not very clear, we slightly prefer not limiting reporting such kind of information which already supported in current positioning protocal. |
| Fujitsu | OK, considerring that AI/ML is mainly used for NLOS scenarios. |

## Other

### Companies’ views

In the following, selected inputs from companies’ contributions are provided.

- Qualcomm (R1-2407020) Observation 16: For AI/ML positioning inference and data collection in Case1/2a/3a, specifying measurements for model input and discussing alternatives for measurement generation are not necessary.

- Huawei, HiSilicon (R1-2406978) Proposal 5: For Case 2b/3b, there is no need to specify the purpose of the measurement report from the UE/gNB to the LMF as the usage of the measurements is LMF implementation and is transparent to the UE/gNB.

InterDigital, Inc. (R1-2406206) Proposal 23: For direct AI/ML based positioning, for case 1, adopt RSRP, RSRPP and DL-RSTD measurement for the UE to determine input for an AIML model for direct AIML positioning

- Samsung (R1-2406638) Proposal 7. Both truncated channel measurement and feature extracted channel measurement can be considered as candidate for the channel measurement in the collected data sample.

# Model output for assisted AI/ML positioning

This section discusses model output for assisted AI/ML positioning and potential measurement enhancements.

## LOS/NLOS indicator for model output

### Companies’ views

In the following, selected inputs from companies’ contributions are provided.

- Qualcomm (R1-2407020) Proposal 20: For Case3a/2a, study enhancements in reporting of earliest path LOS and timing info measurements in which UE/gNB reports multiple-hypotheses soft info of LOS indicator and timing information. Existing additional path reporting (up to 8 paths) could be repurposed to support the multiple-hypothesis LOS reporting.

- Huawei, HiSilicon (R1-2406978) Proposal 6: For Case 3a/2a, LOS/NLOS indicator can reuse the same format as the existing IE “LoS/NLoS Information” in 38.455 and “LOS-NLOS-Indicator-r17” in 37.355, respectively.

- Sony (R1-2406464) Proposal 7: For AI/ML assisted positioning case 2a and case 3a, the positioning measurement reporting includes the indication whether the LOS/NLOS indicator is based on AI/ML computation or not.

- Sharp (R1-2406970) Proposal 11: For Rel-19 AI/ML assisted positioning, RAN1 supports one or more of following cases regarding LOS/NLOS indicator and timing information to be reported: Case 1: LOS/NLOS indicator derived by AI/ML and timing information derived by legacy measurement are reported Case 2: Only timing information derived by AI/ML is reported

- Intel Corporation (R1-2406015) Proposal 18: - For AI/ML assisted positioning Case 2a, when the LOS/NLOS indicator is reported, - the LOS/NLOS indicator provides information on the likelihood of a Line-of-Sight propagation path from the source to the receiver with a value of 1 corresponding to LoS and a value of 0 corresponding to NLoS, where the propagation path corresponds to the associated timing metric that is determined as AI/ML model output and reported in the same report; - the LOS/NLOS indicator reuse the same format as the existing IE LOS-NLOS-Indicator-r17 in 37.355 for Case 2a; - one LOS/NLOS indicator is optionally reported per TRP or per resource as in the existing specification. Proposal 19: - For AI/ML assisted positioning Case 3a, when the LOS/NLOS indicator is reported, - the LOS/NLOS indicator provides information on the likelihood of a Line-of-Sight propagation path from the source to the receiver with a value of 1 corresponding to LoS and a value of 0 corresponding to NLoS, where the propagation path corresponds to the associated timing metric that is determined as AI/ML model output and reported in the same report; - the LOS/NLOS indicator reuse the same format as the existing IE "LoS/NLoS Information" in 38.455; - one LOS/NLOS indicator is optionally reported per UE as in the existing specification.

- vivo (R1-2406173) Proposal 12: The legacy measurement RSTD and LOS/NLOS indicator can be reused as intermediate measurements of AI/ML assisted positioning. An additional indication is needed to indicate that the report measurement comes from AI/ML models.

- Fujitsu (R1-2406306) Proposal 6 Existing LOS/NLOS indicator format defined in TS37.355 and TS38.455 can be re-used for AI/ML positioning case 2a and 3a model output. Both hard and soft values are applicable.

- Apple (R1-2406827) Proposal 23: For AI/ML assisted positioning (i.e., Case 2a and 3a), when LOS/NLOS indicator is reported, the existing IE in 37.355 (LPP for Case 2a) and 38.455 (NRPPa for Case 3a) can be re-used from RAN1 perspective.

- ZTE Corporation, Pengcheng Laboratory (R1-2406055) Proposal 21: For AI/ML-assisted positioning Case 2a/3a, when the LOS/NLOS indicator is reported, from RAN1 perspective, - the LOS/NLOS indicator is associated with the AI/ML model output; - the LOS/NLOS indicator reuses the same format as the existing IE LOS-NLOS-Indicator in TS 37.355/ LoS/NLoS Information in TS 38.455 for Case 2a/3a; - one LOS/NLOS indicator is optionally reported per TRP or per resource as in the existing specification.

- MediaTek Korea Inc. (R1-2407105) Proposal 2b-1: Change the LOS/NLOS indicator definition slightly when AI/ML applies Proposal 2b-2: The reporting which is through the AI/ML model output may need to be indicated to NW

- Ericsson Inc. (R1-2405945) Proposal 28 For AI/ML assisted positioning Case 3a, when the LOS/NLOS indicator is reported, from RAN1 perspective, the LOS/NLOS indicator provides information on the likelihood of a physical Line-of-Sight propagation path from the source to the receiver with a value of 1 corresponding to LoS and a value of 0 corresponding to NLoS; the LOS/NLOS indicator reuse the same format as the existing IE "LoS/NLoS Information" in 38.455; one LOS/NLOS indicator is optionally reported per TRP as in the existing specification. Proposal 29 For AI/ML assisted positioning Case 2a, when the LOS/NLOS indicator is reported, from RAN1 perspective, the LOS/NLOS indicator provides information on the likelihood of a physical Line-of-Sight propagation path from the source to the receiver with a value of 1 corresponding to LoS and a value of 0 corresponding to NLoS; the LOS/NLOS indicator reuse the same format as the existing IE LOS-NLOS-Indicator-r17 in 37.355 for Case 2a; one LOS/NLOS indicator is optionally reported per TRP or per resource as in the existing specification. Proposal 30 For case 3a and 2a, the timing measurement for the first path is associated with a virtual line of sight path. • If a timing measurement is reported together with the LOS/NLOS indicator, the indicator status (for physical LOS/NLOS) is decoupled from the reported timing measurement (for virtual LOS/NLOS) • Note: when the TRP-UE link is LOS, the virtual LOS coincides with the physical LOS. • Details of the association with virtual LOS is up to RAN2/3, e.g. indication in measurement report, or configuration in measurement request. • Send an LS to RAN2/RAN3 with the final agreement.

### 1st round discussion

Based on companies’ input, majority companies support to reuse the existing LOS/NLOS indicator definitions. Thus the following are proposed. Further discussion is needed on whether it is necessary to indicate that the LOS/NLOS indicator is generated by AI/ML.

Proposal 3.1.2-1

For AI/ML assisted positioning Case 3a, when the LOS/NLOS indicator is reported, from RAN1 perspective,

- the LOS/NLOS indicator provides information on the likelihood of a Line-of-Sight propagation path from the source to the receiver with a value of 1 corresponding to LoS and a value of 0 corresponding to NLoS;

- the LOS/NLOS indicator reuse the same format as the existing IE "LoS/NLoS Information" in 38.455;

- one LOS/NLOS indicator is optionally reported per TRP as in the existing specification.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| ZTE | LOS/NLOS indicator is defined as: The IE LOS-NLOS-Indicator provides information on the likelihood of a Line-of-Sight (LOS) propagation path from the source to the receiver. In current 38.455/37.355, LOS/NLOS indicator is associated with each measurement result, and the LOS/NLOS information of measurement result is the same as the real propagation path. However, for AI/ML, the LOS/NLOS information of the measurement result(e.g., RSTD) is different from the real propagation path. I want to make sure all companies have the same understanding. If we support the current proposal, that means, the LOS/NLOS information is associated with the real propagation path, instead of the virtual direct path. |
| InterDigital | The first bullet is not needed. There is no need to introduce a new definition for the LOS/NLOS indicator in the proposal. For the definition, we can reuse the definition from the TS. |
| CATT, CICTCI | Yes when AI/ML model is used to infer LOS/NLOS condition. No when AI/ML model is used to infer timing information. For this case, legacy LOS/NLOS indicator is quite useless since the timing information (e.g. UL RTOA) inferred by AI/ML is assumed under (virtual) LOS condition. Either we change the definition to a useful one, or we just conclude that LOS/NLOS indicator is not reported in this case. |
| mtk | 1, to agree on this means, there could be a need to additionally define a signalling to descrive the virtual LOS path, and there is intention to push this to ran2/ran3 to do it, not in ran1. We dont think this is right 2, the definition of existing LOS/NLOS indicator is to describe for the propagation channel. May be a simple defintion change when AI/ML is enable is enough and there is no need to additionally design NEW signalling for virtual LOS |
| Apple | - „the LOS/NLOS indicator provides information on the likelihood of a Line-of-Sight propagation path from the source to the receiver with a value of 1 corresponding to LoS and a value of 0 corresponding to NLoS; Are we using the hard indicator or also the soft indicator ? if soft, it should say „a range between 0 and 1“ as shown below: indicator This field provides information on the likelihood of a Line-of-Sight propagation path from the source to the receiver and has the following choices: - soft: This field specifies the likelihood of a LOS propagation path in the range between 0 and 1 with 0.1 steps resolution. Value '0' indicates NLOS and values '1' through '10' provide an estimate of the propability for a LOS propagation path between source and receiver. Scale factor 0.1; range 0 to 1. - hard: This field specifies whether the propagation path between source and receiver is estimated to be LOS (true) or NLOS (false). |
| Ericsson | We agree with the proposal. If LOS is associated with the AIML timing estimate, then it should always be set to “full LOS” as the model should always extract the LOS path. For AI ML, we see the LOS indicator as an additional information to the LMF, separate from the timing estimate. The timing estimate should always be for the virtual line of sight path. |
| HW/HiSi | Ok. |
| DOCOMO | Support to reuse the legacy LOS/NLOS indicator. |
| CMCC | The legacy defination for LOS/NLOS indicator could be reused, including the soft and hard value reporting. |
| Lenovo | Perhaps there is no need for the bullet points, since it legacy behaviour. The main bullet could be sufficient. |

Proposal 3.1.2-2

For AI/ML assisted positioning Case 2a, when the LOS/NLOS indicator is reported, from RAN1 perspective,

- the LOS/NLOS indicator provides information on the likelihood of a Line-of-Sight propagation path from the source to the receiver with a value of 1 corresponding to LoS and a value of 0 corresponding to NLoS;

- the LOS/NLOS indicator reuse the same format as the existing IE LOS-NLOS-Indicator-r17 in 37.355 for Case 2a;

- one LOS/NLOS indicator is optionally reported per TRP or per resource as in the existing specification.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| ZTE | Same as Proposal 3.1.2-1 |
| InterDigital | Same comment as in Proposal 3.1.2-1 |
| CATT, CICTCI | Yes when AI/ML model is used to infer LOS/NLOS condition. No when AI/ML model is used to infer timing information. For this case, legacy LOS/NLOS indicator is quite useless since the timing information (e.g. DL RSTD) inferred by AI/ML is assumed under (virtual) LOS condition. Either we change the definition to a useful one, or we just conclude that LOS/NLOS indicator is not reported in this case. |
| Mtk | same view as 3.1.2-1 |
| Ericsson | support |
| HW/HiSi | Ok |
| DOCOMO | Same comment as in Proposal 3.1.2-1 |
| CMCC | Same reply in Proposal 3.1.2-1 |

## Timing information for model output

### Companies’ views

In the following, selected inputs from companies’ contributions are provided.

- Qualcomm (R1-2407020) Proposal 20: For Case3a/2a, study enhancements in reporting of earliest path LOS and timing info measurements in which UE/gNB reports multiple-hypotheses soft info of LOS indicator and timing information. Existing additional path reporting (up to 8 paths) could be repurposed to support the multiple-hypothesis LOS reporting.

- Huawei, HiSilicon (R1-2406978) Proposal 7: For Case 3a/2a, to distinguish the reported timing information from the legacy measurement-based timing information, consider the following alternatives: • Alt.1: Reuse the timing quality indicator, where reported timing information of Case 2a/3a can be represented with high timing quality and NLOS. • Alt.2: Introduce an indicator to indicate the report is subject to measured timing information or non-measured timing information.

InterDigital, Inc. (R1-2406206) Proposal 24: For Case 1, include a timestamp in the measurement forwarded by the LMF in absolute time to indicate when the measurement was made Proposal 25: For Case 1, the timestamp with absolute time should be included per measurement in the measurement forwarded by the LMF Proposal 29: For AIML assisted positioning, support an indication in the measurement report to indicate the reported timing measurement is inferred.

- CATT, CICTCI (R1-2406354) Proposal 22: For AI/ML-assisted positioning, no need to report LOS/NLOS indicator when timing information is reported based on the output of AI/ML model.

- CMCC (R1-2405976) Proposal 7: For AI/ML based positioning, whether the reported measurement is AI based could have an indication.

- Samsung (R1-2406638) Proposal 13. In attached to the model output, the time stamp and quality information is supported. Proposal 14. RAN1 supports TRP to signal the model output to LMF in case 3a.

- Fujitsu (R1-2406306) Proposal 7 Considering potential challenges and imperfections during training and monitoring phase of AI/ML positioning, a more inclusive framework for different types of model output would be desirable (e.g., multiple model output of timing information). Proposal 8 Regarding model output of case 2a and case 3a, timing information with the following enhancement is suggested to be studied:  LMF configure performance criteria to check the reliability/availability of the model output (e.g., virtual LOS related timing information output).

- Ericsson Inc. (R1-2405945) Proposal 31 Reuse the existing measurement reporting in LPP and NRPPa to report timing information for the model output of AI/ML assisted positioning. Proposal 34 For AI/ML assisted positioning at gNB (Case 3a), the model output is uplink relative time of arrival (TUL-RTOA). Proposal 35 For AI/ML assisted positioning at gNB (Case 3a), postprocessing is applied to generate gNB RxTxTimeDiff for measurement reporting of multi-RTT method. Proposal 36 For AI/ML assisted positioning at UE (Case 2a), the model output is downlink relative time of arrival (TDL-RTOA), which is defined relative to DL RTOA Reference Time. Proposal 37 For AI/ML assisted positioning at UE (Case 2a), postprocessing is applied to generate DL RSTD (for DL-TDOA method) and UE RxTxTimeDiff (for multi-RTT method) for measurement reporting.

## Other

### Companies’ views

In the following, selected inputs from companies’ contributions are provided.

- Intel Corporation (R1-2406015) Proposal 17: - For AI/ML-assisted positioning (Cases 2a and 3a), consider support of reporting of the following information from UE and gNB to LMF respectively: - UE to LMF (Case 2a): RSRP/RSRPP for first and additional paths along with associated path timings, e.g., DL RSTD, UE Rx-Tx time difference. - gNB to LMF (Case 3a): AoA and/or RSRP/RSRPP for first and additional paths along with associated path timings, e.g., UL RTOA, gNB Rx-Tx time difference.

- vivo (R1-2406173) Proposal 11: For AI/ML assisted positioning Case 2a and Case 3a, distance ranging between UE and TRP are supported for reporting. - Note: there is no need to introduce the definition of distance ranging in TS 38.215

- Xiaomi (R1-2406270) Proposal 20: In case 1 and case 2a, there is no specification impact on the delivery of input data for inference or the inference output data Proposal 21: For inference in Case 3a, leave RAN3 to assess whether is any specification impact on deliver the input from TRP to the associated gNB and deliver the input from one gNB to another gNB in different AI model deployment options.

- Apple (R1-2406827) Proposal 22: For AI/ML assisted positioning cases 2a and 3a, the following outputs may also be signaled: - “Made with AI” indicator - Time stamp

- ZTE Corporation, Pengcheng Laboratory (R1-2406055) Proposal 20: For cases 2a and 3a, the model output of AI/ML assisted positioning can be angle information. - Other intermediate features in addition to the output listed in TR 38.843 are not precluded. Proposal 22: For reporting model output of AI/ML assisted positioning (Case 3a/2a), an indicator identifying whether the measurement is based on AI/ML model is not needed.

- Nokia (R1-2406587) Proposal 10: In Case 2a, UE may report the weights resulting from LOS/NLOS classification using various channel features (e.g., delay spread, maximum amplitude, etc.), which can be utilized by LMF when calculating the location estimate.

# Training data collection

This section discusses issues for the LCM stage of model training.

## Content of training data

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Huawei, HiSilicon (R1-2406978) Proposal 8: For Case 3b data collection for LMF-side model, the time stamp can reuse legacy reporting. Proposal 9: For indicating the quality of measurement for training (if needed), consider reusing the legacy quality indication reported by the measurement entity, or indicating the quality criterion to the measurement entity. Proposal 10: At least for Case 1, 2b and 3b, if needed, the UE location quality used in legacy reporting can be used for the quality indication of the label for training.

- InterDigital, Inc. (R1-2406206) Proposal 8: A ground truth label quality indicator is associated with a UE or PRU location Proposal 9: Support both hard (1 or 0) and soft indicator (0, 0.1, 0.2, …, 1.0) for a ground truth label quality indicator Proposal 10: A soft or hard ground truth label quality indicator x=1 is associated with PRU location Proposal 11: A soft ground truth label quality indicator 0<x<1 is associated with UE location estimate Proposal 12: A soft or hard ground truth label quality indicator x=0 is associated with UE generated measurements/ground truth using interpolated or extrapolated “virtual” measurements Proposal 13: A ground truth label quality indicator associated with a PRU or UE location needs to be specified at least for Case 1 Proposal 14: In Case 1, the LMF is the only entity that can generate a ground truth label quality indicator associated with location information Proposal 15: In Case 1, the LMF can provide a generated ground truth label quality indicator associated with location information to the UE Proposal 16: In Case 1, support the target UE to request to the LMF a specific level (e.g., above a threshold) of a ground truth label quality for LMF measurement forwarding

- CATT, CICTCI (R1-2406354) Proposal 4: For case 3a, the following methods for ground truth label generated by LMF are considered: - Method 1: UE/PRU provides the location related information to LMF for generating the ground truth label; - Method 2: Multiple gNBs/TRPs provide the SRS-pos measurements (e.g. UL RTOA) to LMF for estimating UE’s location coordinate and the UE’s location coordinate is used to generate the ground truth label.

- CATT, CICTCI (R1-2406354) Proposal 6: For case 2b and case 3b, when LMF side collects training data, LMF side can use a quality indicator condition or criteria to indicate the required quality of the collected data.

- Sharp (R1-2406970) Proposal 8: Legacy quality indicator and time stamp are reused for data collection of AI/ML based positioning. Proposal 9: For training data collection of AI/ML based positioning, quality indicator can be used to request Part A and Part B.

- OPPO (R1-2406255) Proposal 10: Regarding the training data collection for NG-RAN node assisted positioning with gNB-side model (Case 3a) • gNB do measurement based on SRS o The existing procedure/mechanism can be reused for this purpose • gNB can decide itself when/how to start the data collection procedure. • The contents of collected data are up to implementation and no specification is needed in 3GPP • LMF delivers ground-truth label(s) (or the information based on that the label can be derived) to gNB via NRPPa siganling o gNB needs to send request to LMF on what the groud-truth label (or the information based on that the label can be derived) is

- vivo (R1-2406173) Proposal 1: Specify the following two types of training data to support supervised learning and semi-supervised learning - Type 1: both Part A and Part B - Type 2: Part A only Proposal 2: Study and specify mechanisms for reducing the reporting overhead of collected training data, including: - Reduce the number of reported samples. - Reduce the size of each sample, such as truncation. Proposal 3: Channel measurements including CIR, PDP and DP, should be supported for data collection, which can provide more freedom of implementation for model developer. Proposal 4: Further study how to ensure that Part A and Part B in one training sample are associated with a “same” location, particularly when Part B is generated by non-RAT based positioning methods.

- NEC (R1-2406536) Proposal 11: A quality indicator should be defined for a data sample, and it’s determined based on the quality indicator(s), if available, of associated measurement and ground truth label. Proposal 12: Data generation entity can initially report the data fulfilling the quality indicator threshold and then reports supplemental data in case that previously reported quality data is not adequate.

- NVIDIA (R1-2406493) Proposal 3: For AI/ML based positioning, study the definition of the quality indicator of channel measurement and the definition of the quality indicator of label to introduce specification support.

- Apple (R1-2406827) Proposal 13: For part A, the quality indicator of the channel may be mapped to a timing quality, a power quality and a phase quality depending on the type of input Proposal 14: For part B, the quality of the ground truth label may be set as a value from 0 to 1 similar to the LOS/NLOS soft value. Proposal 15: For model training, the following elements shall be specified as part of the data collection procedure: - Part A - Channel Measurement (corresponding to model input), - Quality indicator (for and/or associated with measurement at least for model training) - Time stamp (for and/or associated with measurement) - Part B - Ground truth label, - Quality indicator (for and/or associated with ground truth label) - Time stamp (for and/or associated with ground truth label) - Assistance Information - RS configuration(s), - other necessary information (e.g., scenario identifier. LOS/NLOS condition, timing error, etc.) Proposal 19: To specify feedback of measurement data for both PRS and UL SRS for positioning, the following parameters may need to be configured: - The measurement type: delay Profile (timing only), Power Delay Profile (timing, power), Channel Impulse Response (timing, power and phase) - The number of measurements (taps): 8, 9, 16, 32, 64, [128], [256] (+1) - Feedback Format - First path parameters: first path magnitude, first path phase first path timing, timing granularity factor. - Additional path parameters - Relative time difference for additional paths - Time reporting granularity factor - Time stamp - Part A quality - Part B quality - Additional path magnitude and phase - Option 1: additional path magnitude, additional path phase - Option 2: additional path relative magnitude, additional path phase

- ZTE Corporation, Pengcheng Laboratory (R1-2406055) Proposal 5: The association between part A and part B can be determined by the model training entity based on the collected measurement information and label information. - Part A can be provided by: - TRP measurement result in TS 38.455 (case 3a, case 3b). - SignalMeasurementInformation in TS 37.355 (case 1, case 2a and case 2b). - Part B can be provided by: - ProvideLocationInformation in TS 37.355 (case 2b, case 3b). - ProvideAssistanceData in TS 37.355 (case 1, case 2a). - New assistance information sent from LMF to gNB (case 3a). - Part A and Part B can be associated using: - the timestamp provided in measurement and/or location information. - UE ID associated with the measurement and/or location information. Proposal 6: Whether supporting semi-supervised learning is up to training entity’s implementation.

- ETRI (R1-2406719) Proposal 3: Add ‘classification identifier’ to both Part A and Part B in the collected data sample.

- Fraunhofer IIS, Fraunhofer HHI (R1-2406711) Proposal 6: Support opportunistic/event-based training data collection for follow-up model improvement/adaptation. Data collection can be triggered for example when: - A specific input pattern (e.g., a specific range of SINR values), for which the model does not perform as expected, is detected. - The UE or a known PRU is in a sub-area that has become challenging for the model. - High quality GTLs can be generated. - A monitoring event indicating reduced model performance (and possibly switching to a different model or to fallback) has been reported.

- Nokia (R1-2406587) Proposal 34: For ground truth data collection, to assess the quality of ground truth, LMF may assist UE for obtaining label consistency score/quality of the positioning estimates from one or multiple positioning sources. Proposal 35: For data collection of high-quality ground truth, LMF assists UEs on linking the PRU location (absolute information) with the relative information generated by sensors (e.g. accelerometers, gyros, magnetometers) deployed in the UE. Proposal 36: For training data collection of AI/ML based positioning, an additional data (e.g., TRP ID, beam ID) may be included in Part A along with the data type to indicate if it is mandatory or optional. Proposal 37: In data collection for ground truth generation by the target UE for UE-side models, LMF may indicate UE positioning method(s) (e.g., non-NR and/or NR RAT-dependent and/or NR RAT-independent) with necessary criteria (e.g., required confidence value) for an estimation to be used as ground truth. Proposal 38: For the data collection of Case 2b/3b (LMF-side), for inference input, depending on the channel observation, UE/gNB can determine/select the required reporting configuration to enable the reporting windowing scheme without compromising the DL/UL measurements content.

- Ericsson Inc. (R1-2405945) Proposal 41 For Rel-19 AI/ML based positioning, support collecting labelled and optionally un-labelled training data samples.

### 1st round discussion

Based on companies’ input to RAN1#118, the quality indicator and the time stamp need to be discussed.

For time stamp, companies (e.g., Huawei, HiSilicon, Sharp) suggest to reuse the existing time stamp. The relevant time stamp IEs in the existing specifications 37.355 and 38.455 are copied below.

37.355 (LPP):

–	NR-TimeStamp

The IE NR-TimeStamp defines the UE measurement associated time stamp.

-- ASN1START

NR-TimeStamp-r16 ::= SEQUENCE {

dl-PRS-ID-r16				INTEGER (0..255),

nr-PhysCellID-r16			NR-PhysCellID-r16			OPTIONAL,	-- Need ON

nr-CellGlobalID-r16			NCGI-r15					OPTIONAL,	-- Need ON

nr-ARFCN-r16				ARFCN-ValueNR-r15			OPTIONAL,	-- Need ON

nr-SFN-r16					INTEGER (0..1023),

nr-Slot-r16					CHOICE {

scs15-r16				INTEGER (0..9),

scs30-r16				INTEGER (0..19),

scs60-r16				INTEGER (0..39),

scs120-r16				INTEGER (0..79)

},

...,

[[

nr-Symbol-r18				INTEGER (0..13)			OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

38.455 (NRPPa):

9.2.42		Time Stamp

This information element contains the time stamp.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| System Frame Number | M |  | INTEGER(0..1023) |  |
| CHOICE Slot Index | M |  |  |  |
| >SCS-15 |  |  | INTEGER(0..9) |  |
| >SCS-30 |  |  | INTEGER(0..19) |  |
| >SCS-60 |  |  | INTEGER(0..39) |  |
| >SCS-120 |  |  | INTEGER(0..79) |  |
| Measurement time | O |  | Relative Time 1900 9.2.36 |  |

Proposal 4.1.2-1

For training data collection of AI/ML based positioning, for both time stamp of channel measurement and time stamp of label, the existing IEs are reused.

- For channel measurement and label generated by PRU or non-PRU UE, reuse the existing IE NR-TimeStamp-r16 in TS 37.355;

- For channel measurement generated by TRP/gNB, reuse the existing IE “Time Stamp” in TS 38.455.

|  | Company |
| --- | --- |
| Support | New H3C, Baicells, DOCOMO, TCL, |
| Not support |  |

| Company | Comments |
| --- | --- |
| ZTE | In current 37.355/38.455, a symbol index could be included for phase measurement. This can be determined after whether first path/sample phase is supported. |
| InterDigital | Our concern is whether the current timestamp can be enough to indicate freshness of data. |
| Nokia | Agree with the direction |
| CATT, CICTCI | OK for the format. But we think we still need to discuss how to determine the timestamp of a channel measurement or a label. |
| Ericsson | Support. We can also let the other RAN WGs know about this if there is an agreement. |
| HW/HiSi | Ok |
| Lenovo | Support |
| Fujitsu | OK |
| CEWiT | Fine with the proposal |

Regarding the quality of label, representative proposals are:

- Legacy UE location quality:

  - Huawei, HiSilicon (R1-2406978): “Proposal 10: At least for Case 1, 2b and 3b, if needed, the UE location quality used in legacy reporting can be used for the quality indication of the label for training.”

  - Sharp (R1-2406970): “Proposal 8: Legacy quality indicator and time stamp are reused for data collection of AI/ML based positioning.”

- Value 0 to 1:

  - InterDigital, Inc. (R1-2406206): “ Proposal 9: Support both hard (1 or 0) and soft indicator (0, 0.1, 0.2, …, 1.0) for a ground truth label quality indicator”

  - Apple (R1-2406827): “Proposal 14: For part B, the quality of the ground truth label may be set as a value from 0 to 1 similar to the LOS/NLOS soft value.”

Regarding legacy UE location quality, the existing IEs in 37.355 and 38.455 are copied below.

37.355 (LPP):

–	NR-TimingQuality

The IE NR-TimingQuality defines the quality of a timing value (e.g., of a TOA measurement).

-- ASN1START

NR-TimingQuality-r16 ::= SEQUENCE {

timingQualityValue-r16			INTEGER (0..31),

timingQualityResolution-r16		ENUMERATED {mdot1, m1, m10, m30, ...},

...

}

-- ASN1STOP

NR-TimingQuality field descriptions

timingQualityValue This field provides an estimate of uncertainty of the timing value for which the IE NR-TimingQuality is provided in units of metres.

timingQualityResolution This field provides the resolution used in the timingQualityValue field. Enumerated values mdot1, m1, m10, m30 correspond to 0.1, 1, 10, 30 metres, respectively.

38.455 (NRPPa):

9.2.43	 Measurement Quality

This information element contains the TRP’s best estimate of the quality of the measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Measurement Quality | M |  |  |  |
| >Timing Measurement Quality |  |  |  | Corresponds to information provided in NR-TimingQuality IE as defined in TS 37.355 [14] |
| >>Measurement Quality | M |  | INTEGER(0..31) |  |
| >>Resolution | M |  | ENUMERATED(0.1m, 1m, 10m, 30m, …) |  |
| >Angle Measurement Quality |  |  |  |  |
| >>Azimuth Quality | M |  | INTEGER(0..255) |  |
| >>Zenith Quality | O |  | INTEGER(0..255) |  |
| >>Resolution | M |  | ENUMERATED (0.1deg, …) |  |

For Cases 1, 2b, 3b, the label is UE location. Quality of label could be uncertainty of UE location in meters, similar to NR-TimingQuality. Thus the following is proposed.

Proposal 4.1.2-2

For training data collection of AI/ML based positioning Cases 1, 2b, 3b, quality indicator of label is represented using one of the following options:

- Option 1 (value in meters): reuse the existing IE. That is, IE NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455;

- Option 2 (value 0 to 1): hard (1 or 0) and soft values (0, 0.1, 0.2, …, 1.0), with value 1 indicate the highest quality.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| ZTE | The principle is generally fine. Prefer to reuse the current quality IE. |
| InterDigital2 | Both options can be supported. For Case 1, the UE can be requested to indicate either Option 1 or Option 2. |
| Nokia | To FL, could you clarify Option 2, it looks that it may generate ambiguity if we agree to standardize this option. |
| CATT, CICTCI | Fine in general, though we think Option 2 has issue if it is a hard value report. But fine to discuss. |
| Ericsson | Support. In previous releases, these options were discussed by other RAN groups. |
| DOCOMO | We prefer to reuse the legacy IE. |
| TCL | Option 1 is preferred. In this way, the model training entity can itself decide the data quality based on the value in meters. |

## Assistance data and other signalling on training data collection

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Qualcomm (R1-2407020) Proposal 13: For AI/ML positioning data collection in Case1/2a, support two labelling assistance options from LMF to UE: • Option 1: Ground truth label (e.g., approximate ground truth of UE location coordinates, LOS indicator/timing info of model output) is generated by LMF and provided to UE • Option 2: Position calculation assistance data (e.g., IE NR-PositionCalculationAssistance [TS 37.355]) is provided from LMF to UE Proposal 14: In AI/ML positioning data collection, for Case1/2a, support UE/PRU to request from LMF PRS configuration and activation for data collection. As a starting point, study the following options for dedicated data collection PRS configuration and activation: • Option A: Data collection PRS configuration/activation as part of existing procedure (e.g., on-demand PRS and existing positioning sessions) • Option B: Data collection PRS configuration/activation as part of new data collection dedicated procedure.

- Huawei, HiSilicon (R1-2406978) Proposal 11: For data collection of Case 3a/3b, the legacy configuration of SRS can be reused. • For training data collection of Case 3a, gNB decides autonomously the training data collection of Part A. o The applicability of LMF and gNB coordinated training data collection is not clear. • For training data collection of Case 3b, LMF can trigger/configure the SRS transmission.

- CATT, CICTCI (R1-2406354) Proposal 7: For case 3b, the measuring time difference between Part A and Part B should be restricted to ensure the validity of a training data sample. Proposal 8: For training data collection of AI/ML based positioning, the time stamp can be defined as follows: - For part A, the time stamp of channel measurement could be the time instance at which the channel measurement is performed. - For part B, the time stamp of ground truth label can be: - The time for which the location estimate is valid; - The time when the associated SRS-pos is sent; - The time when the PRS is measured. For case 1 and case 2b, UE recives the reference signal from N TRPs and genernates N measurements as one training data sample, while the training data sample is with only one label. How to match the N measurements with one label (UE’s location) should be considered. Proposal 9: For training data collection of case 1 and case 2b, further study how to match the measurements from multiple TRPs with one label by related time stamps.

- TCL (R1-2406394) Proposal 10: The assistance information for training data association should be discussed when the models for uplink AI/ML based positioning are trained at LMF and Part B is generated by UE. Proposal 11: Down-select the method for training data association: a. UE validates the data association by implementation and sends an availability indicator or causes of unavailability to LMF. b. LMF validates the data association with the information of immobility duration.

- CMCC (R1-2405976) Proposal 6: For UE generates ground truth label based on non-NR and/or NR RAT-dependent positioning methods, the reliability or the positioning accuracy should also be reported.

- Sony (R1-2406464) Proposal 3: The association of part A and part B in the collected data sample (for training data collection of AI/ML based positioning) can be based on timing information or location information.

- CEWiT (R1-2407076) Proposal 6: To associate the measurement and its related data to collected labels and its related data, the data generating entities can record its ID (UE ID/ TRP ID/ LMF ID), RS configurations and IDs, Area ID along with the collected measurement, labels and its corresponding time stamps.

- Sharp (R1-2406970) Proposal 10: For training data collection of AI/ML based positioning, Area ID and/or identifier for reference signal (e.g. DL-PRS-ID) are provided together with training data.

- OPPO (R1-2406255) Proposal 7: In order to facilitate the training data collection at UE side for Case 1 and/or Case 2a, introduce a mechanism to enable that UE can send request for preferred or supported configuration(s) to LMF • FFS: Whether this request is sent via UE capability signaling or other signaling/procedure (e.g., functionality identification). Proposal 8: Regarding the training data collection at UE side for Case 1 and/or Case 2a • Reuse the legacy LPP signaling to configures UE with the corresponding positioning RS for UE-side data collection • The associated ID is signaled along in order to facilitate the insurance of the consistency between training and inference. • No additional signaling/triggering is needed from network to start the data collection procedure at UE side. • The format/content of collected data are up to implementation of UE and no specification is needed in 3GPP Proposal 9: For training data collection at NW side for UE-assisted/LMF-based positioning with LMF-side model (Case 2b), support LPP signaling from LMF to indicate at least the following information • Configuration of Positioning RS • The type of measurement results • How to report the collected data (e.g., periodic reporting, event-triggered reporting) Proposal 11: For training data collection at NW side for NG-RAN node assisted positioning with LMF-side model (Case 3b), support the following mechanisms • gNB reports measurement results (corresponding to model input) and the associated timestamps via NRPPa protocol o The corresponding label(s) can be reported optionally or LMF generates the associated labels based on the know location of the corresponding PRU • NRPPa signaling from LMF to indicate gNB o The type of measurement results o How to report the collected data (e.g., periodic reporting, event-triggered reporting) Proposal 12: For AI-based positioning (including Case 1, Case 2a, Case 2b, Case 3a, Case 3b), Rel-19 is NOT to specify any mechanism to deliver the collected data from the entity that obtains the training data to the training entity when the training entity is not the same entity to obtain label and/or other training data.

- Intel Corporation (R1-2406015) Proposal 5: - For UE-based AI/ML positioning or UE-assisted AI/ML positioning, as part of assistance data regarding DL PRS configuration, the LMF could provide a UE with one or more configurations of measurement time windows during which the UE is expected to perform measurements towards data collection and/or report ground-truth labels. - Reuse/adaptation of the Rel-18 framework for supporting simultaneous DL positioning measurements across UEs is considered as a starting point. Proposal 6: - For UE-based AI/ML positioning or UE-assisted AI/ML positioning, the LMF could provide an NG-RAN node with one or more configurations of time windows to configure and transmit DL PRS with certain requested configuration(s) for data collection for model training. - Reuse/adaptation of the Rel-18 framework for supporting simultaneous DL positioning measurements across UEs is considered as a starting point. Proposal 7: - For UE-based AI/ML positioning or UE-assisted AI/ML positioning, the LMF could provide a UE with configuration of a measurement validity area within which the UE may perform the measurements and/or report ground-truth labels for data collection for model training. - The concept of AreaID-CellList, as described in TS 37.335, to list the NR Cell-IDs of the TRPs belonging to a particular network area within which the UE is to perform measurements and/or report ground-truth labels, can be reused. Proposal 10: - For Cases 1, 2a, 2b, assistance information to generate ground-truth labels based on estimated location coordinates for a target UE are provisioned to a UE. - Use of NR-PositionCalculationAssistance defined in TS 37.355 is considered as a starting point.

- Xiaomi (R1-2406270) Proposal 8: Leave other WGs e.g., RAN2 to discuss the report of collected data Proposal 10: For Case 2b and Case 3b, RAN1 further study whether there is any need for assistance information from UE and what kind of assistance information is needed Proposal 11: For Case 3a, leave RAN3 to assess whether there is specification impact on the collected data format definition, information exchange between TRP and gNB and information exchange between different gNBs considering different training location Proposal 12: For Case 3a, RAN1 is responsible to identify whether there is any need for assistance information from UE and what kind of assistance information is needed

- NEC (R1-2406536) Proposal 10: Study necessary signaling exchange between UE/gNB and LMF to help UE/gNB to pair Part A and Part B. Proposal 13: Support to collect UE coordination information in a zone-based format where the zone size is predefined. Proposal 14: Further study how to match a measurement and a label as a valid instance of training data or monitoring data, if the measurements and the labels are generated by different entities. Proposal 25: Specific the gNB-initiate RS configuration request to facilitate model training/retraining/fine-tuning or ground truth based model monitoring at gNB side in Rel-19, at least for the case where data transfer is not supported. Proposal 26: Specific distinct RS configuration patterns for the different stage of the LCM, e.g., employing different time domain period for data collection between model training and model monitoring.

- Fujitsu (R1-2406306) Proposal 13 For training data collection of case 1/2a, LMF can send pre-configured DL PRS configuration list to UE for further selection, or UE can send request for its preferred DL PRS configuration. Existing on-demand PRS request can be referred to support these two schemes.

- ZTE Corporation, Pengcheng Laboratory (R1-2406055) Proposal 7: For training data collection of case 1 and case 2a, new assistance data associated with training data validity area is not needed. - The current configuration information can be used for training data collection. Proposal 15: For AI/ML positioning, additional assistance information of DL PRS configuration is not necessary to be defined for UE side data collection. Proposal 16: At least for data collection with LMF-side model, support UE/TRP to report more detailed channel measurements, e.g., CIR. Proposal 18: For AI/ML positioning, additional assistance information of UL SRS configuration is not necessary to be defined for gNB side data collection.

- Lenovo (R1-2406441) Proposal 12: AI/ML assistance information may include validity conditions for which a model be valid including area validity, e.g., AreaID, zone ID, ground truth locations and/or temporal validity, e.g., validity time intervals, validity measurement windows. Proposal 13: AI/ML assistance information may also include NW and UE additional conditions, which can be used to ensure consistency of models during training and inference. Proposal 14: For Cases 1 and 2a, consider the support of on-demand PRS for deriving AI/ML model input according to the requirements of the UE-sided model. FFS whether the existing UE-initiated on-demand PRS framework may be re-used or a separate on-demand PRS for AI/ML positioning is considered.

- ETRI (R1-2406719) Proposal 4: For UE-side models managed by UE vendors, UE-initiated assistance signalling is required to facilitate data collection for training and monitoring. Proposal 5: For UE-sided models managed by networks, LMF-initiated assistance signalling is required to facilitate data collection for training and monitoring. Proposal 6: For UE-side models, either the UE or the LMF can initiate assistance signalling to facilitate data collection for inference. Proposal 7: For the gNB-side model (Case 3a), gNB-initiated assistance signaling is required to request uplink configuration, such as UL SRS configuration, for model training data collection.

- NTT DOCOMO, INC. (R1-2406921) Proposal 1: For data collection of case 2b and 3b, the measurement report and related assistance information for AI/ML based positioning is determined by LMF, - For case 2b and case 3b, for data collection of training, inference and performance monitoring, LMF initiates corresponding measurement reporting at UE and gNB respectively. Proposal 2: For data collection of case 1 and 2a, at least following assistance information is indicated from LMF to UE for consistency between training and inference. Further study the necessity of other information. - PRS configuration - Validity area (e.g., AreaID-CellList), where PRS can be assumed to be consistent within a validity areaa Proposal 3: Regarding RS configurations of data collection for each case, - For case 1/2a, UE may send requests to NW for corresponding RS configurations, or NW may send configurations to UE. - For case 2b, NW sends configurations to UE for measurements. - For case3a/3b, NW sends configurations to UE for SRS transmissions. - RS configurations may include/associate with an associated ID to implicitly indicate the NW side additional conditions.

- MediaTek Korea Inc. (R1-2407105) Proposal 3a-1: The associated ID could be provided to UE at training data generation and at inference to discriminate whether there is significant NW side implementation change Proposal 3a-2: The associated ID for positioning use case could be considered at area level. If it is at cell level, there would be a certain number of IDs sent to the UE which is not so efficient

- Ericsson Inc. (R1-2405945) Proposal 44 The training dataset validity area is provided as a part of assistance data, and recorded as a part of metadata of the training dataset. Proposal 45 For training data collection, a training dataset validity area can be defined as a list of TRPs where data is collected. Proposal 46 Training data provided to UE/gNB is organized with the same hierarchy as for legacy assistance data, i.e. per TRP/frequency layers for the UE, and per TRP for the gNB assistance data. Proposal 48 For Part A training data collection of Case 1/2a, the assistance data include: training dataset validity area, PRS assistance data (as in the existing IE NR-DL-PRS-AssistanceData), and associated ID for additional network conditions. Corresponding assistance data is sent during model inference as well. Proposal 49 The assistance data required for PRS/SRS reception is the same across all use cases for both direct and AI/ML assisted positioning.

### 1st round discussion

Based on the proposals and the evaluation results submitted, for Case 1 and 2a, majority companies suggest that assistance data should include training data validity area and DL PRS configuration. Associated ID for NW-side additional conditions is also proposed as a part of assistance data, but this can be discussed after a decision is made on associated ID.

Proposal 4.2.2-1

For training data collection of Case 1 and 2a, at least the following assistance data is provided to the UE for generating Part A:

- training data validity area, i.e., information for determining an area where the training dataset is collected.

  - FFS: details for providing the training data validity area, e.g., use the existing AreaID-CellList as a starting point

- DL PRS configuration

  - The DL PRS configuration can reuse legacy mechanisms.

|  | Company |
| --- | --- |
| Support | DOCOMO |
| Not support | ZTE |

| Company | Comments |
| --- | --- |
| ZTE | The signalling already supported in current spec. Both case 1 and case 2a are UE side model, it’s up to UE’s implementation to determine the model training procedure. |
| InterDigital | We suggest to include “FFS : Othe assistance information”, as existing assistance information such as TRP locations, boresight angles should also be provided. |
| OPPO | In our understanding, validity area can only differential the AI models applicable to different areas. However, it doesn’t consider change in the same area as the time goes on, for example, the configurations of TRPs/gNB may be changed after network optimization or some TRPs or some beams for some TRPs may be turn off. |
| Nokia | In our view, to enable consistency by DL PRS configuration (for Case 1 and 2a) already consider both assistance data suggested in the proposal. In addition, before to discuss potential discussion on „assistance data“, we suggest to finalize the discussion on „approaches to ensure consistency between UE and NW“. |
| CATT, CICTCI | OK. We also think this is a general assistence information not only for part A but also part B. |
| Apple | There are existing IEs in positioning that provide the network side information that can be considered. The mapping of the IE to an associated ID is a separate issue. |
| Ericsson | Support. At least changes in the deployment need to be synched between LMF and UE. For example, even in case 1 if a UE trained using a TRP, the assistance data must match the training, hence the need for validity area. |
| Hw/HiSi | Ok |
| CMCC | A validity area is helpful considering the generalization capability for AI positioning. |
| Qualcomm | Good to start discussing assistance for Case1 and 2a data collection assistance. Let’s not tailor this to Part A only as assistance is needed for data collection in general. -The second bullet looks reasonable. -There are other assistance data needed at UE side, including info on TRP relative time difference, gNB TX timing errors, PRS mapping to TRPs/beams. Let’s include them as well (e.g., NR-RTD-Info-r16, NR-DL-PRS-TRP-TEG-Info, NR-DL-PRS-BeamInfo-r16, nr-TRP-BeamAntennaInfo, etc.) -The training data validity area seems unnecessary. It needs more understanding on the context and underlaying assumptions. Is it intended to let LMF configure UE on where to do data collection or decide areas for which AIML can be supported? Let’s further discuss. Updated proposal: For training data collection of Case 1 and 2a, at least the following assistance data is provided to the UE for generating Part A: - DL PRS configuration - The DL PRS configuration can reuse legacy mechanisms. - Other assistance data, including , including info on TRP relative time difference, gNB TX timing errors, PRS mapping to TRPs/beams. Let’s include them as well (e.g., NR-RTD-Info-r16, NR-DL-PRS-TRP-TEG-Info, NR-DL-PRS-BeamInfo-r16, nr-TRP-BeamAntennaInfo, etc.) - The other assistance data can reuse legacy mechanisms. - FFS necessity of training data validity area, i.e., information for determining an area where the training dataset is collected. - FFS: details for providing the training data validity area, e.g., use the existing AreaID-CellList as a starting point |
| Lenovo | Generally supportive. For common understanding does the legacy DL PRS configuration include the on-demand DL-PRS and DL-PRS preconfiguration aspects ? |
| Fujitsu | OK |
| CEWiT | Similar view as CATT,CICTCI |

Proposal 4.2.2-2

For training data collection of Case 1 and 2a, in terms of DL PRS configuration for collecting Part A of training data, RAN1 study the following options (including specification impacts) on assistance data:

Option A. 	(UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE.

Option B. 	(LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE.

Note: the UE can be a PRU and/or a Non-PRU UE.

Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB.

|  | Company |
| --- | --- |
| Support | Fraunhofer, Sharp, DOCOMO |
| Not support |  |

| Company | Comments |
| --- | --- |
| InterDigital | Regarding Option A, is the starting point on-demand PRS or are we creating a new framework for request for PRS configurations? |
| Sharp | Both options should be supported. |
| Nokia | in our view, this study is not required. However, if there is something else to be clarified, we ask to FL: what is the difference between Option A and Option B with the current specified assistance data considered in LPP ?. |
| CATT, CICTCI | OK. |
| Ericsson | support. We think both options are feasible, and can use the knowledge of validity area. |
| Hw/HiSi | The legacy mechanism could be re-used, in our view there is no additional spec impact. Proposal 4.2.2-2 For training data collection of Case 1 and 2a, in terms of DL PRS configuration for collecting Part A of training data, RAN1 study the following options based on the legacy mechanisms (including specification impacts) on assistance data: Option A. (UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE. Option B. (LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE. Note: the UE can be a PRU and/or a Non-PRU UE. Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB. |
| Qualcomm | We think RAN1 should proceed with Option A with support. For Option B, we do not have to explicitly study it as it is already enabled by existing specifications. |
| CEWiT | Fine with the proposal. It should be upto UE’s implementation and capability for the selection of the Option. |

Proposal 4.2.2-3

For training data collection of Case 3a, for collecting Part A of the training data,

- gNB performs channel measurement based on existing UL SRS.

- The existing procedures are reused in terms of SRS configuration, e.g., coordination between LMF and gNB, configurations to UE for SRS transmissions.

|  | Company |
| --- | --- |
| Support | New H3C, DOCOMO |
| Not support |  |

| Company | Comments |
| --- | --- |
| InterDigital | Ok, and we should include “From RAN1 perspective” in the proposal as details related to the second bullet may be discussed in RAN3. |
| CATT, CICTCI | OK. Just one question: do we need the same first note as previous proposal?: Note: the UE can be a PRU and/or a Non-PRU UE. |
| Ericsson | Support. |
| HW/HiSi | Ok |
| Qualcomm | We are fine. Lets delete Part A as this applicable to data collection in general. |
| TCL | For Case 3a, gNB can be expected to collect Part A of the training data autonomously. So, the necessity of the coordination between LMF and gNB can be further discussed. |

## Other (e.g., model training entity, deliver collected data to model training entity)

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Qualcomm (R1-2407020) Proposal 11: For Rel-19 AI/ML positioning Case1/2a, deprioritize NW-side training. Proposal 12: For Rel-19 data collection in AI/ML positioning use case, agree on working assumptions (from RAN1#116bis) for measurement and data generation entities in Case1/2a.

- CATT, CICTCI (R1-2406354) Proposal 1: Confirm the following working assumption: Working Assumption For training data generation of AI/ML based positioning Case 1, the measurement and its related data (e.g., timestamp) are generated by PRU and/or Non-PRU UE. Proposal 2: Confirm the following working assumption: Working Assumption For training data generation of AI/ML based positioning Case 1, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Proposal 3: Confirm the following working assumptions: Working Assumption For training data generation of AI/ML based positioning Case 2a and 2b, the channel measurement and its related data (e.g., time stamp) are generated by PRU and/or non-PRU UE. Working Assumption For training data generation of AI/ML based positioning Case 2a, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Working Assumption For training data generation of AI/ML based positioning Case 2b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Proposal 5: Confirm the following working assumption: Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF

- ITL (R1-2407166) Proposal 3: For training data generation of AI/ML based positioning Case 1/2a/2b/3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF

- Ruijie Networks Co. Ltd (R1-2406594) Proposal 1: Confirm the working assumption by keeping non-PRU UE with estimated location (under the assumption that user data privacy of non-PRU UE is preserved): Working Assumption Agreement For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved..

- CMCC (R1-2405976) Proposal 3: For AI/ML based positioning, it needs more discussion on the feasibility of obtaining the ground-truth label via PRUs, in which case the training dataset size is large.

- Samsung (R1-2406638) Proposal 9. For data collection in training/finetuning, the data generation entity can be PRU/UE for case 1, TRP for case 3a, and TRP(input)/LMF(label) for case 3b. Proposal 10. For data collection in inference, the data generation entity is UE for case 1 and TRP for case 3a/3b. Proposal 11. RAN1 supports the collected data signalled from PRU (other UE) to UE deployed with AI model. Proposal 12. RAN1 supports to specify data type and size, the data type includes the measurement results and processed measurement results.

- CEWiT (R1-2407076) Proposal 3: Permissions/Privacy and positioning implementation related to non PRU UE needs to be defined and discussed. Proposal 4: Channel Measurements can be performed by both PRU and non PRU UE.

- Xiaomi (R1-2406270) Proposal 5: Confirm the following working assumption for Case 1 - For training data generation of AI/ML based positioning Case 1, the measurement and its related data (e.g., timestamp) are generated by PRU and/or Non-PRU UE. - For training data generation of AI/ML based positioning Case 1, the label and its related data (e.g., time stamp) can be generated by: • PRU • Non-PRU UE with estimated location • LMF Note: transfer of the label and its related data is out of RAN1 scope. Proposal 6: Confirm the following working assumption for Case 2a - For training data generation of AI/ML based positioning Case 2a, the channel measurement and its related data (e.g., time stamp) are generated by PRU and/or non-PRU UE. - For training data generation of AI/ML based positioning Case 2a, the label and its related data (e.g., time stamp) can be generated by: • PRU • Non-PRU UE with estimated location • LMF Note: transfer of label and its related data is out of RAN1 scope.

- NEC (R1-2406536) Proposal 7: Confirm the following working assumptions for the generating the measurement for training data of case 1, case 2a, and case 2b: − For training data generation of AI/ML based positioning Case 1, the measurement and its related data (e.g., timestamp) are generated by PRU and/or Non-PRU UE. − For training data generation of AI/ML based positioning Case 2a and 2b, the channel measurement and its related data (e.g., time stamp) are generated by PRU and/or non-PRU UE. Proposal 8: Confirm the following working assumptions reached in RAN1 #116b meeting for the generating the label for training data of case 1, case 2a, and case 2b: For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: − PRU − Non-PRU UE with estimated location − LMF Proposal 9: Confirm the following working assumptions reached in RAN1 #117 meeting for the generating the label for training data case 3b: For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: − PRU − Non-PRU UE with estimated location − LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved. Proposal 15: Endorse the integration of mixed datasets from diverse drop/clutter parameters, network synchronization errors, and scenarios to train a model with robust generalization capabilities, or for fine-tuning the model to achieve higher accuracy in the target scenario in Rel-19.

- Lenovo (R1-2406441) Proposal 18: Consider the specification of data request and data collection for the enhanced positioning accuracy use case by considering outcomes in the ongoing RAN2 study, and taking into account the following scenarios: - Scenario 1 - LMF-side Model Training: Positioning training dataset transfer is performed using existing 3GPP-signaling, e.g., LPP/NRPPa signalling - Scenario 2 - UE-side Model Training: Positioning training dataset transfer is performed without specification impact using non-3GPP technologies, e.g., proprietary signalling/OAM signalling. In this case training may be performed at the UE or on OTT/OAM side. - Scenario 3 - gNB-side Model Training: Positioning training dataset transfer is performed without specification impact using non-3GPP technologies, e.g., proprietary signalling/OAM signalling. In this case training may be performed at the gNB or on OTT/OAM side.

# Model inference

This section discusses issues for the LCM stage of model training.

## Model inference operation (input, output, configuration)

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Qualcomm (R1-2407020) Proposal 15: In AI/ML positioning inference for Case1/2a, as a starting point, the following information should be supported from LMF to UE: • Requested location info or positioning measurements (e.g., RSTD, UE rx-tx time difference, LOS state, etc,) to be obtained using AI/ML • Reporting configuration for AI/ML-enabled location and/or measurements: start with information provided as part of reporting configuration of existing methods (i.e., NR DL-TdoA report configuration, NR DL-AoD report configuration, NR multi RTT report configuration) Proposal 16: In AI/ML positioning inference for Case1/2a, as a starting point, the following information should be supported from LMF to UE: • Assistance data, including PRS resource configurations: start with information provided as part of assistance data of existing methods (e.g., UE-based DL-TdoA, UE-based DL-AoD, and UE-assisted multi-RTT)

- CATT, CICTCI (R1-2406354) Proposal 23: For case 1 and case 2a, when UE sends a data collection request to LMF, the data collection request contains some assistance information related to the PRS/TRP set expected by UE.

- TCL (R1-2406394) Proposal 9: LMF can select PRS resources of each TRP and indicate the AI-specific resources to UE for measurements.

- Sony (R1-2406464) Proposal 8: Define a set of parameters (e.g., part/all of DL-PRS configuration, received signal quality, etc) representing reference signal characteristics to be used for AI/ML positioning. Proposal 9: Associate the reference signal characteristics to the trained AI/ML model and AI/ML model inference operation.

- NVIDIA (R1-2406493) Proposal 4: For AI/ML based positioning, introduce specification support for assistance signalling and procedure for model configuration, model activation/deactivation, model recovery/termination, and model selection.

- Apple (R1-2406827) Proposal 16: For model inference, the following elements shall be specified as part of the model inference procedure: - Part A - Measurement (corresponding to model input), - Time stamp, - Assistance Information - RS configuration(s), - other necessary information (e.g., scenario identifier. LOS/NLOS condition, timing error, etc.) Proposal 18: For data collection, the key specification impacts are in the transfer of the measurement data in case 2b (UE to LMF) and case 3b (gNB to LMF) for model inference and model monitoring. - This requires an update to support CIR, PDP and DP measurement data. - Additional specification is required to transfer ground truth labels, quality indicators and other necessary information from the measurement entity to the monitoring entity.

- Ericsson Inc. (R1-2405945) Proposal 62 For Case 2a/3a, support UE/gNB to send an indication to the LMF when the AI/ML model is ready for reporting measurements based on model inference. It is up to RAN2/3 to decide which procedure to use to convey the information.

## Ensure consistency between training and inference

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Qualcomm (R1-2407020) Proposal 2: For AI/ML positioning, to ensure consistency between training and inference in Case1/2a, LMF provides UE with assistance data: - For assistance data supported in existing IEs, at least existing assistance data should be provided from LMF to UE. - FFS: Other assistance data not defined in existing IEs NOTE: R1-2407020 also propose to integrate AIML methods as part of existing methods, which implies that existing AD is re-used.

- Huawei, HiSilicon (R1-2406978) Proposal 12: For Case 1 and 2a, the legacy assistance information (regarding area, PRS, data quality, time stamp, error, antenna, TRP, etc.) can be reused by the UE for data categorization and ensuring the consistency between training and inference.

- InterDigital, Inc. (R1-2406206) Proposal 4: At least frequency layer IDs and cell IDs should be the same between training and inference to achieve in configuration and area validity, respectively Proposal 5: Time validity for an AIML model(s) should be specified Proposal 6: Tolerance for difference in assistance information (e.g., network synchronization error, difference in boresight angles) should be specified Proposal 7: RAN1 agrees on the following categorization: “Network conditions” can be found in TS 37.355 while “Network-side additional conditions” are implementation dependent and not specified; “Network conditions” should include at least NR-RTD-Info, NR-DL-PRS-BeamInfo, NR-TRP-LocationInfo.

- CATT, CICTCI (R1-2406354) Proposal 20: For channel measurement reporting in case 2b/3b, the handling of time offset need to be consistent between training and inference phase. Proposal 24: For case 2b, LMF sends the validity area (e.g. some TRP sets) of the collected training data to provide assistance information for UE channel measurement reporting. Proposal 29: For AI/ML-based positioning, both the following concepts of validity area for training data collection can be considered: • Assistance data or measurement validity area configured by LMF. • Validity area (i.e. multiple TRP sets) of collected training data recorded by UE/LMF. Proposal 30: For case 1 and case 2a, if NW-side additional condition is identified as necessary for positioning, the following options can be considered to ensure the consistency between training and inference regarding NW-side additional conditions for inference at UE-side model. - Option 1: Based on associated ID. - FFS on what can be assumed by UE with the same associated ID across training and inference. - Option 2: Based on performance/model monitoring.

- TCL (R1-2406394) Proposal 12: AI-specific or AI model-specific reference signal configurations, including PRS and SRS for positioning, could be introduced and indicated to UE, enabling it to implement the specified model or distinguish the transmitting method of the configured reference signal. Proposal 13: The legacy assistance data provided by LMF, such as TRP information, PRS assistance data and on-demand PRS configurations, can serve as the NW-side additional conditions for UE-side model. Proposal 14: UE can provide cell information and on-demand PRS configurations to LMF to ensure consistency between training and inference for UE-side model. Proposal 15: LMF can indicate the PRS-RSRP threshold or SRS-RSRP threshold to ensure consistency between training and inference for LMF-side model. Proposal 16: UE can provide PRS-RSRP to LMF or gNB can provide SRS-RSRP to LMF along with the measurements for model input to ensure consistency between training and inference for LMF-side model. Proposal 17: The associated ID can be adopted for UE-side model in the use case of AI/ML based positioning for consistency between training and inference. FFS: the format of associated ID(s) for AI/ML based positioning.

- CMCC (R1-2405976) Proposal 10: For AI/ML based positioning, further discuss option 3 and 4. Deprioritize option 1 and 2. Proposal 11: For AI/ML based positioning, further discuss what kinds of RS configurations needs to keep consistency between training and inference. Note: Methods to ensure consistency between training and inference for UE-side model should also be discussed. During the SI, the following options were identified. - 1) Model identification to achieve alignment on the NW-side additional condition between NW-side and UE-side - 2) Model training at NW and transfer to UE, where the model has been trained under the additional condition - 3) Information and/or indication on NW-side additional conditions is provided to UE - 4) Consistency assisted by monitoring (by UE and/or NW, depends on the performance of UE-side candidate models/functionalities to select a model/functionality).

- Samsung (R1-2406638) Proposal 22. Consistence check should be done when/before the entity deciding which model to use. Proposal 23. RAN1 to study the features or characteristics of the training data to be used for consistence check, e.g., the aspects related to generate the data input etc.

- Sony (R1-2406464) Proposal 10: Further study the signalling procedure to ensure the consistency between AI/ML training and AI/ML inference. Proposal 13: UE or gNB to provide an indication of AI/ML model validity to the AI/ML server/management (e.g., LMF).

- CEWiT (R1-2407076) Proposal 7: For all the use cases (Case 1, Case2a, Case 2b, Case 3a, Case 3b), RS configuration and Area validity from which training data is collected should be provided as assistance information to the data collecting entity. Proposal 8: Depending on the deployment scenario, network should provide a range/margin for quality-indicating metrics (e.g., SNR, channel estimation error, etc., for channel measurement), UE's available storage/ UE capability (if the UE is collecting data), expected LOS/NLOS assistance and the type of input given to the model for any required preprocessing of the dataset as assistance information.. Proposal 9: Support use of IE AreaID-CellList for defining the validity of an area for data collection.

- OPPO (R1-2406255) Proposal 5: For AI/ML based positioning of Case 1 and Case 2a, some indication (e.g., in form of associated ID) is signaled from network to ensure the consistency of AI model training and AI model inference (e.g., consistency on NW-sided conditions / additional conditions) e.g., the ID can be a special ID for positioning configurations, and can be indicated to differentiate the associated training data The proprietary information of network should not be disclosed. Proposal 6: For AI/ML based positioning of Case 1 and Case 2a, the combination of the same associated ID, the same cell(s) and the same PRS configuration implicitly the consistency between training and inference (e.g., consistency on NW-sided conditions / additional conditions) • FFS: additional information is needed for the above combination (e.g., the same cell(s), or the same area). Proposal 13: For the model inference for UE-based positioning with UE-side model (Case 1), • The associated ID is signaled from network to ensure the consistency of AI model training and AI model inference • No need to specify the format/contents of AI model input since they are up to UE implementation and transparent from the perspective of air interface • UE can report the estimated location information via existing LPP signaling o Introduce new information to indicate the reported results are generated by legacy method or AI-based method • FFS: whether the current field about the uncertainty can be reused or some new field/IE should be introduced to report the associated quality/probability/confidence of AI model estimated location Proposal 16: For the model inference for NG-RAN node assisted positioning with gNB-side model (Case 3a), • No need to specify the format/contents of AI model inputs since they are up to gNB/TRP implementation and transparent from the perspective of air interface • gNB can report the measurement results that are based on AI model output via existing NRPPa signaling o Introduce new information to indicate the reported results are generated by legacy method or AI-based method • FFS: whether to reuse the existing quality (e.g., Measurement Quality) or introduce a new field/IE to reflect the quality/confidence/probability of reported measurement results that are based on AI model output

- Intel Corporation (R1-2406015) Proposal 26: - To ensure consistency between model training and inference, the measurement validity area configuration that may be provided for model training is assumed to apply also for model inference. Proposal 27: - Consider the consistency between RS configurations as a NW-side additional condition to ensure consistency between training and inference. - FFS details of conditions of consistency between RS configurations Proposal 28: - Consider the consistency between channel/link conditions/characteristics as a NW-side additional condition to ensure consistency between training and inference. - FFS details of conditions of consistency between channel/link conditions/characteristics Proposal 29: - Consider the consistency between model input formats as a NW-side additional condition to ensure consistency between training and inference.

- vivo (R1-2406173) Proposal 20: Consider adopting the following approaches to ensure the consistency between training and inference. - For beam pattern information, NW side provides indication to assist UE in maintaining the consistency between training and inference, such as “associated ID” information or explicit beam pattern information as specified in legacy UE-based AoD positioning. o UE can indicate applicable “associated ID” or beam pattern in other procedures. Proposal 21: Further study how to connect multiple TRPs or cells with associated ID for AI/ML based positioning, and connecting an associated ID with a cell/TRP list could be considered. Proposal 22: Consider adopting the following approaches to ensure the consistency between training and inference. - UE can report supported PRS bandwidth. Proposal 23: Further study how to deal with the potential inconsistency of measurement implementation between regular UE and PRU when supporting PRU to generate data for model training and monitoring.

- Xiaomi (R1-2406270) Proposal 23: Support additional condition alignment is beneficial for both model-based LCM and functionality-based LCM in AI-based positioning use case Proposal 24: RAN1 further investigates the necessity and feasibility for the following potential network additional conditions - Scenario/site information - Measurement configuration - Network synchronization error - Antenna information Proposal 25: Support indication of the network additional condition and unified indication mechanism should be strived for in all cases Proposal 26: For the indication of additional condition, virtual category and/virtual ID can be defined in specification while how to associate with the virtual category and/or virtual ID is up to implementation Proposal 27: Consider solutions to keep consistency regarding UE additional condition for network side model

- NEC (R1-2406536) Proposal 27: An validity area can be introduced to ensure the consistency between training data and inference data. Proposal 28: The legacy AreaID-CellList specified in TS37.355 can be reused as the start point to specify the validity area. Proposal 29: For case 1/2a, following three alternatives can be used as starting point to ensure the consistency between training and inference: − Alt.1: UE requests the expected metadata from LMF. UE can provide the expected metadata to LMF for UE side inference data collection based on the metadata for training data collection such that consistency between training and inference will be ensured. − Alt.2: LMF maintains the association between metadata and UE/model. LMF can ensure consistency between training and inference based on the maintained association. − Alt.3: UE maintains the association between metadata and model. UE firstly determine whether consistency is ensured and perform model inference accordingly.

- Fujitsu (R1-2406306) Proposal 9 For AI/ML positioning Case 1 and 2a, validity area and DL PRS configuration can be reported as either metadata of supported functionality or additional condition of applicable functionality, depending on the granularity of functionalities. Proposal 10 For AI/ML positioning consistency between training and inference, the information exchange can be based on signaling of NW-side conditions/configuration (option 2), and existing information defined in current TS37.355 Assistance Data can be discussed in priority. Proposal 11 In addition to NW-side additional condition/configuration, performance monitoring should be considered as an alternative or supplementary option to achieve AI/ML positioning functionality/model consistency between training and inference. Proposal 12 For training data collection of case 1/2a, existing information of AreaID-CellList defined in TS37.355 can be used to carry the validity area information.

- Apple (R1-2406827) Proposal 9: AI/ML model assistance data - Data that be provided to the AI/ML model at the UE/gNB/LMF before or during an ongoing AI/ML-based positioning session, to be then utilized for potential AI/ML-based positioning by the AI/ML model at a future time - The assistance information can be used to match the UE/network conditions between training data collection, inference and monitoring and may be unicast or broadcast to multiple entities. Proposal 10: For Rel-19 AI/ML positioning, assistance data be signaled as assistance information or configured as part of the procedure. Proposal 11: The information on additional network-side conditions may be sent as The information assistance data may be sent for any one of the LCM stages. They may be: - LCM procedure specific e.g. for data collection for training only, monitoring only, inference only - For use across LCM procedures e.g. to ensure consistency between model training and model inference Proposal 12: For Rel-19 AI/ML positioning, the assistance data be signaled as assistance information or configured as part of the procedure. These conditions can include one or more of the following: - Validity Area Conditions: - Validity/ Geographical area information /Cell-list /scenario/ site information, TRP set - Configuration Conditions: - Reference signal configuration(s) used to generate training data - Data Quality Conditions - Measurement data quality range (e.g. SNR/SINR range), - label data quality range (e.g. mean label positioning error) - Time range when data generated - Hardware Conditions - Network Synchronization Error - Rx-Tx timing error - Phase offset error - Antenna/beam pattern information

- ZTE Corporation, Pengcheng Laboratory (R1-2406055) Proposal 23: There is no need to specify any NW-side additional conditions for AI/ML positioning.

- NTT DOCOMO, INC. (R1-2406921) Proposal 9 For AI/ML based positioning, to ensure consistency between training and inference, as well as to guarantee the consistency of PRS, following impacts can be considered: - An associated ID can be configured within/ together with PRS related configuration - Validity area is configured/indicated, where PRS can be assumed to be consistent within a validity area

- Fraunhofer IIS, Fraunhofer HHI (R1-2406711) Proposal 7: For UE-sided models, support the following assistance information from the NW, to ensure consistency assisted by monitoring between training and inference: - NW indication on a temporal, periodic or permanent change in the radio environment. - NW-sided (LMF) label-based monitoring performance metrics. Proposal 8: For gNB-/LMF-sided models, support the following assistance information from the UE, to ensure consistency assisted by monitoring between training and inference: - UE sensor information, like velocity, orientation or relative displacement. - UE-sided GTL generation for monitoring, e.g., using side-link positioning, landmarks or GNSS. - Information on proximity to landmarks. - A CIR/PDP score indicating how well the report captures the actual channel characteristics.

- Nokia (R1-2406587) Proposal 42: RAN 1 to consider the following approaches to ensure consistency between training and inference regarding NW-side additional conditions: consistency based on PRS configuration along with Associated ID and consistency assisted by monitoring. Proposal 43: For UE-side models, to ensure consistency between training and inference related to NW-additional conditions, a PRS configuration configured for data collection (training) or inference shall be associated with a global cell identity (GCI) and a network-identifier (associated ID). FFS: details of network identifier (associated ID). Proposal 44: For UE-side models, to ensure consistency between training and inference related to NW-additional conditions, RAN1 to study the possibility defining a context scenario (e.g., channel characteristics, TRP patterns) which represent NW-additional conditions as high level information. Proposal 45: To ensure consistency between training and inference, UE can indicate the timeframe of the dataset it used for training, upon which the NW compares the PRS configuration and Associated ID between the indicated timeframe and the upcoming inference time, and informs UE about any changes in the PRS configuration and Associated ID. Proposal 46: To RAN1 further discuss the following options to ensure consistency between training and inference regarding NW-side additional conditions: Option 1 (UE-sided model assessment), Option 2 (UE-sided functionality assessment), and Option 3 (NW-side functionality assessment with UE-assistance).

- Ericsson Inc. (R1-2405945) Proposal 60 When preparing for model inference, the model inference validity area is checked for compatibility with the deployment area. Proposal 61 When preparing for model inference, configurations of reference signal transmission and measurement are checked for compatibility between the trained model and the deployment environment. Proposal 63 For Case 1, to ensure the consistency of NW-side additional condition (e.g., beam configuration of DL PRS) across training and inference, study the following options: • Option 1: based on associated ID of NW conditions/configurations. • Option 2: based on performance monitoring. Proposal 64 For AI/ML based positioning, the consistency between training and inference is checked for each TRP separately. Proposal 65 For a given TRP j, the UE assumes that the NW-side additional conditions with the same associated ID are consistent within the TRP j.

### 1st round discussion

Regarding associated ID for NW-side additional condition, the following agreement was made in RAN1#116bis for beam management (AI 9.1.1):

Agreement Further study, for the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, where the NW-side additional condition may at least impact UE assumption on beams of Set A/Set B: - Opt1: Based on associated ID (Referring to AI 9.1.3.3) - FFS on what can be assumed by UE with the same associated ID across training and inference - FFS on how associated ID is introduced, e.g., within CSI framework, or outside of CSI framework - Opt 2: Performance monitoring based - FFS details - Other options are not precluded.

In RAN1#117, the following working assumption was made under agenda item 9.1.3.3.

Working Assumption (RAN1#117) Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study)

For positioning, majority companies also support to introduce associated ID for NW-side additional condition.

Proposal 5.2.2-1

For AI/ML based positioning Case 1 and 2a, associated ID is introduced to provide NW-side additional condition to ensure the consistency between training and inference.

- FFS: whether the NW-side additional conditions with the same associated ID are consistent at least within a cell, or a TRP, or an area.

|  | Company |
| --- | --- |
| Support | Fraunhofer, DOCOMO, TCL, |
| Not support | ZTE, HW/HiSi |

| Company | Comments |
| --- | --- |
| ZTE | Form our perspective, positioning is different from beam management. For pos, UE is aware of all configuration details and required model output, it’s not necessary to follow beam’s agreement. |
| InterDigital | From the proposal, it is not clear what an associated ID indicates. Before we introduce the ID, we should clarify what the ID points to. |
| OPPO | The principle is fine. The next step is to discuss the relationship with proposal 5.2.2-2. |
| CATT, CICTCI | Support. |
| Apple | One suggestion is to map the associated ID/assoicated IE to a specific set of existing Ies. |
| Ericsson | Support. In our view, we need to find a way to efficiently signal changes in the deployment between training and inference. |
| HW/HiSi | Agree with ZTE and also we should not discuss this at this stage, we should first get a common understanding which parameters are needed to ensure the consistency between training and inference. For example for validity area and PRS/SRS configuration can already be signaled with legacy Ies, other parameters can also already be sigballed or handled by generalization. Then, an associated ID would not be needed. |
| Qualcomm | Agree to ZTE and HW/HiSi. The problem of AIML positioning is different from AIML BM. The context and setup are different. AIML BM only looks into single gNB/TRP and predicts L1-RSRP (including other potential outputs). In positioning, the UE interacts with a number of gNBs/TRPs and context of measurements are different. UE needs to find timing measurements and/or compute location info and it can be highly affected by TRPs relative time difference, TX timing errors, and other NW settings, which are already explicitly provided in existing specifications (e.g., existing IEs: NR-TRP-LocationInfo-r16, NR-RTD-Info-r16, NR-DL-PRS-TRP-TEG-Info,etc.) Information in existing IEs should be the starting point for the discussion. We think associated ID framework can be further studied for information not supported in existing specifications (up to Rel18). The framework of associated ID can be used if no such information is already supported in specs. |
| Lenovo | We would prefer to start with a discussion on the definition of NW-side additional conditions from a positioning perspective. Whether an Associated ID is needed or not would then depend on the definition of NW-side additional conditions. Similar view to InterDigital. |
| Fujitsu | The associated ID is assumed as a cell-level ID in 9.1.3.3, while beyond-cell-level consistence would be needed for positioning. So, whether and how to use the associated ID in PO need to be further studied. |

Proposal 5.2.2-2

For AI/ML based positioning Case 1 and 2a, to ensure the consistency between training and inference, at least the following are checked for consistency:

- Validity Area

- PRS configurations

- Measurement configuration, e.g., sample-based or path-based measurement, PDP or DP, measurement parameters (e.g., {Nt, Nt', k} for sample-based measurement)

- Measurement data quality range, e.g. SNR/SINR range

- Hardware Conditions, e.g., network synchronization error, UE/gNB RX and TX timing error, channel estimation error.

|  | Company |
| --- | --- |
| Support | TCL |
| Not support |  |

| Company | Comments |
| --- | --- |
| ZTE | This can be discussed after Proposal. 5.2.2-1. |
| InterDigital | Agree with (a). Regarding (b), FFS “whether all PRS configurations need to match between training and inference” since it is not clear whether the configurations should be the same at all levels in the PRS configuration hierarchy. We disagree with (c) since we have not made a conclusion yet. It’s not clear whether (d) is needed on top of what we have in specification (e.g., power measurements). Regarding (e), is it part of “additional NW side conditions”? |
| CATT, CICTCI | Y for (a), (b), FFS on (c), (d), (e). It is unclear whether (d) and (e) are controllabel. For case 1 and 2a, it seems (c) is determined by UE itself, no need for spec support to ensure. |
| Ericsson | OK |
| HW/HiSi | What does it mean “checked for consistency”? Is it meant to be specified or for further discussion whether these factors impact consistency? Regarding the measurement configuration, it is pre-mature to add the examples since corresponding agreements have not been made yet, Also for the measurement data quality range, the example could be deleted. Our understanding is that instead the RSRP could be used. This is also a separate discussion. Same for hardware conditions, since they could be overcome with mixed training data sets. |
| DOCOMO | At least (a) and (b) should be included. |
| CMCC | The listed conditions may need further check. Some of them may be a ‘hard‘ condition since if this condition is not satisfied then the AI can not work, but some of them may not. If some of the conditions are not satisfied, AI can still be used and model monitoring can prevent the perforemance. |
| Qualcomm | For (a), (c), and (d), these look like implementation at UE side. For (b) and (e), they make sense to be provided (as they are NW-side info and UE may not be aware of their changes). It should also be worded as (information provided) instead of (checked) |
| Lenovo | Fine with at least a) and b) . |

# Model monitoring

This section discusses issues for the LCM stage of model performance monitoring.

## Model monitoring methods

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- OPPO (R1-2406255) Proposal 18: For functionality/model performance monitoring, Rel-19 is NOT to specify dedicated specification enhancement for the mechanism without ground-truth labels • Functionality/model performance monitoring without ground-truth labels can be done by implementation without any specification enhancement • Note: we also categorize some mechanisms based on “the approximate ground-truth label” in to that without ground-truth labels.

- Intel Corporation (R1-2406015) Proposal 20: - Support both input-based and output-based functionality/model monitoring for AI/ML based positioning.

- vivo (R1-2406173) Proposal 17: For performance monitoring of LMF-side model, the following assistance information can be provided for monitoring: - SNR/SINR provided by UE (Case 2b) or gNB (Case 3b) - Relative distance information based on UE’s assessment e.g., from its own sensors (Case 2b and 3b) Proposal 18: Support both label-based and label-free monitoring for AI/ML based positioning.

- Xiaomi (R1-2406270) Proposal 33: Confirm the necessity of assessment/monitoring of inactive models / functionalities, with the following assumptions as the starting point: - One way to monitor inactive models/functionalities is by activating them and reusing mechanisms defined for monitoring of active models/functionalities. - The following aspects may be considered for further study or in WI to assess the applicability and expected performance of an inactive model/functionality: - Configuring an AI/ML model for monitoring without activation (e.g., monitoring-only mode without reporting predicted beams in BM Case 1 and 2) - Dataset delivery from the network to the UE for assessment/monitoring of the applicability and expected performance of the model/functionality. - NW may provide performance criteria/preference for UE’s model selection. - Other aspects are not precluded for further study or specification.

- Apple (R1-2406827) Proposal 17: For model monitoring, the following elements shall be specified as part of the monitoring procedure: - Part A: - Measurement (corresponding to model input), - Time stamp, - Quality indicator (for and/or associated with ground truth label and/or measurement at least for model monitoring), - Part B - Ground truth label, - Quality indicator (for and/or associated with ground truth label and/or measurement at least for model training), - Time Stamp - Assistance Information - RS configuration(s), - other necessary information (e.g., scenario identifier. LOS/NLOS condition, timing error, etc.). Proposal 40: Summary of specification enhancements for monitoring: - Model monitoring based on provided ground truth label (or its approximation) - Trigger for bursty RS to enable statistics collection (all cases) - If configured and permission is granted, LMF may provide GT information to UE (case 1, case 2a). - Report from LMF to UE of estimated position (or GT from estimated position) for comparison with model output (case 2a). - Model monitoring using at least statistics of measurement(s) without ground truth label - There may be an impact if the statistics block is in a separate physical entity from monitoring comparison/decision block (all cases). - Trigger for bursty RS to enable statistics collection (all cases).

- NTT DOCOMO, INC. (R1-2406921) Proposal 12: For performance monitoring of AI/ML based positioning, at least support LMF as the entity which makes decision of upcoming operations (e.g., functionality activation/deactivation/update/switching, fallback operation). - For case 1/2a and case 3a, the inference output information or the calculated performance metric should be reported to LMF by UE and gNB respectively. Proposal 13: Regarding decision making of the performance monitoring for AI/ML based positioning, for case 1 and 2a, NW indicates UE to trigger corresponding procedures (e.g., activation/deactivation, switching, update, fallback).

- Fraunhofer IIS, Fraunhofer HHI (R1-2406711) Proposal 9: Consider a sequential monitoring process: upon identifying of a potential issue through early metrics, activate a broader set of monitoring metrics to confirm the presence of the issue. Proposal 10: For functionality/model monitoring, support diverse monitoring configurations encompassing: - Offering options for event-based or scheduled monitoring in different time intervals, with the flexibility to adjust the monitoring frequency. - Providing customization for reporting detail, notifying monitoring entity on the level of detail in monitoring reports. - Supporting different methods of generating ground truth labels for monitoring purposes, with varying quality of labels.

- Nokia (R1-2406587) Proposal 16: Performance metric based on ground truth label (or its approximation) is defined as the statistical difference (e.g., MSE) between ground truth label and inference output, where measurements associated with ground truth information can be used as inference input. Proposal 17: Necessary data for performance monitoring with ground truth label (or its approximation) consists of at least: i) ground truth (or its approximation), e.g., collected from PRU; ii) measurements corresponding to inference input, e.g., measurements collected from PRU; and iii) inference output, e.g., UE location estimation. Proposal 18: For UE/gNB-side models, label-free monitoring may consist of: • confidence level associated with model inference output, e.g., UE location estimate, • statistics of measurement(s) compared to the statistics of measurements in the training data set, • standard deviation as monitoring metric based on the historical/buffered/successive inference output. Proposal 22: RAN1 to study relevant aspects as entities and data content of label-free monitoring at least for first priority cases. Proposal 29: For label-free monitoring of UE-side models, for UE to derive monitoring metric, LMF may provide UE necessary data and configuration for monitoring. Proposal 30: For label-free monitoring of UE-side models, for LMF to derive monitoring metric, UE may provide LMF necessary data for monitoring, which may contain: • statistics of UE measurements (e.g., RSRPP), • standard deviation of UE inference output (e.g., UE location estimation in Case 1).

- Ericsson Inc. (R1-2405945) Observation 48 Label-free model performance monitoring is up to implementation of model inference entity. The only potential specification impact is to notify other entities of the model monitoring decision, if necessary. Observation 49 If certain information on the ground truth label can be generated by the model inference entity, then such label-based model performance monitoring is also self-contained and up to implementation of model inference entity. The only potential specification impact is to notify other entities of the model monitoring decision, if necessary. Proposal 50 Regardless of model performance monitoring methods, a same entity performs both monitoring metric calculation and monitoring decision making. Proposal 51 Introduce signaling to provide information on ground truth label only when necessary. Proposal 67 For both direct and assisted AI/ML positioning, the model inference entity is the core entity that performs continuous model monitoring itself, including calculation of model monitoring metrics. Proposal 68 For model inference at UE (Case 1, 2a) or gNB (Case 3a), the LMF can provide assistance information to support the calculation of model monitoring metrics. Proposal 69 LMF interacts with gNB and UE to make decisions on whether to activate or deactivate AI/ML positioning, and which AI/ML positioning method to use.

### 1st round discussion

For decision making of model performance monitoring, companies have different views. For example:

- NTT DOCOMO, INC. (R1-2406921):

  - Proposal 12: For performance monitoring of AI/ML based positioning, at least support LMF as the entity which makes decision of upcoming operations (e.g., functionality activation/deactivation/update/switching, fallback operation).

- Ericsson Inc. (R1-2405945):

  - Proposal 50	Regardless of model performance monitoring methods, a same entity performs both monitoring metric calculation and monitoring decision making.

  - Proposal 69	LMF interacts with gNB and UE to make decisions on whether to activate or deactivate AI/ML positioning, and which AI/ML positioning method to use.

- Huawei, HiSilicon (R1-2406978)

  - Proposal 15: For model monitoring in Case 1, conclude that Option A-4 is sufficient where the monitoring decision is performed in the same UE-side entity that derives the monitoring metric.

  - Proposal 16: For model monitoring in Case 3a, recommend to support Option A where the gNB calculates the metric and makes the monitoring decision itself.

- Xiaomi (R1-2406270)

  - Proposal 28: For performance monitoring in Case 1, -	The performance metric calculation can be performed on UE side; -	The LCM decision making can be performed by UE side

It seems easier to discuss monitoring decision making for each case or scenario individually. See proposals in section 6.2 and 6.3.

Regarding label-free model performance monitoring, several companies (e.g., OPPO, vivo, Nokia, Ericsson) point that this should be supported. Also label-free model performance monitoring can be done by implementation in the model inference entity, with no or small specification impact. OPPO and Ericsson point out that the same procedure applies to label-based monitoring methods where “the approximate ground-truth label” is generated by the model inference entity without receiving information on ground truth label from other entities. These can be called self-monitoring methods, i.e., implementation-based, without receiving information on ground truth label from other entities. For reference, evaluations of label-free model performance monitoring were carried out in Rel-18 study item phase.

Regarding decision making of self-monitoring methods,

- For models hosted by gNB or the target UE, the gNB or UE may make decision/recommendation regarding model activation, deactivation, update, fallback to legacy positioning methods. Further coordination with LMF may be necessary since LMF has high-level control on positioning methods and configuration.

- For models hosted by LMF, the LMF can directly make model management decision based on its own monitoring metric calculation.

A relevant agreement was made in RAN1#116 for LMF-side model (Case 2b, 3b), where many companies have the view that LMF can perform self-monitoring.

Agreement (RAN1#116) For LMF-side model, RAN1 studies whether/what assistance information and/or measurement report may be sent from UE/PRU, and/or gNB to LMF to assist at least for the performance monitoring. - RAN1 understands that it is out of RAN1 scope to define monitoring metric calculation and related model management decisions for LMF-side model.

Proposal 6.1.2-1

For model performance monitoring of AI/ML positioning, self-monitoring methods are supported, where:

- the model inference entity performs monitoring metric calculation for its own model without receiving information on ground truth label from other entities.

- the model inference entity also makes model management decision/recommendation (e.g., model deactivation, update, fallback) based on monitoring metric for its own model.

The specification impact of self-monitoring:

- For Case 1 and 2a, the target UE notifies the LMF of its model management decision/recommendation as a result of model performance monitoring.

- For Case 3a, gNB notifies the LMF of its model management decision/recommendation as a result of model performance monitoring.

- Note: LMF makes the decision on whether to activate or deactivate AI/ML positioning, and which AI/ML positioning method/case or legacy positioning method to use.

Note: for self-monitoring methods, it is up to implementation how to perform the monitoring metric calculation, and how to make model management decision using the calculated monitoring metric.

Note: self-monitoring methods include:

- Label-free methods;

- Label-based monitoring methods where “the approximate ground-truth label” is generated by the model inference entity without receiving information on ground truth label from other entities.

|  | Company |
| --- | --- |
| Support | Fraunhofer |
| Not support |  |

| Company | Comments |
| --- | --- |
| InterDigital | For Case 1, we don’t understand this proposal with relation to the agreement made in RAN1#116b. Which option (A or B or variants of A or B) are we discussing here? |
| CATT, CICTCI | OK. |
| Apple | Clarify that the note means that the switch between case 1,…, case n and non-AI/ML positioning is made by the LMF. However, within each case, the model inference entity can make its own decisions with one or more models. |
| Ericsson | Support. For the specification impact part, we think there are already existing procedure for measurement failure in higher layers, so we should ask RAN2/3 how to adapt / reuse these. |
| HW/HiSi | Not sure whether the first part of the proposal is needed. In any case, it is not clear what the term self-monitoring means or if it is really needed. These procedures have not been discussed yet and needs to more time. |
| Qualcomm | We are partially ok with the direction of the proposal. We think RAN1 should also focus on the assistance needed to allow inference entities handle monitoring for their AIML positioning. For label-based monitoring, the UE still needs assistance from LMF to obtain labels (e.g., position calculation assistance info). Although, if it needs to be called (self-monitoring), this does not mean there is no assistance between entities. |
| Lenovo | In principle the option to do self-monitoring should be supported since it offers flexibility for the inference entity to make the model management decisions. The one aspect of the first note: “for self-monitoring methods, it is up to implementation how to perform the monitoring metric calculation, and how to make model management decision using the calculated monitoring metric” . It seems that the result of the model management needs to be aligned with another entity such as the LMF, e.g., Case 1 or 2a. Would the result of the model management still be up to implementation? |
| Fujitsu | OK with this direction in general. |
| CEWiT | For Case 3a, how gNB will calculate the monitoring metric based on ground truth labels without getting information from LMF or any other entity needs to be discussed. |

## Model monitoring for Case 1

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Qualcomm (R1-2407020) Proposal 17: In AI/ML positioning monitoring for Case1/2a, support the following assistance options (in RAN1#116bis agreement) for label-based monitoring metric calculation: • Option A-1 • Option A-2 • Option B-1 • FFS Option A-3 and A-4(while considering specifications-based PRU data transfer, e.g., using SL) Proposal 18: In AI/ML positioning monitoring for Case1/2a, support UE-initiated monitoring with the following assistance from LMF to UE: • RS resources configurations/activations related to monitoring • Monitoring assistance information (as discussed in Option A monitoring metric calculation in RAN1#116bis agreement) o ground truth label o position calculation assistance data.

- Huawei, HiSilicon (R1-2406978) Proposal 13: Label-based model monitoring for Case 1 should be based on PRU channel measurements and their known locations. Proposal 14: For Case 1, there is no need to involve the LMF for metric calculation, as the LMF may not have the knowledge to monitor/manage the UE-side models. Proposal 15: For model monitoring in Case 1, conclude that Option A-4 is sufficient where the monitoring decision is performed in the same UE-side entity that derives the monitoring metric. • The monitoring is done in implementation manner and a monitoring metric does not need to be specified. • There is no need to restrict the type of measurements used as model input.

- InterDigital, Inc. (R1-2406206) Proposal 17: For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, support Option A-1, A-2 and A-3. Proposal 18: For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, do not support Option A-4 Proposal 19: Prioritize specification of variants of Option A over specification of Option B.

- CATT, CICTCI (R1-2406354) Proposal 26: For case 1 and case 2a, for model performance monitoring metric calculation in label-based model monitoring, at least the following options for generating ground truth label can be considered. - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF.

- TCL (R1-2406394) Proposal 2: The monitoring metrics for UE-side model include error of location, error of speed or acceleration, error of measurement value, F1-score of LOS/NLOS indicator, standard deviation or variance of location and standard deviation or variance of measurement value.

- Sony (R1-2406464) Proposal 11: For case 1, support Option A and the LMF sends criteria (e.g., parameters for performance evaluation, thresholds) to the UE performing performance metrics (PM) computation.

- Sharp (R1-2406970) Proposal 12: RAN1 to further primally discuss benefits and potential specification impact for Option A-3, A-4 and B-2.

- OPPO (R1-2406255) Proposal 20: For UE-based positioning with UE-side model (Case 1), Rel-19 is NOT to specify dedicated specification enhancement for functionality/model performance monitoring • Functionality/model performance monitoring can be done by UE implementation without impact on air interface • The feasibility of Option A-1, A-3, A-4, B-2 has not been justified • For Option A-2, it is not clear what the solution it is. • For Option B-1, there is no additional spec impact and it is not clear how LMF use this information for monitoring metric calculation

- Intel Corporation (R1-2406015) Proposal 23: - For AI/ML-assisted positioning Case 1, for label-based model monitoring at LMF, Options A-1 and A-2 should be considered further. - For Option A-1, ground-truth labels (or their approximations) are determined by the LMF based on legacy measurement metric(s) for RAT-dependent and/or RAT-independent positioning methods to LMF by target UE/NG-RAN node. - For Option A-2, ground-truth labels (or their approximations) are determined by the target UE based on legacy assistance information received from the LMF. - Option A-3 may be considered as a supplementary method in addition to one or both of Options A-1 and A-2. Proposal 24: - For AI/ML-assisted positioning Case 1, for label-based model monitoring at LMF, Option B-1 should be considered further. - Estimated coordinates from AI/ML model at UE side are reported to the LMF by the target UE. - Ground-truth labels (or their approximations) are determined by the LMF based on known coordinates of PRUs or estimated coordinates for non-PRU UEs.

- vivo (R1-2406173) Proposal 13: For model performance monitoring of AI/ML positioning Case 1, the target UE side performs monitoring metric calculation should be supported. Proposal 14: For model performance monitoring of AI/ML positioning Case 1, option A-1 and option A-3 can be supported for the target UE side performs monitoring metric calculation.

- Xiaomi (R1-2406270) Proposal 28: For performance monitoring in Case 1 - The performance metric calculation can be performed on UE side - The LCM decision making can be performed by UE side Proposal 29: Further study the following two options for the case that UE make the LCM decision in Case 1 - Option 1: UE make the LCM decision based on UE’s implementation and then report the LCM decision to LMF - Option 2: LMF provide a performance monitoring metric threshold, UE could make the LCM decision by comparing the calculated performance metric and the configured threshold. o Standardization effort is needed for the performance metric definition

- NEC (R1-2406536) Proposal 20: Support the main bullet of both Option A and Option B for case 1, i.e., both LMF and UE can perform monitoring metric calculation. Proposal 21: Prioritize Option A-3 if performs monitoring metric calculation at the target UE side. Proposal 22: Prioritize Option B-2 if performs monitoring metric calculation at the target LMF side.

- Fujitsu (R1-2406306) Proposal 15 Several issues need to be further studied for Option A-3, e.g., how to report model information (e.g., model input) to facilitate PRU measurements. Proposal 16 Deprioritize the study on Option A-4 for case 1 model performance monitoring. Proposal 17 For Option B-2 of case 1 model performance monitoring, model information exchange under functionality framework should be further studied. Proposal 18 For label-based case 1 performance monitoring, option B-2 is suggested to be studied with priority, or the ground truth label can be obtained by implementations.

- Google (R1-2405951) Proposal 4: The model monitoring should not require the UE to report the model inference result to the NW. • Option A is preferred compared to Option B Proposal 5: For UE side monitoring metric calculation, support UE to determine the model performance based on the correlation for the model inference results within a time window • If the model inference results are uncorrelated within the time window, the UE can determine the model fails.

- Apple (R1-2406827) Proposal 28: RAN1 should support at least Option A-2, A-3 and B-1.

- ZTE Corporation, Pengcheng Laboratory (R1-2406055) Proposal 25: For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, support option B: - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF.

- Lenovo (R1-2406441) Proposal 23: For Option A – target UE model monitoring for AI/ML positioning Case 1, support Options A-1, A-2, A-3 and A-4 in terms of receiving ground truth label information. Proposal 24: For Option B – LMF model monitoring for AI/ML positioning Case 1, further study Option B in terms near-real time latency requirements for performance monitoring.

- ETRI (R1-2406719) Proposal 9: Add ‘Option A-5 The PRU performs monitoring metric calculation.’ to Option A for model performance monitoring of AI/ML positioning Case 1. Proposal 10: In Case 1, in the absence of PRU, investigate methods for assessing the reliability of ground truth label used in label-based performance monitoring.

- NTT DOCOMO, INC. (R1-2406921) Proposal 14: For case 1 and 2a, when monitoring entity is UE, UE calculates monitoring metric following NW indication. - The indication includes at least model ID/functionality information, performance metrics/threshold.

- Nokia (R1-2406587) Proposal 24: To RAN1 defer Option A-4 from discussion and support other remaining options (Option A and Option B) for performance monitoring.

- Ericsson Inc. (R1-2405945) Proposal 52 Label-based performance monitoring proposals under consideration in RAN1 require higher specification complexity, increased implementation and signaling burdens, and elevated operation costs of positioning systems. The effectiveness of the listed label-based monitoring proposals shall be evaluated and compared to that achievable with self contained performance monitoring (either label-free monitoring or certain label-based approaches). The more complicated label-based monitoring solutions should be supported only if the added values are identified to justify the increased complexity. Observation 50 Label-based monitoring can be achieved by setting up legacy positioning procedure parallel to Case 1 AI ML procedure, or with the support of an OTT server.

### 1st round discussion

At RAN1 #116bis meeting, the following was agreed for label-based monitoring for Case 1:

Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded.

Based on the submitted contributions to RAN1#118, Companies’ preference for the Options is as follow (please update/correct if your view is not captured correctly):

|  | Supported by: | Not supported by: |
| --- | --- | --- |
| Option A in general | Sony, Xiaomi, Google, Lenovo, DOCOMO(?) 5 sources |  |
| Option A-1 | QC, InterDigital, CATT/CICTCI, Intel, vivo, Ericsson, Nokia 8 sources | OPPO 1 source |
| Option A-2 | QC, InterDigital, CATT/CICTCI, Intel, Ericsson, Apple, Nokia 8 sources | OPPO (not clear) 1 source |
| Option A-3 | InterDigital, Sharp, vivo, NEC, Apple, Nokia 6 sources | OPPO, Ericsson 2 sources |
| Option A-4 | Huawei/HiSilicon, Sharp 3 sources | InterDigital, OPPO, Fujitsu (deprioritize), Nokia (defer), Ericsson 5 sources |
| Option B in general |  |  |
| Option B-1 | QC, CATT/CICTCI, Intel, Apple, ZTE, Nokia 7 sources | OPPO 1 source |
| Option B-2 | Sharp, ZTE, Nokia, Sony, NEC, Fujitsu 6 sources | OPPO, Ericsson 2 sources |

The above indicates that Option A has more support than Option B. For the detailed options, Option A-4 has the least support and the most objections. Thus, it is proposed that Option A-4 is deprioritized for Rel-19. Other options are open for further discussion.

Proposal 6.2.2-1

For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring,

- Option A-4 is deprioritized

|  | Company |
| --- | --- |
| Support | InterDigital, Apple, DOCOMO |
| Not support | HW/HiSi |

| Company | Comments |
| --- | --- |
| CATT, CICTCI | Support. |
| Ericsson | OK |
| HW/hiSi | Option A-4 is implementation and cannot be deprioritized. PRUs can send their measurements and labels (location) in implementation manner to the UE side. This can be the target UE or the OTT server. As the training can be terminated there (RP-233133 “ For UE-side models, training data can be generated by the UE, while the termination point for training data may include the UE or a UE-side OTT server.”), labels and measurements can be assumed to be available. The monitoring can therefore be carried out by implementation. Option A-4 allows accurate performance monitoring, since PRU locations are known. Proposal 6.2.2-1 Conclusion: For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, Option A-4 can be achieved by implementation and has no specification impact is deprioritized |
| Fujitsu | Support |

Proposal 6.2.2-2

For label-based model performance monitoring of AI/ML positioning Case 1, study the following options:

- Option 1: the target UE makes the management decision/recommendation (e.g., model deactivation, update, fallback) as a result of model performance monitoring.

  - The target UE notifies the LMF of its model management decision/recommendation.

    - Note: LMF makes the decision on whether to activate or deactivate AI/ML positioning, and which AI/ML positioning method/case or legacy positioning method to use.

  - Option 1 is at least applicable if Option A (The target UE side performs monitoring metric calculation) is supported.

- Option 2: LMF makes the model management decision (e.g., model deactivation, update, fallback) as a result of model performance monitoring, and notifies the target UE.

  - Option 2 is at least applicable if Option B (The LMF performs monitoring metric calculation) is supported.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| ZTE | There’s a conflict concerning the following two parts: For label-based model performance monitoring of AI/ML positioning Case 1, study the following options: - Option 1: the target UE makes the management decision/recommendation (e.g., model deactivation, update, fallback) as a result of model performance monitoring. - The target UE notifies the LMF of its model management decision/recommendation. - Note: LMF makes the decision on whether to activate or deactivate AI/ML positioning, and which AI/ML positioning method/case or legacy positioning method to use. The former part mentioned that UE can make the deactivation or fallback decision, while the latter part specifies that LMF make the decision. It’s not clear which part is responsible for making management decision. |
| InterDigital | We don’t understand why we don’t start form the agreement made in RAN1#116b if a further study is pursued. Option A and Option B were clear about whether it’s the UE or LMF which makes a decision about model management. |
| OPPO | Similar view with ZTE. |
| CATT, CICTCI | Support and should take ZTE’s consideration into account. |
| Ericsson | Ok with option 1. For option 2, we think the LMF can deactivate AIML positioning method but for model activation deactivation it should be up tot he UE. |
| HW/HiSi | Could it ålease be clarified: The agreement from 116b is about where the metric is calculated, not what entity makes the management decisions. This proposals here seems to be a separate issue that we first shoudl discuss. |
| DOCOMO | Same view with ZTE. We think at leaset metric calculation/decision by LMF side can be included. |
| Qualcomm | We think option 1 should be enough. |
| Lenovo | At least Option 1 should be considered. |

## Model monitoring for Case 3a

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Huawei, HiSilicon (R1-2406978) Proposal 16: For model monitoring in Case 3a, recommend to support Option A where the gNB calculates the metric and makes the monitoring decision itself.

- InterDigital, Inc. (R1-2406206) Proposal 30: From RAN1 perspective, support Option A (NG-RAN node performs monitoring metric calculation for its own model) for performance monitoring calculation Proposal 31: Study feasibility of obtaining the ground truth for timing information for Case 3a

- CATT, CICTCI (R1-2406354) Proposal 28: For case 3a, for model performance monitoring metric calculation in label-based model monitoring, at least the following options for generating ground truth label can be considered. - Option A. NG-RAN node performs monitoring metric calculation for its own model, and LMF provides the generated ground truth label to NG-RAN node. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node based on the inference results transmitted by gNB/TRP and estimated ground truth label.

- TCL (R1-2406394) Proposal 3: The monitoring metrics for gNB-side model include error of measurement value, F1-score of LOS/NLOS indicator and standard deviation or variance of measurement value.

- Sony (R1-2406464) Proposal 12: For case 3a, support both Option A (NG-RAN node performs monitoring metric) and Option B (LMF performs monitoring metric)..

- CEWiT (R1-2407076) Proposal 10 : How LMF generates the ground truth labels using model output for AI/ML assisted positioning should be discussed. Proposal 11: For label free monitoring for Case 3a, gNB/TRP is preferred to perform the model monitoring metric calculation. Proposal 12: For label-based monitoring for Case 3a, LMF is preferred to perform the model monitoring metric calculation.

- OPPO (R1-2406255) Proposal 23: For NG-RAN node assisted positioning with gNB-side model (Case 3a), in order to facilitate ground-truth-label-based functionality/model performance monitoring at gNB, support the enhancement on NRPPa signaling to enable the delivery of the ground-truth label or the information that can derive the ground-truth label from LMF to gNB • Functionality/model performance monitoring can be done by gNB implementation

- Intel Corporation (R1-2406015) Proposal 21: - For AI/ML-assisted positioning Case 3a, for Option A for label-based model monitoring at NG-RAN node, ground-truth labels (or their approximations) may be determined at the NG-RAN node via implementation, or they may be determined by NG-RAN node based on assistance information received from the LMF via NRPPa for ground-truth labels, including their quality information. - Note: In general, the LMF may not be able to generate accurate ground-truth labels (i.e., intermediate radio metrics) for monitoring based on knowledge regarding the TRP location(s) and PRU/UE locations as the mapping from intermediate radio metrics to location coordinates may not always be an invertible function. Proposal 22: - For AI/ML-assisted positioning Case 3a, for Option B for label-based model monitoring at LMF, model inputs and outputs are provided by the NG-RAN node to the LMF via NRPPa. - Ground-truth labels (or their approximations) can be determined: - (preferred option) by the LMF based on one or more of: known coordinates of PRUs, estimated coordinates for non-PRU UEs, measurements received from NG-RAN node corresponding to non-AI/ML positioning methods. - (supplementary option) by NG-RAN node based on assistance information received from the LMF for ground-truth labels, including their quality information, and reported to the LMF via NRPPa.

- Xiaomi (R1-2406270) Proposal 31: - For input-based performance monitoring o gNB calculates the performance metric and makes the LCM decision for the assisted positioning with multi-TRP construction o gNB calculates the performance metric and LMF makes the LCM decision for assisted positioning with single TRP construction  Standardization effort is needed for the performance metric definition and report - For ground truth-based performance monitoring o LMF calculates the performance metric and makes the LCM decision

- NEC (R1-2406536) Proposal 23: Recommend to RAN3 that prioritize Option B, i.e., LMF performs monitoring metric calculation for the model located at the NG-RAN node, for label-based model monitoring of case 3a.

- Fujitsu (R1-2406306) Proposal 14 For case 3a performance monitoring metrics calculation, wait for inputs from other WGs for further down-selection.

- Apple (R1-2406827) Proposal 31: Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the NG-RAN. - Option A-2. Reuse Rel-18 assistance data transfer framework from LMF to the NG-RAN (based on NRPPa), where the PRU measurement (e.g., legacy measurement) and the corresponding PRU GT are sent via LMF to the NG-RAN. - Note as this is a PRU, the location is known by the LMF. Proposal 32: Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node - Option B-1: at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the NG-RAN node to LMF. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. {non-PRU with estimated position) - Option B-2: at least inference result (i.e., the model output corresponding to PRU channel measurement) “close” to the target UE is sent by the NG-RAN node to LMF. - In one example, PRU and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. Proposal 35: RAN1 should support at least Option A-2, and B-1.

- ZTE Corporation, Pengcheng Laboratory (R1-2406055) Proposal 26: For AI/ML assisted model monitoring, the following options can be considered: - Option 1: Model monitoring is performed based on AI/ML inference output, i.e., intermediate features. - Option 2: Model monitoring is performed based on estimated UE’s location according to AI/ML inference output. Proposal 27: For UE side model and NG-RAN side model, model monitoring metric calculation function and model monitoring function can be located in the same or different entities.

- ETRI (R1-2406719) Proposal 11: In Case 3a, it is preferable to conduct label-based performance monitoring metric calculation at the LMF. Proposal 12: In Case 3a, in the absence of PRU, investigate methods for assessing the reliability of ground truth label used in label-based performance monitoring.

- NTT DOCOMO, INC. (R1-2406921) Proposal 11: RAN1 concludes that no further discussion will be made in RAN1 for performance monitoring of case 3a and 3b. Detailed spec impact is up to RAN3. RAN1 sends LS to RAN2 and RAN3 for clarification, if necessary.

- Nokia (R1-2406587) Proposal 25: For Case 3a, RAN1 indicates the following potential candidates to perform monitoring metric calculation: NG-RAN and LMF. Further details of monitoring is out of the scope of RAN1 WG.

- Ericsson Inc. (R1-2405945) Proposal 53 For AI/ML positioning Case 3a, for model performance monitoring metric calculation in label-based model monitoring, from RAN1 perspective, Option A and Option B are feasible and recommended. Proposal 54 Specification impact for performance monitoring of Case 3a is expected for: (a) (Option A of label-based) LMF to send label-related information to gNB; (b) (Option B of label-based) LMF to send the monitoring metric calculation results to gNB; (c) (label-based and label-free) gNB to notify LMF of action as a consequence of monitoring; (d) SRS configuration and time window for performance monitoring. Proposal 55 Send an LS to RAN2 and RAN3 on the potential specification impact for performance monitoring of Case 3a.

### 1st round discussion

In RAN1 116bis the following agreement was made on model performance monitoring of Case 3a:

Agreement For AI/ML positioning Case 3a, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility of the following options. To provide information on how to generate information on ground truth label for each option. - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope, but RAN1 can make recommendation about the option(s), and potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform the monitoring metric calculation is up to implementation

This is related to RAN2 agreements shown below.

RAN2#125bis: Agreements: … 4 FFS whether there is specification impact associated to gNB-side model monitoring. 5 For POS, RAN2 assumes gNB or LMF could perform performance monitoring for case 3a and LMF is responsible for the performance monitoring for case 3b and wait for any further inputs from other WGs RAN2#126: Agreements for NW-side model 1 RAN2 to await RAN1 progress to determine need for any specification work on AI/ML positioning Case 3a and Case 3b.

Based on the companies’ contributions submitted to RAN1#118, majority view is that both Option A and B are feasible for Case 3a, which is aligned with RAN2 agreement as well. Thus an agreement can be made from RAN1 perspective, and an LS can be sent to RAN2 and RAN3.

Proposal 6.3.2-1

For AI/ML positioning Case 3a, for model performance monitoring metric calculation in label-based model monitoring, from RAN1 perspective, Option A and Option B are feasible and recommended.

- Option A.	NG-RAN node performs monitoring metric calculation for its own model.

- Option B.	LMF performs monitoring metric calculation for the model located at the NG-RAN node.

- Send an LS to RAN2 and RAN3

Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation.

Note: Exact method to perform the monitoring metric calculation is up to implementation.

|  | Company |
| --- | --- |
| Support | Baicells, DOCOMO |
| Not support | InterDigital, Apple |

| Company | Comments |
| --- | --- |
| InterDigital | LS to RAN2 or RAN3 is not needed since agreements in RAN1 can be read by other WGs. |
| CATT, CICTCI | Support |
| Apple | For Option B, explicity indicate that user privacy will be preserved especially for non-PRU UEs. The LMF is not to send label information (e.g. position/timing) to the gNB. |
| Ericsson | OK |
| HW/HiSi | LS does not seem to be needed. |
| Qualcomm | For Option A, the UE/PRU privacy needs to be maintained. We still think it is out of scope of RAN1 to make such recommendation. |
| Lenovo | Ok to support. This proposal seems to have a relation with Proposal 6.1.2-1 |

Proposal 6.3.2-2

For label-based model performance monitoring of AI/ML positioning Case 3a, study the following options:

- Option 1: the gNB makes the management decision/recommendation (e.g., model deactivation, update, fallback) as a result of model performance monitoring.

  - The gNB notifies the LMF of its model management decision/recommendation.

    - Note: LMF makes the decision on whether to activate or deactivate AI/ML positioning, and which AI/ML positioning method/case or legacy positioning method to use.

  - Option 1 is at least applicable if Option A (NG-RAN node performs monitoring metric calculation for its own model) is supported.

- Option 2: LMF makes the model management decision (e.g., model deactivation, update, fallback) as a result of model performance monitoring, and notifies the gNB.

  - Option 2 is at least applicable if Option B (LMF performs monitoring metric calculation for the model located at the NG-RAN node) is supported.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| ZTE | Same as Proposal 6.2.2-1. |
| Baicells | We think for option1, whether gNB can notify LMF of its model management recommendation can be further discussed. The monitoring results of gNB may already carry sufficient information. |
| CATT, CICTCI | Support |
| Ericsson | Prefer option 1, LMF should instead manage the positioning method. |
| DOCOMO | Support to study both options. Option 2 can be considered as a starting point. |

## other proposals for model monitoring

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Huawei, HiSilicon (R1-2406978) Proposal 17: For model monitoring in Case 3b, no further assistance information or measurement report in addition to inference is required to be sent to the LMF. Observation 12: Model monitoring for Case 2a can be done in a similar manner as Case 1, i.e., by implementation at the same UE-side entity that derives the monitoring metric. Proposal 18: For model monitoring in Case 2b, no further assistance information or measurement report in addition to inference is required to be sent to the LMF.

- InterDigital, Inc. (R1-2406206) Proposal 32: For Case 3b, specification enhancement is not needed for assistance information and/or measurement report sent from UE/PRU and/or gNB to LMF to assist LMF for the performance monitoring.

- CATT, CICTCI (R1-2406354) Proposal 25: For label-free monitoring, for all cases, the performance metric calculation is up to implementation. Proposal 26: For case 1 and case 2a, for model performance monitoring metric calculation in label-based model monitoring, at least the following options for generating ground truth label can be considered. - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. Proposal 27: For case 2b and case 3b, for model performance monitoring metric calculation in label-based model monitoring, LMF generates UE’s location as ground truth label and performs monitoring metric calculation. Proposal 28: For case 3a, for model performance monitoring metric calculation in label-based model monitoring, at least the following options for generating ground truth label can be considered. - Option A. NG-RAN node performs monitoring metric calculation for its own model, and LMF provides the generated ground truth label to NG-RAN node. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node based on the inference results transmitted by gNB/TRP and estimated ground truth label.

- TCL (R1-2406394) Proposal 1: It is recommended to perform the monitoring metric calculation at the model inference entity for label-free model monitoring.

- CMCC (R1-2405976) Proposal 8: For performance monitoring is based on the ground-truth labels, further study method to obtain ground-truth label. Proposal 9: For AI/ML based positioning, the relationship between model monitoring and positioning integrity can be considered.

- Samsung (R1-2406638) Proposal 15. RAN1 considers the model output based, model input based and other (measurement) based monitoring metrics. Proposal 16. RAN1 considers to define the measurement conditions (e.g., SNR level, LoS indication etc) to facilitate the performance (model) monitoring. Proposal 17. RAN1 considers the monitoring entity as UE/TRP/LMF for UE side model, TRP, LMF for TRP side model, LMF for LMF side model. Proposal 18. RAN1 considers the monitoring metric related threshold to be configured, dependent on the monitoring metric content. Proposal 19. RAN1 supports the signalling for the monitoring refence signal configuration, monitoring measurement and its report. Proposal 20. Monitoring outcome determination could be based on one time metric or multiple time metrics. Proposal 21. Post monitoring behaviour should be specified.

- Xiaomi (R1-2406270) Proposal 30: For performance monitoring in Case 2a - For label-free based performance monitoring method o UE calculate the performance monitoring metric and make the LCM decision - For the label-based performance monitoring method o LMF calculate the performance monitoring metric and make the LCM decision Proposal 32: Before receiving the request from other WG, there is no need for RAN1 to study the assistance information for performance monitoring note: this is for case 2b/3b

- NEC (R1-2406536) Proposal 24: For model performance monitoring of AI/ML positioning Case 2a, support following options with regard to which entity is designated to generate model monitoring metric for model performance monitoring metric calculation in label-based model monitoring: - Option A. The target UE side performs monitoring metric calculation. - Option B. The LMF performs monitoring metric calculation.

- Fujitsu (R1-2406306) Proposal 19 For case 2a model performance monitoring, label-free method is up to implementation while label-based method can wait for the options’ discussion progress of case 1. Proposal 20 For label-based case 3b model performance monitoring, LMF collects the inference output and ground truth labels. There is no assistance information needed to support LMF-side monitoring. Proposal 22 Model related information exchange can be studied together with model performance monitoring procedures.

- NVIDIA (R1-2406493) Proposal 5: For AI/ML based positioning, introduce specification support for assistance signalling and procedure for model performance monitoring and model update/tuning, including monitored metrics, triggers for model update, dedicated reference signals, measurements, and feedback report.

- Apple (R1-2406827) Proposal 26: the default option is for the monitoring to occur at the entity with the AI/ML model.

# Model identification and model functionality

## Model identification, model ID

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Huawei, HiSilicon (R1-2406978) Proposal 19: Defer discussion on model-ID based LCM in 9.1.2 until end of Q3/2024, since its necessity still is under discussion in 9.1.3.3 and in RAN2.

- Samsung (R1-2406638) Proposal 4. RAN1 supports the indication of determined model from one entity to another entity Proposal 5. RAN1 supports the indication of measurement related threshold/report/feedback and model applicable condition to assist the model determination from one entity to another entity

- OPPO (R1-2406255) Proposal 26: For UE-based positioning with UE-side model (Case 1) and UE-assisted/LMF based positioning with UE-side model (Case 2a), specify the UE capability signaling to report the supported configuration(s) associated with given functionality(ies) - E.g., DL-PRS resources capability, DL-PRS Processing capability, measurement capability Proposal 27: For UE-based positioning with UE-side model (Case 1) and UE-assisted/LMF based positioning with UE-side model (Case 2a), UE can report the applicable functionalities by sending a ProvideCapabilities message to the LMF (namely, triggering the capability indication procedure of TS 37.355) - FFS: whether some enhancement is needed to reduce the signaling overhead

- Xiaomi (R1-2406270) Proposal 13: Support functionality identification for Case 1 and Case 2a Proposal 14: Support model identification for Case 1 and Case 2a Proposal 15: Whether there is functionality identification / model identification-like procedure between gNB and LMF is up to RAN3

- NVIDIA (R1-2406493) Proposal 8: For AI/ML based positioning, introduce specification support for additional conditions to be included in model description information during model identification.

## Model functionality, UE capability reporting

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Qualcomm (R1-2407020) Proposal 19: For AI/ML positioning life cycle management in Case1/2a, RAN1 proceeds with understanding that LMF can request UE to activate, deactivate, and fall-back AI/ML positioning.

- Huawei, HiSilicon (R1-2406978) Proposal 20: Support functionality-based LCM for UE-side model of Case 1/2a.

- InterDigital, Inc. (R1-2406206) Proposal 2: An AIML positioning functionality is defined by input type and output type of an AIML model(s) Proposal 3: Support dynamic UE capability signaling for Case 1 for AIML based positioning to indicate UE capability that can change dynamically (e.g., remaining battery power) to the LMF Proposal 28 : For AI/ML based positioning case 1, at least the following types of measurements should be requested by the LMF to be made by the UE: • timing information; • paired timing information and power information. • phase measurement

- Samsung (R1-2406638) Proposal 2. RAN1 considers the trigger conditions for enabling AI/ML based method for positioning, including potential model applicable conditions or the environment(channel) applicable conditions. Proposal 3. In addition to the entity where the model is deployed, the network can decide or /assist to decide the model used at UE side, the LMF can decide and LMF/UE can assist to decide the model used at TRP side, the TRP/UE can assist to decide the model used at LMF side.

- OPPO (R1-2406255) Proposal 25: For UE-based positioning with UE-side model (Case 1) and UE-assisted/LMF based positioning with UE-side model (Case 2a) • The functionality/model can be activated or disactivated by LPP signaling, which is the same as the legacy positioning procedure • UE can autonomously deactivate the AI operations and fall back to the legacy operations o UE reporting includes some field/IE to indicate whether the results are based on legacy operation or AI model output.

- Intel Corporation (R1-2406015) Proposal 25: - At least for UE-side models, model/functionality selection or switching could be realized via explicit indication from a network entity (e.g., LMF) or implicitly based on configurations related to data collection for training and/or inference, e.g., configurations related to data collection in time dimension or over a certain geographical area.

- Xiaomi (R1-2406270) Proposal 16: Categorize the AI/ML feature from the following 3 aspects for AI-based positioning - Output type of the AI models - The location where AI models are deployed - The positioning RS type i.e., SRS-based or PRS-based Proposal 17: At least measurement related aspects e.g., PRS bandwidth, TRP configuration can be considered as conditions to categorize the AI/ML functionality Proposal 18: Support UE directly report the applicable functionality/model via e.g., an identifier. Proposal 19: Applicable functionality/model report can be supported in both the case that associated AI/ML feature is enabled and the case that associated AI/ML feature is not enabled Proposal 34: The fallback mode including the non-AI positioning approach and related configuration are preconfigured to UE Proposal 35: UE group-based LCM operation (e.g., model activation/deactivation/switch) can be considered

- Fujitsu (R1-2406306) Proposal 21 In RAN1, positioning specific functionality discussion can be postponed until further progress of functionality granularity is achieved in AI9.1.3.3.

- NVIDIA (R1-2406493) Proposal 6: For AI/ML based positioning, introduce specification support for UE capability signalling including model training, model inference and model monitoring. Proposal 7: For AI/ML based positioning, introduce specification support for conditions of a Feature/FG available for functionality.

- Apple (R1-2406827) Proposal 43: Information covered by the UE capability may include: - Positioning types (based on agreed on cases) - Capability may include parameters such as the specific AI/ML case type. - Measurement and Reporting capability: Reference signal configuration such as reference signal resources, measurement capability (for case 1, 2a and 2b) and measurement reporting capability (case 2a e.g. LOS/NLOS and 2b e.g. CIR, PDP, DP) - UE/Network conditions including Scenarios, site, or dataset specific information: As an example, site specific capabilities may be based on physical cell-IDs and even sub-site IDs to accommodate the scenario in which a cell may have to be supported by multiple different models due to differences within the cell. This may be sent as additional conditions for the AI/ML model. - Note that this may require an update to the capabilities procedure as discussed in RAN2 - Monitoring requirements such as ground truth labels and quality indicators

- Fraunhofer IIS, Fraunhofer HHI (R1-2406711) Proposal 13: Support monitoring the events resulting from functionality/model management decisions (fallback or (de)activation/switching) for a pre-defined time window. Proposal 14: Support a monitoring report including the outcome (performance, robustness, overhead/cost, etc.) of functionality or model management decisions when required.

- Nokia (R1-2406587) Proposal 47: RAN 1 to discuss the potential approaches on reusing or enhancing the current LPP to enable the functionality framework for AI/ML positioning. Proposal 48: In positioning use cases 1 and 2a, AI/ML functionality is indicated to UE via the PRS configuration provided in LPP Assistance Data and the measurement configuration provided in LPP Request Location Information. Proposal 49: For all use cases (e.g., Cases 1, 3a, 3b) LMF is the only entity to determine functionality decision (e.g., functionality switching, selection, (de)activation, fallback) based on monitoring outcome.

# Other proposals

## Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Qualcomm (R1-2407020) Proposal 1: For AI/ML positioning cases (Case1 to Case3b), from RAN1 perspective, AI/ML positioning can be part of existing positioning methods and modes as follows: - Case1 as part of UE-based DL-TdoA and DL-AoD positioning - Case2a and Case2b as part of UE-assisted DL-TdoA, UE-assisted DL-AoD, and multi-RTT positioning - Case3a and Case3b as part of NG-RAN-assisted UL-TdoA, NG-RAN-assisted UL-AoA, and multi-RTT positioning

- InterDigital, Inc. (R1-2406206) Proposal 1: RAN1 does not discuss 2nd priority use cases until stage 3 work on 1st priority is completed Proposal 35: For Case 3a and Case 3b, use existing UL or DL & UL positioning methods as the starting point for AIML-based positioning method Proposal 36: For Case 1, enhance the existing UE-based DL-TDOA positioning method to support AIML-based positioning

- Samsung (R1-2406638) Proposal 1. Second prioritized cases strive to have common design with first prioritized cases.

- Sony (R1-2406464) Proposal 4: Support model transfer operation from server node (e.g., LMF) to UE/gNB to support AI/ML-assisted positioning. Proposal 5: Define the AI/ML model structure and parameters for model transfer to support AI/ML-assisted positioning

- CEWiT (R1-2407076) Proposal 5: Semi supervised learning can be used for positioning Non PRU UE in scenarios where lesser PRUs are deployed in an area.

- Xiaomi (R1-2406270) Proposal 7: For data collection in Case 1 and Case 2a - When AI models are developed by the UE side, three is no specification impact on the generation of measurements corresponding to input and the ground truth - When AI models are developed by the network side (if model transfer/delivery is supported), there is need to specify the data format definition and configuration of the data generation - For both cases, RAN1 need to identify whether any assistance information is needed from network.

- NEC (R1-2406536) Proposal 19: Support semi-supervised learning for AI/ML based positioning.

# Proposals for online sessions

## Proposals for Monday online session

Proposal 4.1.2-1

For training data collection of AI/ML based positioning, for both time stamp of channel measurement and time stamp of label, the existing IEs are reused.

- For channel measurement and label generated by PRU or non-PRU UE, reuse the existing IE NR-TimeStamp-r16 in TS 37.355;

- For channel measurement generated by TRP/gNB, reuse the existing IE “Time Stamp” in TS 38.455.

Proposal 6.2.2-1

Conclusion:

For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring,

- Option A-4 is up to implementation and has no specification impact. No further discussion on Option A-4.

Proposal 6.3.2-1

For AI/ML positioning Case 3a, for model performance monitoring metric calculation in label-based model monitoring, from RAN1 perspective, Option A and Option B are feasible and recommended.

- Option A.	NG-RAN node performs monitoring metric calculation for its own model.

- Option B.	LMF performs monitoring metric calculation for the model located at the NG-RAN node.

- Send an LS to RAN2 and RAN3

Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation.

Note: Exact method to perform the monitoring metric calculation is up to implementation.

Note: For Option A, RAN1 assumes that user data privacy can be preserved.

Proposal 2.3.2-1

For AI/ML positioning, for both sample-based measurement (if supported) and path-based measurement (if supported), regarding the power information for determining the model input,

- For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point.

  - For measurement report of DL PRS-RSRPP, reuse the existing measurement report mapping table for PRS-RSRPP in 38.133.

- For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point.

  - For measurement report of UL PRS-RSRPP, reuse the existing measurement report mapping table for SRS-RSRPP in 38.133.

Note: when sample-based measurement is used, “path” and related terms are replaced by “sample” and related terms in the existing definitions.

Proposal 4.1.2-2

For training data collection of AI/ML based positioning Cases 1, 2b, 3b, study and select one of the following options to represent the quality indicator of label:

- Option 1 (value in meters): reuse the existing IE. That is, IE NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455;

- Option 2 (value 0 to 1): hard (1 or 0) and soft values (0, 0.1, 0.2, …, 1.0), with value 1 indicate the highest quality.

## Proposals for Tuesday online session

## Proposals for Wednesday online session

# Conclusion

TBD

# References

RP-240774, "Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface," Qualcomm, 3GPP TSG RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024.

R1-2405945	AI/ML for Positioning Accuracy Enhancement	Ericsson Inc.

R1-2405951	AI/ML based Positioning	Google

R1-2405976	Discussion on specification support for positioning accuracy enhancement	CMCC

R1-2406015	Specification support for AI/ML for positioning accuracy enhancement	Intel Corporation

R1-2406055	Discussion on AI/ML-based positioning enhancement	ZTE Corporation, Pengcheng Laboratory

R1-2406173	Specification support for positioning accuracy enhancement	vivo

R1-2406206	Discussion on support for AIML positioning	InterDigital, Inc.

R1-2406255	On specification for AI/ML-based positioning accuracy enhancements	OPPO

R1-2406270	Discussion on AI/ML-based positioning accuracy enhancement	Xiaomi

R1-2406306	Discussion on specification support for AI/ML positioning accuracy enhancement	Fujitsu

R1-2406354	Specification support for AI/ML-based positioning	CATT, CICTCI

R1-2406394	Discussion on specification support for positioning accuracy enhancement	TCL

R1-2406441	Specification impacts for AI/ML Positioning	Lenovo

R1-2406464	Discussion on AI/ML for positioning accuracy enhancement	Sony

R1-2406493	Specification support for AI-enabled positioning	NVIDIA

R1-2406536	Discussion on specification support for AIML based positioning accuracy enhancement	NEC

R1-2406587	AI/ML for Positioning Accuracy Enhancement	Nokia

R1-2406594	Discussions on specification support for positioning accuracy enhancement	Ruijie Networks Co. Ltd

R1-2406638	Discussion for supporting AI/ML based positioning accuracy enhancement	Samsung

R1-2406711	AI/ML positioning accuracy enhancement	Fraunhofer IIS, Fraunhofer HHI

R1-2406719	Discussion on specification support for positioning accuracy enhancement	ETRI

R1-2406827	Discussion on Specification Support for AI/ML-based positioning	Apple

R1-2406921	Discussion on AI/ML for positioning accuracy enhancement	NTT DOCOMO, INC.

R1-2406970	Discussion on specification support for AI/ML based positioning accuracy enhancements	Sharp

R1-2406978	Discussion on  AI/ML for positioning accuracy enhancement	Huawei, HiSilicon

R1-2407020	Specification support for AI-ML-based positioning accuracy enhancement	Qualcomm Incorporated

R1-2407063	Discussion on Specification Support of AI/ML for Positioning Accuracy Enhancement	Indian Institute of Tech (M), IIT Kanpur

R1-2407076	Discussion on specification support for AI/ML positioning accuracy enhancement	CEWiT

R1-2407105	Design for AI/ML based positioning	MediaTek Korea Inc.

R1-2407166	Discussions on positioning accuracy enhancement for AI/ML	ITL

# Appendix A. Agreements from RAN1#118

Working Assumption

For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by:

- PRU

- Non-PRU UE with estimated location

- LMF

Note: transfer of label and its related data is out of RAN1 scope.

Note: It is assumed that user data privacy of non-PRU UE is preserved.

Note: Previous related working assumption made in RAN1#116bis for training data generation of AI/ML based positioning Case 3b will not need to be confirmed.

Agreement

Sample-based measurement is defined as:

- The measurement is composed of Nt' samples of the estimated channel response in time domain. The timing information for the Nt' samples are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR.

  - The corresponding measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' samples.

- Nt' and k can be signalled

  - FFS: the value range of Nt'; the value range of integer k for the timing granularity T.

- The timing information is defined relative to a reference time

Further discussion is expected on the determination of Nt' and k (including signaling) , and a rule to be introduced for selecting Nt' samples.

Note: It doesn’t imply the definition of Sample-based measurement will be captured into the spec.

Agreement

Path-based measurement refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed).

Agreement

For training data collection of AI/ML based positioning, if a training data sample contains both Part A and Part B, RAN1 assumes that Part A and Part B in one training data sample are:

- for a same UE (PRU or Non-PRU UE), and

- for a same location associated with Part B.

Note: the association can be discussed

Agreement

Draft LS R1-2405577 is endorsed in principle by adding the latest agreements made in this meeting and adding “agreements” to “Note: the working assumptions above are based on RAN1 understanding for RAN work item (NR_AIML_air).”

Agreement

Final LS R1-2405578 is endorsed.

# Appendix B. Report mapping tables for DL PRS-RSRPP and UL SRS RSRPP in 38.133

Table 10.1.38.3.1-1: Measurement report mapping for PRS-RSRPP

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| PRS_RSRPP_0 | PRS-RSRPP<-156 | dBm |
| PRS_RSRPP_1 | -156≤PRS-RSRPP<-155 | dBm |
| PRS_RSRPP_2 | -155≤PRS-RSRPP<-154 | dBm |
| PRS_RSRPP_3 | -154≤PRS-RSRPP<-153 | dBm |
| PRS_RSRPP_4 | -153≤PRS-RSRPP<-152 | dBm |
| PRS_RSRPP_5 | -152≤PRS-RSRPP<-151 | dBm |
| PRS_RSRPP_6 | -151≤PRS-RSRPP<-150 | dBm |
| PRS_RSRPP_7 | -150≤PRS-RSRPP<-149 | dBm |
| PRS_RSRPP_8 | -149≤PRS-RSRPP<-148 | dBm |
| PRS_RSRPP_9 | -148≤PRS-RSRPP<-147 | dBm |
| PRS_RSRPP_10 | -147≤PRS-RSRPP<-146 | dBm |
| PRS_RSRPP_11 | -146≤PRS-RSRPP<-145 | dBm |
| PRS_RSRPP_12 | -145≤PRS-RSRPP<-144 | dBm |
| PRS_RSRPP_13 | -144≤PRS-RSRPP<-143 | dBm |
| PRS_RSRPP_14 | -143≤PRS-RSRPP<-142 | dBm |
| PRS_RSRPP_15 | -142≤PRS-RSRPP<-141 | dBm |
| PRS_RSRPP_16 | -141≤PRS-RSRPP<-140 | dBm |
| PRS_RSRPP_17 | -140≤PRS-RSRPP<-139 | dBm |
| PRS_RSRPP_18 | -139≤PRS-RSRPP<-138 | dBm |
| … | … | … |
| PRS_RSRPP_111 | -46≤PRS-RSRPP<-45 | dBm |
| PRS_RSRPP_112 | -45≤PRS-RSRPP<-44 | dBm |
| PRS_RSRPP_113 | -44≤PRS-RSRPP<-43 | dBm |
| PRS_RSRPP_114 | -43≤PRS-RSRPP<-42 | dBm |
| PRS_RSRPP_115 | -42≤PRS-RSRPP<-41 | dBm |
| PRS_RSRPP_116 | -41≤PRS-RSRPP<-40 | dBm |
| PRS_RSRPP_117 | -40≤PRS-RSRPP<-39 | dBm |
| PRS_RSRPP_118 | -39≤PRS-RSRPP<-38 | dBm |
| PRS_RSRPP_119 | -38≤PRS-RSRPP<-37 | dBm |
| PRS_RSRPP_120 | -37≤PRS-RSRPP<-36 | dBm |
| PRS_RSRPP_121 | -36≤PRS-RSRPP<-35 | dBm |
| PRS_RSRPP_122 | -35≤PRS-RSRPP<-34 | dBm |
| PRS_RSRPP_123 | -34≤PRS-RSRPP<-33 | dBm |
| PRS_RSRPP_124 | -33≤PRS-RSRPP<-32 | dBm |
| PRS_RSRPP_125 | -32≤PRS-RSRPP<-31 | dBm |
| PRS_RSRPP_126 | -31≤PRS-RSRPP | dBm |

Table 13.6.1-1: UL SRS RSRPP report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| SRS_RSRPP_0 | SRS-RSRPP<-156 | dBm |
| SRS_RSRPP_1 | -156≤SRS-RSRPP<-155 | dBm |
| SRS_RSRPP_2 | -155≤SRS-RSRPP<-154 | dBm |
| SRS_RSRPP_3 | -154≤SRS-RSRPP<-153 | dBm |
| SRS_RSRPP_4 | -153≤SRS-RSRPP<-152 | dBm |
| SRS_RSRPP_5 | -152≤SRS-RSRPP<-151 | dBm |
| SRS_RSRPP_6 | -151≤SRS-RSRPP<-150 | dBm |
| SRS_RSRPP_7 | -150≤SRS-RSRPP<-149 | dBm |
| SRS_RSRPP_8 | -149≤SRS-RSRPP<-148 | dBm |
| SRS_RSRPP_9 | -148≤SRS-RSRPP<-147 | dBm |
| SRS_RSRPP_10 | -147≤SRS-RSRPP<-146 | dBm |
| SRS_RSRPP_11 | -146≤SRS-RSRPP<-145 | dBm |
| SRS_RSRPP_12 | -145≤SRS-RSRPP<-144 | dBm |
| SRS_RSRPP_13 | -144≤SRS-RSRPP<-143 | dBm |
| SRS_RSRPP_14 | -143≤SRS-RSRPP<-142 | dBm |
| SRS_RSRPP_15 | -142≤SRS-RSRPP<-141 | dBm |
| SRS_RSRPP_16 | -141≤SRS-RSRPP<-140 | dBm |
| SRS_RSRPP_17 | -140≤SRS-RSRPP<-139 | dBm |
| SRS_RSRPP_18 | -139≤SRS-RSRPP<-138 | dBm |
| … | … | … |
| SRS_RSRPP_111 | -46≤SRS-RSRPP<-45 | dBm |
| SRS_RSRPP_112 | -45≤SRS-RSRPP<-44 | dBm |
| SRS_RSRPP_113 | -44≤SRS-RSRPP<-43 | dBm |
| SRS_RSRPP_114 | -43≤SRS-RSRPP<-42 | dBm |
| SRS_RSRPP_115 | -42≤SRS-RSRPP<-41 | dBm |
| SRS_RSRPP_116 | -41≤SRS-RSRPP<-40 | dBm |
| SRS_RSRPP_117 | -40≤SRS-RSRPP<-39 | dBm |
| SRS_RSRPP_118 | -39≤SRS-RSRPP<-38 | dBm |
| SRS_RSRPP_119 | -38≤SRS-RSRPP<-37 | dBm |
| SRS_RSRPP_120 | -37≤SRS-RSRPP<-36 | dBm |
| SRS_RSRPP_121 | -36≤SRS-RSRPP<-35 | dBm |
| SRS_RSRPP_122 | -35≤SRS-RSRPP<-34 | dBm |
| SRS_RSRPP_123 | -34≤SRS-RSRPP<-33 | dBm |
| SRS_RSRPP_124 | -33≤SRS-RSRPP<-32 | dBm |
| SRS_RSRPP_125 | -32≤SRS-RSRPP<-31 | dBm |
| SRS_RSRPP_126 | -31≤SRS-RSRPP | dBm |