3GPP TSG RAN WG1 #118		  R1-2407375

Maastricht, NL, August 19th - 23rd, 2024

Source: 	Moderator (Samsung)

Title:	Summary of discussion on CSI related operation based on NES capability

Agenda Item:	8.1

Document for:	Discussion and Decision

Introduction

This document is a summary of discussion on the Rel-18 CR on CSI related operation based on NES capability as provided in [1], [2]. Changes proposed in [2] are copied to Appendix directly for easy discussion.

Background

In RAN1#117 meeting, a CR on CSI processing criteria for new NES capability signaling was agreed as R1-2405705. The agreed CR addressed the coexistence issue between legacy and R18 NES UE capabilities related to CPU. Similar to CPU, new parameters for the number of supported simultaneous NZP-CSI-RS resources (maxNumberCSI-ResourcePerCC-r18, maxNumberCSI-ResourceAcrossCC-r18), the number of supported total CSI-RS ports in simultaneous NZP-CSI-RS resources (maxNumberTotalCSI-ResourcePerCC-r18, maxNumberPortsAcrossCC-r18) and the number of supported CSI report setting (totalNumberCSI-Reporting-r18) were introduced for R18 NES. How a UE shall use these new capabilities in consideration with the legacy CSI report capability shall be clarified as UE behavior for achieving common understanding with gNB.

To address the remaining issue, the proposed CR in [2] describes conditions with which legacy or R18 capabilities are used for the number of supported simultaneous NZP-CSI-RS resources, the number of supported total CSI-RS ports in simultaneous NZP-CSI-RS resources and the number of supported CSI report setting.

Discussion

Proposal 1:

- Agree on the following TP for TS 38.214.

- Reason for change: - In Rel-18, new parameters for the number of supported simultaneous NZP-CSI-RS resources (maxNumberCSI-ResourcePerCC-r18, maxNumberCSI-ResourceAcrossCC-r18), the number of supported total CSI-RS ports in simultaneous NZP-CSI-RS resources (maxNumberTotalCSI-ResourcePerCC-r18, maxNumberPortsAcrossCC-r18) and the number of supported CSI report setting (totalNumberCSI-Reporting-r18) are introduced. How a UE shall use these new capabilities in consideration with the legacy CSI report capability shall be clarified as UE behavior for achieving common understanding with gNB. - Summary of change: - The new capability parameters for the number of supported simultaneous NZP-CSI-RS resources/ports and the number of supported CSI report setting as well as corresponding UE behavior which capability parameter(s) to use is clarified. - Consequences if not approved: - The UE behavior is not defined when both new capability parameters and legacy capability parameters are reported. - Other comments: - It is understood that there is no impact on the legacy UE (i.e., no support of the new parameters) on the number of supported simultaneous NZP-CSI-RS resources and the number of supported CSI report setting. - 5.2.1.1 Reporting settings - <Unchanged part omitted> The time domain behavior of the CSI-ReportConfig is indicated by the higher layer parameter reportConfigType and can be set to 'aperiodic', 'semiPersistentOnPUCCH', 'semiPersistentOnPUSCH', or 'periodic'. The UE indicates the number of supported periodic CSI report setting per BWP with parameter maxNumberPeriodicCSI-PerBWP-ForCSI-Report or totalNumberCSI-Reporting-r18. If UE is configured with at least one CSI report setting with sub-configuration in a BWP in a serving cell, UE shall use totalNumberCSI-Reporting-r18 for the BWP; otherwise, UE shall use maxNumberPeriodicCSI-PerBWP-ForCSI-Report for the BWP. The UE indicates the number of supported semi-persistent CSI report setting per BWP with parameter maxNumberSemiPersistentCSI-PerBWP-ForCSI-Report or totalNumberCSI-Reporting-r18. If UE is configured with at least one CSI report setting with sub-configuration in a BWP in a serving cell, UE shall use totalNumberCSI-Reporting-r18 for the BWP; otherwise, UE shall use maxNumberSemiPersistentCSI-PerBWP-ForCSI-Report for the BWP. The UE indicates the number of supported aperiodic CSI report setting per BWP with parameter maxNumberAperiodicCSI-PerBWP-ForCSI-Report or totalNumberCSI-Reporting-r18. If UE is configured with at least one CSI report setting with sub-configuration in a BWP in a serving cell, UE shall use totalNumberCSI-Reporting-r18 for the BWP; otherwise, UE shall use maxNumberAperiodicCSI-PerBWP-ForCSI-Report for the BWP. For 'periodic' and 'semiPersistentOnPUCCH'/'semiPersistentOnPUSCH' CSI reporting, the configured periodicity and slot offset applies in the numerology of the UL BWP in which the CSI report is configured to be transmitted on. The higher layer parameter reportQuantity indicates the CSI-related, L1-RSRP-related, L1-SINR-related, CapabilityIndex-related or TDCP-related quantities to report. The reportFreqConfiguration indicates the reporting granularity in the frequency domain, including the CSI reporting band and if PMI/CQI reporting is wideband or sub-band. The timeRestrictionForChannelMeasurements parameter in CSI-ReportConfig can be configured to enable time domain restriction for channel measurements and timeRestrictionForInterferenceMeasurements can be configured to enable time domain restriction for interference measurements. The CSI-ReportConfig can also contain CodebookConfig, which contains configuration parameters for Type-I, Type II, Enhanced Type II CSI, Further Enhanced Type II Port Selection, Enhanced Type II for coherent joint transmission (CJT), Further Enhanced Type II Port Selection for CJT, Enhanced Type II for predicted PMI, or Further Enhanced Type II Port Selection for predicted PMI including codebook subset restriction when applicable, and configurations of group-based reporting. A UE is not expected to be configured with a CSI report setting associated with a dormant DL BWP if the reportConfigType is set to 'aperiodic'. A CSI-ReportConfig can contain a list of sub-configurations, provided by the higher layer parameter csi-ReportSubConfigToAddModList, where each sub-configuration is identified by CSI-ReportSubConfigId and configured with nzp-CSI-RS-ResourceList which corresponds to a list of one or more CSI-RS resources or configured with portSubsetIndicator which corresponds to a CSI-RS antenna port subset, and/or configured with powerOffset which corresponds to a power offset for PDSCH relative to CSI-RS additional to powerControlOffset of the CSI-RS resource(s). A UE is not expected to be configured with a CSI-ReportConfig that contains a mix of sub-configuration(s) each configured with nzp-CSI-RS-ResourceList which corresponds to a list of one or more CSI-RS resources and some other sub-configuration(s) each configured with portSubsetIndicator which corresponds to CSI-RS antenna port subset. - <Unchanged part omitted> - 5.2.1.6 CSI processing criteria - <Unchanged part omitted> In any slot, the UE is not expected to have more active CSI-RS ports or active CSI-RS resources in active BWPs than reported as capability. The UE indicates the number of supported simultaneous NZP-CSI-RS resources with parameter maxNumberSimultaneousNZP-CSI-RS-PerCC or maxNumberCSI-ResourcePerCC-r18 in a component carrier, and maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC or maxNumberCSI-ResourceAcrossCC-r18 in active BWPs across all component carriers. If UE is configured with at least one CSI report setting with sub-configuration in the component carrier, UE shall use maxNumberCSI-ResourcePerCC-r18; otherwise, UE shall use maxNumberSimultaneousNZP-CSI-RS-PerCC. If UE is configured with at least one CSI reporting setting with sub-configuration in any component carrier, UE shall use maxNumberCSI-ResourceAcrossCC-r18; otherwise, UE shall use maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC. The UE indicates the supported total number of ports in simultaneous NZP-CSI-RS resources with parameter totalNumberPortsSimultaneousNZP-CSI-RS-PerCC or maxNumberTotalCSI-ResourcePerCC-r18 in a component carrier, and totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC or maxNumberPortsAcrossCC-r18 in active BWPs across all component carriers. If UE is configured with at least one CSI report setting with sub-configuration in the component carrier, UE shall use maxNumberTotalCSI-ResourcePerCC-r18; otherwise, UE shall use totalNumberPortsSimultaneousNZP-CSI-RS-PerCC. If UE is configured with at least one CSI reporting setting with sub-configuration in any component carrier, UE shall use maxNumberPortsAcrossCC-r18; otherwise, UE shall use totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC. NZP CSI-RS resource is active in a duration of time defined as follows. For aperiodic CSI-RS, starting from the end of the PDCCH containing the request and ending at the end of the scheduled PUSCH containing the report associated with this aperiodic CSI-RS. When the PDCCH candidates are associated with a search space set configured with searchSpaceLinkingId, for the purpose of determining the NZP CSI-RS resource active duration, the PDCCH candidate that ends later in time among the two linked PDCCH candidates is used. For semi-persistent CSI-RS, starting from the end of when the activation command is applied, and ending at the end of when the deactivation command is applied. For periodic CSI-RS, starting when the periodic CSI-RS is configured by higher layer signalling, and ending when the periodic CSI-RS configuration is released. If a CSI-RS resource is referred N times by one or more CSI Reporting Settings not configured with higher layer parameter csi-ReportSubConfigToAddModList, the CSI-RS resource and the CSI-RS ports within the CSI-RS resource are counted N times. - <Unchanged part omitted>

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | We agree with the comments online that these descriptions are duplicated with UE feature descriptions. This CR seems not needed. |
| Moderator | Thanks for the comment. As far as we know, the current UE capability discussion does not include the description related to this CR. @ZTE, Could you give the exact agreement/description for this part in UE feature? |
| Moderator2 | Some update (in other comment) to clarify that there is no impact on the behavior of legacy UE. |

Conclusion

TBD.

Reference

- R1-2406630, Remaining issues on network energy saving, 3GPP TS RAN1 #118, Samsung, August. 2024.

- R1-2406632, Correction on CSI related operation based on NES capability, 3GPP TS RAN1 #118, Samsung, August. 2024.

Appendix

CR in R1-2406632.