3GPP TSG RAN WG1 #119	                                         	           R1-2409814

Orlando, US, November 18th – 22nd, 2024

Agenda Item:	9.6.2

Source:	Moderator (Apple Inc.)

Title:	Summary #1 on LP-WUS operation in IDLE/INACTIVE mode

Document for:	Discussion/Decision

# Introduction

One of the objectives in Rel-19 WI on LP-WUS/WUR (RP-241824) is to support LP-WUS operation in idle/inactive mode:

- For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

This contribution summarizes the discussions on LP-WUS operation in IDLE/INACTIVE modes in RAN1#118.

Section 2 provides a summary of the agreements as the outcome of RAN1#119 discussions. Section 3 captures the proposals for online sessions. Section 4 documents the detailed discussions. Section 5 provides guidance for RAN1#120 discussions. Agreements from previous meetings and companies’ proposals from the contributions are captured in the Appendix.

# RAN1#118bis Agreements

# Proposals for Online Sessions

## Proposals for Monday 11/18 Online

Proposal 4-1r1

Confirm the following working assumption:

Working Assumption

From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode,

- The UE may start LP-WUS monitoring if

  - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB

  - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

- If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up

- The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if

  - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB

  - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

- FFS the serving cell measurement metrics

- The entry/exit thresholds can be configured separately for different types of LR

- It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB.

- Note: This may be revisited based on the RAN2/RAN4 discussion.

Note: this does not intend to impact any agreements and working assumptions made in RAN1/RAN2 after the working assumption. [The later agreements take precedence in case there is any conflict.]

Proposal 1-11

At least support the case that the number of beams for LP-WUS/LP-SS is the same as the number of SSB beams.

Proposal 1-9A

Do not support one-to-multiple mapping between the LP-WUS/LP-SS beams and the SSB beams.

Proposal 1-9B

Do not support multiple-to-one mapping between the LP-WUS/LP-SS beams and the SSB beams.

Proposal 1-10r1

Each LP-WUS or LP-SS is QCLed with an SSB with ‘typeA’, and when applicable, ‘typeD’ with the same SSB.

Proposal 1-8r1

It is supported that LP-WUS/LP-SS are transmitted only on a subset of the actually transmitted SSB beams via explicit signaling.

Proposal 3-1:

Do not support additional codepoints for LP-WUS.

# LP-WUS Operation in Idle/Inactive Mode

## LP-WUS configuration and monitoring

The following was agreed on the multiple MOs per beam per LO:

Agreement

When K (K>1) LP-WUS MOs are configured for each beam in an LO, down select between

- Option A: K LP-WUS MOs for a beam are divided into M (M >=1) groups of R LP-WUS MOs. A UE monitors all or some of the MO(s) within the K LP-WUS MOs.

  - For each group of R LP-WUS MOs, the same LP-WUS information is transmitted.

    - FFS how the same LP-WUS information is transmitted in the R LP-WUS MOs

  - Different LP-WUS information can be transmitted in different groups of R LP-WUS MOs.

  - M = 1 and M > 1 is supported.

  - FFS: detailed UE monitoring behavior

  - FFS R=1 or R>= 1

- Option B: K LP-WUS MOs for a beam are divided into G (G >= 1) groups of R*M (M >= 1) LP-WUS MOs. A UE monitors all or some of the MO(s) within one group of R*M LP-WUS MOs based on its subgroup ID.

  - Each group of R*M LP-WUS MOs is further divided into M groups of R LP-WUS MOs.

    - For each group of R LP-WUS MOs, the same LP-WUS information is transmitted.

      - FFS how the same LP-WUS information is transmitted in the R LP-WUS MOs

    - Different LP-WUS information can be transmitted in different groups of R LP-WUS MOs.

    - FFS: detailed UE monitoring behavior

  - M = 1 and M > 1 is supported.

  - FFS R=1 or R>=1

  - Note: this achieves the same purpose as “Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO.”

For LO-to-PO mapping, we had the following agreements:

Agreement

It is supported that UEs monitoring the same PO are divided into multiple subgroups, where LP-WUS can provide wake-up indication for each subgroup. Consider the following options:

- Option 1: UEs monitoring the same PO monitor the same LO

- Option 2: UEs corresponding to different POs monitor the same LO

- Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO.

- Combinations of the above options can be considered.

Regarding Options 2/3 on the mapping between LO and PO:

- Option 2: [1] Huawei/HiSi, [2] Nokia, [4] TCL, [5] CMCC, [8] Samsung, [9] Spreadtrum, [12] NEC, [22] Sharp, [23] Ericsson (up to 2 POs per LO)

- Option 3: [3] InterDigital, [4] TCL, [12] NEC, [14] CATT, [18] ETRI, [22] Sharp

[11] Apple: variation of Option 3 to support different LOs for different wake-up delay values for the same PO

Regarding Option A vs Option B:

- Option A: [1] Huawei/HiSi, [4] TCL, [5] CMCC, [7] ZTE, [9] Spreadtrum, [10] vivo, [11] Apple, [13] xiaomi, [22] Sharp, [23] Ericsson

R>=1:  [1] Huawei/HiSi (M>=3), [5] CMCC, [7] ZTE(?), [22] Sharp

R=1: [10] vivo, [23] Ericsson

- Option B: [2] Nokia, [3] InterDigital, [8] Samsung, [14] CATT, [15] Futurewei, [18] ETRI, [19] LG, [21] DCM, [24] QC, [27] Nordic

R>=1: [14] CATT, [15] Futurewei

R=1: [8] Samsung, [27] Nordic

[2] Nokia (M={1, 2}, G={1,2}, may increase the number of subgroups per PO to 64)

- [6] Panasonic: 32 subgroups for Option A, >=64 subgroups for Option B

Note that Option B and Option 3 in the above agreements achieve the same purpose, to support more subgroups per PO than the number of subgroups supported per LP-WUS.

Some companies support both Option B and Option 3, but note that in the end, at most one of them should be supported as they serve the same purpose.

Question 1-1

Please share your views on the need to use different sets of MOs/LOs to serve different subgroups of UEs, in either the form of Option B or Option 3. If yes, please indicate the maximum number of subgroups supported per LP-WUS.

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | We are open to consider but hope no impacts on LP-WUS design. |
| Sharp | If N-1 mapping is not supported, a larger M may be required for some case and it may require UE more power consumption to monitor all MOs, so we can support this case and we prefer option3 for its more simple and clear for the definition of LO. |
| xiaomi | We do not support Option B. With the same number of resource overhead and paging rate, option B has higher blocking probability and may cause longer latency. |
| vivo | Not support both option B and Option 3. We don’t see the benefit for the TDMed subgrouping via different sets of MOs/LOs as analyzed in our contribution. Instead, the required number of MOs per beam will increase as the number of TDMed group increases to meet target <1% FARc (false alarm from common codepoint). In result, it is challenging to reserve sufficient time resource for large number of MOs in a LO. Also, option B and option 3 are more complicit than option A. Hence, we want to support option A and support a LP-WUS to associate with up to 32 subgroups per PO. |
| Samsung | Since only 32 subgroups is supported in one PO, which means 6 + 1 bits is enough to indicate each subgroup and all the subgroup in one LP-WUS, option 3 is not needed. Option B is an optimization for option A, and the motivation of option B is to further reduce the number of UEs in a group corresponding to one subgroup ID it belongs to. It will bring some power saving gain by reducing the number of UEs being awakened which is not required to monitor PO. Besides, the information bit of one LP-WUS can be reduced accordingly since a LP-WUS is only used to indicate one group in G groups associated with one subgroup ID, and UEs belong to different groups associated with one subgroup ID or UEs belong to different subgroup ID has been already divided by different MOs. Even though more MO should be configured in this case, only small groups of LP-WUS MOs will be used to transmit LP-WUS. As a consequence, the resource overhead may even decrease. It should be noticed that the maximum number of subgroups supported per LP-WUS depends on how many subgroups ID associated with one G group in Option B. |
| Qualcomm | This may depend on the LP-WUS design under discussion in agenda 9.6.1. I.e., whether LP-WUS can support 32, less or more UE subgroups when the performance target of 1% FAR and 1% MDR is met. |
| Nokia1 | We think that option B could be helpful to keep the number of UEs per LP-WUS lower, like raised by others. In the end this will depend to the LP-WUS design. For option 3 would not be needed if option B is supported. |

Question 1-2

On whether to support repetitions across MOs (R>1), some companies think this can be achieved by repetitions within a MO, i.e., an MO with a longer duration, but with R=1.  Please share your views on whether to support R>1, and explain why.

| Company | Support R>1? (Y/N) | Comments |
| --- | --- | --- |
| ZTE, Sanechips | Y | Whether it is in the same MO or not depends on how many symbols for LP-WUS occupied to achieve msg3. If 4symbols and 2 repetition is enough to achieve msg3, then it can be defined within a MO. If more resources is required, then it could be in two MOs. |
| Sharp | Y | Channel coding of LP-WUS may be better than simple repetitions within a MO , and R larger than 1 can provide addition diversity gain for LP-WUS reception. |
| xiaomi | Y | We are more inclined to support repetitions with time gaps, since it can provide better time domain diversity. |
| vivo | N | The difference for the LP-WUS repetition across different MOs i.e., inter-MO repetition, and LP-WUS repetition within a MO i.e., intra-MO repetition, is whether to define certain gap between nominal LP-WUS repetitions. In NR, repetitions for PUCCH, PUSCH, and PDSCH are typically continuous in the time domain, occurring back-to-back without gaps for nominal repetitions. However, if there is a collision during one of the repetitions, such as a DL symbol colliding with a PUCCH or PUSCH repetition, the repetition may be postponed or dropped, resulting in a gap between the actual repetitions. Hence, for LP-WUS repetition, no gap between nominal LP-WUS repetitions should be taken as the baseline. Moreover, for inter-LP-WUS MO repetition, the benefit to define gap with value larger than 0 between nominal LP-WUS repetitions is unclear. And the number of LP-WUS MOs and the total length of a LO will increase due to inserted gaps between MOs for repetition. Hence, we want to support intra-MO LP-WUS repetition i.e., R=1 (repetition within a LP-WUS MO). |
| Samsung | N | In our understanding, there are quite a lot method in legacy to achieve a similar result as repetition, e.g., by channel coding. It cost much resources from NW perspective by support R>1, but since UE are not forced to combine these repetitions, the cost doesn't seem worth it. |
| Qualcomm | Y | From LP-WUS design perspective, we need to define a duration for the gNB to transmit just one copy of the LP-WUS. This can be naturally called a LP-WUS MO. On top of this, if replica of LP-WUS is transmitted, we can either define it as a MO or multiple MOs either with a replica. It seems the later is cleaner design. |

Value of M (maximum number of MOs per beam without considering the repetitions per PO, and within a group for Option B) for either Option A or Option B

- >= 3: [1] Huawei/HiSi

- M={1, 2}, G={1,2}: [2] Nokia

- 4: [10] vivo, [15] Futurewei

- 2 or 3: [11] Apple

- >=4: [23] Ericsson

A value of 3 as the middle point is proposed below to start the discussion.

Proposal 1-3

The maximum value of M for Option A or Option B (if either of them is supported) is 3.

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | {2,4} is preferred. |
| Sharp | ok |
| xiaomi | OK with the proposal. |
| vivo | We think 3 for the maximum value of M for option A is not sufficient for some cases. We prefer to support the maximum number of M=4 to address the cases with large total number of UEs or large UE paging rate in the system. |
| Samsung | Based on our analysis, when a LP-WUS associated with one PO with 32 subgroups, at most one subgroup is expected to be waken up in one DRX cycle with 63% cumulative probability and at most 2 subgroups are expected to be waken up in one DRX cycle with 86% cumulative probability by assuming per UE subgroup paging rate is 4% with 1% FAR. Since the probability is only 9.7% for 3 subgroups being paged under a same assumption, we think M=2 is enough to cover the case with higher probability. |
| Qualcomm | Support the proposal. |

Candidate values for wake-up delay:

- {400, 800, 40}: [1] Huawei/HiSi, [5] CMCC,

- {80, 400, 800}: [11] Apple

- {80, 460, 860}: [14] CATT (if iDRX cycle > 160ms)

- {80, 400, 720}: [15] Futurewei

- {20, 400, 800}: [16] OPPO, [27] Nordic

- {~800, ~400, >=30}: [21] DCM

- {10, 400, 800}: [25] MTK

- 400ms: [2] Nokia

- Panasonic: Before designing the exact candidate values, it is proposed to clarify in more detail levels on the impacting factors, especially the UE assumption of the time duration of synchronization in the reported wake-up delay.

- [10] vivo: Proposal 8: The reported value of UE’s wake-up delay capability should not include the sync/re-sync time.

- [24] QC: RAN1 discusses the number of SSBs for UE MR synchronization.

There are proposals to separately discuss the sync/re-sync time, i.e., not including this time into the wake-up delay reported by the UE. But in the earlier agreement, the wake-up delay was clearly defined as “Definition of wake-up delay: Minimum gap time between LP-WUS reception and MR to start PDCCH monitoring”, which implies that sync/resync time is already included. Therefore, the moderator’s suggestion is not to reopen the discussion. Instead, it can be considered when discussing the 3 candidate values for the wake-up delay.

Proposal 1-4

For the 3 candidate values for the wake-up delay capability report, support 400ms and 800ms, and the 3rd value is down-selected from {40, 80}.

| Company | Comments |
| --- | --- |
| CMCC | According to TR 38.869, the ramp-up time (i.e., 400/800ms) does not include the sync/re-sync time of MR: Therefore, we support to add an additional sync/re-sync time (e.g., 200ms as SSB is transmitted per 20ms) on top of ramp-up time for candidate values in ultra-deep sleep mode. |
| ZTE, Sanechips | OK |
| Vodafone | Fine with CMCC suggestion. We would also be interested to understand the assumption on the number of SSB for UE MR synchronization upon waking up. |
| xiaomi | OK with the proposal. |
| vivo | It is unclear to us to support 400ms and 800ms wake up delay capability which include both ramp up time and sync/re-sync time. Since in R18 LP-WUS SI, 400ms and 800ms are only considered for ramp up time. Given that, what is the assumption and how to derive the sync/re-sync time for these two candidate values? Since the similar discussion on whether the sync/re-sync time should be included in the wake-up delay capability occurs in A.I 9.6.3 connected mode procedure, the same logic is applicable for RRC idle/inactive mode procedure. |
| Samsung | Support in general. One clarification question for the proposal, what’s the difference between 40ms and 80ms, or in other words, what’s the intention of the additional 40ms in the case of 80ms? |
| DCM | Support. |
| Qualcomm | We agree with CMCC that it should be clarified whether UE resync time is considered in the wakeup delay capability. |
| Nokia1 | We agree with FL that the reported time should cover time needed for synchronization thus we may need to accommodate some tolerance in the reported capability to it e.g. by extending the time by 90 or 120ms |

For the offset value(s) between Lo and PO/PF, the following was agreed:

Agreement

For the offset value(s) between an LO and a reference PO/PF, consider the following options:

- Definition: The gap between an LO and a PO is considered to be no less than the wake-up delay a UE supports if the gap between the end of the last LP-WUS MO the UE monitors in the LO and the start of the PO is no less than the wake-up delay.

- Option 1: gNB configures a single offset value.

  - If the gap between an LO and the PO associated with the offset is no less than the wake-up delay a UE supports, the UE monitors the PO associated with the offset after receiving a wake-up indication in a LP-WUS.

  - Otherwise,

    - Option 1-1: the UE follows the legacy paging monitoring procedure.

    - Option 1-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay.

- Option 2: gNB configures one or multiple offset values.

  - For the same PO, each offset corresponds to a LO.

    - This does not preclude the possibility that the same LO may correspond to different POs with different offset values.

  - Option 2A: A UE does not expect that the gap between the LO associated with the largest offset and the corresponding PO is less than the wake-up delay the UE supports. The UE monitors the LO associated with one offset that has a gap between the LO and the corresponding PO no less than the wake-up delay.

    - This implies that the gNB needs to configure at least one offset value that is no less than the largest wake-up delay supported by the UEs.

    - FFS exactly how to choose the offset

  - Option 2B:

    - If the gap between the LO associated with the largest offset and the corresponding PO is no less than the wake-up delay a UE supports, the UE monitors the LO associated with one offset that has a gap between the LO and the PO associated with the offset no less than the wake-up delay.

      - FFS exactly how to choose the offset

    - Otherwise,

      - Option 2B-1: the UE follows the legacy paging monitoring procedure.

      - Option 2B-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay.

        - FFS exactly how to choose the offset

  - FFS the UE shall monitor the LO associated with additional offset(s)

Note: The PO mentioned above refers to legacy PO configured for the UE.

This is a summary of companies’ views:

- Option 1: [1] Huawei/HiSi, [2] Nokia, [6] Panasonic, [8] Samsung, [9] Spreadtrum, [12] NEC, [13] xiaomi, [14] CATT, [18] ETRI, [19] LG, [23] Ericsson, [25] MTK, [27] Nordic

  - Option 1-1: [1] Huawei/HiSi, [2] Nokia, [6] Panasonic, [8] Samsung, [9] Spreadtrum, [18] ETRI, [19] LG, [23] Ericsson, [27] Nordic (the UE follows the legacy paging monitoring procedure, if capability is larger than default paging cycle of iDRX)

  - Option 1-2: [12] NEC, [13] xiaomi, [25] MTK

- Option 2: [3] InterDigital, [4] TCL, [7] ZTE, [10] vivo, [11] Apple, [15] Futurewei, [19] LG (if multiple offsets are supported), [17] Sony, [21] DCM, [22] Sharp, [25] MTK

  - Option 2A: [3] InterDigital, [17] Sony, [21] DCM

  - Option 2B-1: [11] Apple, [19] LG (if multiple offsets are supported), [21] DCM

  - Option 2B-2: [10] vivo, [15] Futurewei, [25] MTK

  - Option 2B: [22] Sharp

Further, for Option 2B, ZTE and vivo have the following proposals, where a UE may monitor multiple LOs associated with different offset values for a PO.

| [7] ZTE | Proposal 6: Further clarify the offset selection based on following Alt1: The UE monitors the LO associated with the minimum offset that has a gap between the LO and the corresponding PO no less than the wake-up delay. Alt2: The UE monitors the LO associated with the minimum offset and larger offset that has a gap between the LO and the corresponding PO no less than the wake-up delay. |
| --- | --- |
| [10] vivo | Proposal 9: Support Option 2: gNB configures one or two offset values. If two offsets between LO and PO/PF are configured,  Each offset is configured corresponding to a wake-up delay category, i.e. deep sleep or ultra-deep sleep.  For UE with wake-up delay capability corresponding to ultra-deep sleep, it only monitors LP-WUS at a LO corresponding to the larger offset.  For UE with wake-up delay capability corresponding to deep sleep, it shall monitor LP-WUS in both two LOs corresponding to the two configured offsets. |

There are a good number of supporting companies for both Option 1 and Option 2. Based on the input, the proposal is to first reduce the number of candidate options.

Proposal 1-5

For the offset value(s) between an LO and a reference PO/PF, down-select among Option 1-1, Option 2B-1 and Option 2B-2.

| Company | Comments |
| --- | --- |
| CMCC | Okay and our preference is Option 1-1 which is the most straightforward way. |
| ZTE, Sanechips | Option 1-1 invalidates the UE capability report and option1 can be viewed as a special case of option2. also, option2 provide flexibility for the gNB. |
| Sharp | Ok |
| xiaomi | OK with the proposal. |
| ETRI | OK |
| vivo | At this stage, we don’t want to preclude option 1-2. We need to first consider about the down selection among option 1 and option 2 and then figure out the UE monitoring behavior. Hence, we cannot go with the proposal 1-5. Besides, we support option 2 and to address the concern on the increase of system overhead when two offsets are configured, UE with deep sleep wake-up delay capability shall monitor all LOs corresponding to all the configured offsets, as such, it up to the network to decide the LP-WUS transmission on which LO. And for deep sleep UE, it can get latency benefit at least for the case that only deep sleep UE needs to be paged. |
| Samsung | Agree to the proposal. |
| DCM | Support it. But option 2B-2 should be also precluded later because it conflicts legacy paging monitoring procedure, if UE stops LP-WUS monitoring and performs legacy paging monitoring procedure by exit condition for LP-WUS such as RRM based. |
| Qualcomm | We prefer to down-select among 1-1, 2A and 2B-1. 2B-2 is a redundant solution given that network the multiple offsets are already support by option 2 type of solutions. |

Detailed configurations for LO

| [2] Nokia | Proposal: LO reference time location is defined in relation to PF. One common offset from PF to a reference frame before the PF is configured for all LOs, together with offsets for each PO-LO pair from the reference frame to indicate the start of a LO. Proposal: Define starting symbol for each the LP-WUS MOs in LO. |
| --- | --- |
| [3] InterDigital | Proposal 4: Support LO configuration with the following. • Frame offset from the start of a frame to a paging frame. • Symbol offset from the start of the frame to the start of the first MO. |
| [5] CMCC | Proposal 6. If one LO is associated with multiple POs/PFs, support the reference PO/PF is the first PO/PF within the multiple POs/PFs. |
| [8] Samsung | Proposal 7: The time location of LP-WUS occasion can be determined by a reference point and an offset, wherein the reference point is the start of a reference PF associated with the LO if LO associated with all the POs in a PF, or the reference point is the start of the one or more PO(s) associated with the LO if LO associated with one or more PO(s) in one DRX cycle. |
| [10] vivo | Proposal 3: UE is required to monitor each of M groups per beam until its target LP-WUS is detected. And each of M groups per beam can be used for waking up any of 32 subgroups. |
| [11] Apple | Proposal 5: For multi-beam support of LP-WUS monitoring, the LP WUS MO(s) are defined in a similar way as for paging occasions and PEI occasions. |
| [12] NEC | Proposal 1: for RRC-IDLE/INACTIVE, the LP-WUS configuration indicates one or more LP-WUS frequency bands in a carrier, and for each LP-WUS frequency band, a periodicity and an offset of LOs are comprised in the configuration. |
| [14] CATT | Proposal 8: For Option1-1, the time location of LO can be determined by a reference point and an configured offset. Proposal 9: For multiple LOs are associated with one PO, the time location of LO can be determined based on the configured offsets, subgroup-ID and the number of LO associated with one PO. |
| [16] OPPO | Proposal 5: On the LO configuration for iDRX, if multiple offset values between a LO and a reference PO/PF are supported, similar definition as PEI could be considered, e.g. frame-level offset and symbol-level offset. |
| [23] Ericsson | Proposal 7 The reference PF/PO is the start of a first paging frame associated with the LP-WUS LO. The UE determines the start of the reference frame for LP-WUS LO using the single offset value configured to the UE. |

Miscellaneous

| [1] Huawei/HiSi | Proposal 7: For LP-WUS in IDLE/inactive mode, to minimize the number of codepoints per MO for a UE to monitor, the codepoint corresponding to all the subgroups is only monitored in some MOs. • In the MOs above, the maximum number of codepoints for a UE to monitor is 1 or no larger than the number of codepoints monitored in the other MOs. Proposal 8: To reduce the signalling/resource overhead, consider to configure a MO/LO which is monitored by all the UEs to detect the codepoint corresponding to all the subgroups. Proposal 10: The power offset between SSB and LP-WUS/LP-SS should be specified and can be configured by gNB in system information. |
| --- | --- |
| [5] CMCC | Proposal 7. RAN1 to clarify how the time on sync/re-sync procedure is addressed in the offset value. The following options can be considered: - Option 1: RAN1 to clarify that the time on sync/re-sync procedure is included in the candidate values for ultra-deep sleep mode. - Option 2: Up to gNB implementation to ensure the configured offset value includes a proper time duration for sync/re-sync procedure |
| [7] ZTE | Proposal 11: Discuss and decide using LP-SS or LP-WUS to achieve cell specific wake-up. • Slightly prefer to use LP-SS for cell specific wake-up • If LP-WUS is used to achieve cell specific wake-up, at least, 4POs would be needed to be associated to LO and LP-WUS design and coverage would be further impacted. Proposal 16: Consider the power offset between LP-SS and SSB • FFS the power control for LP=WUS |
| [8] Samsung | Proposal 17: Support configuring the EPRE ratio between LP-SS and SSB. |
| [13] xiaomi | Proposal 6: The power offset of LP-SS/LP-WUS compared to the power of SSB should be configured. Proposal 9: UE can be provided with assistant information about whether PEI provide more subgrouping information besides LP WUS subgrouping indication. |
| [14] CATT | Proposal 14: The UE falls back to legacy mechanism when the LP-WUS is in detection errors. |
| [19] LG | Proposal #16: For UE triggered by LP-WUS to monitor a PO, introducing a time window based on the reception of LP-WUS. The time window has the following characteristics:  UE receives the existing PO only within the time window  UE transits to sleep state at the end of the window  The length of window is determined based on  The number of upcoming paging occasions  The number of slot/frame configured by SIB  FFS: LO mapped to dynamic PO |
| [23] Ericsson | Proposal 4 Type of WUR capability supported in the cell, e.g., support for both OFDM and OOK-based WUR or OFDM-based WUR only, should be configurable by the NW. |

A few companies proposed to make the power offset of LP-SS/LP-WUS w.r.t. the power of SSB configurable. The benefit includes (1) AGC and (2) make the RSRP/RSRQ of LP-SS based measurement and MR measurement comparable. For the 2nd part, if the thresholds configured by the gNB already consider the power difference, it does not seem necessary for the UE to be aware of it?

Question 1-7

Please share your views on the need to configure the EPRE ratio between LP-SS and SSB.

| Company | Comments |
| --- | --- |
| CMCC | This may strongly related to the discussion on coverage enhancement in AI 9.6.1 (i.e. whether power boosting is introduced for LP-SS), We can discuss this later. |
| ZTE, Sanechips | We are OK. |
| Xiaomi | We support to configure the EPRE ratio between LP-SS and SSB. Besides the reasons listed in (1) and (2), we think power offset is also useful for UE to do cell selection. If UE would like to save more power, it can select a cell with LP WUS coverage as large as possible, so it will select a cell with a relatively bigger LP-SS power offset. |
| Qualcomm | We support the configuration of EPRE ratio between LP-SS and SSB. |
| Nokia1 | For RSRP determination we need a reference, and as the reference power is already signaled to SSB, it would seem natural to define this via offset. |

### Multi-beam operation

LP-WUS/LP-SS can be transmitted via a subset of all SSB beams

- Yes: [1] Huawei/HiSi, [3] InterDigital, [10] vivo (explicit signaling), [19] LG

There are 4 companies proposing to support LP-WUS/LP-SS is transmitted on a subset of SSB beams. This was also discussed in the previous meetings without any agreement. Let us aim to conclude on this issue in RAN1#119.

Proposal 1-8

It is supported that LP-WUS/LP-SS are transmitted only on a subset of the SSB beams.

| Company | Comments |
| --- | --- |
| CMCC | Fine with the proposal. |
| ZTE, Sanechips | - The overhead benefits can be implemented by gNB implementation. |
| Sharp | support |
| xiaomi | OK with the proposal. |
| vivo | Support. One of the benefits to support LP-WUS/LP-SS to be transmitted on a subset of the SSB beams is to reduce the overhead and time duration of LP-WUS/LP-SS. |
| Samsung | Support. |
| Qualcomm | We do not see benefit and need to support this. |
| Nokia1 | We would think that there could be merit to have some SSB level granularity for the deployment, but it does not always need to be binary. Effectively, SSB specific offsets could be used to prevent the UE to do LP-WUS monitoring under particular SSB. |
| Moderator | Companies supporting the proposal: InterDigital, HW, CMCC, vivo, Samsung, xiaomi Some companies suggest that gNB can do this by implementation without any spec change. Therefore, the proposal is updated to cover the case of explicit signaling for further discussion. Proposal 1-8r1 It is supported that LP-WUS/LP-SS are transmitted only on a subset of the actually transmitted SSB beams via explicit signaling. |

One-to-multiple association between LP-WUS/LP-SS and SSB: [8] Samsung, [10] vivo

Multiple-to-one-mapping between LP-SS and SSB: [10] vivo

There is very limited interest in supporting one-to-multiple or multiple-to-one mapping between LP-WUS/LP-SS and SSB. Given these are not essential for the operation, moderator’s suggestion is not to consider them further.

[CLOSED] Proposal 1-9

Do not support one-to-multiple or multiple-to-one mapping between the LP-WUS/LP-SS beams and the SSB beams.

| Company | Comments |
| --- | --- |
| CMCC | Okay |
| ZTE, Sanechips | Fine with FL proposal |
| xiaomi | OK with the proposal. |
| Samsung | Support one-to-multiple case can improve the beam sweeping efficiency of LR and to reduce the resource overhead since the coverage for LP-WUS/LP-SS is smaller than SSB. It has the benefit compared with one-to-one mapping. Therefore, we hope to leave a door to discuss it further. |
| Qualcomm | Support the FL proposal. |
| Nokia1 | Support the FL proposal |
| Moderator | The interest seems limited. Samsung, vivo, Panasonic would like to consider these cases further. It may make the discussion easier to separate the two cases, as the motivation is different. Therefore we have Proposal 1-9A and 1-9B below for further discussion. Discussion closed. |

Proposal 1-9A

Do not support one-to-multiple mapping between the LP-WUS/LP-SS beams and the SSB beams.

| Company | Comments |
| --- | --- |
| Moderator | The motivation seems questionable to support one-to-multiple mapping as LP-WUS/LP-SS has limited coverage, and it does not seem to make sense to use wider beam for LP-WUS/LP-SS. |

Proposal 1-9B

Do not support multiple-to-one mapping between the LP-WUS/LP-SS beams and the SSB beams.

| Company | Comments |
| --- | --- |
| Moderator | The main question is whether we want to support narrower beams for LP-WUS/LP-SS, given that it leads to larger overhead. |

On the QCL between LP-WUS/LP-SS and SSB, the following was agreed:

Agreement

Each LP-WUS is QCLed with one SSB. Each LP-SS is QCLed with one SSB.

- FFS QCL Type A or Type C and/or Type D

- FFS implicit QCL determination or some signaling is required

On the QCL type between LP-WUS/LP-SS and SSB:

- Type C, and Type D when applicable: [1] Huawei/HiSi, [2] Nokia, [3] InterDigital (LP-SS), [7] ZTE (CD-SSB)

- Type A, and Type D for FR2: [3] InterDigital (LP-WUS), [5] CMCC (LP-WUS) , [23] Ericsson

- Type C: [13] xiaomi (Type A for LP-WUS vs LP-SS)

- [5] CMCC: Type D for FR2 for LP-SS (FFS for FR1)

- [10] vivo, [21] DCM: at least Type D

- [27] Nordic: Type A

Proposal 1-10

Each LP-WUS or LP-SS is QCLed with an SSB with ‘typeC’, and when applicable, ‘typeD’ with the same SSB.

| Company | Comments |
| --- | --- |
| CMCC | At least support Type D in FR2 scenario Open to discuss whether type A/B/C is applicable. |
| ZTE, Sanechips | Yes. I guess this is for idle/inactive state. |
| xiaomi | OK with the proposal. |
| Samsung | Agree in general. One clarification question for the wording, what does it mean by mentioned “when applicable”, in our understanding, different company may have different understanding for this wording. |
| Qualcomm | We support Type A and Type D when applicable which following paging PDCCH. |
| Nokia1 | We see that we can go either by defining support TypeA and TypeD (where applicable) or follow the wording we have for Type0-CSS that UE may assume these are QCL’ed. |
| Moderator | There was offline comments saying that we should follow the QCL type between PDCCH and SSB, which is Type A. The proposal is updated to the following: Proposal 1-10r1 Each LP-WUS or LP-SS is QCLed with an SSB with ‘typeA’, and when applicable, ‘typeD’ with the same SSB. |

There was an earlier agreement that:

Agreement

At least support 1:1 association between LP-WUS MO(s)/LP-SS transmissions and SSB beams.

But companies have different understanding on this agreement, whether it means the same number of beams for LP-WUS/LP-SS and SSB, or it means each LP-WUS/LP-SS has an associated SSB beam.

Below is a new proposal for discussion.

Proposal 1-11

At least support the case that the number of beams for LP-WUS/LP-SS is the same as the number of SSB beams.

| Company | Comments |
| --- | --- |

## eDRX

Given that now we have a rough idea the LP-WUS design for iDRX case, we can revisit the support for eDRX.

| [1] HW/HiSi | Proposal 12: LP-WUS with e-DRX is deprioritized. |
| --- | --- |
| [2] Nokia | Proposal: If LP-WS operation with eDRX is supported and UE is configured with eDRX and LP-WUS, the LP-WUS monitoring is determined based on paging occasions so that UE can monitor LP-WUS associated to the POs that are within the PTW. |
| [7] ZTE | Proposal 12: For eDRX, discuss • the LO periodicity could be the same as IDRX • the LO periodicity could be the same as eDRX periodicity |
| [13] xiaomi | Proposal 12: Support LP WUS applied for eDRX to reduce paging latency. |
| [14] CATT | Proposal 15: When eDRX is configured, LP-WUS is monitored within PTW. |
| [19] LG | Proposal #17: It is not necessary to consider eDRX for LP-WUS operation. |
| [21] DCM | Proposal 7: • RAN1 firstly discuss LP-WUS operation with I-DRX before eDRX |

Question 2-1

Please share your views on whether to support LP-WUS for eDRX, and explain why.

| Company | Comments |
| --- | --- |
| CMCC | Okay to discuss, but may not the high priority issue at present. |
| ZTE, Sanechips | Should be supported. It is important for RedCap, or IoT cases. Also, the benefits already declared in the TR. |
| Vodafone | Fine to discuss |
| xiaomi | We support LP WUS for e-DRX. The main drawback of e-DRX is the long latency. Using LP WUS to wake up UE outside PTW can reduce latency significantly. |
| vivo | LP-WUS for eDRX should be deprioritized since we still have lots of work to do with LP-WUS procedure based on I-DRX. |
| Samsung | If the UE operates in eDRX with an eDRX cycle no longer than 1024 radio frames, it monitors POs as defined for iDRX with configured eDRX cycle. In this case, LP-WUS should be supported without any changes. If eDRX cycle longer than 1024 radio frames, PTW is configured for PDCCH monitoring, which means only PO located in PTW can be monitored by UE with configured eDRX cycle. Since the PTW is UE-specific and determined by a Paging Hyperframe, LO can be configured to trigger PO monitoring in one or more PTW, which will reduce unnecessary PO monitoring in PTW. Based on the explanations, we think LP-WUS can be supported for eDRX and LP-WUS can be configured to trigger PO monitoring in one or more PTW. |
| Qualcomm | For Rel-19, either LP-WUS is not supported for eDRX or LP-WUS is monitored for POs in the PTW. |
| Nokia1 | Fine to deprioritize. |

## LP-WUS payload size and content

Additional codepoints:

- No: [1] Huawei/HiSi, [3] InterDigital, [10] vivo, [15] Futurewei

- Yes: [7] ZTE (codepoints for two subgroups and possibly others), [22] Sharp (Support subgroup combinations with special codepoint value)

Number of information bits in an LP-WUS

- [3] InterDigital, [4] TCL: 8 bits

- [8] Samsung, [25] MTK, [27] Nordic: 6 bits

- [15] Futurewei: 12 bits

- [17] Sony: 16 bits

Here are a few other proposals related to different :

| [19] LG | Consider extending the maximum number of subgroups per PO to 64 or 128 Proposal #14: UEs with different wake-up delay should be assigned to different UE subgroups Proposal #15: UEs having shorter wake-up delay can be assigned to a specific subgroup(s) separately  The number of those subgroups can be separately configured  Wake-up delays below a certain threshold are considered as shorter wake-up delay |
| --- | --- |
| [23] Ericsson | Proposal 16 The number of LP-WUS information bits per PO is ceil(log2(N+1)) bits, where N is the configured number of UE subgroups per PO. Proposal 17 The maximum number of codepoints for UE subgroup indication per PO is 2*N, where N is the configured number of UE subgroups per PO. |

Moderator does not see the immediate need to discuss the number of information bits given that the code point method has been agreed. Whether we need the concept of the number of information bits at all, or we just discuss the number of the code points to support, is unclear at this point. We can hold this until the LP-WUS signal design gets clearer.

Question 3-1

Please share your views on whether additional codepoints need to be supported for LP-WUS in addition to what were agreed in RAN1#118bis.

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | Yes. At least some of 2 subgroups codepoints should be supported. The first cluster assumes per subgroup paging rate 3% The second cluster assumes per subgroup paging rate 1% It can be seen that 2 subgroups wake-up is not a corner case. |
| xiaomi | Currently we do not see the needs for additional codepoints. |
| vivo | Not support additional codepoints. Marginal benefit to UE power saving gain is observed if using additional codepoints, especially when sufficient MOs, e.g., 4 are configured, which ensures <=1% false wake up rate caused by common codepoints. Moreover, if support additional codepoints, more codepoints shall be detected by UE, resulting in the increase of FARo from noise or LP-WUS length to keep <=1% FARo. |
| Samsung | We would like to know the intention to support additional codepoint. |
| Qualcomm | The benefit of 2 or more UE subgroups is not clear to us. |
| Nokia1 | With M>1 this does not seem necessary |
| Moderator | Given the comment that having M>1 can largely address the issue of indicating multiple subgroups in a LO, the following is proposed: Proposal 3-1: Do not support additional codepoints for LP-WUS. |

## RRM measurement

This is a summary of companies’ views on the RRM measurement related issues:

| [1] Huawei/HiSi | Proposal 11: Confirm the working assumption for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode - The detailed metrics to be used for serving cell measurement can be left to RAN2/RAN4 discussion. Proposal 13: Confirm the working assumption that from RAN1 perspective for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively. Proposal 14: The RRM metrics for OOK-based receiver, including LP-RSRP and LP-RSRQ can be also applied to OFDM-based receiver. |
| --- | --- |
| [2] Nokia | Proposal: Once LP-SS sequence design has been finalized, verify if the LP-RSRP and LP-RSRQ metric definitions need to be revisited. Proposal: LP-RSRP and LP-RSRQ measurements should be associated to a particular beam Proposal: For LP-RSRP and LP-RSRQ measurements, it should be possible to network to configure a sub-set of LP-SS’s from which LP-RSRP and LP-RSRQ can be measured. Proposal: For controlling LP-WUS monitoring in SSB level, consider some flexible methods that would enable UEs in very good conditions to monitor LP-WUS even in cell edge area, and allowing more relaxed conditions for cell centre operation. Proposal: For LR that is capable of receiving PSS/SSS, RAN1 confirms the working assumption of re-using the measurement quantity definition but separates the definitions of measurement quantities for LR measurements based on SSB (from the MR measurement quantities) for configuration clarity. |
| [3] InterDigital | Proposal 13: For the entry condition, define procedures efficiently triggering LR based measurement for identifying whether LR-based thresholds are satisfied. Proposal 14: For the exit condition, the other conditions such as timer are supported to ensure proper UE behavior. Proposal 15: SS-RSRP and SS-RSRQ are reused for OFDM based LR. • LP-SSS-RSRP and LP-SSS-RSRQ are not defined in the specifications. Proposal 16: LP-SS based RRM measurement is supported for OFDM based LR. Proposal 17: Introduce definitions of LP-RSRP and LP-RSRQ for OFDM based LR to support LP-SS based RRM measurement. Proposal 18: Introduce the following definitions of LP-RSRP and LP-RSRQ for OFDM based LR. • LP-RSRP is defined as the linear average over the power contributions (in [W]) of the resource elements that carry LP-SSs. • LP-RSRQ is defined as the ratio of N×LP-RSRP / LP-RSSI, where N is the number of resource blocks in the LP-RSSI measurement bandwidth. |
| [6] Panasonic | Proposal 4: UE starts LP-WUS monitoring, if one or more following conditions are met, depending on the configuration, - the serving cell RSRP measurement performed by the MR is above an entry threshold - the serving cell RSRP measurement gap between MR and LR based measurement is below a threshold - the serving cell RSRP measurement performed by the LR is above an entry threshold - the serving cell RSRQ measurement performed by the LR is above an entry threshold UE falls back to legacy PO monitoring and may stop LP-WUS monitoring, if one or more following conditions are met, depending on the configuration, - the serving cell RSRP measurement performed by the LR is below an exit threshold - the serving cell RSRP measurement performed by the MR is below an exit threshold, assuming relaxed MR-based measurement - the serving cell RSRP measurement gap between MR and LR based measurement is above a threshold, assuming relaxed MR-based measurement - the serving cell RSRQ measurement performed by the LR is below an exit threshold Proposal 5: LR performs serving cell measurement, if one or more following conditions are met, depending on the configuration, - the serving cell RSRP measurement performed by the MR is above a threshold - the serving cell RSRP measurement gap between MR and LR based measurement is below a threshold - the serving cell RSRP measurement performed by the LR is above a threshold - the serving cell RSRQ measurement performed by the LR is above an entry threshold When LR performs serving cell measurement offloaded from NR, for serving cell measurement relaxation performed by MR, both Option 1 and Option 2 are supported, based on configuration and subject to conditions, e.g., the serving cell measurement performed by MR is above or below a threshold o Option 1: MR does not perform serving cell measurement o Option 2: MR performs serving cell measurement with relaxation |
| [7] ZTE | Proposal 1: Consider another exit condition in the WA, UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if • The UE has not received LP-WUS for a long time Proposal 2: • For OFDM WUR, • LP-SSS-RSRP is defined as the linear average over the power contributions (in [W]) of the resource elements that carry secondary synchronization signals. • LP-SSS-RSRQ is defined as the ratio of N×SS-RSRP / NR carrier RSSI • SMTC is not needed for OFDM WUR and OOK WUR for serving cell measurement • OFDM WUR and OOK WUR serving cell measurement can be applied at least for RRC_IDLE intra-frequency and RRC_INACTIVE intra-frequency |
| [8] Samsung | Proposal 14: If the SDT procedure is ongoing in RRC_INACTIVE state, UE is not expected to monitor LP-WUS even though the entry condition is satisfied. Besides, UE should stop LP-WUS monitoring when UE expects to transmit RACH relevant signal/channel or SDT. Proposal 15: A UE stops monitoring LP-WUS when the UE haven’t received LP-WUS for a configured time duration. |
| [11] vivo | Proposal 11: The network configures the measurement time resource for LP-SSS-RSSI, which can be different from NR SMTC. |
| [13] xiaomi | Proposal 11: For OFDM-based LR, reuse the LP-SS based LP-RSRP/LP-RSRQ definition of OOK-based LR. |
| [19] LG | Proposal #18: Whether the UE can receive overlaid OFDM sequence over LP-WUS or not should be considered to specify entry/exit condition of LP-WUS monitoring. Proposal #19: Consider the following aspects about RRM measurement relaxation of the main radio by offloading to LP-WUR.  The UE can measure cell quality by LP-WUR while the main radio is in ultra-deep sleep state.  The existing RRM measurement procedure by the main radio can be triggered based on the serving cell measurement by LP-WUR.  If the UE stops LP-WUS monitoring due to the serving cell measurement which is below exit threshold(s) by LP-WUR, the UE starts to measure either a serving cell or a neighbor cell by the main radio. Proposal #20: If RRM measurement relaxation by using LP-WUR is supported, the UE with OFDM-based LP-WUR can perform RRM measurement by receiving LP-SS and/or PSS/SSS. |
| [21] DCM | Proposal 10:  For the definition of LP-RSRP, LP-RSSI and LP-RSRQ, no need of further discussion Proposal 11:  For RRM metric based on OFDM-based LP-WUR,  No need to introduce LP-SSS-RSRP and LP-SSS-RSRQ in the spec  Confirme the working assumption with following update from legacy SS-RSRP/SS-RSRQ  The measurement time resources(s) restriction by SMTC window duration is not applicable.  The measurement time resource(s) for SS-RSRP/RSSI are confined within a duration, e.g., duration that the OFDM-based LP-WUR receiver is activated  If SS-RSRP or SS-RSRQ is measured by OFDM-based LP-WUR, following is applicable, - RRC_IDLE intra-frequency - RRC_INACTIVE intra-frequency |
| [23] Ericsson | Proposal 20 Entry/exit conditions should be configurable, e.g., as part of the LP-WUS configuration. Proposal 21 It should be possible to configure different entry/exit conditions based on supported WUR types, if any. Proposal 22 Confirm the updated working assumption on entry/exit condition for LP-WUS monitoring where the update is based on latest RAN1 and RAN2 discussions. Proposal 23 If serving cell RRM using LP-SS is supported, only scenarios that are suitable for operation with low LP-SS overhead and NW energy impact should be considered for RRM. |
| [24] QC | Proposal 9: Support the UE monitoring PEI after receiving LP-WUS indicating wakeup to enhance the UE subgrouping granularity. Proposal 10: Confirm the RAN1 #117 working assumption on the RRM measurement metrics based on SSS for OFDM-based LP-WUR. |

Whether to confirm the following working assumption was discussed in RAN1#118bis. It seemed that the main concern was whether this may conflict with any agreements made in RAN2 afterwards. However, no clear conflict has been identified. It is recommended to confirm the working assumption.

Proposal 4-1

Confirm the following working assumption:

Working Assumption

From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode,

- The UE may start LP-WUS monitoring if

  - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB

  - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

- If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up

- The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if

  - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB

  - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

- FFS the serving cell measurement metrics

- The entry/exit thresholds can be configured separately for different types of LR

- It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB.

- Note: This may be revisited based on the RAN2/RAN4 discussion.

Note: this does not impact any agreements and working assumptions made in RAN1/RAN2.

| Company | Comments |  |
| --- | --- | --- |
| CMCC | Generally fine with the proposal. |  |
| ZTE, Sanechips | OK. |  |
| Vodafone | “Serving cell” would need to be revisited for both entry and exit if operation of LR and MR in different carriers is enabled. |  |
| Sharp | On the second sub-bullet of UE action of monitoring legacy PO monitoring , considering the un-sync of LP-WUS monitoring of gNB and UE side, to avoid missing of paging, UE can still monitor legacy PO for some time, and the time can be configured by gNB as hysteresis. |
| xiaomi | OK with the proposal |  |
| ETRI | Support the proposal |  |
| Samsung | Support the proposal |  |
| DCM | Support. |  |
| Qualcomm | RAN2 and RAN4 already discussed our WS and made progress. We prefer to wait for RAN2/4 to send us the LS first. |  |
| Moderator | Based on the offline comments, the note was updated as follows. Note: this does not intend to impact any agreements and working assumptions made in RAN1/RAN2 after the working assumption. [The later agreements take precedence in case there is any conflict.] |  |

It was discussed in RAN1#118bis whether to remove the restriction of using SMTC window for LP-SSS-RSRP/RSSI measurement. This proposal seems to make sense, but there was concern whether this should be decided by RAN4. Let us aim to conclude on this issue in this meeting.

Proposal 4-2

The measurement time resource(s) for LP-SSS-RSRP and LP-SSS-RSSI can be separately configured or defined from that for SS-RSRP and SS-RSSI.

(If you do not agree with this proposal, please comment on whether we can send an LS to RAN4 on this.)

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | Not support this proposal. We can make following conclusion “There is no consensus to support measurement time resource(s) for LP-SSS-RSRP and LP-SSS-RSSI” And send the LS to RAN4. whether support that is up to RAN4. |
| xiaomi | OK with the proposal |
| vivo | Support proposal 4-2. There is no need to restrict the time measurement resource for LP-SSS-RSRP and LP-SSS-RSSI to be within the SMTC. Hence, for flexibility consideration, the network can configure the measurements time resource for LP-SSS-RSRP and LP-SSS-RSSI different from the current NR SMTC window. |
| Samsung | The measurement time resource can be decided in RAN4 and support to send the LS to RAN4. |
| DCM | We are OK with that either it is supported in RAN1 perspective or send LS to RAN4 about it. We are not OK with that it is not supported in RAN1 perspective. |
| Qualcomm | This should be left to RAN4 to discuss. |
| Nokia1 | Maybe it would be good to first discuss how would the measurement time resource be configured without the SMTC or is the intention fully omit the time domain configuration? |

A few companies proposed to add a timer for the exit condition for LP-WUS. Please share your views on the following proposal.

Proposal 4-3

UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if	the UE has not received LP-WUS for a long time.

| Company | Comments |
| --- | --- |
| CMCC | Okay |
| ZTE, Sanechips | Yes. if the LP-WUS can not be received for a long time, but the LP-SS, especially for SSS, measurement still satisfies the requirement, the following situations may happen - The NW has updated the configuration, e.g., threshold, but the UE is in LP-WUS mode and did not receive the SIB update, or the LP-WUS wake-up indication can not reach every UE. - The measurement threshold for SSS/LP-SS can not match the LP-WUS coverage due to the measurement uncertainty especially when the SSS and LP-WUS is not in the same frequency location. Therefore, the FL proposal make sense and provide a safe fallback mechanism. |
| Vodafone | Support this proposal as it provides a straightforward exit condition increasing the robustness of the system. |
| xiaomi | OK with the proposal |
| ETRI | Support the proposal |
| vivo | Not support proposal 4-3. Both LP-WUS and PEI monitoring is up to UE implementation, no need to additional define the exiting condition based on timer/duration except for threshold based i.e. the working assumption given in proposal 4-1. |
| Samsung | The motivation for the proposal is to ensure UE can wake up to monitor PO in case the UE missed LP-WUS. |
| DCM | No necessity. In current understanding, NW does not transmit LP-WUS if there is no UEs to be paged. The proposal procedure would conflict this thought. Moreover, if a UE is in LP-WUS coverage and UE missed to detect LP-WUS, UE can detect it in the next I-DRX cycle. If a UE is not in LP-WUS coverage and UE can not detect LP-WUS by poor condition, UE will stop LP-WUS monitoring and performs legacy paging monitoring procedure by exit condition for LP-WUS based on RRM. Thus, the proposal should not be supported. |
| Qualcomm | This should be left to UE implementation. |
| Nokia1 | We do not seen necessity for this. RAN2 can discuss whether they need some exit conditions. |

## Other

Topics that are expected to be discussed in AI 9.6.1:

- Whether to have preamble for LP-WUS

- Bandwidth of LP-WUS

- Periodicity of LP-SS

- Overlaid OFDM sequence for LP-SS

Topics that are deprioritized in this meeting:

- LR and MR on the same or different carrier/band

#### Question 5-1:

Please provide any other comments/suggestions you may have, e.g., the topics that you think are important to be discussed now.

| Company | Comment |
| --- | --- |
| ZTE, Sanechips | We can discuss if there are some spec impacts when separate band is supported in RAN1. We can discuss whether the cell specific wake-up is needed. According to our observation: - If LP-WUS can indicate all subgroups per PO to wake-up, the system overhead occupied only by LP-WUS could be up to 10% |

# Guidance for RAN1#119 Discussions

# References

- R1-2409424	Procedures and functionalities of LP-WUS in IDLE/INACTIVE mode	Huawei, HiSilicon

- R1-2409445	LP-WUS operation in IDLE/Inactive mode	Nokia

- R1-2409457	Discussion on LP-WUS operation in IDLE/INACTIVE modes	InterDigital, Inc.

- R1-2409477	Discussion on LP-WUS Operation in IDLE/INACTIVE modes	TCL

- R1-2409521	Discussion on LP-WUS operation in IDLE/INACTIVE modes	CMCC

- R1-2409547	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Panasonic

- R1-2409560	Discussion on LP-WUS operation in IDLE/INACTIVE mode	ZTE Corporation, Sanechips

- R1-2409606	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Samsung

- R1-2409645	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Spreadtrum, UNISOC

- R1-2409690	Discussion on LP-WUS operation in IDLE/INACTIVE modes	vivo

- R1-2409812	LP-WUS operation in IDLE/INACTIVE modes	Apple

- R1-2409861	Discussion on LP-WUS operation in RRC IDLE/INACTIVE mode	NEC

- R1-2409905	Discussion on LP-WUS operation in Idle/Inactive modes	Xiaomi

- R1-2409950	System design and procedure of LP-WUS operation for UE in IDLE/Inactive Modes	CATT

- R1-2410037	Discussion on LP-WUS Operation in IDLE/INACTIVE Modes	FUTUREWEI

- R1-2410072	Further consideration on LP-WUS operation in RRC_IDLE/INACTIVE modes	OPPO

- R1-2410232	LP-WUS operation in IDLE / INACTIVE modes	Sony

- R1-2410274	Discussion on LP-WUS operation in IDLE/INACTIVE modes	ETRI

- R1-2410295	Discussion on LP-WUS operation in IDLE/INACTIVE modes	LG Electronics

- R1-2410300	On LR and MR operating frequencies	Vodafone, Deutsche Telekom

- R1-2410398	Discussion on LP-WUS operation in IDLE/INACTIVE modes	NTT DOCOMO, INC.

- R1-2410420	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Sharp

- R1-2410445	LP-WUS operation in IDLE and INACTIVE modes	Ericsson

- R1-2410487	LP-WUR operation in idle and inactive modes	Qualcomm Incorporated

- R1-2410511	LP-WUS operation in IDLE/INACTIVE modes	MediaTek Inc.

- R1-2410556	Discussion on LP-WUS operation in Idle/Inactive modes	Lenovo

- R1-2410568	On LP-WUS operation in IDLE/Inactive	Nordic Semiconductor ASA

# Appendix A: Agreements from previous meetings

## RAN1#116 agreements

Agreement

Multi-beam operations are supported for LP-WUS and LP-SS for idle mode

Agreement

LP-WUS occasions (LOs) are defined for LP-WUS monitoring.

- Each LO has one or more LP-WUS monitoring occasions (MOs), where UE can monitors for LP-WUS transmission in each of the LP-WUS MOs.

  - Different LP-WUS MOs may correspond to different beams in multi-beam operation

  - It is not precluded that FFS whether or not each LO is defined as a time window that covers the corresponding LP-WUS MOs

  - FFS details

- It is at least supported that a UE monitors LOs with a configured periodicity.

- Each UE has a periodicity for LO monitoring, and it is at least supported that a UE monitors one LO per period.

  - FFS: A UE does not expect its LP-WUS monitoring occasions overlapping in time

  - FFS: monitoring of multiple more than one LOs per period e.g. if LP-WUS common to all UEs is supported or in case of eDRX (if supported)

- FFS eDRX, if supported

Agreement

For the case where a UE supports PEI and PEI is configured by the gNB, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not.

Agreement

It is supported that the UE monitors the legacy PO after receiving LP-WUS indicating wake-up.

- FFS: support of UE monitoring dynamic PO

Conclusion

For idle/inactive mode, how to map a UE to a subgroup ID for LP-WUS is left to RAN2 to decide.

## RAN1#116bis agreements

Agreement

For multi-beam operation of LP-WUS, UE assumes the same LP-WUS information payload is repeated in all transmitted beams corresponding to LP-WUS

- the selection of the beam(s) for the reception of the LP-WUS is up to UE implementation

Agreement

Each LO consists of N * K LP-WUS MOs, where N is the number of beams corresponding to LP-WUS, and K is the number of LP-WUS MOs for each beam.

- Option 1: K = 1

- Option 2: K can be larger than or equal to 1

  - FFS if more than 1 LP-WUS is transmitted from the same beam, whether the information in these multiple LP-WUS is always the same or can be different

Agreement

From RAN1 perspective, at least the following metrics can be supported for RRM serving cell measurement performed by OOK-based receiver based on LP-SS:

- LP-RSRP

  - LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols.

    - FFS: How to determine the received power of LP-SS in OOK ON symbols

- LP-RSRQ

  - LP-RSRQ = LP-RSRP/LP-RSSI

  - For the definition of LP-RSSI for determination of LP-RSRQ, further consider the following options:

    - Option 1: LP-RSSI is the linear average of total received power in all LP-SS OOK symbols.

    - Option 2: LP-RSSI is the linear average of total received power in LP-SS OOK OFF symbols.

    - Option 3: LP-RSSI is the linear average of total received power in LP-SS OOK ON symbols.

- FFS: LP-SINR, Power ratio of OOK-ON symbol and OOK-OFF symbol

Note: RAN1 will send an LS to RAN2 and RAN4 on the measurement metrics that can be supported from RAN1 perspective, to facilitate RAN2/RAN4 discussions. The exact metrics for OOK-based receiver to be used and defined in the specifications depend on the outcome of [RAN1]/RAN2/RAN4 discussions.

Working Assumption

From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode,

- The UE may start LP-WUS monitoring if

  - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB, and/or

  - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

- If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up

- The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if

  - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB, and/or

  - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

- FFS the serving cell measurement metrics

- The entry/exit thresholds can be configured separately for different types of LR

- It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB.

- Note: This may be revisited based on the RAN2/RAN4 discussion.

Conclusion

LP-SINR is not considered further as a metric for RRM serving cell measurement for OOK-based receiver.

## RAN1#117 agreements

Conclusion

Regarding the “FFS: How to determine the received power of LP-SS in OOK ON symbols” for LP-RSRP, no additional work in RAN1.

Agreement

It is supported that UEs monitoring the same PO are divided into multiple subgroups, where LP-WUS can provide wake-up indication for each subgroup. Consider the following options:

- Option 1: UEs monitoring the same PO monitor the same LO

- Option 2: UEs corresponding to different POs monitor the same LO

- Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO.

- Combinations of the above options can be considered.

Agreement

For LP-SS based LP-RSRQ, LP-RSRP and LP-RSSI are measured within the same bandwidth.

Agreement

For LP-RSSI definition for LP-RSRQ, the following is agreed

- Option 1: LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols.

Note: Above does not constrain LP-SS sequence design for OOK

Working Assumption

From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively.

- Above is applicable for both time-domain processing or frequency-domain processing

- Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications

Agreement

For idle/inactive mode, the maximum number of information bits (excluding CRC) in a LP-WUS is Z, where Z <= [8 or 16].

- FFS the exact value of Z

Agreement

For idle/inactive mode, the maximum number of subgroups per PO is X, where 8 <= X <= 256.

- FFS the exact value of X.

## RAN1#118 agreements

Agreement

The definitions of LP-RSRP and LP-RSSI for LP-RSRQ are updated as follows:

LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols over the frequency resources defined by the number of REs that carry LP-SS.

LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols over the frequency resources defined by the number of REs that carry LP-SS.

Agreement

At least support 1:1 association between LP-WUS MO(s)/LP-SS transmissions and SSB beams.

Agreement

For the wake-up delay, consider the following options:

Option 2: UE reports one value from X candidate values for the wake-up delay via UE capability reporting.

- FFS: X is 2, 3, 4

Above applies for IDLE/INACTIVE mode.

Definition of wake-up delay: Minimum gap time between LP-WUS reception and MR to start PDCCH monitoring

Agreement

For the maximum number of subgroups per PO (X), down-select from the following options in RAN1#118bis:

- Option 1: X = 8

- Option 2: X = 16

- Option 3: X = 32

- Option 4: X = 64

- Option 5: X = 128

- Option 6: X = 256

For decision in RAN1#118bis, companies are encouraged to provide the maximum number of information bits per LP-WUS (Z), the number of OFDM symbols occupied by LP-WUS per MO, the number of MOs for their preferred option.

Conclusion

There is no consensus in RAN1 on the support of dynamic PO.

Agreement

On the LO configuration for iDRX, offset value(s) between a LO and a reference PO/PF is/are configured.

- FFS: one or multiple offset values, and if multiple offset values are supported, how a UE decides which value to use

- FFS: the exact definition of the reference PF/PO and the detailed procedure for UE to determine the LO(s) corresponding to Option 1, 2, or 3 (if supported)

- (Working Assumption) For each UE, the periodicity of LO is the same as its iDRX cycle.

- If a UE receives a wake-up indication in a LP-WUS, consider the following alternatives

  - Alt 1: it monitors the PO associated with the offset.

  - Alt 2: it monitors the first PO after its reported wake-up delay.

  - Other alternatives are not precluded

Note: The PO mentioned above refers to legacy PO configured for the UE.

Agreement

Send an LS to RAN2 and RAN4 to convey the following

In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE. Request RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs.

Final LS in R1-2407559.

Conclusion

RAN1 will not initiate work on entry/exit conditions based on RRM measurement and RRM measurement offloading/relaxation conditions unless triggered by RAN2 and RAN4

## RAN1#118bis agreements

Agreement

Confirm the following working assumption for iDRX: For each UE, the periodicity of LO is the same as its iDRX cycle.

Agreement

For the mapping between LO and PO, supports at least Option 1 (UEs monitoring the same PO monitor the same LO).

Agreement

From RAN1 perspective, when a UE is monitoring LP-WUS (based on the entry/exit condition for LP-WUS), a UE is not required to monitor a PO if

- it does not detect a LP-WUS on the monitored LO

- or the LP-WUS does not indicate a wake-up indication for the UE’s corresponding subgroup

Agreement

For the offset value(s) between an LO and a reference PO/PF, consider the following options:

- Definition: The gap between an LO and a PO is considered to be no less than the wake-up delay a UE supports if the gap between the end of the last LP-WUS MO the UE monitors in the LO and the start of the PO is no less than the wake-up delay.

- Option 1: gNB configures a single offset value.

  - If the gap between an LO and the PO associated with the offset is no less than the wake-up delay a UE supports, the UE monitors the PO associated with the offset after receiving a wake-up indication in a LP-WUS.

  - Otherwise,

    - Option 1-1: the UE follows the legacy paging monitoring procedure.

    - Option 1-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay.

- Option 2: gNB configures one or multiple offset values.

  - For the same PO, each offset corresponds to a LO.

    - This does not preclude the possibility that the same LO may correspond to different POs with different offset values.

  - Option 2A: A UE does not expect that the gap between the LO associated with the largest offset and the corresponding PO is less than the wake-up delay the UE supports. The UE monitors the LO associated with one offset that has a gap between the LO and the corresponding PO no less than the wake-up delay.

    - This implies that the gNB needs to configure at least one offset value that is no less than the largest wake-up delay supported by the UEs.

    - FFS exactly how to choose the offset

  - Option 2B:

    - If the gap between the LO associated with the largest offset and the corresponding PO is no less than the wake-up delay a UE supports, the UE monitors the LO associated with one offset that has a gap between the LO and the PO associated with the offset no less than the wake-up delay.

      - FFS exactly how to choose the offset

    - Otherwise,

      - Option 2B-1: the UE follows the legacy paging monitoring procedure.

      - Option 2B-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay.

        - FFS exactly how to choose the offset

  - FFS the UE shall monitor the LO associated with additional offset(s)

Note: The PO mentioned above refers to legacy PO configured for the UE.

Agreement

At least the following codepoints are supported for LP-WUS:

- One codepoint corresponding to each of the subgroups that can be indicated by LP-WUS

- One codepoint corresponding to all the subgroups that can be indicated by LP-WUS

- FFS: Additional codepoints

Agreement

Support 3 candidate values for the wake-up delay capability a UE reports, two values for ultra-deep sleep state and one value for deep sleep state.

- FFS the values

- Note: Ultra-deep sleep state and deep sleep state are mentioned above solely for the purpose of RAN1 discussions and it is not intended to be captured in specifications from RAN1 perspective

Agreement

Each LP-WUS is QCLed with one SSB. Each LP-SS is QCLed with one SSB.

- FFS QCL Type A or Type C and/or Type D

- FFS implicit QCL determination or some signaling is required

Agreement

The number of beams for LP-SS is the same as the number of beams for the LP-WUS MOs in an LO.

Agreement

When K (K>1) LP-WUS MOs are configured for each beam in an LO, down select between

- Option A: K LP-WUS MOs for a beam are divided into M (M >=1) groups of R LP-WUS MOs. A UE monitors all or some of the MO(s) within the K LP-WUS MOs.

  - For each group of R LP-WUS MOs, the same LP-WUS information is transmitted.

    - FFS how the same LP-WUS information is transmitted in the R LP-WUS MOs

  - Different LP-WUS information can be transmitted in different groups of R LP-WUS MOs.

  - M = 1 and M > 1 is supported.

  - FFS: detailed UE monitoring behavior

  - FFS R=1 or R>= 1

- Option B: K LP-WUS MOs for a beam are divided into G (G >= 1) groups of R*M (M >= 1) LP-WUS MOs. A UE monitors all or some of the MO(s) within one group of R*M LP-WUS MOs based on its subgroup ID.

  - Each group of R*M LP-WUS MOs is further divided into M groups of R LP-WUS MOs.

    - For each group of R LP-WUS MOs, the same LP-WUS information is transmitted.

      - FFS how the same LP-WUS information is transmitted in the R LP-WUS MOs

    - Different LP-WUS information can be transmitted in different groups of R LP-WUS MOs.

    - FFS: detailed UE monitoring behavior

  - M = 1 and M > 1 is supported.

  - FFS R=1 or R>=1

  - Note: this achieves the same purpose as “Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO.”

Working Assumption

The maximum number of subgroups per PO supported in Rel-19 is 32.