3GPP TSG RAN WG1 #119		R1-2410763

Orlando, USA, 18th-22nd November 2024

Agenda Item 	:	9.11.4

Source 	:	Moderator (Sony)

Title 	:	Final FL Summary for IoT-NTN

Document for	: Discussion

1	Introduction	3

2	WID objectives	4

3	Current and Previous agreements	5

4	NPUSCH	10

4.1	Overall summary of issues raised in Tdocs	10

4.2	OCC schemes to support at 3.75kHz SCS and 15kHz SCS	17

4.3	3.75kHz single-tone OCC scheme	20

4.4	15kHz single-tone OCC scheme	23

4.5	Multi-tone OCC scheme	25

4.6	DMRS	27

4.7	UL gaps	30

4.8	Other features that should work with OCC	32

4.9	Signalling	32

4.9.1	[NEW] Thursday 17 October proposals	34

4.10	Pairing	34

4.11	Downlink Issues and alignment	35

4.11.1	[NEW] Thursday 17 October proposals	36

5	NPRACH	37

5.1	Overall summary of issues raised in Tdocs	37

5.2	Whether NPRACH OCC is supported	39

5.3	NPRACH OCC scheme	40

5.3.1	[NEW] Proposal after offline discussion	42

5.4	RAR impacts	42

5.5	NPRACH partitioning	43

5.6	Signalling	43

6	Offline session: Tuesday 15 October	44

6.1	Background	44

6.2	Proposals for discussion	46

6.3	Updated proposals after offline discussion	48

7	15 October: Proposals for online discussion	48

8	Offline session: Thursday 17 October	49

8.1	Background	49

8.2	Proposals for offline discussion	49

8.3	Updated proposals after offline discussion	51

9	17 October: Proposals for online discussion	51

10	Goal for RAN1#119 Orlando	51

11	Conclusions	52

12	References	53

# Introduction

This is the initial FLS for IoT NTN.

Main Introduction

This document is the Feature Lead Summary document for the Rel-19 IoT-NTN work item [1].

Items to be considered in this version of the feature lead summary are tagged with [FL1].

This FLS contains a set of proposals, which can hopefully be addressed in online meeting time at some stage. The document also contains a set of questions. These questions are intended for the purpose of sharing company views. If there is enough agreement, it might be possible to generate proposals.

NPUSCH

The following issues are discussed for NPUSCH:

- 3.75kHz single-tone OCC scheme. DMRS multiplexing type and supported OCC length.

- 15kHz single tone OCC scheme. Clarification of meaning of “slot-level”, CDM DMRS details and OCC length.

- Multi-tone OCC scheme. Relative priority of this feature.

- UL gaps. How UL transmission gaps (of various types) affect OCC.

- Signalling. Which parameters will need signalling for OCC?

- Features that operate with OCC. Consideration of support of OCC for CG-Msg3 EDT is a pressing issue.

- Alignment and downlink issues. Consider how to align OCC between UEs, with gaps and with NPDCCH transmission.

NPRACH

Issues related to the following have been identified:

- Support or not. Whether NPRACH needs to support OCC.

- Choice of OCC schemes. Details of inter-symbol group scheme.

- RAR impacts.

- NPRACH partitioning. Does NPRACH resource need to be partitioned to support OCC?

- Signalling. Which parameters will need signalling for OCC?

When uploading updated documents with comments, please use the following convention for filenames:

Follow the naming convention in this example:

- IoTNTNFLS1-v000.docx

- IoTNTNFLS1-v001-CompanyA.docx

- IoTNTNFLS1-v002-CompanyA-CompanyB.docx

- IoTNTNFLS1-v003-CompanyB-CompanyC.docx

If needed, you may “lock” a discussion document for 30 minutes by creating a checkout file, as in this example:

- Assume CompanyC wants to update IoTNTNFLS1-v002-CompanyA-CompanyB.docx.

- CompanyC uploads an empty file named IoTNTNFLS1-v003-CompanyB-CompanyC.checkout.

- CompanyC checks that no one else has created a checkout file simultaneously, and if there is a collision, CompanyC tries to coordinate with the company who made the other checkout (see, e.g., contact list below).

- CompanyC then has 30 minutes to upload IoTNTNFLS1-v003-CompanyB-CompanyC.docx.

- If no update is uploaded in 30 minutes, other companies can ignore the checkout file.

In file names, please use the hyphen character (not the underline character) and include ‘v’ in front of the version number, as in the examples above and in line with the general recommendation, otherwise the sorting of the files will be messed up (which can only be fixed by the RAN1 secretary).

To avoid excessive email load on the RAN1 email reflector, please note that there is NO need to send an info email to the reflector just to inform that you have uploaded a new version of this document. Companies are invited to enter the contact info in the table below.

The table below provides a list of points of contact within companies for this WI. Contact details from RAN1#118bis Maastricht use a blue font. Please feel free to update your contact details and convert into a black font.

[FL1] Please consider entering contact info below for the points of contact for this email discussion.

| Company | Point(s) of contact | Email address(es) |
| --- | --- | --- |
| SONY | Martin Beale | martin.beale@sony.com |
| Ericsson | Gerardo Agni Medina Acosta | gerardo.agni.medina.acosta@ericsson.com |
| Lenovo | Zhi Yan | yanzhi1@lenovo.com |
| LGE | Daesung Hwang Seungmin Lee Hanjun Park | daesung.hwang@lge.com edison.lee@lge.com hanjun0128.park@lge.com |
| Gatehouse Satcom | René Brandborg Sørensen | rbs@gatehouse.com |
| Nokia, NSB | Jingyuan Sun | Jingyuan.sun@nokia-sbell.com |
| InterDigital, Inc | Umer Salim | umer.salim@interdigital.com |
| ETRI | Pansoo Kim | pskim@etri.re.kr |
| vivo | Zichao Ji Siqi Liu | jizichao@vivo.com liusiqi@vivo.com |
| Sharp | Zhanping Yin | zyin@sharplabs.com |
| Xiaomi | Xuemei Qiao | qiaoxuemei@xiaomi.com |
| Spreadtrum | Lei Gu | Lei.gu@unisoc.com |
| TCL | Yu Ding Yiwei Deng | yu10.ding@tcl.com yiwei1.deng@tcl.com |
| ZTE | Nan Zhang Fangyu Cui Ziyang Li | zhang.nan152@zte.com.cn cui.fangyu@zte.com.cn Li.ziyang1@zte.com.cn |
| Nordic | Mauri Nissila | Mauri.nissila@nordicsemi.no |
| CMCC | YI ZHENG Yongchang LIU | Zhengyi@chinamobile.com Liuyongchang@chinamobile.com |

# WID objectives

The IoT-NTN WID [1] was updated in RANP#104 and has the following objectives:

- Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4]

# Previous agreements

The following agreements were made in RAN1#116 Athens:

Agreement#116-IoT-NTN #1 For single-tone NPUSCH format 1 transmissions with both 3.75kHz and 15kHz SCS, the following OCC schemes are considered by RAN1 for further study: - Time domain OCC where OCC spreads across: - Symbol-level - Slot-level - Repetition-level - RV-level For multi-tone NPUSCH format 1 transmissions, the following OCC schemes are considered by RAN1 for further study: - Time domain OCC where OCC spreads across: - Symbol-level - Slot-level - Repetition-level - RV-level - Intra-symbol pre-DFT spreading OCC Agreement#116-IoT-2 The following evaluation assumptions are used for the study of OCC for NPUSCH format 1: Parameter; value. scenario; orbit; GEO; LEO600. Elevation angle; 12.5 degree; 30degree. Channel and impairments; carrier frequency; 2GHz. Channel model; NTN-TDL-C The channels from different UE are independent.. Frequency error; Uniform random selection from [-0.1 ppm, +0.1 ppm] for all UEs Variation of frequency error is negligible.. Timing error; Uniform random selection from [-97Ts, +97Ts] for all UEs Timing drift 80us/s for LEO600 and 0 for GEO.. Power imbalance; Uniformly distributed between +Pimb and -Pimb for all UEs Proponent to report the value of Pimb (can be zero) and justification for the chosen value. transmitter; SCS; 3.75KHz and 15KHz; 15kHz. Number of tones; Single tone; Single tone and multi tone up to 12 tones. Waveform; DFT-s-OFDM. Frequency hopping; w/o frequency hopping. MIMO scheme; SISO. DMRS configuration; For baseline evaluations: OS#3 per slot for 3.75kHz OS#4 per slot for 15kHz For OCC evaluations: Up to proponent; For baseline evaluations: OS#4 per slot for 15kHz For OCC evaluations: Up to proponent. Number of resource unit (NRU); Up to proponent; Up to proponent. Modulation order (Qm); Up to proponent; Up to proponent. TBS (ITBS); Up to proponent; Up to proponent. Number of repetitions (Nrep); Up to proponent. OCC length; Up to 4. OCC sequence; Up to proponent. Number of UE; Up to 4. Velocity of UE; 3km/h. receiver; Receiver algorithm; MMSE. Channel estimation; Real channel estimation. KPI; SNR at 10% BLER; Report for baseline and OCC schemes. Aggregated throughput; Total throughput of up to 4 UEs multiplexed.

The following agreements were made in RAN1#116bis Changsha:

Agreement For the NPUSCH evaluation assumptions, update the DMRS configuration, as follows: DMRS configuration; For baseline evaluations: OS#4 per slot for 3.75kHz OS#3 per slot for 15kHz For OCC evaluations: Up to proponent; For baseline evaluations: OS#3 per slot for 15kHz For OCC evaluations: Up to proponent. Agreement At least the following NPRACH OCC schemes are considered by RAN1 for study: - Intra-symbol group OCC - Inter-symbol group(s) OCC - Inter-repetition OCC Agreement The study of OCC for NPRACH does not consider NPRACH format 2. Agreement The following evaluation assumptions are used for the study of OCC for NPRACH: Parameter; value. Scenario; Orbit and elevation angle; GEO at 12.5 degrees; LEO600 at 30 degrees. Channel and impairments; carrier frequency; 2GHz. Channel model; NTN-TDL-C The channels from different UE are independent.. Frequency error; Uniform random selection from [-0.1 ppm, +0.1 ppm] for all UEs Variation of frequency error is negligible.. Timing error; Uniform random selection from [-97Ts, +97Ts] for all UEs Timing drift 80us/s for LEO600 and 0 for GEO.. Power imbalance; Uniformly distributed between +Pimb and -Pimb for all UEs Proponent to report the value of Pimb (can be zero) and justification for the chosen value. Transmitter; NPRACH format; 1 or 0. MIMO scheme; SISO. Number of repetitions (Nrep); Up to proponent. OCC length; Up to proponent. OCC sequence; Up to proponent. Number of UE; Up to proponent. Velocity of UE; 3km/h. Total NPRACH time / frequency resource utilisation; To be reported by proponent.. KPI; Target detection probability; 99%. Target false alarm probability; 0.1%. SNR operating point; Report SNR where target detection probability and false alarm probability are reached for baseline and OCC schemes. Agreement OCC multiplexing is not supported between a UE using NPUSCH format 1 with 3.75kHz SCS and another UE using NPUSCH format 1 with 15kHz SCS. Agreement For OCC of NPUSCH format 1, RAN1 will not consider multiplexing more than 4 UEs. Agreement For single-tone DMRS when OCC is applied to NPUSCH format 1, RAN1 considers at least the following for further study: - TDM of DMRS. The time domain locations of DMRS for different UEs are different. No OCC is applied for the DMRS of different UEs. - FFS: Detailed mapping - CDM of DMRS. The time domain locations of DMRS for different UEs are the same. Different OCCs are applied for the DMRS of different UEs. - FFS: Detailed mapping - Other schemes are not precluded, including combinations of the above Agreement For the NPUSCH evaluation assumptions, update the frequency error assumption, as follows. Frequency error; Uniform random selection from [-0.1 ppm, +0.1 ppm] for all UEs Variation of frequency error is negligible. For GEO, the same frequency error is applied to each subframe of a transport block. For LEO, the same frequency error is applied to each subframe of a segment (if applied in the evaluation). Companies to report their assumption on frequency error across segments..

The following agreements were made in RAN1#117 Fukuoka:

Agreement For 3.75kHz single-tone OCC for NPUSCH format 1, RAN1 supports either symbol-level OCC or slot-level OCC. Other OCC schemes are not pursued. For 15kHz single-tone OCC for NPUSCH format 1, RAN1 supports either symbol-level OCC or slot-level OCC. Other OCC schemes are not pursued. Agreement Inter-repetition OCC for NPRACH is not studied further in RAN1. Agreement - For the time-domain DMRS pattern (including blanked DMRS, if any): - For 15kHz single-tone, RAN1 strives to reuse the Rel-17 DMRS pattern - For 3.75kHz single-tone - RAN1 studies - Rel-17 DMRS pattern - A new DMRS pattern - The DMRS overhead (including blanked DMRS, if any) for OCC is the same as for Rel-17 Agreement The Rel-17 guard period locations and length for NB-IoT 3.75kHz UL slot are preserved when OCC is applied to NPUSCH format 1.

The following agreements were made in RAN#118bis Maastricht:

Agreement RAN1 studies whether the following types of UL transmission gap will impact the design of OCC for IoT-NTN when considering e.g. phase continuity - UL gaps for synchronization (from Rel-13) - Gaps around NPRACH occasions - UL timing adjustment gaps and segmentation for IoT-NTN (from Rel-17) - TDM DMRS that are muted - Guard periods for 3.75kHz UL transmissions Agreement The following combinations are considered for further simulation in RAN1 for 3.75kHz SCS OCC for NPUSCH format 1: - Option 1: OCC2, Symbol-level, TDM DMRS - Option 2: OCC2, Symbol-level, CDM DMRS with new pattern - Option 3: OCC2, Slot-level, TDM DMRS - Option 4: OCC2, Slot-level, CDM DMRS with legacy pattern - Option 6: OCC4, Symbol-level, CDM DMRS with new pattern The following combinations are considered for further simulation in RAN1 for 15kHz SCS OCC for NPUSCH format 1: - Option 1: OCC2, Symbol-level, TDM DMRS - Option 3: OCC2, Slot-level, TDM DMRS - Option 4: OCC2, Slot-level, CDM DMRS with legacy pattern - Option 5: OCC4, Symbol-level, TDM DMRS - Option 7: OCC4, Slot -level, TDM DMRS - Option 8: OCC4, Slot-level, CDM DMRS with legacy pattern Note 1: For TDM, the legacy DMRS pattern, with DMRS symbols appropriately muted/blanked is used. Companies to report their assumption on whether spreading is applied to the legacy DMRS pattern for 15 kHz SCS. Note 2: Companies to report DMRS sequence applied. Agreement For 3.75kHz SCS, NPUSCH format 1 simulations are performed using an appropriate MCS with SNR at least in the range of -8dB to 0dB.

The following agreements were made in RAN1#118bis Hefei:

Agreement At least the following schemes are supported for single-tone: - For 3.75kHz SCS OCC for NPUSCH format 1: - OCC length 2, Symbol-level - FFS: DMRS pattern(s) - For 15kHz SCS OCC for NPUSCH format 1: - OCC length 2, Slot-level, CDM DMRS with legacy pattern - FFS: CDM details, e.g. with or without spreading Agreement For NPRACH transmission, inter-symbol group OCC is not further studied. Agreement For support of single-tone OCC for NPUSCH format 1, RAN1 studies: - The parameters that need to be signalled, considering the following: - OCC codeword - Enabling of OCC feature - FFS: other parameters - For dynamic grant in RRC CONNECTED, study which and whether any parameters are signalled via DCI and which and whether any parameters are signalled by RRC - FFS: whether/how to support cases other than dynamic grant in RRC CONNECTED, e.g. Msg3, PUR, CB Msg3 EDT

# NPUSCH

## Overall summary of issues raised in Tdocs

The following is an overall summary of issues raised by companies in input contributions.

OCC code type

- Walsh

  - Specify [1,1] and [1,-1] as the codewords for OCC2 [Ericsson][MTK]

- DFT [ZTE]

  - Unified design with NR PUCCH format 1

3.75kHz OCC scheme

- DMRS pattern:

  - CDM-legacy: [Sharp]

    - Performance close to baseline [HW]

    - More evaluation needed due to phase change between DMRS locations [CMCC]

    - Phase change at maximum CFO difference is too great [Spreadtrum]

    - Compatibility with legacy UEs OK [ZTE]

    - Multi-cell orthogonality impacted [Nok]

    - Clarify meaning of “legacy slot structure” [Ericsson]

      - Is the DMRS symbol spread [Ericsson]

  - CDM-new: [LGE][Nordic][NEC]

    - Performance close to baseline [HW]

    - Specification impacts [Spreadtrum][OPPO]

      - Different slot formats in different slots since DMRS is located in different positions [OPPO]

    - Implementation complexity increased [ZTE][Nok]

      - eNB needs to implement and switch between two different DMRS receive / channel estimation algorithms for legacy and OCC UEs [Nok]

    - Compatibility with legacy UEs problematic [ZTE][Lenovo][TCL]

    - Multi-cell orthogonality impacted [Nok]

    - Needed to meet CFO requirements / pull-in issue [NEC]

  - TDM: [Spreadtrum][CATT][Lenovo]

    - With consecutive DMRS for same UE [HW][CMCC]

    - DMRS density is reduced [ZTE]

      - Leads to degraded performance [Sharp]

    - Consecutive DMRS active for a UE, then consecutive DMRS blanked [HW]

    - Simulation results from Qualcomm show good performance [CATT]

      - 0.2dB loss to baseline [QC]

    - Previous simulation results show OK performance for TDM [OPPO]

    - RAN4 need to define phase continuity requirement [LGE]

    - RAN4 need to define ON/OFF mask to avoid interference [LGE]

  - Reuse legacy pattern [HW][CMCC][Nok][MTK]

    - Whether TDM or CDM-legacy, prefer to reuse the legacy pattern [HW]

    - Little performance loss with legacy pattern when an advanced CFO estimator used [HW]

    - Reduces implementation complexity (legacy and new schemes would require 2 DMRS receivers at eNB) [CMCC][ZTE]

    - Reduces commercialisation complexity [CMCC]

  - DMRS-sequence based [Nok]

    - Legacy with different DMRS sequences for different UEs [Spreadtrum][vivo][Nok]

    - New seed for DMRS sequence generation [LGE]

    - Doesn’t work [OPPO]

  - Specify as a mask on current DMRS sequences [vivo]

- Supported OCC lengths:

  - Note that RAN1#118bis Hefei agreement stated “At least the following schemes are supported for single-tone” and then listed OCC2 schemes. This implies that OCC4 schemes could still be considered: FL

  - 2: already agreed in RAN1#118 Hefei and doesn’t need to be reconsidered in RAN1#119 Orlando: FL

  - 4:	 QC. Nordic

    - Only 0.47dB loss to baseline [QC]

- Spreading method

  - Use MPUSCH, identical mapping for symbol-based [ZTE]

Performance of DMRS schemes based on 3.75kHz simulations. All of these are based on symbol-level:

OCC2: [HW][QC][CMCC][ZTE]

|  | Loss to baseline |
| --- | --- |
| TDM | HW: 1.96dB (with advanced CFO estimator) CMCC: 0.3dB ZTE: 1.0dB (200Hz differential offset, assuming pairing) QC: 0.29dB |
| CDM legacy pattern | HW: 0.46dB (with advanced CFO estimator) CMCC: 0.1dB ZTE: 0.1dB (200Hz differential offset, assuming pairing) QC: 0.47dB |
| CDM new pattern | HW: 0.42dB (with advanced CFO estimator) CMCC: 0.1dB ZTE: 0dB (200Hz differential offset, assuming pairing) QC: 0dB |

OCC4: [HW][QC]

|  | Loss to baseline |
| --- | --- |
| TDM |  |
| CDM legacy pattern |  |
| CDM new pattern | HW: 1.91dB (with advanced CFO estimator) QC: 0.47dB |

Notes:

- loss to baseline (single UE performance) listed for REP8

3.75kHz DMRS-new pattern

- Note that this only needs to be considered if we make a higher level agreement that we will support “DMRS-new”: FL

- Within cluster separation is x1 symbols, between cluster separation is x2 symbols [QC][NEC][LGE]

  - For OCC2:

    - Option 1: QC

      - OS [0,1]; OS [8,9] every 2 consecutive slots

    - Option 2: [LGE]

      - Slot index mod 2 = 0: DMRS in OS#6

      - Slot index mod 2 = 1: DMRS in OS#0

    - Option 3: [NEC]

      - Consider a range of different locations

  - For OCC4:

    - Option 1: QC

      - OS [8,9,10,11], [16,17,18,19] every 4 consecutive slots

    - Option 2: [LGE]

      - Slot index mod 4 = 0: DMRS in OS#5, OS#6

      - Slot index mod 4 = 1: DMRS in OS#0, OS#1

      - Slot index mod 4 = 2 or 3: no DMRS

    - Option 3: [NEC]

      - Consider a range of different locations

15kHz OCC scheme

- Clarification of slot-level [HW][ZTE]

  - Across-slots [Nok]

  - Across Nslots [HW][Xiaomi][ZTE][Apple]

    - Performance within 0.28dB of baseline [HW]

    - Less specification impact than across-slot [ZTE]

    - Use MPUSCH,identical = 2,4 for single-tone [ZTE]

      - Then the current mapping for multitone in current specs can be reused [ZTE]

  - Performance between slot and Nslot is similar [HW]

- Supported OCC lengths:

  - Note that RAN1#118bis Hefei agreement stated “At least the following schemes are supported for single-tone” and then listed OCC2 schemes. This implies that OCC4 schemes could still be considered: FL

  - 2: already agreed in RAN1#118 Hefei and doesn’t need to be reconsidered in RAN1#119 Orlando: FL

  - 4: QC, Nordic

    - Do not support: Ericsson, Sharp

- DMRS details

  - Note that the RAN1#118bis agreement is “OCC length 2, Slot-level, CDM DMRS with legacy pattern; FFS: CDM details, e.g. with or without spreading”: FL

  - Spreading [OPPO][Lenovo][QC][Samsung]

    - Needed to differentiate UEs [Samsung]

  - No spreading [HW][Spreadtrum]

    - Elements of OCC sequence are multiplied with the DMRS symbols per slot, in addition to the legacy reference signal sequence [HW]

    - Agreement to strive to reuse legacy pattern [Spreadtrum] FL: I think this applies to the time locations, not to the values of the complex symbols in those locations.

  - Note: we need to define the terms “spreading” and “no spreading” in order to choose between them.

  - Specify as a mask on current DMRS sequences [vivo]

  - Use cyclic shifts of DMRS sequence [Ericsson]

Performance of DMRS schemes based on 15kHz simulations. All of these are based on slot-level:

OCC2: [HW][CMCC][ZTE]

|  | Loss to baseline |
| --- | --- |
| CDM spreading |  |
| CDM no spreading |  |

|  | Loss to baseline |
| --- | --- |
| Slot-level | HW: 0.15dB CMCC: 0.28dB ZTE: 0dB (200Hz differential CFO assuming pairing) |
| Nslot-level | HW: 0.28dB (with slot-level DMRS) ZTE: 0dB (200Hz differential CFO assuming pairing) |

Other simulation results: vivo (OCC2, TDM DMRS: 2.56dB loss to baseline)

OCC4: [HW]

|  | Loss to baseline |
| --- | --- |
| CDM spreading |  |
| CDM no spreading |  |

|  | Loss to baseline |
| --- | --- |
| Slot-level | HW: 2.19dB (with slot-level DMRS) |
| Nslot-level | HW: 1.33dB (with slot-level DMRS; poor performance with Nslot-level DMRS) |

Notes:

- loss to baseline (single UE performance) listed for REP8

Multi-tone support:

- No: Ericsson

  - Multi-tone would only be applicable in high SNR conditions [Ericsson]

    - High SNR conditions are not an issue since they do not use many resources [Ericsson]

  - Many different numbers of subcarriers leads to complicated specification updates [Ericsson]

  - Deprioritise multi-tone [Samsung][Interdigital]

    - Only consider if there is time following single-tone discussions [Samsung]

- Yes: HW, CMCC, Spreadtrum, Nok, CATT, Xiaomi, Apple

  - Minimal spec impact [CMCC]

- Common time-domain solution with single-tone [HW] [Nok][OPPO]

  - Nslot [Spreadtrum][Xiaomi]

- DMRS

  - Different cyclic shifts [HW][Spreadtrum][CATT]

- Supported OCC lengths:

  - 2: [HW][Nok]

  - 4 [Nok]

    - Performance is poor [HW: see table below]

    - DMRS for OCC4 can be specified easily [Nok]

OCC2: [HW]

|  | Loss to baseline |
| --- | --- |
| Slot-level | HW: 0.94dB (DMRS with cyclic shifts) |
| Nslot level | HW: 1,17dB (DMRS with cyclic shifts) |

OCC4: [HW]

|  | Loss to baseline |
| --- | --- |
| Slot-level | HW: 2.57dB (DMRS with cyclic shifts) |
| Nslot level | HW: 8.43dB (DMRS with cyclic shifts) |

Features that NPUSCH should work with:

- Connected mode dynamic grant: HW, vivo [QC]

- EDT

  - Yes: [QC][TCL]

  - No:

- PUR:

  - Yes: [QC][TCL][Spreadtrum][LGE][Nok][OPPO][Apple]

    - NPRACH partitioning to indicate UE capability [Apple]

  - No: [HW][Xiaomi]

    - Consider relationship of OCC to “shared PUR” in Rel-16 [Ericsson]

    - Can already support 2 UEs in same T/F resources for 12 tones in R17 [HW]

    - Specification impact [Xiaomi]

- CB-Msg3-EDT (R19)

  - Yes: [LGE] [Spreadtrum][LGE][OPPO][Ericsson][QC][Apple]

    - Max OCC2 to avoid Msg4 becoming bottleneck [Ericsson]

  - No: [HW][Xiaomi]

    - UEs would have to randomly choose OCC [HW][CMCC]

    - Pairing not possible in IDLE mode [HW][CMCC]

  - Further study: CMCC

  - Specification impact [Xiaomi]

- Msg3:

  - Yes: Spreadtrum, OPPO

  - No: HW, CMCC, Xiaomi

    - RAN2 impacts – RAR needs to be updated [HW]

    - Can’t pair UEs as power / CFO imbalance is not known [CMCC]

    - Do not know OCC capability of UE at initial access, unless PRACH partitioning is applied [CMCC]

    - Specification impact [Xiaomi]

- Compatibility and coexistence between OCC and non-OCC UEs [Nok]

- Prioritise CONNECTED mode dynamic grant over other features [vivo]

UL gaps:

- Need to align OCC around transmission gaps [QC]

- Need to accurately align UEs if their NPUSCH starting times cause gaps to occur at different times [QC]

- Align OCC DMRS such that they don’t straddle a gap [QC]

- Approaches to mitigate gaps in OCC transmissions:

  - OCC sequences that are robust to time misalignment [Ericsson]

  - Timing correction / control functionality at UE transmitter [Ericsson]

  - Receiver processing techniques to compensate for distortion in gaps [Ericsson]

  - Periodicity of OCC pattern (in slots) is a divisor of (UL segment size – UL gap size) [LGE]

  - Drop any OCC codeword that spans a “problematic” gap [Nok][Spreadtrum]

    - Note that if gappy-UE transmits part of a codeword, a non-gap UE’s NPUSCH would lose orthogonality in the decoding process: FL

- RAN4 issues

  - For an NR UE, phase continuity is maintained if the gap is 13 OFDM symbols or less. Does this still apply to IoT-NTN? [CMCC]

- UL gaps for synchronization (from Rel-13):

  - Phase continuity broken across gap [vivo]

  - HD-FDD switching leads to loss phase continuity [CMCC]

  - eNB does not schedule UE with OCC if the NPUSCH would span a gap [vivo]

  - Consider in design / further study [CMCC]

  - OCC2 means that OCC codeword should never straddle gap given {256,40}ms structure [Apple]

- Gaps around NPRACH occasions

  - Drop OCC segment that spans gap [Spreadtrum]

  - Consider in design / further study [CMCC]

  - Align OCC scheme around NPRACH gaps [QC]

  - eNB can schedule to avoid NPRACH gaps, hence not a problem [ZTE]

- UL timing adjustment gaps and segmentation for IoT-NTN (from Rel-17)

  - Drop OCC segment that spans gap [Spreadtrum]

  - OCC does not span UL segment gap [vivo][LGE]

  - UL TA gaps are not an issue if segment length is a multiple of OCC length and eNB can decide this [MTK]

  - UL TA gaps not a problem for Nslot as remainder of available non-gapped resource allows compensation [ZTE]

  - Consider in design of OCC scheme [CMCC][Apple][ETRI]

  - Gaps affect phase continuity since TA is changed in gap [CMCC][vivo] [Nok]

- TDM DMRS that are muted

  - No issue [Spreadtrum][vivo][CMCC][Apple]

    - < 13 OFDM symbols (from NR coverage enhancement work), hence phase continuity is maintained [vivo]

- Guard periods for 3.75kHz UL transmissions

  - Affect the design of the OCC multiplexing scheme: [CMCC]

  - Do not affect phase continuity [CMCC][ZTE][ETRI] [Spreadtrum][vivo]

Physical channel mapping

- Resource unit size

  - Increase RU size

    - Super-RU = M RUs [QC]

    - Avoids a reduction of coding rate [QC]

  - Increase RV size

    - Super-RV = NRU super-RUs [QC]

    - Avoids a reduction of coding rate [QC]

    - RV length increased by OCC factor [Lenovo]

    - No: keep RV size in time domain the same [MTK]

  - Reduce number of repetitions

    - Reduced by OCC factor [Lenovo]

  - Physical channel mapping rules need to change [LGE][QC][OPPO][vivo][Lenovo][TCL]

    - Spreading function required in physical channel mapping [Lenovo]

- TBS determination

  - Based on multiplication of slots of allocated RU by OCC length [OPPO]

  - Based on multiplication of symbols in a slot multiplied by OCC length [OPPO]

- Method

  - “pause and repeat across symbol” [OPPO]

    - Note: this is equivalent to “pause and repeat across slot” in current spec [OPPO]

  - Slot-based:

    - MPUSCH_identical = 2,4 also applies to single tone [ZTE][LGE]

    - MPUSCH_identical = 2,4 also applies to single tone and Nslots = 1 for pure slot-based (as opposed to Nslot-based) [OPPO]

  - Symbol-based

    - MPUSCH_identical = 2,4 also applies to single tone [ZTE]

Signalling

- Aspects that need to be signalled:

  - OCC factor (M) [QC][ETRI] [Sharp]

  - OCC codeword [HW][QC][Sharp][TCL][MTK][Nordic]

  - OCC feature enabling [QC][Sharp][TCL][MTK][Nordic]

  - Sequence type (DFT or Walsh) [ETRI]

- RRC [CMCC][ETRI][Spreadtrum]

  - OCC feature enabling [CMCC][QC][TCL][LGE][Nordic][Spreadtrum]

  - OCC factor (M) [QC]

  - OCC codeword [CMCC]

- DCI [HW][Sharp][Spreadtrum]

  - OCC codeword [HW][CMCC][LGE][CATT][QC][Sharp][TCL][Nordic]

    - OCC2: 1 bit needed [HW]

    - Implicit [QC]

  - OCC feature enabling [Sharp]

    - Allows fast switch between OCC scheme and legacy NPUSCH [Sharp]

  - Maintain DCI size [Sharp][TCL][Spreadtrum]

    - Does not increase blind decoding effort at UE [Sharp]

    - Reinterpretation of DCI fields [Sharp][Spreadtrum]

      - Reinterpret REP field (NTN coverage conditions are stable) [Sharp]

      - Reinterpret bits in MCS field [TCL]

- MAC CE [Apple]

  - OCC codeword [Apple]

  - OCC factor (M) [Apple]

- Implicitly derived [QC]

  - OCC codeword [QC]

- CG-Msg3-EDT:

  - OCC codeword linked to virtual NPRACH resource [LGE]

  - OCC codeword linked to contention resolution ID [LGE]

Pairing

- RAN1 study potential loss of orthogonality from pairing UEs [Ericsson]

- Factors to be considered for pairing:

  - Traffic characteristics [Ericsson]

  - Number of repetitions [Ericsson]

  - Modulation schemes [Ericsson]

  - Location [Ericsson]

  - Power [Ericsson]

Downlink issues

- Increase in NPDCCH resource [Ericsson]

  - 4 OCC NPUSCH would require 4 DCIs [Ericsson]

- Alignment of NPUSCH requires staggered NPDCCH, requiring new k0 values (subframes between NPDCCH and NPUSCH) [Ericsson]

  - Additional offset: k0 + {1,2,4,8,16} to allow for NPPDCH repetitions [Ericsson]

  - Additional offset is a function of OCC codeword [Sony]

- NPUSCH from different UEs need alignment [Nok]

Alignment

- Alignment of NPUSCH requires staggered NPDCCH, requiring new k0 values (subframes between NPDCCH and NPUSCH) [Ericsson]

- Reference point in time introduced to align OCC from different UEs [TCL]

- Alignment of adding new UEs with OCC to existing group of UEs already doing OCC [QC][Nok]

- Alignment of codewords around OCC gaps [QC]

- New DMRS schemes will lead to difficulty of alignment of UEs. [Nok]

  - Applies to both TDM and CDM [Nok]

PAPR

- eNB PAPR may be increased with PAPR [Ericsson]

  - Consult RAN4

Evaluation assumptions

- BLER operating point = 1% or less [Viasat]

  - Due to long HARQ RTT and commercial parameters [Viasat]

- Repetition level = 1,2 or 4

  - Based on parameter of commercial network [Viasat]

- RAN1 studies performance when paired UEs have resynchonised at a gap [Sony]

## 3.75kHz single-tone OCC scheme

The following views were expressed about the type of OCC scheme that should be supported for 3.75kHz single-tone:

- DMRS pattern:

  - CDM-legacy: [Sharp]

    - Performance close to baseline [HW]

    - More evaluation needed due to phase change between DMRS locations [CMCC]

    - Phase change at maximum CFO difference is too great [Spreadtrum]

    - Compatibility with legacy UEs OK [ZTE]

    - Multi-cell orthogonality impacted [Nok]

    - Clarify meaning of “legacy slot structure” [Ericsson]

      - Is the DMRS symbol spread [Ericsson]

  - CDM-new: [LGE][Nordic][NEC]

    - Performance close to baseline [HW]

    - Specification impacts [Spreadtrum][OPPO]

      - Different slot formats in different slots since DMRS is located in different positions [OPPO]

    - Implementation complexity increased [ZTE][Nok]

      - eNB needs to implement and switch between two different DMRS receive / channel estimation algorithms for legacy and OCC UEs [Nok]

    - Compatibility with legacy UEs problematic [ZTE][Lenovo][TCL]

    - Multi-cell orthogonality impacted [Nok]

    - Needed to meet CFO requirements / pull-in issue [NEC]

  - TDM: [Spreadtrum][CATT][Lenovo]

    - With consecutive DMRS for same UE [HW][CMCC]

    - DMRS density is reduced [ZTE]

      - Leads to degraded performance [Sharp]

    - Consecutive DMRS active for a UE, then consecutive DMRS blanked [HW]

    - Simulation results from Qualcomm show good performance [CATT]

      - 0.2dB loss to baseline [QC]

    - Previous simulation results show OK performance for TDM [OPPO]

    - RAN4 need to define phase continuity requirement [LGE]

    - RAN4 need to define ON/OFF mask to avoid interference [LGE]

  - Reuse legacy pattern [HW][CMCC][Nok][MTK]

    - Whether TDM or CDM-legacy, prefer to reuse the legacy pattern [HW]

    - Little performance loss with legacy pattern when an advanced CFO estimator used [HW]

    - Reduces implementation complexity (legacy and new schemes would require 2 DMRS receivers at eNB) [CMCC][ZTE]

    - Reduces commercialisation complexity [CMCC]

  - DMRS-sequence based [Nok]

    - Legacy with different DMRS sequences for different UEs [Spreadtrum][vivo][Nok]

    - New seed for DMRS sequence generation [LGE]

    - Doesn’t work [OPPO]

  - Specify as a mask on current DMRS sequences [vivo]

- Supported OCC lengths:

  - Note that RAN1#118bis Hefei agreement stated “At least the following schemes are supported for single-tone” and then listed OCC2 schemes. This implies that OCC4 schemes could still be considered: FL

  - 2: already agreed in RAN1#118 Hefei and doesn’t need to be reconsidered in RAN1#119 Orlando: FL

  - 4:	 QC. Nordic

    - Only 0.47dB loss to baseline [QC]

- Spreading method

  - Use MPUSCH, identical mapping for symbol-based [ZTE]

Performance of DMRS schemes based on 3.75kHz simulations. All of these are based on symbol-level:

OCC2: [HW][QC][CMCC][ZTE]

|  | Loss to baseline |
| --- | --- |
| TDM | HW: 1.96dB (with advanced CFO estimator) CMCC: 0.3dB ZTE: 1.0dB (200Hz differential offset, assuming pairing) QC: 0.29dB |
| CDM legacy pattern | HW: 0.46dB (with advanced CFO estimator) CMCC: 0.1dB ZTE: 0.1dB (200Hz differential offset, assuming pairing) QC: 0.47dB |
| CDM new pattern | HW: 0.42dB (with advanced CFO estimator) CMCC: 0.1dB ZTE: 0dB (200Hz differential offset, assuming pairing) QC: 0dB |

OCC4: [HW][QC]

|  | Loss to baseline |
| --- | --- |
| TDM |  |
| CDM legacy pattern |  |
| CDM new pattern | HW: 1.91dB (with advanced CFO estimator) QC: 0.47dB |

Notes:

- loss to baseline (single UE performance) listed for REP8

3.75kHz DMRS-new pattern

- Note that this only needs to be considered if we make a higher level agreement that we will support “DMRS-new”: FL

- Within cluster separation is x1 symbols, between cluster separation is x2 symbols [QC][NEC][LGE]

  - For OCC2:

    - Option 1: QC

      - OS [0,1]; OS [8,9] every 2 consecutive slots

    - Option 2: [LGE]

      - Slot index mod 2 = 0: DMRS in OS#6

      - Slot index mod 2 = 1: DMRS in OS#0

    - Option 3: [NEC]

      - Consider a range of different locations

  - For OCC4:

    - Option 1: QC

      - OS [8,9,10,11], [16,17,18,19] every 4 consecutive slots

    - Option 2: [LGE]

      - Slot index mod 4 = 0: DMRS in OS#5, OS#6

      - Slot index mod 4 = 1: DMRS in OS#0, OS#1

      - Slot index mod 4 = 2 or 3: no DMRS

    - Option 3: [NEC]

      - Consider a range of different locations

The following agreement was made related to the support of NPUSCH format 1 with 3.75kHz SCS:

Agreement At least the following schemes are supported for single-tone: - For 3.75kHz SCS OCC for NPUSCH format 1: - OCC length 2, Symbol-level - FFS: DMRS pattern(s) - For 15kHz SCS OCC for NPUSCH format 1: - OCC length 2, Slot-level, CDM DMRS with legacy pattern - FFS: CDM details, e.g. with or without spreading

There are two points that need to be addressed in order to progress this agreement:

- The DMRS pattern that will be applied. The choices are TDM, CDM using the legacy DMRS pattern and CDM using a new DMRS pattern

- Whether other schemes are supported at 3.75kHz SCS. This basically comes down to whether OCC length 4 will also be supported

### 3.75kHz DMRS multiplexing scheme

The DMRS patterns under consideration are illustrated in some documents. Figure 1 is taken from [R1-2409408 Huawei] and illustrates the patterns under consideration:

(a) CDM of DMRS with legacy pattern

(b) CDM of DMRS with new pattern

(c) TDM of DMRS with legacy pattern

Figure 1 - Potential DMRS patterns for NPUSCH format 1 at 3.75kHz SCS

From the submitted Tdocs, the advantages and disadvantages of the schemes are summarised in Table 1.

Table 1 - Comparison of DMRS schemes for 3.75kHz NPUSCH format 1

|  | TDM | CDM legacy | CDM new |
| --- | --- | --- | --- |
| Performance loss to baseline | Acceptable (within approx. 0.3 to 1.0dB) | Acceptable (within approx. 0 to 0.5dB) | Acceptable (within approx. 0 to 0.4dB) |
| DMRS density | Reduced, but performance impact not significant | Same | Same |
| Compatibility with legacy UEs | OK | OK, but check orthogonality in multi-cell case | Concerns. DMRS are in different locations for legacy and OCC UEs |
| Specification impacts | Minimal | Small. Spreading of DMRS symbol needs defining | Large. New NPUSCH slot formats required. Slot format changes from slot to slot. |
| Implementation complexity | UE: minimal eNB: channel estimation needs updating | UE: minimal eNB: channel estimation needs updating | UE: more significant eNB: channel estimation needs significant update |
| Multi-cell orthogonality | No impact | Impacted if same DMRS complex values in each CDM-ed symbol | Impacted. DMRS patterns do not align with other cell scheduling legacy UE |
| Phase continuity | Not impacted across a single OFDM symbol | Not impacted | Not impacted |
| Similarity to 15kHz scheme | Different | Same | Different |

It appears that TDM-based DMRS has the better balance of advantages and disadvantages. The performance of the TDM-based DMRS pattern is reported as being acceptable. If we adopt a TDM scheme, we should clarify that the applied TDM pattern is according to Figure 1, where there are consecutive active DMRS symbols followed by consecutive blanked DMRS symbols. This structure allows for more accurate CFO estimation and is the scheme that has been simulated by most companies.

Hence, the following proposal is made:

[FL1] Proposal 4_2_1v1:

3.75kHz SCS OCC for NPUSCH format 1 uses a TDM DMRS pattern:

- Consecutive DMRS symbols are active for a UE followed by consecutive blanked DMRS symbols

Companies are invited to comment on proposal 4_2_1v1 in the table below:

| Company | Comment |
| --- | --- |
| LGE | On the analysis for the compatibility with legacy UEs, it is questionable that the answer for the TDM approach is OK. For instance, the legacy UE will transmit DMRS in all the DMRS symbols, and the advanced UE with TDM approach will transmit DMRS in a subset of DMRS symbols. In this case, the DMRS from the legacy UE and the new UE will be collided. Regarding the implementation complexity, unlike NR specification, the LTE specification does not consider the phase continuity during the non-contiguous transmission. Moreover, this is low-cost/low-complexity NB-IoT UEs. It is also questionable how the UE implementation to support TDM is simple in the LTE NB-IoT context. Furthermore, some RAN4 works including UE requirement for phase continuity and ON/OFF masking should be supported for the TDM approach. |
| CATT | Ok |
| CMCC | Fine with the proposal. Our understanding is that neither the blanked DMRS symbol or the active DMRS symbols should follow the legacy DMRS pattern. |
| Qualcomm | We do not agree with the proposal and analysis. First of all, it is unclear why “multi-cell orthogonality” is different for different schemes, since the details of the sequence are still unknown, so we could always design a pattern that maintains orthogonality across cells. |
| Nokia, NSB | More comparison between the main bullet with minor update of DMRS cyclic shift with legacy DMRS sequence/pattern is needed before RAN1 make agreement. For main bullet, as TDM DMRS will impact the channel estimation, which will also impact to the performance of the IoT NTN UL, which is opposite to the motivation of the capacity enhancement. We do not support the sub-bullet as it will have much impact to the specification and also increasing the distance between the two separate DMRS symbols for CFO estimation. |
| Ericsson | Based on the agreements reached until now, we suggest incorporating the following clarifications: 3.75kHz SCS OCC length 2 for NPUSCH format 1 single-tone uses a TDM DMRS pattern: - Consecutive DMRS symbols are active for a UE followed by consecutive blanked DMRS symbols In relation with the sub-bullet, is the right understanding the before applying the OCC spreading, the slot structure is as per legacy as shown below? CP+Data symbol (275 us); CP+Data symbol (275 us); CP+Data symbol (275 us); CP+Data symbol (275 us); CP+DMRS symbol (275 us); CP+Data symbol (275 us); CP+Data symbol (275 us); Guard period (75 us). Slot (2ms). Then, when the OCC spreading is applied, is the intention applying the spreading only to “data symbols” keeping unspread and untouched the location of the “DMRS symbols”? And then with TDM DMRS in the case of two UEs, is the right understanding that the “DMRS symbol” in even slots would be used for the OCC’d UE1 whereas the “DMRS symbol” in odd slots would be used for the OCC’d UE2? |
| FL2 | It seems like there is no consensus on the proposal. Ericsson> Yes, the figure is correct. Yes, data symbols are spread and DMRS are not. No, UE1 has {active DMRS in slot 1,2; blank in slot 3,4}, UE2 has {blank in slot 1,2; DMRS in slot 3,4}. The proposal might be clearer as: [FL1] Proposal 4_2_1v2: 3.75kHz SCS OCC length 2 for NPUSCH format 1 single-tone uses a TDM DMRS pattern: - A UE with active DMRS in slot n and n+1 does not transmit DMRS in slots n+2 and n+3 |

### 3.75kHz OCC lengths

It has already been agreed that a length 2 OCC scheme will be supported. We need to decide whether an additional OCC length of 4 will be supported. While OCC4 allows for more users to be supported, the performance of OCC4 is reported to be worse by various companies, would have a higher specification impact (including further options for DMRS pattern, for whichever DMRS scheme is decided upon), DL signalling may become a bottleneck (at least for dynamic grant) and UE pairing would become more difficult (it would be harder for the eNB scheduler to find a set of UEs with compatible power, CFO, timing etc.). Only a few companies propose to support an OCC length of 4.

[FL1] Proposal 4_2_2v1:

For 3.75kHz SCS OCC for NPUSCH format 1, the maximum OCC length is 2.

Companies are invited to comment on proposal 4_2_2v1 in the table below:

| Company | Comment |
| --- | --- |
| LGE | In our understanding, considering the segmentation gap, OCC length 4 would not be easy. In this point of view, we are fine to focus on the OCC length 2. |
| CATT | We support it. |
| CMCC | Fine with the proposal. |
| ETRI | We support it |
| Qualcomm | As mentioned in previous meetings, we think that OCC-4 is workable and would provide significant capacity improvements. If DL control is a concern, we can compromise to specify OCC-4 for CB-msg3 only. |
| Nokia, NSB | OK. |
| Ericsson | Yes: Legacy specification already today supports multiplexing a non-negligible number of users. That is, up to 36 simultaneous single-tone UEs when NPRACH (45 kHz) and NPUSCH (135 kHz) co-exist, and up to 48 simultaneous single-tone UEs when NPUSCH uses the entire transmission bandwidth (180 kHz). OCC-based NPUSCH transmissions using an OCC length 2 would lead to support of up 72 single-tone users simultaneously when NPRACH and NPUSCH co-exist, and up 96 simultaneous single-tone users when NPUSCH utilises 180 kHz. Overall, the larger the number of simultaneous OCC users, the larger is the complexity added to the eNodeB’s scheduler and a bottleneck is more likely to be created in DL which is non orthogonal. Thus, even supporting an OCC length 2 is challenging, since it doubles the number of UEs to be handled. |
| FL2 | While not everyone agrees to this proposal, a sufficient number of companies does agree. Hence we could consider it in the online session. |

## 15kHz single-tone OCC scheme

The following views were expressed about the type of OCC scheme that should be supported for 15kHz single-tone:

15kHz OCC scheme

- Clarification of slot-level [HW][ZTE]

  - Across-slots [Nok]

  - Across Nslots [HW][Xiaomi][ZTE][Apple]

    - Performance within 0.28dB of baseline [HW]

    - Less specification impact than across-slot [ZTE]

    - Use MPUSCH,identical = 2,4 for single-tone [ZTE]

      - Then the current mapping for multitone in current specs can be reused [ZTE]

  - Performance between slot and Nslot is similar [HW]

- Supported OCC lengths:

  - Note that RAN1#118bis Hefei agreement stated “At least the following schemes are supported for single-tone” and then listed OCC2 schemes. This implies that OCC4 schemes could still be considered: FL

  - 2: already agreed in RAN1#118 Hefei and doesn’t need to be reconsidered in RAN1#119 Orlando: FL

  - 4: QC, Nordic

    - Do not support: Ericsson, Sharp

- DMRS details

  - Note that the RAN1#118bis agreement is “OCC length 2, Slot-level, CDM DMRS with legacy pattern; FFS: CDM details, e.g. with or without spreading”: FL

  - Spreading [OPPO][Lenovo][QC][Samsung]

    - Needed to differentiate UEs [Samsung]

  - No spreading [HW][Spreadtrum]

    - Elements of OCC sequence are multiplied with the DMRS symbols per slot, in addition to the legacy reference signal sequence [HW]

    - Agreement to strive to reuse legacy pattern [Spreadtrum] FL: I think this applies to the time locations, not to the values of the complex symbols in those locations.

  - Note: we need to define the terms “spreading” and “no spreading” in order to choose between them.

  - Specify as a mask on current DMRS sequences [vivo]

  - Use cyclic shifts of DMRS sequence [Ericsson]

Performance of DMRS schemes based on 15kHz simulations. All of these are based on slot-level:

OCC2: [HW][CMCC][ZTE]

|  | Loss to baseline |
| --- | --- |
| CDM spreading |  |
| CDM no spreading |  |

|  | Loss to baseline |
| --- | --- |
| Slot-level | HW: 0.15dB CMCC: 0.28dB ZTE: 0dB (200Hz differential CFO assuming pairing) |
| Nslot-level | HW: 0.28dB (with slot-level DMRS) ZTE: 0dB (200Hz differential CFO assuming pairing) |

Other simulation results: vivo (OCC2, TDM DMRS: 2.56dB loss to baseline)

OCC4: [HW]

|  | Loss to baseline |
| --- | --- |
| CDM spreading |  |
| CDM no spreading |  |

|  | Loss to baseline |
| --- | --- |
| Slot-level | HW: 2.19dB (with slot-level DMRS) |
| Nslot-level | HW: 1.33dB (with slot-level DMRS; poor performance with Nslot-level DMRS) |

Notes:

- loss to baseline (single UE performance) listed for REP8

The following agreement was made related to the support of NPUSCH format 1 with 15kHz SCS:

Agreement At least the following schemes are supported for single-tone: - For 3.75kHz SCS OCC for NPUSCH format 1: - OCC length 2, Symbol-level - FFS: DMRS pattern(s) - For 15kHz SCS OCC for NPUSCH format 1: - OCC length 2, Slot-level, CDM DMRS with legacy pattern - FFS: CDM details, e.g. with or without spreading

There are several points that need to be addressed in order to progress this agreement:

- Clarification on the meaning of “slot-level”. Specifically, we need to clarify whether spreading operates on a slot basis or on an Nslot basis.

- Whether other schemes are supported at 15kHz SCS. This basically comes down to whether OCC length 4 will also be supported.

- Details of the CDM scheme: whether spreading is applied to the DMRS or not.

### Clarification of meaning of “slot-level”

The following aspect of physical channel mapping is defined in TS36.211 section 10.1.3.6:

After mapping to slots, the slots shall be repeated additional times, before continuing the mapping of to the following slot, where

The smallest unit of repetition at 15kHz SCS is hence Nslots = 2 slots. This unit of repetition has been pretty irrelevant for the single-tone case up until the current release since Midentical,PUSCH = 1 for single-tone. However, for slot-level OCC operation, the slot structure needs repeating in some way in order to provide a spreading operation. There would appear to be two options:

- Option 1: Nslot = 1 spreading. Redefine that for OCC, Nslots = 1 and repeat the slots, for example by changing Midentical,PUSCH = OCC length

- Option 2: Nslot = 2 spreading. Maintain the definition of Nslots = 2 and repeat the Nslots = 2 pair of slots, for example by Midentical,PUSCH = OCC length

These two options are illustrated in Figure 2.

Figure 2 – Spreading options for 15kHz SCS

Companies (ZTE, Huawei) report that the performance of Nslot = 2 level spreading is close to the baseline performance for an OCC length of 2. Companies report that that spreading at the Nslot =2 level has fewer specification impacts than spreading at the Nslot = 1 level, where the spreading function can be defined in the specifications by changing the value of Midentical, NPSUCH for single tone (e.g. to Midentical, NPSUCH = 2 for an OOC length of 2).

[FL1] Proposal 4_3_1v1:

For 15kHz SCS single tone, the slot level spreading scheme is clarified as either:

- Option 1: Nslot = 1 spreading: redefining that Nslot = 1 for OCC and that Midentical, NPSUCH is equal to the OCC length.

- Option 1: Nslot = 2 spreading: the current value of Nslot is maintained for single-tone at 15kHz SCS. Midentical, NPSUCH is equal to the OCC length.

Companies are invited to comment on proposal 4_3_1v1 in the table below:

| Company | Comment |
| --- | --- |
| LGE | We are OK with second Option 1 (that should be Option 2) while the performance is acceptable to minimize the specification change. |
| Vivo1 | Support. Just one typo: option 2: Nslot = 2 spreading Our preference is option2, which has less spec impact |
| CATT | We support Nslot = 2. |
| CMCC | Support the option 2 which has less spec impacts. |
| Qualcomm | The performance with N_slot = 2 would be degraded with respect to N_slot = 1. For multi-tone we would understand the specification simplification (since we could reuse the M_identical for OCC), but this is not the case for single tone where M_identical is equal to 1. |
| Nokia, NSB | We prefer to have minimize impact to specification while we are also fine to use Nslot = 1 spreading considering the performance and impact to spec on OCC for data, DMRS, etc. |
| Ericsson | It seems too early to perform a clarification on this aspect, this seems to be a discussion even touching upon an Editor CR phase. We think that we still have more essential aspects to discuss around the Slot-level scheme such as the “FFS: CDM details, e.g. with or without spreading”, which might impact the decision. |
| FL2 | Yes, the second bullet should be about “Option 2”. Sorry for the typo. Ericsson> I think that this is not just an editorial issue. The difference between option1 and option2 would change the physical channel mapping as shown in the figures. It could also have an impact on performance (as pointed out by Qualcomm). The proposal could be updated to: [FL1] Proposal 4_3_1v2: For 15kHz SCS single tone, RAN1 studies: - Option 1: Nslot = 1 spreading: redefine that Nslot = 1 for OCC and that Midentical, NPSUCH is equal to the OCC length. - Option 2: Nslot = 2 spreading: the current value of Nslot is maintained for single-tone at 15kHz SCS. Midentical, NPSUCH is equal to the OCC length. This does not mean that we would need to select one of these two schemes right now. |

### 15kHz OCC lengths

The arguments for and against support of OCC4 at 15kHz SCS are similar to the arguments for 3.75kHz SCS.

It has already been agreed that a length 2 OCC scheme will be supported. We need to decide whether an additional OCC length of 4 will be supported. While OCC4 allows for more users to be supported, the performance of OCC4 is reported to have a 1-2dB loss to baseline performance (by Huawei), would have a higher specification impact (including further options for DMRS pattern), DL signalling may become a bottleneck (at least for dynamic grant) and UE pairing would become more difficult (it would be harder for the eNB scheduler to find a set of UEs with compatible power, CFO, timing etc.). Only a few companies propose to support an OCC length of 4.

[FL1] Proposal 4_3_2v1:

For 15kHz SCS OCC for NPUSCH format 1, the maximum OCC length is 2.

Companies are invited to comment on proposal 4_3_2v1 in the table below:

| Company | Comment |
| --- | --- |
| LGE | Considering segmentation gap, it would not be easy to support OCC length 4. We are fine with FL’s proposal. |
| CATT | OK |
| CMCC | Fine with the proposal. |
| ETRI | We support it. |
| Qualcomm | OK |
| Nokia, NSB | For 15kHz SCS, to have better capacity, we do not think there is consensus to exclude OCC 4 in this stage. |
| Ericsson | We support the proposal. |

### Details of CDM DMRS with legacy pattern

The following agreement was made at RAN1#117 Fukuoka:

Agreement - For the time-domain DMRS pattern (including blanked DMRS, if any): - For 15kHz single-tone, RAN1 strives to reuse the Rel-17 DMRS pattern - For 3.75kHz single-tone - RAN1 studies - Rel-17 DMRS pattern - A new DMRS pattern - The DMRS overhead (including blanked DMRS, if any) for OCC is the same as for Rel-17

FL’s understanding of the term “RAN1 strives to reuse the Rel-17 DMRS pattern” is that it relates to the locations of the DMRS symbols and not to the complex-values applied to the DMRS symbols.

As part of the “CDM with legacy pattern” scheme, RAN1 needs to decide whether spreading is applied to the DMRS symbols as part of the CDM scheme or not. According to the FL’s understanding, Figure 3 illustrates operation of CDM of the legacy DMRS with and without spreading. In the case of DMRS spreading, the complex-valued DMRS symbol used in slot 1 is copied to slot 2 and OCC is applied across those identical complex-valued DMRS symbols. In the case of operation without DMRS spreading, the complex-valued DMRS symbol from slot 1 is OCC-ed with the different complex-valued DMRS symbol from slot 2.

Figure 3 – CDM of the legacy DMRS pattern with and without spreading

Some companies think that a DMRS spreading function is necessary for the CDM operation to work at all. Other companies [presumably] understand that CDM can work across different complex-value DMRS symbols (the eNB knows what those complex-value symbols are, in contrast to the case for OCC of data symbols) and hence that a “no spreading” mode of operation is possible for DMRS. The “no spreading” operation would lead to fewer changes to DMRS and might lead to less impact on other UEs (through any DMRS interference effect).

[FL1] Proposal 4_3_3v1:

For 15kHz CDM DMRS with legacy pattern, choose between the following clarifications:

- Option 1: DMRS symbols are spread before the OCC is applied.

  - In the spreading function, the complex-valued DMRS symbol from slot 2*n is copied to slot 2*n+1

- Option 2: DMRS symbols are not spread before the OCC is applied.

  - The complex-valued DMRS symbols in slot 2*n and 2*n+1 are according to the legacy DMRS sequence.

Companies are invited to comment on proposal 4_3_3v1 in the table below:

| Company | Comment |
| --- | --- |
| LGE | We are OK with Option 2. Since the DMRS sequence would be the known sequence, there would not be problematic to go with Option 2. For information, according to the current DMRS OCC for cell randomization, DMRS spreading is not considered. |
| Vivo1 | For option2, as FL explained, in the case of operation without DMRS spreading, the complex-valued DMRS symbol from slot 1 is OCC-ed with the different complex-valued DMRS symbol from slot 2, therefore no spreading actually means ‘no OCC is applied for DMRS’. ‘before the OCC is applied’ in the 2nd sub-bullet somehow is ambiguous as it is not clear if it refers to PUSCH OCC or DMRS OCC, thus should be removed. Therefore, we suggest the following revision - Option 2: DMRS symbols are not spread and does not apply OCC before the OCC is applied. |
| CATT | We support option 1. |
| ETRI | We support option 2. |
| Qualcomm | We think both proposals are problematic. Assume two UEs in two different cells start the NPUSCH transmission at the same time (same slot number), so the Gold part of the DMRS sequence is common. For Option 1 essentially downsamples every other sample of the legacy DMRS pattern, so two cells may end up with the exact same sequence. For example, if cell 1 uses the Hadamard [ 1 1 1 1 1 1 ….] and cell 2 uses the Hadamard [ 1 -1 1 -1 …], both cells would end up using the same sequence [1 1 1 1 ….] + the Gold (which is common) and the receiver cannot differentiate between both cells. For option 2, there is a similar issue if you take the same two Hadamards. If a UE in cell 1 uses [1 1 1 1 ….] with OCC [1 1] and another UE in cell 2 uses [1 -1 1 -1 ….] with OCC [1 -1], both DMRS sequences would be identical. We suggest companies to further check these issues. |
| Nokia, NSB | We support Option 2, maybe with some update. We think the minimum impact to spec is to use some sequence shift according to the OCC index, with legacy sequence as baseline. “Option 2: DMRS symbols are not spread before the OCC is applied. The complex-valued DMRS symbols in slot 2*n and 2*n+1 are according to the legacy DMRS sequence. To minimize impact to spec, Shift related processing for DMRS symbols in slot 2*n and 2*n+1 can be further discussed.” |
| Ericsson | The wording is confusing. Is the intention of the sub-bullet under Option1 to reflect the following “the complex-valued DMRS symbol used in slot 1 is copied to slot 2 and OCC is applied across those identical complex-valued DMRS symbols” if so, we are not sure that is correctly reflected in the sub-bullet. |
| FL2 | Ericsson> Yes. That is the intention. Vivo> Isn’t there an OCC function in some sense? The two UEs would end up transmitting different DMRS sequences due to the impact of applying the OCC code on the DMRS sequence. I think this ends up being close to what Nokia has been proposing (that different UEs use different DMRS sequences). We could update the proposal as: [FL1] Proposal 4_3_3v2: For 15kHz CDM DMRS with legacy pattern, RAN1 studies: - Option 1: DMRS symbols are spread before the OCC is applied. - In the spreading function, the complex-valued DMRS symbol used in slot 1 is copied to slot 2 and OCC is applied across those identical complex-valued DMRS symbols - Option 2: DMRS symbols are not spread before the OCC is applied. - OCC is applied to the legacy complex-valued DMRS symbol used in slot 1 and slot 2. |

## Multi-tone OCC scheme

The following views were expressed about the type of OCC scheme that should be supported for multi-tone:

Multi-tone support:

- No: Ericsson

  - Multi-tone would only be applicable in high SNR conditions [Ericsson]

    - High SNR conditions are not an issue since they do not use many resources [Ericsson]

  - Many different numbers of subcarriers leads to complicated specification updates [Ericsson]

  - Deprioritise multi-tone [Samsung][Interdigital]

    - Only consider if there is time following single-tone discussions [Samsung]

- Yes: HW, CMCC, Spreadtrum, Nok, CATT, Xiaomi, Apple

  - Minimal spec impact [CMCC]

- Common time-domain solution with single-tone [HW] [Nok][OPPO]

  - Nslot [Spreadtrum][Xiaomi]

- DMRS

  - Different cyclic shifts [HW][Spreadtrum][CATT]

- Supported OCC lengths:

  - 2: [HW][Nok]

  - 4 [Nok]

    - Performance is poor [HW: see table below]

    - DMRS for OCC4 can be specified easily [Nok]

OCC2: [HW]

|  | Loss to baseline |
| --- | --- |
| Slot-level | HW: 0.94dB (DMRS with cyclic shifts) |
| Nslot level | HW: 1,17dB (DMRS with cyclic shifts) |

OCC4: [HW]

|  | Loss to baseline |
| --- | --- |
| Slot-level | HW: 2.57dB (DMRS with cyclic shifts) |
| Nslot level | HW: 8.43dB (DMRS with cyclic shifts) |

If multi-tone were to be supported for OCC, there seems to be consensus that a slot-based or Nslot-based approach would be suitable, from a performance perspective and from the specification impact that the pre-DFT-based approach would entail.

However, there are concerns that multi-tone transmissions aren’t suitable for NTN since they are more suitable high for SNR conditions (that are not really achieved in NTN). In lower SNR conditions, multiplexing more users can be achieved by scheduling single-tone transmissions.

In previous meetings, a lot of time has been spent on choice of the single-tone scheme. This has meant that there has not been time to discuss the multi-tone scheme. There are concerns over whether multi-tone OCC would actually be used, when there is the opportunity to schedule single-tone instead. This would seem to indicate that multi-tone is a lower priority for many companies. There are views expressed that there should be commonality between the single-tone scheme and any multi-tone scheme. This would suggest that it would be beneficial to firstly decide on a single-tone scheme and then consider a multi-tone tone scheme that has maximum commonality with the single-tone scheme.

Hence, it is proposed to prioritise the single-tone scheme over the multi-tone scheme and come back to multi-tone once a single-tone scheme has been decided on.

[FL1] Proposal 4.4-1v1: RAN1 prioritises the single-tone OCC schemes. Once single-tone schemes have been agreed, RAN1 can revisit multi-tone, aiming to minimise differences from the 15kHz single-tone scheme.

Companies are invited to comment on proposal 4.4-1.

| Company | Comment |
| --- | --- |
| CATT | For multi-tone, it can reuse NR scheme. |
| Nokia, NSB | No support. In WID, it is included “Multi-tone support for 15 kHz SCS should also be considered”. We think single-tone and multi-tone should be discussed and specified at same time, for the motivation of capacity improvement in Rel19. |
| Ericsson | We only support the first part of the proposal “RAN1 prioritises the single-tone OCC schemes”. Applying the same OCC scheme used for single-tone to the multi-tone case does not seem to be possible, mainly because for single-tone the allocation in the frequency-domain is constant (1-tone), whereas for multi-tone the allocation in the frequency-domain is variable (i.e., either 3, 6, or 12 tones). The OCC mapping is foreseen to be impacted. In our view RAN1 should focus on finalizing the design details of OCC for single-tone (both 15 kHz SCs and 3.75 kHz SCS). |
| FL2 | There is no consensus on this proposal at the moment. |

## UL gaps

UL gaps:

- Need to align OCC around transmission gaps [QC]

- Need to accurately align UEs if their NPUSCH starting times cause gaps to occur at different times [QC]

- Align OCC DMRS such that they don’t straddle a gap [QC]

- Approaches to mitigate gaps in OCC transmissions:

  - OCC sequences that are robust to time misalignment [Ericsson]

  - Timing correction / control functionality at UE transmitter [Ericsson]

  - Receiver processing techniques to compensate for distortion in gaps [Ericsson]

  - Periodicity of OCC pattern (in slots) is a divisor of (UL segment size – UL gap size) [LGE]

  - Drop any OCC codeword that spans a “problematic” gap [Nok][Spreadtrum]

    - Note that if gappy-UE transmits part of a codeword, a non-gap UE’s NPUSCH would lose orthogonality in the decoding process: FL

- RAN4 issues

  - For an NR UE, phase continuity is maintained if the gap is 13 OFDM symbols or less. Does this still apply to IoT-NTN? [CMCC]

- UL gaps for synchronization (from Rel-13):

  - Phase continuity broken across gap [vivo]

  - HD-FDD switching leads to loss phase continuity [CMCC]

  - eNB does not schedule UE with OCC if the NPUSCH would span a gap [vivo]

  - Consider in design / further study [CMCC]

  - OCC2 means that OCC codeword should never straddle gap given {256,40}ms structure [Apple]

- Gaps around NPRACH occasions

  - Drop OCC segment that spans gap [Spreadtrum]

  - Consider in design / further study [CMCC]

  - Align OCC scheme around NPRACH gaps [QC]

  - eNB can schedule to avoid NPRACH gaps, hence not a problem [ZTE]

- UL timing adjustment gaps and segmentation for IoT-NTN (from Rel-17)

  - Drop OCC segment that spans gap [Spreadtrum]

  - OCC does not span UL segment gap [vivo][LGE]

  - UL TA gaps are not an issue if segment length is a multiple of OCC length and eNB can decide this [MTK]

  - UL TA gaps not a problem for Nslot as remainder of available non-gapped resource allows compensation [ZTE]

  - Consider in design of OCC scheme [CMCC][Apple][ETRI]

  - Gaps affect phase continuity since TA is changed in gap [CMCC][vivo] [Nok]

- TDM DMRS that are muted

  - No issue [Spreadtrum][vivo][CMCC][Apple]

    - < 13 OFDM symbols (from NR coverage enhancement work), hence phase continuity is maintained [vivo]

- Guard periods for 3.75kHz UL transmissions

  - Affect the design of the OCC multiplexing scheme: [CMCC]

  - Do not affect phase continuity [CMCC][ZTE][ETRI] [Spreadtrum][vivo]

There are several (potentially related) problems with UL gaps. Firstly, it is necessary to ensure that an OCC transmission does not span a gap as this will leave part of the OCC codeword on one side of the gap and the other part of the codeword on the other side of the gap. Secondly, UL gaps may occur at different times for different UEs – there may need to be alignment such that both UEs in an OCC pair are transmitting consistently. Thirdly, there is likely to be phase discontinuity on either side of an UL transmission gap – this will introduce loss of orthogonality between UEs.

The following types of gap have been identified in RAN1#119. FL summary comments on the issues related to these gap types are included.

- UL gaps for synchronization (from Rel-13)

  - Issues identified. Further study required in RAN1. Detailed OCC scheme design may need to account for these gaps. Depending on OCC design parameters, an OCC codeword may never span this type of gap.

- Gaps around NPRACH occasions

  - Issues identified. Further study required in RAN1. Detailed OCC scheme design may need to account for these gaps. Depending on OCC design parameters, an OCC codeword may never span this type of gap.

- UL timing adjustment gaps for NTN (from Rel-17)

  - Issues identified. Further study required in RAN1. Detailed OCC scheme design may need to account for these gaps.

- TDM DMRS that are muted

  - No issue

- Guard periods for 3.75kHz UL transmissions

  - No issue

Based on this summary, it is proposed that RAN1 observes that TDM DMRS (if supported) and guard periods for 3.75kHz UL transmissions do not impact phase continuity.

[FL1] Proposed observation 4.5-1: From the perspective of phase continuity, the following do not affect the design of OCC for IoT-NTN:

- TDM DMRS that are muted

- Guard periods for 3.75kHz UL transmissions

Companies are invited to comment on proposed observation 4.5-1.

| Company | Comment |
| --- | --- |
| LGE | In our understanding, we are not expert to decide it since we don’t have the relevant UE requirement for the phase continuity for LTE NB-IoT. We need to send LS to RAN4 to have the answer of this question. |
| CMCC | Support. |
| ETRI | We support LGE’s view. |
| Qualcomm | It is unclear whether RAN1 can agree on this or we would need feedback from RAN4. The guard periods should not lead to lack of phase continuity, as per legacy. |
| Nokia, NSB | OK. |
| FL2 | It sounds like we should ask RAN4 on whether phase continuity is maintained across the different sorts of UL gap. |

## Other features that should work with OCC

The following features were identified as being features OCC should be compatible with:

- Connected mode dynamic grant: HW, vivo [QC]

- EDT

  - Yes: [QC][TCL]

  - No:

- PUR:

  - Yes: [QC][TCL][Spreadtrum][LGE][Nok][OPPO][Apple]

    - NPRACH partitioning to indicate UE capability [Apple]

  - No: [HW][Xiaomi]

    - Consider relationship of OCC to “shared PUR” in Rel-16 [Ericsson]

    - Can already support 2 UEs in same T/F resources for 12 tones in R17 [HW]

    - Specification impact [Xiaomi]

- CB-Msg3-EDT (R19)

  - Yes: [LGE] [Spreadtrum][LGE][OPPO][Ericsson][QC][Apple]

    - Max OCC2 to avoid Msg4 becoming bottleneck [Ericsson]

  - No: [HW][Xiaomi]

    - UEs would have to randomly choose OCC [HW][CMCC]

    - Pairing not possible in IDLE mode [HW][CMCC]

  - Further study: CMCC

  - Specification impact [Xiaomi]

- Msg3:

  - Yes: Spreadtrum, OPPO

  - No: HW, CMCC, Xiaomi

    - RAN2 impacts – RAR needs to be updated [HW]

    - Can’t pair UEs as power / CFO imbalance is not known [CMCC]

    - Do not know OCC capability of UE at initial access, unless PRACH partitioning is applied [CMCC]

    - Specification impact [Xiaomi]

- Compatibility and coexistence between OCC and non-OCC UEs [Nok]

- Prioritise CONNECTED mode dynamic grant over other features [vivo]

From the RAN1#118bis Hefei meeting, it seems like many companies would be OK to consider the fundamental design of OCC before starting consideration on these other features. However, RAN2 have sent an LS to RAN1 [4: R1-2409343], asking:

RAN2 is working on CB-msg3 EDT in IDLE state and would like to ask RAN1 whether the OCC solution for NPUSCH on which RAN1 is working can be applied to CB-msg3 NPUSCH or not.

For the current meeting, we should decide whether OCC can be applied to CB-Msg3 EDT, due to the need to respond to the LS from RAN2. We can defer discussion on PUR, Msg3 and EDT to a future meeting.

From the input contributions to this meeting, it seems that most companies think that OCC can be applied to CB-Msg3 EDT from the RAN1 perspective. A concern is that the UE would have to randomly choose the OCC codeword. For NPRACH, the NPRACH preamble sequence is selected in the MAC layer, according to TS36.321 section 5.1.2. In other words, the NPRACH preamble selection procedure is defined by RAN2. FL assumes that RAN2 would also select the OCC codeword to apply in CB-Msg3 EDT. Issues of pairing UEs for OCC are up to eNB implementation. It is assumed that if the eNB needed to pair OCC UEs and was unable to do so, it would not configure OCC with CB-Msg3 EDT. Hence, it is FL’s understanding in summary that from a RAN1 perspective, OCC can be applied to CB-Msg3 EDT.

[FL1] Proposal 4.6-1v1: From a RAN1 perspective, OCC can be applied to CB-Msg3 EDT.

Companies are invited to comment on proposal 4.6-1.

| Company | Comment |
| --- | --- |
| Vivo1 | We are fine with this direction, but we think the condition of the feasibility of OCC should be reflected. According to previous discussion, it is agreed that for NPUSCH with 3.75kHz, it can be assumed that Msg3 transmission/reception without Msg1/Msg2 can satisfy the corresponding requirement. While for 15kHz, the timing error may be larger than half CP, which may be problematic for msg3 decoding. Therefore, we think we suggest the following revisions. [FL1] Proposal 4.6-1v1: From a RAN1 perspective, OCC can be applied to CB-Msg3 EDT with 3.75kHz |
| CMCC | Not support. Not only the timing issue, but also the issue of power imbalance should be considered for the performance of the contention based Msg3. Currently no company provides the evaluation results with the consideration of timing mis-alignement and the power imbalance. The timing issue is not exactly the same as we response to RAN2 in the previous meeting. In the previous meeting, we were talking about the whether the PUSCH can be decoded for only single UE. And the currently the LS from RAN2 is the OCC multiplexing which is more sensitive to the timing alignment and power imbalance. More evaluations needs to be considered. |
| ETRI | We need to study the feasibility of considering timing offset and power imbalance issues. |
| Qualcomm | Agree |
| Nokia, NSB | OK. Detail can be further discussed. |
| Ericsson | The proposal is missing a set of considerations under which OCC for “CB-msg3 EDT” could be feasible to be supported: - Using NPUSCH Format 1 single-tone to perform OCC-based transmissions in “CB-msg3 EDT”. This because RAN1 has only agreed OCC schemes for NPUSCH Format 1 single-tone. - Using an OCC length 2. This for avoiding creating a potential bottleneck for Msg4. - The benefits/Gains of using OCC for “CB-msg3 EDT” depend on the collision rate (and successful decoding of the OCC’d UEs), which in turn depends on the design aspects mentioned in the previous bullets and the time-frequency resource framework of “CB-msg3EDT”. |
| FL2 | I have now moved discussion of CG-Msg3 EDT to section 6. My understanding is that there is currently no consensus on whether OCC can be applied to CG-Msg3 EDT. There is still the possibility that consensus could be achieved in the future. |

## Signalling

There are various aspects related to OCC that need to be signalled to the UE. These aspects have been identified in various documents:

- Aspects that need to be signalled:

  - OCC factor (M) [QC][ETRI] [Sharp]

  - OCC codeword [HW][QC][Sharp][TCL][MTK][Nordic]

  - OCC feature enabling [QC][Sharp][TCL][MTK][Nordic]

  - Sequence type (DFT or Walsh) [ETRI]

- RRC [CMCC][ETRI][Spreadtrum]

  - OCC feature enabling [CMCC][QC][TCL][LGE][Nordic][Spreadtrum]

  - OCC factor (M) [QC]

  - OCC codeword [CMCC]

- DCI [HW][Sharp][Spreadtrum]

  - OCC codeword [HW][CMCC][LGE][CATT][QC][Sharp][TCL][Nordic]

    - OCC2: 1 bit needed [HW]

    - Implicit [QC]

  - OCC feature enabling [Sharp]

    - Allows fast switch between OCC scheme and legacy NPUSCH [Sharp]

  - Maintain DCI size [Sharp][TCL][Spreadtrum]

    - Does not increase blind decoding effort at UE [Sharp]

    - Reinterpretation of DCI fields [Sharp][Spreadtrum]

      - Reinterpret REP field (NTN coverage conditions are stable) [Sharp]

      - Reinterpret bits in MCS field [TCL]

- MAC CE [Apple]

  - OCC codeword [Apple]

  - OCC factor (M) [Apple]

- Implicitly derived [QC]

  - OCC codeword [QC]

- CG-Msg3-EDT:

  - OCC codeword linked to virtual NPRACH resource [LGE]

  - OCC codeword linked to contention resolution ID [LGE]

The following agreement was made at RAN1#118bis Hefei:

Agreement For support of single-tone OCC for NPUSCH format 1, RAN1 studies: - The parameters that need to be signalled, considering the following: - OCC codeword - Enabling of OCC feature - FFS: other parameters - For dynamic grant in RRC CONNECTED, study which and whether any parameters are signalled via DCI and which and whether any parameters are signalled by RRC - FFS: whether/how to support cases other than dynamic grant in RRC CONNECTED, e.g. Msg3, PUR, CB Msg3 EDT

We have not yet fully concluded on the OCC schemes (see section 4.2 and 4.3), although it seems that (at the time of writing this section and according to the proposals in this version of the document that):

- There will be a single OCC factor (M) supported: OCC2.

- The sequence type will be defined in the specifications as part of the OCC scheme.

Hence, it is proposed that the only parameters that need signalling for OCC support are OCC codeword and enabling of OCC feature. There is majority consensus that OCC codeword would be signalled by DCI and OCC feature enabling would be signalled by RRC. Hence, the following proposal is made:

[FL1] Proposal 4.7-1v1:

For support of single-tone OCC for NPUSCH format 1:

- The parameters that need to be signalled are:

  - OCC codeword

  - Enabling of OCC feature

- For dynamic grant in RRC CONNECTED, OCC codeword is signalled via DCI and enabling of OCC feature is signalled by RRC

Companies are invited to comment on proposal 4.7-1.

| Company | Comment |
| --- | --- |
| CMCC | Fine with the proposal. |
| ETRI | Support |
| Qualcomm | We think it would be beneficial to support dynamic OCC ON/OFF based on dynamic grant. |
| Nokia, NSB | We think the first sub-bullet can be agreed but not good for other part. The first sub-bullet can be supported for all the cases, not only for single-tone, but also multi-tone. The second sub-bullet should be further discussed before any agreement. |
| Ericsson | We can be ok with it. |
| FL2 | I understand that Qualcomm and Nokia have similar views that the second bullet cannot be agreed. The proposal could be updated as: [FL1] Proposal 4.7-1v2: For support of single-tone OCC for NPUSCH format 1, the parameters that need to be signalled are: - OCC codeword - Enabling of OCC feature |

## Pairing

The following issues related to pairing of UEs were identified:

- RAN1 study potential loss of orthogonality from pairing UEs [Ericsson]

- Factors to be considered for pairing:

  - Traffic characteristics [Ericsson]

  - Number of repetitions [Ericsson]

  - Modulation schemes [Ericsson]

  - Location [Ericsson]

  - Power [Ericsson]

From the RAN1#118bis Hefei meeting, it seems like the issue of pairing needs to be considered in order to determine the OCC length. The OCC length is considered in sections 4.2.2 and 4.3.2. It hence seems like we don’t need to make any specific agreements on pairing for OCC.

## Downlink Issues and alignment

The following issues related to the downlink have been identified:

- Increase in NPDCCH resource [Ericsson]

  - 4 OCC NPUSCH would require 4 DCIs [Ericsson]

- Alignment of NPUSCH requires staggered NPDCCH, requiring new k0 values (subframes between NPDCCH and NPUSCH) [Ericsson]

  - Additional offset: k0 + {1,2,4,8,16} to allow for NPPDCH repetitions [Ericsson]

  - Additional offset is a function of OCC codeword [Sony]

- NPUSCH from different UEs need alignment [Nok]

A related issue is that of alignment of the OCC transmissions in general, where the following issues were identified:

- Alignment of NPUSCH requires staggered NPDCCH, requiring new k0 values (subframes between NPDCCH and NPUSCH) [Ericsson]

- Reference point in time introduced to align OCC from different UEs [TCL]

- Alignment of adding new UEs with OCC to existing group of UEs already doing OCC [QC][Nok]

- Alignment of codewords around OCC gaps [QC]

- New DMRS schemes will lead to difficulty of alignment of UEs. [Nok]

  - Applies to both TDM and CDM [Nok]

The following proposal was not considered at RAN1#118bis Hefei and FL proposes that we further consider this proposal in order to progress alignment and DL signalling issues:

[FL1] Proposal 4.9-1: For NPUSCH format 1 single-tone for both 15kHz and 3.75kHz, RAN1 studies how alignment is supported, including:

- Modification to scheduling delays (k0) between NPDCCH and NPUSCH, considering appropriate repetition values for NPDCCH

- FFS: other methods

Companies are invited to comment on proposal 4.9-1.

| Company | Comment |
| --- | --- |
| LGE | In our understanding, when the NPUSCH repetition is enabled, the NPDCCH would be repeated as well. In this case, the current candidate values for k0 would be sufficient. |
| Nokia, NSB | OK for study. |
| Ericsson | We are ok with the proposal, which we think is a fundamental aspect for OCC to work since DL is not orthogonal. To LG: Could you clarify under which basis the “the current candidate values for k0 would be sufficient”? Could you please tell us what is the number of repetitions that you are assuming for NPDCCH? |

# NPRACH

## Overall summary of issues raised in Tdocs

The following is an overall summary of issues raised by companies in input contributions to RAN1#118bis. Note that there were not many significant changes to the issues identified in RAN1#118.

Support or not:

- Support [QC][Lenovo][NEC][Nordic][Nokia][ETRI][Interdigital]

  - Performance OK for up to OCC5 [ZTE]

  - Performance OK with 0.2dB loss to baseline for 3UEs being OCC-ed [QC]

  - Performance degradation of 1dB for OCC2, 3dB for OCC4 at 99% detection probability [ETRI]

  - WID says that NPRACH OCC is supported [Nokia]

  - Specification impacts are small [Nordic]

  - CB-Msg3-EDT is a separate optional feature and does not impact whether NPRACH OCC is supported [ZTE]

    - Note: this is to counter the argument that NPRACH enhancements are not needed in R19 since CG-Msg3-EDT does not use NPRACH: FL

- Not support [HW][Ericsson][Spreadtrum][TCL][MTK][Samsung][CATT][Ericsson][Xiaomi]

  - Reasons:

    - Backward compatibility / impact on legacy UEs [CMCC] [Ericsson]

    - Specification impact [Ericsson][Spreadtum][HW][vivo][Xiaomi][Samsung]

    - NPRACH is not the bottleneck [HW] [CATT][Spreadtrum][HW][Ericsson]

    - Performance with power imbalance, timing error [HW][CMCC][vivo][CATT]

      - Cannot pair UEs with compatible imperfections in NPRACH [vivo]

    - RAN2 impacts in RAR [HW][Ericsson][TCL]

    - RAR usage will increase and that may become a bottleneck [Ericsson]

    - Potential increase in false alarm rate [CMCC][Ericsson]

    - R19 EDT will not require Msg1, so OCC enhancement not required [MTK]

OCC factors (M)

- 2 [Sharp][NEC]

- 3 [QC]

- 4 [Sharp]

- 5 [ETRI][ZTE][LGE][Sharp]

  - Maximum of 2 UEs mux-ed [Sharp]

- Note: the value chosen will probably depend on the NPRACH scheme (symbol vs SG etc)	[FL]

OCC codewords

- Split symbol group into two sets of 3 [QC][NEC]

  - Concatenated cyclic shifts of length-3 DFT sequence [QC]

  - CP in OS[0,3], symbols [1,2] and [4,5] have OCC code applied [NEC][TCL]

- DFT sequences defined for NR PUCCH [LGE]

- Consider CP issue when designing codewords [ETRI]

- Codeword index hopping [NEC]

Multiplexing of legacy UEs and OCC UEs

- Strive to allow [QC][Nordic]

Features that OCC should work with:

- Initial access [QC]

- EDT [QC]

- PDCCH order [QC]

- Connected mode CBRA [QC]

RAR

- RAR impact of OCC needs to be taken into account [QC][HW][Sharp]

  - RAPID needs to account for OCC [Ericsson][LGE]

    - RAPID calculated by increasing subcarrier index first and then OCC index [LGE]

  - RAR impacts would cause workload in RAN2 [Samsung]

- Separate RA-RNTI for NDPDSCH-RAR for OCC UEs [Sharp]

  - Allows the MAC PDUs for legacy and OCC UEs to be differentiated [Sharp]

- Separate DCI scheduling RAR-PDSCH as a function of OCC [ZTE][Sharp]

NPRACH resource

- Dedicated NPRACH resources for OCC [Apple][Ericsson][ETRI][Interdigital]

  - Avoids clash between legacy UEs and OCC UEs [Apple][Ericsson]

    - Clash occurs when symbols within symbol group are repeated since FH pattern would then be different between legacy and OCC UEs [Ericsson]

    - Separate OCC NPRACH resource for each coverage level [Lenovo]

- Use all-1s OCC codeword for legacy UEs within NPRACH multiplexing scheme [QC]

Performance requirements

- RAN4 performance requirements on false preamble detection need updating [Ericsson]

Signalling

- Repetition and spreading level [ETRI]

- Sequence length [ETRI]

- Channel for configuration

  - NPDCCH / DCI [ETRI]

  - RRC unicast

  - SIB

- Separate configuration for each coverage level [Lenovo]

Anchor and non-anchor carrier selection probability

- Study if the anchor carrier and non-anchor carrier selection probabilities need enhancing [NEC]

  - Account for there being effectively more NPRACH resources if OCC is applied to some of the carriers

## Whether NPRACH OCC is supported

The following views on whether NPRACH OCC should be supported or not were expressed:

- Support [QC][Lenovo][NEC][Nordic][Nokia][ETRI][Interdigital]

  - Performance OK for up to OCC5 [ZTE]

  - Performance OK with 0.2dB loss to baseline for 3UEs being OCC-ed [QC]

  - Performance degradation of 1dB for OCC2, 3dB for OCC4 at 99% detection probability [ETRI]

  - WID says that NPRACH OCC is supported [Nokia]

  - Specification impacts are small [Nordic]

  - CB-Msg3-EDT is a separate optional feature and does not impact whether NPRACH OCC is supported [ZTE]

    - Note: this is to counter the argument that NPRACH enhancements are not needed in R19 since CG-Msg3-EDT does not use NPRACH: FL

- Not support [HW][Ericsson][Spreadtrum][TCL][MTK][Samsung][CATT][Ericsson][Xiaomi]

  - Reasons:

    - Backward compatibility / impact on legacy UEs [CMCC] [Ericsson]

    - Specification impact [Ericsson][Spreadtum][HW][vivo][Xiaomi][Samsung]

    - NPRACH is not the bottleneck [HW] [CATT][Spreadtrum][HW][Ericsson]

    - Performance with power imbalance, timing error [HW][CMCC][vivo][CATT]

      - Cannot pair UEs with compatible imperfections in NPRACH [vivo]

    - RAN2 impacts in RAR [HW][Ericsson][TCL]

    - RAR usage will increase and that may become a bottleneck [Ericsson]

    - Potential increase in false alarm rate [CMCC][Ericsson]

    - R19 EDT will not require Msg1, so OCC enhancement not required [MTK]

Following agreements in previous meetings, the only NPRACH OCC scheme left for study is the Intra-symbol group OCC scheme.

An issue raised by many companies is that NPRACH is not the bottleneck and hence NPRACH is not a priority for UL capacity enhancement. Many companies also raised the specification and TU effort that would be required to support NPRACH OCC, although some companies think that NPRACH OCC is easier to specify than NPUSCH OCC. Simulations seem to show promising performance for NPRACH with cross-symbol OCC, although performance degrades with imbalance (such as power imbalance). There is widespread agreement that NPRACH OCC would impact RAN2 as they would need to deal with RAR-related issues.

It might be possible to agree on the RAN1 aspects of NPRACH OCC. RAN2 can then still determine whether the RAN2 aspects for support of OCC for NPRACH are manageable or not.

[FL1] Proposal 5_2_1: From a RAN1 perspective, OCC for NPRACH using an intra-symbol group OCC scheme is beneficial.

Companies are invited to comment on proposal 5_2_1.

| Company | Comment |
| --- | --- |
| CATT | So far we didn’t find its benefit. |
| CMCC | As we mentioned in the contribution, the impact of OCC to the detection of PRACH of legacy UEs should be considered. |
| ETRI | Support |
| Qualcomm | Agree. Regarding the false alarm / detection of legacy UEs, in our evaluations we have included the evaluation for legacy UEs (OCC codeword [1 1 1]) and the feature is beneficial even in this case. |
| Nokia, NSB | Agree. |
| Ericsson | We not sure it is beneficial, and on the contrary we have identified several impacts on legacy specification (e.g., RAPID). |
| FL2 | It seems like the word “beneficial” is a stumbling block. The issues of RAPID etc are RAN2 issues and are not meant to be covered by this proposal. Maybe we should send an LS to RAN2 asking whether it can be supported also from a RAN2 perspective. [FL1] Proposal 5_2_1v2: From a RAN1 perspective, OCC for NPRACH using an intra-symbol group OCC is feasible. Send an LS to RAN2 asking about feasibility from a RAN2 perspective. |

## NPRACH OCC scheme

The following issues related to the design of a cross-symbol (intra-symbol group) OCC scheme were raised in Tdocs:

OCC factors (M)

- 2 [Sharp][NEC]

- 3 [QC]

- 4 [Sharp]

- 5 [ETRI][ZTE][LGE][Sharp]

  - Maximum of 2 UEs mux-ed [Sharp]

- Note: the value chosen will probably depend on the NPRACH scheme (symbol vs SG etc)	[FL]

OCC codewords

- Split symbol group into two sets of 3 [QC][NEC]

  - Concatenated cyclic shifts of length-3 DFT sequence [QC]

  - CP in OS[0,3], symbols [1,2] and [4,5] have OCC code applied [NEC][TCL]

- DFT sequences defined for NR PUCCH [LGE]

- Consider CP issue when designing codewords [ETRI]

- Codeword index hopping [NEC]

This issue only needs to be progressed if RAN1 determines that NPRACH OCC should be supported, according to section 5.2.

The main proposals for how OCC is implemented for NPRACH centre around splitting the symbol group into two sets of 3 symbols and then applying an OCC operation between those two sets of symbols. If we can agree on this approach, we can at a future time determine details of the OCC scheme, which will inevitably lead to a decision on the OCC factors. FL notes from previous discussion (in section 5.2) that NPRACH is not a particular bottleneck – this observation would motivate a lower value for supported OCC factor (rather than a higher OCC factor).

[FL1] Proposal 5.3-1: For NPRACH OCC, the symbol group is split into two sets of 3 symbols. The OCC operation is applied between the two sets of symbols. FFS: details of OCC operation.

Companies are invited to comment on proposal 5.3-1.

| Company | Comment |
| --- | --- |
| LGE | For clarification, we can add “including CP” after “the symbol group”. Meanwhile, this approach would need to restrict the NPRACH format as well. To be specific, only NPRACH format 1 could be used since the CP length would be different from that of symbols for NPRACH format 0. |
| CATT | Need to discuss the benefit when UL synchronization is not holding in PRACH transmission. |
| ETRI | This approach may be valid for NPRACH format 1. |
| Qualcomm | Based on our evaluation, spreading of 3 is workable with similar specification impact as 2. |
| Nokia, NSB | Not support. The minimized processing is to have OCC for entire symbol group but not splitting into two sets. And the two sets solution will impact more on performance and complexity of NPRACH processing. |
| Ericsson | The proposal needs to include more clarifications about the proposed scheme, for example. Is the intention using an OCC length 3? In our understanding the proposed scheme also requires repeating the OCC sequences and applying a cyclic shift operation. |
| FL2 | It seems like this proposal needs more work. We need to firstly get over the hurdle of whether we are going to support NPRACH OCC (section 5.2). |

## RAR impacts

The following issues related to RAR were raised in Tdocs:

RAR

- RAR impact of OCC needs to be taken into account [QC][HW][Sharp]

  - RAPID needs to account for OCC [Ericsson][LGE]

    - RAPID calculated by increasing subcarrier index first and then OCC index [LGE]

  - RAR impacts would cause workload in RAN2 [Samsung]

- Separate RA-RNTI for NDPDSCH-RAR for OCC UEs [Sharp]

  - Allows the MAC PDUs for legacy and OCC UEs to be differentiated [Sharp]

- Separate DCI scheduling RAR-PDSCH as a function of OCC [ZTE][Sharp]

The RAR currently identifies a UE by the RAPID, indicating the NPRACH resources used by the UE. Since the time-frequency NPRACH resources would be used by multiple UEs with OCC, the RAPID might need to be updated to also indicate the OCC codeword used by the UE. Alternatively (or additionally), a new RA-RNTI could be used for RAR which would stop legacy UEs from parsing the Rel-19 NPDSCH-RAR. Alternatively, separate RAR-PDSCH (where a RAR-PDSCH is linked to an OCC codeword) could be signalled by indicating the OCC codeword in the DCI.

It would seem like there are RAR impacts. These issues are probably best left to RAN2 once the basic NPRACH OCC scheme has been agreed in RAN1.

[FL1] Question 5.4-1: Should RAN2 deal with RAR impacts of NPRACH OCC?

Companies are invited to comment on question 5.4-1.

| Company | Comment |
| --- | --- |
| LGE | In our understanding, when we first design NPRACH and relevant information, RAN1 decided to redefine RAPID for NPRACH case. I believe RAN1 decided whit issue. |
| Qualcomm | Once RAN1 agrees to supporting this feature, RAN2 can look into the details. |
| Nokia, NSB | Yes. RAN2 to deal with it. |
| Ericsson | There are other aspects that need to be discussed in RAN1, for example the random access response won’t be orthogonal (Msg2), thus there is a risk of incurring in a DL bottle neck. |
| FL2 | I think this can be further considered after we resolve the support of NPRACH in section 5.2. |

## NPRACH partitioning

The following issues related to NPRACH partitioning were raised in Tdocs:

NPRACH resource

- Dedicated NPRACH resources for OCC [Apple][Ericsson][ETRI][Interdigital]

  - Avoids clash between legacy UEs and OCC UEs [Apple][Ericsson]

    - Clash occurs when symbols within symbol group are repeated since FH pattern would then be different between legacy and OCC UEs [Ericsson]

    - Separate OCC NPRACH resource for each coverage level [Lenovo]

- Use all-1s OCC codeword for legacy UEs within NPRACH multiplexing scheme [QC]

Many companies think that separate NPRACH resource is required for OCC UEs, meaning that NPRACH partitioning would be necessary.

Qualcomm propose an NPRACH OCC scheme where legacy UEs can use the all-1s OCC codeword (i.e. use the existing sequence in NPRACH) while OCC UEs would use different sequences. In this scheme, both legacy UEs and OCC UEs could share the same NPRACH resource. Whether this approach would be feasible would depend on the details of the NPRACH OCC scheme.

It seems like the necessity of NPRACH partitioning depends on the NPRACH OCC scheme itself. Hence, it is proposed to defer the discussion on NPRACH partitioning for later in the work item.

## Signalling

The following issues related to signalling for NPRACH OCC were raised in Tdocs:

Signalling

- Repetition and spreading level [ETRI]

- Sequence length [ETRI]

- Channel for configuration

  - NPDCCH / DCI [ETRI]

  - RRC unicast

  - SIB

- Separate configuration for each coverage level [Lenovo]

From RAN1#118bis Hefei, there was a general view that more significant issues for NPRACH OCC need to be discussed first (e.g. whether NPRACH OCC is supported – section 5.2 and the NPRACH OCC scheme).

Hence, it is proposed to defer the discussion on signalling for NPRACH OCC to a later meeting.

# CG-Msg3-EDT LS

RAN1 received an LS from RAN2 on whether OCC can be supported in CG-msg3 EDT [4]. RAN2 asked the following question:

# 1. Overall Description: RAN2 is working on CB-msg3 EDT in IDLE state and would like to ask RAN1 whether the OCC solution for NPUSCH on which RAN1 is working can be applied to CB-msg3 NPUSCH or not. # 2. Actions: To RAN1 group: RAN2 respectfully asks RAN1 to take the information above into consideration and provide the feedback.

The following views were expressed related to the LS :

- Feasibility:

  - Any SCS:

    - Yes: Spreadtrum, ZTE, NEC, Ericsson, Qualcomm, MTK

      - Evaluation assumptions for OCC have been consistent with those for CG-Msg3 EDT: QC

    - More evaluation: CMCC, Samsung, Nokia

      - How to ensure sufficient timing accuracy for UE: Nokia

    - No: CATT, HW

      - Potential performance degradation: CATT

      - eNB complexity to decode all OCC candidates: CATT

      - Not clear it is in WID scope: CATT

      - Too many issues that have not yet been evaluated to be able to reach a consensus: HW

  - 3.75kHz

    - Yes: vivo, Apple, OPPO

    - More evaluation: CMCC, Samsung

    - No:

  - 15kHz

    - Yes:

    - More evaluation: CMCC, Samsung, vivo

    - No:

  - Restrictions:

    - Only single-tone (RAN1 has not specified OCC for MT yet): Ericsson

    - OCC2: to avoid Msg4 bottleneck: Ericsson

    - Performance depends on collision rate, which depends on time / frequency resources: Ericsson

- Assumptions / pairing

  - Need to know the assumptions on the following:

    - Power imbalance [CMCC]

    - Timing (DL sync) [CMCC]

    - CFO conditions [CMCC]

- Evaluations

  - Need evaluations of performance considering:

    - Power imbalance [CMCC][HW]

    - Timing (DL sync) [CMCC]

    - CFO conditions [CMCC]

- RAN1 needs more time to decide whether OCC can be supported for CG-Msg3-EDT [Samsung]

- RAN1 needs to progress on OCC for NPUSCH before being able to answer: LGE

- Configuration information needs to be defined: ZTE, Nokia

- More details needed on definition of CG-msg3 EDT: ZTE

- How to make UEs choose different OCC codewords: LGE

- How to define RNTI value to use for scrambling of CG-msg3 EDT: LGE

- How to ensure alignment of OCC codewords for CG-msg3 EDT: LGE

9 companies think that OCC solution can be applied to CG-Msg3 EDT for at least 3.75kHz SCS (and 5 of those companies think that it can be supported at any NB-IoT SCS). However, 5 companies either think that the OCC solution cannot be applied to CG-Msg3 EDT or that more evaluation is required.

It would seem that currently there is no consensus in RAN1 for the application of OCC to CG-Msg3 EDT. It is hence proposed to reply to RAN2 as follows:

[FL2] Proposal 6_1v1: The following response is sent in reply to RAN2 LS R1-2409343

Reply to the Question 1: RAN1 do not currently have consensus to support OCC in the case of CB-msg3 NPUSCH.

Companies are invited to comment on question 5.4-1.

| Company | Comment |
| --- | --- |

# Monday: Proposals for online session

Proposal 4_2_2v1:

For 3.75kHz SCS OCC for NPUSCH format 1, the maximum OCC length is 2.

Proposal 4_3_2v1:

For 15kHz SCS OCC for NPUSCH format 1, the maximum OCC length is 2.

Proposal 4_3_1v2:

For 15kHz SCS single tone, RAN1 studies:

- Option 1: Nslot = 1 spreading: redefine that Nslot = 1 for OCC and that Midentical, NPSUCH is equal to the OCC length.

- Option 2: Nslot = 2 spreading: the current value of Nslot is maintained for single-tone at 15kHz SCS. Midentical, NPSUCH is equal to the OCC length.

Proposal 4_3_3v2:

For 15kHz CDM DMRS with legacy pattern, RAN1 studies:

- Option 1: DMRS symbols are spread before the OCC is applied.

  - In the spreading function, the complex-valued DMRS symbol used in slot 1 is copied to slot 2 and OCC is applied across those identical complex-valued DMRS symbols

- Option 2: DMRS symbols are not spread before the OCC is applied.

  - OCC is applied to the legacy complex-valued DMRS symbol used in slot 1 and slot 2.

Proposal 4.7-1v2:

For support of single-tone OCC for NPUSCH format 1, the parameters that need to be signalled are:

  - OCC codeword

  - Enabling of OCC feature

Proposal 4.9-1:

For NPUSCH format 1 single-tone for both 15kHz and 3.75kHz, RAN1 studies how alignment is supported, including:

- Modification to scheduling delays (k0) between NPDCCH and NPUSCH, considering appropriate repetition values for NPDCCH

- FFS: other methods

Proposal 5_2_1v2:

From a RAN1 perspective, OCC for NPRACH using an intra-symbol group OCC is feasible. Send an LS to RAN2 asking about feasibility from a RAN2 perspective.

# Conclusions

This document is the feature lead summary for IoT-NTN in RAN1#119. It contains the FLS discussion and lists the proposals that were considered in online and offline sessions.

# References

[1]     RP-241624, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, MediaTek Inc. (Rapporteur), RAN#104, June 17-20, 2024

[2]	R1-2409309 “Final FL Summary for IoT-NTN”. RAN1#118bis, Hefei, China. Moderator (Sony)

[3] 	R1-2401298 “Work Plan for Rel-19 IoT NTN”. Mediatek (rapporteur)

[4]	R1-2409343. “LS on OCC for CB-msg3 NPUSCH”. RAN1#119, Orlando, USA. Source (RAN2).

R1-2409408	Discussion on UL capacity enhancements for IoT NTN	Huawei, HiSilicon

R1-2409530	Discussion on the IoT -NTN uplink capacity/throughput enhancements	CMCC

R1-2409617	Discussion on uplink capacity/throughput enhancement for IoT-NTN	Samsung

R1-2409653	Discussion on IoT-NTN uplink capacity/throughput enhancement	Spreadtrum, UNISOC

R1-2409701	Discussion on IoT-NTN uplink capacity enhancement	vivo

R1-2409722	IoT-NTN uplink capacity/throughput enhancement	InterDigital, Inc.

R1-2409729	Discussion on UL capacity enhancement for IoT NTN	ZTE Corporation, Sanechips

R1-2409826	Discussion on IoT-NTN Uplink Capacity Enhancement	Apple

R1-2409834	Discussion on IoT-NTN uplink capacity/throughput enhancement	LG Electronics

R1-2409839	Discussion on the IoT-NTN uplink capacity/throughput enhancements	TCL

R1-2409848	IoT-NTN uplink capacity enhancement	Nokia, Nokia Shanghai Bell

R1-2409872	IoT-NTN uplink capacity/throughput enhancement	NEC

R1-2409886	Discussion on IoT-NTN uplink capacity enhancement	Xiaomi

R1-2409981	Discussion on UL capacity enhancement for IoT NTN	CATT

R1-2410082	Discussion on IoT-NTN uplink capacity/throughput enhancement	OPPO

R1-2410161	IoT NTN OCC methods and signaling for NPUSCH and NPRACH	Sharp

R1-2410240	On IoT-NTN uplink capacity enhancements	Sony

R1-2410280	Discussion on uplink capacity/throughput enhancement for IoT NTN	ETRI

R1-2410306	On uplink capacity enhancements for IoT-NTN	Ericsson

R1-2410365	Discussion on uplink capacity enhancement for IoT NTN	Lenovo

R1-2410498	IOT-NTN uplink capacity/throughput enhancement	Qualcomm Incorporated

R1-2410503	Operator views on KPIs for uplink capacity enhancement for IOT- NTN 	ViaSat Satellite Holdings Ltd

R1-2410506	IoT-NTN uplink capacity/throughput enhancement	Nordic Semiconductor ASA

R1-2410529	IoT-NTN uplink capacity and throughput enhancement	MediaTek Inc.

LS-related Tdocs:

R1-2409343	LS on OCC for CB-msg3 NPUSCH	RAN2, vivo

RAN2 is requesting RAN1 input on the applicability of OCC solution for CB-msg3 NPUSCH. Response needed. To be handled in agenda item 9.11. To be moderated by Martin (Sony).

Relevant tdocs:

R1-2409498	Discussion on LS on OCC for CB-msg3 NPUSCH	CMCC

R1-2409572	Draft reply RAN2 LS on OCC for CB-msg3 NPUSCH	Samsung

R1-2409622	Discussion on LS on OCC for CB-msg3 NPUSCH	Spreadtrum, UNISOC

R1-2409661	Draft reply LS on OCC for CB-msg3 NPUSCH	vivo

R1-2409662	Discussion on LS on OCC for CB-msg3 NPUSCH	vivo

R1-2409723	Discussion on LS on OCC for CB-msg3 NPUSCH	ZTE Corporation, Sanechips

R1-2409785	Draft Reply LS to RAN2 on OCC for CB-msg3 NPUSCH	Apple

R1-2409830	Discussion on LS on OCC for CB-msg3 NPUSCH	LG Electronics

R1-2409850	Discussion on LS from RAN2 on OCC for CB-msg3 NPUSCH	Nokia, Nokia Shanghai Bell

R1-2409869	Discussion on RAN2 LS on OCC for CB-msg3 NPUSCH	NEC

R1-2409983	Discussion on LS reply on OCC for CB-msg3 NPUSCH	CATT

R1-2410084	Discussion on OCC for CB-msg3 NPUSCH	OPPO

R1-2410085	Draft reply LS on OCC for CB-msg3 NPUSCH	OPPO

R1-2410309	Discussion LS on OCC for CB-msg3 NPUSCH	Ericsson

R1-2410328	Discussion on the reply of LS on OCC for CB-msg3 NPUSCH	Huawei, HiSilicon

R1-2410329	Draft reply LS on OCC for CB-msg3 NPUSCH	Huawei, HiSilicon

R1-2410459	Discussion on OCC for CB-msg3 NPUSCH	Qualcomm Incorporated

R1-2410507	Discussion on reply LS on OCC for CB-msg3 NPUSCH in NB-IoT NTN	MediaTek Inc.

R1-2409351	Reply LS to RAN1 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2	RAN4, ZTE Corporation

RAN4 response to R1-2407548 from RAN1#118bis. To be taken into account as part of discussions on Rel-18 IoT-NTN maintenance.