3GPP TSG-RAN WG1 Meeting #119	R1-2410770

Orlando, USA, November 18th – 22nd, 2024

Agenda Item:	7

Source:	Moderator(Huawei)

Title:	Summary#2 for discussion on SCell dormancy indication case 2

Document for:	Discussion and Decision

Introduction

Discussion on the issue of SCell dormancy indication case 2 in case of BWP switching is summarized in this document. Below is the relevant proponent contribution.

| R1-2410608 | Correction on SCell dormancy indication case 2 in case of BWP switching | Huawei, HiSilicon |
| --- | --- | --- |

Background

For SCell dormancy indication Case 2, SCell dormancy indication in a DCI format 1_1 on the PCell is provided by repurposing a few specified DCI fields comprising MCS, NDI, RV, HARQ number process, Antenna port(s), and DMRS sequence initialization. The “invalid FDRA” is used for trigger the implicit SCell dormancy indication in the DCI format 1_1.

The method of DCI format interpretation in the case of BWP switching is presented in the current specification [1]. According to the description below, since the condition does not include whether switching to the indicated BWP occurs, the DCI format interpretation is based on the indicated DL BWP, as long as the indicated DL BWP is different from the active DL BWP, regardless of whether the switch to the indicated BWP occurs.

If a bandwidth part indicator field is configured in a DCI format, the bandwidth part indicator field value indicates the active DL BWP, from the configured DL BWP set, for DL receptions as described in [5, TS 38.212]. If a bandwidth part indicator field is configured in a DCI format, the bandwidth part indicator field value indicates the active UL BWP, from the configured UL BWP set, for UL transmissions as described in [5, TS 38.212]. If a bandwidth part indicator field is configured in a DCI format and indicates an UL BWP or a DL BWP different from the active UL BWP or DL BWP, respectively, the UE shall - for each information field in the DCI format - if the size of the information field is smaller than the one required for the DCI format interpretation for the UL BWP or DL BWP that is indicated by the bandwidth part indicator, the UE prepends zeros to the information field until its size is the one required for the interpretation of the information field for the UL BWP or DL BWP prior to interpreting the DCI format information fields, respectively - if the size of the information field is larger than the one required for the DCI format interpretation for the UL BWP or DL BWP that is indicated by the bandwidth part indicator, the UE uses a number of least significant bits of the DCI format equal to the one required for the UL BWP or DL BWP indicated by bandwidth part indicator prior to interpreting the DCI format information fields, respectively - set the active UL BWP or DL BWP to the UL BWP or DL BWP indicated by the bandwidth part indicator in the DCI format

If a bandwidth part indicator in this DCI format 1_1 indicates a DL BWP which is different from the active DL BWP of PCell, and the value of the FDRA field is invalid, the DCI format interpretation is based on the indicated DL BWP.  In this case, the active DL BWP of this serving cell will not switch to the DL BWP indicated by the bandwidth part indicator, since the value of the FDRA field in this DCI format 1_1 is invalid. However, if the size of the information field in the active BWP is smaller than that in the indicated BWP, prepending zeros is required prior to interpreting the DCI format information fields. This prepending can result in padded zero bit being mapped onto an SCell, accidentally imposing that SCell into dormancy, even though the gNB might not intend to indicate dormancy for these particular SCells.

As illustrated in Figure 1, if the size of Antenna port(s) field in the active DL BWP is smaller than that in the indicated DL BWP, prepending two zero bits is required prior to interpreting the DCI format information fields. This prepending can result in two prepended zero bits being mapped to an SCell, accidentally imposing that SCell into dormancy, such as, SCell#9 and SCell#10, even though the gNB does not intend to indicate dormancy for these particular SCells.

Figure 1 For DCI format 1_1, MCS, NDI, RV, HPN and AP fields interpreted based on the indicated DL BWP which is different from the active DL BWP

The issue was first raised during the R18 single DCI maintenance phase [2]. In the last meeting, it was suggested to discuss it in Rel-16, since the implicit SCell dormancy indication was introduced in Rel-16 and has the same problem. We also acknowledge that this issue exists in legacy NR. Furthermore, if this issue is not addressed in R16, it will have a larger impact on later introduced features, such as R18 DCI format 1_3. Because the sizes of more DCI field information can be configured per BWP through related RRC parameters as listed in Table 1, when the bandwidth part indicator indicates a DL BWP that is different from the active DL BWP, more DCI field information may be affected by prepended zeros, resulting in these zeros being mapped to the SCell.

Table 1 DCI fields in DCI format 1_3 related to implicit SCell dormancy indication and corresponding higher layer parameters

| DCI format 1_3 |  |  |
| --- | --- | --- |
| DCI information field | RRC parameter (in PDSCH-Config) | number of bits |
| MCS | mcs-Table | 5 bits |
| NDI | —— | 1 bits |
| RV | numberOfBitsForRV-DCI-1-3 | 0, 1, 2 bits |
| HARQ process number | harq-ProcessNumberSizeDCI-1-3 | 0, 1, 2, 3, 4 or 5 bits |
| Antenna port(s) | dmrs-Type, maxLength | 4, 5, or 6 bits |

As depicted in Figure 2, the sizes of RV, HPN, and AP are configured with values of 1 bit, 1 bit, and 4 bits, respectively, based on corresponding higher layer parameters in the active DL BWP. In the indicated DL BWP, the sizes of RV, HPN, and AP are configured with values of 2 bits, 2 bits, and 6 bits, respectively, based on corresponding higher layer parameters. The size of the information field in the active DL BWP is smaller than that in the indicated DL BWP, prepending with zeros is required prior to interpreting the DCI format information fields. This padding can result in prepended zero bits being mapped to SCell#7, SCell#9, SCell#11, and SCell#12, imposing dormancy on these SCells.

Figure 2 For DCI format 1_3, MCS, NDI, RV, HPN and AP fields interpreted based on the indicated DL BWP which is different from the active DL BWP

Consequently, if a bandwidth part indicator indicates a DL BWP which is different from the active DL BWP, determining the size of the SCell dormancy indication field based on the indicated BWP may reduce the flexibility of SCell dormancy signaling. Therefore, the issue needs to be resolved.

In the last meeting, we proposed to interpret the field information for SCell dormancy indication case 2 based on the active DL BWP in this case, as presented in Option 1. However, some companies mentioned that the proposed method differs from the legacy mechanism and seems to have a considerable impact on the specification. In order to consist with the legacy mechanism, another method as presented in Option 2 can also be considered. The interpretation method will be consistent with the legacy mechanism, and prepended zeros will not be mapped to SCells. To summarize, if a bandwidth part indicator indicates a DL BWP that is different from the active DL BWP, and the size of the information field in the active DL BWP is smaller than that in the indicated DL BWP, the two options are as follows:

- Option 1: Interpret the field information for SCell dormancy indication case 2 based on the active DL BWP.

- Option 2: Interpret the field information for SCell dormancy indication case 2 based on the indicated DL BWP. Prepended zeros are not mapped to SCells.

Discussion

#### Question#1

Do you agree with the identified issue in R1-2410608 towards specification changes?

| Company | Comment |
| --- | --- |
| CATT | Yes |
| Qualcomm | No |
| Spreadtrum | No According to 38.321, SCell dormancy case 2 cannot indicate BWP switching. BWP switching DCI only can be a PDCCH indicating a downlink assignment or UL grant. 38.321: The BWP switching for a Serving Cell is used to activate an inactive BWP and deactivate an active BWP at a time. The BWP switching is controlled by the PDCCH indicating a downlink assignment or an uplink grant, by the bwp-InactivityTimer, by RRC signalling, or by the MAC entity itself upon initiation of Random Access procedure or upon detection of consistent LBT failure on SpCell.. |
| ZTE | No. Interpreting the DCI fields based on the indicated BWP with adding padding bits or puncturing MSB is the correct UE behavior in the current spec. This is applied to many cases. There is no issue for this scenarios. |
| Huawei, HiSilicon | Yes This change of BWP ID does not directly lead to real BWP switching. From DCI processing perspective, a UE will not switch its BWP if the FDRA is invalid as agreed, while the BWP ID may be different as allowed by current spec. This issue could be a more functional scenario for DCI format 1_3 which schedules both invalid FDRA and valid FDRA of multiple PDSCHs, with BWP ID flexibilly indicated by the same DCI. |
| Samsung | Not an essential correction. The mentioned issue is not a typical case, and can be easily avoided by the NW. |
| NTT DOCOMO | No. We agree with companies that in case of Rel-16, FDRA is invalid for SCell dormancy indication case 2, and hence it is error case if indicated BWP ID is different from current active BWP ID. |

#### Question#2

If your answer is yes for question#1, which option is preferred between Option 1 and Option 2 below? Or else?

- Option 1: Interpret the field information for SCell dormancy indication case 2 based on the active DL BWP.

  - Also see corresponding TP#1 in section 5.

- Option 2: Interpret the field information for SCell dormancy indication case 2 based on the indicated DL BWP. Prepended zeros are not mapped to SCells.

  - Also see corresponding TP#2 in section 5.

| Company | Comment |
| --- | --- |
| CATT | Option 2. However, the proposed text is not clear to address the condition of prepended “0” for the indicated BWP. |
| Qualcomm | This issue can be avoided by gNB. For case 2 dormancy indication DCI, there is no BWP switching due to the invalid FDRA field. Then there is no reason that the gNB would set the BWP indicator field different than the active BWP ID. This issue should be considered as an error case and UE behaviour is not specified. |
| ZTE | The gNB can handle this if the gNB vendor believes this is an issue. |
| Huawei, HiSilicon | This discussion can be different between a legacy DCI format and DCI format 1_3. For legacy DCI format, we can discuss first whether a UE expects BWP ID changes. |

Conclusion

tbd

TP

## TP#1 for Option 1

- 10.3 PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells < Unchanged parts are omitted > If a UE is provided search space sets to monitor PDCCH for detection of DCI format 1_1, or of DCI format 1_3, and if - the CRC of DCI format 1_1 or of DCI format 1_3 is scrambled by a C-RNTI or an MCS-C-RNTI, and if - a one-shot HARQ-ACK request field is not present or has a '0' value, and if - for DCI format 1_3, a HARQ-ACK retransmission indicator field is not present or has a '0' value, and if - the UE detects a DCI format 1_1 on the primary cell that does not include a carrier indicator field, or detects a DCI format 1_1 on the primary cell that includes a carrier indicator field with value equal to 0, or detects a DCI format 1_3 on the primary cell, and if - resourceAllocation = resourceAllocationType0 and all bits of the frequency domain resource assignment field in DCI format 1_1, or for one or more blocks of the frequency domain resource assignment field in DCI format 1_3, are equal to 0, or - resourceAllocation = resourceAllocationType1 and all bits of the frequency domain resource assignment field in DCI format 1_1, or for one or more blocks of the frequency domain resource assignment field in DCI format 1_3, are equal to 1, or - resourceAllocation = dynamicSwitch and all bits of the frequency domain resource assignment field in DCI format 1_1, or for one or more blocks of the frequency domain resource assignment field in DCI format 1_3, are equal to 0 or 1 the UE considers the DCI format 1_1 or the DCI format 1_3 as indicating SCell dormancy, not scheduling a PDSCH reception on a serving cell, where for DCI format 1_3 the serving cell is the one with the smallest index that is associated with a block from the one or more blocks of the frequency domain resource assignment field, and for transport block 1 interprets the sequence of fields of - modulation and coding scheme - new data indicator - redundancy version and of - HARQ process number - antenna port(s) for DCI format 1_1, or for DCI format 1_3 if AntennaPortsDCI1-3 is configured as 'type2' - DMRS sequence initialization for DCI format 1_1 according to the current active DL BWP as described in clause 12, as providing a bitmap to each configured SCell, in an ascending order of the SCell index, where - a '0' value for a bit of the bitmap indicates an active DL BWP, provided by dormantBWP-Id, for the UE for a corresponding activated SCell - a '1' value for a bit of the bitmap indicates - an active DL BWP, provided by firstWithinActiveTimeBWP-Id, for the UE for a corresponding activated SCell, if a current active DL BWP is the dormant DL BWP - a current active DL BWP, for the UE for a corresponding activated SCell, if the current active DL BWP is not the dormant DL BWP - the UE sets the active DL BWP to the indicated active DL BWP If an active DL BWP provided by dormantBWP-Id for a UE on an activated SCell is not a default DL BWP for the UE on the activated SCell, as described in clause 12, the BWP inactivity timer is not used for transitioning from the active DL BWP provided by dormantBWP-Id to the default DL BWP on the activated SCell. < Unchanged parts are omitted >

## TP#2 for Option 2

- 10.3 PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells < Unchanged parts are omitted > If a UE is provided search space sets to monitor PDCCH for detection of DCI format 1_1, and if - the CRC of DCI format 1_1 is scrambled by a C-RNTI or a MCS-C-RNTI, and if - a one-shot HARQ-ACK request field is not present or has a '0' value, and if - the UE detects a DCI format 1_1 on the primary cell that does not include a carrier indicator field, or detects a DCI format 1_1 on the primary cell that includes a carrier indicator field with value equal to 0, and if - resourceAllocation = resourceAllocationType0 and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 0, or - resourceAllocation = resourceAllocationType1 and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 1, or - resourceAllocation = dynamicSwitch and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 0 or 1 the UE considers the DCI format 1_1 as indicating SCell dormancy, not scheduling a PDSCH reception, and for transport block 1 interprets the sequence of fields of - modulation and coding scheme - new data indicator - redundancy version and of - HARQ process number - antenna port(s) - DMRS sequence initialization as providing a bitmap to each configured SCell, except the prepended zero(s), in an ascending order of the SCell index, where - a '0' value for a bit of the bitmap indicates an active DL BWP, provided by dormantBWP-Id, for the UE for a corresponding activated SCell - a '1' value for a bit of the bitmap indicates - an active DL BWP, provided by firstWithinActiveTimeBWP-Id, for the UE for a corresponding activated SCell, if a current active DL BWP is the dormant DL BWP - a current active DL BWP, for the UE for a corresponding activated SCell, if the current active DL BWP is not the dormant DL BWP - the UE sets the active DL BWP to the indicated active DL BWP < Unchanged parts are omitted >