3GPP TSG RAN WG1 #119			R1-2410774

Orlando, US, November 18th – 22nd, 2024

Source:	Moderator (ZTE)

Title:	Summary of the discussion on Type-1 and Type-2 HARQ-ACK codebook

Agenda item:	8.1

Document for:	Discussion and Decision

Introduction

This document provides the summary of Type-1 and Type-2 HARQ-ACK codebook proposed by [1].

R1-2410190	Draft CR on Type-1 and Type-2 HARQ-ACK codebook	ZTE Corporation, Sanechips

Discussion

For Type-1 or Type-2 HARQ-ACK codebook, the HARQ-ACK information for a cell will be skipped when the active UL BWP change occurs on the cell for PUCCH transmission and the dynamic PUCCH cell switching is configured, or the active UL BWP change occurs on the PCell and the dynamic PUCCH cell switching is not configured. In Rel-18 MC, PUCCH cell switching can be dynamically indicated by PUCCH cell indicator in DCI format 1_3 if higher layer parameter pucch-sSCellDynDCI-1-3 is configured. Therefore, the PUCCH cell can be indicated by DCI format 1_1, 1_2 or 1_3.

- Above situation is missing for the second sub-codebook of Type-2 HARQ-ACK codebook generation in multi-cell scheduling.

- Since both the first sub-codebook and the second sub-codebook of the Type-2 HARQ-ACK codebook can be transmitted on a same PUCCH, the above situation is also missing for the first sub-codebook of Type-2 HARQ-ACK codebook generation.

- There is a typo in Type-1 HARQ-ACK codebook generation.

Therefore, the following draft CR is provided for Type-1 codebook and Type-2 HARQ-ACK codebook.

- 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel <Unchanged parts are omitted> In the following pseudo-code, the subslotLengthForPUCCH is for the primary cell if the UE is provided pucch-sSCellPattern; otherwise, subslotLengthForPUCCH is for the serving cell of the PUCCH transmission. Set j=0 - index of occasion for candidate PDSCH reception or SPS PDSCH release or TCI state update Set B=∅ Set MA,c=∅ Set CK1 to the cardinality of set K1 Set k=0 – index of slot timing values K1,k, in descending order of the slot timing values, in set K1 for serving cell c If a UE is not provided ca-SlotOffset for any serving cell of PDSCH receptions and for the serving cell of corresponding PUCCH transmission with HARQ-ACK information while k<CK1 if modnU−K1,k+1,max2μUL−μDL,1=0 or subslotLengthForPUCCH is provided for the HARQ-ACK codebook Set nD=0 – index of a DL slot overlapping with an UL slot Set Nk to a number of DL slots overlapping with UL slot nU−K1,k if subslotLengthForPUCCH is provided for the HARQ-ACK codebook; otherwise, Nk=max2μDL−μUL,1 while nD<Nk if pdsch-TimeDomainAllocationListForMultiPDSCH and timeDomainHARQ-BundlingType1 are provided for serving cell c R=RT; R'=RT'; elseif pdsch-TimeDomainAllocationListForMultiPDSCH is provided and timeDomainHARQ-BundlingType1 is not provided for serving cell c R=RT; else Set R to the set of rows end if Set CR to the cardinality of R Set r=0 – index of row in set R if slot nU starts at a same time as or after a slot for an active DL BWP change on serving cell c or an active UL BWP change on the serving cell of PUCCH transmission if the UE is provided pucch-sSCellDyn or pucch-sSCellDynDCI-1-2 or pucch-sSCellDynDCI-1-3, or an active UL BWP change on the PCell if the UE is not provided pucch-sSCellDyn and pucch-sSCellDynDCI-1-2 and pucch-sSCellDynDCI-1-3, and slot n0,k+nD is before the slot for the active DL BWP change on serving cell c or the active UL BWP change on the serving cell of PUCCH transmission, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and slot n0,k+nD overlaps with UL slot nU−K1,k−1, k>0, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU−K1,k, nD=nD+1; else while r<CR if the UE is not provided timeDomainHARQ-BundlingType1 and is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot n0,k+nD−NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r is configured as UL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated where K1,k is the k-th slot timing value in set K1, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU−K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and the end of the PDSCH time resource for row r is not within any UL slot nU−K1,l, 0≤l<CK1, or if pdsch-TimeDomainAllocationListForMultiPDSCH is provided and HARQ-ACK information for PDSCH time resource derived by row r in slot n0,k+nD cannot be provided in slot nU R=R\r; elseif the UE is provided timeDomainHARQ-BundlingType1 and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot n0,k+nD− ∆K0,r(d), at least one symbol of each PDSCH time resource derived by row r of set R' is configured as UL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, and for each slot from n0,k+nD−NPDSCHrepeat,max+1 to slot n0,k+nD at least one symbol of the PDSCH time resource derived by row r of set R is configured as UL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if the row r of set R belongs to time domain resource allocation table configured for DCI format 1_2, where d = 0,1,…,C∆K0,r−1, ∆K0,r=maxK0K0,r−K0,r, and C∆K0,r is the cardinality of ∆K0,r. R=R\r; R'=R'\r; else r=r+1; end if end while if the UE is not provided with multiPDSCH-perSlotType1-CB and the UE does not indicate a capability to receive more than one unicast PDSCH or multicast PDSCH per slot and R≠∅, or if the UE is provided with multiPDSCH-perSlotType1-CB = 'disabled' and R≠∅, MA,c=MA,c∪j; j=j+1; else Set CR to the cardinality of R Set m to the smallest last OFDM symbol index, as determined by the SLIV, among all rows of R while R≠∅ Set r=0 while r<CR if S≤m for start OFDM symbol index S for row r br,k,nD=j; - index of occasion for candidate PDSCH reception, or SPS PDSCH release, or TCI state update associated with row r R=R\r; B=B∪br,k,nD; else r=r+1; end if end while MA,c=MA,c∪j j=j+1; Set m to the smallest last OFDM symbol index among all rows of R; end while end if nD=nD+1; end if end while end if k=k+1; end while else while k<CK1 if modnU−K1,k+(Nslot,offsetUL2μoffset,UL−Nslot,offset,cDL2μoffset,DL,c)∙2μUL+1,max⁡(2μUL−μDL,1)=0 or subslotLengthForPUCCH is provided for the HARQ-ACK codebook Set nD=0 – index of a DL slot overlapping with an UL slot Set Nk to a number of DL slots overlapping with UL slot nU−K1,k if subslotLengthForPUCCH is provided for the HARQ-ACK codebook; otherwise, Nk=max2μDL−μUL,1 while nD<Nk if pdsch-TimeDomainAllocationListForMultiPDSCH and timeDomainHARQ-BundlingType1 are provided for serving cell c R=RT; R'=RT'; elseif pdsch-TimeDomainAllocationListForMultiPDSCH is provided and timeDomainHARQ-BundlingType1 is not provided for serving cell c R=RT; else Set R to the set of rows end if Set CR to the cardinality of R Set r=0 – index of row in set R if slot nU starts at a same time as or after a slot for an active DL BWP change on serving cell c or an active UL BWP change on the serving cell of PUCCH transmission if the UE is provided pucch-sSCellDyn or pucch-sSCellDynDCI-1-2 or pucch-sSCellDynDCI-1-3, or an active UL BWP change on the PCell if the UE is not provided pucch-sSCellDyn and pucch-sSCellDynDCI-1-2 and pucch-sSCellDynDCI-1-3, and slot n0,k+nD is before the slot for the active DL BWP change on serving cell c or the active UL BWP change on the serving cell of PUCCH transmission where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU−K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and slot n0,k+nD overlaps with UL slot nU−K1,k−1, k>0, nD=nD+1; else while r<CR if the UE is not provided timeDomainHARQ-BundlingType1 and is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot n0,k+nD−NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r is configured as UL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated where K1,k is the k-th slot timing value in set K1, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU−K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and the end of the PDSCH time resource for row r is not within any UL slot nU−K1,l, 0≤l<CK1 or if pdsch-TimeDomainAllocationListForMultiPDSCH is provided and HARQ-ACK information for PDSCH time resource derived by row r in slot n0,k+nD cannot be provided in slot nU R=R\r; elseif the UE is provided timeDomainHARQ-BundlingType1 and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot n0,k+nD− ∆K0,r(d), at least one symbol of each PDSCH time resource derived by row r of set R' is configured as UL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, and for each slot from n0,k+nD−NPDSCHrepeat,max+1 to slot n0,k+nD at least one symbol of the PDSCH time resource derived by row r of set R is configured as UL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if the row r of set R belongs to time domain resource allocation table configured for DCI format 1_2, where d = 0,1,…,C∆K0,r−1. R=R\r; R'=R'\r; else r=r+1; end if end while if the UE is not provided with multiPDSCH-perSlotType1-CB and the UE does not indicate a capability to receive more than one unicast PDSCH or multicast PDSCH per slot and R≠∅, or if the UE is provided with multiPDSCH-perSlotType1-CB = 'disabled' and R≠∅, MA,c=MA,c∪j; j=j+1; else Set CR to the cardinality of R Set m to the smallest last OFDM symbol index, as determined by the SLIV, among all rows of R while R≠∅ Set r=0 while r<CR if S≤m for start OFDM symbol index S for row r br,k,nD=j; - index of occasion for candidate PDSCH reception, or SPS PDSCH release, or TCI state update associated with row r R=R\r; B=B∪br,k,nD; else r=r+1; end if end while MA,c=MA,c∪j; j=j+1; Set m to the smallest last OFDM symbol index among all rows of R; end while end if nD=nD+1; end if end while end if k=k+1; end while end if <Unchanged parts are omitted> - 9.1.3.1 Type-2 HARQ-ACK codebook in physical uplink control channel <Unchanged parts are omitted> Denote by NC−DAIDL the number of bits for the counter DAI and set TD=2NC−DAIDL. Denote by VC−DAI,c,mDL the value of the counter DAI in a DCI format scheduling PDSCH reception, or having associated HARQ-ACK information without scheduling PDSCH reception, on serving cell c in PDCCH monitoring occasion m according to Table 9.1.3-1 or Table 9.1.3-1A. Denote by VT−DAI,mDL the value of the total DAI in a DCI format in PDCCH monitoring occasion m according to Table 9.1.3-1. The UE assumes a same value of total DAI in all DCI formats that include a total DAI field in PDCCH monitoring occasion m. A UE does not expect to multiplex, in a same Type-2 HARQ-ACK codebook, HARQ-ACK information that is in response to detection of DCI formats with different number of bits for the counter DAI field. If the UE transmits HARQ-ACK information in a PUCCH in slot n and for any PUCCH format, the UE determines the o0ACK, o1ACK,⋯,oOACK−1ACK, for a total number of OACK HARQ-ACK information bits, according to the following pseudo-code: Set m=0 – PDCCH, with DCI format scheduling PDSCH reception, or having associated HARQ-ACK information without scheduling a PDSCH reception, monitoring occasion index: lower index corresponds to earlier PDCCH monitoring occasion Set j=0 Set Vtemp=0 Set Vtemp2=0 Set Vs=∅ Set NcellsDL to the number of serving cells configured by higher layers for the UE - if, for an active DL BWP of a serving cell, the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, the serving cell is counted two times where the first time corresponds to the first CORESETs and the second time corresponds to the second CORESETs - if the UE indicates type2-HARQ-ACK-Codebook and receives a number NPDSCH, cm>1 of PDSCHs on a serving cell c that are scheduled by DCI formats in PDCCH receptions at a same PDCCH monitoring occasion m, the serving cell c is counted NPDSCH, cm times for PDCCH monitoring occasion m in increasing order of the PDSCH reception starting time Set M to the number of PDCCH monitoring occasion(s) while m<M Set c=0 – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cell while c<NcellsDL if PDCCH monitoring occasion m is before an active DL BWP change on serving cell c or an active UL BWP change on the serving cell of PUCCH transmission if the UE is provided pucch-sSCellDyn or pucch-sSCellDynDCI-1-2 or pucch-sSCellDynDCI-1-3, or an active UL BWP change on the PCell if the UE is not provided pucch-sSCellDyn and pucch-sSCellDynDCI-1-2 and pucch-sSCellDynDCI-1-3, and the active DL BWP change is not triggered in PDCCH monitoring occasion m, and the PUCCH transmission starts at or after a slot for the active DL BWP change or the active UL BWP change c=c+1; else if there is a PDSCH providing a transport block for a HARQ process with enabled HARQ-ACK information on serving cell c associated with PDCCH in PDCCH monitoring occasion m, or there is a PDCCH providing a DCI format associated with HARQ-ACK information without scheduling PDSCH reception on serving cell c if VC−DAI,c,mDL≤Vtemp j=j+1 end if Vtemp=VC−DAI,c,mDL if VT-DAI,mDL=∅ Vtemp2=VC−DAI,c,mDL else Vtemp2=VT−DAI,mDL end if if harq-ACK-SpatialBundlingPUCCH is not provided and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for at least one configured DL BWP of at least one serving cell, o2⋅TD⋅j+2VC−DAI,c,mDL−1ACK = HARQ-ACK information bit corresponding to the first transport block of this cell o2⋅TD⋅j+2VC−DAI,c,mDL−1+1ACK = HARQ-ACK information bit corresponding to the second transport block of this cell Vs=Vs∪2⋅TD⋅j+2VC−DAI,c,mDL−1, 2⋅TD⋅j+2VC−DAI,c,mDL−1+1 elseif harq-ACK-SpatialBundlingPUCCH is provided to the UE and m is a monitoring occasion for PDCCH with a DCI format that supports PDSCH reception with two transport blocks and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks in at least one configured DL BWP of at least one serving cell, oTD⋅j+VC−DAI,c,mDL−1ACK = binary AND operation of the HARQ-ACK information bits corresponding to the first and second transport blocks of this cell Vs=Vs∪TD⋅j+VC−DAI,c,mDL−1 else oTD⋅j+VC−DAI,c,mDL−1ACK = HARQ-ACK information bit of this cell Vs=Vs∪TD⋅j+VC−DAI,c,mDL−1 end if end if c=c+1 end if end while m=m+1 end while Vtemp=j mod4TD×4TD+Vtemp if UE does not set Vtemp2=VT−DAIUL and TD=2 Vtemp2=Vtemp end if j=j×TD4 if Vtemp2<Vtemp j=j+1 end if if harq-ACK-SpatialBundlingPUCCH is not provided to the UE and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for at least one configured DL BWP of a serving cell, OACK=2⋅4⋅j+Vtemp2 else OACK=4⋅j+Vtemp2 end if oiACK=NACK for any i∈0,1,⋯,OACK−1\Vs <Unchanged parts are omitted> If a UE is provided by MC-DCI-SetofCellsToAddModList a number of sets of serving cells and is provided USS sets to monitor PDCCH for detection of DCI format 1_3, the UE separately applies the following procedures for determining a corresponding second Type-2 HARQ-ACK sub-codebook for scheduling cells associated with DCI format 1_3 that - schedules PDSCH receptions on more than one serving cells from a set of serving cells, and/or - does not include a SCell dormancy indication field or the SCell dormancy indication field is reserved, indicates SCell dormancy, and schedules PDSCH reception on one or more serving cells from the set of serving cells - in the following, and for the purpose of providing HARQ-ACK information corresponding to SCell dormancy indication, the UE assumes that the UE receives a PDSCH on the serving cell associated with fields in DCI format 1_3 used for SCell dormancy indication, as described in Clause 10.3, and that the PDSCH provides one transport block that the UE correctly decodes from the procedures for determining a first Type-2 HARQ-ACK sub-codebook that is associated with unicast SPS PDSCH receptions or with any unicast DCI format scheduling a PDSCH reception on a single serving cell, or has associated HARQ-ACK information without scheduling a PDSCH reception as described in this clause. The UE appends the second Type-2 HARQ-ACK sub-codebook to the first Type-2 HARQ-ACK sub-codebook. Denote by NC−DAIDL the number of bits for the counter DAI field in DCI format 1_3 and set TD=2NC−DAIDL. Denote by VC−DAI,c,mDL the value of the counter DAI in a DCI format 1_3 scheduling PDSCH receptions on more than one serving cells among the more than one serving cells, in PDCCH monitoring occasion m according to Table 9.1.3-1. Denote by VT−DAI,mDL the value of the total DAI in DCI format 1_3 scheduling PDSCH receptions on more than one cells in PDCCH monitoring occasion m according to Table 9.1.3-1. The UE assumes a same value of total DAI in all DCI formats 1_3 in PDCCH monitoring occasion m that schedule more than one PDSCH receptions on respective more than one serving cells from a set of serving cells. The UE determines the o0ACK, o1ACK,⋯,oOACK−1ACK, for a total number of OACK HARQ-ACK information bits in the second Type-2 HARQ-ACK sub-codebook according to the following pseudo-code. Set Ncells,setDL,max to the maximum number of serving cells in ScheduledCell-ListDCI-1-3 of a set of serving cells provided by MC-DCI-SetofCells, across the number of sets of serving cells, that can be scheduled PDSCH receptions by DCI format 1_3 Set NsetsTB,max to the maximum total number of TBs in PDSCH receptions that can be scheduled by a DCI format 1_3 over more than one serving cells in a set of serving cells across the number of sets of serving cells Set NsetsDL to the number of sets of serving cells MC-DCI-SetofCells in a PUCCH group Set NcellsDL to the number of serving cells, across NsetsDL sets of serving cells in the PUCCH group Set c to the index of serving cells, c=0,…, NcellsDL−1, a lower index corresponds to a lower RRC index of a corresponding serving cell - if the UE indicates type2-HARQ-ACK-Codebook and receives NPDSCH, cm>1 PDSCHs on a serving cell c that are scheduled by NPDSCH, cm DCI formats 1_3 in PDCCH receptions at a same PDCCH monitoring occasion m, where - each of the DCI formats 1_3 schedules more than one PDSCH receptions on respective more than one serving cells, - c is the smallest cell index among the respective more than one serving cells, and - c is same across the NPDSCH, cm DCI formats 1_3 the serving cell c is counted NPDSCH, cm times for PDCCH monitoring occasion m in increasing order of the PDSCH reception starting time among the NPDSCH, cm PDSCH receptions Set mc to the index of a serving cell, in a set of indexes of serving cells arranged in ascending order, from the set of Ncells,setDL,max serving cells, mc=0,…, Ncells,setDL,max−1 Set m=0 – PDCCH monitoring occasion index for detection of a DCI format 1_3 scheduling PDSCH receptions on more than one serving cells from a set of serving cells: lower index corresponds to earlier PDCCH monitoring occasion Set j=0 Set Vtemp=0 Set Vtemp2=0 Set Vs=∅ Set M to the number of PDCCH monitoring occasions while m<M c=0 if harq-ACK-SpatialBundlingPUCCH is not provided, while c<NcellsDL if PDCCH monitoring occasion m is before an active UL BWP change on the serving cell of PUCCH transmission if the UE is provided pucch-sSCellDyn or pucch-sSCellDynDCI-1-2 or pucch-sSCellDynDCI-1-3, or an active UL BWP change on the PCell if the UE is not provided pucch-sSCellDyn and pucch-sSCellDynDCI-1-2 and pucch-sSCellDynDCI-1-3, and the PUCCH transmission with the HARQ-ACK information starts at or after a slot for the active UL BWP change c=c+1; else if there is a PDSCH reception on serving cell c that is scheduled by a DCI format scheduling more than one PDSCHs that provide respective more than one transport blocks with enabled HARQ-ACK information on respective more than one serving cells, where the DCI format is associated with a PDCCH reception in PDCCH monitoring occasion m and c is the smallest serving cell index among the more than one serving cells if VC−DAI,c,mDL≤Vtemp j=j+1; end if Vtemp=VC−DAI,c,mDL; if VT-DAI,mDL=∅ Vtemp,2=VC−DAI,c,mDL; else Vtemp,2=VT−DAI,mDL; end if cnt=0; mc=0; while mc<Ncells,setDL,max if the UE is scheduled PDSCH reception on serving cell mc, if any, from the more than one serving cells if PDCCH monitoring occasion m is before an active DL BWP change on serving cell mc, and the active DL BWP change is not triggered in PDCCH monitoring occasion m, and the PUCCH with the HARQ-ACK information starts at or after a slot for the active DL BWP change, if maxNrofCodeWordsScheduledByDCI is 2 for serving cell mc, oNsetsTB,max⋅TD⋅j+NsetsTB,max⋅VC−DAI,c,mDL−1+cntACK = NACK; oNsetsTB,max⋅TD⋅j+NsetsTB,max⋅VC−DAI,c,mDL−1+1+cntACK = NACK; cnt=cnt+2; else oNsetsTB,max⋅TD⋅j+NsetsTB,max⋅VC−DAI,c,mDL−1+cntACK = NACK; cnt=cnt+1; end if else if maxNrofCodeWordsScheduledByDCI is 2 for serving cell mc oNsetsTB,max⋅TD⋅j+NsetsTB,max⋅VC−DAI,c,mDL−1+cntACK = HARQ-ACK information bit corresponding to the first transport block of this cell oNsetsTB,max⋅TD⋅j+NsetsTB,max⋅VC−DAI,c,mDL−1+1+cntACK = HARQ-ACK information bit corresponding to the second transport block of this cell cnt=cnt+2; else oNsetsTB,max⋅TD⋅j+NsetsTB,max⋅VC−DAI,c,mDL−1+cntACK = HARQ-ACK information bit corresponding to the transport block of this cell cnt=cnt+1; end if end if end if mc=mc+1; end while while cnt< NsetsTB,max oNsetsTB,max⋅TD⋅j+NsetsTB,max⋅VC−DAI,c,mDL−1+cntACK= NACK; cnt=cnt+1; end while Vs=Vs∪NsetsTB,max⋅TD⋅j+NsetsTB,max⋅VC-DAI,c,mDL−1, …, NsetsTB,max⋅TD⋅j+NsetsTB,max⋅VC-DAI,c,mDL−1+NsetsTB,max−1; end if c=c+1; end if end while else while c<NcellsDL if PDCCH monitoring occasion m is before an active UL BWP change on the serving cell of PUCCH transmission if the UE is provided pucch-sSCellDyn or pucch-sSCellDynDCI-1-2 or pucch-sSCellDynDCI-1-3, or an active UL BWP change on the PCell if the UE is not provided pucch-sSCellDyn and pucch-sSCellDynDCI-1-2 and pucch-sSCellDynDCI-1-3, and the PUCCH transmission with the HARQ-ACK information starts at or after a slot for the active UL BWP change c=c+1; else if there is a PDSCH reception on serving cell c that is scheduled by a DCI format scheduling more than one PDSCHs that provide respective more than one transport blocks with enabled HARQ-ACK information on respective more than one serving cells, where the DCI format is associated with a PDCCH reception in PDCCH monitoring occasion m and c is the smallest serving cell index among the more than one serving cells if VC−DAI,c,mDL≤Vtemp j=j+1; end if Vtemp=VC−DAI,c,mDL; if VT-DAI,mDL=∅ Vtemp,2=VC−DAI,c,mDL; else Vtemp,2=VT−DAI,mDL; end if cnt=0; mc=0; while mc<Ncells,setDL,max if the UE is scheduled PDSCH reception for transport blocks with enabled HARQ-ACK information on serving cell mc, if any, from the more than one serving cells if PDCCH monitoring occasion m is before an active DL BWP change on serving cell mc, and the active DL BWP change is not triggered in PDCCH monitoring occasion m, and the PUCCH with the HARQ-ACK information starts at or after a slot for the active DL BWP change, oNcells,setDL,max⋅TD⋅j+Ncells,setDL,max⋅(VC-DAI,c,mDL−1)+cntACK = NACK; else if maxNrofCodeWordsScheduledByDCI is 2 for serving cell mc if the PDSCH reception provides two transport blocks oNcells,setDL,max⋅TD⋅j+Ncells,setDL,max⋅VC-DAI,c,mDL−1+cntACK = binary AND operation of the HARQ-ACK information bits corresponding to the first and second transport blocks of this cell else oNcells,setDL,max⋅TD⋅j+Ncells,setDL,max⋅VC-DAI,c,mDL−1+cntACK = HARQ-ACK information bit corresponding to the first transport block of this cell end if else oNcells,setDL,max⋅TD⋅j+Ncells,setDL,max⋅VC-DAI,c,mDL−1+cntACK= HARQ-ACK information bit of this cell end if cnt=cnt+1; end if end if mc=mc+1; end while while cnt< Ncells,setDL,max oNcells,setDL,max⋅TD⋅j+Ncells,setDL,max⋅VC-DAI,c,mDL−1+cntACK= NACK; cnt=cnt+1; end while Vs=Vs∪Ncells,setDL,max⋅TD⋅j+Ncells,setDL,max⋅VC-DAI,c,mDL−1, Ncells,setDL,max⋅TD⋅j+Ncells,setDL,max⋅VC-DAI,c,mDL−1+1…, Ncells,setDL,max⋅TD⋅j+Ncells,setDL,max⋅VC-DAI,c,mDL−1+Ncells,setDL,max−1; end if c=c+1; end if end while end if m=m+1; end while Vtemp=j mod4TD×4TD+Vtemp; if UE does not set Vtemp2=VT−DAIUL and TD=2 Vtemp2=Vtemp; end if j=j×TD4; if Vtemp2<Vtemp j=j+1; end if if harq-ACK-SpatialBundlingPUCCH is not provided, OACK=NsetsTB,max⋅4⋅j+Vtemp2 else OACK=Ncells,setDL,max⋅4⋅j+Vtemp2 end if oiACK=NACK for any i∈0,1,⋯,OACK−1\Vs . If OACK+OSR+OCSI≤11 and NsetsDL>0, for obtaining a PUCCH transmission power as described in clause 7.2.1, the UE determines nHARQ-ACK=nHARQ-ACK,0+nHARQ-ACK,1, where nHARQ-ACK,0 is the value of nHARQ-ACK for the first Type-2 HARQ-ACK sub-codebook and nHARQ-ACK,1 is the value of nHARQ-ACK for the second Type-2 HARQ-ACK sub-codebook that is determined as <Unchanged parts are omitted>

## Question 1:

Do you agree with the draft CR? If not, please provide your comments/reasons.

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Support the draftCR. |
| NTT DOCOMO | We support the intention of the draft CR. Although I’m not sure whether “the UE is not provided A and B and C” is correct English rather than “the UE is not provided A or B or C”, it should be consistent across specs anyway. |
| Qualcomm | OK with the proposal. |
| Spreadtrum | Support. It can be in alignment CR. |
| CATT | OK |
| LGE | OK. It can be in alignment CR. |

Conclusion

Proposal:

The draft CR R1-241019 is agreed in principle for alignment CR.

References

R1-2410190	Draft CR on Type-1 and Type-2 HARQ-ACK codebook	ZTE Corporation, Sanechips