3GPP TSG-RAN WG1#119	R1-24abcde

Orlando, FL, USA, Nov 18th-22nd 2024

Agenda Item:	9.5.3

Source:	Moderator (Ericsson)

Title:	Summary of AI 9.5.3 for R19 NES

Document for:	Discussion

# Introduction

This is the summary for AI 9.5.3 on the adaptation of common signalling for NES based on the views expressed by companies in the contributions listed in the Appendix A and providing topics and proposals for discussion/agreement.

# Discussion on RAN4 LS (R1-2409350)

RAN4 sent an LS (R1-2409350) related to on-demand SSB SCell operation and SSB adaptation. The contents of the LS are as follows. The last three questions are related to SSB adaptation for which proposed answers are to be discussed in this agenda item.

1. Overall Description:

In RAN4#112bis meeting, RAN4 discussed the impacts on RRM requirements for on-demand SSB in Obj. 1 and SSB adaptation in Obj. 3. During the discussion, RAN4 identified the following questions.

[RAN4 Questions]:

Obj. 1 related to on-demand SSB SCell operation_

Based on the discussion, RAN4 identified that the relation between always-on SSB and on-demand SSB (i.e. Case#2 as agreed in RAN1) has impacts on UE’s behavior and requirements in SCell. RAN4 would like RAN1 to clarify at least the following aspects:

- Q1: What is the relation in terms of periodicity between always-on SSB and OD-SSB?

- Q2: What is the relation in terms of time location between always-on SSB and OD-SSB?

- Q3: What is the relation in terms of frequency location between the always-on SSB and OD-SSB?

- Q4: What is the spatial relation between the always-on SSB and OD-SSB?

Obj.3 related to SSB adaptation in time domain

For the SSB adaption in the objective of adaptation of common signal/channel transmits, RAN4 would also like RAN1 to clarify the relation for SSB adaptation at least for following aspects.

- Q1: What is the relation in terms of time location before and after SSB adaptation?

- Q2: What is the relation in terms of frequency location before and after SSB adaptation?

- Q3: What is the spatial relation before and after SSB adaptation?

2. Actions:

To RAN1:

RAN4 respectfully asks RAN1 to answer the questions above.

1. Overall Description:

In RAN4#112bis meeting, RAN4 discussed the impacts on RRM requirements for on-demand SSB in Obj. 1 and SSB adaptation in Obj. 3. During the discussion, RAN4 identified the following questions.

[RAN4 Questions]:

Obj. 1 related to on-demand SSB SCell operation_

Based on the discussion, RAN4 identified that the relation between always-on SSB and on-demand SSB (i.e. Case#2 as agreed in RAN1) has impacts on UE’s behavior and requirements in SCell. RAN4 would like RAN1 to clarify at least the following aspects:

- Q1: What is the relation in terms of periodicity between always-on SSB and OD-SSB?

- Q2: What is the relation in terms of time location between always-on SSB and OD-SSB?

- Q3: What is the relation in terms of frequency location between the always-on SSB and OD-SSB?

- Q4: What is the spatial relation between the always-on SSB and OD-SSB?

Obj.3 related to SSB adaptation in time domain

For the SSB adaption in the objective of adaptation of common signal/channel transmits, RAN4 would also like RAN1 to clarify the relation for SSB adaptation at least for following aspects.

- Q1: What is the relation in terms of time location before and after SSB adaptation?

- Q2: What is the relation in terms of frequency location before and after SSB adaptation?

- Q3: What is the spatial relation before and after SSB adaptation?

2. Actions:

To RAN1:

RAN4 respectfully asks RAN1 to answer the questions above.

Q1: What is the relation in terms of time location before and after SSB adaptation?

Inputs from company contributions for answers to Q1 are captured below.

| Ericsson | Reply to RAN4’s Question 1 on Objective 3 as follows: - At least the case where the time location is same before and after SSB adaptation is supported (i.e., SSB burst periodicity is adapted). - Any additional cases are still under RAN1 discussion. |
| --- | --- |
| CMCC | Proposal 4: The SSB time/frequency location before and after SSB adaptation can be the same or different. |
| ZTE | The time location such as SSB positions in one SSB burst can be same or different before and after SSB adaptation. |
| Samsung | A: There is no explicit indication of the time domain location (e.g., SFN offset and half frame index) for the SSB burst before or after the SSB adaptation, and the UE can determine the time domain location for the SSB burst after adaptation based on the time domain location for the SSB burst before adaptation. |
| Vivo | [RAN1 Answer]: RAN1 agrees that SSB period can only be changed before and after SSB adaptation. Other than period, the time location is not changed before and after SSB adaptation. |
| CATT | - The adapation is achieved via switching between two SSB peridicities, or activating two SSB peridicities at the same time. - The adapation is only about peridicity change, i.e, there is no freuqency change or SSB beam configuration change after the adapation. |
| Nokia | RAN1 response: After SSB adaptation, the periodicity of existing SSB burst transmissions is changed but half-frame index and SFN offset remain the same as before adaptation. |
| Oppo | A1: If “time location” refers to the offset of SSB burst in one SSB periodicity, the offset of SSB bursts can be changed before and after SSB adaptation; If “time location” refers to transmitted SSB indexes, the SSB indexes should be maintained before and after SSB adaptation. |
| Google | A1: The time location may be the same or different before and after SSB adaptation. |
| LG | - Answer to Q1: SSB time domain occasions before adaptation are the subset of SSB time domain occasions after adaptation. |
| Fujitsu | • Except for the SSB periodicity of 5 ms, the location within a frame where an SSB burst is transmitted is the same before and after SSB adaptation • The SFN offset within a period before and after SSB adaptation can be the same or different |
| Huawei/HiSi | RAN1 is discussing details of adaptation of SSB periodicity where the SSB burst periodicity before and after the reception of SSB adaptation signalling may be same or different, depending on the adaptation level among SSB burst, actually transmitted SSBs within a burst or SSB patterns. |
| Futurewei | Q1: The group agreed that “Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) is supported”. Based on the above agreement the burst periodicity of the SSB transmission will change before and after SSB adaptation. |
| Apple | Proposal 5: Time location of an SS/PBCH block within a half frame is not changed before and after adaptation. |
| Interdigital | Proposal 11: In the LS response to RAN4, include the following on the relation of time location before and after SSB adaptation: - The SSB periodicity and the corresponding time locations of the SSB bursts (e.g. in terms of SFN index, half-frame index) changes from before to after SSB adaptation. - For example, before SSB adaptation the SSB bursts may be transmitted in some sparse SFN indexes with a periodicity of 160ms and after SSB adaptation the SSB bursts may be transmitted in denser SFN indexes with periodicity of 20ms. |
| Intel | For Q1, there is no relation in terms of time location before and after SSB adaptation |

- Q1: What is the relation in terms of time location before and after SSB adaptation?

## Proposal 2.1.1

- [Reply to Q1] RAN1 agreed that SSB burst periodicity is adapted. RAN1 has not agreed to change the time location of the SSB burst other than the periodicity.

- Companies are invited to provide their input on the above proposed reply.

| Company | Support (Y/N) | Comment |
| --- | --- | --- |
| CMCC |  | We support SSB burst periodicity can be adapted. Regarding the issue on time location, we think this should be further discussed. |
| CATT |  | We are fine with the proposal. |
| Fujitsu1 |  | We share the same view as CMCC that the change of time location can be further discussed. |
| Tejas | Y | Support |
| Huawei, HiSilicon | N | In RAN1#116 meeting, an agreement on what SSB adaptation mechanisms to study was achieved. Agreement For adaptation of SSB in time-domain, consider the following adaptation mechanisms for further study - Adaptation of SSB burst periodicity - Adaptation based on two SSB configurations where up to two configurations can be active - Adaptation based on skipping/transmitting some SSB bursts non-uniformly with single SSB configuration - Adapting the transmitted number of SSBs within a SSB burst - Cell DTX for SSB adaptation - Whether to support new SSB burst periodicity value(s) - Whether to support new SSB burst(s) (i.e. how SSB transmission is made within a burst) - New compact SSB burst(s) - Adapting the position of SSBs within a SSB burst - Other mechanisms/combinations are not precluded. Among these options, two of them have reached agreement. 1. Adaptation of SSB burst periodicity. Agreed in RAN1#117. 2. Cell DTX for SSB adaptation. Deprioritized in RAN1#118bis. For other options, there is no conclusion yet, and RAN1 can carry on examining them until a consensus is made. For example, new SSB periodicity, e.g. smaller than 5ms for facilitating SCell activation or greater than 160ms for more NW energy savings, is one of the discussion points. Note that, several options, e.g., adapting the transmitted number of SSBs within a SSB burst, make a valid use case for not changing the periodicity before and after SSB periodicity adaptation. Consequently, we suggest the following modifications in blue: RAN1 agreed that SSB burst periodicity is can be adapted. RAN1 has not agreed is still discussing other options to change the time location of the SSB burst other than the periodicity. For example: whether to support new SSB burst periodicity value(s), whether to support new SSB burst(s), Adapting the transmitted number of SSBs within a SSB burst. etc. |
| Panasonic | Y | Support |
| Moderator |  | Proposal is updated in 2.1.1-rev1. |
| Nokia, Nokia Shanghai Bell |  | We are fine to agree the revised proposal. However, we think that adapting other time domain properties than periodicity should not be specified. |

## Proposal 2.1.1-rev1

- Reply to Q1(What is the relation in terms of time location before and after SSB adaptation?):

- RAN1 agreed that at least SSB burst periodicity is adapted.

- There are no RAN1 agreements to adapt the time location of the SSB burst other than the periodicity but RAN1 is still discussing other options.

| Moderator | Closed. |
| --- | --- |

Q2: What is the relation in terms of frequency location before and after SSB adaptation?

Inputs from company contributions on reply to Q2 are captured below.

| Ericsson | Reply to RAN4’s Question 2 on Objective 3 as follows: - At least the case where the frequency location is same before and after SSB adaptation is supported (i.e., SSB burst periodicity is adapted). - Any additional cases are still under RAN1 discussion. |
| --- | --- |
| CMCC | Proposal 4: The SSB time/frequency location before and after SSB adaptation can be the same or different. |
| ZTE | The frequency locations are the same before and after SSB adaptation |
| Samsung | A: The frequency location for SSB before and after SSB adaptation maintains the same. |
| Vivo | [RAN1 Answer]: RAN1 assumes frequency location before and after SSB adaptation is not changed. |
| CATT | - The adapation is achieved via switching between two SSB peridicities, or activating two SSB peridicities at the same time. - The adapation is only about peridicity change, i.e, there is no freuqency change or SSB beam configuration change after the adapation. |
| Nokia | RAN1 response: SSB adaptation is only applied in time domain. Frequency location of SSB transmission does not change due to SSB adaptation |
| Oppo | A2: SSB frequency location should be maintained before and after SSB adaptation. |
| Google | A2: The frequency location is not changed before and after SSB adaptation. |
| LG | - Answer to Q2: The frequency location of SSB before adaptation can be same or different from the frequency location of SSB after adaptation, based on higher layer configuration. |
| Fujitsu | • The frequency location before and after SSB adaptation is the same. |
| Huawei/HiSi | The frequency location before and after SSB adaptation can be the same or different, according to the SSB configuration. |
| Futurewei | Q2: The WID scope is “Adaptation of SSB in time domain, e.g. adapting periodicity”. Thus, the change in terms of frequency location of SSB before and after SSB adaptation is not in the scope of the WID. This implies, in our view, that the frequency location of SSB before and after SSB adaptation does not change. |
| Apple | Proposal 3: Frequency location before and after SSB adaptation is not changed. |
| Interdigital | Proposal 12: In the LS response to RAN4, include the following on the relation of frequency location before and after SSB adaptation: - The frequency location of SSBs is expected to be the same before and after SSB adaptation in time domain |
| Intel | For Q2, same frequency resource is used for the SSB transmision before and after SSB adaptation |

- Q2: What is the relation in terms of frequency location before and after SSB adaptation?

## Proposal 2.1.2

- [Reply to Q2] At least the case where the frequency location is same before and after SSB adaptation is supported.

- Companies are invited to provide their input on the above proposed reply.

| Company | Support (Y/N) | Comment |
| --- | --- | --- |
| CMCC | Y | Okay |
| CATT |  | OK |
| Fujitsu1 | Y |  |
| Tejas | Y | Support |
| Huawei, HiSilicon | Y |  |
| Moderator |  | Proposal is updated in 2.1.2-rev1. |
| Nokia, Nokia Shanghai Bell | Y | We support the updated proposal. |

## Proposal 2.1.2-rev1

- Reply to Q2(What is the relation in terms of frequency location before and after SSB adaptation?):

- The frequency location is same before and after SSB adaptation.

| Moderator | Closed. |
| --- | --- |

Q3: What is the spatial relation before and after SSB adaptation?

Inputs from company contributions on reply to Q3 are captured below.

| Ericsson | Reply to RAN4’s Question 3 on Objective 3 as follows: - At least the case where there is no change to spatial relation before and after SSB adaptation is supported (i.e., SSB burst periodicity is adapted). - Any additional cases are still under RAN1 discussion. |
| --- | --- |
| CMCC | Proposal 5: The spatial relation before and after SSB adaptation can be the same. |
| ZTE | The SSBs with same index before and after SSB adaptation are QCLed. |
| Samsung | A: The spatial information (i.e., actually transmitted SSB in burst) for SSB before and after SSB adaptation maintains the same. |
| Vivo | [RAN1 Answer]: RAN1 assumes spatial location before and after SSB adaptation is not changed. |
| CATT | - The adapation is achieved via switching between two SSB peridicities, or activating two SSB peridicities at the same time. - The adapation is only about peridicity change, i.e, there is no freuqency change or SSB beam configuration change after the adapation. |
| Nokia | RAN1 response: Spatial properties of SSB transmission do not change when SSB is adapted in time domain. |
| Oppo | A3: SSBs corresponding to a same SSB index in a serving cell that across SSB bursts before and after SSB adaptation should be quasi co-located. |
| Google | A3: The spatial relation is not changed before and after SSB adaptation. |
| LG | - Answer to Q3: UE assumes that SSBs are quasi co-located if the corresponding SSB indices within SSB burst before adaptation and within SSB burst after adaptation are the same. |
| Fujitsu | • The spatial relation before and after SSB adaptation is the same. |
| Huawei/HiSi | For more analysis in spatial domain, please refer to Reply to Q4 in Obj. 1. There is no constraint on the spatial relation between always-on SSB and on-demand SSB. The on-demand SSB and always-on SSB can have the same beams or different. RAN1 is discussing and studying the details of how and when (under which scenario) the UE should be informed about this spatial relationship. |
| Futurewei | Q3: As was noted above, only the time adaptation is in the scope of the WID. Therefore, in our view, there shall be no spatial changes before and after SSB adaptation. |
| Apple | Proposal 4: SSBs with the same index before and after SSB adaptation have the same QCL assumption. |
| Interdigital | Proposal 13: In the LS response to RAN4, include the following on spatial relation before and after SSB adaptation: - Spatial relation is not impacted during SSB adaptation in time domain - SSB beams per burst are expected to be the same before and after SSB adaptation |
| Intel | For Q3, same spatial relation is applied when same SSB index is used before and after SSB adaptation |

- Q3: What is the spatial relation before and after SSB adaptation?

## Proposal 2.1.3

- [Reply to Q3] At least the case where there is no change to spatial relation before and after SSB adaptation is supported.

- Companies are invited to provide their input on the above proposed reply.

| Company | Support (Y/N) | Comment |
| --- | --- | --- |
| CMCC | Y | Okay |
| CATT | Y | OK |
| Fujitsu1 | Y |  |
| Tejas | Y | Support |
| Huawei, HiSilicon | Y |  |
| Moderator |  | Proposal is updated in 2.1.3-rev1. |
| Nokia, Nokia Shanghai Bell | Y | We support the updated proposal |

## Proposal 2.1.3-rev1

- Reply to Q3(What is the spatial relation before and after SSB adaptation?):

- There is no change to spatial relation for the same SSB index before and after SSB adaptation.

- At least the case where there is no change to actually transmitted SSBs within a burst before and after SSB adaptation is supported.

| Moderator | Closed. |
| --- | --- |

# Adaptation of SSB in time domain

Topic 2.1.1

For the adaptation mechanisms of SSB in time-domain, several companies discussed which of the scenarios to support:

|  |  | Yes | No |
| --- | --- | --- | --- |
| Rel-19 NES-capable UE’s PCell (Connected mode) | A2 adaptation for SSB that is not CD-SSB | Honor, ZTE, , Tejas, ETRI, Interdigital, Samsung, Huawei/HiSi, Sony, Nokia, Fujitsu (only when CD-SSB on the cell has > 20ms periodicity), Intel, CT | LG, Apple, vivo, CMCC (if cell has CD-SSB with normal periodicity), [Qualcomm (marginal NES gains)?], CATT, Sharp |
| Rel-19 NES-capable UE’s PCell (Connected mode) | A3 adaptation for SSB not on sync raster | Ericsson, Honor, ZTE, ETRI, Interdigital, Samsung, Mediatek, Huawei/HiSi, Sony, Nokia, Fujitsu (only when CD-SSB on the cell has > 20ms periodicity), Intel, Mediatek SPRD (not on sync raster), | LG, Apple, vivo, CMCC (if cell has CD-SSB with normal periodicity), [Qualcomm (marginal NES gains)?], [CT?] CATT, Sharp |
| Rel-19 NES-capable UE’s SCell | B1 adaptation for CD-SSB | Honor, ZTE, DoCoMo, ETRI, [Huawei/HiSi?], Cewit/CT/Sony/Intel (for CD-SSB>20ms periodicity), Nokia, Sharp, NEC (>20ms periodicity), CATT (for SCell that doesn’t support initial cell selection), | LG, Qualcomm, Tejas, vivo, Mediatek Apple/Interdigital(for CD-SSB>20ms periodicity), Transsion (on sync raster), FW, SPRD (SSB is not on sync raster) |

Above table summarizes the company views on the scenarios for SSB adaptation.

For Rel-19 NES-capable UE’s PCell (connected mode), some companies indicate that if the cell has CD-SSB with normal periodicity (<=20ms periodicity), then adapting not CD-SSB may provide only marginal gains (if any) and hence should not be supported. However, some companies think that when CD-SSB on a cell has larger than 20ms periodicity, adaptation of SSB not on sync raster should be supported.

## Proposal 3.1.1

Support adaptation of SSB in time domain for Rel-19 NES-capable UE’s PCell (connected mode) for the following case:

- Adaptation for SSB not on sync raster is supported (A3) which is not cell-defining

- Note: This is applicable for a cell that provides a CD-SSB with larger than 20ms periodicity on the sync raster for initial cell selection. The adaptable SSB is present along with the CD-SSB.

| Company | Comment |
| --- | --- |
| CMCC | We do not support the proposal. From our understanding, PCell shall have a periodically transmitted CD-SSB, If companies want to hold the periodicity for CD-SSB to 20ms, adapting SSB not on sync raster/NCD-SSB will not obtain NES gain. Thus, we don’t think this scenario is valid. |
| CATT | Don’t support. There is always a CD-SSB for PCell. We don’t support to introduce an additional SSB(A3) on top of CD-SSB of PCell, since gNB cann’t save energy by such way. |
| Fujitsu1 | We are fine with the proposal. |
| Huawei, HiSilicon | We are fine with the proposal. However, we think (A3) is more useful for PCell that has OD-SIB1 (NES cell in Obj2). Which can handle the concerns from CATT and CMCC and others by not even requiring to send SSBs on sync rasters in some of the use cases of NES cell. Hence, we propose the following changes in blue to have a package and useful agreement with valid and realistic/beneficial use cases: Support adaptation of SSB in time domain for Rel-19 NES-capable UE’s PCell (connected mode) and Pcell with OD-SIB1 (NES cell in Obj2 for idle/inactive UEs) for the following case: - Adaptation for SSB not on sync raster is supported (A3) - Note: This is applicable for a PCell (connected mode) cell that provides a CD-SSB with larger than 20ms periodicity on the sync raster for initial cell selection. The adaptable SSB is present along with the CD-SSB. - Note: For Pcell with OD-SIB1 (NES cell in Obj2 for idle/inactive UEs), This is applicable for a cell that provides a NCD-SSB with larger than 20ms periodicity on the sync raster for initial cell selection. Or for the NES cell that do not send NCD-SSB on sync raster. Further, for the case of PCell (connected mode) we’d like to extend the applicable case to RRC_INACTIVE UEs. One of the main differences between inactive UEs and idle UEs is that, there is context of RRC configuration stored in UE and gNB, so that UE would know how the adaptation pattern is configured and how SSB could be adapted on this cell, and the negative impact on inactive UE is much smaller than that on idle UE. |
| Panasonic | When the cell is operating fast adaptation on SSB in time-domain, SSB should be not located on synchronization raster or non-cell-defining SSB regardless of PCell or SCell. |
| ZTE, Sanechips | In our opinion, adaptation for NCD-SSB (A2) should also be supported in Rel-19 NES-capable UEs’ PCell. In the scenario that PCell is other UE’s SCell, NCD-SSB adaptation has already been agreed, it should also be supported in PCell, since gNB should transmit the SSB based on single SSB configuration in the same Cell. For the concerns on marginal gains, for one thing, the adaptable SSB can be TDM with CD-SSB in the PCell. For another, if it is applicable for a cell that provide a CD-SSB with larger than 20ms periodicity, the NES gain can be significant. |
| Nokia, Nokia Shanghai Bell | We support the first bullet and agree with ZTE that also adaptation of NCD-SSB on Pcell (A2) should be supported. We think that the note is not needed. Our understanding is that the scenario when additional SSB besides CD-SSB is transmitted in the cell is the case when additional SSB is the different BWP than CD-SSB. According to our simulations there is some NES gain also in the case when CD-SSB is transmitted with 20ms periodicity (FDD cell). |
| ETRI | As commented offline by a couple of companies, we also think that the benefit of the proposal is unclear. The note says, the CD-SSB periodicity is larger than 20 ms. That means this cell cannot support initial access of legacy UEs without impact to them. So the underlying assumption of this proposal is there is no legacy UE in the cell. Then two questions: - Why don’t we directly adapt the CD-SSB on sync raster (A1)? - What is the benefit/purpose of adapting another SSB (A2/A3) while fixing the CD-SSB periodicity? To our understanding, unfortunately, the only way that we can achieve a meaningful NES gain from SSB adaptation is to bar the legacy UEs from the NES cell. Given that, the simple and clean solution is to adapt the CD-SSB. If there is other SSBs (e.g., NCD-SSB) in that cell, those SSBs need be adapted together. From this perspective, to us a possible way forward is: Support SSB time domain adaptation for both CD-SSB (A1) and NCD-SSB (A2/A3) (i.e., any type of SSB) regardless of whether the applicable cell is PCell or SCell. We are not sure whether this proposal is acceptable or not at this stage since this requires reverting the previous RAN1 agreement we already made on the PCell case, however, we feel that it may worth consider it in the current situation. |
| LGE | Based on the offline session discussion, we can accept the scenario where: - Adaptable SSB is non cell-defining SSB and not on sync raster - Non-adaptable SSB (for initial cell selection) and adaptable SSB are in the different BWP. - In the NOTE, we don’t see the necessity of the limitation of periodicity for non-adaptable SSB (i.e., larger than 20 ms) In that sense, we suggest the following modification. Support adaptation of SSB in time domain for Rel-19 NES-capable UE’s PCell (connected mode) for the following case: - Adaptation for SSB not on sync raster is supported (A3), is non cell-defining SSB - Note: This is applicable for a cell that provides a CD-SSB with larger than 20ms periodicity on the sync raster for initial cell selection. The adaptable SSB is present along with the CD-SSB and located in the BWP different from BWP including CD-SSB. |
| Moderator | Closed. |

Regarding adaptation of CD-SSB for Rel-19 NES-capable UE’s SCell, views from companies seem largely unchanged from last RAN1 meeting. Thus, the comments from last meeting’s discussions (Discussion point 2.1.3, R1-2409279) seem still applicable. In below discussion, companies can provide any additional comments that could help clarify the motivation, use case, etc.

## Discussion point 3.1.2

Companies are invited to provide any additional comments (including e.g. clarifying motivation, use case, necessity, UE impact) on whether/how to support adaptation of SSB in time domain for Rel-19 NES-capable UE’s SCell for the following case:

- Adaptation for CD-SSB (B1)

| Company | Comment |
| --- | --- |
| Moderator | Below table shows the comments on a similar discussion point (2.1.3, see tdoc R1-2409279) from RAN1#118bis that companies can also consider/address in the discussion. |
| CMCC | A SCell that have a CD-SSB can be a PCell/camped cell for the other UE. If companies not support CD-SSB adaptation for PCell or IDLE/INACTIVE UE, this scenario may also invalid either. |
| CATT | From network perspective, it can be up to gNB implementation whether the adaptation for SSB on a SCell is cell-defining or not. |
| Fujitsu1 | If it is ensured that the cell with CD-SSB adaptation is a pure SCell, e.g., by setting the cellbarred IE in MIB and SIB1 to barred, CD-SSB adaptation can be supported. |
| Tejas | Same view as CMCC. |
| Huawei, HiSilicon | We are fine with this proposal. In general, we don’t think that B1 is a problematic case to support. If the majority contends that this shall be deprioritized, we can also be okay with it, for the sake of spec progress. |
| Nokia, Nokia Shanghai Bell | We support the proposal as mentioned in our reply in the previous meeting. Regarding the comment that the cell can be Pcell for some other UE we would like to point out that gNB is aware of that and can choose not to adapt CD-SSB in that case. |
| LGE | We do not support the proposal if this CD-SSB is transmitted on sync raster. |
| China Telecom | We support the proposal. We don’t agree with the companies’ concern that there could be impact if SCell with adaptive CD-SSB can be PCell for legacy UEs. Such condition can be avoided by implementation of gNB scheduling, just as that of SSB-less operation. |
| Apple | For a CD-SSB on sync raster, we do not think is can be adapted since it can be used as a PCell. For a CD-SSB off raster, we do not see the need for a CD-SSB off raster , rather a NCD-SSB would be enough. |
| NEC | We think that this case should be supported to ensure higher energy savings for SCell. Currently most of the SCells are generally deployed with CD-SSB and NCD-SSBs are rarely used practically. Hence, if we limit the scope of SSB adaptation to only NCD-SSBs then the feature may not be considered useful. Also, regarding the argument whether CD-SSB of SCell can be used as CD-SSB for another PCell, we think that we should let the network implementation handle this case for proper configuration. |

# Adaptation of PRACH

Additional frequency domain parameters

Below is the agreement regarding frequency domain parameters from RAN1#118bis.

Agreement (from RAN1#118bis)

For adaptation of PRACH in time-domain, the frequency domain resources for the additional PRACH resources and legacy PRACH resources can be same or different

- FFS: applicable case(s) (i.e. case(s) from the RAN1#117 agreement).

- Discuss further following options for signaling

- Option 1: at least the following parameter(s) can be configured separately for the additional PRACH resources.

  - msg1-FrequencyStart at least for 4-step RACH

  - FFS: other applicable legacy frequency domain parameter(s)

- Option 2: Offset to legacy frequency domain parameter(s) are configured for the additional PRACH resources

  - Note: Offset to legacy frequency domain parameter(s) is a new parameter

  - FFS: applicable legacy frequency domain parameter(s)

Agreement (from RAN1#118bis)

For adaptation of PRACH in time-domain, the frequency domain resources for the additional PRACH resources and legacy PRACH resources can be same or different

- FFS: applicable case(s) (i.e. case(s) from the RAN1#117 agreement).

- Discuss further following options for signaling

- Option 1: at least the following parameter(s) can be configured separately for the additional PRACH resources.

  - msg1-FrequencyStart at least for 4-step RACH

  - FFS: other applicable legacy frequency domain parameter(s)

- Option 2: Offset to legacy frequency domain parameter(s) are configured for the additional PRACH resources

  - Note: Offset to legacy frequency domain parameter(s) is a new parameter

  - FFS: applicable legacy frequency domain parameter(s)

Several companies indicated their preference on the option 1 vs option 2.

- Option 1:

  - Huawei/HiSi (also msg1-FDM), Futurewei, DoCoMo, Vivo, CT, Fujitsu, CATT, Tejas, Ericsson, Transsion, OPPO, HONOR, InterDigital, ETRI, Intel, ZTE (less spec impact)

- Option 2

  - Nokia (mitigates overlap issues between legacy and additional ROs), Xiaomi/Sharp/LG/Quectel (less signaling overhead)

## Proposal 4.1.1

At least msg1-FrequencyStart can be configured separately for the additional PRACH resources at least for 4-step RACH.

| Company | Support (Y/N) | Comment |
| --- | --- | --- |
| CMCC | Y | Option 1 is more straightforward. |
| CATT | Y |  |
| Fujitsu1 | Y |  |
| Tejas | Y | Support |
| HW | Y |  |
| Panasonic |  | We do not see clear merit of using offset to legacy frequency domain parameters over configuring frequency domain parameters separately for additional PRACH resources. |
| Nokia/NSB |  | Similar comment to Panasonic. It would be good if proponents provided clear examples of benefits and merit. As a matter of fact, we do not see a strict need to have any frequency domain parameters, but if we need to have any, then having a simple offset operating like msg1-FDM seems more than enough. |
| Moderator |  | Closed. |

DCI

Below working assumption was made in RAN1#118bis.

Working Assumption

For DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode.

- P-RNTI is used

Working Assumption

For DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode.

- P-RNTI is used

Several companies indicated their preference on the confirmation of the working assumption.

- Yes

  - Xiaomi (only DCI 1_0), Fujitsu, NEC, CATT, Interdigital, Honor, ETRI, vivo, Huawei/HiSi, ZTE, Lenovo, SPRD/Unisoc (at least idle/inactive UE), Oppo (update to allow the DCI in Type-3 CSS for connected mode UE)

- No

  - Intel (use new RNTI)

There are also a few companies that proposed that the DCI-based adaptation to supported for other DCIs such as DCI 2_7 with PEI-RNTI, DCI 1_0 with SI-RNTI, DCI 1_0 with C-RNTI (PDCCH order) and DCI 1_0 with new RNTI.

- Additionally in DCI 2_7 with PEI-RNTI : Qualcomm, Huawei/HiSi

- Additionally in DCI 1_0 with SI-RNTI: Ericsson

- In DCI 1_0 with new RNTI (instead of P-RNTI): Intel

- Additionally in DCI 1_0 with C-RNTI (for connected mode UE): Qualcomm, Huawei/HiSi, SPRD/Unisoc, Google (PDCCH order DCI contains a field indicating whether the triggered PRACH resource is from additional RACH resource or legacy PRACH resource), Lenovo

Confirming of the working assumption seems agreeable.

## Proposal 4.1.2

Confirm the following working assumption from RAN1#118bis.

Working Assumption

For DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode.

- P-RNTI is used

| Company | Support (Y/N) | Comment |
| --- | --- | --- |
| CMCC |  | Frankly speaking, we do not think using Paging DCI to indicate PRACH adaptation is feasible since: - In current spec, IDLE/INACTIVE UE may detect Paging DCI on its PO per I-DRX cycle. The periodicity is typically more than 1s; For CONNECTED UE, it only monitors Paging DCI per modification period, which means the period is about several seconds. - Compared with existing mechanism (e.g. SI update procedure), such mechanism cannot bring significant benefits on latency or overhead reduction perspective. Thus, proponents may need to clarify the benefits before we confirm this WA. |
| CATT | Y |  |
| Fujitsu1 | Y |  |
| Tejas | N | It is better to first agree on the type of PRACH resource adaptation method and comeback to this, which is given in the proposal proposal 4.1.6. The number of bits required for PRACH resource adaptation indication depends on the type of resource adaptation method to be agreed in proposal 4.1.6. And the number of reserved bits in DCI format 1_0 may or may not be sufficient for PRACH resource adaptation. We support DCI 1_0 with new RNTI. |
| HW |  | We think that CMCC has a good point. For witch we also think the support of other DCIs (that we agreed to study before) is needed on top of Paging DCI to have for example faster indications to the UEs. i.e. DCI 2_7 and DCI 1_0 with C-RNTI |
| Panasonic |  | When DCI format 1_0 is used for indication of PRACH adaptation, there will be difference in operation between case of network triggered traffic and case of UE initiated traffic. When UE has data traffic to be sent it must receive DCI 1_0 with P-RNTI (new paging DCI/P-RNTI) used for additional PRACH resources availability for certain period. There will be additional latency and additional UE power consumption to wake-up for reception of this DCI. To reduce latency and UE power consumption, information of PRACH adaptation in the DCI 1_0 with P-RNTI last received by the UE can still be valid. Alternatively, such adaptation information can be sent in the SIB1. |
| Nokia/NSB | Y | We agree with Panasonic. We propose to discuss the inclusion of adaptation information in the SIB1 as well. |
| ZTE, Sanechips | Y | We are fine with FL’s proposal. And we are open to consider other DCIs for PRACH adaptation. |
| LGE |  | We prefer to defer the discuss this issue and discuss first the required size of the field and which field(s) are used for indicating additional PRACH resources. |
| NEC |  | We also support the addition of other DCIs, e.g. DCI 2_7 and DCI 2_9. |
| DCM |  | We have same understanding with CMCC if following the legacy manner to monitor DCI format 1_0 with P-RNTI. We believe that monitoring occasion of the DCI for PRACH adaptation can be further studied to be enhanced. |

DCI field

Regarding the DCI field to be used for adaptation indication, several companies identified two main options.

Option 1: Up to six reserved bits from PHY that may be shared with TRS availability indicator may be available.

Option 2: Up to four reserved bits (Bits 5-8) within the short message (from upper layers) may be available.

Several companies (Huawei/HiSi, FW, QC, vivo, Fujitsu, CATT, LG, etc) propose to use the short message field, while some companies (Nokia, etc) suggested agreeing on the size of the field and then discuss which bits of paging DCI should be used. One company (Ericsson) suggested to have the field location within the DCI to be configurable.

## Discussion point 4.1.3

For DCI-based adaptation for additional PRACH resources, companies are invited to provide their views on

- DCI field size (including maximum value) for carrying the adaptation indication in the DCI

- How to handle indication for idle/inactive UE and for connected mode UE, respectively.

| Company | Comment |
| --- | --- |
| CMCC | We may discuss proposal 4.1.2 first and then look into this discussion point. |
| CATT | The DCI is used to indicate whether the additional PRACH resource is available or not, which is a PRACH resource configuration level indication. In this way, only 1 bit information is sufficient for the indication. |
| Fujitsu1 | Regarding the DCI field size, our view is that 1 bit is enough. Regarding the separate handing for idle/inactive UE and for connected UEs, we don’t think it is needed for PRACH resources used for CBRA. For PRACH resources used for CFRA, it can be discussed later after the outcome of the Discussion point 4.1.4. |
| Tejas | We prefer to discuss first proposal 4.1.6 and can decide on the maximum number of bits required for PRACH resource adaptation. |
| HW | 1 bit is enough |
| Nokia/NSB | We should first agree on how the PRACH adaptation is realized in terms of approach, to then identify the number of bits we need (maybe 1 is enough), to finally decide which ones we use. |
| ZTE, Sanechips | From the perspective of DCI field size, it is relevant to DCI-based adaptation for additional PRACH resources (i.e., Proposal 4.1.6). If PRACH resource configuration level (Alt 1) is supported, one bit is enough for DCI-based adaptation. If subset of PRACH resource level (Alt 2) is supported, further study is needed to determine the adaptation granularity in order to decide the DCI field size. In our view, we support Alt 1 in Proposal 4.1.6. As a result, DCI field size is one bit for carry the adaptation indication in the DCI. From the perspective of how to handle indication for idle/inactive UE and for connected mode UE, we think there would be ambiguity if idle/inactive UE and connected mode UE use a single DCI field in the DCI, since the PRACH configuration for idle/inactive UEs may be configured in SIB1, while PRACH configuration for connected mode UEs may be configured in RRC signaling. To this regard, we think two DCI fields for idle/inactive UEs and connected UEs, respectively in the DCI can address this problem. |
| LGE | For DCI field size, we think it depends on the outcome of Proposal 4.1.6. Regarding the handling of indication for idle/inactive/connected mode UEs, we think single DCI format is sufficient. |
| Moderator | Proposals are in 4.1.3 and 4.1.4 below. |

## Proposal 4.1.3

For DCI-based adaptation for additional PRACH resources,

- Select from the following options for carrying the adaptation indication in DCI format 1_0 with P-RNTI

- Option 1: Use reserved bits in the DCI format

  - FFS: relation (if any) to TRS availability bits in the DCI format

- Option 2: Use Bits 5-8 within the Short Message (from upper layers)

  - [Note: Availability should be confirmed by checking with RAN2.]

- Option 3: Use both Option 1 and Option 2

| Moderator | Closed. |
| --- | --- |

## Proposal 4.1.4

For DCI-based adaptation for additional PRACH resources,

- DCI field size for carrying the adaptation indication in the DCI format 1_0 with P-RNTI is down-selected from:

  - Alt 1: 1 bit

  - Alt 2: 2 bits

  - Alt 3: 4 bits

- FFS: whether the field used for connected mode UE and idle/inactive mode UE is same or different.

| Company | Comment |
| --- | --- |
| Apple | As discussed during online, it would be too early to discuss this before the contents are clearer. Currently,if an agreement is really needed, we would like to keep larger number of bits still on the table, e.g. 16bits. |

Applicability to CBRA and/or CFRA

A few companies discussed whether the additional PRACH resources can be associated with CBRA and/or CFRA. (Nokia, Xiaomi, Fujitsu, LG, Google).

Nokia:

There are two types of PRACH that can be configured:

- PRACH associated with CBRA configured by cell-specific signaling.

- PRACH associated with CFRA configured by UE-dedicated signaling.

The agreed PRACH adaptation shall at least target CBRA procedures. PRACH adaptation associated with CFRA procedures should be studied as well, if time allows.

Proposal-7: RAN1 to study at least PRACH adaptation solution(s) that are applied to CBRA. PRACH adaptation applied to CFRA should also be investigated, if time allows, however no CFRA-specific PRACH adaptation solutions should be considered.

Xiaomi:

Proposal 8: The adaptation of PRACH can be applied to CBRA and CFRA, including 4-step RACH and 2-step RACH.

Fujitsu:

Proposal 3. For the configuration of additional PRACH resources, discuss the following aspects:

* Whether to configure additional PRACH resources for 4-step RACH and/or 2-step RACH

* For 2-step RACH, adaptation of the PUSCH resources associated with the PRACH resources should also be considered

* Whether to configure additional PRACH resources for CBRA and/or CFRA

LG:

Proposal #14: Discuss whether additional ROs can be used for CFRA or not.

Google:

Proposal 9: For CFRA, support the NW to configure whether the UE should select the RO from additional PRACH resources for NES or legacy PRACH resources.

Proposal 10: For initial transmission of CBRA, support the UE to select the PRACH resource set based on the measured RSRP

* If the measured RSRP is above a threshold, UE selects the RO from the additional PRACH resources for NES; otherwise, UE selects the RO from legacy PRACH resources

Nokia:

There are two types of PRACH that can be configured:

- PRACH associated with CBRA configured by cell-specific signaling.

- PRACH associated with CFRA configured by UE-dedicated signaling.

The agreed PRACH adaptation shall at least target CBRA procedures. PRACH adaptation associated with CFRA procedures should be studied as well, if time allows.

Proposal-7: RAN1 to study at least PRACH adaptation solution(s) that are applied to CBRA. PRACH adaptation applied to CFRA should also be investigated, if time allows, however no CFRA-specific PRACH adaptation solutions should be considered.

Xiaomi:

Proposal 8: The adaptation of PRACH can be applied to CBRA and CFRA, including 4-step RACH and 2-step RACH.

Fujitsu:

Proposal 3. For the configuration of additional PRACH resources, discuss the following aspects:

* Whether to configure additional PRACH resources for 4-step RACH and/or 2-step RACH

* For 2-step RACH, adaptation of the PUSCH resources associated with the PRACH resources should also be considered

* Whether to configure additional PRACH resources for CBRA and/or CFRA

LG:

Proposal #14: Discuss whether additional ROs can be used for CFRA or not.

Google:

Proposal 9: For CFRA, support the NW to configure whether the UE should select the RO from additional PRACH resources for NES or legacy PRACH resources.

Proposal 10: For initial transmission of CBRA, support the UE to select the PRACH resource set based on the measured RSRP

* If the measured RSRP is above a threshold, UE selects the RO from the additional PRACH resources for NES; otherwise, UE selects the RO from legacy PRACH resources

Based on the company inputs, at least PRACH adaptation mechanism applies to the PRACH associated with CBRA configured by cell-specific signalling. Whether/how to handle the PRACH associated with CFRA configured by UE-specific RRC signaling may need more discussion.

## Discussion point 4.1.4

Companies are invited to provide their views on the following:

- The PRACH adaptation mechanism is applicable to at least for the PRACH associated with CBRA configured by cell-specific signaling.

- Whether/how (including the handling of additional PRACH resources) PRACH adaptation mechanism is applied for the following:

  - for the PRACH associated with CFRA configured by UE-specific RRC signaling

| Company | Comment |
| --- | --- |
| CMCC | We think PRACH adaptation can work with both CBRA and CFRA resource. |
| CATT | We agree to support CBRA. The motivation to support CFRA is unclear. |
| Fujitsu1 | For CFRA, aside from RRC signalling, DCI (such as PDCCH order) can also be considered. |
| Tejas | We support both CBRA and CFRA. |
| HW | Support to prioritize CBRA first. CRFA can be discussed later once CBRA design is finished. For the sub-bullet for CFRA “for the PRACH associated with CFRA configured by UE-specific RRC signaling”, UE-specific RRC signalling is used to configure the additional PRACH resources? |
| LGE | We think at least CBRA should be supported. We are open to discuss whether additional ROs can be used for CFRA or not. |
| China Telecom | At least CBRA should be supported. |
| NEC | PRACH adaptation can work with both CBRA and CFRA resource. |
| DCM | CBRA should be prioritized. |

Further details of configuration options (Alt 1 and Alt 2) for determining additional PRACH resources

Below agreement was made in RAN1#118bis

Agreement (from RAN1#118bis)

For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain,

- For Alt 1 (same PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s),

  - Opt 1-2a: up to N1 additional value(s) of (x, y)

    - FFS: value of N1(>=1)

  - Opt 1-2b: up to N2 additional value(s) of y

    - FFS: value of N2 (>=1)

  - Opt 1-3: Muting/masking ROs

  - Opt 1-4:

    - up to N4_1 additional timing offset(s) at frame-level

    - up to N4_2 additional timing offset(s) at slot-level

    - FFS: values of N4_1(>=1) and N4_2(>=1)

  - Opt 1-5: No additional mechanism is selected.

  - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211)

- For Alt 2 (different PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s),

  - Opt 1-2a: up to N1 additional value(s) of (x, y)

    - FFS: value of N1(>=1)

  - Opt 1-2b: up to N2 additional value(s) of y

    - FFS: value of N2 (>=1)

  - Opt 1-3: Muting/masking ROs

  - Opt 1-4:

    - up to N4_1 additional timing offset(s) at frame-level

    - up to N4_2 additional timing offset(s) at slot-level

    - FFS: values of N4_1(>=1) and N4_2(>=1)

  - Opt 1-5: No additional mechanism is selected.

  - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211)

Agreement (from RAN1#118bis)

For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain,

- For Alt 1 (same PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s),

  - Opt 1-2a: up to N1 additional value(s) of (x, y)

    - FFS: value of N1(>=1)

  - Opt 1-2b: up to N2 additional value(s) of y

    - FFS: value of N2 (>=1)

  - Opt 1-3: Muting/masking ROs

  - Opt 1-4:

    - up to N4_1 additional timing offset(s) at frame-level

    - up to N4_2 additional timing offset(s) at slot-level

    - FFS: values of N4_1(>=1) and N4_2(>=1)

  - Opt 1-5: No additional mechanism is selected.

  - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211)

- For Alt 2 (different PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s),

  - Opt 1-2a: up to N1 additional value(s) of (x, y)

    - FFS: value of N1(>=1)

  - Opt 1-2b: up to N2 additional value(s) of y

    - FFS: value of N2 (>=1)

  - Opt 1-3: Muting/masking ROs

  - Opt 1-4:

    - up to N4_1 additional timing offset(s) at frame-level

    - up to N4_2 additional timing offset(s) at slot-level

    - FFS: values of N4_1(>=1) and N4_2(>=1)

  - Opt 1-5: No additional mechanism is selected.

  - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211)

Below table summarizes the company views.

| Same PRACH config index (Alt 1) | Opt 1-2a | Xiaomi, LG (N1>1), Qualcomm (N1=1), NEC, CMCC, Google, Mediatek, Quectel, CT |
| --- | --- | --- |
|  | Opt 1-2b | Apple (N2=1), ETRI,, CMCC, Transsion, Mediatek, Quectel, ZTE |
|  | Opt 1-3 | Ericsson, Apple (muting happens after SSB-to-RO mapping), CATT, Samsung, DoCoMo, Nokia |
|  | Opt 1-4 | Nokia, Xiaomi, Ericson, Fujitsu, Apple (N4_2=1 for x =1 (10ms periocicity)), CATT, Tejas, ETRI, Intel, DoCoMo, CMCC, Sharp, Oppo, Transsion, Mediatek, ZTE |
|  | Opt 1-5 | Interdigital, Honor, vivo, Huawei/HiSi, CT |
| Different PRACH config index (Alt 2) | Opt 1-2a | LG (N1>1), Qualcomm (N1=1), Mediatek, Quectel, CT |
|  | Opt 1-2b | Apple (N2=1), NEC, ETRI, Transsion, Mediatek, Quectel |
|  | Opt 1-3 | Ericsson, FW, Fujitsu, Apple (muting happens after SSB-to-RO mapping), CATT, Samsung, DoCoMo, Google. Sharp, Oppo, Nokia |
|  | Opt 1-4 | Nokia, Ericsson, Apple (N4_2=1 for x =1 (10ms periodicity)), CATT, Tejas, ETRI, Intel, DoCoMo, Transsion, Mediatek |
|  | Opt 1-5 | Xiaomi, Interdigital, Honor, vivo, Huawei/HiSi, CMCC, CT, ZTE |

Motivations:

- Opt 1-2a/1-2b

  - Yes: can provide frame-level offset

  - No: effectively creates new PRACH configuration

- Opt 1-3 / Opt 1-4:

  - Yes: it facilitates condensed PRACH resources

  - No: effectively creates new PRACH configuration

- Opt 1-5:

  - Yes: There is already sufficient flexibility from the current PRACH configuration table and additional frequency domain parameter

  - No: overlapping resources handling is needed, need a mechanism for condensed PRACH

- From Fujitsu (illustrating the issue of overlaps between legacy and PRACH additional resources) :

- Qualcomm: Define the additional PRACH resources as the PRACH resources that are derived based on the additional PRACH configuration parameter(s) and do not overlap in both time and frequency with PRACH resources configured for legacy UEs.

## Proposal 4.1.5

For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain,

- For Alt 1 (same PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s),

  - Opt 1-2a: up to N1 additional value(s) of (x, y)

    - N1 = [1 or 2 or 3]

  - Opt 1-2b: up to N2 additional value(s) of y

    - N2 = [1 or 2 or 3]

  - Opt 1-3: Muting/masking ROs

  - Opt 1-4:

    - up to N4_1 additional timing offset(s) at frame-level

    - up to N4_2 additional timing offset(s) at slot-level

    - N4_1 = [1 or 2 or 3]

    - N4_2 = [1 or 2 or 3]

  - Opt 1-5: No additional mechanism is selected.

  - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211)

- For Alt 2 (different PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s),

  - Opt 1-2a: up to N1 additional value(s) of (x, y)

    - FFS: value of N1(>=1)

  - Opt 1-2b: up to N2 additional value(s) of y

    - FFS: value of N2 (>=1)

  - Opt 1-3: Muting/masking ROs

    - Muting is after the SSB-to-RO mapping

  - Opt 1-4:

    - up to N4_1 additional timing offset(s) at frame-level

    - up to N4_2 additional timing offset(s) at slot-level

    - N4_1 = [1 or 2 or 3]

    - N4_2 = [1 or 2 or 3]

  - Opt 1-5: No additional mechanism is selected.

  - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211)

| Company | Support (Y/N) | Comment |
| --- | --- | --- |
| Moderator |  | When providing input, companies are invited to also consider and indicate whether/how the mechanism facilitates condensed PRACH resources in time-domain. |
| CMCC | Y |  |
| CATT | Y |  |
| Fujitsu1 |  | Regarding Opt 1-3 in Alt 2, our understanding is that it can address the issue when a subset of additional PRACH resources overlap with legacy PRACH resources in both time and frequency domains. In this case, different SSB-to-RO mapping results for additional PRACH resources that overlap with legacy PRACH resources should be avoided. However, if muting is performed after the SSB-to-RO mapping, it might result in an uneven number of ROs associated with each SSB index. On the other hand, to facilitate condensed PRACH resources in time domain, muting after SSB-to-RO mapping can be considered. Considering that performing muting both before and after SSB-to-RO mapping are possible, we propose to remove the phrase “muting is after the SSB-to-RO mapping”. - Opt 1-3: Muting/masking ROs - Muting is after the SSB-to-RO mapping |
| Tejas | N | According to Tejas N4_1 is similar to frame offset (As given in 38.211: 𝑦IAB = (𝑦 + Δ𝑦) mod 𝑥 where Δ𝑦 is given by the higher-layer parameter prach-ConfigurationFrameOffset-IAB, and 𝑥 is the value used in 𝑛f mod 𝑥 = 𝑦;) given for IAB. And N4_2 is similar to slot offset (As given in 38.211: The subframe number 𝑠n from Tables 6.3.3.2-2 to 6.3.3.2-3 and the slot number 𝑠 n from Table 6.3.3.2-4 shall be replaced by (𝑠n + Δ𝑠) mod 𝐿 where Δ𝑠 ∈ {0,1, … , 𝐿 - 1} is given by the higher-layer parameter prach-ConfigurationSOffset-IAB, and 𝐿 is the number of subframes in a frame when using Tables 6.3.3.2-2 to 6.3.3.2-3 and the number of slots in a frame for 60 kHz subcarrier spacing when using in Table 6.3.3.2-4.) given for IAB. If the above understanding is correct, then N4_1 can have 16 [0,1,2, … 15] values and N4_2 can have 10 values [0,1,2…9] for 15KHz and 40 values [0,1, 2, … ,39] for 60KHz SCS. |
| HW |  | We already agreed to support same/different index and frequency parameters, based on that, how to condensed PRACH resources in time-domain and avoid the overlap is up to gNB implementation. And in the commercial TDD network, the TDD pattern is generally fixed and the uplink resources for PRACH transmission is almost fixed, additional optimization for PRACH is not helpful in this case. |
| Panasonic |  | Alt 1 will always lead to overlapping ROs between legacy and additional PRACH resources. Using frequency offset can solve the issue but, in our understanding, it is simply replicating the same set of resources and not contributing to time domain adaptation. Alt 2 with at least muting/masking (opt 1-3) should be considered. The idea is to have sparse legacy ROs (larger periodicity) and denser additional ROs (smaller periodicity) using Alt 2 and using muting/masking or validity time windows to create condensed ROs. |
| Nokia/NSB |  | We are ok with the spirit of the proposal. While we find it suboptimal (Alt 1-2 variants are not needed in our view) we could support it if Opt 1-5 is removed from the alternatives for Alt. 2. The reason is simple. Different network deployments, gNB’s architectures and schedulers have different requirements and needs. We must create a system that offers at least sufficient flexibility to accommodate all these differences. By the way, we encourage companies not to focus exclusively on the condensed RO use case, since this is only one possible instance of the PRACH adaptation, that comes with plenty of limitations for implementation, scalability of computational capabilities at gNB and scheduler flexibility. We have to be very careful not to create a feature that comes with too many limitations to be implemented… |
| ZTE, Sanechips |  | From our perspective, Alt 2 with Opt 1-5 is the most straightforward method to configure additional PRACH resources. Besides, when considering facilitating condensed PRACH resources in time-domain, Alt 1 with Opt 1-4 can be used. For one thing, N4_1 consecutive additional timing offsets are configured to facilitating condensed PRACH resources. For another, one additional timing offset with a configured N4_1 can also achieve condensed PRACH resources. |
| Moderator |  | Continue discussion. |
| Nokia/NSB |  | We start by proposing to change the sub-bullet of Option 1-3 to “muting/masking”, since technically all PRACH masks in NR operate as active indication, i.e., the RO can be used, and not as a negative indication, i.e., the RO cannot be used. Changing this logic would seem very odd to us, and it is better to avoid implying that the PRACH mask mutes RO… Moving to the merit of the mask, we understand that some companies are a bit confused on how this would work. We will try to provide a comprehensive explanation to clarify the matter. First, we think that masks could focus only on time domain, given that several frequency domain parameters already allow a fine RO configuration in frequency domain, and we have already made on an important agreement during this meeting. Masks according to this principle would not discriminate ROs in the frequency domain and act on the resources as per PRACH configuration index to activate subsets of additional ROs in time domain, according to patterns who strictly depend on the PRACH configuration index itself. We observe that this also allows to preserve even collision probability across all ROs, without resorting to more complex mechanisms. A good approach in this sense would be for the mask to indicate SFNs or association periods according to the legacy indication approach (i.e., all, only one, even/odd, none), to minimize spec impact and allow NW to ensure that once additional ROs are created, a suitable tool can be used to obtain effects like, but not limited to: - Additional ROs occur only in even/odd SFNs/association periods - Additional ROs occur only in specific SFNs/association periods - Additional ROs can occur either on even or odd SFNs, irrespective of whether the configuration period is counted from an even or odd system frame (i.e., as given by the values of x and y as per PRACH configuration index) In other words, the structure and size of existing PRACH mask could be reused (Table 7.4.1 of TS 38.321), and only the interpretation associated to the different codepoints could be changed, i.e., the codepoint of the PRACH mask for NES would correspond to different indications as compared to their existing legacy PRACH mask counterparts. Only NES-capable UEs would be able to interpret this mask to identify a subset of additional ROs that can be used by such UEs, together with the legacy ROs, for performing the PRACH transmission. The applicability of such PRACH mask for NES would not depend on how the additional ROs are created, e.g., via same or different PRACH configuration index, to allow NW to use it to increase the scheduler’s flexibility with no specific constraint. Now, legacy PRACH mask resets at each mapping cycle, when N<1 (up to 1/8), hence a natural extension for NES would be for the mask to reset every N SFNs/association periods, where N could be equal to 8 for simplicity. In this context, a possible mask could be as the one below, which is based on Table 7.4-1: PRACH Mask Index values of TS 38.321, and where N could also be larger than 8 and Reserved codepoints could also be used if RAN1 agrees to this. We observe that this is just an example for the sake of better explaining, and should not be considered as a proposal to be included in any agreement. PRACH Mask Index/ msgA-SSB-SharedRO-MaskIndex/ ssb-SharedRO-MaskIndex; SFs with additional valid ROs. 0; All. 1; mod(SFN,N)=0. 2; mod(SFN,N)=1. 3; mod(SFN,N)=2. 4; mod(SFN,N)=3. 5; mod(SFN,N)=4. 6; mod(SFN,N)=5. 7; mod(SFN,N)=6. 8; mod(SFN,N)=7. 9; Every even SFN. 10; Every odd SFN. 11; Reserved. 12; Reserved. 13; Reserved. 14; Reserved. 15; Reserved. Examples follow, taken from our Tdoc R1-2409714. Assume for simplicity that PRACH configuration index #113 is used for legacy RO configuration and PRACH configuration index #125 is used for additional RO configuration. If codepoint 9 is configured/indicated, this allows NW to distribute subframes with ROs for NES UEs evenly across all subframes, helping NW to reduce resource saturation in the odd subframes: If codepoint 9 is configured/indicated, this allows NW to obtain clustered/condensed ROs for all UEs in cells in odd subframes: If codepoint 5 is configured, then additional ROs occur only on the fifth SF, i.e., the one with SFN4, which could help gNB scheduling specific UL channels/signals transmissions over specific SFNs, since additional ROs are occurring only in one SF. Overall, as we explained during the offline session, it is fundamental for gNB to be able to operate at this level since the number of supported PRACH configuration indices on the field is very limited and the configuration “freedom” for NW would be almost zero in many case, unless we introduce masks (and/or time offset, actually). |
| LGE |  | According to Random access configuration tables (i.e., Table 6.3.3.2-2 to 6.3.3.3.2-4) currently in TS 38.211, configurations with the same y (offset) value and different x (period) values are not fully covered by different RACH configuration indices. In addition, if the multiple candidate (x, y) values can be configured in advance and one of them dynamically indicates through the DCI indicating RO adaptation, the N1 value is preferred to be greater than 1 because the cycle of additional PRACH resources can be flexibly adjusted. In this regard, we support Opt 1-2a with N1>1, and Opt 1-2b and Opt 1-4 can be removed because Opt 1-2a with N1>1 can cover the additional PRACH configurations that can be configured with Opt 1-2b and Opt 1-4. Moreover, it is also desirable for Alt 1 and Alt 2 to have a common additional mechanism, Opt 1-2a should be supported by both Alt 1 and Alt 2 without excluding Opt 1-2a from Alt 2 as well. |
| Moderator |  | Proposal updated in 4.1.5a-rev1 to discuss the Alt 2 first. |
| Apple |  | According to our understanding, there are two issues discussed here, - How to create additional non-overlapping PRACH resources in time domain - How to create condensed RO patterns For the 1st issue, there are cases that when non-overlapping can handled by gNB implementation, so we think the following can be considered: For configuring additional PRACH resources, select from the following options Opt 1-4: up to 1 time offset in frame-level/slot-level Opt 1-6’: Based on NW indication, additional PRACH resources are defined as the PRACH resources that are derived based on the additional PRACH configuration index only parameter(s) andor based on the the additional PRACH configuration index and that do not overlap in both time and frequency with PRACH resources configured for legacy UEs.] This is due to that the non-overlapping resource may always be needed. For the 2nd issue, since the Paging cycle is at least 320ms, and could be as long as 2560ms, if a dense PRACH configuration is configured, the pattern needs to last for at least one paging cycle, which is not energy saving. Therefore, we think the condensed RO is needed to provide enough NES gain. We also think it would be beneficial for NW to be able to adjust the pattern of the condensed RO based on the signaling. We agree that there could be multiple solutions, including multiple offsets, muting/masking ROs before/after SSB-to-RO mapping, we can further discuss on that. For configuring condensed PRACH resources, select from the following options: Option 1-3: Masking ROs Option 1-4: Multiple time offsets in frame-level/slot-level |
| Qualcomm |  | Based on some offline with proponents of Alt 1-5, the following proposal can be a possible compromise: For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain, - For Alt 2 (different PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s), - Opt 1-6A: If an additional PRACH resource overlaps in both time and frequency with a PRACH resource configured for legacy UEs, the UE only considers the PRACH resource configured for legacy UEs in the overlapped resource. - FFS: whether the consideration is before or after SSB-RO mapping |
| Moderator |  | Proposals updated in 4.1.5a-rev2 and 4.1.5b. |

## Proposal 4.1.5a-rev1

For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain,

- For Alt 2 (different PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s),

  - Opt 1-2a: up to N1 additional value(s) of (x, y)

    - FFS: value of N1(>=1)

  - Opt 1-2b: up to N2 additional value(s) of y

    - FFS: value of N2 (>=1)

  - Opt 1-3: Muting/masking ROs

    - Muting is after the SSB-to-RO mapping

  - Opt 1-4:

    - up to N4_1 additional timing offset(s) at frame-level

    - up to N4_2 additional timing offset(s) at slot-level

    - N4_1 = [1 or 2 or 3]

    - N4_2 = [1 or 2 or 3]

      - FFS: For PRACH configuration index (for additional resources) with periodicity larger than 10ms, N4_2 = 0.

  - Opt 1-5: No additional mechanism is selected.

  - [Opt 1-6: Additional PRACH resources are defined as the PRACH resources that are derived based on the additional PRACH configuration parameter(s) and that do not overlap in both time and frequency with PRACH resources configured for legacy UEs.]

## Proposal 4.1.5a-rev2

For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain,

- For Alt 2 (different PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s),

  - Opt 1-2a: up to N1 additional value(s) of (x, y)

    - FFS: value of N1(>=1)

  - Opt 1-2b: up to N2 additional value(s) of y

    - FFS: value of N2 (>=1)

  - Opt 1-3: Muting/masking ROs

    - Muting is after the SSB-to-RO mapping

  - Opt 1-4:

    - up to N4_1 additional timing offset(s) at frame-level

    - up to N4_2 additional timing offset(s) at slot-level

    - N4_1 = [1 or 2 or 3]

    - N4_2 = [1 or 2 or 3]

      - FFS: For PRACH configuration index (for additional resources) with periodicity larger than 10ms, N4_2 = 0.

  - Opt 1-5: No additional mechanism is selected.

  - Opt 1-6A: If an additional PRACH resource (indicated by the PRACH configuration index for the additional PRACH resources) overlaps in both time and frequency with a PRACH resource configured for legacy UEs, the UE only considers the PRACH resource configured for legacy UEs in the overlapped resource.

    - FFS: whether the consideration is before or after SSB-RO mapping

## Proposal 4.1.5b

For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain,

- For Alt 1 (same PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s),

  - Opt 1-2a: up to N1 additional value(s) of (x, y)

    - FFS: value of N1(>=1)

  - Opt 1-2b: up to N2 additional value(s) of y

    - FFS: value of N2 (>=1)

  - Opt 1-3: Muting/masking ROs

  - Opt 1-4:

    - up to N4_1 additional timing offset(s) at frame-level

    - up to N4_2 additional timing offset(s) at slot-level

    - N4_1 = [1 or 2 or 3]

    - N4_2 = [1 or 2 or 3]

  - Opt 1-5: No additional mechanism is selected.

  - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211)

Further details of DCI-based adaptation

Below agreement was made in RAN1#118 meeting.

Agreement

For DCI-based adaptation for additional PRACH resources, select only from the following alternatives

- Alt 1: (PRACH resource configuration level) DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: details

- Alt 2: (subset of PRACH resource level) DCI-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: whether the subset of the additional PRACH resources is in

    - Alt 2-1: RO level per SSB

    - Alt 2-2: SSB-to-RO mapping cycle level

    - Alt 2-3: PRACH association period level

    - Alt 2-4: PRACH association pattern period level

    - Alt 2-5: SFN level

- Alt 3: DCI-based Enhanced/new Cell DRX to indicate whether the enhanced/new Cell DRX is activated or deactivated.

  - If activated, the additional configured PRACH provided by semi-static signalling within non-active period are not available.

  - FFS: whether Alt 1 and/or Alt 2 can be applied to the active period

  - FFS: details

Agreement

For DCI-based adaptation for additional PRACH resources, select only from the following alternatives

- Alt 1: (PRACH resource configuration level) DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: details

- Alt 2: (subset of PRACH resource level) DCI-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: whether the subset of the additional PRACH resources is in

    - Alt 2-1: RO level per SSB

    - Alt 2-2: SSB-to-RO mapping cycle level

    - Alt 2-3: PRACH association period level

    - Alt 2-4: PRACH association pattern period level

    - Alt 2-5: SFN level

- Alt 3: DCI-based Enhanced/new Cell DRX to indicate whether the enhanced/new Cell DRX is activated or deactivated.

  - If activated, the additional configured PRACH provided by semi-static signalling within non-active period are not available.

  - FFS: whether Alt 1 and/or Alt 2 can be applied to the active period

  - FFS: details

Comapany views on this are as follows:

Alt 1: FW, Mediatek, vivo, Xiaomi, Fujitsu, CATT, Honor, Interdigital, Sharp, Ericsson

Alt 2: Nokia (PRACH masking(dynamic)), Panasonic, Fujitsu, Sony, Quectel, Samsung, Qualcomm, Transsion, Cewit, Tejas, Apple

- Alt 2-1: Samsung, Cewit

- Alt 2-2: Qualcomm, Transssion, Cewit

- Alt 2-3: Qualcomm

- Alt 2-5: Xiaomi, Tejas

Alt 3: Samsung, Transsion, NEC

While some companies have provided some upper limits on number of subsets (no more than 2 or 4 or 16), proponents of Alt 2 are encouraged to provide a bit more information on the maximum number of subsets, considering the DCI field size limitations and UE/gNB complexity in handling multiple subsets of additional PRACH resources.

## Proposal 4.1.6

For DCI-based adaptation for additional PRACH resources, select only from the following alternatives:

- Alt 1: (PRACH resource configuration level) DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: details

- Alt 2: (subset of PRACH resource level) DCI-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not

  - Maximum number of subsets of the additional PRACH resources= [2 or 3 or 4 or 16]

  - FFS: whether the subset of the additional PRACH resources is in

    - Alt 2-1: RO level per SSB

    - Alt 2-2: SSB-to-RO mapping cycle level

    - Alt 2-3: PRACH association period level

    - Alt 2-4: PRACH association pattern period level

    - Alt 2-5: SFN level

- Alt 3: DCI-based Enhanced/new Cell DRX to indicate whether the enhanced/new Cell DRX is activated or deactivated.

  - If activated, the additional configured PRACH provided by semi-static signalling within non-active period are not available.

  - FFS: whether Alt 1 and/or Alt 2 can be applied to the active period

  - FFS: details

| Company | Support (Y/N) | Comment |
| --- | --- | --- |
| Moderator |  | When providing input, companies are invited to also indicate the preference on the maximum #subsets, and the sub-alternatives under Alt 2, including any details on how the subsets are determined. |
| CMCC | Y | Prefer Alt 1 as the most straightforward way. |
| CATT |  | Alt1 is preferred. |
| Fujitsu1 | Y | Alt 2 can be considered if it can be realized by using a 1-bit indication to indicate whether the configured subset pattern is activated. |
| Tejas | Y | Alt 2-5 is preferred. DCI indication for the available additional PRACH at SFN level can be indicated for a duration of 160 msec. The number of bits required is dependent on the PRACH configuration periodicity. A minimum of 1-bit is required for 160msec PRACH configuration periodicity and a maximum of 16-bit are required for PRACH configuration of periodicity of 10 msec. Grouping of SF’s can also be considered if there is restriction on the number of available for indication. |
| HW |  | Alt1 is preferred. |
| Panasonic |  | Alt 1 is simply indication of additional PRACH resources ON/OFF whereas Alt 2 allows adaptation within the additional PRACH resources provided by the semi-static signalling. Alt 2 can offer more flexibility of adaptation as subset of PRACH resources can be indicated. The gNB can make use of such flexibility, for example to achieve more condensed active period and longer sleeping duration. |
| Nokia/NSB |  | Agree with Panasonic. Do not see any merit for Alt 2-1 and Alt 2-2 and do not understand why Alt 2-4 is not retained instead. Alt 2-1 and Alt 2-2 are unsuitable for the needs of NES applications, given that they have been designed for indicating resources only when the number of back-to-back ROs mapped to the same SSB is larger than 1. Their applicability is extremely limited and would make the adaptation quite limited as well. Alt 2-4 and Alt. 2-5 offer much more with very limited specification impact. Finally, we note that any alternative 2 can be indicated with only 1 bit in the DCI, similar to Alt. 1, if the subset of ROs to be indicated is configured via SIB1. We should compare apples with apples and not mix things. |
| Moderator |  | Continue discussion based on below agreement from Tuesday morning session. Agreement For DCI-based adaptation for additional PRACH resources, select only from the following alternatives: - Alt 1: (PRACH resource configuration level) DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not - FFS: details - Alt 2: (subset of PRACH resource level) DCI-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not - FFS: Maximum number of subsets of the additional PRACH resources= [2 or 3 or 4 or 16] - FFS: whether the subset of the additional PRACH resources is in - Alt 2-1: RO level per SSB - Alt 2-2: SSB-to-RO mapping cycle level - Alt 2-3: PRACH association period level - Alt 2-4: PRACH association pattern period level - Alt 2-5: SFN level - Alt 2-6: Network configured time period - Alt 3: DCI-based Enhanced/new Cell DRX to indicate whether the enhanced/new Cell DRX is activated or deactivated. - If activated, the additional configured PRACH provided by semi-static signalling within non-active period are not available. - FFS: whether Alt 1 and/or Alt 2 can be applied to the active period - FFS: details |
| Nokia/NSB |  | We would like to ask two questions to all companies: FIRST Let us discuss the Alt2-x alternatives a bit more technically. From our perspective the 6 options can be divided in 2 groups: - Approaches that can be used only when N<1, where N is the number of SSB per RACH occasion: - Alt 2-1: RO level per SSB - Alt 2-2: SSB-to-RO mapping cycle level - Approaches that can be used independently of the value of N: - Alt 2-3: PRACH association period level - Alt 2-4: PRACH association pattern period level - Alt 2-5: SFN level - Alt 2-6: Network configured time period Is any company objecting this split? If yes, why? SECOND If a PRACH mask as per Alt 2-2 to Alt 2-5 or a time period as per Alt 2.6 is configured in SIB1, and is thus part of the PRACH configuration, and is then activated with 1 bit in the DCI, is this solution compatible with an Alt-1 approach? If yes, we should not focus on DCI overhead (yet) but only on the technical merit of the approach, and then decide whether we go for dynamic indication of dynamic activation. If no, what would be the reason? |
| LGE |  | We think that at least Alt 1 should be supported, and we can discuss further whether to support the adaptation a subset of the additional PRACH resources. Regarding Alt 2, we think gNB can configure which level the additional PRACH resources are adapted to: RO level/SSB-to-RO mapping cycle level/PRACH connection cycle level/PRACH connection pattern cycle level and bitmap signaling mechanism in DCI can be determined based on this gNB’s configuration. Therefore, we prefer to support Alt 2-6 (Network configured time period). |
| China Telecom | Y | We prefer Alt1. |
| Apple |  | As we commented for Proposal 4.1.5, since the Paging cycle is at least 320ms, and could be as long as 2560ms, if only a dense PRACH configuration is configured, the pattern needs to last for at least one paging cycle, which is not energy saving. For Therefore, we think it would be beneficial for NW to be able to adjust the RO patterns to achieve different level of energy saving gain while maintaining reasonable latency for UE. In that sense, to move forward, we think the following can be considered: - For DCI-based adaptation for additional PRACH resources, whether to activate all or a subset of PRACH resources can be indicated by NW - NW can configure subset-level of adaptation pattern(s) - FFS: SFN-level, PRACH association period level, or PRACH association pattern period level. |
| NEC | Y | We think that Alt-2 can be beneficial and should be supported. Alt-1 may be used when the DCI size is not sufficient to perform Alt-2 |
| DCM |  | We support Apple’s modification and the SECOND comment from Nokia. It seems good way for both of NES and UE processing to semi-statically configure some subset patterns of additional PRACH resources in a reasonable granularity. |
| Moderator |  | Proposal is updated to 4.1.6-rev1. |

## Proposal 4.1.6-rev1

For DCI-based adaptation for additional PRACH resources, select only from the following alternatives:

- Alt 1: (PRACH resource configuration level) DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not

  - [DCI field size = 1 bit]

  - FFS: details

- Alt 2: (subset of PRACH resource level) DCI-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not

  - Option 1:

    - [DCI field size = 1 bit]

    - A single PRACH mask provided by semi-static signalling is used to identify the subset of the additional PRACH resources

  - Option 2

    - [DCI field size = 4 bits]

    - FFS: How to identify subsets of additional PRACH resources

  - FFS: Maximum number of subsets of the additional PRACH resources= [2 or 3 or 4 or 16]

  - FFS: whether the subset of the additional PRACH resources is in

    - Alt 2-1: RO level per SSB

    - Alt 2-2: SSB-to-RO mapping cycle level

    - Alt 2-3: PRACH association period level

    - Alt 2-4: PRACH association pattern period level

    - Alt 2-5: SFN level

    - Alt 2-6: Network configured time period

# Adaptation of Paging

Many proposals submitted for this are related to the PO/PF determination and paging-related configuration/procedures defined in RAN2 specifications. As per the agreement in earlier RAN1 meeting, such aspects are expected to be handled by RAN2.

Regarding the adaptation, some companies discussed/proposed that using SI update mechanism is sufficient, while some others proposed to consider/support dynamic adaptation of paging. Some companies discussed potential impacts to first PDCCH monitoring occasions, PEI for the new POs. Few companies suggested to wait for RAN2 progress.

It seems more RAN2 progress may be needed before any potential discussions in RAN1.

## Moderator suggestion for RAN1#119: continue to wait for RAN2 progress.

# Other

Some contributions also discussed joint adaptation of more than one common signal/channel.

# Conclusion

# Appendix A (Contributions)

| 1 | R1-2409415 | On common channel/signal adaptation for eNES | Huawei, HiSilicon |
| --- | --- | --- | --- |
| 2 | R1-2409470 | Adaptation of common signal/channel transmissions | Quectel |
| 3 | R1-2409519 | Discussion on adaptation of common signal/channel transmissions | CMCC |
| 4 | R1-2409558 | Discussion on common signal/channel for NES | ZTE Corporation, Sanechips |
| 5 | R1-2409604 | Adaptation of common signal/channel transmissions | Samsung |
| 6 | R1-2409643 | Discussion on adaptation of common signal/channel transmissions | Spreadtrum, UNISOC |
| 7 | R1-2409688 | Discussions on adaptation of common signal/channel transmissions | vivo |
| 8 | R1-2409714 | Adaptation of common signal/channel transmissions | Nokia, Nokia Shanghai Bell |
| 9 | R1-2409737 | Discussion on adaptation of common signal/channel transmission | Intel Corporation |
| 10 | R1-2409756 | Adaptation of common signals/channels | Tejas Networks Limited |
| 11 | R1-2409774 | Discussion on adaptation of common signal/channel transmissions | InterDigital, Inc. |
| 12 | R1-2409810 | On adaptation of common signal/channel for NES enhancements | Apple |
| 13 | R1-2409903 | Discussion on adaptation of common signal and channel transmissions | Xiaomi |
| 14 | R1-2409948 | Discussion on adaptation of common signal/channel transmissions | CATT |
| 15 | R1-2409967 | Discussion on adaptive transmission of common signal or common channel | Transsion Holdings |
| 16 | R1-2410005 | Discussion on common signal/channel adaptation | China Telecom |
| 17 | R1-2410034 | Discussion of the adaptation of common signal/channel transmissions | FUTUREWEI |
| 18 | R1-2410076 | Discussion on adaptation of common signal/channel transmission | OPPO |
| 19 | R1-2410134 | Discussion on adaptation of common signal / channel transmissions | Fujitsu |
| 20 | R1-2410158 | Adaptation of Common Signals | Google |
| 21 | R1-2410180 | Discussion on adaptation of common signal channel transmissions | HONOR |
| 22 | R1-2410211 | Discussion on adaptation of common signal/channel transmissions | NEC |
| 23 | R1-2410230 | Adaptation of common signal/channel transmissions | Sony |
| 24 | R1-2410247 | Discussion on adaptation of common signal/channel transmission | Panasonic |
| 25 | R1-2410273 | Adaptation of common signal/channel transmissions | ETRI |
| 26 | R1-2410293 | Adaptation of common signal/channel transmissions | LG Electronics |
| 27 | R1-2410302 | Adaptation of common signals and channels | Lenovo |
| 28 | R1-2410362 | Discussion on adaptation of common signal/channel transmissions | Sharp |
| 29 | R1-2410396 | Discussion on adaptation of common signal/channel transmissions | NTT DOCOMO, INC. |
| 30 | R1-2410443 | Adaptation of common signal/channel transmissions for NES | Ericsson |
| 31 | R1-2410485 | Adaptation of common channel transmissions | Qualcomm Incorporated |
| 32 | R1-2410523 | Adaptation of common signal/channel transmissions | MediaTek Inc. |
| 33 | R1-2410583 | Discussion on adaptation of common signal and channel transmissions | CEWiT |

Tdocs (from AI 5) related to the LS from RAN4 (R1-2409350)

| 1 | R1-2409350 | LS on SSB relation in On-demand SSB and SSB adaptation on Scell | RAN4, Ericsson |
| --- | --- | --- | --- |
| 2 | R1-2409495 | Discussion on LS on SSB relation in on-demand SSB and SSB adaptation on SCell | CMCC |
| 3 | R1-2409548 | LS reply on SSB relation in On-demand SSB and SSB adaptation | ZTE Corporation, Sanechips |
| 4 | R1-2409549 | Discussion on LS on SSB relation in On-demand SSB and SSB adaptation | ZTE Corporation, Sanechips |
| 5 | R1-2409571 | Draft reply LS on SSB relation in on-demand SSB and SSB adaptation on Scell | Samsung |
| 6 | R1-2410038 | Discussion on LS on SSB relation in on-demand SSB and SSB adaptation on SCell | Nokia, Nokia Shanghai Bell |
| 7 | R1-2410039 | Draft reply LS on SSB relation in on-demand SSB and SSB adaptation on Scell | Nokia, Nokia Shanghai Bell |
| 8 | R1-2410077 | Discussion on SSB relation in On-demand SSB and SSB adaptation operation | OPPO |
| 9 | R1-2410078 | Draft reply LS on SSB relation in On-demand SSB and SSB adaptation operation | OPPO |
| 10 | R1-2410147 | Draft Reply LS on SSB relation in On-demand SSB and SSB adaptation on Scell | Google |
| 11 | R1-2410284 | Discussion on RAN4 LS on SSB relation in On-demand SSB and SSB adaptation on Scell | LG Electronics |
| 12 | R1-2410541 | Draft reply LS on SSB relation in On-demand SSB and SSB adaptation on Scell | CATT |
| 13 | R1-2410609 | Discussion on RAN4 LS on SSB relation in On-demand SSB and SSB adaptation on SCell | Huawei, HiSilicon |

# Appendix B (WI objectives from WID in RP-242354)

# List of RAN1 agreements

## RAN1#116

Agreement

For adaptation of SSB in time-domain, consider the following adaptation mechanisms for further study

- Adaptation of SSB burst periodicity

- Adaptation based on two SSB configurations where up to two configurations can be active

- Adaptation based on skipping/transmitting some SSB bursts non-uniformly with single SSB configuration

- Adapting the transmitted number of SSBs within a SSB burst

- Cell DTX for SSB adaptation

- Whether to support new SSB burst periodicity value(s)

- Whether to support new SSB burst(s) (i.e. how SSB transmission is made within a burst)

  - New compact SSB burst(s)

  - Adapting the position of SSBs within a SSB burst

- Other mechanisms/combinations are not precluded

Agreement

For adaptation of PRACH in time-domain, consider the following adaptation mechanisms for further study

- Adaptation based on configuration of additional[/different] PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any)

  - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs

- For the additional PRACH resources,

  - Adaptation of PRACH resource periodicity/PRACH occasion

  - Adaptation at PRACH configuration/association period/association pattern period level and SSB to RO mapping cycle

  - Adaptation based on extending cell DRX operation for PRACH

  - Concentrating ROs in time domain

- Other options are not precluded

Agreement

For adaptation of paging,

- Study further from RAN1 perspective, techniques for adaptation of paging occasions in time-domain and achievable network energy savings

- Note: Specification details for PO/PF determination and paging-related configuration/procedures to be handled by RAN2

Agreement

For the adaptation mechanisms of SSB in time-domain, study further applicable scenarios and associated legacy UE impact/handling (if any) based on the following:

- Applicability to UE in idle/inactive and/or connected mode

- Applicability to PCell and/or SCell(s)

Agreement

For the adaptation mechanisms of SSB in time-domain, study further following mechanisms:

- Adaptation mechanism indicated or configured by gNB without UE trigger

- Adaptation triggered by UE (if any)

FFS: Details of associated signaling/indication/configuration

Agreement

For the adaptation mechanisms of PRACH in time-domain

- Support at least PRACH adaptation provided by gNB without UE trigger

  - FFS: PRACH adaptation with UE trigger

  - Note: UE trigger means UE requests adaptation of PRACH

- Study at least the following,

  - Dynamic signaling and/or semi-static signaling of PRACH adaptation

  - Adaptation of PRACH transmission according to certain condition

  - Applicability to idle/inactive and/or connected mode UEs

  - Which scenarios the adaptation mechanism is applicable to (e.g. cell with both legacy and Rel-19 UE, cell with only Rel-19 UEs)

## RAN1#116bis

Agreement

For indication of adaptation of SSB in time-domain,

- Support at least SSB adaptation provided by gNB without UE trigger

Agreement

For adaptation of PRACH in time-domain, support at least the following:

- Adaptation based on additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any)

  - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs

  - Configuration of additional PRACH resources is provided by semi-static signalling

    - FFS: details including whether there is overlap of additional PRACH resources and PRACH resources for legacy UEs

  - FFS: adaptation mechanism for additional PRACH resources

  - Note: No change to the existing PRACH configuration tables in 38.211

Agreement

For adaptation of PRACH in time-domain, support the following:

- SSB-RO mapping for the additional PRACH resources is separate from the SSB-RO mapping of the PRACH resources for legacy UEs (if any)

  - FFS: whether/how to handle SSB-RO mapping if the additional PRACH resources overlap in both time and frequency with the PRACH resources for legacy UEs

  - Note: SSB-RO mapping of the PRACH resources for legacy UEs is not impacted if Rel-19 UE uses these PRACH resources

  - FFS: SSB-RO mapping for the additional PRACH resources

Agreement

Support adaptation mechanisms of PRACH in time-domain for following:

- UE in idle/inactive mode

- UE in connected mode

Agreement

Adaptation mechanism(s) of SSB in time-domain is supported at least for one of the following scenario(s):

- For cell with both legacy UEs and Rel-19 NES-capable UEs

  - Rel-19 NES-capable UE’s PCell (Connected mode)

    - Study from the following options:

      - Option A1: adaptation for CD-SSB

      - Option A2: adaptation for SSB that is not CD-SSB

      - Option A3: adaptation for SSB not on sync raster

  - Rel-19 NES-capable UE’s SCell

    - Study from the following options:

      - Option B1: adaptation for CD-SSB

      - Option B2: adaptation for SSB that is not CD-SSB

      - Option B3: adaptation for SSB not on sync raster

  - FFS: Rel-19 NES-capable UE in idle/inactive mode

- Note: Impact to idle/inactive UEs shall be minimized

Agreement

For adaptation of PRACH in spatial domain,

- Study possibility of scenarios with non-uniform distribution of UEs in different beams

    - Note 6: Companies are encouraged to provide details on how they map UEs to different beams

- Study network energy savings gain achieved by non-uniform PRACH resource allocation across SSBs for scenarios with non-uniform distribution of UEs in different beams (if any),

    - Assume the following framework for network energy evaluation in FR1 and companies to report at least the below settings used in the evaluation/simulation

      - 20ms SSB period

      - 30kHz SCS, DDDSU TDD pattern

      - Setting A: SIB1 period (20ms/40ms/160ms)

      - Setting B1: Cell load (Empty/low/medium)

      - Setting B2: Traffic model

      - Setting C: SIB1 PDSCH time domain resource index in 38.214 Table 5.1.2.1.1-2

      - Setting D: CORESET0/SSB multiplexing pattern including controlResourceSetZero (index) in 38.213 Table 13-6, and searchSpaceZero (index) in 38.213 Table 13-11

      - Setting E1: PRACH configurations

        - (legacy) PRACH resources according to the following PRACH configuration for all transmitted SSBs

      - Case A1-1: PRACH configuration #5 (20ms)

      - Case A1-2: PRACH configuration #17 (10ms)

      - Case A2-1: PRACH configuration #0 (160ms)

        - (time-domain PRACH adaptation) Additional and legacy PRACH resources yielding total PRACH resources that are according to one of the following PRACH configuration for all transmitted SSBs

      - Case B1: PRACH configuration #17 (10ms)

      - Case B2: PRACH configuration #0 (160ms)

      - Companies to report details of assumed time domain adaptation mechanism

        - (spatial-domain PRACH adaptation) Additional and legacy PRACH resources yielding total PRACH resources that are according to one of the following PRACH configuration

      - Case C1: PRACH configuration #17 (10ms)

      - Case C2: PRACH configuration #0 (160ms)

      - Companies to report details of assumed spatial domain adaptation mechanism, including details of non-uniform PRACH resource allocation across SSBs

      - Setting F: Cat 1/Cat 2 BS as defined in TR38.864

      - Setting G1: Number of SSB beams: 4,8 SSBs in a SSB burst with SSB pattern case C

      - Note 1: Baseline to compare is Case C1 vs Case B1/A1-1/A1-2, Case C2 vs Case B2/A2-1

      - Note 2: It is up to company to report the SSB-RO mapping ratio and FDMed RO number, etc

      - Note 3: Other PRACH configuration index with different PRACH format other than format 0 is not precluded

      - Note 4: Other SSB/SIB1/RACH periodicity/PRACH resource/configuration assumptions are not precluded (up to companies to report)

    - Other frameworks for network energy evaluation are not precluded, e.g. including for FR2

## RAN1#117

Agreement

For the study of adaptation of PRACH in spatial domain, following network energy savings gains were reported by sources based on the evaluation framework agreed in RAN1#116bis:

- Two sources showed following NES gain for TDD, CAT1 BS power model, case C1 vs A1-1, zero load [R1-2404409, R1-2405107]

  - -4% ~ -45%

- Seven sources showed following NES gain for TDD, CAT1 BS power model, case C1 vs B1/A1-2, zero load [R1-2404225, R1-2404185, R1-2404334, R1-2404123, R1-2404562, R1-2405107, R1-2405163]

  - 0% ~ 31%

  - Note: Five sources assumed that case B1 has same PRACH resources as case A1-2. Remaining two sources evaluated only A1-2.

  - Note: Three sources showed NES gains 0% ~ 10% [R1-2404225, R1-2404185, R1-2404334]

- One source showed following NES gain for TDD, CAT1 BS power model, case C1 vs B1, zero load [R1-2404464]

  - 1.0%~8.8%

  - Note: The evaluation results provide the extra NES gain of spatial domain PRACH adaptation compared to time domain PRACH adaptation, where spatial domain and time domain PRACH adaptations are based on dynamic switching between PRACH resources according to two PRACH configuration indexes.

- One source showed following NES gain for TDD, CAT1 BS power model, case C1 vs B1, zero load [R1-2404626]

  - -48.41%~0%

  - Note: For B1, it was assumed that periodicity of PRACH resources can be adapted. For C1, it was assumed that periodicity of PRACH resources is not adapted and some ROs within a periodicity can be deactivated.

- One source showed following NES gain for TDD, CAT1 BS power model, for case C1 vs A1-2, zero load [R1-2404626]

  - 4.59%~38.04%

  - Note: For C1, it was assumed that periodicity of PRACH resources is not adapted and some ROs within a periodicity can be deactivated.

- Four sources showed following NES gain for TDD, CAT2 BS power model, case C1 vs B1/A1-2, zero load [R1-2404562, R1-2404225, R1-2403943, R1-2404626]

  - 0% ~ 3.5%

  - Note: Three sources assumed that case B1 has same PRACH resources as case A1-2. One source evaluated only A1-2.

- One source showed following NES gain for TDD, CAT2 BS power model, case C1 vs B1, zero load [R1-2404464]

  - 0%~0.2%

  - Note: The evaluation results provide the extra NES gain of spatial domain PRACH adaptation compared to time domain PRACH adaptation, where spatial domain and time domain PRACH adaptations are based on dynamic switching between PRACH resources according to two PRACH configuration indexes

- One source showed following NES gain for TDD, CAT2 BS power model, case C1 vs B1, zero load [R1-2404626]

  - -1.19%~0%

  - Note: For B1, it was assumed that periodicity of PRACH resources can be adapted. For C1, it was assumed that periodicity of PRACH resources is not adapted and some ROs within a periodicity can be deactivated.

- Two sources showed following NES gain for TDD, CAT1 or CAT2 BS power model, case C2 vs B2, zero load [R1-2403943, R1-2405107]

  - Less than 0.2%

- One source showed following NES gain for TDD, CAT1 BS power model, (C1 vs A1-2 with changed PRACH format), PRACH format A, 10ms PRACH periodicity, different loads [R1-2403980]

  - 13.7%/8.7%/4.9%/2.6% for zero/low/light/medium cell load

- One source showed following NES gain for TDD, CAT1 BS power model, (C1 vs B1 with changed PRACH format), PRACH format A, 10ms PRACH periodicity, different loads [R1-2403980]

  - 8.03%/5.1%/3.06%/1.74% for zero/low/light/medium cell load

- One source showed following NES gain for TDD, C1 vs B1/A1-2, different loads [R1-2404562]

  - 16%/4.78% for light/medium cell load for CAT1 BS power model

  - 0.65%/0.29% for light/medium cell load for CAT2 BS power model

- One source showed following NES gain for TDD, C1 vs B1, different loads [R1-2404626]

  - -18.57%~0%/-2.52%~0% for low /medium cell load for CAT1 BS power model

  - -0.81%~0%/-0.42%~0% for low /medium cell load for CAT2 BS power model

  - Note: For B1, it was assumed that periodicity of PRACH resources can be adapted. For C1, it was assumed that periodicity of PRACH resources is not adapted and some ROs within a periodicity can be deactivated.

- One source showed following NES gain for TDD, C1 vs A1-2, different loads [R1-2404626]

  - 3.67%~19.88%/2.29%~5.22% for low /medium cell load for CAT1 BS power model

  - 0.67%~1.75%/0.39%~0.91% for low /medium cell load for CAT2 BS power model

  - Note: For C1, it was assumed that periodicity of PRACH resources is not adapted and some ROs within a periodicity can be deactivated.

- One source showed NES gain for FDD, C1 vs B1, zero load [R1-2404464]

  - 1.4%~7% for CAT1 BS power model

  - 0%~0.3% for CAT2 BS power model

  - Note: The evaluation results provide the extra NES gain of spatial domain PRACH adaptation compared to time domain PRACH adaptation, where spatial domain and time domain PRACH adaptations are based on dynamic switching between PRACH resources according to two PRACH configuration indexes

- One source showed NES gain for FR2, CAT1 BS power model, spatial domain adaptation of PRACH configuration index 75 vs a time domain adaptation of PRACH configuration index 75, zero load [R1-2405163]

  - 4%~7%

- Note 1: About possibility of scenarios with non-uniform distribution of UEs in different beams

  - Several companies indicated (and three companies showed data/analysis) that there can be scenarios with non-uniform distribution of UEs in different beams.

  - Several companies mentioned that for non-uniform UE distribution, it can be addressed by gNB implementation e.g. by adjusting SSB beamwidth, etc. Several companies also mentioned that it is not clear how gNB can predict the distribution of UEs in different beams, especially for Idle/Inactive UEs.

- Note 2: Most sources that showed the NES gains (if any) for adaptation of PRACH in spatial domain compared to A1-2/B1 observed that the gain would be due to reduction in the number of overall ROs in time domain in their evaluations. Most of these companies only accounted for ROs in time domain.

- Note 3: The evaluation results assumed the non-uniform distribution of UE is static during the evaluation time period.

Conclusion

There is no consensus in RAN1 on the support of PRACH adaptation in spatial domain

Agreement

For adaptation of PRACH in time-domain, support at least the following case(s)

- Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs

- Case 2: time-domain overlap but no overlap in frequency domain between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs

- Case 3: additional PRACH resources for NES-capable UEs and legacy PRACH resources overlap neither in time nor frequency domains

- FFS: whether additional conditions are needed to support the above cases

- FFS: Additional case whether full/partial overlap in both time and frequency is allowed

- Above does not preclude discussion for the case where the configuration for additional PRACH resources contains legacy PRACH resources

Agreement

At least for the case where legacy ROs and additional ROs overlap in neither time nor frequency domain, for adaptation of PRACH in time-domain, the SSB-RO mapping rule for additional PRACH resources follows the legacy SSB-RO mapping rule.

- Mapping SS/PBCH block indexes to valid additional PRACH occasions provided by semi-static signalling follows the legacy mapping order for preamble/time resource/frequency/PRACH slot indexes.

  - Note: This mapping is not impacted by time domain PRACH adaptation

- Validation rules for the additional PRACH resources follow the legacy validation rules for PRACH resources configured for legacy UEs.

Agreement

For adaptation of SSB in time-domain, Option 1 is supported

- Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s)

- Note: Using Option 2 to realize Option 1 is not precluded

  - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active]

    - FFS: details of the differences between the two SSB configurations, e.g. two different periodicities

- FFS: Details including applicable scenarios

- FFS: Support of Cell DTX for connected mode UEs for SSB

Agreement

For adaptation of PRACH in time-domain, the additional PRACH resources are configured based on at least:

- a PRACH configuration index

- FFS: whether the PRACH configuration index is same and/or different from the PRACH configuration index for the legacy PRACH resources

Study further the following

- When the PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resource,

  - Additional parameter(s) for determining the additional PRACH resources e.g.

    - Scaled/adjusted PRACH configuration period

    - Additional timing offset

    - Adjusting the parameters (e.g., (x, y) value and slot number) of the PRACH configuration

    - Muting/masking ROs

- When the PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resource

  - Additional mechanisms (if any) for determining the additional PRACH resources e.g.

    - Muting/masking ROs (e.g. for the case when the PRACH configuration index for the additional PRACH resources contains legacy resources)

- Additional parameters to facilitate condensed/cluster RACH resources in time-domain (including whether needed)

Agreement

For the adaptation mechanism for additional PRACH resources, study further the following:

- Option 1: Higher layer signalling (with potential enhancements) based PRACH resource adaptation

- Option 2: L1-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: details

  - Strive to re-use existing DCI format(s)

- Option 3: Adaptation of PRACH transmission according to predefined condition(s)

  - FFS: details

- Option 4-rev1: L1-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: whether the subset of the additional PRACH resources is in RO level / SSB-to-RO mapping cycle level/PRACH association period level/PRACH association pattern period level for time-domain PRACH adaptation

  - Strive to re-use existing DCI format(s)

- Option 5: Enhanced cell DRX

## RAN1#118

Agreement

For adaptation of PRACH in time-domain, select at least one from the following alternatives for configuration of the additional PRACH resources

- Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources and

  - Discuss further additional mechanism(s) for determining the additional PRACH resources, e.g.

    - Opt 1-1: Scaled/adjusted PRACH configuration period

    - Opt 1-2: Adjusting the parameters (e.g., (x, y) value and slot number) of the PRACH configuration

    - Opt 1-3: Muting/masking ROs

    - Opt 1-4: additional timing offset(s)

- Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources,

  - Discuss further additional mechanism(s) for determining the additional PRACH resources, e.g.

    - Opt 2-1: Muting/masking ROs (e.g. for the case when the PRACH configuration index for the additional PRACH resources contains legacy resources)

    - Opt 2-2: Additional timing offset(s)

- FFS: Additional parameters to facilitate condensed/cluster RACH resources in time-domain (including whether needed)

- FFS: Additional frequency domain parameter(s) (e.g., freq. starting offset)

Agreement

Extend the RAN1#117 agreement on SSB-RO mapping rule for additional PRACH resources to Case 1

- Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs

RAN1#117 Agreement

At least for the case where legacy ROs and additional ROs overlap in neither time nor frequency domain, for adaptation of PRACH in time-domain, the SSB-RO mapping rule for additional PRACH resources follows the legacy SSB-RO mapping rule.

- Mapping SS/PBCH block indexes to valid additional PRACH occasions provided by semi-static signalling follows the legacy mapping order for preamble/time resource/frequency/PRACH slot indexes.

  - Note: This mapping is not impacted by time domain PRACH adaptation

- Validation rules for the additional PRACH resources follow the legacy validation rules for PRACH resources configured for legacy UEs.

RAN1#117 Agreement

At least for the case where legacy ROs and additional ROs overlap in neither time nor frequency domain, for adaptation of PRACH in time-domain, the SSB-RO mapping rule for additional PRACH resources follows the legacy SSB-RO mapping rule.

- Mapping SS/PBCH block indexes to valid additional PRACH occasions provided by semi-static signalling follows the legacy mapping order for preamble/time resource/frequency/PRACH slot indexes.

  - Note: This mapping is not impacted by time domain PRACH adaptation

- Validation rules for the additional PRACH resources follow the legacy validation rules for PRACH resources configured for legacy UEs.

Agreement

For SSB-RO mapping rule for additional PRACH resources for Case 2.

- Extend the RAN1#117 and RAN1#118 agreements on SSB-RO mapping

Agreement

For the adaptation mechanism for additional PRACH resources (for CONNECTED mode UE and IDLE/INACTIVE mode UE),

- At least DCI based adaptation is supported. No introduction of new DCI format.

Agreement

For adaptation mechanism(s) of SSB in time-domain,

- For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported

  - FFS: Adaptation for SSB that is not CD-SSB is supported (A2)

  - FFS: Adaptation for SSB not on sync raster is supported (A3)

- For Rel-19 NES-capable UE’s SCell

  - Adaptation of SSB configured for the SCell is supported for the following cases

    - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell

    - Adaptation for SSB that is not CD-SSB on sync raster (B2’)

    - Adaptation for SSB that is not CD-SSB not on sync raster (B3’)

Agreement

For DCI-based adaptation for additional PRACH resources,

- Select from the following DCI format(s) to carry the adaptation indication.

  - DCI format 1_0

  - DCI format 2_7

  - DCI format 2_9

- FFS: existing (P-RNTI, SI-RNTI, CellDTRX-RNTI, PEI-RNTI, C-RNTI) or new RNTI used for detecting the DCI format

Agreement

For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select from following options

- Option 1: One SSB burst periodicity is configured for the UE and UEs assumes SSB transmissions are not present during Cell DTX non-active period

- Option 2: UE assumes SSB transmission with different periodicities during Cell DTX non-active period and during Cell DTX active period

- Option 3: Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior) – no spec impact

Agreement

For DCI-based adaptation for additional PRACH resources, select only from the following alternatives

- Alt 1: (PRACH resource configuration level) DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: details

- Alt 2: (subset of PRACH resource level) DCI-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: whether the subset of the additional PRACH resources is in

    - Alt 2-1: RO level per SSB

    - Alt 2-2: SSB-to-RO mapping cycle level

    - Alt 2-3: PRACH association period level

    - Alt 2-4: PRACH association pattern period level

    - Alt 2-5: SFN level

- Alt 3: DCI-based Enhanced/new Cell DRX to indicate whether the enhanced/new Cell DRX is activated or deactivated.

  - If activated, the additional configured PRACH provided by semi-static signalling within non-active period are not available.

  - FFS: whether Alt 1 and/or Alt 2 can be applied to the active period

  - FFS: details

## RAN1#118bis

Agreement

For adaptation of PRACH in time-domain, the same PRACH preamble format is used for the additional RACH resources and legacy PRACH resources.

Agreement

For adaptation of PRACH in time-domain, support both of the following

- Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources

- Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources

- FFS: Additional details

Working Assumption

For DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode.

- P-RNTI is used

Agreement

For adaptation of PRACH in time-domain, the frequency domain resources for the additional PRACH resources and legacy PRACH resources can be same or different

- FFS: applicable case(s) (i.e. case(s) from the RAN1#117 agreement).

- Discuss further following options for signaling

- Option 1: at least the following parameter(s) can be configured separately for the additional PRACH resources.

  - msg1-FrequencyStart at least for 4-step RACH

  - FFS: other applicable legacy frequency domain parameter(s)

- Option 2: Offset to legacy frequency domain parameter(s) are configured for the additional PRACH resources

  - Note: Offset to legacy frequency domain parameter(s) is a new parameter

  - FFS: applicable legacy frequency domain parameter(s)

Conclusion

There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19

Agreement

For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select Option 3, i.e. Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior).

- No spec impact

Agreement

For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain,

- For Alt 1 (same PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s),

  - Opt 1-2a: up to N1 additional value(s) of (x, y)

    - FFS: value of N1(>=1)

  - Opt 1-2b: up to N2 additional value(s) of y

    - FFS: value of N2 (>=1)

  - Opt 1-3: Muting/masking ROs

  - Opt 1-4:

    - up to N4_1 additional timing offset(s) at frame-level

    - up to N4_2 additional timing offset(s) at slot-level

    - FFS: values of N4_1(>=1) and N4_2(>=1)

  - Opt 1-5: No additional mechanism is selected.

  - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211)

- For Alt 2 (different PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s),

  - Opt 1-2a: up to N1 additional value(s) of (x, y)

    - FFS: value of N1(>=1)

  - Opt 1-2b: up to N2 additional value(s) of y

    - FFS: value of N2 (>=1)

  - Opt 1-3: Muting/masking ROs

  - Opt 1-4:

    - up to N4_1 additional timing offset(s) at frame-level

    - up to N4_2 additional timing offset(s) at slot-level

    - FFS: values of N4_1(>=1) and N4_2(>=1)

  - Opt 1-5: No additional mechanism is selected.

  - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211)