3GPP TSG-RAN WG1 #119                                                 	       R1-2410880

Orlando, US, November 18th- 22nd, 2024

Title 	:  Summary#4 on the channel model adaptation and extension

Source 	:  Moderator (ZTE)

Agenda item	:  9.8.2

Document for	:  Discussion and Decision

- Introduction

Based on the following relevant scope of the SI for 7-24 GHz channel model,

- Adapt/extend as necessary the channel model of TR38.901 at least for 7-24 GHz, including at least the following aspects for applicable scenarios: - Near-field propagation (with consideration being given to consistency between near-field and far-field) - Spatial non-stationarity

this contribution summarizes the proposals in companies’ input under the AI 9.8.2 with following aspects:

- Channel model for Near field propagation

- Channel model for Spatial non-stationarity

- Other aspects

The details of each part are provided in corresponding section below and the agreements achieved in RAN1#116-bis~RAN1#118-bis are listed in Appendix-A for reference.

- Views on the near-field propagation

  - Direct path

In last RAN1#118-bis meeting, following agreements have been achieved regarding the details to model the antenna element-wise channel parameters of direct path between BS and UE:

Agreement For near-field channel, if necessary, to model the following antenna element-wise channel parameters of direct path between TRP and UE, - Angular domain parameters - FFS: Doppler shift, Delay with Option-2 “Determined by the antenna element locations of both TRP and UE”. Note: Angular domain parameters can be optionally modelled. Agreement For near-field channel, if necessary, to model the following antenna element-wise channel parameter of direct path between TRP and UE, - Delay (can be optionally modelled) - Working Assumption - The existing model in Clause 7.6.2 of TR 38.901 is used to model the delay. - with Option-2 “Determined by the antenna element locations of both TRP and UE”.

### 1.1.1. Doppler shift (Closed)

1.1.1.1. Company view (Round-1)

According to the discussion in last meeting, there has a FFS on whether the Doppler shift parameters shall be modeled as antenna element-wise parameters for direct paths, and following are summarized according to companies’ inputs :

- [LGE, ZTE, NVIDIA, Apple, vivo, Intel, CATT, CEWiT] propose that the Doppler shift parameters can be optionally modeled. More specifically,

[LGE, ZTE] highlight that the difference between the Doppler shift term of element-wise model and antenna-wise model can be significant at least in some use cases such as positioning/sensing, UAV, and cases with high UT speed.

[LGE, ZTE, Apple,vivo, CATT] mention that since element-wise angular domain parameters can be optionally modeled, additional burden for the generating element-wise Doppler shift parameter might be marginal.

- [Huawei, HiSilicon, Lenovo, OPPO, Qualcomm] propose that no need to model the antenna element-wise Doppler shift parameters. Specifically,

[Lenovo] mentions the near-field region is expected to model crowded areas, where most devices are handheld devices, in which Doppler shift values are relatively small. And Doppler shift values become notable for high-speed train scenarios and/or highways and vehicle roads, wherein for these scenarios it is reasonable to model the cells to be far enough from the main road to cover a larger area.

[Qualcomm] provides the simulation results to show that under the different configuration of values of velocity and observation time that result in a displacement of no more than 1 meter, the impact of the Doppler term on the near-field channel model is observed to be marginal and can be deprioritized.

[OPPO] provides the analysis to demonstrate that the Doppler shifts discrepancy between far/near-field channel is only <20Hz and can be viewed as negligible.

- [Samsung] proposes that RAN1 shall discuss whether updates for Doppler shift is needed.

[Samsung] highlights that even though the wavefront is non-planar, the small inter-element spacing means that the Doppler shift introduced due to distance differences is negligible.

- Additionally, some companies [ZTE, vivo, CATT, CEWiT, Intel] share the detailed formula of antenna element-wise Doppler shift parameters in the channel coefficient. Specifically,

- [ZTE, vivo, CATT, CEWiT] propose the spherical unit vector in the Doppler shift part can be updated to the spherical unit vector of corresponding antenna element pair, e.g., expj2πrrx,s,u,LOST∙vλ0t, where rrx,LOS,u,s is the spherical unit vector with azimuth arrival angle ϕLOS,AOA,u,s and elevation arrival angle θLOS,ZOA,u,s of direct path between transmit antenna element s and receive antenna element u.

- [Intel] proposes that the impact of Doppler shift in direct path can be given by exp−2πλ×ru,sru,s⋅vt

From FL’s perspective, since the antenna element-wise angular domain parameters for direct path can be optionally modeled, the complexity to model antenna element-wise Doppler shift parameter is limited. Additionally, considering the channel model will also be the basis for all potential technical study of the new use cases, e.g., positioning or sensing use cases, the antenna element-wise Doppler shift parameters can also be optionally modeled for the direct path.

- Thus, following is proposed from FL’s perspective:

Proposal 1-1-1-1:

- For near-field channel, the Doppler shift of direct path between TRP and UE is optionally modeled as antenna element-wise channel parameters with following formula:

- expj2πrrx,LOS,u,sT∙vλ0t

where rrx,LOS,u,s is the spherical unit vector with azimuth arrival angle ϕLOS,AOA,u,s and elevation arrival angle θLOS,ZOA,u,s of direct path between transmit antenna element s and receive antenna element u, and v is the speed of UE.

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| InterDigital | Our first preference to stay with the existing Doppler formulation in TR 38.901, but we are fine with above proposal, i.e., the element-to-element wise Doppler shift parameter. |
| Apple | Fine |
| Huawei, HiSilicon | Generally fine. |
| vivo | Support |
| Samsung | If companies have strong view that the impact of Doppler shift is significant, we are fine with the proposal. |
| QC | Needs more discussion. Don’t think this is critical. We need to discuss relevant velocities of interest in the near-field context. We also need to discuss limits on UE displacement --- velocity x time product cannot be large. |
| LGE | Support, but prefer minor modification (reuse expression in current spec) and v is the speed of UE. and v is UT velocity vector with speed v, travel azimuth angle ϕv, elevation angle θv. |
| Intel | Support |
| CATT | Support |
| Lenovo/ MotM | Agree with InterDigital and QC, not a priority but certainly needs to stay optional if supported |
| Sony | We are supportive in principle. |

1.1.1.2. Summary of the discussion

The following conclusion is made and no further discussion is needed for this aspect:

Conclusion For near-field channel, no changes are expected on following parameter for direct path between TRP and UE: - Doppler shift

### 1.1.2. Delay

1.1.2.1. Company view (Round-1)

As for the delay parameters of direct path, there has a working assumption that “ the existing model in Clause 7.6.2 of TR 38.901 is used to model the element-wise delay”. And following views are summarized according to companies’ inputs:

- [Apple, Qualcomm] propose to confirm the working assumption. Specifically,

[Qualcomm] provides the simulation results to demonstrate that the differences in path lengths using planar wave approximation and actual antenna coordinates are of the order of a few centimeters even for UEs that are very close to the gNB. And such minor differences are unlikely to matter for systems operating using 100s of MHz.

- [ZTE, vivo, CEWiT, Intel] propose to revise the working assumption. Specifically,

[ZTE, CEWiT] proposes the existing methods in section 7.6.2 of TR38.901 is based on far-field assumption, and [ZTE] provides the simulation results to demonstrate that employing the existing methodology to calculate delay results in discrepancies when compared to methods based on the actual distance between each antenna elements pair.

- [vivo, Intel] highlights that only the NLOS ray is considered for the model in Clause 7.6.2 of TR 38.901, hence it cannot be used to model antenna element-wise delay of direct path between TRP and UE.

- [Lenovo] proposes that further clarification is needed on the working assumption regarding the applicability of the additional modeling component in Clause 7.6.2 of TR 38.901 to the near-field channel with antenna element-wise delay modeling.

Additionally, [Samsung] proposes that RAN1 can consider to add the absolute delay term that obtained based on 3D distance and clause 7.6.2 in TR 38.901 in the channel coefficients of LOS. While [Intel] proposes that no additional delay needs to be added to the channel for the direct path, since this is fully captured in the phase of the direct path channel as e−j2πλru,s.

Besides, when considering to revise the working assumption, [ZTE, vivo] further propose the element-wise delay shall be calculated directly based on the actual distance between Rx antenna element u and Tx antenna element s, and share the detailed formula of the antenna element-wise delay in channel coefficient as following:

- [ZTE] proposes that the delay parameters can be updated to:

- δτ−τ1,u,s,

- τ1,u,s=τ1−d3D−|ru,s|c

- where ru,s refers to the vector determined by the location of receive antenna element u and transmit antenna element s, d3D refers to the 3D distance between reference antenna at TRP and UE side.

- [vivo] proposes that the delay part in the channel coefficient can be updated and calculated by:

- τLOS,u,s=ru,s−minru,s/c

- Thus, following is proposed from FL’s perspective according to above summary:

Proposal 1-1-2-1:

- For near-field channel, one of following options is considered to calculate the antenna element-wise delay of direct path between TRP and UE:

- Option-1: The following formula is used to calculate the antenna element-wise delay of direct path:

- τ1,u,s=τ1−d3D−|ru,s|c

- where ru,s refers to the vector determined by the location of receive antenna element u and transmit antenna element s, d3D refers to the 3D distance between reference antenna at TRP and UE side.

- Note: The above formula is applied with the same principle as existing model in Clause 7.6.2 of TR 38.901, and the channel response of LOS ray between Rx antenna u and Tx antenna s at delay τ at time t is given by:

Hu,s,1LOSt;τ=Frx,u,θθLOS,ZOA,u,s,ϕLOS,AOA,u,sFrx,u,ϕθLOS,ZOA,u,s,ϕLOS,AOA,u,sT100−1Ftx,s,θθLOS,ZOD,u,s,ϕLOS,AOD,u,sFtx,s,ϕθLOS,ZOD,u,s,ϕLOS,AOD,u,s

exp⁡(−j2πd3Dλf)exp−j2π|ru,s|−d3Dλfexpj2πrrx,LOS,u,sT∙vλ0tδτ−τ1,u,s

- Option-2: The antenna element-wise delay of direct path is directly modeled based on the distance of direct path between antenna elements at TRP and UE side.

- Following formula is adopted to model the antenna element-wise delay in the channel coefficient as:

- δτ−τ1,u,s,  τ1,u,s=τ1−d3D−|ru,s|c

- where ru,s refers to the vector determined by the location of receive antenna element u and transmit antenna element s, d3D refers to the 3D distance between reference antenna at TRP and UE side.

- Option-3: Confirm the following working assumption made in RAN1#118BIS:

- “The existing model in Clause 7.6.2 of TR 38.901 is used to model the delay.”

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| InterDigital | Support Option 3, we think the delay difference cause by different antenna elements can be ignore with the agreed Rel-19 array aperture size. |
| Apple | It is unclear the difference between Option 1 and Option 2. Can FL clarify it? |
| Huawei, HiSilicon | Support Option-2. |
| vivo | We don’t see the difference between Option-1 and Option-2. In Option-1 and Option-2, without the minimum delay operation, the τ1,u,s could be minus, and therefore, the equation τ1,u,s=ru,s−minru,s/c is necessary. |
| Samsung | One question for τ1. In here, is τ1 normalized one based on (7.5-2) in TR38.901? |
| QC | Prefer Option 3. There is almost no marginal benefit of going beyond this. |
| LGE | Although we can’t figure out what is big different between Option-1 and Option-2 (I guess FL intended Option-2 for vivo’s formula), okay with Option-1 for unification with existing model of TR 38.901 |
| Intel | The difference between Option-1 and Option-2 is unclear. Also, Option-1 is mixing the Angular domain parameters with the delay. We prefer to discuss the two issues separately. |
| CATT | Same view as QC. We need to justify with satisfactory benefit. |
| Lenovo/ MotM | Similar to most companies, more clarity is needed on Option 1 and Option 2 discrepancy |
| Sony | We cannot figure out what is the difference between option 1 and option 2. Also, in option 1 d3D is defined as “3D distance between reference antenna at TRP and UE side.” However, in 38.901 d3D is defined as “the distance between the Tx and the Rx.” We suggest changing the notation to avoid a clash if/when merging with 38.901. |

### 1.1.3. Angular domain parameters (Closed)

1.1.3.1. Company view (Round-1)

- In last meeting, the angular domain parameters have been agreed to be optionally modeled as antenna element-wise channel parameters for the direct path. In this meeting, as for the detailed formula of angular domain parameters in the channel coefficient, [Huawei, HiSilicon, ZTE, Samsung, Apple,vivo, Intel, Qualcomm, CEWiT] propose that angular parameters in the field pattern of transmit antenna element and receive antenna elements can be updated to the antenna element-wise angular domain parameters, i.e.,

- Frx,u,θθLOS,ZOA,u,s,ϕLOS,AOA,u,sFrx,u,ϕθLOS,ZOA,u,s,ϕLOS,AOA,u,sT100−1Ftx,s,θθLOS,ZOD,u,s,ϕLOS,AOD,u,sFtx,s,ϕθLOS,ZOD,u,s,ϕLOS,AOD,u,s

- Thus, following is proposed from FL’s perspective,

###### Proposal 1-1-3-1:

For near-field channel, the following formula is adopted to model the angular domain parameters of direct path between TRP and UE as antenna element-wise channel parameter:

- Frx,u,θθLOS,ZOA,u,s,ϕLOS,AOA,u,sFrx,u,ϕθLOS,ZOA,u,s,ϕLOS,AOA,u,sT100−1Ftx,s,θθLOS,ZOD,u,s,ϕLOS,AOD,u,sFtx,s,ϕθLOS,ZOD,u,s,ϕLOS,AOD,u,s

- Where θLOS,ZOA,u,s, ϕLOS,AOA,u,s, θLOS,ZOD,u,s, ϕLOS,AOD,u,s are the respective antenna element-wise elevation arrival angles, azimuth arrival angles, elevation departure angles and azimuth departure angles of LoS path between the transmit antenna element s and receive antenna element u.

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| InterDigital | We are fine with this proposal for LOS direct path. |
| Apple | Agree |
| Huawei, HiSilicon | Support. |
| vivo | Support |
| Samsung | Support |
| QC | Should be okay. |
| LGE | Support |
| Intel | Support |
| Sony | Support. |

1.1.3.2. Summary of the discussion

In the online discussion, the following agreement is achieved.

Agreement For near-field channel, the following formula is adopted to model the angular domain parameters of direct path between TRP and UE as antenna element-wise channel parameter: Frx,u,θθLOS,ZOA,u,s,ϕLOS,AOA,u,sFrx,u,ϕθLOS,ZOA,u,s,ϕLOS,AOA,u,sT100−1Ftx,s,θθLOS,ZOD,u,s,ϕLOS,AOD,u,sFtx,s,ϕθLOS,ZOD,u,s,ϕLOS,AOD,u,s Where , , , are the respective antenna element-wise elevation arrival angles, azimuth arrival angles, elevation departure angles and azimuth departure angles of LoS path between the transmit antenna element s and receive antenna element u.

  - Non-direct path

In last RAN1#118-bis meeting, following agreements have been achieved regarding the antenna element-wise channel parameters of non-direct path between BS and UE:

Conclusion For near-field channel, no changes are expected on following parameters of the non-direct path between TRP and UE: - Amplitude Agreement For near-field channel, if necessary, the following parameters of the non-direct path between TRP and UE should be modeled as antenna element-wise parameter. - Angular domain parameters, Delay (both can be optionally modelled) - Working Assumption - The existing model in Clause 7.6.2 of TR 38.901 is used to model the delay. - FFS: Doppler shift Agreement For near-field channel, if necessary, the antenna element-wise channel parameters of non-direct path between TRP and UE is modelled by: - For the non-direct paths, the antenna element-wise channel parameters are derived based on at least the distance between the BS/UE and a point associated with cluster, i.e., - d1 is used to determine the 1st point associated with cluster to calculate the parameter at least for BS side; - FFS: d2 is used to determine the 2nd point associated with cluster to calculate the parameter at least for UE side; - FFS: whether one point is sufficient - FFS: the details to generate the d1, d2,e.g., if the d2 is needed, both d1 and d2 will be generated pair wise. - FFS: The ratio of non-direct path which show the phenomenon with spherical wavefront - FFS: The determination of the point associated with cluster is conducted in Cluster-level

    - Methodology

In last meeting, there has an agreement related to the methodology of antenna element-wise channel parameters for non-direct path with some pending issues. Some companies share their views on these pending issues and summarized as following.

1.2.1.1. 2nd point associated with cluster (Closed)

1.2.1.1.1. Company view (Round-1)

Regarding on whether to consider the d2 that used to determine the 2nd point associated with cluster to calculate the parameter at least for UE side. Following views are summarized according companies’ inputs:

- [Huawei, HiSilicon, ZTE, Ericsson, InterDigital, Apple, vivo, Intel, Lenovo, Fujitsu, BUPT, CMCC, MediaTek, CEWiT] propose to consider the 2nd point associated with cluster. More specifically,

[InterDigital] highlights that a propagation typically experiences more than one reflection, therefore, it may be better to consider both d1 and d2 to support more flexibility and accuracy in modelling of the NF channel.

[vivo] provides the simulation results to demonstrates the change of departure angle due to the assumption of single refection point is significant, thus one point is not sufficient to model the antenna element-wise channel parameters of non-direct path between TRP and UE.

[CEWiT] highlights that classification of single bounce cluster / single point or multi-bounce cluster / two or more points can be done with respect to absolute cluster delay threshold and projected departure and arrival angle intersection point.

[ZTE, Ericsson] mention that such distance d2 can be optionally considered, e.g., when the antenna elements at UE side is limited, there is no need to calculate the element-wise channel parameters at UE side [ZTE].

- [CATT, OPPO, SHARP] propose that no need to consider the 2nd point associated with cluster. Specifically,

[CATT] highlights that supporting only one point using the existing spatial consistency procedure A of TR 38.901 has less specs impact and lower computational complexity.

[OPPO] highlights that the antenna array at UE side is very restricted by the device size, antenna architecture and cost, and the near field condition for non-direct paths occurs when the clusters are near gNB antenna array, which means they are usually far from UE antenna array and would result in far field like channel.

From FL’s perspective, although the aperture size at UE side may not be as large as BS side, there may also have some local scatterers that are close to UEs and be affected by the near-field effect, thus FL suggests to further consider the impact of spherical wavefront at UE side, then it’s better to additionally consider the d2 at UE side in the near-field channel model.

Thus, following is proposed from FL’s perspective:

- Proposal 1-2-1-1-1:

- For near-field channel, for the non-direct paths, the impact of spherical wavefront is considered at UE side with following:

- d2 is used to determine the 2nd point associated with cluster to calculate the parameters at least for UE side.

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| InterDigital | Consideration of both d1 and d2 may potentially offer more flexibility and accuracy in modelling the NF channel. However, we are also fine with the single bounce case where only one d1 is modeled. |
| Apple | Fine |
| Huawei, HiSilicon | Support. |
| vivo | Support |
| Samsung | Support |
| QC | Suggest postponing this for now. Lets revisit this question after we have better understanding of the modelling aspects. It is clearly not needed for specular reflections. |
| Intel | Support |
| CATT | No. We don’t agree to model d2. |
| Lenovo/ MotM | Our understanding is that d1 and d2 both correspond to a single scatterer where d1 is the scatterer distance to gNB and d2 is the scatterer distance to UE. If this is not common understanding then more discussion is needed. |
| Sony | Support. If companies have issues, the value d2=∞ can be introduced as an optional value, which is equivalent to consider all clusters as far-field cluster with respect to the UE side. |

1.2.1.1.2. Summary of the discussion

The following working assumption is achieved to conclude this issue and further details are expected to be concluded in RAN1#120.

Working Assumption For near-field channel, for the non-direct paths, the impact of spherical wavefront is optionally considered at UE side.

1.2.1.2. Generation of distance d1 (Closed)

1.2.1.2.1. Company view (Round-1)

As for the details to generate the d1, according to companies’ inputs, following issues shall be considered.

- Sub-issue#1: How to generate the d1

According to companies’ inputs, following views are summarized regarding on how to generate the d1:

- [Huawei, HiSilicon, Samsung,vivo] propose to determine the d1 by solving an optimization problem. More specifically,

[Huawei, HiSilicon,vivo] proposes that a multi-bounce propagation model can be assumed, then the d1 and d2 can be obtained by solving an optimization problem of minimizing the propagation distance between the first and last bounce scatterer. And the multi-bounce assumption falls to single-bounce when there is no solution for the optimization problem.

[Samsung] proposes that if considering the single-bounce, it’s possible to compute the distance by utilizing the triangular relation among BS, UE and a cluster point, and RAN1 shall discuss which approach is appropriate between single bounce and multi-bounce based method.

- [ZTE, Ericsson, Lenovo, OPPO, BUPT, CMCC, MediaTek, CEWiT, InterDigital] propose to determine the d1 according to a specific distribution. More specifically.

[Ericsson] proposes that the distance between the BS and a point associated with cluster can be determined by        d1 =kgNB,n∙Ln, where Ln is the path length that calculated by the absolute delay according to section 7.6.9 of TR39.901, and for the two strongest clusters, set the kgNB,n=1, while for other clusters, kgNB,n is a random variable from a Beta distribution with parameters α=3.81,β=1.62. While d2=kUE,n∙Ln , where kUE,n=1 if kgNB,n=1, or kUE,n=1−kgNB,n if kgNB,n<1.

[ZTE, BUPT, CMCC] propose that the d1 and d2 are randomly generated according to a log-normal distribution, and the upper bound shall be restricted by the absolute delay[ZTE]. And [ZTE] further proposes that for UMi scenarios, d1~Log-N(4.5, 0.72), d2~Log-N(3.6 , 1.22), while for Indoor Office scenarios, d1~Log-N(0.5, 1.42), d2~Log-N(1.5, 0.82).

[InterDigital] proposes that  d1 is generated from a distribution function (e.g. a uniform distribution) and d2 is generated from the cluster delay τn and the d1.

[Lenovo] proposes that the d1 and d2 are respectively generated in a pairwise manner via a stochastic method, conditioned that d1,m+d2,m>dD for each cluster m, where dD is the 3D Euclidean distance between the gNB and the UE.

[OPPO] highlights that to model d1 based on a distribution ( Gaussian or Gamma distribution ) related to the generated delay and power. And a minimum allowed distance is required to avoid generating too near channel components. A maximum allowed distance is also required to ensure the first propagation time does not exceed the generated delay.

[CEWiT] highlights that determination of d1 and d2 for the non-direct paths can be drawn from a uniform random distribution, and should be performed in tandem with the Near-field Boundary.

- [Intel] proposes that the vector d1,n which connects the center of GCS ate BS to the 1st point associated with cluster n for the BS side and the vector d2,n which connects the center of GCS ate UE to the 2nd point associated with cluster n for the UE side are given by

d1,n≜d3D2rtx,LOST⋅rtx,nrtx,n,

d2,n≜d3D2rrx,LOST⋅rrx,nrrx,n,

- [SHARP] proposes that the distance d1 is the absolute time-of-flight of the non-direct paths when considering the ‘Reflection model’.

For the optimization problem-based way to generate d1, [Qualcomm] proposes that cluster generation based on Two-Bounce Cluster Model (TBCM) using the optimization algorithm may not always guarantee a feasible cluster location without requiring changes in the generation of departure/arrival angles. In most cases it may result in a cluster location that is too close to the gNB or UE. This can lead to near-field impact extending to UEs that are far away from the gNB.

Additionally, [Apple] proposes that d1 and d2 depends on the cluster delay generated according to existing step 5 of TR 38.901. And [NVIDIA] proposes that cluster location is directly dropped and generated in a deterministic manner, while [CATT] proposes that the existing spatial consistency procedure A of TR38.901 can be used to calculate the antenna element-wise channel parameters for non-direct paths.

Besides, [Huawei, HiSilicon, ZTE, Ericsson] propose that when generating the d1, d2, the absolute delay should be computed based on the generated relative delay. And [Huawei, HiSilicon, ZTE, Ericsson] highlight that it can be referred to Section 7.6.9 in TR 38.901, where the distribution of the offset ∆τ should be given for all the scenarios [Huawei, HiSilicon, ZTE].

- Sub-issue#2：Ray-level or cluster-level determination

After determining how to generate the d1, there has an remaining issue on whether the determination of the point associated with cluster shall be conducted in cluster-level or ray-level, and following are summarized according to companies’ inputs:

- [Ericsson, ZTE, vivo, Intel, MediaTek] propose to determine the point associated with cluster in cluster-level. More specifically,

[vivo] highlights that the ray-level determination method has higher accuracy but with higher complexity. And since it is agreed that the visible probability or visibility region is modeled per cluster, it is better to be aligned with each other.

[ZTE, vivo] highlights that for the ray-level determination, it may incur that the positions of the scatterers corresponding to different rays associated with the same cluster may be very different.

- [Huawei, HiSilicon] proposes to determine the point associated with cluster in ray-level. More specifically,

[Huawei, HiSilicon] highlights that determine the point associated with cluster per cluster will equivalently eliminate the concept of ray by reusing the same set of channel parameters (e.g., phase, angular domain parameters) for the rays within a cluster, which obviously violates the principle of current TR 38.901.

Additionally, some companies further mentions the detailed ways to obtain the antenna element-wise channel parameters of non-direct paths. Specifically,

- If determined in cluster-level：

[Intel] proposes that the antenna-element specific ray angles θn,m,ZOD,s, ϕn,m,AOD,s, θn,m,ZOA,u, and ϕn,m,AOA,u are obtained by applying the ray offset angles of Table 7.5-3 of TR 38.901 to cluster angles θn,ZOA,u, ϕn,AOA,u, θn,ZOD,s, and ϕn,AOD,s, using equations (7.5-13), (7.5-18), and (7.5-20) of TR 38.901.

[ZTE] proposes that the phase, angular domain parameters, delay and Doppler shift shall be obtained in ray-wise according to the cluster-level d1 and d2, the ray-wise spherical unit vector generated according to existing procedure of TR38.901 as the procedure listed in following figure:

- If determined in ray-level:

[Huawei, HiSilicon] proposes that the ray-wise antenna element-wise delay, phase, angular domain parameters can be directly calculated according to the ray-level d1 and d2, the ray-wise spherical unit vector generated according to existing procedure of TR38.901.

From FL’s perspective, following can be observed according to above summary:

- For the sub-issue#1: It seems that different companies share the different views on the methods to generate the d1.

- For the sub-issue#2: For the ray-level determination, the positions of the scatterers corresponding to different rays associated with the same cluster may be very different. Additionally, consider the complexity, it’s preferred to determine the point associated with cluster in cluster-level.

Thus, following is proposed from FL’s perspective:

- Proposal 1-2-1-2-1:

- For near-field channel, for the non-direct paths, the distance d1 between the BS and the 1st point associated with cluster is generated in cluster level with one of following options:

- Option-1: The distance d1 is generated directly following a specific distribution.

- FFS: the distribution of d1, e.g., log-normal distribution.

- Note: the generation of d1 shall consider the restriction of absolute cluster delay generated according to the procedure defined in Section 7.6.9 in TR 38.901.

- Option-2: The distance d1 is generated by a scaling factor multiplied by the absolute distance corresponding to the absolute cluster delay, where the scaling factor is generated from a specific distribution.

- FFS: distribution of scaling factor.

- The absolute cluster delay is generated according to the procedure defined in Section 7.6.9 in TR 38.901

- Option-3: The distance d1 is generated by resolving an optimization algorithm.

- FFS: The details.

- Note 1: If d2 is agreed to be considered, the generation of d2 can follow the same principle with the generation of d1.

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| InterDigital | We prefer to use a specific distribution to generate d1,n and d2,n i.e., Option-1. |
| Apple | For the distance d1, we think cluster delays are generated in Step 5 of Section 7.5, and can be used in Option 1 and Option 2. |
| Huawei, HiSilicon | Fine with the proposal and support option 3. |
| vivo | We should discuss whether d2 is necessary (i.e., Proposal 1-2-1-1-1) prior to this proposal. This is because d1 in Option-3 cannot be solely solved by an optimization algorithm, and it must be couple with d2. Therefore, Option-3 should be modified as - Option-3: The distance d1 paired with the distance d2 is generated by resolving an optimization algorithm. FFS: The details. |
| Samsung | Support Option 3. We consider that the derivation approach of distance is more intuitive. For distribution method, we are not sure how do we guarantee the relationship between generated distance and each cluster. |
| QC | We need to make it clear that this is only for paths that do not correspond to specular reflection. We expect the dominant indirect paths to correspond to specular reflections. Might be best to split the discussion into two parts; one for specular reflections and the other for non-specular reflections. We can then decide what fraction of paths are specular reflections. |
| CATT | Option1/2 |
| BUPT | Support Option 1. |
| Ericsson | ###### From today’s online discussion, it seems some companies have concern that neither of Option 1 nor Option 2 can model specular reflections. To address this issue, we propose the following updates to the proposal: Proposal 1-2-1-2-1: For near-field channel, for the non-direct paths, the distance d1 between the BS and the 1st point associated with cluster is generated in cluster level with one of following options: - Option-1: The distance d1 is generated directly following a specific distribution as follows: - For the k1 strongest clusters, d1 = speed of light times the absolute cluster delay - For other k2 clusters, the distance d1 is less than the speed of light times the absolute delay of the cluster and generated following a specific distribution - FFS: the specific distribution of d1, e.g., log-normal distribution for UMi - FFS: The values of k1 and k2, e.g., k1=2 and k2=18 for UMa - Note: the generation of d1 shall consider the restriction of absolute cluster delay generated according to the procedure defined in Section 7.6.9 in TR 38.901. - Option-2: The distance d1 is generated by a scaling factor multiplied by the absolute distance corresponding to the absolute cluster delay, where the scaling factor is generated from a specific distribution as follows: - For the k1 strongest clusters, d1 = speed of light times the absolute delay of the cluster (i.e., the scaling factor is 1) - For other k2 clusters, the distance d1 is less than the speed of light times the absolute delay of the cluster and generated following a specific distribution (i.e., the scaling factor is <1) - FFS: distribution of scaling factor, e.g., Beta distribution for UMa - FFS: The values of k1 and k2, e.g., k1=2 and k2=18 for UMa - The absolute cluster delay is generated according to the procedure defined in Section 7.6.9 in TR 38.901 - Option-3: The distance d1 is generated by resolving an optimization algorithm. - FFS: The details. Note 1: If d2 is agreed to be considered, the generation of d2 can follow the same principle with the generation of d1. Note: In our tdoc, we show (based on ray tracing simulations) that about 10% of paths in a UMa scenario are due to specular reflections and that these paths are typically associated with higher gain. |
| Sony | ###### We prefer Ericsson’s rewording. Regarding Option 3, we do not think it stands on the same footing as option 1 and 2. It would be good to add “FFS: the mapping of the k1 clusters to the N generated clusters” to options 1 and 2. |

1.2.1.2.2 Summary of the discussion

After online discussion, the following agreement is made with understanding that down-selection is expected in future meeting.

Agreement For near-field channel, for the non-direct paths, the distance between the BS and the 1st point associated with cluster is generated with one of following options that to be selected in RAN1#120: - Option-1: The distance is generated directly following a specific distribution. - For the clusters, = speed of light times the absolute delay of the cluster - For other clusters, the distance is equal or less than the speed of light times the absolute delay of the cluster and generated following a specific distribution - FFS: The specific distribution, e.g., log-normal distribution for UMi - FFS: The values of and - The generation of absolute delay of the cluster is according to the procedure defined in Section 7.6.9 in TR 38.901 - FFS: The value for UMi, UMa, InH-Office. - Option-2: The distance is generated by a scaling factor multiplied by the absolute distance corresponding to the absolute delay of the cluster, where the scaling factor is generated from a specific distribution. - For the clusters, = speed of light times the absolute delay of the cluster (i.e., the scaling factor is 1) - For other clusters, the distance is equal or less than the speed of light times the absolute delay of the cluster and the scaling factor generated following a specific distribution (i.e., the scaling factor is equal or less than 1) - FFS: Distribution of scaling factor, e.g., Beta distribution for UMa - FFS: The values of and - The generation of absolute delay of the cluster is according to the procedure defined in Section 7.6.9 in TR 38.901 - FFS: The value for UMi, UMa, InH-Office. Note 1: If is agreed to be considered, the generation of can follow the same principle with the generation of .

1.2.1.3. Ratio of non-direct paths with spherical wavefront impact

1.2.1.3.1. Company view (Round-1)

As for the ratio of non-direct path which show the phenomenon with spherical wavefront, [Samsung] highlights RAN1 shall consider the range of impact of clusters taking into account UE’s location, and following detailed views are summarized according to companies’ inputs:

- [Huawei, HiSilicon, ZTE] propose that such a ratio can be directly set as 1, or no need to define such a ratio[ZTE]. More specifically,

[Huawei, HiSilicon] proposes that since the planer-wavefront is originally an approximation of spherical-wavefront when the far-field assumption holds, setting the ratio as 1 can match the propagation characteristic better and contribute to the conservation of spec effort.

[ZTE] highlights that the generated d1 of NLoS path will implicitly determine whether this 1st point associated with cluster shows the near-field or far-field characteristics. When the generated d1 is larger sufficient, it will automatically to be same as the far-field channel model.

- [Apple, Qualcomm, CATT] propose such a ratio is needed. Specifically,

[Apple] proposes that if the cluster delay generated in Step 5 is within the Rayleigh distance, then this non-direct path has spherical wavefront.

[Qualcomm] proposes that the spherical wavefront modelling for indirect paths is applied only to a set of K indirect paths. And K=0 is not precluded as it can be useful when spherical wavefront modelling is only required for the direct path in certain scenarios or environments.

[CATT] proposes that the ratio an be set as either 1 or 0, since the near-field or far-field condition for the non-direct paths between BS and UE can follow the near-field or far-field condition for the direct path.

From FL’s perspective, whether to introduce such a ratio highly depends on whether the same or different implementations are used for near-field and far-field channel realization. If a same implementation is used, there is no need to determine such a ratio. In such, this issue can be discussed together with the issue on whether a same or different implementations are used for near-field and far-field channel realization.

In this meeting, regarding whether the same or different implementations are used for near-field and far-field channel realization, followings are summarized according to companies’ inputs:

- [Huawei, HiSilicon, ZTE, vivo, Intel] propose a same implementations are used. Specifically,

[Huawei, HiSilicon] highlights that considering channel initialization only takes up tiny proportion of overall simulation time, the increase of computational complexity for a same implementation is inconspicuous.

[vivo] provides the analysis to illustrate the additional generation burden can be ignored, and highlights that the unified channel model is used only in case that the near-field propagation needs to be considered in simulation.

[ZTE] highlights that the generated d1 and d2 of NLoS path will implicitly determine whether this cluster shows the near-field or far-field characteristics, and when considering the same model, the consistency, e.g., channel characteristic vs. the mobility of same UE or different UEs in different location can be ensured.

- [CATT] proposes that different implementations are used.

[CATT] highlights that if the same implementation is used for near-field and far-field channel realization, the complexity of generating the channel for far-field would be increased but the gain can be very tiny.

- [Samsung] proposes that RAN1 need further discussion on same/different implementation methods taking into account channel consistency and computational complexity.

Additionally, [OPPO] highlights that smooth transition to far field channel for near-field channel modelling, e.g. at the edge of the near-field region, shall be considered regardless of whether the same or different implementations are used.

From FL’s perspective, following can be observed according to companies’ inputs:

- Compared with different implementations, a same implementations seems to be more preferred without introducing additional issues, e.g., the smooth transition between near-field and far-field, criteria/applicable conditions on when to apply near-field/far-field.

- The additional computational complexity for a same implementation compared to different implementations is marginal.

- For each NLoS path between BS and UE, its near-field/far-field condition will be implicitly determined according to the generated d1 and d2. And if considering to determine such a ratio, the ratio has to be determined per UE and per scenario, which will significantly increase the workload of channel model.

Thus, following is proposed from FL’s perspective:

Proposal 1-2-1-3-1:

The same implementation, e.g., procedures/equations, is used to generate the channel coefficients for all clusters/UEs when the near-field channel model is considered.

- No need to introduce a ratio to distinguish the NLoS cluster with or without spherical wavefront characteristics.

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| InterDigital | We are fine with the proposal. |
| Apple | No. We think different implementation is expected for near-field and far-field. This could simplify the implementation, as far-field case could follow legacy approach. |
| Huawei, HiSilicon | Generally fine. |
| vivo | Support |
| Samsung | Support |
| QC | It is incorrect to assume that all paths will exhibit spherical wavefront properties. For higher frequencies, aberrated wavefronts are likely more typical compared to spherical wavefronts due to imperfections on various environmental surfaces or due to surface textures. It may therefore not be necessary to introduce near field modelling for all paths/rays. We need three parameters: fraction of paths undergoing specular reflections, fraction of paths that exhibit spherical wavefronts while not corresponding to specular reflections and fraction of paths that have aberrated wavefronts, i.e., wavefronts whose spherical characteristic has been degraded/corrupted due to reflector/surface texture or other properties. |
|  | Not support and share similar view with Qualcomm. Since we don’t think we should preclude the case where spherical wavefront modelling is only required for the direct path, we support at least K = 0, 1. FFS whether/how support value of (0, 1) |
| CATT | Agree with Apple, QC. |
| BUPT | Support |
| Sony | Generally fine. |

    - Antenna element-wise channel parameters

#### 1.2.2.1. Doppler shift (Closed)

1.2.2.1.1. Company view (Round-1)

Regarding on whether to model the antenna element-wise Doppler shift parameters for the non-direct path, following are summarized according to companies’ inputs:

- [ZTE, NVIDIA, Apple,vivo, Intel, CATT, CEWiT] propose to optionally model the antenna element-wise Doppler shift parameters for non-direct paths.

[ZTE] highlights that for non-direct path, potential new use cases with higher UE velocity may require the modeling of antenna element-wise Doppler shift.

[vivo, ZTE] highlight that for antenna element-wise Doppler shift parameters, only need to replace the antenna center-wise angle with the antenna element-wise angular domain parameters, there is no extra cost.

- [Huawei, HiSilicon, Lenovo, OPPO, Qualcomm] propose that there is no need to model the antenna element-wise Doppler shift parameters for non-direct paths.

[Lenovo] mentions the near-field region is expected to model crowded areas, where most devices are handheld devices, in which Doppler shift values are relatively small. And Doppler shift values become notable for high-speed train scenarios and/or highways and vehicle roads, wherein for these scenarios it is reasonable to model the cells to be far enough from the main road to cover a larger area.

[Qualcomm] provides the simulation results to show that under the different configuration of values of velocity and observation time that result in a displacement of no more than 1 meter, the impact of the Doppler term on the near-field channel model is observed to be marginal and can be deprioritized.

- Additionally, [ZTE,vivo, Intel, CATT, CEWiT] proposes to update the Doppler shift to expj2πrrx,n,m,u,sT∙vλ0t, where   rrx,n,m,u,s are the spherical unit vector with azimuth arrival angle and elevation arrival angle of ray m in cluster n between the transmit antenna element s and receive antenna element u.

From FL’s perspective, similar as the direct path, since the antenna element-wise angular domain parameters is agreed to be optionally modeled for the non-direct path, no extra cost will be introduced if modeling the antenna element-wise Doppler shift parameters. And considering the potential new use cases with higher UE velocity, it’s better to optionally model the antenna element-wise Doppler shift parameters for non-direct path.

- Thus, following is proposed from FL’s perspective:

Proposal 1-2-2-1-1:

- For near-field channel, the Doppler shift is optionally modeled as antenna element-wise channel parameters of non-direct path between TRP and UE.

- The following formula is adopted to model the antenna element-wise Doppler shift in the channel coefficient:

exp⁡(j2πd2,n∙  rrx,n,m,0,0−drx,u,0d2,n∙ rrx,n,m,0,0−drx,u,0T∙vλ0t)

- where d2,n are the generated distance of the nth cluster.  rrx,n,m,0,0 are the spherical unit vector with azimuth arrival angle and elevation arrival angle for ray m of cluster n between the reference antenna element pair;drx,u,0 are the vector pointing from reference receive antenna element to receive antenna element u.

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| InterDigital | We prefer to continue using the existing TR 38.901 definition. |
| Apple | Okay with the proposal in principle, but we could check the detailed formula in a later stage. |
| Huawei, HiSilicon | Generally fine. |
| vivo | Support |
| Samsung | If companies have strong view that the impact of Doppler shift is significant, we are fine with the proposal. |
| QC | Five companies have said there is no need. Can the FL draft a more fair proposal with two options to better reflect where the group stands? Might be best to not make decisions on these aspects until the basic modelling details are worked out. If an agreement is really required, please leave this as a study so that companies get more time to check the proposed modelling approach. |
| Intel | The parameter d2,n is still not clearly defined/agreed mathematically. We prefer to have a clear definition of d2,n before using it in derivation of other parameters. |
| CATT | Prefer the existing TR 38.901 definition. |
| Lenovo/ MotM | Further discussion is needed to motivate for the need of this proposal |
| Sony | We are not convinced that Doppler shift must be modeled antenna element-wise for non-direct paths. |

1.2.2.1.2 Summary of the discussion

The following conclusion is made and no further discussion is needed for this aspect:

Conclusion For near-field channel, no changes are expected on following parameter for non-direct path between TRP and UE: - Doppler shift

#### 1.2.2.2. Delay

1.2.2.2.1. Company view (Round-1)

As for the delay parameters for non-direct path, there also has a working assumption “the existing model in Clause 7.6.2 of TR 38.901 is used to model the delay”, and following views are summarized according to companies’ inputs:

- [Apple, Intel, Qualcomm] propose to confirm the working assumption.

[Intel] highlights that since the near-field channel is associated with large antenna apertures D, the modeling of the propagation delay in Clause 7.6.2.1 of TR 38.901 can be re-used to calculate the ray-specific delay τn,m.

[Qualcomm] provides the simulation results to demonstrate that the differences in path lengths using planar wave approximation and actual antenna coordinates are of the order of a few centimeters even for UEs that are very close to the gNB. And such minor differences are unlikely to matter for systems operating using 100s of MHz.

- [ZTE,vivo, CATT] propose to revise the working assumption. Specifically:

[ZTE, vivo] highlights that similar as the illustration of LoS path, the existing methods to calculate the ray-level antenna element-wise delay in section 7.6.2 of TR38.901 is based on far-field assumption, and cannot reflect the spherical wavefront characteristics for near-field channel model.

- [Lenovo] proposes that further clarification is needed on the working assumption regarding the applicability of the additional modeling component in Clause 7.6.2 of TR 38.901 to the near-field channel with antenna element-wise delay modeling.

Additionally, [ZTE] highlights that considering in the existing section 7.6.2 of TR38.901, each ray within a cluster for a given Rx and Tx has unique time of arrival when considering the large bandwidth and large antenna array case, thus it will be more accurate to model independent delay of each ray for the near-field channel model.

Besides, when not using the existing model in Clause 7.6.2 of TR 38.901 to model the delay, following companies share the detailed formula to calculate the antenna element-wise delay for non-direct path:

- [ZTE] propose that the antenna element-wise delay parameters can be calculated according to the generated d1 and d2, the location of antenna elements at BS/UE side and the spherical unit vector that generated by existing procedure. The detailed formula is given as following:

τn,m,u,s=τn−d1−d1∙ rtx,n,m,0,0−dtx,s,0c−d2−d2∙ rrx,n,m,0,0−drx,u,0c

where  d1 and d2 are the generated distance of cluster n ;  rtx,n,m,0,0 and  rrx,n,m,0,0 are the spherical unit vector with azimuth departure angle and elevation departure angle, the spherical unit vector with azimuth arrival angle and elevation arrival angle for ray m of cluster n between the reference antenna element pair, respectively; dtx,s,0 and drx,u,0 are the respective vector pointing from reference transmit antenna element to transmit antenna element s, the vector pointing from reference receive antenna element to receive antenna element u.

- [vivo] proposes that the following formula is adopted to model the delay for non-direct paths:

- τn,u,s'=τn+rtx,n,s−rtx,n+ rrx,n,u− rrx,n /c

- τn,u,s=τn,u,s'−min⁡(τn,u,s')

where rtx,n,s refers to the vector determined by the location of antenna element s and first-bounce scatterer, rrx,n,u refers to the vector determined by the location of antenna element u and last-bounce scatterer, rtx,n refers to the vector determined by the location of reference point in the transmitter and first-bounce scatterer, rrx,n refers to the vector determined by the location of reference point in the receiver and last-bounce scatterer.

- [CATT] proposes that the delay can be calculated by τs1,u= rs1,uc , where the distance vector rs1,u between the transmit antenna s1 and reference receive antenna u can be obtained following the existing spatial consistency procedure A of TR38.901.

From FL’s perspective, similar as the existing model in section 7.6.2 of TR38.901 that each ray within a cluster for a given Rx and Tx has unique time of arrival, since the large antenna array is also considered in near-field channel model, it’s more reasonable to model the independent delay of each ray for the non-direct paths. As for how to calculate the antenna element-wise delay for non-direct paths, it seems the different companies share the different views, thus, following is proposed from FL’s perspective:

Proposal 1-2-2-2-1:

- For near-field channel, one of following options is considered to calculate the antenna element-wise delay parameters of non-direct path between TRP and UE:

- Option-1: The existing model in Clause 7.6.2 of TR 38.901 is used to model the antenna element-wise delay with following updates in equation (7.6-4):

τu,s,n,m=τn,m−d1,n−d1,n∙ rtx,n,m,0,0−dtx,s,0c−d2,n−d2,n∙ rrx,n,m,0,0−drx,u,0c

where d1,n and d2,n are the generated distance of the nth cluster. rtx,n,m,0,0 and  rrx,n,m,0,0 are the spherical unit vector with azimuth departure angle and elevation departure angle, the spherical unit vector with azimuth arrival angle and elevation arrival angle for ray m of cluster n between the reference antenna element pair, respectively; dtx,s,0 and drx,u,0 are the respective vector pointing from reference transmit antenna element to transmit antenna element s, the vector pointing from reference receive antenna element to receive antenna element u.

Option-2: Confirm the following working assumption made in RAN1#118BIS:

- “The existing model in Clause 7.6.2 of TR 38.901 is used to model the delay.”

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| InterDigital | We prefer Option-2. |
| Apple | We prefer Option 2. |
| Huawei, HiSilicon | Seems both Options will introduce λf, which is not preferred. |
| vivo | Similar to the delay formula for the direct path, without the minimum delay operation, the τu,s,n,m could be minus, and therefore, the equation τu,s,n,m=τu,s,n,m'−min⁡(τu,s,n,m') is necessary. |
| QC | Needs more discussion. In our view the parameters d_1 and d_2 may not necessarily reflect the distance traversed by a ray if it is assumed to be set equal to the radius of curvature associated with a wavefront. May be good to revisit after the modelling details are resolved. |
| Intel | The parameter d1,n and d2,n are still not clearly defined/agreed mathematically. We prefer to have a clear definition of d2,n before using it in derivation of other parameters. |
| CATT | We prefer Option 2. |

#### 1.2.2.3. Phase and angular domain parameters

1.2.2.3.1. Company view (Round-1)

During the discussion in last several meetings, the phase has been agreed to be modeled as antenna element-wise channel parameters for the non-direct path, and the angular domain parameters have been agreed to be optionally modeled as antenna element-wise channel parameters for the non-direct path.

In such, in this meeting, some companies proposed the detailed formula to update the corresponding parameters in the channel coefficient for non-direct path, and following views are summarized according to companies’ inputs:

- For the phase parameters:

- [Huawei, HiSilicon, ZTE, Qualcomm, CEWiT] propose that the phase is expressed by the distance information between the point associated with clusters and antenna elements of BS/UE, e.g.,

expj2πd1−d1∙ rtx,n,m,0,0−dtx,s,0λ0expj2πd2−d2∙ rrx,n,m,0,0−drx,u,0λ0

where d1 and d2 are the generated distance of cluster n ; rtx,n,m,0,0 and  rrx,n,m,0,0 are the spherical unit vector with azimuth departure angle and elevation departure angle, the spherical unit vector with azimuth arrival angle and elevation arrival angle for ray m of cluster n between the reference antenna element pair, respectively; dtx,s,0 and drx,u,0 are the respective vector pointing from reference transmit antenna element to transmit antenna element s, the vector pointing from reference receive antenna element to receive antenna element u.

- [CEWiT] further proposes that for a single bounce model, the phase can be updated to expj2πdrx,u−dC,n,mλ0expj2πdtx,s−dC,n,mλ0, where dC,n,m is the ray m from the single cluster n.

- [vivo] proposes that the antenna element-wise phase can be updated to:

- exp−j2πλ0rrx,n,u−rrx,nexp−j2πλ0rtx,n,s−rtx,n

- where rtx,n is the distance between the reference point in the transmitter and the first-bounce scatterer,  rtx,n,s is the distance between the transmit antenna element s and the first-bounce scatterer, and rrx,n is the distance between the reference point in the receiver and the last-bounce scatterer, rrx,n,u is the distance between the receive antenna element u and the last-bounce scatterer.

- [Apple, Intel] propose that the phase between TRP antenna element s and UE antenna element u is given by expj2πrrx, n,m,u,sT⋅drx,uλ0⋅expj2πrtx, n,m,u,sT⋅dtx,sλ0, where

rrx, n,m,u,sT=sinθn,m,u, s, ZOA⋅cosϕn,m,u, s, AOAsinθn,m,u, s, ZOA⋅sinϕn,m,u, s, AOAcosθn,m,u, s, ZOA,

rtx, n,m,u,sT=sinθn,m,u, s, ZOD⋅cosϕn,m,u, s, AODsinθn,m,u, s, ZOD⋅sinϕn,m,u, s, AODcosθn,m,u, s, ZOD.

- [CATT] proposes that the element-wise phase can be calculated by expj2πλ0ds1,u−ds,u , where ds1,u=|rs1,u| represents the sum of the distance from one point associated with cluster to the transmit antenna element s1 and to the reference receive antenna element u, and ds,u=|rs,u| represents the sum of the distance from one point associated with cluster to the transmit antenna element s and to the reference receive antenna element u.

- For the angular domain parameters:

- [Huawei, HiSilicon, ZTE, Apple,vivo, Intel, Qualcomm, CEWiT] propose that angular parameters in the field pattern of transmit antenna element and receive antenna elements can be updated to the antenna element-wise angular domain parameters, e.g.,

Frx,u,θθn,m,ZOA,u,s,ϕn,m,AOA,u,sFrx,u,ϕθn,m,ZOA,u,s,ϕn,m,AOA,u,sTexpjΦn,mθθkn,m−1expjΦn,mθϕkn,m−1expjΦn,mϕθexpjΦn,mϕϕFtx,s,θθn,m,ZOD,u,s,ϕn,m,AOD,u,sFtx,s,ϕθn,m,ZOD,u,s,ϕn,m,AOD,u,s

Thus, following is proposed from FL’s perspective:

###### Proposal 1-2-2-3-1:

For near-field channel, the following formula is adopted to model the phase parameters of non-direct path between TRP and UE as antenna element-wise channel parameter:

- expj2πd1,n−d1,n∙ rtx,n,m,0,0−dtx,s,0λ0expj2πd2,n−d2,n∙ rrx,n,m,0,0−drx,u,0λ0

- where d1,n and d2,n are the generated distance of the nth cluster. rtx,n,m,0,0 and rrx,n,m,0,0 are the spherical unit vector with azimuth departure angle and elevation departure angle, the spherical unit vector with azimuth arrival angle and elevation arrival angle for ray m of cluster n between the reference antenna element pair, respectively; dtx,s,0 and drx,u,0 are the respective vector pointing from reference transmit antenna element to transmit antenna element s, the vector pointing from reference receive antenna element to receive antenna element u.

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| InterDigital | We prefer CEWiT formulation for single-bunce model which can be extended to multi-bounce model. |
| Apple | No, we think the direct expression of the phase parameter in terms of antenna pair is clear. Specifically, expj2πrrx, n,m,u,sT⋅drx,sλ0⋅expj2πrtx, n,m,u,sT⋅dtx,uλ0 with rrx, n,m,u,sT=sinθn,m,u, s, ZOA⋅cosϕn,m,u, s, AOAsinθn,m,u, s, ZOA⋅sinϕn,m,u, s, AOAcosθn,m,u, s, ZOA, rtx, n,m,u,sT=sinθn,m,u, s, ZOD⋅cosϕn,m,u, s, AODsinθn,m,u, s, ZOD⋅sinϕn,m,u, s, AODcosθn,m,u, s, ZOD. |
| Huawei, HiSilicon | Generally fine. |
| vivo | Support |
| QC | Depends on what the parameters d_1 and d_2 come to represent. Its not clear if we can use them as equivalent to distance. May be best to get the group to a common understanding on how to model clusters for non-direct paths before proceeding to this. |

###### Proposal 1-2-2-3-2: (Closed)

For near-field channel, the following formula is adopted to model the angular domain parameters of non-direct path between TRP and UE as antenna element-wise channel parameter:

Frx,u,θθn,m,ZOA,u,s,ϕn,m,AOA,u,sFrx,u,ϕθn,m,ZOA,u,s,ϕn,m,AOA,u,sTexpjΦn,mθθkn,m−1expjΦn,mθϕkn,m−1expjΦn,mϕθexpjΦn,mϕϕFtx,s,θθn,m,ZOD,u,s,ϕn,m,AOD,u,sFtx,s,ϕθn,m,ZOD,u,s,ϕn,m,AOD,u,s

where θn,m,ZOA,u,s,ϕn,m,AOA,u,s, θn,m,ZOD,u,s,ϕn,m,AOD,u,s are the ray-wise angular domain parameters of ray m cluster n between the transmit antenna element s and receive antenna element u.

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| InterDigital | We are fine with the principle with the following corrections: Frx,u,θθn,m,ZOA,u,s,ϕn,m,AOA,u,sFrx,u,ϕθn,m,ZOA,u,s,ϕn,m,AOA,u,sTexpjΦn,mθθkn,m−1expjΦn,mθϕkn,m−1expjΦn,mϕθexpjΦn,mϕϕFtx,s,θθn,m,ZOD,u,s,ϕn,m,AOD,u,sFtx,s,ϕθn,m,ZOD,u,s,ϕn,m,AOD,u,s |
| Apple | Okay |
| Huawei, HiSilicon | Prefer InterDigital’s version. |
| vivo | Support |
| LGE | Ok |
| Intel | Support InterDigital’s version. |
| Sony | Fine with InterDigital’s corrections. Note that they also need to be applied to the θ’s and ϕ’s under the formula. |

1.2.2.3.2. Summary of the discussion

In the online discussion, the following agreement is achieved and this issue is concluded.

Agreement For near-field channel, the following formula is adopted to model the angular domain parameters of non-direct path between TRP and UE as antenna element-wise channel parameter: Frx,u,θθn,m,ZOA,u,ϕn,m,AOA,uFrx,u,ϕθn,m,ZOA,u,ϕn,m,AOA,uTexpjΦn,mθθkn,m−1expjΦn,mθϕkn,m−1expjΦn,mϕθexpjΦn,mϕϕFtx,s,θθn,m,ZOD,s,ϕn,m,AOD,sFtx,s,ϕθn,m,ZOD,s,ϕn,m,AOD,s where , are the ray-wise angular domain parameters of ray m cluster n between the transmit antenna element s and receive antenna element u.

1.3. Reference antenna determination of antenna array (Closed)

### 1.3.1 Company view (Round-1)

In this meeting, some companies [Huawei, HiSilicon, ZTE,vivo, CATT, Samsung, OPPO, SHARP, Qualcomm] mention that reference antenna element/point shall be considered when determining the antenna element-wise channel parameters, and [ZTE] highlights that the selection of reference antenna element have impact on the value of calculated channel coefficient, and it’s better to have a unified understanding on the determination of reference antenna element to keep consistency from different companies during the simulation.

Specifically, [vivo, Intel, SHARP, Qualcomm] propose to assume the center of respective antenna array as the reference point at the BS/UE side, and [ZTE] proposes that the antenna element with index ceil(N/2) for the antenna array with N elements at BS/UE side is treated as reference antenna element/point.

From FL’s perspective, when the number of antenna elements at BS/UE side is an even number, it may result in that the center of respective antenna array at the BS/UE side does not refer to a real antenna element. Since the channel parameters that generated following by existing procedure of TR 38.901 shall be treated as the channel parameters of reference antenna, it’s preferred to determine a real antenna element at the BS/UE side as the reference antenna element/point.

Thus, following is proposed from FL’s perspective:

- Proposal 1-3-1:

- For near-field channel, the antenna element in the center of antenna array is assumed as the reference antenna element at the respective BS and UE side, i.e., the reference antenna element at BS/UE side is the antenna element with index ceil(N/2) for the antenna array with N elements.

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| InterDigital | We support to define the reference point as the center of the antenna array. |
| Huawei, HiSilicon | Open to further discuss. |
| vivo | Support. However, the statement of i.e. seems not necessary, and it is up to implementation. |
| QC | As the FL points out, this point may not be at the center of the panel. Not sure if this can lead to any unintended consequences. Might be okay. Good to discuss further. |
| QC | As the FL points out, this point may not be at the center of the panel. Not sure if this can lead to any unintended consequences. Might be okay… |
| CATT | We think center/edge does not have any technical difference. While the definition of center is not clear and need more clarification. In this sense, it’s not good to use ‘center’ as reference. |
| Sony | We prefer to define a reference point as “the center of the antenna array”. However, handheld device might not have antenna arrays, but just antenna elements: How to apply the definition to this case? Moreover, a UE can have more that one antenna array; as our understanding is that the channel generation should common to all such arrays, which array should one pick? Maybe a better definition is: reference point = center of the BS or UE device. As a general comment, the proposal should explain why a reference point is needed (i.e., what is it used for?). |

### 1.3.2 Summary for the discussion

The following agreement is made by defined a reference point and this issue is closed.

Agreement For near-field channel, to generate the channel parameter, the reference point is defined as: - The physical center of the antenna array/center of the device.

1.4. Modelling for link level simulation

1.4.1 Company view (Round-1)

During the last several meetings, the discussion of near-field channel model have been mainly focused on the system level evaluation&procedure. While [ZTE] proposes that the link level evaluation for near-field channel model shall also be considered. And considering that the 3D locations of both TRP and UE is not given in the link level evaluation, [ZTE] proposes that the antenna element-wise channel parameters for the link level evaluation can also be obtained following the same methods of non-direct path to obtain the antenna element-wise channel parameters. Following is an example for the step-wise procedure of the link level evaluation to obtain the antenna element-wise channel parameters proposed by [ZTE].

From FL’s perspective, how to determine the antenna element-wise channel parameters in the link level evaluation is also an important aspect of channel model. This issue shall be considered together with the discussion on the details and methods for the system level evaluation along with the difference between the link level evaluation and system level evaluation realization procedure shall be considered when determining the specific methods.

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| InterDigital | We think this topic could be down prioritized. |
| vivo | We think it is too early to discuss this proposal. |

1.5. Others

In addition, some companies mention the issue on criteria to define the near-field region in this meeting, and following are summarized according to companies’ views:

- [Intel] highlights that RAN1 to consider defining soft criteria for leveraging near-field models and state that near-field modelling is applicable for evaluation scenarios that contain links that meet such criteria.

- [CATT] proposes that the effective Rayleigh distance, a scaled Rayleigh distance with a scaling factor of 0.4 can be used to define the near-field region.

- [OPPO] proposes that the near-field region can be defined based on Rayleigh distance with a scale factor.

- [Sony] compares the various definitions of the boundary between the near and far field, and proposes that the boundary between the near- and far-field of an antenna array should be drawn at distances about an order of magnitude smaller than the Fraunhofer distance, i.e., dF/10 or so.

- [CEWIT] proposes that depending on the density/distribution of the UEs, two options can be considered for the near-field boundary, where one-half of conventional Rayleigh distance (Rd/2) for densely populated UEs, and full conventional Rayleigh distance (Rd) for sparsely populated UEs.

Additionally, regarding the issue on the near-field/far-field condition for the non-direct paths, [Intel] proposes that different granularity, i.e., per TX-cluster and cluster-RX links, one Far-field/near-field determination for TX-cluster links and one far-field/near-field determination for cluster-RX links, per TX-cluster-RX links, entire NLOS channel, or the entire channel, can be considered to determine the near-/far-field condition of non-direct paths, while [OPPO] proposes that the near/far-field condition for non-direct paths is determined per cluster based on the distance between gNB antenna array and cluster.

As for how to determine the near-/far-field condition for the non-direct paths, following options are summarized according to the inputs:

- Option-1: Follow the direct path [CATT].

- Option-2: Assessed by using the distance between antenna array and cluster location [OPPO, InterDigital].

- Option-3: Introduce the near-field probability to determine the near-field condition [LGE].

From FL’s perspective, when considering a same implementation that used for the near-field and far-field channel realization, there is no need to introduce such a criteria to define the near-field region and determine the near/far-field condition for non-direct paths.

companies are encouraged to share your views on the above issues:

| Companies | Comments and Views |
| --- | --- |

Regarding the detailed channel coefficient generation procedure, the companies’ inputs are summarized and shown in Appendix-B.

From FL’s perspective, regarding the proposed updates on the diagram, it can be considered for reference and detailed changes can be discussed later once the details of model are stable.

Companies are encouraged to share your views regarding the above content if any.

| Companies | Comments and Views |
| --- | --- |

In addition, some proposals from individual company are also provided:

- [Samsung] proposes that the existing channel models lack terms representing distinct phases of field patterns for each antenna element, and the update of channel parameters with weighting vectors of each antenna element should be considered.

- [Lenovo] proposes that for non-angular parameters in near-field channels, the parameters are modeled to either have antenna-element common parameter values or antenna group-wise parameters,

Companies are encouraged to share your views regarding the above content if any.

| Companies | Comments and Views |
| --- | --- |

- Views on Spatial non-stationarity

2.1 Methodology for UE side SNS (Closed)

In RAN1#118bis, the following agreement is achieved on the methodology of UE side SNS

Agreement For the modelling of spatial non-stationarity at BS side, if physical blocker-based approach is adopted, for blockage model B, the procedure to determine the number of blockers and locations are same as existing blockage model B. For the modelling of spatial non-stationarity at UE side, if blocker-based approach is adopted, an attenuation per antenna element is introduced, the following options can be considered with down-selection by RAN1#120: - Option-1: generated by leveraging the existing blockage Model-B with potential updates. - At most 2 hand type blockers and one head type blocker for a specific UE is assumed. - Option-2: generated by leveraging the existing blockage Model-A with potential updates - Option-3: generated by a distribution or a fixed value due to the hand grip and/or head proximity - FFS: the details, e.g., how to determine the value of loss for the case as one hand grip, a dual hand grip and/or head proximity

### 2.1.1 Company View (Round-1)

According to the contributions in this meeting, the following views are provided on the 3 methodologies for UE side spatial non-stationarity:

- Option-1: generated by leveraging the existing blockage Model-B with potential updates.

  - [ZTE]: For the modelling of spatial non-stationarity at UE side, Option 1(i.e., leveraging the existing blockage Model-B with potential updates) is supported.

    - For the modelling of spatial non-stationarity, if physical blocker-based approach is adopted, at least for blockage model B, the following new UE side blocker type/size can be introduced in the Table 7.6.4.2-5 in TR 38.901:

|  | Typical set of blockers | Blocker dimensions | Mobility pattern |
| --- | --- | --- | --- |
| Indoor;Outdoor | User hand | Cartesian: w=0.1m; h=0.1m | Stationary |
| Indoor;Outdoor | User head | Cartesian: w=0.24m; h=0.20m | Stationary |

    - Drop at most 2 hand type blockers and at most one head type blocker within a certain distance from UE.

  - [Fujitsu]: Spatial non-stationarity at UE side can be modeled based on the existing Model-B with potential updates.

However, [Lenovo] highlights that for non-stationarity modeling of the attenuation per antenna element due to blockage at the UE side, further discuss Option-2 and Option-3, whereas Option-1 is down prioritized.

- Option-2: generated by leveraging the existing blockage Model-A with potential updates

  - [ZTE]: For the modelling of UE side spatial non-stationarity, if physical blocker-based approach is adopted, for blockage model-A, the following updates are required:

    - The method for selecting the reference element;

    - The method for calculating the location and size of the blocker based on the angular blocking region of the reference element;

    - The method for calculating the angular blocking regions for each element, including:

      - Rotating the blocker for each element;

      - Calculating the angular blocking regions for each element according the rotated blocker;

      - Extending the attenuation determination formula into element wise;

  - [Intel]: UE side antenna element blockage modeling is supported by blockage Model A extension

    - One or no self-blockage region is generated per UE antenna element;

    - The orientation of each self-blockage region is determined based on UE antenna architecture, FFS details;

    - Table 7.6.4.1-1 for self-blocking region parameters in TR 38.901 is used as a baseline, with potential scaling of the region dimensions based on the number of blocked UE antenna elements.

  - [MediaTek]: If necessary, utilize TR 38.901 spatial consistency and blockage A along with multiple antenna arrays/panels (grouping of antenna elements) to model spatial non-stationarity effects.

- Option-3: generated by a distribution or a fixed value due to the hand grip and/or head proximity

  - [vivo]: For the modelling of spatial non-stationarity at UE side, three steps are used

    - Step 1: Determinate the UE usage scenarios according to the typical proportion in Table 2.

    - Step 2: Obtain the per antenna element attenuation values βu based on the UE usage scenario.

    - Step 3: Apply the attenuation value βu for the per element pair channel.

Table 2: The typical proportion of free-space, one hand grip and head and one hand grip in daily life.

| Usage scenario | Free-Space | one hand grip | head and one hand grip |
| --- | --- | --- | --- |
| Duration | 15.5 hours | 8 hours | 0.5 hours |
| Proportion | 65% | 33% | 2% |

Table 3: The attenuation loss caused by hand or head occlusion obtained based on measurements.

| Antenna Port | Added loss in Free-Space | Added loss for one hand grip | Added loss for head and one hand grip |
| --- | --- | --- | --- |
| AP#1 | 0 dB | 0 dB | 3 dB |
| AP#2 | 0 dB | 6 dB | 6 dB |
| AP#3 | 0 dB | 3 dB | 3 dB |
| AP#4 | 0 dB | 1 dB | 4 dB |

  - [InterDigital]: Option 3 is an abstract approach with a similar end results as other options for modelling of self-blockage. Support Option 3, where the value of loss factors per antenna element may be fixed or determined based on a distribution to emulate the effect of hand grip and/or head proximity.

  - [LGE]: For the modelling of spatial non-stationarity at UE side, support to generate by a distribution or a fixed value due to the hand grip and/or head proximity.

  - [Apple]: For the modelling of spatial non-stationarity at UE side, support the power attenuation per antenna element is generated by a fixed value due to hand grip and/or head proximity.

  - [CATT]: For the modelling of spatial non-stationarity at UE side, if blocker-based approach is adopted, support Option 3.

    - Option 3: generated by a distribution or a fixed value due to the hand grip and/or head proximity.

  - [Lenovo]: For non-stationarity modeling of the attenuation per antenna element due to blockage at the UE side, further discuss Option-2 and Option-3, whereas Option-1 is down prioritized.

  - [OPPO]: For the modelling of spatial non-stationarity at UE side, if blocker-based approach is adopted, an attenuation per antenna element is introduced based on option 3.

    - Only attenuation in part of (fixed) antenna elements needs to be modelled for one blocker type

    - Further study whether the attenuation values are generated by a distribution or a fixed value

  - [Sony]: Consider the table below as an input to generate a distribution (Option-3), such as a probability distribution or a lookup table, of the attenuation per antenna element for the modeling of spatial non-stationarity at UE side, if blocker-based approach is adopted.

Table 3. Measured attenuations due to the hand grip and head proximity.

| Antenna element placement | Hand [dB] | Head [dB] |
| --- | --- | --- |
| Top | −1.0 | −4.0 |
| Top side | −1.0 | −4.0 |
| Bottom | −3.6 | −3.3 |
| Bottom side | −3.8 | −3.3 |

  - [Qualcomm]: For the modelling of spatial non-stationarity at UE side, if blocker-based approach is adopted, introduce a new blockage model (C) that generates a loss due to the hand grip and/or head proximity using a distribution. Further, consider the following:

    - Introduce three UE states depending on how a UE is held: one-hand grip, dual-hand grip and head/body proximity.

    - Reuse the UE antenna model for handheld devices to identify the subsets of antenna impacted in different UE states.

    - Assign probability of a UE impacted by UE-side blockage and individual probabilities for each of the three states.

    - Model UE-side blockage using attenuation factors that impact a subset of antennas. Attenuation factors are chosen from a range of values at random.

    - Determine attenuation factors/tables for each state and each frequency range, with consideration to module-based and discrete antenna architectures.

  - [Nokia]: For the modelling of spatial non-stationarity at UE side:

    - RAN1 to specify the proposed near-field element-wise self-blockage model represented in Table 3 (for Option 3)

    - The 3GPP self-blocking model A can be used together with element-wise self-blockage (Option 3) as a far-field body (torso) blockage model with the already defined portrait mode or with an updated blockage region (Option 2).

Table 3: New proposed near-field element-wise self-blockage model for 6G.

| Antenna Port | Added Implementation loss in Free-Space | Added loss for a one hand grip (Portrait) | Added loss for a dual hand grip (Landscape) | Added loss for head and one hand grip |
| --- | --- | --- | --- | --- |
| AP#1 | 3 dB | 0 dB | 9 dB | 1 dB |
| AP#2 | 2 dB | 0 dB | 5 dB | 3 dB |
| AP#3 | 1 dB | 3 dB | 9 dB | 12 dB |
| AP#4 | 0 dB | 12 dB | 5 dB | 3 dB |

  - [ZTE] highlights that different gesture will lead to various attenuation per elements and the impacts on all potential candidate antenna location should be considered.

- According to the above views, Option 1 requires minimum spec effort but a bit more simulation complexity due to the rotation and knife edge attenuation calculation, Option 2 requires additional spec effort since it also needs to drop virtual blocker to calculate the blockage angle, and the rotation should be performed to identify the element-wise blockage angle range. While  majority companies prefer Option 3 due to its simplicity for simulation to define a fixed table or distribution for some specific device usages, e.g. one hand grip, dual hand grip, head and one hand grip. From FL’s perspective, the following is proposed:

###### Proposal 2-1-1:

For the modelling of spatial non-stationarity at UE side, if blocker-based approach is adopted, the Option-3 is supported with following details:

- Three typical cases are introduced: one hand grip, dual hand grip, head and one hand grip.

  - FFS: probability of each case in the simulation.

- For each case, to model the attenuation for each of antenna in the candidate antenna location defined in the UE antenna model for handheld device with one of following alternatives:

  - Alt 1: Introduce a fixed value for each candidate antenna position per case.

  - Alt 2: Introduce a distribution for each candidate antenna position.

  - FFS: distribution per antenna position and per case.

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| Apple | Support. We prefer Alt 1 for simplicity. |
| InterDigital | We are fine with the proposal. |
| Huawei, HiSilicon | Generally fine. |
| vivo | As we pointed out, at least three typical cases (i.e., no blockage, hand grip, and head and hand grip) occur with a certain probability, and each case should be statistically determined by the distribution, which is based on the real user behavior. |
| Samsung | Support |
| LGE | Ok |
| Nokia | In general, we are OK with the proposal. Alt.1 will be a simpler approach, but we are open to discuss Alt.3. However, the companies proposing the values need to clarify how the attenuation values per antenna are derived. In our case, the simulations are following the standardized CTIA grip. |
| Sony | Generally okay. Alt2 should allow for the case that the attenuation experienced by the UE’s antenna ports is correlated. For example, if one antenna port at the bottom of the UE experiences attenuation due to hand grip, then it is likely that any other antenna port also place on the bottom half will experience attenuation. |

### 2.1.2 Summary of the discussion

The following working assumption is achieved and companies are encouraged to provide the inputs (e.g., values) in next meeting to conclude the discussion.

Working Assumption For the modelling of spatial non-stationarity at UE side mainly due to close proximity, if blocker-based approach is adopted, the Option-3 is supported with following details: - Three typical cases are introduced: one hand grip, dual hand grip, head and one hand grip. - FFS: probability of each case in the simulation. - For each case, to model the attenuation for each of antenna in the candidate antenna location defined in the UE antenna model for handheld device with one of following alternatives: - Alt 1: Introduce a fixed value for each candidate antenna position per case. - Alt 2: Introduce a distribution for candidate antenna position. - FFS: distribution per antenna position(s) and per case.

2.2 Methodology for BS side SNS

According to the contributions in this meeting, the following views are provided on the 2 methodologies for BS side spatial non-stationarity:

- Option 1: Introducing per ray/cluster the visible probability, or visibility region for set of antenna element

- Supported by InterDigital, Huawei, LGE, vivo, BUPT, CMCC, Qualcomm, CATT, Apple

- Option 2: Introducing the physical blocker to emulate the blockage impact on the link for each element-pair

- Supported by Intel, Nokia, ZTE, Ericsson, MediaTek, Qualcomm, OPPO, NVIDIA, Samsung, Fujitsu

Similar number of proponents support each option, furthermore, [Qualcomm, Intel] propose to support one stochastic model and one physical blocker based model for BS side SNS.

### 2.2.1 Physical blocker based approach

According to the agreements in last meeting, the power attenuation can reuse the knife edge attenuation model in TR 38.901, and the extension of blockage model B to antenna element-wise model has also been accomplished, therefore, the remaining issues include the following:

- New blocker type/size

- Number and location of blocker

- Applicability of blockage model A

2.2.1.1 Physical blocker discussion (Closed)

2.2.1.1.1 Company View (Round-1)

According to companies’ contributions, the following views are provided on the new blocker type/size:

- [Ericsson]: For the recommended blocker dimension of a building edge, use Cartesian: w=50 m; h=20 m.

  - For blockage by a building edge, use a simplified variant of blockage model B that only considers diffraction around one edge of the blocker, resulting in a blockage loss of LdB=20log100.5−F, where F represents one of Fh1, Fh2, Fv1, and Fv2.

  - Blockage by building edges should only be considered for links involving base stations that are in close vicinity to the building edge.

- [ZTE]: Drop the blockers within a certain distance from BS and consider the K nearest blockers.

- [Intel]: Blockage Model B applicability to be limited to gNB and UE side SnS other than head and hand blockage.

- [Qualcomm]:If spatial non-stationarity at the gNB-side is modeled using blockage models, aim to reuse the existing blockage model B in 38.901 with potential changes to attenuation models to better suit spatial non-stationarity. Further the following recommendations are made:

  - Details related to user hand/head blockage to be considered in a separate section intended to model UE-side SnS and not included within blockage model B.

  - For rectangular screens to model building edges:

    - Locate screens closer to gNB ( < 5m)

    - Screen dimensions: similar height as gNB & width of 25-50m.

- [NVIDIA]: Blockers, including building edges and small objects such as billboard, streetlamp, and pillar, are generated with deterministic locations, shapes and sizes in an environment and used to model spatial non-stationarity across BSs and UEs in the environment.

From FL’s perspective, for physical blocker based BS side SNS, the pending issue includes the dimensions of building edge, corresponding simplification on power variation calculation, and dropping limitations on the BS side blockers. For the dropping rule in the existing blockage model B, the K blockers are selected according to the distance from UE, but for BS side SNS, it’s more reasonable to drop the blockers close to BS instead of UE. Therefore, the following is proposed:

###### Proposal 2-2-1-1-A:

For the modelling of spatial non-stationarity at BS side, if physical blocker-based approach is adopted, the nearest K blockers from BS are selected.

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| InterDigital | We are fine with the proposal. |
| Samsung | Support |

###### Proposal 2-2-1-1-B:

For the modelling of spatial non-stationarity, if physical blocker-based approach is adopted, at least for blockage model B, the following parameters to define the building edge are introduced in the Table 7.6.4.2-5 in TR 38.901:

|  | Typical set of blockers | Blocker dimensions | Mobility pattern |
| --- | --- | --- | --- |
| Outdoor | Building edge | Cartesian: w=50 m; h=20 m | Stationary |

- In the blockage model B, the following equation is used to calculate the attenuation caused by the building edge:

  - LdB=20log100.5−F, where F represents one of Fℎ1, Fℎ2, Fv1, and Fv2 defined in equation 7.6-29 in section 7.6.4.2 of TR 38.901.

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| InterDigital | We are fine with the proposal, with the following correction, For the modelling of spatial non-stationarity at BS side |
| Samsung | Support |
| Sony | Fine. There seems to be a minus sign missing in the expression of LdB when comparing to the original formula in 38.901. (In our understanding, F∈[−.5,+.5], which makes log10.5−F≤0, and thus LdB≤0, but our understanding is that losses are non-negative in 38.901. |

2.2.1.1.2 Summary of the discussion

After online discussion, the following working assumption is achieved and companies are encouraged to check the value of the dimensions in bracket at least.

Working Assumption - For the modelling of spatial non-stationarity at BS side, if physical blocker-based approach is adopted, the nearest K blockers from BS are selected. - For the modelling of spatial non-stationarity at BS side, if physical blocker-based approach is adopted, at least for blockage model B, the following parameters to define the building edge are introduced in the Table 7.6.4.2-5 in TR 38.901: Typical set of blockers; Blocker dimensions; Mobility pattern. Outdoor; Building edge; Cartesian: w=50 m; h=20 m; Stationary. - In the blockage model B, the following equation is used to calculate the attenuation caused by the building edge: - , where represents one of , , , and defined in equation 7.6-29 in section 7.6.4.2 of TR 38.901.

2.2.1.2 Granularity of physical blocker based SNS (Closed)

2.2.1.2.1 Company View (Round-1)

According to companies’ contributions, the following views are provided on the granularity of physical blocker based SNS:

- [ZTE]: For the modelling of spatial non-stationarity, if physical blocker-based approach is adopted, the rotation of blocker is conducted in cluster level.

- [Qualcomm]: If spatial non-stationarity is to be modeled, the concept of visibility region or blockage region is applied on a per cluster basis.

- [InterDigital]: Reuse the existing mechanism for rotating the blocker to ensure the arrival/departure direction at each Receive/Transmit antenna element.

As mentioned by companies, for physical blocker based approach, the rectangular screen is rotated to be perpendicular to each element, then it should be further clarified whether the rotation and power variation calculation is in cluster level or in ray level, e.g. if it’s in cluster level, it means that all rays in one cluster share the same power variation due to SNS, otherwise, different rays may have different rotations and different power variation. It should be noticed that in existing blockage model B, the rotation is conducted in cluster level, so FL suggests to also conduct the power variation calculation in cluster level for SNS.

Therefore, from FL’s perspective, the following is proposed:

###### Proposal 2-2-1-2:

For the modelling of spatial non-stationarity at BS side, if physical blocker-based approach is adopted, the rotation and power variation calculation are conducted in cluster level.

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| InterDigital | We are fine with the proposal. |
| Samsung | Support |

2.2.1.2.2 Summary of the discussion

The following agreement is achieved and this issue is concluded:

Agreement For the modelling of spatial non-stationarity at BS side, if physical blocker-based approach is adopted, the rotation and power variation calculation are conducted in ray level.

2.2.1.3 Applicability of blockage model A

2.2.1.3.1 Company View (Round-1)

According to companies’ contributions, the following views are provided on the applicability of blockage model A:

- [Intel]: The following updates to blockage Model A to incorporate spatial non-stationarity are adopted:

  - Update of the model parameters from gNB perspective defined in Table 7.6.4.1-2 of TR 38.901,

  - Disabling self-blockage when evaluated from gNB perspective,

  - Recalculation of virtual blocking screen location and dimensions for antenna elements not collocated with the antenna reference point

    - For each ray/cluster, rotate the virtual blocker screen to ensure the arrival/departure direction at each Receive/Transmit antenna element is always perpendicular to the screen.

- [MediaTek]: If necessary, utilize TR 38.901 spatial consistency and blockage A along with multiple antenna arrays/panels (grouping of antenna elements) to model spatial non-stationarity effects.

It seems that some companies still want to extend blockage model A for modeling BS side SNS and disable self-blockage in this model. [Intel] further proposes to both adopt blockage model B as a geometry based solution and blockage model A as a stochastic solution.

From FL’s perspective, the following is proposed, the following additional aspects should be further studied and more details are expected to achieve the following aspects:

- How to update of the model parameters from gNB perspective defined in Table 7.6.4.1-2 of TR 38.901

- Disabling self-blockage when evaluated from gNB perspective

- Calculating the location and size of the virtual blocker based on the angular blocking region of the reference element

- Recalculation of virtual blocking screen location and dimensions for antenna elements not collocated with the antenna reference point

  - For each cluster, rotate the virtual blocker screen for each antenna element.

  - Calculating the angular blocking regions for each element according the rotated blocker

- Extending the attenuation determination formula into element wise.

Companies are encouraged to share your views with more detailed views/procedures if the extension of Model-A is needed for SNS at BS side:

| Companies | Comments and Views |
| --- | --- |
| MediaTek | From our view, blockage model A is applicable for SNS modeling at both UE side and BS side, by determining the impacted azimuth and elevation angular ranges caused by a blocking screen. Regarding the issues listed by the FL, we propose the following solutions: - On SNS modelling at UE side, for an antenna element 𝑢, the attenuation of each cluster due to a self-blocking/non-self-blocking region is determined according to AOA, ZOA and the corresponding antenna-wise blocking region parameters of (φk,u,xk,u,θk,u, yk,u,ru). The existing table for self-blocking region parameters and the table 7.6.4.1-2 of non-self-blocking region parameters is leveraged for the reference antenna element. - On SNS modelling at BS side, for an antenna element s, the attenuation of each cluster due to a non-self-blocking region is determined according to AOD, ZOD and the corresponding antenna-wise parameters of (φk,s,xk,s,θk,s, yk,s,rs). - At either UE side or BS side, new blocker types introduced for blockage model B (i.e., outdoor building edge, indoor pillar, streetlamp, billboard, etc.) can be supported by converting the vertical blocking screen with width w and height ℎ to the non-self-blockage region parameters. - For a blocker is near-by the BS/UE (i.e., nearby building, pillar, streetlamp, billboard), the corresponding blocking region is calculated as follow: - For a blocker is right below the BS (i.e., roof top), the corresponding blocking region is calculated as follow: - The antenna-wise blocking region parameters for a BS/UE antenna element is derived based on the blocking region parameters of the BS/UE reference antenna element and the the GCS location relative offset between a BS/UE antenna element and the BS/UE reference antenna element, according to the following steps: - Step-1: Determine the location and size of the virtual blocker based on the angular blocking region of the reference element. - Step-2: Calculate the antenna-wise blocking region parameters for the BS/UE antenna element, according to the location and size of the virtual blocker and the location of the BS/UE antenna element. In summary, we would like to propose: For the modelling of spatial non-stationarity, if physical blocker-based approach is adopted, existing stochastic attenuation model in TR 38.901 blockage model A can be adopted for modeling the power variation for the impacted ray/cluster within the element-pair link - For the modelling of spatial non-stationarity at UE side, for an antenna element u, the attenuation of each cluster due to a self-blocking/non-self-blocking region is determined according to AOA, ZOA and the corresponding antenna-wise blocking region parameters of (φk,u,xk,u,θk,u, yk,u,ru) - Introduce new non-self-blocker types for the outdoor building edge (i.e., near-by building), indoor pillar, street lamp and billboard at UE side - FFS: The value(s) or the statistical distribution of non-self-blocking region for new non-self-blocker types - The antenna-wise blocking region parameters of (φk,u,xk,u,θk,u, yk,u,ru) for the antenna element u is derived based on the blocking region parameters of the UE reference antenna element and the GCS location relative offset between a UE antenna element u and the UE reference antenna element - FFS: The details, e.g., the procedure to derive the antenna-wise blocking region parameters of (φk,u,xk,u,θk,u, yk,u,ru) - For the modeling of spatial non-stationarity at BS side, for an antenna element s, the attenuation of each cluster due to a non-self-blocking region is determined according to AOD, ZOD and the corresponding antenna-wise parameters of (φk,s,xk,s,θk,s, yk,s,rs) - Introduce new non-self-blocker types for the outdoor building edge (i.e., near-by building or roof top), indoor pillar, street lamp and billboard at BS side - FFS: The value(s) or the statistical distribution of non-self-blocking region for new non-self-blocker types - The antenna-wise blocking region parameters of (φk,s,xk,s,θk,s, yk,s,rs) for the antenna element s is derived based on the blocking region parameters of the BS reference antenna element and the GCS location relative offset between the BS antenna element s and the BS reference antenna element - FFS: The details, e.g., the procedure to derive the antenna-wise blocking region parameters of (φk,s,xk,s,θk,s, yk,s,rs). |

### 2.2.2 Visible probability or visibility region

According to contributions, the following views are provided by companies on the general procedure for VP or VR based approach:

- [vivo]: For the modelling of spatial non-stationarity at BS side, if visibility-based approach is adopted, the following steps are used.

  - Step 1: Determinate the visible probability for the cluster with spatial non-stationarity according to the cluster power.

  - Step 2: For the cluster with spatial non-stationarity, determinate its visible elements based on the distance between the cluster and the BS antenna elements, and cluster power.

  - Step 3：Calculate the power variation factor αs,n of the cluster to all antenna element according to the visible probability and distance between antenna element and the cluster.

  - Step 4: Generate the channel impulse response and apply the power variation factor into the channel coefficients.

- [Huawei]: For each cluster n:

  - a) Calculate the VP

  - b) Decide whether the spatial non-stationarity happens.

  - c) Locate the first-bounce scatterer.

  - d) Calculate the distance dn between Tx antenna elements and the first-bounce scatterer.

  - e) Calculate the VR.

  - f) Calculate the power attenuation factor for Tx antenna element s according to the following formula:

- [Qualcomm]: If spatial non-stationarity is to be modeled using visibility regions at the gNB side, the following four components are necessary for adequately modelling the impact of spatial non-stationarity:

  - Probability of spatial non-stationarity impacting a UE at any given location

  - Probability of a cluster being partially visible for a UE determined to be impacted by SnS

  - Attributes of the visibility region, e.g., size of the visibility region

  - Attenuation factors associated with a visibility region

- [ZTE]: For VP or VR based approach, the detailed procedure are illustrated as following:

  - Step 4: Determine the clusters that exhibit SNS feature;

  - Step 5: Generate VR for clusters and update the power (e.g., variation (e.g., reduction)) for invisible antenna element;

  - Step 6: Generate the element-wise channel coefficient;

- [CATT]: A procedure to generate the region based VP is proposed

  - Step 1: Generate the distance from the antenna element to cluster.

  - Step 2: Determine the virtual 3-D region for all visible antenna elements

  - Step 3: Determine the power attenuation factor.

- [BUPT, CMCC]: The VA approach consists of three main steps:

  - Step 1: For each cluster/ray, determine whether SnS occurs based on the cluster spatial non-stationarity probability. If SnS does not occur, the power attenuation factor αs,n/m=1.

  - Step 2: For clusters where SnS occurs, use the visibility probability to determine the visible region and the power attenuation factor for each antenna element.

  - Step 3: Substitute the power attenuation factors into the model to generate channel coefficients with SnS characteristics.

- According to the above views, the procedure of VP or VR based solution from majority companies mainly include the following 3 steps, i.e. Determination of clusters with SNS impact, Visibility region generation and Power attenuation factor determination. In addition to these 3 procedures, how to apply power attenuation factor to channel coefficient realization is common for both VP or VR based solution and physical blocker based solution. From FL’s perspective, it’s beneficial to align companies understanding on the general procedure of VP or VR based solution, so the following is proposed:

###### Proposal 2-2-2: (Closed)

For the modelling of spatial non-stationarity, if the unified visible probability and visibility region based approach is adopted, the following steps are considered:

- Step 1: Determine whether a cluster is impacted by SNS

- Step 2: Generate the visibility region

- Step 3: Calculate the power attenuation factor

FFS: the details of each step

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| InterDigital | We are fine with this proposal in principle but this topic may be deferred. Also, please correct as the following, For the modelling of spatial non-stationarity at BS side |
| Apple | Support |
| Huawei, HiSilicon | Support. |
| vivo | Support |
| LGE | Support |
| CATT | Support. |
| BUPT | Support |
| Sony | If the proposal is restricted to the BS side, that happens then a the UE side? Will there be a separate discussion? Note that modeling SNS at the UE side is supported and therefore it makes sense to apply the VP/VR method at the UE side as well. |

- The detailed solutions for each step are elaborated in the following sub-sections.

2.2.2.0 Summary of the discussion

The following agreement is achieved and proponent of the unified solution is encouraged to provide the details of the solution in following online/offline discussion.

Agreement For the modelling of spatial non-stationarity at BS side, if stochastic based approach, at least for unified visible probability (VP) and visibility region (VR), is adopted, the following steps are considered: - Step 1: Determine whether a cluster is impacted by SNS - Step 2: Generate the visibility region - Step 3: Calculate the power attenuation factor FFS: the details of each step Note: Merge of some of the steps is not precluded

2.2.2.1 Determination of clusters with SNS impact

2.2.2.1.1 Company View (Round-1)

Based on companies’ views, the following details are proposed by companies on determination of clusters with SNS impact:

- [Huawei]:

  - Calculate the VP according to the following formula: Vn=Α∙e−maxPn−Pnλ+Β+δ, where A and λ are the parameters of exponential distribution, Pn is the cluster power, B is the lower bound of VP, and δ~N(0, ξ2). The elaborate parameterization procedure can be found in Appendix B.

  - Decide whether the spatial non-stationarity happens. If the calculated VP is greater than or equal to 1, there doesn’t exist spatial non-stationary effect at corresponding side.

- [vivo]: The occurrence of spatial non-stationarity for each cluster is determined according to the cluster power.

- [ZTE]: For the clusters of a UE, the ratio of clusters with SNS feature follows the normal distribution, e.g., N0.18,0.0072;

- [BUPT, CMCC]: The SnS probability of a cluster can be modeled as:

- or

  - where ϵ~N0,σ2 represents random jitter in the SnS probability, with σ=0.0914, σ1=0.0782 and σ2=0.1054. To determine whether SnS occurs within that cluster:

  - where Sn=1 indicates that SnS has occurred in the cluster; otherwise, SnS does not occur, and αs,n=1. x is a randomly generated variable following a uniform distribution, x~U(0,1).

According to companies’ input, [Huawei, vivo, BUPT, CMCC] propose to determine the clusters with SNS impact according to a function of cluster power, while different sources present different functions. From FL’s perspective, the different solutions can be listed for further study and down-selection, thus the following is proposed:

###### Proposal 2-2-2-1:

For the modelling of spatial non-stationarity, if the unified visible probability and visibility region based approach is adopted, for the Step 1 (whether a cluster is impacted by SNS), down-select to support one of following options:

- Option 1: If the calculated Vn is greater than or equal to 1, the n-th cluster is not impacted by SNS by assuming:

- Vn=Α∙e−maxPn−Pnλ+Β+δ,

- where A and λ are the parameters of exponential distribution, Pn is the cluster power, B is the lower bound of VP, and δ~N(0, ξ2), Pn  is the power of the n-th cluster.

- Option 2: If the randomly generated parameter x(following the uniform distribution as x~U(0,1)) is smaller than Prsns, the cluster is impacted by the SNS, where the Prsns is calculated by one of following alternatives:

- Alt-1:Prsns=min(max(−0.0076Pn−0.48+ε, 0), 1),where ε~N0,σ2 with σ=0.0914, Pn  is the power of the nth cluster.

- Alt-2:Prsns=min(max(−0.0076Pn−0.47+ε1, 0), 1),   Pn≤−118dBmin(max(−0.0182Pn−1.68+ε2, 0), 1),   Pn>−118dB, where ε1,ε2~N0,σ2 with σ1=0.0782 and σ2=0.1054, Pn  is the power of the n-th cluster.

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| Apple | We may hold this proposal, depending on the discussions of Proposal 2-2-2. |
| Huawei, HiSilicon | Generally fine and support Option 1. |
| vivo | Support |
| BUPT | Support option 2. |
| Sony | It would be good to have more time to check the details of this proposal. |

2.2.2.2 Visibility region generation

2.2.2.2.1 Company View (Round-1)

Based on companies’ views, the following details are proposed by companies on visibility region generation based on the following two alternatives:

- Alt 1: VR size is randomly generated as number of elements

  - [Qualcomm]: If spatial non-stationarity is to be modeled on the gNB side, visibility regions are assumed to be rectangular in shape. The determination of visibility regions is not dependent cluster locations. Visibility regions of a cluster are randomly determined with a minimum size limit.

  - [ZTE]: The ratio of visible elements to the total number of antenna elements can be used as a parameter to measure the size of the visible region. For LOS rays, generate the ratio of visible elements based on the probability function:Px=0.0766x2+0.0143x+0.0669; For NLOS clusters, generate the ratio of visible elements based on uniform distribution;

- Alt 2: VR size is derived based on distance between antenna array of BS and UE/cluster

  - [Huawei]: The VR is assumed to be a sphere centered at first-bounce scatterer with radius Rn, where Rn equals to the Vn∙Nallth largest dn and Nall represents the total number of Tx antenna elements.

  - [vivo]: The visible elements are determined by the distance between the cluster and the BS antenna elements, and cluster power.

  - [CATT]: For the modelling of spatial non-stationarity, the 3-D virtual region of visible antennas in the antenna array can be generated by the intersection portion of a cluster specific 3-D region and the antenna array plane. The cluster specific 3-D region can be modeled as a sphere with the radius rn, where

- rn=Ln∙dn,smax−dn,smin+dn,smin

  - [BUPT, CMCC]: For SnS clusters, the VP can be generated as follows:

                - where x is a randomly generated variable that follows a uniform distribution, x~U(0,1). Furthermore, based on the visibility probability, the radius of the visible region is determined. The visible region of the antenna array is a spherical area centered at the first-bounce cluster, with a radius of Rn,m. The VnNall antenna elements closest to the first-bounce cluster within the visible region are considered the visible elements. Therefore, the radius of the visible region can be expressed as:

                - where dnmax and dnmin represent the maximum and minimum distances, respectively, from the first-bounce cluster to antenna elements.

  - [OPPO]: For the modelling of spatial non-stationarity, if visible probability or visibility region is adopted, VR size is derived based on distance between antenna array of BS and cluster/UE.

  - [Apple]: For the modelling of spatial non-stationarity, consider adopting the visible probability (VP) or visibility region (VR) approach, VP and VR size depend on the distance between TRP and cluster (i.e., Alt 2)

Among the alternatives, although slight majority companies prefer Alt 2 to determine the visibility region according to the distance between the antenna element and cluster, and the VR is assumed to be a sphere centered at first-bounce scatterer with radius Rn, further study and comparison seems needed among all options. Therefore, the following is proposed:

###### Proposal 2-2-2-2:

For the modelling of spatial non-stationarity, if the unified visible probability and visibility region based approach is adopted, one of following options is to determine the visibility region for each cluster:

- Option-1: VR size is defined as number of elements generated by a distribution with limitation on the minimum size;

  - Note: the shape of the VR is assumed to be rectangular.

- Option-2: The region is generated by a spherical area centered at the first-bounce scatterer;

  - FFS: the details, e.g., the radius of the spherical area is determined by one of alternatives:

    - Option 1: Rn equals to the Vn∙Nalltℎ largest dn, where dn is the distance from antenna element to the first bounce scatterer of n-th cluster and Nall represents the total number of Tx antenna elements, Vn is generated by a function of cluster power.

    - Option 2: Rn=Vn∙dnmax−dnmin+dnmin, where dnmax and dnmin represent the maximum and minimum distances, respectively, from the first-bounce scatterer of n-th cluster to antenna elements, Vn=min(−μlnx,1), where x is a randomly generated variable that follows a uniform distribution, x~U(0,1), μ=0.42.

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| InterDigital | We prefer to discuss the blocker-based approach first. This topic may be deferred. |
| Apple | We may hold this proposal, depending on the discussions of Proposal 2-2-2. |
| Huawei, HiSilicon | Generally fine and support Option 1 under Option-2. |
| vivo | Support |
| LGE | Support |
| CATT | Support |
| BUPT | Support Option 2 under Option-2. |
| Sony | We are supportive, but similar to others, we prefer to discuss the blocker-based approach first. |
| Huawei, HiSilicon | We suggest the following wording refinement to address the clarification question received offline: - Option-2: The region is generated by a spherical area centered at the 1stpoint associated with the nth cluster; - FFS: the details, e.g., the radius Rn of the spherical area is determined by one of alternatives: - Option 1: Rn equals to the Vn∙Nalltℎ largest dn, where dn is the distance from antenna element to the1stpoint associated with the nth cluster and Nall represents the total number of Tx antenna elements, Vn is visible probability generated by a function of cluster power/delay. |

2.2.2.3 Power attenuation factor

2.2.2.3.1 Company View (Round-1)

Based on companies’ views, the following details are proposed by companies on power attenuation factor:

- [Huawei]: Calculate the power attenuation factor for Tx antenna element s according to the following formula:

            - αs,n=              1           ,ds,n≤RneRn−ds,ndnmax−dnmin∙C,ds,n>Rn

            - where C represents the roll-off coefficient between the visible and invisible region.

  - Generate channel coefficients according to formula (7)-(8) and apply pathloss and shadowing.

Hu,s,1LOSt=αs,1Frx,u,θθLOS,ZOA,u,φLOS,AOA,uFrx,u,ϕθLOS,ZOA,u,φLOS,AOA,uT100−1Ftx,s,θθLOS,ZOD,s,φLOS,AOD,sFtx,s,ϕθLOS,ZOD,s,φLOS,AOD,s

exp−j2πd0,0λ0expj2πd0,0−du,sλ0expj2πrrx,LOST.vλ0t

(6)

Hu,s,n,mNLOSt=αs,nPnMFrx,u,θθn,m,ZOA,u,φn,m,AOA,uFrx,u,φθn,m,ZOA,u,φn,m,AOA,uTexpjΦn,mθθκn,m−1expjΦn,mθφκn,m−1expjΦn,mφθexpjΦn,mφφ

Ftx,s,θθn,m,ZOD,s,φn,m,AOD,sFtx,s,φθn,m,ZOD,s,φn,m,AOD,sexpj2πdrx,n,m,0−drx,n,m,uλ0expj2πdtx,n,m,0−dtx,n,m,sλ0expj2πrrx,n,mT.vλ0t

(7)

- [vivo]: Regarding the power variation factor, three scenarios can be considered as follows:

  - For cluster without spatial non-stationarity, power variation factor is set to 1.

  - For the cluster with spatial non-stationarity, if the distance from a certain antenna element to the cluster ds,n is less than Rn, it would be visible element and the power variation factor is set to 1.

  - If the distance is large than or equal to Rn, then the antenna element would be invisible element and the power variation factor with the range [0, 1] could be calculated based on the visible probability and distance between antenna element and the cluster.

- αs,n=1,                         ds,n<RnFVP(Pn),ds,n, ds,n≥Rn

                - where, ds,n denotes the distance between Tx antenna element and the cluster.  VP is the visible probability of the n-th cluster which related to cluster power.

  - Captured the power variation factor αs,n in the channel impulse response of TR38.901-based step-wise channel coefficient generation procedure, as follows:

                - For NLOS case

                - Hu,sNLOSτ,t=n=1Nαs,nHu,s,nNLOStδτ−τn

                - For LOS case

                - Hu,sLOSτ,t=1KR+1n=1Nαs,nHu,s,nNLOStδτ−τn+KRKR+1αs,1Hu,s,1LOStδτ−τ1

- [CATT]: It is noted that if the region of visible antennas is the whole antenna array, the cluster does not have SNS impact.  The power attenuation factor αs,n of antenna element s in the whole antenna array to cluster n should be 1. If the region of visible antennas is part of the antenna array, the power attenuation factor of the antenna element in the region can be expressed as

- αs,n=11+e(ds,n−ds,nminds,nmax−ds,nmin−Vn)∗C

- [BUPT, CMCC]: Within the visible region, the power of the cluster remains constant. Outside the visible region, the cluster's power decay factor is modeled as a function that gradually decreases to zero, ensuring a smooth transition between the visible and non-visible regions, as follows:

- ,

            - where ds,n represents the distance from the n-th first-bounce cluster to the s-th antenna element of the Tx, and C is the roll-off factor between the visible and invisible regions.

- [InterDigital]: The variation (e.g., reduction) of power for the impacted ray/cluster within the element-pair can be enhanced and introduced for modelling spatial non- stationarity of the channel. In addition, we can introduce a parameter, αu,s, as an indicator of path status, e.g., visible (reflection or direct path), non-visible/blockage or diffraction. For example,

- αu,s=   0,           block/nonvisible    1,             nonblock/visible0<…< 1,   diffraction

- [Qualcomm]: If spatial non-stationarity is to be modeled on the gNB side, consider the following options for attenuation modelling outside the visibility regions:

  - Option A: Portions of the array that fall outside the visibility region of a cluster do not see any paths to/from that cluster i.e., a binary determination.

  - Option B: Portions of the array that fall outside the visibility region of a cluster see paths to/from that cluster with a high attenuation factor, for e.g. 30 dB.

  - FFS: attenuation factors governing the transition across the edges of a visibility region

- [Apple]: For the modelling of spatial non-stationarity with the VR approach, the power attenuation factor is equal to VR over the total number of TRP antennas.

- Moreover, [Ericsson] pointed out that the visibility probability/region approach to spatial non-stationarity lacks measurement data for validation and modeling methods to guarantee continuous power variations across the array.

- From FL’s perspective, majority companies prefer to define the power attenuation factor as a function of distance between antenna element to first bounce scatterer. Then, the following is proposed:

###### Proposal 2-2-2-3:

For the modelling of spatial non-stationarity, if the unified visible probability and visibility region based approach is adopted, the power attenuation factor αs,n is generated by one of the following options:

- Option 1: αs,n=              1           ,ds,n≤Rn11+e(ds,n−dnminds,nmax−ds,nmin−Vn)∗C,ds,n>Rn

- Option 2: αs,n=              1           ,ds,n≤RneVn−ds,n−dnmindnmax−dnmin∙C,ds,n>Rn

- Option 3: αs,n=              1           ,ds,n≤RneRn−ds,ndnmax−dnmin∙C,ds,n>Rn

where ds,n represents the distance from the first-bounce scatterer of n-th cluster to the s-th antenna element of the Tx, and C is the roll-off factor between the visible and invisible regions, dnmax and dnmin represent the maximum and minimum distances from the first-bounce scatterer of n-th cluster to antenna elements, Rn is the radius for generating the visibility region of n-th cluster, Vn is a function of cluster power of n-th cluster.

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| InterDigital | We prefer the binary attenuation for simplicity to avoid the Rn determination, i.e., if the path is blocked then the attenuation value is zero, otherwise it is 1. |
| Apple | We may hold this proposal, depending on the discussions of Proposal 2-2-2. |
| Huawei, HiSilicon | Generally fine and support Option 3. |
| vivo | Support |
| CATT | Support |
| BUPT | Support |

2.3 Others

2.3.1 Cause of SNS

#### 2.3.1.1 Company View (Round-1)

- According to companies’ contributions, the following views are provided:

- [Huawei]: Incomplete reflection, diffraction and scattering can result in spatial non-stationary effect.

- [vivo]: The causes for spatial non-stationary characteristics include: (1). Blockage: the object with relative limited size might not completely block the entire antenna array with a large aperture. (2). Non-blockage: the object with relative limited size in the propagation environment might no longer serve as the complete scatterer for the entire antenna array with a large aperture.

- [LGE]: For the modelling of spatial non-stationarity, impact of incomplete scatterer should be modelled.

- [BUPT, CMCC]: In spatial non-stationarity modeling, both blockage and non-blockage effects need to be considered.

- [Qualcomm]: Spatial non-stationarity may arise due to limited visibility (depending on the dimensions of a reflectors/scatterer) or due to blockage (a physical entity obstructing the view of a portion of the array).

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| LGE | We think it is very important to align the understanding among companies about the causes of the phenomenon before discussing the SnS modeling approach. |

2.3.2 Modelling for link level simulation

#### 2.3.2.1 Company View (Round-1)

In this meeting, in addition to the discussion of system level evaluation, [ZTE] also proposes to study the link level channel model to incorporate the SNS impacts. Since the link level simulation is to emulate the propagation between BS and single UE, e.g., without consideration on consistency cross UEs and BS sites, the following diagram for link level simulation is introduced based on visible probability and visibility region approaches:

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| vivo | Support |

- Proposals for online discussion

Proposal 1-1-2-1:

- For near-field channel, one of following options is considered to calculate the antenna element-wise delay of direct path between TRP and UE:

- Option-1: The following formula is used to calculate the antenna element-wise delay of direct path:

- τ1,u,s=τ1−d3D−|ru,s|c

- where ru,s refers to the vector determined by the location of receive antenna element u and transmit antenna element s, d3D refers to the 3D distance between reference antenna at TRP and UE side.

  - Note: The above formula is applied with the same principle as existing model in Clause 7.6.2 of TR 38.901, and the channel response of LOS ray between Rx antenna u and Tx antenna s at delay τ at time t is given by:

Hu,s,1LOSt;τ=Frx,u,θθLOS,ZOA,u,s,ϕLOS,AOA,u,sFrx,u,ϕθLOS,ZOA,u,s,ϕLOS,AOA,u,sT100−1Ftx,s,θθLOS,ZOD,u,s,ϕLOS,AOD,u,sFtx,s,ϕθLOS,ZOD,u,s,ϕLOS,AOD,u,s

exp⁡(−j2πd3Dλf)exp−j2π|ru,s|−d3Dλfexpj2πrrx,LOS,u,sT∙vλ0tδτ−τ1,u,s

- Option-2: The antenna element-wise delay of direct path is directly modeled based on the distance of direct path between antenna elements at TRP and UE side.

  - Following formula is adopted to model the antenna element-wise delay in the channel coefficient as:

- δτ−τ1,u,s,  τ1,u,s=τ1−d3D−|ru,s|c

- where ru,s refers to the vector determined by the location of receive antenna element u and transmit antenna element s, d3D refers to the 3D distance between reference antenna at TRP and UE side.

- Option-3: Confirm the following working assumption made in RAN1#118BIS:

  - “The existing model in Clause 7.6.2 of TR 38.901 is used to model the delay.”

Proposal 1-2-1-3-1:

The same implementation, e.g., procedures/equations, is used to generate the channel coefficients for all clusters/UEs when the near-field channel model is considered.

- No need to introduce a ratio to distinguish the NLoS cluster with or without spherical wavefront characteristics.

###### Updated Proposal 2-2-2-2:

For the modelling of spatial non-stationarity, if stochastic based approach, at least for unified visible probability (VP) and visibility region (VR), is adopted, one of following options is to determine the visibility region for each cluster:

- Option-1: VR size is defined as number of elements generated by a distribution with limitation on the minimum size;

  - Note: the shape of the VR is assumed to be rectangular.

- Option-2: The region is generated by a spherical area centered at the 1st  point associated with the nth cluster;

  - FFS: the details, e.g., the radius Rn of the spherical area is determined by one of alternatives:

    - Alt-1: Rn equals to the Vn∙Nalltℎ largest dn, where dn is the distance from antenna element to the 1st point associated with the nth cluster and Nall represents the total number of Tx antenna elements, Vn is visible probability generated by a function of cluster power/delay.

    - Alt-2: Rn=Vn∙dnmax−dnmin+dnmin, where dnmax and dnmin represent the maximum and minimum distances, respectively, from the first-bounce scatterer of n-th cluster to antenna elements, Vn=min(−μlnx,1), where x is a randomly generated variable that follows a uniform distribution, x~U(0,1), μ=0.42.

###### Updated Proposal 2-2-2-3:

For the modelling of spatial non-stationarity, if stochastic based approach, at least for unified visible probability (VP) and visibility region (VR), is adopted, the power attenuation factor αs,n is generated by one of the following options:

- Option 1: αs,n=              1           ,ds,n≤Rn11+e(ds,n−dnminds,nmax−ds,nmin−Vn)∗C,ds,n>Rn

- Option 2: αs,n=              1           ,ds,n≤RneVn−ds,n−dnmindnmax−dnmin∙C,ds,n>Rn

- Option 3: αs,n=              1           ,ds,n≤RneRn−ds,ndnmax−dnmin∙C,ds,n>Rn

where ds,n represents the distance from the first-bounce scatterer of n-th cluster point associated with the nth cluster to the s-th antenna element of the Tx, and C is the roll-off factor between the visible and invisible regions, dnmax and dnmin represent the maximum and minimum distances from the first-bounce scatterer point associated with the nth cluster of n-th cluster to antenna elements, Rn is the radius for generating the visibility region of n-th cluster, Vn is a function of cluster power of n-th cluster.

- Option 4: αs,n is determined assuming a linear roll-off in dB domain from the edge of the visibility region until 30 dB attenuation is reached.

  - FFS: value of the roll-off factor

###### Proposal 2-2-2-1:

For the modelling of spatial non-stationarity, if the unified visible probability and visibility region based approach is adopted, for the Step 1 (whether a cluster is impacted by SNS), down-select to support one of following options:

- Option 1: If the calculated Vn is greater than or equal to 1, the n-th cluster is not impacted by SNS by assuming:

- Vn=Α∙e−maxPn−Pnλ+Β+δ,

- where A and λ are the parameters of exponential distribution, Pn is the cluster power, B is the lower bound of VP, and δ~N(0, ξ2), Pn  is the power of the n-th cluster.

- Option 2: If the randomly generated parameter x(following the uniform distribution as x~U(0,1)) is smaller than Prsns, the cluster is impacted by the SNS, where the Prsns is calculated by one of following alternatives:

  - Alt-1:Prsns=min(max(−0.0076Pn−0.48+ε, 0), 1),where ε~N0,σ2 with σ=0.0914, Pn  is the power of the nth cluster.

  - Alt-2:Prsns=min(max(−0.0076Pn−0.47+ε1, 0), 1),   Pn≤−118dBmin(max(−0.0182Pn−1.68+ε2, 0), 1),   Pn>−118dB, where ε1,ε2~N0,σ2 with σ1=0.0782 and σ2=0.1054, Pn  is the power of the n-th cluster.

- Proposals for offline discussion

- Conclusion

Reference

R1-2409403	Considerations on the 7-24GHz channel model extension	Huawei, HiSilicon

R1-2409442	Discussion on adaptation and extension of channel model	Ericsson

R1-2409468	Further Discussion on Channel Model Extension for FR3	InterDigital, Inc.

R1-2409491	Discussion on channel modelling adaptation/extension for 7-24GHz	LG Electronics

R1-2409611	Discussion on channel model adaptation/extension of TR38.901 for 7 - 24 GHz	Samsung

R1-2409695	Views on channel model adaptation/extension of TR38.901 for 7-24GHz	vivo, BUPT

R1-2409725	Discussion on the channel model adaptation and extension	ZTE Corporation, Sanechips

R1-2409779	Channel model adaptation of TR 38901 for 7-24 GHz	NVIDIA

R1-2409821	Channel Model Adaptation and Extension of TR38.901 for 7-24 GHz	Apple

R1-2409955	Discussion on channel model adaptation/extension for 7-24GHz	CATT

R1-2410016	Channel model extension for 7-24 GHz	Lenovo

R1-2410099	Channel model adaptation and extension for 7-24GHz	OPPO

R1-2410131	Discussion on channel model adaptation/extension for 7-24 GHz	Fujitsu

R1-2410214	Discussion on modeling near-field propagation and spatial non-stationarity in TR38.901 for 7-24GHz	BUPT, CMCC

R1-2410237	Further discussion of channel model adaptation/extension of TR38.901 for 7–24GHz	Sony

R1-2410320	Views on channel modelling adaptation/extension for 7-24GHz	Sharp, NYU WIRELESS

R1-2410492	Channel Model Adaptation/Extension of TR38.901 for 7-24GHz	Qualcomm Incorporated

R1-2410520	Discussion on channel modelling enhancements for 7-24GHz for NR	MediaTek Inc.

R1-2410595	Discussion on Channel model adaptation/extension of TR38.901 for 7-24GHz	Nokia

R1-2410628	Channel model adaptation/extension of TR38.901 for 7-24 GHz	CEWiT

R1-2410656  Discussion on channel model adaptation/extension Intel Corporation

Appendix-A

## RAN1#118-bis

Conclusion The spatial non-stationarity characteristics at UE side (e.g., due to the impact of hand(s) and/or head), i.e., the antenna element-wise power variation, is supported in the channel modelling in TR 38.901 with potential updates. Agreement For near-field channel, if necessary, to model the following antenna element-wise channel parameters of direct path between TRP and UE, - Angular domain parameters - FFS: Doppler shift, Delay with Option-2 “Determined by the antenna element locations of both TRP and UE”. Note: Angular domain parameters can be optionally modelled. Conclusion For near-field channel, no changes are expected on following parameters of the non-direct path between TRP and UE: - Amplitude Agreement For near-field channel, if necessary, the antenna element-wise channel parameters of non-direct path between TRP and UE is modelled by: - For the non-direct paths, the antenna element-wise channel parameters are derived based on at least the distance between the BS/UE and a point associated with cluster, i.e., - d1 is used to determine the 1st point associated wit h cluster to calculate the parameter at least for BS side; - FFS: d2 is used to determine the 2nd point associated with cluster to calculate the parameter at least for UE side; - FFS: whether one point is sufficient - FFS: the details to generate the d1, d2,e.g.,if the d2 is needed, both d1 and d2 will be generated pair wise. - FFS: The ratio of non-direct path which show the phenomenon with spherical wavefront - FFS: The determination of the point associated with cluster is conducted in Cluster-level Agreement For near-field channel, if necessary, the following parameters of the non-direct path between TRP and UE should be modeled as antenna element-wise parameter. - Angular domain parameters, Delay (both can be optionally modelled) - Working Assumption - The existing model in Clause 7.6.2 of TR 38.901 is used to model the delay. - FFS: Doppler shift Agreement For near-field channel, if necessary, to model the following antenna element-wise channel parameter of direct path between TRP and UE, - Delay (can be optionally modelled) - Working Assumption - The existing model in Clause 7.6.2 of TR 38.901 is used to model the delay. with Option-2 “Determined by the antenna element locations of both TRP and UE”. Conclusion RAN1 confirms that the modelling of near-field propagation characteristics (i.e., characteristics of spherical wavefront) is taken as an additional modelling component, and the relevant progress (e.g., the agreement(s) with/without “if necessary”) in AI 9.8.2 will be captured in TR 38.901. Agreement For the modelling of spatial non-stationarity, if physical blocker-based approach is adopted, at least for blockage model B, the following new blocker type/size can be introduced in the Table 7.6.4.2-5 in TR 38.901: Typical set of blockers; Blocker dimensions; Mobility pattern. Outdoor; Billboard; Cartesian: w=2.4m; h=3.6m; Stationary. Outdoor; Street lamp; Cartesian: w=0.4m; h=0.8m; Stationary. Outdoor; Building edge; Cartesian: w=X m; h=Y m; Stationary. Indoor; Pillar; Cartesian: w=0.3m; h=3m; Stationary. - FFS: the value of X and Y for the blocker dimensions of building edge is needed. - FFS: the details related to the user hand/head: Indoor;Outdoor; FFS: User hand; Cartesian: w=[0.2]m; h=[0.1]m; Stationary. Indoor;Outdoor; FFS: User head; Cartesian: w=[0.24]m; h=[0.20] m; Stationary. - FFS:The details of blockage model B to implement the impact of user hand and head. - FFS: The location of the user hand/head Agreement For the modelling of spatial non-stationarity at BS side, if physical blocker-based approach is adopted, for blockage model B, the procedure to determine the number of blockers and locations are same as existing blockage model B. For the modelling of spatial non-stationarity at UE side, if blocker-based approach is adopted, an attenuation per antenna element is introduced, the following options can be considered with down-selection by RAN1#120: - Option-1: generated by leveraging the existing blockage Model-B with potential updates. - At most 2 hand type blockers and one head type blocker for a specific UE is assumed. - Option-2: generated by leveraging the existing blockage Model-A with potential updates - Option-3: generated by a distribution or a fixed value due to the hand grip and/or head proximity - FFS: the details, e.g., how to determine the value of loss for the case as one hand grip, a dual hand grip and/or head proximity

## RAN1#118

Agreement Previous agreement made in RAN1#117 is updated as: For the assumption on the aperture size of antenna array, the following is considered for near-field and spatial non-stationarity channel model study, e.g., simulation/measurement and calibration: - Up to 1.5 m for UMa with maximum antenna elements in the array is 5k for single Polarization. - Up to 1 m for UMi with maximum antenna elements in the array is 2.22k for single Polarization. - Up to 0.71 m for Indoor factory with maximum antenna elements in the array is 1.12k for single Polarization. - Up to 0.25 (for rectangular antenna array), 0.5 (for linear antenna array) m for Indoor office with maximum antenna elements in the array is 256, 80 for single Polarization, respectively. Agreement Confirm the following working assumption made in RAN1#117. Working Assumption For the near-field channel modeling, no changes are expected on both value and parameter generation procedure of at least following large-scale parameters in existing TR 38.901: - Pathloss model, SF, LOS probability - FFS:DS, ASA, ASD, ZSA, ZSD, K factor Agreement For the near-field channel modeling, no changes are expected on both value and parameter generation procedure of at least following large-scale parameters in existing TR 38.901: - DS, ASA, ASD, ZSA, ZSD, K factor Agreement The spatial non-stationarity characteristics, i.e., the antenna element-wise power variation at least at BS side, is supported in the channel modelling. FFS: the antenna element-wise power variation at UE side FFS: the causes and details of methodology for modelling the spatial non-stationarity characteristics Agreement For near-field channel, no changes are expected on following parameters of the non-direct path between TRP and UE: - Polarization matrix - FFS: Amplitude Agreement For near-field channel, the following formula is adopted to model the phase of direct path between TRP and UE as antenna element-wise channel parameter: exp⁡(−j2πd3Dλ0)exp−j2π¦ru,s¦−d3Dλ0, where the ru,s refers to the vector determined by the location of antenna element u and s. The d3D refers to the 3D distance between reference antenna at TRP and UE side. Agreement For near-field channel, if necessary, the following parameters of the non-direct path between TRP and UE should be modeled as antenna element-wise parameter. - Phase - FFS: Doppler shift, Angular domain parameters, delay Observation According to the inputs from multiple sources, partial blockage effect may cause the spatial non-stationarity. Agreement For the modelling of spatial non-stationarity, the variation (e.g., reduction) of power for the impacted ray/cluster within the element-pair link should be modelled as: - If visible probability (VP) or visibility region (VR) is adopted, - A power attenuation factor within [0 1] is introduced. - FFS: Details on how to determine the exact value for each cluster - If physical blocker-based approach is adopted: - Adopt the existing knife edge attenuation model in TR 38.901 in blockage Model-B to model the power attenuation per element with following update as: - For each ray/cluster, rotating the blocker to ensure the arrival/departure direction at each Receive/Transmit antenna element is always perpendicular to the screen, respectively. - FFS: applicability and details for blockage Model-A Agreement For the modelling of spatial non-stationarity, if physical blocker-based approach is adopted, the following additional blocker type can be considered for blockage model B: - Building edge for outdoor scenario - Small object, e.g., billboard, street lamp, pillar, for either indoor or outdoor scenario - FFS: UE-side (self-blockage) blocker for both indoor and outdoor scenario - Details, e.g., blocker types such as Single hand grip, dual-hand grip, and head with one hand grip. FFS: the number and the location of the blocker between BS and one specific UE. FFS: applicability and details for blockage Model-A Agreement For near-field channel, if necessary, the antenna element-wise channel parameters of non-direct path between TRP and UE can be determined by one of the following candidate options: - Option-1: The antenna element-wise channel parameters are derived based on at least the distance between the BS/UE and a point associated with cluster. - FFS: How to obtain the distance. - FFS: Other parameters. - FFS: association between a point and a cluster - Option-2: The antenna element-wise channel parameters are determined based on the existing spatial consistency procedure of TR 38.901 with updates. - FFS: Details to obtain the antenna element-wise parameters. Note: Companies are encouraged to check the Option-3 including the similarity/difference with Option-1. Agreement For the modelling of spatial non-stationarity, if visible probability or visibility region is adopted, - Visible probability or visibility region is modelled per cluster - FFS Ratio of UEs and clusters that have SNS impact - Rectangle can be considered as starting point for shape of VR with following alternatives to define the size: - Alt 1: VR size is defined as number of elements generated by a distribution - FFS distribution - Alt 2: VR size is derived based on distance between antenna array of BS and UE/cluster - Note: Cluster location is required in this alternative - Alt 3: VR size is randomly generated with a minimum size limit

## RAN1#117

Agreement For the assumption on the aperture size of antenna array, the following is considered for near-field and spatial non-stationarity channel model study, e.g., simulation/measurement and calibration: - Up to 1.5 m for UMa with maximum antenna elements in the array is [5k] for single Polarization. - Up to 1 m for UMi with maximum antenna elements in the array is [2.22k] for single Polarization. - Up to [0.71] m for Indoor factory with maximum antenna elements in the array is [1.12k] for single Polarization. - Up to [0.25 (for rectangular antenna array), 0.5 (for linear antenna array)] m for Indoor office with maximum antenna elements in the array is [138, 24] for single Polarization, respectively. Working Assumption For the near-field channel modeling, no changes are expected on both value and parameter generation procedure of at least following large-scale parameters in existing TR 38.901: - Pathloss model, SF, LOS probability - FFS:DS, ASA, ASD, ZSA, ZSD, K factor Agreement For near-field channel, if necessary, to model the following antenna element-wise channel parameters of direct path between TRP and UE, - Phase with Option-2 “Determined by the antenna element locations of both TRP and UE”. Agreement For near-field channel, if necessary, to model the following antenna element-wise channel parameters of non-direct path between TRP and UE, - Angular domain parameters (i.e., AoA, AoD, ZoA, ZoD), Delay, phase, Doppler shift, Amplitude - FFS: Impacts on the polarization The following options are considered: - Option-1: The cluster location-based approach, wherein the cluster location is obtained with following alternatives: - Alt-1: cluster location is derived based on at least the distance between the BS/UE and clusters. - FFS: How to obtain the distance. - FFS: Other parameters. - Alt-2: cluster location is directly dropped and generated. - Option-2: The parameter-based approach with following detailed alternatives: - Alt-1: Introduce the model of variation rate of parameter over antenna elements. - Alt-2: Modelling the variation by taking the existing spatial consistency procedure of TR 38.901 as baseline. - Option-3: The curvature-based approach. Agreement For the modelling of spatial non-stationarity, if necessary, the variation (e.g., reduction) of power for the impacted ray/cluster within the element-pair link should be modelled. - FFS: The value for power variation - FFS: Impacts on the phase Agreement For the modelling of spatial non-stationarity, if necessary, if visible probability (VP) or visibility region (VR) is adopted, at least the following aspects should be considered for definition of VR/VP: - Granularity of visible probability or visibility region (e.g., per cluster or per ray) - Determination of visible probability (e.g., distribution) or visibility region (e.g., size, location) Agreement For the modelling of spatial non-stationarity, if necessary, if physical blocker-based approach is adopted, the following aspects should be considered for definition of blocker: - Blocker size/type: - FFS: Additional blocker size/type compared to the Table 7.6.4.2-5 in TR 38.901. - FFS: Different blocker sizes/types are considered to emulate the antenna element-wise blockage effect at the BS and UE side - Blocker location, e.g. distribution of the blocker, relative distance between blocker and BS or UE - FFS: Number of physical blockers to be considered. Agreement To align the understanding of the terminology for channel model study, the following figures are considered as the reference: - For non-direct path: - For direct path: Conclusion For near-field channel, no changes are expected on the following parameters for direct path. - Amplitude, polarization matrix

## RAN1#116-bis

Agreement The antenna array is assumed for the near-field study. Agreement For the study of near-field channel modelling, at least following aspects should be considered: - Whether/How to define the near-field region - The parameters variation for each ray/cluster across different antenna element pairs Agreement The following scenarios defined in TR38.901 should be considered for the study/modelling of near-field. - UMa,UMi, Indoor office and Indoor factory - FFS: RMa and other new scenarios Agreement For the assumption on the aperture size of antenna array, the following is considered as reference for channel model study. - up to [TBD] m, or [TBD] lambda for UMi - up to [TBD] m, or [TBD] lambda for UMa - up to [TBD] m, or [ TBD] lambda for Indoor office - up to [TBD] m, or [TBD] lambda for Indoor factory Agreement For the near-field channel model: - The impact of the assumption of wavefront is only considered from the perspective of antenna array. - The near field for each element within the antenna array is not considered in this SI. Agreement For near-field channel model, RAN1 strives to design a unified model to explicitly reflect the new properties of near- and existing properties of far-field under the structure of existing stochastic model TR 38.901. - FFS: whether the same or different implementations, e.g., procedures/equations, are used for near- and far-field channel realization Agreement The near- or far-field condition should be studied for the direct path and non-direct paths between BS and UE. - The near-/far-field condition for the direct path may be assessed by using the 3D BS-UE distance. - FFS: The determination of near-/far-field condition for the non-direct paths - Note: The direct path is referring to the LoS ray in the TR 38.901 in principle. - Note: The non-direct paths are referring to the cluster/ray(s) without including LoS ray in the TR 38.901. Agreement For near-field channel, if necessary, to model the following antenna element-wise channel parameters of direct path between TRP and UE, - Angular domain parameters (i.e., AoA, AoD, ZoA, ZoD), Delay, initial phase, Doppler shift, Amplitude - FFS: Impacts on the polarization The following options are considered: - Option-1: Determined by the locations of both TRP and UE. - Option-2: Determined by the antenna element locations of both TRP and UE Agreement The following scenarios defined in TR38.901 should be considered for studying/modelling of spatial non-stationarity - UMi, UMa, Indoor office and Indoor factory - FFS: RMa and other new scenarios Agreement For the modelling of spatial non-stationarity, at least the following options can be studied to identify the impacted ray/cluster and element-pair link: - Option 1: Introducing per ray/cluster the visible probability, or visibility region for set of antenna element - Option 2: Introducing the physical blocker to emulate the blockage impact on the link for each element-pair - Note: The consistency across antenna elements and across clusters should be guaranteed.

Appendix-B

- Near-field channel coefficient generation

- ZTE:

- CATT:

Appendix-C

- Spatial non-stationarity channel coefficient generation

- [ZTE]: For system level simulation, consider the following procedure

Figure 20. Channel coefficient generation procedure.

- [ZTE]: For link level simulation, consider the following procedure

Figure 28. Procedure for link level simulation of spatial non-stationarity.

- Step 0-3: Same as the existing procedure for CDL channel generation;

- Step 4: Determine the clusters that exhibit SNS feature;

  - Generate the ratio of clusters with SNS features based on a normal distribution N0.18,0.0072;

  - Randomly identify which clusters/ray possess the SNS feature according to this ratio;

- Step 5: Generate VR for clusters and update the power (e.g., variation (e.g., reduction)) for invisible antenna element;

  - VR size: Use the ratio of visible elements to represent the size of VR.

    - For LOS rays, generate the ratio of visible elements based on the probability function:Px=0.0766x2+0.0143x+0.0669;

    - For NLOS clusters, generate the ratio of visible elements based on uniform distribution;

  - VR shape: Rectangular shape as an example;

  - Power: Generate the power of cluster/ray with SNS impact based on power attenuation factor.

- Step 6: Generate the element-wise channel coefficient;

- [Huawei]:

Step 1-10: Generate the delays, angles, and initial phases for a pair of reference antenna elements.

Step 10.5: For each cluster n:

- Calculate the VP according to the following formula:

Vn=Α∙e−maxPn−Pnλ+Β+δ                                                          (4)

where A and λ are the parameters of exponential distribution, Pn is the cluster power, B is the lower bound of VP, and δ~N(0, ξ2). The elaborate parameterization procedure can be found in Appendix B.

- Decide whether the spatial non-stationarity happens.

If the calculated VP is greater than or equal to 1, there doesn’t exist spatial non-stationary effect at corresponding side.

If the spatial non-stationarity happens, execute the following steps:

- Locate the first-bounce scatterer.

The scatterer locating approach in the section 3.4 of [4] can be referred to, which is described detailedly in Appendix A.

- Calculate the distance dn between Tx antenna elements and the first-bounce scatterer.

- Calculate the VR.

The VR is assumed to be a sphere centered at first-bounce scatterer with radius Rn, where Rn equals to the Vn∙Nallth largest dn and Nall represents the total number of Tx antenna elements.

Figure 4 Exemplified visibility region

- Calculate the power attenuation factor for Tx antenna element s according to the following formula:

αs,n=              1           ,ds,n≤RneRn−ds,ndnmax−dnmin∙C,ds,n>Rn                                                              (5)

where C represents the roll-off coefficient between the visible and invisible region.

Step 11-12: Generate channel coefficients according to formula (7)-(8) and apply pathloss and shadowing.

Hu,s,1LOSt=αs,1Frx,u,θθLOS,ZOA,u,φLOS,AOA,uFrx,u,ϕθLOS,ZOA,u,φLOS,AOA,uT100−1Ftx,s,θθLOS,ZOD,s,φLOS,AOD,sFtx,s,ϕθLOS,ZOD,s,φLOS,AOD,s

exp−j2πd0,0λ0expj2πd0,0−du,sλ0expj2πrrx,LOST.vλ0t

(6)

Hu,s,n,mNLOSt=αs,nPnMFrx,u,θθn,m,ZOA,u,φn,m,AOA,uFrx,u,φθn,m,ZOA,u,φn,m,AOA,uTexpjΦn,mθθκn,m−1expjΦn,mθφκn,m−1expjΦn,mφθexpjΦn,mφφ

Ftx,s,θθn,m,ZOD,s,φn,m,AOD,sFtx,s,φθn,m,ZOD,s,φn,m,AOD,sexpj2πdrx,n,m,0−drx,n,m,uλ0expj2πdtx,n,m,0−dtx,n,m,sλ0expj2πrrx,n,mT.vλ0t

(7)

- [vivo]:

- For NLOS case

Hu,sNLOSτ,t=n=1Nβu,s,nHu,s,nNLOStδτ−τn

- For LOS case

Hu,sLOSτ,t=1KR+1n=1Nβu,s,nHu,s,nNLOStδτ−τn+KRKR+1βu,s,1Hu,s,1LOStδτ−τ1

- [CATT]:

- The boundary of near field and far field under LOS/NLOS is generated in the step of  “Assign propagation condition”.

- Delay and arrival & departure angle are generated for each antenna element in the steps of “Generate delays” and “Generate arrival & departure angle”.

- For the spatial non-stationary, if rVP approach is supported, the generated region of the cluster at the antenna array can be applied in the step of “Generate cluster powers”. All the parameters are generated to model the region for each cluster, such as location and size. Since the granularity of rVP is per cluster, the effect of rVP approach can be modelled of in the step of “Generate arrival & departure angle”.

- [BUPT, CMCC]:

- For LOS case

.

- For NLOS case

.