3GPP TSG RAN WG1 #119			R1-2410900

Orlando, US, November 18th – 22nd, 2024

Agenda item:	8.2

Source: 	Moderator (NTT DOCOMO, INC.)

Title: 	Summary#3 of discussion on Rel-18 NR UE features

Document for:	Discussion and Decision

- Introduction

This document summarizes contributions submitted to AI 8.2 and AI 5 regarding Rel-18 NR UE features for NR_MC_enh, NR_MIMO_evo_DL_UL, NR_Mob_enh2 and NR_pos_enh2.

[119-R18-UE_features] Email discussion on Rel-18 UE features – Hiroki (DOCOMO) - To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc Rel-18 Positioning R1-2409344 LS on Additional Measurements for SL-TDOA and SL-TOA RAN2, Qualcomm RAN2 response to R1-2407387 from RAN1#118bis. RAN2 is asking for details on requested RAN2 action. To be handled as part of Rel-18 UE features discussion (agenda item 8.2). Rel-18 Mobility Enh. R1-2409346 LS on UE capabilities for inter-frequency L1 measurements for LTM RAN2, Huawei RAN2 is requesting RAN1 to check RAN2’s understanding on interFreqL1-MeasConfig-r18 and inform them if there is any issue. To be handled as part of Rel-18 UE features discussion (agenda item 8.2).

- NR_MC_enh

Discussion

Simultaneous Configuration of Rel-18 DL DMRS and DCI format 1_3

Following input is provided in contributions for the RAN1#119 meeting.

| [2] | Huawei, HiSilicon | - Simultaneous configuration of Rel-18 DL DMRS and Rel-18 single DCI According to the current TS38.214 as follows, a DCI format 1_3 can indicate R18 DMRS for PDSCH receptions, which implies that simultaneous configuration of DCI format 1_3 and R18 DL DMRS is supported in the current specification. When receiving PDSCH scheduled by DCI format 1_1 or 1_3 by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI or DCI format 4_2 by PDCCH with CRC scrambled by G-RNTI for multicast or G-CS-RNTI, - the UE may be configured with the higher layer parameter dmrs-Type and/or dmrs-TypeEnh, and the configured DM-RS configuration type is used for receiving PDSCH in as defined in Clause 7.4.1.1 of [4, TS 38.211].. However, in current TS38.212 as follows, DCI format 1_3 does not support scheduling R18 DMRS because the antenna port field in DCI format 1_3 cannot indicate the R18 DMRS-related tables, such as Tables 7.3.1.2.2-7/8/9/10. Therefore, there is a contradiction between the descriptions in TS38.212 and TS38.214 regarding the combination of DCI format 1_3 and R18 DL DMRS. MAr above for the case of antennaPortsDCI-1-3= type1A or each block above for the case of antennaPortsDCI-1-3= type2 is defined by the following: - 4, 5, or 6 bits as defined by Tables 7.3.1.2.2-1/2/3/4 and Tables 7.3.1.2.2-1A/2A/3A/4A, where the number of CDM groups without data of values 1, 2, and 3 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively. The antenna ports p0,…,pv-1 shall be determined according to the ordering of DMRS port(s) given by Tables 7.3.1.2.2-1/2/3/4 or Tables 7.3.1.2.2-1A/2A/3A/4A.. Since RAN1 has not discussed the joint feature of DCI format 1_3 and R18 DL DMRS, and the current specification is incomplete for support of this joint feature, Observation: Current specifications (1) allows simultaneous configurations of DCI format 1_3 and R18 DL DMRS, (2) while do not support DCI format 1_3 dynamic scheduling PDSCH with R18 DL DMRS. While we are open to complete the specification work to enable the dynamic scheduling operation of DCI format 1_3 for PDSCH with R18 DL DMRS, the operation of (1) as a first step requires further discussion as well. In our reading, as cited above, R18 single DCI and R18 DMRS can still be configured/enabled simultaneously without any restriction, as illustrated in Figure 1. Thus, the following case is considered possible by current specifications. Figure 1 Current spec allows simultaneous config and simultaneous monitoring When R18 DL DMRS is enabled or configured, it is understood that additional UE complexity in terms of enhanced DMRS channel estimation for handling the new 4 FD-OCC pattern, e.g., associated parameter estimation would be needed. In multi-carrier scheduling, the UE needs to handle multiple PDSCHs from multiple scheduled cells. Therefore, simultaneous configuration of DCI format 1_3 and R18 DL DMRS will result in higher implementation complexity and require higher UE processing capabilities. Note that the R18 DMRS feature is enabled through higher layer signalling, so the corresponding processing resources are prepared based on the configuration. Similarly, PDCCH processing (e.g., counting of CCE limit) in R18 single DCI is also performed based on feature enablement/configuration. Therefore, regardless of DCI format 1_1 or DCI format 1_3 dynamically scheduling or not, the corresponding processing resources are affected by both of R18 DMRS or R18 single DCI feature, once RRC configured. Simultaneous configurations of both R18 DMRS and R18 single DCI can lead to additional UE implementation preparation/processing. To solve this, a new UE capability is proposed to support the simultaneous configuration of R18 DL DMRS and R18 single DCI. If the UE does not report this capability, when DCI format 1_3 is configured on the scheduling cell, neither the scheduling cell nor the scheduled cells can configure dmrs-TypeEnh with the value ‘enabled’. As a result, DCI format 1_1 cannot indicate R18 DL DMRS for the scheduled cell. Considering configuration and scheduling flexibility, the reporting granularity of the new capability is FeatureSet. Proposal MC-1: Support the following new FG 49-xx; Simultaneous Configuration of Rel-18 DL DMRS and DCI format 1_3; Indicates whether the UE can be configured with both Rel-18 DL DMRS and DCI format 1_3.; one of {49-1, 49-1b}; Yes; n/a; Rel-18 DL DMRS and DCI format 1_3 should not be configured simultaneously regardless of on scheduling cell or on the scheduled cell(s).; Per FS; No; No; n/a; Optional with capability signaling. |
| --- | --- | --- |

(Closed) Proposal 2-1:

Introduce following FG for simultaneous Configuration of Rel-18 DL DMRS and DCI format 1_3.

- FG name

  - Simultaneous Configuration of Rel-18 DL DMRS and DCI format 1_3

- Component

  - Indicates whether the UE can be configured with both Rel-18 DL DMRS and DCI format 1_3

- Prerequisite

  - At least one of {49-1, 49-1b}

- Consequence if not supported

  - Rel-18 DL DMRS and DCI format 1_3 should not be configured simultaneously regardless of on scheduling cell or on the scheduled cell(s).

- Type

  - Per FS

- Note

  - None

- Mandatory or optional

  - Optional with capability signaling

| Company | Comment |
| --- | --- |
| Moderator | At RAN1#118bis meeting, same proposal was provided, and the moderator commented that as the proponent argued “there is a contradiction between the descriptions in TS38.212 and TS38.214 regarding the combination of DCI format 1_3 and R18 DL DMRS”, this issue should be discussed in maintenance session first before discussing the capability [5]. However, it seems there is no corresponding CR in AI8.1 at RAN1#119 meeting. Then, the moderator’s recommendation is to postpone the discussion on this proposal. |
| Huawei, HiSilicon | Strictly speaking, the contradiction could be decoupled with the issue we are discussing here as Fig 1 above illustrates, which is allowed by current spec even if the contradiction remains unsolved. However, in order to better proceed the discussion and align companies understanding, we suggest to have a discussion on this issue in UE feature first, and if needed, to have a clear guidance from the UE feature session discussion on which session to further discuss/resolve the showstopper, e.g. Rel-18 MIMO session or Rel-18 MC session. |
| ZTE | We tend to agree with moderator’s assessment that it is proper to postpone this discussion until some potential outcome can be reached in AI 8.1. |
| Moderator | Based on the input, let’s quickly check companies’ view on which session to further discuss/resolve this issue in online session. Proposed conclusion: Whether the simultaneous configuration of Rel-18 DL DMRS and DCI format 1_3 is supported or not is to be discussed/clarified in {Rel-18 MIMO maintenance session, Rel-18 MCE maintenance session}. |
| Moderator | Based on the discussion, RAN1 can discuss CR in maintenance session and UE capability proposal in UE feature session in future RAN1 meeting. |

- NR_MIMO_evo_DL_UL

Discussion

UE STxMP processing capabilities

Following input is provided in contributions for the RAN1#119 meeting.

| [3] | Samsung | When the UE can support multi-DCI based STx2P PUSCH+PUSCH, the UE will prepare two TBs in parallel to transmit both PUSCH simultaneously. However, depending on UE’s capability for processing time, additional timeline may be required. Therefore, new processing capability for additional timeline of multi-DCI based STx2P PUSCH+PUSCH can be introduced as follow: Index; Feature group; Components; Prerequisite feature groups; Consequence if the feature is not supported by the UE; Type; Need of FR1/FR2 differentiation; Note; Mandatory/Optional. 40-6-3a-1; UE STxMP processing capability for codebook; 1. Require additional timeline to process multiple TBs for codebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG Note. This FG can be applied for CG+DG also if UE can support those FG.; 40-6-3a; UE should process multiple TBs within legacy timeline; Per FSPC; FR2 only; candidate values: UE reports candidate value independently for each SCS in unit of symbols For 15kHz SCS: {1,2} For 30kHz SCS: {1,2,4} For 60kHz SCS: {2,4,8} For 120kHz SCS: {4,8,16} For 480kHz SCS: {16,32,64} For 960kHz SCS: {32,64,128}; Optional with capability signalling. 40-6-3b-2; UE STxMP processing capability for noncodebook; 1. Require additional timeline to process multiple TBs for noncodebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG Note. This FG can be applied for CG+DG also if UE can support those FG.; 40-6-3b; UE should process multiple TBs within legacy timeline; Per FSPC; FR2 only; candidate values: UE reports candidate value independently for each SCS in unit of symbols For 15kHz SCS: {1,2} For 30kHz SCS: {1,2,4} For 60kHz SCS: {2,4,8} For 120kHz SCS: {4,8,16} For 480kHz SCS: {16,32,64} For 960kHz SCS: {32,64,128}; Optional with capability signalling. |
| --- | --- | --- |

(Closed) Proposal 3-1:

Introduce following FG for UE STxMP processing capability for codebook.

- FG name

  - UE STxMP processing capability for codebook

- Component

  - 1. Require additional timeline to process multiple TBs for codebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG

  - Note. This FG can be applied for CG+DG also if UE can support those FG.

- Prerequisite

  - 40-6-3a

- Consequence if not supported

  - UE should process multiple TBs within legacy timeline

- Type

  - Per FSPC

- Need of FR1/FR2 differentiation

  - FR2 only

- Note

  - candidate values:

  - UE reports candidate value independently for each SCS in unit of symbols

  - For 15kHz SCS: {1,2}

  - For 30kHz SCS: {1,2,4}

  - For 60kHz SCS: {2,4,8}

  - For 120kHz SCS: {4,8,16}

  - For 480kHz SCS: {16,32,64}

  - For 960kHz SCS: {32,64,128}

- Mandatory or optional

  - Optional with capability signaling

Introduce following FG for UE STxMP processing capability for non-codebook.

- FG name

  - UE STxMP processing capability for non-codebook

- Component

  - 1. Require additional timeline to process multiple TBs for noncodebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG

  - Note. This FG can be applied for CG+DG also if UE can support those FG.

- Prerequisite

  - 40-6-3b

- Consequence if not supported

  - UE should process multiple TBs within legacy timeline

- Type

  - Per FSPC

- Need of FR1/FR2 differentiation

  - FR2 only

- Note

  - candidate values:

  - UE reports candidate value independently for each SCS in unit of symbols

  - For 15kHz SCS: {1,2}

  - For 30kHz SCS: {1,2,4}

  - For 60kHz SCS: {2,4,8}

  - For 120kHz SCS: {4,8,16}

  - For 480kHz SCS: {16,32,64}

  - For 960kHz SCS: {32,64,128}

- Mandatory or optional

  - Optional with capability signaling

| Company | Comment |
| --- | --- |
| Moderator | Additional timeline for multi-DCI based PUSCH+PUSCH scheduling was proposed at RAN1#115 meeting, and although majority was open to discuss, there were several companies who consider it is not necessary [6]. There has been no discussion and agreement on this issue in MIMO STxMP agenda. In addition, same capability proposal as this meeting was provided at RAN1#116bis and RAN1#117 meetings, but there were only few inputs in the FL summaries and hence it was not discussed. At this meeting, the moderator would like to check companies’s views on the proposal to conclude this issue. |
| Apple | We are fine with both FGs. In terms of the SCS, it is better to clarify whether the SCS is the scheduling DCI, or scheduled PUSCH. We prefer to clarify that SCS is the minimum of two. |
| QC | We are fine with the proposal in general. Besides Apple’s comment (which we agree), we think the candidate values for additional time are relatively too small. We’d like to add larger candidate values highlighted in red into the proposal. For 15kHz SCS: {1,2,4} For 30kHz SCS: {1,2,4,8} For 60kHz SCS: {2,4,8,16} For 120kHz SCS: {4,8,16,32} For 480kHz SCS: {16,32,64,128} For 960kHz SCS: {32,64,128,256} |
| ZTE | Support, and we are find to the update from QC. |
| Moderator | Based on the inputs, it seems companies are ok with the proposal in general. Additional candidate values and clarification on SCS can be discussed in online session. Proposed agreement: Introduce following FG for UE STxMP processing capability for codebook. - FG name - UE STxMP processing capability for codebook - Component - 1. Require additional timeline to process multiple TBs for codebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG - Note. This FG can be applied for CG+DG also if UE can support those FG. - Prerequisite - 40-6-3a - Consequence if not supported - UE should process multiple TBs within legacy timeline - Type - Per FSPC - Need of FR1/FR2 differentiation - FR2 only - Note - candidate values: - UE reports candidate value independently for each SCS in unit of symbols - For 15kHz SCS: {1,2,4} - For 30kHz SCS: {1,2,4,8} - For 60kHz SCS: {2,4,8,16} - For 120kHz SCS: {4,8,16,32} - For 480kHz SCS: {16,32,64,128} - For 960kHz SCS: {32,64,128,256} - Note: SCS above is the minimum between SCS of the scheduling DCI and SCS of the scheduled PUSCH. - Mandatory or optional - Optional with capability signaling Proposed agreement: Introduce following FG for UE STxMP processing capability for non-codebook. - FG name - UE STxMP processing capability for non-codebook - Component - 1. Require additional timeline to process multiple TBs for noncodebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG - Note. This FG can be applied for CG+DG also if UE can support those FG. - Prerequisite - 40-6-3b - Consequence if not supported - UE should process multiple TBs within legacy timeline - Type - Per FSPC - Need of FR1/FR2 differentiation - FR2 only - Note - candidate values: - UE reports candidate value independently for each SCS in unit of symbols - For 15kHz SCS: {1,2,4} - For 30kHz SCS: {1,2,4,8} - For 60kHz SCS: {2,4,8,16} - For 120kHz SCS: {4,8,16,32} - For 480kHz SCS: {16,32,64,128} - For 960kHz SCS: {32,64,128,256} - Note: SCS above is the minimum between SCS of the scheduling DCI and SCS of the scheduled PUSCH. - Mandatory or optional - Optional with capability signaling |
| Moderator | Agreement: Introduce following FG for UE STxMP processing capability for codebook. - FG name - UE STxMP processing capability for codebook - Component - 1. Require additional timeline to process multiple TBs for codebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG - Note. This FG can be applied for CG+DG also if UE can support those FG. - Prerequisite - 40-6-3a - Consequence if not supported - UE should process multiple TBs within legacy timeline - Type - Per FSPC - Need of FR1/FR2 differentiation - FR2 only - Note - candidate values: - UE reports candidate value independently for each SCS in unit of symbols - For 15kHz SCS: {1,2,4} - For 30kHz SCS: {1,2,4,8} - For 60kHz SCS: {2,4,8,16} - For 120kHz SCS: {4,8,16,32} - For 480kHz SCS: {16,32,64,128} - For 960kHz SCS: {32,64,128,256} - Note: SCS above is the minimum between SCS of the scheduling DCI and SCS of the scheduled PUSCH. - Mandatory or optional - Optional with capability signaling Agreement: Introduce following FG for UE STxMP processing capability for non-codebook. - FG name - UE STxMP processing capability for non-codebook - Component - 1. Require additional timeline to process multiple TBs for noncodebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG - Note. This FG can be applied for CG+DG also if UE can support those FG. - Prerequisite - 40-6-3b - Consequence if not supported - UE should process multiple TBs within legacy timeline - Type - Per FSPC - Need of FR1/FR2 differentiation - FR2 only - Note - candidate values: - UE reports candidate value independently for each SCS in unit of symbols - For 15kHz SCS: {1,2,4} - For 30kHz SCS: {1,2,4,8} - For 60kHz SCS: {2,4,8,16} - For 120kHz SCS: {4,8,16,32} - For 480kHz SCS: {16,32,64,128} - For 960kHz SCS: {32,64,128,256} - Note: SCS above is the minimum between SCS of the scheduling DCI and SCS of the scheduled PUSCH. - Mandatory or optional - Optional with capability signaling |

- NR_Mob_enh2

Discussion

Inter-frequency L1 measurement and reports for LTM procedure

Following input is provided in contributions for the RAN1#119 meeting.

| [4] | Ericsson | In [2], RAN2 described their interpretation of what inter-frequency means for FG45-1a, which is reported per BC. This issue was discussed in RAN1#117, and again in RAN3#118. In our view, the RAN2 interpretation is reasonable, but in no way obvious. Therefore, we propose to add notes to clarify that if the UE supports 45-1a, the candidate cell can be on any band where the UE supports LTM, even outside the band combination for which 45-1a is reported: A UE that supports FG 45-1a can perform L1 measurements on any band where it supports LTM. A proposal is provided in Annex A. 45-1a; Inter-frequency L1 measurement and reports for L1-L2 Triggered Mobility (LTM) procedure; 1. Support of inter- frequency L1- RSRP measurement and reporting based on SSB(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra- and inter-frequency L1-RSRP measurement 4. Support of up to L candidate cells and M beams in one report where a SSBRI-RSRP pair is used for each beam report for intra- and inter-frequency L1-RSRP measurement; 2-21 or 2-22 or 2-23 or 2-23a, 45-1; Yes; No; UE does not support inter-frequency L1 measurement and reports for Rel-18 LTM operation; Per BC; No; No; n/a; Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 4 candidate values: L: {1,2,3,4} M: {1,2,3,4} M L: {1,2,3,4, 6, 8, 9, 12, 16} Note: the candidate cell to be measured can be on any band where the UE supports LTM; Optional with capability signalling. |
| --- | --- | --- |

The proposal is related to RAN2 LS in [7], and there are following inputs in AI 5 related to the LS.

| [7] | RAN2 | The UE capability for inter-frequency L1 measurements for LTM (FG 45-1a) is specified as: interFreqL1-MeasConfig-r18 Indicates whether UE supports inter-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s). This capability signalling comprises of the following parameters: - supportedMaxIntraInterFreqCellsConfig-r18 indicates the maximum number of RRC configured candidate cells for intra- and inter-frequency L1-RSRP measurement; - supportedMaxIntraInterFreqCellsPerReport-r18 indicates maximum number of candidate cells in one report where a SSBRI-RSRP pair is used for each beam report for intra- and inter-frequency L1-RSRP measurement; - supportedMaxIntraInterFreqBeamsPerCellReports-r18 indicates maximum number of candidate beams per candidate cell in one report where a SSBRI-RSRP pair is used for each beam report for intra- and inter-frequency L1-RSRP measurement; - supportedMaxIntraInterFreqBeamsReports-r18 indicates maximum number of candidate cells beams in total across all cells in one report where a SSBRI-RSRP pair is used for each beam report for intra- and inter-frequency L1-RSRP measurement; UE supporting this feature shall also indicate support of intraFreqL1-MeasConfig-r18.; BC; No; N/A; N/A. RAN2 understands that, when the UE indicates support of this capability for a BC and the UE is currently operating in this BC (or a subset of it), the SSBs of candidate cells on which the UE can perform inter-frequency L1 measurements according to this UE capabilities can be in any band on which the UE supports LTM. For example, if the UE has serving cells in bands A and B, indicates support of inter-frequency L1 measurements in a BC (A, B, C) and supports LTM in bands A, B, C, D, E, F and G, the UE can perform inter-frequency L1 measurements of SSBs of candidate cells in bands A, B, C, D, E, F and G. RAN2 agreed to introduce a new UE capability to allow the UE to indicate, per UE, that the SSBs of LTM candidate cells on which the UE can perform inter-frequency L1 measurements are limited to the BCs in which the UE indicates support of inter-frequency L1 measurements. RAN2 did not agree yet the final name and description but it could be for example: interFreqL1-OnlyInBC-r18 When included, indicates that, for each BC in which the UE indicates support of interFreqL1-MeasConfig-r18, the UE only supports inter-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s) in the BC i.e., the UE does not support L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s) out of the BC.; UE; No; No; No. In the above example, if the UE also signals interFreqL1-OnlyBC, the UE can perform inter-frequency L1 measurements of SSBs of candidate cells in bands A, B and C, but cannot perform inter-frequency L1 measurements of SSBs of candidate cells in bands D, E, F or G. Such a restriction applies to all UE capabilities related to inter-frequency L1 measurements of SSBs of candidate cells: Capabilities for which interFreqL1-MeasConfig-r18 is a pre-requisite: interFreqSSB-L1-MeasWithoutGaps-r18 (R4 39-2) Indicates whether UE supports SSB based inter-frequency L1-RSRP measurements on SSBs within active DL BWP without measurement gaps (without interruption on serving cell(s)) for LTM. UE supporting this feature shall also indicate support of interFreqL1-MeasConfig-r18.; BC; No; N/A; N/A. ltm-InterFreqMeasGap-r18 (R4 39-2a) Indicates whether the UE supports SSB based inter-frequency L1-RSRP measurements with measurement gaps for LTM. A UE supporting this feature shall also indicate support of interFreqL1-MeasConfig-r18.; UE; No; No; No. Capabilities for which one of the two above is a pre-requisite: multiCellL1-measRTD-greaterThan-CP-r18 (R4 39-1) Indicates the capability of simultaneous L1-RSRP measurements for more than one cell when the max RTD among the cells on the same frequency layer or in the same active BWP is larger than CP length of the cell on the frequency layer or in the same active BWP. A UE supporting this feature shall also indicate support of either intraFreqL1-MeasConfig-r18, interFreqSSB-L1-MeasWithoutGaps-r18 or ltm-InterFreqMeasGap-r18.; BC; No; N/A; N/A. maxFreqLayersL1-Meas-r18 (R4 39-3-1) Indicates the number of frequency layers for L1-RSRP measurement. This capability signalling comprises of the following parameters: - supportedMaxIntraInterFreqLayersWithoutGaps-r18 indicates the maximum number of frequency layers UE can measure for intra- and inter-frequency without measurement gaps L1-RSRP measurement. A UE indicating support for this component shall also indicate support for intraFreqL1-MeasConfig-r18 and/or interFreqSSB-L1-MeasWithoutGaps-r18. - supportedMaxIntraInterFreqLayersWithGaps-r18 indicates the maximum number of frequency layers UE can measure for inter-frequency L1-RSRP measurement with measurement gaps. A UE indicating support for this component shall also indicate support for ltm-InterFreqMeasGap-r18.; BC; No; N/A; N/A. maxNeighCellsPerFreqLayerL1-Meas-r18 (R4 39-3-2) Indicates the number of neighbouring cells per frequency layer for L1-RSRP measurement. This capability signalling comprises of the following parameters: - supportedMaxNeighCellsPerFreqLayersWithoutGaps-r18 indicates the max number of neighbour cells UE can measure for L1-RSRP per frequency layer for intra-frequency or inter-frequency without measurement gaps. A UE indicating support for this component shall also indicate support for intraFreqL1-MeasConfig-r18 or interFreqSSB-L1-MeasWithoutGaps-r18. - supportedMaxNeighCellsPerFreqLayersWithGaps-r18 indicates the max number of neighbour cells UE can measure for L1-RSRP per frequency layer for inter-frequency with measurement gaps. A UE indicating support for this component shall also indicate support for ltm-InterFreqMeasGap-r18.; BC; No; N/A; N/A. supportedMaxCellsWithoutGapsL1-Meas-r18 (R4 39-3-3) Indicates the max number of total cells of serving cells and neighbouring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement. A UE indicating support for this feature shall also indicate support for intraFreqL1-MeasConfig-r18 or interFreqSSB-L1-MeasWithoutGaps-r18.; BC; No; N/A; N/A. supportedMaxSSB-WithinSlotL1-Meas-r18 (R4 39-3-4) Indicates the max number of SSB resources for L1-RSRP measurement that UE can measure within a slot across candidate cells for intra- and inter-frequency without gap L1-RSRP measurement. A UE indicating support for this feature shall also indicate support for intraFreqL1-MeasConfig-r18 or interFreqSSB-L1-MeasWithoutGaps-r18.; BC; No; N/A; N/A. maxSSB-PerFreqLayerL1-Meas-r18 (R4 39-3-5) Indicates the maximum number of SSB resources for L1-RSRP measurement per frequency layer UE can measure. This capability signalling comprises of the following parameters: - supportedMaxSSB-PerFreqLayersWithoutGaps-r18 indicates the max number of SSB resources UE can measure for L1-RSRP per frequency layer for intra-frequency or inter-frequency without measurement gaps. A UE indicating support for this component shall also indicate support for intraFreqL1-MeasConfig-r18 or interFreqSSB-L1-MeasWithoutGaps-r18. - supportedMaxSSB-PerFreqLayersWithGaps-r18 indicates the max number of SSB resources UE can measure for L1-RSRP per frequency layer for inter-frequency with measurement gaps. A UE indicating support for this component shall also indicate support for ltm-InterFreqMeasGap-r18.; BC; No; N/A; N/A. supportedMaxSSB-L1-Meas-r18 (R4 39-3-6) Indicates the max number of total SSB resources of serving cells and neighbouring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement. A UE indicating support for this feature shall also indicate support for intraFreqL1-MeasConfig-r18 or interFreqSSB-L1-MeasWithoutGaps-r18.; BC; No; N/A; N/A. RAN2 plans to agree corresponding CRs for TS 38.306 and TS 38.331 at the next RAN2 meeting. To RAN1, RAN4 ACTION: RAN2 kindly ask RAN1 and RAN4 to take into account the above information and inform RAN2 if they see an issue. |
| --- | --- | --- |
| [8] | CATT | In [2], RAN2 sent an LS to RAN1 and RAN4 to ask two questions related to LTM L1 intra and inter-frequency measurements: 45-1; Intra-frequency L1 measurement and reports for L1-L2 Triggered Mobility (LTM) procedure. 45-1a; Inter-frequency L1 measurement and reports for L1-L2 Triggered Mobility (LTM) procedure. Question 1: Are the above intra-frequency and inter-frequency L1 measurement and reporting features (45-1 and 45-1a) prerequisites to support intra-frequency and inter-frequency LTM, respectively? Question 2: The above features, 45-1 and 45-1a, from RAN1 and related RAN4 features (39-1, 39-2, 39-3-1, 39-3-2, 39-3-3, 39-3-4, 39-3-5, 39-3-6) are defined per BC for both intra-frequency and inter-frequency measurements. RAN2 would like check with RAN1/4 for which BC (e.g. BC of current serving cells, BC including current serving cells and cell to be measured or something else) these capabilities are to be considered for L1 intra-frequency and inter-frequency LTM measurements?. In [3], for Question 2, RAN4 has provided the following response: Reply to the Question 2: RAN4 discussed the following options. Option 1 is supported by majority companies in RAN4, but RAN4 cannot reach consensus on option 1. Meanwhile, RAN4 discussed the compromised option in Option 3, and the feasibility and details are up to RAN2. - Option 1: The BC granularity is BC of serving cells. - Option 2: The BC granularity is BC including serving cells and candidate cells to be measured. - Option 3: The UE capability is based on option 1 and introduce additional UE capability to accommodate option 2. From RAN4 perspective, the UE capabilities can be implemented into the specification based on RAN2 conclusion.. In the latest incoming LS [1], RAN2 agreed to introduce a new UE capability to allow the UE to indicate, per UE, that the SSBs of LTM candidate cells on which the UE can perform inter-frequency L1 measurements are limited to the BCs in which the UE indicates support of inter-frequency L1 measurements. And such a restriction applies to all UE capabilities related to inter-frequency L1 measurements of SSBs of candidate cells. This means RAN2 has decided to support Option 3, i.e., the existing capability interFreqL1-MeasConfig-r18 and the new UE capability are used to accommodate Option 1 and Option 2, respectively. In our view, there is no issue on introducing the new UE capability. Observation 1: There is no issue on the new per UE capability for inter-frequency L1 measurements for LTM and thus no need to provide a reply LS back to RAN2. |
| [9] | Ericsson | FG 45-1a is signalled per BC, but it has been unclear how this BC is related to the frequencies where the LTM measurements are performed. For example, if the UE has serving cells in bands A and B, indicates support of inter-frequency L1 measurements in a BC (A, B, C) and supports LTM in bands A, B, C, D, E, F and G, on what frequencies can the UE perform inter-frequency measurements? In [1], RAN2 decided that for this case, the UE can perform measurements on candidate cells in bands A, B, C, D, E, F and G. This was in accordance with the proposal in [2], which was unfortunately not agreed in RAN1#118bis. RAN2 also agreed to introduce a new FG, which provides the UE with the possibility to signal that it can only perform inter-frequency measurements within the BC. For the example above, the UE would only support LTM measurements on candidate cells in bands A, B and C, but not on candidate cells in bands D, E, F or G. RAN1 discussed the corresponding issue, see e.g. [3][4][5], but without any agreement. Fundamentally, the solutions proposed by RAN2 are fine. However, we still believe that we should clarify the operation of FG 45-1a, stating that if a UE supports FG 45-1a for a BC, it means that it supports inter-frequency measurements in any band it supports: Clarify with a note in FG 45-1a that a UE that supports 45-1a supports inter-frequency measurements on candidate cells on any band where the UE supports LTM. We also propose to include that in an LS response to RAN2: Send an LS response to RAN2 requesting them to clarify FG 45-1a. A draft LS response is provided in the appendix. 1. Overall Description: RAN1 thanks RAN2 for the LS on UE capabilities for inter-frequency L1 measurements for LTM. RAN1 agrees with RAN2’s interpretation that a UE that supports 45-1a for a BC support inter-frequency L1-RSRP measurement and reporting for candidate cells on any band on which the UE supports LTM. However, this should be clarified in the feature description for FG 45-1a (interFreqL1-MeasConfig-r18). ACTION: RAN1 respectfully asks RAN2 to capture this in the feature description for interFreqL1-MeasConfig-r18. |

(Closed) Proposal 4-1:

Add following note for FG45-1a.

- Note: the candidate cell to be measured can be on any band where the UE supports LTM

| Company | Comment |
| --- | --- |
| Moderator | Based on the inputs, the issue was discussed and solved in RAN2, and there would be no company having concern on the RAN2’s solution. So, the discussion point in RAN1 is only about whether the note proposed by Ericsson needs to be captured into the RAN1 UE features list or not. The proposed note is aligned with following example described in RAN2 LS. In that sense, it may be no problem to capture it, while it may not be necessary to capture it. The moderator would like to ask companies’ views on the proposal. In addition, regarding whether RAN1 should provide reply LS on this issue or not, the moderator thinks separate reply LS is not necessary and if RAN1 needs to mention something, it can be captured in the LS on updated RAN1 UE features list. --- For example, if the UE has serving cells in bands A and B, indicates support of inter-frequency L1 measurements in a BC (A, B, C) and supports LTM in bands A, B, C, D, E, F and G, the UE can perform inter-frequency L1 measurements of SSBs of candidate cells in bands A, B, C, D, E, F and G. |
| Nokia | Support with some editorial changes: Note: the SSBs of candidate cell(s) to be measured can be on any band where the UE supports LTM |
| CATT | Ok. Also with Nokia’s modification of the note. |
| ZTE | We are fine with adding the note into FG45-1a, and such general description is enough. |
| NTT DOCOMO | We are fine with adding the note to clarify the operation. |
| Huawei, HiSilicon | Agree with the moderator that a separate reply LS to RAN2 LS is not necessary. As the for the note on FG45-1a, I remembered it was discussed in last meeting but not have agreement on it. I am wondering what is changed after then. As RAN2 will directly work on 38.306, the changing in the RAN1 table may not be necessary because anyway the decision was made by RAN2 and RAN1 may not need to reflect every changes in the table made by other WGs. But we can go with majority if only HW have concern. |
| Moderator | Based on the inputs, it seems companies may be ok with adding the note although it may not be so essential. Let’s check if Nokia’s version is ok for all in online session. Proposed agreement: Add following note for FG45-1a. - Note: the SSBs of candidate cell(s) to be measured can be on any band where the UE supports LTM |
| Moderator | Agreement: Add following note for FG45-1a. - Note: the SSBs of candidate cell(s) to be measured can be on any band where the UE supports LTM |

- NR_pos_enh2

Discussion

Additional Measurements for SL-TDOA and SL-TOA

There is a LS from RAN2 on additional measurement for SL-TDOA and SL-TOA, and there are questions related to RAN1 UE capabilities FG 41-1-7a and 41-1-7b.

| [10] | RAN2 | RAN2 observed that the LS in R2-2407909 (R1-2407387) with the latest RAN1 feature list for Rel-18 NR (R1-2407385) contains the following note: For NR_pos_enh2, with regards to FGs 41-1-7a and FG 41-1-7b, RAN1 respectfully informs RAN2 that UE capability components have been specified, but there is no corresponding report specified in the ProvideLocationInformation message of TDOA and TOA methods.. However, there was no specific action for RAN2 other than “take into account the RAN1 NR UE features”. RAN2 discussed the above issue at RAN2#127bis and have the following questions for RAN1: Question 1: Is RAN2 requested to add signalling to enable a SL-TDOA/SL-TOA capable UE to report additional measurements (up to 3 additional SL-TDOA/SL-TOA measurements according to UE capabilities)? Question 2: If the answer to Question 1 is ‘yes’, what should the additional SL-TDOA/SL-TOA measurements comprise? Do the additional SL-TDOA/SL-TOA measurements include the SL-RSTD/SL-RTOA measurements only (as FGs 41-1-7a and FG 41-1-7b seems to imply), or can the additional SL-TDOA/SL-TOA measurements also include the SL-PRS RSRP/RSRPP measurements together with other currently reported information such as SL-PRS Resource ID (sl-PRS-ResourceId-r18), LOS/NLOS indicator (los-NLOS-Indicator-r18), additional paths measurements (sl-TDOA-AdditionalPathList-r18/sl-TOA-AdditionalPathList-r18), timing quality (sl-TimingQuality-r18), and/or time stamp (sl-TimeStamp-r18)? Question 3: If additional SL-RSTD/SL-RTOA (and probably SL-PRS RSRP/RSRPP) measurement reporting is required, should the encoding of the additional SL-RSTD/SL-RTOA (and probably SL-PRS RSRP/RSRPP) measurements be defined relative to the 1st/main SL-RSTD/SL-RTOA measurement (or 1st/main SL-PRS RSRP/RSRPP measurement)? I.e., analogous to LPP additional measurement reporting where the e.g., additional RSTD measurements are defined relative to the 1st/main RSTD measurement (e.g., nr-RSTD-ResultDiff in LPP). If so, what would be the value range of the additional SL-RSTD/SL-RTOA (and probably SL-PRS RSRP/RSRPP) measurements relative to the 1st/main SL-RSTD/SL-RTOA (and probably SL-PRS RSRP/RSRPP) measurement? To RAN WG1 ACTION: RAN2 respectfully asks RAN1 to provide answers to the above questions. |
| --- | --- | --- |

Since RAN1 answer is required, there are following inputs in AI 5 related to this issue.

| [11] | vivo | For the above question, RAN1 would like to answer one by one. For Q1, RAN1 has different views on whether to add signalling to enable a SL-TDOA/SL-TOA capable UE to report additional measurements (up to 3 additional SL-TDOA/SL-TOA measurements according to UE capabilities). So RAN1 added the note for the FGs 41-1-7a and FG 41-1-7b and up to RAN2 to decide whether to add signalling to enable a SL-TDOA/SL-TOA capable UE to report additional measurements (up to 3 additional SL-TDOA/SL-TOA measurements according to UE capabilities) For Q2: If RAN2 decides to add the signalling, it can include the additional SL-TDOA/SL-TOA measurements, and can also include the SL-PRS RSRP/RSRPP measurements together with other currently reported information such as SL-PRS Resource ID (sl-PRS-ResourceId-r18), LOS/NLOS indicator (los-NLOS-Indicator-r18), additional paths measurements (sl-TDOA-AdditionalPathList-r18/sl-TOA-AdditionalPathList-r18), timing quality (sl-TimingQuality-r18), and/or time stamp (sl-TimeStamp-r18) For Q3: If RAN2 decides to add the signalling, for the SL-TDOA, the differential RSRP (e.g., DL-PRS-RSRP-ResultDiff), differential RSTD (e.g., nr-RSTD-ResultDiff in LPP) relative to the 1st/main SL-RSTD can be directly reused. In addition, the value range can be the same as the value range of additional measurements in LPP. However, differential reporting may not be supported for SL-TOA since differential reporting of additional measurement is not supported in NRPPa. |
| --- | --- | --- |
| [12, 13] | CATT, CICTCI | - FG 41-1-7a, 41-1-7b and 41-1-7a were introduced in the SLPP specification correctly as a UE capability for SL-PRS measurement for SL RSTD, SL RTOA and SL UE Rx – Tx time difference, respectively. The 3rd component of these FGs indicates the maximum number of the measurement reporting for different SL-PRS reception for the same pair of UEs. 41-1-7a; SL PRS measurement for SL-RSTD; 1. Support SL RSTD measurement based on SL-PRS 2. Support SL RSTD measurement reporting 3. Maximum number of SL RSTD measurement reporting for different SL-PRS reception for the same pair of UEs. 41-1-7b; SL PRS measurement for SL RTOA; 1. Support SL RTOA measurement based on SL-PRS 2. Support SL RTOA measurement reporting 3. Maximum number of SL RTOA measurementreporting for different SL-PRS reception for the same pair of UEs. 41-1-7c; SL PRS measurement for UE Rx – Tx time difference without Tx time stamp; 1. Support UE Rx – Tx time difference measurement based on SL PRS 2. Support UE Rx – Tx time difference measurement reporting without Tx time stamp 3. Maximum number of Rx-Tx measurement reporting for different SL-PRS reception for the same pair of UEs. - In RAN1#114, RAN1 reached the following agreement for UE to report multiple Rx – Tx time difference measurements for different SL-PRS reception for the same pair of UEs. Agreement (RAN1#114) Confirm the following working assumption with update: Working assumption Support to indicate to UE(s) with higher layer signaling to report multiple Rx-Tx measurements for the same SL PRS transmission (resp. reception) and different SL PRS receptions (resp. transmissions) for the same pair of UE(s). - FFS: whether the different SL PRS receptions correspond to the same or different SL PRS resources - Note: reporting a single Rx-Tx measurement is also supported - Note: The indicated Rx-Tx time difference measurement is based on actual Tx time... - The 3rd component of 41-1-7c “3. Maximum number of Rx-Tx measurement reporting for different SL-PRS reception for the same pair of UEs” matches the above agreement for SL Rx-Tx measurement reporting. That is, although FG 41-1-7a and 41-1-7b include the 3rd component for UE’s capability of maximum number of SL RSTD/RTOA measurement reporting for different SL-PRS reception for the same pair of UEs, RAN1 actually has no agreement to reporting multiple SL RSTD/RTOA measurements for different SL-PRS reception for the same pair of UEs. The following two options were discussed in RAN1#118 to resolve the problem []: - Proposal: With regards to FG 41-1-7a and FG 41-1-7b, consider the following options: - Option 1: Remove component 3 from both FGs, since there is no corresponding feature specified in SLPP - Option 2: Send an LS to RAN2 to inform them that this UE capability component has been specified, but there is no corresponding report specified in the ProvideLocationInformation message of TDOA and TOA methods. Option 2 was agreed in RAN1#118. However, the wording “there is no corresponding report specified in the ProvideLocationInformation message of TDOA and TOA methods”, causes some confusion in RAN2, which triggered RAN2 to send the LS R1-2409344 (R2-2409256), asking the questions shown in the Introduction. As discussed in RAN1#118 [3], some companies commented that the reporting of multiple SL RSTD/RTOA are already supported in the existing measurement reporting IEs, and thus there is no need to have additional work on the report of the SL RSTD/RTOA measurements. SL-TDOA-SignalMeasurementInformation ::= SEQUENCE { sl-TDOA-MeasList SEQUENCE (SIZE(1..maxNrOfUEs)) OF SL-TDOA-MeasElementPerARP-ID-Rx, ... } SL-TDOA-MeasElementPerARP-ID-Rx ::= SEQUENCE (SIZE(1..4)) OF SL-TDOA-MeasElement We share the similar view that no additional signaling enhancement is needed for reporting SL RSTD/RTOA measurements. To avoid the confusion in other WGs, we proposal to make the following changes on the note included in LS R1-2407387 [1]. In RAN1#118, RAN1 sent the LS R1-2407387 [1] with the latest RAN1 feature list for Rel-18 NR to RAN2, which contains the following note: For NR_pos_enh2, with regards to FGs 41-1-7a and FG 41-1-7b, RAN1 respectfully informs RAN2 that UE capability components have been specified, but there is no corresponding report specified in the ProvideLocationInformation message of TDOA and TOA methods. Proposal 1: Provide the following change to note sent to RAN2: For NR_pos_enh2, with regards to FGs 41-1-7a and FG 41-1-7b, RAN1 respectfully informs RAN2 that UE capability components have been specified corresponding to the report specified in the ProvideLocationInformation message of TDOA and TOA methods. RAN1 does not foresee the need of further signaling enhancement in reporting SL RSTD/RTOA for FGs 41-1-7a and FG 41-1-7b. For the question raised by RAN2, we support providing the following responses: Proposal 2: Provide the following response to RAN2’s question: Question 1: Is RAN2 requested to add signalling to enable a SL-TDOA/SL-TOA capable UE to report additional measurements (up to 3 additional SL-TDOA/SL-TOA measurements according to UE capabilities)? RAN1’s response: No No need to add additional signalling for SL-RSTD/SL-RTOA measurement reporting. The report of SL-TDOA/SL-TOA according to UE capabilities is already supported by existing signalling. Question 2: If the answer to Question 1 is ‘yes’, what should the additional SL-TDOA/SL-TOA measurements comprise? Do the additional SL-TDOA/SL-TOA measurements include the SL-RSTD/SL-RTOA measurements only (as FGs 41-1-7a and FG 41-1-7b seems to imply), or can the additional SL-TDOA/SL-TOA measurements also include the SL-PRS RSRP/RSRPP measurements together with other currently reported information such as SL-PRS Resource ID (sl-PRS-ResourceId-r18), LOS/NLOS indicator (los-NLOS-Indicator-r18), additional paths measurements (sl-TDOA-AdditionalPathList-r18/sl-TOA-AdditionalPathList-r18), timing quality (sl-TimingQuality-r18), and/or time stamp (sl-TimeStamp-r18)? RAN1’s response: No additional SL-RSTD/SL-RTOA measurement reporting is required. Question 3: If additional SL-RSTD/SL-RTOA (and probably SL-PRS RSRP/RSRPP) measurement reporting is required, should the encoding of the additional SL-RSTD/SL-RTOA (and probably SL-PRS RSRP/RSRPP) measurements be defined relative to the 1st/main SL-RSTD/SL-RTOA measurement (or 1st/main SL-PRS RSRP/RSRPP measurement)? I.e., analogous to LPP additional measurement reporting where the e.g., additional RSTD measurements are defined relative to the 1st/main RSTD measurement (e.g., nr-RSTD-ResultDiff in LPP). If so, what would be the value range of the additional SL-RSTD/SL-RTOA (and probably SL-PRS RSRP/RSRPP) measurements relative to the 1st/main SL-RSTD/SL-RTOA (and probably SL-PRS RSRP/RSRPP) measurement? RAN1’s response: No additional SL-RSTD/SL-RTOA measurement reporting is required. |
| [14, 15] | OPPO | According to the updated RAN1 UE features list for Rel-18 NR after RAN1#118 [2], maximum number of SL RSTD/RTOA measurement reporting for different SL-PRS reception for the same pair of UEs is up to 4, as copied below. That is up to 3 additional SL-TDOA/SL-TOA measurements may be needed according to UE capabilities. However, according to the latest SLPP protocol [3] there is no IE in SL-TDOA-ProvideLocationInformation/ SL-TOA-ProvideLocationInformation message for any additional measurement report. Obviously, some IE similar as NR-DL-TDOA-AdditionalMeasurements-r16 in LPP protocol is missing in current SLPP protocol. 41. NR_pos_enh2; 41-1-7a; SL PRS measurement for SL-RSTD; 1. Support SL RSTD measurement based on SL-PRS 2. Support SL RSTD measurement reporting 3. Maximum number of SL RSTD measurement reporting for different SL-PRS reception for the same pair of UEs; 41-1-1; No; No; UE does not support SL PRS measurement for SL-RSTD; Per band; n/a; n/a; n/a; Need for location server/ UE to know if the feature is supported Component 3 candidate values: {1,2,3,4}; Optional with capability signaling. 41. NR_pos_enh2; 41-1-7b; SL PRS measurement for SL RTOA; 1. Support SL RTOA measurement based on SL-PRS 2. Support SL RTOA measurement reporting 3. Maximum number of SL RTOA measurement reporting for different SL-PRS reception for the same pair of UEs; 41-1-1; No; No; UE does not support SL PRS measurement for SL RTOA; Per band; n/a; n/a; n/a; Need for location server/ UE to know if the feature is supported Component 3 candidate values: {1,2,3,4}; Optional with capability signaling. Nevertheless, reporting additional measurements is an optional UE feature, and even additional signalling was not introduced, the measurement for different SL-PRS reception can still be reported with separate SL-TDOA-ProvideLocationInformation/ SL-TOA-ProvideLocationInformation messages. As ASN.1 has already frozen it is not desirable to add signalling to report additional measurements. Proposed reply: - As reporting additional measurements is an optional UE feature, and even additional signalling was not introduced, the measurement for different SL-PRS reception can still be reported with separate SL-TDOA-ProvideLocationInformation/ SL-TOA-ProvideLocationInformation messages. Thus, RAN2 is NOT requested to add signalling to enable a SL-TDOA/SL-TOA capable UE to report additional measurements. |
| [16] | Intel Corporation | RAN1 would like to respond to the questions from RAN2 as below. Question 1: Is RAN2 requested to add signalling to enable a SL-TDOA/SL-TOA capable UE to report additional measurements (up to 3 additional SL-TDOA/SL-TOA measurements according to UE capabilities)? Answer 1: Yes, as pointed out in the earlier RAN1 LS in R2-2407909 (R1-2407387), the signalling support to report up to 3 additional SL-TDOA/SL-TOA measurements is intended to be introduced to TS 38.355. Question 2: If the answer to Question 1 is ‘yes’, what should the additional SL-TDOA/SL-TOA measurements comprise? Do the additional SL-TDOA/SL-TOA measurements include the SL-RSTD/SL-RTOA measurements only (as FGs 41-1-7a and FG 41-1-7b seems to imply), or can the additional SL-TDOA/SL-TOA measurements also include the SL-PRS RSRP/RSRPP measurements together with other currently reported information such as SL-PRS Resource ID (sl-PRS-ResourceId-r18), LOS/NLOS indicator (los-NLOS-Indicator-r18), additional paths measurements (sl-TDOA-AdditionalPathList-r18/sl-TOA-AdditionalPathList-r18), timing quality (sl-TimingQuality-r18), and/or time stamp (sl-TimeStamp-r18)? Answer 2: In addition to the additional SL-RSTD/SL-RTOA measurements, the additional SL-TDOA/SL-TOA measurements should also include the SL-PRS RSRP/RSRPP measurements together with other currently reported information such as SL-PRS Resource ID (sl-PRS-ResourceId), LOS/NLOS indicator (los-NLOS-Indicator), additional paths measurements (sl-TDOA-AdditionalPathList/sl-TOA-AdditionalPathList), timing quality (sl-TimingQuality), time stamp (sl-TimeStamp) and/or sl-POS-ARP-ID-Rx. Question 3: If additional SL-RSTD/SL-RTOA (and probably SL-PRS RSRP/RSRPP) measurement reporting is required, should the encoding of the additional SL-RSTD/SL-RTOA (and probably SL-PRS RSRP/RSRPP) measurements be defined relative to the 1st/main SL-RSTD/SL-RTOA measurement (or 1st/main SL-PRS RSRP/RSRPP measurement)? I.e., analogous to LPP additional measurement reporting where the e.g., additional RSTD measurements are defined relative to the 1st/main RSTD measurement (e.g., nr-RSTD-ResultDiff in LPP). If so, what would be the value range of the additional SL-RSTD/SL-RTOA (and probably SL-PRS RSRP/RSRPP) measurements relative to the 1st/main SL-RSTD/SL-RTOA (and probably SL-PRS RSRP/RSRPP) measurement? Answer 3: Yes, the encoding of the additional SL-RSTD/SL-RTOA as well as SL-PRS RSRP/RSRPP measurements are intended to be defined relative to the first/main SL-RSTD/SL-RTOA/SL-PRS RSRP/RSRPP measurements similar to additional measurement reporting per LPP. The value ranges for differential SL-RSTD and SL-RTOA information can be same as that for nr-RSTD-ResultDiff-r16 (TS 37.355) for reporting of additional NR-DL-TDOA measurements, i.e., k0 INTEGER (0..8191), k1 INTEGER (0..4095), k2 INTEGER (0..2047), k3 INTEGER (0..1023), k4 INTEGER (0..511), k5 INTEGER (0..255) The value range for differential SL-PRS RSRP and SL-PRS RSRPP information can be same as that for nr-DL-PRS-RSRP-ResultDiff-r16, i.e., INTEGER (0..61). Note that the value ranges for the information elements recommended above are from the perspective of RAN1 and are subject to review by RAN4. |
| [17] | Huawei, HiSilicon | Currently, a SL-TDOA/SL-TOA capable UE can obtain multiple SL-TDOA/SL-TOA measurements. UE needs to report additional SL-TDOA/SL-TOA measurements to enable more accurate positioning. From RAN1 point of view, we suggest RAN2 to add signalling to enable a SL-TDOA/SL-TOA capable UE to report additional measurements. If UE reports multiple SL-TDOA/SL-TOA measurements, the additional SL-TDOA/SL-TOA measurements should comprise as follows: - the SL-RSTD/SL-RTOA measurements (as FGs 41-1-7a and FG 41-1-7b seems to imply) - the SL-PRS RSRP/RSRPP measurements - the other currently reported information - SL-PRS Resource ID (sl-PRS-ResourceId-r18) - LOS/NLOS indicator (los-NLOS-Indicator-r18) - additional paths measurements (sl-TDOA-AdditionalPathList-r18/sl-TOA-AdditionalPathList-r18) - timing quality (sl-TimingQuality-r18) and/or time stamp (sl-TimeStamp-r18) In RAN2 specification TS 38.355, there is already a per-ARP reporting framework. It has not been quite clear whether the per-ARP measurement reporting can actually contain the multiple measurement of the same ARP. For example, the filed sl-POS-ARP-ID-Rx may be the same in the list of SL-TDOA-MeasElementPerARP-ID-Rx, which at least seems not to be precluded by the specification. In addition, we think that 4x4=16 measurements per transmit UE is too much for SL positioning, given that UE may not have so many Tx beams as a gNB. Having 4 measurements per transmit UE seems sufficient for SL location fix, and the Rx UE could assign the up to 4 measurements to the same Rx ARP or across multiple Rx ARPs. SL-TDOA-MeasElementPerARP-ID-Rx ::= SEQUENCE (SIZE(1..4)) OF SL-TDOA-MeasElement SL-TDOA-MeasElement ::= SEQUENCE { applicationLayerID OCTET STRING OPTIONAL, -- Cond FirstElement los-NLOS-Indicator LOS-NLOS-Indicator OPTIONAL, -- sl-losNlosIndicator sl-POS-ARP-ID-Rx INTEGER (1..4) OPTIONAL, -- sl-pos-arpID-Rx sl-PRS-ResourceId INTEGER (0..16) OPTIONAL, -- sl-PRS-ResourceId sl-PRS-RSRP-Result INTEGER (0..126) OPTIONAL, -- sl-PRS-RSRP sl-PRS-RSRPP-Result INTEGER (0..126) OPTIONAL, -- sl-PRS-RSRPP sl-RSTD-Result CHOICE { k0 INTEGER (0..1970049), k1 INTEGER (0..985025), k2 INTEGER (0..492513), k3 INTEGER (0..246257), k4 INTEGER (0..123129), k5 INTEGER (0..61565) } OPTIONAL, -- sl-PRS-RSTD sl-TDOA-AdditionalPathList SL-TDOA-AdditionalPathList OPTIONAL, sl-TimeStamp SL-TimeStamp OPTIONAL, -- sl-Timestamp sl-TimingQuality SL-TimingQuality OPTIONAL, -- sl-TimingQuality ... } In light of this, we suggest to follow the per ARP reporting, i.e. no change to existing reporting IE, i.e. up to 4 per Rx ARP measurements, which can be associated with the same Rx ARP ID or different Rx ARP IDs. If additional SL-RSTD/SL-RTOA (and probably SL-PRS RSRP/RSRPP) measurement reporting is required, we suggest the encoding (e.g. absolute/relative) of the additional SL-RSTD/SL-RTOA (and probably SL-PRS RSRP/RSRPP) measurements to follow the per Rx ARP measurement encoding scheme. Based on the above analysis, we propose the following responses to RAN2: Proposal 1: Reply to RAN2 Q1 that RAN1 suggests RAN2 to add signalling to enable a SL-TDOA/SL-TOA capable UE to report additional measurements. Proposal 2: Reply to RAN2 Q2 that RAN1 suggests the additional SL-TDOA/SL-TOA measurements to follow the per ARP reporting, i.e. no change to existing reporting IE, i.e. up to 4 per Rx ARP measurements, which can be associated with the same Rx ARP ID or different Rx ARP IDs. Proposal 3: Reply to RAN2 Q3 that RAN1 suggests the encoding (absolute/relative) of the additional SL-RSTD/SL-RTOA (and probably SL-PRS RSRP/RSRPP) measurements to follow the existing per Rx ARP measurement encoding scheme. |
| [18, 19] | ZTE Corporation, Sanechips | In Rel-16 NR positioning, FG 13-6 is supported for enabling UE to provide up to 3 additional RSTD measurements per pair of TRPs. On top of that, in Rel-17 NR positioning, the maximum number of UE Rx TEG which can be supported and reported by UE is 8 according to FG27-1-1. With that, the maximum number of additional RSTD measurements supported in Rel-17 is extended to 31 (up to 8 measurements of different Rx TEGs for each RSTD measurement). NR-DL-TDOA-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF NR-DL-TDOA-AdditionalMeasurementElement-r16 NR-DL-TDOA-AdditionalMeasurementsExt-r17 ::= SEQUENCE (SIZE (1..maxAddMeasTDOA-r17)) OF NR-DL-TDOA-AdditionalMeasurementElement-r16 maxAddMeasTDOA-r17 INTEGER ::= 31 13-6; DL PRS Measurement Report for DL-TDOA; 1. DL RSTD measurements per pair of TRPs. Values = {1, 2, 3, 4} 2. Support DL PRS-RSRP measurements. Values = {0, 1}; Need for location server to know if the feature is supported.. 27-1-1; UE-RxTEGs for UE-assisted DL TDOA and/or Multi-RTT positioning; 1. Support of UE-RxTEGs for UE-assisted DL TDOA and/or Multi-RTT positioning 2. The maximum number of UE-RxTEG, which is supported and reported by UE for UE assisted DL TDOA and/or Multi-RTT positioning; Component 1 candidate values: {UE-assisted DL TDOA, Multi-RTT positioning, UE-assisted DL TDOA and Multi-RTT positioning} Component 2 candidate values: {1, 2, 3, 4, 6, 8} Note: a single value is reported when both multi-RTT and DL-TDOA are supported Need for location server to know if the feature is supported If the UE does not include RxTEG-ID associated with a measurement, no assumption can be made on the UE Rx timing errors for this measurement Note: The "per band" reporting on this capability does not imply, that the RxTEG IDs in the measurement report are grouped per band; In the measurement report, the RxTEG ID can span from 0, up to 31. - For Question 1 Similarly, for SL positioning, the following FGs are supported for SL measurements. FG 41-1-7a and 41-1-7b support UE reporting up to 4 SL RTOA/RSTD for different SL PRS reception for the same pair of UEs (1 + 3 additional measurements). 41-1-7a; SL PRS measurement for SL-RSTD; 1. Support SL RSTD measurement based on SL-PRS 2. Support SL RSTD measurement reporting 3. Maximum number of SL RSTD measurement reporting for different SL-PRS reception for the same pair of UEs; Need for location server/ UE to know if the feature is supported Component 3 candidate values: {1,2,3,4}. 41-1-7b; SL PRS measurement for SL RTOA; 1. Support SL RTOA measurement based on SL-PRS 2. Support SL RTOA measurement reporting 3. Maximum number of SL RTOA measurement reporting for different SL-PRS reception for the same pair of UEs; Need for location server/ UE to know if the feature is supported Component 3 candidate values: {1,2,3,4}. 41-1-19a; Report of Rx ARP-ID with SL positioning measurements; Maximum number of Rx ARP-IDs with SL positioning measurements; Component candidate values: {2,3,4} Need for location server/UE to know if the feature is supported. However, current spec only captures the capability regarding the maximum number of Rx ARP-IDs with SL positioning measurement. If we follow the logic of Uu positioning considering FG41-1-7a, 41-1-7b, 41-1-19a, the maximum number of SL-TDOA/SL-TOA measurements for a single pair of UEs that can be reported by UE is 4*4=16. SL-TDOA-SignalMeasurementInformation ::= SEQUENCE { sl-TDOA-MeasList SEQUENCE (SIZE(1..maxNrOfUEs)) OF SL-TDOA-MeasElementPerARP-ID-Rx, ... } SL-TDOA-MeasElementPerARP-ID-Rx ::= SEQUENCE (SIZE(1..4)) OF SL-TDOA-MeasElement SL-TOA-ProvideLocationInformation ::= SEQUENCE { sl-TOA-SignalMeasurementInformation SL-TOA-MeasElementPerARP-ID-Rx OPTIONAL, sl-TOA-Error SL-TOA-LocationInformationError OPTIONAL, ... } SL-TOA-MeasElementPerARP-ID-Rx ::= SEQUENCE (SIZE(1..4)) OF SL-TOA-MeasElement - For Question 2 Additional SL-TDOA/SL-TOA measurements can include the SL-RSTD/SL-RTOA measurements, and optionally include the SL-PRS RSRP/RSRPP measurements together with other currently reported information such as SL-PRS Resource ID (sl-PRS-ResourceId-r18), LOS/NLOS indicator (los-NLOS-Indicator-r18), additional paths measurements (sl-TDOA-AdditionalPathList-r18/sl-TOA-AdditionalPathList-r18), timing quality (sl-TimingQuality-r18), time stamp (sl-TimeStamp-r18). NR-DL-TDOA-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF NR-DL-TDOA-AdditionalMeasurementElement-r16 NR-DL-TDOA-AdditionalMeasurementsExt-r17 ::= SEQUENCE (SIZE (1..maxAddMeasTDOA-r17)) OF NR-DL-TDOA-AdditionalMeasurementElement-r16 NR-DL-TDOA-AdditionalMeasurementElement-r16 ::= SEQUENCE { nr-DL-PRS-ResourceID-r16 NR-DL-PRS-ResourceID-r16 OPTIONAL, nr-DL-PRS-ResourceSetID-r16 NR-DL-PRS-ResourceSetID-r16 OPTIONAL, nr-TimeStamp-r16 NR-TimeStamp-r16, nr-RSTD-ResultDiff-r16 CHOICE { k0-r16 INTEGER (0..8191), k1-r16 INTEGER (0..4095), k2-r16 INTEGER (0..2047), k3-r16 INTEGER (0..1023), k4-r16 INTEGER (0..511), k5-r16 INTEGER (0..255), ..., kMinus6-r18 INTEGER (0..524224), kMinus5-r18 INTEGER (0..262112), kMinus4-r18 INTEGER (0..131056), kMinus3-r18 INTEGER (0..65528), kMinus2-r18 INTEGER (0..32764), kMinus1-r18 INTEGER (0..16382) }, nr-TimingQuality-r16 NR-TimingQuality-r16, nr-DL-PRS-RSRP-ResultDiff-r16 INTEGER (0..61) OPTIONAL, nr-AdditionalPathList-r16 NR-AdditionalPathList-r16 OPTIONAL, ..., [[ nr-UE-Rx-TEG-ID-r17 INTEGER (0..maxNumOfRxTEGs-1-r17) OPTIONAL, nr-DL-PRS-FirstPathRSRP-ResultDiff-r17 INTEGER (0..61) OPTIONAL, nr-los-nlos-IndicatorPerResource-r17 LOS-NLOS-Indicator-r17 OPTIONAL, nr-AdditionalPathListExt-r17 NR-AdditionalPathListExt-r17 OPTIONAL ]], [[ nr-MeasBasedOnAggregatedResources-r18 ENUMERATED {true} OPTIONAL, nr-AggregatedDL-PRS-ResourceInfo-List-r18 SEQUENCE (SIZE (2.. 3)) OF NR-AggregatedDL-PRS-ResourceInfo-Element-r18 OPTIONAL, nr-RSCPD-r18 INTEGER (0..3599) OPTIONAL, nr-PhaseQuality-r18 NR-PhaseQuality-r18 OPTIONAL, nr-RSCPD-AdditionalMeasurementsAddSamples-r18 SEQUENCE (SIZE (1..nrNumOfSamples-1-r18 )) OF NR-RSCPD-AdditionalMeasurementSamplesElement-r18 OPTIONAL, nr-ReportDL-PRS-MeasBasedOnSingleOrMultiHopRx-r18 ENUMERATED { singleHop, multipleHop } OPTIONAL ]] - For Question 3 Analogous to LPP additional measurement reporting, additional SL-RSTD/SL-RTOA (and probably SL-PRS RSRP/RSRPP) measurements be defined relative to the 1st/main SL-RSTD/SL-RTOA measurement (or 1st/main SL-PRS RSRP/RSRPP measurement), the value range can be defined as: SL-RSTD/SL-RTOA in additional measurement; k0 INTEGER (0..8191), k1 INTEGER (0..4095), k2 INTEGER (0..2047), k3 INTEGER (0..1023), k4 INTEGER (0..511), k5 INTEGER (0..255),. SL-PRS RSRP/RSRPP in additional measurement; INTEGER (0..61). |
| [20] | Qualcomm Incorporated | RAN1 thanks RAN2’s LS R2-2409256 (R1-2409344) and would like to provide the following RAN1 responses to the following questions: Question 1: Is RAN2 requested to add signalling to enable a SL-TDOA/SL-TOA capable UE to report additional measurements (up to 3 additional SL-TDOA/SL-TOA measurements according to UE capabilities)?. Answer 1: Yes Question 2: If the answer to Question 1 is ‘yes’, what should the additional SL-TDOA/SL-TOA measurements comprise? Do the additional SL-TDOA/SL-TOA measurements include the SL-RSTD/SL-RTOA measurements only (as FGs 41-1-7a and FG 41-1-7b seems to imply), or can the additional SL-TDOA/SL-TOA measurements also include the SL-PRS RSRP/RSRPP measurements together with other currently reported information such as SL-PRS Resource ID (sl-PRS-ResourceId-r18), LOS/NLOS indicator (los-NLOS-Indicator-r18), additional paths measurements (sl-TDOA-AdditionalPathList-r18/sl-TOA-AdditionalPathList-r18), timing quality (sl-TimingQuality-r18), and/or time stamp (sl-TimeStamp-r18)?. Answer 2: The additional measurements can include all the additional information: - SL-PRS RSRP/RSRPP - SL-PRS Resource ID - LOS/NLOS indicator - additional paths measurements - timing quality - time stamp Question 3: If additional SL-RSTD/SL-RTOA (and probably SL-PRS RSRP/RSRPP) measurement reporting is required, should the encoding of the additional SL-RSTD/SL-RTOA (and probably SL-PRS RSRP/RSRPP) measurements be defined relative to the 1st/main SL-RSTD/SL-RTOA measurement (or 1st/main SL-PRS RSRP/RSRPP measurement)? I.e., analogous to LPP additional measurement reporting where the e.g., additional RSTD measurements are defined relative to the 1st/main RSTD measurement (e.g., nr-RSTD-ResultDiff in LPP). If so, what would be the value range of the additional SL-RSTD/SL-RTOA (and probably SL-PRS RSRP/RSRPP) measurements relative to the 1st/main SL-RSTD/SL-RTOA (and probably SL-PRS RSRP/RSRPP) measurement?. Answer 3: There is no need to perform relative encoding of the additional SL-RSTD/SL-RTOA, SL-PRS RSRP/RSRPP measurements. |
| [21, 22] | Ericsson | During RAN1#118, the issue supporting multiple measurement for a given pair of UE was discussed, and comments mentioned the possibility to support multiple measurements can already be supported, since the reporting measurement list contains four possible entries. It was also commented that one issue was that if the measurement list is used to report multiple measurements for a given UE pair and for a given ARP ID, then it is not possible to report multiple measurements over multiple ARP IDs in the same report. However, from our perspective, that would be an optimization at this stage of the release. Regarding Question 1: Is RAN2 requested to add signalling to enable a SL-TDOA/SL-TOA capable UE to report additional measurements (up to 3 additional SL-TDOA/SL-TOA measurements according to UE capabilities)? - RAN1 should respond that RAN2 is not requested to add signalling to support the report of additional measurements Based on the response to question 1, the remaining questions do not need to be answered. |

(Closed) Proposal 5-1:

Alt.1

- Answer to Q1:

  - 1-1-1: Yes, as pointed out in the earlier RAN1 LS in R2-2407909 (R1-2407387), the signalling support to report up to 3 additional SL-TDOA/SL-TOA measurements is intended to be introduced to TS 38.355.

  - 1-1-2: RAN1 suggests RAN2 to add signalling to enable a SL-TDOA/SL-TOA capable UE to report additional measurements.

  - 1-1-3: Yes, similar as what we defined in FG13-6 for DL-TDOA, the FG 41-1-7a and 41-1-7b support UE reporting up to 4 SL RTOA/RSTD for different SL PRS reception for the same pair of UEs (1 + 3 additional measurements). Also considering the support of FG41-1-19a, the number of additional SL RTOA/RSTD for a pair of UEs can be up to 15.

- Answer to Q2:

  - 1-2-1: In addition to the additional SL-RSTD/SL-RTOA measurements, the additional SL-TDOA/SL-TOA measurements should also include the SL-PRS RSRP/RSRPP measurements together with other currently reported information such as SL-PRS Resource ID (sl-PRS-ResourceId),  LOS/NLOS indicator (los-NLOS-Indicator), additional paths measurements (sl-TDOA-AdditionalPathList/sl-TOA-AdditionalPathList), timing quality (sl-TimingQuality), time stamp (sl-TimeStamp) and/or sl-POS-ARP-ID-Rx.

  - 1-2-2: RAN1 suggests the additional SL-TDOA/SL-TOA measurements to follow the per ARP reporting, i.e. no change to existing reporting IE, i.e. up to 4 per Rx ARP measurements, which can be associated with the same Rx ARP ID or different Rx ARP IDs.

  - 1-2-3: Additional SL-TDOA/SL-TOA measurements can include the SL-RSTD/SL-RTOA measurements, and optionally include the SL-PRS RSRP/RSRPP measurements together with other currently reported information such as SL-PRS Resource ID (sl-PRS-ResourceId-r18),  LOS/NLOS indicator (los-NLOS-Indicator-r18), additional paths measurements (sl-TDOA-AdditionalPathList-r18/sl-TOA-AdditionalPathList-r18), timing quality (sl-TimingQuality-r18), time stamp (sl-TimeStamp-r18).

- Answer to Q3:

  - 1-3-1: Yes, the encoding of the additional SL-RSTD/SL-RTOA as well as SL-PRS RSRP/RSRPP measurements are intended to be defined relative to the first/main SL-RSTD/SL-RTOA/SL-PRS RSRP/RSRPP measurements similar to additional measurement reporting per LPP. The value ranges for differential SL-RSTD and SL-RTOA information can be same as that for nr-RSTD-ResultDiff-r16 (TS 37.355) for reporting of additional NR-DL-TDOA measurements. The value range for differential SL-PRS RSRP and SL-PRS RSRPP information can be same as that for nr-DL-PRS-RSRP-ResultDiff-r16. Note that the value ranges for the information elements recommended above are from the perspective of RAN1 and are subject to review by RAN4.

  - 1-3-2: RAN1 suggests the encoding (absolute/relative) of the additional SL-RSTD/SL-RTOA (and probably SL-PRS RSRP/RSRPP) measurements to follow the existing per Rx ARP measurement encoding scheme.

  - 1-3-3: There is no need to perform relative encoding of the additional SL-RSTD/SL-RTOA, SL-PRS RSRP/RSRPP measurements.

Alt.2

- Answer to Q1:

  - 2-1-1: No need to add additional signalling for SL-RSTD/SL-RTOA measurement reporting. The report of SL-TDOA/SL-TOA according to UE capabilities is already supported by existing signalling.

  - 2-1-2: As reporting additional measurements is an optional UE feature, and even additional signalling was not introduced, the measurement for different SL-PRS reception can still be reported with separate SL-TDOA-ProvideLocationInformation/ SL-TOA-ProvideLocationInformation messages. Thus, RAN2 is NOT requested to add signalling to enable a SL-TDOA/SL-TOA capable UE to report additional measurements.

  - 2-1-3: It is RAN1’s opinion that RAN2 does not need to add additional signalling to report additional measurements.

- Answer to Q2/Q3:

  - Based on RAN1 response to Q1, these questions do not need to be treated.

Alt.3

- Answer to Q1:

  - RAN1 has different views on whether to add signalling to enable a SL-TDOA/SL-TOA capable UE to report additional measurements (up to 3 additional SL-TDOA/SL-TOA measurements according to UE capabilities). So RAN1 added the note for the FGs 41-1-7a and FG 41-1-7b and up to RAN2 to decide whether to add signalling to enable a SL-TDOA/SL-TOA capable UE to report additional measurements (up to 3 additional SL-TDOA/SL-TOA measurements according to UE capabilities).

| Company | Comment |
| --- | --- |
| Moderator | <Summary of companies’ views> - Answer to Q1 - Yes: Intel, Huawei/HiSilicon, ZTE/Sanechips, Qualcomm - No: CATT/CICTCI, OPPO, Ericsson - It is up to RAN2: vivo - Answer to Q2 - SL-RSTD/SL-RTOA measurements only: - SL-RSTD/SL-RTOA measurements + SL-PRS RSRP/RSRPP measurements together with other currently reported information such as SL-PRS Resource ID (sl-PRS-ResourceId-r18), LOS/NLOS indicator (los-NLOS-Indicator-r18), additional paths measurements (sl-TDOA-AdditionalPathList-r18/sl-TOA-AdditionalPathList-r18), timing quality (sl-TimingQuality-r18), and/or time stamp (sl-TimeStamp-r18): Intel (+sl-POS-ARP-ID-Rx), Huawei/HiSilicon, ZTE/Sanechips, Qualcomm, vivo(if RAN2 agreed additional signaling) - additional SL-TDOA/SL-TOA measurements to follow the per ARP reporting, i.e. no change to existing reporting IE, i.e. up to 4 per Rx ARP measurements, which can be associated with the same Rx ARP ID or different Rx ARP IDs: Huawei/HiSilicon - No need to answer: CATT/CICTCI, OPPO, Ericsson - Answer to Q3 - Yes, additional SL-RSTD/SL-RTOA/SL-PRS RSRP/RSRPP measurements be defined relative to the 1st/main SL-RSTD/SL-RTOA/SL-PRS RSRP/RSRPP measurement, and value range is same as the value range of additional measurements in LPP: Intel, Huawei/HiSilicon, ZTE/Sanechips, vivo(if RAN2 agreed additional signaling, differential reporting for SL-TDOA, but not for SL-TOA) - No, there is no need to perform relative encoding of the additional SL-RSTD/SL-RTOA, SL-PRS RSRP/RSRPP measurements: Qualcomm - No need to answer: CATT/CICTCI, OPPO, Ericsson Based on the above situation, as described in [11], there are different views in RAN1 on whether to add signalling to enable a SL-TDOA/SL-TOA capable UE to report additional measurements. The moderator would like to try to make RAN1 conclusion, as it might be difficult to make a decision in RAN2 as well. |
| Qualcomm | We think that the previous agreement was evident that there is a UE feature without the corresponding signaling being introduced. We understand that we are late in the CR phase, but this is the definition of an “essential CR”: A feature that was supposed to exist, for which a UE feature has been added, it hasnot been fully specified. The previous LS included: - “ For NR_pos_enh2, with regards to FGs 41-1-7a and FG 41-1-7b, RAN1 respectfully informs RAN2 that UE capability components have been specified, but there is no corresponding report specified in the ProvideLocationInformation message of TDOA and TOA methods. “ This sentence clearly points out that the signaling is missing: “no corresponding report specified”, and therefore, the natural thing to do is to add the corresponding report. |
| ZTE | Same understanding as Qualcomm and Moderator, we prefer to make RAN1 conclusion. We don’t think letting RAN2 decide is helpful since RAN2’s LS shows that they are not clear on how to address the UE feature. we should provide clear response. Q1: ok with either 1-1-1 and 1-1-3 Q2: ok with either 1-2-1 or 1-2-3 Q3: 1-3-1 |
| CATT | Alt.2. Based on the previous RAN1’s agreement, RAN1 does not request “corresponding report” for reporting the additional measurements. Also, R18 can be seen as the first release to introduce SL-Positioning. It is not essential to introduce the enhancements in R18. Similar to NR positioning for regular UEs, the reporting of the additional measurements were not introduced in the first release, but the enhancements of the next release. |
| Moderator | Based on the inputs, further discussion in online session is necessary. We should stick to workable solution with minimum effort as we are in maintenance phase. The constructive discussion to conclude this issue in RAN1 will be appreciated. |
| Huawei, HiSilicon | Q1: We want to clarify our intention that the signaling part is only about “requesting the UE to report”, instead of reporting itself. Q2: 1-2-2 Q3: 1-3-2 or 1-3-3. Currently only absolute value is used for per-ARP measurement. We believe reusing the current signaling for the additional measurement reporting and add the signaling for requesting the additional measurement will suffice with minimum spec changes. |
| Moderator | Agreement: Reply to RAN2 as below. - From RAN1 perspective, to make the additional SL-TDOA/SL-TOA measurement feature work, - following changes are recommended. It is up to RAN2 for final decision. - SL-TOA-MeasElementPerARP-ID-Rx ::= SEQUENCE (SIZE(1..416)) OF SL-TOA-MeasElement - SL-TDOA-MeasElementPerARP-ID-Rx ::= SEQUENCE (SIZE(1..416)) OF SL-TDOA-MeasElement - The signaling of requesting additional measurement is also needed to support above recommendation. FFS: other reply LS contents |

(Closed) Question 5-2:

Based on above agreement, is there any additional content necessary for reply LS to RAN2?

| Company | Comment |
| --- | --- |
| ZTE | Given our agreed Question 1’s reply, we would like to change our view for Question 3: 1-3-3: There is no need to perform relative encoding of the additional SL-RSTD/SL-RTOA, SL-PRS RSRP/RSRPP measurements. |
| Qualcomm | We can comment that for the reply to Q1 and Q2 the above agreed reply is provided, and for Q3, we can clarify that: There is no need to perform relative encoding of the additional SL-RSTD/SL-RTOA, SL-PRS RSRP/RSRPP measurements. |
| Moderator | Based on the inputs, we can check whether following reply LS contents are ok or not. Proposed agreement: Following is captured in LS to RAN2 on Rel-18 NR UE features list. --- RAN1 would like to thank RAN2 for the LS in R1-2409344/R2-2409256. Regarding Question 1 and Question 2 in the LS, RAN1 would like to inform RAN2 about the following RAN1 recommendation. Agreement: Reply to RAN2 as below. - From RAN1 perspective, to make the additional SL-TDOA/SL-TOA measurement feature work, - following changes are recommended. It is up to RAN2 for final decision. - SL-TOA-MeasElementPerARP-ID-Rx ::= SEQUENCE (SIZE(1..416)) OF SL-TOA-MeasElement - SL-TDOA-MeasElementPerARP-ID-Rx ::= SEQUENCE (SIZE(1..416)) OF SL-TDOA-MeasElement - The signaling of requesting additional measurement is also needed to support above recommendation.. Regarding Question 3, it is RAN1’s understanding based on above agreement that there is no need to perform relative encoding of the additional SL-RSTD/SL-RTOA, SL-PRS RSRP/RSRPP measurements. --- |
| Moderator | Agreement: Following is captured in LS to RAN2 on Rel-18 NR UE features list. --- RAN1 would like to thank RAN2 for the LS in R1-2409344/R2-2409256. Regarding Question 1 and Question 2 in the LS, RAN1 would like to inform RAN2 about the following RAN1 recommendation. Agreement: Reply to RAN2 as below. - From RAN1 perspective, to make the additional SL-TDOA/SL-TOA measurement feature work, - following changes are recommended. It is up to RAN2 for final decision. - SL-TOA-MeasElementPerARP-ID-Rx ::= SEQUENCE (SIZE(1..416)) OF SL-TOA-MeasElement - SL-TDOA-MeasElementPerARP-ID-Rx ::= SEQUENCE (SIZE(1..416)) OF SL-TDOA-MeasElement - The signaling of requesting additional measurement is also needed to support above recommendation.. Regarding Question 3, it is RAN1’s understanding based on above agreement that there is no need to perform relative encoding of the additional SL-RSTD/SL-RTOA, SL-PRS RSRP/RSRPP measurements. --- |

- Conclusions

### NR_MIMO_evo_DL_UL

Agreement:

Introduce following FG for UE STxMP processing capability for codebook.

- FG name

  - UE STxMP processing capability for codebook

- Component

  - 1. Require additional timeline to process multiple TBs for codebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG

  - Note. This FG can be applied for CG+DG also if UE can support those FG.

- Prerequisite

  - 40-6-3a

- Consequence if not supported

  - UE should process multiple TBs within legacy timeline

- Type

  - Per FSPC

- Need of FR1/FR2 differentiation

  - FR2 only

- Note

  - candidate values:

  - UE reports candidate value independently for each SCS in unit of symbols

  - For 15kHz SCS: {1,2,4}

  - For 30kHz SCS: {1,2,4,8}

  - For 60kHz SCS: {2,4,8,16}

  - For 120kHz SCS: {4,8,16,32}

  - For 480kHz SCS: {16,32,64,128}

  - For 960kHz SCS: {32,64,128,256}

  - Note: SCS above is the minimum between SCS of the scheduling DCI and SCS of the scheduled PUSCH.

- Mandatory or optional

  - Optional with capability signaling

Agreement:

Introduce following FG for UE STxMP processing capability for non-codebook.

- FG name

  - UE STxMP processing capability for non-codebook

- Component

  - 1. Require additional timeline to process multiple TBs for noncodebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG

  - Note. This FG can be applied for CG+DG also if UE can support those FG.

- Prerequisite

  - 40-6-3b

- Consequence if not supported

  - UE should process multiple TBs within legacy timeline

- Type

  - Per FSPC

- Need of FR1/FR2 differentiation

  - FR2 only

- Note

  - candidate values:

  - UE reports candidate value independently for each SCS in unit of symbols

  - For 15kHz SCS: {1,2,4}

  - For 30kHz SCS: {1,2,4,8}

  - For 60kHz SCS: {2,4,8,16}

  - For 120kHz SCS: {4,8,16,32}

  - For 480kHz SCS: {16,32,64,128}

  - For 960kHz SCS: {32,64,128,256}

  - Note: SCS above is the minimum between SCS of the scheduling DCI and SCS of the scheduled PUSCH.

- Mandatory or optional

  - Optional with capability signaling

### NR_Mob_enh2

Agreement:

Add following note for FG45-1a.

- Note: the SSBs of candidate cell(s) to be measured can be on any band where the UE supports LTM

### NR_pos_enh2

Agreement:

Reply to RAN2 as below.

- From RAN1 perspective, to make the additional SL-TDOA/SL-TOA measurement feature work,

  - following changes are recommended. It is up to RAN2 for final decision.

    - SL-TOA-MeasElementPerARP-ID-Rx ::= SEQUENCE (SIZE(1..416)) OF SL-TOA-MeasElement

    - SL-TDOA-MeasElementPerARP-ID-Rx ::= SEQUENCE (SIZE(1..416)) OF SL-TDOA-MeasElement

  - The signaling of requesting additional measurement is also needed to support above recommendation.

FFS: other reply LS contents

Agreement:

Following is captured in LS to RAN2 on Rel-18 NR UE features list.

---

RAN1 would like to thank RAN2 for the LS in R1-2409344/R2-2409256. Regarding Question 1 and Question 2 in the LS, RAN1 would like to inform RAN2 about the following RAN1 recommendation.

Agreement: Reply to RAN2 as below. - From RAN1 perspective, to make the additional SL-TDOA/SL-TOA measurement feature work, - following changes are recommended. It is up to RAN2 for final decision. - SL-TOA-MeasElementPerARP-ID-Rx ::= SEQUENCE (SIZE(1..416)) OF SL-TOA-MeasElement - SL-TDOA-MeasElementPerARP-ID-Rx ::= SEQUENCE (SIZE(1..416)) OF SL-TDOA-MeasElement - The signaling of requesting additional measurement is also needed to support above recommendation.

Regarding Question 3, it is RAN1’s understanding based on above agreement that there is no need to perform relative encoding of the additional SL-RSTD/SL-RTOA, SL-PRS RSRP/RSRPP measurements.

---

References

[1]	R1-2409043	Updated RAN1 UE features list for Rel-18 NR after RAN1#118bis		Moderators (AT&T, NTT DOCOMO, INC.)

[2]	R1-2409392	Rel-18 UE Features	Huawei, HiSilicon

[3]	R1-2409579	UE features for Rel-18 work items	Samsung

[4]	R1-2410340	Rel-18 UE Features	Ericsson

[5]	R1-2409041	Summary#2 of discussion on UE features for MC enhancements	Moderator (NTT DOCOMO, INC.)

[6]	R1-2312255	Summary #1 on Rel-18 STxMP	Moderator (OPPO)

[7]	R1-2409346	LS on UE capabilities for inter-frequency L1 measurements for LTM	RAN2, Huawei

[8]	R1-2409924	Discussion on UE capabilities for inter-frequency L1 measurements for LTM	CATT

[9]	R1-2410566	Discussion on RAN2 LS on UE capabilities for inter-frequency L1 measurements for LTM	Ericsson

[10]	R1-2409344	LS on Additional Measurements for SL-TDOA and SL-TOA	RAN2, Qualcomm

[11]	R1-2409663	Draft reply LS on additional Measurements for SL-TDOA and SL-TOA	vivo

[12]	R1-2409914	Discussion on additional measurements for SL-TDOA and SL-TOA	CATT, CICTCI

[13]	R1-2409915	Draft reply LS on additional measurements for SL-TDOA and SL-TOA	CATT, CICTCI

[14]	R1-2410067	Discussion on LS about Additional Measurements for SL-TDOA and SL-TOA	OPPO

[15]	R1-2410068	Draft reply LS to RAN2 on Additional Measurements for SL-TDOA and SL-TOA	OPPO

[16]	R1-2410144	Draft Reply LS on Additional Measurements for SL-TDOA and SL-TOA	Intel Corporation

[17]	R1-2410327	Discussion on LS to RAN1 on additional measurements in SL positioning	Huawei, HiSilicon

[18]	R1-2410449	Discussion on additional measurements for SL-TDOA and SL-TOA	ZTE Corporation, Sanechips

[19]	R1-2410450	Draft reply LS on additional measurements for SL-TDOA and SL-TOA	ZTE Corporation, Sanechips

[20]	R1-2410461	Draft Reply for the LS on Additional Measurements for SL-TDOA and SL-TOA	Qualcomm Incorporated

[21]	R1-2410613	Discussion on LS on Additional Measurements for SL-TDOA and SL-TOA	Ericsson

[22]	R1-2410615	Draft reply LS Additional Measurements for SL-TDOA and SL-TOA	Ericsson