3GPP TSG-RAN WG2 Meeting #128                                                               R2-2409539

Orlando, USA, November 18th– 22nd, 2024

Agenda item:	8.8.6

Source:	vivo

Title:	Discussion on Redirection from E-UTRA TN to NR-NTN

Document for:	Discussion and Decision

- Introduction

In the LTE TN to NR NTN idle mobility discussion in RAN2#127bis [1], whether to support the redirection from LTE TN to NR NTN was discussed. However, due to some controversy, the issue was postponed.

Proposal 3: RAN2 confirms the understanding that dedicated NR NTN frequency is not configured in the RRCConnectionRelease message, and confirms that this can be left to NW implementation (w/o Spec impact). - QC is ok with this but thinks it’s fine to clarify this somewhere - Samsung thinks we should separate the case of providing dedicated frequencies from the redirection case. Xiaomi agrees, redirection could be performed providing assistance information in the release message. LG shares similar view as Samsung but thinks the NW does not need to provide assistance info - China Telecom thinks satellite information would be needed to support redirection to NR NTN - QC thinks that either we go for proposal 3 or we need to introduce a UE capability for this. Come back in the next meeting

In the contribution, we will analyze the issue and give our view.

- Discussion

According to TS 36.331 [2], the NR frequency can be configured by NW in the RRCConnectionRelease message [3], and there is no restriction on whether the NR frequency is for TN or NTN.

RedirectedCarrierInfo ::= CHOICE { eutra ARFCN-ValueEUTRA, geran CarrierFreqsGERAN, utra-FDD ARFCN-ValueUTRA, utra-TDD ARFCN-ValueUTRA, cdma2000-HRPD CarrierFreqCDMA2000, cdma2000-1xRTT CarrierFreqCDMA2000, ..., utra-TDD-r10 CarrierFreqListUTRA-TDD-r10, nr-r15 CarrierInfoNR-r15, nr-r17 CarrierInfoNR-r17 }

Besides, even if no satellite assistance information is provided, UE can also perform cell selection and try to camp on the NTN cell if the frequency indicated by NW is for NTN. So from this perspective, the redirection from LTE TN to NR NTN is feasible.

Observation 1: Up to implementation, NW to configure the NTN frequency within the redirection information within the RRCConnectionRelease message.

Proposal 1: According to the current spec, redirection from LTE TN to NR NTN is feasible.

According to the WID of the LTE TN to NR NTN mobility [3], from which it can seen only the enhancement on cell reselection in RRC IDLE state is included. No further enhancement on redirection from LTE TN to NR NTN is required to be supported.

The work item aims at specifying support for the following objectives: - idle mode mobility based on cell reselection from E-UTRA TN to NR NTN, where E-UTRA TN provides satellite information for NR NTN neighbor cells in a System Information Block. [RAN2]

Observation 2: Any enhancement on redirection from LTE to NR NTN is out of the WID.

Proposal 2: No further enhancement to support the further optimization of the redirection from LTE TN to NR NTN.

- Conclusion

In this contribution, we have further discussed the redirection issue from LTE TN to NR NTN. And we make the following observations and proposals:

Observation 1: Up to implementation, NW to configure the NTN frequency within the redirection information within the RRCConnectionRelease message.

Observation 2: Any enhancement on redirection from LTE to NR NTN is out of the WID.

Proposal 1: According to the current spec, redirection from LTE TN to NR NTN is feasible.

Proposal 2: No further enhancement to support the further optimization of the redirection from LTE TN to NR NTN.

- Reference

- R2-2409213, Report from Break-out session on NR-NTN and IoT-NTN.

- TS36.331, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification

- RP-240924, Revised WID: Inter RAT mobility support from E-UTRAN TN to NR-NTN.