3GPP TSG-RAN2 Meeting #128	R2-2409550

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

Agenda Item:	8.5.2

Source:	OPPO

Title:	Discussion on On-Demand SSB

Document for:	Discussion, Decision

- Introduction

This is to discuss Objective-1 of R19 NES, for which R2#127 agreed that

Agreements on OD-SSB

Scenarios on OD-SSB:

- RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

OD-SSB transmission indication:

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

Measurement on OD-SSB:

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

- Discussion

2.1 Issue-1: OD-SSB Activation/Deactivation

Based on the latest R1 progress

Agreement

For a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported. In order to deactivate on-demand SSB transmission from a UE perspective, support at least one of the following options.

- Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication

- Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication

- Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

- Option 3: Configuration/indication of the duration of on-demand SSB transmission window

- Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell

- Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired

- Option 5: On-demand SSB transmission, if any, is deactivated when SCell activation is completed

- Option 6: Explicit indication of deactivation for on-demand SSB via [group-common] DCI

- FFS: Each option is applicable to which Cases or Scenarios

- FFS: Details related to each of the above options

R1 would make a conclusion regarding the deactivation method, so no need for R2 work on it.

- R2 wait for R1 progress for the OD-SSB deactivation method.

For the MAC-CE format design, R2 agreed that

=> Will discuss it later.

=> NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

There are in general two methods: one is to include cell-index of OD-SSB explicitly while the other is to include cell-index implicitly via an ordered bitmap. In general, the latter one is more signaling efficient, considering the saved overhead for cell-index indication (e.g., only 1 byte consumed for typical 7-cell case, and considering the length consumed for OD-SSB parameter (at least including periodicity) indication can be roughly the same between the two methods).

- Adopt the bitmap based method to indicate OD-SSB indication for multiple SCells simultaneously.

Besides, R1 had an FFS point regarding which parameter(s), besides periodicity, has to be carried by MAC-CE

Agreement

For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell.

- Periodicity of the on-demand SSB

- FFS: Any other relevant parameters

So that R2 cannot conclude on the detailed design of the MAC-CE format.

- R2 wait for R1 conclusion on the content of MAC-CE before concluding on the MAC-CE format design.

2.2 Issue-2: Measurement of OD-SSB

For serving cell measurement, 127 concluded that

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

And 127bis discussed that

3. L3 RRM Framework

- Option1: Based on different measurement configuration when OD-SSB is transmitted

- Option2: Based on OD-SSB pattern ignoring SMTC when OD-SSB is transmitted

Need more time to think of.

It would be helpful to firstly clarify the details of case-1, before looking into case-2.

For scenario-2, where the OD-SSB before SCell activation is mainly for fast SCell activation, it is hard to restrict the L3 measurement to SMTC window, considering the SMTC window is more designed for static SSB transmission, while now the OD-SSB can be dynamically (de)activated with various periodicity (FFS other parameters), so it is hard to always restrict the serving cell L3 measurement into a configured SMTC window (with measurement cycle), i.e., cannot wait for a coming SMTC window for L3 measurement, which may further delay the SCell activation. Although it is proposed that there can be multiple sets of SMTC window configurations for different OD-SSB parameters, it is essentially the same as following OD-SSB configurations, since the SMTC window configurations should be align with OD-SSB configurations. So, it seems sufficient at least to design a serving cell L3 measurement mechanism for OD-SSB at least before SCell activation. The same logic holds for scenario-2A, where OD-SSB is triggered upon SCell activation. The logic is similar to R18 FR2 unknown SCell activation, where SSB periodicity is used instead of SMTC window to speed up SCell activation procedure.

And same logic can be applied to case-2, when it comes to L3 measurement based on OD-SSB, yet considering that it is still not super clear whether a combined always-on SSB and OD-SSB based measurement can be done, or rely on either one of the two, it seems better to wait for R1 reply on the LS for the always-on/OD-SSB relation before concluding on this issue. R4 moderator suggested WF as follows for case-2.

  - From moderator’s understanding, this issue can be summarised with the following options.

    - Option 1: The UE is assumed to perform measurement based on OD-SSB only.

    - Option 2: The UE is assumed to perform measurement based on legacy measCycleSCell

    - Option 3: The UE is assumed to perform measurement by combining the always-on SSB and OD-SSB.

On the other hand, after the SCell has been activated, there seems no strong need to use a dense SSB for L3 measurement, spare OD-SSB (case-1) or always-on SSB (case-2) seems sufficient. Furthermore, it is not super clear whether OD-SSB is still needed during this time period, or CSI-RS is sufficient. So, it seems sufficient to limit enhancement to before/during SCell activation phase.

Table 1 Summary of L3 measurement for case-1/2

|  | Before SCell activation | During SCell activation | After SCell activation |
| --- | --- | --- | --- |
| Case#1 | Rely on OD-SSB pattern (ignoring SMTC) | Rely on OD-SSB pattern (ignoring SMTC) | FFS whether need to support OD-SSB |
| Case#2 | FFS whether to rely on always-on or OD-SSB or both | FFS whether to rely on always-on or OD-SSB or both | FFS whether need to support OD-SSB |

- Based on OD-SSB pattern (i.e., ignoring SMTC) to perform L3 measurement for SCell before and during SCell activation, at least for Case-1.

For case-2, the first question is whether to allow combined measurement on always-on SSB and OD-SSB, for which R2 discussed this at 127bis

4. Whether always-on SSB and/or OD-SSB are measured in case 2?

P1 in R2-2408295 (Samsung)

Proposal 1: For case #2, RAN2 to discuss which option is followed by UE for RRM measurements of SCell configured with on demand SSB.

- Option1: UE uses on-demand SSB for RRM measurements when they are transmitted, otherwise UE uses periodic SSB

- Option2: UE uses on-demand SSB for RRM measurements when they are transmitted in addition to periodic SSB for RRM measurements.

=> Wait for next meeting.

From our view, there is anyway scenarios where combination cannot be done, e.g., always-on SSB and OD-SSB are at different frequencies, since OD-SSB is limited non-sync-raster frequencies (as agreed by R1 below), while always-on SSB can be CD-SSB at sync-raster-frequencies.

Agreement

For a cell supporting on-demand SSB SCell operation, at least the following is supported

- On-demand SSB on the cell is not located on synchronization raster.

- On-demand SSB on the cell is non-cell-defining SSB

FFS: Additional support of OD-SSB for CD-SSB located on sync-raster

So either 3GPP define the feasibility of combined measurement case-by-case, e.g., leave it to R4 after R1 replies R4-2416913, or can go for a rough but safer way-out, i.e., not pursue combined-measurement.

- At least support L3 measurement on OD-SSB only for case#2, and leave further decision on supporting combined L3 measurement on both always-on SSB and OD-SSB to R4, e.g., for specific sub-cases.

For L3 measurement, since it is possible that serving cell and neighbouring cell share the same MO configuration, e.g., in an intra-frequency scenario, it is good to understand the behavior for neighboring and serving cell scenario.

For neighbouring cell, it anyway needs to rely on SMTC window to identify the time location of SSB searching. Besides, when it comes to OD-SSB, it is anyway infeasible for UE to know the neighboring cell OD-SSB (de)activation status. Combing the two, it seems more realistic to rely on Case-2, if network would like to configure the UE to perform neighboring cell measurement. Anyway, there seems no much work that R2 can do for the neighbouring cell measurement under OD-SSB objective, and it aligns with R4 conclusion below

Issue 1-1-2: OD-SSB impact on neighbour cell measurement

Agreement:

- For the RRM measurement on neigboring cell, the legacy meaurement framework is applied.

  - Note: The discussion of SMTC configuration is a separate issue.

- This agreement can be revisited if it is not aligned with further agreement in other WGs.

- For L3 measurement of neighbouring cell based on OD-SSB, R2 not pursue further specification work.

Another issue is whether to rely on legacy event-based MR triggering or rely on MAC-CE based MR triggering.

We understand that MAC-CE based MR triggering was introduced for FR2 unknown SCell, while OD-SSB can be sent before SCell activation, and thus enable known case, which would lead to an even quicker SCell activation compared with the “semi-known” case of MAC-CE triggered MR.

- R2 not pursue MAC-CE triggered MR for OD-SSB.

- Conclusion

Proposal 1	R2 wait for R1 progress for the OD-SSB deactivation method.

Proposal 2	Adopt the bit-map based method to indicate OD-SSB indication for multiple SCells simultaneously.

Proposal 3	R2 wait for R1 conclusion on the content of MAC-CE before concluding on the MAC-CE format design.

Proposal 4	Based on OD-SSB pattern (i.e., ignoring SMTC) to perform L3 measurement for SCell before and during SCell activation, at least for Case-1.

Proposal 5	At least support L3 measurement on OD-SSB only for case#2, and leave further decision on supporting combined L3 measurement on both always-on SSB and OD-SSB to R4.

Proposal 6	For L3 measurement of neighbouring cell based on OD-SSB, R2 not pursue further specification work.

Proposal 7	R2 not pursue MAC-CE triggered MR for OD-SSB.