3GPP TSG-RAN WG2 Meeting #128	R2-2409557

Orlando, USA, Nov 18-22, 2024

Agenda item:	8.7.4.2

Source:	Qualcomm Incorporated

Title:	Discussion on DSR enhancements

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion and decision

# Introduction

In this paper, we discuss a few FFS issues on the enhanced DSR design. Namely, co-existence between Rel-18 and Rel-19 DSR MAC CEs, format of the enhanced DSR MAC CE, and whether to report non-delay critical data buffered between delay-critical data in DSR.

# Discussion

At the RAN2#127-bis meeting, the following agreement on the coexistence between Rel-18 and Rel-19 DSR MAC CE was made:

- FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

If there is at least one LCG that is configured with multiple reporting thresholds, it implies that the network supports and prefers Rel-19 enhanced DSR MAC CE. Therefore, a UE which is capable of Rel-19 DSR should prioritize the use of Rel-19 DSR MAC CE whenever possible, even when the LCG has non-zero buffer size associated with only reporting threshold.

Proposal 1.	If there is at least one LCG configured with multiple reporting thresholds, the UE should always use Rel-19 DSR MAC CE, as long as it can be accommodated by the available PUSCH resource.

However, if the UE is configured to use Rel-19 MAC CE but the available PUSCH resource is too small to include it completely, we think Rel-18 DSR MAC CE should be used instead, because it is still useful to send some delay information than sending nothing. In that case, the delay information is calculated based on the triggering threshold, i.e. the reported buffer size equals the amount of delay critical data whose remaining time is below the triggering threshold of that LCG, same as in Rel-18.

Proposal 2.	If available PUSCH resource is not large enough to accommodate a Rel-19 DSR MAC CE, a Rel-18 DSR MAC CE is sent instead, with the delay information calculated based on the trigger threshold.

In Rel-18, UE is allowed to report zero in the buffer size field in a DSR MAC CE. In that case, the remaining time field is reserved. The reason behind this design is to give UE more flexibility in MAC PDU assembly (e.g. relaxation in DSR cancelation).

This flexibility becomes even more important for UE implementation with Rel-19 DSR MAC CE, which may include multiple buffer sizes for an LCG corresponding to different reporting thresholds. If we follow the principle, which has been used in all BSR and DSR MAC CEs so far, that the reported buffer size should be the one after the MAC PDU is built. As a result, when UE starts to build a MAC PDU, it may not be able to know whether all the delay critical data associated with a reporting threshold can be included in the MAC PDU or not.

An implementation friendly approach would be to assume each reporting threshold has data to report. If this assumption is no longer true after the MAC PDU is assembled, UE tries its best to not include reporting thresholds which have no data left. However, this adjustment often requires UE to perform LCP procedure again because the shorter DSR MAC CE frees up some bytes for data. In some cases (e.g. UE has multiple UL grants in the same slot), UE may not have enough time to perform the entire adjustment. Hence it is desirable to allow UE to keep its initial assumption and report zero buffer size instead.

Proposal 3.	Buffer size field in Rel-19 DSR MAC CE may have a value of zero, as in legacy. In that case, the corresponding remaining time field is reserved.

Another FFS issue is whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR. It was also discussed in contributions such as [1][2]. Although this scenario may not be common, we believe it can occur. When it does, the legacy DSR reporting procedure can result in network under-serving that LCH. Therefore, we think such non-delay critical data should be reported in DSR MAC CE as well. When reporting, the remaining time to be reported is the remaining time of the delay critical data which arrived after it. That is because the purpose of including it is to ensure in-time delivery of the delay critical data buffered behind it.

Proposal 4.	In the scenario where non-delay critical data is buffered ahead of delay critical data in the same logical channel, the non-delay-critical data should be reported too, with its remaining time set to the shortest remaining time among the delay critical data which arrived after it.

# Conclusion

Based on the above analysis, we respectively request RAN2 to discuss and agree to the following proposals:

Proposal 1.	If there is at least one LCG configured with multiple reporting thresholds, the UE should always use Rel-19 DSR MAC CE, as long as it can be accommodated by the available PUSCH resource.

Proposal 2.	If available PUSCH resource is not large enough to accommodate a R19 DSR MAC CE, a Rel-18 DSR MAC CE is sent instead, with the delay information calculated based on the trigger threshold.

Proposal 3.	Buffer size field in Rel-19 DSR MAC CE may have a value of zero, as in legacy. In that case, the corresponding remaining time field is reserved.

Proposal 4.	In the scenario where non-delay critical data is buffered ahead of delay critical data in the same logical channel, the non-delay-critical data should be reported too, with its remaining time set to the shortest remaining time among the delay critical data which arrived after it.

# References

- R2-2407047, Discussion on scheduling enhancements, Ericsson, August 2024.

- R2-2407062, Scheduling Enhancements for XR, Nokia, Nokia Shanghai Bell, August 2024.