3GPP SA WG2 Meeting #128	R2-2409560

Orlando, USA, November 18-22, 2024

# Title:	Reply LS on Application-Layer FEC Awareness at RAN

# Response to:	LS S2-2410999 on Application-Layer FEC Awareness at RAN

# Release:	Release 19

# Work Item:	NR_XR_Ph3-Core

Source:	Qualcomm Incorporated (to be RAN2)

To:	SA2

Cc:	SA4

Contact Person:

Name:	Linhai He

E-mail Address:	linhaihe AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

1. Overall Description:

RAN2 thank SA2 for the questions on application-layer FEC awareness at RAN. The following are answers from RAN2.

Question 1 from SA2:

Please provide feedback on feasibility of use of content ratio information for discarding of DL PDUs during congestion for RLC AM mode based on the above principles. Please also provide feedback on applicability of RLC AM for XR services, also considering the RAN-level enhancements of RLC Acknowledged Mode (RLC AM) with small packet delay budget as per RP-240791.

Answer by RAN2:

RAN2 confirm that it is feasible to use content ratio for discarding DL PDUs during congestion for DRBs configured with RLC AM. In RLC AM, the ARQ feedback from UE allows NG-RAN to reliably determine which PDUs have been successfully received.  In a possible implementation, NG-RAN can use this information to identify PDU sets whose original content can be correctly recovered by the UE. Among those PDU sets, NG-RAN can select PDUs to discard, ensuring that no more PDUs are discarded than when no content ratio is provided.

RAN2 confirm that it is feasible to use RLC AM for XR services, especially with the upcoming latency reduction enhancements for RLC AM.

Question 2 from SA2:

Please provide feedback on feasibility of use of content ratio information for discarding of DL PDUs during congestion for RLC UM based on the above principles.

Answer by RAN2:

RAN2 confirm that it is feasible to use content ratio for discarding DL PDUs during congestion for DRBs configured with RLC UM. This is because NG-RAN can determine with very high probability (≥99%, according to TS 38.104) whether a MAC PDU containing a specific RLC PDU is successfully received based on HARQ feedback in its DL HARQ operation. In a possible implementation, this capability allows NG-RAN to determine, with high probability, whether enough PDUs in a PDU set have been successfully received by the UE so that the original content of that PDU set can be correctly recovered. Consequently, NG-RAN can use this information to identify which PDUs it may discard, in the same way as in the case of RLC AM.

2. Actions:

To SA2:

ACTION: 	RAN2 kindly asks SA2 to take the above answers into consideration in their discussions.

3. Date of Next RAN2 Meetings:

RAN2#129	Feb 17-21, 2025				Athens, Greece

RAN2#129-bis	April 7-11, 2025				China