3GPP TSG-RAN WG2 Meeting #128                                        R2-2409574

Orlando, USA, Nov 18th – 22th, 2024

Source:        ZTE Corporation

Title:           Introduction of BDS B2b in A-GNSS positioning

Agenda item:   8.16

Document for:  Discussion and Decision

- Introduction

China has published a BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B2b in [1]. The WID [2] of Rel-19 BDS_B2b is as follows:

Specify A-BDS enhancements in Rel-19, including: – Supporting new signal of BDS B2b in A-GNSS for both E-UTRA and NR [RAN2]; – Specify corresponding assistant information in dedicated signaling [RAN2].

This contribution provides our view on the specification impact of introducing BDS B2b in A-GNSS positioning in TS37.355.

- Discussion

TS37.355 can contain the information of up to 16 GNSS systems wherein BDS is one of them. Each GNSS system can have up to 16 satellites, and each GNSS system can have up to 16 GNSS signals. B2b is a new signal of BDS.

The whole signaling structure of A-GNSS positioning in TS37.355 is as follows:

Figure 1. Signaling structure of A-GNSS positioning in TS37.355

The above signaling differs at different level with respect to different GNSS IDs, different satellite IDs and/or different signal IDs. To be specific:

For A-GNSS-ReqeustCapabilities, it only contains 3 request signalings with the format of BOOLEAN, so there is no specification impact on A-GNSS-ReqeustCapabilities when introducing BDS B2b signal in TS37.355.

For A-GNSS-ProvideCapabilities, UE sends the supported GNSSs (using gnss-ID) and supported GNSS signals (using GNSS-SignalIDs) to LMF. So BDS B2b signal should be introduced in GNSS-SignalIDs.

For A-GNSS-RequestAssistanceData, introducing BDS B2b signal will only impact gnss-GenericAssistDataReq, since it is a per-GNSS request. In the GNSS-GenericAssistDataReqElement, UE requests AD for each GNSS on data bit assistance, almanac assistance or differential correction assistance, etc. Current BDS-DifferentialCorrectionsReq only suits for B1I and B3I signal. RAN2 should discuss whether B2b signal has a different differential correction parameter to be requested independently.

For A-GNSS-ProvideAssistanceData, introducing BDS B2b signal will only impact gnss-GenericAssistData, since it is a per-GNSS AD provision. Each GNSS, e.g., BDS, is associated with a set of assisatance data IEs in gnss-GenericAssistData.

RAN2#127-bis[3] has agreed that the following assistance data parameters should be impacted for BDS B2b:

Agreements: Introduce new parameters for B2b signal in the following affected IEs: ¬ AlmanacReducedKeplerianSet: Add one bit to indicate health state of B2b_I signal. ¬ AlmanacMidiAlmanacSet: Add one bit to indicate health state of B2b_I signal. ¬ BDS-ClockModel2: Add one bit to indicate group delay differential between BDS B2b and B3I signal. ¬ GNSS-SignalID: Use one value to indicate the B2b I signal. ¬ GNSS-SignalIDs: Use one value to indicate the B2b I signal. ¬FFS: Iod: Add one bit B2b signal in ‘GNSS to iod Bit String(11) relation’ table.

According to our observation, at least the following IEs in gnss-GenericAssistData should also consider the specification impact of introducing BDS B2b signals:

- gnss-DataBitAssistance: this IE provides data bit assistance data of each satellite, each GNSS signal of BDS, wherein the GNSS-SignalID should introduce BDS B2b, and the field description of gnss-DataBits should include reference of BDS B2b as [1].

- gnss-UTC-Model: there are 5 models for GNSS-UTC-Model, where model-2 is for B1C and B2a, model-5 is for B1I. RAN2 should discuss which UTC model the B2b corresponds to.

- gnss-AuxiliaryInformation: it contains the BDS B1C and BDS B2a satellite orbit type of each BDS satellite. The satellite orbit type of B2b should also be allowed to indicate in this IE. The field description of satType-r16 should increase B2b signal.

For A-GNSS-RequestLocationInformation, the IE does not contain any GNSS signal-specific request. So there is no specification impact on A-GNSS-RequestLocationInformation when introducing BDS B2b signal in TS37.355.

For A-GNSS-ProvideLocationInformation, the IE provides measurements per satellite (i.e., SV-ID), per GNSS signal (i.e., GNSS-SignalID) and per GNSS system (i.e., gnss-ID). So the only specification impact is to introduce BDS B2b in GNSS-SignalID.

Observation 1: There is no additional spec impact in A-GNSS-ReqeustCapabilities, A-GNSS-ProvideCapabilities, A-GNSS-RequestLocationInformation and A-GNSS-ProvideLocationInformation when introducing BDS_B2b in A-GNSS.

Proposal 1: RAN2 to consider the spec impact of gnss-DataBitAssistance, gnss-UTC-Model and gnss-AuxiliaryInformation when introducing BDS B2b signal.

There is also a GNSS-FrequencyID, which is to indicate different values corresponding to different centre frequency of different GNSS signals, see below figure:

Value & Explanation relation (below GNSS-FrequencyID)

| System | Value | Explanation |  |
| --- | --- | --- | --- |
|  |  | Link | Centre Frequency [MHz] |
| GPS | 0 | L1 | 1575.42 |
|  | 1 | L2 | 1227.60 |
|  | 2 | L5 | 1176.45 |
|  | 3-7 | reserved |  |
| SBAS | 0 | L1 | 1575.42 |
|  | 1 | L5 | 1176.45 |
|  | 2-7 | reserved |  |
| QZSS | 0 | L1 | 1575.42 |
|  | 1 | L2 | 1227.60 |
|  | 2 | L5 | 1176.45 |
|  | 3-7 | reserved |  |
| GLONASS k = -7..13 | 0 | G1 | 1602+k×0.5625 |
|  | 1 | G2 | 1246+k×0.4375 |
|  | 2 | G3 | 1202.025 |
|  | 3-7 | reserved |  |
| Galileo | 0 | E1 | 1575.420 |
|  | 1 | E6 | 1278.750 |
|  | 2 | E5a | 1176.450 |
|  | 3 | E5b | 1207.140 |
|  | 4 | E5 | 1191.795 |
|  | 5-7 | reserved |  |
| BDS | 0 | B1I | 1561.098 |
|  | 1 | B1C | 1575.420 |
|  | 2 | B2 | 1207.140 |
|  | 3 | B3 | 1268.520 |
|  | 4 | B2a | 1176.450 |
|  | 5-7 | reserved |  |
| NavIC | 0 | L5 | 1176.450 |
|  | 1-7 | reserved |  |

However the BDS B2b signal has a centre frequency as 1207.140 MHz, which is same to the B2 in the figure. So there is no need to increase a new row in GNSS-FrequencyID.

Observation 2: GNSS-FrequencyID already has B2 signal which has the same frequency as B2b signal, i.e. 1207.140MHz.

Proposal 2: Add BDS B2b in the table of GNSS-FrequencyID, in the same row with the centre frequency 1207.140MHz.

- Conclusion

In this contribution, we propose the following observation and proposals:

Observation 1: There is no additional spec impact in A-GNSS-ReqeustCapabilities, A-GNSS-ProvideCapabilities, A-GNSS-RequestLocationInformation and A-GNSS-ProvideLocationInformation when introducing BDS_B2b in A-GNSS.

Observation 2: GNSS-FrequencyID already has B2 signal which has the same frequency as B2b signal, i.e. 1207.140MHz.

Proposal 1: RAN2 to consider the spec impact of gnss-DataBitAssistance, gnss-UTC-Model and gnss-AuxiliaryInformation when introducing BDS B2b signal.

Proposal 2: Add BDS B2b in the table of GNSS-FrequencyID, in the same row with the centre frequency 1207.140MHz.

- Reference

- BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B2b (Version 1.0) , China Satellite Navigation Office, 2020-07

- RP-242413, New WID on Introduction of BDS B2b Signal in A-GNSS, CAICT, CATT, 2024-09

- R2-2409214, Report from session on positioning and sidelink relay, Session chair (MediaTek), 2024-10