3GPP TSG-RAN WG2 Meeting #128	R2-2409605

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:	6.1.2.1

Source: 	CATT

Title: 	Discussion on remaining issues for one-shot HARQ Feedback in NR NTN

Document for: 	Discussion and Decision

# 1.	Introduction

This contribution addresses the following remaining issue on whether one-shot HARQ feedback mechanism is applicable to Rel-17 NR NTN [1], and also proposes necessary specification changes in TS 38.321 from our perspective.

R2-2409123 Correction on drx HARQ RTT timer in NTN LG Electronics Inc. CR Rel-17 38.321 17.10.0 1967 - F NR_NTN_solutions-Core - Vivo, Qualcomm explains that we agree to not further clarify. LG thinks that there is a difference between NTN and TN. - Qualcomm and Huawei think that these changes were for URLLC. - Sharp asks if one shot feedback is supported for NTN, as that would be the only reason to align. LG understands that there were no restrictions but if there is we should say that this is only for TN. - Ericsson explains that this was for NR-U and there was an agreement that this is applicable to outside of NR-U. Ericsson supports fixing this. - Qualcomm thinks that to support one shot we would need to support a number of other things that RAN1 should discuss and we shouldn’t make such decisions in RAN2. Check if one shot feedback is supported for NTN The CR is postponed

# 2.	Discussion

  - 2.1	Support of one-shot HARQ Feedback for NR NTN in RAN1 Spec

In RAN1#107-e, there was the below agreement: [2]

Agreement For Type-3 HARQ codebook in NTN, the UE should skip the codebook feedback for a feedback-disabled HARQ processes Note: The Type-3 codebook size is reduced by excluding the bit positions of disabled HARQ processes

Also, according to TS 38.213 [3], the HARQ feedback enabled/disabled feature specific to NR NTN has already been considered in the related procedure for one-shot HARQ feedback since Rel-17:

Table 0: One-shot HARQ feedback procedure in TS 38.213 [3]

### 9.1.4 Type-3 HARQ-ACK codebook determination If a UE is provided pdsch-HARQ-ACK-OneShotFeedback or pdsch-HARQ-ACK-EnhType3ToAddModList, the UE determines o0ACK,o1ACK,…,oOACK-1ACK HARQ-ACK information bits, for a total number of OACK HARQ-ACK information bits, of a Type-3 HARQ-ACK codebook according to the following procedure. If the UE is provided pdsch-HARQ-ACK-EnhType3ToAddModList and a DCI format scheduling PDSCH reception and triggering the Type-3 HARQ-ACK codebook includes an enhanced Type 3 codebook indicator field that provides a value for pdsch-HARQ-ACK-EnhType3Index, the UE determines a size of a set of indicated serving cells NcellsDL,ind and a size of a set of indicated HARQ process numbers NHARQ,cDL,ind for each indicated serving cell and each indicated HARQ process number from the entry in pdsch-HARQ-ACK-EnhType3ToAddModList corresponding to the pdsch-HARQ-ACK-EnhType3Index value. Each bit from MSB to LSB provided by perCC corresponds to a serving cell in ascending order of serving cell index, where value ‘1’ or value ‘0’ indicate HARQ-ACK for the corresponding serving cell is included or not included in the Type 3 HARQ-ACK codebook, respectively. Each bit string provided by perHARQ corresponds to a serving cell in ascending order of serving cell index, and each bit from MSB to LSB within a bit string corresponds to a HARQ process number on a corresponding serving cell in ascending order of HARQ process number, where value ‘1’ or value ‘0’ indicate HARQ-ACK for the corresponding HARQ process number on the corresponding serving cell is included or not included in the Type 3 HARQ-ACK codebook, respectively. If the DCI format does not include the enhanced Type 3 codebook indicator field, the pdsch-HARQ-ACK-EnhType3Index value is zero. […] Set NDIHARQ=0 if pdsch-HARQ-ACK-OneShotFeedbackNDI or pdsch-HARQ-ACK-EnhType3NDI is provided; else set NDIHARQ=1. Set c=0 – serving cell index in the set of serving cells Set h=0 – HARQ process number index in the set of numbers of HARQ processes Set t=0 – TB index Set g=0 – CBG index Set j=0 while c<NcellsDL while h<NHARQ,cDL if downlinkHARQ-FeedbackDisabled is not provided, or is provided and indicates enabled HARQ-ACK information for h, or harq-feedbackEnablingforSPSactive is provided and enabled and h corresponds to a transport block in a first SPS PDSCH reception after an activation of SPS PDSCH receptions if NDIHARQ=0 if NHARQ-ACK,cCBG/TB,max>0 while t<NTB,cDL while g<NHARQ-ACK,cCBG/TB,max ojACK= HARQ-ACK information bit for CBG g of TB t for HARQ process number index h in the set of numbers of HARQ processes of serving cell c, if any; else, ojACK=0 j=j+1 g=g+1 end while […] end while h=0 c=c+1 end while […]

Observation 1: According to RAN1#107-e agreement and current TS38.213, one-shot HARQ feedback feature can be supported for NR NTN since Rel-17.

Also, from the UE capability perspective, this one-shot HARQ feedback feature is specified as a per Band UE capability, and there is no restriction that this capability cannot be signaled for the NR NTN bands, according to TS 38.306: [4]

oneShotHARQ-feedbackTriggeredByDCI-1-2-r17 Indicates whether the UE supports one-shot HARQ ACK feedback triggered by DCI format 1_2, comprised of the following functional components: - Supports feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 scheduling a PDSCH; - Supports feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 without scheduling a PDSCH using a reserved FDRA value. A UE supporting this feature shall also indicate support of oneShotHARQ-feedback-r16 and dci-Format1-2And0-2-r16.; Band; No; N/A; N/A. enhancedType3-HARQ-CodebookFeedback-r17 Indicates whether the UE supports enhanced type 3 HARQ-ACK codebook feedback based on triggering information in DCI 1_1 and DCI 1_2 (for a UE supporting DCI format 1_2 as indicated in dci-Format1-2And0-2-r16) and also supports transmission of enhanced type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config as indicated in twoHARQ-ACK-Codebook-type1-r16). The capability signalling comprises the following parameters: - enhancedType3-HARQ-Codebooks-r17 indicates the maximum number of supported enhanced type 3 HARQ-ACK codebooks; - maxNumberPUCCH-Transmissions-r17 indicates the maximum number of actual PUCCH transmissions for type 3 or enhanced type 3 HARQ-ACK codebook feedback within a slot. UE only supports feedback of a dynamically selected enhanced type 3 HARQ-ACK codebook based on triggering information in DCI 1_1 and DCI 1_2 (for a UE supporting DCI format 1_2 as indicated in dci-Format1-2And0-2-r16) if the UE supports more than one enhanced type 3 HARQ-ACK codebook to be configured (as indicated in enhancedType3-HARQ-Codebooks-r17). A UE that indicates support of this capability shall also indicate support of oneShotHARQ-feedback-r16.; Band; No; N/A; N/A. oneShotHARQ-feedback-r16 Indicates whether the UE supports one shot HARQ ACK feedback comprised of the following functional components: - Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_1 scheduling a PDSCH; - Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_1 without scheduling a PDSCH using a reserved FDRA value. This capability is also applicable to a frequency band that does not require shared spectrum access.; Band; No; N/A; N/A.

Observation 2: Related UE capabilities indicating the support of one-shot HARQ feedback feature are per band capabilities, without restriction that these capabilities cannot be signaled for NR NTN bands since Rel-17.

With above evidence, RAN2 can confirm that one-shot HARQ feedback mechanism has already been supported in NR NTN since Rel-17, based on RAN1 specifications and related RAN1-introduced UE capabilities.

Proposal 1: RAN2 confirms that one-shot HARQ feedback feature can be supported in NR NTN since Rel-17 (based on RAN1 specification and related RAN1-introduced UE capabilities).

  - 2.2	Proposed MAC Spec change

At the last meeting, the change to MAC was proposed in [5], simply aligning the description on HARQ RTT timer handling in NTN (where HARQ feedback disabled/enabled can be configured) with that in legacy TN, for the case of one-shot HARQ Feedback triggered by DCI.

Table 1: Proposed MAC change in [5]

1> if a DRX group is in Active Time: 2> monitor the PDCCH on the Serving Cells in this DRX group as specified in TS 38.213 [6]; 2> if the PDCCH indicates a DL transmission; or 2> if the PDCCH indicates a one-shot HARQ feedback as specified in clause 9.1.4 of TS 38.213 [6]; or 2> if the PDCCH indicates a retransmission of HARQ feedback as specified in clause 9.1.5 of TS 38.213 [6]: 3> if this Serving Cell is configured with downlinkHARQ-FeedbackDisabled: 4> if the corresponding HARQ process(es) is configured with HARQ feedback enabled: 5> set HARQ-RTT-TimerDL-NTN for the corresponding HARQ process(es) equal to drx-HARQ-RTT-TimerDL plus the latest available UE-gNB RTT value; 5> start or restart the HARQ-RTT-TimerDL-NTN for the corresponding HARQ process(es) whose HARQ feedback is reported in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback. 3> else: 4> start or restart the drx-HARQ-RTT-TimerDL for the corresponding HARQ process(es) whose HARQ feedback is reported in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

We found that such simple wording alignment may result in some problems in the specific case that "among all HARQ processes indicated to perform one-shot HARQ Feedback (i.e. by pdsch-HARQ-ACK-OneShotFeedback, pdsch-HARQ-ACK-OneShotFeedbackDCI-1-2 or pdsch-HARQ-ACK-EnhType3ToAddModList), only some of them are configured as HARQ feedback enabled by downlinkHARQ-FeedbackDisabled". This case is supported by RAN1 specification, which can be easily seen from the green-highlighted condition in Table 0 above, where the PHY still needs to filter the HARQ processes configured with "HARQ Feedback enabled", upon entering the loop for one-shot HARQ Feedback procedure. In this specific case, one-shot HARQ feedback is really signaled only for the HARQ processes that are BOTH indicated to perform one-shot HARQ Feedback (by whichever parameter listed above) AND configured with HARQ Feedback enabled. See below Figure 1: it is the HARQ feedback for HARQ processes 1, 2 and 3 that is really signaled via one-shot HARQ feedback transmission, if a DCI indicating one-shot HARQ Feedback is received by UE.

Figure 1: One-shot HARQ feedback operation in TS 38.213 with downlinkHARQ-FeedbackDisabled configured

Observation 3: As per 38.213, there is the case that among all HARQ processes indicated to perform one-shot HARQ Feedback, only some of them are configured with "HARQ Feedback enabled" but the others with "HARQ Feedback disabled". In this case, one-shot HARQ feedback is only signaled for the HARQ process(es) that are both indicated to perform one-shot HARQ Feedback and configured with "HARQ feedback enabled".

Similar logic of PHY operation applies to the MAC that the NTN specific HARQ RTT timer, i.e. HARQ-RTT-TimerDL-NTN, should be (re)started only for HARQ 1, 2, and 3 in the MAC (not all the "corresponding HARQ processes" indicated for one-shot HARQ process transmission).

Observation 3a: In MAC, HARQ-RTT-TimerDL-NTN should be (re)started only for the HARQ processes both indicated to perform one-shot HARQ Feedback and configured with "HARQ feedback enabled" for the case in Observation 3.

In the legacy TN procedure (i.e. the cyan-highlighted part in Table 1, introduced by Rel-17 URLLC/IIoT [6]), the description "corresponding HARQ process(es)" refers to ALL those HARQ processes indicated for one-shot HARQ Feedback, due to no configuration of downlinkHARQ-FeedbackDisabled in TN. Therefore, if the same wording in TN is directly reused into the NTN case as proposed by [5], there is the risk that the description "4> if the corresponding HARQ process(es) is configured with HARQ feedback enabled" is misinterpreted as a condition that requires all the HARQ processes indicated for one-shot HARQ Feedback to be "HARQ Feedback enabled" (e.g. all HARQ processes 1~5 as in Figure 1), in order to enter this loop for to (re)start HARQ-RTT-TimerDL-NTN. This leads to the consequence that the MAC entity has no means to enter the loop and perform the NTN specific HARQ RTT timer normally, for the case in Observation 3.

Observation 4: By reusing the wording in TN, proposed change in [5] may be misinterpreted as allowing the MAC entity to enter the loop for HARQ-RTT-TimerDL-NTN (re)start, only if all corresponding HARQ processes indicated for one-shot HARQ feedback are configured with "HARQ feedback enabled". This leads to ambiguity on whether the MAC entity can still enter this loop normally for the case in Observation 3, and consequently erroneous HARQ-RTT-TimerDL-NTN handling.

Logically, it is straightforward that if at least one of the corresponding HARQ process(es) which is indicated for one-shot HARQ Feedback is configured with HARQ Feedback enabled, the MAC entity shall enter the loop to perform the (re)start operation of HARQ-RTT-TimerDL-NTN. Regarding for which specific HARQ process(es) HARQ-RTT-TimerDL-NTN shall be actually (re)started, note that the existing texts only (re)start HARQ-RTT-TimerDL-NTN for the HARQ process(es) configured with “HARQ feedback Enabled”, not including the HARQ process whose HARQ feedback is signaled due to “harq-feedbackEnablingforSPSactive”. We think this principle should be kept in the one-shot HARQ feedback case, and so the HARQ-RTT-TimerDL-NTN is (re)started only for the HARQ processes whose HARQ Feedback is enabled and really signaled in the corresponding transmission carrying DL HARQ feedback in PHY.

In summary, the intended MAC behavior is proposed as follows, with the TP provided in the Appendix:

Proposal 2: Agree the following intended MAC behavior, in the case that one-shot HARQ feedback is indicated by PDCCH and downlinkHARQ-FeedbackDisabled is also configured:

- If at least one of the corresponding HARQ process(es) is configured with HARQ feedback enabled, start or restart the HARQ-RTT-TimerDL-NTN for the corresponding HARQ process(es) whose HARQ feedback is enabled and reported in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

Proposal 2a: Agree the TP in the Appendix.

Note that the proposed change in Proposal 2/2a can automatically cover the HARQ feedback for normal DL scheduling other than the one-shot HARQ feedback transmission, where the concerned HARQ process, if HARQ feedback enabled, can still meet the condition “at least one of the corresponding process(es)”, so as to not be affected by the change.

  - 2.2	UE capability aspects

Strictly speaking, UE implemented based on current MAC specification (i.e. till V17.10.0 and 18.3.0) cannot support a correct UE behaviour for HARQ-RTT-TimerDL-NTN handling, when both one-shot HARQ feedback and HARQ enabled/disabled are configured in NR NTN. So the combination of the two features should not be configured by the NW based on current Spec. Therefore, if we agree on a change to introduce the correct UE behaviour from now on, the gNB in NR NTN needs to distinguish whether the UE is a new UE implemented according to this new UE behaviour, or an old one lack of the correct UE behaviour to support the co-configuration of the two features, so as to decide whether it can configure both features and thus avoid any inter-operability issue (i.e. configuring both features  to the old UE that misbehaves on HARQ-RTT-TimerDL-NTN handling).

If companies see the need to deal with such an inter-operability issue, a new Rel-17 UE capability needs to be introduced, e.g. indicating whether the UE is capable of HARQ-RTT-TimerDL-NTN in case one-shot HAQR feedback and HARQ feedback enabled/disabled are configured at the same time in NR NTN.

Proposal 3: RAN2 discusses whether a Rel-17 UE capability needs to be introduced, indicating whether the UE is capable of HARQ-RTT-TimerDL-NTN handling in the case of one-shot HARQ feedback transmission.

# 3.	Conclusion

This contribution discusses the remaining issue on whether/how to support one-shot HARQ Feedback in NR NTN. Observations and proposals are listed as follows:

Observation 1: According to RAN1#107-e agreement and current TS38.213, one-shot HARQ feedback feature can be supported for NR NTN since Rel-17.

Observation 2: Related UE capabilities indicating the support of one-shot HARQ feedback feature are per band capabilities, without restriction that these capabilities cannot be signaled for NR NTN bands since Rel-17.

Observation 3: As per 38.213, there is the case that among all HARQ processes indicated to perform one-shot HARQ Feedback, only some of them are configured with "HARQ Feedback enabled" but the others with "HARQ Feedback disabled". In this case, one-shot HARQ feedback is only signaled for the HARQ process(es) that are both indicated to perform one-shot HARQ Feedback and configured with "HARQ feedback enabled".

Observation 3a: In MAC, HARQ-RTT-TimerDL-NTN should be (re)started only for the HARQ processes both indicated to perform one-shot HARQ Feedback and configured with "HARQ feedback enabled" for the case in Observation 3.

Observation 4: By reusing the wording in TN, proposed change in [5] may be misinterpreted as allowing the MAC entity to enter the loop for HARQ-RTT-TimerDL-NTN (re)start, only if all corresponding HARQ processes indicated for one-shot HARQ feedback are configured with "HARQ feedback enabled". This leads to ambiguity on whether the MAC entity can still enter this loop normally for the case in Observation 3, and consequently erroneous HARQ-RTT-TimerDL-NTN handling.

Proposal 1: RAN2 confirms that one-shot HARQ feedback feature can be supported in NR NTN since Rel-17 (based on RAN1 specification and related RAN1-introduced UE capabilities).

Proposal 2: Agree the following intended MAC behavior, in the case that one-shot HARQ feedback is indicated by PDCCH and downlinkHARQ-FeedbackDisabled is also configured:

- If at least one of the corresponding HARQ process(es) is configured with HARQ feedback enabled, start or restart the HARQ-RTT-TimerDL-NTN for the corresponding HARQ process(es) whose HARQ feedback is enabled and reported in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

Proposal 2a: Agree the TP in the Appendix.

Proposal 3: RAN2 discusses whether a Rel-17 UE capability needs to be introduced, indicating whether the UE is capable of HARQ-RTT-TimerDL-NTN handling in the case of one-shot HARQ feedback transmission.

# 4.	Reference

- RAN2 #127bis meeting minutes.

- RAN1 #107-e meeting minutes.

- 3GPP TS 38.213: "NR; Physical layer procedures for control"

- 3GPP TS 38.306: " NR; User Equipment (UE) radio access capabilities"

- R2-2409123	Correction on drx HARQ RTT timer in NTN	LG Electronics Inc.

- R2-2204016	Introduction of enhanced IIoT&URLLC support for NR	Samsung

Appendix: TP to MAC Spec (V17.10.0) based on P2

5.7	Discontinuous Reception (DRX)

The MAC entity may be configured by RRC with a DRX functionality that controls the UE's PDCCH monitoring activity for the MAC entity's C-RNTI, CI-RNTI, CS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, TPC-SRS-RNTI, AI-RNTI, SL-RNTI, SL-CS-RNTI and SL Semi-Persistent Scheduling V-RNTI. When using DRX operation, the MAC entity shall also monitor PDCCH according to requirements found in other clauses of this specification. When in RRC_CONNECTED, if DRX is configured, for all the activated Serving Cells, the MAC entity may monitor the PDCCH discontinuously using the DRX operation specified in this clause; otherwise the MAC entity shall monitor the PDCCH as specified in TS 38.213 [6].

[...]

When DRX is configured, the MAC entity shall:

[...]

1>	if a DRX group is in Active Time:

2>	monitor the PDCCH on the Serving Cells in this DRX group as specified in TS 38.213 [6];

2>	if the PDCCH indicates a DL transmission; or

2>	if the PDCCH indicates a one-shot HARQ feedback as specified in clause 9.1.4 of TS 38.213 [6]; or

2>	if the PDCCH indicates a retransmission of HARQ feedback as specified in clause 9.1.5 of TS 38.213 [6]:

3>	if this Serving Cell is configured with downlinkHARQ-FeedbackDisabled:

4>	if at least one of the corresponding HARQ process(es) is configured with HARQ feedback enabled:

5>	set HARQ-RTT-TimerDL-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerDL plus the latest available UE-gNB RTT value;

5>	start or restart the HARQ-RTT-TimerDL-NTN for the corresponding HARQ process(es) whose HARQ feedback is enabled and reported in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

3>	else:

4>	start or restart the drx-HARQ-RTT-TimerDL for the corresponding HARQ process(es) whose HARQ feedback is reported in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

NOTE 3:	When HARQ feedback is postponed by PDSCH-to-HARQ_feedback timing indicating an inapplicable k1 value, as specified in TS 38.213 [6], the corresponding transmission opportunity to send the DL HARQ feedback is indicated in a later PDCCH requesting the HARQ-ACK feedback.

3>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process(es) whose HARQ feedback is reported;

3>	stop the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process;

3>	if the PDSCH-to-HARQ_feedback timing indicate an inapplicable k1 value as specified in TS 38.213 [6]:

4>	start the drx-RetransmissionTimerDL in the first symbol after the (end of the last) PDSCH transmission (within a bundle) for the corresponding HARQ process.

[...]