3GPP TSG-RAN WG2 Meeting #128	                                                                    R2-2409623

Orlando, USA, Nov. 18th – 22nd, 2024

Source:	CATT

Title:	Discussion on the Random Access for Ambient IoT

Agenda Item:	8.2.4

Document for:	Discussion and Decision

- Introduction

This paper further discusses the remaining issues related to RACH for A-IoT [1]. In details, the following aspects are covered,

- RACH type selection

- D2R data transmission failure handling

- How to perform re-access

Section 2 gives our discussion, and Section 3 summarizes the conclusion of this paper.

- Discussion

- RACH type selection

In the last meeting discussion, it was agreed that the device determines the RACH type to perform the random access according to the A-IoT paging message,

- For all use cases, device determines the random access type from the paging message. FFS whether it is explicitly or implicitly. FFS whether we will down selection for all RA types (2step, 3step, CFRA) or try to unify the design for 2step, 3step

Whether to down select the RACH types or design an unified procedure

During the online discussion, some companies raised the concern that it may cause the complexity to the device if the device needs to support all the 2-step and 3-step CBRA as well as CFRA. Then two potential way forwards are on the table, i.e., make down selection to the RACH types or design an unified procedure.

- Whether to design an unified procedure

One way forward suggested by companies is to design an unified procedure for all RACH types. Then the general principles are summarized as follows,

1. Regardless of whether the device performs 2-step/3-step CBRA or CFRA, the device always sends 16-bit random number in A-IoT Msg1; 2. The device determines whether to send the data along with the random number based on the scheduled size of A-IoT Msg1. If larger than 16-bit, the device can include the data transmission in A-IoT Msg1; 3. The device is not aware of CBRA or CFRA, and it just follows the R2D scheduling for the next D2R transmission.

Regarding the bullet 1, it is not resource-efficient to always send the random number as it is useless for CFRA.

Observation 1: Unified procedure for CBRA and CFRA is not resource-efficient as the device always send the random number even for CFRA.

Regarding the bullet 2, if we further consider the unified procedure for 2-step CBRA and 3-step CBRA, it is indeed an implicitly indication based on the scheduled size of A-IoT Msg1 for the device to determine the CBRA type. That is to say, anyway the device needs to support both 2-step CBRA and 3-step CBRA, which does not make much contribution to the device complexity.

Observation 2: Even with the unified procedure for 2-step CBRA and 3-step CBRA, it is indeed an implicitly indication based on the scheduled size of A-IoT Msg1 for the device to determine the CBRA type, which also requires the device to support both 2-step CBRA and 3-step CBRA.

Therefore, it is proposed that,

Proposal 1: Unified procedure for CBRA and CFRA is not considered in A-IoT.

- Whether to make down selection to the RACH types

Then another key point is whether we need to make down selection to the 2-step/3-step CBRA and CFRA.

The intention to introduce CFRA is that if the reader has sufficient resource to satisfy the number of devices (based on the assistance information from CN) to perform CFRA, the reader can allocate the CFRA resource to the device, which accelerates the completion of this service request due to no contention resolution steps. From the reader point of view, it is not suggested putting restriction on the reader implementation.

Observation 3: It is not suggested putting restriction on the reader implementation to use the CFRA if the reader has sufficient resource to satisfy the number of devices for a certain service request from CN.

From the device point of view, it’s easy to support CFRA if the device supports CBRA, as the device only needs to skip the contention resolution, which introduce no further complexity to the device.

Observation 4: No further complexity to the device is introduced to support CFRA if the device supports CBRA.

Finally, at least for a service request (for example, reader command) targeted for only one device, no conflicts will occur, and thus the device can directly perform data transmission by CFRA.

Observation 5: For a read command targeted for only one device, the device should directly perform data transmission by CFRA due to no conflicts.

Therefore, it is proposed that,

Proposal 2a: RAN2 to confirm supporting CFRA for A-IoT.

Regarding the 2-step CBRA and 3-step CBRA, if both are supported, the reader needs to determine the RACH type based on the evaluation of link quality and then informs it to the device via A-IoT paging message. If the channel quality is good enough, 2-step CBRA can be used by the reader. Otherwise, it should indicate the device with 3-step CBRA. From the device complexity point of view, it requires the device to support the separated formats of A-IoT Msg1 for 2-step CBRA and 3-step CBRA. Therefore, we have a slight prefer on the supporting of only 3-step CBRA.

Proposal 2b: Only 3-step CBRA is supported for contention-based random access for A-IoT.

How to indicate RACH type by the reader

Then the next issue is whether we need the explicit or implicit indication in the A-IoT paging message so that the device can determine the RACH type.

For the CFRA, anyway the association between the dedicated RACH resource and the device information should be provided in the A-IoT paging message, so the device can implicitly know to perform CFRA if there is dedicated RACH resource allocated for itself. Otherwise, it should perform the 3-step CBRA (if needed). No need to introduce explicit indication in the A-IoT paging message.

Proposal 3: The device should perform CFRA if there is dedicated RACH resource associated with the device information in the A-IoT paging message. Otherwise, the device should perform 3-step CBRA (if needed).

Whether to allow parallel CBRA and CFRA among devices

Another issue that needs to clarify is for all devices that need to response the A-IoT paging message, whether to allow the reader to allocate the CFRA resource to part of these devices while the remaining devices perform CBRA. For example, it was agreed in last meeting that,

- If multiple device IDs in single paging is supported, if A-IoT paging message contains multiple device IDs, reader can configure either contention free RA or contention-based RA

How to understand the term “either/or”. Does that means device 1 and device 2 are provided with dedicated CFRA resource associated with the device information in the A-IoT paging message, while the remaining devices not.

If above scenario is allowed, the reader needs to ensure that the devices that perform CBRA should not select the access occasions allocated to the specific devices that perform CFRA. From our perspective, as per the SA1 use cases for the inventory or command services, there is no further requirement that part of devices have higher priority than the others on the access to the reader. Therefore, it is suggested that,

Proposal 4: Do not support parallel CBRA and CFRA among the devices that respond the same service request.

- D2R data transmission failure handling

Re-transmission due to D2R failure

For the D2R failure, it was discussed in last meeting with the following agreements,

- In case of D2R data transmission failure, device follows the reader instruction:  it is supported to re-access in another opportunity controlled/provided by the reader (i.e. retry the random access); and  reader can repeat the R2D “upper layer command” to trigger the device to re-send the same D2R “response” (i.e., device just follows the received R2D to transmit D2R).

For D2R data transmission failure, as can be seen from above agreement, the trigger of re-sending the same D2R response is by the upper layer command, i.e., re-send A-IoT data through retrying the random access. The basic principle should be that the reader re-sends the A-IoT paging message that contains the upper layer message which only triggers the “specific” failed device to re-send the corresponding failed data, instead of triggering all the devices to retry the random access.

Regarding on how to implement this intention, we may need further consideration. The reader is not aware of the device information and the response from the device, as both of which are upper layer message contained in the AS layer. So how the reader make such “specific” scheduling to the devices suffered by D2R transmission failure? After the reception of Msg2, the device is confirmed with a random number or it can be allocated by the reader with an unique “temporary ID” (according to the AS ID discussion in last meeting), thus this temporary ID information is a potential solution by the reader to associate the repeated upper layer command with these “specific” devices.

Proposal 5: When the reader repeats the upper layer command to trigger the device to re-send the same D2R response, the upper layer message contained in the A-IoT paging should only trigger the “specific” failed device to re-send the corresponding D2R data. How to implement this intention can be discussed in WI.

Failure/success feedback indication for following D2R data

Then for the A-IoT Msg3 transmission, an optional feedback mechanism was introduced to handle the failure,

- Support optional explicit R2D failure/success feedback indication for at least MSG3 for re-access purpose. FFS for following D2R data.

The intention of “optional” is if there is subsequent scheduling after A-IoT Msg3 transmission, then this subsequent scheduling message can be used to confirm the reception of A-IoT Msg3, otherwise, the explicit feedback is used to indicate whether A-IoT Msg3 is successfully received by the reader. In details, if the reader successfully receives A-IoT Msg3 and there is no following data to be scheduled, the reader will send this explicit indication to stop the AS operation for this device; if the reader does not successfully receives A-IoT Msg3, as per the previous discussion, the reader will re-send the upper layer command to trigger the device to re-transmit the corresponding data through re-access. The device just needs to follow the reader command.

For the failure/success feedback indication for following D2R data after A-IoT Msg3, the similar principle used for A-IoT Msg3 can be followed as there is no further spec impact, i.e., up to reader implementation (summarized in Table I).

Table I: Reader implementation for D2R transmission failure after A-IoT Msg3

| D2R transmission | Success indication | Failure indication |
| --- | --- | --- |
| Not the last data after A-IoT Msg3 | Implicitly indicated by the following scheduling | Reader re-send the upper layer command to trigger the device to re-transmit the data by re-access |
| The last data after A-IoT Msg3 | Explicit success feedback indication (e.g., 1 bit ACK) |  |

As per above Table I, it’s up to reader implementation whether to include such explicit feedback indication based on the D2R transmission, i.e., no further spec impact is found.

Proposal 6: Support optional explicit feedback indication for D2R transmission after A-IoT Msg3. It’s up to reader implementation whether to use this explicit feedback indication for a D2R transmission after A-IoT Msg3.

- How to perform re-access

For the re-access procedure performed by the device,

- R2D message is used to provide access occasion(s) which can be used for re-access purpose. One option to be captured in the TR is that this R2D message is AIoT paging message. FFS additional indications to differentiate if needed in paging message. FFS if other message can be used.

The A-IoT paging can provide the access occasions for re-access purpose. The intention is to allow the device to perform re-access upon receiving the subsequent A-IoT paging while the device suffers the contention resolution failure upon receiving the previous/current A-IoT paging.

FFS additional indications to differentiate

An issue was raised that whether to introduce further indication to the device so that the device can select some particular access occasions which may not collide with other devices that perform initial access in the same round triggered by the current A-IoT paging. From our view, this is a beneficial way to relieve the device aggregation due to the contention resolution failure in the previous access attempt. However, this can be left to reader implementation, for example, the reader finds only a few number of device messages are successfully detected compared with the approximate inventoried devices, the reader can adjust the number of access occasions by the A-IoT paging, and also provide some particular access occasions for re-access purpose.

Proposal 7: It’s up to reader to optionally adjust the number of access occasions and provide the particular access occasions for re-access purpose by the A-IoT paging.

FFS if other message can be used

Another issue is whether to allow re-access in the same round triggered by the current A-IoT paging, i.e., introduce a new message to adjust the access occasions between two A-IoT paging messages. To our view, there may be the situation where the devices are distributed within some adjacent access occasions. Thus, the reader may want to trigger the device to randomly re-select the access occasions. Along with this intention, up to reader implementation, the reader can also adjust the number of access occasions between the two A-IoT paging messages.

Proposal 8: Reader is allowed to trigger the device to randomly re-select the access occasions between two A-IoT paging messages. RAN2 to introduce a new R2D message for this purpose.

- Conclusion

In this paper, we further discuss the RACH-related issues for A-IoT. The observations and proposals of this paper are summarized as follows,

Observation 1: Unified procedure for CBRA and CFRA is not resource-efficient as the device always send the random number even for CFRA.

Observation 2: Even with the unified procedure for 2-step CBRA and 3-step CBRA, it is indeed an implicitly indication based on the scheduled size of A-IoT Msg1 for the device to determine the CBRA type, which also requires the device to support both 2-step CBRA and 3-step CBRA.

Observation 3: It is not suggested putting restriction on the reader implementation to use the CFRA if the reader has sufficient resource to satisfy the number of devices for a certain service request from CN.

Observation 4: No further complexity to the device is introduced to support CFRA if the device supports CBRA.

Observation 5: For a read command targeted for only one device, the device should directly perform data transmission by CFRA due to no conflicts.

RACH type selection

Proposal 1: Unified procedure for CBRA and CFRA is not considered in A-IoT.

Proposal 2a: RAN2 to confirm supporting CFRA for A-IoT.

Proposal 2b: Only 3-step CBRA is supported for contention-based random access for A-IoT.

Proposal 3: The device should perform CFRA if there is dedicated RACH resource associated with the device information in the A-IoT paging message. Otherwise, the device should perform 3-step CBRA (if needed).

Proposal 4: Do not support parallel CBRA and CFRA among the devices that respond the same service request.

D2R data transmission failure handling

Proposal 5: When the reader repeats the upper layer command to trigger the device to re-send the same D2R response, the upper layer message contained in the A-IoT paging should only trigger the “specific” failed device to re-send the corresponding D2R data. How to implement this intention can be discussed in WI.

Proposal 6: Support optional explicit feedback indication for D2R transmission after A-IoT Msg3. It’s up to reader implementation whether to use this explicit feedback indication for a D2R transmission after A-IoT Msg3.

How to perform re-access

Proposal 7: It’s up to reader to optionally adjust the number of access occasions and provide the particular access occasions for re-access purpose by the A-IoT paging.

Proposal 8: Reader is allowed to trigger the device to randomly re-select the access occasions between two A-IoT paging messages. RAN2 to introduce a new R2D message for this purpose.

- References

[1] Draft_RAN2_127b_Meeting_Report_v1