3GPP TSG-RAN WG2 Meeting #128                                                        R2-2409625

Orlando, USA, Nov. 18th – 22nd, 2024

Source:              CATT

Title:	             Consideration on Random Access in SBFD Symbols

Agenda Item:	8.11.2

Document for:	Discussion and Decision

Introduction

In RAN2#127 and RAN2#127bis meetings, the random access in SBFD symbols was discussed. But there are still some leftover issues. In this contribution, the following leftover issues will be further discussed:

- Whether the working assumption on supported RACH triggers can be confirmed?

- Whether early identification in Msg3 needs to be supported when legacy RO is selected?

- Whether it is possible to support both RACH configurations in one cell?

- Whether addition enabler is needed for RACH configuration Alt1-1 of Option 1?

- For CBRA, how to determine the RO type for initial RACH transmission?

- For CBRA, whether it is possible that UE first selects legacy RO and then performs RO type fallback after a number of RACH attempt?

- For CFRA, how to determine the RO type for initial RACH transmission?

- How to determine the RO type in case of CFRA/CBRA fallback?

- Whether 2-step RA on SBFD symbols needs to be supported?

Discussion

Whether the working assumption on supported RACH triggers can be confirmed?

In RAN2#127 meeting, regarding to the supported RACH triggers, the following working assumption was reached:

- Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

In[1], all the RACH triggers are listed as below:

### 9.2.6 Random Access Procedure The random access procedure is triggered by a number of events: - Initial access from RRC_IDLE; - RRC Connection Re-establishment procedure; - DL or UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure (see clause 18.0) is ongoing, when UL synchronisation status is "non-synchronised"; - UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when there are no PUCCH resources for SR available; - Handover, except for when RACH-less HO is configured; - SR failure; - Explicit request by RRC upon synchronous reconfiguration; - RRC Connection Resume procedure from RRC_INACTIVE; - To establish time alignment for a primary or a secondary TAG; - Request for Other SI (see clause 7.3); - Beam failure recovery; - Consistent UL LBT failure on SpCell; - SDT in RRC_INACTIVE (see clause 18); - Positioning purpose during RRC_CONNECTED requiring random access procedure, e.g., when timing advance is needed for UE positioning; - Early UL synchronization with an LTM candidate cell; - RACH-based LTM cell switch.

From RAN2 perspective, allowing SBFD-aware to use RACH resources in SBFD symbol can reduce the RACH latency and improve RACH capacity. There is no motivation to prohibit SBFD-aware UE to use RACH resource on SBFD symbol for any of the above RACH trigger events. In RAN2#127bis meeting, some companies suggested excluding some RACH triggers which are not delay-sensitive, such as Msg1-based SI request and etc., but the motivation is not persuasive. Hence, the above working assumption reached in RAN2#127 meeting can be confirmed.

Proposal 1: Confirm the working assumption that random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

Whether early identification is needed when legacy RO is selected?

In RAN2#127bis meeting, the following agreements were reached:

RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact. From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework.

Based on the above agreements, it is obvious that if additional RO is selected, early identification via Msg1 is possible. If legacy RO is selected, since SBFD is not introduced as a feature combination, early identification via Msg1 is not supported.

Based on the above background, what needs to be discussed next is whether early identification via Msg3 needs to be supported in case of legacy RO is selected. According to [1], the RRC connection setup procedure is as below:

Figure-1 UE triggered transition from RRC_IDLE to RRC_CONNECTED

If early identification via Msg3 (step 1 in the Figure-1) is supported, the NW can allocate UL resource on SBFD symbols for RRCSetupComplete(step2a), ULInformationTransfer(step5) and securityModeComplete(step7a). It can reduce the UE access latency. On the contrary, if early identification via Msg3 is not supported, the NW can only aware whether the UE supports SBFD via UE capability enquiry procedure which is performed after AS security activation (e.g., after step 7a in Figure-1). It means step 2a/step 5/step 7a can only on transmitted on UL subframes. In order to reduce the access latency, it is slightly preferred to support early identification via Msg3 in case of legacy RO is selected.

Proposal 2: If legacy RO is selected, early identification via Msg3 can be introduced.

Furthermore, early identification via Msg3 has already been supported for other features, such as (e)RedCap UE and PUCCH repetition of Msg4 HARQ-ACK. In order to support early identification via Msg3 for these features, new LCIDs for CCCH were introduced. For SBFD, if early identification via Msg3 needs to be supported, the same early identification method can be reused, that is to introduce new LCIDs for CCCH of SBFD-aware UE.

Proposal 3: If legacy RO is selected and early identification is supported for SBFD-aware UE, new LCIDs for CCCH SDU specific for SBFD-aware UE can be introduced.

Whether it is possible to support both RACH configurations in one cell？

In RAN1#118bis meeting, the following conclusion was reached:

The following two RACH configuration options are considered for SBFD based random access: - Option 1: Use one single RACH configuration based on the existing parameters of the single RACH configuration. Can extend the existing parameters if needed. - Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration For RACH configuration Option 2, RAN2 needs to specify RRC signalling for the new SBFD based RACH configuration with a new set of parameters. The RACH configuration for SBFD is transmitted via SIB1. FFS dedicated RRC signalling detail. FFS whether NW can provide both configurations.

Based on the above agreement, it is still FFS whether the NW can provide both configurations. Regarding to this FFS, there are two alternatives:

- Alternative 1: Only one RACH configuration (either Alt1-1 of Option 1 or Alt2-3 of Option 2) can be supported in one cell.

- Alternative 2: Two RACH configurations (both Alt1-1 of Option 1 and Alt2-3 of Option 2) can be supported in one cell.

But in RAN1#118bis meeting, the following conclusion was reached:

Conclusion There is no consensus in RAN1 to support the following in Rel-19: - Enabling both RACH configuration Option 1 and Option 2 for SBFD random access operation in a cell from network side.

Based on the above RAN1 conclusion, it is obvious that Alternative 2 is excluded. Hence, there is no need to re-open the discussion on this issue in RAN1.

Proposal 4: Follow RAN1 conclusion, RAN2 agreed that providing both RACH configuration Option 1 and Option 2 in a cell is not supported.

Enabler for RACH configuration Alt1-1 of Option 1

For Alt 1-1 of Option 1, additional ROs are derived only based on the legacy RACH configuration. UE cannot differentiate whether RACH on SBFD symbols is enabled or not in NW side. Hence, it is suggested to introduce an explicit indication to indicate that whether RA on SBFD symbol is enabled or not for RACH configuration Alt1-1 of Option 1.

Proposal 5: For RACH configuration Alt1-1 of Option 1, introduce an explicit indication to indicate whether RA on SBFD symbol is enabled or not.

Initial RO type selection for CBRA

In RAN2#127bis, regarding to the RO type selection for CBRA, the following agreements were reached:

Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network.

Based on the above agreement, it is obvious that upon initiation of CBRA, UE should select the RO type, but the detailed principle for selecting the RO type is still FFS. Regarding to this issue, there are mainly four different solutions:

- Solution 1:  NW explicitly indicates the RO type.

- Solution 2:  Based on RSRP threshold configured by NW;

- Solution 3:  SBFD-aware UE always prioritize additional RO;

- Solution 4:  SBFD-aware UE always selects the nearest RO.

For Solution 1, considering UE initiating CBRA may be in RRC_IDLE/INACTIVE state, NW may have no information about it and only a per cell indication is feasible. Hence, it is hard for the NW to determine the proper RO type.

For Solution 2, the same solution is also used for 2/4 step RA selection. If the RSRP of the DL pathloss reference is above the threshold, 2-step RA can be selected; otherwise, 4-step RA can be selected. The reason of using RSRP is that if the RSRP is above the threshold, it means the channel quality is good and more data can be carried in the first RACH message, hence 2-step can be selected. But for SBFD, RAN2 is doubtable when RSRP is above threshold, which RO type is more suitable. Hence, it had better send LS to RAN1 to check whether RO type selection based on RSRP is feasible.

For Solution 3, if all UEs in the cell support SBFD, there is the risk that all UE select the additional RO, which will increase the RACH failure probability in additional RO and also result the resource waste in legacy RO.

For Solution 4, it is same as the current RO selection rule, and the UE can initiate the RACH as soon as possible. But the main problem of this solution is there should be two sets of RACH resource during RACH initialization, which is contradict that only one set of RACH resource is selected during RACH initialization.

Based on the above analysis, Solution 1/3/4 all have obvious shortages, whether Solution 2 is feasible depends on RAN1. Hence, it had better send LS to RAN1.

Proposal 6: For CBRA initial transmission, send LS to RAN1 to check whether RO selection can be based on RSRP.

RO type fallback from legacy RO to additional RO for CBRA

In RAN2#127bis, regarding to the RO type fallback, the following agreement was reached:

For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

Regarding to the FFS, there are two issues needs to be discussed:

- Whether it is possible for an SBFD-aware UE to select legacy RO for initial RO selection?

- If the SBFD-aware UE initially select a legacy RO, whether the RO type fallback to additional RO is allowed for a number of RACH attempt?

For issue 1), the answer tightly depends on which solution is selected in section 2.5. If solution 3 is selected, the answer of issue 1 is No; otherwise, the answer is Yes.

For issue 2), it can be further discussed after the answer of issue 1) is determined.

Proposal 7: For CBRA initial transmission, the discussion on whether legacy RO can be selected firstly and whether RO type fallback can be supported when legacy RO is selected firstly can be postponed until there is conclusion on the initial RO type selection criteria.

Initial RO type selection for CFRA

In RAN1#117 meeting, RAN1 has agreed that for SBFD-aware UEs in RRC_CONNECTED state, support CBRA and CFRA in SBFD symbols. In last RAN2#127bis meeting, it was agreed that for CBRA, RO type selection is needed. But there is no discussion on CFRA.

In NR, CFRA can be triggered due to the following cases:

- Msg1-based SI request

This is applicable for RRC_IDLE/RRC_INACTIVE UE, and the dedicated CFRA resource is configured in SIB1.

- BFR

The dedicated BFR related RACH resources can be configured via dedicated RRC signalling before beam failure.

- RRCReconfigurationWithSync

The dedicated RACH resources are configured via dedicated RRC signalling.

- LTM

The dedicated RACH resources are configured via dedicated RRC signalling.

- PDCCH order

The dedicated RACH resources are configured via PDCCH order.

For RACH configuration Alt2-3 of Option 2, the SSB/RO mapping is separate for legacy RACH configuration and the additional RACH configuration. Hence, when NW indicates the dedicated RACH resource, it had better indicate the RO type directly no matter SIB or dedicated RRC signalling is used. For RACH configuration Alt1-1 of Option 1, the same rule can also be applied.

Proposal 8: The RO type should be indicated by NW for CFRA.

RO type selection when CFRA/CBRA fallback happens

According to [2], the following two kinds of RA type fallbacks are supported for CBRA and CFRA:

- Case 1: CFRA fallbacks to CBRA.

- Case 2: CFRA fallbacks to CBRA, and CBRA fallbacks to CFRA again…….

For Case 1, CFRA fallbacks to CBRA include the following two sub-cases:

- Sub-case 1: CFRA is triggered but there is no suitable SSB/CSI-RS (i.e. no SSB/CSI-RS satisfy the RSRP requirement) or the preamble index indicated in PDCCH order is 0b000000;

- Sub-case 2: CFRA is performed for N(N≥ 1) times and when performs RA re-attempt, no SSB/CSI-RS satisfy the RSRP requirement.

For Case 2, after CFRA fallback to CBRA, for RACH re-attempt, once there is suitable SSB/CSI-RS, it can fallback to CFRA again. And during CFRA, if no SSB/CSI-RS satisfy the RSRP threshold, CFRA can fallback to CBRA again…….

According to the discussion in section 2.7, for CFRA, the RO type can be indicated by NW. In case of CFRA fallback to CBRA, there are two kinds of RO type selection methods:

- Alternative 1:  Always use the RO type indicated by NW for CFRA;

- Alternative 2:  When performing CFRA, uses the RO type indicated by NW; when performs CBRA, follow the RO type determine method proposed in section 2.5.

For Alternative 1, the RO type can be same for CFRA and CBRA. While for Alternative 2, it is possible that the RO type can be different for consecutive two RACH procedure and it needs to discuss how to handle the power ramping. Hence, Alternative 1 is slightly preferred.

Proposal 9: If CFRA/CBRA fallback, it can use the RO type indicated by NW.

Furthermore, for CBRA, it was agreed that RO type fallback can be performed after a number of RA attempts. It should further discuss whether the combination of RA type fallback and RO type fallback needs to be considered. Considering for CFRA, the RO type is indicated by NW. It means the RACH performance using the indicated RO type is better than another RO type. Hence, RO type fallback is unnecessary.

Proposal 10: For CFRA/CBRA fallback, RO type fallback is not considered.

Whether 2-step RA on SBFD symbols needs to be supported?

In Rel-16, 2-step RA was introduced to reduce RA latency. When SBFD is introduced in Rel-19, it should further discuss whether 2-step RA on SBFD symbols needs be supported. Regarding to this issue, in RAN1#116 meeting and in RAN2#127 meeting, the following agreements were reached:

Agreements in RAN1#116 meeting: For SBFD aware UEs in RRC CONNECTED state, support Type-1 random access procedure (4-step RACH) in SBFD symbols. - FFS Type-2 random access procedure (2-step RACH) Agreements in RAN2#127 meeting: RAN2 focus on 4-step RACH for SBFD RA, FFS on 2-step if needed.

The main benefit of introducing 2-step RA is to reduce the latency, which is same as one of the motivation of introducing SBFD. Since both of the two RACH features can achieve the same effect, whether the combination of them needs justification.

Proposal 11: Whether the combination of SBFD and 2-step RA should be supported needs to be justified.

Conclusion

According to the analysis in section 2, it is proposed:

RACH triggers

Proposal 1: Confirm the working assumption that random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

Early identification in Msg3

Proposal 2: If legacy RO is selected, early identification via Msg3 can be introduced.

Proposal 3: If legacy RO is selected and early identification is supported for SBFD-aware UE, new LCIDs for CCCH SDU specific for SBFD-aware UE can be introduced.

Support of both RACH in one cell

Proposal 4: Follow RAN1 conclusion, RAN2 agreed that providing both RACH configuration Option 1 and Option 2 in a cell is not supported.

Additional enabler for RACH configuration Option 1

Proposal 5: For RACH configuration Alt1-1 of Option 1, introduce an explicit indication to indicate whether RA on SBFD symbol is enabled or not.

RO type selection for initial RACH attempt (CBRA)

Proposal 6: For CBRA initial transmission, send LS to RAN1 to check whether RO selection can be based on RSRP.

Proposal 7: For CBRA initial transmission, the discussion on whether legacy RO can be selected firstly and whether RO type fallback can be supported when legacy RO is selected firstly can be postponed until there is conclusion on the initial RO type selection criteria.

RO type selection for initial RACH attempt (CFRA)

Proposal 8: The RO type should be indicated by NW for CFRA.

RO type selection for  CFRA/CBRA fallback

Proposal 9: If CFRA/CBRA fallback, it can use the RO type indicated by NW.

Proposal 10: For CFRA/CBRA fallback, RO type fallback is not considered.

Support of 2-step RA

Proposal 11: Whether the combination of SBFD and 2-step RA should be supported needs to be justified.

Reference

- TS38.300  NR; NR and NG-RAN Overall Description; Stage 2; (Release-18)

- TS38.321  NR; Medium Access Control (MAC) protocol specification (Release-18)