3GPP TSG-RAN WG2 Meeting #128	R2-2409628

# Orlando, USA, Nov. 18th - 22nd, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR | 0529 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on the NOTE in the description of dl-PRS-MeasRRC-Inactive |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Note in the description of dl-PRS-MeasRRC-Inactive in UE capability is ambiguity. The note says these capabilities are the same in RRC_INACTIVE state. However these capabilities mentioned in the note are not always the same because for example dl-RSTD-MeasurementPerPairOfTRP-FR1 can be different from dl-RSTD-MeasurementPerPairOfTRP-FR2. Actually, these capabilities in RRC_INACTIVE state are the same as these in RRC_CONNECTED state. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. NOTE 2 in the dl-PRS-MeasRRC-Inactive for NR-DL-TDOA-MeasurementCapability is clarified as these capabilities in RRC_CONNECTED state are the same as in RRC_INACTIVE respectively. 2. NOTE 2 in the dl-PRS-MeasRRC-Inactive for NR-DL-AoD-MeasurementCapability is clarified as these capabilities in RRC_CONNECTED state are the same as in RRC_INACTIVE respectively. 3. NOTE in the dl-PRS-MeasRRC-Inactive for NR-Multi-RTT-MeasurementCapability is clarified as these capabilities in RRC_CONNECTED state are the same as in RRC_INACTIVE respectively. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The clarification of capabilities in RRC_INACTIVE state is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6 6.5.10, 6.5.11, 6.5.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

6.5.10	NR DL-TDOA Positioning

*******************Skip the unchanged part*************

6.5.10.6a	NR DL-TDOA Capability Information Elements

–	NR-DL-TDOA-MeasurementCapability

The IE NR-DL-TDOA-MeasurementCapability defines the DL-TDOA measurement capability. The UE can include this IE only if the UE supports NR-DL-PRS-ResourcesCapability for DL-TDOA. Otherwise, the UE does not include this IE.

-- ASN1START

NR-DL-TDOA-MeasurementCapability-r16 ::= SEQUENCE {

dl-RSTD-MeasurementPerPairOfTRP-FR1-r16			INTEGER (1..4),

dl-RSTD-MeasurementPerPairOfTRP-FR2-r16			INTEGER (1..4),

supportOfDL-PRS-RSRP-MeasFR1-r16				ENUMERATED { supported}	OPTIONAL,

supportOfDL-PRS-RSRP-MeasFR2-r16				ENUMERATED { supported}	OPTIONAL,

...,

[[

nr-UE-TEG-Capability-r17						NR-UE-TEG-Capability-r17			OPTIONAL,

dl-tdoa-MeasCapabilityBandList-r17				SEQUENCE (SIZE (1..nrMaxBands-r16)) OF

DL-TDOA-MeasCapabilityPerBand-r17	OPTIONAL

]]

}

DL-TDOA-MeasCapabilityPerBand-r17 ::= SEQUENCE {

freqBandIndicatorNR-r17				FreqBandIndicatorNR-r16,

supportOfDL-PRS-FirstPathRSRP-r17	ENUMERATED { supported }			OPTIONAL,

dl-PRS-MeasRRC-Inactive-r17			ENUMERATED { supported }			OPTIONAL,

...

}

-- ASN1STOP

NR-DL-TDOA-MeasurementCapability field descriptions

dl-RSTD-MeasurementPerPairOfTRP-FR1 Indicates number of DL RSTD measurements per pair of TRPs on FR1.

dl-RSTD-MeasurementPerPairOfTRP-FR2 Indicates number of DL RSTD measurements per pair of TRPs on FR2.

supportOfDL-PRS-RSRP-MeasFR1 Indicates whether the UE supports DL-PRS RSRP measurement for DL-TDOA on FR1.

supportOfDL-PRS-RSRP-MeasFR2 Indicates whether the UE supports DL-PRS RSRP measurement for DL-TDOA on FR2.

nr-UE-TEG-Capability Indicates the UE TEG capability.

supportOfDL-PRS-FirstPathRSRP Indicates whether the target device supports DL-PRS RSRPP of first path measurement for DL-TDOA. The UE can include this field only if the UE supports prs-ProcessingCapabilityBandList. Otherwise, the UE does not include this field. The UE supporting additionalPathsReport and supportOfDL-PRS-FirstPathRSRP shall support RSRPP reporting for K=1 or 2 additional paths.

dl-PRS-MeasRRC-Inactive This field, if present, indicates that the target device supports DL-PRS measurement in RRC_INACTIVE state. The UE can include this field only if the UE supports maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer, maxNrOfTRP-AcrossFreqs, maxNrOfPosLayer and dl-PRS-BufferType-RRC-Inactive. Otherwise, the UE does not include this field. NOTE 1: This capability is applicable to both, UE-assisted and UE-based DL-TDOA. NOTE 2: The capabilities NR-DL-PRS-ResourcesCapability, dl-RSTD-MeasurementPerPairOfTRP-FR1, dl-RSTD-MeasurementPerPairOfTRP-FR2, supportOfDL-PRS-RSRP-MeasFR1, supportOfDL-PRS-RSRP-MeasFR2, simul-NR-DL-AoD-DL-TDOA in RRC_INACTIVE state are the same in RRC_CONNECTED state respectively.

NEXT CHANGE

6.5.11	NR DL-AoD Positioning

*******************Skip the unchanged part*************

6.5.11.6a	NR DL-AoD Capability Information Elements

–	NR-DL-AoD-MeasurementCapability

The IE NR-DL-AoD-MeasurementCapability defines the DL-AoD measurement capability. The UE can include this IE only if the UE supports NR-DL-PRS-ResourcesCapability for DL-AoD. Otherwise, the UE does not include this IE;

-- ASN1START

NR-DL-AoD-MeasurementCapability-r16 ::= SEQUENCE {

maxDL-PRS-RSRP-MeasurementFR1-r16		INTEGER (1..8),

maxDL-PRS-RSRP-MeasurementFR2-r16		INTEGER (1..8),

dl-AoD-MeasCapabilityBandList-r16		SEQUENCE (SIZE (1..nrMaxBands-r16)) OF

DL-AoD-MeasCapabilityPerBand-r16,

...,

[[

maxDL-PRS-RSRP-MeasurementFR1-v1730		ENUMERATED { n16, n24 }					OPTIONAL,

maxDL-PRS-RSRP-MeasurementFR2-v1730		ENUMERATED { n16, n24 }					OPTIONAL

]]

}

DL-AoD-MeasCapabilityPerBand-r16 ::= SEQUENCE {

freqBandIndicatorNR-r16					FreqBandIndicatorNR-r16,

simul-NR-DL-AoD-DL-TDOA-r16				ENUMERATED { supported}					OPTIONAL,

simul-NR-DL-AoD-Multi-RTT-r16			ENUMERATED { supported}					OPTIONAL,

...,

[[

maxDL-PRS-FirstPathRSRP-MeasPerTRP-r17	ENUMERATED { n1, n2, n4, n8, n16, n24 }	OPTIONAL,

dl-PRS-MeasRRC-Inactive-r17				ENUMERATED { supported }				OPTIONAL

]]

}

-- ASN1STOP

NR-DL-AoD-MeasurementCapability field descriptions

maxDL-PRS-RSRP-MeasurementFR1 Indicates the maximum number of DL-PRS RSRP measurements on different PRS resources from the same TRP supported by the UE on FR1. If this field with suffix -v1730 is present, the target device should set the field with suffix -r16 to value '8'.

maxDL-PRS-RSRP-MeasurementFR2 Indicates the maximum number of DL-PRS RSRP measurements on different PRS resources from the same TRP supported by the UE on FR2. If this field with suffix -v1730 is present, the target device should set the field with suffix -r16 to value '8'.

simul-NR-DL-AoD-DL-TDOA Indicates whether the UE supports simultaneous processing for DL-AoD and DL-TDOA measurements. The UE can include this field only if the UE supports DL-TDOA and DL-AoD. Otherwise, the UE does not include this field.

simul-NR-DL-AoD-Multi-RTT Indicates whether the UE supports simultaneous processing for DL-AoD and UE Multi-RTT measurements. The UE can include this field only if the UE supports Multi-RTT and DL-AoD. Otherwise, the UE does not include this field.

maxDL-PRS-FirstPathRSRP-MeasPerTRP This field, if present, indicates that the target device supports measuring and reporting the PRS RSRPP of the first path. The enumerated value indicates the maximum number of RSRPP of first path per TRP supported. The UE can include this field only if the UE supports one of maxDL-PRS-RSRP-MeasurementFR1 and maxDL-PRS-RSRP-MeasurementFR2. Otherwise, the UE does not include this field. NOTE 1: The maximum number of first path PRS RSRP per TRP should be less than or equal to the maximum number of PRS RSRP defined in maxDL-PRS-RSRP-MeasurementFR1 and maxDL-PRS-RSRP-MeasurementFR2.

dl-PRS-MeasRRC-Inactive This field, if present, indicates that the target device supports DL-PRS measurement in RRC_INACTIVE state. The UE can include this field only if the UE supports maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer, maxNrOfTRP-AcrossFreqs, maxNrOfPosLayer and dl-PRS-BufferType-RRC-Inactive. Otherwise, the UE does not include this field. NOTE 1: This capability is applicable to both, UE-assisted and UE-based DL-AoD. NOTE 2: The capabilities NR-DL-PRS-ResourcesCapability, simul-NR-DL-AoD-DL-TDOA in RRC_INACTIVE state are the same in RRC_CONNECTED state respectively.

NEXT CHANGE

6.5.12	NR Multi-RTT Positioning

*******************Skip the unchanged part*************

6.5.12.6a	NR Multi-RTT Capability Information Elements

–	NR-Multi-RTT-MeasurementCapability

The IE NR-Multi-RTT-MeasurementCapability defines the Multi-RTT measurement capability. The UE can include this IE only if the UE supports NR-DL-PRS-ResourcesCapability for Multi-RTT. Otherwise, the UE does not include this IE;

-- ASN1START

NR-Multi-RTT-MeasurementCapability-r16 ::= SEQUENCE {

maxNrOfRx-TX-MeasFR1-r16					INTEGER (1..4)						OPTIONAL,

maxNrOfRx-TX-MeasFR2-r16					INTEGER (1..4)						OPTIONAL,

supportOfRSRP-MeasFR1-r16					ENUMERATED { supported }			OPTIONAL,

supportOfRSRP-MeasFR2-r16					ENUMERATED { supported }			OPTIONAL,

srs-AssocPRS-MultiLayersFR1-r16				ENUMERATED { supported }			OPTIONAL,

srs-AssocPRS-MultiLayersFR2-r16				ENUMERATED { supported }			OPTIONAL,

...,

[[

nr-UE-TEG-Capability-r17					NR-UE-TEG-Capability-r17			OPTIONAL,

multi-RTT-MeasCapabilityBandList-r17		SEQUENCE (SIZE (1..nrMaxBands-r16)) OF

Multi-RTT-MeasCapabilityPerBand-r17

OPTIONAL

]]

}

Multi-RTT-MeasCapabilityPerBand-r17 ::= SEQUENCE {

freqBandIndicatorNR-r17					FreqBandIndicatorNR-r16,

supportOfDL-PRS-FirstPathRSRP-r17		ENUMERATED { supported }			OPTIONAL,

dl-PRS-MeasRRC-Inactive-r17				ENUMERATED { supported }			OPTIONAL,

...

}

-- ASN1STOP

NR-Multi-RTT-MeasurementCapability field descriptions

maxNrOfRx-TX-MeasFR1 Indicates the maximum number of UE Rx–Tx time difference measurements corresponding to a single SRS resource/resource set for positioning with each measurement corresponding to a single DL-PRS resource/resource set on FR1.

maxNrOfRx-TX-MeasFR2 Indicates the maximum number of UE Rx–Tx time difference measurements corresponding to a single SRS resource/resource set for positioning with each measurement corresponding to a single DL-PRS resource/resource set on FR2.

srs-AssocPRS-MultiLayersFR1 Indicates whether the UE supports measurements derived on one or more DL-PRS resource/resource sets which may be in different positioning frequency layers for SRS transmitted in a single CC. PRS and SRS may be on different bands. This is for FR1 only.

srs-AssocPRS-MultiLayersFR2 Indicates whether the UE supports measurements derived on one or more DL-PRS resource/resource sets which may be in different positioning frequency layers for SRS transmitted in a single CC. PRS and SRS may be on different bands. This is for FR2 only.

supportOfRSRP-MeasFR1 Indicates whether the UE supports RSRP measurement for Multi-RTT on FR1.

supportOfRSRP-MeasFR2 Indicates whether the UE supports RSRP measurement for Multi-RTT on FR2.

nr-UE-TEG-Capability Indicates the UE TEG capability.

supportOfDL-PRS-FirstPathRSRP Indicates whether the target device supports DL-PRS RSRPP of first path measurement for Multi-RTT. The UE can include this field only if the UE supports prs-ProcessingCapabilityBandList. Otherwise, the UE does not include this field. The UE supporting additionalPathsReport and supportOfDL-PRS-FirstPathRSRP shall support RSRPP reporting for K=1 or 2 additional paths.

dl-PRS-MeasRRC-Inactive This field, if present, indicates that the target device supports DL-PRS measurement in RRC_INACTIVE state. The UE can include this field only if the UE supports maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer, maxNrOfTRP-AcrossFreqs, maxNrOfPosLayer and dl-PRS-BufferType-RRC-Inactive. Otherwise, the UE does not include this field. NOTE: The capabilities NR-DL-PRS-ResourcesCapability, maxNrOfRx-TX-MeasFR1, maxNrOfRx-TX-MeasFR2, supportOfRSRP-MeasFR1, supportOfRSRP-MeasFR2, srs-AssocPRS-MultiLayersFR1, srs-AssocPRS-MultiLayersFR2, simul-NR-DL-AoD-Multi-RTT in RRC_INACTIVE state are the same in RRC_CONNECTED state respectively.

END OF CHANGE