3GPP TSG-RAN WG2 Meeting #128		R2-2409632

Orlando, USA, 18th – 22nd Nov. 2024

Source:	vivo

Title:		Discussion on topology and intermediate relay UE (re)selection

Agenda Item:		8.13.2

Document for:	Discussion and Decision

- Introduction

In RAN2 #127 meeting, agenda ‘Relay discovery and (re)selection’ was discussed and some initial agreements were reached, mainly for general principles and especially which are applicable to remote UE and the last relay UE.

For the discovery/ relay (re)selection at intermediate relay UE, as well as the support for cross-path topologies, they were covered by the following post-email discussion:

[Post127][401][Relay] Multi-hop relay discovery and (re)selection (LG)

In this contribution, we would like to further discuss some issues which are touched in this post-email discussion but we think needs more clarifications, including:

- The relay (re)selection at intermediate relay UE;

- Whether to support some special cases of cross-path topologies, considering the impact of L2 ID(s).

- Discussion

  - Intermediate relay UE reselection

According to the discussion in the post-email [Post127][401], the discovery procedure at intermediate relay UE has already been discussed for the following aspects[1]:

- For mode-A announcement initiation/forwarding at intermediate relay UE

- For mode-B solicitation forwarding at intermediate relay UE

And according to SA2 TR[2], the most relevant conclusions for relay (re)selection are as follows:

- When Model A discovery is performed, the Remote UE may choose the Intermediate Relay based on the announcement message sent by the Intermediate Relay.

- When Model B discovery is performed, the Remote UE selects both the UE-to-Network Relay and the path to reach the UE-to-Network Relay. To perform link management, the DCR message is unicasted between Relays according to the path information included in the message. The path information is an (ordered) list of User Info ID of Relays in the selected path. The Remote UE sends the selected path information to the Intermediate UE-to-Network Relay for communication setup.

- The relay path is selected based on e.g. the PC5 signal strength, number of hops between the Remote UE and the UE-to-Network Relay UE, per-hop or cumulative QoS information. The path selection criteria and the support of these metrics can be determined during the normative phase.

We understand especially for initial relay selection, it can be just left to remote UE to select both the UE-to-Network Relay and the path to reach the UE-to-Network Relay, according to SA2 TR.

Observation 1: Relay selection is performed by remote UE according to SA2 TR 23.700-03.

However, for the relay reselection at intermediate relay UE, which is related to the follow proposal in post-email discussion, we have different understanding:

[Post127][401] Proposal 7: Following relay (re)selection triggering conditions are not supported for intermediate Relay UE. [Relay Triggering conditions for intermediate Relay UE] - If the intermediate Relay UE has no serving cell; - If the intermediate Relay UE does not have a selected last (/serving intermediate) Relay UE; - If the SL-RSRP of the currently selection last (/serving intermediate) Relay UE is below a configured threshold; - If the SD-RSRP of the currently selected last (/serving intermediate) Relay UE is below a configured threshold; - If the upper layer indicates not to use the currently selected last (/serving intermediate) Relay UE; - If the upper layer of the selected last (/serving intermediate) Relay UE requests the release of the PC5-RRC connection; - If the RLF is detected on the PC5-RRC connection with the current last (/serving intermediate) Relay UE; - If the intermediate Relay UE receives a notification message from the last (/serving intermediate) Relay UE caused by one of the following: h-1) if last Relay UE detects Uu RLF h-2) if serving intermediate Relay UE detects PC5 RLF h-3) if last (/serving intermediate) Relay UE receives RRCReconfiguration message for HO h-4) if last (/serving intermediate) Relay UE performs cell reselection h-5) if last (/serving intermediate) Relay UE fails Uu RRC connection establishment/Resume h-6) if last (/serving intermediate) Relay UE receives Uu connection release between gNB and last (/serving intermediate) Relay UE h-7) if serving intermediate Relay UE receives PC5-RRC connection release between last (/the other serving intermediate) Relay UE and the serving intermediate Relay UE

At least for some conditions, e.g. k),l) to trigger reselection, it could be possible that the intermediate relay UE can do the reselection itself, especially considering that we need the extension to multi-hop case, where it may be easier to allow last relay UE reselection by Intermediate Relay UE especially when only the link between them deteriorates. If it is always the remote UE to reselect all relay UEs, it could cause delay and the performance may not be so good as some of the links are actually good enough to be used continually.

Observation 2: Making the intermediate relay UE capable of reselecting relay link by itself can avoid the impact on the good link between remote UE and this intermediate relay UE.

Therefore, we propose:

Proposal 1: One intermediate Relay UE can perform relay reselection for another intermediate relay/last relay UE e.g. when the PC5 RSRP between them is below a threshold.

  - Cross path topologies

The topologies were also discussed in the post-email [Post127][401] discussion[1]. Different cases are defined, taking Case-A and case-B as the example (as case-c and case-d have few supports):

(Figure 2.5A-1)

(Figure 2.5A-2)

(Figure 2.5A-3)

(Figure 2.5B-1)

(Figure 2.5B-2)

(Figure 2.5B-3)

According to WID[3], ‘The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities’

However, the discussions are somehow stuck because the companies have different understanding on the structure especially when different L2 ID(s) are used in those cases.

Observation 3: It is not clear that the ‘single indirect path’ limitation in WID is descripted in perspective of remote UE or intermediate relay UE.

Observation 4: It is not clear about whether it is possible /how different L2 ID(s) can support ‘a single indirect path’ for multi-hop relay.

It is clear that the L2 ID(s) identify the PC5-RRC connections, as follows[4]:

A PC5-RRC connection is a logical connection between two UEs for a pair of Source and Destination Layer-2 IDs which is considered to be established after a corresponding PC5 unicast link is established as specified in TS 23.287 [40]. There is one-to-one correspondence between the PC5-RRC connection and the PC5 unicast link. A UE may have multiple PC5-RRC connections with one or more UEs for different pairs of Source and Destination Layer-2 IDs.

So, from AS perspective, different L2 ID(s) means different PC5-RRC connection(s) and thus different link(s).

Observation 5: From AS perspective, different L2 ID(s) means different PC5-RRC connection(s) and thus different unicast link(s).

Therefore, in our understanding, it is doubtful how the Figure 2.5A-1, Figure 2.5A-2 and even Figure 2.5A-3 can be differentiated by the UE. Same question for Figure 2.5B-2 and Figure 2.5B-3. If we have conclusions on which cases we do not support, we need to discuss whether/how it can be avoided, or is it actually a valid case according to SA2 assumption.

Observation 6: For the cases defined for cross path topologies, it is not clear which one(s) are valid cases, and which one(s) can be avoided if we agree not to support them.

Therefore, we propose to first clarify and discuss about RAN2 understandings, before we agree to support/not support any cases.

Proposal 2: RAN2 to first reach consensus on the following issues before concluding on which cross path topologies are (not) supported:

- Issue 1: ‘single indirect path’ limitation in WID is descripted in perspective of remote UE or intermediate relay UE or both; If only from remote UE perspective, then Figure 2.5B-2 may be supported.

- Issue 2: Whether two pair of L2 ID(s) between intermediate relay UE and last relay UE (e.g. Figure 2.5A-1 and Figure 2.5B-2) lead to multiple indirect path and thus not aligned with WID;

- Issue 3: If only a single pair of L2 ID(s) is supported between intermediate relay UE and last relay UE (e.g. only Figure 2.5B-1 but not Figure 2.5B-2 is supported), whether it is possible and how to achieve it. LS to SA2 may be needed.

At the same time, to make some progress, we could try to reach some general agreement to kill some cases. E.g. all the case where there are two links between last relay UE and gNB, could be unsupported, because it will increase the complexity and they have acquired few supports during email discussion.

Proposal 3: RAN2 to agree that all cases where there are two links between last relay UE and gNB, is NOT supported (i.e. Figure 2.5A-3, Figure 2.5B-3, Figure 2.5C-2, Figure 2.5D-1, Figure 2.5D-2, Figure 2.5D-3).

- Conclusion

In this contribution, we discussed intermediate relay UE (re)selection and the issues which need to be clarified in topology discussion. We have the following Observations and Proposals:

Observation 1: Relay selection is performed by remote UE according to SA2 TR 23.700-03.

Observation 2: Making the intermediate relay UE capable of reselecting relay link by itself can avoid the impact on the good link between remote UE and this intermediate relay UE.

Observation 3: It is not clear that the ‘single indirect path’ limitation in WID is descripted in perspective of remote UE or intermediate relay UE.

Observation 4: It is not clear about whether it is possible /how different L2 ID(s) can support ‘a single indirect path’ for multi-hop relay.

Observation 5: From AS perspective, different L2 ID(s) means different PC5-RRC connection(s) and thus different unicast link(s).

Observation 6: For the cases defined for cross path topologies, it is not clear which one(s) are valid cases, and which one(s) can be avoided if we agree not to support them.

Proposal 1: One intermediate Relay UE can perform relay reselection for another intermediate relay/last relay UE e.g. when the PC5 RSRP between them is below a threshold.

Proposal 2: RAN2 to first reach consensus on the following issues before concluding on which cross path topologies are (not) supported:

- Issue 1: ‘single indirect path’ limitation in WID is descripted in perspective of remote UE or intermediate relay UE or both; If only from remote UE perspective, then Figure 2.5B-2 may be supported.

- Issue 2: Whether two pair of L2 ID(s) between intermediate relay UE and last relay UE (e.g. Figure 2.5A-1 and Figure 2.5B-2) lead to multiple indirect path and thus not aligned with WID;

- Issue 3: If only a single pair of L2 ID(s) is supported between intermediate relay UE and last relay UE (e.g. only Figure 2.5B-1 but not Figure 2.5B-2 is supported), whether it is possible and how to achieve it. LS to SA2 may be needed.

Proposal 3: RAN2 to agree that all cases where there are two links between last relay UE and gNB, is NOT supported (i.e. Figure 2.5A-3, Figure 2.5B-3, Figure 2.5C-2, Figure 2.5D-1, Figure 2.5D-2, Figure 2.5D-3).

- References

- R2-xxxxxx_Report of Post multi-hop relay discovery and (re)selection_P2_summary;

- TR 23.700-03, Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS); Phase 3 (Release 19), V19.0.0;

- RP-242349, Revised WID on NR sidelink multi-hop relay, LG Electronics, InterDigital, FirstNet, 3GPP TSG RAN Meeting #105;

- 3GPP TS 38.300 V18.3.0 (2024-09);