3GPP TSG-RAN WG2 Meeting #128		R2-2409633

Orlando, USA, 18th – 22nd November, 2024

Source:	vivo

Title:		Discussion on CP and SRAP impact for Approach 1

Agenda Item:		8.13.3

Document for:	Discussion and Decision

# Introduction

In the RAN2#127 meeting [1], the following agreements were reached on control plane procedures.

Agreements:

The following connections are assumed as a baseline to be needed:

-	From last Relay UE perspective:

A direct (non-relayed) PC5 connection with the first or an intermediate Relay UE, and

A direct (non-relayed) Uu connection with serving gNB, if in RRC_CONNECTED.

-	From intermediate relay UE perspective (including first relay UE):

A direct (non-relayed) PC5 connection with each of two adjacent (remote or relay) UEs, and

An end-to-end Uu connection with serving gNB, if in RRC_CONNECTED.

FFS what RRC states are supported for the intermediate relay UE.

-	From U2N Remote UE perspective:

A direct (non-relayed) PC5 connection with Intermediate Relay UE, and

An end-to-end Uu RRC connection with serving gNB, if in RRC_CONNECTED.

Agreements:

The multi-hop CP protocol stack is end-to-end for Uu-PDCP and above and hop-by-hop for SRAP and below (as in Rel-17/18).

The multi-hop UP protocol stack is end-to-end for Uu-PDCP and above and hop-by-hop for SRAP and below (as in Rel-17/18).

After the RAN2#127 meeting, an email was kicked off to further discuss different solutions on multi-hop relay control plane [2]. According to the email discussion, there are two main approaches for implementing multi-hop U2N relays (i.e., approach 1 and approach 2), which are described as follows:

- Approach 1: The network needs to directly control each of the intermediate relay UEs via Uu RRC.

- Approach 2: Only the last relay UE requires control by the network via Uu RRC.

In this contribution, we mainly focus on discussing more details on CP and SRAP impact for Approach 1, in order to support two additional hops in an extensible way.

# Discussion

## RRC connection establishment for U2N Remote UE

Figure 1. RRC connection establishment for U2N Remote UE (via two additional hops relays)

- The U2N Remote UE, First Relay UE, Intermediate Relay UE, and Last Relay UE perform discovery procedure, and establish a PC5-RRC connection between each adjacent UE (U2N Remote UE<->First Relay UE, First Relay UE <-> Intermediate Relay UE, Intermediate Relay UE <-> Last Relay UE) using the NR sidelink PC5 unicast link establishment procedure.  [FFS whether to support PC5-RRC connection establishment between some adjacent UEs after transmission of the first RRC message in step 2.]

- The L2 U2N Remote UE sends the first RRC message (i.e., RRCSetupRequest) for its connection establishment with gNB via the First Relay UE, using a specified PC5 Relay RLC channel configuration.  The first Relay UE sends the SidelinkUEInformationNR message to request for the dedicated configurations required to support the multi-hop relay operation for the U2N Remote UE. If the First Relay UE is not in RRC_CONNECTED, it needs to do its own Uu RRC connection establishment via the Intermediate Relay UE (using similar actions as a U2N Remote UE) upon reception of a message from U2N Remote UE on the specified PC5 Relay RLC channel.  The Intermediate Relay UE sends the SidelinkUEInformationNR message to request for the dedicated configurations required to support the multi-hop relay operation for the U2N Remote UE.   If the Intermediate Relay UE is not in RRC_CONNECTED, it needs to do its own Uu RRC connection establishment via the Last Relay UE (using similar actions as a U2N Remote UE) upon reception of a message from the First Relay UE on the specified PC5 Relay RLC channel.  The Last Relay UE sends the SidelinkUEInformationNR message to request for the dedicated configurations required to support the multi-hop relay operation for the U2N Remote UE. If the Last Relay UE is not in RRC_CONNECTED, it needs to do its own Uu RRC connection establishment upon reception of a message from the Intermediate Relay UE on the specified PC5 Relay RLC channel.  In each of the previous sub-steps, if a given relay UE and its parent relay UE both need to enter RRC_CONNECTED, the given relay UE cannot do so until the parent relay UE has completed its own RRC connection establishment. The Last Relay UE receives SRB0 relaying Uu Relay RLC channel configuration for the Intermediate Relay UE from gNB. The Intermediate Relay UE receives SRB0 relaying Relay RLC channel configuration for the First Relay UE from gNB. The gNB configures SRB0 (for U2N Remote UE) relaying RLC channel to the first Relay UE. The gNB responds with an RRCSetup message to U2N Remote UE. The RRCSetup message is sent to the U2N Remote UE using SRB0 relaying Last Relay RLC channel over Uu and the specified PC5 Relay RLC channels over each of the PC5 links. [FFS whether the Last Relay UE can send SUI on behalf of all other relay UEs.]

- The gNB, Last Relay UE, Intermediate Relay UE and First Relay UE perform relaying channel setup procedure over Uu. According to the configuration from the gNB, the First Relay/U2N Remote UE establishes a PC5 Relay RLC channel for relaying of SRB1 towards the U2N Remote UE/First Relay UE over PC5, the Intermediate Relay/First Relay UE establishes a PC5 Relay RLC channel for relaying of SRB1 towards the First Relay UE/Intermediate Relay UE over PC5 and the Last Relay UE/ Intermediate Relay UE establishes a PC5 Relay RLC channel for relaying of SRB1 towards the Intermediate Relay UE/Last Relay UE over PC5.  [FFS if each relay UE can establish RLC channel for relaying of SRB1 at the same time as its connection establishment in step 2].

- The RRCSetupComplete message is sent by the U2N Remote UE to the gNB via the First Relay UE, Intermediate Relay UE and the Last Relay UE using SRB1 relaying channels over PC5 and SRB1 relaying channel configured to the Last Relay UE over Uu. Then the U2N Remote UE is RRC_CONNECTED with the gNB.

- The L2 U2N Remote UE and gNB establish security following the Uu security mode procedure and the security messages are forwarded through the First Relay UE, Intermediate Relay UE, and Last Relay UE.

- The gNB sends an RRCReconfiguration message to the U2N Remote UE via the Last Relay UE, Intermediate Relay UE, and First Relay UE to setup the end-to-end SRB2/DRBs of the U2N Remote UE. The U2N Remote UE sends an RRCReconfigurationComplete message to the gNB via the First Relay UE, Intermediate Relay UE, and Last Relay UE as a response. In addition, the gNB may configure additional Uu Relay RLC channels between the gNB and Last Relay UE, and PC5 Relay RLC channels between each of the Intermediate Relay UE, First Relay UE, and U2N Remote UE for the relaying traffic.

In the email report [2], the figure and description above are proposed to serve as a baseline connection establishment procedure for multi-hop U2N Relays if Approach 1 (all relay UEs must be in RRC_CONNECTED when the remote UE is in RRC_CONNECTED) is adopted.

Observation 1	Based on the email report in [2], the figure and description above serve as a baseline connection establishment procedure for multi-hop U2N Relays in Approach 1.

Based on the above baseline procedures in above Figure 1, there are several detailed issues that need further in-depth analysis.

## Issue 1: How to guarantee same cell modeling in Approach 1

According to legacy SL U2N relay architecture, when the Remote UE enters RRC_CONNECTED mode, both the Remote UE and the Relay UE must have the same serving cell, i.e. the Relay UE’s serving cell. This rule must be kept in multi-hop relay approach 1 architecture, where the Remote UE, the Intermediate Relay UE(s) and the Last Relay UE should have the same serving cell in RRC_CONNECTED, i.e. the Last Relay UE’s serving cell.

Proposal 1	For Approach 1, the Remote UE, the Intermediate relay UE(s) and the Last Relay UE should have the same serving cell in RRC_CONNECTED, i.e. the Last Relay UE’s serving cell.

From multi-hop discovery stage, all relay discovery and selection behaviors should prepare for the final same cell modeling. For example, in below Figure 2, each potential Intermediate Relay UE in RRC_IDLE/OOC can forward discovery message for next relay UE. When all of Relay UEs in the multi-hop path are in RRC_IDLE/OOC, the Remote UE will trigger them enter into RRC_CONNECTED one by one. And the Last Relay UE establishes RRC connection with its serving cell and then other Intermediate Relay UE(s) and the Remote UE in-sequence establishes RRC connection with the same serving cell. The same cell modeling can be guaranteed. Otherwise, if one Intermediate Relay UE is already in RRC_CONNECTED, it may not need to forward discovery message and can directly response its own existing route information whose route is from this relay to the network. The situation will be that Remote UE and first intermediate Relay UE(s) are in RRC_IDLE/OOC and the rest Intermediate Relay UE(s) and the Last Relay UE have RRC connection with same serving cell. If this path is selected, the Remote UE and first IDLE/OOC Intermediate Relay UE(s) will in-sequence establish RRC connection with the same serving cell.

Figure 2. Example of guaranteeing same cell modeling for multi-hop relay

Proposal 2	For Approach 1, the RRC_CONNECTED Intermediate Relay UE will response/broadcast its existing routing for discovery procedure which guarantees the same serving cell modelling for multi-hop path.

## Issue 2: The time point when to establish PC5 RRC connection

In a discovery procedure for a multi-hop U2N path, there may be multiple potential paths. SA2 has agreed that it is Remote UE to decide the final path selection [3]. Before path selection, PC5 RRC connection establishment between adjacent UEs may be useless and wasteful. It is the first valid time point for PC5 RRC connection establishment after path selection is completed. Some companies propose to support PC5 RRC connection establishment between some adjacent UEs after transmission of the first Uu RRC message from the remote UE. However, it is not preferred because it will prolong the RRC establishment procedure.

Proposal 3	For Approach 1, PC5 RRC connection establishment between adjacent UEs should be performed after path selection and before the first Uu RRC message from the Remote UE to reduce the accumulative latency of a multi-hop path establishment as much as possible.

## Issue 3: Whether the Last Relay UE can send SUI on behalf of all other relay UEs

When all of UEs in multi-hop path need to establish RRC connection with the same serving cell, the detailed steps are illustrated in following Figure 3 (different color represents different UE’s procedure and information):

Figure 3. Example of different UE’s procedure and information with different color

According the above Figure 3, after the Last Relay UE4 enters RRC_CONNECTED mode, it should report SUI to NW and request for relaying configurations. In legacy R17 single-hop relay procedure, Relay UE only needs to report its Remote UE’s L2 ID and get Uu relay RLC channel configuration and mapping relationship for Remote UE’s SRB0/1 to prepare for remote UE’s Uu RRC connection establishment.

However, in a multi-hop relay architecture, there are multiple child UEs under the Last Relay UE4. If the last Relay UE4 can report the whole path information to NW and get relaying configuration for all of child UEs, e.g. local IDs for each child, Uu relay RLC channel configuration and mapping relationship for all of children and even PC5 relay RLC channel configuration and mapping relationship for all of grandson UEs between it and its son UE, it can obviously reduce the number of reconfigurations of the last relay UE and shorten the establishment latency.

Each Intermediate Relay UE and Remote UE cannot omit the SUI and request relaying configuration procedure since its own Layer 2 ID must be confirmed and associated with the Uu configuration. With similar way as the Last Relay UE, each Intermediate Relay UE can also report the whole path or the rest of child UEs via SUI and get the whole relay configuration for all of child UEs in upstream PC5 link and downstream PC5 link to fasten the establishment procedure.

Proposal 4	For Approach 1, the Last Relay UE and the Intermediate Relay UE(s) can report the whole path or all of its child UEs, and its own Layer-2 ID via SUI to NW and get the whole relaying configuration for all of child UEs in upstream PC5/Uu link and downstream PC5 link to fasten the RRC establishment procedure.

Proposal 5	For Approach 1, after completing the RRC establishment procedure, each UE should report SUI to NW by itself and get its relaying configuration via its own end-to-end Uu RRC connection.

## System information delivery for U2N Remote UE

In Rel-17, a L2 U2N Relay UE can perform unsolicited SIB1 forwarding to a U2N Remote UE connected with this L2 U2N Relay UE via PC5 link, when the SIB1 has not been sent to this U2N Remote UE before or an updated SIB1 has been received by this L2 U2N Relay UE. The similar function should be certainly supported for multiple-hop L2 U2N relay network as the U2N Remote UE served by L2 multiple-hop U2N Relay network also needs SIB1 for RRC connection setup with the gNB. In multiple-hop L2 U2N Relay network, unicast SIB1 propagation from the last U2N Relay UE to all its downstream UEs (Intermediate U2N Relay UE to U2N Remote UE) respectively should be supported, which comprises:

- The last U2N Relay UE performs unsolicited SIB1 forwarding to the Intermediate U2N Relay UE connected to this last U2N Relay UE via PC5 link;

- Any Intermediate U2N Relay UE performs unsolicited SIB1 forwarding to the U2N Remote UEs connected with this Intermediate U2N Relay UE via PC5 link.

Proposal 6	For Approach 1, a last Relay UE should support unsolicited forwarding of the acquired SIB1 to the Intermediate U2N Relay UEs connected to the last relay UE via PC5 link.

Proposal 7	For Approach 1, an Intermediate Relay UE should support unsolicited forwarding of the acquired SIB1 to the Remote UEs connected to this Intermediate relay UE via PC5 link.

Further, in Rel-17, a L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE, can request the desired SIBs from its parent U2N Relay UE. Naturally, this should also be supported for U2N Remote UE in RRC_IDLE or RRC_INACTIVE in Layer 2 multiple-hop U2N Relay network. When a U2N Remote UE is connected with an Intermediate U2N Relay UE via PC5 link, it can send SI request to its serving Intermediate U2N Relay UE, the serving Intermediate U2N Relay UE can send the requested SIBs to the U2N Remote UE, and the serving Intermediate U2N Relay UE forward the requested SIBs to the U2N remote UE if the requested SIBs are available. Otherwise, the serving Intermediate Relay UE should acquire the request SIBs first either in the similar way as a legacy U2N Remote UE or acquire the requested SIBs in its Uu interface, and forward the acquired SIBs to the U2N Remote UE afterwards.

Proposal 8	For Approach 1, upon reception of SI request from Remote UE, the Intermediate Relay UE can perform one of the following actions:

  - send the requested SIBs, if available, to Remote UE by itself.

  - acquire the requested SIBs, if unavailable, by further forwarding the SI request to the last Relay UE, and forward the requested SIBs to Remote UE accordingly afterwards.

## Paging delivery for U2N Remote UE

For Rel-17 L2 U2N relay, a U2N Remote UE can send its paging information to its Intermediate U2N relay UE to request its serving U2N relay to monitor the paging message for this U2N Remote UE on behalf, wherein the paging information comprises the UE ID information (ng-5G-S-TMSI, or, ng-5G-S-TMSI & fullI-RNTI) and the paging cycle configuration. The same function should also be supported for a U2N Remote UE served by Rel-19 multiple-hop U2N network. In Rel-19 L2 multiple-hop U2N relay network, at least the last U2N Relay UE should be able to monitor the paging message for all downstream UEs, including the downstream Intermediate U2N Relay UEs and the downstream U2N Remote UEs, based on the paging information from these Intermediate U2N Relay UE and U2N Remote UEs. If a U2N Remote UE connected with an Intermediate U2N Relay UE, it should transmit the paging information to the Intermediate U2N Relay UE and the Intermediate U2N Relay UE can either monitor the paging for the U2N Remote UE by itself e.g., if it is in-coverage and support basic paging function), or further forward the paging information of this U2N Remote UE to its last U2N Relay UE to request its last U2N Relay UE to monitor the paging message for this U2N Remote UE on-behalf. In R19 multiple-hop U2N network, in case of an Intermediate U2N Relay UE sending the paging information of a U2N Remote UE to its last U2N Relay UE to request its last U2N Relay UE to monitor the paging message on-behalf, it should forward the received paging message from the last U2N Relay UE to U2N Remote UE accordingly afterwards.

Proposal 9	For Approach 1, a last Relay UE should support monitoring the paging message for a downstream UE (Intermediate U2N Relay UE or U2N Remote UE) based on the paging information (e.g., ng-5G-S-TMSI, fullI-RNTI, paging cycle info) received from the downstream UE.

Proposal 10	For Approach 1, upon reception of the paging information from a Remote UE, a intermediate Relay UE can perform one of the following actions:

  - Monitor paging for the Remote UE, and forward the paging message to the Remote UE by itself, e.g., if it is capable of paging and in-coverage.

  - Further forward the paging information to the last Relay UE, and forward the received paging message from the last Relay UE to Remote UE accordingly afterwards.

A U2N Remote UE in RRC_IDLE/RRC_INACTIVE connected with an Intermediate Relay UE via PC5 link may leave the L2 multiple-hop Relay network due to mobility. In this case, the Intermediate U2N Relay UE should notify the corresponding last U2N Relay UE which monitors the paging information for this U2N Remote UE on-behalf to release the paging information of this U2N Remote UE. This can help the last U2N Relay UE to stop paging message monitoring for this U2N Remote UE, which is beneficial for power saving of the last U2N Relay UE.

Proposal 11	For Approach 1, the last Relay UE should be notified when a Remote UE left so that the last U2N Relay UE can stop paging message monitoring on behalf of the Remote UE.

## TAU/RNAU for U2N Remote UE

In Rel-17, the L2 U2N Remote UE performs TAU/RNAU based on the L2 U2N Relay UE's serving cell when it is connected with the L2 U2N Relay UE. A L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE initiates TAU/RNAU procedure if the serving cell changes (due to cell change by the L2 U2N Relay UE) and the new serving cell is outside of the L2 U2N Remote UE's configured TA/RNA. According to our Proposal 1 above, we assume that the U2N Remote UE and all intermediate Relay UEs should following the last Relay UE's serving cell, and thus a similar principle can be applied to multi-hop Layer-2 UE-to-Network Relay operation for minimum the specification efforts. Therefore,

Proposal 12	For Approach 1, the Remote UE follows the last Relay UE's serving cell and thus performs legacy Rel-17 TAU/RNAU procedure as baseline.

## SRAP impact

Based on the above baseline procedures in above Figure 1, some issues are further analyzed from Uu radio bearer configuration and bearer mapping perspective.

## Issue 1: FFS whether each Relay UE except the Last Relay UE can establish SRB only (w/o DRB)

If a Relay UE is just performing the relaying role for at least one remote UE but doesn’t have its own service, it is natural that the DRB configuration for this UE in not necessary. Moreover, the current RRC specification has already supported that for IAB only SRB1 and SRB2 are setup (w/o DRB), see highlighted blue text as shown in below Figure 4. IAB is also used for relaying purpose, we believe that it is feasible to extend a similar principle to multi-hop relay scenario. In other words, a relay UE except the Last Relay UE can setup SRB only (w/o DRB) during its RRC reconfiguration procedure only if it is acting as the relay UE for at least one remote UE but has no its own service.

Figure 4. Pre-condition for SRB and DRB setup in current RRC specification

Proposal 13	For Approach 1, RAN2 agree that each Relay UE except the Last Relay UE can setup SRB1 and SRB2 only (i.e., w/o DRB) during its RRC reconfiguration procedure if it is acting as the Relay UE for at least one Remote UE but has no its own service.

## Issue 2: SRAP format

Legacy U2N SRAP header only comprises Bearer ID and the local UE ID of the U2N Remote UE, regardless of the SRAP PDU is transmitted from the U2N Remote UE to the gNB or from the gNB to the U2N Remote UE, i.e. any U2N Relay UE cannot identify a received SRAP PDU is for upstream forwarding or downstream forwarding solely based on the SRAP header. If the legacy SRAP header is reused, as for R18 U2N relay, any U2N relay UE relies on the ingress link to determine if the received SRAP PDU is for upstream forwarding or downstream forwarding, i.e.:

- If the SRAP PDU is received in the child link (from Remote UE or child Intermediate U2N Relay UE), the SRAP PDU is for upstream forwarding;

- If the SRAP PDU is received in the parent link (to gNB or parent U2N Relay UE), the SRAP PDU is for downstream forwarding;

Observation 2	If legacy L2 U2N SRAP header is reused, any U2N Relay UE relies on the ingress link in which an SRAP PDU is received to determine the SRAP PDU is for upstream or downstream forwarding.

Proposal 14	For Approach 1, RAN2 assumes that legacy U2N SRAP format is reused as baseline.

## Issue 3: Uu radio bearer mapping configuration

Following the legacy U2N SRAP format and handling, the below Figure 5 shows how an end-to-end Uu radio bearer is mapped to/from the serving gNB from each UE perspective (different color represents different UE’s Uu radio bearer).

Figure 5. Example of different UE’s Uu radio bearer mapping with different color

In accordance with the above illustration, it’s straightforward to have a bulk of proposals for each UE’s Uu radio bearer mapping configuration as follows:

Proposal 15	For Approach 1, from a Remote UE perspective, the dedicated Uu radio bearer mapping configuration at least includes the following information:

  - Local identity of the Remote UE

  - The Remote UE’s Uu SRB/DRB identity to PC5 RLC channel mapping

Proposal 16	For Approach 1, from First Relay UE perspective, the dedicated Uu radio bearer mapping configuration at least includes the following information:

- For relaying at least one Remote UE’s signaling and data

  - Local identity of each Remote UE

  - Each Remote UE’s Uu SRB/DRB identity to child/parent PC5 RLC channel mapping

- For its own signaling

  - Local identity of this Relay UE

  - This Relay UE’s SRB identity to parent PC5 RLC channel mapping

Proposal 17	For Approach 1, from Intermediate Relay UE perspective, the dedicated Uu radio bearer mapping configuration at least includes the following information:

- For relaying at least one Remote UE’s signaling and data

  - Local identity of each Remote UE

  - Each Remote UE’s Uu SRB/DRB identity to child/parent PC5 RLC channel mapping

- For its own signaling

  - Local identity of this Relay UE

  - This Relay UE’s SRB identity to parent PC5 RLC channel mapping

- For relaying its downstream intermediate Relay UE’s signaling

  - Local identity of each downstream intermediate Relay UE

  - Each downstream intermediate Relay UE’s SRB identity to child/parent PC5 RLC channel mapping

Proposal 18	For Approach 1, from Last Relay UE perspective, the dedicated Uu radio bearer mapping configuration at least includes the following information:

- For relaying at least one Remote UE’s signaling and data

  - Local identity of each Remote UE

  - Each Remote UE’s Uu SRB/DRB identity to PC5/Uu RLC channel mapping

- For relaying all intermediate Relay UEs’ signaling

  - Local identity of each intermediate Relay UE

  - Each intermediate Relay UE’s SRB identity to PC5/Uu RLC channel mapping

## Issue 4: SRAP rerouting

Local rerouting is supported for IAB network to ensure robust data transmission. For multiple-hop U2N relay, the backhaul route is rather instable compared to multiple-hop IAB network as IAB node is supposed to be fixedly deployed while the U2N Relay UE(s) could be probably mobile. To ensure the robust data transmission, flexible data rerouting could be considered for SRAP design of multiple-hop U2N Relay, in order to conquer the backhaul route broken, see below Figure 6 as an example for rerouting. One simple improvement could be to introduce UL/DL indication in the SRAP header by reusing the R bit in R18 SRAP header. Based on the UL/DL indication in the SRAP header, an

y U2N relay which received an SRAP PDU with unknown local UE ID can determine upstream/downstream direction to forward the SRAP PDU instead of simply dropping the SRAP PDU.

Figure 6. Example of Rerouting

As multiple-hop U2N Relay could be very instable due to the mobility of U2N relay UEs, such rerouting scheme could be more important than IAB network. To facilitate the data rerouting in multiple-hop U2N Relay network, we propose:

Proposal 19	RAN2 to discuss whether SRAP rerouting needs to be supported for multiple-hop U2N Relay.

## Backward compatibility issue

L2 multiple-hop U2N relay can extend the network coverage even more than the R17/18 L2 U2N Relay network. By the time of L2 multiple-hop U2N relay deployment, legacy U2N Remote UE may have already considerable penetration in the market. Similar to the case that an advanced NR gNB is usually required to serve legacy NR UEs in the market by the operators, an L2 multiple-hop U2N relay network may also be probably required to serve legacy U2N Remote UEs so that the legacy U2N Remote UEs (see Figure 7 below) can also benefit from the coverage extension by the L2 multiple-hop U2N relay network.

Figure 7. Topology to serve legacy L2 U2N remote UE in L2 U2N multi-hop relay network

Proposal 20	RAN2 to discuss whether backward-compatibility of multi-hop L2 U2N relay network is supported, i.e. whether the multi-hop L2 U2N relay network should also be able to serve R17/R18 L2 U2N Remote UEs.

# Conclusion

This contribution mainly discussed the potential CP and SRAP impact for Approach 1. The contribution concludes with:

Observation 1	Based on the email report in [2], the figure and description above serve as a baseline connection establishment procedure for multi-hop U2N Relays in Approach 1.

Observation 2	If legacy L2 U2N SRAP header is reused, any U2N Relay UE relies on the ingress link in which an SRAP PDU is received to determine the SRAP PDU is for upstream or downstream forwarding.

RRC connection establishment

Proposal 1	For Approach 1, the Remote UE, the Intermediate relay UE(s) and the Last Relay UE should have the same serving cell in RRC_CONNECTED, i.e. the Last Relay UE’s serving cell.

Proposal 2	For Approach 1, the RRC_CONNECTED Intermediate Relay UE will response/broadcast its existing routing for discovery procedure which guarantees the same serving cell modelling for multi-hop path.

Proposal 3	For Approach 1, PC5 RRC connection establishment between adjacent UEs should be performed after path selection and before the first Uu RRC message from the Remote UE to reduce the accumulative latency of a multi-hop path establishment as much as possible.

Proposal 4	For Approach 1, the Last Relay UE and the Intermediate Relay UE(s) can report the whole path or all of its child UEs, and its own Layer-2 ID via SUI to NW and get the whole relaying configuration for all of child UEs in upstream PC5/Uu link and downstream PC5 link to fasten the RRC establishment procedure.

Proposal 5	For Approach 1, after completing the RRC establishment procedure, each UE should report SUI to NW by itself and get its relaying configuration via its own end-to-end Uu RRC connection.

System information delivery

Proposal 6	For Approach 1, a last Relay UE should support unsolicited forwarding of the acquired SIB1 to the Intermediate U2N Relay UEs connected to the last relay UE via PC5 link.

Proposal 7	For Approach 1, an Intermediate Relay UE should support unsolicited forwarding of the acquired SIB1 to the Remote UEs connected to this Intermediate relay UE via PC5 link.

Proposal 8	For Approach 1, upon reception of SI request from Remote UE, the Intermediate Relay UE can perform one of the following actions:

  - send the requested SIBs, if available, to U2N Remote UE by itself.

  - acquire the requested SIBs, if unavailable, by further forwarding the SI request to the last Relay UE, and forward the requested SIBs to Remote UE accordingly afterwards.

Paging delivery

Proposal 9	For Approach 1, a last Relay UE should support monitoring the paging message for a downstream UE (Intermediate U2N Relay UE or U2N Remote UE) based on the paging information (e.g., ng-5G-S-TMSI, fullI-RNTI, paging cycle info) received from the downstream UE.

Proposal 10	For Approach 1, upon reception of the paging information from a U2N Remote UE, a U2N intermediate Relay UE can perform one of the following actions:

  - Monitor paging for the U2N Remote UE, and forward the paging message to the U2N Remote UE by itself, e.g., if it is capable of paging and in-coverage.

  - Further forward the paging information to the last U2N Relay UE, and forward the received paging message from the last U2N Relay UE to U2N Remote UE accordingly afterwards.

Proposal 11	For Approach 1, the last U2N Relay UE should be notified when a U2N Remote UE left so that the last U2N Relay UE can stop paging message monitoring on behalf of the U2N Remote UE.

TAU/RNAU

Proposal 12	For Approach 1, the U2N Remote UE follows the last Relay UE's serving cell and thus performs legacy Rel-17 TAU/RNAU procedure as baseline.

SRAP impact

Proposal 13	For Approach 1, RAN2 agree that each Relay UE except the Last Relay UE can setup SRB1 and SRB2 only (i.e., w/o DRB) during its RRC reconfiguration procedure if it is acting as the Relay UE for at least one Remote UE but has no its own service.

Proposal 14	For Approach 1, RAN2 assumes that legacy U2N SRAP format is reused as baseline.

Proposal 15	For Approach 1, from a Remote UE perspective, the dedicated Uu radio bearer mapping configuration at least includes the following information:

  - Local identity of the Remote UE

  - The Remote UE’s Uu SRB/DRB identity to PC5 RLC channel mapping

Proposal 16	For Approach 1, from First Relay UE perspective, the dedicated Uu radio bearer mapping configuration at least includes the following information:

- For relaying at least one Remote UE’s signaling and data

  - Local identity of each Remote UE

  - Each Remote UE’s Uu SRB/DRB identity to child/parent PC5 RLC channel mapping

- For its own signaling

  - Local identity of this Relay UE

  - This Relay UE’s SRB identity to parent PC5 RLC channel mapping

Proposal 17	For Approach 1, from Intermediate Relay UE perspective, the dedicated Uu radio bearer mapping configuration at least includes the following information:

- For relaying at least one Remote UE’s signaling and data

  - Local identity of each Remote UE

  - Each Remote UE’s Uu SRB/DRB identity to child/parent PC5 RLC channel mapping

- For its own signaling

  - Local identity of this Relay UE

  - This Relay UE’s SRB identity to parent PC5 RLC channel mapping

- For relaying its downstream intermediate Relay UE’s signaling

  - Local identity of each downstream intermediate Relay UE

  - Each downstream intermediate Relay UE’s SRB identity to child/parent PC5 RLC channel mapping

Proposal 18	For Approach 1, from Last Relay UE perspective, the dedicated Uu radio bearer mapping configuration at least includes the following information:

- For relaying at least one Remote UE’s signaling and data

  - Local identity of each Remote UE

  - Each Remote UE’s Uu SRB/DRB identity to PC5/Uu RLC channel mapping

- For relaying all intermediate Relay UEs’ singnalling

  - Local identity of each intermediate Relay UE

  - Each intermediate Relay UE’s SRB identity to PC5/Uu RLC channel mapping

Proposal 19	RAN2 to discuss whether SRAP rerouting needs to be supported for multiple-hop U2N Relay.

Backward compatibility issue

Proposal 20	RAN2 to discuss whether backward-compatibility of multi-hop L2 U2N relay network is supported, i.e. whether the multi-hop L2 U2N relay network should also be able to serve R17/R18 L2 U2N Remote UEs.

# References

- RAN2#127 Meeting Chairman Notes.

- R2-2410006, Report of [Post127][402][Relay] Multi-hop relay control plane, InterDigital.

- TR 23.700-03, Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS); Phase 3 (Release 19).