3GPP TSG-RAN WG2 Meeting #128	R2-2409635

Orlando, USA, 18 - 22 Nov, 2024

Agenda item:	8.10.2

Source:	Samsung

Title:	MRO enhancements for Rel-18 mobility features

Document for:	Discussion and Decision

# Introduction

As per [1], one of the objectives of R19 SON WI is to support the MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs and subsequent CPAC. From RAN2, this involves identification and specification of necessary UE reporting to enhance the mobility parameter tuning as given below.

The objective of this work item is to specify data collection enhancement in NR standalone and MR-DC for SON/MDT purpose. The specific objectives of this work item are: - MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]: - Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3] - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

# Discussion

## MRO for LTM

In RAN2#126 [2], the following agreement is made for RLF.

- =>If available, log the L1 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure.

Further in RAN2#127-bis [3], the following agreement is made for SHR.

=>UE logs available L1 measurement results for the serving cell, the target cell and other LTM candidate cells when a successful LTM cell switch triggers SHR.

In NR-DC, there may be two set of LTM candidate cells, one associated with MCG and one associated with SCG. The L1 measurements associated with SCG are not much helpful in RLF and SHR. We also note that there is no information available with MN on whether an LTM cell is configured by SN, as there is no inter-node message defined which shares the LTM configuration from SN to MN. Considering that the measurement reporting is the main overhead for the UE (and the network) in the SON/MDT reports which are stored, it is important to log only the necessary information.

Proposal 1-1: Only the L1 measurement results of the LTM candidate cells associated to MCG are logged in RLF report or SHR.

RAN2 also agreed to provide an explicit indication in RLF report to indicate whether neighbour cell is an LTM candidate cell (unless RAN3 defines a network based solution).

- Unless RAN3 defines a NW-based solution: Introduce an explicit indication in RLF-Report to indicate whether a neighbour cell is an LTM candidate cell.

Our understanding is that this is due to the fact that the RLF report may be kept by the UE for 48 hours and network may have cleared the relevant information by then. This is equally applicable for SHR also.

Proposal 1-2: UE logs an explicit indication whether neighbour cell is an LTM candidate cell in SHR (same as in RLF report).

For both the cases, the explicit indication can be included along with logging of L3 measurements (similar to existing choCandidate flag) in measResultNR. It needs to be logged only if the LTM candidate cell is associated with the MCG.

Proposal 1-3: The explicit indication whether neighbour cell is an LTM candidate cell is included in RLF report or SHR only if the LTM candidate cell is associated with the MCG.

Random Access Information for LTM

We think that it is useful for logging the RA purpose for early synchronisation and LTM cell switch separately.

Proposal 1-4: A new RA purpose is used for LTM cell switch and early synchronisation for TA.

We understand that the random access optimisation for the early TA needs to be done by both the source cell and candidate cell. For e.g. the thresholds at which the early TA is triggered is decided by source cell, while the RA parameters are decided by the candidate cell. The configuration of thresholds will make an impact on whether early TA is successful or not. So UE may include both source cell and candidate cell identifier in the RA report.

Proposal 1-5: UE includes the source and candidate cell identifiers in the RA information for early synchronisation for TA.

It is also important to know whether the random access for early TA is successful and how many attempts are made for early TA.

Proposal 1-6: If the UE has received TA value in LTM cell switch command, UE reports the TA is successful in the RA information.

Finally, it is important to log the RA report in all the cases-RA report, RLF report and SHR.

Proposal 1-7: RA information for early TA is included in RA report, RLF report and SHR.

## MRO for CHO with Candidate SCGs

In RAN2#127 [4], RAN2 agreed to the following regarding CHO with candidate SCG(s).

UE includes following information in RLF report:

- b.	Time information regarding condition fulfilment for CHO with candidate SCGs. Details are FFS. We consider both the case when both CHO condition and associated CPC condition are fulfilled, and the case when CHO (or CPC) is fulfilled but CPC (or CHO) conditions are not fulfilled.

- c.	Measurement results of PCells and PSCells.

In RAN2#127-bis, RAN2 also agreed to the following.

- UE reports the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO), and the first met execution condition (as agreed by RAN3), for a failed CHO with candidate SCGs. Details FFS. - Include the elapsed time between the point in time of the first fulfilled condition and RLF in RLF report. Details FFS.

As it is already agreed in to include the measurement results of PCells and PSCells and the MeasResultNR contains triggered event information (the first triggered event associated to a candidate cell and the time between events) will be available at the network. So the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO) can be the time gap between the first triggered event for PCell and the first triggered event for PSCell (when at least one PCell condition is fulfilled before all conditions of PSCell) or the time gap between the first triggered event of the PSCell and the first triggered event for PCell (when at least one PSCell condition is fulfilled before all conditions of PCell).

Proposal 2-1: UE reports the time gap between the first met condition and the second met condition as follows:

- When at least one PCell condition is fulfilled before all the conditions for PSCell, UE logs the time gap between the first triggered event for PCell and the first triggered event for PSCell.

- When at least one PSCell condition is fulfilled before all the conditions for PCell, UE logs the time gap between the first triggered event for PSCell and the first triggered event for PCell.

It is possible that there are multiple PCells with fulfilled conditions. In the case of RLF, it may be required to identify the elapsed time between the fulfilled condition and RLF for each of the PCells.

Proposal 2-2:	Elapsed time between the first fulfilled condition and RLF is included in RLF report for all the PCells.

The measurement information and time information will be useful for near failure cases of handover. Similarly, upon SCG failure, the UE stops even the evaluation of CHO when the CHO is associated with candidate SCG(s). So these information can be included in SHR and SCGFailureInformation.

Proposal 2-3:	Measurement results of PCells and PSCells and the time information (as agreed for RLF) are included in SHR and SCGFailureInformation also.

# Conclusion

Based on the discussion, we have the following proposals and observations.

MRO for LTM

Proposal 1-1: Only the L1 measurement results of the LTM candidate cells associated to MCG are logged in RLF report or SHR.

Proposal 1-2: UE logs an explicit indication whether neighbour cell is an LTM candidate cell in SHR (same as in RLF report).

Proposal 1-3: The explicit indication whether neighbour cell is an LTM candidate cell is included in RLF report or SHR only if the LTM candidate cell is associated with the MCG.

Proposal 1-4: A new RA purpose is used for LTM cell switch and early synchronisation for TA.

Proposal 1-5: UE includes the source and candidate cell identifiers in the RA information for early synchronisation for TA.

Proposal 1-6: If the UE has received TA value in LTM cell switch command, UE reports the TA is successful in the RA information.

Proposal 1-7: RA information for early TA is included in RA report, RLF report and SHR.

MRO for CHO with SCGs

Proposal 2-1: UE reports the time gap between the first met condition and the second met condition as follows:

- When at least one PCell condition is fulfilled before all the conditions for PSCell, UE logs the time gap between the first triggered event for PCell and the first triggered event for PSCell.

- When at least one PSCell condition is fulfilled before all the conditions for PCell, UE logs the time gap between the first triggered event for PSCell and the first triggered event for PCell.

Proposal 2-2:	Elapsed time between the first fulfilled condition and RLF is included in RLF report for all the PCells.

Proposal 2-3:	Measurement results of PCells and PSCells and the time information (as agreed for RLF) are included in SHR and SCGFailureInformation also.

# References

[1] RP-234038: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4

[2] R2-2406202: RAN2#126 Meeting Report

[3] RAN2#127-bis Chair notes

[4] R2-2409210: RAN2#127 Meeting Report