3GPP TSG-RAN WG2 Meeting #128	R2-2409636

# Orlando, Florida, USA, 18 – 22 November 2024

Agenda item:		8.7.5

Source:	Xiaomi

Title:	RLC AM retransmission enhancements

Document for:		Discussion and Decision

Introduction

In RAN2#127bis meeting, following was agreed regarding RLC retransmission enhancements [1]:

Agreements on RLC timely retransmissions - RAN2 confirm that existing mechanisms are insufficient to resolve the timely RLC retransmission problem and RLC enhancements for timely RLC retransmission are investigated in Rel-19. - Exclude enhanced status reporting. - Focus the discussion on autonomous retransmission and polling enhancements, e.g. we need to understand how each option affects the capacity and packet delay Agreements on avoiding unnecessary retransmissions - RAN2 confirm the previous baseline assumption: the RLC receiving window always advances to any given RLC SN before the transmitting window does. - RAN2 will adopt a “combined” approach for avoiding unnecessary RLC retransmissions, i.e. - TX side stops transmissions of an outdated SDU - RX side abandons the SDU based on a local timer - Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified - FFS if some C-PDU handling is needed to avoid C-PDU discard - FFS if some indication is sent from Tx to Rx. The assumption is this is not a full status report, but something simple (if needed)

In this contribution, we discus RLC AM retransmission enhancements.

Discussion

Avoid unnecessary RLC retransmissions

RAN2 agreed to adopt a “combined” approach for avoiding unnecessary RLC retransmissions. There are several aspects for further discussion.

Tx side stops transmissions of an outdated SDU

Currently RLC SDU discard procedure is specified in TS 38.322, clause 5.4, as below:

When indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity or the transmitting UM RLC entity shall discard the indicated RLC SDU, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity shall not introduce an RLC SN gap when discarding an RLC SDU.

For “combined” approach, when indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity shall discard the indicated RLC SDU, regardless of whether the RLC SDU or a segment has been submitted to the lower layers or not. When discarding an RLC SDU, an RLC SN gap can be introduced. Since Rx side uses status report to inform Tx side, there is no issue of discarding the RLC SDU which has already been submitted to MAC, nor introducing an RLC SN gap.

Proposal 1: Tx side discards the RLC SDUs indicated by PDCP layer for discarding, regardless of whether the RLC SDU or a segment has been submitted to the lower layers or not. An RLC SN gap can be allowed during the discard procedure.

Rx side abandons the SDU based on a local timer

As agreed in RAN2#127 meeting, “In the RX-initiated approach for avoiding unnecessary retransmissions, RLC receiver abandons missing SDUs like already done by PDCP, i.e. based on a timer.”

A timer similar to t-Reordering in PDCP can be introduced in RLC Rx side to move the lower edge of the receiving window (i.e. RLC receive state variable RX_Next). The timer is started when there is a gap among received AMD PDUs, and stopped when such gap does not exist anymore. Upon timer expiry, RX_Next is moved, and status report is triggered.

Proposal 2: A timer is introduced in RLC Rx side. The timer is started when there is a gap among received AMD PDUs, and stopped when such gap does not exist. Upon timer expiry, RX_Next is advanced.

Rx informs Tx side about the abandoned SDUs

As agreed in RAN2#127bis meeting, “as a baseline we assume existing SR can be reused unless issues are identified”. Our understanding is that legacy RLC status report is sufficient. As discussed above, expiry of local timer triggers the update of receive state variable RX_Next (i.e. movement of lower edge of the receiving window). Status report is used to inform the transmitter about the update of RLC receiving window, so that Tx side can advance the transmitting window accordingly. This is aligned with RAN2 assumption that “the RLC receiving window always advances to any given RLC SN before the transmitting window does”.

There was proposal to have explicit RLC indication of RLC SDUs abandoned by the receiver. However, with proper timer configuration, RLC SDUs abandoned by the receiver have already been discarded by the transmitter anyway, so there is no need to explicitly indicate those abandoned SDUs to the transmitter.

Proposal 3: Upon expiry of local timer, legacy status report is triggered. There is no need to introduce explicit RLC indication of RLC SDUs abandoned by the receiver.

FFS

There is one FFS “if some C-PDU handling is needed to avoid C-PDU discard”. This refers to the potential risk of discarding PDCP control PDUs. The timer mechanism introduced for RLC is similar to t-Reordering timer at PDCP. The movement of RLC receiving window will pass / discard those RLC PDUs with sufficiently long transmitting time (related to timer configuration), which means that those abandoned RLC SDUs (including PDCP control PDUs) have sufficient number of transmissions already. With proper setting of the timer by the network, there is no issue of PDCP control PDU handling.

Proposal 4: RAN2 confirms that special handling of PDCP control PDU is not needed.

Another FFS is “if some indication is sent from Tx to Rx. The assumption is this is not a full status report, but something simple (if needed)”. With above discussion of “combined” approach, it is clear that “combined” approach can already achieve the goal of avoiding unnecessary RLC retransmissions with minimal complexity increase. The “combined” approach can also satisfy the requirement that “the RLC receiving window always advances to any given RLC SN before the transmitting window does.” It is not clear what additional benefit can be brought by the indication from Tx to Rx. On the other hand, there is significant standardization efforts and complexity to support the indication from Tx to Rx. From the experience of Rel-18 PDCP SN gap report, following needs to be defined to support such indication:

- Format of control PDU for the notification

- In the Tx side, the triggering condition to send the control PDU

- In the Rx side, update of state variables / timers based on the reception of control PDU

Considering that the benefits are not clear but there are obvious drawbacks, we propose not to support the indication from Tx to Rx.

Proposal 5: The indication from Tx to Rx about the discarded RLC PDUs are not supported.

Timely RLC retransmission

Enhancements to polling

Regarding polling enhancements, it should be noted that the purpose of polling is to inform the peer to transmit status report. In case of polling sent by gNB, polling enhancement can be handled by gNB implementation. In case of polling sent by the UE, the goal is to get status report from gNB in a fast manner. However, gNB should be aware of the PDB requirements of XR traffic, and can send RLC status report in time by implementation. Therefore, evaluation is needed on the benefits of polling enhancements over gNB implementation on the timely handling of polling and RLC status report.

Proposal 6: Polling enhancement is excluded since gNB implementation can handle timely polling and RLC status report taking into account the PDB requirements of XR traffic.

Autonomous retransmission

In current RLC AM, retransmissions are prioritized over initial transmissions, as from TS 38.322 clause 5.2.3.1.1, as copied below:

The transmitting side of an AM RLC entity shall prioritize transmission of AMD PDUs containing previously transmitted RLC SDUs or RLC SDU segments over transmission of AMD PDUs containing not previously transmitted RLC SDUs or RLC SDU segments.

Autonomous retransmissions are based on conditions like timer / counter, instead of actual RLC status report. Therefore prioritizing autonomous retransmission over initial transmission might impact system performance (e.g. capacity, packet latency). It is suggested to differentiate autonomous retransmission when considering prioritization in RLC transmitter side.

Proposal 7: Autonomous retransmission should be differentiated from normal RLC retransmission base on RLC status report when considering prioritization in RLC transmitter side.

Conclusion

In this contribution, we discuss RLC AM retransmission enhancements, and propose the following:

Proposal 1: Tx side discards the RLC SDUs indicated by PDCP layer for discarding, regardless of whether the RLC SDU or a segment has been submitted to the lower layers or not. An RLC SN gap can be allowed during the discard procedure.

Proposal 2: A timer is introduced in RLC Rx side. The timer is started when there is a gap among received AMD PDUs, and stopped when such gap does not exist. Upon timer expiry, RX_Next is advanced.

Proposal 3: Upon expiry of local timer, legacy status report is triggered. There is no need to introduce explicit RLC indication of RLC SDUs abandoned by the receiver.

Proposal 4: RAN2 confirms that special handling of PDCP control PDU is not needed.

Proposal 5: The indication from Tx to Rx about the discarded RLC PDUs are not supported.

Proposal 6: Polling enhancement is excluded since gNB implementation can handle timely polling and RLC status report taking into account the PDB requirements of XR traffic.

Proposal 7: Autonomous retransmission should be differentiated from normal RLC retransmission base on RLC status report when considering prioritization in RLC transmitter side.

- References

[1] R2-2409215, Session chair (Huawei), “Report from session on R18 MBS, R18 QoE and R19 XR”