3GPP TSG-RAN WG2 Meeting #128	R2-2409651

Orlando, USA, Nov 18th – 22nd , 2024

Source:	CATT, Turkcell

Title:	Simulation results of RRM Measurement Prediction

Agenda Item:	8.3.2.1

Document for:	Discussion and Decision

Introduction

In RAN2#126 meeting, 3 scenarios have been agreed as high priority [1] for RRM measurement prediction:

Table 1: Prioritization of evaluation scenarios

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

And the following 3 cell level sub cases were agreed in RAN2#125bis [2]:

1 For cell level measurement prediction model, at least consider the following cases: Case 1: To predict beam level results, then generate cell level results based on the predicted beam results; Case 2: To directly predict cell level results based on cell level results. Case 3: To directly predict cell level results based on beam level results

In this document, we focus on the scenario 2 and scenario 4, and the simulation results of cell level results prediction based on the cell level results (sub case 2) are provided.

Discussion

Before the discussion on the simulation results, some clarification on the definition of Average RSRP Difference in Case 2/4 simulation is provided. In the simulation, one universal AI model addressing all UEs/cells is used (cluster based approach), and the cell level RSRP (per UE per cell at each sample time) is used as AI inputs to predict the cell level RSRP (per UE per cell in the future time sample). Then, the predicated RSRP RSRP_Predicated and RSRP obtained from system simulation RSRP_Simulated are compared to derive the RSRP Difference. Finally, Average RSRP difference is calculated as the average absolute RSRP Difference.

2.1	Case 2: FR1 to FR1 intra-frequency temporal domain case B

Temporal domain case B can be used to predict the measurement result(s) in the future time instance by historical measurement(s). Based on the post #127 email discussion [3], two examples are provided as shown below. In our simulation, we use Example 2 for Case B.

Figure 5.2.1-2 Example 1 of intra-frequency temporal domain case B Figure 5.2.1-3 Example 2 of intra-frequency temporal domain case B

Figure 1: 2 Examples of intra-frequency temporal domain case B

For Case B, the input are a series of measurement results (with a given measurement periodicity), whereas the output is the prediction of RSRP values at a point of time which does not and will not coincide with any measurement. Regarding the AI model, ResNet is one residual learning framework to ease the training of the networks which can be used to reformulate the layers. For temporal domain case B, ResNet shows its good performance on prediction with different window lengths of predication window and observation window. Hence, we use ResNet for FR1 to FR1 intra-frequency temporal domain case B.

In the Post email [3], it was agreed that both sliding L1/L3 filtering and non-sliding L1/L3 filtering options can be used for evaluation. In Case 2, non-sliding L1/L3 filtering is used for simulation.

In this section, we compare the impact of MRRT on RSRP difference. The simulation results are shown in Table 2. And the RSRP difference trend and the accuracy of the predicated best cell are shown in Figure 2.

Table 2: Simulation report table for FR1 to FR1 intra-frequency temporal domain case B considering different OW vs. PW

| Report parameters |  | CATT |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |  |  |  |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 2 |  |  |  |
|  | UE speed (30,60,90,120 Km/h) | 60Km/h |  |  |  |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | N/A |  |  |  |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | N/A |  |  |  |
|  | Whether LOSsoft is modeled or not | Yes |  |  |  |
|  | spatial consistency option (A or B) | Procedure A |  |  |  |
|  | Number of TX beams | 4 |  |  |  |
|  | Number of RX beams | 1 |  |  |  |
|  | Measurement reduction rate(50%,…Note2) | 50% | 66.7% | 75% | 80% |
|  | Observation window(Note3) | N/A |  |  |  |
|  | Prediction window (Note3) | N/A |  |  |  |
|  | Any other parameters (Note 4) | N/A |  |  |  |
| Data Size (Number of Samples) | Training/validity | 13000/ 1625 |  |  |  |
|  | Testing | 1625 |  |  |  |
| AI/ML model input/output | Model input (Note 5) | Cell level RSRP of all cells |  |  |  |
|  | Model output(Note 6) | Cell level RSRP of all cells |  |  |  |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | ResNet |  |  |  |
|  | Model complexity in a number of parameters(M) | 0.024M |  |  |  |
|  | Model complexity in model size (e.g. Mbyte) | 0.093Mbytes |  |  |  |
|  | Computational complexity [FLOPs] | 292.66K |  |  |  |
| Metrics | MRRT | 50% | 66.7% | 75% | 80% |
|  | Average L3 cell level RSRP difference (dBm) | 0.602 | 0.917 | 1.092 | 1.486 |
|  | Other optional KPIs (e.g., L1 beam level RSRP difference,) | N/A |  |  |  |

It is predictable that as the observation window expands, more information can be utilized for prediction. As MRRT grows, the information utilized for AI model predication diminishes. Hence, the RSRP difference is supposed to increase. And the simulation results also verify this trend as illustrated in Figure 2. It can be seen that as the MRRT increases, the RSRP difference increases which also lowers the prediction accuracy of the best cell.

Figure 2: RSRP difference and accuracy of the predicated best cell trend considering MRRT

Observation 1: The RSRP difference increases as the MRRT increases in Case 2: FR1 to FR1 intra-frequency temporal domain case B.

Observation 2: The prediction accuracy of the best cell decreases as the MRRT increases in Case 2: FR1 to FR1 intra-frequency temporal domain case B.

2.2	Case 4: FR2 to FR2 intra-frequency temporal domain case A

In the temporal domain case A, it is mainly employed to predict the measurement result of a cell in the future. Based on the predicted measurement result, some procedures, such as measurement events or RLF, can also be predicted. One of the advantages of this prediction is to enhance the handover  performance. For instance, the target cell can be predicted in advance according to the predicted measurement result, which is beneficial for facilitating network mobility control.

For AI models, neural networks based on long short-term memory (LSTM) have played a significant role in the field of Natural Language Processing. Additionally, they have been widely employed for sequence modeling. The reason LSTMs have been widely used for this purpose is that the model connects back to itself during a forward pass of the samples. Thus, it benefits from the context generated by previous predictions when predicting any new sample. For Case 4, LSTM model is utilized to predict the measurement result. In the Post email [3], it was agreed that both sliding L1/L3 filtering and non-sliding L1/L3 filtering options can be used for evaluation. In Case 4, sliding L1/L3 filtering is used for simulation.

In order to determine whether the prediction accuracy of different prediction points in a PW is consistent, we simulate the case 3N:3N (N represents a measurement period) for Case 4. One example is shown in Figure 3.

Figure 3: RSRP predication for Case 4 with observation window of 3 measurement periods  vs. prediction window of 3 measurement periods

The simulation results are provided in Table 3.

Table 3: Simulation report table for FR2 to FR2 intra-frequency temporal domain case A

| Report parameters |  | CATT |  |  |
| --- | --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |  |  |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 2 |  |  |
|  | UE speed (30,60,90,120 Km/h) | 60 |  |  |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | N/A |  |  |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | N/A |  |  |
|  | Whether LOSsoft is modeled or not | Yes |  |  |
|  | spatial consistency option (A or B) | Procedure A |  |  |
|  | Number of TX beams | 32 |  |  |
|  | Number of RX beams | 4 |  |  |
|  | Measurement reduction rate(50%,…Note2) | N/A |  |  |
|  | Observation window(Note3) | 240ms |  |  |
|  | Prediction window (Note3) | 240ms |  |  |
|  | Any other parameters (Note 4) | N/A |  |  |
| Data Size (Number of Samples) | Training/validity | 208656/23184 |  |  |
|  | Testing | 33810 |  |  |
| AI/ML model input/output | Model input (Note 5) | Cell level RSRP of all cells |  |  |
|  | Model output(Note 6) | Cell level RSRP of all cells |  |  |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM |  |  |
|  | Model complexity in a number of parameters(M) | 0.87M |  |  |
|  | Model complexity in model size (e.g. Mbyte) | 10.5MB |  |  |
|  | Computational complexity [FLOPs] | 1300.08K |  |  |
| Metrics | Prediction points index (m) | m = 1 | m = 2 | m = 3 |
|  | Average L3 cell level RSRP difference (dB) | 0.529 | 0.739 | 1.055 |
|  | Other optional KPIs (e.g., L1 beam level RSRP difference,) | N/A |  |  |

The separate RSRP difference for each prediction point index, which is equal to m times (measurement periods) (where 1 ≤ m ≤ M and m is an integer) within the prediction window, is shown in Figure 4. It can be observed that the RSRP difference increases as m increases. That is to say, the prediction time that is closer to the simulated results in the observation window shows a smaller RSRP difference. As m becomes larger, the RSRP difference increases. This indicates that as the prediction window increases, the prediction accuracy decreases.

Observation 3: As the prediction point index increases within the prediction window, the RSRP difference increases correspondingly.

OW vs. PW = 3N: 3N

Figure 4: RSRP difference trend in predication window

Under the same conditions, the average L3 cell level RSRP difference of case 3N: N is 0.405 (dB). It can be seen that the RSRP difference (0.529) of the first prediction point in case 3N: 3N is larger than that of case 3N: N. In other words, for the prediction window (PW), the longer the duration, the lower the prediction accuracy, and the RSRP difference for each prediction point worsens.

Observation 4: For the prediction window, the longer the duration, the lower the prediction accuracy and the RSRP difference for each prediction point deteriorates.

Conclusion

In this document, we shows the simulation results for Case 2 and Case 4 for RRM measurement, and it can be seen that:

Case 2: FR1 to FR1 intra-frequency temporal domain case B:

Observation 1: The RSRP difference increases as the MRRT increases in Case 2: FR1 to FR1 intra-frequency temporal domain case B.

Observation 2: The prediction accuracy of the best cell decreases as the MRRT increases in Case 2: FR1 to FR1 intra-frequency temporal domain case B.

Case 4: FR2 to FR2 intra-frequency temporal domain case A

Observation 3: As the prediction point index increases within the prediction window, the RSRP difference increases correspondingly.

Observation 4: For the prediction window, the longer the duration, the lower the prediction accuracy and the RSRP difference for each prediction point deteriorates.

Reference

- R2-2406202, RAN2#126 chairman notes;

- R2-2404102, RAN2#125bis chairman notes;

- R2-2407885, Summary of [POST127][030][AI mobility] RRM simulation assumptions (OPPO), Oppo