3GPP TSG-RAN WG2 Meeting #128	R2-2409655

Orlando, USA, November 18th – 22nd, 2024

Agenda item:	8.10.5

Source:	Samsung

Title:	Reporting failure cause for SDT

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss further on the reporting of failure causes for SDT.

# Discussion

In RAN2#126 [1], the below agreement was made

When SDT failure happens, the UE can indicate the failure cause of SDT to the network, e.g. T319a expiration. Details are TBD, e.g. if RSRP and data volume can also be included in such report.

In RAN2#127-bis [2], additional failure causes were deprioritised. However we can confirm that T319a expiration is logged by the UE during SDT failure since it was agreed in RAN2#126.

Proposal 1: UE logs if SDT failure is due toT319a expiration.

In RAN2#127-bis, there were following failure causes proposed by various companies.

1. T319 expiry

2. Random access problem

3. RLC max retransmission

4. CG-SDT timer

5. Integrity failure

6. Cell reselection

7. RRC setup/reject/release

Causes 1, 2, 3 was proposed by all companies which addressed the issue. So it may be straightforward to agree to report these causes.

Proposal 2: UE logs if SDT failure is due to Random access problem or RLC max retransmission.

# Conclusion

Based on the discussion, we have the following proposals:

Proposal 1: UE logs if SDT failure is due toT319a expiration.

Proposal 2: UE logs if SDT failure is due to Random access problem or RLC max retransmission.

# References

[1] R2-2406202: RAN2#126 Meeting Report

[2] RAN2#127-bis meeting report.