3GPP TSG-RAN WG2 Meeting #128	R2- 2409657

Orlando, USA, Oct 18th - Oct 22nd, 2024

Agenda Item:	8.6.3

Source:	MediaTek Inc.

Title:	Discussion on event triggered L1 MR

Document for:	Discussion, Decision

Introduction

In this contribution, the remaining details for event triggered L1 measurement report are discussed:

- The granularity of TTT and report (Beam/cell level)

- The measurement RS type alignment

- The report content

Discussion

Granularity of TTT and report

In the last meeting, the granularity and operation of TTT was discussed. The main focus was on whether TTT should be per beam or per cell. This is related to the topic of TTT operation during beam changes (whether to reset TTT or not) and the granularity of the report (per beam or per cell). Firstly, we would like to clarify the terminology of beam/cell level TTT:

- TTT per beam (beam level TTT): TTT is specific to one beam, so it resets if the beam index changes.

- TTT per cell (cell level TTT): TTT is cross-beam within a cell, so it does not reset if the beam index changes within a cell.

Proposal 1: RAN2 to clarify that beam/cell level TTT means TTT is reset/not reset upon beam changing (within a cell) during event evaluation.

For the beams of serving cell and candidate cell for event evaluation, RAN2 has the following agreement:

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. - Any beam in candidate RS configuration can be used for LTM event evaluation.

Serving cell event

For LTM 2 (serving cell event), only the current beam is considered for event evaluation, so there is only one report for the serving cell. If the beam level TTT is used and current beam changes, TTT will be reset and re-estimate the event for the new beam.

- If TTT is per beam-> TTT is reset if the current beam change -> lower triggering frequency->more stable report.

- If TTT is per cell-> TTT continues if the current beam change -> higher triggering frequency->less stable report.

| TTT per beam or per cell | Stability of each report | More/less likely to trigger | Report number |
| --- | --- | --- | --- |
| Per beam | High | Less likely | Only one (current beam) |
| Per cell | Low | More likely | Only one (current beam) |

Observation 1: For serving cell event (LTM 2), beam level TTT lead to lower triggering frequency and more stable report. Vise versa for cell level TTT.

Observation 2: For serving cell event (LTM 2), no matter beam/cell level TTT is used, there is only one report for serving cell.

Candidate cell event

For LTM 4 (candidate cell event), as agreed, any beam in candidate RS configuration can be used for LTM event evaluation. So multiple beams may be evaluated simultaneously.

- If TTT is per beam-> individual event evaluation for each beam -> multiple reports for one cell -> more stable for each report.

- If the TTT is per cell - > combined event evaluation for the candidate cell -> one report for one cell -> Less stable for each report.

| TTT per beam or per cell | Stability of each report | More/less likely to trigger for each report | Report number (if report) |
| --- | --- | --- | --- |
| Per beam | High | Less likely | More |
| Per cell | Low | More likely | Only one per cell |

Observation 3: For candidate cell event (LTM 4), beam level TTT lead to lower triggering frequency and more stable report. Vise versa for cell level TTT.

Observation 4: For candidate cell event (LTM 4), if beam level TTT is used, the report is per beam. If cell level TTT is used, the report is per cell.

Serving cell event + Candidate cell event

For LTM event like LTM3 or LTM5, both serving cell beams and candidate cell beams qualities are considered. Theoretically, 4 options are possible with beam/cell level TTT in the combination of serving and candidate cells. However, we believe a unified solution for TTT should be used for event evaluation between serving and candidate cells, i.e., TTT per beam or per cell for both. There is no reason to use different solutions (e.g., beam level TTT for serving while cell level for candidate) in one event evaluation procedure.

Proposal 2: For Serving+ Candidate cell event (LTM 3 or LTM 5), a unified solution for TTT should be used between serving and candidate cells, i.e., TTT per beam or per cell for both.

With this assumption, only beam/cell level TTT for both serving cell and candidate cell should be considered.

Summary

As LTM is a kind of cross cell beam switching, network need to know the specific quality of the indicated beam. beam level TTT is specific to one beam, which lead to more stable and convincible report. It may have more signalling overhead if multiple candidate beams are evaluated simultaneously, but we believe the benefits of providing more stable results for network judgment outweigh the drawbacks of signalling overhead. The comparison can be found in the following table:

Table 1 Comparison of beam/cell level TTT for serving/candidate cells

|  | For Serving cell |  | For Candidate cells |  |
| --- | --- | --- | --- | --- |
|  | Stability & Credibility | Overhead | Stability & Credibility | Overhead |
| Beam level TTT |  |  |  |  |
| Cell level TTT |  |  |  |  |

As for the signalling overhead for candidate beam with beam level TTT, it can be further limited by the CSI resource configuration from the network. It may also be beneficial to support multiple event report in one container (MAC-CE) to further reduce the overhead.

Proposal 3: The TTT is reset when the evaluated beam(s) change, no cross-beam TTT is introduced.

Proposal 4: The event triggered L1 MR is reported at beam level. Individual event is defined for each beam.

Measurement RS type alignment

In the last meeting, the RS type alignment was discussed and the following working assumption is made:

Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5). We will decide which option to go next meeting.

Based on the working assumption, the unified RS type for both serving cell and candidate cells should be chosen. The various options discussed at the last meeting can be summarized into these two options.

- Option1: Network only indicate one RS type (SSB or CSI-RS) for event evaluation. UE performs the L1 measurement for the indicated RS type of the serving beam and candidate beams.

- Option2: Two RS type (SSB + CSI-RS) can be configured simultaneously. It is up to UE to select the same RS type for serving cell and candidate cells for event evaluation.

For both option1 and option2, no matter SSB or CSI-RS is indicated or selected, the UE performs L1 measurement for the corresponding RS of the serving beam and the same RS type of candidate beams. It is not necessary for the UE to derive the RS quality (SSB) based on the measured result of associated QCLed with another RS type (CSI-RS).

Proposal 5: The same RS type is used for both serving beam and candidate beams for event evaluation. The UE does not estimate RS quality based on the measured result of associated QCLed RS with another RS type.

In L3 RRM, the network configures either SSB or CSI-RS. The both (SSB and CSI-RS) configuration is not provided in EventTriggerConfig. Also, Option 2 may not work well with the configured candidate RS resource set, which may not contain the corresponding RS resources or may have very few resources of the RS type that UE selects. Considering the configuration complexity and practicality, we suggest reusing L3 RRM and adopting Option 1, where the network configures only one RS type. (However, different RS type for different event evaluations should be allowed if they are configured simultaneously)

Proposal 6: Network only indicate one RS type (SSB or CSI-RS) for one event evaluation. (This does not preclude different RS types configured for different events.)

Report content

In the last meeting, the following information are agreed to be included in the event L1 MR MAC CE:

Basic information included in MR MAC CE: - Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id) - Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams - Triggered event information (e.g., ReportConfigID) MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not). N is configurable by NW. Additional information included in MR MAC CE: - The information and quantity of current beam, based on NW configuration.

Several components are still FFS and need discussion.

Single/multiple result for one beam

Considering event L1 MR is send after TTT expires, UE may perform several L1 measurement during TTT. In that case, there are two options on how to report the beam quantity:

- Option 1a: UE reports each measured result during the event evaluation period (e.g., TTT) of the RS.

- Option 1b: UE only reports one latest value for the measured result for the target RS.

In our understanding, the triggering of LTM depends on the latest measured beam quality and the event. It should be enough for network to make decision based on the reported latest measured beam quality and the event type. Reporting each measured results during TTT seems unnecessary and cause more signalling overhead.

Proposal 7: If multiple samples are measured for the target RS during the event evaluation period, UE only reports the latest measured result.

Beams haven’t fulfilled the event condition (including leaving condition)

- SpCell

In R18 LTM, the spCellInclusion is introduced to indicate the serving cell beam quality to the network, even if it hasn’t fulfilled the event condition. However, it is hard for network to indicate specific beam for UE to measure, and the selection of the serving cell beam is up to UE implementation.

In the last meeting, RAN2 agreed that the UE can include the current beam (configured by the network) but deferred the decision on including multiple beams for the serving cell. Our understanding is that this is unnecessary since UE-initiated beam reporting (NR_MIMO_Ph5) already covers this aspect. Event L1 MR should focus more on candidate cell beam reporting. However, for the one beam of the serving cell, it does not necessarily have to be the current beam; this can reuse the Rel-18 design and be left to UE implementation.

Proposal 8: Reuse Rel-18 spCellInclusion mechanism to leave UE implementation to choose the serving cell beam for reporting.

Proposal 9: Multiple serving cell beams reporting is not introduced.

2. 	Candidate cell

Allowing reporting on beams that have not fulfilled the event condition may mislead the purpose of reporting the correct beam and make the control signalling complicated to specify RS that the UE to report. The UE may report an RS that has not fulfilled the event condition, rather than the one that has fulfilled the event condition.

Moreover, this may introduce extra signalling overhead when no event condition is fulfilled, yet UE is required to report something. Therefore, we suggest focusing on beams that have fulfilled the event condition.

Proposal 10: UE is not required to report beams from candidate cell that have not fulfilled the event condition (including leaving condition).

Other information

The event triggered L1 MR aligns well with conditional LTM and may assist in informing UE decisions to the network. E.g., UE’s decision for LTM cell switch/early synchronization. It is recommended to study the inclusion of conditional LTM information in event-triggered L1 MR.

Proposal 11: RAN2 to further study whether the information of UE decisions in conditional LTM can be included in event triggered L1 MR.

Conclusion

In this contribution, the following observations and proposals are made:

Granularity of TTT and report

Proposal 1: RAN2 to clarify that beam/cell level TTT means TTT is reset/not reset upon beam changing (within a cell) during event evaluation.

Observation 1: For serving cell event (LTM 2), beam level TTT lead to lower triggering frequency and more stable report. Vise versa for cell level TTT.

Observation 2: For serving cell event (LTM 2), no matter beam/cell level TTT is used, there is only one report for serving cell.

Observation 3: For candidate cell event (LTM 4), beam level TTT lead to lower triggering frequency and more stable report. Vise versa for cell level TTT.

Observation 4: For candidate cell event (LTM 4), if beam level TTT is used, the report is per beam. If cell level TTT is used, the report is per cell.

Proposal 2: For Serving+ Candidate cell event (LTM 3 or LTM 5), a unified solution for TTT should be used between serving and candidate cells, i.e., TTT per beam or per cell for both.

Proposal 3: The TTT is reset when the evaluated beam(s) change, no cross-beam TTT is introduced.

Proposal 4: The event triggered L1 MR is reported at beam level. Individual event is defined for each beam.

Measurement RS type alignment

Proposal 5: The same RS type is used for both serving beam and candidate beams for event evaluation. The UE does not estimate RS quality based on the measured result of associated QCLed RS with another RS type.

Proposal 6: Network only indicate one RS type (SSB or CSI-RS) for one event evaluation. (This does not preclude different RS types configured for different events.)

Report content

Proposal 7: If multiple samples are measured for the target RS during the event evaluation period, UE only reports the latest measured result.

Proposal 8: Reuse Rel-18 spCellInclusion mechanism to leave UE implementation to choose the serving cell beam for reporting.

Proposal 9: Multiple serving cell beams reporting is not introduced.

Proposal 10: UE is not required to report beams from candidate cell that have not fulfilled the event condition (including leaving condition).

Proposal 11: RAN2 to further study whether the information of UE decisions in conditional LTM can be included in event triggered L1 MR.

References

- R2_127b_R18 SL_MOB_R19 NES_MOB_Session Notes_EoM

- R2_127_R18 SL_MOB_R19 NES_MOB_Session Notes (Kyeongin Jeong)_EoM

- RP-240299-Revised-WID-R19-Mobility-Enhancements